3GPP TS 25.102 V16.0.0 (2020-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

User Equipment (UE) radio transmission and reception (TDD)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	12

1	Scope	13

2	References	13

3	Definitions, symbols and abbreviations	13

3.1	Definitions	13

3.2	(void)	14

3.3	Abbreviations	14

4	General	16

4.1	Relationship between Minimum Requirements and Test Requirements	16

4.2	Power Classes	16

4.3	Control and monitoring functions	16

4.3.1	Minimum requirement	16

4.4	RF requirements in later releases	16

4.5	Applicability of requirements for MBSFN-only Ues	16

5	Frequency bands and channel arrangement	17

5.1	General	17

5.2	Frequency bands	17

5.3	TX-RX frequency separation	17

5.3.1	3.84 Mcps TDD Option	17

5.3.2	1.28 Mcps TDD Option	17

5.3.3	7.68 Mcps TDD Option	17

5.4	Channel arrangement	18

5.4.1	Channel spacing	18

5.4.1.1	3.84 Mcps TDD Option	18

5.4.1.2	1.28 Mcps TDD Option	18

5.4.1.3	7.68 Mcps TDD Option	18

5.4.2	Channel raster	18

5.4.2.1	3.84 Mcps TDD Option	18

5.4.3	Channel number	18

5.4.4	UARFCN	19

5.4.4.1	3.84 Mcps TDD Option	19

5.4.4.2	1.28 Mcps TDD Option	19

5.4.4.3	7.68 Mcps TDD Option	20

6	Transmitter characteristics	20

6.1	General	20

6.2	Transmit power	20

6.2.1	User Equipment maximum output power	20

6.2.1.1	3.84 Mcps TDD option	20

6.2.1.2	1.28 Mcps TDD option	21

6.2.1.3	7.68 Mcps TDD option	21

6.2.2	UE maximum output, power with E-DCH	22

6.2.2.1	3.84 Mcps TDD option	22

6.2.2.2	1.28 Mcps TDD option	22

6.2.2.3	7.68 Mcps TDD option	22

6.2.3	UE maximum output, power with multi-code	22

6.2.3.1	1.28 Mcps TDD option	22

6.3	UE frequency stability	23

6.3A	UE frequency stability for 1.28Mcps TDD MC-HSUPA	23

6.4	Output power dynamics	23

6.4.1	Power control	23

6.4.1.1	3.84 Mcps option	23

6.4.1.1.1	Initial Accuracy	23

6.4.1.1.2	Differential accuracy, controlled input	23

6.4.1.1.3	Differential accuracy, measured input	24

6.4.1.2	1.28 Mcps TDD Option	24

6.4.1.2.1	Open loop power control	24

6.4.1.2.2	Closed loop power control	24

6.4.1.3	7.68 Mcps option	25

6.4.1.3.1	Initial Accuracy	25

6.4.1.3.2	Differential accuracy, controlled input	25

6.4.1.3.3	Differential accuracy, measured input	26

6.4.2	Minimum output power	26

6.4.2.1	Minimum requirement	26

6.4.2.1.1	3.84 Mcps TDD Option	26

6.4.2.1.2	1.28 Mcps TDD Option	26

6.4.2.1.3	7.68 Mcps TDD Option	26

6.4.2.2	Additional requirement for 1.28Mcps TDD MC-HSUPA	26

6.4.3	Out-of-synchronisation handling of output power	26

6.4.3.1	Requirement for continuous transmission	27

6.4.3.1.1	3.84 Mcps TDD Option	27

6.4.3.1.2	1.28 Mcps TDD Option	28

6.4.3.1.3	7.68 Mcps TDD Option	30

6.4.3.2	Requirement for discontinuous transmission	31

6.4.3.2.1	3.84 Mcps TDD Option	31

6.4.3.2.2	1.28 Mcps TDD Option	33

6.4.3.2.3	7.68 Mcps TDD Option	35

6.5	Transmit ON/OFF power	37

6.5.1	Transmit OFF power	37

6.5.1.1	Minimum Requirement	37

6.5.1.2	Additional requirement for 1.28Mcps TDD MC-HSUPA	37

6.5.2	Transmit ON/OFF Time mask	37

6.5.2.1	Minimum Requirement	38

6.5.2.1.1	3.84 Mcps TDD Option	38

6.5.2.1.2	1.28 Mcps TDD Option	38

6.5.2.1.3	7.68 Mcps TDD Option	38

6.6	Output RF spectrum emissions	39

6.6.1	Occupied bandwidth	39

6.6.1.1	3.84 Mcps TDD Option	39

6.6.1.2	1.28 Mcps TDD Option	39

6.6.1.3	7.68 Mcps TDD Option	39

6.6.1A	Occupied bandwidth for 1.28Mcps TDD MC-HSUPA	39

6.6.2	Out of band emission	39

6.6.2.1	Spectrum emission mask	40

6.6.2.1.1	3.84 Mcps TDD Option	40

6.6.2.1.2	1.28 Mcps TDD Option	40

6.6.2.1.3	7.68 Mcps TDD Option	41

6.6.2.1A	Additional Spectrum emission mask for 1.28Mcps TDD MC-HSUPA	42

6.6.2.1A.1	Minimum requirement	42

6.6.2.2	Adjacent Channel Leakage power Ratio (ACLR)	42

6.6.2.2.1	Minimum requirement	43

6.6.2.2.2	Additional requirement for 1.28Mcps TDD MC-HSUPA	44

6.6.3	Spurious emissions	44

6.6.3.1	Minimum Requirement	44

6.6.3.1.1	3.84 Mcps TDD Option	44

6.6.3.1.2	1.28 Mcps TDD Option	45

6.6.3.1.3	7.68 Mcps TDD Option	46

6.7	Transmit intermodulation	47

6.7.1	Minimum requirement	47

6.7.1.1	3.84 Mcps TDD Option	47

6.7.1.2	1.28 Mcps TDD Option	48

6.7.1.3	7.68 Mcps TDD Option	48

6.7.2	Additional requirement for 1.28Mcps TDD MC-HSUPA	48

6.8	Transmit Modulation	48

6.8.1	Transmit pulse shape filter	49

6.8.2	Error Vector Magnitude	49

6.8.2.1	Minimum Requirement	49

6.8.2.2	Additional requirement for 1.28Mcps TDD MC-HSUPA	49

6.8.2A	In-band Emissions for 1.28Mcps TDD MC-HSUPA	49

6.8.3	Peak Code Domain Error	50

6.8.3.1	Minimum Requirement	50

7	Receiver characteristics	50

7.1	General	50

7.2	Diversity characteristics	51

7.3	Reference sensitivity level	51

7.3.1	Minimum Requirements	51

7.3.1.1	3.84 Mcps TDD Option	51

7.3.1.2	1.28 Mcps TDD Option	51

7.3.1.3	7.68 Mcps TDD Option	52

7.3.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option	52

7.4	Maximum input level	52

7.4.1	Minimum Requirements for DPCH reception	52

7.4.1.1	3.84 Mcps TDD Option	52

7.4.1.2	1.28 Mcps TDD Option	52

7.4.1.3	7.68 Mcps TDD Option	53

7.4.2	Minimum Requirements for HS-PDSCH reception	53

7.4.2.1	3.84 Mcps TDD Option	53

7.4.2.2	1.28 Mcps TDD Option	53

7.4.2.2.1 Minimum requirement for 16QAM	53

7.4.2.2.2	Minimum requirement for 64QAM	54

7.5	Adjacent Channel Selectivity (ACS)	54

7.5.1	Minimum Requirement	54

7.5.1.1	3.84 Mcps TDD Option	54

7.5.1.2	1.28 Mcps TDD Option	55

7.5.1.3	7.68 Mcps TDD Option	55

7.5.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option	56

7.6	Blocking characteristics	56

7.6.1	Minimum Requirement	56

7.6.1.1	3.84 Mcps TDD Option	56

7.6.1.2	1.28 Mcps TDD Option	58

7.6.1.3	7.68 Mcps TDD Option	59

7.6.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option	61

7.7	Spurious response	62

7.7.1	Minimum Requirement	62

7.7.1.1	3.84 Mcps TDD Option	62

7.7.1.2	1.28 Mcps TDD Option	62

7.7.1.3	7.68 Mcps TDD Option	62

7.7.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option	63

7.8	Intermodulation characteristics	63

7.8.1	Minimum Requirements	63

7.8.1.1	3.84 Mcps TDD Option	63

7.8.1.2	1.28 Mcps TDD Option	64

7.8.1.3	7.68 Mcps TDD Option	64

7.8.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option	65

7.9	Spurious emissions	65

7.9.1	Minimum Requirement	65

7.9.1.1	3.84 Mcps TDD Option	65

7.9.1.2	1.28 Mcps TDD Option	65

7.9.1.3	7.68 Mcps TDD Option	67

8	Performance requirement	67

8.1	General	67

8.2	Demodulation in static propagation conditions	68

8.2.1	Demodulation of DCH	68

8.2.1.1	Minimum requirement	68

8.2.1.1.1	3.84 Mcps TDD Option	68

8.2.1.1.2	1.28 Mcps TDD Option	68

8.2.1.1.3	7.68 Mcps TDD Option	69

8.3	Demodulation of DCH in multipath fading conditions	70

8.3.1	Multipath fading Case 1	70

8.3.1.1	Minimum requirement	70

8.3.1.1.1	3.84 Mcps TDD Option	70

8.3.1.1.2	1.28 Mcps TDD Option	71

8.3.1.1.3	7.68 Mcps TDD Option	71

8.3.2	Multipath fading Case 2	72

8.3.2.1	Minimum requirement	72

8.3.2.1.1	3.84 Mcps TDD Option	72

8.3.2.1.2	1.28 Mcps TDD Option	73

8.3.2.1.3	7.68 Mcps TDD Option	73

8.3.3	Multipath fading Case 3	74

8.3.3.1	Minimum requirement	74

8.3.3.1.1	3.84 Mcps TDD Option	74

8.3.3.1.2	1.28 Mcps TDD Option	75

8.3.3.1.3	7.68 Mcps TDD Option	75

8.3A	Demodulation of DCH in high speed train condition	76

8.3A.1	General	76

8.3A.2	Minimum requirement	76

8.3A.2.1	3.84 Mcps TDD Option	76

8.3A.2.2	1.28 Mcps TDD Option	76

8.3A.2.3	7.68 Mcps TDD Option	78

8.4	Base station transmit diversity mode for 3.84 Mcps TDD Option	78

8.4.1	Demodulation of BCH in SCTD mode	78

8.4.1.1	Minimum requirement	78

8.5	Power control in downlink	78

8.5.1	Power control in downlink, constant BLER target	78

8.5.1.1	Minimum requirements 3.84 Mcps TDD option	78

8.5.1.2	Minimum requirements 1.28 Mcps TDD option	79

8.5.2	Power control in downlink, wind up effects	80

8.5.2.1	Minimum requirements 3.84 Mcps TDD option	80

8.5.2.2	Minimum requirements 1.28 Mcps TDD option	80

8.5.3	Power control in the downlink, initial convergence	80

8.5.3.1	Minimum requirements 3.84 Mcps TDD option	80

8.5.3.2	Minimum requirements 1.28 Mcps TDD option	80

8.6	Uplink Power Control for 3.84 Mcps TDD Option	81

8.6.1	Test Conditions	81

8.6.2	Performance	82

8.7	Demodulation of DCH in moving conditions	82

8.7.1	Minimum requirement	82

8.7.1.1	3.84 Mcps TDD Option	82

8.7.1.2	1.28 Mcps TDD Option	83

8.7.1.3	7.68 Mcps TDD Option	83

8.8	Demodulation of DCH in birth-death conditions	83

8.8.1	Minimum requirement	83

8.8.1.1	3.84 Mcps TDD Option	83

8.8.1.2	1.28 Mcps TDD Option	84

8.8.1.3	7.68 Mcps TDD Option	84

9	Performance requirements (HSDPA)	84

9.1	Performance requirement for 3.84 Mcps TDD option	84

9.1.1	HS-DSCH throughput for fixed reference channels	84

9.1.1.1	Minimum requirement QPSK, Fixed Reference Channel, 7,3 Mbps - Category 8 - UE	85

9.1.1.2	Minimum requirement 16QAM, Fixed Reference Channel, 7,3 Mbps - Category 8 - UE	85

9.1.2	HS-DSCH throughput for Variable Reference Channels	86

9.1.2.1	Minimum requirement Variable Reference Channel, 7,3 Mbps - Category 8 - UE	86

9.1.3	Reporting of Channel Quality Indicator	87

9.1.3.1	Minimum requirement Channel Quality Indicator, 7,3 Mbps - Category 8 - UE	87

9.1.4	HS-SCCH Detection Performance	88

9.1.4.1	Minimum Requirements for HS-SCCH Detection	88

9.2	Performance requirements for 1.28 Mcps TDD option	89

9.2.1	HS-DSCH throughput for fixed reference channels	89

9.2.1.1	Category 1, 0.5Mbps UE class	89

9.2.1.2	Category 4, 1.1Mbps UE class	90

9.2.1.3	Category 7, 1.6Mbps UE class	91

9.2.1.4	Category 10, 2.2Mbps UE class	92

9.2.1.5	Category 13, 2.8Mbps UE class	93

9.2.1.6	Category 16-24	94

9.2.1.7	Category 25	95

9.2.1.8	Category 26	96

9.2.1.9	Category 27	97

9.2.1.10	Category 28	98

9.2.1.11	Category 29	99

9.2.1.12	Category 30	100

9.2.1A	HS-DSCH throughput for fixed reference channels for MU-MIMO	101

9.2.1A.1	Category 1-3	102

9.2.1A.2	Category 4-6	102

9.2.1A.3	Category 7-9	103

9.2.1A.4	Category 10-12	104

9.2.1A.5	Category 13-15	105

9.2.2	HS-DSCH throughput for Variable Reference Channels	106

9.2.2.1	Category 1, 0.5Mbps UE class	106

9.2.2.2	Category 4, 1.1Mbps UE class	107

9.2.2.3	Category 7, 1.6Mbps UE class	108

9.2.2.4	Category 10, 2.2 Mbps UE class	109

9.2.2.5	Category 13, 2.8 Mbps UE class	110

9.2.3	Reporting of Channel Quality Indicator	111

9.2.3.1	Minimum Requirement-UE categories 1-24	111

9.2.3.2	Minimum Requirement-UE categories 25-27	112

9.2.3.3	Minimum Requirement-UE categories 28-30	113

9.2.4	HS-SCCH Detection Performance	114

9.2.4.1	Minimum Requirements for HS-SCCH Type 1 Detection	114

9.2.4.2	Minimum Requirements for HS-SCCH Type 4/5 Detection	115

9.2.4.3	Minimum Requirements for HS-SCCH Type 6/7/8/9 Detection	116

9.2.5	PLCCH Detection Performance	117

9.2.5.1	Minimum Requirements	117

9.3	Performance requirement for 7.68 Mcps TDD option	118

9.3.1	HS-DSCH throughput for fixed reference channels	118

9.3.1.1	Minimum requirement QPSK, Fixed Reference Channel, 5,3 Mbps - Category 8 - UE	118

9.3.1.2	Minimum requirement 16 QAM, Fixed Reference Channel, 5,3 Mbps - Category 8 - UE	119

9.3.2	(void)	120

9.3.3	(void)	120

9.3.4	HS-SCCH Detection Performance	120

9.3.4.1	Minimum Requirements for HS-SCCH Detection	120

10	Performance requirements (MBMS)	121

10.1	Demodulation of MCCH	121

10.1.1	Minimum requirement	121

10.1.1.1	3.84 Mcps TDD Option	121

10.1.1.2	1.28 Mcps TDD Option	122

10.1.1.3	7.68 Mcps TDD Option	122

10.1.2	MBSFN capable UE	123

10.1.2.1	3.84 Mcps TDD Option	123

10.1.2.1.1	Non-IMB	123

10.1.2.1.2	IMB	123

10.1.2.2	1.28 Mcps TDD Option	124

10.1.2.3	7.68 Mcps TDD Option	125

10.2	Demodulation of MTCH	125

10.2.1	Minimum requirement	125

10.2.1.1	3.84 Mcps TDD Option	125

10.2.1.2	1.28 Mcps TDD Option	126

10.2.1.3	7.68 Mcps TDD Option	126

10.2.2	MBSFN capable UE	127

10.2.2.1	3.84 Mcps TDD Option	127

10.2.2.1.1	Non-IMB	127

10.2.2.1.2	IMB	127

10.2.2.2	1.28 Mcps TDD Option	128

10.2.2.3	7.68 Mcps TDD Option	128

10.2.3	MBSFN TDD & FDD same platform sharing	129

10.2.3.1	3.84 Mcps TDD Option	129

10.2.3.1.1	Non-IMB	129

10.2.3.1.2	IMB	130

10.2.3.2	(void)	130

10.2.3.3	7.68 Mcps TDD Option	130

10.3	Demodulation of MTCH and cell identification	131

10.3.1 	Minimum requirement	131

10.3.1.1	(void)	131

10.3.1.2	1.28 Mcps TDD Option	131

11	Performance requirement (E-DCH)	132

11.1	Detection of E-DCH HARQ ACK Indicator Channel (E-HICH)	132

11.1.1	Minimum requirement	132

11.1.1.1	3.84 Mcps TDD Option	132

11.1.1.2	1.28 Mcps TDD Option	132

11.1.1.3	7.68 Mcps TDD Option	133

11.2	Demodulation of E-DCH Absolute Grant Channel (E-AGCH)	133

11.2.1	Minimum requirement	133

11.2.1.1	3.84 Mcps TDD Option	133

11.2.1.2	1.28 Mcps TDD Option	134

11.2.1.3	7.68 Mcps TDD Option	135

12	Performance requirement under multiple-cell scenario	135

12.1	General	135

12.2	Demodulation of DCH in static propagation conditions	136

12.2.1	Minimum requirement	136

12.2.1.1	3.84 Mcps TDD Option	136

12.2.1.2	1.28 Mcps TDD Option	136

12.2.1.3	7.68 Mcps TDD Option	138

12.3	Demodulation of DCH in Multipath fading Case 1 conditions	139

12.3.1	Minimum requirement	139

12.3.1.1	3.84 Mcps TDD Option	139

12.3.1.2	1.28 Mcps TDD Option	139

12.2.1.3	7.68 Mcps TDD Option	141

12.4	Demodulation of DCH in Multipath fading Case 3 conditions	141

12.4.1	Minimum requirement	141

12.4.1.1	3.84 Mcps TDD Option	141

12.4.1.2	1.28 Mcps TDD Option	141

12.4.1.3	7.68 Mcps TDD Option	143

Annex A (normative):	Measurement channels	144

A.1	(void)	144

A.2	Reference measurement channel	144

A.2.1	UL reference measurement channel (12.2 kbps)	144

A.2.1.1	3.84 Mcps TDD Option	144

A.2.1.2	1.28 Mcps TDD Option	145

A.2.1.3	7.68 Mcps TDD Option	146

A.2.2	DL reference measurement channel (12.2 kbps)	147

A.2.2.1	3.84 Mcps TDD Option	147

A.2.2.2	1.28 Mcps TDD Option	148

A.2.2.3	7.68 Mcps TDD Option	149

A.2.3	DL reference measurement channel (64 kbps)	150

A.2.3.1	3.84 Mcps TDD Option	150

A.2.3.2	1.28 Mcps TDD Option	151

A.2.3.3	7.68 Mcps TDD Option	152

A.2.4	DL reference measurement channel (144 kbps)	154

A.2.4.1	3.84 Mcps TDD Option	154

A.2.4.2	1.28 Mcps TDD Option	156

A.2.4.3	7.68 Mcps TDD Option	157

A.2.5	DL reference measurement channel (384 kbps)	159

A.2.5.1	3.84 Mcps TDD Option	159

A.2.5.2	1.28 Mcps TDD Option	161

A.2.5.3	7.68 Mcps TDD Option	162

A.2.6	BCH reference measurement channel	163

A.2.6.1	3.84 Mcps TDD Option	163

A.2.6.2	1.28 Mcps TDD Option	164

A.2.6.3	7.68 Mcps TDD Option	164

A.2.7	UL multi code reference measurement channel (12.2 kbps)	165

A.2.7.1	3.84 Mcps TDD Option	165

A.2.7.2	1.28 Mcps TDD Option	166

A.2.7.3	7.68 Mcps TDD Option	167

A.2.8	DL reference measurement channel (2 Mbps)	169

A.2.8.1	3.84 Mcps TDD Option	169

A.2.8.2	1.28 Mcps TDD Option	171

A.2.8.3	7.68 Mcps TDD Option	172

A.2.9	DL reference measurement channel for MBSFN only Ues	173

A.2.9.1	3.84 Mcps TDD Option	173

A.2.9.1.1	Non-IMB	173

A.2.9.1.2 IMB	175

A.2.9.2	VOID	176

A.2.9.3	7.68 Mcps TDD Option	176

A.3	HSDPA reference measurement channels	178

A.3.1	HSDPA reference measurement channels for 3,84 Mcps TDD option	178

A.3.1.1	Reference measurement channels for 7,3 Mbps - Category 8 - UE	178

A.3.1.1.1	QPSK modulation scheme for test 1, 2, 3	178

A.3.1.1.2	QPSK modulation scheme for test 4	179

A.3.1.1.3	16QAM modulation scheme for test 1, 2, 3	180

A.3.1.1.4	16QAM modulation scheme for test 4	181

A.3.2	HSDPA reference measurement channels for 1.28 Mcps TDD option	182

A.3.2.1	Reference measurement channels for 0.5 Mbps UE class	182

A.3.2.1.1	QPSK modulation scheme	182

A.3.2.2	Reference measurement channels for 1.1 Mbps UE class	183

A.3.2.2.1	QPSK modulation scheme	183

A.3.2.2.2	16QAM modulation scheme	183

A.3.2.3	Reference measurement channels for 1.6 Mbps UE class	184

A.3.2.3.1	QPSK modulation scheme	184

A.3.2.3.2	16QAM modulation scheme	185

A.3.2.4	Reference measurement channels for 2.2 Mbps UE class	185

A.3.2.4.1	QPSK modulation scheme	185

A.3.2.4.2	16QAM modulation scheme	186

A.3.2.5	Reference measurement channels for 2.8 Mbps UE class	187

A.3.2.5.1	QPSK modulation scheme	187

A.3.2.5.2	16QAM modulation scheme	188

A.3.2.6	PLCCH reference measurement channel	189

A.3.2.7	 Reference measurement channels for Category 16-24 UE	190

A.3.2.7.1	Reference measurement channel for category 16-18 UE	190

A.3.2.7.2	Reference measurement channel for category 19-21 UE	191

A.3.2.7.3	Reference measurement channel for category 22-24 UE	192

A.3.2.8	Reference measurement channel of 48kbps	193

A.3.2.9	Void	193

A.3.2.10	Reference Measurement Channel for category 25 UE	194

A.3.2.10.1	QPSK modulation scheme	194

A.3.2.10.2	16QAM modulation scheme	195

A.3.2.11	Reference Measurement Channel for category 26 UE	197

A.3.2.11.1	QPSK modulation scheme	197

A.3.2.11.2	16QAM modulation scheme	198

A.3.2.12	Reference Measurement Channel for category 27 UE	200

A.3.2.12.1	QPSK modulation scheme	200

A.3.2.12.2	16QAM modulation scheme	202

A.3.2.13	Reference Measurement Channel for category 28 UE	204

A.3.2.13.1	64QAM modulation scheme	204

A.3.2.14	Reference Measurement Channel for category 29 UE	206

A.3.2.14.1	64QAM modulation scheme	206

A.3.2.15	Reference Measurement Channel for category 30 UE	208

A.3.2.15.1	64QAM modulation scheme	208

A.3.2A	HSDPA reference measurement channels for 7,68 Mcps TDD option	210

A.3.2A.1	Reference measurement channels for 5,3 Mbps - Category 8 - UE	210

A.3.2A.1.1	QPSK modulation scheme for test 1, 2, 3 & 4	210

A.3.2A.1.2	16QAM modulation scheme for test 1, 2, 3 & 4	211

A.3.3	Variable Reference Channel definition for 3,84 Mcps and 1,28 Mcps TDD options	212

A.3.4	HSDPA reference measurement channels for 1.28 Mcps TDD option for MU-MIMO	213

A.3.4.1	Reference measurement channels for category 1-3	213

A.3.4.1.1	QPSK modulation scheme	213

A.3.4.2	Reference measurement channels for category 4-6	214

A.3.4.2.1	QPSK modulation scheme	214

A.3.4.2.2	16QAM modulation scheme	215

A.3.4.3	Reference measurement channels for category 7-9	216

A.3.4.3.1	QPSK modulation scheme	216

A.3.4.3.2	16QAM modulation scheme	217

A.3.4.4	Reference measurement channels for category 10-12	218

A.3.4.4.1	QPSK modulation scheme	218

A.3.4.4.2	16QAM modulation scheme	219

A.3.4.5	Reference measurement channels for category 13-15	220

A.3.4.5.1	QPSK modulation scheme	220

A.3.4.5.2	16QAM modulation scheme	221

A.4	Downlink reference parameter for MBMS tests	222

A.4.1	MCCH	222

A.4.1.1	3.84 Mcps TDD Option	222

A.4.1.1.1	Non-IMB	222

A.4.1.1.2	IMB	222

A.4.1.2	1.28 Mcps TDD Option	223

A.4.1.3	7.68 Mcps TDD Option	224

A.4.2	MTCH	224

A.4.2.1	3.84 Mcps TDD Option	224

A.4.2.1.1	Non-IMB	224

A.4.2.1.2	IMB	225

A.4.2.2	1.28 Mcps TDD Option	226

A.4.2.3	7.68 Mcps TDD Option	227

A.5 HSUPA reference measurement channels for 1.28Mcps TDD option	227

A.5.1 Fixed reference channel 1(FRC1) for 16QM	227

A.5.2 Fixed reference channel for MC-HSUPA	228

Annex B (normative):	Propagation conditions	230

B.1	Static propagation condition	230

B.2	Multi-path fading propagation conditions	230

B.2.1	3.84 Mcps TDD Option	230

B.2.2	1.28 Mcps TDD Option	232

B.2.3	7.68 Mcps TDD Option	234

B.3	MIMO propagation conditions	236

B.3.1	3.84 Mcps TDD Option	236

B.3.2	1.28 Mcps TDD Option	236

B.3.2.1	MIMO Dual Stream Static Orthogonal Conditions	236

B.3.3	7.68 Mcps TDD Option	237

B.4	High speed train condition	237

B.5	Moving propagation conditions	238

B.6	Birth-Death propagation conditions	239

Annex C (normative):	Environmental conditions	240

C.1	General	240

C.2	Environmental requirements for the UE	240

C.2.1	Temperature	240

C.2.2	Voltage	240

C.2.3	Vibration	241

Annex D (informative):	Terminal capabilities (TDD)	242

Annex E (informative):	Change history	243

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

Z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

This document establishes the minimum RF characteristics of all three options of the TDD mode of UTRA. The three options are the 3.84 Mcps, 1.28 Mcps and 7.68 Mcps options respectively. The requirements are listed in different subsections only if the parameters deviate.

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI ETR 273-1-2: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement of radiated methods of measurement (using test sites) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[2]	3GPP TS 25.306: "UE Radio Access capabilities definition".

[3]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[4]	3GPP TS 25.307: "Requirements on User Equipments (Ues) supporting a release-independent frequency band".

[5]	3GPP TS 25.346: "Introduction of the Multimedia Broadcast/Multicast Service (MBMS) in the Radio Access Network (RAN)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following definitions apply:

Power Spectral Density: The units of Power Spectral Density (PSD) are extensively used in this document. PSD is a function of power versus frequency and when integrated across a given bandwidth, the function represents the mean power in such a bandwidth. When the mean power is normalised to (divided by) the chip-rate it represents the mean energy per chip. Some signals are directly defined in terms of energy per chip, (DPCH_Ec, Ec, and P-CCPCH_Ec) and others defined in terms of PSD (Io, Ioc, Ior and Îor). There also exist quantities that are a ratio of energy per chip to PSD (DPCH_Ec/Ior, Ec/Ior etc.). This is the common practice of relating energy magnitudes in communication systems.

It can be seen that if both energy magnitudes in the ratio are divided by time, the ratio is converted from an energy ratio to a power ratio, which is more useful from a measurement point of view. It follows that an energy per chip of X dBm/3.84 MHz (3.84 Mcps TDD option) or X dBm/1.28 MHz (1.28 Mcps TDD option) can be expressed as a mean power per chip of X dBm. Similarly, a signal PSD of Y dBm/3.84 MHz (3.84 Mcps TDD option) or Y dBm/1.28 MHz (1.28 Mcps TDD option) can be expressed as a signal power of Y dBm.

Maximum Output Power: This is a measure of the maximum power the UE can transmit (i.e. the actual power as would be measured assuming no measurement error) in a bandwidth of at least (1+  times the chip rate of the radio access mode. The period of measurement shall be a transmit timeslot excluding the guard period.

Mean Power: When applied to a CDMA modulated signal this is the power (transmitted or received) in a bandwidth of at least (1+  times the chip rate of the radio access mode. The period of measurement shall be a transmit timeslot excluding the guard period unless otherwise stated.

RRC Filtered Mean Power: The mean power as measured through a root raised cosine filter with roll-off factor  and a bandwidth equal to the chip rate of the radio access mode.

Nominal Maximum Output Power: This is the nominal power defined by the UE power class. The period of measurement shall be a transmit timeslot excluding the guard period.

Received Signal Code Power (RSCP): Given only signal power is received, the RRC filtered mean power of the received signal after despreading and combining.

Interference Signal Code Power (ISCP): Given only interference power is received, the RRC filtered mean power of the received signal after despreading to the code and combining. Equivalent to the RSCP value but now only interference is received instead of signal.

Multi-carrier reception: For 1.28Mcps TDD Option, it refers to the HS-DSCH reception on multiple carriers in a TTI for a UE. The assigned carriers for a UE should be contiguous.

Multi-carrier transmission: For 1.28Mcps TDD Option, it refers to transmission on multiple carriers simultaneously for a UE. The assigned carriers for a UE should be contiguous.

NOTE 1:	The RRC filtered mean power of a perfectly modulated CDMA signal is 0.246 dB lower than the mean power of the same signal.

NOTE 2:	The roll-off factor  is defined in section 6.8.1.

MBSFN-only UE:  A UE operable in receive mode only (for the purpose of MBSFN reception).

## 3.2	(void)

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACIR	Adjacent Channel Interference Ratio

ACLR	Adjacent Channel Leakage power Ratio

ACS	Adjacent Channel Selectivity

BS	Base Station

BER	Bit Error Ratio

BLER	Block Error Ratio

CQI	Channel Quality Indicator

CW	Continuous wave (unmodulated signal)

DL	Down link (forward link)

DTX	Discontinuous Transmission

DPCH	Dedicated physical channel

DPCH_Ec	Average energy per PN chip for DPCH

The ratio of the average energy per PN chip of the DPCH to the total transmit power spectral density of the downlink at the BS antenna connector

The ratio of the sum of DPCH_Ec for one service in case of multicode to the total transmit power spectral density of the downlink at the BS antenna connector

E-DCH	Enhanced Dedicated Channel

E-AGCH	E-DCH Absolute Grant Channel

E-HICH	E-DCH HARQ ACK Indicator Channel

EIRP	Effective Isotropic Radiated Power

FDD	Frequency Division Duplexing

FER	Frame Error Ratio

Fuw	Frequency of unwanted signal. This is specified in bracket in terms of an absolute frequency(s) or frequency offset from the assigned channel frequency. For multi-carrier reception of 1.28Mcps TDD Option, negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used.

Hybrid ARQ	Hybrid Automatic Repeat reQuest

HSDPA	High Speed Downlink Packet Access

HS-DSCH	High Speed Downlink Shared Channel

HS-PDSCH	High Speed Physical Downlink Shared Channel

HS-SCCH	High Speed Shared Control Channel

IMB	Integrated Mobile Broadcast

Ioc	The power spectral density (integrated in a noise bandwidth equal to the chip rate and normalized to the chip rate) of a band limited white noise source (simulating interference from other cells) as measured at the UE antenna connector. For multi-carrier reception of 1.28Mcps TDD Option, Ioc is defined for each of the carrier individually and is assumed to be equal for all carriers unless explicitly stated per carrier.

Ior	The total transmit power spectral density (integrated in a bandwidth of (1+α) times the chip rate and normalized to the chip rate) of the downlink signal at the BS antenna connector. For multi-carrier reception of 1.28Mcps TDD Option, Ior is defined for each of the carrier individually and is assumed to be equal for all carriers unless explicitly stated per carrier.

The received power spectral density (integrated in a bandwidth of (1+α) times the chip rate and normalized to the chip rate) of the downlink signal as measured at the UE antenna connector. For multi-carrier reception of 1.28Mcps TDD Option,  is defined for each of the carrier individually and is assumed to be equal for all carriers unless explicitly stated per carrier.

MBMS	Multimedia Broadcast and Multicast Service

MBSFN         	MBMS over a Single Frequency Network

MC-HSDPA	Multi-carrier HSDPA

MC-HSUPA	Multi-carrier HSUPA

MCCH	MBMS point-to-multipoint Control Channel

MTCH	MBMS point-to-multipoint Traffic Channel

Orthogonal Channel Noise Simulator, a mechanism used to simulate the users or control signals on the other orthogonal channels of a downlink link.

P-CCPCH	Primary Common Control Physical Channel

PCH	Paging Channel

PPM	Parts Per Million

RACH	Random Access Channel

RSSI	Received Signal Strength Indicator

R	Number of information bits per second excluding CRC bits successfully received on HS-DSCH by a HSDPA capable UE.

RU	Resource Unit

SCTD	Space Code Transmit Diversity

SIR	Signal to Interference ratio

TDD	Time Division Duplexing

TPC	Transmit Power Control

UE	User Equipment

UL	Up link (reverse link)

UTRA	UMTS Terrestrial Radio Access

ΔFOOB	Δ Frequency of Out Of Band emission

# 4	General

## 4.1	Relationship between Minimum Requirements and Test Requirements

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification 34.122 Annex F defines Test Tolerances. These Test Tolerances are individually calculated for each test. The Test Tolerances are used to relax the Minimum Requirements in this specification to create Test Requirements. The measurement results returned by the test system are compared - without any modifications - against the Test Requirements as defined by the shared risk principle.

The Shared Risk principle is defined in ETR 273 Part 1 sub-part 2 section 6.5.

## 4.2	Power Classes

For UE power classes 1 and 4, a number of RF parameter are not specified. It is intended that these are part of a later release.

## 4.3	Control and monitoring functions

This requirement verifies that the control and monitoring functions of the UE prevent it from transmitting if no acceptable cell can be found by the UE.

### 4.3.1	Minimum requirement

The power of the UE, as measured with a thermal detector, shall not exceed -30dBm if no acceptable cell can be found by the UE.

## 4.4	RF requirements in later releases

The standardisation of new frequency bands may be independent of a release. However, in order to implement a UE that conforms to a particular release but supports a band of operation that is specified in a later release, it is necessary to specify some extra requirements. TS 25.307 [4] specifies requirements on Ues supporting a frequency band that is independent of release.

NOTE:	For terminals conforming to the 3GPP release of the present document, some RF requirements in later releases may be mandatory independent of whether the UE supports the bands specified in later releases or not. The set of requirements from later releases that is also mandatory for Ues conforming to the 3GPP release of the present document is determined by regional regulation.

## 4.5	Applicability of requirements for MBSFN-only Ues

Only relevant sections are applicable to MBSFN-only UE operation (which also includes IMB [5]). Furthermore, for the case of IMB, only the 3.84Mcps TDD option shall apply.

# 5	Frequency bands and channel arrangement

## 5.1	General

The information presented in this section is based on the chip rates of 3.84 Mcps Option, 1.28 Mcps Option and 7.68 Mcps Option.

NOTE:	Other chip rates may be considered in future releases.

## 5.2	Frequency bands

UTRA/TDD is designed to operate in the following bands;

a)	1900 - 1920 MHz:	Uplink and downlink transmission
2010 - 2025 MHz	Uplink and downlink transmission

b)	1850 - 1910 MHz:	Uplink and downlink transmission
1930 - 1990 MHz:	Uplink and downlink transmission

c)	1910 - 1930 MHz:	Uplink and downlink transmission

d)	2570 - 2620 MHz: Uplink and downlink transmission

e)   2300—2400 MHz: Uplink and downlink transmission

f)	1880 - 1920 MHz: Uplink and downlink transmission

Note 1:　Deployment in existing or other frequency bands is not precluded.

Note 2:　In China, Band a only includes 2010 - 2025 MHz for 1.28 Mcps TDD option.

## 5.3	TX-RX frequency separation

### 5.3.1	3.84 Mcps TDD Option

No TX-RX frequency separation is required as Time Division Duplex (TDD) is employed. Each TDMA frame consists of 15 timeslots where each timeslot can be allocated to either transmit or receive.

### 5.3.2	1.28 Mcps TDD Option

No TX-RX frequency separation is required as Time Division Duplex (TDD) is employed. Each subframe consists of 7 main timeslots where all main timeslots (at least the first one) before the single switching point are allocated DL and all main timeslots (at least the last one) after the single switching point are allocated UL.

### 5.3.3	7.68 Mcps TDD Option

No TX-RX frequency separation is required as Time Division Duplex (TDD) is employed. Each TDMA frame consists of 15 timeslots where each timeslot can be allocated to either transmit or receive.

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

#### 5.4.1.1	3.84 Mcps TDD Option

The nominal channel spacing is 5 MHz, but this can be adjusted to optimise performance in a particular deployment scenario.

#### 5.4.1.2	1.28 Mcps TDD Option

The nominal channel spacing is 1.6 MHz, but this can be adjusted to optimise performance in a particular deployment scenario.

#### 5.4.1.3	7.68 Mcps TDD Option

The nominal channel spacing is 10 MHz, but this can be adjusted to optimise performance in a particular deployment scenario.

### 5.4.2	Channel raster

The channel raster is 200 kHz for all bands, which means that the carrier frequency must be a multiple of 200 kHz.

#### 5.4.2.1	3.84 Mcps TDD Option

In addition a number of additional centre frequencies are specified according to table 5.1, which means that the centre frequencies for these channels are shifted 100 kHz relative to the general raster.

### 5.4.3	Channel number

The carrier frequency is designated by the UTRA absolute radio frequency channel number (UARFCN). The value of the UARFCN in the IMT2000 band is defined in the general case as follows:

Nt = 5*F 	0.0 MHz  F 3276.6 MHz

where F is the carrier frequency in MHz

Additional channels applicable to operation in the frequency band defined in sub-clause 5.2(d) are defined via the following UARFCN definition:

Nt = 5 * (F - 2150.1 MHz)	2572.5 MHz  F 2617.5 MHz

### 5.4.4	UARFCN

#### 5.4.4.1	3.84 Mcps TDD Option

The following UARFCN range shall be supported for each band:

- Table 5.1: UTRA Absolute Radio Frequency Channel Number 3.84 Mcps TDD Option

| Frequency Band | Frequency Range | UARFCN Uplink and Downlink transmission | Additional UARFCN Uplink and Downlink transmission |
| --- | --- | --- | --- |
| For operation in frequency band as defined in subclause 5.2 (a) | 1900-1920 MHz 2010-2025 MHz | 9512 to 9588 10062 to 10113 | - |
| For operation in frequency band as defined in subclause 5.2 (b) | 1850-1910 MHz 1930-1990 MHz | 9262 to 9538 9662 to 9938 | - |
| For operation in frequency band as defined in subclause 5.2 (c) | 1910-1930 MHz | 9562 to 9638 | - |
| For operation in frequency band as defined in subclause 5.2 (d) | 2570-2620 MHz | 12862 to 13088 | 2112, 2137, 2162, 2187, 2212, 2237, 2262, 2287, 2312, 2337 |

#### 5.4.4.2	1.28 Mcps TDD Option

The following UARFCN range shall be supported for each band:

- Table 5.2: UTRA Absolute Radio Frequency Channel Number 1.28 Mcps TDD Option

| Frequency Band | Frequency Range | UARFCN Uplink and Downlink transmission |
| --- | --- | --- |
| For operation in frequency band as defined in subclause 5.2 (a) | 1900-1920 MHz 2010-2025 MHz | 9504 to 9596 10054 to 10121 |
| For operation in frequency band as defined in subclause 5.2 (b) | 1850-1910 MHz 1930-1990 MHz | 9254 to 9546 9654 to 9946 |
| For operation in frequency band as defined in subclause 5.2 (c) | 1910-1930 MHz | 9554 to 9646 |
| For operation in frequency band as defined in subclause 5.2 (d) | 2570-2620 MHz | 12854 to 13096 |
| For operation in frequency band as defined in subclause 5.2 (e) | 2300-2400 MHz | 11504 to 11996 |
| For operation in frequency band as defined in subclause 5.2 (f) | 1880-1920 MHz | 9404 to 9596 |

#### 5.4.4.3	7.68 Mcps TDD Option

The following UARFCN range shall be supported for each band:

- Table 5.3: UTRA Absolute Radio Frequency Channel Number 7.68 Mcps TDD Option

| Frequency Band | Frequency Range | UARFCN Uplink and Downlink transmission | Additional UARFCN Uplink and Downlink transmission |
| --- | --- | --- | --- |
| For operation in frequency band as defined in subclause 5.2 (a) | 1900-1920 MHz 2010-2025 MHz | 9512 to 9588 10062 to 10113 | - |
| For operation in frequency band as defined in subclause 5.2 (b) | 1850-1910 MHz 1930-1990 MHz | 9262 to 9538 9662 to 9938 | - |
| For operation in frequency band as defined in subclause 5.2 (c) | 1910-1930 MHz | 9562 to 9638 | - |
| For operation in frequency band as defined in subclause 5.2 (d) | 2570-2620 MHz | 12874 to 13076 | - |

# 6	Transmitter characteristics

## 6.1	General

Unless detailed the transmitter characteristic are specified at the antenna connector of the UE. For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed. Transmitter characteristics for UE(s) with multiple antennas/antenna connectors are FFS. For 1.28Mcps TDD MIMO capable Ues, transmitter characteristics are specified at each of the two antenna connectors,

The UE antenna performance has a significant impact on system performance and minimum requirements on the antenna efficiency are therefore intended to be included in future versions of this specification. It is recognised that different requirements and test methods are likely to be required for the different types of UE.

All the parameters in section 6 are defined using the UL reference measurement channel (12.2 kbps) specified in Annex A.2.1 unless explicitly stated otherwise.

For 1.28Mcps TDD, Ues supporting MC-HSUPA shall support both minimum requirements, as well as additional requirements for MC-HSUPA.

For the additional requirements for MC-HSUPA, all the parameters in clause 6 are defined using the UL E-DCH reference measurement channel, specified in subclause A.5.2. For the additional requirements for MC-HSUPA, the spacing of the adjacent carrier frequencies shall be 1.6 MHz.

## 6.2	Transmit power

### 6.2.1	User Equipment maximum output power

The nominal maximum output power defined is the broadband transmit power of the UE, i.e. the power in a bandwidth of at least (1+α) times the chip rate of the radio access mode. The period of measurement shall be a transmit timeslot excluding the guard period.

#### 6.2.1.1	3.84 Mcps TDD option

The power classes in Table 6.1 define the nominal maximum output power for 3.84 Mcps TDD options.

- Table 6.1: UE power classes

| Power Class | Nominal maximum output power | Tolerance |
| --- | --- | --- |
| 1 | +30 dBm | +1 dB / -3 dB |
| 2 | +24 dBm | +1 dB / -3 dB |
| 3 | +21 dBm | +2 dB / -2 dB |
| 4 | +10 dBm | +4 dB / -4 dB |

- NOTE:

1)	For multi-code operation the nominal maximum output power will be reduced by the difference of peak to average ratio between single and multi-code transmission.

2)	The tolerance allowed for the nominal maximum power applies even at the multi code transmission mode.

3)	For UE using directive antennas for transmission, a class dependent limit will be placed on the maximum EIRP (Equivalent Isotropic Radiated Power).

#### 6.2.1.2	1.28 Mcps TDD option

The power classes in Table 6.2 define the nominal maximum output power for 1.28 Mcps TDD option. For MC-HSUPA, the nominal transmit power is defined by the sum of the broadband transmit power of each carrier in the UE.

- Table 6.2: UE power classes for 1.28 Mcps TDD

| Power Class | Nominal maximum output power | Tolerance |
| --- | --- | --- |
| 1 | +33 dBm | +1 dB / -3 dB |
| 2 | +24 dBm | +1 dB / -3 dB |
| 3 | +21 dBm | +2 dB / -2 dB |
| 4 | +27 dBm | +1 dB / -3 dB |

NOTE 1:	The tolerance allowed for the nominal maximum power applies even at the multi code transmission mode.

NOTE 2:	For UE using directive antennas for transmission, a class dependent limit will be placed on the maximum EIRP (Equivalent Isotropic Radiated Power).

NOTE3:	For multi-carrier transmission, the nominal maximum output power will be reduced by the corresponding cubic metric value.

#### 6.2.1.3	7.68 Mcps TDD option

The power classes in Table 6.1 define the nominal maximum output power for 7.68 Mcps TDD options.

- Table 6.3: UE power classes

| Power Class | Nominal maximum output power | Tolerance |
| --- | --- | --- |
| 1 | +30 dBm | +1 dB / -3 dB |
| 2 | +24 dBm | +1 dB / -3 dB |
| 3 | +21 dBm | +2 dB / -2 dB |
| 4 | +10 dBm | +4 dB / -4 dB |

NOTE 1:		For multi-code operation the nominal maximum output power will be reduced by the difference of peak to average ratio between single and multi-code transmission.

NOTE 2:	The tolerance allowed for the nominal maximum power applies even at the multi code transmission mode.

NOTE 3:	For UE using directive antennas for transmission, a class dependent limit will be placed on the maximum EIRP (Equivalent Isotropic Radiated Power).

### 6.2.2	UE maximum output, power with E-DCH

#### 6.2.2.1	3.84 Mcps TDD option

[FFS]

#### 6.2.2.2	1.28 Mcps TDD option

The Maximum Power Reduction (MPR) for the nominal maximum output power defined in 6.2 is specified in table 6.4.

- Table 6.4  UE maximum output power with E-DCH

| UE transmit channel configuration | CM (dB) | MPR (dB) |
| --- | --- | --- |
| E-DCH and E-UCCH | 0  CM  1.5 | CM |

- Where Cubic Metric (CM) is based on the UE transmit channel configuration and is given by

CM = CEIL{[20 * log10 ((v_norm 3) rms) - 20 * log10 ((v_norm_ref 3) rms)] / k,0.5}

Where

-	 CEIL{X,0.5}means rounding upwards to closest 0.5dB, i.e. CM[0, 0.5, 1, 1,5]

-	v_norm is the normalized voltage waveform of the input signal

-	v_norm_ref is the normalized voltage waveform of the reference signal (12.2 kbps AMR Speech)

-    k is 1.94

-	20 * log10 ((v_norm_ref 3) rms) = 1.22 dB

#### 6.2.2.3	7.68 Mcps TDD option

[FFS]

### 6.2.3	UE maximum output, power with multi-code

#### 6.2.3.1	1.28 Mcps TDD option

The Maximum Power Reduction (MPR) for the nominal maximum output power defined in 6.2 is specified in table 6.2C.

- Table 6.2C  UE maximum output power with multi-code

| UE transmit channel configuration | CM (dB) | MPR (dB) |
| --- | --- | --- |
| For some combinations of; DPCH and HS-SICH/DPCH | 0  CM  2.5 | CM |

- Where Cubic Metric (CM) is based on the UE transmit channel configuration and is given by

CM = CEIL{[20 * log10 ((v_norm 3) rms) - 20 * log10 ((v_norm_ref 3) rms)] / k, 0.5}

Where

-   CEIL{X, 0.5} means rounding upwards to closest 0.5dB, i.e. CM [0, 0.5, 1, 1.5, 2, 2.5]

-	v_norm is the normalized voltage waveform of the input signal

-	v_norm_ref is the normalized voltage waveform of the reference signal (12.2 kbps AMR Speech)

-    k is 1.68

-	20 * log10 ((v_norm_ref 3) rms) = 1.22 dB

## 6.3	UE frequency stability

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of one timeslot compared to carrier frequency received from the BS. These signals will have an apparent error due to BS frequency error and Doppler shift. In the later case, signals from the BS must be averaged over sufficient time that errors due to noise or interference are allowed for within the above ±0.1PPM figure. The UE shall use the same frequency source for both RF frequency generation and the chip clock.

## 6.3A	UE frequency stability for 1.28Mcps TDD MC-HSUPA

For multi-carrier transmission, the UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of one timeslot compared to the average of the carrier frequencies received from the BS. These signals will have an apparent error due to BS frequency error and Doppler shift. In the later case, signals from the BS must be averaged over sufficient time that errors due to noise or interference are allowed for within the above ±0.1PPM figure. The UE shall use the same frequency source for both RF frequency generation and the chip clock.

## 6.4	Output power dynamics

Power control is used to limit the interference level.

### 6.4.1	Power control

#### 6.4.1.1	3.84 Mcps option

Uplink power control is the ability of the UE transmitter to sets its output power in accordance with measured downlink path loss, values determined by higher layer signalling and path loss weighting parameter  as defined in TS 25.331. The output power is defined as the RRC filtered mean power of the transmit timeslot.

##### 6.4.1.1.1	Initial Accuracy

The UE power control initial accuracy error shall be less than +/-9dB under normal conditions and +/- 12dB under extreme conditions.

##### 6.4.1.1.2	Differential accuracy, controlled input

The power control differential accuracy, controlled input, is defined as the error in the UE transmitter power step as a result of a step in SIRTARGET when the path loss weighting parameter =0. The step in SIRTARGET shall be rounded to the closest integer dB value. The power control error resulting from a change in IBTS or DPCH Constant Value shall not exceed the values defined in Table 6.3.

- Table 6.3: Transmitter power step tolerance as a result of control power step

| SIRTARGET [dB] | Transmitter power step tolerance [dB] |
| --- | --- |
| SIRTARGET  1 |  0.5 |
| 1 < SIRTARGET  2 |  1 |
| 2 < SIRTARGET  3 |  1.5 |
| 3 < SIRTARGET  10 |  2 |
| 10 < SIRTARGET  20 |  4 |
| 20 < SIRTARGET  30 |  6 |
| 30 < SIRTARGET |  9 (note 1) |
| NOTE 1: Value is given for normal conditions. For extreme conditions value is 12 |  |

##### 6.4.1.1.3	Differential accuracy, measured input

The power control differential accuracy, measured input, is defined as the error in UE transmitter power step change as a result of a step change in path loss LPCCPCH.

The error shall not exceed the sum of the following two errors:

-	The power control error, resulting from a change in the path loss (LPCCPCH), the same tolerances as defined in table 6.3 shall apply,

-	and the errors in the PCCPCH RSCP measurement as defined in TS 25.123.

#### 6.4.1.2	1.28 Mcps TDD Option

##### 6.4.1.2.1	Open loop power control

Open loop power control is the ability of the UE transmitter to sets its output power to a specific value. The open loop power control tolerance is given in Table 6.3A

6.4.1.2.1.1	Minimum requirement

The UE open loop power is defined as the RRC filtered mean power in a timeslot or ON power duration, whichever is available.

- Table 6.3A: Open loop power control tolerance

| Normal conditions | ± 9 dB |
| --- | --- |
| Extreme conditions | ± 12 dB |

6.4.1.2.1.2	Additional requirement for MC-HSUPA

The open loop power control tolerance per carrier is given in Table 6.3A.

##### 6.4.1.2.2	Closed loop power control

Closed loop power control in the Uplink is the ability of the UE transmitter to adjust its output power in accordance with one or more TPC commands received in the downlink.

6.4.1.2.2.1	Power control steps

The power control step is the change in the UE transmitter output power in response to a single TPC command, TPC_cmd, arrived at the UE.

# 6.4.1.2.2.1.1	Minimum requirement

The UE transmitter shall have the capability of changing the output power with a step size of 1, 2 and 3 dB according to the value of TPC or RP-TPC, in the slot immediately after the TPC_cmd can be arrived.

a)	The transmitter output power step due to closed loop power control shall be within the range shown in Table 6.3B.

b)	The transmitter average output power step due to closed loop power control shall be within the range shown in Table 6.3C. Here a TPC_cmd group is a set of TPC_cmd values derived from a corresponding sequence of TPC commands of the same duration.

The closed loop power is defined as the relative power differences between RRC filtered mean power of original (reference) timeslot and RRC filtered mean power of the target timeslot without transient duration.

- Table 6.3B: Transmitter power control range

| TPC_cmd | Transmitter power control range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 dB step size |  | 2 dB step size |  | 3 dB step size |  |
|  | Lower | Upper | Lower | Upper | Lower | Upper |
| Up | +0.5 dB | +1.5 dB | +1 dB | +3 dB | +1.5 dB | +4.5 dB |
| Down | -0.5 dB | -1.5 dB | -1 dB | -3 dB | -1.5 dB | -4.5 dB |

- Table 6.3C: Transmitter average power control range

| TPC_cmd group | Transmitter power control range after 10 equal TPC_ cmd groups |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 dB step size |  | 2 dB step size |  | 3 dB step size |  |
|  | Lower | Upper | Lower | Upper | Lower | Upper |
| Up | +8 dB | +12 dB | +16 dB | +24 dB | +24 dB | +36 dB |
| Down | -8 dB | -12 dB | -16 dB | -24 dB | -24 dB | -36 dB |

The UE shall meet the above requirements for inner loop power control over the power range bounded by the Minimum output power as defined in subclause 6.4.3, and the Maximum output power supported by the UE (i.e. the actual power as would be measured assuming no measurement error). This power shall be in the range specified for the power class of the UE in subclause 6.2.1.

# 6.4.1.2.2.1.2	Additional requirement for MC-HSUPA

The UE transmitter shall have the capability of changing the output power with a step size of 1, 2 and 3 dB according to the value of TPC or RP-TPC, in the slot immediately after the TPC_cmd for the corresponding carrier can be arrived.

a)	The transmitter output power step due to closed loop power control in each assigned carrier in the uplink shall be within the range shown in Table 6.3B, when the total transmit power in each of the assigned carriers is equal to each other.

b)	The transmitter average output power step due to closed loop power control in each assigned carrier in the uplink shall be within the range shown in Table 6.3C, when the total transmit power in each of the assigned carriers is equal to each other. Here a TPC_cmd group is a set of TPC_cmd values derived from a corresponding sequence of TPC commands of the same duration.

c)	The requirements can be tested by sending the same TPC commands for each of the assigned carriers, assuming that the signal powers for the carriers (in terms of total power) have been aligned prior to the beginning of the test procedure.

The closed loop power is defined as the relative power differences between RRC filtered mean power of original (reference) timeslot and RRC filtered mean power of the target timeslot without transient duration.

#### 6.4.1.3	7.68 Mcps option

Uplink power control is the ability of the UE transmitter to sets its output power in accordance with measured downlink path loss, values determined by higher layer signalling and path loss weighting parameter  as defined in TS 25.331. The output power is defined as the RRC filtered mean power of the transmit timeslot.

##### 6.4.1.3.1	Initial Accuracy

The UE power control initial accuracy error shall be less than +/-9dB under normal conditions and +/- 12dB under extreme conditions.

##### 6.4.1.3.2	Differential accuracy, controlled input

The power control differential accuracy, controlled input, is defined as the error in the UE transmitter power step as a result of a step in SIRTARGET when the path loss weighting parameter =0. The step in SIRTARGET shall be rounded to the closest integer dB value. The power control error resulting from a change in IBTS or DPCH Constant Value shall not exceed the values defined in Table 6.3D.

- Table 6.3D: Transmitter power step tolerance as a result of control power step

| SIRTARGET [dB] | Transmitter power step tolerance [dB] |
| --- | --- |
| SIRTARGET  1 |  0.5 |
| 1 < SIRTARGET  2 |  1 |
| 2 < SIRTARGET  3 |  1.5 |
| 3 < SIRTARGET  10 |  2 |
| 10 < SIRTARGET  20 |  4 |
| 20 < SIRTARGET  30 |  6 |
| 30 < SIRTARGET |  9 (1) |
| Note 1: Value is given for normal conditions. For extreme conditions value is 12 |  |

##### 6.4.1.3.3	Differential accuracy, measured input

The power control differential accuracy, measured input, is defined as the error in UE transmitter power step change as a result of a step change in path loss LPCCPCH.

The error shall not exceed the sum of the following two errors:

-	The power control error, resulting from a change in the path loss (LPCCPCH), the same tolerances as defined in table 6.3 shall apply,

-	and the errors in the PCCPCH RSCP measurement as defined in TS 25.123.

### 6.4.2	Minimum output power

The minimum controlled output power of the UE is when the power is set to a minimum value.

#### 6.4.2.1	Minimum requirement

##### 6.4.2.1.1	3.84 Mcps TDD Option

The minimum output power is defined as the mean power in one time slot excluding the guard period. The minimum output power shall be less than -44 dBm.

##### 6.4.2.1.2	1.28 Mcps TDD Option

The minimum output power is defined as the mean power in one time slot excluding the guard period. The minimum output power shall be less than -49 dBm.

##### 6.4.2.1.3	7.68 Mcps TDD Option

The minimum output power is defined as the mean power in one time slot excluding the guard period. The minimum output power shall be less than -41 dBm.

#### 6.4.2.2	Additional requirement for 1.28Mcps TDD MC-HSUPA

The minimum output power is defined as the mean power in one time slot in each carrier excluding the guard period. The minimum output power in each carrier shall be less than -49 dBm, when the transmissions in all carriers are set to minimum output power.

### 6.4.3	Out-of-synchronisation handling of output power

The UE shall monitor the DPCH quality in order to detect a loss of the signal on Layer 1, as specified in TS 25.224. The thresholds Qout, Qin, Qsbout and Qsbin specify at what DPCH quality levels the UE shall shut its power off and when it shall turn its power on, respectively. The thresholds are not defined explicitly, but are defined by the conditions under which the UE shall shut its transmitter off and turn it on, as stated in this clause.

#### 6.4.3.1	Requirement for continuous transmission

##### 6.4.3.1.1	3.84 Mcps TDD Option

# 6.4.3.1.1.1	Minimum requirement

When the UE estimates the DPCH quality over the last 160 ms period to be worse than a threshold Qout, the UE shall shut its transmitter off within 40 ms. The UE shall not turn its transmitter on again until the DPCH quality exceeds an acceptable level Qin. When the UE estimates the DPCH quality over the last 160 ms period to be better than a threshold Qin, the UE shall again turn its transmitter on within 40 ms.

The UE transmitter shall be considered "off" if the transmitted power is below the level defined in subclause 6.5.1 (Transmit off power). Otherwise the transmitter shall be considered as "on".

# 6.4.3.1.1.2	Test case

This subclause specifies a test case, which provides additional information for how the minimum requirement should be interpreted for the purpose of conformance testing in case of continuous transmission.

The conditions for the continuous test case are as follows:

The handover triggering level shall be set very high to ensure that the beacon channel power never exceeds the value of 10dB above it. Therefore the averaging time for signal quality will always be 160 milliseconds.

The quality levels at the thresholds Qout and Qin correspond to different signal levels depending on the downlink conditions DCH parameters. For the conditions in Table 6.4, a signal with the quality at the level Qout can be generated by a ΣDPCH_Ec/Ior ratio of -13 dB, and a signal with Qin by a ΣDPCH_Ec/Ior ratio of -9 dB. In this test, the DL reference measurement channel (12.2) kbps specified in subclauseA.2.2, where the CRC bits are replaced by data bits, and with static propagation conditions is used.

Table 6.4: DCH parameters for the of Out-of-synch handling test case - 3.84 Mcps TDD option - continuous transmission

| Parameter | Unit |  | Value |
| --- | --- | --- | --- |
|  |  | dB | 1.1 |
|  |  | dBm/3.84 MHz | -60 |
|  | dB |  | See figure 6.1 |
| Information Data Rate | kbps |  | 13 |
| TFCI | - |  | On |

Figure 6.1 shows an example scenario where the ΣDPCH_Ec/Ior ratio varies from a level where the DPCH is demodulated under normal conditions, down to a level below Qout where the UE shall shut its power off and then back up to a level above Qin where the UE shall turn the power back on.

Figure 6.1: Test case for out-of-synch handling in the UE. - 3.84 Mcps TDD option - continuous transmission

In this test case, the requirements for the UE are that

1)	The UE shall not shut its transmitter off before point B.

2)	The UE shall shut its transmitter off before point C, which is Toff = 200 ms after point B

3)	The UE shall not turn its transmitter on between points C and E.

4)	The UE shall turn its transmitter on before point F, which is Ton = 200 ms after Point E.

##### 6.4.3.1.2	1.28 Mcps TDD Option

# 6.4.3.1.2.1	Minimum Requirement

When the UE estimates the DPCH quality over the last 160 ms period to be worse than a threshold Qout, the UE shall shut its transmitter off within 40 ms. The UE shall not turn its transmitter on again until the DPCH quality exceeds an acceptable level Qin. When the UE estimates the DPCH quality over the last 160 ms period to be better than a threshold Qin, the UE shall again turn its transmitter on within 40 ms.

The DPCH quality shall be monitored in the UE and compared to the thresholds Qout and Qin for the purpose of monitoring synchronisation. The threshold Qout should correspond to a level of DPCH quality where no reliable detection of the TPC commands transmitted on the downlink DPCH can be made. This can be at a TPC command error ratio level of e.g. 30%. The threshold Qin should correspond to a level of DPCH quality where detection of the TPC commands transmitted on the downlink DPCH is significantly more reliable than at Qout. This can be at a TPC command error ratio level of e.g. 20%.

The UE transmitter shall be considered "off" if the transmitted power is below the level defined in subclause 6.5.1 (Transmit off power). Otherwise the transmitter shall be considered as "on".

# 6.4.3.1.2.2	Test case

This subclause specifies a test case, which provides additional information for how the minimum requirement should be interpreted for the purpose of conformance testing in case of continuous transmission for 1.28 Mcps TDD option.

The conditions for the continuous test case are as follows:

The handover triggering level shall be set very high to ensure that the beacon channel power never exceeds the value of 10dB above it. Therefore the averaging time for signal quality will always be 160 milliseconds.

The quality levels at the thresholds Qout and Qin correspond to different signal levels depending on the downlink conditions DCH parameters. For the conditions in Table 6.4, a signal with the quality at the level Qout can be generated by a ΣDPCH_Ec/Ior ratio of -15 dB, and a signal with Qin by a ΣDPCH_Ec/Ior ratio of -4.5 dB. In this test, the DL reference measurement channel (12.2) kbps specified in subclause A.2.2, where the CRC bits are replaced by data bits, and with static propagation conditions is used.

Table 6.4AA: DCH parameters for the of Out-of-synch handling test case - 1.28 Mcps TDD option - continuous transmission

| Parameter | Unit |  | Value |
| --- | --- | --- | --- |
|  |  | dB | -1 |
|  |  | dBm/1.28 MHz | -60 |
|  | dB |  | See figure 6.1AA |
| Information Data Rate | kbps |  | 12.2 |
| TFCI | - |  | On |

Figure 6.1AA shows an example scenario where the ΣDPCH_Ec/Ior ratio varies from a level where the DPCH is demodulated under normal conditions, down to a level below Qout where the UE shall shut its power off and then back up to a level above Qin where the UE shall turn the power back on.

Figure 6.1AA: Test case for out-of-synch handling in the UE - 1.28 Mcps TDD option - continuous transmission

In this test case, the requirements for the UE are that:

1)	The UE shall not shut its transmitter off before point B.

2)	The UE shall shut its transmitter off before point C, which is Toff = 200 ms after point B

3)	The UE shall not turn its transmitter on between points C and E.

4)	The UE shall turn its transmitter on before point F, which is Ton = 200 ms after Point E.

##### 6.4.3.1.3	7.68 Mcps TDD Option

# 6.4.3.1.3.1	Minimum requirement

When the UE estimates the DPCH quality over the last 160 ms period to be worse than a threshold Qout, the UE shall shut its transmitter off within 40 ms. The UE shall not turn its transmitter on again until the DPCH quality exceeds an acceptable level Qin. When the UE estimates the DPCH quality over the last 160 ms period to be better than a threshold Qin, the UE shall again turn its transmitter on within 40 ms.

The UE transmitter shall be considered "off" if the transmitted power is below the level defined in subclause 6.5.1 (Transmit off power). Otherwise the transmitter shall be considered as "on".

# 6.4.3.1.3.2	Test case

This subclause specifies a test case, which provides additional information for how the minimum requirement should be interpreted for the purpose of conformance testing in case of continuous transmission.

The conditions for the continuous test case are as follows:

The handover triggering level shall be set very high to ensure that the beacon channel power never exceeds the value of 10dB above it. Therefore the averaging time for signal quality will always be 160 milliseconds.

The quality levels at the thresholds Qout and Qin correspond to different signal levels depending on the downlink conditions DCH parameters. For the conditions in Table 6.4AB, a signal with the quality at the level Qout can be generated by a ΣDPCH_Ec/Ior ratio of -16 dB, and a signal with Qin by a ΣDPCH_Ec/Ior ratio of -12 dB. In this test, the DL reference measurement channel (12.2) kbps specified in subclause A.2.2, where the CRC bits are replaced by data bits, and with static propagation conditions is used.

Table 6.4AB: DCH parameters for the of Out-of-synch handling test case - 7.68 Mcps TDD option - continuous transmission

| Parameter | Unit |  | Value |
| --- | --- | --- | --- |
|  |  | dB | 1.1 |
|  |  | dBm/7.68 MHz | -60 |
|  | dB |  | See Figure 6.1BB |
| Information Data Rate | kbps |  | 13 |
| TFCI | - |  | On |

Figure 6.1AB shows an example scenario where the ΣDPCH_Ec/Ior ratio varies from a level where the DPCH is demodulated under normal conditions, down to a level below Qout where the UE shall shut its power off and then back up to a level above Qin where the UE shall turn the power back on.

Figure 6.1AB: Test case for out-of-synch handling in the UE. - 7.68 Mcps TDD option - continuous transmission

In this test case, the requirements for the UE are that

1)	The UE shall not shut its transmitter off before point B.

2)	The UE shall shut its transmitter off before point C, which is Toff = 200 ms after point B

3)	The UE shall not turn its transmitter on between points C and E.

4)	The UE shall turn its transmitter on before point F, which is Ton = 200 ms after Point E.

#### 6.4.3.2	Requirement for discontinuous transmission

##### 6.4.3.2.1	3.84 Mcps TDD Option

# 6.4.3.2.1.1	Minimum Requirement

During DTX, there are periods when the UE will receive no data from the UTRAN. As specified in TS 25.224, in order to keep synchronization, Special Bursts shall be transmitted by the UTRAN during these periods of no data.

During these periods, the conditions for when the UE shall shut its transmitter on or off are defined by the power level of the received Special Bursts.

When the UE does not detect at least one special burst with a quality above a threshold Qsbout over the last 160 ms period, the UE shall shut its transmitter off within 40 ms. The UE shall not turn its transmitter on again until the special burst quality exceeds an acceptable level Qsbin. When the UE estimates the special burst quality to be better than a threshold Qsbin over the last 160 ms, the UE shall again turn its transmitter on within 40 ms.

The UE transmitter shall be considered "off" if the transmitted power is below the level defined in subclause 6.5.1 (Transmit off power). Otherwise the transmitter shall be considered as "on".

# 6.4.3.2.1.2	Test case

This subclause specifies a test case, which provides additional information for how the minimum requirement should be interpreted for the purpose of conformance testing in case of discontinuous transmission.

The conditions for the discontinuous test case are as follows:

The handover triggering level shall be set very high to ensure that the beacon channel power never exceeds the value of 10dB above it. Therefore the averaging time for signal quality will always be 160 milliseconds.

The UTRAN transmits Special Bursts as specified in TS 25.224. The Special Burst Scheduling Parameter, SBSP = 4, which means that UTRAN sends a Special Burst at every fourth frame with no data. Therefore, the UTRAN sends a Special Burst in the first frame without data transmission, followed by 3 frames with no transmission; followed by a Special Burst, etc.

The DCH parameters are shown in Table 6.4A.

The quality levels at the thresholds Qsbout and Qsbin correspond to different signal levels depending on the downlink conditions DCH parameters. For the conditions in Table 6.4A, a signal with the quality at the level Qsbout can be generated by a DPCH_Ec/Ior ratio during received special bursts of -16 dB, and a signal with Qsbin by a DPCH_Ec/Ior ratio during received special bursts of -12 dB.

Table 6.4A: DCH parameters for the of Out-of-synch handling test case - 3.84 Mcps TDD option - discontinuous transmission

| Parameter | Unit | Value |
| --- | --- | --- |
|  | dB | 1.1 |
|  | dBm/3.84 MHz | -60 |
|  | dB | See figure 6.1A |
| Bits/burst (including TFCI bits) | bits | 244 |
| TFCI | - | On |

Figure 6.1A shows an example scenario where the special burst quality varies from a level above Qsbin , down to a level below Qsbout where the UE shall shut its power off and then back up to a level above Qsbin where the UE shall turn the power back on.

While the normal data is transmitted using two channelization codes, the Special Burst is transmitted with only one channelization code. Therefore the total energy per chip during Special Bursts is 3 dB lower than for continuous data transmission. The Special Bursts are represented by "SBs" in Figure 6.1A.

During the period of 3 frames with no data, the UE will receive a very low power, which is not shown in the figure. The power shown in the figure is the power of the Special Burst.

Figure 6.1A. Test case for out-of-synch handling in the UE - 3.84 Mcps TDD option - discontinuous transmission

In this test case, he requirements for the UE are that:

1)	The UE shall not shut its transmitter off before point B.

2)	The UE shall shut its transmitter off before point C, which is Toff = 200 ms after point B.

3)	The UE shall not turn its transmitter on between points C and E.

4)	The UE shall turn its transmitter on before point F, which is Ton = 200 ms after Point E.

##### 6.4.3.2.2	1.28 Mcps TDD Option

# 6.4.3.2.2.1	Minimum Requirement

During DTX, there are periods when the UE will receive no data from the UTRAN. As specified in TS 25.224, in order to keep synchronization, Special Bursts shall be transmitted by the UTRAN during these periods of no data.

The DPCH quality shall be monitored in the UE and compared to the thresholds Qsbout and Qsbin for the purpose of monitoring synchronisation during downlink DTX. The threshold Qsbout should correspond to a level of DPCH quality where no reliable detection of the TPC commands transmitted on the downlink DPCH can be made. This can be at a TPC command error ratio level of e.g. 30. The threshold Qsbin should correspond to a level of DPCCH quality where detection of the TPC commands transmitted on the downlink DPCH is significantly more reliable than at Qsbout. This can be at a TPC command error ratio level of e.g. 20%.

When the UE does not detect at least one special burst with a quality above a threshold Qsbout over the last 160 ms period, the UE shall shut its transmitter off within 40 ms. The UE shall not turn its transmitter on again until the special burst quality exceeds an acceptable level Qsbin. When the UE estimates the special burst quality to be better than a threshold Qsbin over the last 160 ms, the UE shall again turn its transmitter on within 40 ms.

The UE transmitter shall be considered "off" if the transmitted power is below the level defined in subclause 6.5.1 (Transmit off power). Otherwise the transmitter shall be considered as "on".

# 6.4.3.2.2.2	Test case

This subclause specifies a test case, which provides additional information for how the minimum requirement should be interpreted for the purpose of conformance testing in case of discontinuous transmission.

The conditions for the discontinuous test case are as follows :

The handover triggering level shall be set very high to ensure that the beacon channel power never exceeds the value of 10dB above it. Therefore the averaging time for signal quality will always be 160 milliseconds.

The UTRAN transmits Special Bursts as specified in TS 25.224. The Special Burst Scheduling Parameter, SBSP = 4, which means that UTRAN sends a Special Burst at every fourth frame with no data. Therefore, the UTRAN sends a Special Burst in the first frame without data transmission, followed by 3 frames with no transmission; followed by a Special Burst, etc. Additionally, the Special Burst will be sent in both subframes of the relevant frame designated for the Special Burst.

The DCH parameters are shown in Table 6.4B.

The quality levels at the thresholds Qsbout and Qsbin correspond to different signal levels depending on the downlink conditions DCH parameters. For the conditions in Table 6.4B, a signal with the quality at the level Qsbout can be generated by a DPCH_Ec/Ior ratio during received special bursts of -18 dB, and a signal with Qsbin by a DPCH_Ec/Ior ratio during received special bursts of -7,5 dB.

Table 6.4B: DCH parameters for the of Out-of-synch handling test case - 1.28 Mcps TDD option - discontinuous transmission

| Parameter | Unit | Value |
| --- | --- | --- |
|  | dB | -1 |
|  | dBm/1.28 MHz | -60 |
|  | dB | See figure 6.1B |
| Bits/burst (including TFCI bits) | bits | 88 in each subframe |
| TFCI | - | On |

Figure 6.1B shows an example scenario where the DPCH_Ec/Ior ratio during received special bursts varies from a level where the DPCH in DTX mode is demodulated under normal conditions, down to a level below Qsbout where the UE shall shut its power off and then back up to a level above Qsbin where the UE shall turn the power back on.

While the normal data is transmitted using two channelization codes, the Special Burst is transmitted with only one channelization code. Therefore the total energy per chip during Special Bursts is 3 dB lower than for continuous data transmission. The Special Bursts are represented by "SBs" in the figure.

During the period of 3 frames with no data, the UE will receive a very low power, which is not shown in the figure. In the fourth frame the Special Burst will be sent in both subframes designated to carry the Special Burst during DTX. The power shown in the figure is the power of the Special Burst.

Figure 6.1B: Test case for out-of-synch handling in the UE -1.28 Mcps TDD option - discontinuous transmission

In this test case, the requirements for the UE are that:

1)	The UE shall not shut its transmitter off before point B.

2)	The UE shall shut its transmitter off before point C, which is Toff = 200 ms after point B.

3)	The UE shall not turn its transmitter on between points C and E.

4)	The UE shall turn its transmitter on before point F, which is Ton = 200 ms after Point E.

##### 6.4.3.2.3	7.68 Mcps TDD Option

# 6.4.3.2.3.1	Minimum Requirement

During DTX, there are periods when the UE will receive no data from the UTRAN. As specified in TS 25.224, in order to keep synchronization, Special Bursts shall be transmitted by the UTRAN during these periods of no data.

During these periods, the conditions for when the UE shall shut its transmitter on or off are defined by the power level of the received Special Bursts.

When the UE does not detect at least one special burst with a quality above a threshold Qsbout over the last 160 ms period, the UE shall shut its transmitter off within 40 ms. The UE shall not turn its transmitter on again until the special burst quality exceeds an acceptable level Qsbin. When the UE estimates the special burst quality to be better than a threshold Qsbin over the last 160 ms, the UE shall again turn its transmitter on within 40 ms.

The UE transmitter shall be considered "off" if the transmitted power is below the level defined in subclause 6.5.1 (Transmit off power). Otherwise the transmitter shall be considered as "on".

# 6.4.3.2.3.2	Test case

This subclause specifies a test case, which provides additional information for how the minimum requirement should be interpreted for the purpose of conformance testing in case of discontinuous transmission.

The conditions for the discontinuous test case are as follows:

The handover triggering level shall be set very high to ensure that the beacon channel power never exceeds the value of 10dB above it. Therefore the averaging time for signal quality will always be 160 milliseconds.

The UTRAN transmits Special Bursts as specified in TS 25.224. The Special Burst Scheduling Parameter, SBSP = 4, which means that UTRAN sends a Special Burst at every fourth frame with no data. Therefore, the UTRAN sends a Special Burst in the first frame without data transmission, followed by 3 frames with no transmission; followed by a Special Burst, etc.

The DCH parameters are shown in Table 6.4C.

The quality levels at the thresholds Qsbout and Qsbin correspond to different signal levels depending on the downlink conditions DCH parameters. For the conditions in Table 6.4C, a signal with the quality at the level Qsbout can be generated by a DPCH_Ec/Ior ratio during received special bursts of -19 dB, and a signal with Qsbin by a DPCH_Ec/Ior ratio during received special bursts of -15 dB.

Table 6.4C: DCH parameters for the of Out-of-synch handling test case - 7.68 Mcps TDD option - discontinuous transmission

| Parameter | Unit | Value |
| --- | --- | --- |
|  | dB | 1.1 |
|  | dBm/7.68 MHz | -60 |
|  | dB | See Figure 6.1C |
| Bits/burst (including TFCI bits) | bits | 244 |
| TFCI | - | On |

Figure 6.1C shows an example scenario where the special burst quality varies from a level above Qsbin , down to a level below Qsbout where the UE shall shut its power off and then back up to a level above Qsbin where the UE shall turn the power back on.

While the normal data is transmitted using two channelization codes, the Special Burst is transmitted with only one channelization code. Therefore the total energy per chip during Special Bursts is 3 dB lower than for continuous data transmission. The Special Bursts are represented by "SBs" in Figure 6.1C.

During the period of 3 frames with no data, the UE will receive a very low power, which is not shown in the figure. The power shown in the figure is the power of the Special Burst.

Figure 6.1C. Test case for out-of-synch handling in the UE - 7.68 Mcps TDD option - discontinuous transmission

In this test case, he requirements for the UE are that:

1)	The UE shall not shut its transmitter off before point B.

2)	The UE shall shut its transmitter off before point C, which is Toff = 200 ms after point B.

3)	The UE shall not turn its transmitter on between points C and E.

4)	The UE shall turn its transmitter on before point F, which is Ton = 200 ms after Point E.

## 6.5	Transmit ON/OFF power

### 6.5.1	Transmit OFF power

Transmit OFF power is defined as the RRC filtered mean power measured over one chip when the transmitter is off. The transmit OFF power state is when the UE does not transmit.

#### 6.5.1.1	Minimum Requirement

The requirement for transmit OFF power shall be less than 65dBm.

#### 6.5.1.2	Additional requirement for 1.28Mcps TDD MC-HSUPA

The transmit OFF power is defined per carrier as the RRC filtered mean power in a duration of at least one timeslot excluding any transient periods. The requirement for the transmit OFF power in each carrier shall be less than -65 dBm, when the transmissions in all carriers are turned off.

### 6.5.2	Transmit ON/OFF Time mask

The time mask transmit ON/OFF defines the ramping time allowed for the UE between transmit OFF power and transmit ON power.

#### 6.5.2.1	Minimum Requirement

##### 6.5.2.1.1	3.84 Mcps TDD Option

The transmit power level versus time shall meet the mask specified in figure 6.2, where the transmission period refers to the burst without guard period for a single transmission slot, and to the period from the beginning of the burst in the first transmission slot to the end of the burst without guard period in the last transmission timeslot for consecutive transmission slots.

- Figure 6.2: Transmit ON/OFF template for 3.84 Mcps TDD Option

##### 6.5.2.1.2	1.28 Mcps TDD Option

The transmit power level versus time shall meet the mask specified in figure 6.2A, where the transmission period refers to the burst without guard period for a single transmission slot, and to the period from the beginning of the burst in the first transmission slot to the end of the burst without guard period in the last transmission timeslot for consecutive transmission slots.

- Figure 6.2A: Transmit ON/OFF template for 1.28 Mcps TDD Option

##### 6.5.2.1.3	7.68 Mcps TDD Option

The transmit power level versus time shall meet the mask specified in Figure 6.2B, where the transmission period refers to the burst without guard period for a single transmission slot, and to the period from the beginning of the burst in the first transmission slot to the end of the burst without guard period in the last transmission timeslot for consecutive transmission slots.

- Figure 6.2B: Transmit ON/OFF template for 7.68 Mcps TDD Option

## 6.6	Output RF spectrum emissions

### 6.6.1	Occupied bandwidth

#### 6.6.1.1	3.84 Mcps TDD Option

Occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum, centred on the assigned channel frequency. The occupied channel bandwidth shall be less than 5 MHz based on a chip rate of 3.84 Mcps.

#### 6.6.1.2	1.28 Mcps TDD Option

Occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum, centred on the assigned channel frequency. The occupied channel bandwidth shall be less than 1.6 MHz based on a chip rate of 1.28 Mcps.

#### 6.6.1.3	7.68 Mcps TDD Option

Occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum, centred on the assigned channel frequency. The occupied channel bandwidth shall be less than 10 MHz based on a chip rate of 7.68 Mcps.

### 6.6.1A	Occupied bandwidth for 1.28Mcps TDD MC-HSUPA

In the case of multi-carrier transmission, occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum, centered at the center of the assigned channel frequencies. The occupied channel bandwidth shall be less than N*1.6 MHz based on a chip rate of 1.28 Mcps, in which N is the number of assigned carrier frequencies.

### 6.6.2	Out of band emission

Out of band emissions are unwanted emissions immediately outside the nominal channel resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and adjacent channel leakage power ratio (ACLR).

#### 6.6.2.1	Spectrum emission mask

##### 6.6.2.1.1	3.84 Mcps TDD Option

The spectrum emission mask of the UE applies to frequencies, which are between 2.5 MHz and 12.5MHz from the UE centre carrier frequency. The out of channel emission is specified relative to theRRC filtered mean power of the UE carrier.

6.6.2.1.1.1	Minimum Requirement

The power of any UE emission shall not exceed the levels specified in table 6.5.

- Table 6.5: Spectrum Emission Mask Requirement (3.84 Mcps TDD Option)

| Δf* in MHz | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 2.5 - 3.5 |  | 30 kHz ** |
| 3.5 - 7.5 |  | 1 MHz *** |
| 7.5 - 8.5 |  | 1 MHz *** |
| 8.5 - 12.5 | -49 dBc | 1 MHz *** |
| * f is the separation between the carrier frequency and the centre of the measuring filter. |  |  |
| ** The first and last measurement position with a 30 kHz filter is at f equals to 2.515 MHz and 3.485 MHz |  |  |
| *** The first and last measurement position with a 1 MHz filter is at f equals to 4 MHz and 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |
| Note: The lower limit shall be -50dBm/3.84 MHz or the minimum requirement presented in this table which ever is the higher. |  |  |

##### 6.6.2.1.2	1.28 Mcps TDD Option

The spectrum emission mask of the UE applies to frequencies, which are between 0.8MHz and 4.0MHz from the UE centre carrier frequency. The out of channel emission is specified relative to the RRC filtered mean power of the UE carrier.

6.6.2.1.2.1	Minimum Requirement

The power of any UE emission shall not exceed the levels specified in table 6.5A. The requirements assume that the UE output power shall be maximum level.

- Table 6.5A: Spectrum Emission Mask Requirement (1.28 Mcps TDD Option)

| Δf* in MHz | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 0.8-1.8 |  | 30 kHz ** |
| 1.8-2.4 | −49−17⋅ΔfMHz−1.8dBc | 30 kHz ** |
| 2.4 - 4.0 | -44 dBc | 1MHz *** |
| * f is the separation between the carrier frequency and the centre of the measuring filter. |  |  |
| ** The first and last measurement position with a 30 kHz filter is at f equals to 0.815 MHz and 2.385 MHz. |  |  |
| *** The first and last measurement position with a 1 MHz filter is at f equals to 2.9MHz and 3.5MHz .As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |
| Note: The lower limit shall be -55dBm/1.28 MHz or the minimum requirement presented in this table which ever is the higher. |  |  |

##### 6.6.2.1.3	7.68 Mcps TDD Option

The spectrum emission mask of the UE applies to frequencies, which are between 5 MHz and 25MHz from the UE centre carrier frequency. The out of channel emission is specified relative to the RRC filtered mean power of the UE carrier.

# 6.6.2.1.3.1	Minimum Requirement

The power of any UE emission shall not exceed the levels specified in Table 6.5B.

- Table 6.5B: Spectrum Emission Mask of higher chip rate reference configuration

| Δf* in MHz | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 5.0 - 5.75 | −38−10.67⋅ΔfMHz−5.0dBc | 30 kHz ** |
| 5.75 - 7.0 | −46−5.6⋅ΔfMHz−5.75dBc | 30 kHz** |
| 7.0 - 15 | −38−0.5⋅ΔfMHz−7.0dBc | 1 MHz *** |
| 15.0 - 17.0 | −42−5.0⋅ΔfMHz−15.0dBc | 1 MHz *** |
| 17.0 - 25.0 | -53 dBc | 1 MHz *** |
| * f is the separation between the carrier frequency and the centre of the measuring filter. |  |  |
| ** The first and last measurement position with a 30 kHz filter is at f equals to 5.015 MHz and 6.985 MHz |  |  |
| *** The first and last measurement position with a 1 MHz filter is at f equals to 7.5 MHz and 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |
| Note: The lower limit shall be -47dBm/7.68 MHz or the minimum requirement presented in this table which ever is the higher. |  |  |

#### 6.6.2.1A	Additional Spectrum emission mask for 1.28Mcps TDD MC-HSUPA

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.5C the spurious requirements in clause 6.6.3.1A are applicable.

##### 6.6.2.1A.1	Minimum requirement

The power of any UE emission shall not exceed the levels specified in Table 6.5C for the specified transmission carrier number.

- Table 6.5C: Spectrum emission mask for MC-HSUPA

| Spectrum emission limit (dBm) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | Transmission Carrier Number |  |  | Measurement bandwidth |
|  | 2 | 3 | 6 |  |
|  0-1 | -13 | -15 | -18 | 30 kHz |
|  1-2.5 | -10 | -10 | -10 | 1 MHz |
|  2.5-2.8 | -10 | -10 | -10 | 1 MHz |
|  2.8-5 | -10 | -10 | -10 | 1 MHz |
|  5-6 | -25 | -13 | -13 | 1 MHz |
|  6-10 |  | -25 | -13 | 1 MHz |
|  10-15 |  |  | -25 | 1 MHz |

#### 6.6.2.2	Adjacent Channel Leakage power Ratio (ACLR)

In the case a single carrier is assigned on the uplink, Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the RRC filtered mean power centered on the assigned channel frequency to the RRC filtered mean power centered on an adjacent channel frequency.

In the case multiple adjacent carriers are assigned on the uplink, ACLR is the ratio of the sum of the RRC filtered mean power centered on each assigned channel frequencies to the RRC filtered mean powers centered on an adjacent channel frequency.

##### 6.6.2.2.1	Minimum requirement

# 6.6.2.2.1.1	3.84 Mcps TDD Option

If the adjacent channel RRC filtered mean power is greater than -50dBm then the ACLR shall be higher than the value specified in Table 6.6.

- Table 6.6:UE ACLR (3.84 Mcps TDD Option)

| Power Class | adjacent channel | ACLR limit |
| --- | --- | --- |
| 2, 3 | UE channel ± 5 MHz | 33 dB |
| 2, 3 | UE channel ± 10 MHz | 43 dB |

- NOTE:

1)	The requirement shall still be met in the presence of switching transients.

2)	The ACLR requirements reflect what can be achieved with present state of the art technology.

3)	Requirement on the UE shall be reconsidered when the state of the art technology progresses.

# 6.6.2.2.1.2	1.28 Mcps TDD Option

If the adjacent channel RRC filtered mean power is greater than -55dBm then the ACLR shall be higher than the value specified in Table 6.6A.

- Table 6.6A: UE ACLR (1.28 Mcps TDD Option)

| Power Class | adjacent channel | ACLR limit |
| --- | --- | --- |
| 2, 3 | UE channel ± 1.6 MHz | 33 dB |
| 2, 3 | UE channel ± 3.2 MHz | 43 dB |

- NOTE:

1)	The requirement shall still be met in the presence of switching transients.

2)	The ACLR requirements reflect what can be achieved with present state of the art technology.

3)	Requirement on the UE shall be reconsidered when the state of the art technology progresses.

# 6.6.2.2.1.3	7.68 Mcps TDD Option

If the adjacent channel RRC filtered mean power is greater than -50dBm measured with a 3.84 Mcps RRC filter then the ACLR shall be higher than the value specified in Table 6.6B.

- Table 6.6B: UE ACLR of higher chip rate reference configuration

| Power Class | adjacent channel | Chip Rate for RRC Measurement Filter | ACLR limit |
| --- | --- | --- | --- |
| 2, 3 | UE channel ± 7.5 MHz | 3.84 MHz | 33 dB |
| 2, 3 | UE channel ± 12.5 MHz | 3.84 MHz | 43 dB |
| 2 ,3 | UE channel ± 10.0 MHz | 7.68 MHz | 33 dB |
| 2 ,3 | UE channel ± 20.0 MHz | 7.68 MHz | 43 dB |

- NOTE:

1)	The requirement shall still be met in the presence of switching transients.

2)	The ACLR requirements reflect what can be achieved with present state of the art technology.

##### 6.6.2.2.2	Additional requirement for 1.28Mcps TDD MC-HSUPA

If the adjacent channel RRC filtered mean power is greater than -55dBm then the ACLR shall be higher than the value specified in Table 6.6C.

Table 6.6C: UE ACLR for multi-carrier transmission

| Power Class | Adjacent channel frequency relative to the center of two assigned channel frequencies | ACLR limit |
| --- | --- | --- |
| 2, 3 | + (N*1.6 + 0.8)MHz or - (N*1.6 + 0.8)MHz | 33 dB |
| 2, 3 | + (N*1.6 + 2.4)MHz or - (N*1.6 + 2.4)MHz | 36 dB |
| Note: N is the number of assigned transmission carriers. |  |  |

NOTE:

1)	The requirement shall still be met in the presence of switching transients.

2)	The ACLR requirements reflect what can be achieved with present state of the art technology.

3)	Requirement on the UE shall be reconsidered when the state of the art technology progresses.

### 6.6.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The frequency boundary and the detailed transitions of the limits between the requirement for out band emissions and spectrum emissions are based on ITU-R Recommendations SM.329 [3].

#### 6.6.3.1	Minimum Requirement

##### 6.6.3.1.1	3.84 Mcps TDD Option

These requirements are only applicable for frequencies which are greater than 12.5 MHz away from the UE center carrier frequency.

- Table 6.7A: General Spurious emissions requirements (3.84 Mcps TDD Option)

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- |
| 9 kHz  f < 150 kHz | 1 kHz | -36 dBm |
| 150 kHz  f < 30 MHz | 10 kHz | -36 dBm |
| 30 MHz  f < 1000 MHz | 100 kHz | -36 dBm |
| 1 GHz  f < 12.75 GHz | 1 MHz | -30 dBm |

- Table 6.7B: Additional Spurious emissions requirements (3.84 Mcps TDD Option)

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- |
| 921 MHz  f < 925 MHz | 100 kHz | -60 dBm (note 1) |
| 925 MHz  f  935 MHz | 100 kHz | -67 dBm (note 1) |
| 935 MHz < f  960 MHz | 100 kHz | -79 dBm (note 1) |
| 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm (note 1) |
| 2620 MHz  f  2690 MHz | 3.84 MHz | -37 dBm (note 1) |
| 1884.5 MHz  f  1915.7 MHz | 300 kHz | -41 dBm (note 2) |
| NOTE 1: The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.7A are permitted for each UARFCN used in the measurement. NOTE 2: Applicable for transmission in 2010-2025 MHz as defined in subclause 5.2 (a). |  |  |

##### 6.6.3.1.2	1.28 Mcps TDD Option

These requirements are only applicable for frequencies which are greater than 4 MHz away from the UE center carrier frequency.

- Table 6.7C: General Spurious emissions requirements (1.28 Mcps TDD Option)

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- |
| 9 kHz  f < 150 kHz | 1 kHz | -36 dBm |
| 150 kHz  f < 30 MHz | 10 kHz | -36 dBm |
| 30 MHz  f < 1000 MHz | 100 kHz | -36 dBm |
| 1 GHz  f < 12.75 GHz | 1 MHz | -30 dBm |

- Table 6.7D: Additional Spurious emissions requirements (1.28 Mcps TDD Option)

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| a | 703 MHz  f < 803 MHz | 1 MHz | -50 dBm (note 3) |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz  f  935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm (note1) |
|  | 2010 MHz  f  2025 MHz | 1MHz | -65 dBm (Note2) |
|  | 1880 MHz  f  1920 MHz | 1MHz | -65 dBm (Note 3) |
|  | 2300 MHz  f  2400 MHz | 1MHz | -65 dBm (note 3) |
|  | 2496 MHz  f  2690 MHz | 1MHz | -50dBm (note 3) |
|  | 3400 MHz  f < 3600 MHz | 1 MHz | -50 dBm (note 3) |
| b | 1850 MHz  f  1910 MHz | 1 MHz | -65 dBm (Note 4) |
|  | 1930 MHz  f  1990 MHz | 1 MHz | -65 dBm (Note 5) |
|  | 2010 MHz  f  2025 MHz | 1MHz | -65 dBm |
| c | 2010 MHz  f  2025 MHz | 1 MHz | -65 dBm |
| d | 1900 MHz  f  1920 MHz | 1 MHz | -65 dBm |
|  | 2010 MHz  f  2025 MHz | 1 MHz | -65 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -37 dBm |
| e | 703 MHz  f < 803 MHz | 1 MHz | -50 dBm (note1) |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz  f  935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm (note1) |
|  | 1880 MHz  f  1920 MHz | 1 MHz | -65 dBm |
|  | 2010 MHz  f  2025 MHz | 1 MHz | -65 dBm |
|  | 2496 MHz  f  2690 MHz | 1MHz | -50dBm |
|  | 3400 MHz  f < 3600 MHz | 1 MHz | -50 dBm |
| f | 703 MHz  f < 803 MHz | 1 MHz | -50 dBm (note1) |
|  | 921 MHz f<925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz< f <935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz< f <960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz  f  1850 MHz | 100 kHz | -71 dBm (note1) |
|  | 2010 MHz  f  2025 MHz | 1MHz | -65 dBm |
|  | 2300 MHz  f  2400 MHz | 1MHz | -65 dBm |
|  | 2496 MHz  f  2690 MHz | 1MHz | -50dBm |
|  | 3400 MHz  f < 3600 MHz | 1 MHz | -50 dBm |
| Note 1 The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.7c are permitted for each UARFCN used in the measurement. Note 2: This requirement is only applicable when UE operating in 1900-1920MHz of band a. Note 3: This requirement is only applicable when UE operating in 2010-2025MHz of band a. Note 4: This requirement is only applicable when UE operating in 1930-1990MHz of band b. Note 5: This requirement is only applicable when UE operating in 1850-1910MHz of band b. |  |  |  |

##### 6.6.3.1.3	7.68 Mcps TDD Option

These requirements are only applicable for frequencies which are greater than 25 MHz away from the UE center carrier frequency.

- Table 6.7E: General Spurious emissions requirements (7.68 Mcps TDD Option)

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- |
| 9 kHz  f < 150 kHz | 1 kHz | -36 dBm |
| 150 kHz  f < 30 MHz | 10 kHz | -36 dBm |
| 30 MHz  f < 1000 MHz | 100 kHz | -36 dBm |
| 1 GHz  f < 12.75 GHz | 1 MHz | -30 dBm |

- Table 6.7F: Additional Spurious emissions requirements (7.68 Mcps TDD Option)

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- |
| 921 MHz  f < 925 MHz | 100 kHz | -60 dBm (note 1) |
| 925 MHz  f  935 MHz | 100 kHz | -67 dBm (note 1) |
| 935 MHz < f  960 MHz | 100 kHz | -79 dBm (note 1) |
| 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm (note 1) |
| 2620 MHz  f  2690 MHz | 3.84 MHz | -37 dBm (note 1) |
| 1884.5 MHz  f  1915.7 MHz | 300 kHz | -41 dBm (note 2) |
| NOTE 1: The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.7E are permitted for each UARFCN used in the measurement. NOTE 2: Applicable for transmission in 2010-2025 MHz as defined in subclause 5.2 (a). |  |  |

- 6.6.3.2	Additional requirement for 1.28Mcps TDD MC-HSUPA

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.7G from the edge of the channel bandwidth and are only applicable for multi-carrier transmission.

- Table 6.7G:  Boundary between ΔfOOB and spurious emission domain

| - Channel bandwidth | - Transmission Carrier Number |  |  |
| --- | --- | --- | --- |
|  | - 2 | - 3 | - 6 |
| - ΔfOOB (MHz) | - 6 | - 10 | - 15 |

The spurious emission limits in Table 6.7C and Table 6.7D apply for all transmission carrier number configurations.

NOTE:	In order that the measurement of spurious emissions falls within the frequency ranges that are more than ΔfOOB (MHz) from the edge of the channel bandwidth, the minimum offset of the measurement frequency from each edge of the channel should be ΔfOOB + 0.8.

## 6.7	Transmit intermodulation

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

### 6.7.1	Minimum requirement

User Equipment(s) transmitting in close vicinity of each other can produce intermodulation products, which can fall into the UE, or BS receive band as an unwanted interfering signal. The UE intermodulation attenuation is defined by the ratio of the RRC filtered mean power of the wanted signal to the RRC filtered mean power of the intermodulation product when an interfering CW signal is added at a level below the wanted signal.

#### 6.7.1.1	3.84 Mcps TDD Option

The requirement of transmitting intermodulation for carrier spacing 5 MHz is prescribed in Table 6.8.

- Table 6.8: Transmit Intermodulation (3.84 Mcps TDD Option)

| Interference Signal Frequency Offset | 5MHz | 10MHz |
| --- | --- | --- |
| Interference Signal Level | -40 dBc |  |
| Minimum Requirement | -31dBc | -41dBc |

#### 6.7.1.2	1.28 Mcps TDD Option

The requirement of transmitting intermodulation for carrier spacing 1.6 MHz is prescribed in Table 6.8A.

- Table 6.8A: Transmit Intermodulation (1.28 Mcps TDD Option)

| Interference signal frequency offset | 1.6MHz | 3.2MHz |
| --- | --- | --- |
| Interference signal level | -40dBc |  |
| Minimum requirement of intermodulation products | -31 dBc | -41 dBc |

#### 6.7.1.3	7.68 Mcps TDD Option

The requirement of transmitting intermodulation for carrier spacing 10 MHz is prescribed in Table 6.8B.

- Table 6.8B: Transmit Intermodulation (7.68 Mcps TDD Option)

| Interference Signal Frequency Offset | 10MHz | 20MHz |
| --- | --- | --- |
| Interference Signal Level | -40 dBc |  |
| Minimum Requirement | -31dBc | -41dBc |

### 6.7.2	Additional requirement for 1.28Mcps TDD MC-HSUPA

The UE intermodulation attenuation is defined by the ratio of the sum of the RRC filtered mean powers of the wanted signal on the assigned N carriers to the sum of the RRC filtered mean powers of the intermodulation product on adjacent N carriers when an interfering CW signal is added at a level below the wanted signal. N is the number of carriers for multi-carrier transmission.

- Table 6.8C: Transmit Intermodulation requirement for 1.28Mcps TDD MC-HSUPA

| - Transmission Carrier Number (UL) | - 2 |  | - 3 |  | - 6 |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Interference Signal Frequency Offset | - 3.2MHz | - 6.4MHz | - 4.8MHz | - 9.6MHz | - 9.6MHz | - 19.2MHz |
| - Interference Signal Level | - -40dBc |  |  |  |  |  |
| - Intermodulation Product | - -31dBc | - -41dBc | - -31dBc | - -41dBc | - -31dBc | - -41dBc |

## 6.8	Transmit Modulation

Transmit modulation defines the modulation quality for expected in-channel RF transmissions from the UE. The requirements apply to all transmissions.

### 6.8.1	Transmit pulse shape filter

- The transmit pulse-shaping filter is a root-raised cosine (RRC) with roll-off =0.22 in the frequency domain. The impulse response of the chip impulse filter RC0(t) is

- Where the roll-off factor =0.22 and Tc is the chip duration

### 6.8.2	Error Vector Magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Both waveforms pass through a matched Root Raised Cosine filter with bandwidth corresponding to the considered chip rate and roll-off =0,22. One of the waveforms is then further modified by selecting the frequency, absolute phase, absolute amplitude and chip clock timing so as to minimise the error vector. The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. The period of measurement shall be one transmit timeslot excluding the guard period. See Annex B of TS 34.122 for further details.

#### 6.8.2.1	Minimum Requirement

When 16QAM modulation is not used on any of the uplink code channels, the Error Vector Magnitude shall not exceed 17.5 % for the parameters specified in Table 6.9.

When 16QAM modulation is used on any of the uplink code channels, the modulation accuracy requirement shall not exceed 14% for the parameters specified in Table 6.9

- Table 6.9: Test parameters for Error Vector Magnitude/Peak Code Domain Error

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -20 |
| Operating conditions |  | Normal conditions |
| Power control step size | dB | 1 |

#### 6.8.2.2	Additional requirement for 1.28Mcps TDD MC-HSUPA

When 16QAM modulation is not used on any of the uplink code channels in a carrier, the Error Vector Magnitude in that carrier shall not exceed 17.5 % for the parameters specified in Table 6.9.

When 16QAM modulation is used on any of the uplink code channels in a carrier, the modulation accuracy requirement shall not exceed 14% for the parameters specified in Table 6.9.

### 6.8.2A	In-band Emissions for 1.28Mcps TDD MC-HSUPA

The in-band emission is measured as the ratio of the UE output power in one carrier to the UE output power in another, where the power in the former carrier shall be set to the minimum output power and the power in the other carrier to the maximum output power. These two carriers were placed in the two edges respectively in the transmission carriers. All the other carriers are set to OFF power during the test.

The basic in-band emission measurement interval is defined over one slot in the time domain.

- Table 6.10: Minimum requirements for in-band emissions

| Parameter Description | Unit | Limit |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| IQ Image | dB | -25 |  | 1.28 MHz |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | 180 kHz (Note 1) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm  Output power < -30 dBm |  |
| Note 1: For Carrier leakage, the limit is defined as ratio between the power measured in a 180 kHz bandwidth around the center of transmission bandwidth, divided with the power measured in a 1.28MHz bandwidth centered around the primary carrier while all the other carriers are OFF. |  |  |  |  |

### 6.8.3	Peak Code Domain Error

This specification is applicable for multi-code transmission only.

The code domain error is computed by projecting the error vector power onto the code domain at a specific spreading factor. The error power for each code is defined as the ratio to the mean power of the reference waveform expressed in dB. The Peak Code Domain Error is defined as the maximum value for Code Domain Error. The period of measurement shall be one transmit timeslot excluding the guard period, and the midamble.

#### 6.8.3.1	Minimum Requirement

The peak code domain error shall not exceed -21 dB at spreading factor 16 for the parameters specified in Table 6.9.

The peak code domain error for 7.68 Mcps option shall not exceed -24 dB at spreading factor 32 for the parameters specified in Table 6.9.

The requirements are defined using the UL reference measurement channel specified in subclause A.2.7.

# 7	Receiver characteristics

## 7.1	General

Unless detailed the receiver characteristic are specified at the antenna connector of the UE. For UE with an integral antenna only, a reference antenna with a gain of 0 dBi is assumed. UE with an integral antenna may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For Ues with more than one receiver antenna connector the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

The UE antenna performance has a significant impact on system performance, and minimum requirements on the antenna efficiency are therefore intended to be included in future versions of this specification. It is recognised that different requirements and test methods are likely to be required for the different types of UE.

For 3.84Mcps TDD Option and 7.68Mcps TDD Option, all the parameters in Section 7 are defined using the DL reference measurement channel specified in Annex A.2.2

For 1.28Mcps TDD Option, UE supporting multi-carrier reception shall support both minimum requirements, as well as additional requirements for multi-carrier reception. For minimum requirements, all the parameters in Section 7 are defined using the DL reference measurement channel specified in Annex A.2.2; For UE supporting multi-carrier reception, all the parameters in Section 7 are defined using the DL reference measurement channel specified in Annex A.3.2.8. For the additional requirements for multi-carrier reception, the spacing between the two adjacent carriers shall be 1.6 MHz.

For Ues supporting only MBSFN reception, the DL reference measurement channel specified in Annex A.2.9 is used. For the purposes of clause 7, the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the DL reference measurement channel in use, irrespective of its particular physical channel type (DPCH or not).

## 7.2	Diversity characteristics

A suitable receiver structure using coherent reception in both channel impulse response estimation, and code tracking procedures is assumed. Three forms of diversity are considered to be available in UTRA/TDD:

- Table 7.1: Diversity characteristics for UTRA/TDD

| Time diversity | Channel coding and interleaving in both up link and down link |
| --- | --- |
| Multi-path diversity | Rake receiver or other suitable receiver structure with maximum combining. Additional processing elements can increase the delay-spread performance due to increased capture of signal energy. |
| Antenna diversity | Antenna diversity with maximum ratio combing in the base station and optionally in the mobile stations. Possibility for downlink transmit diversity in the base station. |

## 7.3	Reference sensitivity level

The reference sensitivity level is the minimum mean power received at the UE antenna port at which the the specified minimum requirement shall be met.

### 7.3.1	Minimum Requirements

#### 7.3.1.1	3.84 Mcps TDD Option

For non-IMB operation, the BER shall not exceed 0.001 for the parameters specified in Table 7.2.

For IMB operation, the BLER shall not exceed 0.01 for the parameters specified in Table 7.2.

- Table 7.2: Test parameters for reference sensitivity (3.84 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 * | dB |
|  | -105 | dBm/3.84 MHz |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |

#### 7.3.1.2	1.28 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in Table 7.2A.

- Table 7.2A: Test parameters for reference sensitivity (1.28 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
|  | -108 | dBm/1.28 MHz |

#### 7.3.1.3	7.68 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in Table 7.2B.

- Table 7.2B: Test parameters for reference sensitivity (7.68 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
|  | -105 | dBm/7.68 MHz |

### 7.3.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option

The BLER measured on each carrier shall not exceed 0.1 for the parameters specified in Table 7.2AA.

- Table 7.2AA: Test parameters for reference sensitivity of multi-carrier reception

| Parameter | Level | Unit |
| --- | --- | --- |
| Σ HS-PDSCH_EcIor | 0 | dB |
|  | -104.8 | dBm/1.28 MHz |

## 7.4	Maximum input level

The maximum input level is defined as the maximum mean power received at the UE antenna port which does not degrade the specified minimum requirement.

### 7.4.1	Minimum Requirements for DPCH reception

#### 7.4.1.1	3.84 Mcps TDD Option

For non-IMB operation, the BER shall not exceed 0.001 for the parameters specified in Table 7.3.

For IMB operation, the BLER shall not exceed 0.01 for the parameters specified in Table 7.3.

- Table 7.3: Maximum input level (3.84 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | -7 * | dB |
|  | -25 | dBm/3.84 MHz |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |

#### 7.4.1.2	1.28 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in Table 7.3A

- Table 7.3A: Maximum input level (1.28 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | -7 | dB |
|  | -25 | dBm/1.28 MHz |

#### 7.4.1.3	7.68 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in Table 7.3B.

- Table 7.3B: Maximum input level (7.68 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | -10 | dB |
|  | -25 | dBm/7.68 MHz |

### 7.4.2	Minimum Requirements for HS-PDSCH reception

#### 7.4.2.1	3.84 Mcps TDD Option

<Void>

#### 7.4.2.2	1.28 Mcps TDD Option

##### 7.4.2.2.1 Minimum requirement for 16QAM

The throughput shall be ≥ 90% of the maximum throughput of the reference measurement channels as specified in Table 7.3CA for different UE categories with the parameters specified in Table 7.3C. For multi-carrier reception, DL reference channel specified in Table 7.3CA and the minimum requirements shall be applied to each carrier simultaneously.

- Table 7.3C

| Parameter | Level | Unit |
| --- | --- | --- |
| HS−PDSCHEcIor | 0 | dB |
|  | -25 | dBm/1.28 MHz |
| Redundancy and constellation version | 6 | - |
| Maximum number of HARQ transmissions | 1 | - |

- Table 7.3CA

| UE Category | Reference Channel |
| --- | --- |
| Category 4-6 | A.3.2.2.2 |
| Category 7-9 | A.3.2.3.2 |
| Category 10-12 | A.3.2.4.2 |
| Category 13-15 | A.3.2.5.2 |

##### 7.4.2.2.2	Minimum requirement for 64QAM

The throughput shall be ≥ 90% of the maximum throughput of the reference measurement channels as specified in Table 7.3E for different UE categories with the parameters specified in Table 7.3D. For multi-carrier reception, DL reference channel specified in Table 7.3E and the minimum requirements shall be applied to each carrier simultaneously.

- Table 7.3D

| - Parameter | - Level | - Unit |
| --- | --- | --- |
| - HS−PDSCHEcIor | - 0 | - dB |
|  | - -25 | - dBm/1.28 MHz |
| - Redundancy and constellation version | - 6 | - - |
| - Maximum number of HARQ transmissions | - 1 | - - |

- Table 7.3E

| UE Category | Reference Channel |
| --- | --- |
| Category 16-18 | A.3.2.7.1 |
| Category 19-21 | A.3.2.7.2 |
| Category 22-24 | A.3.2.7.3 |
| Category 25/26/27 | Note1 |
| Category 28 (Note2) | A.3.2.13.1 |
| Category 29 (Note2) | A.3.2.14.1 |
| Category 30 (Note2) | A.3.2.15.1 |
| Note1: Category 25/26/27 Ues are configured to non-MIMO mode and the requirements of Category 18/21/24 Ues are applied respectively. Note2: The FRCs for the first stream is used. |  |

## 7.5	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity is a measure of a receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receiver filter attenuation on the adjacent channel(s).

### 7.5.1	Minimum Requirement

#### 7.5.1.1	3.84 Mcps TDD Option

The ACS shall be better than the value indicated in Table 7.4 for the test parameters specified in Table 7.5 where for non-IMB operation the BER shall not exceed 0.001 and for IMB operation, the BLER shall not exceed 0.01.

- Table 7.4: Adjacent Channel Selectivity (3.84 Mcps TDD Option)

| Power Class | Unit | ACS |
| --- | --- | --- |
| 2 | dB | 33 |
| 3 | dB | 33 |
| Note: For the case of an MBSFN-only UE, no power class may be applicable. In this case the same ACS requirement of 33dB shall apply. |  |  |

- Table 7.5: Test parameters for Adjacent Channel Selectivity (3.84 Mcps TDD Option)

| Parameter | Unit | Level |
| --- | --- | --- |
|  | dB | 0 * |
| Îor | dBm/3.84 MHz | -91 |
| Ioac mean power (modulated) | dBm | -52 |
| Fuw offset | MHz | +5 or -5 |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |

#### 7.5.1.2	1.28 Mcps TDD Option

The UE shall fulfil the minimum requirement specified in Table 7.4A for all values of an adjacent channel interferer up to –25 dBm.

However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5A where the BER shall not exceed 0.001.

- Table7.4A: Adjacent Channel Selectivity (1.28 Mcps TDD Option)

| Power Class | Unit | ACS |
| --- | --- | --- |
| 2 | dB | 33 |
| 3 | dB | 33 |
| Note: For the case of an MBSFN-only UE, no power class may be applicable. In this case the same ACS requirement of 33dB shall apply. |  |  |

- Table 7.5A: Test parameters for Adjacent Channel Selectivity (1.28 Mcps TDD Option)

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
|  | dB | 0 | 0 |
| Îor | dBm/1.28MHz | -91 | -62 |
| Ioac mean power (modulated) | dBm | -54 | -25 |
| Fuw offset | MHz | +1.6 or -1.6 | +1.6 or -1.6 |

#### 7.5.1.3	7.68 Mcps TDD Option

The ACS shall be better than the value indicated in Table 7.4B for the test parameters specified in 7.5B where the BER shall not exceed 0.001

- Table 7.4B: Adjacent Channel Selectivity (7.68 Mcps TDD Option)

| Power Class | Unit | ACS |
| --- | --- | --- |
| 2 | dB | 33 |
| 3 | dB | 33 |
| Note: For the case of an MBSFN-only UE, no power class may be applicable. In this case the same ACS requirement of 33dB shall apply. |  |  |

- Table 7.5B: Test parameters for Adjacent Channel Selectivity (7.68 Mcps TDD Option)

| Parameter | Unit | Level |
| --- | --- | --- |
|  | dB | 0 |
| Îor | dBm/7.68 MHz | -91 |
| Ioac mean power (modulated) | dBm | -52 |
| Fuw offset (3.84 Mcps Modulated) | MHz | +7.5 or -7.5 |
| Fuw offset (7.68 Mcps Modulated) | MHz | +10 or -10 |

### 7.5.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option

The ACS shall be better than the value indicated in table 7.4A for the test parameters specified in table 7.5AA where the BLER measured on each carrier shall not exceed 0.1.

- Table 7.5AA: Test parameters for Adjacent Channel Selectivity of multi-carrier reception

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Σ HS-PDSCH_EcIor | dB | 0 | 0 |
| Îor | dBm/1.28MHz | -87.8 | -58.8 |
| Ioac mean power (modulated) | dBm | -54 | -25 |
| Fuw offset(Note) | MHz | +1.6 or -1.6 | +1.6 or -1.6 |
| Note: Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used. |  |  |  |

## 7.6	Blocking characteristics

The blocking characteristics is a measure of the receiver ability to receive a wanted signal at is assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6.1	Minimum Requirement

#### 7.6.1.1	3.84 Mcps TDD Option

For non-IMB operation, the BER shall not exceed 0.001 for the parameters specified in table 7.6, table 7.7 and table 7.7AA. For IMB operation, the BLER shall not exceed 0.01 for the parameters specified in table 7.6, table 7.7 and table 7.7AA. For table 7.7 and 7.7AA up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. Additional requirement in table 7.7AA is applied for Band a) UE operating on 2010-2025MHz.

- Table 7.6: In-band blocking (3.84 Mcps TDD Option)

| Parameter | Level |  | Unit |
| --- | --- | --- | --- |
| ΣDPCHEcIor | 0 * |  | dB |
| Ior | -102 |  | dBm/3.84 MHz |
| Iouwmean power (modulated) | -56 (for Fuw offset ±10 MHz) | -44 (for Fuw offset ±15 MHz) | dBm |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |  |

- Table 7.7: Out of band blocking (3.84 Mcps TDD Option)

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
|  | 0 * | 0 * | 0 * | dB |
|  | -102 | -102 | -102 | dBm/3.84 MHz |
| (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands as definded in subclause 5.2(a) | 1840 <f <1885 1935 <f <1995 2040 <f <2085 | 1815 <f 1840 2085 f <2110 | 1< f 1815 2110 f <12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(b) | 1790 < f < 1835 2005 < f < 2050 | 1765 < f  1790 2050  f < 2075 | 1 < f  1765 2075  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(c) | 1850 < f < 1895 1945 < f < 1990 | 1825 < f  1850 1990  f < 2015 | 1 < f  1825 2015  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(d) | 2510 <f< 2555 2635 <f< 2680 | 2485 <f 2510 2680 f< 2705 | 1 <f 2485 2705 f< 12750 | MHz |
| 1. For operation referenced in 5.2(a), from 1885 f  1935 MHz, 1995 f 2040 MHz, the appropriate in-band blocking in table 7.6 or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |
| 2. For operation referenced in 5.2(b), from 1835  f  2005 MHz, the appropriate in-band blocking in table 7.6 or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |
| 3. For operation referenced in 5.2(c), from 1895  f  1945 MHz, the appropriate in-band blocking in table 7.6 or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |
| 4. For operation referenced in 5.2(d), from 2555  f  2635 MHz, the appropriate in-band blocking in table 7.6 or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |  |  |

- Table 7.7AA: Additional Out of band blocking (3.84 Mcps TDD Option)

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
| ΣDPCHEcIor | 0 * | 0 * | 0 * | dB |
| Ior | -102 | -102 | -102 | dBm/3.84 MHz |
| Iouw (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands in 2010-2025 MHz as definded in subclause 5.2(a) | 1840 <f <1995 2040 <f <2085 | 1815 <f 1840 2085 f <2110 | 1< f 1815 2110  f <12750 | MHz |
| NOTE 1: Additional requirement is applied for Band a) UE operating on 2010-2025MHz. NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |  |  |

#### 7.6.1.2	1.28 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in table 7.6A and table 7.7A. For table 7.7A up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size.

- Table 7.6A: In-band blocking (1.28 Mcps TDD Option)

| Parameter | Level |  | Unit |
| --- | --- | --- | --- |
|  | 0 |  | dB |
|  | -105 |  | dBm/1.28 MHz |
| mean power (modulated) | -61 (for Fuw offset ±3.2 MHz) | -49 (for Fuw offset ±4.8 MHz) | dBm |

- Table 7.7A: Out of band blocking (1.28 Mcps TDD Option)

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | dB |
|  | -105 | -105 | -105 | dBm/1.28 MHz |
| (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands as definded in subclause 5.2(a) | 1840 <f <1895.2 1924.8 <f <2005.2 2029.8 <f <2085 | 1815 <f 1840 2085 f <2110 | 1< f 1815 2110  f <12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(b) | 1790 < f < 1845.2 1914.8 < f < 1925.2 1994.8 < f < 2050 | 1765 < f  1790 2050  f < 2075 | 1 < f  1765 2075  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(c) | 1850 < f < 1905.2 1934.8 < f < 1990 | 1825 < f  1850 1990  f < 2015 | 1 < f  1825 2015  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(d) | 2510 < f < 2565.2 2624.8 < f < 2680 | 2485 < f  2510 2680  f < 2705 | 1< f  2485 2705  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(e) | 2240 < f < 2295.2 2404.8 < f < 2460 | 2215 < f  2240 2460  f < 2485 | 1< f  2215 2485  f < 12750 | MHz |
| Fuw For operation in frequency bands as defined in subclause 5.2(f) | 1820 < f <1875.2 1924.8 < f <1980 | 1795 < f  1820 1980  f < 2005 | 1 < f  1795 2005 < f <12750 | MHz |
| 1. For operation referenced in 5.2(a), from 1895.2 f  1924.8 MHz, 2005.2 f 2029.8 MHz, the appropriate in-band blocking in table 7.6A or adjacent channel selectivity in section 7.5.1.2shall be applied. |  |  |  |  |
| 2. For operation referenced in 5.2(b), from 1845.2  f < 1914.8 MHz, and 1925.2< f < 1994.8 MHz, the appropriate in-band blocking in table 7.6A or adjacent channel selectivity in section 7.5.1.2 shall be applied. |  |  |  |  |
| 3. For operation referenced in 5.2(c), from 1905.2  f  1934.8 MHz, the appropriate in-band blocking in table 7.6A or adjacent channel selectivity in section 7.5.1.2 shall be applied. |  |  |  |  |
| 4. For operation referenced in 5.2(d), from 2565.2  f  2624.8 MHz, the appropriate in-band blocking in table 7.6 or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |
| 5. For operation referenced in 5.2(e), from 2295.2  f  2404.8 MHz, the appropriate in-band blocking in table 7.6A or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |
| 6. For operation referenced in 5.2(f), from 1875.2  f  1924.8 MHz, the appropriate in-band blocking in table 7.6A or adjacent channel selectivity in section 7.5.1 shall be applied. |  |  |  |  |

#### 7.6.1.3	7.68 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in table 7.6B, 7.7B and table 7.7CC. For table 7.7B and 7.7CC up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. Additional requirement in table 7.7CC is applied for Band a) UE operating on 2010-2025MHz.

- Table 7.6B: In-band blocking

| Parameter | Level |  | Unit |
| --- | --- | --- | --- |
| ΣDPCHEcIor | 0 |  | dB |
| Ior | -102 |  | dBm/7.68 MHz |
| Iouwmean power (modulated) | -53 (for Fuw offset ±20 MHz) | -41 (for Fuw offset ±30 MHz) | dBm |

- Table 7.7B: Out of band blocking

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | dB |
|  | -102 | -102 | -102 | dBm/7.68 MHz |
| (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands as definded in subclause 5.2(a) | 1840 <f <1870 1950 <f <1980 2055 <f <2085 | 1815 <f 1840 2085 f <2110 | 1< f 1815 2110 f <12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(b) | 1790 < f < 1820 2020 < f < 2050 | 1765 < f  1790 2050  f < 2075 | 1 < f  1765 2075  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(c) | 1850 < f < 1880 1960 < f < 1990 | 1825 < f  1850 1990  f < 2015 | 1 < f  1825 2015  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(d) | 2510 <f< 2540 2650 <f< 2680 | 2485 <f 2510 2680 f< 2705 | 1 <f 2485 2705 f< 12750 | MHz |
| 1. For operation referenced in 5.2(a), from 1870 f  1950 MHz, 1980 f  2055 MHz, the appropriate in-band blocking in table 7.6B or adjacent channel selectivity in section 7.5.1.3 shall be applied. |  |  |  |  |
| 2. For operation referenced in 5.2(b), from 1820  f  2020 MHz, the appropriate in-band blocking in table 7.6B or adjacent channel selectivity in section 7.5.1.3 shall be applied. |  |  |  |  |
| 3. For operation referenced in 5.2(c), from 1880  f  1960 MHz, the appropriate in-band blocking in table 7.6B or adjacent channel selectivity in section 7.5.1.3 shall be applied. |  |  |  |  |
| 4. For operation referenced in 5.2(d), from 2540  f  2650 MHz, the appropriate in-band blocking in table 7.6B or adjacent channel selectivity in section 7.5.1.3 shall be applied. |  |  |  |  |

- Table 7.7CC: Additional Out of band blocking (7.68 Mcps TDD Option)

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
| ΣDPCHEcIor | 0 | 0 | 0 | dB |
| Ior | -102 | -102 | -102 | dBm/3.84 MHz |
| Iouw (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands in 2010-2025 MHz as definded in subclause 5.2(a) | 1840 <f <1995 2040 <f <2085 | 1815 <f 1840 2085 f <2110 | 1< f 1815 2110  f <12750 | MHz |
| NOTE 1: Additional requirement is applied for Band a) UE operating on 2010-2025MHz. |  |  |  |  |

### 7.6.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option

The BLER measured on each carrier shall not exceed 0.1 for the parameters specified in table 7.6AA and table 7.7AAA. For table 7.7AAA up to 24 exceptions for each carrier are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size.

- Table 7.6AA: In-band blocking of multi-carrier reception

| Parameter | Level |  | Unit |
| --- | --- | --- | --- |
| Σ HS-PDSCH_EcIor | 0 |  | dB |
|  | -101.8 |  | dBm/1.28 MHz |
| mean power (modulated) | -61 (for Fuw offset ±3.2 MHz)(Note) | -49 (for Fuw offset ±4.8 MHz)(Note) | dBm |
| Note: Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used. |  |  |  |

- Table 7.7AAA: Out of band blocking of multi-carrier reception

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | dB |
|  | -101.8 | -101.8 | -101.8 | dBm/1.28 MHz |
| (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands as definded in subclause 5.2(a) | 1840 <f <1895.2 1924.8 <f <2005.2 2029.8 <f <2085 | 1815 <f 1840 2085 f <2110 | 1< f 1815 2110  f <12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(b) | 1790 < f < 1845.2 1914.8 < f < 1925.2 1994.8 < f < 2050 | 1765 < f  1790 2050  f < 2075 | 1 < f  1765 2075  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(c) | 1850 < f < 1905.2 1934.8 < f < 1990 | 1825 < f  1850 1990  f < 2015 | 1 < f  1825 2015  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(d) | 2510 < f < 2565.2 2624.8 < f < 2680 | 2485 < f  2510 2680  f < 2705 | 1< f  2485 2705  f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(e) | 2240 < f < 2295.2 2404.8 < f < 2460 | 2215 < f  2240 2460  f < 2485 | 1< f  2215 2485  f < 12750 | MHz |
| Note: 1. For operation referenced in 5.2(a), from 1895.2 f  1924.8 MHz, 2005.2 f 2029.8 MHz, the appropriate in-band blocking in table 7.6AA or adjacent channel selectivity in section 7.5.2 shall be applied. |  |  |  |  |
| 2. For operation referenced in 5.2(b), from 1845.2  f < 1914.8 MHz, and 1925.2< f < 1994.8 MHz, the appropriate in-band blocking in table 7.6AA or adjacent channel selectivity in section 7.5.2 shall be applied. |  |  |  |  |
| 3. For operation referenced in 5.2(c), from 1905.2  f  1934.8 MHz, the appropriate in-band blocking in table 7.6AA or adjacent channel selectivity in section 7.5.2 shall be applied. |  |  |  |  |
| 4. For operation referenced in 5.2(d), from 2565.2  f  2624.8 MHz, the appropriate in-band blocking in table 7.6AA or adjacent channel selectivity in section 7.5.2 shall be applied. |  |  |  |  |
| 5. For operation referenced in 5.2(e), from 2295.2  f  2404.8 MHz, the appropriate in-band blocking in table 7.6AA or adjacent channel selectivity in section 7.5.2 shall be applied. |  |  |  |  |

## 7.7	Spurious response

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the blocking limit is not met.

### 7.7.1	Minimum Requirement

#### 7.7.1.1	3.84 Mcps TDD Option

For non-IMB operation, the BER shall not exceed 0.001 for the parameters specified in Table 7.8.

For IMB operation, the BLER shall not exceed 0.01 for the parameters specified in Table 7.8.

- Table 7.8: Spurious Response (3.84 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 * | dB |
|  | -102 | dBm/3.84 MHz |
| (CW) | -44 | dBm |
| Fuw | Spurious response frequencies | MHz |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |

#### 7.7.1.2	1.28 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in Table 7.8A.

- Table 7.8A:	Spurious Response (1.28 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
|  | -105 | dBm/1.28 MHz |
| (CW) | -44 | dBm |
| Fuw | Spurious response frequencies | MHz |

#### 7.7.1.3	7.68 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in Table 7.8B.

- Table 7.8B: Spurious Response

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
|  | -102 | dBm/7.68 MHz |
| (CW) | -44 | dBm |
| Fuw | Spurious response frequencies | MHz |

### 7.7.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option

The BLER measured on each carrier shall not exceed 0.1 for the parameters specified in Table 7.8AA.

- Table 7.8AA:	Spurious Response of multi-carrier reception

| Parameter | Level | Unit |
| --- | --- | --- |
| Σ HS-PDSCH_EcIor | 0 | dB |
|  | -101.8 | dBm/1.28 MHz |
| (CW) | -44 | dBm |
| Fuw | Spurious response frequencies | MHz |

## 7.8	Intermodulation characteristics

- Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

### 7.8.1	Minimum Requirements

#### 7.8.1.1	3.84 Mcps TDD Option

For non-IMB operation, the BER shall not exceed 0.001 for the parameters specified in table 7.9.

For IMB operation, the BLER shall not exceed 0.01 for the parameters specified in table 7.9.

- Table 7.9: Receive intermodulation characteristics

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 * | dB |
| Îor | -102 | dBm/3.84 MHz |
| Iouw1 (CW) | -46 | dBm |
| Iouw2 mean power (modulated) | -46 | dBm |
| Fuw1 (CW) | ±10 | MHz |
| Fuw2 (modulated) | ±20 | MHz |
| NOTE *: Subtract 0.77dB when using the IMB DL reference measurement channel. For IMB the term DPCH_Ec refers to the sum of the energy of the physical channels comprising the IMB DL reference measurement channel. |  |  |

#### 7.8.1.2	1.28 Mcps TDD Option

- The BER shall not exceed 0.001 for the parameters specified in table 7.9A

- Table 7.9A: Receive intermodulation characteristics (1.28 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
| Îor | -105 | dBm/1.28 MHz |
| Iouw1 (CW) | -46 | dBm |
| Iouw2 mean power (modulated) | -46 | dBm/1.28 MHz |
| Fuw1 (CW) | ±3.2 | MHz |
| Fuw2 (modulated) | ±6.4 | MHz |

#### 7.8.1.3	7.68 Mcps TDD Option

The BER shall not exceed 0.001 for the parameters specified in table 7.9B.

- Table 7.9B: Receive intermodulation characteristics

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
| Îor | -102 | dBm/7.68 MHz |
| Iouw1 (CW) | -46 | dBm |
| Iouw2 mean power (modulated) | -46 | dBm |
| Fuw1 (CW) | ±20 | MHz |
| Fuw2 (modulated) | ±40 | MHz |

### 7.8.2	Additional requirement of multi-carrier reception for 1.28Mcps TDD Option

- The BLER measured on each carrier shall not exceed 0.1 for the parameters specified in table 7.9AA

- Table 7.9AA: Receive intermodulation characteristics of multi-carrier reception

| Parameter | Level | Unit |
| --- | --- | --- |
| Σ HS-PDSCH_EcIor | 0 | dB |
| Îor | -101.8 | dBm/1.28 MHz |
| Iouw1 (CW) | -46 | dBm |
| Iouw2 mean power (modulated) | -46 | dBm/1.28 MHz |
| Fuw1 (CW)(Note) | ±3.2 | MHz |
| Fuw2 (modulated) | ±6.4 | MHz |
| Note: Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used. |  |  |

## 7.9	Spurious emissions

The Spurious Emissions Power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

### 7.9.1	Minimum Requirement

#### 7.9.1.1	3.84 Mcps TDD Option

The power of any spurious emission shall not exceed:

- Table 7.10: Receiver spurious emission requirements (3.84 Mcps TDD Option)

| Band | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz - 1.9 GHz and 1.92 GHz - 2.01 GHz and 2.025 GHz - 2.11 GHz and 2.17 GHz - 2.57 GHz | -47 dBm | 1 MHz |  |
| 1.9 GHz - 1.92 GHz and 2.01 GHz - 2.025 GHz and 2.11 GHz - 2.170 GHz and 2.57 GHz - 2.69 GHz | -60 dBm | 3.84 MHz |  |
| 2.69 GHz - 12.75 GHz | -47 dBm | 1 MHz |  |

#### 7.9.1.2	1.28 Mcps TDD Option

The power of any spurious emission shall not exceed the maximum level specified in Table 7.10A-1 and Table 7.10A-2:

- Table 7.10A-1: Receiver spurious emission requirements (1.28 Mcps TDD Option)

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz  f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz  f  12.75 GHz | 1 MHz | -47 dBm |  |

- Table 7.10A-2: Additional receiver spurious emission requirements (1.28 Mcps TDD Option)

| Band | Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- | --- |
| a | 703 MHz  f < 803 MHz | 1 MHz | -50 dBm |  |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm | Note 1 |
|  | 925 MHz  f  935 MHz | 100 kHz | -67 dBm | Note 1 |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm | Note 1 |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm | Note 1 |
|  | 2010 MHz  f 2025 MHz | 1MHz | -65dBm | Note 2 |
|  | 2300 MHz  f 2400 MHz | 1MHz | -65dBm | Note 3 |
|  | 1880 MHz  f 1920 MHz | 1MHz | -65dBm | Note 3 |
|  | 2496 MHz  f 2690 MHz | 1MHz | -50dBm |  |
|  | 3400 MHz f < 3600 MHz | 1 MHz | -50 dBm |  |
| b | 1850 MHz  f 1910 MHz | 1.28MHz | -64dBm |  |
|  | 1910 MHz  f 1990 MHz | 1.28MHz | -64dBm |  |
| c | 1910 MHz  f 1930 MHz | 1.28MHz | -64dBm |  |
| d | 2570 MHz  f 2620 MHz | 1.28MHz | -64dBm |  |
|  | 2010 MHz  f 2025 MHz | 1.28MHz | -64dBm |  |
|  | 2110 MHz  f 2170 MHz | 3.84MHz | -60dBm |  |
|  | 2620 MHz  f 2690 MHz | 3.84MHz | -60dBm |  |
| e | 703 MHz  f < 803 MHz | 1 MHz | -50 dBm |  |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm | Note 1 |
|  | 925 MHz  f  935 MHz | 100 kHz | -67 dBm | Note 1 |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm | Note 1 |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm | Note 1 |
|  | 2300 MHz  f 2400 MHz | 1.28MHz | -64dBm |  |
|  | 2010 MHz  f 2025 MHz | 1MHz | -65dBm |  |
|  | 1880 MHz  f 1920 MHz | 1MHz | -65dBm |  |
|  | 2496 MHz  f 2690 MHz | 1MHz | -50dBm |  |
|  | 3400 MHz  f < 3600 MHz | 1 MHz | -50 dBm |  |
| f | 703 MHz  f < 803 MHz | 1 MHz | -50 dBm |  |
|  | 921 MHz f<925 MHz | 100 kHz | -60 dBm | Note 1 |
|  | 925 MHz< f <935 MHz | 100 kHz | -67 dBm | Note 1 |
|  | 935 MHz< f <960 MHz | 100 kHz | -79 dBm | Note 1 |
|  | 1805 MHz  f  1850 MHz | 100 kHz | -71 dBm | Note 1 |
|  | 1880 MHz  f 1920 MHz | 1MHz | -65dBm |  |
|  | 2010 MHz  f 2025 MHz | 1MHz | -65dBm |  |
|  | 2300 MHz  f 2400 MHz | 1.28MHz | -65dBm |  |
|  | 2496 MHz  f 2690 MHz | 1MHz | -50dBm |  |
|  | 3400 MHz  f < 3600 MHz | 1 MHz | -50 dBm |  |
| Note 1 The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.7c are permitted for each UARFCN used in the measurement. Note 2: This requirement is only applicable when UE operating in 1900-1920MHz of band a. Note 3: This requirement is only applicable when UE operating in 2010-2025MHz of band a. |  |  |  |  |

#### 7.9.1.3	7.68 Mcps TDD Option

The power of any spurious emission shall not exceed:

- Table 7.10B: Receiver spurious emission requirements

| Band | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz - 1.9 GHz and 1.92 GHz - 2.01 GHz and 2.025 GHz - 2.11 GHz 2.17 GHz - 2.57 GHz | -47 dBm | 1 MHz |  |
| 1.9 GHz - 1.92 GHz and 2.01 GHz - 2.025 GHz and 2.11 GHz - 2.170 GHz 2.57 Ghz - 2.69 GHz | -57 dBm | 7.68 MHz |  |
| 2.69 GHz - 12.75 GHz | -47 dBm | 1 MHz |  |

# 8	Performance requirement

## 8.1	General

The performance requirements for the UE in this section are specified for the measurement channels specified in Annex A and the propagation condition specified in Annex B. Unless otherwise stated the receiver characteristics are specified at the antenna connector of the UE. For UE(s) with an integral antenna only, a reference antenna with a gain of 0 dBi is assumed. UE with an integral antenna may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For Ues with more than one receiver antenna connector the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

- Table 8.1: Summary of UE performance targets

| Test Chs. | Information Data Rate | Static | Multi-path Case 1 | Multi-path Case 2 | Multi-path Case 3 | High speed train* |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER< 10-2 |
|  | 64 kbps | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER<10-1, 10-2, 10-3 | BLER< 10-1, 10-2 |
|  | 144 kbps | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER<10-1, 10-2, 10-3 | - |
|  | 384 kbps | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER<10-1, 10-2, 10-3 | - |
|  | 2048 kbps | BLER < 10-1, 10-2 | BLER< 10-1, 10-2 | BLER< 10-1, 10-2 | BLER<10-1, 10-2, 10-3 | - |
| BCH | 12.3kbps |  | BLER< 10-2 |  |  | - |

## 8.2	Demodulation in static propagation conditions

### 8.2.1	Demodulation of DCH

The performance requirement of DCH in static propagation conditions is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

#### 8.2.1.1	Minimum requirement

##### 8.2.1.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 8.2 the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.3. These requirements are applicable for TFCS size 16.

- Table 8.2: DCH parameters in static propagation conditions (3.84 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- | --- |
|  | dB | -6 | -3 | 0 | 0 | 0 |
| Ioc | dBm/3.84 MHz | -60 |  |  |  |  |
| Cell Parameter* |  | 0,1 |  |  |  | - |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1 . .5 | C(i,16) i=1 . .9 | C(i,16) i=1 . .8 | - |
| OCNS Channelization Code* | C(k,Q) | C(3,16) | C(6,16) | - | - | - |
| Information Data Rate | kbps | 12.2 | 64 | 144 | 384 | 2048 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |  |  |  |  |

- Table 8.3: Performance requirements in AWGN channel (3.84 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 1.1 | 10-2 |
| 2 | 3.5 | 10-1 |
|  | 3.8 | 10-2 |
| 3 | 3.4 | 10-1 |
|  | 3.6 | 10-2 |
| 4 | 2.7 | 10-1 |
|  | 3.0 | 10-2 |
| 5 | 3.5 | 10-1 |
|  | 3.6 | 10-2 |

##### 8.2.1.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.2A the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.3A.

- Table 8.2A: DCH parameters in static propagation conditions (1.28 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Number of DPCHo |  | 8 | 2 | 2 | 0 |
| Scrambling code and basic midamble code number* |  | 0 | 0 | 0 | 0 |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1…8 | C(i,16) i=1…8 | C(i,16) i=1…9 |
| DPCHo Channelization Codes* | C(k,Q) | C(i,16) 3≤ i ≤10 | C(i,16) 9≤ i ≤10 | C(i,16) 9≤ i ≤10 | - |
|  | dB | -10 | -10 | -10 | 0 |
| Ioc | DBm/1.28MHz | -60 |  |  |  |
| Information Data Rate | Kbps | 12.2 | 64 | 144 | 384 |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 8.3A: Performance requirements in AWGN channel (1.28 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 3.6 | 10-2 |
| 2 | 2.4 | 10-1 |
|  | 2.7 | 10-2 |
| 3 | 2.8 | 10-1 |
|  | 3.2 | 10-2 |
| 4 | 4.6 | 10-1 |

##### 8.2.1.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 8.2B the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.3B. These requirements are applicable for TFCS size 16.

- Table 8.2B: DCH parameters in static propagation conditions (7.68 Mcps TDD Option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
|  | dB | -9 |
| Ioc | dBm/7.68 MHz | -60 |
| Cell Parameter* | - | 0,1 |
| DPCH Channelization Codes* | C(k, Q) | C(i, 32), i = 1,2 |
| OCNS Channelization Code* | C(k, Q) | C(3, 32) |
| Information Data Rate | kbps | 12.2 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |

- Table 8.3B: Performance requirements in AWGN channel (7.68 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 1.1 | 10-2 |

## 8.3	Demodulation of DCH in multipath fading conditions

### 8.3.1	Multipath fading Case 1

The performance requirement of DCH is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

#### 8.3.1.1	Minimum requirement

##### 8.3.1.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 8.4 the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.5. These requirement are applicable for TFCS size 16.

- Table 8.4: DCH parameters in multipath Case 1 channel (3.84 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- | --- |
|  | DB | -6 | -3 | 0 | 0 | 0 |
| Ioc | dBm/3.84 MHz | -60 |  |  |  |  |
| Cell Parameter* |  | 0,1 |  |  |  | - |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1 . .5 | C(i,16) i=1 . .9 | C(i,16) i=1 . .8 | - |
| OCNS Channelization Code* | C(k,Q) | C(3,16) | C(6,16) | - | - | - |
| Information Data Rate | kbps | 12.2 | 64 | 144 | 384 | 2048 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |  |  |  |  |

- Table 8.5: Performance requirements in multipath Case 1 channel (3.84 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 13.9 | 10-2 |
| 2 | 13.7 | 10-1 |
|  | 19.8 | 10-2 |
| 3 | 14.1 | 10-1 |
|  | 20.6 | 10-2 |
| 4 | 13.8 | 10-1 |
|  | 20.0 | 10-2 |
| 5 | 13.2 | 10-1 |
|  | 17.8 | 10-2 |

##### 8.3.1.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.4A the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.5A.

- Table 8.4A: DCH parameters in multipath Case 1 channel (1.28 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Number of DPCHo |  | 8 | 2 | 2 | 0 |
| Scrambling code and basic midamble code number* |  | 0 | 0 | 0 | 0 |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1…8 | C(i,16) i=1…8 | C(i,16) i=1…9 |
| DPCHo Channelization Codes* | C(k,Q) | C(i,16) 3≤ i ≤10 | C(i,16) 9≤ i ≤10 | C(i,16) 9≤ i ≤10 | - |
|  | DB | -10 | -10 | -10 | 0 |
| Ioc | dBm/1.28MHz | -60 |  |  |  |
| Information Data Rate | Kbps | 12.2 | 64 | 144 | 384 |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 8.5A: Performance requirements in multipath Case 1 channel (1.28 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 22.4 | 10-2 |
| 2 | 15.8 | 10-1 |
|  | 22.9 | 10-2 |
| 3 | 16.6 | 10-1 |
|  | 23.9 | 10-2 |
| 4 | 15.6 | 10-1 |
|  | 21.4 | 10-2 |

##### 8.3.1.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 8.4B the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.5B. These requirement are applicable for TFCS size 16.

- Table 8.4B: DCH parameters in multipath Case 1 channel (7.68 Mcps TDD Option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
|  | dB | -9 |
| Ioc | dBm/7.68 MHz | -60 |
| Cell Parameter* | - | 0,1 |
| DPCH Channelization Codes* | C(k, Q) | C(i, 32), i = 1,2 |
| OCNS Channelization Code* | C(k, Q) | C(3, 32) |
| Information Data Rate | kbps | 12.2 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |

- Table 8.5B: Performance requirements in multipath Case 1 channel (7.68 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 13.9 | 10-2 |

### 8.3.2	Multipath fading Case 2

The performance requirement of DCH is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

#### 8.3.2.1	Minimum requirement

##### 8.3.2.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 8.6 the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.7. These requirements are applicable for TFCS size 16.

- Table 8.6: DCH parameters in multipath Case 2 channel (3.84 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- | --- |
|  | DB | -3 | 0 | 0 | 0 | 0 |
| Ioc | dBm/3.84 MHz | -60 |  |  |  |  |
| Cell Parameter* |  | 0,1 |  |  |  | - |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1 . .5 | C(i,16) i=1 . .9 | C(i,16) i=1 . .8 | - |
| OCNS Channelization Code* | C(k,Q) | C(3,16) | - | - | - | - |
| Information Data Rate | kbps | 12.2 | 64 | 144 | 384 | 2048 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |  |  |  |  |

- Table 8.7: Performance requirements in multipath Case 2 channel (3.84 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 5.8 | 10-2 |
| 2 | 5.7 | 10-1 |
|  | 9.2 | 10-2 |
| 3 | 9.3 | 10-1 |
|  | 12.7 | 10-2 |
| 4 | 8.8 | 10-1 |
|  | 12.0 | 10-2 |
| 5 | 10.3 | 10-1 |
|  | 12.7 | 10-2 |

##### 8.3.2.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.6A the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.7A.

- Table 8.6A: DCH parameters in multipath Case 2 channel (1.28 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Number of DPCHo |  | 8 | 2 | 2 | 0 |
| Scrambling code and basic midamble code number* |  | 0 | 0 | 0 | 0 |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1…8 | C(i,16) i=1…8 | C(i,16) i=1…9 |
| DPCHo Channelization Codes* | C(k,Q) | C(i,16) 3≤ i ≤10 | C(i,16) 9≤ i ≤10 | C(i,16) 9≤ i ≤10 | - |
|  | dB | -10 | -10 | -10 | 0 |
| Ioc | dBm/1.28MHz | -60 |  |  |  |
| Information Data Rate | Kbps | 12.2 | 64 | 144 | 384 |
| *Note Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 8.7A: Performance requirements in multipath Case 2 channel (1.28 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 13.6 | 10-2 |
| 2 | 9.8 | 10-1 |
|  | 13.9 | 10-2 |
| 3 | 10.3 | 10-1 |
|  | 14.4 | 10-2 |
| 4 | 11.4 | 10-1 |
|  | 15.0 | 10-2 |

##### 8.3.2.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 8.6B the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.7B. These requirements are applicable for TFCS size 16.

- Table 8.6B: DCH parameters in multipath Case 2 channel (7.68 Mcps TDD Option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
|  | dB | -6 |
| Ioc | dBm/7.68 MHz | -60 |
| Cell Parameter (note) | - | 0,1 |
| DPCH Channelization Codes (note) | C(k, Q) | C(i, 32), i = 1,2 |
| OCNS Channelization Code (note) | C(k, Q) | C(3, 32) |
| Information Data Rate | kbps | 12.2 |
| NOTE: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |

- Table 8.7B: Performance requirements in multipath Case 2 channel (7.68 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 5.8 | 10-2 |

### 8.3.3	Multipath fading Case 3

The performance requirement of DCH is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

#### 8.3.3.1	Minimum requirement

##### 8.3.3.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 8.8 the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.9. These requirements are applicable for TFCS size 16.

- Table 8.8: DCH parameters in multipath Case 3 channel (3.84 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- | --- |
|  | dB | -3 | 0 | 0 | 0 | 0 |
| Ioc | dBm/3.84 MHz | -60 |  |  |  |  |
| Cell Parameter* |  | 0,1 |  |  |  | - |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1 . .5 | C(i,16) i=1 . .9 | C(i,16) i=1 . .8 | - |
| OCNS Channelization Code* | C(k,Q) | C(3,16) | - | - | - | - |
| Information Data Rate | kbps | 12.2 | 64 | 144 | 384 | 2048 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |  |  |  |  |

- Table 8.9: Performance requirements in multipath Case 3 channel (3.84 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 4.8 | 10-2 |
| 2 | 5.8 | 10-1 |
|  | 8.5 | 10-2 |
|  | 10.7 | 10-3 |
| 3 | 10.3 | 10-1 |
|  | 13.3 | 10-2 |
|  | 16.0 | 10-3 |
| 4 | 8.9 | 10-1 |
|  | 11.5 | 10-2 |
|  | 13.6 | 10-3 |
| 5 | 9.4 | 10-1 |
|  | 11.5 | 10-2 |
|  | 13.6 | 10-3 |

##### 8.3.3.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.8A the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.9A.

- Table 8.8A: DCH parameters in multipath Case 3 channel (1.28 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Number of DPCHo |  | 8 | 2 | 2 | 0 |
| Scrambling code and basic midamble code number* |  | 0 | 0 | 0 | 0 |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1…8 | C(i,16) i=1…8 | C(i,16) i=1…9 |
| DPCHo Channelization Codes* | C(k,Q) | C(i,16) 3≤ i ≤10 | C(i,16) 9≤ i ≤10 | C(i,16) 9≤ i ≤10 | - |
|  | dB | -10 | -10 | -10 | 0 |
| Ioc | dBm/1.28MHz | -60 |  |  |  |
| Information Data Rate | Kbps | 12.2 | 64 | 144 | 384 |
| *Note Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 8.9A: Performance requirements in multipath Case 3 channel (1.28 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 11.7 | 10-2 |
| 2 | 9.0 | 10-1 |
|  | 11.7 | 10-2 |
|  | 14.3 | 10-3 |
| 3 | 9.1 | 10-1 |
|  | 11.2 | 10-2 |
|  | 12.7 | 10-3 |
| 4 | 9.9 | 10-1 |
|  | 11.2 | 10-2 |
|  | 12.4 | 10-3 |

##### 8.3.3.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 8.8B the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.9B. These requirements are applicable for TFCS size 16.

- Table 8.8B: DCH parameters in multipath Case 3 channel (7.68 Mcps TDD Option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
|  | dB | -6 |
| Ioc | dBm/7.68 MHz | -60 |
| Cell Parameter* | - | 0,1 |
| DPCH Channelization Codes* | C(k, Q) | C(i, 32), i = 1,2 |
| OCNS Channelization Code* | C(k, Q) | C(3, 32) |
| Information Data Rate | kbps | 12.2 |
| *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |

- Table 8.9B: Performance requirements in multipath Case 3 channel (7.68 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 4.8 | 10-2 |

## 8.3A	Demodulation of DCH in high speed train condition

### 8.3A.1	General

The performance requirement of DCH in high speed train conditions is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

### 8.3A.2	Minimum requirement

##### 8.3A.2.1	3.84 Mcps TDD Option

<void>

##### 8.3A.2.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.9C, the average downlink IorIoc power ratio shall be below the specified BLER shown in Table 8.9D.

- Table 8.9C: DCH parameters in high speed train condition

| - Parameters | - Unit | - Test 1 | - Test 2 |
| --- | --- | --- | --- |
| - Number of DPCHo |  | - 8 | - 2 |
| - Scrambling code and basic midamble code number* |  | - 0 | - 0 |
| - DPCH Channelization Codes* | - C(k,Q) | - C(i,16) - i=1,2 | - C(i,16) - i=1…8 |
| - DPCHo Channelization Codes* | - C(k,Q) | - C(i,16) - 3≤ i ≤10 | - C(i,16) - 9≤ i ≤10 |
|  | - dB | - -10 | - -10 |
| - Ioc | - dBm/1.28MHz | - -60 |  |
| - Information Data Rate | - Kbps | - 12.2 | - 64 |
| - *Note: Refer to TS 25.223 for definition of channelization codes and cell parameter. |  |  |  |

- Table 8.9D: DCH requirements in high speed train condition

| Test Number | IorIoc | BLER |
| --- | --- | --- |
| 1 | 8.5 | 10-2 |
| 2 | 6.2 | 10-1 |
|  | 8.5 | 10-2 |

#### 8.3A.2.3	7.68 Mcps TDD Option

<void>

## 8.4	Base station transmit diversity mode for 3.84 Mcps TDD Option

### 8.4.1	Demodulation of BCH in SCTD mode

The performance requirement of BCH is determined by the maximum Block Error Rate (BLER). The BLER is specified for the BCH. BCH is mapped into the Primary Common Control Physical Channel (P-CCPCH).

#### 8.4.1.1	Minimum requirement

For the parameters specified in Table 8.10 the BLER should not exceed the BLER specified in Table 8.11.

NOTE:	This requirement doesn't need to be tested.

- Table 8.10: P-CCPCH parameters in multipath Case 1 channel

| Parameters | Unit | Test 1 |
| --- | --- | --- |
|  | dB | -3 |
| I | dBm/3.84 MHz | -60 |
| Information Data Rate | Kbps | 12.3 |

- Table 8.11: Performance requirements in multipath Case 1 channel

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 8.4 | 10-2 |

## 8.5	Power control in downlink

Power control in the downlink is the ability of the UE receiver to converge to the required link quality set by the network while using minimum downlink power.

### 8.5.1	Power control in downlink, constant BLER target

#### 8.5.1.1	Minimum requirements 3.84 Mcps TDD option

For the parameters specified in Table 8.12 the downlink  averaged over one timeslot shall be below the specified value in Table 8.13 more than 90% of the time. BLER shall be as shown in Table 8.13. Downlink power control is ON during the test.

Table 8.12: Test parameters for downlink power control - constant BLER Target (3.84 Mcps TDD option)

| Parameter | Unit | Test 1 |
| --- | --- | --- |
|  | dB | 0 |
|  | dBm/3.84 MHz | -60 |
| Information Data Rate | kbps | 12.2 |
| Target quality value on DTCH | BLER | 0.01 |
| Propagation condition |  | Case 1 |
| DL Power Control step size, TPC | dB | 1 |
| Maximum_DL_power * | dB | 0 |
| Minimum_DL_power * | dB | -27 |
| *Note: Refer to TS 25.224 for description and definition |  |  |

- Table 8.13: Requirements for downlink power control - constant BLER Target (3.84 Mcps TDD option)

| Parameter | Unit | Test 1 |
| --- | --- | --- |
|  | dB | 8.5 |
| Measured quality on DTCH | BLER | 0.01±30% |

#### 8.5.1.2	Minimum requirements 1.28 Mcps TDD option

For the parameters specified in Table 8.13A the downlink  averaged over one timeslot, shall be below the specified value in Table 8.13B more than 90% of the time. BLER shall be as shown in table 8.13B. Downlink power control is ON during the test.

Table 8.13A: Test parameters for downlink power control - constant BLER Target (1.28 Mcps TDD option)

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
|  | dBm/1.28 Mhz | -60 | -60 | -60 | -60 | -60 | -60 |
| Information data rate | kbps | 12.2 | 12.2 | 64 | 64 | 64 | 64 |
| Target quality on DTCH | BLER | 0.01 | 0.01 | 0.1 | 0.1 | 0.001 | 0.001 |
| Propagation condition |  | Case 1 | Case 3 | Case 1 | Case 3 | Case 1 | Case 3 |
| DL Power Control step size, TPC | dB | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum_DL_power * | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Minimum_DL_power * | dB | -27 | -27 | -27 | -27 | -27 | -27 |

- NOTE:	Power is compared to P-CCPCH power

Table 8.13B: Requirements for downlink power control - constant BLER Target (1,28 Mcps TDD option)

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | dB | 7.5 | 4.8 | 9.1 | 8.9 | 17.9 | 13.1 |
| Measured quality on DTCH | BLER | 0.01±30% | 0.01±30% | 0.1±30% | 0.1±30% | 0.001±30% | 0.001±30% |

### 8.5.2	Power control in downlink, wind up effects

#### 8.5.2.1	Minimum requirements 3.84 Mcps TDD option

Void

#### 8.5.2.2	Minimum requirements 1.28 Mcps TDD option

This test is run in three stages where stage 1 is for convergence of the power control loop. In stage two the maximum downlink power for the dedicated channel is limited not to be higher than the value specified in Table 8.13C. All parameters used in the three stages are specified in Table 8.13C. The downlinkI Ior/Ioc power ratio measured values, which are averaged over one timeslot, during stage 3 shall be lower than the value specified in Table 8.13D more than 90% of the time.

Power control of the UE is ON during the test.

- Table 8.13C: Test parameter for downlink power control, wind-up effects

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Stage 1 | Stage 2 | Stage 3 |
| Time in each stage | s | 5 | 40 | 5 |
|  | dBm/1.28 MHz | -60 |  |  |
| Information Data Rate | kbps | 12.2 |  |  |
| Quality target on DTCH | BLER | 0.01 |  |  |
| Propagation condition |  | Case 1 |  |  |
| Maximum_DL_Power | dB | 0 | P( Note 1) | 0 |
| Minimum_DL_Power | dB | -27 |  |  |
| DL Power Control step size, TPC | dB | 1 |  |  |
| Note 1: P is the level corresponding to the average Ior/Ioc power ratio - 3 dB compared to the P-CCPCH level. The average Ioc/Ioc power ratio is measured during the initialisation stage after the power control loop has converged before the actual test starts. |  |  |  |  |

- Table 8.13D: Requirements in downlink power control, wind-up effects

| Parameter | Unit | Test 1, stage 3 |
| --- | --- | --- |
| Ior/Ioc | dB | 9.1 |

### 8.5.3	Power control in the downlink, initial convergence

This requirement verifies that DL power control works properly during the first seconds after DPCH connection is established

#### 8.5.3.1	Minimum requirements 3.84 Mcps TDD option

Void

#### 8.5.3.2	Minimum requirements 1.28 Mcps TDD option

For the parameters specified in Table 8.13E the downlink Ior/Ioc power ratio measured values, which are averaged over 50 ms, shall be within the range specified in Table 8.13F more than 90% of the time. T1 equals to 5 s and it starts 100 ms after the DPCH physical channel is considered established and the first uplink frame is transmitted. T2 equals to 5 s and it starts when T1 has expired. Power control is ON during the test.

The first 100 ms shall not be used for averaging, ie the first sample to be input to the averaging filter is at the beginning of T1. The averaging shall be performed with a sliding rectangular window averaging filter. The window size of the averaging filter is linearly increased from 0 up to 50 ms during the first 50 ms of T1, and then kept equal to 50ms.

- Table 8.13E: Test parameters for downlink power control, initial convergence

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Target quality value on DTCH | BLER | 0.01 | 0.01 | 0.1 | 0.1 |
| Initial Ior/Ioc | dB | 5 | -15 | 9.4 | -10.6 |
| Information Data Rate | kbps | 12.2 | 12.2 | 64 | 64 |
|  | dBm/1.28 MHz | -60 |  |  |  |
| Propagation condition |  | Static |  |  |  |
| Maximum_DL_Power | dB | 0 |  |  |  |
| Minimum_DL_Power | dB | -27 |  |  |  |
| DL Power Control step size, TPC | dB | 1 |  |  |  |

- Table 8.13F: Requirements in downlink power control, initial convergence

| Parameter | Unit | Test 1 and Test 2 | Test 3 and Test 4 |
| --- | --- | --- | --- |
| Ior/Iocduring T1 | dB | -8.5  Ior/Ioc  0 | -4.1  Ior/Ioc  4.4 |
| Ior/Iocduring T2 | dB | -8.5  Ior/Ioc  -3 | -4.1  Ior/Ioc  1.4 |

## 8.6	Uplink Power Control for 3.84 Mcps TDD Option

Power control in the uplink is the ability of the UE to converge to the required link quality set by the network while using minimum uplink power.

### 8.6.1	Test Conditions

During period T1, the PCCPCH and a second Beacon Channel are transmitted in the DL in designated slots within each frame and at the same power level.

The UE transmits, using the channel of TS25.105, Annex A.2.1 UL reference measurement channel (12.2 kbps) in one UL slot. For different parts of the test, different UL slots will be designated.

The values of table 8.14, period T1 shall be selected. Then, with the received PCCPCH and Beacon power set at -60 dBm, the value of DPCH constant value shall be adjusted so that the mean UE output power is 5 dBm. These conditions are held steady during period T1.

Periods T1 and T2 are each 5 seconds long.

- Table 8.14: UL Power Control Test Conditions

|  |  | Period T1 | Period T2 |
| --- | --- | --- | --- |
| IBTS all slots | dBm | -60 |  |
| PCCPCH Power -Broadcast | dBm | 18 |  |
| PCCPCH power - Received | dBm | -60 | -70 |
| Mean UE transmit power | dBm | 5 | According to tables 8.15 and 8.16 |
| SIRTARGET | dB | 6 |  |
| Ioc in PCCPCH and Beacon Slots | dBm | -60 |  |
| IE (information element) Alpha | As defined in 25.331 | 1.0 |  |
| PCCPCH slot position | Integer 0 -14 | 0 |  |
| Beacon slot position | Integer 0-14 | 8 |  |

- Figure 8.1

### 8.6.2	Performance

At the end of period T1, the PCCPCH and Beacon Received power shall be simultaneously decreased by 10 dB. These conditions are summarized in table 8.14, period T2.

For the first frame including the change in received power the UE output power shall satisfy the values in table 8.15.

For the 20th frame after the change in received power the UE output power shall satisfy the values in table 8.16.

- Table 8.15: Required UE Output Power, Frame Containing Power Level Change

| Parameter | Units | Value |  |
| --- | --- | --- | --- |
| UL transmission slot position |  | 1,9 | 7,14 |
| UE output power | dBm | 15 ±4.0 | 5 ±0.5 |

- Table 8.16: Required UE Output Power, 20 Frames after Power Level Change

| Parameter | Units | Value |  |
| --- | --- | --- | --- |
| UL transmission slot position |  | 1,9 | 7,14 |
| UE output power | dBm | 15 ±4.0 | 15 ±4.0 |

## 8.7	Demodulation of DCH in moving conditions

The receive characteristics of the Dedicated Channel (DCH) in dynamic moving propagation conditions are determined by the Block Error Ratio (BLER) values. BLER is measured for the each of the individual data rate specified for the DPCH. DCH is mapped into Dedicated Physical Channel (DPCH).

### 8.7.1	Minimum requirement

#### 8.7.1.1	3.84 Mcps TDD Option

Void

#### 8.7.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.17 the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.18.

- Table 8.17: DCH parameters in moving propagation conditions (1.28 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Number of DPCHo |  | 8 | 2 |
| Scrambling code and basic midamble code number* |  | 0 | 0 |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1…8 |
| DPCHo Channelization Codes* | C(k,Q) | C(i,16) 3≤ i ≤10 | C(i,16) 9≤ i ≤10 |
|  | dB | -10 | -10 |
| Ioc | DBm/1.28MHz | -60 |  |
| Information Data Rate | Kbps | 12.2 | 64 |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |

- Table 8.18: Performance requirements in moving propagation conditions (1.28 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 7.1 | 10-2 |
| 2 | 6.7 | 10-2 |

#### 8.7.1.3	7.68 Mcps TDD Option

Void

## 8.8	Demodulation of DCH in birth-death conditions

The receive characteristics of the Dedicated Channel (DCH) in birth-death propagation conditions are determined by the Block Error Ratio (BLER) values. BLER is measured for the each of the individual data rate specified for the DPCH. DCH is mapped into in Dedicated Physical Channel (DPCH).

### 8.8.1	Minimum requirement

#### 8.8.1.1	3.84 Mcps TDD Option

Void

#### 8.8.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 8.19 the BLER should not exceed the piece-wise linear BLER curve specified in Table 8.20.

- Table 8.19: DCH parameters in brith-death propagation conditions (1.28 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Number of DPCHo |  | 8 | 2 |
| Scrambling code and basic midamble code number* |  | 0 | 0 |
| DPCH Channelization Codes* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1…8 |
| DPCHo Channelization Codes* | C(k,Q) | C(i,16) 3≤ i ≤10 | C(i,16) 9≤ i ≤10 |
|  | dB | -10 | -10 |
| Ioc | DBm/1.28MHz | -60 |  |
| Information Data Rate | Kbps | 12.2 | 64 |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |

- Table 8.20: Performance requirements in birth-death propagation conditions (1.28 Mcps TDD Option)

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 7.3 | 10-2 |
| 2 | 6.5 | 10-2 |

#### 8.8.1.3	7.68 Mcps TDD Option

Void

# 9	Performance requirements (HSDPA)

## 9.1	Performance requirement for 3.84 Mcps TDD option

The requirements are stated for the HSDPA UE reference combination classes specified in [2] and under the multipath propagation conditions specified in Annex B. The performance metric for HS-DSCH requirements in multi-path propagation conditions is the throughput R measured on HS-DSCH.

### 9.1.1	HS-DSCH throughput for fixed reference channels

The performance requirements in this subclause apply for the reference measurement channels specified in Annex A.3.2.

During the Fixed Reference Channel tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-SICH is specified in Table 9.1:

- Table 9.1: Node-B Emulator Behaviour in response to ACK/NACK/DTX

| HS-SICH ACK/NACK Field State | Node-B Emulator Behaviour |
| --- | --- |
| ACK | ACK: new transmission using 1st redundancy version (RV) |
| NACK | NACK: retransmission using the next RV (up to the maximum permitted number or RV’s) |
| DTX | DTX: retransmission using the RV previously transmitted to the same H-ARQ process |

#### 9.1.1.1	Minimum requirement QPSK, Fixed Reference Channel, 7,3 Mbps - Category 8 - UE

For the parameters specified in Table 9.2, the measured throughput R shall exceed the throughput specified in Table 9.3 for each radio condition.

Table 9.2: Test parameters for fixed reference measurement channel requirements for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option) QPSK

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  |  |  |
| Scrambling code and basic midamble code number* | - | 0, 1 |  |  |  |
| Number of TS | - | 8 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |  | C(i,16) i=1..14 |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| Redundancy and constellation version coding sequence** | - | {0,0,0,0} s=1, R=0, b=0 |  |  |  |
| HS−PDSCHIor | dB | -12,04 |  |  | -11.46 |
| HS−PDSCHIor | dB | 0 |  |  |  |
| Ioc | dBm/3,84MHz | -60 |  |  |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note **: This sequence implies Chase combining |  |  |  |  |  |

Table 9.3: Performance requirements for fixed reference measurement channel requirement in multipath channels for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option) QPSK

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8,5 | 1300 |
| 2 | PB3 | 9,0 | 1300 |
| 3 | VA30 | 9,75 | 1300 |
| 4 | VA120 | 11,5 | 1400 |

#### 9.1.1.2	Minimum requirement 16QAM, Fixed Reference Channel, 7,3 Mbps - Category 8 - UE

For the parameters specified in Table 9.4, the measured throughput R shall exceed the throughput specified in Table 9.5 for each radio condition.

Table 9.4: Test parameters for fixed reference measurement channel requirements for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option) 16QAM

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 16QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 0, 1 |  |  |  |
| Number of TS | - | 8 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |  | C(i,16) i=1..14 |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| Redundancy and constellation version coding sequence** | - | {0,0,0,0} s=1, r=0 |  |  |  |
| HS−PDSCHIor | dB | -12,04 |  |  | -11,46 |
| HS−PDSCHIor | dB | 0 |  |  |  |
| Ioc | dBm/3,84MHz | -60 |  |  |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note **: This sequence implies Chase combining |  |  |  |  |  |

Table 9.5: Performance requirements for fixed reference measurement channel requirement in multipath channels for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option) 16QAM

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 16,0 | 2600 |
| 2 | PB3 | 17,5 | 2600 |
| 3 | VA30 | 18,5 | 2600 |
| 4 | VA120 | 14,5 | 1600 |

### 9.1.2	HS-DSCH throughput for Variable Reference Channels

#### 9.1.2.1	Minimum requirement Variable Reference Channel, 7,3 Mbps - Category 8 - UE

For the parameters specified in Table 9.6 the measured throughput R shall exceed the throughput specified in Table 9.7 for each radio condition. The Variable Reference Channel is specified in Annex A.3.3.

Table 9.6: Test parameters for variable reference measurement channel requirements for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Scrambling code and basic midamble code number* | - | 0, 1 |  |  |  |
| Number of TS | - | 8 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |  |  |
| Number of Hybrid ARQ processes** | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 1 |  |  |  |
| Redundancy and constellation version coding sequence | (Xrv, s, r, b) | (0, 1, 0, 0) |  |  |  |
| HS-PDSCHi_Ec/Ior | dB | -12,04 |  |  |  |
| 1iHS−PDSCHEciIor | dB | 0 |  |  |  |
| Ioc | dBm/3,84MHz | -60 |  |  |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note **: For timing requirements, HARQ is not active |  |  |  |  |  |

Table 9.7: Performance requirements for variable reference measurement channel requirement in multipath channels for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option)

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8,8 | 1240 |
|  |  | 14,8 | 2500 |
|  |  | 18,8 | 3600 |
|  |  | 24,8 | 5000 |
| 2 | PB3 | 8,8 | 1220 |
|  |  | 14,8 | 2430 |
|  |  | 20,8 | 4030 |
|  |  | 24,8 | 5080 |
| 3 | VA30 | 10,1 | 1190 |
|  |  | 16,1 | 2290 |
|  |  | 20,1 | 3220 |
|  |  | 24,1 | 4260 |
| 4 | VA120 | 7,1 | 590 |
|  |  | 11,1 | 1180 |
|  |  | 15,1 | 1840 |
|  |  | 19,1 | 2390 |

### 9.1.3	Reporting of Channel Quality Indicator

The reporting accuracy of channel quality indicator (CQI) under AWGN environments is determined by the reporting variance and BLER performance using the transport format indicated by the reported median CQI.

#### 9.1.3.1	Minimum requirement Channel Quality Indicator, 7,3 Mbps - Category 8 - UE

For the parameters specified in Table 9.7A the reported CQI value shall be within the range of +/- 10 of the allowable CQIs of the reported median CQI more than 90% of the time. The BLER for the reported median CQI shall be less than 10%.

Table 9.7A: Test parameters for variable reference measurement channel requirements for 7,3 Mbps - Category 8 - UE (3,84 Mcps TDD Option)

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Scrambling code and basic midamble code number* | - | 0, 1 |  |
| Number of TS | - | 8 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |
| Number of Hybrid ARQ processes** | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 1 |  |
| Redundancy and constellation version coding sequence | (Xrv, s, r, b) | (0, 1, 0, 0) |  |
| HS-PDSCHi_Ec/Ior | dB | -12,04 |  |
| 1iHS−PDSCHEciIor | dB | 0 |  |
|  | dB | 5 | 10 |
| Ioc | dBm/3,84MHz | -60 |  |
| Note*: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note**: For timing requirements, HARQ is not active |  |  |  |

### 9.1.4	HS-SCCH Detection Performance

The detection performance of the HS-SCCH is determined by the probability of event Em, which is declared when the UE is signaled on HS-SCCH, but DTX is observed in the corresponding HS-SICH ACK/NACK field. The probability of event Em is denoted P(Em).

#### 9.1.4.1	Minimum Requirements for HS-SCCH Detection

For the test parameters in Table 9.7B, for each value of HS-SCCH-1 Ec/Ior specified in Table 9.7C, the measured P(Em) shall be less than or equal to the corresponding specified value of P(Em).

- Table 9.7B: Test parameters for HS-SCCH detection (3.84 Mcps TDD option)

| Parameter | Unit |  | Test 1 |  | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of TS under test |  | - |  | 1 |  |  |
| Number of HS-SCCH codes per timeslot |  | - |  | 4 |  |  |
| HS-SCCH UE Identity (,, …, ) |  | - |  | UE1 = 0000000000000000 (UE1 under test) UE2 = 0101010101010101 UE3 = 1010101010101010 UE4 = 1111111111111111 |  |  |
| HS-SCCH Channelization Codes* |  | C(k,Q) |  | HS-SCCH-1 = C(1, 16), for UE1 (UE under test) HS-SCCH-2 = C(2, 16) for UE2 HS-SCCH-3 = C(3, 16) for UE3 HS-SCCH-4 = C(4, 16) for UE4 |  |  |
| HS-SCCH Ec/Ior |  | dB |  | HS-SCCH-2_Ec/Ior = HS-SCCH-3_Ec/Ior = HS-SCCH-4_Ec/Ior, Where,  HS-SCCH-X_Ec/Ior = 1, where X = 1, 2, 3, 4 |  |  |

- Table 9.7C: Minimum requirement for HS-SCCH detection (3.84 Mcps TDD option)

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-SCCH-1 (dB) | (dB) |  |
| 1 | PA3 | -1.6 | 0 | 0.05 |
| 2 | PA3 | -3.0 | 5 | 0.01 |
| 3 | VA30 | -2.5 | 0 | 0.01 |

## 9.2	Performance requirements for 1.28 Mcps TDD option

The requirements are stated for the HSDPA UE reference combination classes specified in [2] and under the multipath propagation conditions specified in Annex B. The performance metric for HS-DSCH requirements in multi-path propagation conditions is the throughput R measured on HS-DSCH.

For multi-carrier reception, the performance metric for HS-DSCH requirements is the throughput R measured on HS-DSCH on each carrier and the spacing between the two adjacent carriers shall be 1.6 MHz.

Unless otherwise stated the performance requirements are specified at the antenna connector of the UE. For UE(s) with an integral antenna only, a reference antenna with a gain of 0 dBi is assumed. UE with an integral antenna may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For Ues with more than one antenna connector testing the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

### 9.2.1	HS-DSCH throughput for fixed reference channels

The performance requirements in this subclause apply for the reference measurement channels specified in Annex A.3.2.

During the Fixed Reference Channel tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-SICH is specified in Table 9.8

- Table 9.8: Node-B Emulator Behaviour in response to ACK/NACK/DTX

| HS-SICH ACK/NACK Field State | Node-B Emulator Behaviour |
| --- | --- |
| ACK | ACK: new transmission using 1st redundancy and constellation version (RV) |
| NACK | NACK: retransmission using the next RV (up to the maximum permitted number or RV’s) |
| DTX | DTX: retransmission using the RV previously transmitted to the same H-ARQ process |

NOTE:	Performance requirements in this section assume a sufficient power allocation to HS-SCCH so that probability of reporting DTX is very low.

#### 9.2.1.1	Category 1, 0.5Mbps UE class

For the parameters specified in Table 9.9, the measured throughput R shall exceed the throughput specified in Table 9.10 for each radio condition.

- Table 9.9: Test parameters for fixed reference measurement channel, QPSK

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK* |  |  |  |
| Scrambling code and basic midamble code number** | - | 1 |  |  |  |
| Number of TS | - | 2 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..10 |  |  |  |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  |  |  |
| HS−PDSCHIor | dB | -10 |  |  |  |
| Ioc*** | dBm/1.28MHz | -60 |  |  |  |
| * Note Only QPSK is supported for this category UE. **Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. ***Note: For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |  |

- Table 9.10: Performance requirements for fixed reference channel, QPSK

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 160 |
| 2 | PB3 | 10 | 170 |
| 3 | VA30 | 10 | 161 |
| 4 | VA120 | 10 | 153 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.1.2	Category 4, 1.1Mbps UE class

For the parameters specified in Table 9.9-1, the measured throughput R shall exceed the throughput specified in Table 9.10-1 for each radio condition.

- Table 9.9-1: Test parameters for fixed reference measurement channel, 16QAM

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 16QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 1 |  |  |  |
| Number of TS | - | 2 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..12 |  |  |  |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| Redundancy and constellation version coding sequence | - | {6,2,1,5} |  |  |  |
| HS−PDSCHIor | dB | -10.8 |  |  |  |
| Ioc** | dBm/1.28MHz | -60 |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |  |

- Table 9.10-1: Performance requirements for QPSK, fixed reference channel, 16QAM

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 15 | 388 |
| 2 | PB3 | 15 | 347 |
| 3 | VA30 | 15 | 316 |
| 4 | VA120 | 15 | 274 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

* Note: Test case in 9.2.1.1.1 can be used to test this kind of UE in case of QPSK.

#### 9.2.1.3	Category 7, 1.6Mbps UE class

For the parameters specified in Table 9.9-2, the measured throughput R shall exceed the throughput specified in Table 9.10-2 for each radio condition.

- Table 9.9-2: Test parameters for fixed reference measurement channel

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 | Test 7 | Test 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  |  |  | 16QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 1 |  |  |  |  |  |  |  |
| Number of TS | - | 3 |  |  |  |  |  |  |  |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |  |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |  |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..10 |  |  |  | C(i,16) i=1..12 |  |  |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  |  |  | {6,2,1,5} |  |  |  |
| HS−PDSCHIor | dB | -10 |  |  |  | -10.8 |  |  |  |
| Ioc** | dBm/ 1.28MHz | -60 |  |  |  |  |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |  |  |  |  |  |

- Table 9.10-2: Performance requirements for fixed reference channel

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 270 |
| 2 | PB3 | 10 | 278 |
| 3 | VA30 | 10 | 259 |
| 4 | VA120 | 10 | 242 |
| 5 | PA3 | 15 | 488 |
| 6 | PB3 | 15 | 471 |
| 7 | VA30 | 15 | 431 |
| 8 | VA120 | 15 | 377 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.1.4	Category 10, 2.2Mbps UE class

For the parameters specified in Table 9.9-3, the measured throughput R shall exceed the throughput specified in Table 9.10-3 for each radio condition.

- Table 9.9-3: Test parameters for fixed reference measurement channel

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 | Test 7 | Test 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  |  |  | 16QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 1 |  |  |  |  |  |  |  |
| Number of TS | - | 4 |  |  |  |  |  |  |  |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |  |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |  |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..10 |  |  |  | C(i,16) i=1..12 |  |  |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  |  |  | {6,2,1,5} |  |  |  |
| HS−PDSCHIor | dB | -10 |  |  |  | -10.8 |  |  |  |
| Ioc** | dBm/ 1.28MHz | -60 |  |  |  |  |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |  |  |  |  |  |

- Table 9.10-3: Performance requirements for fixed reference channel

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 360 |
| 2 | PB3 | 10 | 343 |
| 3 | VA30 | 10 | 320 |
| 4 | VA120 | 10 | 275 |
| 5 | PA3 | 15 | 615 |
| 6 | PB3 | 15 | 606 |
| 7 | VA30 | 15 | 554 |
| 8 | VA120 | 15 | 493 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.1.5	Category 13, 2.8Mbps UE class

For the parameters specified in Table 9.9-4, the measured throughput R shall exceed the throughput specified in Table 9.10-4 for each radio condition.

- Table 9.9-4: Test parameters for fixed reference measurement channel

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 | Test 7 | Test 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  |  |  | 16QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 1 |  |  |  |  |  |  |  |
| Number of TS | - | 5 |  |  |  |  |  |  |  |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |  |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |  |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..10 |  |  |  | C(i,16) i=1..12 |  |  |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  |  |  | {6,2,1,5} |  |  |  |
| HS−PDSCHIor | dB | -10 |  |  |  | -10.8 |  |  |  |
| Ioc** | dBm/ 1.28MHz | -60 |  |  |  |  |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |  |  |  |  |  |

- Table 9.10-4: Performance requirements for fixed reference channel

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 461 |
| 2 | PB3 | 10 | 470 |
| 3 | VA30 | 10 | 438 |
| 4 | VA120 | 10 | 409 |
| 5 | PA3 | 15 | 890 |
| 6 | PB3 | 15 | 810 |
| 7 | VA30 | 15 | 730 |
| 8 | VA120 | 15 | 630 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.1.6	Category 16-24

For the parameters specified in Table 9.9-5, the measured throughput R shall exceed the throughput specified in Table 9.10-5 for each reference measurement channel in annex A.3.2.7.

- Table 9.9-5: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 (Category 16-18) | Test 2 (Category 19-21) | Test 3 (Category 22-24) |  |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 64QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 1 |  |  |  |
| Number of TS | - | 3 | 4 |  | 5 |
| Number of Hybrid ARQ processes | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..14 |  |  |  |
| Redundancy and constellation version coding sequence | - | {6,5,4,0} |  |  |  |
| HS−PDSCHIor | dB | -11.46 |  |  |  |
| Ioc** | dBm/ 1.28MHz | -60 |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |  |

- Table 9.10-5: Performance requirements for fixed reference measurement channels

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 18 | 660 |
| 2 | PA3 | 18 | 875 |
| 3 | PA3 | 18 | 1090 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.1.7	Category 25

The requirements in this section apply when MIMO is configured. If MIMO is not configured, a category 25 UE should have the capability of category 18 according to [2].

For the parameters specified in Table 9.9-6, the measured throughput R shall exceed the throughput specified in Table 9.10-6 for the reference measurement channels in annex A.3.2.10.

For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 and the HS-PDSCH_Ec/Ior should be 0dB in dual stream transmission, other parameters and the performance requirements are the same.

- Table 9.9-6: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  | 16QAM |  |
| Scrambling code and basic midamble code number* | - | 0 |  |  |  |
| Number of TS | - | 3 |  |  |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  | C(i,16) i=1..16 |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  | {6,2,1,5} |  |
| HS−PDSCHIor | dB | -12.04 |  | -12.04 |  |
| Stream Number Configuration | - | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) |
| Ioc | dBm/ 1.28MHz | -60 |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 9.10-6: Performance requirements for fixed reference channels

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 390 |
| 2 | PA3 | 6 | 160 |
| 3 | PA3 | 16 | 860 |
| 4 | PA3 | 12 | 370 |

#### 9.2.1.8	Category 26

The requirements in this section apply when MIMO is configured. If MIMO is not configured, a category 26 UE should have the capability of category 21 according to [2].

For the parameters specified in Table 9.9-7, the measured throughput R shall exceed the throughput specified in Table 9.10-7 for the reference measurement channels in annex A.3.2.11.

For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 and the HS-PDSCH_Ec/Ior should be 0dB in dual stream transmission, other parameters and the performance requirements are the same.

- Table 9.9-7: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  | 16QAM |  |
| Scrambling code and basic midamble code number* | - | 0 |  |  |  |
| Number of TS | - | 4 |  |  |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  | C(i,16) i=1..16 |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  | {6,2,1,5} |  |
| HS−PDSCHIor | dB | -12.04 |  | -12.04 |  |
| Stream Number Configuration | - | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) |
| Ioc | dBm/ 1.28MHz | -60 |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 9.10-7: Performance requirements for fixed reference channels

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 650 |
| 2 | PA3 | 6 | 220 |
| 3 | PA3 | 16 | 950 |
| 4 | PA3 | 12 | 380 |

#### 9.2.1.9	Category 27

The requirements in this section apply when MIMO is configured. If MIMO is not configured, a category 27 UE should have the capability of category 24 according to [2].

For the parameters specified in Table 9.9-8, the measured throughput R shall exceed the throughput specified in Table 9.10-8 for the reference measurement channels in annex A.3.2.12.

For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 and the HS-PDSCH_Ec/Ior should be 0dB in dual stream transmission, other parameters and the performance requirements are the same.

- Table 9.9-8: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  | 16QAM |  |
| Scrambling code and basic midamble code number* | - | 0 |  |  |  |
| Number of TS | - | 5 |  |  |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  | C(i,16) i=1..16 |  |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |  | {6,2,1,5} |  |
| HS−PDSCHIor | dB | -12.04 |  | -12.04 |  |
| Stream Number Configuration | - | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) |
| Ioc | dBm/ 1.28MHz | -60 |  |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 9.10-8: Performance requirements for fixed reference channels

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 10 | 850 |
| 2 | PA3 | 6 | 280 |
| 3 | PA3 | 16 | 1200 |
| 4 | PA3 | 12 | 500 |

#### 9.2.1.10	Category 28

For the parameters specified in Table 9.9-9, the measured throughput R shall exceed the throughput specified in Table 9.10-9 for the reference measurement channels in annex A.3.2.13.

For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 and the HS-PDSCH_Ec/Ior should be 0dB in dual stream transmission, other parameters and the performance requirements are the same.

- Table 9.9-9: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 64QAM |  |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Number of TS | - | 3 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |
| Redundancy and constellation version coding sequence | - | {6,5,4,0} |  |
| HS−PDSCHIor | dB | -12.04 |  |
| Stream Number Configuration | - | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) |
| Ioc | dBm/ 1.28MHz | -60 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |

- Table 9.10-9: Performance requirements for fixed reference channels

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 20 | 800 |
| 2 | PA3 | 18 | 540 |

#### 9.2.1.11	Category 29

For the parameters specified in Table 9.9-10, the measured throughput R shall exceed the throughput specified in Table 9.10-10 for the reference measurement channels in annex A.3.2.14.

For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 and the HS-PDSCH_Ec/Ior should be 0dB in dual stream transmission, other parameters and the performance requirements are the same.

- Table 9.9-10: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 64QAM |  |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Number of TS | - | 4 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |
| Redundancy and constellation version coding sequence | - | {6,5,4,0} |  |
| HS−PDSCHIor | dB | -12.04 |  |
| Stream Number Configuration | - | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) |
| Ioc | dBm/ 1.28MHz | -60 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |

- Table 9.10-10: Performance requirements for fixed reference channels

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 20 | 1200 |
| 2 | PA3 | 18 | 780 |

#### 9.2.1.12	Category 30

For the parameters specified in Table 9.9-11, the measured throughput R shall exceed the throughput specified in Table 9.10-11 for the reference measurement channels in annex A.3.2.15.

For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 and the HS-PDSCH_Ec/Ior should be 0dB in dual stream transmission, other parameters and the performance requirements are the same.

- Table 9.9-11: Test parameters for fixed reference measurement channels

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 64QAM |  |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Number of TS | - | 5 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |  |
| Redundancy and constellation version coding sequence | - | {6,5,4,0} |  |
| HS−PDSCHIor | dB | -12.04 |  |
| Stream Number Configuration | - | Fixed Dual Stream | Fixed Single Stream (2nd Stream is not used) |
| Ioc | dBm/ 1.28MHz | -60 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |

- Table 9.10-11: Performance requirements for fixed reference channels

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 20 | 1570 |
| 2 | PA3 | 18 | 1000 |

### 9.2.1A	HS-DSCH throughput for fixed reference channels for MU-MIMO

The performance requirements in this subclause apply for the reference measurement channels specified in Annex A.3.4.

The Ior defined in this section include both the user’s signal and interfering user’s signal. Suppose user’s signal power is Ior1and interference user’s signal power is Ior2, then Ior=Ior1+Ior2. In addition, we have the following definitionAttenuationFactor=Ior1Ior2.

During the Fixed Reference Channel tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-SICH is specified in Table 9.11.

- Table 9.11: Node-B Emulator Behaviour in response to ACK/NACK

| HS-SICH ACK/NACK Field State | Node-B Emulator Behaviour |
| --- | --- |
| ACK | ACK: new transmission using 1st redundancy and constellation version (RV) |
| NACK | NACK: retransmission using the next RV (up to the maximum permitted number or RV’s) |
| DTX | DTX: retransmission using the RV previously transmitted to the same H-ARQ process |

NOTE:	Performance requirements in this section assume a sufficient power allocation to HS-SCCH so that probability of reporting DTX is very low.

#### 9.2.1A.1	Category 1-3

For the parameters specified in Table 9.12-1, the measured throughput R shall exceed the throughput specified in Table 9.11-2 for user under test. The reference measurement channels in annex A.3.4.1 applied to both user under test and the interference user.

- Table 9.12-1: Test parameters for fixed reference measurement channel for MU-MIMO

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |
| Scrambling code and basic midamble code number* | - | 0 |
| Midamble |  | Special Default Midamble |
| Number of TS | - | 2 |
| Number of Hybrid ARQ processes per stream | - | 4 |
| Maximum number of Hybrid ARQ transmissions | - | 4 |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} |
| Ioc | dBm/ 1.28MHz | -60 |
| Interference User’s Number |  | 1 |
| AttenuationFactor** | dB | 15 |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: AttenuationFactor = (DUT’s power / Interfering user power) |  |  |

- Table 9.12-2: Performance requirements for fixed reference channels for MU-MIMO

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8 | 199 |

#### 9.2.1A.2	Category 4-6

For the parameters specified in Table 9.12-3, the measured throughput R shall exceed the throughput specified in Table 9.11-4 for user under test. The reference measurement channels in annex A.3.4.2 applied to both user under test and the interference user.

- Table 9.12-3: Test parameters for fixed reference measurement channel for MU-MIMO

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK | 16QAM |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Midamble |  | Special Default Midamble |  |
| Number of TS | - | 2 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 | C(i,16) i=1..16 |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} | {6,2,1,5} |
| Ioc | dBm/ 1.28MHz | -60 |  |
| Interference User’s Number | - | 1 |  |
| AttenuationFactor** | dB | 15 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: AttenuationFactor = (DUT’s power / Interfering user power) |  |  |  |

- Table 9.12-4: Performance requirements for fixed reference channels for MU-MIMO

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8 | 198 |
| 2 | PA3 | 15 | 391 |

#### 9.2.1A.3	Category 7-9

For the parameters specified in Table 9.12-5, the measured throughput R shall exceed the throughput specified in Table 9.11-6 for user under test. The reference measurement channels in annex A.3.4.3 applied to both user under test and the interference user.

- Table 9.12-5: Test parameters for fixed reference measurement channel for MU-MIMO

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK | 16QAM |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Midamble |  | Special Default Midamble |  |
| Number of TS | - | 3 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 | C(i,16) i=1..16 |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} | {6,2,1,5} |
| Ioc | dBm/ 1.28MHz | -60 |  |
| Interference User’s Number | - | 1 |  |
| AttenuationFactor** | dB | 15 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: AttenuationFactor = (DUT’s power / Interfering user power) |  |  |  |

- Table 9.12-6: Performance requirements for fixed reference channels for MU-MIMO

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8 | 298 |
| 2 | PA3 | 15 | 598 |

#### 9.2.1A.4	Category 10-12

For the parameters specified in Table 9.12-7, the measured throughput R shall exceed the throughput specified in Table 9.11-8 for user under test. The reference measurement channels in annex A.3.4.4 applied to both user under test and the interference user.

- Table 9.12-7: Test parameters for fixed reference measurement channel for MU-MIMO

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK | 16QAM |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Midamble |  | Special Default Midamble |  |
| Number of TS | - | 4 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 | C(i,16) i=1..16 |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} | {6,2,1,5} |
| Ioc | dBm/ 1.28MHz | -60 |  |
| Interference User’s Number | - | 1 |  |
| AttenuationFactor** | dB | 15 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: AttenuationFactor = (DUT’s power / Interfering user power) |  |  |  |

- Table 9.12-8: Performance requirements for fixed reference channels for MU-MIMO

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8 | 398 |
| 2 | PA3 | 15 | 759 |

#### 9.2.1A.5	Category 13-15

For the parameters specified in Table 9.12-9, the measured throughput R shall exceed the throughput specified in Table 9.11-10 for user under test. The reference measurement channels in annex A.3.4.5 applied to both user under test and the interference user.

- Table 9.12-9: Test parameters for fixed reference measurement channel for MU-MIMO

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK | 16QAM |
| Scrambling code and basic midamble code number* | - | 0 |  |
| Midamble |  | Special Default Midamble |  |
| Number of TS | - | 5 |  |
| Number of Hybrid ARQ processes per stream | - | 4 |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) i=1..16 | C(i,16) i=1..16 |
| Redundancy and constellation version coding sequence | - | {0,0,0,0} | {6,2,1,5} |
| Ioc | dBm/ 1.28MHz | -60 |  |
| Interference User’s Number | - | 1 |  |
| AttenuationFactor** | dB | 15 |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: AttenuationFactor = (DUT’s power / Interfering user power) |  |  |  |

- Table 9.12-10: Performance requirements for fixed reference channels for MU-MIMO

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 8 | 498 |
| 2 | PA3 | 15 | 947 |

### 9.2.2	HS-DSCH throughput for Variable Reference Channels

#### 9.2.2.1	Category 1, 0.5Mbps UE class

For the parameters specified in Table 9.13 the measured throughput R shall exceed the throughput specified in Table 9.14 for each radio condition.

- Table 9.13: Test parameters for variable reference channel, 0.5Mbps UE class

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| HS-PDSCH Modulation and TBS | - | * |  |  |
| Scrambling code and basic midamble code Number ** | - | 1 |  |  |
| Number of TS | - | 2 |  |  |
| Number of DPCHo | - | 0 |  |  |
| Number of HARQ Process | - | 4 |  |  |
| Number of transmission | - | 1 |  |  |
| Redundancy and constellation version coding sequence | Xrv | 0 |  |  |
| HS-PDSCH Channelization Codes** | C(k,Q) | C(i,16) 1i10 |  |  |
| HS-PDSCHi_Ec/Ior | dB | -10 |  |  |
| Ioc**** | dBm | -60 |  |  |
| * Note 1 As requested by the last received CQI report **Note 2 Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. ***Note 3 If the indicated CQI is 0, the Node-B emulator shall format the next HS-PDSCH transmission with the transport block size and the modulation scheme that were previously used. ****Note 4 For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |

- Table 9.14: Performance requirements for variable reference channel, 0.5Mbps UE class

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 15 | 242 |
| 2 | PB3 | 15 | 244 |
| 3 | VA30 | 15 | 211 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2 For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.2.2	Category 4, 1.1Mbps UE class

For the parameters specified in Table 9.13-1 the measured throughput R shall exceed the throughput specified in Table 9.14-1 for each radio condition.

- Table 9.13-1: Test parameters for variable reference channel, 1.1Mbps UE class

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| HS-PDSCH Modulation and TBS | - | * |  |  |
| Scrambling code and basic midamble code Number ** | - | 1 |  |  |
| Number of TS | - | 2 |  |  |
| Number of DPCHo | - | 0 |  |  |
| Number of HARQ Process | - | 4 |  |  |
| Number of transmission | - | 1 |  |  |
| Redundancy and constellation version coding sequence | Xrv | 0 |  |  |
| HS-PDSCH Channelization Codes** | C(k,Q) | C(i,16) 1i10 |  |  |
| HS-PDSCHi_Ec/Ior | dB | -10 |  |  |
| Ioc**** | dBm | -60 |  |  |
| * Note 1 As requested by the last received CQI report **Note 2 Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. ***Note 3 If the indicated CQI is 0, the Node-B emulator shall format the next HS-PDSCH transmission with the transport block size and the modulation scheme that were previously used. ****Note 4 For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |

- Table 9.14-1: Performance requirements for variable reference channel, 1.1 Mbps UE class

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 15 | 318 |
| 2 | PB3 | 15 | 323 |
| 3 | VA30 | 15 | 213 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.2.3	Category 7, 1.6Mbps UE class

For the parameters specified in Table 9.13-2 the measured throughput R shall exceed the throughput specified in Table 9.14-2 for each radio condition.

- Table 9.13-2: Test parameters for variable reference channel, 1.6Mbps UE class

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| HS-PDSCH Modulation and TBS | - | * |  |  |
| Scrambling code and basic midamble code Number ** | - | 1 |  |  |
| Number of TS | - | 3 |  |  |
| Number of DPCHo | - | 0 |  |  |
| Number of HARQ Process | - | 4 |  |  |
| Number of transmission | - | 1 |  |  |
| Redundancy and constellation version coding sequence | Xrv | 0 |  |  |
| HS-PDSCH Channelization Codes** | C(k,Q) | C(i,16) 1i10 |  |  |
| HS-PDSCHi_Ec/Ior | dB | -10 |  |  |
| Ioc**** | dBm | -60 |  |  |
| * Note 1 As requested by the last received CQI report **Note 2 Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. ***Note 3 If the indicated CQI is 0, the Node-B emulator shall format the next HS-PDSCH transmission with the transport block size and the modulation scheme that were previously used. ****Note 4 For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |

- Table 9.14-2: Performance requirements for variable reference channel, 1.6Mbps UE class

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 15 | 480 |
| 2 | PB3 | 15 | 483 |
| 3 | VA30 | 15 | 323 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.2.4	Category 10, 2.2 Mbps UE class

For the parameters specified in Table 9.13-3 the measured throughput R shall exceed the throughput specified in Table 9.14-3 for each radio condition.

- Table 9.13-3: Test parameters for variable reference channel, 2.2Mbps UE class

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| HS-PDSCH Modulation and TBS | - | * |  |  |
| Scrambling code and basic midamble code Number ** | - | 1 |  |  |
| Number of TS | - | 4 |  |  |
| Number of DPCHo | - | 0 |  |  |
| Number of HARQ Process | - | 4 |  |  |
| Number of transmission | - | 1 |  |  |
| Redundancy and constellation version coding sequence | Xrv | 0 |  |  |
| HS-PDSCH Channelization Codes** | C(k,Q) | C(i,16) 1i10 |  |  |
| HS-PDSCHi_Ec/Ior | dB | -10 |  |  |
| Ioc**** | dBm | -60 |  |  |
| * Note 1 As requested by the last received CQI report **Note 2 Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. ***Note 3 If the indicated CQI is 0, the Node-B emulator shall format the next HS-PDSCH transmission with the transport block size and the modulation scheme that were previously used. ****Note 4 For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |

- Table 9.14-3: Performance requirements for variable reference channel, 2.2Mbps UE class

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 15 | 625 |
| 2 | PB3 | 15 | 631 |
| 3 | VA30 | 15 | 418 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

#### 9.2.2.5	Category 13, 2.8 Mbps UE class

For the parameters specified in Table 9.13-4 the measured throughput R shall exceed the throughput specified in Table 9.14-4 for each radio condition.

- Table 9.13-4: Test parameters for variable reference channel, 2.8Mbps UE class

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| HS-PDSCH Modulation and TBS | - | * |  |  |
| Scrambling code and basic midamble code Number ** | - | 1 |  |  |
| Number of TS | - | 5 |  |  |
| Number of DPCHo | - | 0 |  |  |
| Number of HARQ Process | - | 4 |  |  |
| Number of transmission | - | 1 |  |  |
| Redundancy and constellation version coding sequence | Xrv | 0 |  |  |
| HS-PDSCH Channelization Codes** | C(k,Q) | C(i,16) 1i10 |  |  |
| HS-PDSCHi_Ec/Ior | dB | -10 |  |  |
| Ioc**** | dBm | -60 |  |  |
| * Note 1 As requested by the last received CQI report **Note 2 Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. ***Note 3 If the indicated CQI is 0, the Node-B emulator shall format the next HS-PDSCH transmission with the transport block size and the modulation scheme that were previously used. ****Note 4 For multi-carrier reception, it refers to the interference power on each carrier. |  |  |  |  |

- Table 9.14-4: Performance requirements for variable reference channel, 2.8 Mbps UE class

| Test Number | Propagation conditions | [dB](Note1) | R (Throughput) [kbps](Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 15 | 783 |
| 2 | PB3 | 15 | 792 |
| 3 | VA30 | 15 | 544 |
| Note 1: For multi-carrier reception, it refers to on each carrier. Note 2: For multi-carrier reception, R refers to throughput on each carrier. |  |  |  |

## 9.2.3	Reporting of Channel Quality Indicator

The reporting accuracy of channel quality indicator (CQI) under AWGN and static orthogonal environments is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median.

#### 9.2.3.1	Minimum Requirement-UE categories 1-24

For the parameters specified in Table 9.15, the reported CQI value shall be within +/- x, as specified in Table 9.16, of the reported median CQI for more than Y%, also specified in Table 9.16, of the time.

Table 9.15: Test parameters for CQI reporting measurement channel requirements (1.28 Mcps TDD Option)

|  |  | Category 1-3 | Category 4-6 |  | Category 7-9 |  | Category 10-12 |  | Category 13-15 |  | Category 16-18 | Category 19-21 | Category 22-24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 | Test 7 | Test 8 | Test 9 | Test 10 | Test 11 | Test 12 |
| Number of TS | - | 2 | 2 |  | 3 |  | 4 |  | 5 |  | 3 | 4 | 5 |
| Number of HS-PDSCH codes per TS | - | 10 | 10 |  | 10 |  | 10 |  | 10 |  | 14 | 14 | 14 |
| HS-PDSCHi_Ec/Ior | dB | -10 | -10 |  | -10 |  | -10 |  | -10 |  | -11.46 | -11.46 | -11.46 |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,16) 1i10 | C(i,16) 1i10 |  | C(i,16) 1i10 |  | C(i,16) 1i10 |  | C(i,16) 1i10 |  | C(i,16) 1i14 | C(i,16) 1i14 | C(i,16) 1i14 |
| Number of DPCHo | - | 0 |  |  |  |  |  |  |  |  |  |  |  |
| Number of HARQ Process | - | 4 |  |  |  |  |  |  |  |  |  |  |  |
| Number of transmission | - | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Ioc** | dBm/1.28MHz | -60 |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 1 | 8 | 1 | 8 | 1 | 8 | 1 | 8 | 18 | 18 | 18 |
| Propagation Channel | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| *Note 1 Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note 2 For multi-carrier reception, it refers to on each carrier. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 9.16: Performance requirements for CQI reporting measurement channel requirements (1.28 Mcps TDD Option)

| Test | Permitted CQI range from median (x) | % of time that CQI must be within +/- x of median (Y) | Maximum BLER for median reported CQI |
| --- | --- | --- | --- |
| Test 1 | +/- 3 | 90 | 10% |
| Test 2 | +/- 3 | 90 |  |
| Test 3 | +/- 2 | 90 |  |
| Test 4 | +/- 2 | 90 |  |
| Test 5 | +/-2 | 90 |  |
| Test 6 | +/-2 | 90 |  |
| Test 7 | +/-2 | 90 |  |
| Test 8 | +/-2 | 90 |  |
| Test 9 | +/-2 | 90 |  |
| Test 10 | +/-2 | 90 |  |
| Test 11 | +/-2 | 90 |  |
| Test 12 | +/-2 | 90 |  |

#### 9.2.3.2	Minimum Requirement-UE categories 25-27

For the parameters specified in Table 9.16A, the reported CQI value shall be within +/- x, as specified in Table 9.16B, of the reported median CQI for more than Y%, also specified in Table 9.16B, of the time.

The MIMO dual stream static orthogonal propagation conditions are defined in subclause B.3.2.1. For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 in dual stream transmission, and the HS-PDSCHi_Ec/Ior should be 0dB.

Table 9.16A: Test parameters for CQI reporting measurement channel requirements (1.28 Mcps TDD Option)

|  |  | Category 25 |  | Category 26 |  |  | Category 27 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |  | Test 5 |  | Test 6 |  |
| Number of TS | - | 3 |  | 4 |  |  |  | 5 |  |  |
| Number of HS-PDSCH codes per TS | - | 16 |  | 16 |  |  |  | 16 |  |  |
| Number of HS-PDSCH codes per TS | - | 16 |  |  |  |  |  |  |  |  |
| HS-PDSCHi_Ec/Ior | dB | -12.04 |  |  |  |  |  |  |  |  |
| HS-PDSCH Channelization Codes | C(k,Q) | C(i,16) 1i16 |  |  |  |  |  |  |  |  |
| Number of DPCHo | - | 0 |  |  |  |  |  |  |  |  |
| Number of HARQ Process perstream | - | 4 |  |  |  |  |  |  |  |  |
| Number of transmission | - | 1 |  |  |  |  |  |  |  |  |
| Ioc | dBm | -60 |  |  |  |  |  |  |  |  |
|  | dB | 8 | 10 | 8 |  | 10 |  | 8 |  | 10 |
| Stream Number configuration | - | Single Stream | Dual Stream | Single Stream |  | Dual Stream |  | Single Stream |  | Dual Stream |
| Propagation Channel | - | AWGN | Static Orthogonal | AWGN |  | Static Orthogonal |  | AWGN |  | Static Orthogonal |

Table 9.16B: Performance requirements for CQI reporting measurement channel requirements (1.28 Mcps TDD Option)

| Test | Permitted CQI range from median (x) | % of time that CQI must be within +/- x of median (Y) | Maximum BLER for median reported CQI |
| --- | --- | --- | --- |
| Test 1 | +/- 2 | 90 | 10% |
| Test 2 | +/- 2 | 90 |  |
| Test 3 | +/- 2 | 90 |  |
| Test 4 | +/- 2 | 90 |  |
| Test 5 | +/-2 | 90 |  |
| Test 6 | +/-2 | 90 |  |

#### 9.2.3.3	Minimum Requirement-UE categories 28-30

For the parameters specified in Table 9.16C, the reported CQI value shall be within +/- x, as specified in Table 9.16D, of the reported median CQI for more than Y%, also specified in Table 9.16D, of the time.

The MIMO dual stream static orthogonal propagation conditions are defined in subclause B.3.2.1. For UE supporting Spreading Factor 1 only in dual stream transmission, the number of HS-PDSCH codes per TS should be configured to 1 in dual stream transmission, and the HS-PDSCHi_Ec/Ior should be 0dB.

Table 9.16C: Test parameters for CQI reporting measurement channel requirements (1.28 Mcps TDD Option)

|  |  | Category 28 |  | Category 29 |  |  | Category 30 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |  | Test 5 |  | Test 6 |  |
| Number of TS | - | 3 |  | 4 |  |  |  | 5 |  |  |
| Number of HS-PDSCH codes per TS | - | 16 |  | 16 |  |  |  | 16 |  |  |
| Number of HS-PDSCH codes per TS | - | 16 |  |  |  |  |  |  |  |  |
| HS-PDSCHi_Ec/Ior | dB | -12.04 |  |  |  |  |  |  |  |  |
| HS-PDSCH Channelization Codes | C(k,Q) | C(i,16) 1i16 |  |  |  |  |  |  |  |  |
| Number of DPCHo | - | 0 |  |  |  |  |  |  |  |  |
| Number of HARQ Process perstream | - | 4 |  |  |  |  |  |  |  |  |
| Number of transmission | - | 1 |  |  |  |  |  |  |  |  |
| Ioc | dBm | -60 |  |  |  |  |  |  |  |  |
|  | dB | 16 | 18 | 16 |  | 18 |  | 16 |  | 18 |
| Stream Number | - | Single Stream | Dual Stream | Single Stream |  | Dual Stream |  | Single Stream |  | Dual Stream |
| Propagation Channel | - | AWGN | Static Orthogonal | AWGN |  | Static Orthogonal |  | AWGN |  | Static Orthogonal |

Table 9.16D: Performance requirements for CQI reporting measurement channel requirements (1.28 Mcps TDD Option)

| Test | Permitted CQI range from median (x) | % of time that CQI must be within +/- x of median (Y) | Maximum BLER for median reported CQI |
| --- | --- | --- | --- |
| Test 1 | +/- 2 | 90 | 10% |
| Test 2 | +/- 2 | 90 |  |
| Test 3 | +/- 2 | 90 |  |
| Test 4 | +/- 2 | 90 |  |
| Test 5 | +/- 2 | 90 |  |
| Test 6 | +/- 2 | 90 |  |

### 9.2.4	HS-SCCH Detection Performance

The detection performance of the HS-SCCH is determined by the probability of event Em, which is declared when the UE is signaled on HS-SCCH, but DTX is observed in the corresponding HS-SICH ACK/NACK field. The probability of event Em is denoted P(Em).

#### 9.2.4.1	Minimum Requirements for HS-SCCH Type 1 Detection

For the test parameters specified in Table 9.17, for each value of HS-SCCH  specified in Table 9.18 the measured P(Em) shall be less than or equal to the corresponding specified value of P(Em).

- Table 9.17: Test parameters for HS-SCCH type 1 detection (1.28Mcps TDD option)

| Parameter | Unit |  | Test 1 |  | Test2 |
| --- | --- | --- | --- | --- | --- |
| Number of TS under test |  | - |  | 1 |  |
| Number of HS-SCCH codes per timeslot |  | - |  | 8 (4 x2) |  |
| Scrambling code and basic midamble code number* |  | - |  | 0 |  |
| Number of DPCHo |  | - |  | 2 |  |
| Number of H-ARQ process |  | - |  | 4 |  |
| HS-SCCH UE Identity (,, …, ) |  | - |  | UE1 = 0000000000000000 (UE1 under test) UE2 = 0101010101010101 UE3 = 1010101010101010 UE4 = 1111111111111111 |  |
| HS-SCCH Channelization Codes* |  | C(k,Q) |  | C(i,16) 1i8 |  |
| HS-SCCH Channelization Codes for UE under test |  | C(k,Q) |  | C(i,16) 1i2 |  |
| DPCHo Channelization Codes |  | C(k,Q) |  | C(i,16) 9i10 |  |
| Power control for HS-SCCH of UE 1 |  | - |  | OFF |  |
| HS−SCCHIor |  | dB |  | -10 |  |
| Ioc** |  | dBm/1.28MHz |  | -60 |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note ** For multi-carrier reception, it refers to on each carrier |  |  |  |  |  |

- Table 9.18: Minimum requirement for HS-SCCH type 1 detection (1.28Mcps TDD option)

| Test Number | Propagation Conditions | (dB)(Note1) | (Note2) |
| --- | --- | --- | --- |
| 1 | PA3 | 16 | 0.01 |
| 2 | VA30 | 12 | 0.01 |
| Note1 For multi-carrier reception, it refers to on each carrier. Note2 For multi-carrier reception, it refers to on each carrier. |  |  |  |

#### 9.2.4.2	Minimum Requirements for HS-SCCH Type 4/5 Detection

For the test parameters specified in Table 9.18AA, for each value of HS-SCCH  specified in Table 9.18AA the measured P(Em) shall be less than or equal to the corresponding specified value of P(Em). Minimum performance requirements specified in Table 9.18AB are based on receiver diversity.

The requirments for HS-SCCH Type4 in this section does not applicable to UE which only support MU-MIMO but do not support SU-MIMO.

- Table 9.18AA: Test parameters for HS-SCCH Type 4/5 detection (1.28Mcps TDD option)

| Parameter | Unit |  | Test 1 |  | Test2 |
| --- | --- | --- | --- | --- | --- |
| Number of TS under test |  | - |  | 1 |  |
| Number of HS-SCCH codes per timeslot |  | - |  | 8 (4 x2) |  |
| Scrambling code and basic midamble code number* |  | - |  | 0 |  |
| Number of DPCHo |  | - |  | 2 |  |
| Number of H-ARQ process |  | - |  | 4 |  |
| HS-SCCH Channelization Codes* |  | C(k,Q) |  | C(i,16) 1i8 |  |
| HS-SCCH Channelization Codes for UE under test |  | C(k,Q) |  | C(i,16) 1i2 |  |
| DPCHo Channelization Codes |  | C(k,Q) |  | C(i,16) 9i10 |  |
| Power control for HS-SCCH of UE 1 |  | - |  | OFF |  |
| HS−SCCHIor |  | dB |  | -10 |  |
| Ioc |  | dBm/1.28MHz |  | -60 |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 9.18AB: Minimum requirement for HS-SCCH Type 4/5 detection (1.28Mcps TDD option)

| Test Number | Propagation Conditions | (dB) |  |
| --- | --- | --- | --- |
| 1 | PA3 | 12.3 | 0.01 |
| 2 | VA30 | 9.2 | 0.01 |

#### 9.2.4.3	Minimum Requirements for HS-SCCH Type 6/7/8/9 Detection

For the test parameters specified in Table 9.18AC, for each value of HS-SCCH  specified in Table 9.18AD the measured P(Em) shall be less than or equal to the corresponding specified value of P(Em). Minimum performance requirements specified in Table 9.18AD are based on receiver diversity.

- Table 9.18AC: Test parameters for HS-SCCH Type 6/7/8/9 detection (1.28Mcps TDD option)

| Parameter | Unit |  | Test 1 |  | Test2 |
| --- | --- | --- | --- | --- | --- |
| Number of TS under test |  | - |  | 1 |  |
| Number of HS-SCCH codes per timeslot |  | - |  | 8 (4 x2) |  |
| Scrambling code and basic midamble code number* |  | - |  | 0 |  |
| Number of DPCHo |  | - |  | 2 |  |
| Number of H-ARQ process |  | - |  | 4 |  |
| HS-SCCH Channelization Codes* |  | C(k,Q) |  | C(i,16) 1i8 |  |
| HS-SCCH Channelization Codes for UE under test |  | C(k,Q) |  | C(i,16) 1i2 |  |
| DPCHo Channelization Codes |  | C(k,Q) |  | C(i,16) 9i10 |  |
| Power control for HS-SCCH of UE 1 |  | - |  | OFF |  |
| HS−SCCHIor |  | dB |  | -10 |  |
| Ioc |  | dBm/1.28MHz |  | -60 |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. |  |  |  |  |  |

- Table 9. 18AD: Minimum requirement for HS-SCCH Type 6/7/8/9 detection (1.28Mcps TDD option)

| Test Number | Propagation Conditions | (dB) |  |
| --- | --- | --- | --- |
| 1 | PA3 | 12.5 | 0.01 |
| 2 | VA30 | 9.4 | 0.01 |

### 9.2.5	PLCCH Detection Performance

The detection performance of the PLCCH is determined by the BER of the received PLCCH.

#### 9.2.5.1	Minimum Requirements

For the test parameters in Table 9.18A, for the value of Îor/Ioc specified in Table 9.18B, the measured BER should be equal or less than the corresponding specified BER value.

- Table 9.18A: Test parameters for PLCCH detection (1.28Mcps TDD option)

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Number of PLCCH | - | 1 |
| Number of interfering codes/timeslot | - | 1 × SF16 |
| Number of timeslot | - | 1 |
| PLCCH information bit pattern | - | Alternating 1 and 0 starting with 1 (101010….) |
| Ioc | dBm/1.28 MHz | -60 |
| PLCCH_Ec/Ior | dB | -3 |
| PLCCH channelization codes | C(k, Q) | C(1, 16) |
| OCNS channelization code | C(k, Q) | C(2, 16) |
| Midamble allocation | - | Common |
| Power control | - | OFF |
| Propagation condition | - | VA30 |

- Table 9.18B: Minimum requirement for PLCCH detection (1.28Mcps TDD option)

| Test Number | (dB) | BER |
| --- | --- | --- |
| 1 | 0.3 | 0.04 |

## 9.3	Performance requirement for 7.68 Mcps TDD option

The requirements are stated for the HSDPA UE reference combination classes specified in [2] and under the multipath propagation conditions specified in Annex B. The performance metric for HS-DSCH requirements in multi-path propagation conditions is the throughput R measured on HS-DSCH.

### 9.3.1	HS-DSCH throughput for fixed reference channels

The performance requirements in this subclause apply for the reference measurement channels specified in Annex A.3.2.

During the Fixed Reference Channel tests the behaviour of the Node-B emulator in response to the ACK/NACK signalling field of the HS-SICH is specified in Table 9.19:

- Table 9.19: Node-B Emulator Behaviour in response to ACK/NACK/DTX

| HS-SICH ACK/NACK Field State | Node-B Emulator Behaviour |
| --- | --- |
| ACK | ACK: new transmission using 1st redundancy version (RV) |
| NACK | NACK: retransmission using the next RV (up to the maximum permitted number or RV’s) |
| DTX | DTX: retransmission using the RV previously transmitted to the same H-ARQ process |

#### 9.3.1.1	Minimum requirement QPSK, Fixed Reference Channel, 5,3 Mbps - Category 8 - UE

For the parameters specified in Table 9.20, the measured throughput R shall exceed the throughput specified in Table 9.21 for each radio condition.

Table 9.20: Test parameters for fixed reference measurement channel requirements for 5,3 Mbps - Category 8 - UE (7,68 Mcps TDD Option) QPSK

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | QPSK |  |  |  |
| Scrambling code and basic midamble code number* | - | 0, 1 |  |  |  |
| Number of TS | - | 4 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,32) i=1..32 |  |  |  |
| Number of Hybrid ARQ processes | - | 3 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| Redundancy and constellation version coding sequence** | - | {0,0,0,0} s=1, R=0, b=0 |  |  |  |
| HS−PDSCHIor | dB | -15,05 |  |  |  |
| HS−PDSCHIor | dB | 0 |  |  |  |
| Ioc | dBm/7,68MHz | -60 |  |  |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note **: This sequence implies Chase combining |  |  |  |  |  |

Table 9.21: Performance requirements for fixed reference measurement channel requirement in multipath channels for 5,3 Mbps - Category 8 - UE (7,68 Mcps TDD Option) QPSK

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 5,2 | 880 |
| 2 | PB3 | 5,5 | 880 |
| 3 | VA30 | 6,2 | 880 |
| 4 | VA120 | 6,2 | 880 |

#### 9.3.1.2	Minimum requirement 16 QAM, Fixed Reference Channel, 5,3 Mbps - Category 8 - UE

For the parameters specified in Table 9.22, the measured throughput R shall exceed the throughput specified in Table 9.23 for each radio condition.

Table 9.22: Test parameters for fixed reference measurement channel requirements for 5,3 Mbps - Category 8 - UE (7,68 Mcps TDD Option) 16QAM

| Parameters | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Modulation | - | 16QAM |  |  |  |
| Scrambling code and basic midamble code number* | - | 0, 1 |  |  |  |
| Number of TS | - | 4 |  |  |  |
| HS-PDSCH Channelization Codes* | C(k,Q) | C(i,32) i=1..32 |  |  |  |
| Number of Hybrid ARQ processes | - | 3 |  |  |  |
| Maximum number of Hybrid ARQ transmissions | - | 4 |  |  |  |
| Redundancy and constellation version coding sequence** | - | {0,0,0,0} s=1, R=0, b=0 |  |  |  |
| HS−PDSCHIor | dB | -15,05 |  |  |  |
| HS−PDSCHIor | dB | 0 |  |  |  |
| Ioc | dBm/7,68MHz | -60 |  |  |  |
| Note *: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. Note **: This sequence implies Chase combining |  |  |  |  |  |

Table 9.23: Performance requirements for fixed reference measurement channel requirement in multipath channels for 5,3 Mbps - Category 8 - UE (7,68 Mcps TDD Option) 16QAM

| Test Number | Propagation conditions | [dB] | R (Throughput) [kbps] |
| --- | --- | --- | --- |
| 1 | PA3 | 11,1 | 1765 |
| 2 | PB3 | 13,2 | 1765 |
| 3 | VA30 | 13,7 | 1765 |
| 4 | VA120 | 13,6 | 1765 |

### 9.3.2	(void)

### 9.3.3	(void)

### 9.3.4	HS-SCCH Detection Performance

The detection performance of the HS-SCCH is determined by the probability of event Em, which is declared when the UE is signaled on HS-SCCH, but DTX is observed in the corresponding HS-SICH ACK/NACK field. The probability of event Em is denoted P(Em).

#### 9.3.4.1	Minimum Requirements for HS-SCCH Detection

For the test parameters in Table 9.24, for each value of HS-SCCH-1 Ec/Ior specified in Table 9.25, the measured P(Em) shall be less than or equal to the corresponding specified value of P(Em).

- Table 9.24: Test parameters for HS-SCCH detection (7.68 Mcps TDD option)

| Parameter | Unit |  | Test 1 |  | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of TS under test |  | - |  | 1 |  |  |
| Number of HS-SCCH codes per timeslot |  | - |  | 4 |  |  |
| HS-SCCH UE Identity (,, …, ) |  | - |  | UE1 = 0000000000000000 (UE1 under test) UE2 = 0101010101010101 UE3 = 1010101010101010 UE4 = 1111111111111111 |  |  |
| HS-SCCH Channelization Codes* |  | C(k,Q) |  | HS-SCCH-1 = C(1, 32), for UE1 (UE under test) HS-SCCH-2 = C(2, 32) for UE2 HS-SCCH-3 = C(3, 32) for UE3 HS-SCCH-4 = C(4, 32) for UE4 |  |  |
| HS-SCCH Ec/Ior |  | dB |  | HS-SCCH-2_Ec/Ior = HS-SCCH-3_Ec/Ior = HS-SCCH-4_Ec/Ior, Where,  HS-SCCH-X_Ec/Ior = 1, where X = 1, 2, 3, 4 |  |  |

- Table 9.25: Minimum requirement for HS-SCCH detection (7.68 Mcps TDD option)

| Test Number | Propagation Conditions | Reference value |  |  |
| --- | --- | --- | --- | --- |
|  |  | HS-SCCH-1 (dB) | (dB) |  |
| 1 | PA3 | -6.0 | 0 | 0.05 |
| 2 | PA3 | -7.5 | 5 | 0.01 |
| 3 | VA30 | -6.0 | 0 | 0.01 |

# 10	Performance requirements (MBMS)

Unless otherwise stated the receiver characteristics are specified at the antenna connector of the UE. For UE(s) with an integral antenna only, a reference antenna with a gain of 0 dBi is assumed. UE with an integral antenna may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For Ues with more than one receiver antenna connector the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

## 10.1	Demodulation of MCCH

The receive characteristic of the MCCH is determined by the RLC SDU error rate (RLC_SDU_ER). The requirement is valid for all RRC states for which the UE has capabilities.

### 10.1.1	Minimum requirement

#### 10.1.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 10.1, the measured average downlink S-CCPCH_Ec/Ior power ratio shall be below the specified value for the RLC_SDU_ER shown in Table 10.2.

- Table 10.1: Test parameters for MCCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/3.84 MHz | -60 |
|  | dB | -3 |
| Number of Interfering codes/timeslot | - | 7 × SF16 |
| MCCH Data Rate | kbps | 7.2 |
| Propagation condition | - | VA3 |
| Slot Format #i | - | 3 |

- Table 10.2: Test requirements for MCCH detection

| Test Number | S-CCPCH_Ec/Ior (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | -1.25 | 0.01 |

#### 10.1.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 10.3, the measured average downlink  power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.4.

- Table 10.3: Test parameters for MCCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/1.28 MHz | -60 |
| Number of codes per timeslot | - | 2xSF16 |
| Number of interfering codes per timeslot | - | 0 |
| MCCH Data Rate | kbps | 7.6 |
| Propagation condition | - | VA3 |

- Table 10.4: Test requirements for MCCH detection

| Test Number | (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | 5.8 | 0.01 |

#### 10.1.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 10.4A, the measured average downlink S-CCPCH_Ec/Ior power ratio shall be below the specified value for the RLC_SDU_ER shown in Table 10.4B.

- Table 10.4A: Test parameters for MCCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/7.68 MHz | -60 |
|  | dB | -3 |
| Number of Interfering codes/timeslot | - | 15 × SF32 |
| MCCH Data Rate | kbps | 7.2 |
| Propagation condition | - | VA3 |
| Slot Format #i | - | 3 |

- Table 10.4B: Test requirements for MCCH detection

| Test Number | S-CCPCH_Ec/Ior (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | -4.7 | 0.01 |

### 10.1.2	MBSFN capable UE

This requirement is applicable for Ues that are capable of receiving MBSFN.

#### 10.1.2.1	3.84 Mcps TDD Option

##### 10.1.2.1.1	Non-IMB

The test is only applicable for Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

For the parameters specified in Table 10.4C, the measured average downlink S-CCPCH_Ec/Ior power ratio shall be below the specified value for the RLC_SDU_ER shown in Table 10.4D.

- Table 10.4C: Test parameters for MCCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/3.84 MHz | -60 |
|  | dB | 12 |
| Number of Interfering codes/timeslot | - | 7 × SF16 |
| MCCH Data Rate | kbps | 7.2 |
| Propagation condition | - | Extended delay spread (see Appendix B) |
| Slot Format #i | - | 21 |

- Table 10.4D: Test requirements for MCCH detection (at least two receiver antennas)

| Test Number | S-CCPCH_Ec/Ior (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | -19.29 | 0.01 |

##### 10.1.2.1.2	IMB

The test is only applicable for Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

For the parameters specified in Table 10.4DA, the measured average downlink S-CCPCH_Ec/Ior power ratio shall be below the specified value for the RLC_SDU_ER shown in Table 10.4DB.

- Table 10.4DA: Test parameters for MCCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/3.84MHz | -60 |
|  | dB | 12 |
| MCCH Data Rate | kbps | 6.4 (see Annex A.4.1.1.2) |
| Configuration of other physical channels | - | See Annex A.4.1.1.2 |
| Propagation condition | - | Extended Delay Spread (see Annex B.2.1, Table B.1D) |

- Table 10.4DB: Test requirements for MCCH detection (at least two receiver antennas)

| Test Number | S-CCPCH_Ec/Ior (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | -27 | 0.01 |

#### 10.1.2.2	1.28 Mcps TDD Option

For the parameters specified in Table 10.4E, the measured average downlink  power ratio shall be below the specified value for the RLC_SDU_ER shown in Table 10.4F.

- Table 10.4E: Test parameters for MCCH detection

| Parameters | Unit | Test 11 | Test 21 |
| --- | --- | --- | --- |
| Ioc | dBm/1.28 MHz | -60 | -60 |
| Rx antenna | - | 1 | 2 |
| Number of codes/Timeslot | - | 3 | 3 |
| Number of Interfering codes/timeslot | - | 5XSF16 | 5XSF16 |
| MCCH Data Rate | kbps | 7.6 | 7.6 |
| Propagation condition | - | MBSFN channel model 2 (Annex B) | MBSFN channel model 2 (Annex B) |
| Slot Format # | - | 103 | 103 |
| NOTE1: The tests are only applicable for the UE supporting extended delay spread. NOTE2: In the case of Rx diversity, the fading of the signal and AWGN signals applied to each receiver antenna connector shall be uncorrelated. NOTE3: See Table 8Ha in TS25.221. |  |  |  |

- Table 10.4F: Test requirements for MCCH detection

| Test Number | (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | 9.1 | 0.01 |
| 2 | 4.5 | 0.01 |

#### 10.1.2.3	7.68 Mcps TDD Option

The test is only applicable for Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

For the parameters specified in Table 10.4G, the measured average downlink S-CCPCH_Ec/Ior power ratio shall be below the specified value for the RLC_SDU_ER shown in Table 10.4H.

- Table 10.4G: Test parameters for MCCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/7.68 MHz | -60 |
|  | dB | 12 |
| Number of Interfering codes/timeslot | - | 15 × SF32 |
| MCCH Data Rate | kbps | 7.2 |
| Propagation condition | - | Extended delay spread (see Appendix B) |
| Slot Format #i | - | 21 |

- Table 10.4H: Test requirements for MCCH detection (at least two receiver antennas)

| Test Number | S-CCPCH_Ec/Ior (dB) | RLC_SDU_ER |
| --- | --- | --- |
| 1 | -22.71 | 0.01 |

## 10.2	Demodulation of MTCH

The receive characteristic of the MTCH is determined by RLC SDU error rate (RLC SDU ER). RLC SDU ER is specified for each individual data rate of the MTCH. The requirement is valid for all RRC states for which the UE has capabilities for MBMS.

### 10.2.1	Minimum requirement

#### 10.2.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 10.5 the average downlink  power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.6.

- Table 10.5: Parameters for MTCH detection

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| (S-CCPCH_Ec)/Ior per active timeslot | dB | 0 | 0 |
| (S-CCPCH_Ec)/Ior per active timeslot | dB | 0 | 0 |
| MTCH Data Rate | kbps | 128 | 256 |
| Propagation condition | - | VA3 |  |
| Number of Radio Links | - | 2 | 3 |

- Table 10.6: Test requirements for MTCH detection

| Test Number | (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | 5.7 | 0.1 |
| 2 | 5.5 | 0.1 |

#### 10.2.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 10.7 the average downlink  power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.8.

- Table 10.7: Parameters for MTCH detection

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Ioc | dBm/1.28 MHz | -60 |  |
| MTCH Data Rate | kbps | 64 | 128 |
| Number of codes per timeslot | - | 8xSF16 | 14xSF16 |
| Number of interfering codes per timeslot | - | 0 | 0 |
| Propagation condition | - | VA3 |  |
| Number of Radio Links | - | 3 | 3 |

- Table 10.8: Test requirements for MTCH detection

| Test Number | (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | 4.8 | 0.1 |
| 2 | 6.0 | 0.1 |

#### 10.2.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 10.9 the average downlink  power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.10.

- Table 10.9: Parameters for MTCH detection

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Ioc | dBm/7.68 MHz | -60 |  |
| (S-CCPCH_Ec)/Ior per active timeslot | dB | -3 | -3 |
| MTCH Data Rate | Kbps | 128 | 256 |
| Number of interfering codes/timeslot | - | 16 × SF32 | 16 × SF32 |
| Propagation condition | - | VA3 |  |
| Number of Radio Links | - | 2 | 3 |

- Table 10.10: Test requirements for MTCH detection

| Test Number | (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | 6.1 | 0.1 |
| 2 | 5.0 | 0.1 |

### 10.2.2	MBSFN capable UE

This requirement is applicable for Ues that are capable of receiving MBSFN.

#### 10.2.2.1	3.84 Mcps TDD Option

##### 10.2.2.1.1	Non-IMB

The test is only applicable for Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

For the parameters specified in Table 10.10A the average downlink power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.10B.

- Table 10.10A: Parameters for MTCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/3.84 MHz | -60 |
| (S-CCPCH_Ec)/Ior per active timeslot | dB | 0 |
| MTCH Data Rate | kbps | 512 |
| Propagation condition | - | Extended delay spread (see Appendix B) |
| Number of Radio Links | - | 1 |
| S-CCPCH Modulation | - | 16QAM |

- Table 10.10B: Test requirements for MTCH detection (at least two receiver antennas)

| Test Number | (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | 14.58 | 0.1 |

##### 10.2.2.1.2	IMB

The test is only applicable for Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

For the parameters specified in Table 10.10AA the measured average downlink S-CCPCH_Ec/Ior power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.10AB.

- Table 10.10AA: Parameters for MTCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/3.84MHz | -60 |
|  | dB | 12 |
| MTCH Data Rate | kbps | 512 (see Annex A.4.2.1.2) |
| Configuration of other physical channels | - | See Annex A.4.2.1.2 |
| Propagation condition | - | Extended Delay Spread (see Annex B.2.1, Table B.1D) |

- Table 10.10AB: Test requirements for MTCH detection (at least two receiver antennas)

| Test Number | S-CCPCH_Ec/Ior (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | -3.5 | 0.1 |

#### 10.2.2.2	1.28 Mcps TDD Option

For the parameters specified in Table 10.10C the average downlink power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.10D.

- Table 10.10C: Parameters for MTCH detection

| Parameters | Unit | Test 11 | Test 21 | Test 32 | Test 42 |
| --- | --- | --- | --- | --- | --- |
| MTCH Data rate | Kbps | 192 | 384 | 192 | 384 |
| Rx antenna | - | 1 | 2 | 1 | 2 |
| Modulation | - | QPSK | 16QAM | QPSK | 16QAM |
| Ioc | dBm/1.28 MHz | -60 | -60 | -60 | -60 |
| (S-CCPCH_Ec)/Ior | dB | 0 | 0 | 0 | 0 |
| Propagation condition | - | MBSFN channel model 1 (Annex B) | MBSFN channel model 1 (Annex B) | MBSFN channel model 2 (Annex B) | MBSFN channel model 2 (Annex B) |
| Slot Format # | - | 04 | 24 | 44 | 74 |
| NOTE1: Test 1 and Test 2 are specified for the UE supporting normal delay spread. NOTE2: Test 3 and Test 4 are specified for the UE supporting extended delay spread. NOTE3: In the case of Rx diversity, the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. NOTE4: See Table 8Ha in TS25.221. |  |  |  |  |  |

- Table 10.10D: Test requirements for MTCH detection

| Test Number | (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | 13.3 | 0.1 |
| 2 | 14.7 | 0.1 |
| 3 | 13.3 | 0.1 |
| 4 | 15.1 | 0.1 |

#### 10.2.2.3	7.68 Mcps TDD Option

The test is only applicable for Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

For the parameters specified in Table 10.10E the average downlink  power ratio shall be below the specified value for the RLC SDU ER shown in Table 10.10F.

- Table 10.10E: Parameters for MTCH detection

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/7.68 MHz | -60 |
| (S-CCPCH_Ec)/Ior per active timeslot | dB | -3 |
| MTCH Data Rate | kbps | 512 |
| Number of interfering codes/timeslot | - | 16 × SF32 |
| Propagation condition | - | Extended delay spread (see Appendix B) |
| Number of Radio Links | - | 1 |
| S-CCPCH Modulation | - | 16QAM |

- Table 10.10F: Test requirements for MTCH detection (at least two receiver antennas)

| Test Number | (dB) | RLC SDU ER |
| --- | --- | --- |
| 1 | 14.21 | 0.1 |

### 10.2.3	MBSFN TDD & FDD same platform sharing

This test case is to ensure that a simulataneous demodulation of MTCH and FDD transmission is possible for a MBSFN TDD UE sharing the same platform with a FDD UE. The test is only applicable for TDD Ues with at least two receiver antenna connectors where the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated.

#### 10.2.3.1	3.84 Mcps TDD Option

##### 10.2.3.1.1	Non-IMB

For the parameters specified in Table 10.10G the average downlink Îor power shall be below the specified value for the RLC SDU ER shown in Table 10.10H.

- Table 10.10G: Parameters for MTCH detection sharing same platform with FDD

| Parameters | Unit |  | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| FDD UE Tx Pwr | dBm/ 3.84 MHz |  | Nominal Maximum Output Power | Nominal Maximum Output Power |
| Ioc | dBm/ 3.84 MHz |  | -infinity | -infinity |
| (S-CCPCH_Ec)/Ior per active timeslot | dB |  | 0 | 0 |
| MTCH Data Rate | kbps |  | 512 | 512 |
| Number of interfering codes/timeslot | - |  | 0 | 0 |
| Propagation condition |  | - | Extended Delay Spread (see Appendix B) | Extended Delay Spread (see Appendix B) |
| Number of Radio Links | - |  | 1 | 1 |
| S-CCPCH Modulation | - |  | 16QAM | 16QAM |
| TDD operating frequencies | MHz |  | 1900-1920 | 2570-2620 |
| FDD operating band | - |  | Band I | Band VII |
| TDD/FDD carrier frequencies | - |  | Applicable for all combinations of TDD and FDD carrier frequencies except for combinations where the carrier frequency separation is less than 15 MHz | Applicable for all combinations of TDD and FDD carrier frequencies except for combinations where the carrier frequency separation is less than 15 MHz |

Table 10.10H: Test requirements for MTCH detection sharing same platform with FDD (TDD UE has at least two receiver antennas)

| Test Number | Îor (dBm) | RLC SDU ER |
| --- | --- | --- |
| 1 | -83.42 | 0.1 |
| 2 | -83.42 | 0.1 |

##### 10.2.3.1.2	IMB

[Editor’s note: FFS]

#### 10.2.3.2	(void)

#### 10.2.3.3	7.68 Mcps TDD Option

For the parameters specified in Table 10.10K the average downlink Îor power shall be below the specified value for the RLC SDU ER shown in Table 10.10L.

- Table 10.10K: Parameters for MTCH detection sharing same platform with FDD

| Parameters | Unit |  | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| FDD UE Tx Pwr | dBm/ 3.84 MHz |  | Nominal Maximum Output Power | Nominal Maximum Output Power |
| Ioc | dBm/ 7.68 MHz |  | -infinity | -infinity |
| (S-CCPCH_Ec)/Ior per active timeslot | dB |  | -3 | -3 |
| MTCH Data Rate | kbps |  | 512 | 512 |
| Number of interfering codes/timeslot | - |  | 16 × SF32 | 16 × SF32 |
| Propagation condition |  | - | Extended Delay Spread (see Appendix B) | Extended Delay Spread (see Appendix B) |
| Number of Radio Links | - |  | 1 | 1 |
| S-CCPCH Modulation | - |  | 16QAM | 16QAM |
| TDD operating frequencies | MHz |  | 1900-1920 | 2570-2620 |
| FDD operating band | - |  | Band I | Band VII |
| TDD/FDD carrier frequencies | - |  | Applicable for all combinations of TDD and FDD carrier frequencies except for combinations where the carrier frequency separation is less than 17.5 MHz | Applicable for all combinations of TDD and FDD carrier frequencies except for combinations where the carrier frequency separation is less than 17.5 MHz |

Table 10.10L: Test requirements for MTCH detection sharing same platform with FDD (TDD UE has at least two receiver antennas)

| Test Number | Îor (dBm) | RLC SDU ER |
| --- | --- | --- |
| 1 | -80.79 | 0.1 |
| 2 | -80.79 | 0.1 |

## 10.3	Demodulation of MTCH and cell identification

MBMS combining is not controlled by a network but instead it is autonomously handled by a terminal. UE has to be able to receive MTCH and identify intra-frequency neighbour cells according to the requirements. The requirement for MBMS receiving combined with cell identification is determined by RLC SDU error rate.

### 10.3.1 	Minimum requirement

#### 10.3.1.1	(void)

#### 10.3.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 10.11, the average downlink Ior/Ioc power ratio shall be below the specified value for the RLC SDU error rate shown in Table 10.12. The cell reselection parameters are given in clause A.4.2.2.

- Table 10.11 parameters for MTCH demodulation requirements with cell identification

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Stage 1 | Stage 2 | Stage 3 |
| Time in each stage | s | 2s | 800ms | 2s |
|  | dBm/1.28MHz | -60 |  |  |
| Propagation condition |  | VA 3 |  |  |
| MTCH Data Rate | kbps | 64kbps |  |  |
| Number of Radio Links |  | Cell 1, Cell 2 | Cell 1, 2, 3 | Cell 1, Cell 3 |

- Table 10.12: Requirements for MTCH detection

| Test Number |  | RLC SDU ER |
| --- | --- | --- |
| 1 | 6.1 | 0.05 |

# 11	Performance requirement (E-DCH)

Unless otherwise stated the receiver characteristics are specified at the antenna connector of the UE. For UE(s) with an integral antenna only, a reference antenna with a gain of 0 dBi is assumed. UE with an integral antenna may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For Ues with more than one receiver antenna connector the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

## 11.1	Detection of E-DCH HARQ ACK Indicator Channel (E-HICH)

The performance of the E-HICH detection is determined by the false ACK probability (probability of detecting an ACK given that a NACK was sent) and the false NACK probability (probability of detecting a NACK given that an ACK was sent).

### 11.1.1	Minimum requirement

#### 11.1.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 11.1 the average downlink E-HICH Ec/Ior power ratio shall be below the specified value for the false ACK and false NACK probabilities shown in Table 11.2.

- Table 11.1: Test parameters for E-HICH detection (3.84 Mcps TDD option)

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Ioc | dBm/3.84 MHz | -60 |  |
|  | dB | 0 |  |
| Number of Interfering codes/timeslot | - | 7 × SF16 (all codes have equal powers) |  |
| E-HICH signalling pattern | - | 100% NACK | 100% ACK |
| Propagation condition | - | VA30 |  |

- Table 11.2: Test requirements for E-HICH detection (3.84 Mcps TDD option)

| Test Number | E-HICH Ec/Ior (dB) | Parameter | Probability |
| --- | --- | --- | --- |
| 1 | -18.5 | False ACK | 2E-3 |
| 2 | -18.5 | False NACK | 2E-2 |

#### 11.1.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 11.3 the average downlink E-HICH Ec/Ior power ratio shall be below the specified value for the false ACK and false NACK probabilities shown in Table 11.4.

- Table 11.3: Test parameters for E-HICH detection (1.28 Mcps TDD option)

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Ioc | dBm/1.28 MHz | -60 |  |
|  | dB | 0 |  |
| Number of Interfering codes/timeslot | - | 7 × SF16 (all codes have equal powers) |  |
| Midamble | - | Common midamble |  |
| E-HICH signalling pattern | - | 100% NACK | 100% ACK |
| Propagation condition | - | VA30 |  |

- Table 11.4: Test requirements for E-HICH detection (1.28 Mcps TDD option)

| Test Number | E-HICH Ec/Ior (dB) | Parameter | Probability |
| --- | --- | --- | --- |
| 1 | -7.5 | False ACK | 2E-3 |
| 2 | -7.5 | False NACK | 2E-2 |

#### 11.1.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 11.5 the average downlink E-HICH Ec/Ior power ratio shall be below the specified value for the false ACK and false NACK probabilities shown in Table 11.6.

- Table 11.5: Test parameters for E-HICH detection (7.68 Mcps TDD option)

| Parameters | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Ioc | dBm/7.68 MHz | -60 |  |
|  | dB | 0 |  |
| Number of Interfering codes/timeslot | - | 15 × SF32 (all codes have equal powers) |  |
| E-HICH signalling pattern | - | 100% NACK | 100% ACK |
| Propagation condition | - | VA30 |  |

- Table 11.6: Test requirements for E-HICH detection (7.68 Mcps TDD option)

| Test Number | E-HICH Ec/Ior (dB) | Parameter | Probability |
| --- | --- | --- | --- |
| 1 | -21.7 | False ACK | 2E-3 |
| 2 | -21.7 | False NACK | 2E-2 |

## 11.2	Demodulation of E-DCH Absolute Grant Channel (E-AGCH)

The performance of the E-AGCH detection is determined by the missed detection probability.

### 11.2.1	Minimum requirement

#### 11.2.1.1	3.84 Mcps TDD Option

For the parameters specified in Table 11.7 the average downlink E-AGCH Îor/Ioc power ratio shall be below the specified value for the missed detection probability shown in Table 11.8.

- Table 11.7: Test parameters for E-AGCH detection (3.84 Mcps TDD option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/3.84 MHz | -60 |
| EcIor | dB | -6.02 |
| Number of Interfering codes/timeslot | - | 3 × SF16 |
| Total bits in Timeslot Resource Related Information (TRRI) | bits | 6 |
| Total bits in Resource Duration Indicator (RDI) | bits | 3 |
| Total bits in E-AGCH | bits | 38 |
| Propagation condition | - | VA30 |

- Table 11.8: Test requirements for E-AGCH detection (3.84 Mcps TDD option)

| Test Number | E-AGCH Îor/Ioc (dB) | Missed Detection Probability |
| --- | --- | --- |
| 1 | 1.6 | 0.01 |

#### 11.2.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 11.9 the average downlink E-AGCH type 1 Îor/Ioc power ratio shall be below the specified value for the missed detection probability shown in Table 11.10.

- Table 11.9: Test parameters for E-AGCH type 1 detection (1.28 Mcps TDD option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/1.28 MHz | -60 |
| EcIor | dB | -3 |
| Number of Interfering codes/timeslot | - | 2 × SF16 |
| Total bits in Timeslot Resource Related Information (TRRI) | bits | 5 |
| Total bits in Resource Duration Indicator (RDI) | bits | 3 |
| Total bits in E-AGCH | bits | 26 |
| Midamble | - | Common midamble |
| Propagation condition | - | VA30 |

- Table 11.10: Test requirements for E-AGCH type 1 detection (1.28 Mcps TDD option)

| Test Number | E-AGCH Îor/Ioc (dB) | Missed Detection Probability |
| --- | --- | --- |
| 1 | 8 | 0.01 |

For the parameters specified in Table 11.9A the average downlink E-AGCH type 2 Îor/Ioc power ratio shall be below the specified value for the missed detection probability shown in Table 11.10A.

- Table 11.9A: Test parameters for E-AGCH type 2 detection (1.28 Mcps TDD option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/1.28 MHz | -60 |
| EcIor | dB | -3 |
| Number of Interfering codes/timeslot | - | 2 × SF16 |
| Total bits in E-AGCH | bits | 30 |
| Midamble | - | Common midamble |
| Propagation condition | - | VA30 |

- Table 11.10A: Test requirements for E-AGCH type 2 detection (1.28 Mcps TDD option)

| Test Number | E-AGCH Îor/Ioc (dB) | Missed Detection Probability |
| --- | --- | --- |
| 1 | 8.5 | 0.01 |

#### 11.2.1.3	7.68 Mcps TDD Option

For the parameters specified in Table 11.11 the average downlink E-AGCH Îor/Ioc power ratio shall be below the specified value for the missed detection probability shown in Table 11.12.

- Table 11.11: Test parameters for E-AGCH detection (7.68 Mcps TDD option)

| Parameters | Unit | Test 1 |
| --- | --- | --- |
| Ioc | dBm/7.68 MHz | -60 |
| EcIor | dB | -9.03 |
| Number of Interfering codes/timeslot | - | 7 × SF32 |
| Total bits in Timeslot Resource Related Information (TRRI) | bits | 6 |
| Total bits in Resource Duration Indicator (RDI) | bits | 3 |
| Total bits in E-AGCH | bits | 39 |
| Propagation condition | - | VA30 |

- Table 11.12: Test requirements for E-AGCH detection (7.68 Mcps TDD option)

| Test Number | E-AGCH Îor/Ioc (dB) | Missed Detection Probability |
| --- | --- | --- |
| 1 | 1.2 | 0.01 |

# 12	Performance requirement under multiple-cell scenario

## 12.1	General

The performance requirements for the UE in this section are specified for the measurement channels specified in Annex A and the propagation condition specified in Annex B. Unless otherwise stated the receiver characteristics are specified at the antenna connector of the UE. For UE(s) with an integral antenna only, a reference antenna with a gain of 0 dBi is assumed. UE with an integral antenna may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For Ues with more than one receiver antenna connector the fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

- Table 12.1: Summary of UE performance targets

| Test Chs. | Information Data Rate | Performance metric |  |  |
| --- | --- | --- | --- | --- |
|  |  | Static | Multi-path Case 1 | Multi-path Case 3 |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 |
|  | 64 kbps | BLER<10-1 | BLER<10-1 | BLER<10-1 |

## 12.2	Demodulation of DCH in static propagation conditions

The performance requirement of DCH in static propagation conditions is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

### 12.2.1	Minimum requirement

#### 12.2.1.1	3.84 Mcps TDD Option

[FFS]

#### 12.2.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 12.2 and Table 12.3 the BLER should not exceed the piece-wise linear BLER curve specified in Table 12.4.

- Table 12.2: DCH parameters in static propagation conditions (12.2 kbps)

| Parameters | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Number of DPCHo |  | 4 | 12 | 28 |
| Scrambling code and basic midamble code number of SS#1* |  | 19 | 19 | 19 |
| Scrambling code and basic midamble code number of SS#2* |  | 58 | 58 | 58 |
| Scrambling code and basic midamble code number of SS#3* |  | 85 | 85 | 85 |
| DPCH Channelization Codes of SS#1* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1,2 | C(i,16) i=1,2 |
| DPCHo Channelization Codes of SS#2* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| DPCHo Channelization Codes of SS#3* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| of SS#2 | dB | 10 | 5 | 0 |
| of SS#3 | dB | 4 | -1 | -6 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#2 | chip | 0 | 0 | 0 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#3 | chip | 0 | 0 | 0 |
| Power of SS#2** | dBm | -67 | -67.22 | -68.54 |
| Power of SS#3** | dBm | -73 | -73.22 | -74.54 |
| Ioc | dBm/1,28MHz | -80 |  |  |
| Midamble |  | Default midamble (Kcell = 8) |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: Power of SS can be calculated from and Ioc. |  |  |  |  |

- Table 12.3: DCH parameters in static propagation conditions (64 kbps)

| Parameters | Unit | Test 4 | Test 5 | Test 6 |
| --- | --- | --- | --- | --- |
| Number of DPCHo |  | 4 | 12 | 28 |
| Scrambling code and basic midamble code number of SS#1* |  | 19 | 19 | 19 |
| Scrambling code and basic midamble code number of SS#2* |  | 58 | 58 | 58 |
| Scrambling code and basic midamble code number of SS#3* |  | 85 | 85 | 85 |
| DPCH Channelization Codes of SS#1* | C(k,Q) | C(i,16) 1≤ i ≤8 | C(i,16) 1≤ i ≤8 | C(i,16) 1≤ i ≤8 |
| DPCHo Channelization Codes of SS#2* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| DPCHo Channelization Codes of SS#3* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| of SS#2 | dB | 10 | 5 | 0 |
| of SS#3 | dB | 4 | -1 | -6 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#2 | chip | 0 | 0 | 0 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#3 | chip | 0 | 0 | 0 |
| Power of SS#2** | dBm | -67 | -67.22 | -68.54 |
| Power of SS#3** | dBm | -73 | -73.22 | -74.54 |
| Ioc | dBm/1,28MHz | -80 |  |  |
| Midamble |  | Default midamble (Kcell = 8) |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: Power of SS can be calculated from and Ioc. |  |  |  |  |

- Table 12.4: Performance requirements in static propagation conditions

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | -0.3 | 10-2 |
| 2 | 2.8 | 10-2 |
| 3 | 8.7 | 10-2 |
| 4 | 4.1 | 10-1 |
| 5 | 10.7 | 10-1 |
| 6 | 12.9 | 10-1 |

#### 12.2.1.3	7.68 Mcps TDD Option

[FFS]

## 12.3	Demodulation of DCH in Multipath fading Case 1 conditions

The performance requirement of DCH in Multipath fading Case 1 conditions is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

### 12.3.1	Minimum requirement

#### 12.3.1.1	3.84 Mcps TDD Option

[FFS]

#### 12.3.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 12.5 and Table 12.6 the BLER should not exceed the piece-wise linear BLER curve specified in Table 12.7.

- Table 12.5: DCH parameters in Multipath fading Case 1 conditions (12.2 kbps)

| Parameters | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Number of DPCHo |  | 4 | 12 | 28 |
| Scrambling code and basic midamble code number of SS#1* |  | 19 | 19 | 19 |
| Scrambling code and basic midamble code number of SS#2* |  | 58 | 58 | 58 |
| Scrambling code and basic midamble code number of SS#3* |  | 85 | 85 | 85 |
| DPCH Channelization Codes of SS#1* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1,2 | C(i,16) i=1,2 |
| DPCHo Channelization Codes of SS#2* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| DPCHo Channelization Codes of SS#3* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| of SS#2 | dB | 10 | 5 | 0 |
| of SS#3 | dB | 4 | -1 | -6 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#2 | chip | 0 | 0 | 0 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#3 | chip | 0 | 0 | 0 |
| Power of SS#2** | dBm | -67 | -67.22 | -68.54 |
| Power of SS#3** | dBm | -73 | -73.22 | -74.54 |
| Ioc | dBm/1,28MHz | -80 |  |  |
| Midamble |  | Default midamble (Kcell = 8) |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: Power of SS can be calculated from and Ioc. |  |  |  |  |

- Table 12.6: DCH parameters in Multipath fading Case 1 conditions (64 kbps)

| Parameters | Unit | Test 4 | Test 5 | Test 6 |
| --- | --- | --- | --- | --- |
| Number of DPCHo |  | 4 | 12 | 28 |
| Scrambling code and basic midamble code number of SS#1* |  | 19 | 19 | 19 |
| Scrambling code and basic midamble code number of SS#2* |  | 58 | 58 | 58 |
| Scrambling code and basic midamble code number of SS#3* |  | 85 | 85 | 85 |
| DPCH Channelization Codes of SS#1* | C(k,Q) | C(i,16) 1≤ i ≤8 | C(i,16) 1≤ i ≤8 | C(i,16) 1≤ i ≤8 |
| DPCHo Channelization Codes of SS#2* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| DPCHo Channelization Codes of SS#3* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| of SS#2 | dB | 10 | 5 | 0 |
| of SS#3 | dB | 4 | -1 | -6 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#2 | chip | 0 | 0 | 0 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#3 | chip | 0 | 0 | 0 |
| Power of SS#2** | dBm | -67 | -67.22 | -68.54 |
| Power of SS#3** | dBm | -73 | -73.22 | -74.54 |
| Ioc | dBm/1,28MHz | -80 |  |  |
| Midamble |  | Default midamble (Kcell = 8) |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: Power of SS can be calculated from and Ioc. |  |  |  |  |

- Table 12.7: Performance requirements in Multipath fading Case 1 conditions

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 11.8 | 10-2 |
| 2 | 15.2 | 10-2 |
| 3 | 19.5 | 10-2 |
| 4 | 13.3 | 10-1 |
| 5 | 18.4 | 10-1 |
| 6 | 21.1 | 10-1 |

#### 12.2.1.3	7.68 Mcps TDD Option

[FFS]

## 12.4	Demodulation of DCH in Multipath fading Case 3 conditions

The performance requirement of DCH in Multipath fading Case 3 conditions is determined by the maximum Block Error Ratio (BLER). The BLER is specified for each individual data rate of the DCH. DCH is mapped into the Dedicated Physical Channel (DPCH).

### 12.4.1	Minimum requirement

#### 12.4.1.1	3.84 Mcps TDD Option

[FFS]

#### 12.4.1.2	1.28 Mcps TDD Option

For the parameters specified in Table 12.8 and Table 12.9 the BLER should not exceed the piece-wise linear BLER curve specified in Table 12.10.

- Table 12.8: DCH parameters in Multipath fading Case 3 conditions (12.2 kbps)

| Parameters | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Number of DPCHo |  | 4 | 12 | 28 |
| Scrambling code and basic midamble code number of SS#1* |  | 19 | 19 | 19 |
| Scrambling code and basic midamble code number of SS#2* |  | 58 | 58 | 58 |
| Scrambling code and basic midamble code number of SS#3* |  | 85 | 85 | 85 |
| DPCH Channelization Codes of SS#1* | C(k,Q) | C(i,16) i=1,2 | C(i,16) i=1,2 | C(i,16) i=1,2 |
| DPCHo Channelization Codes of SS#2* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| DPCHo Channelization Codes of SS#3* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| of SS#2 | dB | 10 | 5 | 0 |
| of SS#3 | dB | 4 | -1 | -6 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#2 | chip | 0 | 0 | 0 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#3 | chip | 0 | 0 | 0 |
| Power of SS#2** | dBm | -67 | -67.22 | -68.54 |
| Power of SS#3** | dBm | -73 | -73.22 | -74.54 |
| Ioc | dBm/1,28MHz | -80 |  |  |
| Midamble |  | Default midamble (Kcell = 8) |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: Power of SS can be calculated from and Ioc. |  |  |  |  |

- Table 12.9: DCH parameters in Multipath fading Case 3 conditions (64 kbps)

| Parameters | Unit | Test 4 | Test 5 | Test 6 |
| --- | --- | --- | --- | --- |
| Number of DPCHo |  | 4 | 12 | 28 |
| Scrambling code and basic midamble code number of SS#1* |  | 19 | 19 | 19 |
| Scrambling code and basic midamble code number of SS#2* |  | 58 | 58 | 58 |
| Scrambling code and basic midamble code number of SS#3* |  | 85 | 85 | 85 |
| DPCH Channelization Codes of SS#1* | C(k,Q) | C(i,16) 1≤ i ≤8 | C(i,16) 1≤ i ≤8 | C(i,16) 1≤ i ≤8 |
| DPCHo Channelization Codes of SS#2* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| DPCHo Channelization Codes of SS#3* | C(k,Q) | C(i,16) 1≤ i ≤2 | C(i,16) 1≤ i ≤6 | C(i,16) 1≤ i ≤14 |
| of SS#2 | dB | 10 | 5 | 0 |
| of SS#3 | dB | 4 | -1 | -6 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#2 | chip | 0 | 0 | 0 |
| SFN-SFN Observed Timing Difference Type 2 between SS#1 and SS#3 | chip | 0 | 0 | 0 |
| Power of SS#2** | dBm | -67 | -67.22 | -68.54 |
| Power of SS#3** | dBm | -73 | -73.22 | -74.54 |
| Ioc | dBm/1,28MHz | -80 |  |  |
| Midamble |  | Default midamble (Kcell = 8) |  |  |
| *Note: Refer to TS 25.223 for definition of channelization codes, scrambling code and basic midamble code. **Note: Power of SS can be calculated from and Ioc. |  |  |  |  |

- Table 12.10: Performance requirements in Multipath fading Case 3 conditions

| Test Number | [dB] | BLER |
| --- | --- | --- |
| 1 | 6.5 | 10-2 |
| 2 | 8.8 | 10-2 |
| 3 | 11.6 | 10-2 |
| 4 | 10.9 | 10-1 |
| 5 | 14.3 | 10-1 |
| 6 | 17.0 | 10-1 |

#### 12.4.1.3	7.68 Mcps TDD Option

[FFS]

Annex A (normative):
Measurement channels

# A.1	(void)

# A.2	Reference measurement channel

## A.2.1	UL reference measurement channel (12.2 kbps)

### A.2.1.1	3.84 Mcps TDD Option

- Table A.1

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 2 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 10% / 0% |

- Figure A.1

### A.2.1.2	1.28 Mcps TDD Option

- Table A.1A

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 1TS (1*SF8) = 2RU/5ms |
| Midamble | 144 |
| Interleaving | 20 ms |
| Power control | 4 Bit/user/10ms |
| TFCI | 16 Bit/user/10ms |
| 4 Bit reserved for future use (place of SS) | 4 Bit/user/10ms |
| Inband signalling DCCH | 2.4 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 33% / 33% |

- Figure A.1A

### A.2.1.3	7.68 Mcps TDD Option

- Table A.1B

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 1024 chips |
| Interleaving | 20 ms |
| Power control | 2 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 10% / 0% |

- Figure A.1B

## A.2.2	DL reference measurement channel (12.2 kbps)

### A.2.2.1	3.84 Mcps TDD Option

- TableA.2

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 5% / 0 % |

- Figure A.2

### A.2.2.2	1.28 Mcps TDD Option

- Table A.2A

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU's allocated | 1TS (2*SF16) = 2RU/5ms |
| Midamble | 144 |
| Interleaving | 20 ms |
| Power control (TPC) | 4 Bit/user/10ms |
| TFCI | 16 Bit/user/10ms |
| Synchronisation Shift (SS) | 4 Bit/user/10ms |
| Inband signalling DCCH | 2.4 kbps |
| Puncturing level at Code rate 1/3: DCH of the DTCH / DCH of the DCCH | 33% / 33% |

- Figure A.2A

### A.2.2.3	7.68 Mcps TDD Option

- TableA.2B

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 1024 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 5% / 0 % |

- Figure A.2B

## A.2.3	DL reference measurement channel (64 kbps)

### A.2.3.1	3.84 Mcps TDD Option

- Table A.3

| Parameter | Value |
| --- | --- |
| Information data rate | 64 kbps |
| RU´s allocated | 5 codes SF16 = 5RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate : 1/3 DCH of the DTCH / ½ DCH of the DCCH | 41.1% / 10% |

- Figure A.3

### A.2.3.2	1.28 Mcps TDD Option

- Table A.3A

| Parameter | Value |
| --- | --- |
| Information data rate | 64 kbps |
| RU's allocated | 1TS (8*SF16) = 8RU/5ms |
| Midamble | 144 |
| Interleaving | 20 ms |
| Power control (TPC) | 4 Bit/user/10ms |
| TFCI | 16 Bit/user/10ms |
| Synchronisation Shift (SS) | 4 Bit/user/10ms |
| Inband signalling DCCH | 2.4 kbps |
| Puncturing level at Code rate: 1/3 DCH of the DTCH / ½ DCH of the DCCH | 32% / 0 |

- Figure A.3A

### A.2.3.3	7.68 Mcps TDD Option

- Table A.3B

| Parameter | Value |
| --- | --- |
| Information data rate | 64 kbps |
| RU´s allocated | 5 codes SF32 = 5RU |
| Midamble | 1024 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate : 1/3 DCH of the DTCH / ½ DCH of the DCCH | 41.1% / 10% |

- Figure A.3B

## A.2.4	DL reference measurement channel (144 kbps)

### A.2.4.1	3.84 Mcps TDD Option

- Table A.4

| Parameter | Value |
| --- | --- |
| Information data rate | 144 kbps |
| RU´s allocated | 9 codes SF16 = 9RU |
| Midamble | 256 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate: 1/3 DCH of the DTCH / ½ DCH of the DCCH | 44.5% / 16.6% |

- Figure A.4

### A.2.4.2	1.28 Mcps TDD Option

- Table A.4A

| Parameter | Value |
| --- | --- |
| Information data rate | 144 kbps |
| RU's allocated | 2TS (8*SF16) = 16RU/5ms |
| Midamble | 144 |
| Interleaving | 20 ms |
| Power control (TPC) | 8 Bit/user/10ms |
| TFCI | 32 Bit/user/10ms |
| Synchronisation Shift (SS) | 8 Bit/user/10ms |
| Inband signalling DCCH | 2.4 kbps |
| Puncturing level at Code rate: 1/3 DCH of the DTCH / ½ DCH of the DCCH | 38% / 7% |

- Figure A.4A

### A.2.4.3	7.68 Mcps TDD Option

- Table A.4B

| Parameter | Value |
| --- | --- |
| Information data rate | 144 kbps |
| RU´s allocated | 9 codes SF32 = 9RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate: 1/3 DCH of the DTCH / ½ DCH of the DCCH | 44.5% / 16.6% |

- Figure A.4B

## A.2.5	DL reference measurement channel (384 kbps)

### A.2.5.1	3.84 Mcps TDD Option

- Table A.5

| Parameter | Value |
| --- | --- |
| Information data rate | 384 kbps |
| RU´s allocated | 8*3TS = 24RU |
| Midamble | 256 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate : 1/3 DCH of the DTCH / ½ DCH of the DCCH | 43.4% / 15.3% |

- Figure A.5

### A.2.5.2	1.28 Mcps TDD Option

- Table A.5A

| Parameter | Value |
| --- | --- |
| Information data rate | 384 kbps |
| RU's allocated | 4TS (9*SF16) = 36RU/5ms |
| Midamble | 144 |
| Interleaving | 20 ms |
| Power control (TPC) | 16 Bit/user/10ms |
| TFCI | 64 Bit/user/10ms |
| Synchronisation Shift (SS) | 16 Bit/user/10ms |
| Inband signalling DCCH | 2.4 kbps |
| Puncturing level at Code rate: 1/3 DCH of the DTCH / ½ DCH of the DCCH | 47% / 12% |

- Figure A.5A

### A.2.5.3	7.68 Mcps TDD Option

- Table A.5B

| Parameter | Value |
| --- | --- |
| Information data rate | 384 kbps |
| RU´s allocated | 8*3TS = 24RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate : 1/3 DCH of the DTCH / ½ DCH of the DCCH | 43.4% / 15.3% |

- Figure A.5B

## A.2.6	BCH reference measurement channel

[mapped to 1 code SF16]

### A.2.6.1	3.84 Mcps TDD Option

- Table A.6

| Parameter | Value |
| --- | --- |
| Information data rate: | 12.3 kbps |
| RU´s allocated | 1 RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 0 bit |
| TFCI | 0 bit |
| Puncturing level | 10% |

- Figure A.6

### A.2.6.2	1.28 Mcps TDD Option

- Table A.6A

| Parameter | Value |
| --- | --- |
| Information data rate: | 12.3 kbps |
| RU´s allocated | 2 RU |
| Midamble | 144 chips |
| Interleaving | 20 ms |
| Power control | 0 bit |
| TFCI | 0 bit |
| Puncturing level | 13% |

- Figure A.6A

### A.2.6.3	7.68 Mcps TDD Option

- Table A.6

| Parameter | Value |
| --- | --- |
| Information data rate: | 12.3 kbps |
| RU´s allocated | 1 RU |
| Midamble | 1024 chips |
| Interleaving | 20 ms |
| Power control | 0 bit |
| TFCI | 0 bit |
| Puncturing level | 10% |

- Figure A.6B

## A.2.7	UL multi code reference measurement channel (12.2 kbps)

### A.2.7.1	3.84 Mcps TDD Option

- Table A.7

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 512 chips |
| Interleaving | 20 ms |
| Power control | 2 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 5% / 0 % |

- Figure A.7

### A.2.7.2	1.28 Mcps TDD Option

- Table A.7A

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU's allocated | 1TS (2*SF16) = 2RU/5ms |
| Midamble | 144 |
| Interleaving | 20 ms |
| Power control (TPC) | 4 Bit/user/10ms |
| TFCI | 16 Bit/user/10ms |
| 4 Bit reserved for future use (place of SS) | 4 Bit/user/10ms |
| Inband signalling DCCH | 2.4 kbps |
| Puncturing level at Code rate 1/3: DCH of the DTCH / DCH of the DCCH | 33% / 33% |

- Figure A.7A

### A.2.7.3	7.68 Mcps TDD Option

- Table A.7B

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 1024 chips |
| Interleaving | 20 ms |
| Power control | 2 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 5% / 0 % |

- Figure A.7B

## A.2.8	DL reference measurement channel (2 Mbps)

### A.2.8.1	3.84 Mcps TDD Option

- Table A.8

| Parameter | Value |
| --- | --- |
| Information data rate | 2048 kbps |
| RU´s allocated | 16*12TS = 192RU |
| Midamble | 256 chips |
| Interleaving | 10 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 13.9% / 0% |

- Figure A.8

### A.2.8.2	1.28 Mcps TDD Option

- Table A.8A

| Parameter | Value |
| --- | --- |
| Information data rate | 2048 kbps |
| RU's allocated | 5TS (1*SF1) = 80RU/5ms |
| Midamble | 144 |
| Interleaving | 10 ms |
| Power control (TPC) | 6 Bit/user/10ms |
| TFCI | 48 Bit/user/10ms |
| Synchronisation Shift (SS) | 6 Bit/user/10ms |
| Inband signalling DCCH | no |
| Coding | no |
| Modulation | 8PSK |

- Figure A.8A

### A.2.8.3	7.68 Mcps TDD Option

- Table A.8B

| Parameter | Value |
| --- | --- |
| Information data rate | 2048 kbps |
| RU´s allocated | 16*12TS = 192RU |
| Midamble | 512 chips |
| Interleaving | 10 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Inband signalling DCCH | 2 kbps |
| Puncturing level at Code rate 1/3 : DCH of the DTCH / DCH of the DCCH | 13.9% / 0% |

- Figure A.8B

## A.2.9	DL reference measurement channel for MBSFN only Ues

### A.2.9.1	3.84 Mcps TDD Option

#### A.2.9.1.1	Non-IMB

- TableA.8C

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 320 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Repetition level at code rate 1/3: FACH of the MTCH | 29% |

Information data

CRC attachment

Turbo coding 1/3

Trellis termination

1st Interleaving

RF segmentation

Repetition rate-matching (29%)

244

244

16

260 x 3 = 780

780

12

792 bits/20ms

396

512

2nd Interleaving

512

TFCI

512

16

Timeslot segmentation:

Burst type 4

2 codes, SF = 16

132

8

320

8

124

124

Radio Frame #1

396

512

512

512

16

132

320

132

8

320

8

124

124

132

320

Radio Frame #2

- Figure A.8C

#### A.2.9.1.2 IMB

- TableA.8CA: DL reference measurement channel, physical parameters (IMB)

| Parameter | Value |
| --- | --- |
| Information data rate | 28 kbps |
| Number of physical channels in the S-CCPCH type 2 sub-frame | 2 |
| Slot format #i | 2 and 3 |
| TFCI | On |

- TableA.8CB: DL reference measurement channel, transport parameters (IMB)

| Parameter | Value |
| --- | --- |
| Transport channel number | 1 |
| Transport block size | 552 |
| Transport block set size | 552 |
| Transmission Time Interval | 20 ms |
| Type of Error Protection | Turbo Coding |
| Coding Rate | 1/3 |
| Rate Matching Attribute | 256 |
| Size of CRC | 16 |

Table A.8CC defines the physical channels that are transmitted simultaneously with the IMB DL reference measurement channel.  Table A.8CC is applicable for all measurements on the receiver characteristics (clause 7).  OCNS physical channels are applicable only in the case of subclause 7.4.

- TableA.8CC: Additional downlink physical channels transmitted simultanouesly with the IMB DL reference measurement channel

| Physical Channel | Ec / Ior | Notes |
| --- | --- | --- |
| P-CPICH | -10 dB |  |
| T-CPICH | -0.457 dB |  |
| P-CCPCH | -12 dB |  |
| SCH | -12 dB |  |
| OCNS1 | Necessary power so that total transmit power spectral density of Node B (Ior) adds to one | OCNS consists of 8 physical channels each using SF16 and QPSK modulation. Each OCNS code has equal power. |
| NOTE 1: Applicable only in the case of sub-clause 7.4 |  |  |

- Figure A.8CA

### A.2.9.2	VOID

### A.2.9.3	7.68 Mcps TDD Option

- TableA.8D

| Parameter | Value |
| --- | --- |
| Information data rate | 12.2 kbps |
| RU´s allocated | 2 RU |
| Midamble | 640 chips |
| Interleaving | 20 ms |
| Power control | 0 Bit/user |
| TFCI | 16 Bit/user |
| Repetition level at code rate 1/3: FACH of the MTCH | 29% |

Information data

CRC attachment

Turbo coding 1/3

Trellis termination

1st Interleaving

RF segmentation

Repetition rate-matching (29%)

244

244

16

260 x 3 = 780

780

12

792 bits/20ms

396

512

2nd Interleaving

512

TFCI

512

16

Timeslot segmentation:

Burst type 4

2 codes, SF = 32

132

8

640

8

124

124

Radio Frame #1

396

512

512

512

16

132

640

132

8

640

8

124

124

132

640

Radio Frame #2

- Figure A.8D

# A.3	HSDPA reference measurement channels

## A.3.1	HSDPA reference measurement channels for 3,84 Mcps TDD option

### A.3.1.1	Reference measurement channels for 7,3 Mbps - Category 8 - UE

#### A.3.1.1.1	QPSK modulation scheme for test 1, 2, 3

Table A.9: HS-PDSCH fixed reference channel for the PA3, PB3, and VA30 Channel models - 
Category 8

| Parameter | Unit |  | Value |  |
| --- | --- | --- | --- | --- |
| Maximum information bit throughput |  | Mbps |  | 2,6496 |
| Number of HARQ Processes |  | Processes |  | 4 |
| Information Bit Payload () |  | Bits |  | 26496 |
| Number Code Blocks |  | Blocks |  | 6 |
| Total Available of Soft Channel bits in UE |  | Bits |  | 353280 |
| Number of Soft Channel bit per HARQ Proc. |  | Bits |  | 88320 |
| Number of coded bits per TTI |  | Bits |  | 35328 |
| Coding Rate |  |  |  | 3/4 |
| Number of HS-PDSCH Timeslots |  | Slots |  | 8 |
| Number of HS-PDSCH codes per TS |  | Codes |  | 16 |
| Spreading factor |  | SF |  | 16 |

Figure A.9: Coding for HS-PDSCH fixed reference channel with QPSK modulation for the PA3, PB3, and VA30 Channels - Category 8

#### A.3.1.1.2	QPSK modulation scheme for test 4

- Table A.10: HS-PDSCH fixed reference channel for the VA120 Channel model - Category 8

| Parameter | Unit |  | Value |  |
| --- | --- | --- | --- | --- |
| Maximum information bit throughput |  | Mbps |  | 2,3176 |
| Number of HARQ Processes |  | Processes |  | 4 |
| Information Bit Payload () |  | Bits |  | 23176 |
| Number Code Blocks |  | Blocks |  | 5 |
| Total Available of Soft Channel bits in UE |  | Bits |  | 353280 |
| Number of Soft Channel bit per HARQ Proc. |  | Bits |  | 88320 |
| Number of coded bits per TTI |  | Bits |  | 30912 |
| Coding Rate |  |  |  | 3/4 |
| Number of HS-PDSCH Timeslots |  | Slots |  | 8 |
| Number of HS-PDSCH codes per TS |  | Codes |  | 14 |
| Spreading factor |  | SF |  | 16 |

Figure A.10: Coding for HS-PDSCH fixed reference channel with QPSK modulation for the VA120 Channel - Category 8

#### A.3.1.1.3	16QAM modulation scheme for test 1, 2, 3

Table A.11: HS-PDSCH fixed reference channel for the PA3, PB3, and VA30 Channel models - Category 8

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16-QAM |
| Maximum information bit throughput | Mbps | 5,2996 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 52996 |
| Number Code Blocks | Blocks | 11 |
| Total Available of Soft Channel bits in UE | Bits | 353280 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 88320 |
| Number of coded bits per TTI | Bits | 70656 |
| Coding Rate |  | 3/4 |
| Number of HS-PDSCH Timeslots | Slots | 8 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Figure A.11: Coding for HS-PDSCH fixed reference channel with 16-QAM modulation for the PA3 PB3, and VA30 Channels - Category 8

#### A.3.1.1.4	16QAM modulation scheme for test 4

Table A.12: HS-PDSCH fixed reference channel for the PA3, PB3, and VA30 Channel models - Category 8

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16-QAM |
| Maximum information bit throughput | Mbps | 3,4773 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 34773 |
| Number Code Blocks | Blocks | 7 |
| Total Available of Soft Channel bits in UE | Bits | 353280 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 88320 |
| Number of coded bits per TTI | Bits | 61824 |
| Coding Rate |  | 9/16 |
| Number of HS-PDSCH Timeslots | Slots | 8 |
| Number of HS-PDSCH codes per TS | Codes | 14 |
| Spreading factor | SF | 16 |

Figure A.12: Coding for HS-PDSCH fixed reference channel with 16-QAM modulation for the VA120 Channel - Category 8

## A.3.2	HSDPA reference measurement channels for 1.28 Mcps TDD option

### A.3.2.1	Reference measurement channels for 0.5 Mbps UE class

#### A.3.2.1.1	QPSK modulation scheme

- Table A.13-1

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | Kbps | 199.2 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 996 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 11264 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 2816 |
| Number of coded bits per TTI | Bits | 1760 |
| Coding Rate | - | 0.5795 |
| Number of HS-DSCH Timeslots | Slots | 2 |
| Number of HS-PDSCH codes per TS | Codes | 10 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-1

### A.3.2.2	Reference measurement channels for 1.1 Mbps UE class

#### A.3.2.2.1	QPSK modulation scheme

Reference channel in A.3.2.1.1 applies.

#### A.3.2.2.2	16QAM modulation scheme

- Table A.13-2

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | Kbps | 578.6 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 2893 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 22528 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 5632 |
| Number of coded bits per TTI | Bits | 4224 |
| Coding Rate | - | 0.69 |
| Number of HS-DSCH Timeslots | Slots | 2 |
| Number of HS-PDSCH codes per TS | Codes | 12 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-2

### A.3.2.3	Reference measurement channels for 1.6 Mbps UE class

#### A.3.2.3.1	QPSK modulation scheme

- Table A.13-3

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | Kbps | 357.4 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 1787 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 33792 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 8448 |
| Number of coded bits per TTI | Bits | 2640 |
| Coding Rate | - | 0.686 |
| Number of HS-DSCH Timeslots | Slots | 3 |
| Number of HS-PDSCH codes per TS | Codes | 10 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-3

#### A.3.2.3.2	16QAM modulation scheme

- Table A.13-4

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | Kbps | 634.6 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 3173 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 33792 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 8448 |
| Number of coded bits per TTI | Bits | 6336 |
| Coding Rate | - | 0.505 |
| Number of HS-DSCH Timeslots | Slots | 3 |
| Number of HS-PDSCH codes per TS | Codes | 12 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure A.13-4

### A.3.2.4	Reference measurement channels for 2.2 Mbps UE class

#### A.3.2.4.1	QPSK modulation scheme

- Table A.13-5

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | Kbps | 539 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 2695 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 45056 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 11264 |
| Number of coded bits per TTI | Bits | 3520 |
| Coding Rate | - | 0.772 |
| Number of HS-DSCH Timeslots | Slots | 4 |
| Number of HS-PDSCH codes per TS | Codes | 10 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-5

#### A.3.2.4.2	16QAM modulation scheme

- Table A.13-6

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | Kbps | 782.2 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 3911 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 45056 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 11264 |
| Number of coded bits per TTI | Bits | 8448 |
| Coding Rate | - | 0.4658 |
| Number of HS-DSCH Timeslots | Slots | 4 |
| Number of HS-PDSCH codes per TS | Codes | 12 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-6

### A.3.2.5	Reference measurement channels for 2.8 Mbps UE class

#### A.3.2.5.1	QPSK modulation scheme

- Table A.13-7

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | Kbps | 621 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 3105 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 56320 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 14080 |
| Number of coded bits per TTI | Bits | 4400 |
| Coding Rate | - | 0.711 |
| Number of HS-DSCH Timeslots | Slots | 5 |
| Number of HS-PDSCH codes per TS | Codes | 10 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-7

#### A.3.2.5.2	16QAM modulation scheme

- Table A.13-8

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | Kbps | 1278.6 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 6393 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 56320 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 14080 |
| Number of coded bits per TTI | Bits | 10560 |
| Coding Rate | - | 0.6077 |
| Number of HS-DSCH Timeslots | Slots | 5 |
| Number of HS-PDSCH codes per TS | Codes | 12 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure 13-8

### A.3.2.6	PLCCH reference measurement channel

- Table A.14A

| Parameter | Unit | Value |
| --- | --- | --- |
| Information bits | bits | 42 |
| Encoded bits | bits | 88 |
| Number of codes |  | 1 |
| Number of timeslots |  | 1 |
| TTI | ms | 5 |
| Spreading Factor | SF | 16 |
| Coding |  | Repetition encoding |

### A.3.2.7	 Reference measurement channels for Category 16-24 UE

#### A.3.2.7.1	Reference measurement channel for category 16-18 UE

- Table A.15A reference measurement channel for category 16-18 UE

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 64QAM |
| Maximum information bit throughput | Mbps | 1.2496 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 6248 |
| Number Code Blocks | Blocks | 2 |
| Total Available of Soft Channel bits in UE | Bits | 50688 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 12672 |
| Number of coded bits per TTI | Bits | 11088 |
| Coding Rate | - | 0.5635 |
| Number of HS-DSCH Timeslots | Slots | 3 |
| Number of HS-PDSCH codes per TS | Codes | 14 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure A.15A Fixed reference measurement channel for category 16 UE

#### A.3.2.7.2	Reference measurement channel for category 19-21 UE

- Table A.16A Fixed reference measurement channel for category 19-21 UE

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 64QAM |
| Maximum information bit throughput | Mbps | 1.6976 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 8488 |
| Number Code Blocks | Blocks | 2 |
| Total Available of Soft Channel bits in UE | Bits | 67584 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 16896 |
| Number of coded bits per TTI | Bits | 14784 |
| Coding Rate | - | 0.57 |
| Number of HS-DSCH Timeslots | Slots | 4 |
| Number of HS-PDSCH codes per TS | Codes | 14 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure A.16A Fixed reference measurement channel for category 19 UE

#### A.3.2.7.3	Reference measurement channel for category 22-24 UE

- Table A.17A Fixed reference measurement channel for category 22-24 UE

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 64QAM |
| Maximum information bit throughput | Mbps | 2.0464 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 10232 |
| Number Code Blocks | Blocks | 3 |
| Total Available of Soft Channel bits in UE | Bits | 84480 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 21120 |
| Number of coded bits per TTI | Bits | 18480 |
| Coding Rate | - | 0.55 |
| Number of HS-DSCH Timeslots | Slots | 5 |
| Number of HS-PDSCH codes per TS | Codes | 14 |
| Spreading factor | SF | 16 |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

- Figure A.17A Fixed reference measurement channel for category 22 UE

### A.3.2.8	Reference measurement channel of 48kbps

- Table A.18A

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | kbps | 48 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 240 |
| Number Code Blocks | Blocks | 1 |
| Number of coded bits per TTI | Bits | 804 |
| Coding Rate | - | 0.375 |
| Number of HS-DSCH Timeslots | Slots | 1 |
| Number of HS-PDSCH codes per TS | Codes | 8 |
| Spreading factor | SF | 16 |
| HS-PDSCHi_Ec/Ior | dB | -9.03 |
| Number of DPCHo | - | 0 |
| DPCH0i_Ec/Ior | dB | - |
| Note: For multi-carrier reception, the reference measurement channel is applied to each of the carriers. |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

240

240

24

264

792

704

704

704

12

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

88bits/HS-PDSCH/TTI

- Figure A.18A

### A.3.2.9	Void

### A.3.2.10	Reference Measurement Channel for category 25 UE

#### A.3.2.10.1	QPSK modulation scheme

- Table A.20A Reference Measurement Channel for Category 25 (QPSK)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | QPSK | QPSK |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 1.0976 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 564.8 | 532.8 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 2824 | 2664 |
| Number Code Blocks | Blocks | 1 | 1 |
| Total Available of Soft Channel bits in UE | Bits | 202752 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 25344 | 25344 |
| Number of coded bits per TTI | Bits | 4224 | 4224 |
| Coding Rate | - | 0.6697 | 0.6323 |
| Number of HS-DSCH Timeslots | Slots | 3 | 3 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

2824

2824

24

2848

8544

4224

4224

4224

Slot segmentation

12

1408bits/TS

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

- Figure A.20A Reference Measurement Channel for Category 25 (QPSK) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

2664

2664

24

2688

8064

4224

4224

4224

Slot segmentation

12

1408bits/TS

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

- Figure A.21A Reference Measurement Channel for Category 25 (QPSK) - Second Stream

#### A.3.2.10.2	16QAM modulation scheme

- Table A.21A Reference Measurement Channel for Category 25 (16QAM)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | 16QAM | 16QAM |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 1.8416 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 947.2 | 894.4 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 4736 | 4472 |
| Number Code Blocks | Blocks | 1 | 1 |
| Total Available of Soft Channel bits in UE | Bits | 202752 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 25344 | 25344 |
| Number of coded bits per TTI | Bits | 8448 | 8448 |
| Coding Rate | - | 0.561 | 0.529 |
| Number of HS-DSCH Timeslots | Slots | 3 | 3 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

4736

4736

24

4760

14280

8448

8448

8448

Slot segmentation

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.22A Reference Measurement Channel for Category 25 (16QAM) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

4472

4472

24

4496

13488

8448

8448

8448

Slot segmentation

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.23A Reference Measurement Channel for Category 25 (16QAM) - Second Stream

### A.3.2.11	Reference Measurement Channel for category 26 UE

#### A.3.2.11.1	QPSK modulation scheme

- Table A.22A Reference Measurement Channel for Category 26 (QPSK)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | QPSK | QPSK |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 1.5152 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 780.8 | 734.4 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 3904 | 3672 |
| Number Code Blocks | Blocks | 1 | 1 |
| Total Available of Soft Channel bits in UE | Bits | 270336 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 33792 | 33792 |
| Number of coded bits per TTI | Bits | 5632 | 5632 |
| Coding Rate | - | 0.693 | 0.652 |
| Number of HS-DSCH Timeslots | Slots | 4 | 4 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

3904

3904

24

3928

11784

5632

5632

5632

Slot segmentation

12

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

1408bits/TS

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

- Figure A.24A Reference Measurement Channel for Category 26 (QPSK) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

3672

3672

24

3696

11088

5632

5632

5632

Slot segmentation

12

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

1408bits/TS

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

- Figure A.25A Reference Measurement Channel for Category 26 (QPSK) - Second Stream

#### A.3.2.11.2	16QAM modulation scheme

- Table A.23A Reference Measurement Channel for Category 26 (16QAM)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | 16QAM | 16QAM |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 2.8192 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 1452.8 | 1366.4 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 7264 | 6832 |
| Number Code Blocks | Blocks | 2 | 2 |
| Total Available of Soft Channel bits in UE | Bits | 270336 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 33792 | 33792 |
| Number of coded bits per TTI | Bits | 11264 | 11264 |
| Coding Rate | - | 0.645 | 0.607 |
| Number of HS-DSCH Timeslots | Slots | 4 | 4 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

7264

7264

24

3644

10932

11264

11264

11264

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

3644

12

10932

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.26A Reference Measurement Channel for Category 26 (16QAM) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

6832

6832

24

3428

10284

11264

11264

11264

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

3428

12

10284

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.27A Reference Measurement Channel for Category 26 (16QAM) - Second Stream

### A.3.2.12	Reference Measurement Channel for category 27 UE

#### A.3.2.12.1	QPSK modulation scheme

- Table A.24A Reference Measurement Channel for Category 27 (QPSK)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | QPSK | QPSK |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 1.7808 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 920 | 860.8 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 4600 | 4304 |
| Number Code Blocks | Blocks | 1 | 1 |
| Total Available of Soft Channel bits in UE | Bits | 337920 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 42240 | 42240 |
| Number of coded bits per TTI | Bits | 7040 | 7040 |
| Coding Rate | - | 0.653 | 0.611 |
| Number of HS-DSCH Timeslots | Slots | 5 | 5 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

4600

4600

24

4624

13872

7040

7040

7040

Slot segmentation

12

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

1408bits/TS

- Figure A.28A Reference Measurement Channel for Category 27 (QPSK) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

4304

4304

24

4328

12984

7040

7040

7040

Slot segmentation

12

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

1408bits/TS

- Figure A.29A Reference Measurement Channel for Category 27 (QPSK) - Second Stream

#### A.3.2.12.2	16QAM modulation scheme

- Table A.25A Reference Measurement Channel for Category 27 (16QAM)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | 16QAM | 16QAM |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 3.4336 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 1772.8 | 1660.8 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 8864 | 8304 |
| Number Code Blocks | Blocks | 2 | 2 |
| Total Available of Soft Channel bits in UE | Bits | 337920 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 42240 | 42240 |
| Number of coded bits per TTI | Bits | 14080 | 14080 |
| Coding Rate | - | 0.630 | 0.590 |
| Number of HS-DSCH Timeslots | Slots | 5 | 5 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

8864

8864

24

4444

13332

14080

14080

14080

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

4444

12

13332

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.30A Reference Measurement Channel for Category 27 (16QAM) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

8304

8304

24

4164

12492

14080

14080

14080

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

4164

12

12492

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.31A Reference Measurement Channel for Category 27 (16QAM) - Second Stream

### A.3.2.13	Reference Measurement Channel for category 28 UE

#### A.3.2.13.1	64QAM modulation scheme

- Table A.26A Reference Measurement Channel for Category 28 (64QAM)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | 64QAM | 64QAM |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 3.128 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 1614.4 | 1513.6 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 8072 | 7568 |
| Number Code Blocks | Blocks | 2 | 2 |
| Total Available of Soft Channel bits in UE | Bits | 304128 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 38016 | 38016 |
| Number of coded bits per TTI | Bits | 12672 | 12672 |
| Coding Rate | - | 0.637 | 0.597 |
| Number of HS-DSCH Timeslots | Slots | 3 | 3 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

8072

8072

24

4048

12144

12672

12672

12672

12

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

4048

12

12144

4224bits /TS

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

- Figure A.32A Reference Measurement Channel for Category 28 (64QAM) - First Stream

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

Slot segmentation

7568

7568

24

3796

11388

12672

12672

12672

12

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

3796

12

11388

4224bits /TS

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

132

132

144c

- Figure A.33A Reference Measurement Channel for Category 28 (64QAM) - Second Stream

### A.3.2.14	Reference Measurement Channel for category 29 UE

#### A.3.2.14.1	64QAM modulation scheme

- Table A.27A Reference Measurement Channel for Category 29 (64QAM)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | 64QAM | 64QAM |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 4.032 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 2084.8 | 1947.2 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 10424 | 9736 |
| Number Code Blocks | Blocks | 3 | 2 |
| Total Available of Soft Channel bits in UE | Bits | 405504 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 50688 | 50688 |
| Number of coded bits per TTI | Bits | 16896 | 16896 |
| Coding Rate | - | 0.617 | 0.576 |
| Number of HS-DSCH Timeslots | Slots | 4 | 4 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

- Figure A.34A Reference Measurement Channel for Category 29 (64QAM) - First Stream

- Figure A.35A Reference Measurement Channel for Category 29 (64QAM) - Second Stream

### A.3.2.15	Reference Measurement Channel for category 30 UE

#### A.3.2.15.1	64QAM modulation scheme

- Table A.28A Reference Measurement Channel for Category 30 (64QAM)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Stream |  | 1st stream | 2nd stream |
| Modulation | - | 64QAM | 64QAM |
| Combined Nominal Avg. Inf. Bit Rate | Mbps | 4.9072 |  |
| Nominal Avg. Inf. Bit Rate per stream | kbps | 2542.4 | 2364.8 |
| Number of HARQ Processes | Processes | 4 | 4 |
| Information Bit Payload () | Bits | 12712 | 11824 |
| Number Code Blocks | Blocks | 3 | 3 |
| Total Available of Soft Channel bits in UE | Bits | 506880 |  |
| Number of Soft Channel bit per HARQ Proc. | Bits | 63360 | 63360 |
| Number of coded bits per TTI | Bits | 21120 | 21120 |
| Coding Rate | - | 0.602 | 0.560 |
| Number of HS-DSCH Timeslots | Slots | 5 | 5 |
| Number of HS-PDSCH codes per TS | Codes | 16 | 16 |
| Spreading factor | SF | 16 | 16 |
| Note: For UE support SF=1 only in dual stream transmission, both the number of HS-PDSCH codes per TS and spreading factor in the FRC should be changed to 1. |  |  |  |

- Figure A.36A Reference Measurement Channel for Category 30 (64QAM) - First Stream

- Figure A.37A Reference Measurement Channel for Category 30 (64QAM) - Second Stream

## A.3.2A	HSDPA reference measurement channels for 7,68 Mcps TDD option

### A.3.2A.1	Reference measurement channels for 5,3 Mbps - Category 8 - UE

#### A.3.2A.1.1	QPSK modulation scheme for test 1, 2, 3 & 4

Table A.15: HS-PDSCH fixed reference channel for the PA3, PB3, VA30 and VA120 Channel models - 
Category 8

| Parameter | Unit |  | Value |  |
| --- | --- | --- | --- | --- |
| Maximum information bit throughput |  | Mbps |  | 1.7612 |
| Number of HARQ Processes |  | Processes |  | 3 |
| Information Bit Payload () |  | Bits |  | 17612 |
| Number Code Blocks |  | Blocks |  | 4 |
| Total Available of Soft Channel bits in UE |  | Bits |  | 211968 |
| Number of Soft Channel bit per HARQ Proc. |  | Bits |  | 70656 |
| Number of coded bits per TTI |  | Bits |  | 35328 |
| Coding Rate |  |  |  | 1/2 |
| Number of HS-PDSCH Timeslots |  | Slots |  | 4 |
| Number of HS-PDSCH codes per TS |  | Codes |  | 32 |
| Spreading factor |  | SF |  | 32 |

Figure A.15: Coding for HS-PDSCH fixed reference channel with QPSK modulation for the PA3, PB3, VA30 and VA120 Channels - Category 8

#### A.3.2A.1.2	16QAM modulation scheme for test 1, 2, 3 & 4

Table A.16: HS-PDSCH fixed reference channel for the PA3, PB3, VA30 and VA120 Channel models - Category 8

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16-QAM |
| Maximum information bit throughput | Mbps | 3.5066 |
| Number of HARQ Processes | Processes | 3 |
| Information Bit Payload () | Bits | 35066 |
| Number Code Blocks | Blocks | 7 |
| Total Available of Soft Channel bits in UE | Bits | 211968 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 70656 |
| Number of coded bits per TTI | Bits | 70656 |
| Coding Rate |  | ½ |
| Number of HS-PDSCH Timeslots | Slots | 4 |
| Number of HS-PDSCH codes per TS | Codes | 32 |
| Spreading factor | SF | 32 |

Figure A.16: Coding for HS-PDSCH fixed reference channel with 16-QAM modulation for the PA3 PB3, VA30 and VA120 Channels - Category 8

## A.3.3	Variable Reference Channel definition for 3,84 Mcps and 1,28 Mcps TDD options

The variable reference measurement channels are defined by:

a)	The maximum information bit payload that is determined by the UE capability class under test and the allocated resource units (and hence implicitly by the CQI table applicable to the UE under test as derived from TS25.321).

b)	The most recently received UE CQI report.

## A.3.4	HSDPA reference measurement channels for 1.28 Mcps TDD option for MU-MIMO

### A.3.4.1	Reference measurement channels for category 1-3

#### A.3.4.1.1	QPSK modulation scheme

- Table A.3.4.1.1

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | kbps | 360.8 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 1804 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 11264 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 2816 |
| Number of coded bits per TTI | Bits | 2816 |
| Coding Rate | - | 0.6406 |
| Number of HS-DSCH Timeslots | Slots | 2 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

1804

1804

24

1828

5484

2816

2816

2816

Slot segmentation

12

1408bits/TS

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

- Figure A.3.4.1.1

### A.3.4.2	Reference measurement channels for category 4-6

#### A.3.4.2.1	QPSK modulation scheme

- Table A.3.4.2.1

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | kbps | 348 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 1740 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 22528 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 5632 |
| Number of coded bits per TTI | Bits | 2816 |
| Coding Rate | - | 0.6179 |
| Number of HS-DSCH Timeslots | Slots | 2 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

1740

1740

24

1764

5292

2816

2816

2816

Slot segmentation

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

12

1408bits/TS

- Figure A.3.4.2.1

#### A.3.4.2.2	16QAM modulation scheme

- Table A.3.4.2.2

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | kbps | 640.4 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 3202 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 22528 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 5632 |
| Number of coded bits per TTI | Bits | 5632 |
| Coding Rate | - | 0.5685 |
| Number of HS-DSCH Timeslots | Slots | 2 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

12

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

3202

3202

24

3226

9678

5632

5632

5632

Slot segmentation

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.3.4.2.2

### A.3.4.3	Reference measurement channels for category 7-9

#### A.3.4.3.1	QPSK modulation scheme

- Table A.3.4.3.1

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | kbps | 504.4 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 2522 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 33792 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 8448 |
| Number of coded bits per TTI | Bits | 4224 |
| Coding Rate | - | 0.5971 |
| Number of HS-DSCH Timeslots | Slots | 3 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

2522

2522

24

2546

7638

4224

4224

4224

12

1408bits/TS

Slot segmentation

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

- Figure A.3.4.3.1

#### A.3.4.3.2	16QAM modulation scheme

- Table A.3.4.3.2

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | kbps | 1004.2 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 5021 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 33792 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 8448 |
| Number of coded bits per TTI | Bits | 8448 |
| Coding Rate | - | 0.5943 |
| Number of HS-DSCH Timeslots | Slots | 3 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

5021

5021

24

5045

15315

8448

8448

8448

Slot segmentation

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

12

- Figure A.3.4.3.2

### A.3.4.4	Reference measurement channels for category 10-12

#### A.3.4.4.1	QPSK modulation scheme

- Table A.3.4.4.1

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | kbps | 691 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 3455 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 45056 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 11264 |
| Number of coded bits per TTI | Bits | 5632 |
| Coding Rate | - | 0.6135 |
| Number of HS-DSCH Timeslots | Slots | 4 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

3455

3455

24

3479

10437

5632

5632

5632

Slot segmentation

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

1408bits/TS

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

12

- Figure A.3.4.4.1

#### A.3.4.4.2	16QAM modulation scheme

- Table A.3.4.4.2

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | kbps | 1284.8 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 6424 |
| Number Code Blocks | Blocks | 2 |
| Total Available of Soft Channel bits in UE | Bits | 45056 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 11264 |
| Number of coded bits per TTI | Bits | 11264 |
| Coding Rate | - | 0.5703 |
| Number of HS-DSCH Timeslots | Slots | 4 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

6424

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

12

Slot segmentation

6424

24

3224

9672

11264

11264

11264

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

3224

9672

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.3.4.4.2

### A.3.4.5	Reference measurement channels for category 13-15

#### A.3.4.5.1	QPSK modulation scheme

- Table A.3.4.5.1

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | QPSK |
| Maximum information bit throughput | kbps | 862.2 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 4311 |
| Number Code Blocks | Blocks | 1 |
| Total Available of Soft Channel bits in UE | Bits | 56320 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 14080 |
| Number of coded bits per TTI | Bits | 7040 |
| Coding Rate | - | 0.6124 |
| Number of HS-DSCH Timeslots | Slots | 5 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

4311

4311

24

4335

13005

7040

7040

7040

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

Slot segmentation

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

44

44

144c

1408bits/TS

12

- Figure A.3.4.5.1

#### A.3.4.5.2	16QAM modulation scheme

- Table A.3.4.5.2

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation | - | 16QAM |
| Maximum information bit throughput | kbps | 1557 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload () | Bits | 7785 |
| Number Code Blocks | Blocks | 2 |
| Total Available of Soft Channel bits in UE | Bits | 56320 |
| Number of Soft Channel bit per HARQ Proc. | Bits | 14080 |
| Number of coded bits per TTI | Bits | 14080 |
| Coding Rate | - | 0.5529 |
| Number of HS-DSCH Timeslots | Slots | 5 |
| Number of HS-PDSCH codes per TS | Codes | 16 |
| Spreading factor | SF | 16 |

Information data

CRC attachment

Code Block Segmentation

Rate matching

Turbo Coding

Bit Scrambling

Interleaving

Physical channel mapping

7785

7785

24

Slot segmentation

3905

11715

14080

14080

14080

12

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

3905

12

11715

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

2816bits/TS

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

88

88

144c

- Figure A.3.4.5.2

# A.4	Downlink reference parameter for MBMS tests

## A.4.1	MCCH

### A.4.1.1	3.84 Mcps TDD Option

#### A.4.1.1.1	Non-IMB

The parameters for the MCCH demodulation tests are specified in Table A.41 and Table A.42.

- Table A.41: Physical channel parameters for S-CCPCH

| Parameter | Unit | Level | Level |
| --- | --- | --- | --- |
| Channel bit rate | kbps | 22.8 | 22.8 |
| Channel symbol rate | ksps | 11.4 | 11.4 |
| Slot Format #i | - | 3 | 21 |
| TFCI | - | ON | ON |

- Table A.42: Transport channel parameters for S-CCPCH

| Parameter | MCCH |
| --- | --- |
| User Data Rate | 7.2 kbps |
| Number Transport Channel | 1 |
| Transport Block Size | 581 |
| Transport Block Set Size | 581 |
| RLC SDU block size | 4088 |
| Transmission Time Interval | 80 ms |
| Repetition period | 640 ms |
| Modification period | 1280 ms |
| Type of Error Protection | Turbo |
| Coding Rate | 1/3 |
| Rate Matching attribute | 256 |
| Size of CRC | 16 |

#### A.4.1.1.2	IMB

The parameters for the MCCH demodulation tests are specified in Table A.41A and Table A.41B and Table A.41C..

- Table A.41A: Physical channel parameters for S-CCPCH frame type 1

| Parameter | Unit | Level |
| --- | --- | --- |
| Channel bit rate | kbps | 30 |
| Channel symbol rate | ksps | 15 |
| Slot format #i | - | 1 |
| TFCI | - | ON |

- Table A.41B: Transport channel parameters for S-CCPCH frame type 1

| Parameter | MCCH |
| --- | --- |
| User Data Rate | 6.4 kbps |
| Transport Block Size | 72 |
| Transport Block Set Size | 72 |
| RLC SDU block size | 4088 |
| Transmission Time Interval | 10 ms |
| Repetition Period | 640 ms |
| Modification Period | 1280 ms |
| Type of Error Protection | Convolutional Coding |
| Coding Rate | 1/3 |
| Rate matching Attribute | 256 |
| Size of CRC | 16 |

- Table A.41C: Configuration of other physical channels during MCCH test

| Physical Channel | Power Ratio (Ec/Ior) | NOTE |
| --- | --- | --- |
| P-CPICH | -10 dB |  |
| T-CPICH | -0.457 dB |  |
| P-CCPCH | -12 dB |  |
| SCH | -12 dB | This power shall be divided equally between Primary and Secondary Synchronous channels |
| OCNS | Necessary power so that total transmit power spectral density of Node B (Ior) adds to one | OCNS interference consists of 15 codes of equal power, each code using SF16 and QPSK modulation |

### A.4.1.2	1.28 Mcps TDD Option

The parameters for the MCCH demodulation tests are specified in Table A.43 and Table A.44.

- Table A.43: Physical channel parameters for S-CCPCH

| Parameter | Unit | Level1 | Level2 |
| --- | --- | --- | --- |
| Channel bit rate | kbps | 17.6 | 19.2 |
| Channel symbol rate | ksps | 8.8 | 9.6 |
| Slot Format |  | No TPC, SS | No TPC SS |
| SF | - | 16 | 16 |
| TFCI | - | ON | ON |
| NOTE1: used for MCCH test in section 10.1.1.2. NOTE2: used for MCCH test in section 10.1.2.2. |  |  |  |

- Table A.44: Transport channel parameters for S-CCPCH

| Parameter | MCCH |
| --- | --- |
| User Data Rate | 7.6 kbps |
| Number Transport Channel | 1 |
| Transport Block Size | 72 |
| Transport Block Set Size | 72 |
| RLC SDU block size | 4088 |
| Transmission Time Interval | 10 ms |
| Repetition period | 640 ms |
| Modification period | 1280 ms |
| Type of Error Protection | Convolutional code 1/3 |
| Coding Rate | 1/3 |
| Rate Matching attribute | 160 |
| Size of CRC | 16 |
| TFCI | ON |

### A.4.1.3	7.68 Mcps TDD Option

The parameters for the MCCH demodulation tests are specified in Table A.44A and Table A.44B.

- Table A.44A: Physical channel parameters for S-CCPCH

| Parameter | Unit | Level | Level |
| --- | --- | --- | --- |
| Channel bit rate | kbps | 22.8 | 22.8 |
| Channel symbol rate | ksps | 11.4 | 11.4 |
| Slot Format #i | - | 3 | 21 |
| TFCI | - | ON | ON |

- Table A.44B: Transport channel parameters for S-CCPCH

| Parameter | MCCH |
| --- | --- |
| User Data Rate | 7.2 kbps |
| Number Transport Channel | 1 |
| Transport Block Size | 581 |
| Transport Block Set Size | 581 |
| RLC SDU block size | 4088 |
| Transmission Time Interval | 80 ms |
| Repetition period | 640 ms |
| Modification period | 1280 ms |
| Type of Error Protection | Turbo |
| Coding Rate | 1/3 |
| Rate Matching attribute | 256 |
| Size of CRC | 16 |

## A.4.2	MTCH

### A.4.2.1	3.84 Mcps TDD Option

#### A.4.2.1.1	Non-IMB

The parameters for the MTCH demodulation tests are specified in Table A.45 and Table A.46.

- Table A.45: Physical channel parameters for S-CCPCH

| Parameter | Unit | Level | Level | Level |
| --- | --- | --- | --- | --- |
| User Data Rate | kpbs | 512 | 256 | 128 |
| Modulation | - | 16QAM | QPSK | QPSK |
| Channel bit rate | kbps | 1547.8 | 388.8 | 388.8 |
| Channel symbol rate | ksps | 386.95 | 194.4 | 194.4 |
| Slot Format #i | - | 23 and 22 | 3 and 0 | 3 and 0 |
| TFCI | - | ON | ON | ON |
| Physical resources | - | 16 codes x SF16 1 timeslot/ frame | 16 codes x SF16 1 timeslot/ frame | 16 codes x SF16 1 timeslot/ frame |

- Table A.46: Transport channel parameters for S-CCPCH

| Parameter | MTCH |  |  |
| --- | --- | --- | --- |
| User Data Rate | 512 kbps | 256 kbps | 128 kbps |
| Number of Transport Channel | 1 | 1 | 1 |
| Transport Block Size | 2561 | 2561 | 2561 |
| Transport Block Set Size | 40976 | 10244 | 5122 |
| Nr of transport blocks/TTI | 16 | 4 | 2 |
| RLC SDU block size | 40688 | 10160 | 5072 |
| Transmission Time Interval | 80ms | 40 ms | 40 ms |
| Type of Error Protection | Turbo | Turbo | Turbo |
| Coding Rate | 1/3 | 1/3 | 1/3 |
| Rate Matching attribute | 256 | 256 | 256 |
| Size of CRC | 16 | 16 | 16 |
| Puncturing limit | 0.52 | 1.0 | 1.0 |

### A.4.2.1.2	IMB

The parameters for the MTCH demodulation tests are specified in Table A.46A, Table A.46B and Table A.46C.

- Table A.46A: Physical channel parameters for S-CCPCH frame type 2

| Parameter | Unit | Level |
| --- | --- | --- |
| User Data Rate | kbps | 512 |
| Modulation | - | 16QAM |
| Channel bit rate | kbps | 960 |
| Channel symbol rate | ksps | 240 |
| Slot Format #i | - | 4 and 5 |
| TFCI | - | ON |
| Physical resources | - | 5 codes x SF16 1 x 2ms sub-frame |

- Table A.46B: Transport channel parameters for S-CCPCH frame type 2

| Parameter | MTCH |
| --- | --- |
| User Data Rate | 512 kbps |
| Number of Transport Channel | 1 |
| Transport Block Size | 2536 |
| Transport Block Set Size | 40576 |
| Nr of transport blocks/TTI | 16 |
| RLC SDU block size | 40304 |
| Transmission Time Interval | 80 ms |
| Type of Error Protection | Turbo |
| Coding Rate | 1/3 |
| Rate matching Attribute | 256 |
| Size of CRC | 16 |

- Table A.46C: Configuration of other physical channels during MTCH test

| Physical Channel | Power Ratio (Ec/Ior) | NOTE |
| --- | --- | --- |
| P-CPICH | -10 dB |  |
| T-CPICH | -0.457 dB |  |
| P-CCPCH | -12 dB |  |
| S-CCPCH frame type 1 | -24 dB |  |
| SCH | -12 dB | This power shall be divided equally between Primary and Secondary synchronous channels |
| OCNS | Necessary power so that total transmit power spectral density of Node B (Ior) adds to one | OCNS interference consists of 10 codes of equal power, each code using SF16 and QPSK modulation |

### A.4.2.2	1.28 Mcps TDD Option

The parameters for the MTCH demodulation tests are specified in Table A.47 and Table A.48.

- Table A.47: Physical channel parameters for S-CCPCH

| Parameter | Unit | Level |  | Level |  | Level | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| User Data Rate | kpbs | 384 |  | 192 |  | 128 | 64 |
| Channel bit rate | kbps | 614.41 | 563.22 | 307.21 | 281.62 | 246.4 | 140.8 |
| Channel symbol rate | ksps | 153.61 | 140.82 | 153.61 | 140.82 | 123.2 | 70.4 |
| Slot Format | - | No TPC SS |  | No TPC SS |  | No TPC, SS | No TPC, SS |
| TFCI | - | ON |  | ON |  | ON | ON |
| Note1 used for test 3 and test 4 in section 10.2.2.2 Note2 used for test 1 and test 2 in section 10.2.2.2 |  |  |  |  |  |  |  |

- Table A.48: Transport channel parameters for S-CCPCH

| Parameter | MTCH |  |  |  |
| --- | --- | --- | --- | --- |
| User Data Rate | 384kbps | 192kbps | 128 kbps | 64 kbps |
| Number of Transport Channel | 1 | 1 | 1 | 1 |
| Transport Block Size | 2561 | 2561 | 2561 | 1281 |
| Transport Block Set Size | 15366 | 7683 | 5122 | 2562 |
| Nr of transport blocks/TTI | 6 | 3 | 2 | 2 |
| RLC SDU block size | 15248 | 7616 | 5072 | 2512 |
| Transmission Time Interval | 40ms | 40ms | 40 ms | 40 ms |
| Type of Error Protection | Turbo | Turbo | Turbo | Turbo |
| Coding Rate | 1/3 | 1/3 | 1/3 | 1/3 |
| Rate Matching attribute | 256 | 256 | 256 | 256 |
| Size of CRC | 16 | 16 | 16 | 16 |

Parameters for combined MTCH demodulation and cell identification requirements are defined in Table A.49.

- Table A.49: Cell reselection parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Serving cell in the initial condition | - | Cell 1 |
| Neighbour cells | - | Cell 2 and cell 3 |
| Cell_selection_and_reselection_quality_measure | - | P-CCPCH |
| Qrxlevmin | dBm | -103 |
| UE_TXPWR_MAX_ RACH | dBm | 21 |
| Treselection | seconds | 4 |
| Sintrasearch | dB | not sent |
| IE "FACH Measurement occasion info" | - | not sent |

### A.4.2.3	7.68 Mcps TDD Option

The parameters for the MTCH demodulation tests are specified in Table A.49a and Table A.50.

- Table A.49a: Physical channel parameters for S-CCPCH

| Parameter | Unit | Level | Level | Level |
| --- | --- | --- | --- | --- |
| User Data Rate | kpbs | 512 | 256 | 128 |
| Modulation | - | 16QAM | QPSK | QPSK |
| Channel bit rate | kbps | 1547.8 | 388.8 | 388.8 |
| Channel symbol rate | ksps | 386.95 | 194.4 | 194.4 |
| Slot Format #i | - | 23 and 22 | 3 and 0 | 3 and 0 |
| TFCI | - | ON | ON | ON |
| Physical resources | - | 16 codes x SF32 1 timeslot/ frame | 16 codes x SF32 1 timeslot/ frame | 16 codes x SF32 1 timeslot/ frame |

- Table A.50: Transport channel parameters for S-CCPCH

| Parameter | MTCH |  |  |
| --- | --- | --- | --- |
| User Data Rate | 512 kbps | 256 kbps | 128 kbps |
| Number of Transport Channel | 1 | 1 | 1 |
| Transport Block Size | 2561 | 2561 | 2561 |
| Transport Block Set Size | 40976 | 10244 | 5122 |
| Nr of transport blocks/TTI | 16 | 4 | 2 |
| RLC SDU block size | 40688 | 10160 | 5072 |
| Transmission Time Interval | 80ms | 40 ms | 40 ms |
| Type of Error Protection | Turbo | Turbo | Turbo |
| Coding Rate | 1/3 | 1/3 | 1/3 |
| Rate Matching attribute | 256 | 256 | 256 |
| Size of CRC | 16 | 16 | 16 |
| Puncturing limit | 0.52 | 1.0 | 1.0 |

# A.5 HSUPA reference measurement channels for 1.28Mcps TDD option

## A.5.1 Fixed reference channel 1(FRC1) for 16QM

- Table A.51: E-DCH Fixed reference channel 1 (1.28Mcps TDD option)

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum information bit throughput | kbps | 342.4 |
| Information Bit Payload () | Bits | 1712 |
| Number Code Blocks | Blocks | 1 |
| Number of coded bits per TTI | Bits | 1736 |
| Coding Rate |  | 0.623 |
| Modulation |  | 16QAM |
| Number of E-DCH Timeslots | Slots | 1 |
| Number of E-DCH codes per TS | Codes | 1 |
| Spreading factor | SF | 1 |
| Number of E-UCCH per TTI |  | 1 |

- Figure A.17: Coding for E-DCH FRC1 (1.28 Mcps TDD Option)

### A.5.2 Fixed reference channel for MC-HSUPA

- Table A.52: E-DCH FRC for MC-HSUPA

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum information bit throughput | kbps | 4.6 |
| Information Bit Payload () | Bits | 23 |
| Number Code Blocks | Blocks | 1 |
| Number of coded bits per TTI | Bits | 47 |
| Coding Rate |  | 0.0414 |
| Modulation |  | QPSK |
| Number of E-DCH Timeslots | Slots | 1 |
| Number of E-DCH codes per TS | Codes | 1 |
| Spreading factor | SF | 1 |
| Number of E-UCCH per TTI |  | 8 |

- Figure A.18: Coding for E-DCH FRC for 1.28Mcps TDD MC-HSUPA

Annex B (normative):
Propagation conditions

# B.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading and multi-paths exist for this propagation model.

# B.2	Multi-path fading propagation conditions

## B.2.1	3.84 Mcps TDD Option

Table B.1 shows propagation conditions that are used for the performance measurements in multi-path fading environment. All taps have classical Doppler spectrum.

Table B.1: Propagation Conditions for Multi path Fading Environmentsfor operations referenced in 5.2 a), 5.2 b) and 5.2 c)

| Case 1 speed 3km/h |  | Case 2 speed 3 km/h |  | Case 3 speed 120 km/h |  | CASE 4 speed 50 km/h (note) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 976 | -10 | 976 | 0 | 260 | -3 | 976 | -10 |
|  |  | 12000 | 0 | 521 | -6 |  |  |
|  |  |  |  | 781 | -9 |  |  |
| NOTE: Case 4 is only used in TS25.123. |  |  |  |  |  |  |  |

Table B.1A: Propagation Conditions for Multi-Path Fading Environments for HSDPA Performance Requirements for operations referenced in 5.2 a), 5.2 b) and 5.2 c)

| ITU Pedestrian A Speed 3km/h (PA3) |  | ITU Pedestrian B Speed 3Km/h (PB3) |  | ITU vehicular A Speed 30km/h (VA30) |  | ITU vehicular A Speed 120km/h (VA120) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 110 | -9.7 | 200 | -0.9 | 310 | -1.0 | 310 | -1.0 |
| 190 | -19.2 | 800 | -4.9 | 710 | -9.0 | 710 | -9.0 |
| 410 | -22.8 | 1200 | -8.0 | 1090 | -10.0 | 1090 | -10.0 |
|  |  | 2300 | -7.8 | 1730 | -15.0 | 1730 | -15.0 |
|  |  | 3700 | -23.9 | 2510 | -20 | 2510 | -20 |

Table B.1B: Propagation Conditions for Multi path Fading Environments 
for operations referenced in 5.2 d)

| Case 1 speed 2.3km/h |  | Case 2 speed 2.3 km/h |  | Case 3 speed 92 km/h |  | Case 4 speed 38 km/h * |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 976 | -10 | 976 | 0 | 260 | -3 | 976 | -10 |
|  |  | 12000 | 0 | 521 | -6 |  |  |
|  |  |  |  | 781 | -9 |  |  |
| NOTE: Case 4 is only used in TS25.123. |  |  |  |  |  |  |  |

Table B.1C: Propagation Conditions for Multi-Path Fading Environments for HSDPA Performance Requirements for operations referenced in 5.2 d)

| ITU Pedestrian A Speed 2.3km/h (PA3) |  | ITU Pedestrian B Speed 2.3Km/h (PB3) |  | ITU vehicular A Speed 23 km/h (VA30) |  | ITU vehicular A Speed 92 km/h (VA120) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 110 | -9.7 | 200 | -0.9 | 310 | -1.0 | 310 | -1.0 |
| 190 | -19.2 | 800 | -4.9 | 710 | -9.0 | 710 | -9.0 |
| 410 | -22.8 | 1200 | -8.0 | 1090 | -10.0 | 1090 | -10.0 |
|  |  | 2300 | -7.8 | 1730 | -15.0 | 1730 | -15.0 |
|  |  | 3700 | -23.9 | 2510 | -20 | 2510 | -20 |

Table B.1D: Propagation Conditions for Multi-Path Fading Environments for Performance Requirements under an extended delay spread environment

| Extended Delay Spread |  |  |  |
| --- | --- | --- | --- |
| Operations referenced in 5.2 a), 5.2 b) and 5.2 c) Speed 3km/h (EDS) |  | Operations referenced in 5.2 d) Speed 2.3km/h (EDS) |  |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 |
| 310 | -1 | 310 | -1 |
| 710 | -9 | 710 | -9 |
| 1090 | -10 | 1090 | -10 |
| 1730 | -15 | 1730 | -15 |
| 2510 | -20 | 2510 | -20 |
| 12490 | -10 | 12490 | -10 |
| 12800 | -11 | 12800 | -11 |
| 13200 | -19 | 13200 | -19 |
| 13580 | -20 | 13580 | -20 |
| 14220 | -25 | 14220 | -25 |
| 15000 | -30 | 15000 | -30 |
| 27490 | -20 | 27490 | -20 |
| 27800 | -21 | 27800 | -21 |
| 28200 | -29 | 28200 | -29 |
| 28580 | -30 | 28580 | -30 |
| 29220 | -35 | 29220 | -35 |
| 30000 | -40 | 30000 | -40 |

## B.2.2	1.28 Mcps TDD Option

Table B.2 shows propagation conditions that are used for the general performance measurements in multi-path fading environment. Table B.3 shows propagation conditions that are used for HSDPA and multi-carrier HSDPA performance measurements in multi-path fading environments. For multi-carrier HSDPA requirements, the fading of the signals for each carrier shall be independent. All taps in both tables have classical Doppler spectrum.

- Table B.2: Propagation Conditions for Multi-Path Fading Environments operations

| Case 1 |  | Case 2 |  | Case 3 |  |
| --- | --- | --- | --- | --- | --- |
| Speed for operating band a, b, c, f: 3km/h |  | Speed for operating band a, b, c, f: 3km/h |  | Speed for operating band a, b, c, f: 120km/h |  |
| Speed for operating band d: 2.3km/h |  | Speed for operating band d: 2.3km/h |  | Speed for operating band d: 92km/h |  |
| Speed for operating band e: 2.6km/h |  | Speed for operating band e: 2.6km/h |  | Speed for operating band e: 102km/h |  |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 2928 | -10 | 2928 | 0 | 781 | -3 |
|  |  | 12000 | 0 | 1563 | -6 |
|  |  |  |  | 2344 | -9 |

Table B.3: Propagation Conditions for Multi-Path Fading Environments for HSDPA Performance Requirements

| ITU Pedestrian A |  | ITU Pedestrian B |  | ITU vehicular A |  | ITU vehicular A |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed for operating band a, b, c, f: 3km/h |  | Speed for operating band a, b, c, f: 3km/h |  | Speed for operating band a, b, c, f: 30km/h |  | Speed for operating band a, b, c, f: 120km/h |  |
| Speed for operating band d: 2.3km/h |  | Speed for operating band d: 2.3km/h |  | Speed for operating band d: 23km/h |  | Speed for operating band d: 92km/h |  |
| Speed for operating band e: 2.6km/h |  | Speed for operating band e: 2.6km/h |  | Speed for operating band e: 26km/h |  | Speed for operating band e: 102km/h |  |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 110 | -9.7 | 200 | -0.9 | 310 | -1.0 | 310 | -1.0 |
| 190 | -19.2 | 800 | -4.9 | 710 | -9.0 | 710 | -9.0 |
| 410 | -22.8 | 1200 | -8.0 | 1090 | -10.0 | 1090 | -10.0 |
|  |  | 2300 | -7.8 | 1730 | -15.0 | 1730 | -15.0 |
|  |  | 3700 | -23.9 | 2510 | -20 | 2510 | -20 |

Table B.3B shows propagation conditions that are used for MBSFN demodulation performance measurements in multi-path fading environment. All taps have classical Doppler spectrum.

In the case of Rx diversity, the fading of the signals and the AWGN signals provided in each receiver antenna port shall be independent.

Table B.3B: Propagation Conditions for Multi-Path Fading Environments for MBSFN Demodulation Performance Requirements

| MBSFN channel model 1 |  | MBSFN channel model 2 |  |
| --- | --- | --- | --- |
| Speed for Band a, b, c, f: 30 km/h |  | Speed for Band a, b, c, f: 30 km/h |  |
| Speed for Band d: 23 km/h |  | Speed for Band d: 23 km/h |  |
| Speed for Band e: 26 km/h |  | Speed for Band e: 26km/h |  |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0.0 | 0 | 0.0 |
| 310 | -1.0 | 310 | -1.0 |
| 710 | -9.0 | 710 | -9.0 |
| 1090 | -10.0 | 1090 | -10.0 |
| 1730 | -15.0 | 1730 | -15.0 |
| 2510 | -20.0 | 2510 | -20.0 |
| 2734 | -6.6 | 5859 | -6.8 |
| 3044 | -7.6 | 6169 | -7.8 |
| 3444 | -15.6 | 6569 | -15.8 |
| 3824 | -16.6 | 6949 | -16.8 |
| 4464 | -21.6 | 7589 | -21.8 |
| 5469 | -8.5 | 10938 | -13.3 |
| 5779 | -9.5 | 11248 | -14.3 |
| 6179 | -17.5 | 11648 | -22.3 |
| 6559 | -18.5 | 12028 | -23.3 |
| 8428 | -12.6 | 15459 | -15.0 |
| 8738 | -13.6 | 15769 | -16.0 |
| 9138 | -21.6 | 16169 | -24.0 |

## B.2.3	7.68 Mcps TDD Option

Table B.4 and Table B.5 show propagation conditions that are used for the performance measurements in multi-path fading environment. All taps have classical Doppler spectrum.

Table B.4: Propagation Conditions for Multi path Fading Environmentsfor operations referenced in 5.2 a), 5.2 b) and 5.2 c)

| Case 1 speed 3km/h |  | Case 2 speed 3 km/h |  | Case 3 speed 120 km/h |  | CASE 4 speed 50 km/h * |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 976 | -10 | 976 | 0 | 260 | -3 | 976 | -10 |
|  |  | 12000 | 0 | 521 | -6 |  |  |
|  |  |  |  | 781 | -9 |  |  |

*NOTE:	Case 4 is only used in TS25.123.

Table B.5: Propagation Conditions for Multi-Path Fading Environments for HSDPA Performance Requirements for operations referenced in 5.2 a), 5.2 b) and 5.2 c)

| ITU Pedestrian A Speed 3km/h (PA3) |  | ITU Pedestrian B Speed 3Km/h (PB3) |  | ITU vehicular A Speed 30km/h (VA30) |  | ITU vehicular A Speed 120km/h (VA120) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 110 | -9.7 | 200 | -0.9 | 310 | -1.0 | 310 | -1.0 |
| 190 | -19.2 | 800 | -4.9 | 710 | -9.0 | 710 | -9.0 |
| 410 | -22.8 | 1200 | -8.0 | 1090 | -10.0 | 1090 | -10.0 |
|  |  | 2300 | -7.8 | 1730 | -15.0 | 1730 | -15.0 |
|  |  | 3700 | -23.9 | 2510 | -20 | 2510 | -20 |

Table B.6: Propagation Conditions for Multi path Fading Environments 
for operations referenced in 5.2 d)

| Case 1 speed 2.3km/h |  | Case 2 speed 2.3 km/h |  | Case 3 speed 92 km/h |  | Case 4 speed 38 km/h * |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 976 | -10 | 976 | 0 | 260 | -3 | 976 | -10 |
|  |  | 12000 | 0 | 521 | -6 |  |  |
|  |  |  |  | 781 | -9 |  |  |

*NOTE:	Case 4 is only used in TS 25.123.

Table B.7: Propagation Conditions for Multi-Path Fading Environments for HSDPA Performance Requirements for operations referenced in 5.2 d)

| ITU Pedestrian A Speed 2.3km/h (PA3) |  | ITU Pedestrian B Speed 2.3Km/h (PB3) |  | ITU vehicular A Speed 23 km/h (VA30) |  | ITU vehicular A Speed 92 km/h (VA120) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 110 | -9.7 | 200 | -0.9 | 310 | -1.0 | 310 | -1.0 |
| 190 | -19.2 | 800 | -4.9 | 710 | -9.0 | 710 | -9.0 |
| 410 | -22.8 | 1200 | -8.0 | 1090 | -10.0 | 1090 | -10.0 |
|  |  | 2300 | -7.8 | 1730 | -15.0 | 1730 | -15.0 |
|  |  | 3700 | -23.9 | 2510 | -20 | 2510 | -20 |

Table B.8: Propagation Conditions for Multi-Path Fading Environments for Performance Requirements under an extended delay spread environment

| Extended Delay Spread |  |  |  |
| --- | --- | --- | --- |
| Operations referenced in 5.2 a), 5.2 b) and 5.2 c) Speed 3km/h (EDS) |  | Operations referenced in 5.2 d) Speed 2.3km/h (EDS) |  |
| Relative Delay [ns] | Relative Mean Power [dB] | Relative Delay [ns] | Relative Mean Power [dB] |
| 0 | 0 | 0 | 0 |
| 310 | -1 | 310 | -1 |
| 710 | -9 | 710 | -9 |
| 1090 | -10 | 1090 | -10 |
| 1730 | -15 | 1730 | -15 |
| 2510 | -20 | 2510 | -20 |
| 12490 | -10 | 12490 | -10 |
| 12800 | -11 | 12800 | -11 |
| 13200 | -19 | 13200 | -19 |
| 13580 | -20 | 13580 | -20 |
| 14220 | -25 | 14220 | -25 |
| 15000 | -30 | 15000 | -30 |
| 27490 | -20 | 27490 | -20 |
| 27800 | -21 | 27800 | -21 |
| 28200 | -29 | 28200 | -29 |
| 28580 | -30 | 28580 | -30 |
| 29220 | -35 | 29220 | -35 |
| 30000 | -40 | 30000 | -40 |

# B.3	MIMO propagation conditions

## B.3.1	3.84 Mcps TDD Option

<void>

## B.3.2	1.28 Mcps TDD Option

MIMO propagation conditions are defined for a 2x2 antenna configuration. The resulting propagation channel shall be characterized by a complex 2x2 matrix termed

H=h11h12h21h22

### B.3.2.1	MIMO Dual Stream Static Orthogonal Conditions

The channel coefficients of the resulting propagation channnel under MIMO dual stream conditions shall be given by

H=h11h12h21h22=111−1

The generation of the resulting channel coefficients for MIMO dual stream conditions and the association with the transmitter and receiver ports are depicted Figure B.1. Figure B.1 does not restrict test system implementation.

- Figure B.1: Test setup under MIMO Dual Stream Static Orthogonal Conditions

## B.3.3	7.68 Mcps TDD Option

<void>

# B.4	High speed train condition

The high speed train condition for the test of the baseband performance is a non fading propagation channel with one tap. Doppler shift is given by

fst=fdcosθt	(B.1)

where fst is the Doppler shift and fd is the maximum Doppler frequency. The cosine of angle θtis given by

cosθt=Ds/2−vtDmin2+Ds/2−vt2, 0≤t≤Ds/v	(B.2)

cosθt=−1.5Ds+vtDmin2+−1.5Ds+vt2, Ds/v<t≤2Ds/v                        (B.3)

cosθt=cosθt  mod(2Ds/v), t>2Ds/v	(B.4)

where Ds/2 is the initial distance of the train from BS, and Dmin is BS-Railway track distance, both in meters; v is the velocity of the train in m/s, t is time in seconds.

Doppler shift and cosine angle is given by equation B.1 and B.2-B.4 respectively, where the required input parameters listed in table B.9 and the resulting Doppler shift shown in Figure B.2 are applied for all frequency bands.

- Table B.9

| Parameter | Value |
| --- | --- |
| Ds | 300 m |
| Dmin | 2 m |
| v | 300 km/h |
| fd | 560 Hz |

NOTE1:	Parameters for HST conditions in table B.9 including fd and Doppler shift trajectories presented on figure B.2 were derived for Band a).

- Figure B.2: Doppler shift trajectory

# B.5	Moving propagation conditions

The dynamic propagation conditions for the test of the baseband performance are non fading channel models with two taps. The moving propagation condition has two taps, one static, Path0, and one moving, Path1. The time difference between the two paths is according Equation (B.5). The taps have equal strengths and equal phases.

- Figure B.3: The moving propagation conditions

Δτ=B+A21+sinΔω⋅t	(B.5)

The parameters in the equation are shown in.

| A | 5 ms |
| --- | --- |
| B | 1 ms |
| Dw | 40*10-3 s-1 |

# B.6	Birth-Death propagation conditions

The dynamic propagation conditions for the test of the baseband performance is a non fading propagation channel with two taps. The moving propagation condition has two taps, Path1 and Path2 while alternate between 'birth' and 'death'. The positions the paths appear are randomly selected with an equal probability rate and are shown in figure B.4.

- Figure B.4: Birth death propagation sequence

NOTE1:	Two paths, Path1 and Path2 are randomly selected from the group [-3, -2, -1, 0, 1, 2, 3] chip(781.25ns). The paths have equal strengths and equal phases.

NOTE 2:	After 191 ms, Path1 vanishes and reappears immediately at a new location randomly selected from the group [-3, -2, -1, 0, 1, 2, 3]chip but excludes the point Path2.

NOTE 3:	After additional 191 ms, Path2 vanishes and reappears immediately at a new location randomly selected from the group [-3, -2, -1, 0, 1, 2, 3] chip but excludes the point Path1.

NOTE 4:	The sequence in 2) and 3) is repeated.

Annex C (normative):
Environmental conditions

# C.1	General

This normative annex specifies the environmental requirements of the UE. Within these limits the requirements of this specifications shall be fulfilled.

# C.2	Environmental requirements for the UE

The requirements in this clause apply to all types of UE(s)

## C.2.1	Temperature

The UE shall fulfil all the requirements in the full temperature range of:

- Table C.1

| +15C - +35 C | for normal conditions (with relative humidity of 25 % to 75 %); |
| --- | --- |
| -10C - +55C | for extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in S25.102 for extreme operation.

## C.2.2	Voltage

The UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

- Table C.2

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0,9 * nominal | 1,1 * nominal | nominal |
| Regulated lead acid battery | 0,9 * nominal | 1,3 * nominal | 1,1 * nominal |
| Non regulated batteries: Leclanché/lithium Mercury/nickel cadmium | 0,85 * nominal 0,90 * nominal | Nominal Nominal | Nominal Nominal |

Outside this voltage range the UE if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in S4.01A for extreme operation. In particular, the UE shall inhibit all RF transmissions when the power supply voltage is below the manufacturer declared shutdown voltage.

## C.2.3	Vibration

The UE shall fulfil all the requirements when vibrated at the following frequency/amplitudes:

- Table C.3

| Frequency | ASD (Acceleration Spectral Density) random vibration |
| --- | --- |
| 5 Hz to 20 Hz | 0,96 m2/s3 |
| 20 Hz to 500 Hz | 0,96 m2/s3 at 20 Hz, thereafter -3 dB/Octave |

Outside the specified frequency range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in S4.01A for extreme operation.

Annex D (informative):
Terminal capabilities (TDD)

Void

[Note: All UE capabilities can be found in 3GPP TS 25.306]

Annex E (informative):
Change history

| TSG | 29 | CR | R | Title | Cat | Curr | New | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RP-29 |  |  |  | Rel-7 version created based on v6.2.0 |  |  | 7.0.0 |  |
| RP-29 | RP-050501 | 0153 |  | Introduction of UMTS 2.6GHz operating band for TDD | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-29 | RP-050501 | 0154 |  | UMTS 2.6 GHz TDD Propagation Conditions | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-29 | RP-050501 | 0155 |  | UMTS 2.6 GHz TDD UE Receiver Specifications | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-29 | RP-050501 | 0156 | 2 | Channel Raster and UARFCN for 3.84 Mcps TDD in UMTS 2.6 GHz | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-29 | RP-050501 | 0157 |  | Introduction of UMTS 2.6 GHz new operating band for 1.28Mcps TDD | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-29 | RP-050501 | 0158 |  | Introduction of Propagation Conditions for UMTS 2.6 GHz for 1.28Mcps TDD | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-29 | RP-050501 | 0159 |  | Introduction of UMTS 2.6 GHz requirements for blocking and spurious emission of UE receiver for 1.28Mcps TDD | B | 6.1.0 | 7.0.0 | RinImp-UMTS2600TDD |
| RP-30 | RP-050740 | 0160 | 1 | Additional UE Tx Spurious Emission for co-existence with 2.6 GHz FDD | B | 7.0.0 | 7.1.0 | RinImp-UMTS2600TDD |
| RP-31 | RP-060104 | 0162 | 1 | Modifications to HSDPA test parameters for 1.28Mcps TDD | A | 7.1.0 | 7.2.0 | TEI6 |
| RP-31 | RP-060113 | 0163 | 1 | MBMS Requirements for MCCH & MTCH channels | B | 7.1.0 | 7.2.0 | MBMS-RAN-RF-TDD |
| RP-32 | RP-060313 | 0165 | 2 | 1.28Mcps TDD MBMS UE performance requirements | B | 7.2.0 | 7.3.0 | MBMS-RAN-RF-TDD |
| RP-32 | RP-060309 | 0166 | 1 | 7.68 Mcps Frequency Band & Channel Arrangement | B | 7.2.0 | 7.3.0 | VHCRTDD-RF |
| RP-32 | RP-060309 | 0167 | 2 | 7.68 Mcps UE Transmitter Characteristics | B | 7.2.0 | 7.3.0 | VHCRTDD-RF |
| RP-32 | RP-060309 | 0168 |  | 7.68 Mcps UE Receiver Characteristics | B | 7.2.0 | 7.3.0 | VHCRTDD-RF |
| RP-32 | RP-060309 | 0169 |  | 7.68 Mcps - Channel Performance | B | 7.2.0 | 7.3.0 | VHCRTDD-RF |
| RP-32 | RP-060309 | 0170 | 1 | 7.68 Mcps Measurement Channels & Propagation Conditions | B | 7.2.0 | 7.3.0 | VHCRTDD-RF |
| RP-33 | RP-060522 | 0180 |  | HS-SCCH performance requirement for 3.84 Mcps TDD option and 7.68 Mcps TDD option |  | 7.3.0 | 7.4.0 | TEI7 |
| RP-33 | RP-060516 | 0185 | 1 | Out of band blocking for 3.84 Mcps and 7.68 Mcps TDD UE operating in 2010-2025 MHz of band (a) in Japan. |  | 7.3.0 | 7.4.0 | TEI7 |
| RP-33 | RP-060517 | 0190 |  | Clarification of Tx spurious emission level from 3.84 Mcps and 7.68 MCps TDD UE into PHS band |  | 7.3.0 | 7.4.0 | TEI7 |
| RP-33 | RP-060529 | 0192 |  | Editorial corrections to 3.84 Mcps TDD UE performances on MBMS. |  | 7.3.0 | 7.4.0 | TEI7 |
| RP-33 | RP-060528 | 0193 |  | Performance requirements for 3.84 Mcps E-DCH associated downlink signalling channels: E-AGCH and E-HICH |  | 7.3.0 | 7.4.0 | TEI7 |
| RP-33 | RP-060526 | 0194 |  | 7.68 Mcps Operations in 2.6 GHz band |  | 7.3.0 | 7.4.0 | TEI7 |
| RP-33 | RP-060530 | 0195 |  | Clarification of 7.68 Mcps TDD UE ACLR at +/- 10 MHz offset. |  | 7.3.0 | 7.4.0 | MBMS-RAN-RF-TDD |
| RP-33 | RP-060530 | 0196 |  | Performance requirements for 3.84 Mcps E-DCH associated downlink signalling channels: E-AGCH and E-HICH |  | 7.3.0 | 7.4.0 | EDCHTDD-RF |
| RP-34 | RP-060810 | 0200 |  | Combined MBMS demodulation and Cell identification requirement for 1.28 Mcps TDD | A | 7.4.0 | 7.5.0 | TEI6 |
| RP-34 | RP-060818 | 0198 |  | Performance requirements for 7.68 Mcps E-DCH associated downlink signalling channels: E-AGCH and E-HICH | B | 7.4.0 | 7.5.0 | TEI7 |
| RP-34 | RP-060816 | 0197 |  | PLCCH Performance Requirement | B | 7.4.0 | 7.5.0 | RANimp-RABSE-CodOptLCRTDD |
| RP-35 | RP-070081 | 0209 |  | Modificaiton to SEM for 1.28Mcps TDD | A | 7.5.0 | 7.6.0 | TEI4 |
| RP-35 | RP-070082 | 0201 |  | Performance requirements for 7.68 Mcps E-DCH associated downlink signalling channels: E-AGCH and E-HICH | B | 7.5.0 | 7.6.0 | TEI7 |
| RP-35 | RP-070082 | 0203 |  | Corrections & clarifications on 7.68 Mcps TDD MTCH demodulation test case. | F | 7.5.0 | 7.6.0 | TEI7 |
| RP-35 | RP-070085 | 0202 |  | Performance requirements for MTCH using 16QAM in an extended delay spread environment | B | 7.5.0 | 7.6.0 | MBMSE-RANPhysTDD |
| RP-35 | RP-070085 | 0211 |  | Performance requirement for MCCH in an extended delay spread environment | B | 7.5.0 | 7.6.0 | MBMSE-RANPhysTDD |
| RP-36 | RP-070376 | 0225 |  | Performance requirements for MCCH in an extended delay spread environment. | F | 7.6.0 | 7.7.0 | MBMSE-RANPhysTDD |
| RP-36 | RP-070376 | 0212 | 1 | MCCH & MTCH Channels Performances in TDD MBSFN | B | 7.6.0 | 7.7.0 | MBMSE-RANPhysTDD |
| RP-36 | RP-070376 | 0224 | 1 | Performance requirements for MTCH using 16QAM in an extended delay spread environment. | F | 7.6.0 | 7.7.0 | MBMSE-RANPhysTDD |
| RP-36 | RP-070376 | 0213 | 1 | Performance requirement for MTCH in a MBSFN TDD UE sharing the same platform with a FDD UE | B | 7.6.0 | 7.7.0 | MBMSE-RANPhysTDD |
| RP-36 | RP-070377 | 0223 | 1 | Performance requirements for 1.28 Mcps E-DCH associated downlink signalling channels: E-AGCH and E-HICH | B | 7.6.0 | 7.7.0 | LCRTDD-EDCH-RF |
| RP-36 | RP-070373 | 0222 | 1 | Updating of HSDPA demodulation performance requirements for 1.28Mcps TDD | A | 7.6.0 | 7.7.0 | TEI7 |
| RP-36 | RP-070373 | 0216 |  | Updating of HSDPA demodulation performance requirements for 1.28Mcps TDD-FRC | A | 7.6.0 | 7.7.0 | TEI7 |
| RP-36 | RP-070373 | 0219 |  | Updating of HSDPA demodulation performance requirements for 1.28Mcps TDD-VRC | A | 7.6.0 | 7.7.0 | TEI7 |
| RP-37 | RP-070651 | 0243 |  | Change to HSDPA for 1.28 Mcps TDD | F | 7.7.0 | 7.8.0 | TEI7 |
| RP-37 | RP-070651 | 0240 |  | Correction of UE maximum output power classes for 1.28 Mcps TDD option | F | 7.7.0 | 7.8.0 | TEI7 |
| RP-37 | RP-070651 | 0230 |  | Inclusion of 7.68 Mcps in the scope of document | D | 7.7.0 | 7.8.0 | TEI7 |
| RP-37 | RP-070651 | 0232 |  | Clarification of MBMS test for LCR TDD | A | 7.7.0 | 7.8.0 | TEI7 |
| RP-37 | RP-070652 | 0229 |  | Requirements for maximum Input level for HS-PDSCH reception | A | 7.7.0 | 7.8.0 | TEI7 |
| RP-37 | RP-070654 | 0234 |  | Performance Requirements for TDD MBSFN Channels | B | 7.7.0 | 7.8.0 | MBMSE-RANPhysTDD |
| RP-38 | RP-070935 | 0244 |  | LCR TDD MBSFN UE demodulation performance requirements | B | 7.8.0 | 7.9.0 | MBMSE-RANPhysLCRTDD |
| RP-38 | RP-070937 | 0245 |  | Relative delay corrections in Extended Delay Spread propagation condition | F | 7.8.0 | 7.9.0 | TEI7 |
| RP-39 | RP-080118 | 0250 |  | Adding EVM requirement for UL 16QAM | F | 7.9.0 | 7.10.0 | LCRTDD-EDCH-RF |
| RP-39 | RP-080118 | 0251 | 1 | Adding requirements for MBSFN capable UE (dedicated carrier case) | F | 7.9.0 | 7.10.0 | MBMS-RANPhysLCRTDD |
| RP-39 | RP-080119 | 0246 |  | Omissions of minimum requirements for blocking characteristics | F | 7.9.0 | 7.10.0 | TEI6 |
| RP-39 | RP-080119 | 0249 |  | Deleting redundant notes for receiver spurious emissions | F | 7.9.0 | 7.10.0 | TEI6 |
| RP-40 | RP-080324 | 0263 |  | Clarification of MCCH Physical Channel for MBSFN | F | 7.10.0 | 7.11.0 | MBMSE-RANPhysTDD |
| RP-40 | RP-080324 | 0262 |  | Correction to MTCH parameters for demodulation test in TDD MBSFN | F | 7.10.0 | 7.11.0 | MBMSE-RANPhysTDD |
| RP-40 | RP-080324 | 0257 |  | Corrections for LCR TDD MBMS | F | 7.10.0 | 7.11.0 | MBMSE-RANPhysTDD |
| RP-40 | RP-080324 | 0256 |  | MBSFN Reference Channel | F | 7.10.0 | 7.11.0 | MBMSE-RANPhysTDD |
| RP-40 | RP-080384 | 0260 |  | UMTS2.3 GHz TDD: UE receiver characteristics & propagation conditions for 1.28Mcps TDD | B | 7.11.0 | 8.0.0 | RinImp8-UMTS2300TDD |
| RP-40 | RP-080384 | 0259 |  | UMTS2.3 GHz TDD: UE transmitter Characteristics for 2.3GHz LCR TDD | B | 7.11.0 | 8.0.0 | RinImp8-UMTS2300TDD |
| RP-40 | RP-080384 | 0258 |  | New band introduction for 25.102 | B | 7.11.0 | 8.0.0 | RinImp8-UMTS2300TDD |
| RP-41 | RP-080 | 270 |  | Demodulation requirements of fixed reference channels for 1.28Mcps TDD option 64QAM DL | F | 8.0.0 | 8.1.0 | RANimp-64Qam1.28TDD |
| RP-41 | RP-080628 | 0269 |  | RF requirements in later releases | A | 8.0.0 | 8.1.0 | RinImp8-UMTS2300TDD |
| RP-41 | RP-080628 | 0264 | 1 | UE RF capabilitiy information update | F | 8.0.0 | 8.1.0 | RinImp8-UMTS2300TDD |
| RP-42 | RP-080899 | 0279 | 1 | UE reference measurement channel and performance requirement for 384kbps service | A | 8.1.0 | 8.2.0 | TEI4 |
| RP-42 | RP-080939 | 0280 |  | Introduction of band 1880-1920MHz for 25.102 | B | 8.1.0 | 8.2.0 | Rimp9-UMTS1880TDD |
| RP-42 | RP-080946 | 0281 |  | Adding the demodulation requirements for 1.28Mcps TDD Option 64QAM DL. | B | 8.1.0 | 8.2.0 | RANimp-64Qam1.28TDD |
| RP-42 | RP-080946 | 0282 |  | Adding the requirement of maximum input level for 1.28Mcps TDD option 64QAM DL | B | 8.1.0 | 8.2.0 | RANimp-64Qam1.28TDD |
| RP-42 | RP-080941 | 0273 |  | Additional minimum requirements for LCR TDD UE Adjacent Channel Selectivity | F | 8.1.0 | 8.2.0 | TEI8 |
| RP-43 | RP-090169 | 0288 |  | Correction on MBSFN MCCH Slot Format | A | 8.2.0 | 8.3.0 | MBMSE-RANPhysTDD |
| RP-43 | RP-090169 | 0284 |  | Correction on MBSFN MCCH Slot Format | F | 8.2.0 | 8.3.0 | TEI7 |
| RP-43 | RP-090194 | 0289 |  | Introduction of 3.84Mcps TDD MBSFN IMB | B | 8.2.0 | 8.3.0 | MBSFN-DOB |
| RP-43 | RP-090197 | 0285 |  | UMTS1880MHz: Transmitter spurious emission | F | 8.2.0 | 8.3.0 | Rlnlmp9-UMTS1880TDD |
| RP-43 | RP-090197 | 0286 |  | UMTS1880MHz: Receiver characteristic and propagation condition for UE | F | 8.2.0 | 8.3.0 | Rlnlmp9-UMTS1880TDD |
| RP-44 | RP-090539 | 0300 |  | Correction concerning scope of applicability for Extended Delay Spread propagation conditions | A | 8.3.0 | 8.4.0 | MBMSE-RANPhysTDD |
| RP-44 | RP-090554 | 0290 |  | HS-DSCH performance requirements for 1.28Mcps TDD MIMO | F | 8.3.0 | 8.4.0 | RANimp-LCRMIMO |
| RP-44 | RP-090554 | 0291 |  | HS-SCCH performance requirements for 1.28Mcps TDD MIMO | F | 8.3.0 | 8.4.0 | RANimp-LCRMIMO |
| RP-44 | RP-090554 | 0292 |  | CQI reporting performance requirements for 1.28Mcps TDD MIMO | F | 8.3.0 | 8.4.0 | RANimp-LCRMIMO |
| RP-44 | RP-090556 | 0294 |  | Correction on 64QAM Reference measurement channel for 1.28Mcps TDD. (Technically Endorsed CR in R4-50bis - R4-091184) | F | 8.3.0 | 8.4.0 | TEI8 |
| RP-44 | RP-090557 | 0296 |  | Definition of DL reference measurement channel for IMB | F | 8.3.0 | 8.4.0 | MBSFN-DOB |
| RP-44 | RP-090557 | 0295 | 1 | Accommodation of the IMB reference bearer in the receiver characteristics of clause 7 | F | 8.3.0 | 8.4.0 | MBSFN-DOB |
| RP-44 | RP-090557 | 0301 |  | Addition of Performance Requirements for IMB MCCH | F | 8.3.0 | 8.4.0 | MBSFN-DOB |
| RP-44 | RP-090557 | 0302 |  | Addition of Performance Requirements for IMB MTCH | F | 8.3.0 | 8.4.0 | MBSFN-DOB |
| RP-44 | RP-090560 | 0303 |  | Addition of new requirement of new E-AGCH type 2 | B | 8.3.0 | 8.4.0 | RANimp-LCRCPC |
| RP-45 | RP-090818 | 0306 |  | Correction to TS25.102 defining the abbreviations MCCH and MTCH | F | 8.4.0 | 8.5.0 | MBSFN-DOB |
|  |  | 0307 |  | Addition of performance requirements for IMB MTCH | F | 8.4.0 | 8.5.0 | MBSFN-DOB |
|  |  | 0309 | 1 | Addition of Performance Requirements for IMB MCCH | F | 8.4.0 | 8.5.0 | MBSFN-DOB |
| RP-45 | RP-090821 | 0305 |  | Correction of reference channel for category 29-30 UE | F | 8.4.0 | 8.5.0 | RANimp-MIMOLCR |
|  |  | 0308 | 1 | Clarification of test configuration for UE with multiple antennas | F | 8.4.0 | 8.5.0 | RANimp-MIMOLCR |
| RP-45 | RP-090825 | 0304 |  | Revision of 64QAM Reference channel | F | 8.4.0 | 8.5.0 | TEI8 |
| RP-46 | RP-091285 | 0313 |  | UE performance requirements in high speed train condition for LCR TDD (Technically endorsed at RAN 4 52bis in R4-093542) | B | 8.5.0 | 9.0.0 | RinImp9-LCRTDD350 |
| RP-47 | RP-100256 | 0327 | 1 | Maximum output power with multi-code for TDD | A | 9.0.0 | 9.1.0 | TEI7 |
| RP-47 | RP-100256 | 0324 |  | Demodulation of DCH in moving conditions for TDD | A | 9.0.0 | 9.1.0 | TEI7 |
| RP-47 | RP-100256 | 0321 |  | Demodulation of DCH in birth-death conditions for TDD | A | 9.0.0 | 9.1.0 | TEI7 |
| RP-47 | RP-100248 | 0328 |  | Modification to IMB receiver characteristic requirements | A | 9.0.0 | 9.1.0 | MBSFN-DOB |
| RP-47 | RP-100273 | 0318 |  | Additional performance requirements in high speed train conditions for LCR TDD | F | 9.0.0 | 9.1.0 | RinImp9- LCRTDD350 |
| RP-49 | RP-100913 | 0331 |  | Correction of 1.28Mcps TDD spectrum emission mask requirement | A | 9.1.0 | 9.2.0 | TEI7 |
| RP-50 | RP-101326 | 0344 |  | Introduction of new constant BLER test cases | A | 9.2.0 | 9.3.0 | TEI7 |
| RP-50 | RP-101339 | 0336 | 1 | Modification of transport block size to DL reference measurement channel for IMB MBSFN only Ues | A | 9.2.0 | 9.3.0 | MBSFN-DOB |
| RP-50 | RP-101339 | 0338 | 1 | Modification of transport block size to performance requirements for demodulation of IMB MBSFN MTCH | A | 9.2.0 | 9.3.0 | MBSFN-DOB |
| RP-50 | RP-101339 | 0340 | 1 | Correcting the data rate naming of performance requirements for demodulation of IMB MBSFN MCCH | A | 9.2.0 | 9.3.0 | MBSFN-DOB |
| RP-50 | RP-101351 | 0341 | 2 | Introduction of UE requirements for 1.28Mcps TDD MC-HSUPA | B | 9.3.0 | 10.0.0 | TDD_MC_HSUPA |
| RP-51 | RP-110341 | 00350 | - | Correction of Maximum Input Level Test for HS-PDSCH Transmission for 1.28Mcps TDD | A | 10.0.0 | 10.1.0 | TEI8 |
| RP-51 | RP-110335 | 00354 | - | Introduction of new DL power control TC, wind up effects for 1.28Mcps TDD | A | 10.0.0 | 10.1.0 | TEI7 |
| RP-52 | RP-110798 | 0355 |  | Introduction of performance requirements for 1.28Mcps TDD MU-MIMO | B | 10.1.0 | 10.2.0 | MUMIMO_LCR_TDD-Perf |
| RP-53 | RP-111245 | 0359 | 1 | Introduction of new DL power control TC, initial convergence for 1.28Mcps TDD | A | 10.2.0 | 10.3.0 | TEI7 |
| RP-54 | RP-111694 | 0361 |  | UE demodulation performance requirements under multiple-cell scenario for 1.28Mcps TDD | F | 10.3.0 | 11.1.0 | LCR_TDD_UE_demod_mc-Perf |
| RP-56 | RP-120765 | 0365 |  | Clarification of the scope of Band a for 1.28 Mcps TDD option | A | 11.1.0 | 11.2.0 | TEI8 |
| RP-56 | RP-120793 | 0366 |  | Introduction of Band 44 | B | 11.1.0 | 11.2.0 | LTE_APAC700-Core |
| RP-56 | RP-120765 | 0370 |  | Additional spurious emissions requirements for PHS | A | 11.1.0 | 11.2.0 | TEI8 |
| RP-57 | RP-121296 | 0374 |  | Correction of frequency band number in Table 5.2 in 25.102 (R11) | A | 11.2.0 | 11.3.0 | TEI8 |
| RP-59 | RP-130287 | 377 |  | Update of UE co-existence requirement towards UTRA TDD bands in China | F | 11.3.0 | 11.4.0 | TEI11 |
| RP-59 | RP-130287 | 376 |  | Correction of UE co-existence requirement towards UTRA TDD bands in China | F | 11.3.0 | 11.4.0 | TEI11 |
| RP-60 | RP-130764 | 378 |  | Co-existence around 3500 MHz | F | 11.4.0 | 11.5.0 | RInImp8-UMTSLTE3500 |
| RP-62 | RP-131939 | 379 |  | Receiver spurious emissions corrections | F | 11.5.0 | 11.6.0 | TEI11 |
| SP-65 | - | - | - | Update to Rel-12 version (MCC) | - | 11.6.0 | 12.0.0 |  |
| SP-70 | - | - | - | Update to Rel-13 version (MCC) | - | 12.0.0 | 13.0.0 |  |
| RP-75 | - | - | - | Update to Rel-14 version (MCC) | - | 13.0.0 | 14.0.0 |  |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-06 | SA#80 | - | - | - | - | Update to Rel-15 version (MCC) | 15.0.0 |
| 2020-06 | SA#88 | - | - | - | - | Update to Rel-16 version (MCC) | 16.0.0 |