3GPP TS 25.211 V16.0.0 (2020-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Physical channels and mapping of transport channels

onto physical channels (FDD)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, radio, layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Symbols, abbreviations and definitions	7

3.1	Symbols	7

3.2	Abbreviations	7

3.3	Definitions	8

4	Services offered to higher layers	9

4.1	Transport channels	9

4.1.1	Dedicated transport channels	9

4.1.1.1	DCH - Dedicated Channel	9

4.1.1.2	E-DCH – Enhanced Dedicated Channel	9

4.1.2	Common transport channels	9

4.1.2.1	BCH - Broadcast Channel	9

4.1.2.2	FACH - Forward Access Channel	9

4.1.2.3	PCH - Paging Channel	9

4.1.2.4	RACH - Random Access Channel	9

4.1.2.5	Void	10

4.1.2.6	Void	10

4.1.2.7	HS-DSCH – High Speed Downlink Shared Channel	10

4.1.2.7A	E-DCH - Enhanced Dedicated Channel	10

4.2	Indicators	10

5	Physical channels and physical signals	10

5.1	Physical signals	10

5.2	Uplink physical channels	11

5.2.1	Dedicated uplink physical channels	11

5.2.1.1	DPCCH, DPCCH2, S-DPCCH and DPDCH	11

5.2.1.2	HS-DPCCH	15

5.2.1.3	E-DPCCH and E-DPDCH	16

5.2.1.3A	S-E-DPCCH and S-E-DPDCH	17

5.2.2	Common uplink physical channels	17

5.2.2.1	Physical Random Access Channel (PRACH)	17

5.2.2.1.1	Overall structure of random-access transmission	17

5.2.2.1.2	RACH preamble part	18

5.2.2.1.3	RACH message part	18

5.2.2.2	Void	20

5.3	Downlink physical channels	20

5.3.1	Downlink transmit diversity	20

5.3.1.1	Open loop transmit diversity	22

5.3.1.1.1	Space time block coding based transmit antenna diversity (STTD)	22

5.3.1.1.2	Time Switched Transmit Diversity for SCH (TSTD)	24

5.3.1.2	Closed loop transmit diversity	24

5.3.2	Dedicated downlink physical channels	24

5.3.2.1	STTD for DPCH, F-DPCH and F-TPICH	28

5.3.2.2	Dedicated channel pilots with closed loop mode transmit diversity	29

5.3.2.3	Void	30

5.3.2.4	E-DCH Relative Grant Channel	30

5.3.2.5	E-DCH Hybrid ARQ Indicator Channel	32

5.3.2.6	Fractional Dedicated Physical Channel (F-DPCH)	32

5.3.2.7	Fractional Transmitted Precoding Indicator Channel (F-TPICH)	33

5.3.3	Common downlink physical channels	34

5.3.3.1	Common Pilot Channel (CPICH)	34

5.3.3.1.1	Primary Common Pilot Channel (P-CPICH)	35

5.3.3.1.2	Secondary Common Pilot Channel (S-CPICH)	35

5.3.3.1.3	Demodulation Common Pilot Channel (D-CPICH)	36

5.3.3.2	Downlink phase reference	36

5.3.3.3	Primary Common Control Physical Channel (P-CCPCH)	38

5.3.3.3.1	Primary CCPCH structure with STTD encoding	38

5.3.3.4	Secondary Common Control Physical Channel (S-CCPCH)	39

5.3.3.4.1	Secondary CCPCH structure with STTD encoding	41

5.3.3.5	Synchronisation Channel (SCH)	41

5.3.3.5.1	SCH transmitted by TSTD	42

5.3.3.6	Void	43

5.3.3.7	Acquisition Indicator Channel (AICH)	43

5.3.3.8	Void	46

5.3.3.9	Void	46

5.3.3.10	Paging Indicator Channel (PICH)	46

5.3.3.11	Void	47

5.3.3.12	Shared Control Channel (HS-SCCH)	47

5.3.3.13	High Speed Physical Downlink Shared Channel (HS-PDSCH)	47

5.3.3.14	E–DCH Absolute Grant Channel (E-AGCH)	48

5.3.3.14B	E-DCH Rank and Offset Channel (E-ROCH)	48

5.3.3.15	MBMS Indicator Channel (MICH)	48

5.3.3.16	Common E-DCH Relative Grant Channel	49

6	Mapping and association of physical channels	50

6.1	Mapping of transport channels onto physical channels	50

6.2	Association of physical channels and physical signals	51

7	Timing relationship between physical channels	51

7.1	General	51

7.2	PICH/S-CCPCH timing relation	53

7.2A	PICH/HS-SCCH timing relation	53

7.3	PRACH/AICH timing relation	54

7.3A	UL/DL timing relation for Enhanced Uplink in CELL_FACH state and IDLE mode	55

7.4	Void	56

7.5	Void	56

7.6	DPCCH/DPDCH timing relations	56

7.6.1	Uplink	56

7.6.2	Downlink	56

7.6.3	Uplink/downlink timing at UE	56

7.7	Uplink DPCCH/HS-DPCCH/HS-PDSCH timing at the UE	56

7.7.1	Timing when Multiflow is not configured	56

7.7.2	Timing when Multiflow is configured	57

7.8	HS-SCCH/HS-PDSCH timing	58

7.9	MICH/S-CCPCH timing relation	59

7.10	E-HICH/P-CCPCH/DPCH timing relation	59

7.11	E-RGCH/P-CCPCH/DPCH timing relation	60

7.12	E-AGCH/P-CCPCH timing relation	61

7.12A	E-ROCH/P-CCPCH timing relation	61

7.13	E-DPDCH/E-DPCCH/DPCCH timing relation	61

7.14	S-DPCCH/DPCCH timing relation	61

7.15	DPCH/F-DPCH/F-TPICH timing relations in softer handover	61

7.16	S-E-DPDCH/S-E-DPCCH/DPCCH timing relation	61

7.17	DPCCH2/DPCCH timing relation	62

Annex A (informative):	Change history	63

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes the characteristics of the Layer 1 transport channels and physicals channels in the FDD mode of UTRA. The main objectives of the document are to be a part of the full description of the UTRA Layer 1, and to serve as a basis for the drafting of the actual technical specification (TS).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 25.201: "Physical layer - general description".

[2]	3GPP TS 25.211: "Physical channels and mapping of transport channels onto physical channels (FDD)".

[3]	3GPP TS 25.212: "Multiplexing and channel coding (FDD)".

[4]	3GPP TS 25.213: "Spreading and modulation (FDD)".

[5]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[6]	3GPP TS 25.221: "Transport channels and physical channels (TDD)".

[7]	3GPP TS 25.222: "Multiplexing and channel coding (TDD)".

[8]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[9]	3GPP TS 25.224: "Physical layer procedures (TDD)".

[10]	3GPP TS 25.215: "Physical layer - Measurements (FDD)".

[11]	3GPP TS 25.301: "Radio Interface Protocol Architecture".

[12]	3GPP TS 25.302: "Services Provided by the Physical Layer".

[13]	3GPP TS 25.401: "UTRAN Overall Description".

[14]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[15]	3G TS 25.427: "UTRAN Overall Description :UTRA Iub/Iur Interface User Plane Protocol for DCH data streams".

[16]	3GPP TS 25.435: "UTRAN Iub Interface User Plane Protocols for Common Transport Channel Data Streams".

[17]	3GPP TS 25.331: "Radio Resource Control (RRC)".

[18]	3GPP TS 25.321: "MAC protocol specification".

# 3	Symbols, abbreviations and definitions

## 3.1	Symbols

Ndata1	The number of data bits per downlink slot in Data1 field.

Ndata2	The number of data bits per downlink slot in Data2 field. If the slot format does not contain a Data2 field, Ndata2 = 0.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

16QAM	16 Quadrature Amplitude Modulation

4PAM	4 Pulse-Amplitude Modulation

64QAM	64 Quadrature Amplitude Modulation

8PAM	8 Pulse-Amplitude Modulation

AI	Acquisition Indicator

AICH	Acquisition Indicator Channel

BCH	Broadcast Channel

BPSK	Binary Phase Shift Keying

CCPCH	Common Control Physical Channel

CCTrCH	Coded Composite Transport Channel

CLTD	Closed Loop Transmit Diversity

CPICH	Common Pilot Channel

CQI	Channel Quality Indicator

DCH	Dedicated Channel

DPCCH	Dedicated Physical Control Channel

DPCCH2	Dedicated Physical Control Channel 2

DPCH	Dedicated Physical Channel

DPDCH	Dedicated Physical Data Channel

DTX	Discontinuous Transmission

E-AGCH	E-DCH Absolute Grant Channel

E-DCH	Enhanced Dedicated Channel

E-DPCCH	E-DCH Dedicated Physical Control Channel

E-DPDCH	E-DCH Dedicated Physical Data Channel

E-HICH	E-DCH Hybrid ARQ Indicator Channel

E-RGCH	E-DCH Relative Grant Channel

E-ROCH	E-DCH Rank and Offset Channel

FACH	Forward Access Channel

FBI	Feedback Information

F-DPCH	Fractional Dedicated Physical Channel

F-TPICH	Fractional Transmitted Precoding Indicator Channel

FSW	Frame Synchronization Word

HS-DPCCH	Dedicated Physical Control Channel (uplink) for HS-DSCH

HS-DSCH	High Speed Downlink Shared Channel

HS-PDSCH	High Speed Physical Downlink Shared Channel

HS-SCCH	Shared Control Channel for HS-DSCH

ICH	Indicator Channel

MBSFN	MBMS over a Single Frequency Network

MICH	MBMS Indicator Channel

MIMO	Multiple Input Multiple Output

MUI	Mobile User Identifier

NI	MBMS Notification Indicator

PCH	Paging Channel

P-CCPCH	Primary Common Control Physical Channel

PICH	Page Indicator Channel

PRACH	Physical Random Access Channel

PSC	Primary Synchronisation Code

QPSK	Quadrature Phase Shift Keying

RACH	Random Access Channel

RNC	Radio Network Controller

S-CCPCH	Secondary Common Control Physical Channel

SCH	Synchronisation Channel

S-E-DPCCH	Secondary Dedicated Physical Control Channel for E-DCH

S-E-DPDCH	Secondary Dedicated Physical Data Channel for E-DCH

S-DPCCH	Secondary Dedicated Physical Control Channel

SF	Spreading Factor

SFN	System Frame Number

SSC	Secondary Synchronisation Code

STTD	Space Time Transmit Diversity

TFCI	Transport Format Combination Indicator

TSTD	Time Switched Transmit Diversity

TPC	Transmit Power Control

TPI	Transmitted Precoding Indicator

UE	User Equipment

UTRAN	UMTS Terrestrial Radio Access Network

## 3.3	Definitions

Assisting secondary serving HS-DSCH Cell: In addition to the serving HS-DSCH cell, a cell in the secondary downlink frequency, where the UE is configured to simultaneously monitor a HS-SCCH set and receive HS-DSCH if it is scheduled in that cell.

Assisting serving HS-DSCH Cell: In addition to the serving HS-DSCH cell, a cell in the same frequency, where the UE is configured to simultaneously monitor a HS-SCCH set and receive HS-DSCH if it is scheduled in that cell.

DL FET: DL FET refers to early termination of DL DPCH transmission upon receiving an acknowledgement message. In this context, a DL FET ACK/NACK message represents an acknowledge message sent on UL DPCCH for DL FET.

DL_DCH_FET_Config: Higher layers signal this configuration parameter to indicate enhanced DCH physical layer configuration. The possible values are 0 and 1. The value 0 indicates Mode 0 configuration where DL transport channels concatenation and DL FET ACK/NACK signalling on UL are not configured. The value 1 indicates Mode 1 where DL transport channel concatenation and DL FET ACK/NACK signalling on UL are configured.

HS-DSCH cell set: A set of cells that can be configured together as the serving and secondary serving HS-DSCH cells for a UE. This term is applicable also to non-serving cells in an active set.

MIMO mode: This term refers to the downlink MIMO configuration with two transmit antennas

MIMO mode with four transmit antennas: This term refers to the downlink MIMO configuration with four transmit antennas

Multiflow mode: The UE is configured in Multiflow mode when it is configured with an assisting serving HS-DSCH cell.

Non-time reference cell: An HS-DSCH cell configured for a UE in Multiflow mode that has a different timing than the time reference cell. If the time reference cell is the Assisting Serving HS-DSCH cell then the non-time reference cell is the Serving HS-DSCH cell. If the time reference cell is the Serving HS-DSCH Cell, then the non-time reference cell is the Assisting Serving HS-DSCH cell.

Time reference cell: The (Serving or Assisting Serving, but not Secondary Serving or Assisting Secondary Serving) HS-DSCH cell acting as the time reference for the uplink HS-DPCCH when the UE is configured in Multiflow mode. There is only one Time reference cell.

UL DPCH 10ms Mode: When configured by higher layers for the TTI to be transmitted [18], UL DPCH follows physical channel procedures specific to this mode. UL DPCH 10ms Mode can only happen when DL_DCH_FET_Config is configured.

UL 20ms Compression Interval (CI): A time interval of 20ms duration aligned to a 20ms TTI defined for UL DPCH physical layer procedures when DL_DCH_FET_Config is configured by higher layers.

# 4	Services offered to higher layers

## 4.1	Transport channels

Transport channels are services offered by Layer 1 to the higher layers. General concepts about transport channels are described in [12].

A transport channel is defined by how and with what characteristics data is transferred over the air interface. A general classification of transport channels is into two groups:

-	Dedicated channels, using inherent addressing of UE;

-	Common channels, using explicit addressing of UE if addressing is needed.

### 4.1.1	Dedicated transport channels

There exists two types of dedicated transport channel, the Dedicated Channel (DCH) and the Enhanced Dedicated Channel (E-DCH).

#### 4.1.1.1	DCH - Dedicated Channel

The Dedicated Channel (DCH) is a downlink or uplink transport channel. The DCH is transmitted over the entire cell or over only a part of the cell using e.g. beam-forming antennas.

#### 4.1.1.2	E-DCH – Enhanced Dedicated Channel

The Enhanced Dedicated Channel (E-DCH) is an uplink transport channel in CELL DCH.

### 4.1.2	Common transport channels

There are six types of common transport channels: BCH, FACH, PCH, RACH, HS-DSCH and E-DCH.

#### 4.1.2.1	BCH - Broadcast Channel

The Broadcast Channel (BCH) is a downlink transport channel that is used to broadcast system- and cell-specific information. The BCH is always transmitted over the entire cell and has a single non-zero transport format. In cells configured with broadcast distribution, a first BCH mapped to P-CCPCH is always present and one additional BCH mapped to S-CCPCH can be configured.

#### 4.1.2.2	FACH - Forward Access Channel

The Forward Access Channel (FACH) is a downlink transport channel. The FACH is transmitted over the entire cell. The FACH can be transmitted using power setting described in [16].

#### 4.1.2.3	PCH - Paging Channel

The Paging Channel (PCH) is a downlink transport channel. The PCH is always transmitted over the entire cell. The transmission of the PCH is associated with the transmission of physical-layer generated Paging Indicators, to support efficient sleep-mode procedures.

#### 4.1.2.4	RACH - Random Access Channel

The Random Access Channel (RACH) is an uplink transport channel. The RACH is always received from the entire cell. The RACH is characterized by a collision risk and by being transmitted using open loop power control.

#### 4.1.2.5	Void

#### 4.1.2.6	Void

#### 4.1.2.7	HS-DSCH – High Speed Downlink Shared Channel

The High Speed Downlink Shared Channel is a downlink transport channel shared by several UEs. The HS-DSCH can be associated with one downlink DPCH or F-DPCH, and one or several Shared Control Channels (HS-SCCH). The HS-DSCH is transmitted over the entire cell or over only part of the cell using e.g. beam-forming antennas.

#### 4.1.2.7A	E-DCH - Enhanced Dedicated Channel

The Enhanced Dedicated Channel (E-DCH) is an uplink transport channel in CELL_FACH state and IDLE mode.

## 4.2	Indicators

Indicators are means of fast low-level signalling entities which are transmitted without using information blocks sent over transport channels. The meaning of indicators is specific to the type of indicator.

The indicators defined in the current version of the specifications are: Acquisition Indicator (AI), Page Indicator (PI) and MBMS Notification Indicator (NI).

Indicators may be either boolean (two-valued) or three-valued. Their mapping to indicator channels is channel specific.

Indicators are transmitted on those physical channels that are indicator channels (ICH).

# 5	Physical channels and physical signals

Physical channels are defined by a specific carrier frequency, scrambling code, channelization code (optional), time start & stop (giving a duration) and, on the uplink, relative phase (0 or /2). The downlink E-HICH and E-RGCH are each further defined by a specific orthogonal signature sequence. Scrambling and channelization codes are specified in [4]. Time durations are defined by start and stop instants, measured in integer multiples of chips. Suitable multiples of chips also used in specification are:

Radio frame:	A radio frame is a processing duration which consists of 15 slots. The length of a radio frame corresponds to 38400 chips.

Slot:	A slot is a duration which consists of fields containing bits. The length of a slot corresponds to 2560 chips.

Sub-frame:	A sub-frame is the basic time interval for E-DCH and HS-DSCH transmission and E-DCH and HS-DSCH-related signalling at the physical layer. The length of a sub-frame corresponds to 3 slots (7680 chips).

The default time duration for a physical channel is continuous from the instant when it is started to the instant when it is stopped. Physical channels that are not continuous will be explicitly described.

Transport channels are described (in more abstract higher layer models of the physical layer) as being capable of being mapped to physical channels. Within the physical layer itself the exact mapping is from a composite coded transport channel (CCTrCH) to the data part of a physical channel. In addition to data parts there also exist channel control parts and physical signals.

## 5.1	Physical signals

Physical signals are entities with the same basic on-air attributes as physical channels but do not have transport channels or indicators mapped to them. Physical signals may be associated with physical channels in order to support the function of physical channels.

## 5.2	Uplink physical channels

### 5.2.1	Dedicated uplink physical channels

There are seven types of uplink dedicated physical channels, the uplink Dedicated Physical Data Channel (uplink DPDCH), the uplink Dedicated Physical Control Channel (uplink DPCCH), the uplink Secondary Dedicated Physical Control Channel (uplink S-DPCCH), the uplink Dedicated Physical Control Channel 2 (uplink DPCCH2), the uplink E-DCH Dedicated Physical Data Channel (uplink E-DPDCH), the uplink E-DCH Dedicated Physical Control Channel (uplink E-DPCCH) and the uplink Dedicated Control Channel associated with HS-DSCH transmission (uplink HS-DPCCH).

The DPDCH, the DPCCH, the DPCCH2, the S-DPCCH, the E-DPDCH, the E-DPCCH and the HS-DPCCH are I/Q code multiplexed (see [4]).

#### 5.2.1.1	DPCCH, DPCCH2, S-DPCCH and DPDCH

The uplink DPDCH is used to carry the DCH transport channel. There may be zero, one, or several uplink DPDCHs on each radio link.

The uplink DPCCH is used to carry control information generated at Layer 1. The Layer 1 control information consists of known pilot bits to support channel estimation for coherent detection, transmit power-control (TPC) commands, feedback information (FBI), and an optional transport-format combination indicator (TFCI). The transport-format combination indicator informs the receiver about the instantaneous transport format combination of the transport channels mapped to the simultaneously transmitted uplink DPDCH radio frame. There is one and only one uplink DPCCH on each radio link. The uplink DPCCH may also carry DL FET ACK/NACK signalling information for DL FET operation when DL_DCH_FET_Config = 1.

The uplink DPCCH2 is used to carry control information generated at Layer 1. The Layer 1 control information consists of known pilot bits to support channel estimation for coherent detection, and transmit power-control (TPC) commands. When DPCCH2 is configured there is one and only one uplink DPCCH2 for all radio links. When DPCCH2 is configured, the UE shall transmit DPCCH2 only in the slots in which DPCCH is transmitted.

The uplink S-DPCCH is used to carry control information generated at Layer 1. The Layer 1 control information consists of known pilot bits to support channel sounding and channel estimation for coherent detection. There is up to one uplink S-DPCCH on each radio link in the case that UL_CLTD_Enabled as defined in [5] is TRUE. Figure 1 shows the frame structure of the uplink DPDCH, the uplink DPCCH, the uplink DPCCH2 and the uplink S-DPCCH. Each radio frame of length 10 ms is split into 5 subframes, each of 3 slots, each of length Tslot = 2560 chips, corresponding to one power-control period. The DPDCH, DPCCH, S-DPCCH and DPCCH2 are always frame aligned with each other.

Figure 1: Frame structure for uplink DPDCH/DPCCH/S-DPCCH/DPCCH2

The parameter k in figure 1 determines the number of bits per uplink DPDCH slot. It is related to the spreading factor SF of the DPDCH as SF = 256/2k. The DPDCH spreading factor may range from 256 down to 4. The spreading factor of the uplink DPCCH, the uplink DPCCH2 and the uplink S-DPCCH is always equal to 256, i.e. there are 10 bits per uplink DPCCH/S-DPCCH/DPCCH2 slot.

The exact number of bits of the uplink DPDCH and the different uplink DPCCH fields (Npilot, NTFCI, NFBI, and NTPC) is given by table 1 and table 2. What slot format to use is configured by higher layers and can also be reconfigured by higher layers. The exact number of bits of the uplink S-DPCCH is given by table 2A.

The uplink DPCCH2 reuses slot format #1 of DPCCH, and the exact number of bits is given in table 2.

The channel bit and symbol rates given in table 1, table 2 and table 2A are the rates immediately before spreading. The pilot patterns are given in table 3 and table 4, the TPC bit pattern is given in table 5.

The FBI bits are used to support techniques requiring feedback from the UE to the UTRAN Access Point for operation of closed loop mode transmit diversity. The use of the FBI bits is described in detail in [5].

Table 1: DPDCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Ndata |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 15 | 256 | 150 | 10 | 10 |
| 1 | 30 | 30 | 128 | 300 | 20 | 20 |
| 2 | 60 | 60 | 64 | 600 | 40 | 40 |
| 3 | 120 | 120 | 32 | 1200 | 80 | 80 |
| 4 | 240 | 240 | 16 | 2400 | 160 | 160 |
| 5 | 480 | 480 | 8 | 4800 | 320 | 320 |
| 6 | 960 | 960 | 4 | 9600 | 640 | 640 |

There are two types of uplink dedicated physical channels; those that include TFCI (e.g. for several simultaneous services) and those that do not include TFCI (e.g. for fixed-rate services). These types are reflected by the duplicated rows of table 2. It is the UTRAN that determines if a TFCI should be transmitted and it is mandatory for all UEs to support the use of TFCI in the uplink. The mapping of TFCI bits onto slots is described in [3].

DPCCH slot format 5 is used when DL_DCH_FET_Config is configured by higher layers. In this case, over a 20ms Compression Interval (CI) consisting of 30 slots, the bits in the TFCI field carry TFCI information in the first 10 slots, and may carry an ACK/NACK indicator for DL FET in the remaining 20 slots. The ACK/NACK for DL FET is sent when DL_DCH_FET_Config = 1.

In compressed mode, DPCCH slot formats with TFCI fields are changed. There are two possible compressed slot formats for normal slot formats 0 and 2. They are labelled A and B and the selection between them is dependent on the number of slots that are transmitted in each frame in compressed mode. When DL_DCH_FET_Config is configured by higher layers, UL DPCCH slot format 5 is used in compressed mode. In compressed mode when UL_DCH_FET_Config is configured by higher layers, TFCI bits represent TFCI information in the first 10 slots that are not in compressed-mode gaps in a 20ms CI. The TFCI bits in subsequent slots that are not in the compressed mode gaps represent ACK/NACK indication for DL FET when DL_DCH_FET_Config = 1.

If UL_DTX_Active is TRUE (see [5]), the number of transmitted slots per radio frame may be less than the number shown in Table 2 and Table 2A.

Table 2: DPCCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Npilot | NTPC | NTFCI | NFBI | Transmitted slots per radio frame |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 15 | 256 | 150 | 10 | 6 | 2 | 2 | 0 | 15 |
| 0A | 15 | 15 | 256 | 150 | 10 | 5 | 2 | 3 | 0 | 10-14 |
| 0B | 15 | 15 | 256 | 150 | 10 | 4 | 2 | 4 | 0 | 8-9 |
| 1 | 15 | 15 | 256 | 150 | 10 | 8 | 2 | 0 | 0 | 8-15 |
| 2 | 15 | 15 | 256 | 150 | 10 | 5 | 2 | 2 | 1 | 15 |
| 2A | 15 | 15 | 256 | 150 | 10 | 4 | 2 | 3 | 1 | 10-14 |
| 2B | 15 | 15 | 256 | 150 | 10 | 3 | 2 | 4 | 1 | 8-9 |
| 3 | 15 | 15 | 256 | 150 | 10 | 7 | 2 | 0 | 1 | 8-15 |
| 4 | 15 | 15 | 256 | 150 | 10 | 6 | 4 | 0 | 0 | 8-15 |
| 5 | 15 | 15 | 256 | 150 | 10 | 6 | 2 | 2* | 0 | 8-15 |

* NOTE:	In a 20ms CI of 30 slots, TFCI bits represent TFCI information in the first 10 slots and may represent DL FET ACK/NACK indication for DL FET in the remaining slots. Slot format 5 is also used in compressed mode for UL DPCCH.

Table 2A: S-DPCCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Npilot | Nfixed | Transmitted slots per radio frame |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 256 | 150 | 10 | 8 | 2 | 8-15 |

The pilot bit pattern for S-DPCCH is the same as that for uplink DPCCH with Npilot = 8. The Nfixed bits in the S-DPCCH are fixed to "10".

The pilot bit pattern for DPCCH2 is the same as that for uplink DPCCH with Npilot = 8.

The pilot bit patterns are described in table 3 and table 4. The shadowed column part of pilot bit pattern is defined as FSW and FSWs can be used to confirm frame synchronization. (The value of the pilot bit pattern other than FSWs shall be "1".)

Table 3: Pilot bit patterns for uplink DPCCH with Npilot = 3, 4, 5 and 6

|  | Npilot = 3 |  |  | Npilot = 4 |  |  |  | Npilot = 5 |  |  |  |  | Npilot = 6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit # | 0 | 1 | 2 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 0 | 1 | 2 | 3 | 4 | 5 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 0 0 0 1 0 0 1 1 0 1 0 1 1 | 0 0 1 0 1 0 0 0 0 1 1 1 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 0 0 0 1 0 0 1 1 0 1 0 1 1 | 0 0 1 0 1 0 0 0 0 1 1 1 0 1 1 |

Table 4: Pilot bit patterns for uplink DPCCH with Npilot = 7 and 8

|  | Npilot = 7 |  |  |  |  |  |  | Npilot = 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 0 0 0 1 0 0 1 1 0 1 0 1 1 | 0 0 1 0 1 0 0 0 0 1 1 1 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 0 0 0 1 0 0 1 1 0 1 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 0 0 1 0 1 0 0 0 0 1 1 1 0 1 1 |

The relationship between the TPC bit pattern and transmitter power control command is presented in table 5.

Table 5: TPC Bit Pattern

| TPC Bit Pattern |  | Transmitter power control command |
| --- | --- | --- |
| NTPC = 2 | NTPC = 4 |  |
| 11 00 | 1111 0000 | 1 0 |

Multi-code operation is possible for the uplink dedicated physical channels. When multi-code transmission is used, several parallel DPDCH are transmitted using different channelization codes, see [4]. However, there is only one DPCCH per radio link, one DPCCH2 if DPCCH2 is configured and up to one S-DPCCH in the case that UL_CLTD_Enable is TRUE.

A period of uplink DPCCH transmission prior to the start of the uplink DPDCH transmission (uplink DPCCH power control preamble) shall be used for initialisation of a DCH. The length of the power control preamble is a higher layer parameter, Npcp , signalled by the network [5]. The UL DPCCH shall take the same slot format in the power control preamble as afterwards, as given in table 2. When Npcp > 0 the pilot patterns of table 3 and table 4 shall be used. The timing of the power control preamble is described in [5], subclause 4.3.2.3. The TFCI field is filled with "0" bits.

#### 5.2.1.2	HS-DPCCH

Figure 2A illustrates the frame structure of the HS-DPCCH. The HS-DPCCH carries uplink feedback signalling related to downlink HS-DSCH transmission and to HS-SCCH orders according to subclause 6A.1.1 in [5]. The feedback signalling consists of Hybrid-ARQ Acknowledgement (HARQ-ACK) and Channel-Quality Indication (CQI), in case the UE is configured in MIMO mode or in MIMO mode with four transmit antennas Precoding Control Indication (PCI) as well and in case the UE is configured in MIMO mode with four transmit antennas the number of transport blocks preferred (NTBP) as well [3]. Each sub frame of length 2 ms (3*2560 chips) consists of 3 slots, each of length 2560 chips. The HARQ-ACK is carried in the first slot of the HS-DPCCH sub-frame. The CQI, in case the UE is configured in MIMO mode also the PCI, and in case the UE is configured in MIMO mode with four transmit antennas also the PCI and the number of UE preferred transport blocks are carried in the second and third slot of a HS-DPCCH sub-frame. There is at most one HS-DPCCH on each radio link if Secondary_Cell_Enabled as defined in [5] is less than 4 in case the UE is not configured in MIMO mode with four transmit antennas, 2 in case the UE is configured in MIMO mode with four transmit antennas and at most two HS-DPCCHs otherwise. If DPCCH2 is not configured, the HS-DPCCH(s) can only exist together with an uplink DPCCH. If DPCCH2 is configured, the HS-DPCCH(s) can only exist together with an uplink DPCCH2. The timing of the HS-DPCCH relative to the uplink DPCCH is shown in section 7.7 for the case where one HS-DPCCH exists. In the case where two HS-DPCCH exist, both HS-DPCCHs have identical timing.

Figure 2A: Frame structure for uplink HS-DPCCH

The slot formats for uplink HS-DPCCH are defined in Table 5A.

Table 5A: HS-DPCCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Subframe | Bits/ Slot | Transmitted slots per Subframe |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 15 | 256 | 30 | 10 | 3 |
| 1 | 30 | 30 | 128 | 60 | 20 | 3 |

#### 5.2.1.3	E-DPCCH and E-DPDCH

The E-DPDCH is used to carry the E-DCH transport channel. There may be zero, one, or several E-DPDCH on each radio link.

The E-DPCCH is a physical channel used to transmit control information associated with the E-DCH. There is at most one E-DPCCH on each radio link.

E-DPDCH and E-DPCCH are always transmitted simultaneously, except for the following cases when E-DPCCH is transmitted without E-DPDCH:

-	when E-DPDCH but not E-DPCCH is DTXed due to power scaling as described in [5] section 5.1.2.6, or

-	during the ndtx E-DPDCH idle slots if nmax>ntx1 as described in [3] section 4.4.5.2.

E-DPCCH shall not be transmitted in a slot unless DPCCH is also transmitted in the same slot.

Figure 2B shows the E-DPDCH and E-DPCCH (sub)frame structure. Each radio frame is divided in 5 subframes, each of length 2 ms; the first subframe starts at the start of each radio frame and the 5th subframe ends at the end of each radio frame.

An E-DPDCH may use BPSK, 4PAM or 8PAM modulation symbols. In figure 2B, M is the number of bits per modulation symbol i.e. M=1 for BPSK, M=2 for 4PAM and M=3 for 8PAM.

The E-DPDCH slot formats, corresponding rates and number of bits are specified in Table 5B. The E-DPCCH slot format is listed in Table 5C.

Figure 2B: E-DPDCH frame structure

Table 5B: E-DPDCH slot formats

| Slot Format #i | Channel Bit Rate (kbps) | Bits/Symbol M | SF | Bits/ Frame | Bits/ Subframe | Bits/Slot Ndata |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 1 | 256 | 150 | 30 | 10 |
| 1 | 30 | 1 | 128 | 300 | 60 | 20 |
| 2 | 60 | 1 | 64 | 600 | 120 | 40 |
| 3 | 120 | 1 | 32 | 1200 | 240 | 80 |
| 4 | 240 | 1 | 16 | 2400 | 480 | 160 |
| 5 | 480 | 1 | 8 | 4800 | 960 | 320 |
| 6 | 960 | 1 | 4 | 9600 | 1920 | 640 |
| 7 | 1920 | 1 | 2 | 19200 | 3840 | 1280 |
| 8 | 1920 | 2 | 4 | 19200 | 3840 | 1280 |
| 9 | 3840 | 2 | 2 | 38400 | 7680 | 2560 |
| 10 | 2880 | 3 | 4 | 28800 | 5760 | 1920 |
| 11 | 5760 | 3 | 2 | 57600 | 11520 | 3840 |

Table 5C: E-DPCCH slot formats

| Slot Format #i | Channel Bit Rate (kbps) | SF | Bits/ Frame | Bits/ Subframe | Bits/Slot Ndata |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 | 256 | 150 | 30 | 10 |

#### 5.2.1.3A	S-E-DPCCH and S-E-DPDCH

The S-E-DPDCH is used to carry the E-DCH transport channel. When UL_MIMO_Enabled is set to TRUE and rank-2 transmission takes place on a radio link, the number of S-E-DPDCH channels on that radio link is 4, otherwise, it is zero.

The S-E-DPCCH is a physical channel used to transmit control information associated with the S-E-DPDCH. There is at most one S-E-DPCCH on each radio link.

S-E-DPDCH and S-E-DPCCH are always transmitted simultaneously.

S-E-DPCCH shall not be transmitted in a slot unless DPCCH and S-DPCCH is also transmitted in the same slot.

The S-E-DPDCH frame structure is the same as the E-DPDCH frame structure. The S-E-DPCCH frame structure is the same as the E-DPCCH frame structure.

An S-E-DPDCH may use BPSK, 4PAM or 8PAM modulation symbols.

The S-E-DPDCH slot formats, corresponding rates and number of bits are as specified for slot formats 6-11 in table 5B for E-DPDCH. Slot formats 0-5 are not applicable for S-E-DPDCH.

The S-E-DPCCH slot format is as specified for E-DPCCH in Table 5C.

### 5.2.2	Common uplink physical channels

#### 5.2.2.1	Physical Random Access Channel (PRACH)

The Physical Random Access Channel (PRACH) is used to carry the RACH.

##### 5.2.2.1.1	Overall structure of random-access transmission

The random-access transmission is based on a Slotted ALOHA approach with fast acquisition indication. The UE can start the random-access transmission at the beginning of a number of well-defined time intervals, denoted access slots. There are 15 access slots per two frames and they are spaced 5120 chips apart, see figure 3. The timing of the access slots and the acquisition indication is described in subclause 7.3. Information on what access slots are available for random-access transmission is given by higher layers.

Figure 3: RACH access slot numbers and their spacing

The structure of the random-access transmission is shown in figure 4. The random-access transmission consists of one or several preambles of length 4096 chips and a message of length 10 ms or 20 ms.

Figure 4: Structure of the random-access transmission

##### 5.2.2.1.2	RACH preamble part

Each preamble is of length 4096 chips and consists of 256 repetitions of a signature of length 16 chips. There are a maximum of 16 available signatures, see [4] for more details.

##### 5.2.2.1.3	RACH message part

Figure 5 shows the structure of the random-access message part radio frame. The 10 ms message part radio frame is split into 15 slots, each of length Tslot = 2560 chips. Each slot consists of two parts, a data part to which the RACH transport channel is mapped and a control part that carries Layer 1 control information. The data and control parts are transmitted in parallel. A 10 ms message part consists of one message part radio frame, while a 20 ms message part consists of two consecutive 10 ms message part radio frames. The message part length is equal to the Transmission Time Interval of the RACH Transport channel in use. This TTI length is configured by higher layers.

The data part consists of 10*2k bits, where k=0,1,2,3. This corresponds to a spreading factor of 256, 128, 64, and 32 respectively for the message data part.

The control part consists of 8 known pilot bits to support channel estimation for coherent detection and 2 TFCI bits. This corresponds to a spreading factor of 256 for the message control part. The pilot bit pattern is described in table 8. The total number of TFCI bits in the random-access message is 15*2 = 30. The TFCI of a radio frame indicates the transport format of the RACH transport channel mapped to the simultaneously transmitted message part radio frame. In case of a 20 ms PRACH message part, the TFCI is repeated in the second radio frame.

Figure 5: Structure of the random-access message part radio frame

Table 6: Random-access message data fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Ndata |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 15 | 256 | 150 | 10 | 10 |
| 1 | 30 | 30 | 128 | 300 | 20 | 20 |
| 2 | 60 | 60 | 64 | 600 | 40 | 40 |
| 3 | 120 | 120 | 32 | 1200 | 80 | 80 |

Table 7: Random-access message control fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Npilot | NTFCI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 15 | 256 | 150 | 10 | 8 | 2 |

Table 8: Pilot bit patterns for RACH message part with Npilot = 8

|  | Npilot = 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 0 0 1 1 1 1 0 1 0 1 1 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 0 1 0 0 1 1 0 1 1 1 0 0 0 0 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 0 0 0 1 0 0 1 1 0 1 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 0 0 1 0 1 0 0 0 0 1 1 1 0 1 1 |

#### 5.2.2.2	Void

## 5.3	Downlink physical channels

### 5.3.1	Downlink transmit diversity

Table 10 summarises the possible application of open and closed loop transmit diversity modes on different downlink physical channel types. Simultaneous use of STTD and closed loop modes on the same physical channel is not allowed. In addition, if Tx diversity is applied on any of the downlink physical channels allocated to a UE(s) that is configured to use P-CPICH as a phase reference on both antennas, then Tx diversity shall also be applied on P-CCPCH and SCH. If Tx diversity is applied on SCH it shall also be applied on P-CCPCH and vice versa. Regarding CPICH transmission in case of transmit diversity used on SCH and P-CCPCH, see subclause 5.3.3.1. An S-CCPCH carrying BCH shall be transmitted with the same diversity mode as P-CCPCH.

With respect to the usage of Tx diversity for DPCH or F-DPCH on different radio links within an active set, the following rules apply:

-	Different Tx diversity modes (STTD and closed loop) shall not be used on the radio links within one active set.

-	No Tx diversity on one or more radio links shall not prevent UTRAN to use Tx diversity on other radio links within the same active set.

-	If STTD is activated on one or several radio links in the active set, the UE shall operate STTD on only those radio links where STTD has been activated. Higher layers inform the UE about the usage of STTD on the individual radio links in the active set.

-	Regarding the usage of Tx diversity for DPCH on different radio links within an active set, if closed loop TX diversity is activated on one or several radio links in the active set, the UE shall operate closed loop TX diversity on only those radio links where closed loop TX diversity has been activated. Higher layers inform the UE about the usage of closed loop TX diversity on the individual radio links in the active set.

Furthermore, if the UE is not configured in MIMO mode and in MIMO mode with four transmit antennas in a cell the following restrictions apply in this cell:

-	If a DPCH is associated with an HS-PDSCH subframe in the same cell, the transmit diversity mode used for the HS-PDSCH subframe shall be the same as the transmit diversity mode used for the DPCH associated with this HS-PDSCH subframe.

-	If an F-DPCH is associated with an HS-PDSCH subframe in the same cell, the transmit diversity mode used for the HS-PDSCH subframe shall be the same as the transmit diversity mode signalled for the F-DPCH associated with this HS-PDSCH subframe.

-	If neither DPCH nor F-DPCH is associated with an HS-PDSCH subframe the transmit diversity mode used for the HS-PDSCH subframe shall be the STTD if the P-CCPCH in the cell is using transmit diversity. Otherwise, no transmit diversity is used for the HS-PDSCH subframe.

-	If the UE is configured with a secondary serving HS-DSCH cell not associated with either DPCH or F-DPCH in the same cell, the diversity mode used for the HS-PDSCH subframe of that cell is configured by higher layers and independent from that used in the serving HS-DSCH cell.

If the UE is configured in MIMO mode in a cell then a DPCH or F-DPCH associated with an HS-PDSCH subframe can be either in transmit diversity mode or in no transmit diversity mode in this cell.

Regardless of whether or not the UE is configured in MIMO mode or in MIMO mode with four transmit antennas in a cell,

-	If the DPCH associated with an HS-SCCH subframe in the same cell is using either open or closed loop transmit diversity on the radio link transmitted from the HS-DSCH serving cell, the HS-SCCH subframe from this cell shall be transmitted using STTD, otherwise no transmit diversity shall be used for this HS-SCCH subframe.

-	If an F-DPCH for which STTD is signalled is associated with an HS-SCCH subframe in the same cell, the HS-SCCH subframe shall be transmitted using STTD, otherwise no transmit diversity shall be used for this HS-SCCH subframe.

-	If neither DPCH nor F-DPCH is associated with an HS-SCCH subframe the transmit diversity mode used for the HS-SCCH subframe shall be the STTD if the P-CCPCH in the cell is using transmit diversity. Otherwise, no transmit diversity is used for the HS-SCCH subframe.

-	If the UE is configured with a secondary serving HS-DSCH cell not associated with either DPCH or F-DPCH in the same cell, the diversity mode used for the HS-SCCH subframe of that cell is configured by higher layers and independent from that used in the serving HS-DSCH cell.

-	If DPCCH2 is configured and if more than one F-DPCH is transmitted in the same cell, then the transmit diversity mode is the same for all the F-DPCHs. If DPCCH2 is configured and if both DPCH and F-DPCH are transmitted in the same cell, then the transmit diversity modes for the DPCH and F-DPCH can be configured independently.

The transmit diversity mode on the associated DPCH or F-DPCH may not change during a HS-SCCH and or HS-PDSCH subframe and within the slot prior to the HS-SCCH subframe. This includes any change between no Tx diversity and either open loop or closed loop mode.

If the UE is receiving a DPCH on which transmit diversity is used from a cell, or if the UE is receiving an F-DPCH for which STTD is signalled from a cell, the UE shall assume that the E-AGCH, E-ROCH, E-RGCH, E-HICH, and F-TPICH from the same cell are transmitted using STTD.

If DPCCH2 is configured and if the UE is receiving both DPCH and F-DPCH from a cell, then the UE shall assume that the E-AGCH, E-ROCH, E-RGCH, E-HICH, and F-TPICH from the same cell are transmitted using the transmit diversity mode signalled for the F-DPCH.

Table 10: Application of Tx diversity modes on downlink physical channel types
"X" – can be applied, "–" – not applied

| Physical channel type |  | Open loop mode |  |  | Closed loop mode |
| --- | --- | --- | --- | --- | --- |
|  | TSTD |  | STTD | Mode 1 |  |
| P-CCPCH | – |  | X | – |  |
| SCH | X |  | – | – |  |
| S-CCPCH | – |  | X | – |  |
| DPCH (when DPCH slot format has pilot bits) | – |  | X | X |  |
| DPCH (DPCH slot-format does not have pilot bits) | – |  | – | – |  |
| F-DPCH | – |  | X | – |  |
| PICH | – |  | X | – |  |
| MICH | – |  | X | – |  |
| HS-PDSCH (UE not in MIMO mode and not in MIMO mode with four transmit antennas, UE configured without a secondary serving HS-DSCH cell) | – |  | X | X |  |
| HS-PDSCH (UE not in MIMO mode and not in MIMO mode with four transmit antennas in this cell, UE configured with a secondary serving HS-DSCH cell) (*1) (*2) | – |  | X | – |  |
| HS-PDSCH (UE in MIMO mode or in MIMO mode with four transmit antennas in this cell ) (*2) | – |  | – | – |  |
| HS-SCCH (*1) | – |  | X | – |  |
| E-AGCH | – |  | X | – |  |
| E-ROCH | – |  | X | – |  |
| E-RGCH | – |  | X | – |  |
| E-HICH | – |  | X | – |  |
| AICH | – |  | X | – |  |
| F-TPICH | – |  | X | – |  |

NOTE *1:	The Tx diversity mode can be configured independently across cells.

NOTE *2:	The MIMO mode or MIMO mode with four transmit antennas can be configured independently across cells.

#### 5.3.1.1	Open loop transmit diversity

##### 5.3.1.1.1	Space time block coding based transmit antenna diversity (STTD)

The open loop downlink transmit diversity employs a space time block coding based transmit diversity (STTD).

The STTD encoding is optional in UTRAN. STTD support is mandatory at the UE.

A block diagram of a generic STTD encoder is shown in the figure 8, figure 8A and figure 8B below. Channel coding, rate matching and interleaving are done as in the non-diversity mode. For QPSK, the STTD encoder operates on 4 symbols b0, b1, b2, b3 as shown in figure 8. For AICH, E-RGCH, E-HICH the bi are real valued signals, and biis defined as −bi. For channels other than AICH, E-RGCH, E-HICH the bi are 3-valued digits, taking the values 0, 1, "DTX", and biis defined as follows: if bi = 0 then bi= 1, if bi = 1 then bi= 0, otherwise bi = bi.

Figure 8: Generic block diagram of the STTD encoder for QPSK

For 16QAM, STTD operates on blocks of 8 consecutive symbols b0, b1, b2, b3, b4, b5, b6, b7 as shown in figure 8A below.

Figure 8A: Generic block diagram of the STTD encoder for 16QAM

For 64QAM, STTD operates on blocks of 12 consecutive symbols b0, b1, b2, b3, b4, b5, b6, b7, b8, b9, b10, b11 as shown in figure 8B below.

Figure 8B: Generic block diagram of the STTD encoder for 64QAM

##### 5.3.1.1.2	Time Switched Transmit Diversity for SCH (TSTD)

Transmit diversity, in the form of Time Switched Transmit Diversity (TSTD), can be applied to the SCH. TSTD for the SCH is optional in UTRAN, while TSTD support is mandatory in the UE. TSTD for the SCH is described in subclause 5.3.3.5.1.

#### 5.3.1.2	Closed loop transmit diversity

Closed loop transmit diversity is described in [5]. Closed loop transmit diversity mode 1 shall be supported at the UE and may be supported in the UTRAN.

### 5.3.2	Dedicated downlink physical channels

There are five types of downlink dedicated physical channels, the Downlink Dedicated Physical Channel (downlink DPCH), the Fractional Dedicated Physical Channel (F-DPCH), the E-DCH Relative Grant Channel (E-RGCH), the E-DCH Hybrid ARQ Indicator Channel (E-HICH), and the Fractional Transmitted Precoding Indicator Channel (F-TPICH).

The F-DPCH is described in subclause 5.3.2.6.

Within one downlink DPCH, dedicated data generated at Layer 2 and above, i.e. the dedicated transport channel (DCH), is transmitted in time-multiplex with control information generated at Layer 1 (known pilot bits, TPC commands, and an optional TFCI). The downlink DPCH can thus be seen as a time multiplex of a downlink DPDCH and a downlink DPCCH, compare subclause 5.2.1.

Figure 9 shows the frame structure of the downlink DPCH. Each frame of length 10 ms is split into 15 slots, each of length Tslot = 2560 chips, corresponding to one power-control period.

Figure 9: Frame structure for downlink DPCH

The parameter k in figure 9 determines the total number of bits per downlink DPCH slot. It is related to the spreading factor SF of the physical channel as SF = 512/2k. The spreading factor may thus range from 512 down to 4.

The exact number of bits of the different downlink DPCH fields (Npilot, NTPC, NTFCI, Ndata1 and Ndata2) is given in table 11. What slot format to use is configured by higher layers and can also be reconfigured by higher layers.

There are basically two types of downlink Dedicated Physical Channels; those that include TFCI (e.g. for several simultaneous services) and those that do not include TFCI (e.g. for fixed-rate services). These types are reflected by the duplicated rows of table 11. It is the UTRAN that determines if a TFCI should be transmitted and it is mandatory for all UEs to support the use of TFCI in the downlink. The mapping of TFCI bits onto slots is described in [3].

In compressed frames, a different slot format is used compared to normal mode. There are two possible compressed slot formats that are labelled A and B. Slot format B shall be used in frames compressed by spreading factor reduction and slot format A shall be used in frames compressed by higher layer scheduling. The channel bit and symbol rates given in table 11 are the rates immediately before spreading.

Table 11: DPDCH and DPCCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Slot | DPDCH Bits/Slot |  | DPCCH Bits/Slot |  |  | Transmitted slots per radio frame NTr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NData1 | NData2 | NTPC | NTFCI | NPilot |  |
| 0 | 15 | 7.5 | 512 | 10 | 0 | 4 | 2 | 0 | 4 | 15 |
| 0A | 15 | 7.5 | 512 | 10 | 0 | 4 | 2 | 0 | 4 | 8-14 |
| 0B | 30 | 15 | 256 | 20 | 0 | 8 | 4 | 0 | 8 | 8-14 |
| 1 | 15 | 7.5 | 512 | 10 | 0 | 2 | 2 | 2 | 4 | 15 |
| 1B | 30 | 15 | 256 | 20 | 0 | 4 | 4 | 4 | 8 | 8-14 |
| 2 | 30 | 15 | 256 | 20 | 2 | 14 | 2 | 0 | 2 | 15 |
| 2A | 30 | 15 | 256 | 20 | 2 | 14 | 2 | 0 | 2 | 8-14 |
| 2B | 60 | 30 | 128 | 40 | 4 | 28 | 4 | 0 | 4 | 8-14 |
| 3 | 30 | 15 | 256 | 20 | 2 | 12 | 2 | 2 | 2 | 15 |
| 3A | 30 | 15 | 256 | 20 | 2 | 10 | 2 | 4 | 2 | 8-14 |
| 3B | 60 | 30 | 128 | 40 | 4 | 24 | 4 | 4 | 4 | 8-14 |
| 4 | 30 | 15 | 256 | 20 | 2 | 12 | 2 | 0 | 4 | 15 |
| 4A | 30 | 15 | 256 | 20 | 2 | 12 | 2 | 0 | 4 | 8-14 |
| 4B | 60 | 30 | 128 | 40 | 4 | 24 | 4 | 0 | 8 | 8-14 |
| 5 | 30 | 15 | 256 | 20 | 2 | 10 | 2 | 2 | 4 | 15 |
| 5A | 30 | 15 | 256 | 20 | 2 | 8 | 2 | 4 | 4 | 8-14 |
| 5B | 60 | 30 | 128 | 40 | 4 | 20 | 4 | 4 | 8 | 8-14 |
| 6 | 30 | 15 | 256 | 20 | 2 | 8 | 2 | 0 | 8 | 15 |
| 6A | 30 | 15 | 256 | 20 | 2 | 8 | 2 | 0 | 8 | 8-14 |
| 6B | 60 | 30 | 128 | 40 | 4 | 16 | 4 | 0 | 16 | 8-14 |
| 7 | 30 | 15 | 256 | 20 | 2 | 6 | 2 | 2 | 8 | 15 |
| 7A | 30 | 15 | 256 | 20 | 2 | 4 | 2 | 4 | 8 | 8-14 |
| 7B | 60 | 30 | 128 | 40 | 4 | 12 | 4 | 4 | 16 | 8-14 |
| 8 | 60 | 30 | 128 | 40 | 6 | 28 | 2 | 0 | 4 | 15 |
| 8A | 60 | 30 | 128 | 40 | 6 | 28 | 2 | 0 | 4 | 8-14 |
| 8B | 120 | 60 | 64 | 80 | 12 | 56 | 4 | 0 | 8 | 8-14 |
| 9 | 60 | 30 | 128 | 40 | 6 | 26 | 2 | 2 | 4 | 15 |
| 9A | 60 | 30 | 128 | 40 | 6 | 24 | 2 | 4 | 4 | 8-14 |
| 9B | 120 | 60 | 64 | 80 | 12 | 52 | 4 | 4 | 8 | 8-14 |
| 10 | 60 | 30 | 128 | 40 | 6 | 24 | 2 | 0 | 8 | 15 |
| 10A | 60 | 30 | 128 | 40 | 6 | 24 | 2 | 0 | 8 | 8-14 |
| 10B | 120 | 60 | 64 | 80 | 12 | 48 | 4 | 0 | 16 | 8-14 |
| 11 | 60 | 30 | 128 | 40 | 6 | 22 | 2 | 2 | 8 | 15 |
| 11A | 60 | 30 | 128 | 40 | 6 | 20 | 2 | 4 | 8 | 8-14 |
| 11B | 120 | 60 | 64 | 80 | 12 | 44 | 4 | 4 | 16 | 8-14 |
| 12 | 120 | 60 | 64 | 80 | 12 | 48 | 4 | 8* | 8 | 15 |
| 12A | 120 | 60 | 64 | 80 | 12 | 40 | 4 | 16* | 8 | 8-14 |
| 12B | 240 | 120 | 32 | 160 | 24 | 96 | 8 | 16* | 16 | 8-14 |
| 13 | 240 | 120 | 32 | 160 | 28 | 112 | 4 | 8* | 8 | 15 |
| 13A | 240 | 120 | 32 | 160 | 28 | 104 | 4 | 16* | 8 | 8-14 |
| 13B | 480 | 240 | 16 | 320 | 56 | 224 | 8 | 16* | 16 | 8-14 |
| 14 | 480 | 240 | 16 | 320 | 56 | 232 | 8 | 8* | 16 | 15 |
| 14A | 480 | 240 | 16 | 320 | 56 | 224 | 8 | 16* | 16 | 8-14 |
| 14B | 960 | 480 | 8 | 640 | 112 | 464 | 16 | 16* | 32 | 8-14 |
| 15 | 960 | 480 | 8 | 640 | 120 | 488 | 8 | 8* | 16 | 15 |
| 15A | 960 | 480 | 8 | 640 | 120 | 480 | 8 | 16* | 16 | 8-14 |
| 15B | 1920 | 960 | 4 | 1280 | 240 | 976 | 16 | 16* | 32 | 8-14 |
| 16 | 1920 | 960 | 4 | 1280 | 248 | 1000 | 8 | 8* | 16 | 15 |
| 16A | 1920 | 960 | 4 | 1280 | 248 | 992 | 8 | 16* | 16 | 8-14 |
| 17 | 30 | 15 | 256 | 20 | 18 | 0 | 2 | 0 | 0 | 8-15 |
| 18 | 60 | 30 | 128 | 40 | 38 | 0 | 2 | 0 | 0 | 8-15 |

* If TFCI bits are not used, then DTX shall be used in TFCI field.

NOTE 1:	Compressed mode is only supported through spreading factor reduction for SF=512 with TFCI.

NOTE 2:	Compressed mode by spreading factor reduction is not supported for SF=4.

NOTE 3:	If the Node B receives an invalid combination of data frames for downlink transmission, the procedure specified in [15], sub-clause 5.1.2, may require the use of DTX in both the DPDCH and the TFCI field of the DPCCH.

NOTE 4:  Slot formats 17 and 18 are only used when DL_DCH_FET_Config is configured by higher layers.

The pilot bit patterns are described in table 12. The shadowed column part of pilot bit pattern is defined as FSW and FSWs can be used to confirm frame synchronization. (The value of the pilot bit pattern other than FSWs shall be "11".) In table 12, the transmission order is from left to right.

In downlink compressed mode through spreading factor reduction, the number of bits in the TPC and Pilot fields are doubled. Symbol repetition is used to fill up the fields. Denote the bits in one of these fields in normal mode by x1, x2, x3, …, xX. In compressed mode the following bit sequence is sent in corresponding field: x1, x2, x1, x2, x3, x4, x3, x4,…, xX,.

Table 12: Pilot bit patterns for downlink DPCCH with Npilot = 2, 4, 8 and 16

|  | Npilot = 2 | Npilot = 4 (*1) |  | Npilot = 8 (*2) |  |  |  | Npilot = 16 (*3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # | 0 | 0 | 1 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 10 01 00 01 10 00 00 10 11 01 11 00 11 11 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 10 01 00 01 10 00 00 10 11 01 11 00 11 11 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 11 10 01 11 01 10 10 00 00 11 00 01 00 10 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 00 00 10 11 01 11 00 11 11 10 10 01 00 01 |

NOTE *1:	This pattern is used except slot formats 2B and 3B.

NOTE *2:	This pattern is used except slot formats 0B, 1B, 4B, 5B, 8B, and 9B.

NOTE *3:	This pattern is used except slot formats 6B, 7B, 10B, 11B, 12B, and 13B.

NOTE:	For slot format nB where n = 0, …, 15, the pilot bit pattern corresponding to Npilot/2 is to be used and symbol repetition shall be applied.

The relationship between the TPC symbol and the transmitter power control command is presented in table 13.

Table 13: TPC Bit Pattern

| TPC Bit Pattern |  |  | Transmitter power control command |
| --- | --- | --- | --- |
| NTPC = 2 | NTPC = 4 | NTPC = 8 |  |
| 11 00 | 1111 0000 | 11111111 00000000 | 1 0 |

Multicode transmission may be employed in the downlink, i.e. the CCTrCH (see [3]) is mapped onto several parallel downlink DPCHs using the same spreading factor. In this case, the Layer 1 control information is transmitted only on the first downlink DPCH. DTX bits are transmitted during the corresponding time period for the additional downlink DPCHs, see figure 10.

In case there are several CCTrCHs mapped to different DPCHs transmitted to the same UE different spreading factors can be used on DPCHs to which different CCTrCHs are mapped. Also in this case, Layer 1 control information is only transmitted on the first DPCH while DTX bits are transmitted during the corresponding time period for the additional DPCHs.

Note :	support of multiple CCTrChs of dedicated type is not part of the current release.

Figure 10: Downlink slot format in case of multi-code transmission

#### 5.3.2.1	STTD for DPCH, F-DPCH and F-TPICH

The pilot bit pattern for the DPCH channel transmitted on antenna 2 is given in table 14.

-	For Npilot = 8, 16 the shadowed part indicates pilot bits that are obtained by STTD encoding the corresponding (shadowed) bits in Table 12. The non-shadowed pilot bit pattern is orthogonal to the corresponding (non-shadowed) pilot bit pattern in table 12.

-	For Npilot = 4, the diversity antenna pilot bit pattern is obtained by STTD encoding both the shadowed and non-shadowed pilot bits in table 12.

-	For Npilot  = 2, the diversity antenna pilot pattern is obtained by STTD encoding the two pilot bits in table 12 with the last two bits (data or DTX) of the second data field (data2) of the slot. Thus for Npilot = 2 case, the last two bits of the second data field (data 2) after STTD encoding, follow the diversity antenna pilot bits in Table 14.

-	For Npilot = 0, transmit diversity is not used.

STTD encoding for the DPDCH, TPC, and TFCI fields is done as described in subclause 5.3.1.1.1. For the SF=512 DPCH, the first two bits in each slot, i.e. TPC bits, are not STTD encoded and the same bits are transmitted with equal power from the two antennas. The remaining four bits are STTD encoded.

For F-DPCH, the TPC bits are not STTD encoded and the same bits are transmitted with equal power from the two antennas.

For F-TPICH, the TPI bits are not STTD encoded and the same bits are transmitted with equal power from the two antennas.

For compressed mode through spreading factor reduction and for Npilot > 4, symbol repetition shall be applied to the pilot bit patterns of table 14, in the same manner as described in 5.3.2. For slot formats 2B and 3B, i.e. compressed mode through spreading factor reduction and Npilot = 4, the pilot bits transmitted on antenna 2 are STTD encoded, and thus the pilot bit pattern is as shown in the most right set of table 14.

Table 14: Pilot bit patterns of downlink DPCCH for antenna 2 using STTD

|  | Npilot = 2 (*1) | Npilot = 4 (*2) |  | Npilot = 8 (*3) |  |  |  | Npilot = 16 (*4) |  |  |  |  |  |  |  | Npilot = 4 (*5) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # | 0 | 0 | 1 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 01 10 11 10 00 01 01 00 11 01 11 00 00 10 10 | 01 10 11 10 00 01 01 00 11 01 11 00 00 10 10 | 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 10 10 00 01 11 01 10 01 01 00 00 11 10 11 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 10 11 00 10 00 11 11 01 01 10 01 00 01 11 | 01 10 11 10 00 01 01 00 11 01 11 00 00 10 10 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 |

NOTE *1:	The pilot bits precede the last two bits of the data2 field.

NOTE *2:	This pattern is used except slot formats 2B and 3B.

NOTE *3:	This pattern is used except slot formats 0B, 1B, 4B, 5B, 8B, and 9B.

NOTE *4:	This pattern is used except slot formats 6B, 7B, 10B, 11B, 12B, and 13B.

NOTE *5:	This pattern is used for slot formats 2B and 3B.

NOTE:	For slot format nB where n = 0, 1, 4, 5, 6, …, 15, the pilot bit pattern corresponding to Npilot/2 is to be used and symbol repetition shall be applied.

#### 5.3.2.2	Dedicated channel pilots with closed loop mode transmit diversity

In closed loop mode 1 orthogonal pilot patterns are used between the transmit antennas. Closed loop mode 1 shall not be used with DPCH slot formats for which Npilot=2. Pilot patterns defined in the table 12 will be used on antenna 1 and pilot patterns defined in the table 15 on antenna 2. This is illustrated in the figure 11 a which indicates the difference in the pilot patterns with different shading.

Table 15: Pilot bit patterns of downlink DPCCH for antenna 2 using closed loop mode 1

|  |  | Npilot = 4 |  | Npilot = 8 (*1) |  |  |  | Npilot = 16 (*2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # |  | 0 | 1 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 |  | 01 10 11 10 00 01 01 00 11 01 11 00 00 10 10 | 10 10 10 10 10 10 10 10 10 10 10 10 10 10 10 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 10 10 00 01 11 01 10 01 01 00 00 11 10 11 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 10 11 00 10 00 11 11 01 01 10 01 00 01 11 |  |  |

NOTE *1:	This pattern is used except slot formats 0B, 1B, 4B, 5B, 8B, and 9B.

NOTE *2:	This pattern is used except slot formats 6B, 7B, 10B, 11B, 12B, and 13B.

NOTE:	For slot format nB where n = 0, 1, 4, 5, 6, …, 15, the pilot bit pattern corresponding to Npilot/2 is to be used and symbol repetition shall be applied.

Figure 11: Slot structures for downlink dedicated physical channel diversity transmission.
Structure (a) is used in closed loop mode 1.
Different shading of the pilots indicate orthogonality of the patterns

#### 5.3.2.3	Void

#### 5.3.2.4	E-DCH Relative Grant Channel

An E-DCH Relative Grant Channel (E-RGCH) is a fixed rate (SF=128) dedicated downlink physical channel carrying the uplink E-DCH relative grants for one uplink E-DCH associated with the E-RGCH by higher layer signalling. Figure 12A illustrates the structure of the E-RGCH. A relative grant is transmitted using 3, 12 or 15 consecutive slots and in each slot a sequence of 40 ternary values is transmitted. The 3 and 12 slot duration shall be used on an E-RGCH transmitted to UEs for which the cell transmitting the E-RGCH is in the E-DCH serving  radio link set and for which the E-DCH TTI is respectively 2 and 10 ms. The 15 slot duration shall be used on an E-RGCH transmitted to UEs for which the cell transmitting the E-RGCH is not in the E-DCH serving radio link set.

The sequence bi,0, bi,1, …, bi,39 transmitted in slot i in Figure 12A is given by bi,j = a Css,40,m(i),j. In a serving E-DCH radio link set, the relative grant a is set to +1, 0, or -1 and in a radio link not belonging to the serving E-DCH radio link set, the relative grant a is set to 0 or -1. The orthogonal signature sequences Css,40, m(i) is given by Table 16A and the index m(i) in slot i is given by Table 16B. The E-RGCH signature sequence index l in Table 16B is given by higher layers.

In case STTD-based open loop transmit diversity is applied for E-RGCH, STTD encoding according to subclause 5.3.1.1.1 is applied to the sequence bi,j.

Figure 12A: E-RGCH and E-HICH structure

Table 16A: E-RGCH and E-HICH signature sequences

| Css,40,0 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Css,40,1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 |
| Css,40,2 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| Css,40,3 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 |
| Css,40,4 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| Css,40,5 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 |
| Css,40,6 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| Css,40,7 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 |
| Css,40,8 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 |
| Css,40,9 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 |
| Css,40,10 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 |
| Css,40,11 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 |
| Css,40,12 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| Css,40,13 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 |
| Css,40,14 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| Css,40,15 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 |
| Css,40,16 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 |
| Css,40,17 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 |
| Css,40,18 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 |
| Css,40,19 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 |
| Css,40,20 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 |
| Css,40,21 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 |
| Css,40,22 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 |
| Css,40,23 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 |
| Css,40,24 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 |
| Css,40,25 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 |
| Css,40,26 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 |
| Css,40,27 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 |
| Css,40,28 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 |
| Css,40,29 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| Css,40,30 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 |
| Css,40,31 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 |
| Css,40,32 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 |
| Css,40,33 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| Css,40,34 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 |
| Css,40,35 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| Css,40,36 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 |
| Css,40,37 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 |
| Css,40,38 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 |
| Css,40,39 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 |

The bits are transmitted in order from left to right, i.e., column 2 corresponds to index j=0 and the rightmost column corresponds to index j=39.

Table 16B: E-HICH and E-RGCH signature hopping pattern

| Sequence index l | Row index m(i) for slot i |  |  |
| --- | --- | --- | --- |
|  | imod3=0 | imod3=1 | imod3=2 |
| 0 | 0 | 2 | 13 |
| 1 | 1 | 18 | 18 |
| 2 | 2 | 8 | 33 |
| 3 | 3 | 16 | 32 |
| 4 | 4 | 13 | 10 |
| 5 | 5 | 3 | 25 |
| 6 | 6 | 12 | 16 |
| 7 | 7 | 6 | 1 |
| 8 | 8 | 19 | 39 |
| 9 | 9 | 34 | 14 |
| 10 | 10 | 4 | 5 |
| 11 | 11 | 17 | 34 |
| 12 | 12 | 29 | 30 |
| 13 | 13 | 11 | 23 |
| 14 | 14 | 24 | 22 |
| 15 | 15 | 28 | 21 |
| 16 | 16 | 35 | 19 |
| 17 | 17 | 21 | 36 |
| 18 | 18 | 37 | 2 |
| 19 | 19 | 23 | 11 |
| 20 | 20 | 39 | 9 |
| 21 | 21 | 22 | 3 |
| 22 | 22 | 9 | 15 |
| 23 | 23 | 36 | 20 |
| 24 | 24 | 0 | 26 |
| 25 | 25 | 5 | 24 |
| 26 | 26 | 7 | 8 |
| 27 | 27 | 27 | 17 |
| 28 | 28 | 32 | 29 |
| 29 | 29 | 15 | 38 |
| 30 | 30 | 30 | 12 |
| 31 | 31 | 26 | 7 |
| 32 | 32 | 20 | 37 |
| 33 | 33 | 1 | 35 |
| 34 | 34 | 14 | 0 |
| 35 | 35 | 33 | 31 |
| 36 | 36 | 25 | 28 |
| 37 | 37 | 10 | 27 |
| 38 | 38 | 31 | 4 |
| 39 | 39 | 38 | 6 |

#### 5.3.2.5	E-DCH Hybrid ARQ Indicator Channel

An E-DCH Hybrid ARQ Indicator Channel (E-HICH) is a fixed rate (SF=128) dedicated downlink physical channel carrying the uplink E-DCH hybrid ARQ acknowledgement indicator for one uplink E-DCH.

The uplink E-DCH is associated with the E-HICH signature sequence(s) provided by higher layer signalling. Figure 12A illustrates the structure of the E-HICH. A hybrid ARQ acknowledgement indicator is transmitted using 3 or 12 consecutive slots and in each slot a sequence of 40 binary values is transmitted. The 3 and 12 slot duration shall be used for UEs which E-DCH TTI is set to respectively 2 ms and 10 ms.

The sequence bi,0, bi,1, …, bi,39 transmitted in slot i in Figure 12A is given by bi,j = a Css,40, m(i),j. In a radio link set containing the serving E-DCH radio link set, the hybrid ARQ acknowledgement indicator a is set to +1 or –1, and in a radio link set not containing the serving E-DCH radio link set the hybrid ARQ indicator a is set to +1 or 0. The orthogonal signature sequences Css,40,m(i) is given by Table 16A and the index m(i) in slot i is given by Table 16B. The E-HICH signature sequence index l is given by higher layers.

In case STTD-based open loop transmit diversity is applied for E-HICH, STTD encoding according to subclause 5.3.1.1.1 is applied to the sequence bi,j

If UL_MIMO_Enabled is TRUE, the UE is configured with a second E-HICH signature sequence associated with HARQ acknowledgments for the transport block transmitted on S-E-DPDCHs.

#### 5.3.2.6	Fractional Dedicated Physical Channel (F-DPCH)

An F-DPCH carries control information generated at layer 1 (TPC commands) for one uplink DPCCH associated with the F-DPCH by higher layer signalling. It is a special case of downlink DPCCH.

If DPCCH2 is configured, an additional F-DPCH carries control information generated at layer 1 (TPC commands) for one uplink DPCCH2 associated with the F-DPCH by higher layer signalling.

Figure 12B shows the frame structure of the F-DPCH. Each frame of length 10ms is split into 15 slots, each of length Tslot = 2560 chips, corresponding to one power-control period.

Figure 12B: Frame structure for F-DPCH

The exact number of bits of the OFF periods and of the F-DPCH fields (NTPC) is described in table 16C. Each slot format corresponds to a different set of OFF periods within the F-DPCH slot.

If DPCCH2 is configured, the slot format of F-DPCH associated with the DPCCH shall be different from the slot format of F-DPCH associated with the DPCCH2.

Table 16C: F-DPCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Slot | NOFF1 Bits/Slot | NTPC Bits/Slot | NOFF2 Bits/Slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1.5 | 256 | 20 | 2 | 2 | 16 |
| 1 | 3 | 1.5 | 256 | 20 | 4 | 2 | 14 |
| 2 | 3 | 1.5 | 256 | 20 | 6 | 2 | 12 |
| 3 | 3 | 1.5 | 256 | 20 | 8 | 2 | 10 |
| 4 | 3 | 1.5 | 256 | 20 | 10 | 2 | 8 |
| 5 | 3 | 1.5 | 256 | 20 | 12 | 2 | 6 |
| 6 | 3 | 1.5 | 256 | 20 | 14 | 2 | 4 |
| 7 | 3 | 1.5 | 256 | 20 | 16 | 2 | 2 |
| 8 | 3 | 1.5 | 256 | 20 | 18 | 2 | 0 |
| 9 | 3 | 1.5 | 256 | 20 | 0 | 2 | 18 |

In compressed frames, F-DPCH is not transmitted in downlink transmission gaps given by transmission gap pattern sequences signalled by higher layers.

The relationship between the TPC symbol and the transmitter power control command is according to table 13.

#### 5.3.2.7	Fractional Transmitted Precoding Indicator Channel (F-TPICH)

An F-TPICH carries transmitted precoding indicator generated at layer 1 for uplink CLTD operation.

Figure 12C shows the frame structure of the F-TPICH. Each frame of length 10ms is split into 15 slots, each of length Tslot = 2560 chips. A TPI is transmitted using the first two consecutive slots within one subframe, and in each slot one symbol is transmitted using the same slot format.

Figure 12C: Frame structure for F-TPICH

The exact number of bits of the OFF periods and of the F-TPICH fields (NTPI) is described in table 16D. Each slot format corresponds to a different set of OFF periods within the F-TPICH slot.

Table 16D: F-TPICH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Slot | NOFF1 Bits/Slot | NTPI Bits/Slot | NOFF2 Bits/Slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1.5 | 256 | 20 | 2 | 2 | 16 |
| 1 | 3 | 1.5 | 256 | 20 | 4 | 2 | 14 |
| 2 | 3 | 1.5 | 256 | 20 | 6 | 2 | 12 |
| 3 | 3 | 1.5 | 256 | 20 | 8 | 2 | 10 |
| 4 | 3 | 1.5 | 256 | 20 | 10 | 2 | 8 |
| 5 | 3 | 1.5 | 256 | 20 | 12 | 2 | 6 |
| 6 | 3 | 1.5 | 256 | 20 | 14 | 2 | 4 |
| 7 | 3 | 1.5 | 256 | 20 | 16 | 2 | 2 |
| 8 | 3 | 1.5 | 256 | 20 | 18 | 2 | 0 |
| 9 | 3 | 1.5 | 256 | 20 | 0 | 2 | 18 |

The relationship between TPI and bit pattern on F-TPICH is defined in [5].

For a given UE, the TPI feedback bits are not transmitted on F-TPICH bits that overlap with the TPC bits on the F-DPCH channel.

In compressed frames, if one of the two slots corresponding to the TPI indication overlaps with the downlink transmission gaps given by transmission gap pattern sequences signalled by higher layers, neither of the slots is transmitted.

### 5.3.3	Common downlink physical channels

#### 5.3.3.1	Common Pilot Channel (CPICH)

The CPICH is a fixed rate (30 kbps, SF=256) downlink physical channel that carries a pre-defined bit sequence. Figure 13 shows the frame structure of the CPICH.

Figure 13: Frame structure for Common Pilot Channel

In case transmit diversity is used on P-CCPCH and SCH, the CPICH shall be transmitted from both antennas using the same channelization and scrambling code. In this case, the pre-defined bit sequence of the CPICH is different for Antenna 1 and Antenna 2, see figure 14. In case of no transmit diversity, the bit sequence of Antenna 1 in figure 14 is used.

Figure 14: Modulation pattern for Common Pilot Channel

There are two types of Common pilot channels, the Primary and Secondary CPICH. They differ in their use and the limitations placed on their physical features.

##### 5.3.3.1.1	Primary Common Pilot Channel (P-CPICH)

The Primary Common Pilot Channel (P-CPICH) has the following characteristics:

-	The same channelization code is always used for the P-CPICH, see [4];

-	The P-CPICH is scrambled by the primary scrambling code, see [4];

-	There is one and only one P-CPICH per cell;

-	The P-CPICH is broadcast over the entire cell.

##### 5.3.3.1.2	Secondary Common Pilot Channel (S-CPICH)

A Secondary Common Pilot Channel (S-CPICH) has the following characteristics:

-	An arbitrary channelization code of SF=256 is used for the S-CPICH, see [4];

-	An S-CPICH is scrambled by either the primary or a secondary scrambling code, see [4];

-	There may be zero, one, or several S-CPICH per cell;

-	An S-CPICH may be transmitted over the entire cell or only over a part of the cell;

-	An S-CPICH that is intended to be used as phase reference for the second, third or fourth transmit antenna by UEs configured in MIMO mode or in MIMO mode with four transmit antennas shall be transmitted over the entire cell using the primary scrambling code and the antenna 1 pattern.

##### 5.3.3.1.3	Demodulation Common Pilot Channel (D-CPICH)

A Demodulation Common Pilot Channel (D-CPICH) has the following characteristics:

-	An arbitrary channelization code of SF=256 is used for the D-CPICH, see [4];

-	A D-CPICH is scrambled by the primary scrambling code see [4];

-	There may be zero or two D-CPICH per cell;

-	A D-CPICH shall be transmitted over the entire cell;

-	Carries a pre defined bit sequence same as that of Antenna 1 in Figure 14;

-	A D-CPICH is non precoded and is transmitted from third or fourth  transmit antenna;

-	The UE for which the two D-CPICH are activated may assume that the D-CPICHs are present in the HS-DSCH TTIs in which the UE is scheduled to receive HS-PDSCHs.

#### 5.3.3.2	Downlink phase reference

Table 17 specifies the channels which the UE may use as a phase reference for each downlink physical channel type in a cell; it also specifies whether the channels which the UE may use as a phase reference for a channel of a particular type shall be assumed to be the same as the ones which the UE may use as a phase reference for the associated DPCH or F-DPCH in the same cell, if configured.

For the DPCH or F-DPCH and the associated downlink physical channels the following always applies:

-	The UE may use the DPCH pilot bits as a phase reference.

-	In addition, the UE may use either the primary CPICH or a secondary CPICH as a phase reference.

-	By default (i.e. without any indication by higher layers) the UE may use the primary CPICH as a phase reference.

-	When a UE is not configured in MIMO mode and not in MIMO mode with four transmit antennas in a cell: The UE is informed by higher layers when it may use a secondary CPICH as a phase reference. In this case the UE shall not use the primary CPICH as a phase reference. Indication that a secondary CPICH may be a phase reference is also applicable when open loop or closed loop TX diversity is enabled for a downlink physical channel in which case Antenna 1 and Antenna 2 secondary CPICH shall be used as phase references.

-	When the UE is configured in MIMO mode in a cell: The UE is informed by higher layers when it may use a secondary CPICH as a phase reference for a second transmit antenna in addition to the primary CPICH which will be transmitted from the first antenna. In addition, if the UE supports open loop Tx diversity on associated physical channels with the combination of primary CPICH and secondary CPICH, this combination of phase references can also be used as phase references for associated physical channels configured with open loop Tx diversity. If the UE does not support open loop Tx diversity on associated physical channels with the combination of primary CPICH and secondary CPICH and if secondary CPICH is used as a phase reference for second transmit antenna, UE may assume that associated physical channels are not in Tx diversity mode. If no secondary CPICH is signalled as phase reference, the UE may use the Antenna 1 and Antenna 2 primary CPICH as phase references.

-	When the UE is configured in MIMO mode with four transmit antennas in a cell: The UE is informed by higher layers when it should use any of the secondary CPICH as a phase reference for a second, third or fourth transmit antenna in addition to the primary CPICH which will be transmitted from the first antenna. In addition, if the UE supports open loop Tx diversity on associated physical channels with the combination of primary CPICH and secondary CPICH on second transmit antenna, this combination of phase references can also be used as phase references for associated physical channels configured with open loop Tx diversity. If the UE does not support open loop Tx diversity on associated physical channels with the combination of primary CPICH and secondary CPICH, then the UE may assume that associated physical channels are not in Tx diversity mode.

Table 17: Phase references for downlink physical channel types in a cell
"X" – Applicable, "–" – Not applicable

| Physical channel type | DPCH Dedicated pilot (never as the sole phase reference) | Primary -CPICH | Secondary -CPICH | Demodulation-CPICH | Same as associated DPCH or F-DPCH |
| --- | --- | --- | --- | --- | --- |
| P-CCPCH | - | X | - | - | - |
| SCH | - | X | - | - | - |
| S-CCPCH | - | X | - | - | - |
| DPCH* | X | X | X | - | - |
| F-DPCH* | - | X | X | - | - |
| PICH | - | X | – | - | - |
| MICH | - | X | – | - | - |
| HS-PDSCH* (UE not in MIMO mode, not in MIMO mode with four transmit antennas and associated DPCH or F-DPCH is configured) | - | - | - | - | X |
| HS-PDSCH* (UE in MIMO mode) | - | X | X | - | - |
| HS-PDSCH** (UE in MIMO mode with four transmit antennas) | - | X | X | X | - |
| HS-PDSCH (if no associated DPCH or F-DPCH is configured) | - | X | - | - | - |
| HS-SCCH* (if associated DPCH or F-DPCH is configured) | - | - | - | - | X |
| HS-SCCH (if no associated DPCH or F-DPCH is configured) | - | X | - | - | - |
| E-AGCH* | - | - | - | - | X |
| E-RGCH* (if associated DPCH or F-DPCH is configured) | - | - | - | - | X |
| E-RGCH (if no associated DPCH or F-DPCH is configured) | - | X | - | - | - |
| E-HICH* | - | - | - | - | X |
| E-HICH (if no associated DPCH or F-DPCH is configured) | - | X | - | - | - |
| AICH | - | X | - | - | - |
| F-TPICH | - | - | - | - | X |

Note *:	A secondary CPICH should not be configured as a phase reference for DPCH or F-DPCH when a UE simultaneously receives S-CCPCHs on different radio links and DPCH or F-DPCH. The UE behavior is undefined if this configuration is used. The support for simultaneous reception of S-CCPCHs on different radio links and DPCH or F-DPCH is optional in the UE.

**:  When the UE is configured in MIMO mode with four transmit antennas, Secondary CPICHs on Antennas 2, 3, and 4 and Demodulation CPICHs on Antennas 3 and 4 are used as phase reference.

Dedicated pilot bits are never the sole phase reference for any physical channel, but the UE may always use dedicated pilot bits as a phase reference for DPCH.

Furthermore, during a DPCH or F-DPCH frame overlapping with any part of an associated HS-DSCH or HS-SCCH subframe, the phase reference on this DPCH or F-DPCH shall not change.

#### 5.3.3.3	Primary Common Control Physical Channel (P-CCPCH)

The Primary CCPCH is a fixed rate (30 kbps, SF=256) downlink physical channels used to carry the BCH transport channel.

Figure 15 shows the frame structure of the Primary CCPCH. The frame structure differs from the downlink DPCH in that no TPC commands, no TFCI and no pilot bits are transmitted. The Primary CCPCH is not transmitted during the first 256 chips of each slot. Instead, Primary SCH and Secondary SCH are transmitted during this period (see subclause 5.3.3.5).

Figure 15: Frame structure for Primary Common Control Physical Channel

##### 5.3.3.3.1	Primary CCPCH structure with STTD encoding

In case the diversity antenna is present in UTRAN and the P-CCPCH is to be transmitted using open loop transmit diversity, the data bits of the P-CCPCH are STTD encoded as given in subclause 5.3.1.1.1. The last two data bits in even numbered slots are STTD encoded together with the first two data bits in the following slot, except for slot #14 where the two last data bits are not STTD encoded and instead transmitted with equal power from both the antennas, see figure 16. Higher layers signal whether STTD encoding is used for the P-CCPCH or not. In addition the presence/absence of STTD encoding on P-CCPCH is indicated by modulating the SCH, see 5.3.3.4. During power on and hand over between cells the UE can determine the presence of STTD encoding on the P-CCPCH, by either receiving the higher layer message, by demodulating the SCH channel, or by a combination of the above two schemes.

Figure 16: STTD encoding for the data bits of the P-CCPCH

#### 5.3.3.4	Secondary Common Control Physical Channel (S-CCPCH)

The Secondary CCPCH is used to carry the FACH and PCH, and can also be configured to carry a BCH. There are two types of Secondary CCPCH: those that include TFCI and those that do not include TFCI. It is the UTRAN that determines if a TFCI should be transmitted, hence making it mandatory for all UEs to support the use of TFCI. The set of possible rates for the Secondary CCPCH is the same as for the downlink DPCH, see subclause 5.3.2. The frame structure of the Secondary CCPCH is shown in figure 17.

Figure 17: Frame structure for Secondary Common Control Physical Channel

The parameter k in figure 17 determines the total number of bits per downlink Secondary CCPCH slot. It is related to the spreading factor SF of the physical channel as SF = 256/2k. The spreading factor range is from 256 down to 4.

The values for the number of bits per field are given in Table 18. The channel bit and symbol rates given in Table 18 are the rates immediately before spreading. The slot formats applicable to QPSK with pilot bits are not supported in this release. The pilot patterns for the slot formats applicable to QPSK are given in Table 19. DTX shall be used in the pilot field of the 16QAM slot formats, i.e. no pilot bits are used in this release. A BCH mapped to Secondary CCPCH uses the specific fixed Secondary CCPCH slot format in Table 18, and this slot format can only be used for carrying BCH.

The FACH and PCH can be mapped to the same or to separate Secondary CCPCHs. If FACH and PCH are mapped to the same Secondary CCPCH, they can be mapped to the same frame. A BCH mapped to Secondary CCPCH can only be mapped to a separate Secondary CCPCH without multiplexing with other transport channels. The main difference between a CCPCH and a downlink dedicated physical channel is that a CCPCH is not inner-loop power controlled. The main difference between the Primary and Secondary CCPCH is that the transport channel mapped to the Primary CCPCH (BCH) can only have a fixed predefined transport format combination, while the Secondary CCPCH supports multiple transport format combinations on the transport channels mapped to it and offers the possibility of using TFCI.

Table 18: Secondary CCPCH fields

| Slot Format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ Frame | Bits/ Slot | Ndata1 | Npilot | NTFCI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 30 | 15 | 256 | 300 | 20 | 20 | 0 | 0 |
| 1 | 30 | 15 | 256 | 300 | 20 | 12 | 8 | 0 |
| 2 | 30 | 15 | 256 | 300 | 20 | 18 | 0 | 2 |
| 3 | 30 | 15 | 256 | 300 | 20 | 10 | 8 | 2 |
| 4 | 60 | 30 | 128 | 600 | 40 | 40 | 0 | 0 |
| 5 | 60 | 30 | 128 | 600 | 40 | 32 | 8 | 0 |
| 6 | 60 | 30 | 128 | 600 | 40 | 38 | 0 | 2 |
| 7 | 60 | 30 | 128 | 600 | 40 | 30 | 8 | 2 |
| 8 | 120 | 60 | 64 | 1200 | 80 | 72 | 0 | 8* |
| 9 | 120 | 60 | 64 | 1200 | 80 | 64 | 8 | 8* |
| 10 | 240 | 120 | 32 | 2400 | 160 | 152 | 0 | 8* |
| 11 | 240 | 120 | 32 | 2400 | 160 | 144 | 8 | 8* |
| 12 | 480 | 240 | 16 | 4800 | 320 | 312 | 0 | 8* |
| 13 | 480 | 240 | 16 | 4800 | 320 | 296 | 16 | 8* |
| 14 | 960 | 480 | 8 | 9600 | 640 | 632 | 0 | 8* |
| 15 | 960 | 480 | 8 | 9600 | 640 | 616 | 16 | 8* |
| 16 | 1920 | 960 | 4 | 19200 | 1280 | 1272 | 0 | 8* |
| 17 | 1920 | 960 | 4 | 19200 | 1280 | 1256 | 16 | 8* |
| 18*** | 60 | 15 | 256 | 600 | 40 | 36 | 0 | 4 |
| 19*** | 120 | 30 | 128 | 1200 | 80 | 76 | 0 | 4 |
| 20*** | 240 | 60 | 64 | 2400 | 160 | 144 | 0 | 16* |
| 21*** | 480 | 120 | 32 | 4800 | 320 | 272 | 32** | 16* |
| 22*** | 960 | 240 | 16 | 9600 | 640 | 560 | 64** | 16* |
| 23*** | 1920 | 480 | 8 | 19200 | 1280 | 1136 | 128** | 16* |
| BCH**** | 30 | 15 | 256 | 300 | 20 | 18 | 0 | 0 |

* If TFCI bits are not used, then DTX shall be used in TFCI field.

** If pilot bits are not used, then DTX shall be used in pilot field.

*** Slot formats applicable to 16QAM. See subclause 4.3.5.1.1 in [3] for mapping TFCI bits on 16QAM slot formats.

**** Slot format used for S-CCPCH carrying BCH. The first two data bits (256 chips) out of the 20 data bits in each slot are DTXed, resulting in a frame structure identical to P-CCPCH.

- NOTE 1: The slot formats 18 to 23 in Table 18 are only applicable for MBSFN operations with 16QAM.

- NOTE 2: The modulation used in MBSFN operations is signalled by higher layers.

The pilot symbol pattern described in Table 19 is not supported in this release. The shadowed part can be used as frame synchronization words. (The symbol pattern of pilot symbols other than the frame synchronization word shall be "11"). In Table 19, the transmission order is from left to right. (Each two-bit pair represents an I/Q pair of QPSK modulation.)

Table 19: Pilot Symbol Pattern

|  | Npilot = 8 |  |  |  | Npilot = 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 10 01 00 01 10 00 00 10 11 01 11 00 11 11 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 00 01 00 10 11 11 10 01 11 01 10 10 00 00 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 10 01 00 01 10 00 00 10 11 01 11 00 11 11 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 11 11 10 01 11 01 10 10 00 00 11 00 01 00 10 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 10 00 00 10 11 01 11 00 11 11 10 10 01 00 01 |

For slot formats using TFCI, the TFCI value in each radio frame corresponds to a certain transport format combination of the FACHs and/or PCHs currently in use. This correspondence is (re-)negotiated at each FACH/PCH addition / removal. The mapping of the TFCI bits onto slots is described in [3].

##### 5.3.3.4.1	Secondary CCPCH structure with STTD encoding

In case the diversity antenna is present in UTRAN and the S-CCPCH does not carry BCH and is to be transmitted using open loop transmit diversity, the data and TFCI bits of the S-CCPCH are STTD encoded as given in subclause 5.3.1.1.1. The pilot symbol pattern for antenna 2 for the S-CCPCH given in Table 20 is not supported in this release. If S-CCPCH carries BCH and open loop transmit diversity is to be applied, the STTD encoding is done the same way as for P-CCPCH as described in subclause 5.3.3.3.1.

Table 20: Pilot symbol pattern for antenna 2 when STTD encoding is used on the S-CCPCH

|  | Npilot = 8 |  |  |  | Npilot = 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol # | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Slot #0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 00 11 10 11 00 10 10 00 01 11 01 10 01 01 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 01 00 01 11 10 10 11 00 10 00 11 11 01 01 | 11 11 11 11 11 11 11 11 11 11 11 11 11 11 11 | 00 10 10 00 01 11 01 10 01 01 00 00 11 10 11 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 10 10 11 00 10 00 11 11 01 01 10 01 00 01 11 |

#### 5.3.3.5	Synchronisation Channel (SCH)

The Synchronisation Channel (SCH) is a downlink signal used for cell search. The SCH consists of two sub channels, the Primary and Secondary SCH. The 10 ms radio frames of the Primary and Secondary SCH are divided into 15 slots, each of length 2560 chips. Figure 18 illustrates the structure of the SCH radio frame.

Figure 18: Structure of Synchronisation Channel (SCH)

The Primary SCH consists of a modulated code of length 256 chips, the Primary Synchronisation Code (PSC) denoted cp in figure 18, transmitted once every slot. The PSC is the same for every cell in the system.

The Secondary SCH consists of repeatedly transmitting a length 15 sequence of modulated codes of length 256 chips, the Secondary Synchronisation Codes (SSC), transmitted in parallel with the Primary SCH. The SSC is denoted csi,k in figure 18, where i = 0, 1, …, 63 is the number of the scrambling code group, and k = 0, 1, …, 14 is the slot number. Each SSC is chosen from a set of 16 different codes of length 256. This sequence on the Secondary SCH indicates which of the code groups the cell's downlink scrambling code belongs to.

The primary and secondary synchronization codes are modulated by the symbol a shown in figure 18, which indicates the presence/ absence of STTD encoding on the P-CCPCH and is given by the following table:

| P-CCPCH STTD encoded | a = +1 |
| --- | --- |
| P-CCPCH not STTD encoded | a = -1 |

##### 5.3.3.5.1	SCH transmitted by TSTD

Figure 19 illustrates the structure of the SCH transmitted by the TSTD scheme. In even numbered slots both PSC and SSC are transmitted on antenna 1, and in odd numbered slots both PSC and SSC are transmitted on antenna 2.

Figure 19: Structure of SCH transmitted by TSTD scheme

#### 5.3.3.6	Void

#### 5.3.3.7	Acquisition Indicator Channel (AICH)

The Acquisition Indicator channel (AICH) is a fixed rate (SF=256) physical channel used to carry Acquisition Indicators (AI) and Extended Acquisition Indicators (EAI). Acquisition Indicator AIs corresponds to signature s on the PRACH. Extended Acquisition Indicators represent a set of values corresponding to a set of E-DCH resource configurations.

Figure 21 illustrates the structure of the AICH. The AICH consists of a repeated sequence of 15 consecutive access slots (AS), each of length 5120 chips. Each access slot consists of two parts, an Acquisition-Indicator (AI) part consisting of 32 real-valued signals a0, …, a31 and a part of duration 1024 chips with no transmission that is not formally part of the AICH. The part of the slot with no transmission is reserved for possible future use by other physical channels.

The spreading factor (SF) used for channelisation of the AICH is 256.

The phase reference for the AICH is the Primary CPICH.

Figure 21: Structure of Acquisition Indicator Channel (AICH)

The real-valued signals a0, a1, …, a31 in figure 21 are given by

where AIs, taking the values +1, -1, and 0, is the acquisition indicator corresponding to signature s and the sequence bs,0, …, bs,31 is given by Table 22, EAIs', taking the values +1, -1, and 0, is the extended acquisition indicator corresponding to signature s' and the sequence cs',0, …, cs',31 is given by Table 22B. The EAIs' has the same relative transmit power as the AIs; the relative transmit power is indicated by higher layers. If the signature s is not a member of the set of available signatures for all the Access Service Class (ASC) for the corresponding PRACH (cf [5]) then AIs shall be set to 0.

The use of acquisition indicators is described in [5]. The meaning of acquisition indicators depends on whether a UE sends an access preamble signature corresponding to a PRACH message or corresponding to an E-DCH transmission. Furthermore, if a UE sends an access preamble signature corresponding to an E-DCH transmission, the meaning of the acquisition indicator depends on whether EAI is configured or not. The following rules apply for one PRACH preamble scrambling code. If multiple PRACH preamble scrambling codes are defined in a cell, then for each of them the following rules are used independently.

-	If the UE sends an access preamble signature corresponding to a PRACH message, then;

-	if an Acquisition Indicator is set to +1, it represents a positive acknowledgement,

-	if an Acquisition Indicator is set to -1, it represents a negative acknowledgement.

-	If the UE sends an access preamble signature corresponding to an E-DCH transmission, then;

-	if the corresponding Acquisition Indicator is set to +1, it represents a positive acknowledgement and the associated default E-DCH resource configuration is allocated to the UE,

-	if the corresponding Acquisition Indicator is set to -1 and EAI is not configured, then it represents a negative acknowledgement,

-	if the corresponding Acquisition Indicator is set to -1 and EAI is configured, then the UE detects which one of the Extended Acquisition Indicator signatures is present.

The association between the AI and the default E-DCH resource index is such that, for 10 ms TTI length:

X = SigInd mod Y,

The association between the AI and the default E-DCH resource index is such that, for 2 ms TTI length:

X = (SigInd mod (Y - Concurrent TTI partition Index)) + Concurrent TTI partition Index,

where X is the Default E-DCH resource index, "Concurrent TTI partition Index" is signalled by higher layers or otherwise is set to zero, Y is the total number of E-DCH resources configured in the cell for Enhanced Uplink in CELL_FACH state and IDLE mode, and SigInd is the Nth PRACH preamble signature corresponding to the AI that is configured available in the cell and corresponding to E-DCH transmission for Enhanced Uplink in CELL_FACH state and IDLE mode. PRACH preamble signatures are renumbered as specified in [17] subclause 8.6.6.29.

The use of Extended Acquisition Indicators is described in [5]. If s'=0 and an EAI0 is set to +1, it represents a negative acknowledgement. The mapping between the non-zero EAIs' and the E-DCH resource configuration index is presented in Table 22A where X is the index of the default E-DCH resource as defined above and Y is the total number of E-DCH resources configured in the cell.

The real-valued signals, aj, are spread and modulated in the same fashion as bits when represented in {+1, -1 } form.

In case STTD-based open-loop transmit diversity is applied to AICH, STTD encoding according to subclause 5.3.1.1.1 is applied to each sequence bs,0, bs,1, ..., bs,31 separately and sequence cs',0, cs',1, …, cs',31 before the sequences are combined into AICH signals a0, …, a31.

Table 22: AI signature patterns

| s | bs,0, bs,1…, bs,31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 2 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 3 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 |
| 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 5 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| 6 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 |
| 7 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 9 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| 10 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 |
| 11 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 13 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 14 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 15 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 |

Table 22A: EAI and resource configuration mapping

| EAIs' | Signature s' | E-DCH Resource configuration index |
| --- | --- | --- |
| +1 | 0 | NACK |
| -1 |  | (X + 1) mod Y |
| +1 | 1 | (X + 2) mod Y |
| -1 |  | (X + 3) mod Y |
| +1 | 2 | (X + 4) mod Y |
| -1 |  | (X + 5) mod Y |
| +1 | 3 | (X + 6) mod Y |
| -1 |  | (X + 7) mod Y |
| +1 | 4 | (X + 8) mod Y |
| -1 |  | (X + 9) mod Y |
| +1 | 5 | (X + 10) mod Y |
| -1 |  | (X + 11) mod Y |
| +1 | 6 | (X + 12) mod Y |
| -1 |  | (X + 13) mod Y |
| +1 | 7 | (X + 14) mod Y |
| -1 |  | (X + 15) mod Y |
| +1 | 8 | (X + 16) mod Y |
| -1 |  | (X + 17) mod Y |
| +1 | 9 | (X + 18) mod Y |
| -1 |  | (X + 19) mod Y |
| +1 | 10 | (X + 20) mod Y |
| -1 |  | (X + 21) mod Y |
| +1 | 11 | (X + 22) mod Y |
| -1 |  | (X + 23) mod Y |
| +1 | 12 | (X + 24) mod Y |
| -1 |  | (X + 25) mod Y |
| +1 | 13 | (X + 26) mod Y |
| -1 |  | (X + 27) mod Y |
| +1 | 14 | (X + 28) mod Y |
| -1 |  | (X + 29) mod Y |
| +1 | 15 | (X + 30) mod Y |
| -1 |  | (X + 31) mod Y |

Table 22B: EAI signature patterns

| s' | cs',0, cs',1…, cs',31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 2 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 3 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 4 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 5 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 6 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 7 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 |
| 8 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 9 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 10 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 11 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 |
| 12 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 13 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 14 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 15 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 |

#### 5.3.3.8	Void

#### 5.3.3.9	Void

#### 5.3.3.10	Paging Indicator Channel (PICH)

The Paging Indicator Channel (PICH) is a fixed rate (SF=256) physical channel used to carry the paging indicators. The PICH is associated either with an S-CCPCH to which a PCH transport channel is mapped, or with a HS-SCCH associated with the HS-PDSCH(s) to which a HS-DSCH transport channel carrying paging messages is mapped.

Figure 24 illustrates the frame structure of the PICH. One PICH radio frame of length 10 ms consists of 300 bits (b0, b1, …, b299). Of these, 288 bits (b0, b1, …, b287) are used to carry paging indicators. The remaining 12 bits are not formally part of the PICH and shall not be transmitted (DTX). The part of the frame with no transmission is reserved for possible future use.

Figure 24: Structure of Paging Indicator Channel (PICH)

In each PICH frame, Np paging indicators {P0, …, PNp-1} are transmitted, where Np=18, 36, 72, or 144.

The PI calculated by higher layers for use for a certain UE, is associated to the paging indicator Pq, where q is computed as a function of the PI computed by higher layers, the SFN of the P-CCPCH radio frame during which the start of the PICH radio frame occurs, and the number of paging indicators per frame (Np):

Further, the PI calculated by higher layers is associated with the value of the paging indicator Pq. If a paging indicator in a certain frame is set to "1" it is an indication that UEs associated with this paging indicator and PI should read either the corresponding frame of the associated S-CCPCH, or the corresponding subframes of the associated HS-SCCH

The PI bitmap in the PCH data frames over Iub contains indication values for all higher layer PI values possible. Each bit in the bitmap indicates if the paging indicator associated with that particular PI shall be set to 0 or 1. Hence, the calculation in the formula above is to be performed in Node B to make the association between PI and Pq.

The mapping from {P0, …, PNp-1} to the PICH bits {b0, …, b287} are according to Table 24.

Table 24: Mapping of paging indicators Pq to PICH bits

| Number of paging indicators per frame (Np) | Pq = 1 | Pq = 0 |
| --- | --- | --- |
| Np=18 | {b16q, …, b16q+15} = {1, 1,…, 1} | {b16q, …, b16q+15} = {0, 0,…, 0} |
| Np=36 | {b8q, …, b8q+7} = {1, 1,…, 1} | {b8q, …, b8q+7} = {0, 0,…, 0} |
| Np=72 | {b4q, …, b4q+3} = {1, 1,…, 1} | {b4q, …, b4q+3} = {0, 0,…, 0} |
| Np=144 | {b2q, b2q+1} = {1, 1} | {b2q, b2q+1} = {0, 0} |

When transmit diversity is employed for the PICH, STTD encoding is used on the PICH bits as described in subclause 5.3.1.1.1.

#### 5.3.3.11	Void

#### 5.3.3.12	Shared Control Channel (HS-SCCH)

The HS-SCCH is a fixed rate (60 kbps, SF=128) downlink physical channel used to carry downlink signalling related to HS-DSCH transmission. Figure 26A illustrates the sub-frame structure of the HS-SCCH.

Figure 26A: Subframe structure for the HS-SCCH

#### 5.3.3.13	High Speed Physical Downlink Shared Channel (HS-PDSCH)

The High Speed Physical Downlink Shared Channel (HS- PDSCH) is used to carry the High Speed Downlink Shared Channel (HS-DSCH).

A HS-PDSCH corresponds to one channelization code of fixed spreading factor SF=16 from the set of channelization codes reserved for HS-DSCH transmission.  Multi-code transmission is allowed, which translates to UE being assigned multiple channelisation codes in the same HS-PDSCH subframe, depending on its UE capability.

The subframe and slot structure of HS-PDSCH are shown in figure 26B.

Figure 26B: Subframe structure for the HS-PDSCH

An HS-PDSCH may use QPSK, 16QAM or 64QAM modulation symbols. In figure 26B, M is the number of bits per modulation symbols i.e. M=2 for QPSK, M=4 for 16QAM and M=6 for 64QAM. The slot formats are shown in table 26.

Table 26: HS-DSCH fields

| Slot format #i | Channel Bit Rate (kbps) | Channel Symbol Rate (ksps) | SF | Bits/ HS-DSCH subframe | Bits/ Slot | Ndata |
| --- | --- | --- | --- | --- | --- | --- |
| 0(QPSK) | 480 | 240 | 16 | 960 | 320 | 320 |
| 1(16QAM) | 960 | 240 | 16 | 1920 | 640 | 640 |
| 2(64QAM) | 1440 | 240 | 16 | 2880 | 960 | 960 |

All relevant Layer 1 information is transmitted in the associated HS-SCCH i.e. the HS-PDSCH does not carry any Layer 1 information.

#### 5.3.3.14	E–DCH Absolute Grant Channel (E-AGCH)

An E-DCH Absolute Grant Channel (E-AGCH) is a fixed rate (30 kbps, SF=256) downlink physical channel carrying uplink E-DCH absolute grants for uplink E-DCHs associated with the E-AGCH by higher layer signalling. Figure 26C illustrates the frame and sub-frame structure of the E-AGCH.

An E-DCH absolute grant shall be transmitted over one E-AGCH sub-frame or one E-AGCH frame. The transmission over one E-AGCH sub-frame and over one E-AGCH frame shall be used for UEs for which E-DCH TTI is set to respectively 2 ms and 10 ms.

Figure 26C: Sub-frame structure for the E-AGCH

#### 5.3.3.14B	E-DCH Rank and Offset Channel (E-ROCH)

An E-DCH Rank and Offset Channel (E-ROCH) has the same sub-frame structure as the E-AGCH defined in section 5.3.3.14. The transmission shall always take place over one E-ROCH sub-frame. The E-ROCH is only transmitted to a UE for which the E-DCH TTI is set to 2 ms and the UL_MIMO_Enabled is set to TRUE.

The E-ROCH and E-AGCH can only be transmitted simultaneously to a UE if they are configured with different channelization codes.

#### 5.3.3.15	MBMS Indicator Channel (MICH)

The MBMS Indicator Channel (MICH) is a fixed rate (SF=256) physical channel used to carry the MBMS notification indicators. The MICH is always associated with an S-CCPCH to which a FACH transport channel is mapped.

Figure 26D illustrates the frame structure of the MICH. One MICH radio frame of length 10 ms consists of 300 bits (b0, b1, …, b299). Of these, 288 bits (b0, b1, …, b287) are used to carry notification indicators. The remaining 12 bits are not formally part of the MICH and shall not be transmitted (DTX).

Figure 26D: Structure of MBMS Indicator Channel (MICH)

In each MICH frame, Nn notification indicators {N0, …, NNn-1} are transmitted, where Nn=18, 36, 72, or 144.

The NI calculated by higher layers is associated to the index q of the notification indicator Nq, where q is computed as a function of the NI computed by higher layers, the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs, and the number of notification indicators per frame (Nn):

q=C×NI⊕C×SFNmodGmodG×NnG

where G = 216 , C = 25033 and NI is the 16 bit Notification Indicator calculated by higher layers.

The set of NI signalled over Iub indicates all higher layer NI values for which the associated notification indicator on MICH shall be set to 1 during the corresponding modification period. Hence, the calculation in the formula above shall be performed in the Node B every MICH frame for each NI signalled over Iub to make the association between NI and q and set the related Nq to 1. All other notification indicators on MICH shall be set to 0.

The mapping from {N0, …, NNn-1} to the MICH bits {b0, …, b287} are according to table 27.

Table 27: Mapping of notification indicators Nq to MICH bits

| Number of notification indicators per frame (Nn) | Nq = 1 | Nq = 0 |
| --- | --- | --- |
| Nn=18 | {b16q, …, b16q+15} = {1, 1,…, 1} | {b16q, …, b16q+15} = {0, 0,…, 0} |
| Nn=36 | {b8q, …, b8q+7} = {1, 1,…, 1} | {b8q, …, b8q+7} = {0, 0,…, 0} |
| Nn=72 | {b4q, …, b4q+3} = {1, 1,…, 1} | {b4q, …, b4q+3} = {0, 0,…, 0} |
| Nn=144 | {b2q, b2q+1} = {1, 1} | {b2q, b2q+1} = {0, 0} |

When transmit diversity is employed for the MICH, STTD encoding is used on the MICH bits as described in subclause 5.3.1.1.1.

#### 5.3.3.16	Common E-DCH Relative Grant Channel

The Common E-RGCH is a downlink physical channel for UEs in CELL_FACH state as described in sub clause 6B.2 of [5]. There is no DPCH or F-DPCH associated with the Common E-RGCH.

The structure, STTD encoding and timing relationship of the Common E-RGCH follow the rules of the dedicated E-RGCH for which the cell transmitting the E-RGCH is not in the E-DCH serving radio link set (sub clause 5.3.2.4 and 7.11).

# 6	Mapping and association of physical channels

## 6.1	Mapping of transport channels onto physical channels

Figure 27 summarises the mapping of transport channels onto physical channels.

Figure 27: Transport-channel to physical-channel mapping

The DCHs are coded and multiplexed as described in [3], and the resulting data stream is mapped sequentially (first-in-first-mapped) directly to the physical channel(s). The mapping of BCH and FACH/PCH is equally straightforward, where the data stream after coding and interleaving is mapped sequentially to the CCPCHs. A BCH can be mapped to either Primary CCPCH or Secondary CCPCH, while FACH/PCH is always mapped to Secondary CCPCH. Also for the RACH, the coded and interleaved bits are sequentially mapped to the physical channel, in this case the message part of the PRACH. The E-DCH is coded as described in [3], and the resulting data stream is mapped sequentially (first-in-first-mapped) directly to the physical channel(s).

## 6.2	Association of physical channels and physical signals

Figure 28 illustrates the association between physical channels and physical signals.

Figure 28: Physical channel and physical signal association

# 7	Timing relationship between physical channels

## 7.1	General

The P-CCPCH, on which the cell SFN is transmitted, is used as timing reference for all the physical channels, directly for downlink and indirectly for uplink.

Figure 29 describes the frame timing of some of the downlink physical channels; the timing of the remaining downlink physical channels and of the uplink physical channels is specified in the remaining subclauses. For the AICH the access slot timing is included. Transmission timing for uplink physical channels is given by the received timing of downlink physical channels.

Figure 29: Radio frame timing and access slot timing of downlink physical channels

The following applies:

-	SCH (primary and secondary), CPICH (primary and secondary and demodulation CPICHs) and P-CCPCH have identical frame timings.

-	The S-CCPCH timing may be different for different S-CCPCHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. S-CCPCH,k = Tk  256 chip, Tk  {0, 1, …, 149}. For S-CCPCH carrying BCH Tk = 0. For MBSFN operations using slot formats 21 to 23 in table 18, the offset shall be set in accordance with S-CCPCH,k = 256 +  Tk /10.  2560 chip.

-	If the PICH is associated to the S-CCPCH, the PICH timing is PICH = 7680 chips prior to its corresponding S-CCPCH frame timing, i.e. the timing of the S-CCPCH carrying the PCH transport channel with the corresponding paging information, see also subclause 7.2. If the PICH is associated to the HS-SCCH, the PICH frame timing is the same as the HS-SCCH frame timing.

-	AICH access slots #0 starts the same time as P-CCPCH frames with (SFN modulo 2) = 0. The AICH/PRACH timing is described in subclauses 7.3 and 7.4 respectively.

-	The DPCH timing may be different for different DPCHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. DPCH,n = Tn  256 chip, Tn  {0, 1, …, 149}. The DPCH (DPCCH/DPDCH) timing relation with uplink DPCCH/DPDCHs is described in subclause 7.6.

-	The F-DPCH timing may be different for different F-DPCHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. F-DPCH,p = Tp  256 chip, Tp  {0, 1, …, 149}. All F-DPCHs transmitted to a UE from the same HS-DSCH cell set have the same timing. The F-DPCH timing relation with uplink DPCCH/DPDCHs is described in subclause 7.6.

-	In case dual-cell E-DCH is configured together with DPDCH, DPCH (associated with the primary uplink) and F-DPCH (associated with the secondary uplink) transmitted to a UE from the same HS-DSCH cell set have the same timing.

-	In the case that DPCCH2 is configured, the F-DPCH associated with the uplink DPCCH2 has the same timing as DPCH or F-DPCH associated with the uplink DPCCH.

-	The start of HS-SCCH subframe #0 is aligned with the start of the P-CCPCH frames. The relative timing between a HS-PDSCH and the corresponding HS-SCCH is described in subclause 7.8.

-	The E-HICH, E-RGCH, E-AGCH and E-ROCH downlink timing are respectively specified in subclause 7.10, 7.11, 7.12 and 7.12A. The E-DPCCH and E-DPDCH uplink timing are specified in subclause 7.13.

-	For a secondary serving HS-DSCH cell, the nominal radio frame timing for CPICH and timing reference are the same as the radio frame timing for CPICH and timing reference for the serving HS-DSCH cell.

-	The F-TPICH timing may be different for different F-TPICHs, but the offset from the P-CCPCH frame timing is a multiple of 256 chips, i.e. F-TPICH,m = Tm  256 chip, Tm  {0, 1, …, 149}.

## 7.2	PICH/S-CCPCH timing relation

Figure 30 illustrates the timing between a PICH frame and its associated single S-CCPCH frame, i.e. the S-CCPCH frame that carries the paging information related to the paging indicators in the PICH frame. A paging indicator set in a PICH frame means that the paging message is transmitted on the PCH in the S-CCPCH frame starting PICH chips after the transmitted PICH frame. PICH is defined in subclause 7.1.

Figure 30: Timing relation between PICH frame and associated S-CCPCH frame

## 7.2A	PICH/HS-SCCH timing relation

Figure 30a illustrates the timing between a PICH frame and its set of 5 associated HS-SCCH subframes. The first associated subframe of the associated HS-SCCH starts one HS-SCCH sub-frame after the transmitted PICH frame and is the HS-SCCH subframe number 1 as defined in subclause 7.1. A paging indicator set in a PICH frame means that one or more HS-DSCH subframes may be transmitted to the UE on the HS-PDSCH(s) associated with the HS-SCCH subframes associated with the PICH as defined in [5].

Figure 30a: Timing relation between PICH frame and associated HS-SCCH subframes

## 7.3	PRACH/AICH timing relation

The downlink AICH is divided into downlink access slots, each access slot is of length 5120 chips. The downlink access slots are time aligned with the P-CCPCH as described in subclause 7.1.

The uplink PRACH is divided into uplink access slots, each access slot is of length 5120 chips. Uplink access slot number n is transmitted from the UE p-a chips prior to the reception of downlink access slot number n, n = 0, 1, …, 14.

Transmission of downlink acquisition indicators may only start at the beginning of a downlink access slot. Similarly, transmission of uplink RACH preambles and RACH message parts may only start at the beginning of an uplink access slot.

The PRACH/AICH timing relation is shown in figure 31.

Figure 31: Timing relation between PRACH and AICH as seen at the UE

The preamble-to-preamble distance p-p shall be larger than or equal to the minimum preamble-to-preamble distance
p-p,min, i.e. p-p  p-p,min.

In addition to p-p,min, the preamble-to-AI distance p-a and preamble-to-message distance p-m are defined as follows:

-	when AICH_Transmission_Timing is set to 0, then

p-p,min = 15360 chips (3 access slots)

p-a = 7680 chips

p-m = 15360 chips (3 access slots)

-	when AICH_Transmission_Timing is set to 1, then

p-p,min = 20480 chips (4 access slots)

p-a = 12800 chips

p-m = 20480 chips (4 access slots)

The parameter AICH_Transmission_Timing is signalled by higher layers.

## 7.3A	UL/DL timing relation for Enhanced Uplink in CELL_FACH state and IDLE mode

Figure 31A: UL/DL timing relation for Enhanced Uplink in CELL_FACH state and IDLE mode as seen at the UE

The PRACH preamble and Acquisition indication timings p-p and p-a are as defined in section 7.3.

If Coffset is not configured by higher layers or the TTI length is 10ms, then

F-DPCH slot format number (according to Table 16C) = 0

F-DPCH = [(5120 * AICH access slot # with the AI) + 10240 + 256 * Soffset] mod 38400

a-m = 10240 + 256 * Soffset + 0 chips, where

Soffset = a symbol offset, configured by higher layers, {0,…,9}.

0 = 1024 chips defining the DL to UL frame timing difference.

If Coffset is configured by higher layers and the TTI length is 2ms, then

F-DPCH slot format number (according to Table 16C) = (Soffset - Coffset) mod 10

F-DPCH = [(5120 * AICH access slot # with the AI) + 10240 + 2560 * Toffset + 256 * Coffset] mod 38400

a-m = 10240 + 2560 * Toffset + 256 * Coffset + 0 chips, where

Toffset=0, if the preamble was transmitted in access slot 0, 3, ..., 121, if the preamble was transmitted in access slot 1, 4, ..., 132, if the preamble was transmitted in access slot 2, 5, ..., 143, if the preamble was transmitted in access slot 0, 3, ..., 12using an even preamble signature 0, 2, ..., 14using an odd preamble signature 1, 3,  ..., 15
Coffset = a cell-specific symbol offset, configured by higher layers, {0, …, 29}

## 7.4	Void

## 7.5	Void

## 7.6	DPCCH/DPDCH timing relations

### 7.6.1	Uplink

In uplink all the DPCCHs and all the DPDCHs transmitted from one UE have the same frame timing.

### 7.6.2	Downlink

In downlink, the DPCCH and all the DPDCHs carrying CCTrCHs of dedicated type to one UE have the same frame timing.

Note:	support of multiple CCTrChs of dedicated type is not part of the current release.

### 7.6.3	Uplink/downlink timing at UE

At the UE, the uplink DPCCH/DPDCH frame transmission takes place approximately T0 chips after the reception of the first detected path (in time) of the corresponding downlink DPCCH/DPDCH or F-DPCH frame. T0 is a constant defined to be 1024 chips. The first detected path (in time) is defined implicitly by the relevant tests in [14]. More information about the uplink/downlink timing relation and meaning of T0 can be found in [5].

## 7.7	Uplink DPCCH/HS-DPCCH/HS-PDSCH timing at the UE

### 7.7.1	Timing when Multiflow is not configured

Figure 34 shows the timing offset between the uplink DPCH, the HS-PDSCH and the HS-DPCCH at the UE. An HS-DPCCH sub-frame starts m×256 chips after the start of an uplink DPCH frame that corresponds to the DL DPCH or F-DPCH frame from the HS-DSCH serving cell containing the beginning of the related HS-PDSCH subframe with m calculated as

m = (TTX_diff /256 ) + 101

where TTX_diff is the difference in chips (TTX_diff =0, 256, ....., 38144), between

-	the transmit timing of the start of the related HS-PDSCH subframe (see sub-clauses 7.8 and 7.1)

and

-	the transmit timing of the start of the downlink DPCH or F-DPCH frame from the HS-DSCH serving cell that contains the beginning of the HS-PDSCH subframe (see sub-clause 7.1).

At any one time, m therefore takes one of a set of five possible values according to the transmission timing of HS-DSCH sub-frame timings relative to the DPCH or F-DPCH frame boundary. The UE and Node B shall only update the set of values of m in connection to UTRAN reconfiguration of downlink timing.

More information about uplink timing adjustments can be found in [5].

Figure 34: Timing structure at the UE for HS-DPCCH control signalling

### 7.7.2	Timing when Multiflow is configured

The uplink DPCH, the HS-PDSCH of the time reference cell, and uplink HS-DPCCH follow the same time as defined for the non-Multiflow case in section 7.7.1.

The timing relationship between the non-time reference cell's HS-PDSCH subframe and the related HS-DPCCH subframe is derived from HS-PDSCH frame timing difference DIFF of the two cells, where -20 ≤ DIFF ≤ 3860 chips. The time reference cell for HS-DPCCH is indicated by higher layers.

If DIFF is not within the specified limits:

-	HARQ-ACK reporting for the time-reference cell and CQI reporting for both cells are not interrupted.

-	The UE is not required to transmit a valid HARQ-ACK for the non-time reference cell.

If the UE is not configured in MIMO mode in any cell, or it is configured in MIMO mode in at least one cell and the UE indicated not requiring a Longer HARQ Processing Time in higher layer capability signalling, then:

Figure 34a shows the timing offset between the uplink DPCH, the HS-PDSCHs and the HS-DPCCH at the UE when the UE is configured with Multiflow. In this case τDIFF = τnon-time reference cell - τtime reference cell.

If the UE is configured in MIMO mode in at least one cell and the UE indicated requiring a Longer HARQ processing Time in higher layer capability signalling, then:

Figure 34b shows the timing offset between the uplink DPCH, the HS-PDSCHs and the HS-DPCCH at the UE when the UE is configured with Multiflow. In this case τDIFF = τtime reference cell - τ non-time reference cell.

τtime reference cell and τnon-time reference cell above refer to the end times of the HS-PDSCH subframes of the time reference cell and the non-time reference cell respectively, with the corresponding HARQ-ACK fields in the same HS-DPCCH sub-frame.

Figure 34a: Timing structure at the UE for HS-DPCCH control signalling with Multiflow, when the UE is not configured in MIMO mode, as well as when the UE is configured in MIMO mode in any cell and the UE indicated not requiring a Longer HARQ processing Time in higher layers

Figure 34b: Timing of delayed HARQ-ACK with Multiflow, when the UE is configured in MIMO mode in any cell, and the UE indicated requiring a Longer HARQ processing Time in higher layers

TTX_diff1 corresponds to the smallest TTX_diff value of the time reference cell as configured by higher layers. One TTX_diff value of the non-time reference cell shall be selected, denoted as TTXdiff2,, where the associated HS-PDSCH subframe of the time reference cell with TTX_diff1 and the associated HS-PDSCH subframe of the non-time reference cell with TTXdiff2,, correspond to the same HARQ-ACK field in the same HS-DPCCH sub-frame. TTX_diff2 is defined as in Table 28.

Table 28: Definition of TTX_diff2 for the non-time reference cell

| Condition: | TTX_diff2 defined as: |
| --- | --- |
| When TTXdiff2,<2∗7680 | TTXdiff2, |
| When TTXdiff2,≥2∗7680 | TTXdiff2,−38400 |

## 7.8	HS-SCCH/HS-PDSCH timing

Figure 35 shows the relative timing between the HS-SCCH and the associated HS-PDSCH for one HS-DSCH sub-frame. The HS-PDSCH starts HS-PDSCH = 2Tslot = 5120 chips after the start of the HS-SCCH.

Figure 35: Timing relation between the HS-SCCH and the associated HS-PDSCH.

## 7.9	MICH/S-CCPCH timing relation

Figure 36 illustrates the timing between the MICH frame boundaries and the frame boundaries of the associated S-CCPCH, i.e. the S-CCPCH that carries the MBMS control information related to the notification indicators in the MICH frame. The MICH transmission timing shall be such that the end of radio frame boundary occurs MICH chips before the associated S-CCPCH start of radio frame boundary. MICH is equal to 7680 chips.

The MICH frames during which the Node B shall set specific notification indicators and the S-CCPCH frames during which the Node B shall transmit the corresponding MBMS control data is defined by higher layers.

- Figure 36: Timing relation between MICH frame and associated S-CCPCH frame

## 7.10	E-HICH/P-CCPCH/DPCH timing relation

The timing of the E-HICH relative to the P-CCPCH is illustrated in figure 37.

When the E-DCH TTI is 10 ms the E-HICH frame offset relative to P-CCPCH shall be E-HICH,n chips with

τE−HICH,n=5120+7680×τDPCH,n/256−7030

When the E-DCH TTI is 2 ms the E-HICH frame offset relative to P-CCPCH shall be E-HICH,n chips with

τE−HICH,n=5120+7680×τDPCH,n/256+5030

When a downlink F-DPCH is configured, τDPCH,n=τF−DPCH,n.

Figure 37: E-HICH timing

## 7.11	E-RGCH/P-CCPCH/DPCH timing relation

The timing of the E-RGCH relative to the P-CCPCH is illustrated in figure 38.

When transmitted to a UE for which the cell transmitting the E-RGCH is in the E-DCH serving  radio link set, the E-RGCH frame offset shall be as follows:

-	if the E-DCH TTI is 10 ms, the E-RGCH frame offset relative to P-CCPCH shall be E-RGCH,n chips with

τE−RGCH,n=5120+7680×τDPCH,n/256−7030

-	if the E-DCH TTI is 2 ms the E-RGCH frame offset relative to P-CCPCH shall be E-RGCH,n chips with

τE−RGCH,n=5120+7680×τDPCH,n/256+5030

When a downlink F-DPCH is configured, τDPCH,n=τF−DPCH,n.

When transmitted to a UE for which the cell transmitting the E-RGCH is not in the E-DCH serving radio link set, the E-RGCH frame offset relative to P-CCPCH shall be E-RGCH = 5120 chips.

Figure 38: E-RGCH timing

## 7.12	E-AGCH/P-CCPCH timing relation

The E-AGCH frame offset relative to P-CCPCH shall be E-AGCH = 5120 chips as illustrated in figure 39.

Figure 39: E-AGCH timing

## 7.12A	E-ROCH/P-CCPCH timing relation

The E-ROCH frame offset relative to P-CCPCH shall be E-ROCH = 5120 chips as illustrated in figure 39A.

Figure 39A: E-ROCH timing

## 7.13	E-DPDCH/E-DPCCH/DPCCH timing relation

The frame timing of all the E-DPCCHs and all the E-DPDCHs transmitted from one UE shall be the same as the uplink DPCCH frame timing.

## 7.14	S-DPCCH/DPCCH timing relation

The frame timing of uplink S-DPCCH transmitted from one UE shall be the same as the uplink DPCCH frame timing.

## 7.15	DPCH/F-DPCH/F-TPICH timing relations in softer handover

When UE is in softer handover and F-TPICH is transmitted from multiple radio links as defined in [5], the F-TPICH radio frame starts 512 chips after the start of the DPCH/F-DPCH radio frame that corresponds to the same radio link.

## 7.16	S-E-DPDCH/S-E-DPCCH/DPCCH timing relation

The frame timing of the S-E-DPCCH and all the S-E-DPDCHs transmitted from one UE shall be the same as the uplink DPCCH frame timing.

## 7.17	DPCCH2/DPCCH timing relation

The frame timing of the uplink DPCCH2 transmitted from one UE shall be the same as the uplink DPCCH frame timing.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
|  | RAN_05 | RP-99587 | - |  | Approved at TSG RAN #5 and placed under Change Control | - | 3.0.0 |
| 14/01/00 | RAN_06 | RP-99676 | 001 | 1 | Removal of superframe notation | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99677 | 002 | - | Use of CPICH in case of open loop Tx | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99677 | 003 | 2 | CPCH power control preamble length | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99684 | 005 | 1 | Editorial corrections | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99676 | 006 | - | Change to the description of TSTD for SCH | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99678 | 007 | 1 | Introduction of compressed mode by higher layer scheduling | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99676 | 008 | 1 | Modifications to STTD text | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99684 | 009 | 1 | 20 ms RACH message length | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99676 | 010 | - | Update to AICH description | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99678 | 011 | 1 | Sliding paging indicators | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99677 | 016 | - | TAB structure and timing relation for USTS | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99677 | 017 | - | Timing for initialisation procedures | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99677 | 022 | - | Modification of the STTD encoding scheme on DL DPCH with SF 512 | 3.0.0 | 3.1.0 |
| 14/01/00 | - | - | - |  | Change history was added by the editor | 3.1.0 | 3.1.1 |
| 31/03/00 | RAN_07 | RP-000060 | 013 | 6 | Addition of a downlink channel indicating CPCH status | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 023 | 6 | CPCH-related editorial changes, technical changes and additions to 25.211 and some clarifications to 7.4 PCPCH/AICH timing relation. | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 024 | 1 | Additional description of TX diversity for PDSCH | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 025 | 1 | Consistent numbering of scrambling code groups | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 026 | - | Minor corrections to timing section | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 028 | 1 | Timing of PDSCH | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 029 | 1 | Modifications to STTD text | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 031 | 4 | CD/CA-ICH for dual mode CPCH | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 033 | - | Clarification of frame synchronization word and its usage | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 034 | 1 | Editorial updates to 25.211 | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 036 | - | PDSCH multi-code transmission | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 037 | - | Clarification of pilot bit patterns for CPCH and slot formats for CPCH PC-P and message part | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 039 | - | Further restrictions on the application of the Tx diversity modes in DL | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 040 | - | Clarification of downlink pilot bit patterns | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 041 | - | Clarification of DCH initialisation | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 044 | 2 | Emergency Stop of CPCH transmission and Start of Message Indicator | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000060 | 046 | - | Clean up of USTS related specifications | 3.1.1 | 3.2.0 |
| 26/06/00 | RAN_08 | RP-000265 | 047 | 4 | Clarifications to power control preamble sections | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 048 | - | Propagation delay for PCPCH | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 049 | 1 | PICH undefined bits and AICH, AP-ICH, CD/CA-ICH non-transmitted chips | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 051 | 1 | Bit value notation change for PICH and CSICH | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 053 | 1 | Revision of notes in sections 5.3.2 and 5.3.2.1 | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 054 | 5 | Slot format clarification for CPCH | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 055 | 3 | Physical channel nomenclature in FDD | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 056 | 3 | Clarification for the PDSCH channelisation code association with DPCH in 25.211 | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 057 | 2 | Clarification for the PDSCH channelisation code association with DPCH in 25.211 | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 058 | - | Clarification of spreading factor for AICH | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000265 | 060 | - | Explicit mention of slot format reconfiguration also for uplink | 3.2.0 | 3.3.0 |
| 23/09/00 | RAN_09 | RP-000340 | 065 | - | Correction of reference | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 066 | 4 | Clarification of paging indicator mapping | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 068 | - | Editorial modification of the 25.211 about the CD/CA-ICH | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 070 | 1 | Support of closed loop transmit diversity modes | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 071 | - | DPCH initialisation procedure | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 072 | 3 | Correction on indicators | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 074 | - | Correction of STTD for DPCH | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 075 | - | Clarification of first significant path | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 076 | - | Clarification of SCH transmitted by TSTD | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000340 | 077 | 1 | Clarification of FBI field | 3.3.0 | 3.4.0 |
| 15/12/00 | RAN_10 | RP-000537 | 079 | 2 | Clarification of downlink phase reference | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000537 | 083 | 1 | DL Transmission in the case of invalid data frames | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000537 | 084 | - | Clarification of figure 28 | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000537 | 087 | - | RACH message part length | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000537 | 088 | - | Clarifications on power control preambles | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000537 | 089 | 1 | Proposed CR to 25.211 for transfer of CSICH Information from Layer 3 Specification | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000537 | 090 | - | PCPCH/DL-DPCCH Timing Relationship | 3.4.0 | 3.5.0 |
| 16/03/01 | RAN_11 | - | - | - | Approved as Release 4 specification (v4.0.0) at TSG RAN #11 | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010058 | 091 | - | DSCH reading indication | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010058 | 092 | 1 | Clarification of the S-CCPCH frame carrying paging information | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010255 | 095 | 3 | Phase Reference for Secondary CCPCH carrying FACH | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010058 | 096 | - | Uplink power control preamble | 3.5.0 | 4.0.0 |
| 15/06/01 | RAN_12 | RP-010331 | 098 | - | Downlink Phase Reference for DL-DPCCH for CPCH | 4.0.0 | 4.1.0 |
| 15/06/01 | RAN_12 | RP-010331 | 100 | - | Removal of out-of-date reference to FACH beamforming | 4.0.0 | 4.1.0 |
| 15/06/01 | RAN_12 | RP-010331 | 102 | - | Correction of compressed mode by puncturing | 4.0.0 | 4.1.0 |
| 15/06/01 | RAN_12 | RP-010331 | 104 | - | Correction of the representation of slot format | 4.0.0 | 4.1.0 |
| 15/06/01 | RAN_12 | RP-010331 | 106 | 1 | Clarification of PDSCH definition | 4.0.0 | 4.1.0 |
| 21/09/01 | RAN_13 | RP-010518 | 111 | 2 | Correction to DPCH/PDSCH timing | 4.1.0 | 4.2.0 |
| 21/09/01 | RAN_13 | RP-010518 | 121 | 1 | Clarification of the usage of Tx diversity modes in Soft HOV | 4.1.0 | 4.2.0 |
| 21/09/01 | RAN_13 | RP-010709 | 114 | 2 | Removal of another reference to FACH beamforming | 4.1.0 | 4.2.0 |
| 21/09/01 | RAN_13 | RP-010518 | 118 | 1 | Clarification of STTD | 4.1.0 | 4.2.0 |
| 14/12/01 | RAN_14 | RP-010904 | 116 | 2 | Clarification of the pilot bits on CPCH message part and S-CCPCH | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010736 | 123 | - | Addition of pilot bit patterns table of downlink DPCCH for antenna 2 using closed loop mode 1 | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010736 | 125 | - | Slot format for the CPCH | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010736 | 127 | 1 | Clarification of Tx diversity with PDSCH, AP-AICH, CD/CA-ICH and DL-DPCCH associated to CPCH | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010736 | 129 | 1 | Interaction between DSCH scheduling and phase reference modification | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010736 | 131 | - | Support of multiple CCTrChs of dedicated type | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010736 | 133 | - | Removal of slow power control from TS 25.211 | 4.2.0 | 4.3.0 |
| 14/12/01 | RAN_14 | RP-010932 | 135 | - | Restriction to simultaneous use of SSDT and closed loop mode TX diversity | 4.2.0 | 4.3.0 |
| 08/03/02 | RAN_15 | RP-020046 | 139 | 1 | Clarification of different diversity modes used in the same active set | 4.3.0 | 4.4.0 |
| 08/03/02 | RAN_15 | RP-020058 | 146 | - | Specification of HS-DSCH for Release 5 in 25.211 | 4.3.0 | 5.0.0 |
| 07/06/02 | RAN_16 | RP-020307 | 149 | 1 | SCCPCH structure with STTD encoding | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020307 | 153 | - | Downlink bit mapping | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020437 | 147 | 4 | Specification of TX diversity for HSDPA | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020316 | 150 | 1 | Adding section on HS-SCCH/HS-PDSCH timing relation | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020316 | 155 | - | HSDPA subframe definition | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020316 | 157 | 1 | Clarification for uplink HS-DPCCH/HS-PDSCH timing | 5.0.0 | 5.1.0 |
| 14/09/02 | RAN_17 | RP-020591 | 161 | 1 | Phase reference for HSDPA | 5.1.0 | 5.2.0 |
| 14/09/02 | RAN_17 | RP-020571 | 164 | - | Reversal of unwanted corrections resulting from CR 25.211-122 | 5.1.0 | 5.2.0 |
| 14/09/02 | RAN_17 | RP-020581 | 168 | 1 | Numbering corrections | 5.1.0 | 5.2.0 |
| 14/09/02 | RAN_17 | RP-020590 | 169 |  | TX diversity on radio links in the active set | 5.1.0 | 5.2.0 |
| 14/09/02 | RAN_17 | RP-020588 | 170 | 1 | HS-DPCCH timing correction | 5.1.0 | 5.2.0 |
| 14/09/02 | RAN_17 | RP-020587 | 171 |  | Inclusion of closed loop transmit diversity for HSDPA | 5.1.0 | 5.2.0 |
| 14/09/02 | RAN_17 | RP-020581 | 172 |  | Physical channel mapping | 5.1.0 | 5.2.0 |
| 20/12/02 | RAN_18 | RP-020845 | 173 | - | Correction of the number of transport channels in clause 4.1 | 5.2.0 | 5.3.0 |
| 20/12/02 | RAN_18 | RP-020845 | 175 | - | HSDPA Tx diversity of closed loop transmit diversity mode 2 use with HS-PDSCH/HS-SCCH | 5.2.0 | 5.3.0 |
| 21/06/03 | RAN_20 | RP-030271 | 178 | - | Alignment of the terminology, "subframe" | 5.3.0 | 5.4.0 |
| 21/06/03 | RAN_20 | RP-030271 | 179 | - | Correction of AICH description | 5.3.0 | 5.4.0 |
| 21/06/03 | RAN_20 | RP-030271 | 180 | - | Correction of description of TTX_diff | 5.3.0 | 5.4.0 |
| 21/09/03 | RAN_21 | RP-030462 | 186 | 1 | Removal of the combination of TxAA Mode 1 with HS-SCCH | 5.4.0 | 5.5.0 |
| 13/01/04 | RAN_22 | - | - | - | Created for M.1457 update | 5.5.0 | 6.0.0 |
| 09/06/04 | RAN_24 | RP-040231 | 189 | 1 | Re-Introduction of S-CPICH in combination with Closed Loop TxDiversity | 6.0.0 | 6.1.0 |
| 09/06/04 | RAN_24 | RP-040231 | 190 | - | Clarification of NTFCI field of DL-DPCCH power preamble for CPCH | 6.0.0 | 6.1.0 |
| 07/09/04 | RAN_25 | RP-040317 | 192 | - | Correction for the slot range of DL DPCCH power control preamble for CPCH | 6.1.0 | 6.2.0 |
| 13/12/04 | RAN_26 | RP-040449 | 195 | 1 | Introduction of E-DCH | 6.2.0 | 6.3.0 |
| 13/12/04 | RAN_26 | RP-040450 | 193 | 1 | Introduction of MICH | 6.2.0 | 6.3.0 |
| 14/03/05 | RAN_27 | RP-050043 | 197 | 1 | E-HICH/E-RGCH Signature Sequences | 6.3.0 | 6.4.0 |
| 14/03/05 | RAN_27 | RP-050043 | 198 | 1 | E-HICH/E-RGCH Signature Sequence Hopping | 6.3.0 | 6.4.0 |
| 14/03/05 | RAN_27 | RP-050090 | 202 | 2 | E-HICH/E-RGCH/E-AGCH timing | 6.3.0 | 6.4.0 |
| 14/03/05 | RAN_27 | RP-050088 | 200 | 1 | Introduction of F-DPCH without pilot field | 6.3.0 | 6.4.0 |
| 16/06/05 | RAN_28 | RP-050357 | 203 | 2 | Correction of text on E-RGCH duration | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050250 | 205 | 1 | Feature Clean Up: Removal of "CPCH" | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050248 | 207 | - | Feature Clean Up: Removal of DSCH (FDD mode) | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050252 | 210 | 1 | Clarification on E-AGCH transmission interval | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050256 | 211 | 2 | Clarification on phase reference for downlink channels | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050252 | 212 | 1 | Clarification on E-DCH timing | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050244 | 214 | - | Feature Clean Up: Removal of "SSDT" | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050247 | 217 | - | Feature clean up: Removal of the 'TX diversity closed loop mode 2' | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050249 | 219 | - | Feature clean up: Removal of the 'compressed mode by puncturing' | 6.4.0 | 6.5.0 |
| 16/06/05 | RAN_28 | RP-050246 | 221 | - | Feature Clean Up: Removal of dedicated pilot as sole phase reference | 6.4.0 | 6.5.0 |
| 26/09/05 | RAN_29 | RP-050450 | 0222 | - | SF max for E-DPDCH | 6.5.0 | 6.6.0 |
| 26/09/05 | RAN_29 | RP-050450 | 0223 | 1 | DPCCH, E-DPCCH, E-DPDCH combinations | 6.5.0 | 6.6.0 |
| 26/09/05 | RAN_29 | RP-050543 | 0244 | - | Correcting the accidential removal of F-DPCH, MICH, E-AGCH, E-RGCH and E-HICH from Tx Diversity applicability table | 6.5.0 | 6.6.0 |
| 12/12/05 | RAN_30 | RP-050726 | 0224 | - | Clean up due to removal of NTPC=1 | 6.6.0 | 6.7.0 |
| 12/12/05 | RAN_30 | RP-050727 | 0225 | 2 | Combination of DPCCH and E-DCH | 6.6.0 | 6.7.0 |
| 12/12/05 | RAN_30 | RP-050725 | 0227 | - | Clean up due to removal of CSICH | 6.6.0 | 6.7.0 |
| 20/03/06 | RAN_31 | - | - | - | Creation of Release 7 specification (v7.0.0) at RAN#31 | 6.7.0 | 7.0.0 |
| 07/03/07 | RAN_35 | RP-070114 | 0238 | - | Transmit diversity operation in MIMO mode | 7.0.0 | 7.1.0 |
| 07/03/07 | RAN_35 | RP-070115 | 0230 | 2 | Support of CPC feature | 7.0.0 | 7.1.0 |
| 07/03/07 | RAN_35 | RP-070115 | 0231 | - | Support of CPC feature: addition of subframe numbering | 7.0.0 | 7.1.0 |
| 07/03/07 | RAN_35 | RP-070116 | 0234 | 2 | Introduction of 64QAM for HSDPA | 7.0.0 | 7.1.0 |
| 30/05/07 | RAN_36 | RP-070388 | 0235 | 2 | Introduction of 16QAM for HSUPA | 7.1.0 | 7.2.0 |
| 30/05/07 | RAN_36 | RP-070384 | 0237 | 3 | Support for DL only SFN operation for MBMS FDD | 7.1.0 | 7.2.0 |
| 30/05/07 | RAN_36 | RP-070390 | 0239 | 3 | Introduction of PICH to HS-SCCH timing relation and Tx diversity definition for HS-DSCH without associated DL dedicated channel | 7.1.0 | 7.2.0 |
| 30/05/07 | RAN_36 | RP-070387 | 0240 | - | Definition of abbreviation "MIMO" | 7.1.0 | 7.2.0 |
| 30/05/07 | RAN_36 | RP-070389 | 0241 | - | Clarification for CPC feature | 7.1.0 | 7.2.0 |
| 11/09/07 | RAN_37 | RP-070639 | 0246 | 1 | Clarification on MICH | 7.2.0 | 7.3.0 |
| 11/09/07 | RAN_37 | RP-070649 | 0232 | 3 | Enhanced F-DPCH | 7.2.0 | 7.3.0 |
| 11/09/07 | RAN_37 | RP-070643 | 0242 | 1 | PICH associated HS-SCCH for Enhanced CELL_FACH | 7.2.0 | 7.3.0 |
| 11/09/07 | RAN_37 | RP-070641 | 0243 | - | Clarification for CPC feature | 7.2.0 | 7.3.0 |
| 11/09/07 | RAN_37 | RP-070646 | 0245 | - | Clarifications on the use of S-CCPCH pilot bits for MBSFN FDD feature | 7.2.0 | 7.3.0 |
| 27/11/07 | RAN_38 | RP-070940 | 0250 | - | Correction to E-DPCCH transmission | 7.3.0 | 7.4.0 |
| 27/11/07 | RAN_38 | RP-070941 | 0248 | - | Correction to transmit diversity specification in MIMO mode | 7.3.0 | 7.4.0 |
| 27/11/07 | RAN_38 | RP-070941 | 0251 | 1 | Mention PCI as part of HS-DPCCH structure | 7.3.0 | 7.4.0 |
| 04/03/08 | RAN_39 | RP-080142 | 0252 | - | Correction to tie use of transmit diversity on SCH and P-CCPCH | 7.4.0 | 7.5.0 |
| 04/03/08 | RAN_39 | - | - | - | Release 8 version created further to RAN_39 decision | 7.5.0 | 8.0.0 |
| 28/05/08 | RAN_40 | RP-080351 | 0255 | 1 | Correction to E-DCH control channel timing | 8.0.0 | 8.1.0 |
| 10/09/08 | RAN_41 | RP-080672 | 0256 | 2 | Introduction of the Enhanced Uplink for CELL_FACH state | 8.1.0 | 8.2.0 |
| 03/12/08 | RAN_42 | RP-080989 | 257 | 3 | Introduction of Dual-Cell HSDPA Operation on Adjacent Carriers | 8.2.0 | 8.3.0 |
| 03/12/08 | RAN_42 | RP-080986 | 259 | 1 | Removal of a reference to E-AICH | 8.2.0 | 8.3.0 |
| 03/03/09 | RAN_43 | RP-090229 | 261 | 1 | Clarifications to the S-CPICH usage with MIMO | 8.3.0 | 8.4.0 |
| 03/03/09 | RAN_43 | RP-090227 | 263 | 1 | Clarification of ACK transmission in response to HS-SCCH order | 8.3.0 | 8.4.0 |
| 15/09/09 | RAN_45 | RP-090890 | 268 | - | Clarification on STTD encoding description for AICH | 8.4.0 | 8.5.0 |
| 15/09/09 | RAN_45 | RP-090883 | 270 | 2 | Clarification of the applicability of Transmit Diversity | 8.4.0 | 8.5.0 |
| 15/09/09 | RAN_45 | RP-090884 | 273 | - | Clarification of associated channel for HS-DSCH | 8.4.0 | 8.5.0 |
| 18/09/09 | RAN_45 | - | - | - | Release 9 version created further to RAN_45 decision | 8.5.0 | 9.0.0 |
| 01/12/09 | RAN_46 | RP-091170 | 274 | 2 | Introduction of DC-HSUPA | 9.0.0 | 9.1.0 |
| 01/12/09 | RAN_46 | RP-091171 | 278 | 2 | Clarification of Tx diversity mode and phase reference for DC-HSDPA and MIMO operation | 9.0.0 | 9.1.0 |
| 01/12/09 | RAN_46 | RP-091160 | 281 | - | Clarification to MIMO phase references | 9.0.0 | 9.1.0 |
| 14/09/10 | RAN_49 | RP-100892 | 287 | 1 | Clarification of PICH/HS-SCCH timing relation | 9.1.0 | 9.2.0 |
| 16/09/10 | RAN_49 | RP-100902 | 284 | 2 | Release 10 created with the introduction of 4C-HSDPA | 9.2.0 | 10.0.0 |
| 05/12/11 | RAN_54 | RP-111671 | 288 | 5 | Introduction of 8C-HSDPA | 10.0.0 | 11.0.0 |
| 05/12/11 | RAN_54 | RP-111672 | 289 | - | Introduction of Uplink Closed Loop Transmit Diversity for HSPA | 10.0.0 | 11.0.0 |
| 04/09/12 | RAN_57 | RP-121271 | 295 | 2 | Introduction of Multiflow | 11.0.0 | 11.1.0 |
| 04/09/12 | RAN_57 | RP-121270 | 298 | 1 | Introduction of Further Enhancements to CELL_FACH | 11.0.0 | 11.1.0 |
| 04/09/12 | RAN_57 | RP-121384 | 299 | - | Correction of F-TPICH transmission | 11.0.0 | 11.1.0 |
| 04/12/12 | RAN_58 | RP-121845 | 296 | 5 | Introduction of uplink MIMO and 64QAM | 11.1.0 | 11.2.0 |
| 04/12/12 | RAN_58 | RP-121843 | 297 | 6 | Introduction of 4Tx_HSDPA in 25.211 | 11.1.0 | 11.2.0 |
| 04/12/12 | RAN_58 | RP-121838 | 305 | 1 | Removal of the relative amplitude weighting factor between EAI and AI | 11.1.0 | 11.2.0 |
| 04/12/12 | RAN_58 | RP-121844 | 306 | 1 | Clarifications and corrections to HSDPA Multiflow | 11.1.0 | 11.2.0 |
| 04/12/12 | RAN_58 | RP-121842 | 307 | 1 | Clarification of TTI Alignment | 11.1.0 | 11.2.0 |
| 04/12/12 | RAN_58 | RP-121842 | 308 | - | Correction to common E-RGCH | 11.1.0 | 11.2.0 |
| 26/02/13 | RAN_59 | RP-130251 | 310 | 1 | UE behaviour when non-time reference cell is out of time window | 11.2.0 | 11.3.0 |
| 11/06/13 | RAN_60 | RP-130745 | 312 | 3 | Correction to the Four Branch MIMO Transmit Diversity | 11.3.0 | 11.4.0 |
| 11/06/13 | RAN_60 | RP-130746 | 313 | 1 | Signature and default common E-DCH resource mapping | 11.3.0 | 11.4.0 |
| 10/06/14 | RAN_64 | RP-140860 | 317 | - | Correction of DRX operation for Multiflow | 11.4.0 | 11.5.0 |
| 10/09/14 | RAN_65 | RP-141482 | 315 | 2 | Introduction of Enhanced Broadcast of System Information | 11.5.0 | 12.0.0 |
| 10/09/14 | RAN_65 | RP-141480 | 316 | 3 | Introduction of DCH Enhancements | 11.5.0 | 12.0.0 |
| 10/09/14 | RAN_65 | RP-141481 | 318 | 1 | Introduction of UMTS Heterogeneous Networks enhancements | 11.5.0 | 12.0.0 |
| 08/12/14 | RAN_66 | RP-142101 | 320 | 1 | Correction of DL FET | 12.0.0 | 12.1.0 |
| 08/12/14 | RAN_66 | RP-142102 | 321 | 1 | Clarifications to Heterogeneous Networks | 12.0.0 | 12.1.0 |
| 07/12/15 | RAN_70 | RP-152023 | 322 | 4 | Introduction of Dual Carrier HSUPA enhancements for UTRAN CS in TS 25.211 | 12.1.0 | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-09 | RP-73 | RP-161568 | 0324 | 1 | A | On adding the TS 25.321 to the list of references of TS 25.211, which used in the definition of "UL DPCH 10ms Mode" | 13.1.0 |
| 2017-03 | RP-75 | - | - | - | - | Promotion to Release 14 without technical change (MCC) | 14.0.0 |
| 2018-06 | RP-80 | - | - | - | - | Promotion to Release 15 without technical change (MCC) | 15.0.0 |
| 2020-07 | RP-88e | - | - | - | - | Upgrade to Rel-16 version without technical change | 16.0.0 |