3GPP TS 25.222 V16.0.0 (2020-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Multiplexing and channel coding (TDD)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, radio, mux

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	11

2	References	11

3	Definitions, symbols and abbreviations	11

3.1	Definitions	11

3.2	Symbols	12

3.3	Abbreviations	12

4	Multiplexing, channel coding and interleaving for the 1.28 Mcps, 3.84 Mcps and 7.68 Mcps options	14

4.1	General	14

4.2	General coding/multiplexing of TrCHs	14

4.2.1	CRC attachment	18

4.2.1.1	CRC calculation	18

4.2.1.2	Relation between input and output of the CRC attachment block	18

4.2.2	Transport block concatenation and code block segmentation	19

4.2.2.1	Concatenation of transport blocks	19

4.2.2.2	Code block segmentation	19

4.2.3	Channel coding	20

4.2.3.1	Convolutional coding	21

4.2.3.2	Turbo coding	22

4.2.3.2.1	Turbo coder	22

4.2.3.2.2	Trellis termination for Turbo coder	23

4.2.3.2.3	Turbo code internal interleaver	23

4.2.3.2.3.1	Bits-input to rectangular matrix with padding	24

4.2.3.3	Concatenation of encoded blocks	27

4.2.4	Radio frame size equalisation	27

4.2.5	1st interleaving	27

4.2.5.1	Relation between input and output of 1st interleaving	28

4.2.6	Radio frame segmentation	28

4.2.7	Rate matching	29

4.2.7.1	Determination of rate matching parameters	30

4.2.7.1.1	Uncoded and convolutionally encoded TrCHs	31

4.2.7.1.2	Turbo encoded TrCHs	31

4.2.7.2	Bit separation and collection for rate matching	32

4.2.7.2.1	Bit separation	34

4.2.7.2.2	Bit collection	35

4.2.7.3	Rate matching pattern determination	35

4.2.8	TrCH multiplexing	36

4.2.9	Bit Scrambling	37

4.2.10	Physical channel segmentation	37

4.2.11	2nd interleaving	37

4.2.11.1	Frame related 2nd interleaving	38

4.2.11.2	Timeslot related 2nd interleaving	39

4.2.11A	Sub-frame segmentation for the 1.28 Mcps option	41

4.2.12	Physical channel mapping	41

4.2.12.1	Physical channel mapping for the 3.84 Mcps and 7.68Mcps options	41

4.2.12.1.1	Mapping scheme	42

4.2.12.2	Physical channel mapping for the 1.28 Mcps option	43

4.2.12.2.1	Mapping scheme	43

4.2.13	Multiplexing of different transport channels onto one CCTrCH, and mapping of one CCTrCH onto physical channels	44

4.2.13.1	Allowed CCTrCH combinations for one UE	45

4.2.13.1.1	Allowed CCTrCH combinations on the uplink	45

4.2.13.1.2	Allowed CCTrCH combinations on the downlink	45

4.2.14	Transport format detection	45

4.2.14.1	Blind transport format detection	45

4.2.14.2	Explicit transport format detection based on TFCI	45

4.2.14.2.1	Transport Format Combination Indicator (TFCI)	45

4.3	Coding for layer 1 control for the 3.84 Mcps and 7.68Mcps TDD options	46

4.3.1	Coding of transport format combination indicator (TFCI)	46

4.3.1.1	Coding of long TFCI lengths	46

4.3.1.2	Coding of short TFCI lengths	47

4.3.1.2.1	Coding very short TFCIs by repetition	47

4.3.1.2.2	Coding short TFCIs using bi-orthogonal codes	47

4.3.1.3	Mapping of TFCI code word	48

4.3.2	Coding and Bit Scrambling of the Paging Indicator	48

4.3.3	Coding and Bit Scrambling of the MBMS Notification Indicator	49

4.4	Coding for layer 1 control for the 1.28 Mcps option	49

4.4.1	Coding of transport format combination indicator (TFCI) for QPSK and 16QAM	49

4.4.1.1	Mapping of TFCI code word	49

4.4.2	Coding of transport format combination indicator (TFCI) for 8PSK	51

4.4.2.1	Coding of long TFCI lengths	51

4.4.2.2	Coding of short TFCI lengths	53

4.4.2.2.1	Coding very short TFCIs by repetition	53

4.4.2.2.2	Coding short TFCIs using bi-orthogonal codes	53

4.4.2.3	Mapping of TFCI code word	54

4.4.3	Coding and Bit Scrambling of the Paging Indicator	55

4.4.4	Coding of the Fast Physical Access Channel (FPACH) information bits	55

4.4.5	Coding and Bit Scrambling of the MBMS Notification Indicator	56

4.4.6	Coding of PLCCH	56

4.5	Coding for HS-DSCH	57

4.5.1	CRC attachment for HS-DSCH	59

4.5.2	Code block segmentation for HS-DSCH	59

4.5.3	Channel coding for HS-DSCH	60

4.5.4	Hybrid ARQ for HS-DSCH	60

4.5.4.1	HARQ bit separation	60

4.5.4.2	HARQ First Rate Matching Stage	60

4.5.4.3	HARQ Second Rate Matching Stage	61

4.5.4.4	HARQ bit collection	62

4.5.5	Bit scrambling	63

4.5.6	Interleaving for HS-DSCH	63

4.5.7	Constellation re-arrangement for 16 QAM and 64 QAM	64

4.5.8	Physical channel mapping for HS-DSCH	65

4.6	Coding/Multiplexing for HS-SCCH	67

4.6.1	HS-SCCH information field mapping	69

4.6.1.1	Channelisation code set information mapping	69

4.6.1.1.1	1.28Mcps TDD and 3.84Mcps TDD	69

4.6.1.1.2	7.68Mcps TDD	70

4.6.1.2	Timeslot information mapping	71

4.6.1.2.1	1.28 Mcps TDD	71

4.6.1.2.2	3.84 Mcps TDD and 7.68Mcps TDD	71

4.6.1.3	Modulation scheme information mapping	72

4.6.1.4	Redundancy and constellation version information mapping	73

4.6.1.5	HS-SCCH cyclic sequence number	73

4.6.1.6	UE identity	73

4.6.1.7	HARQ process identifier mapping	73

4.6.1.8	Transport block size index mapping	73

4.6.2	Multiplexing of HS-SCCH information	73

4.6.3	CRC attachment for HS-SCCH	74

4.6.4	Channel coding for HS-SCCH	74

4.6.5	Rate matching for HS-SCCH	74

4.6.6	Interleaving for HS-SCCH	74

4.6.7	Physical Channel Segmentation for HS-SCCH	74

4.6.8	Physical channel mapping for HS-SCCH	74

4.6A	Coding/Multiplexing for HS-SCCH orders type A	75

4.6A.1	HS-SCCH orders type A information field mapping	75

4.6A.1.1	Order type mapping	75

4.6A.1.2	UE identity mapping	75

4.6B	Coding/Multiplexing for HS-SCCH type 2 (1.28 Mcps TDD only)	75

4.6B.1	HS-SCCH type 2 information field mapping	77

4.6B.1.1	Type flag 1 mapping	77

4.6B.1.2	Resource repetition pattern index mapping	77

4.6B.1.3	Type flag 2 mapping	78

4.6B.1.4	Transport block size index mapping	78

4.6B.1.5	Timeslot information mapping	78

4.6B.1.6	Channelisation code set information mapping	78

4.6B.1.7	Modulation scheme information mapping	79

4.6B.1.8	HS-SICH indicator mapping	79

4.6B.1.9	HS-SCCH cyclic sequence number	79

4.6B.1.10	UE identity	79

4.6B.2	Multiplexing of HS-SCCH type 2 information	79

4.6B.3	CRC attachment for HS-SCCH type 2	80

4.6B.4	Channel coding for HS-SCCH type 2	80

4.6B.5	Rate matching for HS-SCCH type 2	80

4.6B.6	Interleaving for HS-SCCH type 2	80

4.6B.7	Physical Channel Segmentation for HS-SCCH type 2	80

4.6B.8	Physical channel mapping for HS-SCCH type 2	80

4.6C	Coding/Multiplexing for HS-SCCH type 3 (1.28 Mcps TDD only)	80

4.6C.1	HS-SCCH type 3 information field mapping	82

4.6C.1.1	Type flag 1 mapping	82

4.6C.1.2	Resource repetition pattern index mapping	82

4.6C.1.3	Type flag 2 mapping	82

4.6C.1.4	Transport block size index mapping	82

4.6C.1.5	Timeslot information mapping	83

4.6C.1.6	Channelisation code set information mapping	83

4.6C.1.7	Modulation scheme information mapping	83

4.6C.1.8	Redundancy version information mapping	83

4.6C.1.9	Pointer to the previous transmission mapping	83

4.6C.1.10	HS-SCCH cyclic sequence number	83

4.6C.1.11	UE identity	83

4.6C.2	Multiplexing of HS-SCCH type 3 information	83

4.6C.3	CRC attachment for HS-SCCH type 3	84

4.6C.4	Channel coding for HS-SCCH type 3	84

4.6C.5	Rate matching for HS-SCCH type 3	84

4.6C.6	Interleaving for HS-SCCH type 3	84

4.6C.7	Physical Channel Segmentation for HS-SCCH type 3	84

4.6C.8	Physical channel mapping for HS-SCCH type 3	84

4.6D	Coding/Multiplexing for HS-SCCH type 4 (1.28 Mcps TDD only)	85

4.6D.1	HS-SCCH type 4 information field mapping	86

4.6D.1.1	Type flag 1 mapping	86

4.6D.1.2	Type flag 2 mapping	86

4.6D.1.3	Channelisation code set information mapping	86

4.6D.1.4	Timeslot information mapping	87

4.6D.1.5	Modulation scheme information mapping	87

4.6D.1.6	Transport block size index mapping	87

4.6D.1.7	HARQ process identifier mapping	87

4.6D.1.8	Redundancy version information mapping	87

4.6D.1.9	HS-SCCH cyclic sequence number	87

4.6D.1.10	UE identity	88

4.6D.1.11	Midamble allocation scheme flag	88

4.6D.2	Multiplexing of HS-SCCH type 4 information	88

4.6D.3	CRC attachment for HS-SCCH type 4	88

4.6D.4	Channel coding for HS-SCCH type 4	88

4.6D.5	Rate matching for HS-SCCH type 4	88

4.6D.6	Interleaving for HS-SCCH type 4	89

4.6D.7	Physical Channel Segmentation for HS-SCCH type 4	89

4.6D.8	Physical channel mapping for HS-SCCH type 4	89

4.6E	Coding/Multiplexing for HS-SCCH type 5 (1.28 Mcps TDD only)	89

4.6E.1	HS-SCCH type 5 information field mapping	90

4.6E.1.1	Type flag 1 mapping	90

4.6E.1.2	Type flag 2 mapping	90

4.6E.1.3	Timeslot information mapping	90

4.6E.1.4	Modulation scheme information mapping	90

4.6E.1.5	Transport block size offset information mapping	91

4.6E.1.6	Transport block size index mapping	91

4.6E.1.7	HARQ process identifier mapping	91

4.6E.1.8	Redundancy version information mapping	91

4.6E.1.9	HS-SCCH cyclic sequence number	91

4.6E.1.10	UE identity	91

4.6E.2	Multiplexing of HS-SCCH type 5 information	91

4.6E.3	CRC attachment for HS-SCCH type 5	92

4.6E.4	Channel coding for HS-SCCH type 5	92

4.6E.5	Rate matching for HS-SCCH type 5	92

4.6E.6	Interleaving for HS-SCCH type 5	92

4.6E.7	Physical Channel Segmentation for HS-SCCH type 5	92

4.6E.8	Physical channel mapping for HS-SCCH type 5	92

4.6F	Coding/Multiplexing for HS-SCCH type 6 (1.28 Mcps TDD only)	92

4.6F.1	HS-SCCH type 6 information field mapping	94

4.6F.1.1	Type flag mapping	94

4.6F.1.2	Channelisation code set information mapping	94

4.6F.1.3	Timeslot information mapping	94

4.6F.1.4	Modulation scheme information mapping	94

4.6F.1.5	Resource repetition pattern index mapping	94

4.6F.1.6	Transport block size index mapping	95

4.6F.1.7	HS-SICH indicator mapping	95

4.6F.1.8	HS-SCCH cyclic sequence number	95

4.6F.1.9	UE identity	95

4.6F.2	Multiplexing of HS-SCCH type 6 information	95

4.6F.3	CRC attachment for HS-SCCH type 6	95

4.6F.4	Channel coding for HS-SCCH type 6	95

4.6F.5	Rate matching for HS-SCCH type 6	96

4.6F.6	Interleaving for HS-SCCH type 6	96

4.6F.7	Physical Channel Segmentation for HS-SCCH type 6	96

4.6F.8	Physical channel mapping for HS-SCCH type 6	96

4.6G	Coding/Multiplexing for HS-SCCH type 7 (1.28 Mcps TDD only)	96

4.6G.1	HS-SCCH type 7 information field mapping	97

4.6G.1.1	Type flag mapping	97

4.6G.1.2	Channelisation code set information mapping	97

4.6G.1.3	Timeslot information mapping	97

4.6G.1.4	Modulation scheme information mapping	97

4.6G.1.5	Resource repetition pattern index mapping	98

4.6G.1.6	Transport block size index mapping	98

4.6G.1.7	Redundancy version information mapping	98

4.6G.1.8	Pointer to the previous transmission mapping	98

4.6G.1.9	HS-SCCH cyclic sequence number	98

4.6G.1.10	UE identity	98

4.6G.2	Multiplexing of HS-SCCH type 7 information	98

4.6G.3	CRC attachment for HS-SCCH type 7	98

4.6G.4	Channel coding for HS-SCCH type 7	99

4.6G.5	Rate matching for HS-SCCH type 7	99

4.6G.6	Interleaving for HS-SCCH type 7	99

4.6G.7	Physical Channel Segmentation for HS-SCCH type 7	99

4.6G.8	Physical channel mapping for HS-SCCH type 7	99

4.6H	Coding/Multiplexing for HS-SCCH type 8 (1.28 Mcps TDD only)	99

4.6H.1	HS-SCCH type 8 information field mapping	100

4.6H.1.1	Channelisation code set information mapping	100

4.6H.1.2	Transport block size index mapping	101

4.6H.1.3	Modulation scheme information mapping	102

4.6H.1.4	Timeslot information mapping	102

4.6H.1.5	Redundancy version information mapping	102

4.6H.1.6	Type flag mapping	102

4.6H.1.7	Field flag mapping	102

4.6H.1.8	Special Information mapping	102

4.6H.1.9	HARQ process identifier mapping	102

4.6H.1.10	HS-SCCH cyclic sequence number	102

4.6H.1.11	UE identity	102

4.6H.2	Multiplexing of HS-SCCH type 8 information	102

4.6H.3	CRC attachment for HS-SCCH type 8	103

4.6H.4	Channel coding for HS-SCCH type 8	103

4.6H.5	Rate matching for HS-SCCH type 8	103

4.6H.6	Interleaving for HS-SCCH type 8	103

4.6H.7	Physical Channel Segmentation for HS-SCCH type 8	103

4.6H.8	Physical channel mapping for HS-SCCH type 8	103

4.6I	Coding/Multiplexing for HS-SCCH type 9 (1.28 Mcps TDD only)	104

4.6I.1	HS-SCCH type 9 information field mapping	105

4.6I.1.1	Channelisation code set information mapping	105

4.6I.1.2	Transport block size offset information mapping	106

4.6I.1.3	Modulation scheme information mapping	106

4.6I.1.4	Timeslot information mapping	106

4.6I.1.5	Redundancy version information mapping	106

4.6I.1.6	HARQ process identifier mapping	106

4.6I.1.9	HS-SCCH cyclic sequence number	106

4.6I.1.10	UE identity	106

4.6I.2	Multiplexing of HS-SCCH type 9 information	106

4.6I.3	CRC attachment for HS-SCCH type 9	107

4.6I.4	Channel coding for HS-SCCH type 9	107

4.6I.5	Rate matching for HS-SCCH type 9	107

4.6I.6	Interleaving for HS-SCCH type 9	107

4.6I.7	Physical Channel Segmentation for HS-SCCH type 9	107

4.6I.8	Physical channel mapping for HS-SCCH type 9	107

4.6J	Coding/Multiplexing for HS-SCCH orders type B (1.28Mcps TDD only)	107

4.6J.1	HS-SCCH orders type B information field mapping	108

4.6J.1.1	Order type mapping	108

4.6J.1.2	UE identity mapping	108

4.7	Coding for HS-SICH	108

4.7.1	HS-SICH information field mapping	109

4.7.1.1	RMF information mapping	109

4.7.1.2	RTBS information mapping	109

4.7.1.3	ACK/NACK information mapping	109

4.7.2	Coding for HS-SICH	110

4.7.2.1	Field Coding of ACK/NACK	110

4.7.2.2	Field Coding of CQI	110

4.7.2.2.1	Field Coding of CQI for 1.28 Mcps TDD	110

4.7.2.2.2	Field Coding of CQI for 3.84 Mcps TDD and 7.68Mcps TDD	111

4.7.3	Multiplexing of HS-SICH information fields	111

4.7.4	Interleaver for HS-SICH	111

4.7.5	Physical channel mapping for HS-SICH	111

4.7A	Coding for HS-SICH type 2 (1.28 Mcps TDD only)	111

4.7A.1	HS-SICH type 2 information field mapping	112

4.7A.1.1	RMF information mapping	112

4.7A.1.2	RTBS information mapping	112

4.7A.1.3	ACK/NACK information mapping	113

4.7A.2	Coding for HS-SICH type 2	113

4.7A.2.1	Field Coding of ACK/NACK	113

4.7A.2.2	Field Coding of CQI	113

4.7A.3	Multiplexing of HS-SICH type 2 information fields	113

4.7A.4	Interleaver for HS-SICH type 2	114

4.7A.5	Physical channel mapping for HS-SICH type 2	114

4.8	Coding for E-DCH	114

4.8.1	CRC attachment for E-DCH	115

4.8.2	Code block segmentation for E-DCH	115

4.8.3	Channel coding for E-DCH	116

4.8.4	Physical layer HARQ functionality and rate matching for E-DCH	116

4.8.4.1	Determination of SF, modulation and number of physical channels	116

4.8.4.2	HARQ bit separation	116

4.8.4.3	HARQ Rate Matching Stage	116

4.8.4.4	HARQ bit collection	117

4.8.5	Bit scrambling	117

4.8.6	Interleaving for E-DCH	117

4.8.7	Constellation re-arrangement for 16 QAM	117

4.8.8	Physical channel mapping for E-DCH	117

4.9	Coding for E-UCCH	118

4.9.1	Coding for E-UCCH for the 3.84Mcps and 7.68Mcps TDD options	118

4.9.1.1	Overview	118

4.9.1.2	E-UCCH part 1	119

4.9.1.2.1	Information field mapping of E-TFCI	119

4.9.1.2.2	Channel coding for E-UCCH part 1	119

4.9.1.2.3	Physical channel mapping for E-UCCH part 1	119

4.9.1.3	E-UCCH part 2	119

4.9.1.3.1	Information field mapping of retransmission sequence number	119

4.9.1.3.2	Information field mapping of HARQ process ID	119

4.9.1.3.3	Multiplexing of E-UCCH part 2 information	120

4.9.1.3.4	Channel coding for E-UCCH part 2	120

4.9.1.3.5	Physical channel mapping for E-UCCH part 2	120

4.9.2	Coding for E-UCCH for the 1.28Mcps TDD option	120

4.9.2.1	E-UCCH information field mapping	121

4.9.2.1.1	Information field mapping of E-TFCI	121

4.9.2.1.2	RSN information mapping	121

4.9.2.1.3	HARQ information mapping	122

4.9.2.2	Multiplexing for E-UCCH	122

4.9.2.3	Coding for E-UCCH	122

4.9.2.4	Physical channel mapping for E-UCCH	123

4.10	Coding for E-AGCH	123

4.10.1	Information Field Mapping	124

4.10.1.1	Mapping of the Absolute Grant (Power) Value	124

4.10.1.2	Mapping of the Code Resource Related Information	126

4.10.1.3	Mapping of the Timeslot Resource Related Information	127

4.10.1.4	Mapping of the E-AGCH Cyclic Sequence Number (ECSN)	127

4.10.1.5	Mapping of the Resource Duration Indicator	127

4.10.1.6	Mapping of the E-HICH Indicator (1.28Mcps option only)	128

4.10.1.7	Mapping of the E-UCCH Number Indicator (1.28Mcps option only)	128

4.10.2	Field Multiplexing	128

4.10.3	CRC attachment	129

4.10.4	Channel Coding	129

4.10.5	Rate Matching	129

4.10.6	Interleaving	129

4.10.7	Physical Channel Segmentation	129

4.10.8	Physical Channel Mapping	129

4.10A	Coding for E-AGCH type 2 (1.28Mcps TDD only)	129

4.10A.1	E-AGCH type 2 Information Field Mapping	131

4.10A.1.1	Mapping of the Absolute Grant (Power) Value	131

4.10A.1.2	Mapping of the Code Resource Related Information	131

4.10A.1.3	Mapping of the Timeslot Resource Related Information	131

4.10A.1.4	Mapping of the E-AGCH Cyclic Sequence Number (ECSN)	131

4.10A.1.5	Mapping of the Field Flag	131

4.10A.1.6	Mapping of the Special Information 1	131

4.10A.1.7	Mapping of the Special Information 2	132

4.10A.1.8	Mapping of the E-UCCH Number Indicator	132

4.10A.2	Field Multiplexing of E-AGCH type 2	132

4.10A.3	CRC attachment for E-AGCH type 2	133

4.10A.4	Channel Coding for E-AGCH type 2	133

4.10A.5	Rate Matching for E-AGCH type 2	133

4.10A.6	Interleaving for E-AGCH type 2	133

4.10A.7	Physical Channel Segmentation for E-AGCH type 2	133

4.10A.8	Physical Channel Mapping for E-AGCH type 2	133

4.10B	Coding for E-AGCH orders	133

4.10B.1	E-AGCH orders information field mapping	133

4.10B.1.1	Order type mapping	133

4.10B.1.2	UE identity mapping	134

4.11	Coding for E-HICH ACK/NACK	134

4.11.1	Coding for E-HICH ACK/NACK for the 3.84Mcps and 7.68Mcps options	134

4.11.1.1	Overview	134

4.11.1.2	Coding of the HARQ acknowledgement indicator	134

4.11.1.3	Bit scrambling of the E-HICH	136

4.11.1.4	Physical channel mapping of the E-HICH	136

4.11.2	Coding for E-HICH for the1.28Mcps option only	136

4.11.2.1	Overview	136

4.11.2.2	Coding of the HARQ acknowledgement indicator and TPC/SS	137

4.11.2.3	Bit scrambling and Physical channel mapping of the E-HICH	139

4.12	Coding for E-RUCCH	139

4.12.1	CRC attachment for E-RUCCH	140

4.12.2	Channel coding for E-RUCCH	140

4.12.3	Rate matching for E-RUCCH	140

4.12.4	Bit scrambling for E-RUCCH	140

4.12.5	Interleaving for E-RUCCH	140

4.12.6	Physical channel mapping for E-RUCCH	140

5	Multiplexing, channel coding and interleaving for the 3.84 Mcps MBSFN IMB option	140

5.1	General	140

5.2	General coding/multiplexing of TrCHs	141

5.2.1	CRC attachment	141

5.2.2	Transport block concatenation and code block segmentation	141

5.2.3	Channel coding	141

5.2.4	1st Interleaving	141

5.2.5	Radio frame segmentation	141

5.2.6	Rate matching	141

5.2.7	TrCH multiplexing	141

5.2.8	Insertion of discontinuous transmission (DTX) indication bits	142

5.2.9	Physical channel segmentation	142

5.2.10	2nd Interleaving	142

5.2.11	Physical channel mapping	142

5.2.12	Restrictions on different types of CCTrCHs	143

5.2.12.1	Broadcast channel (BCH)	143

5.2.12.2	Forward access channel (FACH)	143

5.2.13	Multiplexing of different TrCHs into one CCTrCH, and mapping of one CCTrCH onto physical channels	143

5.3	Transport format detection	143

5.3.1	Transport format detection based on TFCI	143

5.3.2	Coding of Transport-Format-Combination Indicator (TFCI)	143

5.3.3	Mapping of TFCI words	143

5.3.3.1	Mapping of TFCI bits for Secondary CCPCH	143

Annex A (informative):	Change history	144

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes multiplexing, channel coding and interleaving for UTRA Physical Layer TDD mode.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 25.202: "UE capabilities".

[2]	3GPP TS 25.211: "Transport channels and physical channels (FDD)".

[3]	3GPP TS 25.212: "Multiplexing and channel coding (FDD)".

[4]	3GPP TS 25.213: "Spreading and modulation (FDD)".

[5]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[6]	3GPP TS 25.215: "Physical layer – Measurements (FDD)".

[7]	3GPP TS 25.221: "Transport channels and physical channels (TDD)".

[9]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[10]	3GPP TS 25.224: "Physical layer procedures (TDD)".

[11]	3GPP TS 25.225: "Measurements".

[12]	3GPP TS 25.331: "RRC Protocol Specification".

[13]	3GPP TS 25.308: "High Speed Downlink Packet Access (HSDPA): Overall description (stage 2)".

[14]	ITU-T Recommendation X.691 (12/97) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[15]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification"

[16]	3GPP TS 25.302: "Services provided by the physical layer"

[17]	3GPP TS 25.306: "UE Radio Access Capabilities"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

TrCH number: The transport channel number identifies a TrCH in the context of L1. The L3 transport channel identity (TrCH ID) maps onto the L1 transport channel number. The mapping between the transport channel number and the TrCH ID is as follows: TrCH 1 corresponds to the TrCH with the lowest TrCH ID, TrCH 2 corresponds to the TrCH with the next lowest TrCH ID and so on.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

x	round towards , i.e. integer such that x  x  < x+1

x	round towards -, i.e. integer such that x-1 < x   x

x	absolute value of x

Unless otherwise is explicitly stated when the symbol is used, the meaning of the following symbols are:

i	TrCH number

j	TFC number

k	Bit number

l	TF number

m	Transport block number

n	Radio frame number

p	PhCH number

r	Code block number

I	Number of TrCHs in a CCTrCH.

Ci	Number of code blocks in one TTI of TrCH i.

Fi	Number of radio frames in one TTI of TrCH i.

Mi	Number of transport blocks in one TTI of TrCH i.

NTCFI code word	Number of TFCI code word bits after TFCI encoding

P	Number of PhCHs used for one CCTrCH.

PL	Puncturing Limit. Signalled from higher layers

RMi	Rate Matching attribute for TrCH i. Signalled from higher layers.

Temporary variables, i.e. variables used in several (sub)clauses with different meaning.

x, X

y, Y

z, Z

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

<ACRONYM>	<Explanation>

ARQ	Automatic Repeat on Request

BCH	Broadcast Channel

BER	Bit Error Rate

BS	Base Station

BSS	Base Station Subsystem

CBR	Constant Bit Rate

CCCH	Common Control Channel

CCTrCH	Coded Composite Transport Channel

CDMA	Code Division Multiple Access

CFN	Connection Frame Number

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

DCA	Dynamic Channel Allocation

DCCH	Dedicated Control Channel

DCH	Dedicated Channel

DL	Downlink

DRX	Discontinuous Reception

DSCH	Downlink Shared Channel

DTX	Discontinuous Transmission

E-AGCH	E-DCH Absolute Grant Channel

ECSN	E-AGCH Cyclic Sequence Number

E-DCH	Enhanced Dedicated Channel

E-HICH	E-DCH Hybrid ARQ Indicator Channel

E-PUCH	E-DCH Physical Uplink Channel

E-RUCCH	E-DCH Random Access Uplink Control Channel

E-TFCI	E-DCH Transport Format Combination Indicator

E-UCCH	E-DCH Uplink Control Channel

FACH	Forward Access Channel

FDD	Frequency Division Duplex

FDMA	Frequency Division Multiple Access

FEC	Forward Error Control

FER	Frame Error Rate

GF	Galois Field

HARQ	Hybrid Automatic Repeat reQuest

HS-DSCH	High Speed Downlink Shared Channel

HS-PDSCH	High Speed Physical Downlink Shared Channel

HS-SCCH	Shared Control Channel for HS-DSCH

HS-SICH	Shared Information Channel for HS-DSCH

IMB	Integrated Mobile Broadcast

JD	Joint Detection

L1	Layer 1

L2	Layer 2

LLC	Logical Link Control

MA	Multiple Access

MAC	Medium Access Control

MBSFN	MBMS over a Single Frequency Network

MICH	MBMS Indicator Channel

MIMO	single user Multiple Input Multiple Output

MS	Mobile Station

MT	Mobile Terminated

MU-MIMO	Multi-User Multiple Input Multiple Output

NRT	Non-Real Time

OVSF	Orthogonal Variable Spreading Factor

PC	Power Control

PCCC	Parallel Concatenated Convolutional Code

PICH	Paging Indicator Channel

PCH	Paging Channel

PhCH	Physical Channel

PI	Paging Indicator (value calculated by higher layers)

PLCCH	Physical Layer Common Control Channel

Pq	Paging Indicator (indicator set by physical layer)

QoS	Quality of Service

QPSK	Quaternary Phase Shift Keying

RACH	Random Access Channel

RF	Radio Frequency

RLC	Radio Link Control

RMF	Recommended Modulation Format

RRC	Radio Resource Control

RRM	Radio Resource Management

RSC	Recursive Systematic Convolutional Coder

RSN	Retransmission Sequence Number

RT	Real Time

RTBS	Recommended Transport Block Size

RU	Resource Unit

RV	Redundancy Version

SCCC	Serial Concatenated Convolutional Code

SCH	Synchronization Channel

SNR	Signal to Noise Ratio

TCH	Traffic channel

TDD	Time Division Duplex

TDMA	Time Division Multiple Access

TFC	Transport Format Combination

TFCI	Transport Format Combination Indicator

TFRI	Transport Format Resource Indicator

TPC	Transmit Power Control

TrBk	Transport Block

TrCH	Transport Channel

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunications System

USCH	Uplink Shared Channel

UTRA	UMTS Terrestrial Radio Access

VBR	Variable Bit Rate

# 4	Multiplexing, channel coding and interleaving for the 1.28 Mcps, 3.84 Mcps and 7.68 Mcps options

In the case of the 3.84 Mcps option, clause 4 applies only for non-MBSFN-IMB operation.  Multiplexing, channel coding and interleaving for 3.84 Mcps MBSFN IMB operation is described in sub-clause 5.

## 4.1	General

Data stream from/to MAC and higher layers (Transport block / Transport block set) is encoded/decoded to offer transport services over the radio transmission link. Channel coding scheme is a combination of error detection, error correcting (including rate matching), and interleaving and transport channels mapping onto/splitting from physical channels.

In the UTRA-TDD mode for the 1.28 Mcps, 3.84 Mcps and 7.68 Mcps options, the total number of basic physical channels (a certain time slot one spreading code on a certain carrier frequency) per frame is given by the maximum number of time slots and the maximum number of CDMA codes per time slot.

## 4.2	General coding/multiplexing of TrCHs

This section only applies to the transport channels: DCH, RACH, DSCH, USCH, BCH, FACH and PCH. Other transport channels which do not use the general method are described separately below.

Figure 1 illustrates the overall concept of transport-channel coding and multiplexing. Data arrives to the coding/multiplexing unit in form of transport block sets, once every transmission time interval. The transmission time interval is transport-channel specific from the set {5 ms(*1), 10 ms, 20 ms, 40 ms, 80 ms}.

Note: (*1)	may be applied for PRACH for 1.28 Mcps TDD

The following coding/multiplexing steps can be identified:

-	add CRC to each transport block (see subclause 4.2.1);

-	TrBk concatenation / Code block segmentation (see subclause 4.2.2);

-	channel coding (see subclause 4.2.3) ;

-	radio frame size equalization (see subclause 4.2.4);

-	interleaving (two steps, see subclauses 4.2.5 and 4.2.11 );

-	radio frame segmentation (see subclause 4.2.6);

-	rate matching (see subclause 4.2.7);

-	multiplexing of transport channels (see subclause 4.2.8);

-	bit scrambling (see subclause 4.2.9);

-	physical channel segmentation (see subclause 4.2.10);

-	sub-frame segmentation(see subclause 4.2.11A only for 1.28Mcps TDD)

-	mapping to physical channels (see subclause 4.2.12).

The coding/multiplexing steps for uplink and downlink are shown in figures 1 and 1A.

Figure 1: Transport channel multiplexing structure for uplink and downlink for 3.84Mcps and 7.68 Mcps TDD

Figure 1A: Transport channel multiplexing structure for uplink and downlink of 1.28Mcps TDD

Primarily, transport channels are multiplexed as described above, i.e. into one data stream mapped on one or several physical channels. However, an alternative way of multiplexing services is to use multiple CCTrCHs (Coded Composite Transport Channels), which corresponds to having several parallel multiplexing chains as in figures 1 and 1A, resulting in several data streams, each mapped to one or several physical channels.

### 4.2.1	CRC attachment

Error detection is provided on transport blocks through a Cyclic Redundancy Check (CRC). The size of the CRC is 24, 16, 12, 8 or 0 bits and it is signalled from higher layers what CRC size that should be used for each transport channel.

#### 4.2.1.1	CRC calculation

The entire transport block is used to calculate the CRC parity bits for each transport block. The parity bits are generated by one of the following cyclic generator polynomials:

gCRC24(D) = D24 + D23 + D6 + D5 + D + 1

gCRC16(D) = D16 + D12 + D5 + 1

gCRC12(D) = D12 + D11 + D3 + D2 + D + 1

gCRC8(D) = D8 + D7 + D4 + D3 + D + 1

Denote the bits in a transport block delivered to layer 1 by , and the parity bits by . Ai is the size of a transport block of TrCH i, m is the transport block number, and Li is the number of parity bits. Li can take the values 24, 16, 12, 8, or 0 depending on what is signalled from higher layers.

The encoding is performed in a systematic form, which means that in GF(2), the polynomial:

yields a remainder equal to 0 when divided by gCRC24(D), polynomial:

yields a remainder equal to 0 when divided by gCRC16(D), polynomial:

yields a remainder equal to 0 when divided by gCRC12(D) and the polynomial:

yields a remainder equal to 0 when divided by gCRC8(D).

If no transport blocks are input to the CRC calculation (Mi = 0), no CRC attachment shall be done. If transport blocks are input to the CRC calculation (Mi  0) and the size of a transport block is zero (Ai = 0), CRC shall be attached, i.e. all parity bits equal to zero.

#### 4.2.1.2	Relation between input and output of the CRC attachment block

The bits after CRC attachment are denoted by , where Bi = Ai + Li. The relation between aimk and bimk is:

k = 1, 2, 3, …, Ai

k = Ai + 1, Ai + 2, Ai + 3, …, Ai + Li

### 4.2.2	Transport block concatenation and code block segmentation

All transport blocks in a TTI are serially concatenated. If the number of bits in a TTI is larger than the maximum size of a code block, then code block segmentation is performed after the concatenation of the transport blocks. The maximum size of the code blocks depends on whether convolutional, turbo coding or no coding is used for the TrCH.

#### 4.2.2.1	Concatenation of transport blocks

The bits input to the transport block concatenation are denoted by  where i is the TrCH number, m is the transport block number, and Bi is the number of bits in each block (including CRC). The number of transport blocks on TrCH i is denoted by Mi. The bits after concatenation are denoted by , where i is the TrCH number and Xi=MiBi. They are defined by the following relations:

k = 1, 2, …, Bi

k = Bi + 1, Bi + 2, …, 2Bi

k = 2Bi + 1, 2Bi + 2, …, 3Bi

k = (Mi – 1)Bi + 1, (Mi – 1)Bi + 2, …, MiBi

#### 4.2.2.2	Code block segmentation

Segmentation of the bit sequence from transport block concatenation is performed if Xi>Z. The code blocks after segmentation are of the same size. The number of code blocks on TrCH i is denoted by Ci. If the number of bits input to the segmentation, Xi, is not a multiple of Ci, filler bits are added to the beginning of the first block. If turbo coding is selected and Xi < 40, filler bits are added to the beginning of the code block. The filler bits are transmitted and they are always set to 0. The maximum code block sizes are:

-	convolutional coding: Z = 504;

-	turbo coding: Z = 5114;

-	no channel coding: Z = unlimited.

The bits output from code block segmentation, for Ci  0, are denoted by , where i is the TrCH number, r is the code block number, and Ki is the number of bits per code block.

Number of code blocks:

Number of bits in each code block (applicable for Ci  0 only):

if Xi < 40 and Turbo coding is used, then

Ki = 40

else

Ki = Xi / Ci

end if

Number of filler bits: Yi = CiKi – Xi

for k = 1 to Yi	-- Insertion of filler bits

end for

for k = Yi+1 to Ki

end for

r = 2	-- Segmentation

while r  Ci

for k = 1 to Ki

end for

r = r+1

end while

### 4.2.3	Channel coding

Code blocks are delivered to the channel coding block. They are denoted by , where i is the TrCH number, r is the code block number, and Ki is the number of bits in each code block. The number of code blocks on TrCH i is denoted by Ci. After encoding the bits are denoted by , where Yi is the number of encoded bits. The relation between oirk and yirk and between Ki and Yi is dependent on the channel coding scheme.

The following channel coding schemes can be applied to transport channels:

-	convolutional coding;

-	turbo coding;

-	no coding.

Usage of coding scheme and coding rate for the different types of TrCH is shown in tables 1 and 1A. The values of Yi in connection with each coding scheme:

-	convolutional coding with rate 1/2: Yi = 2*Ki + 16; rate 1/3: Yi = 3*Ki + 24;

-	turbo coding with rate 1/3: Yi = 3*Ki + 12;

-	no coding: Yi = Ki.

Table 1: Usage of channel coding scheme and coding rate for 3.84Mcps TDD

| Type of TrCH | Coding scheme | Coding rate |
| --- | --- | --- |
| BCH | Turbo coding | 1/3 |
|  | Convolutional coding | 1/2 |
| PCH |  |  |
| RACH |  |  |
| DCH, DSCH, FACH, USCH |  | 1/3, 1/2 |
|  | Turbo coding | 1/3 |
|  | No coding |  |

Note: Rate 1/3 Turbo coding is only applied to BCH when the beacon timeslot uses burst type 4; rate 1/2 convolutional coding is only applied to BCH when the beacon timeslot uses burst type 1. For the MBSFN FACH, only rate 1/3 Turbo coding shall be applied.

Table 1A: Usage of channel coding scheme and coding rate for 1.28Mcps TDD

| Type of TrCH | Coding scheme | Coding rate |
| --- | --- | --- |
| BCH | Convolutional coding | 1/3 |
| PCH |  | 1/3, 1/2 |
| RACH |  | 1/2 |
| DCH, DSCH, FACH, USCH |  | 1/3, 1/2 |
|  | Turbo coding | 1/3 |
|  | No coding |  |

Note: For the MBSFN FACH, only rate 1/3 Turbo coding shall be applied.

#### 4.2.3.1	Convolutional coding

Convolutional codes with constraint length 9 and coding rates 1/3 and 1/2 are defined.

The configuration of the convolutional coder is presented in figure 2.

Output from the rate 1/3 convolutional coder shall be done in the order output 0, output 1, output 2, output 0, output 1, output 2, output 0,…,output 2. Output from the rate 1/2 convolutional coder shall be done in the order output 0, output 1, output 0, output 1, output 0, …, output 1.

8 tail bits with binary value 0 shall be added to the end of the code block before encoding.

The initial value of the shift register of the coder shall be "all 0" when starting to encode the input bits.

Figure 2: Rate 1/2 and rate 1/3 convolutional coders

#### 4.2.3.2	Turbo coding

##### 4.2.3.2.1	Turbo coder

The scheme of Turbo coder is a Parallel Concatenated Convolutional Code (PCCC) with two 8-state constituent encoders and one Turbo code internal interleaver. The coding rate of Turbo coder is 1/3. The structure of Turbo coder is illustrated in figure 3.

The transfer function of the 8-state constituent code for PCCC is:

G(D)= ,

where

g0(D) = 1 + D2 + D3,

g1 (D) = 1 + D + D3.

The initial value of the shift registers of the 8-state constituent encoders shall be all zeros when starting to encode the input bits.

Output from the Turbo coder is , Y'(0), X(1), Y(1), Y'(1), etc:

x1, z1, z'1, x2, z2, z'2, …, xK, zK, z'K,

where x1, x2, …, xK are the bits input to the Turbo coder i.e. both first 8-state constituent encoder and Turbo code internal interleaver, and K is the number of bits, and z1, z2, …, zK and z'1, z'2, …, z'K are the bits output from first and second 8-state constituent encoders, respectively.

The bits output from Turbo code internal interleaver are denoted by x'1, x'2, …, x'K, and these bits are to be input to the second 8-state constituent encoder.

Figure 3: Structure of rate 1/3 Turbo coder (dotted lines apply for trellis termination only)

##### 4.2.3.2.2	Trellis termination for Turbo coder

Trellis termination is performed by taking the tail bits from the shift register feedback after all information bits are encoded. Tail bits are padded after the encoding of information bits.

The first three tail bits shall be used to terminate the first constituent encoder (upper switch of figure 3 in lower position) while the second constituent encoder is disabled. The last three tail bits shall be used to terminate the second constituent encoder (lower switch of figure 3 in lower position) while the first constituent encoder is disabled.

The transmitted bits for trellis termination shall then be:

xK+1, zK+1, xK+2, zK+2, xK+3, zK+3, x'K+1, z'K+1, x'K+2, z'K+2, x'K+3, z'K+3.

##### 4.2.3.2.3	Turbo code internal interleaver

The Turbo code internal interleaver consists of bits-input to a rectangular matrix with padding, intra-row and inter-row permutations of the rectangular matrix, and bits-output from the rectangular matrix with pruning. The bits input to the Turbo code internal interleaver are denoted by , where K is the integer number of the bits and takes one value of 40  K  5114. The relation between the bits input to the Turbo code internal interleaver and the bits input to the channel coding is defined by  and K = Ki.

The following subclause specific symbols are used in subclauses 4.2.3.2.3.1 to 4.2.3.2.3.3 :

K	Number of bits input to Turbo code internal interleaver

R	Number of rows of rectangular matrix

C	Number of columns of rectangular matrix

p	Prime number

v	Primitive root

Base sequence for intra-row permutation

qi	Minimum prime integers

ri	Permuted prime integers

Inter-row permutation pattern

Intra-row permutation pattern of i-th row

i	Index of row number of rectangular matrix

j	Index of column number of rectangular matrix

k	Index of bit sequence

###### 4.2.3.2.3.1	Bits-input to rectangular matrix with padding

The bit sequence  input to the Turbo code internal interleaver is written into the rectangular matrix as follows.

(1)	Determine the number of rows of the rectangular matrix, R, such that:

.

The rows of rectangular matrix are numbered 0, 1, …, R - 1 from top to bottom.

(2)	Determine the prime number to be used in the intra-permutation, p, and the number of columns of rectangular matrix, C, such that:

if (481  K  530) then

p = 53 and C = p.

else

Find minimum prime number p from table 2 such that

,

and determine C such that

.

end if

The columns of rectangular matrix are numbered 0, 1, …, C - 1 from left to right.

Table 2: List of prime number p and associated primitive root v

| p | v | p | v | p | v | p | v | p | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 3 | 47 | 5 | 101 | 2 | 157 | 5 | 223 | 3 |
| 11 | 2 | 53 | 2 | 103 | 5 | 163 | 2 | 227 | 2 |
| 13 | 2 | 59 | 2 | 107 | 2 | 167 | 5 | 229 | 6 |
| 17 | 3 | 61 | 2 | 109 | 6 | 173 | 2 | 233 | 3 |
| 19 | 2 | 67 | 2 | 113 | 3 | 179 | 2 | 239 | 7 |
| 23 | 5 | 71 | 7 | 127 | 3 | 181 | 2 | 241 | 7 |
| 29 | 2 | 73 | 5 | 131 | 2 | 191 | 19 | 251 | 6 |
| 31 | 3 | 79 | 3 | 137 | 3 | 193 | 5 | 257 | 3 |
| 37 | 2 | 83 | 2 | 139 | 2 | 197 | 2 |  |  |
| 41 | 6 | 89 | 3 | 149 | 2 | 199 | 3 |  |  |
| 43 | 3 | 97 | 5 | 151 | 6 | 211 | 2 |  |  |

(3)	Write the input bit sequence  into the R C rectangular matrix row by row starting with bit y1 in column 0 of row 0:

.

where yk = xk for k = 1, 2, …, K and if RCK, the dummy bits are padded such that  for k = K + 1, K + 2, …, RC. These dummy bits are pruned away from the output of the rectangular matrix after intra-row and inter-row permutations.

4.2.3.2.3.2	Intra-row and inter-row permutations

After the bits-input to the RC rectangular matrix, the intra-row and inter-row permutations for the RC rectangular matrix are performed stepwise by using the following algorithm with steps (1) – (6).

(1)	Select a primitive root v from table 2 in section 4.2.3.2.3.1, which is indicated on the right side of the prime number p.

(2)	Construct the base sequence  for intra-row permutation as:

,	j = 1, 2,… (p - 2), and s(0) = 1.

(3)	Assign q0 = 1 to be the first prime integer in the sequence , and determine the prime integer qi in the sequence  to be a least prime integer such that g.c.d(qi, p - 1) = 1, qi > 6, and qi > q(i - 1) for each i = 1, 2, …, R – 1. Here g.c.d. is greatest common divisor.

(4)	Permute the sequence  to make the sequence  such that

rT(i) = qi,  i = 0, 1, …. , R - 1,

where  is the inter-row permutation pattern defined as the one of the four kind of patterns, which are shown in table 3, depending on the number of input bits K.

Table 3: Inter-row permutation patterns for Turbo code internal interleaver

| Number of input bits K | Number of rows R | Inter-row permutation patterns <T(0), T(1), …, T(R - 1)> |
| --- | --- | --- |
| (40K159) | 5 | <4, 3, 2, 1, 0> |
| (160K200) or (481K530) | 10 | <9, 8, 7, 6, 5, 4, 3, 2, 1, 0> |
| (2281K2480) or (3161K3210) | 20 | <19, 9, 14, 4, 0, 2, 5, 7, 12, 18, 16, 13, 17, 15, 3, 1, 6, 11, 8, 10> |
| K = any other value | 20 | <19, 9, 14, 4, 0, 2, 5, 7, 12, 18, 10, 8, 13, 17, 3, 1, 16, 6, 15, 11> |

(5)	Perform the i-th (i = 0,1, …, R - 1) intra-row permutation as:

if (C = p) then

,    j = 0, 1, …, (p - 2), and Ui(p - 1) = 0,

where Ui(j) is the original bit position of j-th permuted bit of i-th row.

end if

if (C = p + 1) then

,     j = 0, 1, …, (p - 2).  Ui(p - 1) = 0, and Ui(p) = p,

where Ui(j) is the original bit position of j-th permuted bit of i-th row, and

if (K =RC) then

Exhange UR-1(p) with UR-1(0).

end if

end if

if (C = p - 1) then

,    j =0, 1, …, (p - 2),

where Ui(j) is the original bit position of j-th permuted bit of i-th row.

end if

(6)	Perform the inter-row permutation for the rectangular matrix based on the pattern ,

where T(i) is the original row position of the i-th permuted row.

4.2.3.2.3.3	Bits-output from rectangular matrix with pruning

After intra-row and inter-row permutations, the bits of the permuted rectangular matrix are denoted by y'k:

The output of the Turbo code internal interleaver is the bit sequence read out column by column from the intra-row and inter-row permuted R  C rectangular matrix starting with bit y'1 in row 0 of column 0 and ending with bit y'CR in row R - 1 of column C - 1. The output is pruned by deleting dummy bits that were padded to the input of the rectangular matrix before intra-row and inter row permutations, i.e. bits y'k that corresponds to bits yk with k > K are removed from the output. The bits output from Turbo code internal interleaver are denoted by x'1, x'2, …, x'K, where x'1 corresponds to the bit y'k with smallest index k after pruning, x'2 to the bit y'k with second smallest index k after pruning, and so on. The number of bits output from Turbo code internal interleaver is K and the total number of pruned bits is:

RC – K.

#### 4.2.3.3	Concatenation of encoded blocks

After the channel coding for each code block, if Ci is greater than 1, the encoded blocks are serially concatenated so that the block with lowest index r is output first from the channel coding block, otherwise the encoded block is output from channel coding block as it is. The bits output are denoted by , where i is the TrCH number and Ei = CiYi. The output bits are defined by the following relations:

k = 1, 2, …, Yi

k = Yi + 1, Yi + 2, …, 2Yi

k = 2Yi + 1, 2Yi + 2, …, 3Yi

k = (Ci - 1)Yi + 1, (Ci - 1)Yi + 2, …, CiYi

If no code blocks are input to the channel coding (Ci = 0), no bits shall be output from the channel coding, i.e. Ei = 0.

### 4.2.4	Radio frame size equalisation

Radio frame size equalisation is padding the input bit sequence in order to ensure that the output can be segmented in Fi data segments of same size as described in the subclause 4.2.6.

The input bit sequence to the radio frame size equalisation is denoted by, where i is TrCH number and Ei the number of bits. The output bit sequence is denoted by, where Ti is the number of bits. The output bit sequence is derived as follows:

tik = cik, for k = 1… Ei  and

tik = {0 , 1} for k= Ei +1… Ti, if Ei < Ti

where

Ti = Fi * Ni  and

is the number of bits per segment after size equalisation.

### 4.2.5	1st interleaving

The 1st interleaving is a block interleaver with inter-column permutations. The input bit sequence to the block interleaver is denoted by , where i is TrCH number and Xi the number of bits. Here Xi is guaranteed to be an integer multiple of the number of radio frames in the TTI. The output bit sequence from the block interleaver is derived as follows:

1)	select the number of columns C1 from table 4 depending on the TTI. The columns are numbered 0, 1, …, C1 - 1 from left to right.

2)	determine the number of rows of the matrix, R1 defined as

R1 = Xi / C1.

The rows of the matrix are numbered 0, 1, …, R1 - 1 from top to bottom.

3)	write the input bit sequence into the R1  C1 matrix row by row starting with bit  in column 0 of row 0 and ending with bit  in column C1 - 1 of row R1 – 1:

4)	Perform the inter-column permutation for the matrix based on the pattern  shown in table 4, where P1C1(j) is the original column position of the j-th permuted column. After permutation of the columns, the bits are denoted by yi,k:

5)	Read the output bit sequence  of the block interleaver column by column from the inter-column permuted R1  C1 matrix. Bit  corresponds to row 0 of column 0 and bit  corresponds to row R1 - 1 of column C1 - 1.

Table 4 Inter-column permutation patterns for 1st interleaving

| TTI | Number of columns C1 | Inter-column permutation patterns <P1C1(0), P1C1(1), …, P1C1(C1-1)> |
| --- | --- | --- |
| 5ms(*1), 10 ms | 1 | <0> |
| 20 ms | 2 | <0,1> |
| 40 ms | 4 | <0,2,1,3> |
| 80 ms | 8 | <0,4,2,6,1,5,3,7> |

(*1) can be used for PRACH for 1.28 Mcps TDD

#### 4.2.5.1	Relation between input and output of 1st interleaving

The bits input to the 1st interleaving are denoted by , where i is the TrCH number and Ti the number of bits. Hence, xi,k = ti,k and Xi = Ti.

The bits output from the 1st interleaving are denoted by , and di,k = yi,k.

### 4.2.6	Radio frame segmentation

When the transmission time interval is longer than 10 ms, the input bit sequence is segmented and mapped onto consecutive Fi radio frames. Following radio frame size equalisation the input bit sequence length is guaranteed to be an integer multiple of Fi.

The input bit sequence is denoted by  where i is the TrCH number and Xi is the number bits. The Fi output bit sequences per TTI are denoted by where ni is the radio frame number in current TTI and Yi is the number of bits per radio frame for TrCH i. The output sequences are defined as follows:

= , ni = 1…Fi, k = 1…Yi

where

Yi = (Xi / Fi) is the number of bits per segment.

The ni –th segment is mapped to the ni –th radio frame of the transmission time interval.

The input bit sequence to the radio frame segmentation is denoted by , where i is the TrCH number and Ti the number of bits. Hence, xik = dik and Xi = Ti.

The output bit sequence corresponding to radio frame ni is denoted by , where i is the TrCH number and Ni is the number of bits. Hence, and Ni = Yi.

### 4.2.7	Rate matching

Rate matching means that bits on a TrCH are repeated or punctured. Higher layers assign a rate-matching attribute for each TrCH. This attribute is semi-static and can only be changed through higher layer signalling. The rate-matching attribute is used when the number of bits to be repeated or punctured is calculated.

The number of bits on a TrCH can vary between different transmission time intervals. When the number of bits between different transmission time intervals is changed, bits are repeated to ensure that the total bit rate after TrCH multiplexing is identical to the total channel bit rate of the allocated physical channels.

If no bits are input to the rate matching for all TrCHs within a CCTrCH, the rate matching shall output no bits for all TrCHs within the CCTrCH.

Notation used in subclause 4.2.7 and subclauses:

Nij :	Number of bits in a radio frame before rate matching on TrCH i with transport format combination j.

:	If positive – number of bits to be repeated in each radio frame on TrCH i with transport format combination j.

If negative – number of bits to be punctured in each radio frame on TrCH i with transport format combination j.

RMi :	Semi-static rate matching attribute for TrCH i. Signalled from higher layers.

PL :	Puncturing limit. This value limits the amount of puncturing that can be applied in order to minimise the number of physical channels. Signalled from higher layers. The allowed puncturing in % is actually equal to (1-PL)*100.

Ndata,j :	Total number of bits that are available for a CCTrCH in a radio frame with transport format combination j.

P :	number of physical channels used in the current frame.

Pmax :	maximum number of physical channels allocated for a CCTrCH.

Up :	Number of data bits in the physical channel p with p = 1...P during a radio frame.

I :	Number of TrCHs in a CCTrCH.

Zij :	Intermediate calculation variable.

Fi :	Number of radio frames in the transmission time interval of TrCH i.

ni :	Radio frame number in the transmission time interval of TrCH i (0  ni < Fi).

q :	Average puncturing or repetition distance(normalised to only show the remaining rate matching on top of an integer number of repetitions).

P1F(ni) :	The column permutation function of the 1st interleaver, P1F(x) is the original position of column with number x after permutation. P1 is defined on table 4 of section 4.2.5 (note that P1F self-inverse).

S[n] :	The shift of the puncturing or repetition pattern for radio frame ni when .

TFi(j) :	Transport format of TrCH i for the transport format combination j.

TFS(i) :	The set of transport format indexes l for TrCH i.

eini :	Initial value of variable e in the rate matching pattern determination algorithm of subclause 4.2.7.3.

eplus :	Increment of variable e in the rate matching pattern determination algorithm of subclause 4.2.7.3.

eminus :	Decrement of variable e in the rate matching pattern determination algorithm of subclause 4.2.7.3.

b :	Indicates systematic and parity bits.

b=1: Systematic bit. X(t) in subclause 4.2.3.2.1.

b=2: 1st parity bit (from the upper Turbo constituent encoder). Y(t) in subclause 4.2.3.2.1.

b=3: 2nd parity bit (from the lower Turbo constituent encoder). Y'(t) in subclause 4.2.3.2.1.

Note: when the TTI is 5msec for 1.28Mcps, the above notation refers to a sub-frame rather than a radio frame. In this case, Fi = 1 and ni = 0.

#### 4.2.7.1	Determination of rate matching parameters

The following relations, defined for all TFC j, are used when calculating the rate matching pattern:

for all i = 1 ... I (1)

for all i = 1 ... I

Puncturing can be used to minimise the required transmission capacity. The maximum amount of puncturing that can be applied is 1-PL, PL is signalled from higher layers. The possible values for Ndata depend on the number of physical channels Pmax , allocated to the respective CCTrCH, and on their characteristics (spreading factor, length of midamble and TFCI code word, usage of TPC and multiframe structure), which is given in [7].

For each physical channel an individual minimum spreading factor Spmin is transmitted by means of the higher layers. Denote the number of data bits in each physical channel by Up,Sp , where p indicates the sequence number 1 p Pmax and Sp indicates the spreading factor of this physical channel: Sp takes the possible values {16, 8, 4, 2, 1} for 1.28Mcps TDD and 3.84Mcps TDD, Sp takes the possible values {32, 16, 8, 4, 2, 1} for 7.68Mcps TDD. The index p is described in section 4.2.12 with the following modifications: spreading factor (Q) is replaced by the minimum spreading factor Spmin and k is replaced by the channelization code index at Q= Spmin. Then, for Ndata one of the following values in ascending order can be chosen:

Optionally, if indicated by higher layers for the UL the UE shall vary the spreading factor autonomously, so that Ndata is one of the following values in ascending order:

Ndata, j for the transport format combination j is determined by executing the following algorithm:

SET1 = { Ndata such that  is non negative }

Ndata, j = min SET1

The number of bits to be repeated or punctured, Ni,j, within one radio frame (one sub-frame when the TTI is 5msec) for each TrCH i is calculated with the relations given at the beginning of this subclause for all possible transport format combinations j and selected every radio frame (sub-frame). The number of physical channels corresponding to Ndata, j, shall be denoted by P.

If Ni,j = 0 then the output data of the rate matching is the same as the input data and the rate matching algorithm of subclause 4.2.7.3 does not need to be executed.

Otherwise, the rate matching pattern is calculated with the algorithm described in subclause 4.2.7.3. For this algorithm the parameters eini, eplus, eminus, and Xi are needed, which are calculated according to the equations in subclauses 4.2.7.1.1 and 4.2.7.1.2.

##### 4.2.7.1.1	Uncoded and convolutionally encoded TrCHs

a = 2

Ni = Ni,j

Xi = Ni,j

R = Ni,j mod Ni,j -- note: in this context Ni,j mod Ni,j is in the range of 0 to Ni,j-1  i.e. -1 mod 10 = 9.

if R  0 and 2R  Ni,j

then q = Ni,j / R

else

q = Ni,j / (R - Ni,j)

endif

NOTE 1:	q is a signed quantity.

If q is even

then q' = q + gcd(q, Fi) / Fi -- where gcd (q, Fi) means greatest common divisor of q and Fi

NOTE 2:	q' is not an integer, but a multiple of 1/8.

else

q' = q

endif

for x = 0 to Fi-1

S[xq' mod Fi] = (x*q' div Fi)

end for

eini = (a  S[P1Fi(ni)]  |Ni | + 1) mod (a  Ni,j)

eplus = a  Xi

eminus = a  |Ni|

puncturing for Ni <0, repetition otherwise.

##### 4.2.7.1.2	Turbo encoded TrCHs

If repetition is to be performed on turbo encoded TrCHs, i.e. Ni,j >0, the parameters in subclause 4.2.7.1.1 are used.

If puncturing is to be performed, the parameters below shall be used. Index b is used to indicate systematic (b=1), 1st parity (b=2), and 2nd parity bit (b=3).

a = 2 when b=2

a = 1 when b=3

If  is calculated as 0 for b=2 or b=3, then the following procedure and the rate matching algorithm of subclause 4.2.7.3 don't need to be performed for the corresponding parity bit stream.

Xi = Ni,j /3 ,

q = Xi /|Ni| 

if(q2)

for r=0 to Fi-1

S[(3r+b-1) mod Fi] = r mod 2;

end for

else

if q is even

then q = q – gcd(q, Fi)/ Fi   -- where gcd (q, Fi) means greatest common divisor of q and Fi

NOTE:	q is not an integer, but a multiple of 1/8.

else	q = q

endif

for x=0 to Fi –1

r = xq' mod Fi;

S[(3r+b-1) mod Fi] = xq div Fi;

endfor

endif

For each radio frame, the rate-matching pattern is calculated with the algorithm in subclause 4.2.7.3, where:

Xi is as above,

eini = (aS[P1 Fi (ni)]|Ni| + Xi) mod (aXi), if eini = 0 then eini = aXi

eplus = aXi

eminus = a|Ni|

#### 4.2.7.2	Bit separation and collection for rate matching

The systematic bits of turbo encoded TrCHs shall not be punctured, the other bits may be punctured. The systematic bits, first parity bits, and second parity bits in the bit sequence input to the rate matching block are therefore separated into three sequences.

The first sequence contains:

-	All of the systematic bits that are from turbo encoded TrCHs.

-	From 0 to 2 first and/or second parity bits that are from turbo encoded TrCHs. These bits come into the first sequence when the total number of bits in a block after radio frame segmentation is not a multiple of three.

-	Some of the systematic, first parity and second parity bits that are for trellis termination.

The second sequence contains:

-	All of the first parity bits that are from turbo encoded TrCHs, except those that go into the first sequence when the total number of bits is not a multiple of three.

-	Some of the systematic, first parity and second parity bits that are for trellis termination.

The third sequence contains:

-	All of the second parity bits that are from turbo encoded TrCHs, except those that go into the first sequence when the total number of bits is not a multiple of three.

-	Some of the systematic, first parity and second parity bits that are for trellis termination.

The second and third sequences shall be of equal length, whereas the first sequence can contain from 0 to 2 more bits. Puncturing is applied only to the second and third sequences.

The bit separation function is transparent for uncoded TrCHs, convolutionally encoded TrCHs, and for turbo encoded TrCHs with repetition. The bit separation and bit collection are illustrated in figures 4 and 5.

Figure 4: Puncturing of turbo encoded TrCHs

Figure 5: Rate matching for uncoded TrCHs, convolutionally encoded TrCHs,
and for turbo encoded TrCHs with repetition

The bit separation is dependent on the 1st interleaving and offsets are used to define the separation for different TTIs. b indicates the three sequences defined in this section, with b=1 indicating the first sequence, b = 2 the second one, and b = 3 the third one.

The offsets b for these sequences are listed in table 5.

Table 5: TTI dependent offset needed for bit separation

| TTI (ms) | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 5, 10, 40 | 0 | 1 | 2 |
| 20, 80 | 0 | 2 | 1 |

The bit separation is different for different radio frames in the TTI. A second offset is therefore needed. The radio frame number for TrCH i is denoted by ni. and the offset by .

Table 6: Radio frame dependent offset needed for bit separation

| TTI (ms) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5, 10 | 0 | NA | NA | NA | NA | NA | NA | NA |
| 20 | 0 | 1 | NA | NA | NA | NA | NA | NA |
| 40 | 0 | 1 | 2 | 0 | NA | NA | NA | NA |
| 80 | 0 | 1 | 2 | 0 | 1 | 2 | 0 | 1 |

##### 4.2.7.2.1	Bit separation

The bits input to the rate matching are denoted by , where i is the TrCH number and Ni is the number of bits input to the rate matching block. Note that the transport format combination number j for simplicity has been left out in the bit numbering, i.e. Ni=Nij. The bits after separation are denoted by . For turbo encoded TrCHs with puncturing, b indicates the three sequences defined in section 4.2.7.2, with b=1 indicating the first sequence, and so forth. For all other cases b is defined to be 1. Xi is the number of bits in each separated bit sequence. The relation between ei,k and xb,i,k is given below.

For turbo encoded TrCHs with puncturing:

k = 1, 2, 3, …, Xi	Xi  = Ni /3

k = 1, …, Ni mod 3	Note: When (Ni mod 3) = 0 this row is not needed.

k = 1, 2, 3, …, Xi	Xi  = Ni /3

k = 1, 2, 3, …, Xi	Xi = Ni /3

For uncoded TrCHs, convolutionally encoded TrCHs, and turbo encoded TrCHs with repetition:

k = 1, 2, 3, …, Xi	Xi = Ni

##### 4.2.7.2.2	Bit collection

The bits xb,i,k are input to the rate matching algorithm described in subclause 4.2.7.3. The bits output from the rate matching algorithm are denoted .

Bit collection is the inverse function of the separation. The bits after collection are denoted by . After bit collection, the bits indicated as punctured are removed and the bits are then denoted by , where i is the TrCH number and Vi = Ni,j+Ni,j. The relations between yb,i,k, zb,i,k, and fi,k are given below.

For turbo encoded TrCHs with puncturing (Yi=Xi):

k = 1, 2, 3, …, YI

k = 1, …, Ni mod 3	Note: When (Ni mod 3) = 0 this row is not needed.

k = 1, 2, 3, …, Yi

k = 1, 2, 3, …, Yi

After the bit collection, bits zi,k with value , where {0, 1}, are removed from the bit sequence. Bit fi,1 corresponds to the bit zi,k with smallest index k after puncturing, bit fi,2 corresponds to the bit zi,k with second smallest index k after puncturing, and so on.

For uncoded TrCHs, convolutionally encoded TrCHs, and turbo encoded TrCHs with repetition:

k = 1, 2, 3, …, Yi

When repetition is used, fi,k=zi,k and Yi=Vi.

When puncturing is used, Yi=Xi and bits zi,k with value , where {0, 1}, are removed from the bit sequence. Bit fi,1 corresponds to the bit zi,k with smallest index k after puncturing, bit fi,2 corresponds to the bit zi,k with second smallest index k after puncturing, and so on.

#### 4.2.7.3	Rate matching pattern determination

The bits input to the rate matching are denoted by , where i is the TrCH and Xi is the parameter given in subclauses 4.2.7.1.1 and 4.2.7.1.2.

NOTE:	The transport format combination number j for simplicity has been left out in the bit numbering.

The rate matching rule is as follows:

if puncturing is to be performed

e = eini	-- initial error between current and desired puncturing ratio

m = 1	-- index of current bit

do while m <= Xi

e = e – eminus	-- update error

if e <= 0 then	-- check if bit number m should be punctured

set bit xi,m to  where {0, 1}

e = e + eplus	-- update error

end if

m = m + 1	--  next bit

end do

else

e = eini	-- initial error between current and desired puncturing ratio

m = 1	-- index of current bit

do while m <= Xi

e = e – eminus	-- update error

do while e <= 0	-- check if bit number m should be repeated

repeat bit xi,m

e = e + eplus	-- update error

end do

m = m + 1	--  next bit

end do

end if

A repeated bit is placed directly after the original one.

### 4.2.8	TrCH multiplexing

Every 10 ms, one radio frame from each TrCH is delivered to the TrCH multiplexing. These radio frames are serially multiplexed into a coded composite transport channel (CCTrCH). If the TTI is smaller than 10ms, then no TrCH multiplexing is performed.

The bits input to the TrCH multiplexing are denoted by , where i is the TrCH id number and Vi is the number of bits in the radio frame of TrCH i. The number of TrCHs is denoted by I. The bits output from TrCH multiplexing are denoted by , where S is the number of bits, i.e. . The TrCH multiplexing is defined by the following relations:

k = 1, 2, …, V1

k = V1+1, V1+2, …, V1+V2

k = (V1+V2)+1, (V1+V2)+2, …, (V1+V2)+V3

k = (V1+V2+…+VI-1)+1, (V1+V2+…+VI-1)+2, …, (V1+V2+…+VI-1)+VI

### 4.2.9	Bit Scrambling

The bits output from the TrCH multiplexer are scrambled in the bit scrambler. The bits input to the bit scrambler are denoted by, where S is the number of bits input to the bit scrambling block equal to the total number of bits on the CCTrCH. The bits after bit scrambling are denoted .

Bit scrambling is defined by the following relation:

and  results from the following operation:

;  ; ;

### 4.2.10	Physical channel segmentation

When more than one PhCH is used, physical channel segmentation divides the bits among the different PhCHs. The bits input to the physical channel segmentation are denoted by , where S is the number of bits input to the physical channel segmentation block. The number of PhCHs after rate matching is denoted by P, as defined in subclause 4.2.7.1.

The bits after physical channel segmentation are denoted , where p is PhCH number and Up is the in general variable number of bits in the respective radio frame for each PhCH. The relation between sk and up,k is given below.

Bits on first PhCH after physical channel segmentation:

k = 1, 2 , …, U1

Bits on second PhCH after physical channel segmentation:

k = 1, 2 , …, U2

…

Bits on the Pth PhCH after physical channel segmentation:

k = 1, 2 , …, UP

### 4.2.11	2nd interleaving

The 2nd interleaving is a block interleaver and consists of bits input to a matrix with padding, the inter-column permutation for the matrix and bits output from the matrix with pruning. The 2nd interleaving can be applied jointly to all data bits transmitted during one frame, or separately within each timeslot, on which the CCTrCH is mapped. The selection of the 2nd interleaving scheme is controlled by higher layer.

#### 4.2.11.1	Frame related 2nd interleaving

In case of frame related 2nd interleaving, the bits input to the block interleaver are denoted by , where U is the total number of bits after TrCH multiplexing transmitted during the respective radio frame with .

The relation between xk and the bits up,k in the respective physical channels is given below:

k = 1, 2 , …, U1

k = 1, 2 , …, U2

…

k = 1, 2 , …, UP

The following steps have to be performed once for each CCTrCH:

(1)	Assign C2 = 30 to be the number of columns of the matrix. The columns of the matrix are numbered 0, 1, 2, …, C2 - 1 from left to right.

(2)	Determine the number of rows of the matrix, R2, by finding minimum integer R2 such that:

U  R2 X C2.

The rows of rectangular matrix are numbered 0, 1, 2, …, R2 - 1 from top to bottom.

(3)	Write the input bit sequence  into the R2  C2 matrix row by row starting with bit  in column 0 of row 0:

where  for k = 1, 2, …, U and if R2  C2 > U, the dummy bits are padded such that  = 0 or 1 for k = U + 1, U + 2, …, R2  C2. These dummy bits are pruned away from the output of the matrix after the inter-column permutation.

(4)	Perform the inter-column permutation for the matrix based on the pattern  that is shown in table 7, where P2(j) is the original column position of the j-th permuted column. After permutation of the columns, the bits are denoted by .

(5)	The output of the block interleaver is the bit sequence read out column by column from the inter-column permuted R2  C2 matrix. The output is pruned by deleting dummy bits that were padded to the input of the matrix before the inter-column permutation, i.e. bits  that corresponds to bits  with k > U are removed from the output. The bits at the output of the block interleaver are denoted by z1,z2,…,zU, where z1 corresponds to the bit  with smallest index k after pruning, z2 to the bit  with second smallest index k after pruning, and so on.

The bits z1,z2,…,zU shall be segmented as follows:

u1,k=zk	k = 1, 2 , …, U1

u2,k=z(k+U1)	k = 1, 2 , …, U2

…

uP,k=z(k+U1+…+UP−1)	k = 1, 2 , …, UP

The bits after frame related 2nd interleaving are denoted by vt,1,vt,2,...,vt,Ut, where t refers to the timeslot sequence number and Ut is the number of bits transmitted in this timeslot during the respective radio frame.

Let T be the number of time slots in a CCTrCH during the respective radio frame (where for 1.28Mcps TDD, the respective radio frame includes subframes 1 and 2), and t=1,…,T. The physical layer shall assign the time slot sequence number t in ascending order of the allocated time slots in the CCTrCH in the respective radio frame. In time slot t, Rt refers to the number of physical channels within the respective time slot and r=1,…,Rt. The relation between r and t and the physical channel sequence number p as detailed in 4.2.12.1 is given by:

p=r t=1

p=R1+R2,…,Rt−1+r 1<t≤T

Defining the relation  ut,r,k = up,k   and denoting Utr as the number of bits for physical channel r in time slot t, the relation between vt,k and ut,r,k  is given below:

vt,k = ut,1,k	k = 1, 2 , …, Ut1

vt,(k+Ut1) = ut,2,k	k = 1, 2 , …, Ut2

…

vt,(k+Ut1+ ... +UtRt−1) = ut,Rt,k	k = 1, 2 , …, UtRt

#### 4.2.11.2	Timeslot related 2nd interleaving

In case of timeslot related 2nd interleaving, the bits input to the block interleaver are denoted by , where t is the timeslot sequence number, and Ut is the number of bits transmitted in this timeslot during the respective radio frame.

Let T be the number of time slots in a CCTrCH during the respective radio frame (where for 1.28Mcps TDD, the respective radio frame includes subframes 1 and 2), andt=1,…,T. The physical layer shall assign the time slot sequence number t in ascending order of the allocated time slots in the CCTrCH in the respective radio frame. In timeslot t, Rt refers to the number of physical channels within the respective timeslot and r=1,…,Rt. The relation between r and t and the physical channel sequence number p as detailed in 4.2.12.1 is given by:

p=r t=1

p=R1+R2,…,Rt−1+r 1<t≤T

Defining the relation ut,r,k = up,k  and  denoting Utr as the number of bits for physical channel r in time slot t, the relation between xt,k and ut,r,k is given below:

k = 1, 2 , …,

k = 1, 2 , …,

…

xt,(k+Ut1+ ... +UtRt−1) = ut,Rt,k	k = 1, 2 , …, UtRt

The following steps have to be performed for each timeslot t, on which the respective CCTrCH is mapped:

(1)	Assign C2 = 30 to be the number of columns of the matrix. The columns of the matrix are numbered 0, 1, 2, …, C2 - 1 from left to right.

(2)	Determine the number of rows of the matrix, R2, by finding minimum integer R2 such that:

Ut  R2  C2.

The rows of rectangular matrix are numbered 0, 1, 2, …, R2 - 1 from top to bottom.

(3)	Write the input bit sequence  into the R2  C2 matrix row by row starting with bit  in column 0 of row 0:

where  for k = 1, 2, …, Ut and if R2  C2 > Ut, the dummy bits are padded such that  = 0 or 1 for k = Ut + 1, Ut + 2, …, R2  C2. These dummy bits are pruned away from the output of the matrix after the inter-column permutation.

(4)	Perform the inter-column permutation for the matrix based on the pattern  that is shown in table 7, where P2(j) is the original column position of the j-th permuted column. After permutation of the columns, the bits are denoted by .

(5)	The output of the block interleaver is the bit sequence read out column by column from the inter-column permuted R2  C2 matrix. The output is pruned by deleting dummy bits that were padded to the input of the matrix before the inter-column permutation, i.e. bits  that corresponds to bits  with k > Ut are removed from the output. The bits after time slot 2nd interleaving are denoted by , where vt,1 corresponds to the bit  with smallest index k after pruning, vt,2 to the bit  with second smallest index k after pruning, and so on.

Table 7 Inter-column permutation pattern for 2nd interleaving

| Number of Columns C2 | Inter-column permutation pattern < P2(0), P2(1), …, P2(C2-1) > |
| --- | --- |
| 30 | <0, 20, 10, 5, 15, 25, 3, 13, 23, 8, 18, 28, 1, 11, 21, 6, 16, 26, 4, 14, 24, 19, 9, 29, 12, 2, 7, 22, 27, 17> |

### 4.2.11A	Sub-frame segmentation for the 1.28 Mcps option

In the 1.28Mcps TDD, it is needed to add a sub-frame segmentation unit between 2nd interleaving unit and physical channel mapping unit when the TTI of the CCTrCh is greater than 5msec. In this case, the operation of rate-matching guarantees that the size of bit streams is an even number and can be subdivided into 2 sub-frames. The transport channel multiplexing structure for uplink and downlink is shown in figure 1A.

The input to the sub-frame segmentation unit is segmented into timeslot chunks, where each timeslot chunk contains all of the bits that are to be transmitted in a given timeslot position in both of the sub-frames.

The input bit sequence is denoted by  where i is the timeslot number and Xi is the number of bits transmitted in timeslot i in a radio frame. The two output bit sequences per radio frame are denoted by   yi,n,1, yi,n,2, yi,n,3, …, yi,n,Yi where n is the sub-frame number in current radio frame and Yi is the number of bits per sub-frame for timeslot i. The output sequences are defined as follows:

yi,n,k= xi,((n−1).Yi)+k, n = 1 or 2, k = 1…Yi

where

Yi = (Xi / 2) is the number of bits in timeslot i per sub-frame,

is the kth bit of the input bit sequence and

yi,n,kis the kth bit of the output bit sequence corresponding to the nth sub-frame

### 4.2.12	Physical channel mapping

#### 4.2.12.1	Physical channel mapping for the 3.84 Mcps and 7.68Mcps options

The PhCH for both uplink and downlink is defined in [7]. The bits after physical channel mapping are denoted by , where p is the PhCH number corresponding to the sequence number 1 p P of this physical channel as detailed below, Up is the number of bits in one radio frame for the respective PhCH, and P. Pmax. The bits wp,k are mapped to the PhCHs so that the bits for each PhCH are transmitted over the air in ascending order with respect to k.

The physical layer shall assign the physical channel sequence number p to the physical channels of the CCTrCH in the respective radio frame, treating each allocated timeslot in ascending order.  If within a timeslot there are multiple physical channels they shall first be ordered in ascending order of the spreading factor (Q) and subsequently by channelisation code index (k), as shown in [9].

The mapping of the bits vt,1,vt,2,...,vt,Ut is performed like block interleaving, writing the bits into columns, but a PhCH with an odd number is filled in forward order, whereas a PhCH with an even number is filled in reverse order.

The mapping scheme, as described in the following subclause, shall be applied individually for each timeslot t used in the current frame. Therefore, the bits  are assigned to the bits of the physical channels  in each timeslot.

In uplink there are at most two codes allocated (P2). If there is only one code, the same mapping as for downlink is applied. Denote SF1 and SF2 the spreading factors used for code 1 and 2, respectively. For the number of consecutive bits to assign per code bsk the following rule is applied:

if

SF1 >= SF2  then bs1 = 1  ; bs2 =  SF1/SF2 ;

else

SF2 > SF1  then bs1 = SF2/SF1; bs2 = 1 ;

end if

In the downlink case bsp is 1 for all physical channels.

##### 4.2.12.1.1	Mapping scheme

Notation used in this subclause:

P t:	number of physical channels for timeslot t , Pt = 1..2 for uplink ; Pt = 1...16 for downlink for 1.28Mcps TDD and 3.84Mcps TDD, Pt = 1...32 for 7.68Mcps TDD

Ut,p:	capacity in bits for the physical channel p in timeslot t

Ut.:	total number of bits to be assigned for timeslot t

bsp:	number of consecutive bits to assign per code

for downlink all bsp = 1

for uplink	if SF1 >= SF2  then bs1 = 1  ; bs2 =  SF1/SF2 ;

if SF2 > SF1  then bs1 = SF2/SF1; bs2 = 1 ;

fbp:	number of already written bits for each code

pos:	intermediate calculation variable

for  p=1  to P t	-- reset number of already written bits for every physical channel

fbp = 0

end for

p = 1	-- start with PhCH #1

for k=1  to Ut.

do while (fbp == Ut,p)	-- physical channel filled up already ?

p = (p mod Pt) + 1;

end do

if (p mod 2) == 0

pos = Ut,p - fbp	-- reverse order

else

pos = fbp + 1	-- forward order

endif

wt,p,pos = vt,k	-- assignment

fbp = fbp + 1	-- Increment number of already written bits

if (fbp mod bsp) == 0	-- Conditional change to the next physical channel

p = (p mod Pt) + 1;

end if

end for

#### 4.2.12.2	Physical channel mapping for the 1.28 Mcps option

The bit streams from the sub-frame segmentation unit are mapped onto code channels of time slots in sub-frames.

The bits after physical channel mapping are denoted by , where p is the PhCH number and Up is the number of bits in one sub-frame for the respective PhCH. The bits wpk are mapped to the PhCHs so that the bits for each PhCH are transmitted over the air in ascending order with respect to k.

The mapping of the bits yt,n,1,  yt,n,2, yt,n,3, …, yt,n,Ut is performed like block interleaving, writing the bits into columns, but a PhCH with an odd number is filled in forward order, were as a PhCH with an even number is filled in reverse order.

The mapping scheme, as described in the following subclause, shall be applied individually for each timeslot t used in the current subframe. Therefore, the bits yt,n,1,  yt,n,2, yt,n,3, …, yt,n,Ut are assigned to the bits of the physical channels  in each timeslot.

In uplink there are at most two codes allocated (P2). If there is only one code, the same mapping as for downlink is applied. Denote SF1 and SF2 the spreading factors used for code 1 and 2, respectively. For the number of consecutive bits to assign per code bsk the following rule is applied:

if

SF1 >= SF2  then bs1 = 1  ; bs2 =  SF1/SF2 ;

else

SF2 > SF1  then bs1 = SF2/SF1; bs2 = 1 ;

end if

In the downlink case bsp is 1 for all physical channels.

##### 4.2.12.2.1	Mapping scheme

Notation used in this subclause:

P t:	number of physical channels for timeslot t , Pt = 1..2 for uplink ; Pt = 1...16 for downlink

Utp:	capacity in bits for the physical channel p in timeslot t in the current sub-frame

Ut.:	total number of bits to be assigned for timeslot t in the current sub-frame

n = index of the current sub-frame (1 or 2)

bsp:	number of consecutive bits to assign per code

for downlink all bsp = 1

for uplink	if SF1 >= SF2  then bs1 = 1  ; bs2 =  SF1/SF2 ;

if SF2 > SF1  then bs1 = SF2/SF1; bs2 = 1 ;

fbp:	number of already written bits for each code

pos:	intermediate calculation variable

for  p=1 to P t	-- reset number of already written bits for every physical channel

fbp = 0

end for

p = 1	-- start with PhCH #1

for k=1  to Ut.

do while (fbp == Ut,p)	-- physical channel filled up already ?

p = (p mod P t)  + 1 ;

end do

if (p mod 2) == 0

pos = Ut,p - fbp	-- reverse order

else

pos = fbp + 1	-- forward order

end if

wtp,pos =yt,n,k	-- assignment

fbp = fbp + 1	-- Increment number of already written bits

If (fbp mod bsp) == 0	-- Conditional change to the next physical channel

p = (p mod P t)  + 1 ;

end if

end for

### 4.2.13	Multiplexing of different transport channels onto one CCTrCH, and mapping of one CCTrCH onto physical channels

Different transport channels can be encoded and multiplexed together into one Coded Composite Transport Channel (CCTrCH). The following rules shall apply to the different transport channels which are part of the same CCTrCH:

1)	Transport channels multiplexed into one CCTrCh shall have co-ordinated timings. When the TFCS of a CCTrCH is changed because one or more transport channels are added to the CCTrCH or reconfigured within the CCTrCH, or removed from the CCTrCH, the change may only be made at the start of a radio frame with CFN fulfilling the relation

CFN mod Fmax = 0,

where Fmax denotes the maximum number of radio frames within the transmission time intervals of all transport channels which are multiplexed into the same CCTrCH, including any transport channels i which are added reconfigured or have been removed, and CFN denotes the connection frame number of the first radio frame of the changed CCTrCH.

After addition or reconfiguration of a transport channel i within a CCTrCH, the TTI of transport channel i may only start in radio frames with CFN fulfilling the relation

CFNi mod Fi = 0.

2)	Different CCTrCHs cannot be mapped onto the same physical channel.

3)	One CCTrCH shall be mapped onto one or several physical channels.

4)	Dedicated Transport channels and common transport channels cannot be multiplexed into the same CCTrCH.

5)	For the common transport channels, only the FACH and PCH may belong to the same CCTrCH.

6)	Each CCTrCH carrying a BCH shall carry only one BCH and shall not carry any other Transport Channel.

7)	Each CCTrCH carrying a RACH shall carry only one RACH and shall not carry any other Transport Channel.

Hence, there are two types of CCTrCH.

CCTrCH of dedicated type, corresponding to the result of coding and multiplexing of one or several DCH.

CCTrCH of common type, corresponding to the result of the coding and multiplexing of a common channel, i.e. RACH and USCH in the uplink and DSCH, BCH, FACH or PCH in the downlink, respectively.

Transmission of TFCI is possible for CCTrCH containing Transport Channels of:

-	dedicated type;

-	USCH type;

-	DSCH type;

-	FACH and/or PCH type.

#### 4.2.13.1	Allowed CCTrCH combinations for one UE

##### 4.2.13.1.1	Allowed CCTrCH combinations on the uplink

The following CCTrCH combinations for one UE are allowed, also simultaneously:

1)	several CCTrCH of dedicated type;

2)	several CCTrCH of common type.

##### 4.2.13.1.2	Allowed CCTrCH combinations on the downlink

The following CCTrCH combinations for one UE are allowed, also simultaneously:

3)	several CCTrCH of dedicated type;

4)	several CCTrCH of common type.

### 4.2.14	Transport format detection

Transport format detection can be performed both with and without Transport Format Combination Indicator (TFCI). If a TFCI is transmitted, the receiver detects the transport format combination from the TFCI. When no TFCI is transmitted, so called blind transport format detection may be used, i.e. the receiver side uses the possible transport format combinations as a priori information.

#### 4.2.14.1	Blind transport format detection

Blind Transport Format Detection is optional both in the UE and the UTRAN. Therefore, for all CCTrCH a TFCI shall be transmitted, including the possibility of a TFCI code word length zero, if only one TFC is defined.

#### 4.2.14.2	Explicit transport format detection based on TFCI

##### 4.2.14.2.1	Transport Format Combination Indicator (TFCI)

The Transport Format Combination Indicator (TFCI) informs the receiver of the transport format combination of the CCTrCHs. As soon as the TFCI is detected, the transport format combination, and hence the individual transport channels' transport formats are known, and decoding of the transport channels can be performed.

## 4.3	Coding for layer 1 control for the 3.84 Mcps and 7.68Mcps TDD options

### 4.3.1	Coding of transport format combination indicator (TFCI)

Encoding of the TFCI depends on its length. If there are 6-10 bits of TFCI the channel encoding is done as described in subclause 4.3.1.1. Also specific coding of less than 6 bits is possible as explained in subclause 4.3.1.2.

#### 4.3.1.1	Coding of long TFCI lengths

The TFCI is encoded using a (32, 10) sub-code of the second order Reed-Muller code. The coding procedure is as shown in figure 6.

Figure 6: Channel coding of the TFCI bits

If the TFCI consists of less than 10 bits, it is padded with zeros to 10 bits, by setting the most significant bits to zero. TFCI is encoded by the (32,10) sub-code of second order Reed-Muller code. The code words of the (32,10) sub-code of second order Reed-Muller code are linear combination of some among 10 basis sequences. The basis sequences are as follows in table 8.

Table 8: Basis sequences for (32,10) TFCI code

| I | Mi,0 | Mi,1 | Mi,2 | Mi,3 | MI,4 | Mi,5 | Mi,6 | Mi,7 | Mi,8 | Mi,9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 2 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 3 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 4 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 5 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 6 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 7 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 8 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 9 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 10 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 11 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 12 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 13 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 14 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 15 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 16 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 17 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 18 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 |
| 19 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 20 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 21 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 |
| 22 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 23 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 24 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 25 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 26 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 27 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |
| 28 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 29 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 30 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 31 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |

The TFCI bits a0 , a1 , a2 , a3 , a4 , a5 , a6 , a7 , a8 , a9 (where a0 is LSB and a9 is MSB) shall correspond to the TFC index (expressed in unsigned binary form) defined by the RRC layer to reference the TFC of the CCTrCH in the associated radio frame.

The output TFCI code word bits bi are given by:

where i = 0,…,31. NTFCI code word = 32.

#### 4.3.1.2	Coding of short TFCI lengths

##### 4.3.1.2.1	Coding very short TFCIs by repetition

If the number of TFCI bits is 1 or 2, then repetition will be used for coding. In this case each bit is repeated to a total of 4 times giving 4-bit transmission (NTFCI code word =4) for a single TFCI bit and 8-bit transmission (NTFCI code word =8) for 2 TFCI bits. The TFCI bit(s) a0 (or a0 and a1 where a0 is the LSB) shall correspond to the TFC index (expressed in unsigned binary form) defined by the RRC layer to reference the TFC of the CCTrCH in the associated radio frame.

In the case of NTFCI code word=4, the TFCI codeword {b0, b1, b2, b3} is equal to the sequence {a0, a0, a0, a0}.

In the case of NTFCI code word=8, the TFCI codeword {b0, b1, … , b7} is equal to the sequence {a0, a1, a0, a1, a0, a1, a0, a1}.

##### 4.3.1.2.2	Coding short TFCIs using bi-orthogonal codes

If the number of TFCI bits is in the range 3 to 5 the TFCI is encoded using a (16, 5) bi-orthogonal (or first order Reed-Muller) code. The coding procedure is as shown in figure 7.

Figure 7: Channel coding of short length TFCI bits

If the TFCI consists of less than 5 bits, it is padded with zeros to 5 bits, by setting the most significant bits to zero. The code words of the (16,5) bi-orthogonal code are linear combinations of 5 basis sequences as defined in table 9.

Table 9: Basis sequences for (16,5) TFCI code

| i | Mi,0 | Mi,1 | Mi,2 | Mi,3 | Mi,4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 |
| 2 | 1 | 1 | 0 | 0 | 1 |
| 3 | 0 | 0 | 1 | 0 | 1 |
| 4 | 1 | 0 | 1 | 0 | 1 |
| 5 | 0 | 1 | 1 | 0 | 1 |
| 6 | 1 | 1 | 1 | 0 | 1 |
| 7 | 0 | 0 | 0 | 1 | 1 |
| 8 | 1 | 0 | 0 | 1 | 1 |
| 9 | 0 | 1 | 0 | 1 | 1 |
| 10 | 1 | 1 | 0 | 1 | 1 |
| 11 | 0 | 0 | 1 | 1 | 1 |
| 12 | 1 | 0 | 1 | 1 | 1 |
| 13 | 0 | 1 | 1 | 1 | 1 |
| 14 | 1 | 1 | 1 | 1 | 1 |
| 15 | 0 | 0 | 0 | 0 | 1 |

The TFCI bits a0 , a1 , a2 , a3 , a4 (where a0 is LSB and a4 is MSB) shall correspond to the TFC index (expressed in unsigned binary form) defined by the RRC layer to reference the TFC of the CCTrCH in the associated radio frame.

The output code word bits bj are given by:

where i = 0,…,15. NTFCI code word = 16.

#### 4.3.1.3	Mapping of TFCI code word

The mapping of the TFCI code word to the TFCI bit positions in a timeslot shall be as follows.

Denote the number of bits in the TFCI code word by NTFCI code word, denote the TFCI code word bits by bk where k=0… NTFCI code word -1.

Figure 8: Mapping of TFCI code word bits to timeslot

The locations of the first and second parts of the TFCI code word in the timeslot is defined in [7].

If the shortest transmission time interval of any constituent TrCH is at least 20 ms the successive TFCI code words in the frames in the TTI shall be identical. If TFCI is transmitted on multiple timeslots in a frame each timeslot shall have the same TFCI code word.

### 4.3.2	Coding and Bit Scrambling of the Paging Indicator

The paging indicator Pq, q = 0, ..., NPI-1, Pq  {0, 1} is an identifier to instruct the UE whether there is a paging message for the groups of mobiles that are associated to the PI, calculated by higher layers, and the associated paging indicator Pq. The length LPI of the paging indicator is LPI=2, LPI=4 or LPI=8 symbols. NPIB = 2*NPI*LPI bits are used for the paging indicator transmission in one radio frame. The mapping of the paging indicators to the bits ei, i = 1, ..., NPIB is shown in table 10.

Table 10: Mapping of the paging indicator

| Pq | Bits {e2Lpi*q+1, e2Lpi*q+2, ... ,e2Lpi*(q+1) } | Meaning |
| --- | --- | --- |
| 0 | {0, 0, ..., 0} | There is no necessity to receive the PCH |
| 1 | {1, 1, ..., 1} | There is the necessity to receive the PCH |

If the number S of bits in one radio frame available for the PICH is bigger than the number NPIB of bits used for the transmission of paging indicators, the sequence e = {e1, e2, ..., eNPIB} is extended by S-NPIB bits that are set to zero, resulting in a sequence h = {h1, h2, ..., hS}:

hk=ek, k=1, ..., NPIBhk=0, k=NPIB+1, ..., S

The bits hk , k = 1, ..., S on the PICH then undergo bit scrambling as defined in section 4.2.9.

The bits sk, k = 1, ..., S output from the bit scrambler are then transmitted over the air as shown in [7].

### 4.3.3	Coding and Bit Scrambling of the MBMS Notification Indicator

The MBMS notification indicator Nq, q = 0, ..., Nn-1, Nq  {0, 1}, is an identifier to instruct UEs whether there is an MBMS notification indication for the groups of MBMS services that are associated to the NI, calculated by higher layers, and the associated MBMS notification indicator Nq. The length LNI of the MBMS notification indicator is LNI=2, LNI=4 or LNI=8 symbols. NNIB = 2*Nn*LNI bits are used for the MBMS notification indicator transmission in one MICH. The mapping of the MBMS notification indicators to the bits ei, i = 1, ..., NNIB is shown in table 10A.

Table 10A: Mapping of the MBMS notification indicator

| Nq | Bits {e2Lni*q+1, e2Lni*q+2, ... ,e2Lni*(q+1) } |
| --- | --- |
| 0 | {0, 0, ..., 0} |
| 1 | {1, 1, ..., 1} |

If the number S of bits available for the MICH is bigger than the number NNIB of bits used for the transmission of MBMS notification indicators, the sequence e = {e1, e2, ..., eNNIB} is extended by S-NNIB bits that are set to zero, resulting in a sequence h = {h1, h2, ..., hS}:

hk=ek, k=1, ..., NNIBhk=0, k=NNIB+1, ..., S

The bits hk , k = 1, ..., S on the MICH then undergo bit scrambling as defined in section 4.2.9.

The bits sk, k = 1, ..., S output from the bit scrambler are then transmitted over the air as shown in [7].

## 4.4	Coding for layer 1 control for the 1.28 Mcps option

### 4.4.1	Coding of transport format combination indicator (TFCI) for QPSK and 16QAM

The coding of TFCI for 1.28Mcps TDD is same as that of 3.84Mcps TDD.cf.[4.3.1 'Coding of transport format combination indicator'].

#### 4.4.1.1	Mapping of TFCI code word

Denote the number of bits in the TFCI code word by NTFCI code word, and denote the TFCI code word bits by bk, where k = 0, …, NTFCI code word -1

When the number of bits in the TFCI code word is 8, 16, 32, the mapping of the TFCI code word to the TFCI bit positions shall be as follows:

Figure 9: Mapping of TFCI code word bits to TFCI position in 1.28 Mcps TDD option, 
where N = NTFCI code word.

For MBSFN transmissions with 16QAM, the coded bits bk, are mapped to the transmitted TFCI bits according to the following two group formulas:

Formula a, mapping onto the outer-corners of the 16QAM constellation

d4k = b2k ,

d4k+1 = b2k+1 ,

d4k+2 = 1,

d4k+3 = 1,

Formula b, mapping onto the inner-corners of the 16QAM constellation

d4k = b2k ,

d4k+1 = b2k+1 ,

d4k+2 =0,

d4k+3 = 0,

The 1st, the 3rd, the 5th and the 7th part of TFCI code word will use the Mapping Formula a, and the 2nd, the 4th, the 6th and the 8th part of TFCI code word will use the Mapping Formula b. As the TTI of S-CCPCH is 40ms or 80ms, the TFCI bits will be repeated with a period of 20ms frame. In the second 20ms frame, the 1st, the 3rd, the 5th and the 7th part of TFCI code word will use the Mapping Formula b, and the 2nd, the 4th, the 6th and 8th part of TFCI code word will use the Mapping Formula a, and so on for the consecutive frames. The mapping of the TFCI code word to the TFCI bit positions shall be as follows:

Figure 9A: Mapping of TFCI code word bits to TFCI position in 1.28 Mcps TDD option for downlink MBSFN, where N = NTFCI code word.

When the number of bits of the TFCI code word is 4 , then the TFCI code word is equally divided into two parts for the consecutive two subframe and mapped onto the end of the first data field in each of the consecutive subframes.The mapping for NTFCI code word =4 is shown in figure 10:

Figure 10: Mapping of TFCI code word bits to TFCI position in 1.28 Mcps TDD option, 
when NTFCI code word.=4

The location of the 1st to 4th parts of the TFCI code word in the timeslot is defined in [7].

If the shortest transmission time interval of any constituent TrCH is at least 20 ms, then successive TFCI code words in the frames within the TTI shall be identical. If a TFCI is transmitted on multiple timeslots in a frame each timeslot shall have the same TFCI code word.

### 4.4.2	Coding of transport format combination indicator (TFCI) for 8PSK

Encoding of TFCI bits depends on the number of them and the modulation in use. When 2 Mcps service is transmitted, 8PSK modulation is applied in 1.28 Mcps TDD option. The encoding scheme for TFCI when the number of bits are 6 – 10, and less than 6 bits is described in section 4.4.2.1 and 4.4.2.2, respectively.

#### 4.4.2.1	Coding of long TFCI lengths

When the number of TFCI bits is 6 – 10, the TFCI bits are encoded by using a (64,10) sub-code of the second order Reed-Muller code, then 16 bits out of 64 bits are punctured (Puncturing positions are 0, 4, 8, 13, 16, 20, 27, 31, 34, 38, 41, 44, 50, 54, 57, 61st bits). The coding procedure is shown in Figure 11.

Figure 11: Channel coding of long TFCI bits for 8PSK

If the TFCI consists of less than 10 bits, it is padded with zeros to 10 bits, by setting the most significant bits to zero. The code words of the punctured (48,10) sub-code of the second order Reed-Muller codes are linear combination of 10 basis sequences. The basis sequences are shown in Table 11.

Table 11: Basis sequences for (48,10) TFCI code

| I | Mi,0 | Mi,1 | Mi,2 | Mi,3 | MI,4 | Mi,5 | Mi,6 | MI,7 | MI,8 | Mi,9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 2 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 3 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 4 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 5 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 7 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 8 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 9 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 10 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 11 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 12 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 13 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 14 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 15 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 16 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 17 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 18 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 19 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 20 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 21 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 22 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 23 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 24 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 25 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |
| 26 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 27 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 28 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 29 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 30 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 31 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 32 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 33 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 34 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 35 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 36 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| 37 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |
| 38 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 39 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 40 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 41 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 42 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 43 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 44 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 45 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 46 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 47 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

Let's define the TFCI bits as a0 , a1 , a2 , a3 , a4 , a5 , a6 , a7 , a8 , a9, where a0 is the LSB and a9 is the MSB. The TFCI bits shall correspond to the TFC index (expressed in unsigned binary form) defined by the RRC layer to reference the TFC of the CCTrCH in the associated radio frame.

The output TFCI code word bits bi are given by:

where i=0…47. NTFCI code word =48.

#### 4.4.2.2	Coding of short TFCI lengths

##### 4.4.2.2.1	Coding very short TFCIs by repetition

When the number of TFCI bits is 1 or 2, then repetition will be used for the coding. In this case, each bit is repeated to a total of 6 times giving 6-bit transmission (NTFCI code word = 6) for a single TFCI bit and 12-bit transmission (NTFCI code word = 12) for 2 TFCI bits. The TFCI bit(s) a0 (or a0 and a1 where a0 is the LSB) shall correspond to the TFC index (expressed in unsigned binary form) defined by the RRC layer to reference the TFC of the CCTrCH in the associated radio frame.

In the case of NTFCI code word=6, the TFCI codeword {b0, b1, b2, b3, b4, b5} is equal to the sequence {a0, a0, a0, a0, a0, a0}.

In the case of NTFCI code word=12, the TFCI codeword {b0, b1, … , b11} is equal to the sequence {a0, a1, a0, a1, a0, a1, a0, a1, a0, a1, a0, a1}.

##### 4.4.2.2.2	Coding short TFCIs using bi-orthogonal codes

If the number of TFCI bits is in the range of 3 to 5, the TFCI bits are encoded using a (32,5) first order Reed-Muller code, then 8 bits out of 32 bits are punctured (Puncturing positions are 0, 1, 2, 3, 4, 5, 6, 7th bits). The coding procedure is shown in Figure 12.

Figure 12: Channel coding of short TFCI bits for 8PSK

If the TFCI consists of less than 5 bits, it is padded with zeros to 5 bits, by setting the most significant bits to zero. The code words of the punctured (32,5) first order Reed-Muller codes are linear combination of 5 basis sequences shown in Table 12.

Table 12: Basis sequences for (24,5) TFCI code

| I | Mi,0 | Mi,1 | Mi,2 | Mi,3 | Mi,4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 | 0 |
| 3 | 1 | 1 | 0 | 1 | 0 |
| 4 | 0 | 0 | 1 | 1 | 0 |
| 5 | 1 | 0 | 1 | 1 | 0 |
| 6 | 0 | 1 | 1 | 1 | 0 |
| 7 | 1 | 1 | 1 | 1 | 0 |
| 8 | 0 | 0 | 0 | 0 | 1 |
| 9 | 1 | 0 | 0 | 0 | 1 |
| 10 | 0 | 1 | 0 | 0 | 1 |
| 11 | 1 | 1 | 0 | 0 | 1 |
| 12 | 0 | 0 | 1 | 0 | 1 |
| 13 | 1 | 0 | 1 | 0 | 1 |
| 14 | 0 | 1 | 1 | 0 | 1 |
| 15 | 1 | 1 | 1 | 0 | 1 |
| 16 | 0 | 0 | 0 | 1 | 1 |
| 17 | 1 | 0 | 0 | 1 | 1 |
| 18 | 0 | 1 | 0 | 1 | 1 |
| 19 | 1 | 1 | 0 | 1 | 1 |
| 20 | 0 | 0 | 1 | 1 | 1 |
| 21 | 1 | 0 | 1 | 1 | 1 |
| 22 | 0 | 1 | 1 | 1 | 1 |
| 23 | 1 | 1 | 1 | 1 | 1 |

Let's define the TFCI bits as a0 , a1 , a2 , a3 , a4, where a0 is the LSB and a4 is the MSB. The TFCI bits shall correspond to the TFC index (expressed in unsigned binary form) defined by the RRC layer to reference the TFC of the CCTrCH in the associated radio frame.

The output code word bits bi are given by:

where i=0…23. NTFCI code word =24.

#### 4.4.2.3	Mapping of TFCI code word

Denote the number of bits in the TFCI code word by NTFCI code word, and denote the TFCI code word bits by bk, where k = 0, …, NTFCI code word -1.

When the number of bits in the TFCI code word is 12, 24 or 48, the mapping of the TFCI code word to the TFCI bit positions in a time slot shall be as follows.

Figure 13: Mapping of TFCI code word bits to timeslot in 1.28 Mcps TDD option, 
where N = NTFCI code word.

When the number of bits in the TFCI code word is 6, the TFCI code word is equally divided into two parts for the consecutive two sub-frames and mapped onto the first data field in each of the consecutive sub-frames. The mapping of the TFCI code word to the TFCI bit positions in a time slot shall be as shown in figure 14.

Figure 14: Mapping of TFCI code word bits to timeslot in 1.28 Mcps TDD option when NTFCI code word = 6

The location of the 1st to 4th parts of the TFCI code word in the timeslot is defined in [7].

### 4.4.3	Coding and Bit Scrambling of the Paging Indicator

The paging indicator Pq, q = 0, ..., NPI-1, Pq  {0, 1} is an identifier to instruct the UE whether there is a paging message for the groups of mobiles that are associated to the PI, calculated by higher layers, and the associated paging indicator Pq. The length LPI of the paging indicator is LPI=2, LPI=4 or LPI=8 symbols. NPIB = 2*NPI*LPI bits are used for the paging indicator transmission in one radio frame. The mapping of the paging indicators to the bits ei, i = 1, ..., NPIB is shown in table 13.

Table 13: Mapping of the paging indicator

| Pq | Bits {e2LPI*q+1, e2LPI*q+2, ... ,e2LPI*(q+1) } | Meaning |
| --- | --- | --- |
| 0 | {0, 0, ..., 0} | There is no necessity to receive the PCH |
| 1 | {1, 1, ..., 1} | There is the necessity to receive the PCH |

If the number S of bits in one radio frame available for the PICH is bigger than the number NPIB of bits used for the transmission of paging indicators, the sequence e = {e1, e2, ..., eNPIB} is extended by S-NPIB bits that are set to zero, resulting in a sequence h = {h1, h2, ..., hS}:

The bits hk , k = 1, ..., S on the PICH then undergo bit scrambling as defined in section 4.2.9.

The bits sk, k = 1, ..., S output from the bit scrambler are then transmitted over the air as shown in [7].

### 4.4.4	Coding of the Fast Physical Access Channel (FPACH) information bits

The FPACH burst is composed by 32 information bits which are block coded and convolutional coded, and then delivered in one sub-frame as follows:

1.	The 32 information bits are protected by 8 parity bits for error detection as described in sub-clause	4.2.1.1.

2.	Convolutional code with constraint length 9 and coding rate ½ is applied as described in sub-clause	4.2.3.1. The size of data block c(k) after convolutional encoder is 96 bits.

3.	To adjust the size of the data block c(k) to the size of the FPACH burst, 8 bits are punctured as described in sub-clause 4.2.7 with the following clarifications:

-	Ni;j =96 is the number of bits in a radio sub-frame before rate matching

-	Ni,,j = -8 is the number of bits to punctured in a radio sub-frame

-	eini = a x Nij

The 88 bits after rate matching are then delivered to the intra-frame interleaving.

4.	The bits in input to the interleaving unit are denoted as {x(0), …, x(87)}. The coded bits are block	rectangular interleaved according to the following rule: the input is written row by row, the output is	read column by column.

Hence, the interleaved sequence is denoted by y (i) and are given by:

y(0), y(1), …, y(87)=x(0), x(8), …,x(80),x(1), …, x(87).

### 4.4.5	Coding and Bit Scrambling of the MBMS Notification Indicator

The MBMS notification indicator Nq, q = 0, ..., Nn-1, Nq  {0, 1} is an identifier to instruct the UE whether there is an MBMS notification indication for the groups of MBMS services that are associated to the NI, calculated by higher layers, and the associated MBMS notification indicator Nq. The length LNI of the MBMS notification indicator is LNI=2, LNI=4 or LNI=8 symbols. NNIB = 2*Nn*LNI bits are used for the MBMS notification indicator transmission in one MICH. The mapping of the MBMS notification indicators to the bits ei, i = 1, ..., NNIB is shown in table 13A.

Table 13A: Mapping of the MBMS notification indicator

| Nq | Bits {e2LNI*q+1, e2LNI*q+2, ... ,e2LNI*(q+1) } |
| --- | --- |
| 0 | {0, 0, ..., 0} |
| 1 | {1, 1, ..., 1} |

If the number S of bits available for the MICH is bigger than the number NNIB of bits used for the transmission of MBMS notification indicators, the sequence e = {e1, e2, ..., eNNIB} is extended by S-NNIB bits that are set to zero, resulting in a sequence h = {h1, h2, ..., hS}:

The bits hk , k = 1, ..., S on the MICH then undergo bit scrambling as defined in section 4.2.9.

The bits sk, k = 1, ..., S output from the bit scrambler are then transmitted over the air as shown in [7].

### 4.4.6	Coding of PLCCH

The PLCCH is a Node-B terminated channel used to carry dedicated (UE-specific) TPC and SS information to multiple UEs. Each TPC/SS command pair for a given UE is mapped to 3 bits as shown in table 13B.

Table 13B: Mapping of the TPC/SS pair

| 3-bit TPC/SS command (MSB on left) | TPC command | SS command |
| --- | --- | --- |
| 000 | 'DOWN' | 'DOWN' |
| 100 | 'UP' | 'DOWN' |
| 011 | 'DOWN' | 'UP' |
| 111 | 'UP' | 'UP' |
| 001 | 'DOWN' | 'Do Nothing' |
| 101 | 'UP' | 'Do Nothing' |

Let I=14 be the number of TPC/SS command pairs that can be carried by a single PLCCH. The 3 bits corresponding to the ith TPC/SS command pair ( i=1…I ), are denoted {Β0i, Β1i, Β2i} where Β0i is the MSB.

The bit sequence corresponding to the I=14 TPC/SS command pairs is denoted αk (k = 0,1,2,…41).  Βni (n=0,1,2) is mapped to αk such that:

α3i−1+n=Βni

The PLCCH burst is composed of 44 information bits {bplcch(0), bplcch(1),…,bplcch(43)} which are repetition coded, and then delivered in one sub-frame as follows:

1.	bplcch(m)= αm (for m=0,1,…41) and bplcch(m) = 0 (for m=42,43)

2.	Repetition coding with code rate ½ is applied to the sequence {bplcch(0),…,bplcch(43)} in order to form the sequence {x(0), x(1), x(2), x(3),…, x(86), x(87)}. The size of the data block after the repetition encoder is 88 bits.  The encoded codeword {x(0), x(1), x(2), x(3),…,x(86), x(87)} is equal to {bplcch (0), bplcch (0), bplcch (1), bplcch (1),…,bplcch (43), bplcch (43)}.

3.	The bits output from the repetition encoder {x(0), …, x(87)} are input to an interleaving unit. The coded bits are block rectangular interleaved according to the following rule: the input is written row by row, the output is read column by column.

Hence, the interleaved sequence is denoted by y(i) and is given by:

y(0), y(1), …, y(87) = x(0), x(8), …, x(80), x(1), …, x(87).

The bit sequence y(0), y(1), … y(87) is mapped to the PLCCH burst in order of bit index, with the lowest bit index being the first bit (in time) to be transmitted.

## 4.5	Coding for HS-DSCH

Figure 15 illustrates the overall concept of transport-channel coding for HS-DSCH. Data arrives at the coding unit in the form of one transport block once every TTI. The TTI is 5 ms for 1.28 Mcps TDD and 10 ms for 3.84 Mcps TDD / 7.68Mcps TDD.

For 1.28 Mcps TDD, in the case of multiple-frequency transmission in one TTI, a number of transport blocks arrive at the coding unit, in which the number of transport blocks equals the number of frequencies used. Each transport block for each frequency shall be coded separately as the following coding step.

For 1.28Mcps TDD, in the case of MIMO dual stream transmission in one TTI, two transport blocks arrive at the coding unit. Each transport block for each data stream shall be coded separately as the following coding step.

The following coding steps for HS-DSCH can be identified:

-	add CRC to each transport block (see subclause 4.5.1);

-	code block segmentation (see subclause 4.5.2);

-	channel coding (see subclause 4.5.3);

-	hybrid ARQ (see subclause 4.5.4);

-	bit scrambling (see subclause 4.5.5);

-	interleaving for HS-DSCH (see subclause 4.5.6);

-	constellation re-arrangement for 16QAM and 64QAM (see subclause 4.5.7);

-	mapping to physical channels (see subclause 4.5.8).

The coding steps for HS-DSCH are shown in figure 15.

Figure 15: Coding chain for HS-DSCH

In the following the number of transport blocks is always one. When referencing non HS-DSCH formulae which are used in correspondence with HS-DSCH formulae the convention is used that transport block subscripts may be omitted (e.g. Xi when i is always 1 may be written X).

### 4.5.1	CRC attachment for HS-DSCH

A CRC of size 24 bits is calculated and added per HS-DSCH TTI. The CRC polynomial is defined in 4.2.1.1 with the following specific parameters: i = 1, L1 = 24 bits.

### 4.5.2	Code block segmentation for HS-DSCH

Code block segmentation for the HS-DSCH transport channel shall be done with the general method described in 4.2.2.2 above with the following specific parameters.

There will only be one transport block, i = 1. The bits bim1, bim2, bim3,…bimB input to the block are mapped to the bits xi1, xi2, xi3,…xiX1 directly. It follows that X1 = B. Note that the bits x referenced here refer only to the internals of the code block segmentation function. The output bits from the code block segmentation function are oir1, oir2, oir3,…oirK.

The value of Z = 5114 for turbo coding shall be used.

### 4.5.3	Channel coding for HS-DSCH

Channel coding for the HS-DSCH transport channel shall be done with the general method described in 4.2.3 above with the following specific parameters.

There will be a maximum of one transport block, i = 1. The rate 1/3 turbo coding shall be used.

### 4.5.4	Hybrid ARQ for HS-DSCH

The hybrid ARQ functionality matches the number of bits at the output of the channel coder to the total number of bits of the HS-PDSCH set to which the HS-DSCH is mapped. The hybrid ARQ functionality is controlled by the redundancy version (RV) parameters. The exact set of bits at the output of the hybrid ARQ functionality depends on the number of input bits, the number of output bits, and the RV parameters.

The hybrid ARQ functionality consists of two rate-matching stages and a virtual buffer as shown in the figure below.

The first rate matching stage matches the number of input bits to the virtual IR buffer, information about which is provided by higher layers. Note that, if the number of input bits does not exceed the virtual IR buffering capability, the first rate-matching stage is transparent.

The second rate matching stage matches the number of bits after first rate matching stage to the number of physical channel bits available in the HS-PDSCH set in the TTI.

Figure 16: HS-DSCH hybrid ARQ functionality

#### 4.5.4.1	HARQ bit separation

The HARQ bit separation function shall be performed in the same way as bit separation for turbo encoded TrCHs in 4.2.7.2 above.

#### 4.5.4.2	HARQ First Rate Matching Stage

HARQ first stage rate matching for the HS-DSCH transport channel shall be done with the general method described in 4.2.7.1.2 above with the following specific parameters.

The maximum number of soft channel bits available in the virtual IR buffer is NIR which is signalled from higher layers for each HARQ process. The number of coded bits in a TTI before rate matching is NTTI this is deduced from information signalled from higher layers and parameters signalled on the HS-SCCH for each TTI. Note that HARQ processing and physical layer storage occurs independently for each HARQ process currently active.

If NIR is greater than or equal to NTTI (i.e. all coded bits of the corresponding TTI can be stored) the first rate matching stage shall be transparent. This can, for example, be achieved by setting eminus = 0. Note that no repetition is performed.

If NIR is smaller than NTTI the parity bit streams are punctured as in 4.2.7.1.2 above by setting the rate matching parameter ΔNilTTI=NIR−NTTIwhere the subscripts i and l refer to transport channel and transport format in the referenced sub-clause. Note the negative value is expected when the rate matching implements puncturing. Bits selected for puncturing which appear as  in the algorithm in 4.2.7 above shall be discarded and not counted in the totals for the streams through the virtual IR buffer.

#### 4.5.4.3	HARQ Second Rate Matching Stage

HARQ second stage rate matching for the HS-DSCH transport channel shall be done with the general method described in 4.2.7.3 above with the following specific parameters. Bits selected for puncturing which appear as  in the algorithm in 4.2.7.3 above shall be discarded and are not counted in the streams towards the bit collection.

The parameters of the second rate matching stage depend on the value of the RV parameters s and r. The parameter s can take the value 0 or 1 to distinguish between transmissions that prioritise systematic bits (s = 1) and non systematic bits (s = 0). The parameter r (range 0 to rmax-1) changes the initial error variable eini in the case of puncturing. In case of repetition both parameters r and s change the initial error variable eini. The parameters Xi, eplus and eminus are calculated as per table 14below.

Denote the number of bits before second rate matching as Nsys for the systematic bits, Np1 for the parity 1 bits, and Np2 for the parity 2 bits, respectively. For the HS-DSCH, denote the number of timeslots used as T, the number of codes per timeslot as C and the number of bits available in timeslot t as Ut, where Ut = p=1CUt,p and Ut,p is the number of bits available in physical channel p of timeslot t as defined in [7]. Ndata is the number of bits available to the HS-DSCH in one TTI and is defined as Ndata=t=1TUt. The rate matching parameters are determined as follows.

For Ndata≤Nsys+Np1+Np2, puncturing is performed in the second rate matching stage. The number of transmitted systematic bits in a transmission is Nt,sys=minNsys,Ndata for a transmission that prioritises systematic bits and Nt,sys=maxNdata−Np1+Np2,0 for a transmission that prioritises non systematic bits.

For Ndata>Nsys+Np1+Np2 repetition is performed in the second rate matching stage. A similar repetition rate in all bit streams is achieved by setting the number of transmitted systematic bits to Nt,sys=Nsys⋅NdataNsys+2Np1.

The number of parity bits in a transmission is: Nt,p1=Ndata−Nt,sys2   and   Nt,p2=Ndata−Nt,sys2 for the parity 1 and parity 2 bits, respectively.

Table 14 below summarizes the resulting parameter choice for the second rate matching stage.

Table 14: Parameters for HARQ second rate matching

|  | Xi | eplus | eminus |
| --- | --- | --- | --- |
| Systematic RM S | Nsys | Nsys | Nsys−Nt,sys |
| Parity 1 RM P1_2 | Np1 | 2⋅Np1 | 2⋅Np1−Nt,p1 |
| Parity 2 RM P2_2 | Np2 | Np2 | Np2−Nt,p2 |

The rate matching parameter eini  is calculated for each bit stream according to the RV parameters r and s using

eini(r)=Xi−r⋅eplusrmax−1 modeplus+1 in the case of puncturing, i.e.,Ndata≤Nsys+Np1+Np2, and

eini(r)=Xi−(s+2⋅r)⋅eplus(2⋅rmax)−1 modeplus+1 for repetition, i.e.,Ndata>Nsys+Np1+Np2.

Where and  is the total number of redundancy versions allowed by varying  as defined in 4.6.1.4.

Note that rmax varies depending on the modulation mode, i.e. for 16QAM and 64QAM rmax = 2 and for QPSK rmax = 4.

Note: For the modulo operation the following clarification is used: the value of (x mod y) is strictly in the range of 0 to y-1 (i.e. -1 mod 10 = 9).

#### 4.5.4.4	HARQ bit collection

The HARQ bit collection is achieved using a rectangular interleaver of size .

The number of rows and columns are determined from:

Nrow=6 for 64QAM

Nrow=4 for 16QAM

Nrow=2 for QPSK

Ncol=NdataNrow

where Ndata is used as defined in 4.5.4.3 above.

Data is written into the interleaver column by column, and read out of the interleaver column by column, starting from the first column.

Nt,sys is the number of transmitted systematic bits. Intermediate values Nr and Nc are calculated using:

Nr=Nt,sysNcol and Nc=Nt,sys−Nr⋅Ncol.

If Nc = 0 and Nr > 0, the systematic bits are written into rows 1…Nr.

Otherwise systematic bits are written into rows 1…Nr+1 in the first Nc columns and, if Nr > 0, also into rows 1…Nr in the remaining Ncol-Nc columns.

The remaining space is filled with parity bits. The parity bits are written column wise into the remaining rows of the respective columns. Parity 1 and 2 bits are written in alternating order, starting with a parity 2 bit in the first available column with the lowest index number.

In the case of 64QAM for each column the bits are read out of the interleaver in the order row1, row2, row3, row4, row 5, row6. In the case of 16QAM for each column the bits are read out of the interleaver in the order row 1, row 2, row 3, row 4. In the case of QPSK for each column the bits are read out of the interleaver in the order row1, row2.

### 4.5.5	Bit scrambling

The bit scrambling for HS-DSCH shall be done with the general method described in subclause 4.2.9.

### 4.5.6	Interleaving for HS-DSCH

The interleaving for TDD is done over all bits in the TTI, as shown in figure 17 when QPSK modulation is being used for the HS-DSCH, and figure 18 when 16-QAM modulation is being used, and figure 18a when 64QAM modulation is being used. The bits input to the block interleaver are denoted by s1, s2, s3, ..., sR , where R is the number of bits in one TTI.

Figure 17: Interleaver structure for HS-DSCH with QPSK modulation

For QPSK, the interleaver is a block interleaver and consists of bits input to a matrix with padding, the inter-column permutation for the matrix and bits output from the matrix with pruning. The output bit sequence from the block interleaver is derived as follows:

(1)	The number of columns of the matrix is 30. The columns of the matrix are numbered 0, 1, 2, …, 29 from left to right.

(2)	Determine the number of rows of the matrix, R2, by finding minimum integer R2 such that R  30  R2. The rows of rectangular matrix are numbered 0, 1, 2, …, R2 – 1 from top to bottom.

(3)	Write the input bit sequence s1, s2, s3, ..., sR into the R2  30 matrix row by row starting with bit y1 in column 0 of row 0:

y1y2y3...y30y31y32y33...y60⋮⋮⋮⋮y30⋅(R2−1)+1y30⋅(R2−1)+2y30⋅(R2−1)+3...y30⋅R2

where yk = sk for k = 1, 2, …, R and, if R < 30R2, dummy bits are inserted for k = R+1, R+2, …, 30R2. These dummy bits are pruned away from the output of the matrix after the inter-column permutation.

(4)	Perform the inter-column permutation for the matrix based on the pattern P2(j)j∈0,1,...,29 that is shown in Table 7, where P2(j) is the original column position of the j-th permuted column. After permutation of the columns, the bits are denoted by y'k.

y'1y'R2+1y'2⋅R2+1...y'29⋅R2+1y'2y'R2+2y'2⋅R2+2...y'29⋅R2+2⋮⋮⋮⋮y'R2y'2⋅R2y'3⋅R2...y'30⋅R2

(5)	The output of the block interleaver is the bit sequence read out column by column from the inter-column permuted R230 matrix. The output is pruned by deleting dummy bits that were padded to the input of the matrix before the inter-column permutation, i.e. bits y'k that corresponds to bits yk with k > R are removed from the output. The bits after interleaving are denoted by v1, v2, v3, ..., vR, where v1 corresponds to the bit y'k with smallest index k after pruning, v2 to the bit y'k with second smallest index k after pruning, and so on.

Figure 18: Interleaver structure for HS-DSCH with 16-QAM modulation

For 16QAM, a second identical interleaver operates in parallel to the first. For both interleavers, R2 is chosen to be the minimum integer that satisfies R ≤ 60  R2. The output bits from the bit scrambling operation are divided pairwise between the interleavers: bits sk and sk+1 go to the first interleaver and bits sk+2 and sk+3 go to the second interleaver, where k mod 4 = 1. Bits are collected pairwise from the interleavers: bits vk and vk+1 are obtained from the first interleaver and bits vk+2 and vk+3 are obtained from the second interleaver, where again k mod 4 = 1.

Figure 18a: Interleaver structure for HS-DSCH with 64-QAM modulation

The interleaving for TDD is done over all bits in the TTI, as shown in figure 18a when 64QAM modulation is being used for the HS-DSCH. The bits input to the block interleaver are denoted by s1, s2, s3, ..., sR , where R is the number of bits in one TTI.

For 64QAM, two identical interleavers operate in parallel to the first. For all interleavers, R2 is chosen to be the minimum integer that satisfies R ≤ 90  R2. The output bits from the bit scrambling operation are divided into three parts of different interleavers: bits sk and sk+1 go to the first interleaver, bits sk+2 and sk+3 go to the second interleaver and bits sk+4 and sk+5 go to the third interleaver, where k mod 6 = 1. Bits are collected accordingly from the interleavers: bits vk and vk+1 are obtained from the first interleaver, bits vk+2 and vk+3 are obtained from the second interleaver and bits vk+4 and vk+5 are obtained from the third interleaver, where again k mod 6 = 1.

### 4.5.7	Constellation re-arrangement for 16 QAM and 64 QAM

This function only applies to 16 QAM modulated bits. In case of QPSK it is transparent.

The following table 15 describes the operations that produce the different rearrangements.

The bits of the input sequence are mapped in groups of 4 so that vk, vk+1, vk+2, vk+3 are used, where k mod 4 = 1.

Table 15: Constellation re-arrangement for 16 QAM

| Constellation version parameter b | Output bit sequence | Operation |
| --- | --- | --- |
| 0 | vvk+1vvk+3 | None |
| 1 | vvk+3vvk+1 | Swapping MSBs with LSBs |
| 2 | vvk+1vvk+3 | Inversion of the logical values of LSBs |
| 3 | vvk+3vvk+1 | Swapping MSBs with LSBs, and inversion of the logical values of LSBs |

The output bit sequences from the table above map to the output bits in groups of 4, i.e. rk, rk+1, rk+2, rk+3, where k mod 4 = 1.

The following table 15a describes the operations that produce the different rearrangements of 64 QAM.

The bits of the input sequence are mapped in groups of 6 so that vk, vk+1, vk+2, vk+3, vk+4, vk+5 are used, where k mod 6 = 1.

Table 15a: Constellation re-arrangement for 64 QAM

| Constellation version parameter b | Output bit sequence | Operation |
| --- | --- | --- |
| 0 | vvk+1vvk+3vk+4vk+5 | None |
| 1 | vk+4vk+5vvk+3vvk+1 | Swapping MSBs and LSBs. Inversion of Middle SBs |
| 2 | vk+2vk+3vvk+5vvk+1 | Left circular shift of pair of SBs. Inversion of Middle SBs |
| 3 | vvk+1vvk+3vk+4vk+5 | Inversion of Middle SBs |

The output bit sequences from the table above map to the output bits in groups of 6, i.e. rk, rk+1, rk+2, rk+3, rk+4, rk+5, where k mod 6 = 1.

### 4.5.8	Physical channel mapping for HS-DSCH

The HS-PDSCH is defined in [7]. The bits input to the physical channel mapping are denoted by r1, r2, ..., rR, where R is the number of physical channel bits in the allocation for the current TTI. These bits are mapped to the physical channel bits, {wt,p,j : t = 1, 2, ..., T; p = 1, 2, ..., C; j = 1, 2, ..., Ut,p}, where t is the timeslot index, T is the number of timeslots in the allocation message, p is the physical channel index, C is the number of codes per timeslot in the allocation message, j is the physical channel bit index and Ut,p is the number of bits for the physical channel p in timeslot t. The timeslot index, t, increases with increasing timeslot number; the physical channel index, p, increases with increasing channelisation code index, and the physical channel bit index, j, increases with increasing physical channel bit position in time. If TS0 is included in the allocation message, TS0 has the maximum timeslot index while the timeslot index of any other timeslot increases with increasing timeslot number.

The bits rk shall be mapped to the PhCHs according to the following rule :

Define {yt,k : k = 1, 2, ..., p=1CUt,p} to be the set of bits to be transmitted in timeslot t as follows :

y1,k=rk	for k = 1, 2, ..., p=1CU1,p

y2,k=rk+p=1CU1,p	for k = 1, 2, ..., p=1CU2,p

…

yT,k=rk+t=1T−1p=1CUt,p	for k = 1, 2, ..., p=1CUT,p

When the modulation level applied to the physical channels is 16- QAM :

The physical channel p used to transmit the kth bit in the sequence yt,k is :

if (k <= Ut,1·C)

p=k−14modC+1

If p is odd then :

wt,p,j=yt,k wherej=4⋅k−14⋅C+(k−1)mod4+1

If p is even then :

wt,p,j=yt,k wherej=Ut,p−4⋅k−14⋅C−3+(k−1)mod4

else

p=k− Ut,1⋅C - 14mod(C−1)+2

If p is odd then :

wt,p,j=yt,k wherej=4⋅k−Ut,1⋅C−14⋅(C−1)+(k−Ut,1⋅C−1)mod4+Ut,1+1

If p is even then :

wt,p,j=yt,k wherej=Ut,p−4⋅k−Ut,1⋅C−14⋅(C−1)−3−Ut,1+(k−Ut,1C−1)mod4

Otherwise, when the modulation level applied to the physical channels is QPSK :

The physical channel p used to transmit the kth bit in the sequence yt,k is :

if (k <= Ut,1·C)

p=k−1modC+1

If p is odd then :

wt,p,j=yt,kwhere j=k−1C+1

If p is even then :

wt,p,j=yt,kwherej=Ut,p−k−1Cj=Ut−k−1C

else

p=k− Ut,1⋅C - 1mod(C−1)+2

If p is odd then :

wt,p,j=yt,kwherej=k−Ut,1⋅C−1C−1+Ut,1+1

If p is even then :

wt,p,j=yt,kwherej=Ut,p−k−Ut,1⋅C−1C−1−Ut,1

When the modulation level applied to the physical channels is 64-QAM:

The physical channel p used to transmit the kth bit in the sequence yt,k is:

p=k−16modC+1

If p is odd then :

wt,p,j=yt,k where j=6⋅k−16⋅C+(k−1)mod6+1

If p is even then :

wt,p,j=yt,k where j=Ut,p−6⋅k−16⋅C−5+(k−1)mod6

## 4.6	Coding/Multiplexing for HS-SCCH

For 1.28 Mcps TDD, HS-SCCH shall be of type 1 when the following two conditions are both true:

-	the UE is not configured in MIMO mode, and

-	the variable HS_DSCH_SPS_STATUS is FALSE.

HS-SCCH type 1 may be used when the following two conditions are both true:

-	the UE is not configured in MIMO mode, and

-	the variable HS_DSCH_SPS_STATUS is TRUE.

In this section, the terms "HS-SCCH" and "HS-SCCH type 1" are used interchangeably.

The following information, provided by higher layers, is transmitted by means of the HS-SCCH physical channel.

For 1.28 Mcps TDD, in the case of multi-frequency HS-DSCH transmission in one TTI, HS-PDSCH on each frequency shall be configured with associated HS-SCCH(s) which is coded and multiplexed as following.

-	Channelisation-code-set information (q bits where q = 8 for 1.28Mcps TDD / 3.84Mcps TDD and q = 10 for 7.68Mcps TDD)): xccs,1, xccs,2, …, xccs, q

-	Time slot information (n bits where n = 5 for 1.28 Mcps TDD and n = 13 for 3.84 Mcps TDD / 7.68Mcps TDD): 
xts,1, xts,2, …, xts,n

-	Modulation scheme information (1 bit): xms,1

-	Transport-block size information (m bits where m = 6 for 1.28 Mcps TDD and m = 9 for 3.84 Mcps TDD / 7.68Mcps TDD): 
xtbs,1, xtbs,2, …, xtbs,m

-	Hybrid-ARQ process information (3 bits): xhap,1, xhap,2, xhap,3

-	Redundancy version information (3 bits): xrv,1, xrv,2,xrv,3

-	New data indicator (1 bit): xnd,1

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

For an HS-SCCH order type A for 1.28Mcps TDD,

-	xccs,1, xccs,2, …, xccs, q are reserved

-	xts,1, xts,2, …, xts,n shall be set to '00000'

-	xms,1, xtbs,1, xtbs,2 shall be set to xodt,1, xodt,2, xodt,3

-	xtbs,3, xtbs,4, …, xtbs,m, xhap,1, xhap,2, xhap,3 ,xrv,1, xrv,2,xrv,3 ,xnd,1, xhcsn,1, xhcsn,2, xhcsn,3 are reserved

where xodt,1, xodt,2, xodt,3 are defined in subclause 4.6A.

The following coding/multiplexing steps can be identified:

-	multiplexing of HS-SCCH information (see subclause 4.6.2)

-	CRC attachment (see subclause 4.6.3);

-	channel coding (see subclause 4.6.4);

-	rate matching (see subclause 4.6.5);

-	interleaving for HS-SCCH (see subclause 4.6.6);

-	mapping to physical channels (see subclauses 4.6.7 and 4.6.8).

The general coding/multiplexing flow is shown in Figure 19.

Figure 19 Coding and Multiplexing for HS-SCCH

### 4.6.1	HS-SCCH information field mapping

#### 4.6.1.1	Channelisation code set information mapping

#### 4.6.1.1.1	1.28Mcps TDD and 3.84Mcps TDD

HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code. The start code kstart is signalled by the bits xccs,1, xccs,2, xccs,3, xccs,4 and the stop code kstop by the bits xccs,5, xccs,6, xccs,7, xccs,8. The mapping in Table 16 below applies.

Table 16: Channelisation code set information mapping for 1.28Mcps and 3.84Mcps TDD

| kstart | xccs,1 | xccs,2 | xccs,3 | xccs,4 | kstop | xccs,5 | xccs,6 | xccs,7 | xccs,8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 1 | 2 | 0 | 0 | 0 | 1 |
| 3 | 0 | 0 | 1 | 0 | 3 | 0 | 0 | 1 | 0 |
| 4 | 0 | 0 | 1 | 1 | 4 | 0 | 0 | 1 | 1 |
| 5 | 0 | 1 | 0 | 0 | 5 | 0 | 1 | 0 | 0 |
| 6 | 0 | 1 | 0 | 1 | 6 | 0 | 1 | 0 | 1 |
| 7 | 0 | 1 | 1 | 0 | 7 | 0 | 1 | 1 | 0 |
| 8 | 0 | 1 | 1 | 1 | 8 | 0 | 1 | 1 | 1 |
| 9 | 1 | 0 | 0 | 0 | 9 | 1 | 0 | 0 | 0 |
| 10 | 1 | 0 | 0 | 1 | 10 | 1 | 0 | 0 | 1 |
| 11 | 1 | 0 | 1 | 0 | 11 | 1 | 0 | 1 | 0 |
| 12 | 1 | 0 | 1 | 1 | 12 | 1 | 0 | 1 | 1 |
| 13 | 1 | 1 | 0 | 0 | 13 | 1 | 1 | 0 | 0 |
| 14 | 1 | 1 | 0 | 1 | 14 | 1 | 1 | 0 | 1 |
| 15 | 1 | 1 | 1 | 0 | 15 | 1 | 1 | 1 | 0 |
| 16 | 1 | 1 | 1 | 1 | 16 | 1 | 1 | 1 | 1 |

If a value of kstart = 16 and kstop = 1 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop shall be treated as an error by the UE.

If NON_RECTANGULAR_Resource_ ALLOCATION_STATUS is FALSE, HS-PDSCH channelization codes of all the allocated timeslots are indicated by channelisation-code-set information field.

If NON_RECTANGULAR_Resource_ ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot with the maximal timeslot index among all the timeslots scheduled to the UE and HS-PDSCH channelisation codes of the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of timeslot 0 are signalled via higher layer signalling if timeslot 0 is scheduled to the UE. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of each timeslot is scheduled to the UE with SF=1.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, HS-PDSCH channelisation codes in the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of the timeslot is scheduled to the UE with SF=1.

##### 4.6.1.1.2	7.68Mcps TDD

HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code. The start code kstart is signalled by the bits xccs,1, xccs,2, xccs,3, xccs,4, xccs,5  and the stop code kstop by the bits xccs,6, xccs,7, xccs,8, xccs,9, xccs,10. The mapping in Table 16A below applies.

Table 16A: Channelisation code set information mapping for 7.68Mcps TDD

| kstart | xccs,1 | xccs,2 | xccs,3 | xccs,4 | xccs,5 | kstop | xccs,6 | xccs,7 | xccs,8 | xccs,9 | xccs,10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 1 | 2 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0 | 0 | 0 | 1 | 0 | 3 | 0 | 0 | 0 | 1 | 0 |
| 4 | 0 | 0 | 0 | 1 | 1 | 4 | 0 | 0 | 0 | 1 | 1 |
| 5 | 0 | 0 | 1 | 0 | 0 | 5 | 0 | 0 | 1 | 0 | 0 |
| 6 | 0 | 0 | 1 | 0 | 1 | 6 | 0 | 0 | 1 | 0 | 1 |
| 7 | 0 | 0 | 1 | 1 | 0 | 7 | 0 | 0 | 1 | 1 | 0 |
| 8 | 0 | 0 | 1 | 1 | 1 | 8 | 0 | 0 | 1 | 1 | 1 |
| 9 | 0 | 1 | 0 | 0 | 0 | 9 | 0 | 1 | 0 | 0 | 0 |
| 10 | 0 | 1 | 0 | 0 | 1 | 10 | 0 | 1 | 0 | 0 | 1 |
| 11 | 0 | 1 | 0 | 1 | 0 | 11 | 0 | 1 | 0 | 1 | 0 |
| 12 | 0 | 1 | 0 | 1 | 1 | 12 | 0 | 1 | 0 | 1 | 1 |
| 13 | 0 | 1 | 1 | 0 | 0 | 13 | 0 | 1 | 1 | 0 | 0 |
| 14 | 0 | 1 | 1 | 0 | 1 | 14 | 0 | 1 | 1 | 0 | 1 |
| 15 | 0 | 1 | 1 | 1 | 0 | 15 | 0 | 1 | 1 | 1 | 0 |
| 16 | 0 | 1 | 1 | 1 | 1 | 16 | 0 | 1 | 1 | 1 | 1 |
| 17 | 1 | 0 | 0 | 0 | 0 | 17 | 1 | 0 | 0 | 0 | 0 |
| 18 | 1 | 0 | 0 | 0 | 1 | 18 | 1 | 0 | 0 | 0 | 1 |
| 19 | 1 | 0 | 0 | 1 | 0 | 19 | 1 | 0 | 0 | 1 | 0 |
| 20 | 1 | 0 | 0 | 1 | 1 | 20 | 1 | 0 | 0 | 1 | 1 |
| 21 | 1 | 0 | 1 | 0 | 0 | 21 | 1 | 0 | 1 | 0 | 0 |
| 22 | 1 | 0 | 1 | 0 | 1 | 22 | 1 | 0 | 1 | 0 | 1 |
| 23 | 1 | 0 | 1 | 1 | 0 | 23 | 1 | 0 | 1 | 1 | 0 |
| 24 | 1 | 0 | 1 | 1 | 1 | 24 | 1 | 0 | 1 | 1 | 1 |
| 25 | 1 | 1 | 0 | 0 | 0 | 25 | 1 | 1 | 0 | 0 | 0 |
| 26 | 1 | 1 | 0 | 0 | 1 | 26 | 1 | 1 | 0 | 0 | 1 |
| 27 | 1 | 1 | 0 | 1 | 0 | 27 | 1 | 1 | 0 | 1 | 0 |
| 28 | 1 | 1 | 0 | 1 | 1 | 28 | 1 | 1 | 0 | 1 | 1 |
| 29 | 1 | 1 | 1 | 0 | 0 | 29 | 1 | 1 | 1 | 0 | 0 |
| 30 | 1 | 1 | 1 | 0 | 1 | 30 | 1 | 1 | 1 | 0 | 1 |
| 31 | 1 | 1 | 1 | 1 | 0 | 31 | 1 | 1 | 1 | 1 | 0 |
| 32 | 1 | 1 | 1 | 1 | 1 | 32 | 1 | 1 | 1 | 1 | 1 |

If a value of kstart = 32 and kstop = 1 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop shall be treated as an error by the UE.

#### 4.6.1.2	Timeslot information mapping

##### 4.6.1.2.1	1.28 Mcps TDD

For 1.28 Mcps, the timeslots to be used for HS-PDSCH resources are signalled by the bits xts,1, xts,2, …, xts,5, where bit xts,n carries the information for timeslot n+1. Timeslot 1 cannot be used for HS-DSCH resources. If the signalling bit is set (i.e. equal to 1), then the corresponding timeslot shall be used for HS-PDSCH resources. Otherwise, the timeslot shall not be used. If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, all used timeslots shall use the same channelisation code set, as signalled by the channelisation code set information bits. Otherwise, the used timeslots may use different channelization code sets as described in 4.6.1.1.1.

When indicated by the higher layer that Timeslot 0 can be used for HS-PDSCH, bit xts,1 carries the information for timeslot 0. If xts,1 is set (i.e. equal to 1), Timeslot 0 shall be used for HS-PDSCH resource. Otherwise, Timeslot 0 shall not be used.

##### 4.6.1.2.2	3.84 Mcps TDD and 7.68Mcps TDD

For 3.84 Mcps, the timeslots to be used for HS-PDSCH resources are signalled by the bits xts,1, xts,2, …, xts,13, where bit xts,n carries the information for the nth available timeslot for HS-PDSCH resources, where the order of the timeslots available for HS-PDSCH resources shall be the same as the order of the 15 time slots within each frame with the following two slots removed:

-	The slot containing the P-CCPCH

-	The first slot in a frame containing the PRACH

If the P-CCPCH and/or PRACH are assigned to some, but not all frames, then the corresponding time slots shall remain unavailable for these frames as well..

If the bit is set (i.e. equal to 1), then the corresponding timeslot shall be used for HS-PDSCH resources. Otherwise, the timeslot shall not be used. All used timeslots shall use the same channelisation code set, as signalled by the channelisation code set information bits.

#### 4.6.1.3	Modulation scheme information mapping

The modulation scheme to be used by the HS-PDSCH resources shall be signalled by bit xms,1. If 64QAM is not supported by the UE, the mapping scheme in Table 17 shall apply.

Table 17: Modulation scheme information mapping

| xms,1 | Modulation Scheme |
| --- | --- |
| 0 | QPSK |
| 1 | 16-QAM |

If 64QAM is supported by the UE, the mapping scheme in Table 17a shall apply.

Table 17a: Modulation scheme information mapping

| xms,1 | Modulation Scheme |
| --- | --- |
| 0 | QPSK or 64QAM |
| 1 | 16-QAM |

The method of determining the modulation scheme by UE with 64QAM capability is as following:

If xms,1 = 1, then modulation scheme is 16QAM;

Else if xms,1 = 0, then

Step 1, UE first calculates the physical resource bearer capability and the transmission bit rate. The physical resource bearer capability is the maximum bit rate at which RAN can transmit with the physical resources assigned in HS-SCCH and the QPSK modulation. The physical resource bearer capability can be calculated by the channelization-code-set information and time slot information of HS-SCCH.

The transmission bit rate is the bit rate to which the transport-block size indicated in HS-SCCH corresponds.

Step 2, if the physical resource bearer capability multiplied by R is less than the transmission bit rate, and R belongs to [0, 1],

then modulation scheme is 64QAM;

else modulation scheme is QPSK.

Note: According to the simulation results the value of the transmission bit rate divided by the physical resource bearer capability according to 64QAM should be more than 1/3, where the value of R is equal to 1.

If 64QAM is configured by UE, the method of determining recommended modulation scheme by NodeB can be similar to 64QAM indication in HS-SCCH. The details are:

If xms,1 =1, NodeB should determine the recommended modulation scheme as 16QAM,

Else if xms,1 =0, NodeB calculates the physical resource bearer capability assigned in HS-SCCH corresponding to RMF and RTBS in HS-SICH, and calculates the transmission bit rate according to the RTBS in HS-SICH, and then determines whether the recommended modulation scheme is 64QAM or QPSK.

#### 4.6.1.4	Redundancy and constellation version information mapping

The redundancy version (RV) parameters r, s and constellation version parameter b are mapped jointly to produce the value Xrv. Xrv is alternatively represented as the sequence xrv,1, xrv,2, xrv,3 where xrv,1 is the MSB. This is done according to the following tables according to the modulation mode used:

Table 18: RV mapping for 16 QAM and 64 QAM

| Xrv (value) | s | r | b |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 |
| 2 | 1 | 1 | 1 |
| 3 | 0 | 1 | 1 |
| 4 | 1 | 0 | 1 |
| 5 | 1 | 0 | 2 |
| 6 | 1 | 0 | 3 |
| 7 | 1 | 1 | 0 |

Table 19: RV mapping for QPSK

| Xrv (value) | s | r |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 2 | 1 | 1 |
| 3 | 0 | 1 |
| 4 | 1 | 2 |
| 5 | 0 | 2 |
| 6 | 1 | 3 |
| 7 | 0 | 3 |

#### 4.6.1.5	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6.1.6	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

#### 4.6.1.7	HARQ process identifier mapping

The hybrid-ARQ process information  xhap,1, xhap,2, xhap,3  is unsigned binary representation of the HARQ process identifier where xhap,1 is MSB.

#### 4.6.1.8	Transport block size index mapping

The transport-block size information  xtbs,1, xtbs,2, …, xtbs,m  is unsigned binary representation of the transport block size index where xtbs,1 is MSB.

### 4.6.2	Multiplexing of HS-SCCH information

The information carried on the HS-SCCH is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1,a2...aq=xccs,1,xccs,2...xccs,q

aq+1,aq+2...aq+n=xts,1,xts,2...xts,n

aq+n+1=xms,1

aq+n+2,aq+n+3...aq+n+m+1=xtbs,1,xtbs,2...xtbs,m

aq+n+m+2,aq+n+m+3,aq+n+m+4=xhap,1,xhap,2,xhap,3

aq+n+m+5,aq+n+m+6,aq+n+m+7=xrv,1,xrv,2,xrv,3

aq+n+m+8=xnd,1

aq+n+m+9,aq+n+m+10,a19+n+m+11=xhcsn,1,xhcsn,2,xhcsn,3

### 4.6.3	CRC attachment for HS-SCCH

From the sequence of bits a1,a2,...aAa 16 bit CRC is calculated according to Section 4.2.1.1. This gives a sequence of bits y1,y2,...y16 where

yk=pim17−k	k = 1,2,…16

This latter sequence of bits is then masked with the UE identity and appended to the sequence of bits a1,a2,...aA. The bits at the output of the CRC attachment block is the sequence of bits b1,b2,...bB, where

bi=ai	i=1,2,…,A

bi=yi−A+xue,i−Amod2	i=A+1…B

### 4.6.4	Channel coding for HS-SCCH

Channel coding for the HS-SCCH shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH.

### 4.6.5	Rate matching for HS-SCCH

Rate matching for HS-SCCH shall be done with the general method described in 4.2.7.

### 4.6.6	Interleaving for HS-SCCH

Interleaving for HS-SCCH shall be done with the general method described in 4.2.11.1.

### 4.6.7	Physical Channel Segmentation for HS-SCCH

Physical channel segmentation for HS-SCCH shall be done with the general method described in 4.2.10. For 1.28 Mcps TDD, the HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2; for 3.84 Mcps TDD and 7.68Mcps TDD the HS-SCCH only uses one physical channel, see [7].

### 4.6.8	Physical channel mapping for HS-SCCH

Physical channel mapping for the HS-SCCH shall be done with the general method described in subclause 4.2.12.

## 4.6A	Coding/Multiplexing for HS-SCCH orders type A

HS-SCCH orders type A are commands sent to the UE using HS-SCCH. No HS-PDSCH is associated with HS-SCCH orders.

For 1.28 Mcps TDD, HS-SCCH order type A may be used when any of the following conditions is true:

-	HS_DSCH_RECEPTION_CELL_FACH_STATE is TRUE;

-	CONTROL_CHANNEL_DRX_STATUS is TRUE, and UE is not configured in MIMO mode;

-	CONTROL_CHANNEL_DRX_STATUS is TRUE, and UE is configured in MIMO mode while the variable  MIMO SF mode for HS-PDSCH dual stream is SF1

-	the variable HS_DSCH_SPS_STATUS is TRUE, and UE is not configured in MIMO mode;

-	the variable HS_DSCH_SPS_STATUS is TRUE, and UE is configured in MIMO mode while the variable  MIMO SF mode for HS-PDSCH dual stream is SF1.

The following information is transmitted by means of the HS-SCCH order type A physical channel.

-	Order type (3 bits):	xodt,1, xodt,2, xodt,3

-	UE identity (16 bits):	xue,1, xue,2, …, xue,16

The coding for HS-SCCH orders type A is specified in subclause 4.6.

### 4.6A.1	HS-SCCH orders type A information field mapping

#### 4.6A.1.1	Order type mapping

If xodt,1, xodt,2, xodt,3= '000', then the HS-SCCH order is an uplink synchronization establishment order when UE is in CELL_FACH or CELL_PCH state.

If xodt,1, xodt,2, xodt,3= '001', then the HS-SCCH order is an order to release the allocated semi-persistent HS-PDSCH resources when UE is in CELL_DCH state.

If xodt,1, xodt,2, xodt,3= '010', then the HS-SCCH order is DRX Activation order when UE is in CELL_DCH state.

If xodt,1, xodt,2, xodt,3= '011', then the HS-SCCH order is DRX De-activation order when UE is in CELL_DCH state.

#### 4.6A.1.2	UE identity mapping

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

## 4.6B	Coding/Multiplexing for HS-SCCH type 2 (1.28 Mcps TDD only)

HS-SCCH shall be of type 2 when any of the following conditions is met:

-	the variable HS_DSCH_SPS_STATUS is TRUE, and UE is not configured in MIMO mode;

-	the variable HS_DSCH_SPS_STATUS is TRUE, and UE is configured in MIMO mode while the variable  MIMO SF mode for HS-PDSCH dual stream is SF1.

HS-SCCH type 2 is used to allocate semi-persistent HS-PDSCH resources for the initial transmissions. The following information is transmitted by means of the HS-SCCH type 2 physical channels.

-	Type flag 1 (2 bits): xflag1,1, xflag1,2

-	Resource repetition pattern index (2bits): xrrpi,1, xrrpi,2

-	Type flag 2 (2 bits): xflag2,1, xflag2,2

-	Transport-block size information (2 bits): xtbs,1, xtbs,2

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Channelisation-code-set information (6 bits): xccs,1, xccs,2, …, xccs, 6

-	Modulation scheme information (1 bit): xms,1

-	HS-SICH indicator (2bits): xHI,1, xHI,2

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

-	Redundancy and constellation version (0 bit): Xrv =0 (see subclause 4.6.1.4)

-    Reserved (5 bits): xres,1, xres,2,… , xres,5

For an HS-SCCH order type A,

-	xflag1,1, xflag1,2, xrrpi,1, xrrpi,2 , xflag2,1, xflag2,2, xtbs,1, xtbs,2 are reserved

-	xts,1, xts,2, …, xts,5 shall be set to '00000'

-	xccs,1, xccs,2, xccs,3 shall be set to xodt,1, xodt,2, xodt,3

-	xccs,4, xccs,5, xccs,6, xms,1, xHI,1, xHI,2, xhcsn,1, xhcsn,2, xhcsn,3, xres,1, xres,2, xres,3 , xres,4, xres,5 are reserved

where xodt,1, xodt,2, xodt,3 are defined in subclause 4.6A.

The following coding/multiplexing steps for HS-SCCH type 2 can be identified:

-	multiplexing of HS-SCCH type 2 information (see subclause 4.6B.2)

-	CRC attachment for HS-SCCH type 2 (see subclause 4.6B.3);

-	channel coding for HS-SCCH type 2 (see subclause 4.6B.4);

-	rate matching for HS-SCCH type 2 (see subclause 4.6B.5);

-	interleaving for HS-SCCH type 2 (see subclause 4.6B.6);

-	mapping to physical channels for HS-SCCH type 2 (see subclauses 4.6B.7 and 4.6B.8).

The general coding/multiplexing flow for HS-SCCH type 2 is shown in Figure 19A.

Figure 19A: Coding and Multiplexing for HS-SCCH type 2

### 4.6B.1	HS-SCCH type 2 information field mapping

#### 4.6B.1.1	Type flag 1 mapping

The type flag 1 xflag1,1, xflag1,2 are mapped such that xflag1,1 ='1'corresponds to the MSB and xflag1,2 ='1' to the LSB. The type flag 1 is used to distinguish HS-SCCH type 2 from other types.

#### 4.6B.1.2	Resource repetition pattern index mapping

The resource repetition pattern index xrrpi,1, xrrpi,2 is the unsigned binary representation of a reference to one of  repetition patterns of the assigned semi-persistent HS-PDSCH resources configured by higher layers. The resource repetition pattern index xrrpi,1, xrrpi,2 are mapped such that xrrpi,1corresponds to the MSB and xrrpi,2 to the LSB. The mapping of xrrpi,1 xrrpi,2 ='00' is not used.

| xrrpi,1, xrrpi,2 | Resource repetition pattern index as signalled in the variable HS_DSCH_SPS_PARAMS[12] |
| --- | --- |
| '01' | 1st entry |
| '10' | 2nd entry |
| '11' | 3rd entry |

#### 4.6B.1.3	Type flag 2 mapping

The type flag 2 xflag2,1, xflag2,2 are mapped such that xflag2,1 ='1'corresponds to the MSB and xflag2,2 ='0' to the LSB. The type flag 2 is used to distinguish HS-SCCH type 2 from other types.

#### 4.6B.1.4	Transport block size index mapping

The transport-block size information xtbs,1, xtbs,2 is the unsigned binary representation of a reference to one of the four Transport-block sizes configured by higher layers.

| xtbs,1, xtbs,2 | Transport Block size as signalled in the variable HS_DSCH_SPS_PARAMS[12] |
| --- | --- |
| '00' | 1st entry |
| '01' | 2nd entry |
| '10' | 3rd entry |
| '11' | 4th entry |

#### 4.6B.1.5	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6B.1.6	Channelisation code set information mapping

HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code. The start code kstart is signalled by the bits xccs,1, xccs,2, xccs,3 and the stop code kstop by the bits xccs,4, xccs,5, xccs,6. The mapping in Table 16B below applies.

If a value of kstart = 9 and kstop = 6 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop are not used.

Table 16B: Channelisation code set information mapping

| kstart | xccs,1 | xccs,2 | xccs,3 | kstop | xccs,4 | xccs,5 | xccs,6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 2 | 0 | 0 | 0 |
| 3 | 0 | 0 | 1 | 4 | 0 | 0 | 1 |
| 5 | 0 | 1 | 0 | 6 | 0 | 1 | 0 |
| 7 | 0 | 1 | 1 | 8 | 0 | 1 | 1 |
| 9 | 1 | 0 | 0 | 10 | 1 | 0 | 0 |
| 11 | 1 | 0 | 1 | 12 | 1 | 0 | 1 |
| 13 | 1 | 1 | 0 | 14 | 1 | 1 | 0 |
| 15 | 1 | 1 | 1 | 16 | 1 | 1 | 1 |

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, HS-PDSCH channelization codes of all the allocated timeslots are indicated by channelisation-code-set information field.

If NON_RECTANGULAR_Resource_ ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot with the maximal timeslot index among all the timeslots scheduled to the UE and HS-PDSCH channelisation codes of the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of timeslot 0 are signalled via higher layer signalling if timeslot 0 is scheduled to the UE. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of each timeslot is scheduled to the UE with SF=1.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, HS-PDSCH channelisation codes in the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of the timeslot is scheduled to the UE with SF=1.

#### 4.6B.1.7	Modulation scheme information mapping

The mapping of the modulation scheme information xms,1 is performed according to table 17 in section 4.6.1.3.

#### 4.6B.1.8	HS-SICH indicator mapping

The HS-SICH indicator consists of 2 bits used to indicate the UE which HS-SICH will be used to convey the CQI and the acknowledgement indicator for the received data on the semi-persistent HS-PDSCH resources. The bits xHI,1, xHI,2 are mapped such that xHI,1 corresponds to the MSB and xHI,2 to the LSB.

| xHI,1, xHI,2 | HS-SICH indicator as signalled in the variable HS_DSCH_SPS_PARAMS[12] |
| --- | --- |
| '00' | 1st entry |
| '01' | 2nd entry |
| '10' | 3rd entry |
| '11' | 4th entry |

#### 4.6B.1.9	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6B.1.10	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6B.2	Multiplexing of HS-SCCH type 2 information

The information carried on the HS-SCCH type 2 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1,a2=xflag1,1,xflag1,2

a3,a4=xrrpi,1,xrrpi,2

a5,a6=xflag2,1,xflag2,2

a7,a8=xtbs,1,xtbs,2

a9,a10...a13=xts,1,xts,2...xts,5

a14,a15...a19=xccs,1,xccs,2...xccs,6

a20=xms,1

a21,a22=xHI,1,xHI,2

a23,a24,a25=xhcsn,1,xhcsn,2,xhcsn,3

a26,a27...a30=xres,1,xres,2...xres,5

### 4.6B.3	CRC attachment for HS-SCCH type 2

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6B.4	Channel coding for HS-SCCH type 2

Channel coding for the HS-SCCH type 2 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 2.

### 4.6B.5	Rate matching for HS-SCCH type 2

Rate matching for HS-SCCH type 2 shall be done with the general method described in 4.6.5.

### 4.6B.6	Interleaving for HS-SCCH type 2

Interleaving for HS-SCCH type 2 shall be done with the general method described in 4.2.11.1.

### 4.6B.7	Physical Channel Segmentation for HS-SCCH type 2

Physical channel segmentation for HS-SCCH type 2 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6B.8	Physical channel mapping for HS-SCCH type 2

Physical channel mapping for the HS-SCCH type 2 shall be done with the general method described in subclause 4.2.12.

## 4.6C	Coding/Multiplexing for HS-SCCH type 3 (1.28 Mcps TDD only)

HS-SCCH shall be of type 3 when any of the following conditions is met:

-	the variable HS_DSCH_SPS_STATUS is TRUE, and UE is not configured in MIMO mode;

-	the variable HS_DSCH_SPS_STATUS is TRUE, and UE is configured in MIMO mode while the variable MIMO SF mode for HS-PDSCH dual stream is SF1.

HS-SCCH type 3 is used to allocate one subframe HS-PDSCH resources for retransmissions of HS-DSCH semi-persistent scheduling operation. The following information is transmitted by means of the HS-SCCH type 3 physical channels.

-	Type flag 1 (2 bits): xflag1,1, xflag1,2

-	Resource repetition pattern index (2bits): xrrpi,1, xrrpi,2

-	Type flag 2 (2 bits): xflag2,1, xflag2,2

-	Transport-block size information (2 bits): xtbs,1, xtbs,2

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Channelisation-code-set information (6 bits): xccs,1, xccs,2, …, xccs, 6

-	Modulation scheme information (1 bit): xms,1

-	Redundancy version information (2 bits): xrv,1, xrv,2

-	Pointer to the previous transmission (4 bits):	xptr,1, xptr,2, xptr,3, xptr,4

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	Reserved (1 bit): xres,1

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

The following coding/multiplexing steps for HS-SCCH type 3 can be identified:

-	multiplexing of HS-SCCH type 3 information (see subclause 4.6C.2)

-	CRC attachment for HS-SCCH type 3 (see subclause 4.6C.3);

-	channel coding for HS-SCCH type 3 (see subclause 4.6C.4);

-	rate matching for HS-SCCH type 3 (see subclause 4.6C.5);

-	interleaving for HS-SCCH type 3 (see subclause 4.6C.6);

-	mapping to physical channels for HS-SCCH type 3 (see subclauses 4.6C.7 and 4.6C.8).

The general coding/multiplexing flow for HS-SCCH type 3 is shown in Figure 19B.

Figure 19B: Coding and Multiplexing for HS-SCCH type 3

### 4.6C.1	HS-SCCH type 3 information field mapping

#### 4.6C.1.1	Type flag 1 mapping

The type flag 1 xflag1,1, xflag1,2 are mapped such that xflag1,1 ='1'corresponds to the MSB and xflag1,2 ='1' to the LSB. The type flag 1 is used to distinguish HS-SCCH type 3 from other types.

#### 4.6C.1.2	Resource repetition pattern index mapping

The resource repetition pattern index xrrpi,1, xrrpi,2 are mapped such that xrrpi,1corresponds to the MSB and xrrpi,2 to the LSB. The mapping xrrpi,1, xrrpi,2='00' is used to indicate one subframe HS-PDSCH resources assigned for retransmissions of semi-persistent scheduling operation.

#### 4.6C.1.3	Type flag 2 mapping

The type flag 2 xflag2,1, xflag2,2 are mapped such that xflag2,1 ='1'corresponds to the MSB and xflag2,2 ='0' to the LSB. The type flag 2 is used to distinguish HS-SCCH type 3 from other types.

#### 4.6C.1.4	Transport block size index mapping

The transport-block size information xtbs,1, xtbs,2 is the unsigned binary representation of a reference to one of the four Transport-block sizes configured by higher layers. The mapping is performed according to section 4.6B.1.4.

#### 4.6C.1.5	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6C.1.6	Channelisation code set information mapping

The mapping of the channelisation code set information xccs,1, xccs,2, xccs,3, xccs,4, xccs,5, xccs,6 is performed according to section 4.6B.1.6.

#### 4.6C.1.7	Modulation scheme information mapping

The mapping of the modulation scheme information xms,1 is performed according to table 17 in section 4.6.1.3.

#### 4.6C.1.8	Redundancy version information mapping

The redundancy version (RV) parameters r, s and constellation version parameter b are mapped jointly to produce the value Xrv. Xrv is alternatively represented as the sequence xrv,1, xrv,2 where xrv,1 is the MSB. This is done according to the following tables according to the modulation mode used.

Table 18A: RV mapping for 16QAM and 64QAM

| Xrv (value) | Nsys / Ndata < 1/2 |  |  | Nsys / Ndata ≥ 1/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | s | r | b | s | r | b |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 2 | 1 | 0 | 2 | 0 | 0 | 0 |
| 3 | 1 | 0 | 3 | 1 | 0 | 2 |

Table 19A: RV mapping for QPSK

| Xrv (value) | Nsys / Ndata <1/2 |  | Nsys / Ndata ≥ 1/2 |  |
| --- | --- | --- | --- | --- |
|  | s | r | s | r |
| 0 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 2 | 1 | 2 | 0 | 3 |
| 3 | 1 | 3 | 1 | 2 |

#### 4.6C.1.9	Pointer to the previous transmission mapping

Pointer to the previous transmission xptr,1, xptr,2, xptr,3, xptr,4 is the unsigned binary representation of s, such that the previous transmission of the same transport block started (4+s) subframes before the start of this transmission. And xptr,1 corresponds to the MSB and xptr,4  to the LSB.

#### 4.6C.1.10	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6C.1.11	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6C.2	Multiplexing of HS-SCCH type 3 information

The information carried on the HS-SCCH type 3 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1,a2=xflag1,1,xflag1,2

a3,a4=xrrpi,1,xrrpi,2

a5,a6=xflag2,1,xflag2,2

a7,a8=xtbs,1,xtbs,2

a9,a10...a13=xts,1,xts,2...xts,5

a14,a15...a19=xccs,1,xccs,2...xccs,6

a20=xms,1

a21,a22=xrv,1,xrv,2

a23,a24...a26=xptr,1,xptr,2...xptr,4

a27,a28,a29=xhcsn,1,xhcsn,2,xhcsn,3

a30=xres,1

### 4.6C.3	CRC attachment for HS-SCCH type 3

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6C.4	Channel coding for HS-SCCH type 3

Channel coding for the HS-SCCH type 3 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 3.

### 4.6C.5	Rate matching for HS-SCCH type 3

Rate matching for HS-SCCH type 3 shall be done with the general method described in 4.6.5.

### 4.6C.6	Interleaving for HS-SCCH type 3

Interleaving for HS-SCCH type 3 shall be done with the general method described in 4.2.11.1.

### 4.6C.7	Physical Channel Segmentation for HS-SCCH type 3

Physical channel segmentation for HS-SCCH type 3 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6C.8	Physical channel mapping for HS-SCCH type 3

Physical channel mapping for the HS-SCCH type 3 shall be done with the general method described in subclause 4.2.12.

## 4.6D	Coding/Multiplexing for HS-SCCH type 4 (1.28 Mcps TDD only)

HS-SCCH shall be of type 4 when any of the following conditions is met:

-	The UE is configured in MIMO mode, and the variable MIMO SF mode for HS-PDSCH dual stream is SF1.

-	The UE is configured in MU-MIMO mode, and UE is not configured in MIMO mode.

-	The UE is configured in MU-MIMO mode, and the UE is configured in MIMO mode with the variable MIMO SF mode for HS-PDSCH dual stream being SF1.

Note :	For the UEs configured in MU-MIMO mode staying in CELL-FACH state, HS-SCCH shall be of type 1 for BCCH transmission [10].

HS-SCCH type 4 is used for single stream transmission in MIMO mode or in MU-MIMO mode. The following information is transmitted by means of the HS-SCCH type 4 physical channels.

-	Type flag 1 (1 bit): xflag1,1

-	Channelisation-code-set information (6 bits): xccs,1, xccs,2, …, xccs, 6

-	Type flag 2 (1 bit): xflag2,1

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Modulation scheme information (1 bit): xms,1

-	Transport-block size information (6 bits): xtbs,1, xtbs,2, …, xtbs,6

-	Hybrid-ARQ process information (4 bits): xhap,1, xhap,2, xhap,3, xhap,4,

-	Redundancy version information (2 bits): xrv,1, xrv,2

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	Midamble allocation scheme flag (1 bit): xflag,1

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

The following coding/multiplexing steps for HS-SCCH type 4 can be identified:

-	multiplexing of HS-SCCH type 4 information (see subclause 4.6D.2)

-	CRC attachment for HS-SCCH type 4 (see subclause 4.6D.3);

-	channel coding for HS-SCCH type 4 (see subclause 4.6D.4);

-	rate matching for HS-SCCH type 4 (see subclause 4.6D.5);

-	interleaving for HS-SCCH type 4 (see subclause 4.6D.6);

-	mapping to physical channels for HS-SCCH type 4 (see subclauses 4.6D.7 and 4.6D.8).

The general coding/multiplexing flow for HS-SCCH type 4 is shown in Figure 19C.

Figure 19C: Coding and Multiplexing for HS-SCCH type 4

### 4.6D.1	HS-SCCH type 4 information field mapping

#### 4.6D.1.1	Type flag 1 mapping

The type flag 1 bit xflag1,1 is mapped as xflag1,1 ='0'. The type flag 1 is used to distinguish HS-SCCH type 4 from other types.

#### 4.6D.1.2	Type flag 2 mapping

The type flag 2 xflag2,1 is mapped as xflag2,1 ='1'. The type flag 2 is used to distinguish HS-SCCH type 4 from other types.

#### 4.6D.1.3	Channelisation code set information mapping

When the midamble allocation scheme flag bit xflag,1='0', the mapping of the channelisation code set information xccs,1, xccs,2, xccs,3, xccs,4, xccs,5, xccs,6 is performed according to section 4.6B.1.6.

When the midamble allocation scheme flag bit xflag,1='1', the bits xccs,1, xccs,2  are comprised of the special default midamble pattern indicator (xmpi,1, xmpi,2) and xccs,1= xmpi,1 and xccs,2= xmpi,2. The mapping is shown in Table 24A below.

The start code kstart is signalled by the bits xccs,3, xccs,4 and the stop code kstop by the bits xccs,5, xccs,6. The mapping in Table 24B below applies. If a value of kstart = 13 and kstop = 4 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop are not used.

Table 24A: Special default midamble pattern mapping

| Special default Mid-amble pattern | xmpi,1 | xmpi,2 |
| --- | --- | --- |
| pattern 1A | 0 | 0 |
| pattern 1B | 0 | 1 |
| pattern 2A | 1 | 0 |
| pattern 2B | 1 | 1 |

Table 24B: Channelisation code set information mapping

| kstart | xccs,3 | xccs,4 | kstop | xccs,5 | xccs,6 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 4 | 0 | 0 |
| 5 | 0 | 1 | 8 | 0 | 1 |
| 9 | 1 | 0 | 12 | 1 | 0 |
| 13 | 1 | 1 | 16 | 1 | 1 |

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, HS-PDSCH channelization codes of all the allocated timeslots are indicated by channelisation-code-set information field.

If NON_RECTANGULAR_Resource_ ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot with the maximal timeslot index among all the timeslots scheduled to the UE and HS-PDSCH channelisation codes of the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of timeslot 0 are signalled via higher layer signalling if timeslot 0 is scheduled to the UE. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of each timeslot is scheduled to the UE with SF=1.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, HS-PDSCH channelisation codes in the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of the timeslot is scheduled to the UE with SF=1.

#### 4.6D.1.4	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6D.1.5	Modulation scheme information mapping

The mapping of the modulation scheme information xms,1 is performed according to section 4.6.1.3.

#### 4.6D.1.6	Transport block size index mapping

The transport-block size information xtbs,1, xtbs,2, …, xtbs,6 is the unsigned binary representation of the transport block size index where xtbs,1 is MSB. The mapping is performed according to section 4.6.1.8.

#### 4.6D.1.7	HARQ process identifier mapping

The hybrid-ARQ process information xhap,1, xhap,2, xhap,3, xhap,4 is unsigned binary representation of the HARQ process identifier where xhap,1 is MSB.

#### 4.6D.1.8	Redundancy version information mapping

The redundancy version (RV) parameters r, s and constellation version parameter b are mapped jointly to produce the value Xrv. Xrv is alternatively represented as the sequence xrv,1, xrv,2 where xrv,1 is the MSB. The mapping is performed according to section 4.6C.1.8. If Xrv =0, the UE shall treat the corresponding transport block as an initial transmission.

#### 4.6D.1.9	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6D.1.10	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

#### 4.6D.1.11	Midamble allocation scheme flag

When the midamble allocation scheme flag bit xflag,1='0', the default midamble allocation scheme  is used. Otherwise when the midamble allocation scheme flag bit xflag,1='1', the special default midamble allocation scheme with four patterns is used.

### 4.6D.2	Multiplexing of HS-SCCH type 4 information

The information carried on the HS-SCCH type 4 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1=xflag1,1

a2,a3,a4=xccs,1,xccs,2,xccs,3

a5=xflag2,1

a6,a7,a8=xccs,4,xccs,5,xccs,6

a9,a10...a13=xts,1,xts,2...xts,5

a14=xms,1

a15,a16...a20=xtbs,1,xtbs,2...xtbs,6

a21,a22...a24=xhap,1,xhap,2...xhap,4

a25,a26=xrv,1,xrv,2

a27,a28,a29=xhcsn,1,xhcsn,2,xhcsn,3

a30=xres,1

### 4.6D.3	CRC attachment for HS-SCCH type 4

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6D.4	Channel coding for HS-SCCH type 4

Channel coding for the HS-SCCH type 4 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 4.

### 4.6D.5	Rate matching for HS-SCCH type 4

Rate matching for HS-SCCH type 4 shall be done with the general method described in 4.6.5.

### 4.6D.6	Interleaving for HS-SCCH type 4

Interleaving for HS-SCCH type 4 shall be done with the general method described in 4.2.11.1.

### 4.6D.7	Physical Channel Segmentation for HS-SCCH type 4

Physical channel segmentation for HS-SCCH type 4 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6D.8	Physical channel mapping for HS-SCCH type 4

Physical channel mapping for the HS-SCCH type 4 shall be done with the general method described in subclause 4.2.12.

## 4.6E	Coding/Multiplexing for HS-SCCH type 5 (1.28 Mcps TDD only)

HS-SCCH shall be of type 5 when the following two conditions are both true:

-	the UE is configured in MIMO mode, and

-	the variable MIMO SF mode for HS-PDSCH dual stream is SF1.

HS-SCCH type 5 is used for dual stream transmission in MIMO mode. The following information is transmitted by means of the HS-SCCH type 5 physical channels.

-	Type flag 1(1 bit): xflag1,1

-	Modulation scheme information for stream 2 (1 bit): xms2,1

-	Transport-block size offset information for stream 2 (5 bits): xtbs2,1, xtbs2,2, …, xtbs2,5

-	Type flag 2 (1 bit): xflag2,1

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Transport-block size information for stream 1 (6 bits): xtbs1,1, xtbs1,2, …, xtbs1,6

-	Modulation scheme information for stream 1 (1 bit): xms1,1

-	Hybrid-ARQ process information (3 bits): xhap,1, xhap,2, xhap,3

-	Redundancy version information for stream 1 (2 bits): xrv1,1, xrv1,2

-	Redundancy version information for stream 2 (2 bits): xrv2,1, xrv2,2

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

The following coding/multiplexing steps for HS-SCCH type 5 can be identified:

-	multiplexing of HS-SCCH type 5 information (see subclause 4.6E.2)

-	CRC attachment for HS-SCCH type 5 (see subclause 4.6E.3);

-	channel coding for HS-SCCH type 5 (see subclause 4.6E.4);

-	rate matching for HS-SCCH type 5 (see subclause 4.6E.5);

-	interleaving for HS-SCCH type 5 (see subclause 4.6E.6);

-	mapping to physical channels for HS-SCCH type 5 (see subclauses 4.6E.7 and 4.6E.8).

The general coding/multiplexing flow for HS-SCCH type 5 is shown in Figure 19D.

Figure 19D: Coding and Multiplexing for HS-SCCH type 5

### 4.6E.1	HS-SCCH type 5 information field mapping

#### 4.6E.1.1	Type flag 1 mapping

The type flag 1 bit xflag1,1 is mapped as xflag1,1 ='1'. The type flag 1 is used to distinguish HS-SCCH type 5 from other types.

#### 4.6E.1.2	Type flag 2 mapping

The type flag 2 bits xflag2,1 is mapped as xflag2,1 ='0'. The type flag 2 is used to distinguish HS-SCCH type 5 from other types.

#### 4.6E.1.3	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6E.1.4	Modulation scheme information mapping

The mapping of the modulation scheme information for each stream (xms1,1 for stream 1 or xms2,1 for stream 2 ) is performed according to section 4.6.1.3.

#### 4.6E.1.5	Transport block size offset information mapping

The transport-block size offset information for stream 2 xtbs2,1, xtbs2,2, …, xtbs2,5 is the unsigned binary representation of the transport block size index offset for the stream 2 where xtbs2,1 is MSB. The transport-block size offset for the stream 2 subtracted from the transport-block size for the stream 1 gives the transport-block size for stream 2.

#### 4.6E.1.6	Transport block size index mapping

The transport-block size information for stream 1 xtbs1,1, xtbs1,2, …, xtbs1,6  is the unsigned binary representation of the transport block size index where xtbs,1 is MSB. The mapping is performed according to section 4.6.1.8.

#### 4.6E.1.7	HARQ process identifier mapping

The hybrid-ARQ process information xhap,1, xhap,2, xhap,3 is unsigned binary representation of the HARQ process identifier HAP1 for stream 1 where xhap,1 is MSB. The HARQ process identifier for stream 2 is HAP1+Nproc/2, where Nproc is the number of HARQ processes configured by higher layers.

#### 4.6E.1.8	Redundancy version information mapping

The redundancy version (RV) parameters r, s and constellation version parameter b are mapped jointly to produce the value Xrv. Xrv1 is alternatively represented as the sequence for stream 1 xrv1,1, xrv1,2 where xrv1,1 is the MSB. Xrv2 is alternatively represented as the sequence for stream 2 xrv2,1, xrv2,2 where xrv2,1 is the MSB. The mapping of the redundancy version for each stream is performed according to section 4.6C.1.8. If Xrv1= 0 or Xrv2 = 0, the UE shall treat the corresponding transport block as an initial transmission.

#### 4.6E.1.9	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6E.1.10	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6E.2	Multiplexing of HS-SCCH type 5 information

The information carried on the HS-SCCH type 5 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1=xflag1,1

a2=xms2,1

a3,a4=xtbs2,1,xtbs2,2

a5=xflag2,1

a6,a7,a8=xtbs2,3,xtbs2,4,xtbs2,5

a9,a10...a13=xts,1,xts,2...xts,5

a14,a15...a19=xtbs1,1,xtbs1,2...xtbs1,6

a20=xms1,1

a21,a22,a23=xhap,1,xhap,2,xhap,3

a24,a25=xrv1,1,xrv1,2

a26,a27=xrv2,1,xrv2,2

a28,a29,a30=xhcsn,1,xhcsn,2,xhcsn,3

### 4.6E.3	CRC attachment for HS-SCCH type 5

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6E.4	Channel coding for HS-SCCH type 5

Channel coding for the HS-SCCH type 5 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 5.

### 4.6E.5	Rate matching for HS-SCCH type 5

Rate matching for HS-SCCH type 5 shall be done with the general method described in 4.6.5.

### 4.6E.6	Interleaving for HS-SCCH type 5

Interleaving for HS-SCCH type 5 shall be done with the general method described in 4.2.11.1.

### 4.6E.7	Physical Channel Segmentation for HS-SCCH type 5

Physical channel segmentation for HS-SCCH type 5 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6E.8	Physical channel mapping for HS-SCCH type 5

Physical channel mapping for the HS-SCCH type 5 shall be done with the general method described in subclause 4.2.12.

## 4.6F	Coding/Multiplexing for HS-SCCH type 6 (1.28 Mcps TDD only)

HS-SCCH shall be of type 6 when the following two conditions are both met:

-	the variable HS_DSCH_SPS_STATUS is TRUE, and

-	UE is configured in MIMO mode while the variable MIMO SF mode for HS-PDSCH dual stream is SF1/SF16.

HS-SCCH type 6 is used to allocate semi-persistent HS-PDSCH resources for the initial transmissions. The following information is transmitted by means of the HS-SCCH type 6 physical channels.

-	Type flag (3 bits): xflag,1, xflag,2, xflag,3

-	Channelisation-code-set information (8 bits): xccs,1, xccs,2, …, xccs, 8

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Modulation scheme information (1 bit): xms,1

-	Resource repetition pattern index (2bits): xrrpi,1, xrrpi,2

-	Transport-block size information (2 bits): xtbs,1, xtbs,2

-	HS-SICH indicator (2bits): xHI,1, xHI,2

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	Reserved (8 bits): xres,1, xres,2,…,  xres,8

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

-	Redundancy and constellation version (0 bit ):  Xrv =0 (see 4.6.1.4)

For an HS-SCCH order type B,

-	xflag,1, xflag,2, xflag,3, xccs,1, xccs,2, xccs,3 , xccs,4, xccs,5, xccs,6, xccs,7, xccs,8 are reserved

-	xts,1, xts,2, …, xts,5 shall be set to '00000'

-	xms,1, xrrpi,1, xrrpi,2 shall be set to xodt,1, xodt,2, xodt,3

-	xtbs,1, xtbs,2, xHI,1, xHI,2, , xhcsn,1, xhcsn,2, xhcsn,3, xres,1, xres,2, xres,3, xres,4, xres,5, xres,6, xres,7, xres,8 are reserved

where xodt,1, xodt,2, xodt,3 are defined in subclause 4.6J.

The following coding/multiplexing steps for HS-SCCH type 6 can be identified:

-	multiplexing of HS-SCCH type 6 information (see subclause 4.6F.2)

-	CRC attachment for HS-SCCH type 6 (see subclause 4.6F.3);

-	channel coding for HS-SCCH type 6 (see subclause 4.6F.4);

-	rate matching for HS-SCCH type 6 (see subclause 4.6F.5);

-	interleaving for HS-SCCH type 6 (see subclause 4.6F.6);

-	mapping to physical channels for HS-SCCH type 6 (see subclauses 4.6F.7 and 4.6F.8).

The general coding/multiplexing flow for HS-SCCH type 6 is shown in Figure 19E.

Figure 19E: Coding and Multiplexing for HS-SCCH type 6

### 4.6F.1	HS-SCCH type 6 information field mapping

#### 4.6F.1.1	Type flag mapping

The type flag xflag,1, xflag,2, xflag,3 are mapped such that xflag,1 ='1'corresponds to the MSB and xflag,3 ='0' to the LSB. The bit xflag,2 is reserved. The type flag is used to distinguish HS-SCCH type 6 from other types.

#### 4.6F.1.2	Channelisation code set information mapping

The mapping of the channelisation code set information xccs,1, xccs,2, xccs,3, xccs,4, xccs,5, xccs,6, xccs,7, xccs,8 is performed according to section 4.6.1.1.1.

#### 4.6F.1.3	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6F.1.4	Modulation scheme information mapping

The mapping of the modulation scheme information xms,1 is performed according to table 17 in section 4.6.1.3.

#### 4.6F.1.5	Resource repetition pattern index mapping

The mapping of the resource repetition pattern index xrrpi,1, xrrpi,2 is performed according to section 4.6B.1.2.

#### 4.6F.1.6	Transport block size index mapping

The mapping of the transport-block size information xtbs,1, xtbs,2 is performed according to section 4.6B.1.4.

#### 4.6F.1.7	HS-SICH indicator mapping

The mapping of the HS-SICH indicator xHI,1, xHI,2 is performed according to section 4.6B.1.9.

#### 4.6F.1.8	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6F.1.9	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6F.2	Multiplexing of HS-SCCH type 6 information

The information carried on the HS-SCCH type 6 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1,a2,a3=xflag,1,xflag,2,xflag,3

a4,a5...a11=xccs,1,xccs,2...xccs,8

a12,a13...a16=xts,1,xts,2...xts,5

a17=xms,1

a18,a19=xrrpi,1,xrrpi,2

a20,a21=xtbs,1,xtbs,2

a22,a23=xHI,1,xHI,2

a24,a25,a26=xhcsn,1,xhcsn,2,xhcsn,3

a27,a28,...,a34=xres,1,xres,2,...xres,8

### 4.6F.3	CRC attachment for HS-SCCH type 6

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6F.4	Channel coding for HS-SCCH type 6

Channel coding for the HS-SCCH type 6 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 6.

### 4.6F.5	Rate matching for HS-SCCH type 6

Rate matching for HS-SCCH type 6 shall be done with the general method described in 4.6.5.

### 4.6F.6	Interleaving for HS-SCCH type 6

Interleaving for HS-SCCH type 6 shall be done with the general method described in 4.2.11.1.

### 4.6F.7	Physical Channel Segmentation for HS-SCCH type 6

Physical channel segmentation for HS-SCCH type 6 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6F.8	Physical channel mapping for HS-SCCH type 6

Physical channel mapping for the HS-SCCH type 6 shall be done with the general method described in subclause 4.2.12.

## 4.6G	Coding/Multiplexing for HS-SCCH type 7 (1.28 Mcps TDD only)

HS-SCCH shall be of type 7 when the following two conditions are both met:

-	the variable HS_DSCH_SPS_STATUS is TRUE, and

-	UE is configured in MIMO mode while the variable MIMO SF mode for HS-PDSCH dual stream is SF1/SF16.

HS-SCCH type 7 is used to allocate one subframe HS-PDSCH resources for retransmissions of HS-DSCH semi-persistent scheduling operation. The following information is transmitted by means of the HS-SCCH type 7 physical channels.

-	Type flag (3 bits): xflag,1, xflag,2, xflag,3

-	Channelisation-code-set information (8 bits): xccs,1, xccs,2, …, xccs, 8

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Modulation scheme information (1 bit): xms,1

-	Resource repetition pattern index (2bits): xrrpi,1, xrrpi,2

-	Transport-block size information (2 bits): xtbs,1, xtbs,2

-	Redundancy version information (2 bits): xrv,1, xrv,2

-	Pointer to the previous transmission (4 bits):	xptr,1, xptr,2, xptr,3, xptr,4

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	Reserved (4 bits): xres,1, xres,2, xres,3, xres,4,

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

The following coding/multiplexing steps for HS-SCCH type 7 can be identified:

-	multiplexing of HS-SCCH type 7 information (see subclause 4.6G.2)

-	CRC attachment for HS-SCCH type 7 (see subclause 4.6G.3);

-	channel coding for HS-SCCH type 7 (see subclause 4.6G.4);

-	rate matching for HS-SCCH type 7 (see subclause 4.6G.5);

-	interleaving for HS-SCCH type 7 (see subclause 4.6G.6);

-	mapping to physical channels for HS-SCCH type 7 (see subclauses 4.6G.7 and 4.6G.8).

The general coding/multiplexing flow for HS-SCCH type 7 is shown in Figure 19F.

Figure 19F: Coding and Multiplexing for HS-SCCH type 7

### 4.6G.1	HS-SCCH type 7 information field mapping

#### 4.6G.1.1	Type flag mapping

The type flag xflag,1, xflag,2, xflag,3 are mapped such that xflag,1 ='1'corresponds to the MSB and xflag,3 ='0' to the LSB. The bit xflag,2 is reserved. The type flag is used to distinguish HS-SCCH type 7 from other types.

#### 4.6G.1.2	Channelisation code set information mapping

The mapping of the channelisation code set information xccs,1, xccs,2, xccs,3, xccs,4, xccs,5, xccs,6, xccs,7, xccs,8 is performed according to section 4.6.1.1.1.

#### 4.6G.1.3	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6G.1.4	Modulation scheme information mapping

The mapping of the modulation scheme information xms,1 is performed according to table 17 in section 4.6.1.3.

#### 4.6G.1.5	Resource repetition pattern index mapping

The mapping of the resource repetition pattern index bits xrrpi,1, xrrpi,2 is performed according to section 4.6C.1.2.

#### 4.6G.1.6	Transport block size index mapping

The mapping of the transport-block size information xtbs,1, xtbs,2 is performed according to section 4.6B.1.4.

#### 4.6G.1.7	Redundancy version information mapping

The mapping of the redundancy version xrv,1, xrv,2 is performed according to section 4.6C.1.8.

#### 4.6G.1.8	Pointer to the previous transmission mapping

The mapping of the pointer to the previous transmission xptr,1, xptr,2, xptr,3 is performed according to section 4.6C.1.9.

#### 4.6G.1.9	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6G.1.10	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6G.2	Multiplexing of HS-SCCH type 7 information

The information carried on the HS-SCCH type 7 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1,a2,a3=xflag,1,xflag,2,xflag,3

a4,a5...a11=xccs,1,xccs,2...xccs,8

a12,a13...a16=xts,1,xts,2...xts,5

a17=xms,1

a18,a19=xrrpi,1,xrrpi,2

a20,a21=xtbs,1,xtbs,2

a22,a23=xrv,1,xrv,2

a24,a25...a27=xptr,1,xptr,2...xptr,4

a28,a29,a30=xhcsn,1,xhcsn,2,xhcsn,3

a31,a32,a33,a34=xres,1,xres,2,xres,3,xres,4

### 4.6G.3	CRC attachment for HS-SCCH type 7

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6G.4	Channel coding for HS-SCCH type 7

Channel coding for the HS-SCCH type 7 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 7.

### 4.6G.5	Rate matching for HS-SCCH type 7

Rate matching for HS-SCCH type 7 shall be done with the general method described in 4.6.5.

### 4.6G.6	Interleaving for HS-SCCH type 7

Interleaving for HS-SCCH type 7 shall be done with the general method described in 4.2.11.1.

### 4.6G.7	Physical Channel Segmentation for HS-SCCH type 7

Physical channel segmentation for HS-SCCH type 7 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6G.8	Physical channel mapping for HS-SCCH type 7

Physical channel mapping for the HS-SCCH type 7 shall be done with the general method described in subclause 4.2.12.

## 4.6H	Coding/Multiplexing for HS-SCCH type 8 (1.28 Mcps TDD only)

HS-SCCH shall be of type 8 when any of the following conditions is met:

-	The UE is configured in MIMO mode, and the variable MIMO SF mode for HS-PDSCH dual stream is SF1/SF16.

-	The UE is configured in MU-MIMO mode, and the UE is configured in MIMO mode with the variable MIMO SF mode for HS-PDSCH dual stream being SF1/SF16.

HS-SCCH type 8 is used for single stream transmission in MIMO mode or in MU-MIMO mode. The following information is transmitted by means of the HS-SCCH type 8 physical channels.

-	Channelisation-code-set information (4 bits): xccs,1, xccs,2…, xccs,4

-	Transport-block size information (6 bits ): xtbs,1, xtbs,2, …, xtbs,6

-	Modulation scheme information (1 bit): xms,1

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Redundancy version information (2 bits): xrv,1, xrv,2

-	Type flag 1 (6 bits): xflag1,1, xflag1,2, xflag1,3, xflag1,4, xflag1,5, xflag1,6,

-	Field flag 2 (1 bit): xflag2,1

-	Special Information (2 bits): xinfo,1, xinfo,2

-	Hybrid-ARQ process information (4 bits): xhap,1, xhap,2, xhap,3, xhap,4,

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

The following coding/multiplexing steps for HS-SCCH type 8 can be identified:

-	multiplexing of HS-SCCH type 8 information (see subclause 4.6H.2)

-	CRC attachment for HS-SCCH type 8 (see subclause 4.6H.3);

-	channel coding for HS-SCCH type 8 (see subclause 4.6H.4);

-	rate matching for HS-SCCH type 8 (see subclause 4.6H.5);

-	interleaving for HS-SCCH type 8 (see subclause 4.6H.6);

-	mapping to physical channels for HS-SCCH type 8 (see subclauses 4.6H.7 and 4.6H.8).

The general coding/multiplexing flow for HS-SCCH type 8 is shown in Figure 19G.

Figure 19G: Coding and Multiplexing for HS-SCCH type 8

### 4.6H.1	HS-SCCH type 8 information field mapping

#### 4.6H.1.1	Channelisation code set information mapping

HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code.

If the field flag xflag2,1='0', the special information xinfo,1, xinfo,2 are comprised of the channelisation code set extended information xccs,5, xccs,6 and xinfo,1= xccs,5, xinfo,2= xccs,6. The start code kstart is signalled by the bits xccs,1, xccs,2, xccs,5 and the stop code kstop by the bits xccs,3, xccs,4, xccs,6. The mapping in Table 16Ba below applies.

If a value of kstart = 5 and kstop = 2 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop are not used.

If the field flag xflag2,1='1', the bits xinfo,1, xinfo,2 are comprised of the special default midamble pattern indicator (xmpi,1, xmpi,2) and xinfo,1= xmpi,1 and xinfo,2= xmpi,2. The mapping in Table 16Bb below applies. The start code kstart is signalled by the bits xccs,1, xccs,2 and the stop code kstop by the bits xccs,3, xccs,4. The mapping in Table 16Bc below applies.

If a value of kstart =13 and kstop = 4 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop are not used

Table 16Ba: Channelisation code set information mapping

| kstart | xccs,1 | xccs,2 | xccs,5 | kstop | xccs,3 | xccs,4 | xccs,6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 2 | 0 | 0 | 0 |
| 3 | 0 | 0 | 1 | 4 | 0 | 0 | 1 |
| 5 | 0 | 1 | 0 | 6 | 0 | 1 | 0 |
| 7 | 0 | 1 | 1 | 8 | 0 | 1 | 1 |
| 9 | 1 | 0 | 0 | 10 | 1 | 0 | 0 |
| 11 | 1 | 0 | 1 | 12 | 1 | 0 | 1 |
| 13 | 1 | 1 | 0 | 14 | 1 | 1 | 0 |
| 15 | 1 | 1 | 1 | 16 | 1 | 1 | 1 |

Table 16Bb: Special default midamble pattern mapping

| Special default Mid-amble pattern | xmpi,1 | xmpi,2 |
| --- | --- | --- |
| pattern 1A | 0 | 0 |
| pattern 1B | 0 | 1 |
| pattern 2A | 1 | 0 |
| pattern 2B | 1 | 1 |

Table 16Bc: Channelisation code set information mapping

| kstart | xccs,1 | xccs,2 | kstop | xccs,3 | xccs,4 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 4 | 0 | 0 |
| 5 | 0 | 1 | 8 | 0 | 1 |
| 9 | 1 | 0 | 12 | 1 | 0 |
| 13 | 1 | 1 | 16 | 1 | 1 |

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, HS-PDSCH channelization codes of all the allocated timeslots are indicated by channelisation-code-set information field.

If NON_RECTANGULAR_Resource_ ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot with the maximal timeslot index among all the timeslots scheduled to the UE and HS-PDSCH channelisation codes of the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of timeslot 0 are signalled via higher layer signalling if timeslot 0 is scheduled to the UE. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of each timeslot is scheduled to the UE with SF=1.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, HS-PDSCH channelisation codes in the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of the timeslot is scheduled to the UE with SF=1.

#### 4.6H.1.2	Transport block size index mapping

The transport-block size information xtbs,1, xtbs,2, …, xtbs,6 is the unsigned binary representation of the transport block size index where xtbs,1 is MSB. The mapping is performed according to section 4.6.1.8.

#### 4.6H.1.3	Modulation scheme information mapping

The mapping of the modulation scheme information xms,1 is performed according to section 4.6.1.3.

#### 4.6H.1.4	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6H.1.5	Redundancy version information mapping

The redundancy version (RV) parameters r, s and constellation version parameter b are mapped jointly to produce the value Xrv. Xrv is alternatively represented as the sequence xrv,1, xrv,2 where xrv,1 is the MSB. The mapping is performed according to section 4.6C.1.8. If Xrv =0, the UE shall treat the corresponding transport block as an initial transmission.

#### 4.6H.1.6	Type flag mapping

The type flag xflag1,1, xflag1,2, xflag1,3, xflag1,4, xflag1,5, xflag1,6 is mapped as '000000'. The type flag is used to distinguish HS-SCCH type 8 from other types.

#### 4.6H.1.7	Field flag mapping

The field flag is used to distinguish the mapping of special information xinfo,1, xinfo,2.

If xflag2,1='0' , the special information x,info,1, xinfo,2 are comprised of the channelisation code set extended information xccs,5, xccs,6 and xinfo,1= xccs,5, xinfo,2= xccs,6.

If xflag2,1='1' , the special information x,info,1, xinfo,2 are comprised of the special default midamble pattern indicator (xmpi,1, xmpi,2).

#### 4.6H.1.8	Special Information mapping

The special information x,info,1, xinfo,2 is mapped such that x,info,1 corresponds to the MSB and xinfo,2 to the LSB.

If xflag2,1='0' , the special information x,info,1, xinfo,2 are comprised of the channelisation code set extended information xccs,5, xccs,6 and xinfo,1= xccs,5, xinfo,2= xccs,6.

If xflag2,1='1' , the special information x,info,1, xinfo,2 are comprised of the special default midamble pattern indicator (xmpi,1, xmpi,2).

#### 4.6H.1.9	HARQ process identifier mapping

The hybrid-ARQ process information xhap,1, xhap,2, xhap,3, xhap,4 is unsigned binary representation of the HARQ process identifier where xhap,1 is MSB.

#### 4.6H.1.10	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6H.1.11	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6H.2	Multiplexing of HS-SCCH type 8 information

The information carried on the HS-SCCH type 8 is multiplexed onto the bits a1,a2,...aAaccording to the following rule:

a1,a2,...,a4=xccs,1,xccs,2,...,xccs,4

a5,a6...a10=xtbs,1,xtbs,2...xtbs,6

a11=xms,1

a12,a13...a16=xts,1,xts,2...xts,5

a17,a18=xrv,1,xrv,2

a19,a20,...,a24=xflag1,1,xflag1,2,...,xflag1,6

a25=xflag2,1

a26,a27=xinfo,1,xinfo,2

a28,a29...a31=xhap,1,xhap,2...xhap,4

a32,a33,a34=xhcsn,1,xhcsn,2,xhcsn,3

### 4.6H.3	CRC attachment for HS-SCCH type 8

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6H.4	Channel coding for HS-SCCH type 8

Channel coding for the HS-SCCH type 8 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 8.

### 4.6H.5	Rate matching for HS-SCCH type 8

Rate matching for HS-SCCH type 8 shall be done with the general method described in 4.6.5.

### 4.6H.6	Interleaving for HS-SCCH type 8

Interleaving for HS-SCCH type 8 shall be done with the general method described in 4.2.11.1.

### 4.6H.7	Physical Channel Segmentation for HS-SCCH type 8

Physical channel segmentation for HS-SCCH type 8 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6H.8	Physical channel mapping for HS-SCCH type 8

Physical channel mapping for the HS-SCCH type 8 shall be done with the general method described in subclause 4.2.12.

## 4.6I	Coding/Multiplexing for HS-SCCH type 9 (1.28 Mcps TDD only)

HS-SCCH shall be of type 9 when the following conditions are true:

-	the UE is configured in MIMO mode, and

-	the variable MIMO SF mode for HS-PDSCH dual stream is SF1/SF16.

HS-SCCH type 9 is used for dual stream transmission in MIMO mode. The following information is transmitted by means of the HS-SCCH type 9 physical channels.

-	Channelisation-code-set information (4 bits): xccs,1, xccs,2…, xccs,4

-	Transport-block size information for stream 1 (6 bits): xtbs1,1, xtbs1,2, …, xtbs1,6

-	Modulation scheme information for stream 1 (1 bit): xms1,1

-	Time slot information (5bits): xts,1, xts,2, …, xts,5

-	Redundancy version information for stream 1 (2 bits): xrv1,1, xrv1,2

-	Transport-block size information for stream 2 (6 bits): xtbs2,1, xtbs2,2, …, xtbs2,6

-	Modulation scheme information for stream 2 (1 bit): xms2,1

-	Redundancy version information for stream 2 (2 bits): xrv2,1, xrv2,2

-	Hybrid-ARQ process information (4 bits): xhap,1, xhap,2, xhap,3, xhap,4,

-	HS-SCCH cyclic sequence number (3 bits): xhcsn,1, xhcsn,2, xhcsn,3

-	UE identity (16 bits): xue,1, xue,2, …, xue,16

The following coding/multiplexing steps for HS-SCCH type 9 can be identified:

-	multiplexing of HS-SCCH type 9 information (see subclause 4.6I.2)

-	CRC attachment for HS-SCCH type 9 (see subclause 4.6I.3);

-	channel coding for HS-SCCH type 9 (see subclause 4.6I.4);

-	rate matching for HS-SCCH type 9 (see subclause 4.6I.5);

-	interleaving for HS-SCCH type 9 (see subclause 4.6I.6);

-	mapping to physical channels for HS-SCCH type 9 (see subclauses 4.6I.7 and 4.6I.8).

The general coding/multiplexing flow for HS-SCCH type 9 is shown in Figure 19H.

Figure 19H: Coding and Multiplexing for HS-SCCH type 9

### 4.6I.1	HS-SCCH type 9 information field mapping

#### 4.6I.1.1	Channelisation code set information mapping

HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code. The start code kstart is signalled by the bits xccs,1, xccs,2 and the stop code kstop by the bits xccs,3, xccs,4. The mapping in Table 16 C below applies.

If a value of kstart = 5 and kstop = 4 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources. Other than this case, kstart > kstop are not used.

Table 16C: Channelisation code set information mapping

| kstart | xccs,1 | xccs,2 | kstop | xccs,3 | xccs,4 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 4 | 0 | 0 |
| 5 | 0 | 1 | 8 | 0 | 1 |
| 9 | 1 | 0 | 12 | 1 | 0 |
| 13 | 1 | 1 | 16 | 1 | 1 |

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, HS-PDSCH channelization codes of all the allocated timeslots are indicated by channelisation-code-set information field.

If NON_RECTANGULAR_Resource_ ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot with the maximal timeslot index among all the timeslots scheduled to the UE and HS-PDSCH channelisation codes of the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of timeslot 0 are signalled via higher layer signalling if timeslot 0 is scheduled to the UE. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of each timeslot is scheduled to the UE with SF=1.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, HS-PDSCH channelisation codes in the specific timeslot is indicated by channelisation-code set information field. The HS-PDSCH channelisation codes of other scheduled timeslots are predefined, i.e. the entire resource of the timeslot is scheduled to the UE with SF=1.

#### 4.6I.1.2	Transport block size offset information mapping

The transport-block size information for stream 1 xtbs1,1, xtbs1,2, …, xtbs1,6 is the unsigned binary representation of the transport block size index where xtbs1,1 is MSB. The mapping is performed according to section 4.6.1.8.

The transport-block size information for stream 2 xtbs2,1, xtbs2,2, …, xtbs2,6 is the unsigned binary representation of the transport block size index where xtbs2,1 is MSB. The mapping is performed according to section 4.6.1.8.

#### 4.6I.1.3	Modulation scheme information mapping

The mapping of the modulation scheme information for each stream (xms1,1 for stream 1 or xms2,1 for stream 2 ) is performed according to section 4.6.1.3.

#### 4.6I.1.4	Timeslot information mapping

The mapping of the time slot information xts,1, xts,2, … xts,5 is performed according to section 4.6.1.2.1.

#### 4.6I.1.5	Redundancy version information mapping

The mapping of the redundancy version for each stream ( xrv1,1, xrv1,2 for stream 1 and xrv2,1, xrv2,2 for stream 2 ) is performed according to section 4.6E.1.8.

#### 4.6I.1.6	HARQ process identifier mapping

The hybrid-ARQ process information xhap,1, xhap,2, xhap,3, xhap,4 is unsigned binary representation of the HARQ process identifier where xhap,1 is MSB.

For dual stream transmission, two transport blocks are transmitted on the associated HS-PDSCH(s), and the mapping relationship between the hybrid-ARQ processes and the transport blocks is such that when the HARQ-process with identifier HAP1is mapped to the transport block on stream 1, the HARQ-process with the identifier given by HAP1+Nproc/2modNproc shall be mapped to the transport block on stream 2, where Nproc is the number of HARQ processes configured by higher layers.

#### 4.6I.1.9	HS-SCCH cyclic sequence number

The HS-SCCH cyclic sequence number xhcsn,1, xhcsn,2, xhcsn,3 is mapped such that xhcsn,1 corresponds to the MSB and xhcsn,3 to the LSB.

#### 4.6I.1.10	UE identity

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

### 4.6I.2	Multiplexing of HS-SCCH type 9 information

The information carried on the HS-SCCH type 9 is multiplexed onto the bits a1,a2,...aAaccording to the following rule :

a1,a2,...,a4=xccs,1,xccs,2,...,xccs,4

a5,a6...a10=xtbs1,1,xtbs1,2...xtbs1,6

a11=xms1,1

a12,a13...a16=xts,1,xts,2...xts,5

a17,a18=xrv1,1,xrv1,2

a19,a20,...,a24=xtbs2,1,xtbs2,2,...,xtbs2,6

a25=xms2,1

a26,a27=xrv2,1,xrv2,2

a28,a29...a31=xhap,1,xhap,2...xhap,4

a32,a33,a34=xhcsn,1,xhcsn,2,xhcsn,3

### 4.6I.3	CRC attachment for HS-SCCH type 9

The sequence of bits b1,b2,...bB, is calculated according to subclause 4.6.3.

### 4.6I.4	Channel coding for HS-SCCH type 9

Channel coding for the HS-SCCH type 9 shall be done with the general method described in 4.2.3 with the following specific parameters:

The rate 1/3 convolutional coding shall be used for HS-SCCH type 9.

### 4.6I.5	Rate matching for HS-SCCH type 9

Rate matching for HS-SCCH type 9 shall be done with the general method described in 4.6.5.

### 4.6I.6	Interleaving for HS-SCCH type 9

Interleaving for HS-SCCH type 9 shall be done with the general method described in 4.2.11.1.

### 4.6I.7	Physical Channel Segmentation for HS-SCCH type 9

Physical channel segmentation for HS-SCCH type 9 shall be done with the general method described in 4.2.10. The HS-SCCH consists of two physical channels HS-SCCH1 and HS-SCCH2.

### 4.6I.8	Physical channel mapping for HS-SCCH type 9

Physical channel mapping for the HS-SCCH type 9 shall be done with the general method described in subclause 4.2.12.

## 4.6J	Coding/Multiplexing for HS-SCCH orders type B (1.28Mcps TDD only)

HS-SCCH orders type B are commands sent to the UE using HS-SCCH. No HS-PDSCH is associated with HS-SCCH orders.

HS-SCCH order type B may be used when any of the following two conditions is met:

-	CONTROL_CHANNEL_DRX_STATUS is TRUE and UE is configured in MIMO mode while the variable MIMO SF mode for HS-PDSCH dual stream is SF1/SF16;

-	The variable HS_DSCH_SPS_STATUS is TRUE, and UE is configured in MIMO mode while the variable MIMO SF mode for HS-PDSCH dual stream is SF1/SF16.

The following information is transmitted by means of the HS-SCCH order type B physical channel.

-	Order type (3 bits):	xodt,1, xodt,2, xodt,3

-	UE identity (16 bits):	xue,1, xue,2, …, xue,16

The coding for HS-SCCH orders type B is specified in subclause 4.6F.

### 4.6J.1	HS-SCCH orders type B information field mapping

#### 4.6J.1.1	Order type mapping

If xodt,1, xodt,2, xodt,3= '001', then the HS-SCCH order is an order to release the allocated semi-persistent HS-PDSCH resources when UE is in CELL_DCH state.

If xodt,1, xodt,2, xodt,3= '010', then the HS-SCCH order is DRX Activation order when UE is in CELL_DCH state.

If xodt,1, xodt,2, xodt,3= '011', then the HS-SCCH order is DRX De-activation order when UE is in CELL_DCH state.

#### 4.6J.1.2	UE identity mapping

The UE identity is the HS-DSCH Radio Network Identifier (H-RNTI) defined in [12]. This is mapped such that xue,1 corresponds to the MSB and xue,16 to the LSB, cf. [14].

## 4.7	Coding for HS-SICH

The following information, provided by higher layers, is transmitted by means of the HS-SICH physical channel.

For 1.28 Mcps TDD, HS-SICH type 1 is not used for dual stream transmission when UE is configured in MIMO mode.

In this section, the terms "HS-SICH" and "HS-SICH type 1" are used interchangeably.

For 1.28 Mcps TDD, in the case of multi-frequency HS-DSCH transmission in one TTI, HS-PDSCH on each frequency shall be configured with associated HS-SICH(s) which is coded as following.

-	Recommended Modulation Format (RMF) (1 bit): xrmf,1

-	Recommended Transport-block size (RTBS) (n bits where n = 6 for 1.28 Mcps TDD and n = 9 for 3.84 Mcps TDD and 7.68Mcps TDD): xtbs,1, xtbs,2, …, xtbs,n

-	Hybrid-ARQ information ACK/NACK (1 bit): xan,1

The following coding/multiplexing steps can be identified:

-	separate coding of RMF, RTBS and ACK/NACK (see subclause 4.7.2);

-	multiplexing of HS-SICH information (see subclause 4.7.3);

-	interleaving for HS-SICH (see subclause 4.7.4);

-	mapping to physical channels (see subclause 4.7.5).

The general coding/multiplexing flow is shown in the figure 20.

Figure 20: Coding and multiplexing for HS-SICH

### 4.7.1	HS-SICH information field mapping

#### 4.7.1.1	RMF information mapping

The RMF information bit, xrmf,1, shall be mapped according to the mapping specified in subclause 4.6.1.3.

#### 4.7.1.2	RTBS information mapping

The RTBS information bits, xtbs,1, xtbs,2, …, xtbs,n, shall be mapped according to the same mapping as is used for the transport block size information bits in subclause 4.6. This mapping is defined by higher layers [12].

#### 4.7.1.3	ACK/NACK information mapping

The ACK/NACK information bit xan,1 shall be mapped according to the mapping given in Table 20 below.

Table 20: ACK/NACK information mapping

| ACK/NACK | xan,1 |
| --- | --- |
| ACK | 1 |
| NACK | 0 |

### 4.7.2	Coding for HS-SICH

#### 4.7.2.1	Field Coding of ACK/NACK

The ACK/NACK bit xan,1 shall be repetition coded to 36 bits. The coded bits are defined as c1...c36.

#### 4.7.2.2	Field Coding of CQI

##### 4.7.2.2.1	Field Coding of CQI for 1.28 Mcps TDD

The quality information consists of Recommended Transport Block Size (RTBS) and Recommended Modulation Format (RMF) fields. The 6 bits of the RTBS field are coded to 32 bits using a (32, 6) 1st order Reed-Muller code. The coding procedure is as shown in figure 21.

Figure 21: Field coding of RTBS information bits

The coding uses a subset basis sequences as the TFCI coder as described in subclause 4.3.1.1. The basis sequences that are used for RTBS coding are as follows in table 21.

Table 21: Basis sequences for (32,6) RTBS code

| i | Mi,0 | Mi,1 | Mi,2 | Mi,3 | Mi,4 | Mi,5 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 2 | 1 | 1 | 0 | 0 | 0 | 1 |
| 3 | 0 | 0 | 1 | 0 | 0 | 1 |
| 4 | 1 | 0 | 1 | 0 | 0 | 1 |
| 5 | 0 | 1 | 1 | 0 | 0 | 1 |
| 6 | 1 | 1 | 1 | 0 | 0 | 1 |
| 7 | 0 | 0 | 0 | 1 | 0 | 1 |
| 8 | 1 | 0 | 0 | 1 | 0 | 1 |
| 9 | 0 | 1 | 0 | 1 | 0 | 1 |
| 10 | 1 | 1 | 0 | 1 | 0 | 1 |
| 11 | 0 | 0 | 1 | 1 | 0 | 1 |
| 12 | 1 | 0 | 1 | 1 | 0 | 1 |
| 13 | 0 | 1 | 1 | 1 | 0 | 1 |
| 14 | 1 | 1 | 1 | 1 | 0 | 1 |
| 15 | 1 | 0 | 0 | 0 | 1 | 1 |
| 16 | 0 | 1 | 0 | 0 | 1 | 1 |
| 17 | 1 | 1 | 0 | 0 | 1 | 1 |
| 18 | 0 | 0 | 1 | 0 | 1 | 1 |
| 19 | 1 | 0 | 1 | 0 | 1 | 1 |
| 20 | 0 | 1 | 1 | 0 | 1 | 1 |
| 21 | 1 | 1 | 1 | 0 | 1 | 1 |
| 22 | 0 | 0 | 0 | 1 | 1 | 1 |
| 23 | 1 | 0 | 0 | 1 | 1 | 1 |
| 24 | 0 | 1 | 0 | 1 | 1 | 1 |
| 25 | 1 | 1 | 0 | 1 | 1 | 1 |
| 26 | 0 | 0 | 1 | 1 | 1 | 1 |
| 27 | 1 | 0 | 1 | 1 | 1 | 1 |
| 28 | 0 | 1 | 1 | 1 | 1 | 1 |
| 29 | 1 | 1 | 1 | 1 | 1 | 1 |
| 30 | 0 | 0 | 0 | 0 | 0 | 1 |
| 31 | 0 | 0 | 0 | 0 | 1 | 1 |

The output RTBS code word bits {zi : i = 1, ... , 32} are given by:

zi=n=16xtbs,n⋅Mi−1,n−1mod2

The RMF bit xrmf,1 is repetition coded to 16 bits to produce the bits . z33,z34,...,znCQI where nCQI = 48.

##### 4.7.2.2.2	Field Coding of CQI for 3.84 Mcps TDD and 7.68Mcps TDD

RTBS and RMF bits are multiplexed onto the bits y1,y2...y10according to the following rule :

y1=xrmf,1

y2,y3...y10=xtbs,1,xtbs,2...xtbs,9

The bits y1,y2...y10are coded to produce the bits w1,w2,…w32 using a (32,10) sub-code of the second order Reed-Muller code as defined in subclause 4.3.1.1..

The bits w1,w2,…w32 are used to produce the CQI bits z1,z2...znCQI using a (4,1) repetition code, where nCQI=128, such that:

zn, zn+32, zn+64, zn+96 = wn      n=1…32

### 4.7.3	Multiplexing of HS-SICH information fields

The CQI bits z1,z2...znCQIare multiplexed with the repetition coded ACK/NACK bits c1...c36 to produce the bits d1,d2...dUwhere U	is the number of physical channel bits carried by HS-SICH, according to the following rule.:

d1,d2...dnCQI=z1,z2...znCQI

dnCQI+1,dnCQI+2...dnCQI+36=c1,c2...c36

dnCQI+37,dnCQI+38...dU=0,0....0

### 4.7.4	Interleaver for HS-SICH

Interleaver for HS-SICH shall be done with the general method described in 4.2.11.1.

### 4.7.5	Physical channel mapping for HS-SICH

Physical channel mapping for HS-SICH shall be done with the general method described in 4.2.12.

## 4.7A	Coding for HS-SICH type 2 (1.28 Mcps TDD only)

HS-SICH type 2 is used for dual stream transmission when UE is configured in MIMO mode.

The following information, provided by higher layers, is transmitted by means of the HS-SICH type 2 physical channels.

-	Recommended Modulation Format (RMF) for stream 1 (1 bit): xrmf1,1

-	Recommended Modulation Format (RMF) for stream 2 (1 bit): xrmf2,1

-	Recommended Transport-block size (RTBS) for stream 1 (n=6 bits): xtbs1,1, xtbs1,2, …, xtbs1,n

-	Recommended Transport-block size (RTBS) for stream 2 (n=6 bits): xtbs2,1, xtbs2,2, …, xtbs2,n

-	Hybrid-ARQ information ACK/NACK for stream 1 (1 bit): xan1,1

-	Hybrid-ARQ information ACK/NACK for stream 2 (1 bit): xan2,1

The following coding/multiplexing steps can be identified:

-	separate coding of RMF, RTBS and ACK/NACK (see subclause 4.7A.2);

-	multiplexing of HS-SICH information (see subclause 4.7A.3);

-	interleaving for HS-SICH (see subclause 4.7A.4);

-	mapping to physical channels (see subclause 4.7A.5).

The general coding/multiplexing flow for HS-SICH type 2 is shown in the figure 20A.

Figure 20A: Coding and multiplexing for HS-SICH type 2

### 4.7A.1	HS-SICH type 2 information field mapping

#### 4.7A.1.1	RMF information mapping

The RMF information bit for each stream, xrmf1,1 for stream 1 or xrmf2,1for stream 2, shall be mapped according to the mapping given in subclause 4.7.1.1 respectively.

#### 4.7A.1.2	RTBS information mapping

The RTBS information bits for each stream , xtbs1,1, xtbs1,2, …, xtbs1,n for stream 1 or xtbs2,1, xtbs2,2, …, xtbs2,n for stream 2, shall be mapped according to the mapping given in subclause 4.7.1.2 respectively.

#### 4.7A.1.3	ACK/NACK information mapping

The ACK/NACK information bit for each stream, xan1,1 for stream 1 or xan2,1 for stream 2, shall be mapped according to the mapping given in subclause 4.7.1.3 respectively.

### 4.7A.2	Coding for HS-SICH type 2

#### 4.7A.2.1	Field Coding of ACK/NACK

The ACK/NACK bit for stream 1, xan1,1 , shall be repetition coded to 36 bits. The coded bits are defined as c1,1...c1,36.

The ACK/NACK bit for stream 2, xan2,1 , shall be repetition coded to 36 bits. The coded bits are defined as c2,1...c2,36.

#### 4.7A.2.2	Field Coding of CQI

The quality information for each stream consists of Recommended Transport Block Size (RTBS) and Recommended Modulation Format (RMF) fields. The 6 bits of the RTBS field for each stream are coded to 32 bits using a (32, 6) 1st order Reed-Muller code respectively. The coding procedure for each stream is performed according to section 4.7.2.2 shown in figure 21A and figure 21B.

Figure 21A: Field coding of RTBS information bits for stream 1

Figure 21B: Field coding of RTBS information bits for stream 2

The RMF bit for stream 1, xrmf1,1 , is repetition coded to 16 bits to produce the bits . z1,33,z1,34,...,z1,nCQI where nCQI = 48.

The RMF bit for steam 2, xrmf2,1 , is repetition coded to 16 bits to produce the bits . z2,33,z2,34,...,z2,nCQI where nCQI = 48.

### 4.7A.3	Multiplexing of HS-SICH type 2 information fields

The CQI bits for each stream, z1,1,z1,2...z1,nCQI orz2,1,z2,2...z2,nCQI, are multiplexed with the repetition coded ACK/NACK bits for each stream, c1,1...c1,36 or c2,1...c2,36, to produce the bits d1,d2...dUwhere Uis the number of physical channel bits carried by HS-SICH, according to the following rule:

d1,d2...dnCQI=z1,1,z1,2...z1,nCQI

dnCQI+1,dnCQI+2...dnCQI+36=c1,1,c1,2...c1,36

dnCQI+37,dnCQI+38...d2∗nCQI+36=z2,1,z2,2...z2,nCQI

d2∗nCQI+37,d2∗nCQI+38...d2∗nCQI+72=c2,1,c2,2...c2,36

d2∗nCQI+73,d2∗nCQI+74...dU=0,0....0

### 4.7A.4	Interleaver for HS-SICH type 2

Interleaver for HS-SICH shall be done with the general method described in 4.2.11.1.

### 4.7A.5	Physical channel mapping for HS-SICH type 2

Physical channel mapping for HS-SICH shall be done with the general method described in 4.2.12.

## 4.8	Coding for E-DCH

Figure 22 shows the processing structure for the E-DCH transport channel mapped onto a separate CCTrCH. Data arrives to the coding unit in form of a maximum of one transport block once every transmission time interval (TTI).  A 10ms TTI is used for E-DCH for 3.84Mcps and 7.68Mcps TDD whilst for 1.28Mcps TDD, a TTI of 5ms will be used.

For 1.28Mcps TDD multi-carrier E-DCH transmission, a number of transport blocks may arrive at the coding unit in one TTI, where the number of the transport blocks equals to the number of the scheduled E-DCH carriers. Each transport block for each scheduled E-DCH carrier shall be coded separately. The following coding steps for E-DCH on one carrier can be identified:

-	add CRC to each transport block (see subclause 4.8.1);

-	code block segmentation (see subclause 4.8.2);

-	channel coding (see subclause 4.8.3);

-	hybrid ARQ (see subclause 4.8.4);

-	bit scrambling (see subclause 4.8.5);

-	interleaving for E-DCH (see subclause 4.8.6);

-	constellation re-arrangement for 16QAM (see subclause 4.8.7);

-	mapping to physical channels (see subclause 4.8.8).

Figure 22. Coding chain for E-DCH

In the following the number of transport blocks is always one. When referencing non E-DCH formulae which are used in correspondence with E-DCH formulae the convention is used that transport block subscripts may be omitted (e.g. Xi when i is always 1 may be written X).

### 4.8.1	CRC attachment for E-DCH

CRC attachment for the E-DCH transport channel shall be performed according to the general method described in 4.2.1 above with the following specific parameters.

- The CRC length shall always be L1=24 bits.

### 4.8.2	Code block segmentation for E-DCH

Code block segmentation for the E-DCH transport channel shall be performed according to the general method described in 4.2.2.2 with the following specific parameters.

- There is a maximum of one transport block. The bits bim1,bim2,bim3,...,bimBi input to the block are mapped to the bits xi1,xi2,xi3,...,xiXi directly. It follows that Xi = Bi. Note that the bits x referenced here refer only to the internals of the code block segmentation function. The output bits from the code block segmentation function are oir1, oir2, oir3, …, oirK.

- The value of Z = 5114 for turbo coding shall be used.

### 4.8.3	Channel coding for E-DCH

Channel coding for the E-DCH transport channel shall be performed according to the general method described in section 4.2.3 above with the following specific parameters.

- There is a maximum of one transport block, i=1. The rate 1/3 turbo coding shall be used.

### 4.8.4	Physical layer HARQ functionality and rate matching for E-DCH

The hybrid ARQ functionality matches the number of bits at the output of the channel coder to the total number of bits of the E-PUCH set to which the E-DCH transport channel is mapped. The hybrid ARQ functionality is controlled by the redundancy version (RV) parameters.

Figure 23: E-DCH hybrid ARQ functionality

#### 4.8.4.1	Determination of SF, modulation and number of physical channels

The SF, modulation type and number of E-PUCHs in the E-PUCH set is determined by higher layers (see [15]).  These correspond to a value of Ne,data,j.

#### 4.8.4.2	HARQ bit separation

The HARQ bit separation function shall be performed in the same way as bit separation for turbo encoded TrCHs with puncturing in 4.2.7.2.1 above.

#### 4.8.4.3	HARQ Rate Matching Stage

The hybrid ARQ rate matching for the E-DCH transport channel shall be done with the general method described in 4.2.7.3 with the following specific parameters.

The parameters of the rate matching stage depend on the value of the RV parameters s and r. The s and r combinations corresponding to each RV allowed for the E-DCH are listed in table 22 below.

Table 22: RV for E-DCH

| E-DCH RV Index | s | r |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 2 | 1 | 1 |
| 3 | 0 | 1 |

The parameter eplus, eminus and eini are calculated with the general method described in 4.5.4.3 above. The following parameters are used as input:

Nsys = Np1 = Np2 = Ne,j/3

Ndata = Ne,data,j

rmax = 2 (for both QPSK and 16-QAM)

#### 4.8.4.4	HARQ bit collection

HARQ bit collection for E-DCH shall be performed according to the general method described for HS-DSCH in subclause 4.5.4.4.

### 4.8.5	Bit scrambling

The bit scrambling for E-DCH shall be performed in accordance with the general method described in subclause 4.2.9.

### 4.8.6	Interleaving for E-DCH

Interlevaing for E-DCH shall be performed in accordance with the general method described for HS-DSCH in subclause 4.5.6.

### 4.8.7	Constellation re-arrangement for 16 QAM

Constellation rearrangement shall be performed in the case of 16-QAM in accordance with the general method described for HS-DSCH in subclause 4.5.7.  For QPSK this function is transparent.

For 3.84Mcps and 7.68Mcps, the constellation version parameter b is associated with the E-DCH RV index as shown in table 23 below.

Table 23: Mapping of RV to constellation rearrangement parameter b for E-DCH (3.84Mcps and 7.68Mcps options)

| E-DCH RV Index | b |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

For 1.28Mcps option, the constellation version parameter b is associated with the retransmission sequence number (RSN). The mapping between RSN and b parameters for constellation re-arrangement is listed in table 25A in subclause 4.9.2.1.2.

### 4.8.8	Physical channel mapping for E-DCH

The E-PUCH is defined in [7]. The bits input to the physical channel mapping are denoted by r1, r2, ..., rR, where R= Ne,data,j and is the number of physical channel data bits to be transmitted in the current TTI.  These bits are mapped to the physical channel bits, {wt,k : t = 1, 2, ..., T; and k = 1, 2, ..., Ut}, where t is the timeslot index, T is the number of timeslots in the allocation message, k is the physical channel bit index and Ut is the number of bits in the E-PUCH physical channel in timeslot t. The timeslot index, t, increases with increasing timeslot number and the physical channel bit index, k, increases with increasing physical channel bit position in time.

The bits r1, r2, ..., rR shall be mapped to the physical channel bits wt,k according to the following rule :

w1,k=rk	for k = 1, 2, ..., U1

w2,k=rk+U1	for k = 1, 2, ..., U2

…

wT,k=rk+t=1T−1Ut	for k = 1, 2, ..., UT

## 4.9	Coding for E-UCCH

### 4.9.1	Coding for E-UCCH for the 3.84Mcps and 7.68Mcps TDD options

E-UCCH is coded in two parts, E-UCCH part 1 and E-UCCH part 2.  Both parts of the E-UCCH are transmitted on the E-PUCH (see [7]).

The following information is transmitted by means of E-UCCH part 1:

-	E-TFCI information:	xtfci,1, xtfci,2, ..., xtfci,7

The following information is transmitted by means of E-UCCH part 2:

-	Retransmission sequence number (RSN):	xrsn,1, xrsn,2

-	HARQ process ID (HARQ_ID):	xharq,1, xharq,2

#### 4.9.1.1	Overview

Figure 24 below illustrates the overall coding chain for E-UCCH.

Figure 24: Coding chain for E-UCCH parts 1 and 2

#### 4.9.1.2	E-UCCH part 1

##### 4.9.1.2.1	Information field mapping of E-TFCI

The E-TFCI is mapped such that xtfci,1 corresponds to the MSB.

##### 4.9.1.2.2	Channel coding for E-UCCH part 1

The E-TFCI bits xtfci,1,xtfci,2,...,xtfci,7are zero-padded and coded to produce the bits y0, y1,…y31 using a (32,10) sub-code of the second order Reed-Muller code as defined in subclause 4.3.1.1.  Bits xtfci,1,xtfci,2,...,xtfci,7 correspond to bits a6,a5,...,a0 of subclause 4.3.1.1, and bits a9,a8,a7 of subclause 4.3.1.1 are set to zero.

##### 4.9.1.2.3	Physical channel mapping for E-UCCH part 1

E-UCCH part 1 is described in [7]. The sequence of bits y0, y1, ..., y31 output from the E-UCCH part 1 channel coding is mapped to the E-UCCH part 1 indicator field of each E-PUCH of the E-DCH TTI configured to carry E-UCCH. The bits yi {i=0,1,…31} are mapped in an identical manner to that described for TFCI in subclause 4.3.1.3 such that they are transmitted over the air in ascending order with respect to i.

#### 4.9.1.3	E-UCCH part 2

##### 4.9.1.3.1	Information field mapping of retransmission sequence number

To indicate the redundancy version (RV) of each HARQ transmission and to assist the Node B soft buffer management a two bit retransmission sequence number (RSN) is signalled from the UE to the Node B. The Node B can avoid soft buffer corruption by flushing the soft buffer associated to one HARQ process in case the last received RSN for that HARQ process is incompatible with the current one.

The RSN is set by higher layers as described in [15].  For a given HARQ process, once the maximum RSN value of 3 is reached, the RSN alternates between the values of 2 and 3 for any further retransmissions.  The RSN thus follows the pattern 0,1,2,3,2,3,2,3,2,3,…  The RSN is transmitted by the physical layer in E-UCCH part 2.

The bits (xrsn,1, xrsn,2) of the RSN field of E-UCCH part 2 are mapped such that xrsn,1 corresponds to the MSB of the RSN and xrsn,2 corresponds to the LSB of the RSN.

The applied E-DCH RV index specifying the used RV (s and r parameter) and in the case of 16-QAM, also the used constellation rearrangement parameter (b) both depend only on the values of RSN, and on Nsys / Ne,data,j as shown in table 25 below.

Table 25: Relation between RSN value and E-DCH RV Index

| RSN Value | Nsys / Ne,data,j <1/2 | 1/2 ≤ Nsys / Ne,data,j |
| --- | --- | --- |
|  | E-DCH RV Index | E-DCH RV Index |
| 0 | 0 | 0 |
| 1 | 2 | 3 |
| 2 | 0 | 2 |
| 3 | 2 | 1 |

The UE shall use either:

-	an RV index as indicated in Table 25 and according to the value of RSN

-	or, if signalled by higher layers only E-DCH RV index 0 independently of the value of RSN.

##### 4.9.1.3.2	Information field mapping of HARQ process ID

The HARQ process ID bits (xharq,1, xharq,2) transmitted on E-UCCH part 2 correspond to the two LSBs of the 3-bit HARQ ID indicated by higher layers (the MSB of the higher layer HARQ ID is not transmitted).  Thus, with the higher layer HARQ ID represented as h1, h2, h3 (with h1 as the MSB), bits xharq,1, xharq,2 correspond to bits h2, h3.

##### 4.9.1.3.3	Multiplexing of E-UCCH part 2 information

The retransmission sequence number information (xrsn,1, xrsn,2) and the HARQ process ID information (xharq,1, xharq,2) are multiplexed together to give a sequence of bits x1, x2, …, x4 where:

xk = xrsn,k	k=1,2

xk = xharq,k-2	k=3,4

##### 4.9.1.3.4	Channel coding for E-UCCH part 2

The bits x1,x2,...,x4are zero-padded and coded to produce the bits z0, z1,…z31 using a (32,10) sub-code of the second order Reed-Muller code as defined in subclause 4.3.1.1.  Bits x1,x2,...,x4 correspond to bits a3,a2,...,a0 of subclause 4.3.1.1, and bits a9,a8,...,a4 of subclause 4.3.1.1 are set to zero.

##### 4.9.1.3.5	Physical channel mapping for E-UCCH part 2

E-UCCH part 2 is described in [7]. The sequence of bits z0, z1, ..., z31 output from the E-UCCH part 2 channel coding is mapped to the E-UCCH part 2 indicator field of each E-PUCH of the E-DCH TTI configured to carry E-UCCH. The bits zi {i=0,1,…31} are mapped such that they are transmitted over the air in ascending order with respect to i.

### 4.9.2	Coding for E-UCCH for the 1.28Mcps TDD option

The E-UCCH on one carrier is used to convey the following information for the E-DCH on the same carrier:

-	The modulation type of the selected E-TFC – 0 bits (see note 1)

-	The transport block size of the selected E-TFC – 6 bits

-	The retransmission sequence number (RSN) – 2 bits

-	The HARQ process ID – 2 bits

Note 1:	The occupied modulation type is not explicitly signaled, which is inferred from the transport block size.

The E-UCCHs on the different carriers are coded independently. The E-UCCH on one carrier is transmitted on the E-PUCH on the same carrier and is coded using a (32, 10) sub code of the second order Reed Muller code as defined in subclause 4.3.1.1

Figure 24A below illustrates the overall coding chain for E-UCCH on one carrier.

Figure 24A: Coding chain for E-UCCH

#### 4.9.2.1	E-UCCH information field mapping

##### 4.9.2.1.1	Information field mapping of E-TFCI

The E-TFCI is mapped such that xtfci,1 corresponds to the MSB.

##### 4.9.2.1.2	RSN information mapping

To indicate the redundancy version (RV) of each HARQ transmission and to assist the Node B soft buffer management a two bit retransmission sequence number (RSN) is signalled from the UE to the Node B. The Node B can avoid soft buffer corruption by flushing the soft buffer associated to one HARQ process in case the last received RSN for that HARQ process is incompatible with the current one.

The RSN is set by higher layers as described in [15].  For a given HARQ process, once the maximum RSN value of 3 is reached, the RSN alternates between the values of 2 and 3 for any further retransmissions.  The RSN thus follows the pattern 0,1,2,3,2,3,2,3,2,3,….

The bits (xrsn,1, xrsn,2) of the RSN field are mapped such that xrsn,1 corresponds to the MSB of the RSN and xrsn,2 corresponds to the LSB of the RSN.

The used RV is implicitly linked to the transmitted RSN, as such the Node-B is always able to determine the correct RV if the RSN information is correctly obtained.

The constellation rearrangement parameter linkage with RSN is shown in Table 25A below

Table 25A: mapping between RSN and b parameters for CoRe

| RSN | Nsys / Ne,data,j <1/2 | 1/2 ≤ Nsys / Ne,data,j |
| --- | --- | --- |
|  | b | b |
| 0 | 0 | 0 |
| 1 | 2 | 3 |
| 2 | 3 | 0 |
| 3 | 1 | 1 |

In addition to being associated with the value of RSN, the redundancy version (RV) of the E-DCH transmission is also associated with the coding rate of the E-DCH transmission according to Table 25B and Table 25C below.

Table 25B: Relation between RSN and E-DCH RV index for QPSK

| RSN | Nsys / Ne,data,j <1/2 | 1/2 ≤ Nsys / Ne,data,j |
| --- | --- | --- |
|  | E-DCH RV Index | E-DCH RV Index |
| 0 | 0 | 0 |
| 1 | 2 | 3 |
| 2 | 0 | 2 |
| 3 | 2 | 1 |

Table 25C: Relation between RSN and E-DCH RV index for 16QAM

| RSN | Nsys / Ne,data,j <1/2 | 1/2 ≤ Nsys / Ne,data,j |
| --- | --- | --- |
|  | E-DCH RV Index | E-DCH RV Index |
| 0 | 0 | 0 |
| 1 | 0 | 3 |
| 2 | 2 | 1 |
| 3 | 2 | 2 |

The UE shall use either:

-	an RV index as indicated in Table 25A, 25B, 25C and according to the value of RSN

-	or, if signalled by higher layers only E-DCH RV index 0 independently of the value of RSN.

##### 4.9.2.1.3	HARQ information mapping

The bits (xharq,1, xharq,2) of the HARQ field are mapped such that xharq,1 corresponds to the MSB of the HARQ process ID and xharq,2 corresponds to the LSB of the HARQ process ID.

#### 4.9.2.2	Multiplexing for E-UCCH

The transport block size information (xtbs,1,…, xtbs,6), retransmission sequence number information (xrsn,1, xrsn,2) and the HARQ process ID information (xharq,1, xharq,2) are multiplexed together to give a sequence of bits x1, x2, …, x10 where:

xk = xtbs,k	k=1,2,3,4,5,6

xk = xrsn,k-6	k=7,8

xk = xharq,k-8	k=9,10

#### 4.9.2.3	Coding for E-UCCH

The bits are coded to produce the bits z0, z1,…z31 using a (32,10) sub-code of the second order Reed-Muller code as defined in subclause 4.3.1.1.

#### 4.9.2.4	Physical channel mapping for E-UCCH

The E-UCCH is described in [7]. The sequence of bits z0, z1, ..., z31 output from the E-UCCH channel coding is mapped with E-DCH to E-PUCH of the E-DCH TTI configured to carry E-UCCH.

## 4.10	Coding for E-AGCH

The E-AGCHs on the different carriers are coded independently.

For the 1.28 Mcps TDD, E-AGCH type 1 is used when the variable E_DCH_SPS_STATUS=FALSE and the UE is not configured in MU-MIMO mode by higher layers for a UE (as defined in [12]).

In this section, the terms "E-AGCH" and "E-AGCH type 1" are used interchangeably.

The E-AGCH carries the following fields multiplexed into w bits xag,1, xag,2, … xag,w.  w is within the range 14 to 28 bits (for the 3.84Mcps option) and 15 to 29 bits (for the 7.68Mcps option) and 23 to 26 bits (for the 1.28Mcps option)

-	Absolute grant (power) value (xpg,1, xpg,2, … xpg,5) {5 bits},

-	Code resource related information (xc,1, xc,2, … xc,Nc), {Nc=5 bits for the 1.28Mcps and 3.84Mcps options, Nc=6 bits for the 7.68Mcps option}

-	Timeslot resource related information (xt,1, xt,2, … xt,nTRRI), {nTRRI bits} [nTRRI =5 for 1.28Mcps, and is configured by higher layers for 3.84Mcps and 7.68Mcps options]

-	E-AGCH Cyclic Sequence Number (ECSN) (xe,1, xe,2, xe,3) (3 bits)

-	Resource duration indicator (3 bits if present) (xr,1, xr,2, xr,3) – [the presence of this field is configured by higher layers]

-	E-HICH Indicator  (xEI,1,xEI,2) (2 bits) (for 1.28Mcps TDD only)

-	E-UCCH Number Indicator (xENI,1,xENI,2, xENI,3) (3 bits) (for 1.28Mcps TDD only)

Figure 25 illustrates the overall coding chain for the E-AGCH on one carrier.

Figure 25 – TrCH processing of E-AGCH

### 4.10.1	Information Field Mapping

#### 4.10.1.1	Mapping of the Absolute Grant (Power) Value

The absolute grant (power) value (xpg,1, xpg,2,…xpg,5) is represented by 5 bits and corresponds to a dB value as specified in table 26 for 3.84Mcps and 7.68 Mcps TDD and in table 26A for 1.28Mcps TDD below.  The values are mapped such that xpg,1 corresponds to the MSB of the index.

Table 26: Mapping of Absolute Grant Value (for 3.84Mcps and 7.68 Mcps TDD)

| Absolute Grant Value | Index |
| --- | --- |
| 31 dB | 31 |
| 30 dB | 30 |
| 29 dB | 29 |
| 28 dB | 28 |
| 27 dB | 27 |
| 26 dB | 26 |
| 25 dB | 25 |
| 24 dB | 24 |
| 23 dB | 23 |
| 22 dB | 22 |
| 21 dB | 21 |
| 20 dB | 20 |
| 19 dB | 19 |
| 18 dB | 18 |
| 17 dB | 17 |
| 16 dB | 16 |
| 15 dB | 15 |
| 14 dB | 14 |
| 13 dB | 13 |
| 12 dB | 12 |
| 11 dB | 11 |
| 10 dB | 10 |
| 9 dB | 9 |
| 8 dB | 8 |
| 7 dB | 7 |
| 6 dB | 6 |
| 5 dB | 5 |
| 4 dB | 4 |
| 3 dB | 3 |
| 2 dB | 2 |
| 1 dB | 1 |
| 0 dB | 0 |

Table 26A: Mapping of Absolute Grant Value (for 1.28Mcps TDD)

| Absolute Grant Value | Index |
| --- | --- |
| 19 dB | 31 |
| 18 dB | 30 |
| 17 dB | 29 |
| 16 dB | 28 |
| 15 dB | 27 |
| 14 dB | 26 |
| 13 dB | 25 |
| 12 dB | 24 |
| 11 dB | 23 |
| 10 dB | 22 |
| 9 dB | 21 |
| 8 dB | 20 |
| 7 dB | 19 |
| 6 dB | 18 |
| 5 dB | 17 |
| 4 dB | 16 |
| 3 dB | 15 |
| 2 dB | 14 |
| 1 dB | 13 |
| 0 dB | 12 |
| -1 dB | 11 |
| -2 dB | 10 |
| -3 dB | 9 |
| -4 dB | 8 |
| -5 dB | 7 |
| -6 dB | 6 |
| -7 dB | 5 |
| -8 dB | 4 |
| -9 dB | 3 |
| -10 dB | 2 |
| -11 dB | 1 |
| -12 dB | 0 |

#### 4.10.1.2	Mapping of the Code Resource Related Information

The code resource related information (xc,1, xc,2,…xc,Nc) indicates which node on the OVSF code tree has been allocated and is represented by Nc bits where Nc=5 for 1.28Mcps and 3.84Mcps and Nc=6 for 7.68Mcps.  The mapping between the allocated OVSF and the enumerated node 0…30 (for 1.28Mcps and 3.84Mcps) and 0…62 (for 7.68Mcps) on the OVSF code tree is as given in table 27 below, in which channelisation code "i" with spreading factor "Q" is denoted as Ci(Q).  xc,1 corresponds to the MSB of the enumerated node.  The right-most column of table 27 is only applicable for the 7.68Mcps option.

Table 27 – Channelisation code mapping

| C1(1) [0] | C1(2) [1] | C1(4) [3] | C1(8) [7] | C1(16) [15] | C1(32) [31] |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | C2(32) [32] |
|  |  |  |  | C2(16) [16] | C3(32) [33] |
|  |  |  |  |  | C4(32) [34] |
|  |  |  | C2(8) [8] | C3(16) [17] | C5(32) [35] |
|  |  |  |  |  | C6(32) [36] |
|  |  |  |  | C4(16) [18] | C7(32) [37] |
|  |  |  |  |  | C8(32) [38] |
|  |  | C2(4) [4] | C3(8) [9] | C5(16) [19] | C9(32) [39] |
|  |  |  |  |  | C10(32) [40] |
|  |  |  |  | C6(16) [20] | C11(32) [41] |
|  |  |  |  |  | C12(32) [42] |
|  |  |  | C4(8) [10] | C7(16) [21] | C13(32) [43] |
|  |  |  |  |  | C14(32) [44] |
|  |  |  |  | C8(16) [22] | C15(32) [45] |
|  |  |  |  |  | C16(32) [46] |
|  | C2(2) [2] | C3(4) [5] | C5(8) [11] | C9(16) [23] | C17(32) [47] |
|  |  |  |  |  | C18(32) [48] |
|  |  |  |  | C10(16) [24] | C19(32) [49] |
|  |  |  |  |  | C20(32) [50] |
|  |  |  | C6(8) [12] | C11(16) [25] | C21(32) [51] |
|  |  |  |  |  | C22(32) [52] |
|  |  |  |  | C12(16) [26] | C23(32) [53] |
|  |  |  |  |  | C24(32) [54] |
|  |  | C4(4) [6] | C7(8) [13] | C13(16) [27] | C25(32) [55] |
|  |  |  |  |  | C26(32) [56] |
|  |  |  |  | C14(16) [28] | C27(32) [57] |
|  |  |  |  |  | C28(32) [58] |
|  |  |  | C8(8) [14] | C15(16) [29] | C29(32) [59] |
|  |  |  |  |  | C30(32) [60] |
|  |  |  |  | C16(16) [30] | C31(32) [61] |
|  |  |  |  |  | C32(32) [62] |

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, the OVSF code of all the allocated timeslots is indicated by Code Resource Related Information field.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot closest to the timeslot in which HS-SICH and/or UL DPCH is allocated among all the uplink timeslot scheduled to the UE. The OVSF code of the specific timeslot is indicated by Code Resource Related Information field. The OVSF code of other scheduled timeslots is predefined, i.e. node 0.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, the OVSF code in the specific timeslot is indicated by Code Resource Related Information field. The OVSF code of other scheduled timeslots is predefined, i.e. node 0.

#### 4.10.1.3	Mapping of the Timeslot Resource Related Information

For 3.84Mcsp and 7.68Mcps options, the timeslot resource related information (xt,1, xt,2,…xt,nTRRI) is a bitmap of length nTRRI indicating which of the timeslots configured for E-DCH use by higher layers have been allocated.  The length of the field (nTRRI) is configured by higher layers up to a maximum of 12 bits. The bitmap is arranged such that the corresponding timeslots are in ascending numerical order, with xt,1 corresponding to LSB and lowest-numbered timeslot configured for E-DCH use.

For 1.28Mcps option, the timeslot resource related information (xt,1, xt,2,…xt,nTRRI) is also a bitmap of length 5 indicating the allocation for E-DCH resources from TS1 to TS5. If the bit is set (i.e. equal to 1), then the corresponding timeslot shall be used for E-DCH resources. The bitmap is arranged such that the corresponding timeslots are in ascending numerical order, with xt,1 corresponding to MSB.

#### 4.10.1.4	Mapping of the E-AGCH Cyclic Sequence Number (ECSN)

The E-AGCH cyclic sequence number is mapped such that xe,1 corresponds to the MSB and xe,3 to the LSB.

#### 4.10.1.5	Mapping of the Resource Duration Indicator

The resource duration indicator (xr,1, xr,2,xr,3) is mapped such that xr,1 corresponds to the MSB of the resource duration index described in [15].

#### 4.10.1.6	Mapping of the E-HICH Indicator (1.28Mcps option only)

The E-HICH indicator consists of 2 bits used to indicate the UE which E-HICH will be used to convey the acknowledgement indicator in the following schedule period for 1.28Mcps TDD only. The bits (xEI,1, xEI,2) are mapped such that xEI,1 corresponds to the MSB and xEI,2 to the LSB.

#### 4.10.1.7	Mapping of the E-UCCH Number Indicator (1.28Mcps option only)

The E-UCCH number indicator is composed of 3bits which is used to calculate the number of E-UCCH for 1.28Mcps TDD only. The bits (xENI,1, xENI,2, xENI,3) are mapped such that xENI,1 corresponds to the MSB and xENI,3 to the LSB. The number of the used E-UCCH is equal to ENI+1.

### 4.10.2	Field Multiplexing

The absolute grant (power) value, code resource related information, timeslot resource related information, resource duration indicator (if present), ECSN, E-HICH indicator (for1.28Mcps TDD only) and E-UCCH number indicator (for1.28Mcps TDD only) are multiplexed together to give a sequence of bits xag,1, xag,2, …, xag,w where:

For the 1.28Mcps option:

xag,k = xpg,k	k=1, 2, .., 5

xag,k = xc,k-5	k=6, 7, ..., 10

xag,k = xt,k-10	k=11, ..., 15

xag,k = xe,k-15	k=16,17,18

if resource duration indicator (RDI) present:

xag,k = xr,k-18	k=19,20,21

xag,k = xEI,k-21	k=22,23

xag,k = xENI,k-23	k=24,25,26

or if resource duration indicator (RDI) does not present:

xag,k = xEI,k-18	k=19,20

xag,k = xENI,k-20	k=21,22,23

For the 3.84Mcps option:

xag,k = xpg,k	k=1, 2, .., 5

xag,k = xc,k-5	k=6, 7, ..., 10

xag,k = xt,k-10	k=11, ..., 10+nTRRI

xag,k = xe,k-10-nTRRI	k=11+nTRRI, ..., 13+nTRRI

xag,k = xr,k-13-nTRRI	k=14+nTRRI, …, 16+nTRRI {if resource duration indicator present}

and for the 7.68Mcps option:

xag,k = xpg,k	k=1, 2, .., 5

xag,k = xc,k-5	k=6, 7, ..., 11

xag,k = xt,k-11	k=12, ..., 11+nTRRI

xag,k = xe,k-11-nTRRI	k=12+nTRRI, ..., 14+nTRRI

xag,k = xr,k-14-nTRRI	k=15+nTRRI, …, 17+nTRRI {if resource duration indicator present}

### 4.10.3	CRC attachment

The E-RNTI (xid,1, xid,2, ..., xid,16) is the E-DCH Radio Network Identifier defined in [12]. It is mapped such that xid,1 corresponds to the MSB.

From the sequence of bits xag,1, xag,2, ..., xag,w a 16 bit CRC is calculated according to section 4.2.1.1. This gives the sequence of bits c1, c2, ..., c16 where:

ck=pim(17−k)	k=1,2,…,16

This sequence of bits is then masked with xid,1, xid,2, ..., xid,16 and appended to the sequence of bits xag,1, xag,2, ..., xag,w to form the sequence of bits y1, y2, ..., yw+16 where

yi=xag,i	i=1,2, ...,w

yi=(ci-w + xid,i-w) mod 2	i=w+1, ..., w+16

### 4.10.4	Channel Coding

1/3 rate convolutional channel coding is applied to the sequence y1, y2,…, yw+16 in accordance with section 4.2.3.1, resulting in the sequence of bits z1, z2, …, z3(w+24).

### 4.10.5	Rate Matching

Rate matching is applied to the input sequence z1, z2, …, z3(w+24) to obtain the output sequence r1, r2, …, rU, where U = 242 for burst type 1 and U = 274 for burst type 2 for 3.84Mcps and 7.68Mcps options while U=172 for 1.28Mcps option.

### 4.10.6	Interleaving

Interleaving is performed as per section 4.2.11.1 (frame-related 2nd interleaving).

### 4.10.7	Physical Channel Segmentation

Physical channel segmentation is performed as per section 4.2.10.  For 1.28Mcps TDD, the E-AGCH consists of two physical channels E-AGCH1 and E-AGCH2,whereas for 3.84Mcps and 7.68Mcps TDD the E-AGCH only uses one physical channel, see[7].Note that physical channel segmentation is transparent when only one physical channel exists

### 4.10.8	Physical Channel Mapping

Physical channel mapping is performed as per section 4.2.12.

## 4.10A	Coding for E-AGCH type 2 (1.28Mcps TDD only)

For the 1.28 Mcps TDD, E-AGCH type 2 is used when any of the following conditions is met:

-	the variable E_DCH_SPS_STATUS =TRUE for a UE (as defined in [12]).

-	the UE is configured in MU-MIMO mode.

The E-AGCH type 2 carries the following fields multiplexed into w bits xag,1, xag,2, … xag,w and w is 30 bits.

-	Absolute grant (power) value (xpg,1, xpg,2, … xpg,5) {5 bits}

-	Code resource related information (xc,1, xc,2, … xc,Nc), {Nc=4 bits }

-	Timeslot resource related information (xt,1, xt,2, … xt,nTRRI), {nTRRI =5bits}

-	E-AGCH Cyclic Sequence Number (ECSN) (xe,1, xe,2, xe,3) (3 bits)

-	Field flag (xflag,1, xflag,2) {2 bits}

-	Special information 1 (x,info1,1, xinfo1,2) {2 bits}

-	Special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) {5 bits}

-	E-UCCH Number Indicator (xENI,1,xENI,2, xENI,3) {3 bits}

-	Reserved (xres,1) {1 bit}

For an E-AGCH order,

-	xpg,1, xpg,2, xpg,3, xpg,4, xpg,5 , xc,1, xc,2, xc,3, xc,Nc are reserved

-	xt,1, xt,2, … xt,nTRRI shall be set to '00000'

-	xe,1, xe,2, xe,3 shall be set to xodt,1, xodt,2, xodt,3

-	xflag,1, xflag,2, x,info1,1, xinfo1,2, x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,, xENI,1,xENI,2, xENI,3, xres,1are reserved

where xodt,1, xodt,2, xodt,3 are defined in subclause 4.10B.

Figure 25A illustrates the overall coding chain for the E-AGCH type 2.

Figure 25A– TrCH processing of E-AGCH type2

### 4.10A.1	E-AGCH type 2 Information Field Mapping

#### 4.10A.1.1	Mapping of the Absolute Grant (Power) Value

The mapping of the absolution grant (power) value (xpg,1, xpg,2,…xpg,5) is performed as per section 4.10.1.1.

#### 4.10A.1.2	Mapping of the Code Resource Related Information

The code resource related information (xc,1, xc,2,…xc,Nc) indicates which node on the OVSF code tree has been allocated and is represented by Nc bits where Nc=4. The mapping between the allocated OVSF and the enumerated node 0…14 on the OVSF code tree is as given in table 27, in which channelisation code "i" with spreading factor "Q" is denoted as Ci(Q). xc,1 corresponds to the MSB of the enumerated node.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is FALSE, the OVSF code of all the allocated timeslots is indicated by Code Resource Related Information field.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is not configured via higher layer signalling, the specific timeslot refers to the timeslot closest to the timeslot in which HS-SICH and/or UL DPCH is allocated among all the uplink timeslot scheduled to the UE. The OVSF code of the specific timeslot is indicated by Code Resource Related Information field. The OVSF code of other scheduled timeslots is predefined, i.e. node 0.

If NON_RECTANGULAR_RESOURCE_ALLOCATION_STATUS is TRUE and non-rectangular resource specific timeslot set is configured via higher layer signalling, the OVSF code in the specific timeslot is indicated by Code Resource Related Information field. The OVSF code of other scheduled timeslots is predefined, i.e. node 0.

#### 4.10A.1.3	Mapping of the Timeslot Resource Related Information

The mapping of the timeslot resource related information (xt,1, xt,2, … xt,nTRRI) is performed as per section 4.10.1.3.

#### 4.10A.1.4	Mapping of the E-AGCH Cyclic Sequence Number (ECSN)

The mapping of E-AGCH cyclic sequence number (xe,1, xe,2, xe,3) is performed as per section 4.10.1.4.

#### 4.10A.1.5	Mapping of the Field Flag

The field flag is used to distinguish the mapping of special information 1 and special information 2. The bits (xflag,1, xflag,2) are mapped such that xflag,1 corresponds to the MSB and xflag,2 to the LSB.

If xflag,1, xflag,2='00', the special information 1(x,info1,1, xinfo1,2) are comprised of the E-HICH indicator(xEI,1, xEI,2) and the special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) are comprised of the resource duration indicator (xr,1, xr,2, xr,3) and xinfo2,4, x,info2,5 are reserved.

If xflag,1, xflag,2='01', the special information 1 (x,info1,1, xinfo1,2) are comprised of the resource repetition pattern index (xrrpi,1, xrrpi,2 ) and the special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) are reserved.

If xflag,1, xflag,2='10', the special default midamble allocation is used. The special information 1 (x,info1,1, xinfo1,2) are comprised of the E-HICH indicator (xEI,1, xEI,2) and the special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) are comprised of the resource duration indicator (xr,1, xr,2, xr,3) and the special default midamble pattern indicator (xmpi,1, xmpi,2).

xflag,1, xflag,2='11' is reserved.

#### 4.10A.1.6	Mapping of the Special Information 1

The special information 1 (x,info1,1, xinfo1,2) is mapped such that x,info1,1 corresponds to the MSB and xinfo1,2 to the LSB.

If the special information 1 are comprised of the E-HICH indicator (xEI,1, xEI,2), then x,info1,1= xEI,1and xinfo1,2= xEI,2. And the mapping of the E-HICH indicator is performed as per section 4.10.1.6.

If the special information 1 are comprised of the resource repetition pattern index (xrrpi,1, xrrpi,2 ), then x,info1,1= xrrpi,1and xinfo1,2= xrrpi,2. The resource repetition pattern index xrrpi,1, xrrpi,2 is the unsigned binary representation of a reference to one of  repetition patterns of the assigned semi-persistent E-PUCH resources configured by higher layers. The resource repetition pattern index xrrpi,1, xrrpi,2 are mapped such that xrrpi,1corresponds to the MSB and xrrpi,2 to the LSB. And if xrrpi,1 xrrpi,2 ='00', one subframe E-PUCH resource is assigned.

| xrrpi,1, xrrpi,2 | Resource repetition pattern index as signalled in the variable E_DCH _SPS_PARAMS[12] |
| --- | --- |
| '01' | 1st entry |
| '10' | 2nd entry |
| '11' | 3rd entry |

#### 4.10A.1.7	Mapping of the Special Information 2

The special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) is mapped such that x,info2,1 corresponds to the MSB and xinfo2,5 to the LSB.

If the special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) are comprised of the resource duration indicator (xr,1, xr,2,xr,3) and xinfo2,4, x,info2,5 are reserved. Then x,info2,1= xr,1, x,info2,2= xr,2and  x,info2,3= xr,3.

The mapping of the resource duration indicator is performed as per section 4.10.1.5.

If  the special information 2 (x,info2,1, xinfo2,2, x,info2,3, xinfo2,4, x,info2,5,) are comprised of the resource duration indicator (xr,1, xr,2, xr,3) and the special default midamble pattern indicator (xmpi,1, xmpi,2), then x,info2,1= xr,1, x,info2,2= xr,2, x,info2,3= xr,3,  x,info2,4= xmpi,1 and x,info2,5= xmpi,2. The mapping in Table 25A below applies.

Table 25A: Special default midamble pattern mapping

| Special default Mid-amble pattern | xmpi,1 | xmpi,2 |
| --- | --- | --- |
| pattern 1A | 0 | 0 |
| pattern 1B | 0 | 1 |
| pattern 2A | 1 | 0 |
| pattern 2B | 1 | 1 |

#### 4.10A.1.8	Mapping of the E-UCCH Number Indicator

The mapping of E-UCCH number indicator (xENI,1, xENI,2, xENI,3) is performed as per section of 4.10.1.7.

### 4.10A.2	Field Multiplexing of E-AGCH type 2

The absolute grant (power) value, code resource related information, timeslot resource related information, ECSN, field flag, special information 1, special information 2 and E-UCCH number indicator are multiplexed together to give a sequence of bits xag,1, xag,2, …, xag,w where:

xag,k = xpg,k	k=1, 2, .., 5

xag,k = xc,k-5	k=6, 7, ..., 9

xag,k = xt,k-9	k=10,11, ..., 14

xag,k = xe,k-14	k=15,16,17

xag,k = xflag,k-17	k=18,19

xag,k = xinfo1,k-19	k=20,21

xag,k = xinfo2,k-21	k=22,23,...,26

xag,k = xENI,k-26	k=27,28,29

xag,k = xres,k-29	k=30

### 4.10A.3	CRC attachment for E-AGCH type 2

The E-RNTI (xid,1, xid,2, ..., xid,16) is the E-DCH Radio Network Identifier defined in [12]. It is mapped such that xid,1 corresponds to the MSB.

The sequence of bits y1, y2, …, yw+16, is calculated according to section 4.10.3.

### 4.10A.4	Channel Coding for E-AGCH type 2

Channel coding is performed as per section 4.10.4.

### 4.10A.5	Rate Matching for E-AGCH type 2

Rate matching is performed as per section 4.10.5.

### 4.10A.6	Interleaving for E-AGCH type 2

Interleaving is performed as per section 4.10.6.

### 4.10A.7	Physical Channel Segmentation for E-AGCH type 2

Physical channel segmentation is performed as per section 4.10.7.

### 4.10A.8	Physical Channel Mapping for E-AGCH type 2

Physical channel mapping is performed as per section 4.10.8.

## 4.10B	Coding for E-AGCH orders

E-AGCH orders are commands sent to the UE using E-AGCH. No E-PUCH resources grants are associated with E-AGCH orders.

The following information is transmitted by means of the E-AGCH order physical channel.

-	Order type (3 bits):	xodt,1, xodt,2, xodt,3

-	UE identity (16 bits):	xid,1, xid,2, …, xid,16

The coding for E-AGCH orders is specified in subclause 4.10A.

### 4.10B.1	E-AGCH orders information field mapping

#### 4.10B.1.1	Order type mapping

If xodt,1, xodt,2, xodt,3= '000', then the E-AGCH order is an order to release the allocated semi-persistent E-PUCH resources when UE is in CELL_DCH state.

#### 4.10B.1.2	UE identity mapping

The E-RNTI (xid,1, xid,2, ..., xid,16) is the E-DCH Radio Network Identifier defined in [12]. It is mapped such that xid,1 corresponds to the MSB.

## 4.11	Coding for E-HICH ACK/NACK

### 4.11.1	Coding for E-HICH ACK/NACK for the 3.84Mcps and 7.68Mcps options

#### 4.11.1.1	Overview

The ACK/NACK is transmitted on the E-HICH as described in [7].

The value of a binary HARQ acknowledgement indicator for user h is denoted "ah" and may assume the value 0 or 1.  The value of the indicator is mapped as shown in table 28.

Table 28 – Mapping of HARQ acknowledgement indicator

| Command | HARQ acknowledgement indicator value (ah) |
| --- | --- |
| NACK | 0 |
| ACK | 1 |

A HARQ acknowledgement indicator is mapped to one of 240 signature sequences of length 240 bits and represented by the bit sequence bh,0, bh,1, …, bh,239 for the hth acknowledgement indicator.  The signature sequence number "r" is selected as described in [7].

The signature sequence bh,0, bh,1, …, bh,239 is constructed via coding of ah followed by bit scrambling.  Spare bits are inserted during the physical channel mapping stage to produce the output sequence dh,0, dh,1, …, dh,U,.

#### 4.11.1.2	Coding of the HARQ acknowledgement indicator

Bit ah is used to form the sequence s2,v (v=0,1,…,239) via a two-stage serialised binary spreading process as shown in figure 26.

Figure 26

The output of the first spreading stage is s1,k=ah⊕C1,i,k , where k=0, 1, 2,…, 19.

The output of the second spreading stage is s2,v=s1,w⊕C2,j,m , where v=0, 1,…, 239 and where w=v12 and, m=v mod 12.

The binary sequences selected for the first (C1,i,k) and second (C2,j,m) spreading operations are derived as a function of the HARQ acknowledgement sequence number r (see [7]) such that:

i=r12

j=r mod 12

The first orthogonal sequence set (C1,i,k) is given by table 29 and the second orthogonal sequence set (C2,j,m) is given by table 30.

Table 29 – Primary code sequences for HARQ acknowledgement indicator

| k | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1,0,k | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| C1,1,k | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| C1,2,k | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| C1,3,k | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| C1,4,k | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| C1,5,k | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| C1,6,k | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| C1,7,k | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| C1,8,k | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| C1,9,k | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| C1,10,k | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| C1,11,k | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| C1,12,k | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| C1,13,k | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| C1,14,k | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| C1,15,k | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| C1,16,k | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| C1,17,k | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| C1,18,k | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| C1,19,k | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |

Table 30 – Secondary code sequences for HARQ acknowledgement indicator

| m | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C2,0,m | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| C2,1,m | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| C2,2,m | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| C2,3,m | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| C2,4,m | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| C2,5,m | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| C2,6,m | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| C2,7,m | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| C2,8,m | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| C2,9,m | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| C2,10,m | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| C2,11,m | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |

#### 4.11.1.3	Bit scrambling of the E-HICH

The bit sequence bh,0,bh,1,…,bh,239 is formed by applying bit scrambling (as defined in subclause 4.2.9) to the sequence s2,v.

#### 4.11.1.4	Physical channel mapping of the E-HICH

The bit sequence bh,0,bh,1,…,bh,239 is segmented into two halves, bh,0,, …, bh,119, and bh,120,…,bh,239.  A sequence of U spare bits zu (u=0…U-1) are inserted between the first and second half of the sequence to form:

dh = {bh,0, bh,1, … , bh,119, z0, z1, … zU-1, bh,120, bh,121, … , bh,239}

U is equal to 4 or 36 dependant on the burst type (see [7]).  The spare bit sequence zu is not defined.

### 4.11.2	Coding for E-HICH for the1.28Mcps option only

#### 4.11.2.1	Overview

The scheduled and non-Scheduled transmissions on different E-HICHs are described in [7]. The acknowledgement indicators for the E-DCH semi-persistent scheduling operation can be transmitted on the same E-HICH carrying indicators for scheduled traffic or the E-HICH carrying indicators for non-scheduled traffic as described in [7].

For 1.28Mcps TDD multi-carrier E-DCH transmission, the acknowledgement indicators for the E-PUCH on one carrier is associated with the E-HICHs on the same carrier. The E-HICHs on the different carriers are coded independently.

The value of a binary HARQ acknowledgement indicator for user h is denoted "ah" and may assume the value 0 or 1.  The value of the indicator is mapped as same as that of 3.84Mcps shown in subclause 4.11.1.1.

Construction of the bit sequence for the hth acknowledgement indicator is achieved via a spreading process using an orthogonal sequence which is the row of an orthogonal matrix of order 80. This orthogonal matrix (C80) is Kronecker tensor product of one Hadamard matrix of order 20 (C20) and another Hadamard matrix of order 4 (C4),

is Kronecker tensor product. (note: Kronecker product is not commutative, i.e. ). The element "0" in Hadamard C20 and C4 shall be replaced by "-1" before the Kronecker tensor product operation. And after the operation the elements "-1" in C80 should be converted back into "0".

These two Hadamard matrices are given by table 31 and table 32.

Table 31: Hadamard matrix of order 4

| m | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| C4,0,m | 1 | 1 | 1 | 1 |
| C4,1,m | 1 | 0 | 1 | 0 |
| C4,2,m | 1 | 1 | 0 | 0 |
| C4,3,m | 0 | 1 | 1 | 0 |

Table 32: Hadamard matrix of order 20

| k | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C20,0,k | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| C20,1,k | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| C20,2,k | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| C20,3,k | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| C20,4,k | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| C20,5,k | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| C20,6,k | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| C20,7,k | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| C20,8,k | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| C20,9,k | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| C20,10,k | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| C20,11,k | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| C20,12,k | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| C20,13,k | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| C20,14,k | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| C20,15,k | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| C20,16,k | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| C20,17,k | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| C20,18,k | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| C20,19,k | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |

The binary orthogonal sequence (C80,r,n) used for spreading operation is selected from the rth row of the orthogonal matrix of order 80 (C80). A HARQ acknowledgement indicator is synchronously linked with the E-DCH TTI transmission to which it relates. There is thus a one-to-one association between an E-DCH TTI transmission and its respective HARQ acknowledgement indicator.

#### 4.11.2.2	Coding of the HARQ acknowledgement indicator and TPC/SS

For scheduled transmissions, E-HICHs carry HARQ acknowledgement indicators only.

When the special default midamble allocation scheme is not used for E-PUCH, a logical allocation resource tag ID "r" (r=0, 1, 2,…, 79) is calculated first for the E-DCH resource allocation associated with the HARQ acknowledgement indicator.

r=16(t0−1)+q0−116Q0

where:

t0 is the last (highest-numbered) allocated timeslot (1,2,..,5)

q0 is the lowest-numbered channelisation code index allocated in timeslot t0 (1,2,…, Q0)

Q0 is the spreading factor of the lowest-numbered channelisation code index allocated in timeslot t0

When the special default midamble allocation scheme is used for E-PUCH, a logical allocation resource tag ID "r" (r=0, 1, 2,…, 79) is calculated first associated with the HARQ acknowledgement indicator.

where:

t0, q0 and Q0 have the same definition as above .

Offset is decided by the special default midamble pattern indicator on E-AGCH and the mapping in Table33 below applies.

Table 33: Offset mapping table

| offset | xmpi,1 | xmpi,2 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 1 |
| 2 | 1 | 0 |
| 3 | 1 | 1 |

The logical resource tag ID r is then mapped to a physical allocation resource tag ID r',

r'=P(r,SFN',MidambleCode),

where P is a permutation function depends on the logical signature index r, system sub-frame number SFN' of E-HICH and the cell specific basic midamble code sequence. A 7-tap linear feedback shift register (LFSR) is used to generate pseudo-random numbers which are then used to generate the pseudo-random permutation P. The generator polynomial of the 7-tap LFSR is g(x)=1+x3+x7, as shown in Figure 27.

Figure 27: Structure of PN register

The pseudo-random permutation is generated according to the following procedures:

1. Initialization

(a) Initialize M = 80, initialize P as P(m) = m, m = 0, 1, …, M-1;

(b) Initialize N = 7;

(c) Initialize PN register with seed s, where s = LSB(SFN', N)⊕LSB(MidambleCode, N) and s6, s5, … s0 are put into the register in the order as shown in figure 27;

(d) Initialize i = 0.

2. Repeat the following steps while i <= M – 3.

(a) Find the smallest p such that M – i – 1 < 2p;

(b) Clock the PN register N times to obtain an N-bit pseudorandom number x. Set k = LSB(x, p);

(c) If k > M – i – 1, set k = k – (M – i);

(d) Swap the i-th and the (k+i)-th element of P, i.e., tmp = P(i), P(i) = P(k+i), P(k+i) = tmp;

(e) Increment i by 1.

where "LSB(x, n)" means the right most n bits of x, "⊕" means modulo 2 addition, and the first output bit from the PN register is the MSB, while the final output bit is the LSB . The resulting P is the output permutation and the physical signature sequence index is given by r'=P(r).

The output of the spreading stage is equal to bh,n=ahΘC80,r',n, where n=0,1,…,79 and Θ is the Xor operation.

For Non-Scheduled transmissions and E-DCH semi-persistent scheduling operation,, E-HICHs carry HARQ acknowledgement indicators and TPC/SS commands. The 80 orthogonal sequences are divided into 20 groups while each group includes 4 sequences of contiguous logical resource tag ID. The mapping between the logical resource tag ID and the physical tag ID is same as scheduled transmissions. Each non-scheduled user is assigned one group by higher layer to indicate the HARQ acknowledgement indicator and TPC/SS command. The first one of the four sequences is used for the acknowledgement indicator's spreading operation and one of the other three is used to indicate TPC/SS command implicitly. The mapping relations between them are described in [7].

The HARQ acknowledgement indicator is spread by the assigned orthogonal sequence (C80,s',n), where s' is the physical resource tag ID. The output of the spreading stage is equal toch,n=ahΘC80,s',n, where n=0,1,…,79. The sequence chosen to indicate TPC/SS command is denoted as "eh,n", eh,n=phΘC80,t',n, where ph is the same as the parameter B defined in [7].

#### 4.11.2.3	Bit scrambling and Physical channel mapping of the E-HICH

For scheduled transmission, the bit sequence bh,0,bh,1,…,bh,79 is segmented into two halves, bh,0,, …, bh,39, and bh,40,…,bh,79.  8 spare bits zu (u=0…7) are inserted between the first and second half of the sequence to form:

dh = {bh,0, bh,1, … , bh,39, z0, z1, … z7, bh,40, bh,41, … , bh,79}

The spare bit sequence zu is not defined.For Non-Scheduled transmission, the corresponding output bit sequences are:

dh1 = {ch,0, ch,1, … , ch,39, z0, z1, … z7, ch,40, ch,41, … , ch,79}

dh2 = {eh,0, eh,1, … , eh,39, z0, z1, … z7, eh,40, eh,41, … , eh,79}

Then the corresponding bit sequence dh or dh1/dh2 is formed by applying bit scrambling (as defined in subclause 4.2.9) to the sequence sh,n or sh1,n/sh2,n, n= 0,1,…,87.

## 4.12	Coding for E-RUCCH

For 1.28Mcps TDD, E-RUCCH supports both a 5ms and a 10ms TTI. For 3.84Mcps and 7.68Mcps TDD, E-RUCCH supports a TTI of 10ms only. The sequence of 39 information bits to be transmitted on one E-RUCCH as supplied by higher layers is defined as bits xe−rucch,1,ae−rucch,2,...ae−rucch,39.

The following processing steps (in sequential order) are defined for E-RUCCH:

-	CRC attachment

-	Channel coding

-	Rate matching

-	Bit scrambling

-	Frame related 2nd interleaving

-	Mapping to physical channels

### 4.12.1	CRC attachment for E-RUCCH

From the sequence of Ai=39 bits (xe−rucch,1,ae−rucch,2,...ae−rucch,39) a CRC of length Li=16 is appended according to Section 4.2.1.1. The bits output from CRC attachment are denoted bim,k, where k=1,2,3, … Ai+Li and Ai = 39

For 3.84Mcps and 7.68Mcps TDD, each parity bit bim,k , (where k=Ai+1,Ai+2, … Ai+Li) is then XOR'd with 1 such that its polarity is reversed.

### 4.12.2	Channel coding for E-RUCCH

1/3 rate convolutional channel coding is applied to the sequence b1, b2, …, bim,k in accordance with section 4.2.3.1, resulting in the sequence of bits z1, z2, …, z3(k+8).

### 4.12.3	Rate matching for E-RUCCH

Rate matching for E-RUCCH shall be applied to the input sequence z1, z2, …, z3(k+8) to obtain the output sequence r1, r2, …, rU, where U = 176 for 1.28Mcps and U= 232 for 3.84Mcps and 7.68Mcps TDD, according to  the general method described in subclause 4.2.7.

### 4.12.4	Bit scrambling for E-RUCCH

Bit scrambling for E-RUCCH shall be performed in accordance with the general method described in subclause 4.2.9.

### 4.12.5	Interleaving for E-RUCCH

Frame-related 2nd interleaving for E-RUCCH shall be performed in accordance with the general method described in subclause 4.2.11.1.

### 4.12.6	Physical channel mapping for E-RUCCH

Physical channel mapping for the E-RUCCH shall be performed in accordance with the general method described in subclause 4.2.12.1.

# 5	Multiplexing, channel coding and interleaving for the 3.84 Mcps MBSFN IMB option

## 5.1	General

Data streams from/to MAC and higher layers (Transport block / Transport block set) are encoded/decoded to offer transport services over the radio transmission link. The channel coding scheme is a combination of error detection, error correcting, rate matching, interleaving and the mapping of transport channels onto physical channels.

## 5.2	General coding/multiplexing of TrCHs

This section only applies to the transport channels BCH and FACH.

Data arrives to the coding/multiplexing unit in form of transport block sets once every transmission time interval. The transmission time interval is transport-channel specific from the set {10 ms, 20 ms, 40 ms, 80 ms}. An overview of the coding/multiplexing steps applicable to MBSFN IMB is shown in figure 2 of subclause 4.2 in [3].

### 5.2.1	CRC attachment

Error detection is provided on transport blocks through a Cyclic Redundancy Check (CRC). CRC attachment shall be performed in accordance with the general method described in subclause 4.2.1 of [3].

### 5.2.2	Transport block concatenation and code block segmentation

Transport blocks, with attached CRCs, are concatenated and segmentated into code blocks. Transport block concatenation and code block segmentation shall be performed in accordance with the general method described in subclause 4.2.2 of [3].

### 5.2.3	Channel coding

Code blocks are delivered to the channel coding. Channel coding shall be performed in accordance with the general method described in subclause 4.2.3 of [3]. The following restrictions apply:

-	Convolutional coding shall be applied to FACH transport channel mapped on S-CCPCH frame type 1 only.

-	Turbo coding shall be applied to FACH transport channels mapped on S-CCPCH frame type 2 only.

### 5.2.4	1st Interleaving

First interleaving shall be performed in accordance with the general methods described in subclause 4.2.5 of [3].

### 5.2.5	Radio frame segmentation

Radio frame segmentation shall be performed in accordance with the general method described in subclause 4.2.6 of [3].

### 5.2.6	Rate matching

Rate matching shall be performed in accordance with the general method described in subclause 4.2.7 of [3], in which the downlink description is only relevant. Only flexible positions of TrCHs are supported for the MBSFN IMB option.

For S-CCPCH frame type 2, the number of bits available to the CCTrCH in one radio frame using P physical channels is defined as Ndata,* = 3Ndata1+ (P-1)3 N'data1, where Ndata1 is defined in [2] and N'data1 is the data size of the corresponding slot format with zero TFCI bits.

### 5.2.7	TrCH multiplexing

Transport channel multiplexing shall be performed in accordance with the general method described in subclause 4.2.8 of [3].

### 5.2.8	Insertion of discontinuous transmission (DTX) indication bits

DTX is used to fill up the radio frame with bits. The DTX indication bits inserted in this step shall be placed at the end of the radio frame, corresponding to flexible positions of TrCHs in the radio frame [3]. Note that the DTX indication bits will be distributed over all slots after 2nd interleaving.

The bits input to the DTX insertion block are denoted by, where S is the number of bits from TrCH multiplexing. The bits output from the DTX insertion block are denoted byw1,w2,w3,…,wWand defined by the following relations:

k = 1, 2, 3, …,S

k = S+1, S+2, S+3, …,W

where DTX indication bits are denoted by . Note that bits wk are three valued and that W = Ndata,*.

### 5.2.9	Physical channel segmentation

For the 3.84 Mcps MBSFN IMB option, physical channel segmentation is applicable to S-CCPCH frame type 2 only.

When more than one PhCH is used, physical channel segmentation divides the bits among the different PhCHs. The bits input to the physical channel segmentation are denoted byw1,w2,w3,…,wW, where W is the number of bits input to the physical channel segmentation block. The number of PhCHs is denoted by P.

The bits after physical channel segmentation are denoted, where p is PhCH number and Up is the number of bits in one radio frame of the pth PhCH. The relation between wk and up,k is given below.

Bits on first PhCH after physical channel segmentation:

u1,k=wk	k = 1, 2 , …, U1

Bits on second PhCH after physical channel segmentation:

u2,k=w(k+U1)	k = 1, 2 , …, U2

…

Bits on the Pth PhCH after physical channel segmentation:

uP,k=w(k+U1+…+UP−1)	k = 1, 2 , …, UP

The number of bits Up in one radio frame of the pth PhCH is 3Ndata1 for p=1 and 3N'data1 for p = 2,3,…, P, where Ndata1 is defined in [2] and N'data1 is the data size of the corresponding slot format with zero TFCI bits.

### 5.2.10	2nd Interleaving

Second interleaving is performed in accordance with the general method described in section 4.2.11 of [3]. For S-CCPCH frame type 2, 2nd interleaving is performed per physical channel of the CCTrCH.

### 5.2.11	Physical channel mapping

Physical channel mapping shall be performed in accordance with the general method described in subclause 4.2.12 in [3], in which the downlink description is only relevant.

### 5.2.12	Restrictions on different types of CCTrCHs

Restrictions on the different types of CCTrCHs are described in general terms in [16]. In this subclause those restrictions are given with layer 1 notation.

#### 5.2.12.1	Broadcast channel (BCH)

The restrictions of subclause 4.2.13.6 of [3] apply.

#### 5.2.12.2	Forward access channel (FACH)

-	The maximum value of the number of TrCHs I in a CCTrCH and the maximum value of the number of transport blocks Mi on each transport channel are given from the UE capability class.

-	Only one Secondary CCPCH frame type 1 is used per CCTrCH.

-	The maximum number of Secondary CCPCHs of frame type 2 used per CCTrCH is defined in [17].

### 5.2.13	Multiplexing of different TrCHs into one CCTrCH, and mapping of one CCTrCH onto physical channels

Multiplexing of different transport channels into one coded composite transport channel shall be performed in accordance with the general method described in 4.2.14 of [3].

## 5.3	Transport format detection

For MBSFN IMB operations, the transport format combination is signalled using the TFCI field of an S-CCPCH slot and only TFCI based detection shall be used.

### 5.3.1	Transport format detection based on TFCI

When a TFCI is available, then TFCI based detection shall be applicable to all TrCHs within the CCTrCH. The TFCI informs the receiver about the transport format combination of the CCTrCHs. As soon as the TFCI is detected, the transport format combination, and hence the transport formats of the individual transport channels are known.

### 5.3.2	Coding of Transport-Format-Combination Indicator (TFCI)

The number of TFCI bits to be encoded per CCTrCH shall always be within the range 1 to 5. These bits shall be encoded according to the general method of subclause 4.3.1.2.2 to produce 16 encoded bits b0 to b15. This procedure is valid also for the case where the number of TFCI bits to be encoded is less than 3.

### 5.3.3	Mapping of TFCI words

#### 5.3.3.1	Mapping of TFCI bits for Secondary CCPCH

The 16 encoded TFCI bits are directly mapped to the S-CCPCH slots of the radio frame. Within a slot the bit with lower index is transmitted before the bit with higher index. The coded bits bk, are mapped to the transmitted TFCI bits dk, according to the following formula:

dk = bk mod 16

For S-CCPCH frame type 1, k = 0, 1, 2, …, 29. For S-CCPCH frame type 2 using QPSK data modulation, k = 0, 1, 2, …, 47 whilst for S-CCPCH frame type 2 using 16-QAM data modulation, k = 0, 1, 2, …, 23.

The set of TFCI bits dk shall be modulated as defined in [9] and mapped to slots of the radio frame such that they are transmitted in ascending order of k.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 14/01/00 | RAN_05 | RAN_05 | - |  | Approved at TSG RAN #5 and placed under Change Control | - | 3.0.0 |
| 14/01/00 | RAN_06 | RP-99694 | 001 | 3 | Correction of rate matching parameters for repetition after 1st Interleaving in 25.222 | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99694 | 002 | 1 | Clarification of bit separation and collection | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99694 | 003 | - | Changing the initial offset value for convolutional code rate matching | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99693 | 004 | 1 | Editorial corrections to TS 25.222 | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99694 | 007 | - | Update of rate matching rule for TDD | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99694 | 009 | 1 | Modified physical channel mapping scheme | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99694 | 013 | - | Introduction of TFCI for S-CCPCH in TDD mode | 3.0.0 | 3.1.0 |
| 14/01/00 | RAN_06 | RP-99694 | 015 | - | TFCI coding and mapping in TDD | 3.0.0 | 3.1.0 |
| 14/01/00 | - | - | - |  | Change history was added by the editor | 3.1.0 | 3.1.1 |
| 31/03/00 | RAN_07 | RP-000068 | 017 | - | Corrections to TS 25.222 | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 018 | - | Refinements of Physical Channel Mapping | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 019 | 1 | TFCI coding specification in TDD | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 021 | - | Modification of Turbo code internal interleaver | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 023 | - | Update of TS 25.222 - clarification of BTFD for TDD | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 025 | - | Change of TFCI basis for TDD | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 026 | - | Padding Function for Turbo coding of small blocks | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 027 | - | Editorial modification of shifting parameter calculation for turbo code puncturing | 3.1.1 | 3.2.0 |
| 31/03/00 | RAN_07 | RP-000068 | 029 | 1 | Editorial changes of channel coding section | 3.1.1 | 3.2.0 |
| 26/06/00 | RAN_08 | RP-000272 | 030 | - | Parity bit attachment to 0 size transport block | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000272 | 031 | - | Correction of the mapping formula | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000272 | 034 | - | Alignment of Multiplexing for TDD | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000272 | 036 | 2 | Bit separation of the Turbo encoded data | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000272 | 038 | 2 | Revision of code block segmentation description | 3.2.0 | 3.3.0 |
| 26/06/00 | RAN_08 | RP-000272 | 039 | - | Editorial corrections in channel coding section | 3.2.0 | 3.3.0 |
| 23/09/00 | RAN_09 | RP-000345 | 040 | 1 | Update of TS 25.222 | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000345 | 041 | 1 | Editorial corrections in Turbo code internal interleaver section | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000345 | 042 | - | Paging Indicator Terminology | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000345 | 043 | 1 | Bit separation and collection for rate matching | 3.3.0 | 3.4.0 |
| 23/09/00 | RAN_09 | RP-000345 | 048 | - | Puncturing Limit definition in WG1 specification | 3.3.0 | 3.4.0 |
| 15/12/00 | RAN_10 | RP-000543 | 049 | - | Clarification on the Ci formula | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000543 | 050 | - | Correction on TFCI & TPC Transmission | 3.4.0 | 3.5.0 |
| 15/12/00 | RAN_10 | RP-000543 | 053 | 1 | Editorial corrections in TS 25.222 | 3.4.0 | 3.5.0 |
| 16/03/01 | RAN_11 | - | - | - | Approved as Release 4 specification (v4.0.0) at TSG RAN #11 | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010063 | 051 | 1 | Bit Scrambling for TDD | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010063 | 054 | 1 | Corrections & Clarifications for TS25.222 | 3.5.0 | 4.0.0 |
| 16/03/01 | RAN_11 | RP-010071 | 055 | 1 | Inclusion of 1.28Mcps TDD in TS 25.222 | 3.5.0 | 4.0.0 |
| 21/09/01 | RAN_13 | RP-010523 | 057 | - | TFCI Terminology | 4.0.0 | 4.1.0 |
| 21/09/01 | RAN_13 | RP-010529 | 058 | - | 5ms TTI for PRACH for 1.28 Mcps TDD | 4.0.0 | 4.1.0 |
| 21/09/01 | RAN_13 | RP-010529 | 060 | - | A correction on the meaning of FPACH in TS 25.222 | 4.0.0 | 4.1.0 |
| 14/12/01 | RAN_14 | RP-010747 | 059 | - | Bit Scrambling for TDD | 4.1.0 | 4.2.0 |
| 14/12/01 | RAN_14 | RP-010747 | 061 | - | Corrections in clause 4.1 and 4.2 of TS 25.222 | 4.1.0 | 4.2.0 |
| 08/03/02 | RAN_15 | RP-020050 | 063 | 1 | Correction to addition of padding zeros to PICH in TDD | 4.2.0 | 4.3.0 |
| 08/03/02 | RAN_15 | RP-020050 | 065 | 3 | Clarification of the requirement for the determination of the rate matching parameters and editorial corrections to 25.222 | 4.2.0 | 4.3.0 |
| 08/03/02 | RAN_15 | RP-020058 | 066 | 2 | Inclusion of HSDPA in 25.222 | 4.2.0 | 5.0.0 |
| 07/06/02 | RAN_16 | RP-020311 | 077 | - | Second Stage Interleaving and Physical Channel Mapping | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020311 | 076 | 1 | Zero padding for TFCI (3.84Mcps TDD) | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020314 | 073 | - | Correction to addition of padding zeros to PICH in 1.28 Mcps TDD | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020314 | 086 | - | Zero padding for TFCI (1.28Mcps TDD) | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020317 | 078 | 2 | Removal of inconsistencies and ambiguities in the HARQ description | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020317 | 079 | 4 | Corrections to HS-DSCH coding | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020317 | 082 | 1 | Corrections to HSDPA Multiplexing and Coding | 5.0.0 | 5.1.0 |
| 07/06/02 | RAN_16 | RP-020317 | 084 | - | Introduction of HS-SCCH Cyclic Sequence Counter for TDD | 5.0.0 | 5.1.0 |
| 19/09/02 | RAN_17 | RP-020570 | 097 | 1 | Clarification of the definition of layer 1 transport channel numbers | 5.1.0 | 5.2.0 |
| 19/09/02 | RAN_17 | RP-020585 | 093 | - | HS-DSCH Interleaving for TDD | 5.1.0 | 5.2.0 |
| 19/09/02 | RAN_17 | RP-020585 | 091 | 1 | HS-SCCH corrections for TDD | 5.1.0 | 5.2.0 |
| 19/09/02 | RAN_17 | RP-020585 | 089 | 1 | Clarification of TFRI bits for 3.84Mcps HSDPA TDD | 5.1.0 | 5.2.0 |
| 15/10/02 | - | - | - | - | Moving of misplaced sections 4.6.1.5 and 4.6.1.6 | 5.2.0 | 5.2.1 |
| 22/12/02 | RAN_18 | RP-020843 | 100 | - | Corrections to TFCI encoding of very short TFCI lengths | 5.2.1 | 5.3.0 |
| 22/12/02 | RAN_18 | RP-020843 | 102 | - | Corrections to TFCI encoding of very short TFCI lengths | 5.2.1 | 5.3.0 |
| 22/12/02 | RAN_18 | RP-020849 | 103 | - | Correction of editorial Error | 5.2.1 | 5.3.0 |
| 22/12/02 | RAN_18 | RP-020849 | 104 | - | Miscellaneous Minor HSDPA Corrections | 5.2.1 | 5.3.0 |
| 22/12/02 | RAN_18 | RP-020852 | 107 | - | Editorial modification to the section numberings | 5.2.1 | 5.3.0 |
| 25/03/03 | RAN_19 | RP-030139 | 108 | 1 | HSDPA corrections | 5.3.0 | 5.4.0 |
| 25/03/03 | RAN_19 | RP-030139 | 109 | 3 | Miscellaneous Corrections | 5.3.0 | 5.4.0 |
| 24/06/03 | RAN_20 | RP-030276 | 111 | - | Corrections to field coding of CQI for HS-SICH (3.84Mcps TDD) | 5.4.0 | 5.5.0 |
| 24/06/03 | RAN_20 | RP-030276 | 112 | - | Correction to definition of number of bits available to HS-DSCH in one TTI. | 5.4.0 | 5.5.0 |
| 06/01/04 | RAN_22 | RP-030646 | 118 | 1 | Correction of subframe segmentation, physical channel mapping & rate matching for 1.28Mcps TDD | 5.5.0 | 5.6.0 |
| 06/01/04 | RAN_22 | RP-030650 | 119 | - | Clarification of the CRC attachment procedure for HS-SCCH (TDD) | 5.5.0 | 5.6.0 |
| 06/01/04 | RAN_22 | RP-030660 | 120 |  | HARQ process identifier mapping | 5.5.0 | 5.6.0 |
| 06/01/04 | RAN_22 | RP-030712 | 121 |  | Alignment of "Soft channel bits" terminology with 25.306 | 5.5.0 | 5.6.0 |
| 13/01/04 | RAN_22 | - | - | - | Created for M.1457 update | 5.6.0 | 6.0.0 |
| 07/09/04 | RAN_25 | RP-040315 | 124 | 1 | Correction of symbol Xi defined in sub-frame segmentation step | 6.0.0 | 6.1.0 |
| 13/12/04 | RAN_26 | RP-040451 | 125 | - | Introduction of MICH | 6.1.0 | 6.2.0 |
| 20/03/06 | RAN_31 | RP-060078 | 0126 | - | Introduction of the Physical Layer Common Control Channel (PLCCH) | 6.2.0 | 7.0.0 |
| 20/03/06 | RAN_31 | RP-060079 | 0127 | - | Introduction of 7.68Mcps TDD option | 6.2.0 | 7.0.0 |
| 29/09/06 | RAN_33 | RP-060492 | 0129 | - | Introduction of E-DCH for 3.84Mcps and 7.68Mcps TDD | 7.0.0 | 7.1.0 |
| 29/09/06 | RAN_33 | RP-060491 | 0130 | - | Correction to coding of PLCCH for 1.28Mcps TDD | 7.0.0 | 7.1.0 |
| 13/03/07 | RAN_35 | RP-070118 | 0131 | 2 | Introduction of E-DCH for 1.28Mcps TDD | 7.1.0 | 7.2.0 |
| 30/05/07 | RAN_36 | RP-070385 | 0132 | 1 | Support for MBSFN operation | 7.2.0 | 7.3.0 |
| 30/05/07 | RAN_36 | RP-070383 | 0133 | 1 | Modification on HARQ process ID signalling for 1.28Mcps TDD | 7.2.0 | 7.3.0 |
| 30/05/07 | RAN_36 | RP-070385 | 0135 | - | Support for 1.28Mcps TDD MBSFN operation | 7.2.0 | 7.3.0 |
| 11/09/07 | RAN_37 | RP-070650 | 0136 | - | Introduction of multi-frequency operation for 1.28Mcps TDD | 7.3.0 | 7.4.0 |
| 11/09/07 | RAN_37 | RP-070647 | 0137 | - | TFCI mapping for S-CCPCH and 16QAM for 1.28Mcps TDD MBSFN | 7.3.0 | 7.4.0 |
| 11/09/07 | RAN_37 | RP-070640 | 0138 | 1 | Improved E-HICH signature sequence allocation for LCR TDD | 7.3.0 | 7.4.0 |
| 27/11/07 | RAN_38 | RP-070943 | 0141 | 1 | More improvement on dedicated carrier for 1.28Mcps TDD MBMS | 7.4.0 | 7.5.0 |
| 04/03/08 | RAN_39 | RP-080140 | 0144 | 1 | Some corrections for 1.28Mcps TDD EUL | 7.5.0 | 7.6.0 |
| 04/03/08 | RAN_39 | RP-080141 | 0145 | 1 | Clarification on TFCI mapping for 1.28Mcps TDD MBMS | 7.5.0 | 7.6.0 |
| 04/03/08 | RAN_39 | - | - | - | Creation of Release 8 further to RAN_39 decision | 7.6.0 | 8.0.0 |
| 28/05/08 | RAN_40 | RP-080356 | 0150 | 1 | Introduction 64QAM for 1.28Mcps TDD HSDPA | 8.0.0 | 8.1.0 |
| 09/09/08 | RAN_41 | RP-080661 | 151 | 1 | Clarification on E-HICH coding for 1.28Mcps TDD | 8.1.0 | 8.2.0 |
| 09/09/08 | RAN_41 | RP-080661 | 153 | 1 | Clarification of E-UCCH Number indicator on E-AGCH for 1.28Mcps TDD | 8.1.0 | 8.2.0 |
| 09/09/08 | RAN_41 | RP-080661 | 154 | - | Clarification of TRRI on E-AGCH for 1.28Mcps TDD EUL | 8.1.0 | 8.2.0 |
| 09/09/08 | RAN_41 | RP-080661 | 155 | - | Correction of E-HICH coding for 1.28 Mcps TDD EUL | 8.1.0 | 8.2.0 |
| 03/12/08 | RAN_42 | RP-080976 | 159 | - | Clarification of E-DCH RV index selection for 1.28Mcps TDD | 8.2.0 | 8.30 |
| 03/12/08 | RAN_42 | RP-080980 | 161 | 1 | Correction of description about 64QAM for LCR TDD | 8.2.0 | 8.30 |
| 03/12/08 | RAN_42 | RP-080987 | 162 | - | Introduction of the Enhanced CELL_FACH, CELL_PCH, URA_PCH state for 1.28Mcps TDD | 8.2.0 | 8.30 |
| 03/12/08 | RAN_42 | RP-081118 | 163 | 1 | Support for 3.84 Mcps MBSFN IMB operation | 8.2.0 | 8.30 |
| 03/03/09 | RAN_43 | RP-090230 | 165 | - | Correction of the absolute grant (power) value mapping for 1.28Mcps TDD EUL | 8.3.0 | 8.4.0 |
| 03/03/09 | RAN_43 | RP-090240 | 166 | 1 | Introducing of CPC for 1.28Mcps TDD | 8.3.0 | 8.4.0 |
| 03/03/09 | RAN_43 | RP-090241 | 167 | 1 | Introducing of MIMO for 1.28Mcps TDD | 8.3.0 | 8.4.0 |
| 03/03/09 | RAN_43 | RP-090239 | 168 | - | Corrections to 25.222 for 3.84Mcps MBSFN IMB | 8.3.0 | 8.4.0 |
| 03/03/09 | RAN_43 | RP-090239 | 169 | - | TFCI for Secondary CCPCH frame type 2 with 16QAM | 8.3.0 | 8.4.0 |
| 03/03/09 | RAN_43 | RP-090231 | 173 | - | Editorial correction for 1.28Mcps TDD | 8.3.0 | 8.4.0 |
| 26/05/09 | RAN_44 | RP-090533 | 174 | - | Modification of CCS information mapping tables for LCR TDD | 8.4.0 | 8.5.0 |
| 26/05/09 | RAN_44 | RP-090526 | 175 | 1 | Corrections to the coding of E-RUCCH for 1.28 Mcps TDD | 8.4.0 | 8.5.0 |
| 26/05/09 | RAN_44 | RP-090530 | 177 | - | Clarification of HS-DSCH coding for MIMO dual stream transmission for 1.28Mcps TDD | 8.4.0 | 8.5.0 |
| 15/09/09 | RAN_45 | RP-090893 | 178 | 2 | Correction to the channel mapping and rate matching for HS-DSCH in CPC for 1.28Mcs TDD | 8.5.0 | 8.6.0 |
| 15/09/09 | RAN_45 | RP-090893 | 180 | - | Modification of HS-SCCH types for CPC in 1.28Mcps TDD | 8.5.0 | 8.6.0 |
| 15/09/09 | RAN_45 | RP-090893 | 182 | 1 | Correction to Xptr field definition for LCR TDD | 8.5.0 | 8.6.0 |
| 15/09/09 | RAN_45 | RP-090893 | 183 | 1 | Correction to HS-SCCH Type 2 definition for LCR TDD | 8.5.0 | 8.6.0 |
| 15/09/09 | RAN_45 | RP-090893 | 184 | - | Clarification of HS-SCCH Order types against HS-SCCH types | 8.5.0 | 8.6.0 |
| 01/12/09 | RAN_46 | RP-091166 | 185 | 1 | Correction to the Resource repetition period of SPS operation for 1.28Mcps TDD | 8.6.0 | 8.7.0 |
| 01/12/09 | RAN_46 | RP-091176 | 192 | 1 | Modification to HS-SCCH for TS0 for 1.28Mcps TDD | 8.7.0 | 9.0.0 |
| 16/03/10 | RAN_47 | RP-100202 | 195 | - | Correction to the resource repetition period index for 1.28Mcps TDD | 9.0.0 | 9.1.0 |
| 16/03/10 | RAN_47 | RP-100202 | 201 | - | Corrections to the Physical channel mapping for HS-DSCH for 1.28Mcps TDD | 9.0.0 | 9.1.0 |
| 01/06/10 | RAN_48 | RP-100585 | 205 | - | Corrections to the Physical channel mapping of 64QAM for HS-DSCH for 1.28Mcps TDD | 9.1.0 | 9.2.0 |
| 07/12/10 | RAN_50 | RP-101319 | 208 | - | Introduction of MU-MIMO for 1.28Mcps TDD | 9.2.0 | 10.0.0 |
| 15/03/11 | RAN_51 | RP-110253 | 209 | 1 | Correction to TS 25.222 for MC HSUPA for 1.28Mcps TDD | 10.0.0 | 10.1.0 |
| 05/12/11 | RAN_54 | RP-111665 | 211 | 1 | Physical channel mapping for HS-DSCH for secondary carrier TS0 for LCR TDD | 10.1.0 | 10.2.0 |
| 2012-09 | SP_57 | - | - | - | Update to Rel-11 version (MCC) | 10.2.0 | 11.0.0 |
| 2013-12 | RP_62 | RP-131895 | 213 | 1 | Introduction of HSPA signalling enhancements for more efficient resource usage for LCR TDD | 11.0.0 | 12.0.0 |
| 2015-12 | SP_70 | - | - | - | Update to Rel-13 version (MCC) | 12.0.0 | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-03 | RP-75 | - | - | - | - | Promotion to Release 14 without technical change (MCC) | 14.0.0 |
| 2018-06 | RP-80 | - | - | - | - | Promotion to Release 15 without technical change (MCC) | 15.0.0 |
| 2020-07 | RP-88e | - | - | - | - | Upgrade to Rel-16 version without technical change | 16.0.0 |