## 5.2	LP-based Coding

In general terms, speech dominated content is encoded using Analysis-by-Synthesis Linear Prediction (LP) paradigm. At some low bitrates configurations, the LP-based coding is used also for generic audio. On the other hand, LP prediction is not used above 64 kb/s. The LP-based coding consists in encoding the LP excitation signal and the speech spectral envelope, represented by the LP filter coefficients. Depending on the particular characteristics of a speech frames, different flavours of the excitation coding are used to encode voiced or unvoiced speech frames, audio frames, inactive frames etc.

The internal sampling rate of the LP-based coding is rather independent of the input signal sampling rate. Instead, it depends on the encoded bitrate to optimize coding efficiency. In the EVS, there are two different internal sampling rates used – 12.8 kHz is used up to 13.2 kb/s inclusively, and 16 kHz sampling rate is used for higher bitrates. It means that up to 13.2 kb/s, the LP-based encoding covers first 6.4 kHz of the input signal while from 16.4 kb/s and up the LP-based encoding covers 8 kHz of the input. For NB signals, the sampling rate is always 12.8 kHz.

For other than NB signals, the upper bandwidth (not covered with the LP-based coding) is then encoded using bandwidth extension (BWE) technologies, ranging from blind BWE at the lowest bitrates, parametric BWEs optimized to different content at higher bitrates, up to full encoding of the upper bandwidth spectrum at the highest bitrate (64 kb/s).

The basic block for the LP excitation coding is a subframe. The size of the subframe in samples is independent of the internal sampling rate. It equals to 64 samples. It means that at 12.8 kHz internal sampling rate, EVS uses 4 subframes of 5 ms while at 16 kHz internal sampling rate, EVS uses 5 subframes of 4 ms.

### 5.2.1	Perceptual weighting

The encoding parameters, such as adaptive codebook delay and gain, algebraic codebook index and gain are searched by minimizing the error between the input signal and the synthesized signal in a perceptually weighted domain. Perceptual weighting is performed by filtering the signal through a perceptual weighting filter, derived from the LP filter coefficients. The perceptually is similar to the weighting also used in open-loop pitch analysis. However, an adaptive perceptual weighting is used in case of LP-based excitation coding.

The traditional perceptual weighting filter Wz=Az/γ1/Az/γ2 has inherent limitations in modelling the formant structure and the required spectral tilt concurrently. The spectral tilt is more pronounced in wideband signals due to the wide dynamic range between low and high frequencies. A solution to this problem is to introduce a pre-emphasis filter at the input and enhance the high frequency content in case of wideband signals. The LP filter coefficients are then found by means of LP analysis on the pre-emphasized signal. Subsequently, they are used to form a perceptual weighting filter. Its transfer function is the same as the LP filter transfer function but with the denominator having fixed coefficients (similar to the pre-emphasis filter). In this way, the weighting in formant regions is decoupled from the spectral tilt as shown below. Finally, the pre emphasized signal is filtered through the perceptual filter to obtain a perceptually weighted signal, which is used further.

The perceptual weighting filter has the following form

Wz=Az/γ1Hde−emphz=Az/γ1/1−β1z−1	(470)

where

Hde−emph (z)=1(1−β1z−1)	(471)

and β1is equal to 0.68.

Because Az is computed based on the pre-emphasized signal spren, the tilt of the filter 1/Az/γ1is less pronounced compared to the case when Az is computed based on the original signal (as the pre-emphasized signal itself exhibits less spectral tilt than the original wideband signal). Since de-emphasis is performed in the decoder, it can be shown that the quantization error spectrum is shaped by a filter having a transfer function 1/WzHde−emphz=1/Az/γ1. Thus, the spectrum of the quantization error is shaped by a filter whose transfer function is 1/Az/γ1, with Az computed based on the pre-emphasized signal. The perceptual weighting is performed on a frame basis while the LP filter coefficients are calculated on a subframe basis using the principle of LSP interpolation, described in subclause 5.1.9.6. For a subframe of size L = 64, the weighted speech is given by

sh(n)=spren+i=116ai γ1i spren−i+β1 shn−1, n=0,…, L−1	(472)

### 5.2.2	LP filter coding and interpolation

#### 5.2.2.1	LSF quantization

##### 5.2.2.1.1	LSF weighting function

For frame-end LSF quantization, the weighting given by equation (473)  is defined by combining the magnitude weighting, frequency weighting, IHM and squared IHM.

As shown in figure 1, since the spectral analysis and LP analysis use similar temporal sections, the FFT spectrum of the second analysis window can be reused to find the best weighting function for the frame-end LSF quantizer.

Figure 19: LSF weighting computation with FFT spectrum

Figure 2 is a block diagram of a spectral analysis module that determines a weighting function. The spectral analysis computation is performed by a pre-processing module and the output is a linear scale spectrum magnitude which is obtained by FFT.

Figure 20: Block diagram of LSF weighting computation

In the Normalization block, the LSFs are normalized to a range of 0 to K-1. The LSFs generally span the range of 0 to π. For a 12.8 kHz internal sampling frequency, K is 128 and for a 16 kHz internal sampling frequency, K is 160.

The Find magnitude weighting for each normalized LSF block determines the magnitude weighting function W1(n) using the spectrum analysis information and the normalized LSF.

The magnitude weighting function is determined using the magnitude of the spectral bins corresponding to the frequency of the normalized LSFs and the additional two magnitudes of the neighbouring spectral bins (+1 and -1 of the spectral bin corresponding to the frequency of the normalized LSFs) around the spectral bin.

The spectral magnitude is obtained by a 128-point FFT and its bandwidth corresponds to the range of 0 to 6400 Hz. If the internal sampling frequency is16 kHz, the number of spectral magnitudes is extended to 160. Because the spectrum magnitude for the range of 6400 to 8000 Hz is missing, the spectrum magnitude for this range will be generated by the input spectrum. More specifically, the average value of the last 32 spectrum magnitudes which correspond to the bandwidth of 4800 to 6400 Hz are repeated to fill in the missing spectrum.

The final magnitude function determines the weighting function of each magnitude associated with a spectral envelope by extracting the maximum magnitude among the three spectral bins.

W1(n)=wf(n)−Min+2,for n=0,…,M-1	(473)

where Min is the minimum value of wf(n) and

wf(n)=10logEmax(n), for n=0,…,M-1	(474)

where M=16 and the Emax(n) is the maximum magnitude among the three spectral bins for each LSF.

In the Find frequency weighting for each normalized LSF block, the frequency weighting function W2(n) is determined by using frequency information from the normalized LSF.

The function determines the weighting function of each frequency using the predetermined weighting graph which is selected by using the input bandwidth and coding mode. There are two predetermined weighting graphs, as shown in figure 3, which are determined by perceptual characteristics such as Bark scale and a formant distribution of the input signal.

The function corresponding to graph (a) in figure 4 is as follows.

W2(n)=0.5+sinπ⋅fn(n)122, for fn(n)=[0,5], 1fn(n)−6121+1, for fn(n)=[6,127] for WB and [6,159] for WB16kHz.	(475)

The function corresponding to graph (b) in figure 21 is as follows.

W2(n)=0.5+sinπ⋅fn(n)122, for fn(n)=[0,5], 1.0, for fn(n)=[6,20], 13fn(n)−20107+1, for fn(n)=[21,127] for WB and [21,159] for WB16kHz.	(474)

Figure 21: Frequency weighting functions

Next, the FFT weighting function Wf(n) is determined by combining the magnitude weighting function and the frequency weighting function. Computing the FFT weighting function Wf(n) for frame-end LSF quantization is performed as follows:

Wf(n)=W1(n)⋅W2(n),  n=0,…,M-1	(477)

The FFT weighting function uses different types of frequency and magnitude weighting functions depending on frequency bandwidth (NB, WB or WB16 kHz) and coding modes (UC or others such as VC, GC, AC, IC and TC).

Along with the FFT weightings Wf, another weighting function called the inverse harmonic mean (IHM) is computed and defined as:

WIHM(n)=1lsfn−lsfn−1+1lsfn+1−lsfn,  n=0,…,M-1	(478)

The LSFs lsfnare normalized between 0 and π, where the first and the last weighting coefficients are calculated with this pseudo LSFs lsf0=0and lsfM=π. M is the order 16 of the LP model.

IHM approximates the spectral sensitivity of LSFs by measuring how close adjacent LSFs come. If two LSF parameters are close together the signal spectrum has a peak near that frequency. Hence a LSF that is close to one of its neighbours has a high scalar sensitivity and should be given a high weight. The sensitivity of close neighbours LSF is even enhanced by computing the squared of IHM:

WIHM2(n)=WIHM(n)⋅WIHM(n),  n=0,…,M-1	(479)

The three set of weightings, Wf, WIHM, and WIHM2 are gathered  into an M by 4 matrix as follows:

E=1WIHM(0)WIHM2(0)Wf(0)1WIHM(1)WIHM2(1)Wf(1)⋮⋮⋮⋮1WIHM(M−1)WIHM2(M−1)Wf(M−1)	(480)

The set of weightings are combined linearly by multiplying the matrix E by a constant column vector P of dimension M:

W=E⋅P	(481)

The vector P is different for NB, WB/SWB at internal sampling rate 12.8 kHz and WB/SWB at internal sampling rate 16 kHz. The vectors P are derived off-line over a training data by minimizing the distance of the linear combination W and the weightings derived mathematically based on Gardner and Rao method, weightings near-optimal  but too complex for being computed on-line compared to an heuristic approach.

##### 5.2.2.1.2	Bit allocation

The frame-end LSF quantization codebooks and bit allocations depend on the selected coding mode. In addition, different codebooks are used for NB, WB and WB 16kHz modes. This means there is a separate, optimized codebook for each coding mode and for each input bandwidth. In NB mode the LSF vectors are in the range of 0-6400Hz although the input signal has content only up to 4kHz. The WB mode corresponds to the mode where the LSF parameters are estimated in the 0-6400Hz range. The WB2 mode corresponds to the mode where the LSF parameters are estimated in the 0-8000Hz range and it is used in general for the higher bitrates.

Table 1 shows the bit allocation for frame-end LSF quantization for each coding mode.

Table 23: Bit allocation for LSF vectors

| ACELP core bitrate (kbps) | Inactive | Unvoiced | Voiced | Generic | Transition | Audio |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6 | 0 | 27 | 16 | 22 | 0 | 0 |  |
| 7.2 | 22 | 37 | 31 | 29 | 31 | 22 |  |
| 8.0 | 22 | 40 | 36 | 33 | 34 | 22 |  |
| 9.6 | 31 | 31 | 31 | 31 | 0 | 0 |  |
| 13.20 | 31 | 0 | 38 | 38 | 40 | 31 |  |
| 16.40 | 31 | 0 | 31 | 31 | 0 | 31 |  |
| 24.40 | 31 | 0 | 31 | 31 | 0 | 31 |  |
| 32.00 | 41 | 0 | 0 | 41 | 41 | 0 |  |
| 64.00 | 41 | 0 | 0 | 41 | 41 | 0 |  |

##### 5.2.2.1.3	Predictor allocation

There are three possible cases.  In safety net only the mean removed LSF vectors are quantized with the multi stage quantizer. In MA predictive quantization the MA prediction error is quantized with the MSVQ. In switched safety net /AR predictive there is a selection between quantizing the mean removed LSF vector and the AR prediction error. Table 24 specifies for each coding type and each bandwidth which quantization scheme is used. The values in the table indicate safety net (0), MA prediction (1), and AR prediction combined with safety net (2). The value  “-1“ indicates that the corresponding mode is not used. The coding modes that employ switched safety net/ AR prediction use one bit to signal which one of the two variants is used.

Table 25: Predictive mode type for LSF quantizer

|  | Inactive | Unvoiced | Voiced | Generic | Transition | Audio |
| --- | --- | --- | --- | --- | --- | --- |
| Narrowband | 1 | 1 | 2 | 2 | 0 | 2 |
| Wideband <9.6kbps | 1 | 1 | 2 | 2 | 0 | 2 |
| Wideband 16kHz | 1 | -1 | 2 | 1 | 0 | 1 |
| Wideband >=9.6kbps | 1 | 1 | 2 | 1 | 0 | 1 |

The predictor values are optimized for all quantizer modes. For a given coding mode and bandwidth, all bitrates use the same predictor values. In general LSF values for voiced speech are considered quite stable over several consecutive frames. Consequently the corresponding AR predictor has the highest coefficient values. Other AR predictor coefficients are slightly lower. For the MA predictor the same value of 1/3 is used everywhere. The value is significantly lower than for AR coefficients since the quantization error starts oscillating over time if the MA coefficient is too large. The value is experimentally chosen to provide reasonable prediction efficiency, stability and good error recovery.

##### 5.2.2.1.4	LSF quantizer structure

A safety net, predictive or switched safety-net predictive multi-stage vector quantizer (MSVQ) is used to quantize the full length frame-end LSF vector for all modes except voiced mode at 16 kHz internal sampling frequency. The last stage of the MSVQ is a multiple scale lattice vector quantizer (MSLVQ) [22]. For each coding mode number of 1 to 4 unstructured VQ stages are used followed by a MSLVQ stage. The number of stages, number of bits per each stage and the codebook names for each coding mode are detailed in table 26. The codebook names are mentioned to illustrate how some of the codebooks are reused between modes.

Table 27: Optimized codebooks and their bit allocation for LSF quantizers

| Coding mode | Bits VQ safety net | Bits in VQ stages – safety net | Codebooks | Bits VQ predictive mode | Bits in VQ stages predictive mode | Codebooks |
| --- | --- | --- | --- | --- | --- | --- |
| Inactive NB | - | - | - | 5 | 5 | IAA_MA1 |
| Unvoiced NB | - | - | - | 8 | 4+4 | UVD_MA1 UVD_MA2 |
| Voiced NB | 8 | 4+4 | SVNB_SN1 SVNB_SN2 | 6 | 3+3 | GESVNB_AR1 GESVNB_AR2 |
| Generic NB | 9 | 5+4 | GETRNB_SN1 GETRNB_SN2 | 6 | 3+3 | GESVNB_AR1 GESVNB_AR2 |
| Transition NB | 9 | 5+4 | GETRNB_SN1 GETRNB_SN2 | - | - | - |
| Audio NB | 4 | 4 | AUNB_SN1 | 0 | 0 | - |
| Inactive WB | - | - | - | 5 | 5 | IAA_MA1 |
| Unvoiced WB | - | - | - | 12 | 4+4+4 | UVD_MA1 UVD_MA2 UVWB_MA3 |
| Voiced WB | 8 | 4+4 | SVWB_SN1 SVWB_SN2 | 6 | 3+3 | GESVWB_AR1 GESVWB_AR2 |
| Generic WB | 9 | 5+4 | GETRWB_SN1 GETRWB_SN2 | 6 | 3+3 | GESVWB_AR1 GESVWB_AR2 |
| Transition WB | 9 | 5+4 | GETRWB_SN1 GETRWB_SN2 | - | - | - |
| Audio WB | 4 | 4 | AUWB_SN1 | 0 | 0 | - |
| Inactive WB2 | - | - | - | 5 | 5 | IAA_MA1 |
| Unvoiced WB2 | - | - | - | - | - | - |
| Voiced WB2 | - | - | BC-TCVQ | - | - | BC-TCVQ |
| Generic WB2 | - | - | - | 5 | 5 | GEWB2_MA1 |
| Transition WB2 | 8 | 4+4 | TRWB2_SN1 TRWB2_SN2 | - | - | - |
| Audio WB2 | - | - | - | 5 | 5 | AUWB2_MA1 |
| CNG | 4 | 4 | CNG_SN1 | - | - | - |
| Generic WB >= 9.6kbps | - | - | - | 5 | 5 | GEWB_MA1 |

The WB2 voiced mode is using BC-TCVQ technology detailed in subclause 5.2.2.1.5.

Overall the optimized VQ codebooks use 14,368 kBytes and the MSLVQ parameters use 9.304 kBytes, including CNG mode.

The remaining LSF quantizer bits are used for the MSLVQ stage. The quantization in all the stages is done such that it minimizes a weighted Euclidean distortion. The calculation of the weights is detailed in subclause 5.2.2.2.1. The search in the multi-stage quantizer is done such that at most 2 candidates are kept per stage. For each candidate obtained in the search in the unstructured optimized VQ, a residual LSF vector is formed by subtracting from the LSF vector the codevectors obtained in each unstructured VQ stage. If there is one optimized VQ stage two residual LSF vectors are obtained, if there are two optimized VQ stages, 4 candidates are obtained and so on.

Each residual LSF vector is split into two 8-dimensional sub vectors. Each sub vector is coded as follows. The lattice codebook obtained through the reunion of three D8+ lattice truncations differently scaled. Each lattice truncation has a different number of leader classes. The leader classes contained in the lattice truncations are given in table 28.

Table 29: Lattice leader class vectors

| Leader class index | Leader class vector | Leader class index | Leader class vector |
| --- | --- | --- | --- |
| 0 | 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 25 | 3.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0 |
| 1 | 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 26 | 3.0, 2.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 2 | 1.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, | 27 | 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5 |
| 3 | 2.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 28 | 2.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5 |
| 4 | 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 29 | 2.5, 2.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 5 | 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0 | 30 | 3.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 6 | 2.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 31 | 2.0, 2.0, 2.0, 1.0, 1.0, 1.0, 1.0, 0.0 |
| 7 | 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 32 | 2.0, 2.0, 2.0, 2.0, 0.0, 0.0, 0.0, 0.0 |
| 8 | 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0 | 33 | 3.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0 |
| 9 | 2.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0 | 34 | 3.0, 2.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0 |
| 10 | 2.0, 2.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 35 | 4.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 11 | 1.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 36 | 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 0.5 |
| 12 | 2.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 37 | 2.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5 |
| 13 | 2.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0 | 38 | 2.5, 2.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 14 | 2.0, 2.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0 | 39 | 3.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 15 | 3.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 40 | 2.0, 2.0, 2.0, 2.0, 1.0, 1.0, 0.0, 0.0 |
| 16 | 1.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5 | 41 | 3.0, 2.0, 1.0, 1.0, 1.0, 1.0, 1.0, 0.0 |
| 17 | 2.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 42 | 3.0, 2.0, 2.0, 1.0, 0.0, 0.0, 0.0, 0.0 |
| 18 | 2.0, 2.0, 1.0, 1.0, 1.0, 1.0, 0.0, 0.0 | 43 | 3.0, 3.0, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 19 | 2.0, 2.0, 2.0, 0.0, 0.0, 0.0, 0.0, 0.0 | 44 | 4.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0, 0.0 |
| 20 | 3.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0 | 45 | 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5 |
| 21 | 1.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5, 0.5 | 46 | 2.5, 1.5, 1.5, 1.5, 1.5, 1.5, 0.5, 0.5 |
| 22 | 2.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 | 47 | 2.5, 2.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5 |
| 23 | 2.0, 2.0, 1.0, 1.0, 1.0, 1.0, 1.0, 1.0 | 48 | 3.5, 1.5, 1.5, 0.5, 0.5, 0.5, 0.5, 0.5 |
| 24 | 2.0, 2.0, 2.0, 1.0, 1.0, 0.0, 0.0, 0.0 | 49 |  |

Given the bitrate available for the lattice codebook, the codebook is thus defined by a set of three integers representing the number of leader vectors for each truncation and three positive real number representing the scale for each lattice truncation. For instance a multiple scale lattice structure is defined by the number of leaders (20, 14, 5, 16, 10, 0) and the scales (1.057, 1.794, 2.896, 1.154, 1.860, 0.0). It means that the first subvector is quantized with a structure having three lattice truncations having 20, 14, and 5 leader classes respectively, which are scaled with the scales 1.057, 1.794, 2.896, respectively. The second subvector has only two truncations having 16 and 10 leader classes respectively. The truncations are ordered such that, for each subvector, their number of leader classes is descendingly ordered.

The difference in number of bits between the total number of bits for LSF end encoding, the prediction bit if needed and the number of bits used for unstructured VQ is used for the MSLVQ stage. The quantization in all the stages is done such that it minimizes a weighted Euclidean distortion. The calculation of the weights is detailed in subclause 5.2.2.1.1.

Suppose x is the current LSF 8-dimensional sub vector and w its corresponding weight vector. The vector x is normalized, i.e. component wise multiplied with the inverse of the off line estimated standard deviation.  The resulting vector x is further sorted in descending order based on the absolute value of its components and the weights vector is arranged following the same order. Let x' be the vector of descendingly sorted absolute values of x and w' the correspondingly sorted weights vector. The weighted distance to the best codevector of each leader class corresponds to:

x'−sjlkw'2=i=18x'2(i)w'(i)−2sji=18w'(i)x'(i)lk(i)+sj2i=18w'(i)lk2(i)	(475)

where lkis the leader vector corresponding to class k and sj is the scale of the truncation j. Each lattice codebook has at most 3 truncations with their corresponding scales. Each truncation has a given number of leader vector classes. The sum of cardinalities of the classes for the truncations forming the codebook for the first LSF subvector and for the second subvector are within the number of bits for the considered operating point given by the overall bitrate and bandwidth. Computing in the transformed input space the second and the third terms from equation (476) directly gives a relative measure of goodness for the best codevector from the leader class k and truncation j which may be considered as a potential codevector for the truncation j and the leader class k.

dkj=−2sji=18w'(i)x'(i)lk(i)+sj2i=18w'(i)lk2(i)	(477)

The part of equation (478) that is independent of the scale is calculated only once for all the leader classes from the first truncation, which is the one having the highest number of leader classes. When adding the last term to the first sum of equation (483) the product x'(8)lk(8)is considered with negative sign if the parity constraint of the leader k is not obeyed by the signs of the vector x. The contribution of the scale values is considered only afterwards in order to obtain the value dkj. The leader class vector k and the truncation j having the smallest dkj correspond to the codevector of the current input vector. The inverse permutation of the sorting operation on the input vector applied on the winning leader vector lk gives the lattice codevector after applying also the corresponding signs. If the parity of leader vector lk is 0 the signs are identical to the signs of the input vector. If the parity is 1 the signs are similar to the signs of the input vector with the constraint that the number of negative components is even. If the parity is -1 the signs are similar with signs of the input vector with the constraint that the number of negative components is odd. The final codevector is obtained after multiplication with the scale sj and with the inverse of the component-wise off-line computed standard deviation. The standard deviations are individually estimated for each coding mode and bandwidth.

The candidate quantized LSF vectors are obtained by adding each lattice quantized residual to the corresponding candidates from the upper stages. The obtained candidates are increasingly sorted. For each sorted candidate the weighted Euclidean distortion with respect to the original LSF vector is calculated. The candidate that minimizes this distortion is selected as codevector to be encoded. The indexes corresponding to the first unstructured optimized VQ codebooks together with the index in the lattice codebook are written in the bitstream. The index for the lattice codebook is obtained as described in subclause 5.2.2.1.4.2.

For the CNG mode, using a total of 29 bits for the LSF quantization, the multiple scale lattice codebook structure is specific to each of the 16 codevectors obtained in the first stage. In addition based on the value of the last component of the 16 dimensional LSF vector only part of the first stage codebook is searched. If the last component of x is larger than 6350 then the search is done only for the first 6 codevectors of the first stage and the LSF vector corresponds to internal sampling frequency of 16kHz, otherwise the search is performed within the last 10 codevectors of the first stage.

5.2.2.1.4.1	Selection between safety net and predictive mode

For the modes where switched safety-net prediction is allowed the selection between the two is done as follows. For frame error concealment reasons safety net is imposed, and variable forcesf set to 1, under the following conditions:

- first three ACELP frames after an HQ frame

- in voiced class signals, if the frame erasure mode LSF estimate of the next frame based on the current frame is at a distance from the current frame LSF vector larger than 0.25. The distance, or stability factor, is calculated as:

sf=1.25−256400000Dframelen	(479)

where frame_len is the frame length of the current frame and D is the Euclidean distance between the current frame LSF vector and the FER estimate for the next frame. In this case forcesfcalculated at the current frame is stored in memory for use at the subsequent frame, thereby forcing the safety net decision for the subsequent frame when forcesfis equal to 1.

- some cases of rate switching

Safety net usage is decided by the following code line:

if ( force_sf || Err[0] < abs_threshold || Err[0]*(*streaklimit) < 1.05 * Err[1])

Thus the safety net mode is selected if force_sf is enabled or if for the quantized safety net codevector the quantization distortion (weighted Euclidean distance) is smaller than abs_threshold of 41000 for NB or 45000 for WB frames. For these relatively low error values the quantization is already transparent to original LSF values and it makes sense from the error recovery point of view to use safety-net as often as possible. Finally the safety net quantized error is compared to the predictively quantized error, with scaling of 1.05 to prefer safety net usage as well using *streaklimit multiplying factor that is adaptive to the number of consecutive predictive frames. The *streaklimit factor gets smaller, when the streak of continuous predictive frames gets longer. This is done in order to restrict the very long usage streaks of predictive frames for frame-erasure concealment reasons. For voiced speech longer predictive streaks are allowed than for other speech types. In voiced mode streak limiting starts after 6 frames, in other modes after 3 frames.

5.2.2.1.4.2	Indexing of the lattice codevector

The indexes of each one of the two multiple scale lattice codevectors is composed of the following entities:

- scale indexes j1,j2 for the two 8-dimensional subvectors

- leader class index, k1,k2for the two 8-dimensional subvectors

- leader permutation index, Il1,Il2 unsigned permutation index

- sign index with parity constraint, Is1,Is2

- scale offset Os(ji),i=1,2 the number of codevectors corresponding to the truncations with smaller scale indexes

- leader offset Ol(ki),i=1,2 the number of codevectors corresponding to leader classes with smaller leader indexes

- π0(ki), i=1,2 cardinality of unsigned leader class, i.e. number of unsigned permutations in the class, shown in table 30.

- N2 is the number of codevectors for the second subvector

The index for each subvector is calculated using

Ii=Os(ji)+Ol(ki)+(Isiπ0(ki)+Ili) for i=1,2	(480)

The indexes Ili and Isi are obtained  using the position encoding based on counting the binomial coefficients and the sign encoding described in [26].

Table 31: Cardinality of unsigned leader vector permutations

| Leader vector index | π0 | Leader vector index | π0 | Leader vector index | π0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 28 | 17 | 56 | 34 | 1120 |
| 1 | 1 | 18 | 420 | 35 | 8 |
| 2 | 70 | 19 | 56 | 36 | 8 |
| 3 | 8 | 20 | 280 | 37 | 280 |
| 4 | 8 | 21 | 56 | 38 | 168 |
| 5 | 28 | 22 | 168 | 39 | 56 |
| 6 | 168 | 23 | 28 | 40 | 420 |
| 7 | 28 | 24 | 560 | 41 | 336 |
| 8 | 1 | 25 | 168 | 42 | 840 |
| 9 | 280 | 26 | 336 | 43 | 28 |
| 10 | 28 | 27 | 28 | 44 | 168 |
| 11 | 56 | 28 | 280 | 45 | 1 |
| 12 | 8 | 29 | 28 | 46 | 168 |
| 13 | 56 | 30 | 8 | 47 | 420 |
| 14 | 420 | 31 | 280 | 48 | 168 |
| 15 | 56 | 32 | 70 |  |  |
| 16 | 70 | 33 | 8 |  |  |

The binomial encoding used for calculating Il1 and Il2 uses the fact that the cardinality of an unsigned leader class with distinct values v0,…,vn-1, each having the number of occurrences k0,…,kn-1 is given by:

Sk0...kn−1=Sk0S−k0k1...S−kikn−1.	(481)

The distinct values for each leader class vector and the number of each value in each leader class vector are given in the following table:

Table 32: Leader vector distinct values, their number of occurrences, and leader vector parities

| Leader class index | Distinct values | Number of occurrences | Parity | Leader class index | Distinct values | Number of occurrences | Parity |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1, 0, | 2,6 | 0 | 25 | 3.0, 1.0, 0.0 | 1,5,2 | 0 |
| 1 | 0.5 | 8 | 1 | 26 | 3.0, 2.0, 1.0, 0.0 | 1,1,1,5 | 0 |
| 2 | 1, 0 | 4,4 | 0 | 27 | 1.5, 0.5 | 6,2 | 1 |
| 3 | 2, 0 | 1,7 | 0 | 28 | 2.5, 1.5, 0.5, | 1,3,4 | -1 |
| 4 | 1.5, 0.5 | 1,7 | -1 | 29 | 2.5, 0.5 | 2,6 | 1 |
| 5 | 1.0, 0.0 | 6,2 | 0 | 30 | 3.5, 0.5, | 1,7 | -1 |
| 6 | 2.0, 1.0, 0.0, | 1,2,5 | 0 | 31 | 2.0, 1.0, 0.0 | 3,4,1 | 0 |
| 7 | 1.5, 0.5 | 2,6 | 1 | 32 | 2.0, 0.0 | 4,4 | 0 |
| 8 | 1.0 | 8 | 0 | 33 | 3.0, 1.0 | 1,7 | 0 |
| 9 | 2.0, 1.0, 0.0 | 1,4,3 | 0 | 34 | 3.0, 2.0, 1.0, 0.0 | 1,1,3,3 | 0 |
| 10 | 2.0, 0.0 | 2,6 | 0 | 35 | 4.0, 0.0, | 1,7 | 0 |
| 11 | 1.5, 0.5 | 3,5 | -1 | 36 | 1.5, 0.5 | 7,1 | -1 |
| 12 | 2.5, 0.5 | 1,7 | 1 | 37 | 2.5, 1.5, 0.5 | 1,4,3 | 1 |
| 13 | 2.0, 1.0, 0.0 | 1,6,1 | 0 | 38 | 2.5, 1.5, 0.5 | 2,1,5 | -1 |
| 14 | 2.0, 1.0, 0.0 | 2,2,4 | 0 | 39 | 3.5, 1.5, 0.5 | 1,1,6 | 1 |
| 15 | 3.0, 1.0, 0.0 | 1,1,6 | 0 | 40 | 2.0, 1.0, 0.0 | 4,2,2 | 0 |
| 16 | 1.5, 0.5 | 4,4 | 1 | 41 | 3.0, 2.0, 1.0, 0.0 | 1,1,5,1 | 0 |
| 17 | 2.5, 1.5, 0.5 | 1,1,6 | -1 | 42 | 3.0, 2.0, 1.0, 0.0 | 1,2,1,4 | 0 |
| 18 | 2.0, 1.0, 0 | 2,4,2 | 0 | 43 | 3.0, 0.0 | 2,6 | 0 |
| 19 | 2.0, 0.0 | 3,5 | 0 | 44 | 4.0, 1.0, 0.0 | 1,2,5 | 0 |
| 20 | 3.0, 1.0, 0.0, | 1,3,4 | 0 | 45 | 1.5 | 8 | 1 |
| 21 | 1.5, 0.5 | 5,3 | -1 | 46 | 2.5, 1.5, 0.5 | 1,5,2 | -1 |
| 22 | 2.5, 1.5, 0.5 | 1,2,5 | 1 | 47 | 2.5, 1.5, 0.5 | 2,2,4 | 1 |
| 23 | 2.0, 1.0 | 2,6 | 0 | 48 | 3.5, 1.5, 0.5 | 1,2,5 | -1 |
| 24 | 2.0, 1.0, 0.0 | 3,2,3 | 0 | 49 |  |  |  |

The index for the two multiple scale lattice codevectors corresponding to the two residual LSF subvectors are combined in a single index, I, which is written in the bitstream.

I=N2I1+I2.	(482)

##### 5.2.2.1.5	LSFQ for voiced coding mode at 16 kHz internal sampling frequency : BC-TCVQ

5.2.2.1.5.1	Block-constrained trellis coded vector quantization (BC-TCVQ)

The VC mode operating at 16 kHz internal sampling frequency has two decoding rates: 31 bits per frame and 40 bits per frame.  The VC mode is quantized by a 16-state and 8 stage block-constrained trellis coded vector quantization (BC-TCVQ) scheme.

Trellis coded vector quantization (TCVQ) [42] generalizes trellis coded quantization (TCQ) to allow vector codebooks and branch labels. The main feature of TCVQ is the partitioning of an expanded set of VQ symbols into subsets and the labelling of the trellis branches with these subsets. TCVQ is based on a rate-1/2 convolutional code, which has N=2v trellis states and two branches entering/leaving each trellis state. Given a block of m source vectors, the Viterbi algorithm (VA) is used to find the minimum distortion path. This encoding procedure allows the best trellis path to begin in any of N initial states and end in any of N terminal states.  In TCVQ, the codebook has 2(R+R)L vector codewords. R is referred to as “codebook expansion factor” (in bits per dimension) since the codebook has 2RLtimes as many codewords as a nominal rate-R VQ. The encoding is accomplished in the following two steps.

Step 1. For each input vector, find the closest codeword and corresponding distortion in each subset.

Step 2. Let the branch metric for a branch labelled with subset S be the distortion found in step 1 and use the VA to find the minimum distortion path through the trellis.

BC-TCVQ is a low-complexity approach that requires exactly one bit per source sample to specify the trellis path. Figure 5 shows the concept of ‘block constrained’ and illustrates the search process of the Viterbi algorithm with a 4-state and 8 stages trellis structure, which selects ‘00’ and ‘10’ as initial states. When the initial state is ‘00’, the terminal state is selected to be one of ‘00’ or ‘01’ and when the initial state is ‘10’, the terminal state is selected to be  one of ‘10’ or ‘11’. As an example, the survival path from the initial stage with state ‘00’ to the stages (NS−log24) with state ‘00’ is shown by a dotted line. In this case, the only two possible trellis paths for the last two stages are toward states ‘00’ and ‘01’. This example uses one bit for the initial state and one bit for the terminal state.  If the terminal state is decided, the path information for the last two stages is not needed.

Figure 22: Block constrained concept in 4-state and 8 stages trellis structure for BC-TCVQ encoding

For any 0≤k≤υ, consider a BC-TCVQ structure that allows 2k initial trellis states and exactly 2v−k terminal trellis states for each allowed initial trellis state. A single VA encoding, starting from the allowed initial trellis states, proceeds in the normal way up to the vector stage m−k. It takes k bits to specify the initial state, and m−k bits to specify the path to vector stage m−k. A unique terminating path, possibly dependent on the initial trellis state, is pre-specified for each trellis state at vector stage m−k through vector stage m. Regardless of the value of k, the encoding complexity is only a single VA search of the trellis, and exactly m bits are required to specify an initial trellis state and a path through the trellis.

The BC-TCVQ for VC mode at a 16kHz internal sampling frequency utilizes 16-state (N=16) and 8-stage (NS=8) TCVQ with 2-dimensional (L=2) vector. LSF subvectors with two elements are allocated to each stage. Table 33 shows the initial states and terminal states for 16-state BC-TCVQ. In this case the parameters k and v are 2 and 4, respectively. Four bits are used for both the initial state and terminal state.

Table 34: Initial state and terminal state for 16-state BC-TCVQ

| Initial state | Terminal state |
| --- | --- |
| 0 | 0, 1, 2, 3 |
| 4 | 4, 5, 6, 7 |
| 8 | 8, 9, 10, 11 |
| 12 | 12, 13, 14, 15 |

5.2.2.1.5.2	Bit Allocations and codebook size for BC-TCVQ

The bit allocations for the LSF quantizer at 31 and 40 bits/frame are summarized in tables 35 and 36.

Table 37: Bit allocation for the LSF quantizer at 31 bits/frame

| Parameters |  | Bit allocation |
| --- | --- | --- |
| BC-TCVQ | Path information (Initial states + path + final states) | 2+4+2 |
|  | Subset codewords | 4 bits 2 (Stages 1 to 2) 3 bits 2 (Stages 3 to 4) 2 bits 4 (Stages 5 to 8) |
| Scheme selection |  | 1 |
| Total |  | 31 |

Table 38: Bit allocation for the LSF quantizer at 40 bits/frame

| Parameters |  | Bit allocations |
| --- | --- | --- |
| BC-TCVQ | Path information (Initial states + path + final states) | 2+4+2 |
|  | Subset codewords | 4 bits 2 (Stages 1 to 2) 3 bits 2 (Stages 3 to 4) 2 bits 4 (Stages 5 to 8) |
| SVQ | Subset codewords | 5 (1st vector with dim.=8) 4 (2nd vector with dim.=8) |
| Scheme selection |  | 1 |
| Total |  | 40 |

Figures 6 and 7 show the LSF quantizer at 31 and 40 bits/frame, respectively. The 1st and 2nd BC-TCVQ use the same bit allocation but different codebook entries. The 3rd and 4th SVQ use the same bit allocation and codebooks. The 31 bit LSF quantizer uses BC-TCVQ and the 40 bit LSF quantizer uses both BC-TCVQ and SVQ.

The following table summarizes the codebook size for BC-TCVQ and SVQ. The overall codebook size is 2,432 words. In addition, there are several tables for BC-TCVQ such as intra-prediction coefficients (56 words), scale information (32 words) and branch information (192 words). The total codebook size is 2,712 words.

Table 39: Codebook size for BC-TCVQ and SVQ

|  | 1st stage | 2nd stage | 3rd stage | 4th stage | 5th stage | 6th stage | 7th stage | 8th stage | Total per frame |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits for BC-TCVQ subcodebook | 4 | 4 | 3 | 3 | 2 | 2 | 2 | 2 |  |
| Scalars for Predictive | 256 | 256 | 128 | 128 | 64 | 64 | 64 | 64 | 1,024 |
| Scalars for Safety-net | 256 | 256 | 128 | 128 | 64 | 64 | 64 | 64 | 1,024 |
| Bits for SVQ subcodebook | 5 |  |  |  | 4 |  |  |  |  |
| Scalars | 256 |  |  |  | 128 |  |  |  | 384 |
| Total |  |  |  |  |  |  |  |  | 2,432 |

5.2.2.1.5.3	Quantization scheme selection

The quantization scheme for the VC mode consists of Safety-net and Predictive schemes. The quantization scheme is selected in an open-loop manner as shown in the figures 8 and  9. The scheme selection is done by calculating the prediction error of unquantized LSFs.

The prediction error (Ek) of the kth frame is obtained from the inter-frame prediction contributionpki, the weighting functionwendi, and a mean-removed unquantized LSF zki as

Ek=i=0M−1wend(i)zk(i)−pk(i)2	(483)

where

pk(i)=ρ(i)zk−1(i), for i=0,…,M	(484)

and ρiis the selected AR prediction coefficients for VC mode and zk−1i is the mean-removed quantized LSF of the previous frame and M is the LPC order.

When Ek is bigger than a threshold, it implies the tendency of the current frame to be non-stationary. Then the safety-net scheme is a better choice. Otherwise the predictive scheme is selected. In addition, the streak limit (streaklimit) prevents the consecutive selection of the predictive scheme.

The quantization scheme selection is shown by the following pseudo-code.

If Ek > streaklimit * op_loop_thr

safety_net = 1;

else

safety_net = 0;

where Ek is the prediction error of the kth frame and the open-loop threshold (op_loop_thr) is 3,784,536.3.

If the safety-net flag (safety_net) is set to 1, the safety-net scheme is selected, and if the safety-net flag (safety_net) is set to 0, the predictive scheme is selected. The scheme selection is encoded using a single bit.

5.2.2.1.5.4	31 bit LSF quantization by the predictive BC-TCVQ with safety-net

Figure 10 shows the predictive BC-TCVQ with safety-net for an encoding rate of 31 bits.

Figure 23: Block diagram of the predictive BC-TCVQ with safety-net for an encoding rate of 31bits/frame

The operation of the 31 bit LSF quantizer is described as follows. If the safety-net scheme is selected, the mean-removed LSF vector, zk(i), is quantized by the 1st BC-TCVQ and 1st intra-frame prediction with 30 bits. If the predictive scheme is selected, the prediction error, rk(i), which is the difference between the mean-removed LSF vector zk(i) and the prediction vector pk(i) is quantized by the 2nd BC-TCVQ and 2nd intra-frame prediction with 30 bits.

An optimal index for each stage of BC-TCVQ is obtained by searching for an index which minimizes Ewerr(p) of equation (485).

Ewerr(p)=i=01wend(2(j−1)+i)tk'(2(j−1)+i)−cjp(i)2, for p=1,…, Pj and j=1,…, M/2	(486)

where Pj is the number of codevectors in the jth sub-codebook, cjp is the pth codevector of jth the subcodebook, wend(i) is a weighting function, and tk'=[tkt(0),tkt(1),...,tkt(M2−1)].

Intra-frame correlation typically remains in the inter-frame AR prediction error vectors. The presence of significant intra-frame correlation motivates the introduction of an intra-predictive coding scheme for the AR prediction error vector, as shown in figure 11, in order to increase the coding gain. The intra-frame prediction uses the quantized elements of the previous stage. The difference between zk(i) and its prediction is then quantized. The prediction is formed for each trellis node using the output codevectors specified by the survivor path associated with the particular node.

The prediction coefficients used for the intra-frame prediction is predefined by the codebook training process. The prediction coefficients are two-by-two matrices for the 2-dimensional vector. The intra-frame prediction process of BC-TCVQ is as follows. The prediction residual vector, tk(i), which is the input of the 1st BC-TCVQ, is computed as

tk(0)=zk(0)

tk(i)=zk(i)−zk(i), for i=1,…, M/2-1	(487)

where

zk(i)=Aizk(i-1), for i=1,…, M/2-1	(488)

where zk(i)is the estimation of zk(i), zk(i−1) is the quantized vector of zk(i−1), and Ai is the prediction matrix with 22 which is computed as

Ai=R01i[R11i]−1, for i=1,…, M/2-1,	(489)

where

and 	(494)

and M is the LPC order.

Then

zk(i)=tk(i)+zk(i), for i=0,…, M/2-1.	(490)

The prediction residual, tk(i), is quantized by the 1st BC-TCVQ. The 1st BC-TCVQ and the 1st intra-frame prediction are repeated to quantize zk(i). Table 40 represents the designed prediction coefficients Ai for the BC-TCVQ in the safety-net scheme.

Table 41: Intra-frame prediction coefficients for the BC-TCVQ in the safety-net scheme

| Coefficient Number | Coefficient Value |
| --- | --- |
| A1 | −0.4523240.808759−0.5242980.305544 |
| A2 | 0.0096630.606028−0.0132080.421115 |
| A3 | 0.1448770.6734950.0809630.580317 |
| A4 | 0.2082250.6331440.2159580.584520 |
| A5 | 0.0508220.7678420.0768790.416693 |
| A6 | 0.0050580.550614−0.0067860.296984 |
| A7 | −0.0238600.611144−0.1627060.576228 |

For the predictive scheme, rk(i) is quantized by the 2nd BC-TCVQ and the 2nd intra-frame prediction. An optimal index for each stage of BC-TCVQ is obtained by searching for an index which minimizes Ewerr(p) in equation (491).

The intra-frame prediction uses the same process with different prediction coefficients as that of the safety-net scheme. Then

rk(i)=tk(i)+rk(i), for i=0,…,M/2-1.	(492)

The prediction residual, tk(i), is quantized by the 2nd BC-TCVQ. The 2nd BC-TCVQ and the 2nd intra-frame prediction are repeated to quantize. Table 42 represents the designed prediction coefficients Ai for the BC-TCVQ in the predictive scheme.

Table 43: Intra-frame prediction coefficients for the BC-TCVQ in the predictive scheme

| Coefficient Number | Coefficient Value |
| --- | --- |
| A1 | −0.2924790.676331−0.4226480.217490 |
| A2 | 0.0489570.50047610.0873010.287286 |
| A3 | 0.1994810.5027840.1067620.420907 |
| A4 | 0.2404590.4405040.2142550.396496 |
| A5 | 0.1931610.4948500.1586900.306771 |
| A6 | 0.0934350.3706620.0655260.148231 |
| A7 | 0.0374170.336906−0.0242460.187298 |

5.2.2.1.5.5	40 bit LSF quantization using the predictive BC-TCVQ/SVQ with safety-net

Figure 12 shows the predictive BC-TCVQ/split-VQ(SVQ) with safety-net for an encoding rate of 40 bits. Both 31 bit LSF quantizer and 40 bit LSF quantizer use the same codebook for BC-TCVQ.

Figure 13: Block diagram of the predictive BC-TCVQ/SVQ with safety-net for an encoding rate of 40 bits/frame

In the LSF quantization for an encoding rate of 40 bit/frame, the difference between the mean-removed LSF and its BC-TCVQ output is quantized by the 3rd and 4th SVQ, as shown in figure 14. The scheme selection, 1st and 2nd BC-TCVQ, and 1st and 2nd intra-frame prediction blocks of the 40 bit LSF quantizer are exactly same as those of the 31 bit LSF quantizer. Both LSF quantizers use same codebooks for the BC-TCVQ.

If the current coding mode in the scheme selection block is selected as the predictive scheme, the prediction error posen_min=argmin(fenvband(j))j=0,...,3 is derived by subtracting p from the mean-removed LSF z. It is quantized by the 2nd BC-TCVQ and the 2nd intra-frame prediction. The residual signal r2  is obtained by subtracting r1from r. The residual signal r2 is then split into two sub-vectors of dimensions 8 and 8, and is quantized using the 4th SVQ. Since the low band is perceptually more important than the high band, five bits are allocated to the 1st 8-dimensional VQ and four bits are allocated to the 2nd 8-dimensional VQ. r2 is quantized by  the 4th SVQ to produce r2. r is then obtained by adding  r2 to r1. Finally the predictive scheme output z is derived by adding p to r(n).

If the current coding mode is selected as the safety-net scheme, the mean-removed LSF z is quantized by the 1st BC-TCVQ and the 1st intra-frame prediction. The residual signal z2  is extracted by subtracting z1from z, and it is quantized by the 3rd SVQ to produce z2. The 3rd SVQ is exactly same as the 4th SVQ. That is, both SVQ quantizers use same codebooks. Because the input distribution of the 3rd SVQ is different from that of the 4th SVQ, scaling factors are used to compensate the difference. Scaling factors are computed by considering the distribution of both residual signals z2 and r2. To minimize the computational complexity in in an actual implementation, the input signal z2 of the 3rd SVQ is divided by the scaling factor, and the resulting signal is quantized by the 3rd SVQ. The quantized signal z2 of the 3rd SVQ is obtained by multiplying the quantized output with the scaling factor. Table 44 shows the scaling factors for the quantization and de-quantization. Finally, the quantized mean-removed LSF z is derived by adding  z2 to z1.

Table 45: Scaling factor for the SVQ

| Dimension | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inverse scale factor for quantization | 0.5462 | 0.5434 | 0.5553 | 0.5742 | 0.5800 | 0.5725 | 0.6209 | 0.6062 |
| Scale factor for de-quantization | 1.8307 | 1.8404 | 1.8009 | 1.7416 | 1.7240 | 1.7467 | 1.6106 | 1.6497 |
| Dimension | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Inverse scale factor for quantization | 0.6369 | 0.6432 | 0.6351 | 0.6173 | 0.6397 | 0.6562 | 0.6331 | 0.6404 |
| Scale factor for de-quantization | 1.5702 | 1.5548 | 1.5745 | 1.6199 | 1.5633 | 1.5239 | 1.5796 | 1.5615 |

##### 5.2.2.1.6	Mid-frame LSF quantizer

For a more accurate representation of the spectral envelope during signal transitions, the encoder quantizes mid-frame LSF coefficients. In contrast to the frame-end LSF vector, the mid-frame LSF vector is not quantized directly. Instead, a weighting factor is searched in a codebook to calculate a weighted average between the quantized LSF vectors of the current and the previous frames. Only 2-6 bits are required depending on the bitrate and the coding mode (see Table 35a).

Table 35a: Bit allocation in mid-frame LSF quantization

| Bitrate [bps] | IC | UC | VC | GC | TC | AC |
| --- | --- | --- | --- | --- | --- | --- |
| 7200 | 2 | 5 | 4 | 5 | 5 | 2 |
| 8000 | 2 | 5 | 4 | 5 | 5 | 2 |
| 9600 | 2 | 5 | 4 | 5 | 0 | 0 |
| 13200 | 2 | 0 | 5 | 5 | 5 | 2 |
| 16400 | 4 | 0 | 5 | 5 | 0 | 0 |
| 24400 | 5 | 0 | 5 | 5 | 0 | 0 |
| 32000 | 5 | 0 | 0 | 5 | 5 | 5 |
| 64000 | 5 | 0 | 0 | 5 | 5 | 5 |

Before searching the codebook, the unquantized mid-frame LSF vector is weighted with the LSF weighting function defined in Equation (481). For simplicity, the following description will be provided by using LSP vectors instead of LSF vectors. These two vectors are related by the following simple relation q(k)=cos(ω(k)) where q(k) is the kth LSP coefficient and ω(k) is kth LSF coefficient. The mid-frame LSP weighting can be expressed using the following formula

qwmid(k)=W(k)qmid(k), for k=0,…,M-1.	(496a)

where qmid(k) is the kth unquantized LSP coefficient and W(k) is kth weighting factor of the function defined in Equation (481). Note, that this is not the weighting factor which is quantized. This weighting is based on the FFT spectrum where more weight is put on perceptually important part of the spectrum and less weight elsewhere.

The weighting factor to be quantized is a vector of size M that is searched in a closed-loop fashion such that the error between the quantized mid-frame LSP coefficients and this weighted representation is minimized in a mean-square sense. That is

Emid=k=0M−1qwmid(k)−(1−fmid(k))qwend[−1](k)+fmid(k)qwend(k)2	(496b)

where qwend(k) is kth quantized weighted end-frame LSP coefficient and fmid is the mid-frame weighting vector taken from the codebook. To save computation complexity, both operations are combined. That is

Emid=k=0M−1wf(k)qmid(k)−(1−fmid(k))qend[−1](k)+fmid(k)qend(k)2	(496c)

Once the winning weighting factor is found, the quantized LSP vector is reordered to maintain a stable LP filter. After the quantization, the end-frame and the mid-frame LSF vectors are used to determine the quantized LP parameters in each subframe. This is done in the same way as for unquantized LP parameters (see Equation (58) in Clause 5.1.96).

### 5.2.3	Excitation coding

The excitation signal coding depends on the coding mode. In general it can be stated that in the absence of DTX/CNG operation, the excitation signal is coded per subframes of 64 samples. This means that it is encoded four times per frame in case of 12.8 kHz internal sampling rate and five times per frame in case of 16 kHz internal sampling rate. The exception is the GSC coding where longer subframes can be used to encode some components of the excitation signal, especially at lower bitrates.

The excitation coding will be described in the following subclauses, separately for each coding mode. The description of excitation coding starts with the GC and VC modes. For the UC, TC, and GSC modes, it will be described in subsequent subclauses with references to this subclause.

#### 5.2.3.1	Excitation coding in the GC, VC and high rate IC/UC modes

The GC,VC and high rate IC/UC modes are very similar and are described together. The VC mode is used in stable voiced segments where the pitch is evolving smoothly within an allowed range as described in subclause 5.1.13.2. Thus, the major difference between the VC and GC modes is that more bits are assigned to the algebraic codebook and less to the adaptive codebook in case of the VC mode as the pitch is not allowed to evolve rapidly in the VC mode. The high-rate IC and UC modes are similar and are used for signalling inactive frames where only a background noise is detected, and unvoiced frames, respectively. The two modes differ from GC mode mainly by their specific gain coding codebook. The GC mode is then used in frames not assigned to a specific coding mode during the signal classification procedure and is aimed at coding generic speech and audio frames. The principle of excitation coding is shown in a schematic diagram in figure 15. The individual blocks and operations are described in detail in the following subclauses.

Figure 25: Schematic diagram of the excitation coding in GC and VC mode

##### 5.2.3.1.1	Computation of the LP residual signal

To keep the processing flow similar for all coding modes, the LP residual signal is computed for the whole frame in the first processed subframe of each frame, as this is needed in the TC mode. For each subframe, the LP residual is given by

rn=spren+i=116aispren−i, n=0,…,63	(497)

where spren is the pre-emphasized input signal, defined in subclause 5.1.4  and aiare the quantized LP filter coefficients, described in subclause 5.2.2.1.

In DTX operation the computed LP residual signal  is attenuated by multiplying an attenuation factor  for all input bandwidths except NB. The attenuation factor is calculated as

(497a)

where  as determined in subclause 5.6.2.1.1 is upper limited by ,  if the bandwidth is not WB or the latest bitrate used for actively encoded frames   is larger than 16.4 kbps. Otherwise  is determined from a hangover attenuation table as defined in Table 35b. is only updated in the first SID frame after an active signal period if two criteria are both fulfilled. The first criterion is satisfied if AMR-WB IO mode is used or the bandwidth=WB. The second criterion is met if the number of consecutive active frames in the latest active signal segment was at least  number of frames or if the current SID is the very first encoded SID frame. The attenuation factor  is finally lower limited to.

Table 35b: Attenuation floor

| Latest active bitrate [kbps] |  |
| --- | --- |
|  | 0.5370318 |
|  | 0.6165950 |
|  | 0.6839116 |
|  | 0.7079458 |
|  | 0.7079458 |

##### 5.2.3.1.2	Target signal computation

The target signal for adaptive codebook search is usually computed by subtracting a zero-input response of the weighted synthesis filter WzHz=Az/γ1Hde−emphz/Azfrom the weighted pre-emphasized input signal. This is performed on a subframe basis. An equivalent procedure for computing the target signal, which is used in this codec, is filtering of the residual signal, rn, through the combination of the synthesis filter Hz=1/Azand the weighting filterWz=Az/γ1Hde−emphz. After determining the excitation signal for a given subframe, the initial states of these filters are updated by filtering the difference between the LP residual signal and the excitation signal. The memory update of these filters is explained in subclause 5.2.3.1.8. The residual signal, rn, which is needed for finding the target vector, is also used in the adaptive codebook search to extend the past excitation buffer. This simplifies the adaptive codebook search procedure for delays less than the subframe size of 64 as will be explained in the next subclause. The target signal in a given subframe is denoted as xn.

##### 5.2.3.1.3	Impulse response computation

The impulse response, hn, of the weighted synthesis filter

WzHz=Az/γ1Hde−emphz/Az	(498)

is computed for each subframe. Note that hnis not the impulse response of the filterHz, but of the filter WzHz. In the equation above,Az, is the quantized LP filter, the coefficients of which are ai (see subclause 5.2.2.1). This impulse response is needed for the search of adaptive and algebraic codebooks. The impulse responsehnis computed by filtering the vector of coefficients of the filterAz/γ1, extended by zeros, through the two filters: 1/AzandHde−emphz.

##### 5.2.3.1.4	Adaptive codebook

5.2.3.1.4.1	Adaptive codebook search

The adaptive codebook search consists of performing a closed-loop pitch search, and then computing the adaptive codevector, vn, by interpolating the past excitation at the selected fractional pitch lag. The adaptive codebook parameters (or pitch parameters) are the closed-loop pitch, TCL, and the pitch gain, gp(adaptive codebook gain), calculated for each subframe. In the search stage, the excitation signal is extended by the LP residual signal to simplify the closed-loop search. The adaptive codebook search is performed on a subframe basis. The bit allocation is different for the different modes.

In the first and third subframes of a GC, UC or IC frame, the fractional pitch lag is searched with a resolution in the range [34, 91½], and with integer sample resolution in the range [92, 231]depending on the bit-rate and coding mode. Closed-loop pitch analysis is performed around the open-loop pitch estimates. Always bounded by the minimum and maximum pitch period limits, the range [d0–8, d0+7] is searched in the first subframe, while the range [d1–8, d1+7] is searched in the third subframe. The pitch period quantization limits are summarized in table 46.

Table 36: Pitch period quantization limits

| Rates (kbps) | Sampling rate of the limits (kHz) | IC/UC | VC | GC |
| --- | --- | --- | --- | --- |
| 7.2 | 12.8 | n.a. | [17; 231] | [34; 231] |
| 8.0 | 12.8 | n.a. | [17; 231] | [20; 231] |
| 9.6 | 12.8 | n.a. | [29; 231] | [29; 231] |
| 13.2 | 12.8 | n.a. | [17; 231] | [20; 231] |
| 16.4 | 16 | n.a. | [36; 289] | [36; 289] |
| 24.4 | 16 | n.a. | [36; 289] | [36; 289] |
| 32 | 16 | [21; 289] | n.a. | [21; 289] |
| 64 | 16 | [21; 289] | n.a. | [21; 289] |

For the second and fourth subframes, a pitch resolution depending on the bit-rate and coding mode is used and the closed-loop pitch analysis is performed around the closed-loop pitch estimates, selected in the preceding (first or third) subframe. If the closed-loop pitch fraction in the preceding subframe is 0, the pitch is searched in the range [TI–8, TI+7½], whereTI=TCLpis the integer part of the fractional pitch lag of the preceding subframe (p is either 0, to denote the first subframe, or 3 to denote the third subframe). If the fraction of the pitch in the previous subframe is 1/2, the pitch is searched in the range [TI–7, TI+8½]. The pitch delay is encoded as follows. In the first and third subframe, absolute values of the closed-loop pitch lags are encoded. In the third and fourth subframe, only relative values with respect to the absolute ones are encoded.

In the VC mode, the closed-loop pitch lag is encoded absolutely in the first subframe and relatively in the following 3 subframes. If the fraction of the closed-loop pitch of the preceding subframe is 0, the pitch is searched in the interval [TI–4, TI+3½]. If the fraction of the closed-loop pitch lag in the preceding subframe is 1/2, the pitch is searched in the range [TI–3, TI+4½].

The closed-loop pitch search is performed by minimizing a mean-squared weighted error between the target signal and the past filtered excitation (past excitation, convolved withhn). This is achieved by maximizing the following correlation

CCL=xnyknyknykn	(499)

where xn is the target signal andyknis the past filtered excitation at delay k. Note that negative indices refer to the past signal. Note also that the search range is limited around the open loop pitch lags, as explained earlier. The convolution of the past excitation signal withhnis computed only for the first delay in the searched range. For other delays, it is updated using the recursive relation

ykn=yk−1n−1+u−khn, n=0,…,63	(500)

where un, n=−231+17,…,63, is the excitation buffer. Note that in the search stage, the samplesun, n=0,…,63, are unknown and they are needed for pitch delays less than 64. To simplify the search, the LP residual signal, rn, is copied tounfor n=0,…,63, in order to make the relation in equation (501) valid for all delays. If the optimum integer pitch lag is in the range [34, 91], the fractions around that integer value are tested. The fractional pitch search is performed by interpolating the normalized correlation of equation (502) and searching for its maximum. The interpolation is performed using an FIR filter for interpolating the term in equation (503) using a Hamming windowed sinc function truncated at±17. The filter has its cut off frequency (–3 dB) at 5050 Hz and –6 dB at 5760 Hz in the down-sampled domain, which means that the interpolation filter exhibits low-pass frequency response. Note that the fraction is not searched if the selected best integer pitch coincides with the lower end of the searched interval.

Once the fraction is determined, the initial adaptive codevector, v'n, is computed by interpolating the past excitation signal uknat the given phase (fraction). In the following text, the fractional pitch lags (not the fractions) in all subframes will be denoted asdfri, where the index i=0,1,2,3denotes the subframe.

In order to enhance the coding performance, a low-pass filter can be applied to the adaptive codevector. This is important since the periodicity doesn’t necessarily extend over the whole spectrum. The low pass filter is of the form bLPFz=a+b⋅z−1+a⋅z−2. Thus, the adaptive codevector is given by

vn=i=−11bLPFi+1v'n+i, n=0,…,63	(504)

where bLPF=0.18, 0.64, 0.18 for srcelp=12800 for rates at and above 32kbps  and  bLPF=0.21, 0.48, 0.21 otherwise.

An adaptive selection is possible by sending 1 bit per sub-frame. There are then two possibilities to generate the excitation, the adaptive codebook v(n), v(n)=v'(n) in the first path, or its low pass-filtered version as described above in the second path. The path which results in minimum energy of the target signal x(n) is selected for the filtered adaptive codebook vector.

Alternatively, the first or the second path can be used without any adaptive selection. Table 37 summarizes the strategy for the different combinations.

Table 38: Adaptive codebook filtering configuration

| Rates (kbps) | IC/UC | VC | GC |
| --- | --- | --- | --- |
| 7.2 | n.a. | Non-filtered | LP filtered |
| 8.0 | n.a. | Non-filtered | LP filtered |
| 9.6 | n.a. | Non-filtered | LP filtered |
| 13.2 | n.a. | Adaptive selection | Adaptive selection |
| 16.4 | LP-filtered | Adaptive selection | LP filtered |
| 24.4 | LP-filtered | Adaptive selection | LP filtered |
| 32 | n.a. | n.a. | Adaptive selection |
| 64 | n.a. | n.a. | Adaptive selection |

5.2.3.1.4.2	Computation of adaptive codevector gain

The adaptive codevector gain (pitch gain) is then found by

gp=n=063xnynn=063ynyn, constrained by 0≤gp≤1.2	(505)

where yn=vn∗hn is the filtered adaptive codevector (zero-state response of WzHztovn).

To avoid instability in case of channel errors, gp is limited by 0.95, if the pitch gains of the previous subframes have been close to 1 and the LP filters of the previous subframes have been close to being unstable (highly resonant).

The instability elimination method tests two conditions: resonance condition using the LP spectral parameters (minimum distance between adjacent LSFs), and gain condition by testing for high valued pitch gains in the previous frames. The method works as follows. First, a minimum distance between adjacent LSFs is computed as…

At 9.6, 16.4 and 24.4 kbps, the gain is further constrained. It is done for helping the recovery after the loss of a previous frame.

gp=min(gp,0.8n=063xnynn=063ynyn)	(506)

##### 5.2.3.1.5	Algebraic codebook

5.2.3.1.5.1	Adaptive pre-filter

An important feature of this codebook is that it is a dynamic codebook, whereby the algebraic codevectors are filtered through an adaptive pre-filterFz. The transfer function of the adaptive pre-filter varies in time in relation to parameters representative of spectral characteristics of the signal. The pre-filter is used to shape the frequency characteristics of the excitation signal to damp frequencies perceptually annoying to the human ear. Here, a pre-filter relevant to WB signals is used which consists of two parts: a periodicity enhancement part 1/1−0.85z−Tand a tilt part 1−β1z−1. That is,

F0z=1−β1z−11−0.85z−T	(507)

The periodicity enhancement part of the filter colours the spectrum by damping inter-harmonic frequencies, which are annoying to the human ear in case of voiced signals. T is the integer part of the closed-loop pitch lag in a given subframe (representing the fine spectral structure of the speech signal) rounded to the ceiling, i.e.,dfri, where i denotes the subframe.

The factorβ1of the tilt part of the pre-filter is related to the voicing of the previous subframe. At 16.4 and 24.4 kbps it is bounded by [0.28, 0.56] and it computed as

β1=0.28+0.28Ev−1Ev−1+Ec−1	(508)

Otherwise it is bounded by [0.0, 0.5] and is given by

β1=0.5Ev−1Ev−1+Ec−1	(509)

whereEv−1and Ec−1are the energies of the scaled pitch codevector and the scaled algebraic codevector of the previous subframe, respectively. The role of the tilt part is to reduce the excitation energy at low frequencies in case of voiced frames.

Depending on bitrates, coding mode and the estimated level of background noise, the adaptive pre-filter also includes a filter based on the spectral envelope, which colours the spectrum by damping frequencies between the formant regions. The final form of the adaptive pre filter Fzis given by

Fz=F0zAz/η1Az/η2	(510)

where η1=0.75and η2=0.9if srcelp=12800Hz and η1=0.8and η2=0.92if srcelp=16000Hz.

The codebook search is performed in the algebraic domain by combining the pre-filter, Fz, with the weighted synthesis filter prior to the codebook search. Thus, the impulse response hnof the weighted synthesis filter must be modified to include the pre-filter Fz. That is, hn←hn∗fn, where fnis the impulse response of the pre-filter.

5.2.3.1.5.2	Overview of Algebraic codebooks used in EVS

Depending on the bitrate and rendered bandwidth, algebraic codebooks of different sizes are used in the EVS codec. The following tables summarize the codebooks used in each subframe at different bitrates of the EVS codec

Table 38: NB Algebraic codebook configurations (bits/subframe)

| Rate (kbps) | IC | UC | VC | GC |
| --- | --- | --- | --- | --- |
| 7.2 | n.a. | n.a. | 12/12/12/20 | 12/12/12/20 |
| 8.0 | n.a. | n.a. | 12/20/12/20 | 12/20/12/20 |
| 9.6 | 30/32/32/32 | 30/32/32/32 | 28/28/28/28 | 24/26/24/26 |
| 13.2 | n.a. | n.a. | 36/43/36/43 | 36/36/36/43 |
| 16.4 | 56/58/56/58 | 56/58/56/58 | 56/56/56/58 | 55/56/55/56 |
| 24.4 | 96/98/96/98 | 96/98/96/98 | 96/96/96/98 | 94/96/96/96 |

Table 39: WB Algebraic codebook configurations (bits/subframe)

| Rate (kbps) | IC | UC | VC | GC | VC-FEC | GC-FEC | GSC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.2 | n.a. | n.a. | 12/12/12/20 | 12/12/12/20 | n.a. | n.a. | n.a. |
| 8.0 | n.a. | n.a. | 12/20/12/20 | 12/20/12/20 | n.a. | n.a. | n.a. |
| 9.6 | 28/28/28/28 | 28/28/28/28 | 26/26/26/28 | 20/26/24/24 | n.a. | n.a. | n.a. |
| 13.2 | n.a. | n.a. | 28/36/36/36 (TD BWE) 36/36/36/43 (FD BWE) | 28/36/28/36 (TD BWE) 36/36/36/36 (FD BWE) | n.a. | n.a. | n.a. |
| 16.4 | 43/43/43/43/43 | 43/43/43/43/43 | 40/43/43/43/43 | 40/43/40/43/43 | n.a. | n.a. | n.a. |
| 24.4 | 75/75/75/75/75 | 75/75/75/75/75 | 73/75/73/75/75 | 73/73/73/75/73 | 73/73/73/73/75 | 70/75/73/73/73 | n.a. |
| 32 | 12/12/12/12/12 | n.a. | n.a. | 36/36/36/36/36 | n.a. | n.a. | n.a. |
| 64 | 12/12/12/12/12 | n.a. | n.a. | 36/36/36/36/36 | n.a. | n.a. | n.a. |

Table 40: SWB Algebraic codebook configurations (bits/subframe)

| Rate (kbps) | IC | UC | VC | GC | VC-FEC | GC-FEC | GSC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9.6 | 24/26/24/26 | 24/26/24/26 | 20/26/24/24 | 20/20/20/20 | n.a. | n.a. | n.a. |
| 13.2 | n.a. | n.a. | 28/36/28/36 | 28/28/28/36 | n.a. | n.a. | n.a. |
| 16.4 | 36/36/36/36/36 | 36/36/36/36/36 | 34/36/36/36/36 | 34/36/34/36/36 | n.a. | n.a. | n.a. |
| 24.4 | 62/65/62/65/62 | 62/65/62/65/62 | 62/62/62/65/62 | 62/62/62/62/62 | 62/62/62/62/62 | 61/61/62/61/62 | n.a. |
| 32 | 12/12/12/12/12 | n.a. | n.a. | 36/28/28/36/36 | n.a. | n.a. | n.a. |
| 64 | 12/12/12/12/12 | n.a. | n.a. | 36/36/36/36/36 | n.a. | n.a. | n.a. |

Table 41: FB Algebraic codebook configurations (bits/subframe)

| Rate (kbps) | IC | UC | VC | GC | VC-FEC | GC-FEC |
| --- | --- | --- | --- | --- | --- | --- |
| 16.4 | 36/36/36/36/36 | 36/36/36/36/36 | 34/36/36/34/36 | 34/34/36/34/36 | n.a. | n.a. |
| 24.4 | 62/62/65/62/65 | 62/62/65/62/65 | 62/62/62/62/62 | 62/62/62/62/62 | 61/62/61/62/62 | 61/61/61/61/61 |
| 32 | 12/12/12/12/12 | n.a. | n.a. | 36/28/28/36/36 | n.a. | n.a. |
| 64 | 12/12/12/12/12 | n.a. | n.a. | 36/36/36/36/36 | n.a. | n.a. |

VC-FEC and GC-FEC are specific configurations for which 4 bits are reserved to transmit LPC-based information exploited by the decoder in case of error of the previous frame.

5.2.3.1.5.3	Codebook structure and pulse indexing of the 7-bit codebook

In the 7-bit codebook, the algebraic vector contains only 1 non-zero pulse at one of 64 positions. The pulse position is encoded with 6 bits and the sign of the pulse is encoded with 1 bit. This gives a total of 7 bits for the algebraic code. The sign index here is set to 1 for positive signs and 0 for negative signs.

5.2.3.1.5.4	Codebook structure and pulse indexing of the 12-bit codebook

In the 12-bit codebook, the algebraic vector contains only 2 non-zero pulses. The 64 positions in a subframe are divided into 2 tracks, where each track contains one pulse, as shown in table 39.

Table 40: Potential positions of individual pulses in the 12-bit algebraic codebook

| Track | Pulse | Positions |
| --- | --- | --- |
| 1 | 0 | 0, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62 |
| 2 | 1 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19, 21, 23, 25, 27, 29, 31, 33, 35, 37, 39, 41, 43, 45, 47, 49, 51, 53, 55, 57, 59, 61, 63 |

Each pulse position in one track is encoded with 5 bits and the sign of the pulse in the track is encoded with 1 bit. This gives a total of 12 bits for the algebraic code. The sign index here is set to 0 for positive signs and 1 for negative signs.

The index of the signed pulse is given by

I=p+s⋅2M	(511)

where p is the position index, s is the sign index, and M=5 is the number of bits per track. For example, a pulse at position 31 has a position index of 31/2 = 15 and it belongs to the track with index 1 (second track).

5.2.3.1.5.5	Codebook structure and pulse indexing of the 20-bit and larger codebooks

In the 20-bit or larger codebooks, the codevector contains 4 non-zero pulses. All pulses can have the amplitudes +1 or –1. The 64 positions in a subframe are divided into 4 tracks, where each track contains one pulse, as shown in table 41.

Table 43: Potential positions of individual pulses in the 20-bit algebraic codebook

| Track | Pulse | Positions |
| --- | --- | --- |
| 1 | 0 | 0, 4, 8, 12, 16, 20, 24, 28, 32 36, 40, 44, 48, 52, 56, 60 |
| 2 | 1 | 1, 5, 9, 13, 17, 21, 25, 29, 33, 37, 41, 45, 49, 53, 57, 61 |
| 3 | 2 | 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62 |
| 4 | 3 | 3, 7, 11, 15, 19, 23, 27, 31, 35, 39, 43, 47, 51, 55, 59, 63 |

Each pulse position in one track is encoded with 4 bits and the sign of the pulse in the track is encoded with 1 bit. This gives a total of 20 bits for the algebraic code.

5.2.3.1.5.6	Pulse indexing of the algebraic codebook

The objective is to enumerate all possible constellations of pulses in a vector c which corresponds to one track of length L within a sub-frame. That is, vector c has signed integer values such that its norm-1 is |x|1=p, whereby we say that c contains p pulses.

We can then partition the vector c into two parts, c=c1, c2 such that the partitions are of length L1 and L2=L−L1 and contain p1 and p2=p−p1 pulses respectively. The number of different constellations for the original vector c can then be determined by the recursive formulae:

fp,L=pp1=0fp1,L1fp−p1,L2,p>0,L>1p>1f0,L=1L>0.	(512)

For computational efficiency, the values of this function can be pre-calculated and placed in a table.

Above equation gives the number of possible states for given p and L. We can then enumerate a specific state, where c1 and c2 have π1 and π2=p−π1 pulses respectively. The number of states that have less pulses than π1 in partition c1 is

sc1,c2=π1−1p1=0fp1,L1f(p−p1,L−L1).	(513)

We can then define that overall state has sc≥sc1,c2, whereby the overall state can be encoded with the recursion

sc=sc1,c2+sc1+fp1,L1sc2,	(514)

where the boundary conditions are

sc=0,L=1, c1≥01,L=1,c1<0.	(515)

The state can be decoded by the algorithm

- Set p1:=1 and choose partitioning length L1≥1 and L2≥1.

- Calculate sc1,c2 with p1.

- If sc<sc1,c2 then π1=p1−1. Otherwise, set p1:=p1+1 and go to 2.

The states of the partitions sc2 and sc1 can then be calculated from the integer and reminder parts of the fraction sc−sc1,c2fp1,L1. We can then recursively determine the state of each position in the vector c until a partition has Lk=1, whereby

ck1=+πk,for sck=0−πk,for sck=1.	(516)

Observe that both the number of states fp,L as well as the state sc are integer numbers which can become larger than 32 bits. We must therefore employ arithmetic operations which support long integers throughout the algorithm.

5.2.3.1.5.7	Pulse indexing of the 43-bit codebook

The joint indexing encoding procedure of three pulses on two tracks is described as follows:

For 3 pulses on a track, the occurrence probability of 3 different pulse positions on a track is the highest, and the occurrence probability of 2 different pulse positions on a track is the second highest, and even the pulses have a higher occurrence probability on the left position of the track than on the right position of the track because the algebraic codebooks need to compensate for the boundary leap of adaptive codebooks between two neighbour sub-frame. So the case of the first pulse with lower position order will be encoded with a smaller index value and the case of more different pulse positions with higher occurrence probability will also be encoded with a smaller index value. The rule is same in case of more than 3 pulses on a track. This rule can be used to save bit in the multi-track joint indexing encoding.

- Firstly, the pulse information for each track is indexed as follows: (here we suppose that Q(Q=3) pulses are assigned for each track, and the total quantity of positions on the track is M)

- Analyse the statistics about the positions of the Q pulses to be encoded on a track and obtain pulse distribution on the track, it includes: quantity (namely posnum=N) of pulse positions with pulses in it, the pulse position distribution which includes pulse position vector: P(N)=p(0),p(1),...,p(N−1), N is the quantity of pulse positions , p(i) is the ith pulse positions with pulse in it on the track, and quantity of pulses in each pulse position with pulse in it which includes pulse number vector SU(N)=su(0),su(1),...,su(N−1), su(0)+su(1)+⋯+su(N−1)=Q, where Q is the number of pulses per track, su(i) is the number of pulses in position p(i), and pulse sign vector S(N)=s(0),s(1),...,s(N−1), s(i) is the ith sign in position p(i). If there are pulses having the same positions (pulses with the same positions have the same signs), they are merged into one pulse and the number of pulses for each pulse position as well as the pulse sign is saved. Pulse position are sorted in ascend order, the pulse sign is also adjusted based on the order of pulse position.

- Compute the offset index I1(N) according to the quantity of pulse positions, the offset index is saved in a table and used in both encoder and decoder sides. Each offset index in the table indicate a unique number of pulse positions in the track, in case Q=N, the offset index only indicate a pulse distribution of pulse positions on the track P(N), in case Q>N, the offset index indicate many SU(N) which have a same pulse distribution of pulse positions on the track P(N).

- Compute the pulse- position index I2(N) according to the pulse distribution of pulse positions on the track (0≤I2(N)<CMN). The I2(N)only indicate a pulse distribution of pulse positions on the track P(N) among all the pulse distribution of I1(N). Permuting serial numbers of the positions P(N)=p(0),p(1),...,p(N−1) and all possible values of P(N) are ordered from a smaller value to a greater value p(0)<p(1)<⋯<p(N−1), N refers to the quantity of positions with pulses in it, M is the total quantity of positions on the track. Compute I2(N) by using the permutation method as follows:

I2(N)=CMN−CM−p(0)N+n=1N−1[CM−p(n−1)−1N−n−CM−p(n)N−n], 0≤I2(N)<CMN, 0≤p(0) < p(1) <⋯< p(N - 1)<15	(514)

wherein p(n) represents a position serial number of an nth position that has pulses on it, n∈[0,N−1], p(0)∈[0,M−N], p(n)∈[p(n−1)+1,M−N+n], p(0)<p(1)<⋯<p(N−1). For 43bit mode, 3 pulses on a track, M=16, 0<N≤3.

I2(N)=C163−C16−p(0)3+C15−p(0)2−C16−p(1)2+C15−p(1)1−C16−p(2)1	(515)

Compute the pulse-number index I3(N) according to the quantity of pulses in each pulse position as follows:

I3(N) is determined according to SU(N) which represents the quantity of pulses in each position with pulses. In order to determine correspondence between SU(N) and I3(N) through algebraic calculation, a calculation method of the third index I3(N) is provided below:

For a track, situations that a track with N pulse positions and Q pulses are mapped to situations that a N positions track have Q−N pulses, where Q represents the total number of pulses that are required to be encoded and on the track. For example, in the condition of 6-pulse 4-position (Q =6, N=4) situations, SU(N) is {1, 2, 1, 2}, 1 is subtracted from the number of pulses in each position (because each position has at least one pulse) to obtain {0, 1, 0, 1}, that is, information of SU(N) is mapped to a 2-pulse 4-position (Q =2, M=4) encoding situation. Figure 16 gives an example of the mapping for I3(N).

Figure 17: Example of mapping for I3(N)

According to set order, all possible distribution situations of Q−N pulses on N positions are arrayed, and an arrayed serial number is used as the indexI3(N) indicating the number of pulses on a position that has pulse.

A calculation formula reflecting the foregoing calculation method is:

I3(N)=CPPTΔN−CPPT−q(0)ΔN+h=1ΔN−1[CPPT−h−q(h−1)ΔN−h−CPPT−h−q(h)ΔN−h]	(517)

wherein ΔN=Q−N,PPT=Q−1, q(h) represents a position serial number of an (h + 1)th pulse, h∈[0,ΔN−1], q(h)∈[0,N−1], q(0)≤q(1)≤...≤q(ΔN−1), and ∑ indicates summation.

Compute the pulse-sign indexI4(N) based on the N pulse sign information.

The pulse sign represented by s(i) may be a positive value or a negative value. A simple coding mode is generally applied, s(i)=0 represents a positive pulse and s(i)=1 represents a negative pulse.

Generate the global index I. Combine the indices I1(N), I2(N), I3(N) and I4(N) to get the global index I as follows :

I=I1(N)+I23×2N+I4(N),1≤N≤M,I∈[0,W−1]	(518)

I23=I3(N)×CMN+I2(N)	(519)

Here W is the upper range of I which is also the number of total permutations of Q pulses.

2. Combine the index of the two 3-pulse tracks together which is encoded as in step 1, suppose the indexes of the two tracks are Ind1 and Ind2 respectively, Ind1∈[0,W1−1] and Ind2∈[0,W2−1], then the Joint_index is as below:

Joint_index=Ind1∗W2+Ind2	(520)

3. Encode the joint index Joint_index. (Suppose encode with 25 bits). In order to reduce the number of bits used for pulse indexing, a threshold THR is set at 3611648 for 3-pulses, according to the the pulse number, combination of the occurrence probability and the number of bits that may be saved. If the joint index Joint_index is smaller than THR, 24 bits will be used to encode the joint index Joint_index. If the joint index Joint_index is bigger than or equal to THR, THR will be added into the joint index Joint_index and 25 bits will be used to encode the joint index Joint_index. This procedure is described as below:

If (Joint_index<THR)

{

Joint_index is encoded with 24 bits.

}

Else

{

Joint_index=Joint_index+THR

Joint_index is encoded with 25 bits.

}

For two pulses on the other track, the index for each track is encoded just as pulse indexing of the 20-bit codebook, but there is no joint indexing procedure, then the index for each track is transmitted one by one.

- 5.2.3.1.5.8	Multi-track joint coding of pulse indexing

The codebook for more than three pulses on a track have idle space in difference ratio, joint encoding for more than two tracks may enable idle codebook spaces in single-track encoding to be combined, and once combined idle spaces are sufficient, one actual encoding bit may be reduced. If several encoding indexes are directly combined, the final encoding length may be very large, or even may exceed the bit width (such as 64 bits) generally used for operating, so a general solution is to split each encoding index into two part and only all the high part is combined together in order to avoid directly combining.

The method is described as follows: the value range of the original index Indt is divided into several intervals by a factor SLFt, correspondingly the original index Indt is split into two indexes Indt0 and Indt1 by the factor SLFt, the length of each interval is not greater than SLFt, SLFt is a positive integer, Indt0 denotes a serial number of an interval to which Indt belongs, and Indt1 denotes a serial number of Indt in the interval to which Indtbelongs (apparently,Indt1≤SLFt), and: Indt≤Indt0×SLFt+Indt1;

The most economical case of splitting is performed as following:

Indt0=Int(IndtSLFt), where Int() denotes rounding down to an integer, and

Indt1=Indt%SLFt, where % denotes taking a remainder.

If a combined index needs to achieve better effect of saving encoding bits, it is needed to select a split index that retains the space characteristics of Indt as much as possible, and therefore, for the track t providing a split index to participate in combination,

if SLFt=2Kt, it is appropriate to select Indt0 to participate in combination, and

if SLFt=Int(Indtmax2Kt), it is appropriate to select Indt1 to participate in combination.

Figure 18: The split factor selection and the corresponding codebook space section

Each track may adopt different SLFt, according to the pulse number on it

Table 42: the parameters for multi-track joint coding

| pulse | bits | Codebook space |  | Hi Bit |  |  | effective ratio | re-back bits |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Dec | Hex | value | bits | range |  | 8bit | 16bit | 24bit |
| 1 | 5 | 32 |  |  |  |  |  |  |  |  |
| 2 | 9 | 512 | 200 | 1 | 1 | 2 | 1.00 | 0 |  |  |
| 3 | 13 | 5472 | 1560 | A8 | 8 | 172 | 0.6875 | 3 |  |  |
| 4 | 16 | 44032 | AC00 | AC | 9 | 345 | 0.67578125 | 1 | 9 |  |
| 5 | 19 | 285088 | 459A0 | 8B | 8 | 140 | 0.546875 |  | 5 |  |
| 6 | 21 | 1549824 | 17A600 | BD | 8 | 190 | 0.7421875 |  | 3 |  |
| 7 | 23 | 7288544 | 6F36E0 | DE | 8 | 223 | 0.875 |  | 1 | 9 |
| 8 | 25 | 30316544 | 1CE9800 | 1CE | 9 | 463 | 0.904297 |  |  | 8 |
| 9 | 27 | 113461024 | 6C34720 | 6C3 | 11 | 1732 | 0.845704 |  |  | 8 |

Multi-track joint coding processing is described as following:

Calculate an encoding index Indt of each track, (subscript t denotes the tth track), split Indt into two split indexes hitand track according to a set factor SLFt combine a split index hitof each track to generate a combined index hiSLPt. The combined index hiSLPt is split into recombined indexes ht according to the re-back bits length, and each recombined index ht and an un-combined split index track of a corresponding track are respectively combined, then obtain the final recombined indexfinalindext with fixed length 8,16 or 24 bits.

For 4 track in a sub-frame, the algebraic codebook 94bit(8777)~108bit(9999) use 24 bits mode joint en/decoding，the algebraic codebook 62bit(4444)~92bit(7777) use 16 bits mode joint en/decoding，the algebraic codebook  40bit(3222)~61bit(4443) use 8 bits mode  joint en/decoding.

All the encoding steps are described as following:

  - Get the parameter from table 43 according to the pulse number of each track, include the index bitst, HiBitbitst, HiBitranget, re−backbitst. And get the 8/16/24 mode also according to the pulse number of all track.

  - The index Indt of trackt is split into hit and track, the SLFt is 2(bitst−HiBitbitst), and length of hit is HiBitbitst, length of track is (bitst−HiBitbitst),

  - Combine thehi0 and hi1 into hiSLP0 as following:

hiSLP0=hi0×HiBitrange1+hi1	(521)

  - Split the low part of hiSLP0and get the h0, and the length of  h0 is re−backbits0,which get from table 44 in step 1, the h0 and track are combine into a finalindex0with the length of 8,16 or 24 bits.

  - The high part hiSLP0H of  hiSLP0continue combining with the next hi2as following:

hiSLP1=hiSLP0H×HiBitrange2+hi2	(522)

  - Split the low part of hiSLP1and get the h1, and the length of  h1 is re−backbits1,which get from table 45 in step 1, the h1 and track are combine into a finalindex1with the length of 8,16 or 24 bits.

  - The high part hiSLP1H of  hiSLP1continue combining with the next hi3as following:

hiSLP2=hiSLP1H×HiBitrange3+hi3	(523)

  - Split the low part of hiSLP2and get the h2, and the length of  h2 is re−backbits2,which get from table 46 in step 1, the h2 and track are combine into a finalindex2with the length of 8,16 or 24 bits.

  - The high part hiSLP2H of  hiSLP1is split into two parts.  The low part of hiSLP2His used as the h3, and the length of  h3 is re−backbits3 which is obtained from table 47 in step 1, the h3 and track are combined into a finalindex3with the length of 8,16 or 24 bits.

  - Finally, the high part trackhiof hiSLP2Htogether withfinalindex0, finalindex1, finalindex2 and finalindex3 are the outputs of multi-track joint coding and  stored into the stream in 16 bits unit.

Figure 19: Schematic diagram of 4-track joint coding

5.2.3.1.5.9	The search criterion at lower bitrates

The algebraic codebook is searched by minimizing the error between an updated target signal and a scaled filtered algebraic codevector. The updated target signal is given by

x11n=xn−gpyn, n=0,…,63	(524)

where yn=vn∗hnis the filtered adaptive codevector and gp is the unquantized adaptive codebook gain. Thus, the updated target signal is obtained by subtracting the adaptive contribution from the initial target signal, xn.

Let a matrix Hbe defined as a lower triangular Toeplitz convolution matrix with the main diagonal h0and lower diagonals h1,…,h63, and d=HTx11 (also known as the backward filtered target vector) be the correlation between the updated signal x11nand the impulse response hn. Furthermore, let Φ=HTH be the matrix of correlations of hn. Here, hnis the impulse response of the combination of the synthesis filter, the weighting filter and the pre-filter Fzwhich includes a long-term filter.

The elements of the vector dnare computed by

dn=i=n63x11ihi−n, n=0,…,63	(525)

and the elements of the symmetric matrix Φ are computed by

ϕi,j=n=j63hn−ihn−j, i=0,…,63, j=i,…,63	(526)

Let ck the k-th algebraic codevector. The algebraic codebook is searched by maximizing the following criterion:

Qk=x11THck2ckTHTHck=dTck2ckTΦck=Rk2Ek	(527)

The vector dnand the matrix Φare usually computed prior to the codebook search.

The algebraic structure of the codebooks allows for very fast search procedures since the algebraic codevector, The algebraic structure of the codebooks allows for very fast search procedures since the algebraic codevector, ckn, contains only a few non-zero pulses. The correlation in the numerator of equation (528) is given by

R=i=0Np−1sidmi	(529)

where miis the position of the i-th pulse, siis its amplitude (sign), and Npis the number of pulses. The energy in the denominator of equation (530) is given by

E=i=0Np−1ϕmi,mi+2i=0Np−1j=i+1Np−1sisjϕmi,mj	(531)

For saving the search load along with a better search result in the 12-bit codebook, the pulse amplitudes are predetermined based on a high-pass filtered dn. The high-pass filter is a three-tap MA (moving-average)-type filter, and its filter coefficients are { -0.35, 1.0, -0.35 }. The sign of a pulse in a position n is set to negative when the high-pass filtered dn is negative, otherwise the sign is set to positive. To simplify the search, dn and Φk,h are modified to incorporate the predetermined signs.

5.2.3.1.5.10	The search criterion at higher bitrates

The following search criterion is used for bit rates at and above 16.4 kbps. It allows limiting the increase of complexity for high number of pulses.

Let N be the sub-frame length, and let matrices H and C, respectively, denote the N×N lower triangular Toeplitz convolution matrix and the N+K−1×N full-size convolution matrix, both defined for the filter h(n). Here, h(n) is the length K impulse response of the combination of the synthesis filter, the weighting filter and the pre-filter F(z)which includes a long-term filter. The target residual is q=H−1x11 and Φ=CTC is the autocorrelation matrix of filter h(n).

The elements of the autocorrelation matrix can be calculated by

Φk,h=φk−h=Kn=0hnhn−k+h,  n=0,…,N−1	(532)

and the target residual by

qn=x11n−nk=1hkqn−k,n=0,…,N−1.	(533)

The final target is then d=Rq which can be calculated by

dn=N−1k=0φn−kqk,n=0,…,N−1.	(534)

Let ck be the kth algebraic codevector. The algebraic codebook is searched by maximizing the following criterion:

Qk=qTΦck2ckTΦck=dTck2ckTΦck=Rk2Ek	(535)

##### 5.2.3.1.6	Combined algebraic codebook

In general the computational complexity of the algebraic codebook increases with the codebook size. In order to keep the complexity reasonable while providing better performance and scalability at high EVS ACELP bit-rates, an efficient combined algebraic codebook structure is employed. The combined algebraic codebook combines usually a frequency-domain coding in a first stage followed by a time-domain ACELP codebook in a second stage.

The frequency-domain coding of the first stage, denoted as a pre-quantizer in figure 20, uses a Discrete Cosine Transform (DCT) as the frequency representation and an Algebraic Vector Quantizer (AVQ) (see subclause 5.2.3.1.6.9)  to quantize the frequency-domain coefficients of the DCT. The pre-quantizer parameters are set at the encoder in such a way that the ACELP codebook (second stage of the combined algebraic codebook) is applied to an excitation residual with more regular spectral dynamics than the pitch residual.

Figure 21: Schematic diagram of the ACELP encoder using a combined algebraic codebook in GC mode at high bit-rates

At the encoder, the first stage, or pre-quantizer, operates as follows. In a given subframe (aligned to the subframe of the ACELP codebook in the second stage) the excitation residual qin(n) after applying the adaptive codebook is computed as

qin(n)=r(n)−gp⋅v(n)	(536)

where r(n) is the target vector in residual domain. Further, v(n) is the adaptive codevector and gp the adaptive codevector gain.

The excitation residual qin(n) after applying the adaptive codebook is de-emphasized with a filter Fp(z). A difference equation for such a de-emphasis filter Fp(z) is given by

qin,d(n)=qin(n)+α⋅qin,d(n−1)	(537)

where qin,d(n) is the de-emphasized residual and coefficient α=0.3 controls the level of de-emphasis.

Further a DCT is applied to the de-emphasized excitation residual qin,d(n)using a rectangular non-overlapping window. Depending on the bit rate, all blocks or only some blocks of DCT coefficients Qin,d(k)  usually corresponding to lower frequencies are quantized using the AVQ encoder. The other (not quantized) DCT coefficients Qd(k) are set to 0 (not quantized). To obtain the excitation residual for the second (ACELP) stage of the combined algebraic codebook, the quantized DCT coefficients Qd(k) are inverse transformed, and then a pre-emphasis filter 1Fp(z) is applied to obtain the time-domain contribution from the pre-quantizer q(n). The pre-emphasis filter has the inverse transfer function of the de-emphasis filter Fp(z).

5.2.3.1.6.1	Quantization

The AVQ encoder produces quantized transform-domain DCT coefficients Qd(k). The indices of the quantized and coded DCT coefficients from the AVQ encoder are transmitted as a pre-quantizer parameters to the decoder.

In every sub-frame, a bit-budget allocated to the AVQ is composed as a sum of a fixed bit-budget and a floating number of bits. Depending on the used AVQ sub-quantizers of the encoder, the AVQ usually does not consume all of the allocated bits, leaving a variable number of bits available in each sub-frame. These bits are floating bits employed in the following sub-frame. The floating number of bits is equal to 0 in the first sub-frame and the floating bits resulting from the AVQ in the last sub-frame in a given frame remain unused when coding WB signals or are re-used in coding of upper band (see subclause 5.2.6.3).

5.2.3.1.6.2	Computation of pre-quantizer gain

Once the pre-quantizer contribution is computed, the pre-quantizer gain is obtained as

gq=k=0N−1Qin,d(k)Qd(k)k=0N−1Qd(k)Qd(k)	(538)

where Qin,d(k) are the AVQ input frequency coefficients and Qd(k) the AVQ output (quantized) frequency coefficients where k=0,...,K−1 is the transform-domain coefficient index and K=64being the number of DCT transform coefficients.

5.2.3.1.6.3	Quantization of pre-quantizer gain

The pre-quantizer gain gq is quantized as follows. First, the gain is normalized by the predicted innovation energy Epred as follows:

(539)

where the predicted innovation energy Epred is obtained as described in subclause 5.2.3.1.7.1.

Then the normalized gain gq,norm is quantized by a scalar quantizer in a logarithmic domain and finally de-normalized resulting in a quantized pre-quantizer gain. Specifically 6-bit scalar quantizer is used whereby the quantization levels are uniformly distributed in the log domain. The index of the quantized pre-quantizer gain is transmitted as a pre-quantizer parameter to the decoder.

5.2.3.1.6.4	Refinement of target vector

The pre-quantizer contribution q(n) is used to refine the original target vector for adaptive codebook search x(n) as

xupdt(n)=x(n)−gq⋅w(n),	(537)

and to refine the adaptive codebook gain using equation (505) with xupdt(n) used instead of x(n). When the pre-quantizer is used, the computation of the target vector for algebraic codebook search x11(n) is done using

x11(n)=x(n)−gq⋅w(n)−gp,updt⋅y(n)	(538)

where w(n) is the filtered pre-quantizer contribution, i.e. the zero-state response of the weighted synthesis filter to the pre-quantizer contribution q(n), and  gp,updt is the refined adaptive codebook gain.

Similarly, the target vector in residual domain r(n) is updated for the algebraic codebook search (the second-stage of the combined algebraic codebook) as

rupdt(n)=r(n)−gq⋅q(n)−gp,updt⋅v(n).	(539)

5.2.3.1.6.5	Combined algebraic codebook in GC mode

In the EVS codec, the combined algebraic codebook structure as from figure 21 is used at bit-rates of 32 kbps and 64 kbps. In both cases the algebraic codebook search uses 36-bit codebooks and the rest of the bit-budget is employed by the AVQ to quantize the pre-quantizer coefficients.

At 32 kbps, the available fixed bit-budget for the AVQ (116, 115, 115, 115, 155 bits for every of five subframes) is sometimes too low to properly encode all input signal frames. Consequently in GC mode at 32kbps, the DCT and iDCT stages of pre-quantizer computation are omitted when the input signal is not classified as a harmonic one. The classification is based on a harmonicity counter  harmacelp updated every frame in the pre-processing module. If in a given frame the harmonicity counter harmacelp≤2 the frame is classified as non-harmonic and the AVQ is applied directly on the time-domain signal qin,d(n) and similarly producing directly the time-domain signal qd(n) in figure 21.

5.2.3.1.6.6	Combined algebraic codebook in TC mode

The combined algebraic codebook structure is used also in TC mode at 32kbps and 64kbps. In this mode the algebraic codebook from figure 21 is replaced by glottal shape codebook but the structure of the pre-quantizer remains the same as in the GC mode. In TC mode @32kbps, the DCT and iDCT stages of the pre-quantizer are always employed.

Figure 22: Schematic diagram of the ACELP encoder using a combined algebraic codebook in IC mode at high bit-rates

5.2.3.1.6.7	Combined algebraic codebook in IC mode

Depending on the input signal characteristics, the ACELP encoder using a combined algebraic codebook from figure 21 is further adaptively changed. Specifically in coding of inactive speech segments, the order of the combined algebraic codebook stages is changed. I.e. the modified combined algebraic codebook combines a time-domain ACELP codebook in a first stage followed by a frequency-domain de-quantizer coding in a second stage as shown in figure 22. The first stage algebraic codebook employs very small codebooks, specifically 12 bits per subframe.

At the encoder, the de-quantizer in IC mode operates as follows. In a given subframe, the target signal x3(n) after subtracting the scaled filtered adaptive excitation and the scaled filtered algebraic excitation is computed as

x3(n)=x(n)−gp⋅y(n)−gc⋅z(n).	(540)

The target signal in speech domain x3(n) is filtered through the inverse of the weighted synthesis filter with zero states resulting in the target in residual domain uin(n).

Similarly to the combined algebraic codebook in GC mode, the signal uin(n) is first de-emphasized with a filter Fp(z) to enhance the low frequencies. A DCT is applied to the de-emphasized signal uin,d(n) using rectangular non-overlapping window. Usually all blocks of DCT coefficients Uin,d(k) are quantized using the AVQ encoder. The quantized DCT coefficients Ud(k) in some bands can be however set to zero.

The quantized DCT coefficients Ud(k) are further inverse transformed using iDCT, and then a pre-emphasis filter 1Fp(z) is applied to obtain the time-domain contribution from the frequency-domain quantizer u(n) where the pre-emphasis filter has the inverse transfer function of the de-emphasis filter Fp(z).

5.2.3.1.6.8	Computation and quantization of de-quantizer gain

Once the de-quantizer contribution is computed, the de-quantizer gain is obtained as

gq=k=0N−1Uin,d(k)Ud(k)k=0N−1Ud(k)Ud(k)	(541)

where Uin,d(k) are the AVQ input transform-domain coefficients and Ud(k) are the AVQ output (quantized) transform-domain coefficients.

The de-quantizer gain gq is quantized using the normalization by the algebraic codebook gain gc. Specifically a 6-bit scalar quantizer is used whereby the quantization levels are uniformly distributed in the linear domain. The indice of the quantized de-quantizer gain gq is transmitted as a de-quantizer parameter to the decoder.

When coding the inactive signal segments the adaptive codebook excitation contribution is limited to avoid a strong periodicity in the synthesis. In practice a limiter is applied in the adaptive codebook search to constrain the adaptive codebook gain by 0≤gp≤0.65.

5.2.3.1.6.9	AVQ quantization with split multi-rate lattice VQ

Prior to the AVQ quantization, the time domain or transform-domain 64 coefficients, here denoted as S'(k), are split into 8 consecutive sub-bands of 8 coefficients each. The sub-bands are quantized with an 8-dimensional multi-rate algebraic vector quantizer. The AVQ codebooks are subsets of the Gosset lattice, referred to as the RE8 lattice.

5.2.3.1.6.9.1	Multi-rate AVQ with the Gosset Lattice RE8

5.2.3.1.6.9.1.1	Gosset Lattice RE8

The Gosset lattice RE8 is defined as the following union:

RE8=2D8∪2D8+1,1,1,1,1,1,1,1	(542)

where D8 is the 8-dimensional lattice composed of all points with integers components with the constraint that the sum of the 8 components is even. The lattice 2D8 is simply the D8 lattice scaled by 2. This implies that the sum of the components of a lattice point in 2D8 is an integer multiple of 4. Therefore, the 8 components of a RE8 lattice point have the same parity (either all even or all odd) and their sum is a multiple of 4.

All points in the lattice RE8 lie on concentric spheres of radius 8nj, nj being the codebook number in sub-band j. Each lattice point on a given sphere can be generated by permuting the coordinates of reference points called “leaders”. There are very few leaders on a sphere compared to the total number of lattice points which lie on the sphere.

5.2.3.1.6.9.1.2	Multi-rate codebooks in Gosset Lattice RE8

To form a vector codebook at a given rate, only lattice points inside a sphere in 8 dimensions of a given radius are taken. Codebooks of different bit rates can be constructed by including only spheres up to a given radius. Multi-rate codebooks are formed by taking subsets of lattice points inside spheres of different radii.

5.2.3.1.6.9.1.2.1	Base codebooks

First, base codebooks are designed. A base codebook contains all lattice points from a given set of spheres up to a number nj. Four base codebooks , noted Q0, Q2, Q3, and Q4, are used. There are 36 non-null absolute leaders plus the zero leader (the origin): Table 48 gives the list of these leaders and indicates to which codebook a leader belongs. Q0, Q2, Q3, and Q4 are constructed with respectively 0, 8, 12, and 16 bits. Hence codebook Qnj requires 4nj bits to index any point in that codebook.

5.2.3.1.6.9.1.2.2	Voronoi extensions

From a base codebook CAVQ (i.e. a codebook containing all lattice points from a given set of spheres up to a number nj), an extended codebook can be generated by multiplying the elements of CAVQ by a factor Mjv, and adding a second-stage codebook called the Voronoi extension. This construction is given by

cj = Mjv⋅ zj+vj	(543)

where Mjv is the scaling factor, zj is a point in a base codebook CAVQ and vj is a point in the Voronoi extension. The extension is computed in such a way that any point cj from equation (544) is also a lattice point in RE8. The scaling factor Mjv is a power of 2 (Mjv=2rjv), where rjv is called the Voronoi extension order.

Such extended codebooks include lattice points that extend further out from the origin than the base codebook. When a given lattice point cj is not included in a base codebook CAVQ (Q0, Q2, Q3 or Q4), the so-called Voronoi extension is applied, using the Q3 or Q4 base codebook part.

Giving the available bit-budget in particular layers, the maximum Voronoi extension order is rjv=2. Therefore, for Q3 or Q4, two extension orders are used: rjv=1 or 2 (Mjv=2 or 4).

When rjv=0, there is no Voronoi extension, and only a base codebook is used.

5.2.3.1.6.9.1.2.3	Codebook rates

There are 8 codebooks: the first 4 are base codebooks without Voronoi extension and the last four with Voronoi extension. The codebook number nj is encoded as a unary code with nj "1" bits and a terminating "0". Table 49 gives for each of the 8 codebooks, its base codebook, its Voronoi extension order (rjv=0 indicates that there is not Voronoi extension), and its unary code.

Table 50: Multi-rate codebooks in RE8 lattice

| Codebook number nj | Base Codebook | Voronoi extension order rjv | Unary code for nj |
| --- | --- | --- | --- |
| 0 | Q0 | 0 | 0 |
| 2 | Q2 | 0 | 10 |
| 3 | Q3 | 0 | 110 |
| 4 | Q4 | 0 | 1110 |
| 5 | Q3 | 1 | 11110 |
| 6 | Q4 | 1 | 111110 |
| 7 | Q3 | 2 | 1111110 |
| 8 | Q4 | 2 | 11111110 |

For the base codebook Q0, (nj=0), there is only one point in the codebook and 1 bit is used to transmit the unary code corresponding to nj.

For the other three base codebooks Qnj (nj=2,3, or 4) without Voronoi extension:

- nj bits are used to transmit the unary code corresponding to nj,,

- 4nj bits are required to index a point in Qnj

- thus 5nj bits are used in total.

For codebooks with Voronoi extension (nj>4):

- nj bits are used to transmit the unary code corresponding to the base codebook number Q3 (respectively Q4) if nj is even (respectively odd) and the Voronoi extension order rjv is 1 if nj<7, or 2 otherwise),

- 12 bits (respectively 16 bits) are required to index the point zj in the base codebook Q3 (respectively Q4)

- 8rjv bits are required to index the 8-dimensional point vj in the Voronoi extension of order rjv

- thus, 5nj bits are used in total.

In the codebook number encoding, a simple bit overflow check is performed: in case when the last AVQ coded sub-band of the spectrum S'(k) is quantized, nj>0 and only 5nj−1 bits are available for the quantization, the terminating "0" in the codebook number coding is not encoded. At the decoder, the same bit overflow check enables the right decoding of the codebook number in this sub-band.

5.2.3.1.6.9.2	Quantization with RE8 lattice

In lattice quantization, the operation of finding the nearest neighbour of the input spectrum S'(k) among all codebook points is reduced to a few simple operations, involving rounding the components of spectrum S'(k) and verifying a few constraints. Hence, no exhaustive search is carried out as in stochastic quantization, which uses stored tables. Once the best lattice codebook point is determined, further calculations are also necessary to compute the index that will be sent to the decoder. The larger the components of the input spectrum S'(k), the more bits will be required to encode the index of its nearest neighbour in the lattice codebook. Hence, to remain within a pre-defined bit-budget, a gain-shape approach has to be used, where the input spectrum is first scaled down by the AVQ gain, then each 8-dimensional block of spectrum coefficients is quantized in the lattice and finally scaled up again to produce the quantized spectrum.

5.2.3.1.6.9.2.1	AVQ gain estimation

Prior to the quantization (nearest neighbour search and indexation of the nearest neighbour), the input spectrum has to be scaled down to ensure that the total bit consumption will remain within the available bit-budget.

A first estimation of the total bit-budget nbits without scaling (i.e. with an AVQ gain equals to 1) is performed:

nbits=jRj	(545)

where Rj is a first estimate of the bit budget to encode the sub-band j given by:

Rj=5log2Ej2	(546)

with Ei being the energy (with a lower limit set to 2) of each sub-band S'(8j):

Ej=max2,i=07S'(8j+i)2	(547)

This gain estimation is performed in an iterative procedure described below.

Let NB_BITS be the number of bits available for the quantization process and NB_SBANDS the number of 8-dimensional sub-bands to be quantized:

| Initialization: fac = 128, offset = 0, nbitsmax = 0.95 (NB_BITS – NB_SBANDS) for i = 1:10 offset = offset + fac nbits=j=1NB_SBANDSmax0,Rj−offset if nbits ≤ nbitsmax, then offset = offset – fac fac = fac / 2 |  |
| --- | --- |

After the 10th iteration, the AVQ gain is equal to 10exp0.1offset⋅log102 and is used to obtain the scaled spectrum Snorm'(k):

Snorm'k=S'k/10exp0.1⋅offset⋅log102	(548)

5.2.3.1.6.9.2.2	Nearest neighbour search

The search of the nearest neighbour in the lattice RE8 is equivalent to searching for the nearest neighbour in the lattice 2D8 and for the nearest neighbour in the lattice 2D8+(1,1,1,1,1,1,1,1), and finally selecting among those two lattice points the closest to Snorm'(8j)) as its quantized version S'(8j).

Based on the definition of RE8, the following fast algorithm is used to search the nearest neighbour of an 8-dimensional sub-band Snorm'(8j) among all lattice points in RE8:

| Search of the nearest neighbour y1j in 2D8 of Snorm'(8j): Compute zj=0.5⋅Snorm'(8j). Round each component of zj to the nearest integer to generate zj'. Compute y1j=2zj'. Calculate the sum S of the 8 components of y1j. if S is not an integer multiple of 4, then modify its Ith component as follows: y1jI=y1jI− 2,if zjI–y1jI<0,y1jI + 2,otherwise. where I=argmaxzj(i)−y1j(i) Search of the nearest neighbour y2j in 2D8+(1,1,1,1,1,1,1,1) of Snorm'(8j): Compute zj=0.5⋅Snorm'(8j)−1.0 where 1.0 denotes an 8-dimensional vector with all ones. Round each component of zj to the nearest integer to generate zj'. Compute y2j=2zj'. Calculate the sum S of the 8 components of y2j. if S is not an integer multiple of 4 then modify its Ith component as follows: y2jI=y2jI− 2,if zjI–y2jI<0,y2jI + 2,otherwise. where I=argmaxzj(i)−y2j(i) Compute y2j=y2j+1.0. Select between y1j and y2j as the closest point S'(8j) in RE8 to Snorm'(8j): S'(8j)=y1j,if e1j>e2j,y2j,otherwise. where e1j=Snorm'(8j)−y1j2 and e2j=Snorm'(8j)−y2j2. |  |
| --- | --- |

5.2.3.1.6.9.3	Indexation

The quantized scaled sub-band S'(8j) of Snorm'(8j)) is a point cj in a RE8 lattice codebook, an index for each cj  has to be computed and later inserted into the bitstream.

This index is actually composed of three parts:

- a codebook number nj;

- a vector index Ij, which uniquely identifies a lattice vector in a base codebook CAVQ;

- and if nj>4, an 8-dimensional Voronoi extension index Ijv that is used to extend the base codebook when the selected point in the lattice is not in a base codebook CAVQ.

The calculation of an index for a given point cj in the RE8 lattice is performed as follows:

First, it is verified whether cj is in a base codebook CAVQ by identifying its sphere and its leader:

- if cj is in a base codebook, the index used to encode cj is thus the codebook number nj plus the index Ij of the lattice point cj in Qnj.

Otherwise, the parameters of the Voronoi extension (see equation (549)) have to determined: the scaling factor Mv, the base codebook CAVQ (Q3 or Q4), the point zj in this base codebook, and the point vj in the Voronoi extension. Then, the index used to encode is composed of the codebook number nj (nj>4) plus the index Ij of the lattice point zj in the base codebook CAVQ (Q3 or Q4), and the index Ijv of vj. in the Voronoi extension.

5.2.3.1.6.9.3.1	Indexing a codebook number

As explained in subclause 5.2.3.1.6.9.1.2.3 – Codebook rates, the codebook index nj is unary encoded with nj bits except for nj=0 that is coded with one bit (see table 51).

5.2.3.1.6.9.3.2	Indexing of codevector in base codebook

The index Ij indicates the rank of codevector zj in j-th sub-band, i.e., the permutation to be applied to a specific leader to obtain zj. The index computation is done in several steps, as follows:

1)	The input codevector zj is decomposed into a sign vector s0 and an absolute vector y0 following a two-path procedure.

2)	The sign vector is encoded, the associated index bitssign(s0) and the number of non-zero components in zj are obtained. More details are given in subsequent subclauses.

3)	The absolute vector is encoded using a multi-level permutation-based index encoding method, and the associated index rank(y0) is obtained.

4)	The absolute vector index rank(y0) and the sign index bitssign(s0) are added together in order to obtain the input vector rank: rank(zj).

rank(zj)=rank(y0)⋅2signnb(s0)+bitssign(s0)	(550)

5)	Finally, the offset leadoffset(zj) is added to the rank. The index Ij is obtained by

Ij=leadoffset(zj)+rank(zj)	(551)

The indexing of codevector in base codebook is done in two steps. First the sign vector is encoded.

The number of bits required for encoding the sign vector elements is equal to the number of non-zero elements in the codevector. "1" represents a negative sign and "0" a positive sign. As lattice RE8 quantization is used, the sum of all the elements in a codevector is an integer multiple of 4. If there is any change of sign in the non-zero element, the sum may not be a multiple of 4 anymore, in that case, the last element sign in the sign vector will be omitted. For example, the sign vector of the input vector (–1, –1, 1, 1, 1, 1, –1, –1) in leader 1 (see table 52) has seven bits and its value is 0x1100001.

In the second step the absolute vector and its position vector is encoded

The encoding method for the absolute vector works as follows. The absolute vector is first decomposed into MLmax levels. The highest-level vector L0 is the original absolute vector. The n value for Ln is initialized to zero. Then:

1)	First the intermediate absolute value vector of L1 is obtained by removing the most frequent element as given in the decomposition order column of table 56 from the original absolute vector ofL0. Sequentially the remaining elements are built into a new absolute vector forL1; it has a position order related to the level L0 original absolute vector. All position valuesqi of the remainder elements are used to build a position vector (q0,q1,q2,...,qm1−1)ofL1.

The relationship between the original absolute vector ofL0and the new absolute vector ofL1is that: the original absolute vector ofL0is the upper-level vector of the new absolute vector ofL1, and the new absolute vector ofL1is  the lower-level vector of the original absolute vector ofL0.The relationship between any two neighbour level absolute vector  is the same. The detail relationship is described as following:

Figure 23: Example processing of first level forKa=20.

2)	Then the position vector (q0,q1,q2,...,qm1−1)of the new absolute vector ofL1 related to the original absolute vector ofL0 is indexed based on a permutation and combination function, the indexing result being called the middle index Imid,1. For the new absolute vector inL1, the position vector indexing is computed as follows:

Imid,1=Cm0m1−Cm0−q0m1+i=1i<m1Cm0−qi−1m1−i−Cm0−qim1−i	(552)

Ifinal=Ifinal⋅Cm0m1+Imid,1	(553)

where Ifinal is initialized to zero before the first step at the beginning of the procedure, m0 is the dimension of the original absolute vector ofL0, m1 is the dimension of the new absolute vector ofL1.

If there is more than one type of element in the new absolute vector, the new absolute vector, named the upper-level vector, will be encoded using the multi-level permutation-based index encoding method as following step:

3)	Increment the n value. At level Ln, 0<Ln<MLmax, the intermediate absolute value vector is obtained by removing the most frequent element as given in the decomposition order column of table 53 from the upper-level vector. Sequentially the remaining elements are built into a new absolute vector for the current level; it has a position order related to the level Ln−1 absolute vector. All position values of the remainder elements are used to build a position vector.

4)	The position vector of the current lower-level vector related to its upper-level vector is indexed based on a permutation and combination function, the indexing result being called the middle index Imid,n. For the absolute vector in the current lower level, the position vector indexing is computed as follows:

Imid,n=Cmn−1mn−Cmn−1−q0mn+i=1i<mnCmn−1−qi−1mn−i−Cmn−1−qimn−i	(554)

Ifinal=Ifinal⋅Cmn−1mn+Imid,n	(555)

The elements q0,q1,q2... are the element values in the Ln level position vector ranged from left to right according to their level, mn−1 is the dimension of the upper-level absolute vector, mn is the dimension of the current-level absolute vector, Cqm represents the permutation and combination formula Cqm=q!m!(p−q)!, q,m=1,2,3,4,8, and q>m. All the values forCqmcan be stored in a simple table in order to avoid calculation of factorials. The Ln−1 level final-index, Ifinal, is multiplied by the possible index value number, Cmn−1mn, in the current level and is added to the index, Imid,n, in the current level, to obtain the final index, Ifinal, of the current level.

5)	Repeat steps 3 and 4 until there is only one type of element left in the current absolute vector. The Ifinal for the lowest level is the rank of the absolute vector called rank(y0). Table 54  is a sample extracted from the 36 leader table case. The leaders are indexed by Ka. The decomposition order corresponds to the level order. The decomposition order column gives the order in which the element will be removed from the higher level. The last column gives the three class parameters, the first one is the number of sign bits, Sn, the second one is the number of decomposition levels and equals the number of element types in the leader, Vc, from the third one to the last one they represent the absolute vector dimension in each lower level except the highest level, m1,m2,m3 (note that the dimension for the highest level is eight, but is not listed in table 55).

Table 56: List of leaders in base codebooks Qnj with their decomposition order and set parameter of multi-level permutation-based encoding

| Ka | Leader | Decomposition order | Sn, Vc, m1,m2,m3 | Q0 | Q2 | Q3 | Q4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | {0,0,0,0,0,0,0,0} |  |  | X |  |  |  |
| 0 | {1,1,1,1,1,1,1,1} | {1} | {7,1} |  | X | X |  |
| 1 | {2,2,0,0,0,0,0,0} | {0,2} | {2,2,2} |  | X | X |  |
| 2 | {2,2,2,2,0,0,0,0} | {0,2} | {4,2,4} |  |  | X |  |
| 3 | {3,1,1,1,1,1,1,1} | {1,3} | {7,2,1} |  |  | X |  |
| 4 | {4,0,0,0,0,0,0,0} | {0,4} | {1,2,1} |  | X | X |  |
| 5 | {2,2,2,2,2,2,0,0} | {2,0} | {6,2,2} |  |  |  | X |
| 6 | {3,3,1,1,1,1,1,1} | {1,3} | {7,2,2} |  |  |  | X |
| 7 | {4,2,2,0,0,0,0,0} | {0,2,4} | {3,3,3,1} |  |  | X |  |
| 8 | {2,2,2,2,2,2,2,2} | {2} | {8,1} |  |  |  | X |
| 9 | {3,3,3,1,1,1,1,1} | {1,3} | {7,2,3} |  |  |  | X |
| 10 | {4,2,2,2,2,0,0,0} | {2,0,4} | {5,3,4,1} |  |  |  | X |
| 11 | {4,4,0,0,0,0,0,0} | {0,4} | {2,2,2} |  |  | X |  |
| 12 | {5,1,1,1,1,1,1,1} | {1,5} | {7,2,1} |  |  |  | X |
| 13 | {3,3,3,3,1,1,1,1} | {1,3} | {7,2,4} |  |  |  | X |
| 14 | {4,2,2,2,2,2,2,0} | {2,0,4} | {7,3,2,1} |  |  |  | X |
| 15 | {4,4,2,2,0,0,0,0} | {0,2,4} | {4,3,4,2} |  |  |  | X |
| 16 | {5,3,1,1,1,1,1,1} | {1,3,5} | {7,3,2,1} |  |  |  | X |
| 17 | { 6,2,0,0,0,0,0,0} | {0,2,6} | {2,3,2,1} |  |  | X |  |
| 18 | { 4,4,4,0,0,0,0,0} | {0,4} | {3,2,3} |  |  |  | X |
| 19 | { 6,2,2,2,0,0,0,0} | {0,2,6} | {4,3,4,1} |  |  |  | X |
| 20 | { 6,4,2,0,0,0,0,0} | {0,2,4,6} | {3,4,3,2,1} |  |  |  | X |
| 21 | { 7,1,1,1,1,1,1,1} | {1,7} | {7,2,1} |  |  |  | X |
| 22 | { 8,0,0,0,0,0,0,0} | {0,8} | {1,2,1} |  |  |  | X |
| 23 | {6,6,0,0,0,0,0,0} | {0,6} | {2,2,2} |  |  |  | X |
| 24 | {8,2,2,0,0,0,0,0} | {0,2,8} | {3,3,3,1} |  |  |  | X |
| 25 | {8,4,0,0,0,0,0,0} | {0,4, 8} | {2,3,2,1} |  |  |  | X |
| 26 | {9,1,1,1,1,1,1,1} | {1,9} | {7,2,1} |  |  |  | X |
| 27 | {10,2,0,0,0,0,0,0} | {0,2,10} | {2,3,2,1} |  |  |  | X |
| 28 | {8,8,0,0,0,0,0,0} | {0,8} | {2,2,2} |  |  |  | X |
| 29 | {10,6,0,0,0,0,0,0} | {0,6,10} | {2,3,2,1} |  |  |  | X |
| 30 | {12,0,0,0,0,0,0,0} | {0,12} | {1,2,1} |  |  |  | X |
| 31 | {12,4,0,0,0,0,0,0} | {0,4,12} | {2,3,2,1} |  |  |  | X |
| 32 | {10,10,0,0,0,0,0,0} | {0,10} | {2,2,2} |  |  |  | X |
| 33 | {14,2,0,0,0,0,0,0} | {0,2,14} | {2,3,2,1} |  |  |  | X |
| 34 | {12,8,0,0,0,0,0,0} | {0,8,12} | {2,3,2,1} |  |  |  | X |
| 35 | {16,0,0,0,0,0,0,0} | {0,16} | {1,2,1} |  |  |  | X |

The last value of the decomposition order for the leader Ka=20 is stored separately because this leader is the only one with 4 different values, the second dimension of the decomposition order being thus reduced from 4 to 3.

Figure 24 gives an encoding example for the leader Ka=20.

Figure 32: Example processing for .

For example, in case the input vector is {0,–2,0,0,4,0,6,0}, the absolute input vector will be {0,2,0,0,4,0,6,0}, its associated leader can be found for Ka=20Ka. The set of decomposition order is {0,2,4,6}. For the highest level L0, element "0" is removed first from the absolute vector. The first level L1 absolute vector is {2,4,6}, its position vector is {1,4,6}. The second element which will be removed is "2", the second level L2 absolute vector is {4,6}, its position vector is {1,2}. The third element which will be removed is "4", the third level L3 absolute vector is {6}, its position vector is {1}.

The absolute vectors that have only two different values, out of which the most frequent is zero, are treated separately in a less complex procedure combining the encoding of the position vector with the sign encoding. These vectors have generally higher probability of occurrence. Example of such vectors are those derived for instance from the leaders: (2,2,0,0,0,0,0,0), (2,2,2,2,0,0,0,0). For these vectors there is a single level for the creation of the index and the first level remaining elements are the non-null components which are the significant elements for the sign encoding. The determination of the remaining elements and the creation of the sign index can be done thus in a single loop.

5.2.3.1.6.9.4	Voronoi extension determination and indexing

If the nearest neighbour cj is not in the base codebook, then the Voronoi extension has to be determined through the following steps.

(a)	Set the Voronoi extension order rjv=1 and the scaling factor Mjv=2rjv.

(b)	Compute the Voronoi index Ijv of the lattice point cj that depends on the extension order rjv and the scaling factor Mjv. The Voronoi index is computed via component-wise modulo operations such that Ijv depends only on the relative position of cj in a scaled and translated Voronoi region:

Ijv=modMjv(cjG−1)	(556)

where G is the RE8 generator matrix. Hence, the Voronoi index Ijv is a vector of integers with each component in [0,Mjv−1].

(c)	Compute the Voronoi codevector vj from the Voronoi index Ijv. The Voronoi codevector is obtained as

vj=y1j−Mjv⋅y2j	(557)

where y2j is the nearest neighbour of y2j in infinite RE8 (see subclause 5.2.3.1.6.9.2.2for search details) and y1j and y2j are defined as

y1j=Ijv4000000022000000202000002002000020002000200002002000002011111111	(558)

and

y2j=y1j−[2,0,0,0,0,0,0,0]/Mjv	(559)

(d)	Compute the difference vector wj=cj−vj. This difference vector wj always belongs to the scaled lattice Mjv⋅RE8. Compute zj=wj/Mjv, i.e. apply the inverse scaling to the difference vector wj. The codevector zj belongs to the lattice RE8 since wj belongs to Mjv⋅RE8 lattice.

(e)	Verify whether zj is in the base codebook CAVQ (i.e. in Q3 or Q4).

If zj is not in C, increment the extension order rjv by 1, multiply the scaling factor Mjv by 2, and go back to sub-step (b).

Otherwise, if zj is in C, then the Voronoi extension order rjv has been found and the scaling factor Mjv=2rjv is sufficiently large to encode the index of cj.

5.2.3.1.6.9.3	Insertion of AVQ parameters into the bitstream

The parameters of the AVQ in each sub-band j consist of the codebook number nj, the vector index in base codebook Ij and the 8-dimensional Voronoi index Ijv. The codebook numbers nj are in the set of integers {0, 2, 3, 4, 5, 6, 7, 8} and the size of its unary code representation is nj bits with the exception of Q0 that requires 1 bit and a possible overflow in the last AVQ coded sub-band. The size of each index Ij and Ijv is given by 4nj bits and 8rjv bits, respectively.

The AVQ parameters nj, Ij, Ijv, are written sequentially in groups corresponding to the same sub-band into the corresponding bitstream as

[(n0I0I0v)(n1I1I1v)...].	(560)

Note that if the lattice point in the block j is in the base codebook C, the Voronoi extension is not searched and consequently the index Ijv is not written into the bitstream in this group.

The actual bit-budget needed to encode AVQ parameters in current frame varies from sub-frame to sub-frame. The difference of bits between the allocated bits and actually spent bits are unused bits that can be employed in the subsequent sub-frame or high-rate higher band coding.

##### 5.2.3.1.7	Gain quantization

5.2.3.1.7.1	Memory-less quantization of the gains

The adaptive codebook gain (pitch gain) and the algebraic codebook gain are quantized jointly in each subframe, using a 5-bit vector quantizer. While the adaptive codebook gain is quantized directly, the algebraic codebook gain is quantized indirectly, using a predicted energy of algebraic codevector. Note that, in this case, the prediction does not use any past information which limits the effect of frame-erasure propagation.

First, energy of residual signal in dB is calculated in each subframe as

Eri=10log164i=063r2i, for i=0, 1, 2, 3,	(559)

where idenotes the subframe and rnis the residual signal, defined in subclause 5.2.3.1.1. Then, average residual signal energy is calculated for the whole frame as

Er=0.25i=03Eri	(560)

which is further modified by subtracting an estimate of the adaptive codebook contribution. That is

Ei=Er−0.5Cnorm0+Cnorm1	(561)

whereCnorm0 and Cnorm1, are as defined in subclause 5.1.10.4, are the normalized correlations of the first and the second half-frames, respectively. The result of equation (562), Ei, serves as a prediction of the algebraic codevector energy and is quantized with 3 bits once per frame. The quantized value of the predicted algebraic codevector energy is defined as

kind=min7k=0Ei−EbookkEi=Ebookkind	(563)

where Ebookk, k=0,…,2nis the n-bit codebook for the predicted algebraic codevector energy and kindis the index minimizing the criterion above. The bit allocation n is bit-rate and mode dependant and is given in Table 57

Table 47: Predictor energy codebook bit allocation

| Rate (kbps) | VC | GC | TC | IC/UC |
| --- | --- | --- | --- | --- |
| 7.2 | n.a. | n.a. | 4 | n.a. |
| 8 | n.a. | n.a. | 4 | n.a. |
| 9.6 | 3 | 3 | n.a. | n.a. |
| 13.2 | 5 | 4 | 4 | n.a. |
| 16.4 | 3 | 3 | n.a. | 3 |
| 24.4 | 3 | 3 | n.a. | 3 |
| 32 | n.a. | 5 | 5 | 5 |
| 64 | n.a. | 5 | 5 | 5 |

Now, let Ecdenote the algebraic codebook excitation energy in dB in a given subframe, which is given by

Ec=10log164n=063c2n	(564)

In the equation above, cn is the filtered algebraic codevector, found in subclause 5.2.3.1.5.

Using the predicted algebraic codevector energy and the calculated algebraic codebook excitation energy, we may estimate the algebraic codebook gain as

gc'=100.05Ei−Ec	(565)

A correction factor between the true algebraic codebook gain, gc, and the estimated one, gc', is given by

γ=gcgc'	(566)

The pitch gain,gp, and correction factor γare jointly vector-quantized using a n-bit codebook, where n is dependent on the bit-rate and coding mode as shown in Table 48

Table 49: Gain codebook bit allocation per subframe

| Rate (kbps) | VC | GC | UC/IC |
| --- | --- | --- | --- |
| 7.2 | 7/6/6/6 | 6/6/6/6 | n.a. |
| 8 | 8/7/6/6 | 8/7/6/6 | n.a. |
| 9.6 | 5/5/5/5 | 5/5/5/5 | n.a. |
| 13.2 | 6/6/6/6 | 6/6/6/6 | n.a. |
| 16.4 | 7/7/7/7/7 | 7/7/7/7/7 | 6/6/6/6/6 |
| 24.4 | 7/7/7/7/7 | 7/7/7/7/7 | 6/6/6/6/6 |
| 32 | 6/6/6/6/6 | 6/6/6/6/6 | 6/6/6/6/6 |
| 64 | 12/12/12/12/12 | 12/12/12/12/12 | 6/6/6/6/6 |

The gain codebook search is performed by minimizing a mean-squared weighted error between the original and the reconstructed signal, which is given by

E=xTx+gp2yTy+gc2zTz−2gpxTy−2gcxTz+2gpgcyTz	(567)

where xis the target vector, yis the filtered adaptive codevector, and zis the filtered algebraic codevector. The quantized value of the pitch gain is denoted asgpand the quantized value of the algebraic codebook gain is denoted as gc=γ gc' , where γ is the quantized value of the factor γ.

Furthermore, if pitch gain clipping is detected (as described in subclause 5.2.3.1.4.2), the last 13 entries in the codebook are skipped in the quantization procedure since the pitch gain in these entries is higher than 1.

5.2.3.1.7.2	Memory-less joint gain coding at lowest bit-rates

For the lowest bitrates of 7.2 and 8.0 kbps, slightly different memory-less joint gain coding scheme is used. This is due to the fact that there are not enough bits to cover the dynamic range of the target vector for algebraic search.

In the first subframe of the current frame, the estimated (predicted) gain of algebraic codebook is given by

gc0[0]=10a0+a1CT−log10(Ec)	(568)

where CT is a signal classification parameter (the coding mode), selected for the current frame in the pre-processing part, and Ec is the energy of the filtered algebraic codevector, calculated in equation (569). The inner term inside the logarithm corresponds to the gain of innovation vector. The constants a0 and a1 are found by means of MSE minimization on a large signal database. The only parameter in the equation above is the coding mode CT which is constant for all subframes of the current frame. The superscript [0] denotes the first subframe of the current frame. The estimation process for the first subframe is schematically depicted in the figure below.

Figure 33: Schematic description of the calculation process of algebraic gain in the first subframe

All subframes following the first subframes use slightly different estimation scheme. The difference is in the fact that in these subframes, the quantized gains of both the adaptive and the algebraic codebook from previous subframe(s) are used as auxiliary estimation parameters to increase the efficiency. The estimated value of the algebraic codebook gain in kth subframe, k>0 is given by

gc0[k]=10b0+b1CT+i=1kbi+1log10(gc[i−1])+i=1kbk+1+igp[i−1]	(570)

where k=1,2,3. Note, that the terms in the first and in the second sum of the exponent, there are quantized gains of algebraic and adaptive excitation of previous subframes, respectively. Note that the term including the gain of innovation vector log10(Ec) is not subtracted. The reason is in the use of the quantized values of past algebraic codebook gains which are already close enough to the optimal gain and thus it is not necessary to subtract this gain again. The estimation constants b0,…,b2k+1 are found again through MSE minimization on a large signal database. The gain estimation process for the second and the following subframes is schematically depicted in the figure below.

Figure 34: Schematic description of the calculation process of algebraic gain in the following subframes

The gain quantization is done both at the encoder and at the decoder by searching the gain codebook and evaluating the MMSE between the target signal and the filtered adaptive codeword. In each subframe, the codebook is searched completely, i.e. for q=0,..,Q-1 where Q is the number of codebook entries. It is possible to limit the searching range in case ĝp is mandated to lie below certain threshold. To allow reducing the search range, the codebook entries are sorted in ascending order according to the value of ĝp.

The gain quantization is performed by calculating the following MMSE criterion for each codebook entry

E=c0gp2+c1gp+c2[γgc0]2+c3γgc0+c4gpγgc0+c5	(571)

where the constants c0, c1, c2, c3, c4 and c5 are calculated as

c0=yTy, c1=xTy, c2=zTz, c3=xTz, c4=yTz, c5=xTx	(572)

in which x(i) is the target signal, y(i) is the filtered adaptive excitation signal and z(i) is the filtered algebraic excitation signal. The codevector leading to the lowest energy is chosen as the winning codevector and its entries correspond to the quantized values of gp and .

Before the gain quantization process it is assumed that both the filtered adaptive and innovation codewords are already known. The gain quantization at the encoder is performed by searching the designed gain codebook in the MMSE sense. Each entry in the gain codebook consists of two values: the quantized gain of the adaptive part and the correction factor γ for the algebraic part of the excitation. The estimation of the algebraic gain excitation is done beforehand and the resulting gc0 is used to multiply the correction factor selected from the codebook. In each subframe the gain codebook is searched completely, i.e. for q=0,..,Q-1. It is possible to limit the search range if the quantized gain of the adaptive part of the excitation is mandated to be below certain threshold. To allow for reducing the search range, the codebook entries are sorted in ascending order according to the value of gp. The gain quantization process is schematically depicted in the figure below.

Figure 35: Schematic diagram of the gain quantization process in the encoder

The gain quantization is performed by minimizing the energy of the error signal e(i) The error energy is given by

E=eTe=(x−gpy−gcz)T(x−gpy−gcz)	(573)

By replacing gc by γgc0 we obtain

E=c5+gp2c0−2gpc1+γ2gc02c2−2γgc0c3+2gpγgc0c4	(574)

The constants c0, c1, c2, c3, c4 and c5 and the estimated gain gc0 are computed before the search of the gain codebook. The error energy E is calculated for each codebook entry. The codevector [gp;γ] leading to the lowest error energy is selected as the winning codevector and its entries correspond to the quantized values of gp and . The quantized value of the fixed codebook gain is then calculated as

gc=gc0.γ	(575)

In the decoder, the received index is used to retrieve the values of the quantized gain of the adaptive excitation and the quantized correction factor of the estimated gain of the algebraic excitation. The estimated gain for the algebraic part of the excitation is done in the same way as in the encoder.

5.2.3.1.7.3	Scalar gain coding at highest bit-rates

At the bit-rate of 64kbps and at the last subframe of TC7 and TC165 (see later in subclause 5.2.3.2.2), the adaptive codebook gain (pitch gain) and the algebraic codebook gain are quantized using a scalar quantizers. The adaptive codebook gain is quantized using a uniform scalar quantizer according to MMSE criterion in the range between [0; 1.22]. In contrast the quantized algebraic codebook gain is obtained as a product of a correction factor γ and the estimated algebraic codebook gain gc', see equation 566, where the correction factor is quantized in log domain in the range between [0.02; 5.0].

At 64 kbps, both the adaptive codebook gain and the algebraic codebook gain are quantized by means of 6 bits each. In the last subframe of TC configurations TC7 and TC165, they are quantized by means of 6-8 bits depending on the bit-rate.

- 5.2.3.1.8	Update of filter memories

An update of the states of the synthesis and weighting filters is needed in order to compute the target signal in the next subframe.

After the two gains have been quantized, the excitation signal,u'n, in the present subframe is found by

u'n=gp vn+gc cn, n=0,…,63	(576)

where gpandgcare the quantized adaptive and algebraic codebook gains, respectively, vnis the adaptive codevector (interpolated, low-pass filtered past excitation), and cnis the algebraic codevector (including pre-filtering). The states of the filters can be updated by filtering the signal rn−u'n (difference between the residual signal and the excitation signal) through the filters 1/Azand Az/γ1Hde−emphzand saving the states of the filters. This would require 3 stages of filtering. A simpler approach, which requires only one filtering, is as follows. The local synthesis signal (without excitation post-processing) from layer 1, s1nis computed by filtering the excitation signal through1/Az. The output of the filter due to the input rn−u'nis equivalent tox1n=sn−s1n. So, the states of the synthesis filter1/Azare given by x1n, n=48,…,63.

The updating of the states of the filterAz/γ1Hde−emphzcan be done by filtering the error signal x1nthrough this filter to find the perceptually weighted error x1,wn. However, the signal x1,wncan be equivalently found by

x1,wn=xn−gp yn−gc zn	(577)

where xnis the adaptive codebook search target signal, ynis the filtered adaptive codebook vector, and znis the filtered algebraic codebook vector. Since the signals xn, yn, and znare available, the states of the weighting filter are updated by computing x1,wnas in equation 578 for  n=48,…,63. This saves two stages of filtering.

#### 5.2.3.2	Excitation coding in TC mode

The principle of excitation coding in TC mode is shown on a schematic diagram in Figure 36. The individual blocks and operations are described in detail in the following clauses.

##### 5.2.3.2.1	Glottal pulse codebook search

The TC mode improves the robustness of the codec to frame erasures. It also encodes frames with an outdated past excitation buffer, e.g. after switching from HQ core frame.

The TC mode in the current frame is selected based on the classification algorithm described in subclause 5.1.13. The increased robustness, or the excitation building when the past excitation is outdated, is achieved by replacing the adaptive codebook (inter-frame long-term prediction) with a codebook of glottal impulse shapes (glottal-shape codebook) [19], which is independent from past excitation. The glottal-shape codebook consists of quantized normalized shapes of the truncated glottal impulses placed at specific positions. The codebook search consists of both the selection of the best shape and the best position.

To select the best codevector, the mean-squared error between the target signal, xn (the same target signal as used for the adaptive codebook search described in subclause 5.2.3.1.2), and the contribution signal, yn, is minimized for all candidate glottal-shape codevectors. The glottal-shape codebook search has been designed in a similar way as the algebraic codebook search, described in subclause 5.2.3.1.5.9. In this approach, each glottal shape is represented as an impulse response of a shaping filterGz. This impulse response can be integrated in the impulse response of the weighted synthesis filter WzHzprior to the search of the optimum impulse position. The searched codevectors can then be represented by vectors containing only one non-zero element corresponding to candidate impulse positions, and they can be searched very efficiently. Once selected, the position codevector is convolved with the impulse response of the shaping filter. This procedure needs to be repeated for all the candidate shapes and the best shape-position combination will form the excitation signal.

Figure 37: Schematic diagram of the excitation coding in TC mode

In the following, all vectors are supposed to be column vectors. Let pk' be a position codevector with one non-zero element at a positionk', and qk' the corresponding glottal-shape codevector with index k'representing the centre of the glottal shape. Index k'is chosen from the range [0, 63], where 64 is the subframe length. Note that, due to the non-causal nature of the shaping filter, its impulse response is truncated for positions in the beginning and at the end of the subframe. The glottal shape codevector qk'can be expressed in a matrix form as qk'=Gpk', where Gis a Toeplitz matrix representing the glottal impulse shape. Similarly to the algebraic codebook search, we can write

ℑk'=xTy2yTy=xTHqk'2qk'THTHqk'=xTHGpk'2pk'TGTHTHGpk'=xTZpk'2pk'TZTZpk'=dgTpk'2pk'TΦgpk'	(579)

where His a lower triangular Toeplitz convolution matrix of the weighted synthesis filter. The rows of a convolution matrix ZTcorrespond to the filtered shifted version of the glottal impulse shape or its truncated representation.

Because of the fact that the position codevector pk'has only one non-zero sample, the computation of the criterion (580) is very simple and can be expressed as

ℑk'=dgk'2Φgk',k'	(581)

As it can be seen from criterion (582), only the diagonal of the correlation matrix Φgfrom criterion (583) needs to be computed.

The codebook consists of 8 prototype glottal impulse shapes of length L=17samples placed at all subframe positions. Note that, since Lis shorter than the subframe length, the remaining samples in the subframe are set to zero.

In general, the coding efficiency of the glottal-shape codebook is lower than the efficiency of the long-term prediction, and more bits are generally needed to assure good synthesized speech quality.

However, the glottal-shape codebook does not need to be used in all subframes. First, there is no reason to use this codebook in subframes that do not contain any significant glottal impulse in the residual signal. Second, the glottal-shape codebook search is important only in the first pitch period in a frame. The following pitch periods can be encoded using the more efficient standard adaptive codebook search as it does not use the excitation of the past frame anymore. To satisfy the constant bit-rate requirement, the glottal-shape codebook is used in the EVS codec only in one of the four subframes in a frame. This leads to a highly structured coding mode where the bit allocation is dependent on the position of the first glottal impulse and the pitch period. The subframe where the glottal-shape codebook is used is chosen as the subframe with the maximum sample in the residual signal in the range 0,Top+2, where Top is the open-loop pitch period estimated over the first half of the frame. The other subframes are processed as described in subclause 5.2.3.2.2.

Criterion (584) is typically used in the algebraic codebook search by pre-computing the backward filtered target vector dg and the correlation matrix Φg. Given the non-causal nature of the filter Gz, the matrix Zis not triangular and Toeplitz anymore, and this approach cannot be efficiently applied for the first L1/2=8positions in the glottal-shape codebook search.

Let zk' be the k'+1th row of the matrix ZT, where ZT=GTHTis computed in two steps to minimize the computational complexity. In the first step, the first L1/2+1rows of this matrix ZTare calculated that correspond to the positions k'from the range 0,L1/2. In the second step, the criterion (585) is used in a similar way as in the algebraic codebook search for the remaining part of ZT (the last 64−L1/2−1rows of the matrix ZT).

In the first step, the convolution between the glottal-shape codebook entry for position k'=0and the impulse response hnis first computed using

z0n=i=0ngn−ihi for n=0,…,63	(586)

where we take advantage of the fact that the filter Gzhas only L1/2+1non-zero coefficients.

Next, the convolution z1nbetween the glottal-shape codebook entry for the position k'=1and the impulse response hnis computed, reusing the values of z0n. For the following rows, the recursion is reused, resulting in

zk'0=g−k'h0,zk'n=zk'−1n−1+g−k'hn for n=1,…,63	(587)

The recursion (588) is repeated for allk'≤L1/2.

Now, the criterion (589) can be computed for all positions k'from the range 0,L1/2in the form

ℑk'=i=063zk'i.x1i2i=063zk'i.zk'i	(590)

In the second step, we take advantage of the fact that rows L1/2+1,…,63of the matrix ZTare built using the coefficients of the convolution   that are already computed as described by recursion (591) for k'=L1/2. That is, each row corresponds to the previous row shifted to the right by 1 with a zero added at the beginning

zk'0=0,zk'n=zk'−1n−1 for n=1,…,63	(592)

and this is repeated for k' from the range L1/2+1,…,63.

Next, the target vector dgand the diagonal of the matrix Φgneed to be computed. First, we evaluate the numerator and the denominator of the criterion (593) for the last position k'=63

dg63=i=0L1/2x63−L1/2+i zL1/2i	(594)

and

Φg63,63=i=0L1/2zL1/2i zL1/2i	(595)

For the remaining positions, the numerator is computed using equation (596), but with the summation index changed. In the computation of the denominator, some of the previously computed values can be reused. For example, for the position k'=62, the denominator of criterion (597) is computed using

Φg62,62=Φg63,63+zL1/2L1/2+1 zL1/2L1/2+1	(598)

Similarly, we can continue to compute the numerator and the denominator of criterion (599) for all positions k'>L1/2.

The search continues using the previously described procedure for all other glottal impulse shapes and the codevector corresponding to the best combination of glottal-shape and position is selected. To maintain the complexity low, the computation described above is further reduced by limiting the position search to ±4 samples around the maximum absolute value of the residual signal.

The last parameter to be determined is the gain of the glottal-shape codebook excitation. The gain is quantized in two steps. First, a roughly quantized gain of the glottal-shape codevector, gm, is found. Then, after both the first-stage contribution (glottal-shape codevector) and the second-stage contribution (algebraic codevector) of the excitation signal are found, the gain gpof the first-stage contribution signal is jointly quantized with the second-stage contribution gain, gc. This is done using the memory-less gain vector quantization, as described in subclause 5.2.3.1.7.1. The found glottal shape codevector qk'n is thus the position codevector pk'n filtered through the shaping filter Gzthat represents the best found glottal shape. When scaling the glottal-shape codevector qk'nwith the signed quantized gaingm, we finally obtain the first stage excitation codevector, vn.

The glottal-shape gain gmis quantized using a quantization table as follows. First, an unquantized gain in the current glottal-shape subframe is found as

gm=n=0N−1xn.yn0.01+n=0N−1yn.yn	(600)

where N is the subframe length, xnis the target signal and ynis the glottal-shape codevector qk'nfiltered through the weighted synthesis filterWz/Az. Further, the sign of the glottal-shape gain is set to 0 if gm<0and 1 otherwise, and written to the bitstream. Finally, the glottal-shape gain quantization index Igmis found as the maximum value of Igmthat satisfies tIgm<gm, where tis the glottal-shape gain quantization table of dimension 8. The signed quantized glottal-shape gaingmis thus found as gm=tIgmand its value is quantized using 4 bits (1 bit for sign, 3 bits for the value).

It should be noted that the closed-loop pitch period, T0, does not need to be transmitted anymore in a subframe which uses the glottal-shape codebook search with the exception of subframes containing more than one glottal impulse, i.e., whenk'+T0<64+L1/2. There are situations where the pitch period of the input signal is shorter than the subframe length and, in this case, we have to transmit its value. Given the pitch period length limitations and the subframe length, a subframe cannot contain more than two impulses. In the situation that the glottal-shape codevector contains two impulses, an adaptive codebook search is used in a part of the subframe. The first T0samples of the glottal-shape codevector qk'nare built using the glottal-shape codebook search and then the other samples in the subframe are built using the adaptive search as shown in Figure 38.

Figure 39: Glottal-shape codevector with two impulses construction

The described procedure is used even if the second glottal impulse appears in one of the first L1/2positions of the next subframe. In this situation, only a few samples (less than L1/2+1) of the glottal shape are used at the end of the current subframe. This approach has a limitation because the pitch period value transmitted in these situations is limited to T0<N, if it is bigger, it is not transmitted.

In order to enhance the coding performance, a low-pass filter is applied to the first stage excitation signal vn. In all subframes after the glottal-shape codebook subframe, the low-pass filtered first stage excitation is found as described in subclause 5.2.3.1.4.2.

##### 5.2.3.2.2	TC frame configurations

5.2.3.2.2.1	TC frame configurations at 12.8 kHz internal sampling

At bit-rates with 12.8 kHz internal sampling rate the glottal-shape codebook is used in one out of four subframes. The other subframes in a TC frame (not encoded with the use of the glottal-shape codebook) are processed as follows. If the subframe with glottal-shape codebook search is not the first subframe in the frame, the excitation signal in preceding subframes is encoded using the algebraic CELP codebook only, this means that the first stage contribution signal is zero. If the glottal-shape codebook subframe is not the last subframe in the frame, the following subframes are processed by the standard CELP coding (i.e., using the adaptive and the algebraic codebook search). Thus, the first stage excitation signal is the scaled glottal-shape codevector, the adaptive codevector or the zero codevector.

In order to further increase encoding efficiency and to optimize bit allocation, different processing is used in particular subframes of a TC frame dependent on the pitch period. When the first subframe is chosen as a TC subframe, the subframe with the 2nd glottal impulse in the LP residual signal is determined. This determination is based on the pitch period value and the following four situations then can appear. In the first situation, the 2nd glottal impulse is in the 1st subframe, and the 2nd, 3rd and 4th subframes are processed using the standard CELP coding (adaptive and algebraic codebook search). In the second situation, the 2nd glottal impulse is in the 2nd subframe, and the 2nd, 3rd and 4th subframes are processed using the standard CELP coding again. In the third case, the 2nd glottal impulse is in the 3rd subframe. The 2nd subframe is processed using algebraic codebook search only as there is no glottal impulse in the 2nd subframe of the LP residual signal to be searched for using the adaptive codebook. The 3rd and 4th subframes are processed using the standard CELP coding. In the last (fourth) case, the 2nd glottal impulse is in the 4th subframe (or in the next frame), the 2nd and 3rd subframes are processed using the algebraic codebook search only, and the 4th subframe is processed using the standard CELP coding. Table 50 shows all possible coding configurations in the EVS codec at 12.8 kHz internal sampling rate.

The TC configuration is transmitted in the bit-stream using a Huffman-style coding and its bit sequence is show in the Table 50 in the column bitstream.

Table 50: TC configurations used in the EVS codec at 12.8 kHz internal sampling rate

| Coding configuration | Bitstream | Positions of the first (and the second, if relevant) glottal impulse(s) in the frame | Type of codebook used (GS = glottal-shape, Ada = adaptive, Alg = algebraic) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st subfr. | 2nd subfr. | 3rd subfr. | 4th subfr. |
| TC1 | 1 |  | GS + Alg | Ada + Alg | Ada + Alg | Ada + Alg |
| TC2 | 0101 |  | GS + Alg | Ada + Alg | Ada + Alg | Ada + Alg |
| TC3 | 0100 |  | GS + Alg | Alg | Ada + Alg | Ada + Alg |
| TC4 | 011 |  | GS + Alg | Alg | Alg | Ada + Alg |
| TC5 | 001 |  | Alg | GS + Alg | Ada + Alg | Ada + Alg |
| TC6 | 0001 |  | Alg | Alg | GS + Alg | Ada + Alg |
| TC7 | 0000 |  | Alg | Alg | Alg | GS + Alg |

5.2.3.2.2.2	TC frame configurations at 16 kHz internal sampling

At bit-rates with 16 kHz internal sampling rate the glottal-shape codebook is used in one out of five subframes. If the subframe with glottal-shape codebook search is not the first subframe in the frame, the excitation signal in preceding subframes is encoded using the algebraic CELP codebook only. If the glottal-shape codebook subframe is not the last subframe in the frame, the following subframes are processed by the standard CELP coding.

As the bit-rates with 16 kHz internal sampling rate are with high bit-budget, the number of TC configurations is reduced compared to the 12.8 kHz internal sampling rate. Table 51 shows all possible coding configurations in the EVS codec at 16 kHz internal sampling rate

Table 51: TC configurations used in the EVS codec at 16 kHz internal sampling rate

| Coding configuration | Bitstream | Positions of the first glottal impulse in the frame | Type of codebook used (GS = glottal-shape, Ada = adaptive, Alg = algebraic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st subfr. | 2nd subfr. | 3rd subfr. | 4th subfr. | 5th subfr. |
| TC161 | 00 |  | GS + Alg | Ada + Alg | Ada + Alg | Ada + Alg | Ada + Alg |
| TC162 | 01 |  | Alg | GS + Alg | Ada + Alg | Ada + Alg | Ada + Alg |
| TC163 | 10 |  | Alg | Alg | GS + Alg | Ada + Alg | Ada + Alg |
| TC164 | 110 |  | Alg | Alg | Alg | GS + Alg | Ada + Alg |
| TC165 | 111 |  | Alg | Alg | Alg | Alg | GS + Alg |

5.2.3.2.2.3	Pitch period and gain coding in the TC mode

When using the TC, it is not necessary to transmit the pitch period for certain subframes. Further, it is not necessary to transmit both pitch gain, gp, and the algebraic codebook gain, gc, for subframes where there is no important glottal impulse, and only the algebraic codebook contribution is computed (the first stage excitation is the zero vector).

In subframes, where the glottal-shape, or adaptive, search is used, the first stage excitation gain (pitch gain), gp, and the second stage excitation gain (algebraic gain), gc, are quantized at bit-rates ≤ 32 kbps using the memory-less vector gain quantization described in subclause 5.2.3.1.7.1. At 64 kbps bit-rate, gains are scalar quantized as described in subclause 5.2.3.1.7.3. In glottal-shape subframes, the first stage gain, gp, is found in the same manner as described in subclause 5.2.3.1.4.2.

When only an algebraic gain is quantized in the current frame (the first stage excitation is the zero vector), the following scalar quantization process is used. First, an optimal algebraic gain in the current subframe is found as

gc=n=0N−1xn.zn0.01+n=0N−1zn.zn	(601)

where Nis the subframe length, xnis the target signal and znis the algebraic codevector cnfiltered through the weighted synthesis filterWz/Azwith the pre-filter Fz. The predictive algebraic energy calculated once per frame is employed as described in subclause 5.2.3.1.7. Further the algebraic codebook gain, gc', and the correction factor, γ, are given by equations (602) and (603), respectively. Finally, the correction factor quantization index, Iγ, is found as the maximum value of Iγthat satisfies

gc<tIγ+1+tIγ2.gc'	(604)

where tis the algebraic gain quantization table of dimension 8. The correction factor γ is quantized with 3 bits using the quantization tabletand the quantized algebraic gain is obtained by

gc=tIγ . gc'	(605)

The following is a list of all TC configurations corresponding to Table 52 and Table 53.

Configuration TC1

In this configuration, two first glottal impulses appear in the first subframe that is processed using the glottal-shape codebook search. This means that the pitch period value in the 1st subframe can have the maximum value less than the subframe length, i.e., 34<T0<N. Here, T0is the closed-loop pitch period and N the subframe length. With the ½ sample resolution it can be coded with 6 bits. The pitch periods in the next subframes are found using – depending on the bit-rate – a 5- or 6-bit delta search with a fractional resolution.

Configuration TC2

When configuration TC2 is used, the first subframe is processed using the glottal-shape codebook search. The pitch period is not needed and all following subframes are processed using the adaptive codebook search. Because we know that the 2nd subframe contains the second glottal impulse, the pitch period maximum value holds T0<2.N−1. This maximum value can be further reduced thanks to the knowledge of the glottal impulse position value k'. The pitch period value in the 2nd subframe is then coded using 7 bits with a fractional resolution in the whole range of minN−k',34,2.N−1−k'. In the 3rd and 4th subframes, a delta search using 6 bits is used with a fractional resolution.

Configuration TC3

When configuration TC3 is used, the first subframe is processed using the glottal-shape codebook search with no use of the pitch value again. But because the 2nd subframe of the LP residual signal contains no glottal impulse and the adaptive search is useless, the first stage contribution signal is replaced by zeros in the 2nd subframe. The adaptive codebook parameters (T0 and gp) are not transmitted in the 2nd subframe. The first stage contribution signal in the 3rd subframe is constructed using the adaptive codebook search with the pitch period maximum value 3.N−1−k'and the minimum value 2.N−1−k', thus only a 7-bit coding of the pitch value with fractional resolution in all range is needed. The 4th subframe is processed using the adaptive search with – depending on the bit-rate – a 5- or 6-bit delta search coding of the pitch period value.

In the 2nd subframe, only the algebraic codebook gain, gc, is transmitted. Consequently, only 3 bits are needed for gain quantization in this subframe as described at the beginning of this subclause.

Configuration TC4

When configuration TC4 is used, the first subframe is processed using the glottal-shape codebook search. Again, the pitch period does not need to be transmitted. But because the LP residual signal contains no glottal impulse in the 2nd and also in the 3rd subframe, the adaptive search is useless for both these subframes. Again, the first stage excitation signal in these subframes is replaced by zeros. The pitch period value is transmitted only in the 4th subframe by means of 7 bits and its minimum value is 3.N−k'.The maximum value of the pitch period is limited by the Tmax=231value only. It does not matter if the second glottal impulse will appear in the 4th subframe or not (the second glottal impulse can be present in the next frame if k'+Tmax≥N).

Note that the absolute value of the pitch period is necessary at the decoder for the frame concealment; therefore, it is transmitted also in the situation when the second glottal impulse appears in the next frame. When a frame mpreceding the TC frame m+1is missing, the correct knowledge of the pitch period value from the frames m−1 and m+1helps to reconstruct the missing part of the synthesis signal in the frame msuccessfully.

The algebraic codebook gain,gc, is quantized with 3 bits in the 2nd subframe and 3rd subframe.

Configuration TC5

When the first glottal impulse appears in the 2nd subframe, the pitch period is transmitted only for the 3rd and 4th subframe. 3rd subframe, the pitch value is coded using 9-bit absolute search while in the 4th subframe using – depending on the bit-rate – 5- or 6- bits delta search. In this case, only algebraic codebook parameters are transmitted in the 1st subframe (with the algebraic codebook gain, gc, quantized with 3 bits).

Configuration TC6

When the first glottal impulse appears in the 3rd subframe, the pitch period does not need to be transmitted for the TC technique. In this case, only algebraic codebook parameters are transmitted in the 1st and 2nd subframe with the algebraic codebook gain,gc, quantized with 3 bits in both subframes. Nevertheless, the pitch period is transmitted in the 4th subframe by means of 9 bit absolute search coding for the reason of better frame erasure concealment in the frame after the TC frame. Also, the pitch period is transmitted for the 3rd subframe by means of 5 bit absolute search coding although it is not usually necessary.

Configuration TC7

When the first glottal impulse appears in the 4th subframe, the pitch period value information is not usually used in this subframe. However, its value is necessary for the frame concealment at the decoder (this value is used for the missing frame reconstruction when the frame preceding or following the TC frame is missing) or in case of strong onsets at the frame-end and very short pitch period. Thus, the pitch value is transmitted only in the 4th subframe by means of 9-bit absolute search coding and only algebraic codebook parameters are transmitted in the first three subframes (the gain pitch, gp, is not essential). The algebraic codebook gain, gc, is quantized with 3 bits in the 1st, 2nd and 3rd subframes. The scalar gain quantization is employed only at the 4th subframe in this configuration to encode the gain pitch and the algebraic codebook gain.

Configuration TC161

In this configuration, one or two first glottal impulses appear in the first subframe that is processed using the glottal-shape codebook search. This means that the pitch period value in the 1st subframe can have the maximum value less than the subframe length N, i.e., 42<T0<N and it is coded with 6 bits. Then the pitch period in the 2nd subframe is found using 8-bit absolute search on the interval 42<T0<2∗N. Finally the pitch period in the 3rd subframe is coded using 10-bit absolute search and in the 4th and 5th subframe using 6-bit delta search.

The gain pitch and the algebraic codebook gain are coded in all subframes using 6-bit VQ at 32 kbps resp. 12-bit SQ at 64 kbps.

Configuration TC162

The first glottal impulse appears in the 2nd subframe and the pitch period is transmitted for the 3rd, 4th and 5th subframe. In the 3rd subframe, the pitch value is coded using 10-bit absolute search while in the 4th and 5th subframe using 6- bits delta search. The pitch period is transmitted also in the 2nd subframe by means of 6 bits and serves in case when two first glottal impulses appears in the second subframe.

The gain pitch and the algebraic codebook gain are coded in the 2nd, 3rd, 4th and 5th subframe using 6-bit VQ at 32 kbps resp. 12-bit SQ at 64 kbps. The algebraic codebook gain,gc, is quantized in the 1st subframe with 3 bits at 32 kbps resp. 6 bits at 64 kbps

Configuration TC163

The first glottal impulse appears in the 3rd subframe. In this case, only algebraic codebook parameters are transmitted in the 1st and 2nd subframe with the algebraic codebook gain,gc, quantized in both subframes with 3 bits at 32 kbps resp. 6 bits at 64 kbps. Then the pitch period is coded by means of 10-bit absolute search in the 3rd subframe and by means of 6-bit delta search in the 4th and 5th subframe.

The gain pitch and the algebraic codebook gain are coded in the 3rd, 4th and 5th subframe using 6-bit VQ at 32 kbps resp. 12-bit SQ at 64 kbps.

Configuration TC164

The first glottal impulse appears in the 4th subframe. In this case, only algebraic codebook parameters are transmitted in the 1st, 2nd and 3rd subframe with the algebraic codebook gain,gc, quantized in all these subframes with 3 bits at 32kbps resp. 6 bits at 64kbps. Then the pitch period is coded by means of 10-bit absolute search in the 4th subframe and by means of 6-bit delta search in the 5th subframe.

The gain pitch and the algebraic codebook gain are coded in the 4th and 5th subframe using 6-bit VQ at 32 kbps resp. 12-bit SQ at 64 kbps.

Configuration TC165

When the first glottal impulse appears in the 5th subframe, the pitch period value information is not usually used in this subframe. However, its value is necessary for the frame concealment at the decoder (this value is used for the missing frame reconstruction when the frame preceding or following the TC frame is missing) or in case of strong onsets at the frame-end and very short pitch period. Thus, the pitch value is transmitted only in the 5th subframe by means of 10-bit absolute search coding and only algebraic codebook parameters are transmitted in the first four subframes. The algebraic codebook gain, gc, is quantized in the 1st, 2nd and 3rd subframes with 3 bits at 32kbps and 6 bits at 64 kbps. The gain pitch and the algebraic codebook gain are coded only in 5th subframe using a scalar gain quantizer.

5.2.3.2.2.4	Update of filter memories

In TC mode, the memories of the synthesis and weighting filter are updated as described in subclause 5.2.3.1.8. Note that signals in equation (606) are the first stage excitation signal vn (i.e., the glottal-shape codevector, the low-pass filtered adaptive codevector, or the zero codevector) and the algebraic codevector cn (including pre-filtering).

#### 5.2.3.3	Excitation coding in UC mode at low rates

The principle of excitation coding in UC mode is shown in a schematic diagram in Figure 40. The individual operations are described in detail in the following clauses.

Figure 38: Schematic diagram of the excitation coding in UC mode

##### 5.2.3.3.1	Structure of the Gaussian codebook

In UC mode, a Gaussian codebook is used for representing the excitation. To simplify the search and reduce the codebook memory requirement, an efficient structure is used whereby the excitation codevector is derived by the addition of 2 signed vectors taken from a table containing 64 Gaussian vectors of dimension 64 (the subframe size). Let ti denote the ith 64-dimensional Gaussian vector in the table. Then, a codevector is constructed by

c=s1tp1+s2tp2	(607)

where s1and s2are the signs, equal to –1 or 1, and p1and p2are the indices of the Gaussian vectors from the table. In order to reduce the table memory, a shift-by-2 table is used, thus only 64 + 63 × 2 = 190 values are needed to represent the 64 vectors of dimension 64.

To encode the codebook index, one has to encode 2 signs, s1and s2, and two indices, p1and p2. The values of p1and p2are in the range 0,63, so they need 6 bits each, and the signs need 1 bit each. However, 1 bit can be saved since the order of the vectors viand vjis not important. For example, choosing v16as the first vector and v25as the second vector is equivalent to choosing v25as the first vector and v16as the second vector. Thus, similar to the case of encoding two pulses in a track, only one bit is needed for both signs. The ordering of the vector indices is such that the other sign information can be easily deduced. This gives a total of 13 bits. To better explain this procedure, let us assume that the two vectors have the indices p1and p2with sign indices σ1and σ2, respectively (σ=0if the sign is positive and σ=1if the sign is negative). The codevector index is given by

I=σ1+2×p1×6+p2	(608)

If p1≤p2then σ1=σ2; otherwise σ2 is different from σ1. Thus, when constructing the codeword (index of codevector), if the two signs are equal then the smaller index is assigned to p1and the larger index to p2, otherwise the larger index is assigned to p1and the smaller index to p2.

##### 5.2.3.3.2	Correction of the Gaussian codebook spectral tilt

In UC mode, the Gaussian codebook spectral tilt is corrected by a modification factor, which is encoded using 3 bits per subframe. First, the tilt of the target vector xnis computed as

etiltxx=i=163xixi−1i=163xi2	(609)

and the tilt of the filtered Gaussian codebook ynis computed as

etiltyy=i=1189yiyi−1i=1189yi2	(610)

The filtered Gaussian codebook, yn, is the initial Gaussian codebook, tn, convolved with the weighted filter, hn. Note that vector tnrepresents the whole codebook, i.e., n=0,…,189.

The spectral tilt modification factor is found by

δ=1−etiltyy . etiltxx2 . etiltyy + etiltxx	(611)

and the integer quantization index is found by

Iδ=16 δ	(612)

where the operator  .  returns the integer part of a floating point number. The integer quantization index is limited to [0, 7].

Finally, the quantized spectral tilt modification factor is used to adapt the tilt of the initial Gaussian codebook. That is

t'n=tn for n=0,t'n=tn−δ.tn−11+δ2 for n=1,…,189	(613)

where the quantized spectral tilt modification factor is found as

δ=Iδ16	(614)

In the following, the adapted Gaussian codebook t'n, n=0,…,189, is searched to obtain the best two codevectors and signs which form the final codevector, cn, of dimension 64. In the following, we assume tn←t'n.

##### 5.2.3.3.3	Search of the Gaussian codebook

The goal of the search procedure is to find the indices p1and p2of the two best random vectors and their corresponding signs, s1and s2. This is achieved by maximizing the following search criterion

Q=xTz2zTz=xTH c2zTz=dTc2zTz	(615)

where xnis the target vector and z=Hcis the filtered final codevector. Note that in the numerator of the search criterion, the dot product between xnand zn, n=0,…,63, is equivalent to the dot product between dnand cn, where d=HTxis the backward filtered target vector which is also the correlation between dnand the impulse response hn. The elements of the vector dnare found by

dn=xn∗h−n=i=n63xi hi−n for n=1,…,63	(616)

Since dnis independent of the codevector cn, it is computed only once, which simplifies the computation of the numerator for different codevectors.

After computing the vector dn, a predetermination process is used to identify Kout of the 64 random vectors in the random table, so that the search process is then confined to those Kvectors. The predetermination is performed by testing the numerator of the search criterion Qkfor the Kvectors which have the largest absolute dot product (or squared dot product) between dnand vin, i=0,…,63. That is, the dot productsχithat are given by

χi=n=063dn vin	(617)

are computed for all random vectors vinand the indices of the Kvectors which result in the Klargest values of χiare retained. These indices are stored in the index vector mi, i=0,…,K−1. To further simplify the search, the sign information corresponding to each predetermined vector is also preset. The sign corresponding to each predetermined vector is given by the sign of χifor that vector. These preset signs are stored in the sign vector si, i=0,…,K−1.

The codebook search is now confined to the pre-determined Kvectors with their corresponding signs. Here, the value K=8is used, thus the search is reduced to finding the best 2 vectors among 8 random vectors instead of finding them among 64 random vectors. This reduces the number of tested vector combinations from 64×652=2080to 8×92=36.

Once the most promising Kvectors and their corresponding signs are predetermined, the search proceeds with the selection of 2 vectors among those Kvectors which maximize the search criterion Q.

We first start by computing and storing the filtered vectors wj, j=0,…,K−1corresponding to the Kpredetermined vectors. This can be performed by convolving the predetermined vectors with the impulse response of the weighted synthesis filter, hn. The sign information is also included in the filtered vectors. That is

wjn=sji=0ntmji hn−i for n=0,…,63, j=0,…,K−1	(618)

We then compute the energy of each filtered pre-determined vector as

εj=wjTwj=n=063wj2n for j=0,…,K−1	(619)

and its dot product with the target vector

ρj=xTwj=n=063wjn xn for j=0,…,K−1	(620)

Note that ρjand εjcorrespond to the numerator and denominator of the search criterion due to each predetermined vector. The search proceeds now with the selection of 2 vectors among the Kpredetermined vectors by maximizing the search criterion Q. Note that the final codevector is given in equation (621).

The filtered codevector znis given by

z=Hc=s1Htp1+s2Htp2=wp1+wp2	(622)

Note that the predetermined signs are included in the filtered predetermined vectors wj. The search criterion in equation (623) can be expressed as

Q=xTz2zTz=xTwp1+xTwp22wp1+wp2Twp1+wp2=ρp1+ρp22εp1+εp2+2wp1Twp2	(624)

The vectors wjand the values ofρjand εjare computed before starting the codebook search. The search is performed in two nested loops for all possible positions p1and p2that maximize the search criterion Q. Only the dot products between the different vectors wjneed to be computed inside the loop.

At the end of the two nested loops, the optimum vector indices p1and p2will be known. The two indices and the corresponding signs are then encoded as described above. The gain of the final Gaussian codevector is computed based on a combination of waveform matching and energy matching. The gain is given by

gc=0.6gw+0.4ge	(625)

where gwis the gain that matches the waveforms of the vectors xnand znand is given bygw=xTz/zTzand geis the gain that matches the energies of the vectors xnand znand is given by ge=xTz/zTz . Here, xnis the target vector and znis the filtered codevector cn, n=0,…,63.

##### 5.2.3.3.4	Quantization of the Gaussian codevector gain

In UC mode, the adaptive codebook is not used and only the Gaussian codevector gain needs to be quantized. The Gaussian codevector gain in dB is given by

gcdB=20loggc	(626)

is uniformly quantized betweenΓminandΓmaxwith the step size given by

δ=Γmax−ΓminL	(627)

where Lis the number of quantization levels. The quantization index kis given by the integer part of

k=gcdB−Γminδ+0.5	(628)

Finally, the quantized gain in dB is given by

gcdB=k×δ+Γmin	(629)

and the quantized gain is given by

gc=10gcdB/20	(630)

In every subframe, 7 bits are used to quantize the gain. Thus, L=128and the quantization step is δ=1.71875dB with the quantization boundariesΓmin=−30andΓmax=190. The quantized gain, gc, is finally used to form the total excitation in the UC mode by multiplying each sample of the codevector, cn, bygc.

##### 5.2.3.3.5	Other parameters in UC mode

In UC mode, the SAD and noisiness parameters are encoded to modify the excitation vector in stationary inactive segments. The noisiness parameter is required for an anti-swirling technique used in the decoder for enhancing the background noise representation during inactive speech.

The noisiness parameter is defined as the ratio between low- and high-order LP residual variances:

ν=σe,22σe,162	(631)

where σe,22 and σe,162denote the LP residual variances for second-order and 16th-order LP filters, respectively. The LP residual variances are readily obtained as a by-product of the Levinson-Durbin procedure, described in subclause 5.1.9.4.

The noisiness parameter is normalized to the interval [0, 1] within which it is linearly quantized with 32 levels. That is

ν=μ.ν−132	(632)

where μis a normalization factor, which is different for WB and NB signals. For WB signals, μ=2, otherwise μ=0.5.

##### 5.2.3.3.6	Update of filter memories

In UC mode, the memories of the synthesis and weighting filter are updated as described in subclause 5.2.3.1.8. Note that the excitation component vnis missing in equation (633) for UC mode.

#### 5.2.3.4	Excitation coding in IC and UC modes at 9.6 kbps

At 9.6 kbps, the IC and UC modes are coded with a hybrid coding embedding two stages of innovative codebooks, the algebraic pulse codebook and a Gaussian noise-like excitation. Since the long term prediction gain is expected to be very low for such frames, the adaptive codebook is not used. The principle is depicted in figure 41.

Figure 39: Schematic diagram of the excitation coding in UC and IC modes at 9.6 kbps

##### 5.2.3.4.1	Algebraic codebook

5.2.3.4.1.1	Adaptive pre-filter

For UC mode, the adaptive pre-filter is performed similarly as in subclause 5.2.3.1.5.1. Additional the pre-filter Fz is amended with a phase scrambling filter as follows:

Fz=Fz⋅(0.7+z−11+0.7z−1)	(613)

For NB IC mode, the filter is designed as follows:

Fz=F0zAz/η1Az/η2	(614)

where F0is defined in subclause 5.2.3.1.5.1 with T=64, β1 is also defined in subclause 5.2.3.1.5.1, and  η1=0.75and η2=0.9.

For WB IC mode, Fz is defined as:

Fz=(1−(a⋅β1+b⋅γ)z−11−0.85z−T	(615)

where T=64, a=0.5, b=0.25and γ represents the tilt of following filter:

F(1)z=Az/η1Az/η2	(616)

The tilt is computed as:

γ=−n=163f(1)(n−1)f(1)(n)(f(1))2(n)	(617)

β1 is bounded by [0.25 0.5] and given is by:

β1=0.25+0.25Ev−1Ev−1+Ec−1	(618)

whereEv−1and Ec−1are the energies of the scaled pitch codevector and the scaled algebraic codevector of the previous subframe, respectively.

##### 5.2.3.4.2	Gaussian noise generation

The Gaussian noise excitation is a second excitation added to the first innovative excitation from the algebraic codebook. This second contribution is only computed and added in WB.

The Gaussian noise excitation is produced by calling three times a random generator with a uniform distribution between -1 and +1. It follows the Central Limit Theorem.

c2(n)=i=02rand(n)	(619)

The Gaussian noisy excitation c2(n)is spectrally shaped by applying the pre-filter Fz defined in subclause 5.2.3.4.1.1.

##### 5.2.3.4.3	Gain coding

For NB only one gain has to be quantized, the gain of the algebraic codebook gc. It is quantized using a 6-bit quantizer.

For WB, the two gains gc and gc2 are quantized jointly in each subframe, using a 7-bit vector quantizer.

In both cases the optimal algebraic codeword gain is computed as follows:

gc=n=0N−1xn.znn=063zn.zn	(620)

In the equation above, zn is the algebraic codevector cn filtered through the weighted synthesis filter Wz/Az with the pre-filter Fz.is the filtered algebraic codevector.

The algebraic codebook excitation energy in dB, Ec, is also computed as follows:

Ec=10log164n=063c2n	(621)

5.2.3.4.3.1	Innovative codebook gain coding (NB)

The algebraic codevector gain in dB is given by

gcdB=20loggc	(622)

is uniformly quantized between -30 dB and 90dB with the step size of 1.9dB. The quantization index k is given by the integer part of

k=gcdB+Ec+301.9+0.5	(623)

Finally, the quantized gain in dB is given by

gcdB=k×1.9−30	(624)

and the quantized gain is given by

gc=10(gcdB−Ec)/20	(625)

5.2.3.4.3.2	Joint gain coding (WB)

The algebraic codebook gain is quantized indirectly, using a predicted energy of algebraic codevector. The energy of residual signal in dB is calculated.

Then, average residual signal energy is calculated for the whole frame and serves as a prediction of the algebraic codevector energy. It is quantized on 4 bits once per frame. The quantized value of the predicted algebraic codevector energy is defined as

kind=min15k=0Er−EbookkEi=Ebookkind	(626)

where Ebookk, k=0,…,15is the 4-bit codebook for the predicted algebraic codevector energy and kind is the index minimizing the criterion above.

Using the predicted algebraic codevector energy, we may estimate the algebraic codebook gain as

gc'=100.05Ei−Ec	(627)

A correction factor between the true algebraic codebook gain, gc, and the estimated one, gc', is given by

γ=gcgc'	(628)

The correction factor γ is uniformly quantized on 5 bits between -20 dB and 20dB with the step size of 1.25dB. The quantization index kis given by the integer part of

k=γ+201.25+0.5	(629)

Finally, the quantized gain in dB is given by

gcdB=k×1.25−20−Ec+Ei	(630)

and the quantized gain is given by

gc=10gcdB/20	(631)

The gain of Gaussian noise excitation is quantized on 2 bits. Unlike the algebraic codeword gain, the Gaussian noise excitation gain is optimized in order to minimize the energy mismatch between the target signal and reconstructed signal. The following criterion is minimized:

mingc2C⋅n=063x2(n)−n=063(gcz(n)+gc2z2(n))2	(632)

where C is an attenuation factor set to 1 for clean speech, where high dynamic of energy is perceptually important and set to 0.8 for noisy speech where the noise excitation is made more conservative for avoiding fluctuation in the output energy between unvoiced and non-unvoiced frames

The quantized gain is expressed as follows where the index k2 of the optimal gain is sent on 2 bits:

gc2=(0.25k2+0.25)⋅gc⋅n=063c2(n)n=063c22(n)	(633)

##### 5.2.3.4.4	Memory update

The update of the filter memories is performed as described in subclause 5.2.3.1.8 except that there is no adaptive codebook contribution. The Gaussian noise excitation is not taken into account for the update and for computing the next subframe signal target.

#### 5.2.3.5	Excitation coding in GSC mode

In the GSC mode, the excitation is encoded using mixed time-domain/frequency-domain coding technique. This mode is aimed at encoding generic audio signals at low bit rates without introducing more delay than the ACELP structure requires. The GSC mode is used only at 12.8 kHz internal sampling rate, and the excitation could be encoded with 4 subframes, 2 subframes, or 1 subframe per frame depending on the bit rate or the signal type.

Figure 40 is a schematic block diagram showing the general concept of coding the excitation in the GSC mode. The speech/music selector is used to choose between coding the excitation signal in the GSC mode or the other ACELP modes described above. The selector mainly consists of the speech music classification (as described in subclause 5.1.13.5), where GSC is used in case music signals are detected. A further detector (as described in subclause 5.1.13.5.3) is used to verify if a detected music contains a temporal attack. In such a case the time domain transient coding mode is used to code only the attack.

When encoding in the GSC mode, the time-domain excitation contribution is first computed. In case of 4 subframes, the time-domain excitation consists of both adaptive codebook and fixed codebook as in ordinary ACELP. In case 1 or 2 subframes are used, the time-domain excitation consists only of the adaptive codebook contribution. Then the time-domain contribution and residual signal are both converted to the transform domain (using DCT). The transform-domain signals are used to determine a cut-off frequency (the upper band still containing significant pitch contribution). The time-domain excitation contribution is then filtered by removing the frequency content above the cut-off frequency. The filtered time-domain contribution in the frequency domain is subtracted from the frequency-domain residual signal, and difference signal is quantized in the frequency domain using PVQ. The quantized difference signal is then added to the filtered transformed time-domain excitation contribution, and the resulting signal is converted back to the time domain to obtain the total excitation signal.

The GSC mode is used for encoding audio signals at 7.2, and 13.2 kbit/s for WB inputs. It is also used to encode unvoiced active speech and some audio signal at 13.2 kbit/s in case of SWB inputs. Further, the GSC mode is used to encode inactive signals in case of NB, WB, SWB, and FB signals (in case DTX is off) at 7.2, 8 and 13.2kbit/s.

Figure 41: GSC encoder overview

##### 5.2.3.5.1	Determining the subframe length

The subframe length, or number of subframes per frame, is determined depending on the bit rate and nature of encoded signal. In case of SWB unvoiced mode at 13.2 kbit.s, 4 subframes are used. For NB and WB signals at 13.2 kbit/s, 2 subframes are used in case of inactive signals or when the high frequency dynamic range flag Fhf is 0, where Fhfis an indicator when set to 1 is indicates the presence of high frequency spectral correlation and is computed in subclause 5.1.11.2.6. Otherwise 1 subframe is used (audio signal where long-time support is needed to get better frequency resolution).

For the bit rates of 7.2 and 8 kbit/s, 1 subframe is always used (NB, WB, and SWB audio signals and inactive speech signals). The number of subframe information is encoded at 13.2 kbps with 1 bit.

##### 5.2.3.5.2	Computing time-domain excitation contribution

For the SWB unvoiced mode at 13.2 kbit/s, 4 subframes are used and the excitation is computed similar to ACELP Generic coding mode using both adaptive and fixed codebooks (see subclause 5.2.3.1).  The signal is encoded using GENERIC coding type at 7.2 kbit/s, and the remaining bits are used to encode the frequency domain contribution. The target signal for FCB search is computed without low-pass filtering of ACB excitation.

In other modes where 1 or 2 subframes are used the time-domain excitation contributions consists only of the adaptive codebook contribution. This is determined using ordinary closed-loop pitch search as in subclause 5.2.3.1.4.1.

When only 1 or 2 subframe are used,  for example at 7.2 and 8 kbit/s rates, the adaptive codebook excitation and pitch gain quantization use the AUDIO coding type. The pitch found is quantized using 10 bits for the first subframe and 6 bit for the following subframe, if any. In these case, he pitch gain is quantized using a 4 bits vector quantizer.

The total excitation is finally constructed based on both ACB and FCB at 13.2 UC mode or only ACB contribution for other modes using a total between 14 and 24 bits in case of 1 or 2 subframe up to 106 bits for the 4 subframe case..

##### 5.2.3.5.3	Frequency transform of residual and time-domain excitation contribution

In the frequency-domain coding of the mixed time-domain / frequency-domain GSC mode, the residual signal and the time-domain excitation contribution are transformed to frequency domain. The time-to-frequency transform is performed using a 256-point Type IV discrete cosine transform (DCTIV) giving a resolution of 25 Hz at the internal sampling frequency of 12.8 kHz.

The DCTIV, y(k), of a signal x(n)of length Lis defined by the following equation:

y(k)=n=0L−1x(n)cosn+12k+12πL, k=0,...,L−1	(634)

Here L=256 and x(n) refers to either residual signal r(n) or time-domain excitation contribution u(n) with DCT output y(k) corresponding to frequency transformed signals fr(k) and fu(k), respectively.

5.2.3.5.3.1	eDCT for DCTIV

The efficient eDCT is built upon a discrete cosine transform type IV (DCTIV) but the eDCT requires less storage and has lower complexity.

- The DCTIV formula in above subclause can be rewritten as:

y(2q)=ReZ(q)	(635)

where the values Zqare given by

Z(q)=W8L−3⋅W4L2q+1p=0L2−1z(p)⋅W4L2p+1WL2pq,q=0,1,…,L2−1	(636)

- and z(p)=x(2p)+jx(L−1−2p), p=0,1,…,L2−1, and WN=e−j2πN=cos(2πN)−jsin(2πN).

Hence, the eDCT is computed using a Fast Fourier Transform (FFT) of L2 points on the pre-rotated data z(p):

A complex DFT with length L2 is applied to the rotated data z(p):

rk=z(p)WL2pk k=0,⋯,L2−1	(637)

Here, when L2=320 or 160,  a simple power-2 DFT is not suitable, so it is implemented with the following low complexity 2-dimensional (L2=P×Q) DFT, where P=64 (L2=320) or 32 (L2=160) and Q=5 are coprime factors.

To reduce complexity, an address table is introduced. It can be calculated by:

I(n1,n2)=K1⋅n1+K2⋅n2 mod L2 n1=0,⋯,P−1, n2=0,⋯,Q−1	(638)

where K1, K2 are coprime and satisfy the condition (K1⋅K2) mod (L2)=0.

Here, K1=65,K2=256 (P=64) or 96 (P=32). The address table I is stored for low complexity 2-dimentional DFT, and is used to indicate which samples are used for P-point DFT or Q-point DFT following:

a)	Applying P-point DFT to z(p) for Q times based on the address table I.

The input data to the i-th (i=0,..., Q−1)P-point DFT is found by seeking their addresses stored in the address table I. For the i-th P-point DFT, the addresses of the input data are the P continuous elements starting from the i⋅P's element in table I. For every time of P-point DFT, the resulting data need to be applied a circular shift with a step of c1, where c1 is the re-ordered index, which satisfies (c1⋅((K12Q)mod P))mod P=1.

The output of step a) is:

wk=DFTP(z(I+iP))c1 i=0,⋯,Q−1	(639)

For the i-th (i=0,..., Q−1)P-point DFT, the addresses of the input data are the P continuous elements starting from I[Pi], and the results are circular shifted with c1=5. Here is an example of circular shift, the original vector is Z=[z0 z1 z2 z3 z4], the new vector with 2 circular shift is Z=[z0 z2 z4 z1 z3].

b)	Applying Q-point DFT to wk for P times based on the address table I.

The input data to the i-th (i=0,..., P−1)Q-point DFT is found by seeking their addresses stored in the address table I. For the i-th Q-point DFT, the addresses of the input data are the Q elements starting from the i-th element in table I, each of which separated by a step of P. For every time of Q-point DFT, the resulting data need to be applied a circular shift with a step of c2. c2 is the re-ordered index, which satisfies (c2⋅((K22P)mod Q))mod Q=1.

The output of step b) is:

rk=DFTQ(w(I+i))c2 i=0,⋯,P−1	(640)

For the i-th (i=0,..., P−1)Q-point DFT, the addresses of the input data are the Q continuous elements starting from I[i], each of which is separated by a step of P, and the results are circular shifted with c2=4 (P=64) or 2 (P=32). Finally, the coefficients are output according to the stored address corresponding to the address table I.

##### 5.2.3.5.4	Computing energy dynamics of transformed residual and quantization of noise level

The DCT of the residual is divided into 16 bands (0 to 15) of length 16 bins. For bands 7 to 14, the energy dynamic per band is computed as the square of the maximum value divided by the average value per band, scaled by a factor 10. Then the average value Dav over the 8 band (from 7 to 14) is computed.

A long-term dynamic Dlt is updated as

Dlt=0.2Dlt+0.8Dav,when Dav>Dlt	(641)

Dlt is quantized with 8 levels in the rage 50-82 (50, 54, 58, 62, 66, 70, 74, 78) with quantization index iD from 0 to 7.

The noise level is computed as Nlev=15−iD

For the bit rates of 7.2 and 8 kbit/s, the noise level is low limited to 12. Thus only values 12, 13, 14, 15 are permitted and Nlev is quantized with 2 bits). For UC SWB mode at 13.2 kbit/s Nlev is set to 15, otherwise Nlev is quantized with 3 bits (values 8 to 15).

##### 5.2.3.5.6	Find and encode the cut-off frequency

The cut-off frequency consists of the last band with significant pitch contribution (the frequency after which coding improvement brought by the time-domain excitation contribution becomes too low to be valuable). Finding the cut-off frequency starts by computing the normalized cross-correlation for each frequency band between the frequency-transformed LP residual fr(k)and the frequency-transformed time-domain excitation contribution fu(k). The 256-sample DCT spectrum is divided into the 16 bands with the following number of frequency bins per band

Bb(i)=8,8,16,16,16,16,16,16,16,16,16,16,16,16,16,32	(642)

with cumulative frequency bins per band

CBb(i)=0,8,16,32,48,64,80,96,112,128,144,160,176,192,208,224	(643)

The last frequency Lf included in each of the 16 frequency bands are defined in Hz as:

Lf(i)=175, 375, 775, 1175, 1575, 1975, 2375, 2775, 3175, 3575, 3975, 4375, 4775, 5175, 5575, 6375	(644)

The normalized correlation per band is defined as

Cc(i)=j=CBb(i)CBb(i)+Bb(i)fr(i)fu(i)Sfr'(i)Sfu'(i)	(645)

Where Sfr'(i)=fr(i)fr(i) and Sfu'(i)=fu(i)fu(i).

The cross-correlation vector is then smoothed between the different frequency bands using the following relation

Cc2i=2⋅(min(0.5,α⋅Cc(0)+δ⋅Cc(1))−0.5),for i=0	(646)

where α=0.95, δ=1−α and β=δ2

The average of the smoothed cross-correlation vector is computed over the first 13 bands (representing 5575 Hz). It is then limited to a minimum value of 0.5 normalised between 0 and 1.

A first estimate of the cut-off frequency is obtained by finding the last frequency of a frequency band Lf which minimizes the difference between the last frequency of a frequency band Lf and the normalized average Cc2 of the smoothed cross-correlation vector multiplied by the width of the spectrum of the input sound signal. That is

imin=minLf(i)−Cc2(Fs2),for 0≤i<12	(647)

and the first estimate of the cut-off frequency is given by

ftc1=Lf(imin)	(648)

where Fs=12800 Hz.

At 7.2 and 8 kbit/s, where the normalized average Cc2 is never really high, or to artificially increase the value of ftc1 to give a little more weight to the time domain contribution, the value of Cc2 is upscaled with a factor of 2.

The 8th pitch harmonic is computed from the minimum or lowest pitch lag value of the time-domain excitation contribution of all sub-frames, and the frequency band containing the 8th harmonic is determined. The final cut-off frequency is given by the higher value between the first estimate of the cut-off frequency ftc1 and the last frequency of the frequency band in which the 8th harmonic is located Lf(i8).

The cut-off frequency is quantized with a maximum of 4 bits using the values {0, 1175, 1575, 1975, 2775, 3175, 3575, 3975, 4375, 4775, 5175, 5575, 6375}.

Some hangover is added to stabilize the decision and prevent the cut-off frequency to switch between 0 (meaning no temporal contribution) and something else too often. First for the temporal contribution to be allowed, the average normalized correlation Cnorm2and the long-term correlation Cnorm2as computed in subclause 5.1.13.5.3, the long term average pitch gain of the GSC temporal contribution Gpltand the last value of the cut-off frequency are compared to some threshold to decide if it is allowed to remove all the temporal contribution (cut-off frequency would be 0). In addition a hangover logic is used to diminish any undesired switching to a complete frequency model where the cut-off frequency would be 0.

For the lowest bitrate, 7.2 and 8.0 kbit/s, only 1 bit is used to send the cut-off frequency information when the coding mode is INACTIVE otherwise, the cut-off frequency is considered as greater than 0 (meaning the temporal contribution is used) and the length of the contribution is deduced from the pitch information. At 13 kbps, 4 bits are used to send the cut-off frequency allowing all the possible cut-off frequency values.

Once the cut-off frequency is determined, the transform of the time-domain excitation contribution is filtered in the frequency domain by zeroing the frequency bins situated above the cut-off frequency supplemented with a smooth transition region. The transition region is situated above the cut-off frequency and below the zeroed bins, and it allows for a smooth spectral transition between the unchanged spectrum below ftc and the zeroed bins in higher frequencies.

##### 5.2.3.5.7	Band energy computation and quantization

The filtered time-domain contribution in the frequency domain fu'(k) is subtracted from the frequency-domain residual signal fr(k), and the resulting difference signal in the frequency domainfd(k)is quantized with the PVQ. Before the quantization is done, some gains per frequency band Bb, as defined above, are computed and quantized using a split VQ. First the gain per band on the difference signal Gbd is computed as :

Gbdi=log102.0⋅k=CBbik=CBbi+Bbifdk,for 0≤i<2log10k=CBbik=CBbi+Bbifdk,for 2≤i<15	(649)

where CBband Bbare defined in subclause 5.2.3.5.6.

In case of NB content, only the first 10 bands are quantized using a split VQ. For other bandwidth, the number of band quantized depends on the bitrate. At low bit rate only 12 bands are quantized, being the band 0 to 8 plus the bands 10, 12 and 14. The band 9, 11, 13 and 15 being interpolated based on the quantized bands 8, 10, 12, and 14. The codebook used for the vector quantization are different depending of the bitrate and the bandwidth of the input signal giving a total 4 different set of codebooks.

In all cases, prior to the vector quantitation of the bands, the average gain of all the bands is subtract from the bands and vector quantized as well using 6 bits. In total between 21 and 26 bits are used to get the gain per band quantized Gbd depending of the bitrate.

##### 5.2.3.5.8	PVQ Bit allocation

The PVQ is a coding technic that is flexible in its bit allocation. To decide where bits should be allocated inside the difference spectrum to quantize, some parameters are analyse as the bitrate, the cut-off frequencyftc1, the noise levelNlev, the coding mode (INACTIVE, AUDIO or active UC), the bit budget available and the bandwidth.

First, only a subset of bands will be sent to the PVQ for quantization. The minimum number of band is 5 out of 16. To determine the number of band, a first criteria is the bit rate, a second criteria is the cut-off frequency and another criteria is noise level. When the number of band is decided, a minimum amount of bit is spread over the number of band decided with an emphasis on the low frequencies. If some bits remain after the minimum bit allocation, then the remaining bits are split among the bands. When the number of bands and its bit allocation are found, the bands are picked from the initial spectrum of the difference signal fd(k)based on the quantized gain of this band. The 5 first bands are always sent to the PVQ, the choice of the other bands on the energy associated to that band and the high frequency flag indicatorFhf.

##### 5.2.3.5.9	Quantization of difference signal

Once the bit allocation the number of band to quantize and their position in the spectrum is defined, a new vector is concatenated containing all the chosen bands. The values are then passed to the PVQ for quantization to obtain the quantized difference spectrumfd(k). The PVQ quantization scheme is described in subclause 5.3.4.2.7.

##### 5.2.3.5.10	Spectral dynamic and noise filling

After the quantization by the PVQ, some band are empty and many more bins are zeroed due to the low inherent to the GSC technology available. To make the frequency model as robust as possible on speech like content, the spectral dynamic is revised and some noise filling is added to the difference spectrum.

For INACTIVE content below 13.2 kbit/s, the quantized spectrum above 1.6kHz is multiplied by a factor of 0.15. For INACTIVE content at 13.2 kHz, the quantized spectrum above 2.0kHz is multiplied by a factor of 0.25. Otherwise the scaling factor for the spectral dynamicSfsd and the frequency bin where the scaling of the spectral dynamic Ffsdis applied is computed as follow:

Sfsd=14−Nlev10−0.4	(650)

and

Ffsd=112+14−Nlev∗16	(651)

Furthermore, for frequencies above 3.2 kHz, the spectral dynamic is limited to an amplitude of ±1 for bitrate below 13.2 kbit/s and to ± 1.5 otherwise.

This scaling is then applied to the quantized difference spectrumfd(k), to obtain its scaled versionfds(k).

A noise filling is then applied to the whole difference spectrum. The noise level added is based on the bitrate, the coding mode and the spectral dynamicNlev to obtain the scaled difference spectrum with noisefdsn(k)on which the gain will be applied on.

##### 5.2.3.5.11	Quantized gain addition, temporal and frequency contributions combination

Once dynamic of the quantized difference spectrum has been scaled and the noise fill has be performed, the gain of each bands is computed exactly as in subclause 5.2.3.5.7 to get gain of the quantized spectrumGbd2. The gain per band to apply Gai consists as:

Gai=10Gdbi−Gdb2i	(652)

This gain is applied to both the scaled difference spectrum with noisefdsn(k) and the scaled difference spectrumfds(k)and both vectors are added to the temporal contribution to get two different spectral representation of the quantized excitation in the frequency domain, one with noise fillfu left (i right )} { and the other withoutfu2 left (i right )} { as shown below.

fu left (i right )=f rSub { size 8{u} } ' left (i right )+ { hat  {f}} rSub { size 8{ ital dsn} }  \( i \)  cdot G rSub { size 8{a} }  left (k right ) \rline  rSub { size 8{i=C rSub { size 6{ ital Bb} }  \( k \) +B rSub { size 6{b} }  \( k \) ,``0 <= k<16} } } {	(653)

and

fu2 left (i right )=f rSub { size 8{u} } ' left (i right )+ { hat  {f}} rSub { size 8{ ital ds} }  \( i \)  cdot G rSub { size 8{a} }  left (k right ) \rline  rSub { size 8{i=C rSub { size 6{ ital Bb} }  \( k \) +B rSub { size 6{b} }  \( k \) ,``0 <= k<16} } } {	(654)

##### 5.2.3.5.12	Specifics for wideband 8kbps

The available bits are allocated to the bands of the frequency excitation signal according to the bit allocation algorithm as described in subclause 5.2.3.5.8, where the frequency excitation signal is the output of DCTIV as described in subclause 5.2.3.5.3. If the index Ihighbit of the highest frequency band with bit allocation is more than a given threshold, the bit allocation for the frequency excitation bands will be adjusted: Decrease the number of the allocated bits of the bands with more bits, and increase the number of the allocated bits of the band Ihighbit and the bands near to Ihighbit. And then, encode the frequency excitation signal with the allocated bits, where the given threshold is determined by the available bits and the resolution of the frequency excitation signal.

The details are described as follows:

- Allocate most of the available bits to the 5 lower frequency bands by the pre-determined bit allocation table;

- Allocate the remaining bits to those bands excluding the lower frequency 5 bands  which have the largest band energy, if there are remaining bits after the first step;

- Search the index Ihighbitof the highest frequency band with bit allocation.

If the index Ihighbit of the highest frequency band with bit allocation is more than a given thresholdTr1=7, the bit allocation for the frequency excitation bands will be adjusted:

- Allocate bits to some more bands whose index is above Ihighbit. The number of the newly bit allocated bandsNextr is determined by the noise levelNlevand the coding mode.

- For the newly bit allocated bands, allocate 5 bits to each band. If the number of the newly bit allocated bandsNextr≤2, allocate 1 more bit to each band whose index starts from 4 to Ihighbit+Nextr.

- The total number of newly allocated bitsRextr is 5⋅Nextr+Ihighbit+Nextr−4. Rextris obtained by decreasing the number of the bits allocated to the 4 lower frequency bands.

Otherwise, the original number of allocated bits to each band is not changed.

Finally, quantize and encode the frequency excitation signal according to the allocated bits.

Then, reconstruct the frequency excitation signal based on the quantized parameters. The reconstructed frequency excitation signal is corresponding to the decoded frequency excitation signal in decoder.

For the reconstructed frequency excitation signal, if the index Ilastbin of the highest frequency band with bit allocation is more than a given threshold, or there is the temporal contribution in the reconstructed frequency excitation signal, the frequency excitation signal above CBb(Ihighbit') will be reconstructed by the reconstructed frequency excitation signal; otherwise, the frequency excitation signal above CBb(Ihighbit')will be reconstructed by noise filling.

The detailed descriptions are as follows:

When the coding mode of previous frame is AC mode, if the last sub-band index of bit allocation Ilastbinis larger than Tr2=8 or there is the temporal contribution in the reconstructed frequency excitation signal, the BWE flag FBWEis set to 1. It should be noted that the BWE flag FBWE is initialized to 0 and calculated for every frame. Ilastbinis then refined by:

Ilastbin=max(Ilastbin,10)ifFBWE=1	(655)

If FBWE=1, the frequency excitation signal below CBb(Ilastbin) will be reconstructed as described in subclause 5.2.3.5.11, and the frequency excitation signal above CBb(Ilastbin) will be reconstructed as follows:

fd(255−k)=fu2 left (159 - k right )0 <= k<255 -  left (16 cdot I rSub { size 8{ ital last_ ital bin} }  - 1 right )} {	(656)

And then the frequency excitation signal fd(255−k)0≤k<255−16⋅Ilastbin−1 is scaled by the quantized gains to obtain the scaled frequency excitation signal fu2 \( 255 - k \) 0 <= k<255 -  left (16 cdot I rSub { size 8{ ital last_ ital bin} }  - 1 right )} {.

When the energy ratio between the current frame and the previous frame is in the range (0.5, 2), for any band with index range is [4, 9], if the band is bit allocated in the current frame or in the previous frame, the coefficients in the band are smoothed by weighting the coefficients of the current frame and the previous frame.

For the scaled frequency excitation signal above CBb(Ilastbin), estimate the position of the formant by LSF parameters. If the magnitudes of the coefficients near to the formant are larger than a threshold, the magnitudes are decreased to improve the perceptual quality.

Otherwise, If FBWE=0, the un-quantized coefficients, i.e. un-decoded coefficients at the decoder side will be reconstructed by noise filling as described in subclause 5.2.3.5.10.

##### 5.2.3.5.13	Inverse DCT

After the gain has been applied and the combination in the frequency domain done, both frequency representations of the coded excitation are convert back to time domain using the exact same DCT as in subclause 5.2.3.5.3. The inverse transform is performed to get the quantized excitation un which is the temporal representation of fu left (i right )} { and u2nwhich is the temporal representation offu2 left (i right )} {. un will be used to update the TDBWE while u2n is used to update the internal CELP state as the adaptive codebook memory.

##### 5.2.3.5.14	Remove pre-echo in case of onset detection

Compute the energy of the excitation unover each 4 samples using a 4-sample sliding window, and find the more energetic section to determine a possible attack (onset). If the attack is larger than the previous frame energy plus 6 dB, the algorithm finds the energy before the attack (excluding the section where the attack has been detected) and it scales it to the level of the previous frame energy plus 6dB.