3GPP TS 32.256 V16.3.0 (2022-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Charging management;

5G connection and mobility domain charging;

stage 2

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	Architecture considerations	9

4.1	High-level 5G System architecture	9

4.1.1	Non-roaming reference architecture	9

4.1.2	Roaming reference architecture	10

4.2	5G connection and mobility domain converged charging architecture	11

4.2.1	Non-roaming	11

4.2.2	Roaming	11

5	5G connection and mobility charging principles and scenarios	12

5.1	5G connection and mobility charging principles	12

5.1.1	General	12

5.1.2	Requirements	12

5.1.3	CHF selection	13

5.1.4	UE Presence in Presence Reporting Area (PRA)	13

5.1.5	Roaming	13

5.1.5.1	General	13

5.1.5.2	CHF selection	13

5.2	5G connection and mobility converged online and offline charging scenarios	14

5.2.1	Basic principles	14

5.2.1.1	General	14

5.2.1.2	Applicable Triggers in the AMF	14

5.2.1.2.1	General	14

5.2.1.2.2	AMF Charging profile	15

5.2.1.2.3	Independent triggers activation	16

5.2.1.3	UE Presence in Presence Reporting Area (PRA)	16

5.2.2	Message flows	17

5.2.2.1	General	17

5.2.2.2	Registration management charging from AMF	17

5.2.2.2.1	General	17

5.2.2.2.2	General Registration – Event offline charging	17

5.2.2.2.3	General Registration – IEC	19

5.2.2.2.4	General Registration – ECUR	20

5.2.2.2.5	Registration with AMF re-allocation – Event offline charging	21

5.2.2.2.6	Registration with AMF re-allocation – IEC	21

5.2.2.2.7	Registration with AMF re-allocation – ECUR	21

5.2.2.2.8	Deregistration – Event offline charging	21

5.2.2.2.9	Registration – Untrusted non-3GPP access - Event offline charging	23

5.2.2.2.10	Registration – Untrusted non-3GPP access - IEC	24

5.2.2.2.11	Registration – Untrusted non-3GPP access - ECUR	25

5.2.2.2.12	Registration for SMS over NAS – Event offline charging	26

5.2.2.2.13	Registration for SMS over NAS – IEC	27

5.2.2.2.14	Registration for SMS over NAS – ECUR	28

5.2.2.2.15	Deregistration for SMS over NAS – Event offline charging	29

5.2.2.3	Connection management charging from AMF	29

5.2.2.3.1	General	29

5.2.2.3.2	UE Triggered Service Request	30

5.2.2.3.3	Network Triggered Service Request	30

5.2.2.3.4	AN release	30

5.2.2.4	Location reporting charging	31

5.2.2.4.1	General	31

5.2.2.4.2	Location reporting	31

5.2.2.5	Roaming - Registration management charging from AMF	33

5.2.2.5.1	General	33

5.2.2.5.2	Registration - PEC to HPLMN	33

5.2.2.5.3	Registration – IEC to HPLMN	34

5.2.2.5.4	Registration – ECUR to HPLMN	36

5.2.2.5.5	Deregistration – PEC to HPLMN	38

5.2.3	CDR generation	39

5.2.3.1	Introduction	39

5.2.3.2	Triggers for CHF CDR	39

5.2.3.2.1	General	39

5.2.3.2.2	Triggers for CHF CDR generation	39

5.2.3.2.3	Triggers for CHF CDR opening	39

5.2.3.2.4	Triggers for CHF CDR closure	39

5.2.4	Ga record transfer flows	39

5.2.5	Bam CDR file transfer	39

6.	Definition of charging information	40

6.1	Data description for 5G connection and mobility charging	40

6.1.1	Message contents	40

6.1.1.1	General	40

6.1.1.2	Charging Data Request message	41

6.1.1.3	Charging data response message	42

6.1.2	Ga message contents	42

6.1.3	CDR description on the Bam interface	42

6.1.3.1	General	42

6.1.3.2	Registration charging CHF CDR data	42

6.1.3.3	N2 connection charging Information CHF CDR data	43

6.1.3.4	Location reporting charging CHF CDR data	43

6.2	5G connection and mobility charging specific parameters	44

6.2.1	Definition of 5G connection and mobility charging information	44

6.2.1.1	General	44

6.2.1.2	Definition of Registration charging information	45

6.2.1.3	Definition of N2 Connection charging information	46

6.2.1.4	Definition of Location Reporting charging information	47

6.2.2	Formal 5G connection and mobility charging parameter description	47

6.2.2.1	5G connection and mobility CHF CDR parameters	47

6.2.2.2	5G connection and mobility resources attributes	47

6.2.3	Detailed message format for converged charging	47

6.3	Bindings for 5G connection and mobility converged charging	49

Annex A (normative): Charging Characteristics	49

A.1	General	49

Annex B (informative): Change history	51

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is part of a series of documents that specify charging functionality and charging management in 3GPP networks. The 3GPP core network charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management TSs that specify:

-	the content of the CDRs per domain / subsystem / service (offline and converged charging);

-	the content of real-time charging messages per domain / subsystem / service (online and converged charging);

-	the functionality of online, offline and converged charging for those domains / subsystems / services;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the converged offline and online charging description for the 5G connection and mobility domain based on the functional stage 2 description in TS 23.501 [200], TS 23.502 [201] and TS 23.503 [202].

This charging description includes the converged offline and online charging architecture and scenarios specific to the 5G connection and mobility domain, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [1] onto the 5G connection and mobility domain.

It further specifies the structure and content of the CDRs for offline charging, and the charging events for converged online and offline charging. The present document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240 [1].

-	The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298 [51].

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [54].

-	The file based mechanism used to transfer the CDRs from the network to the operator's billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297 [52].

-	The services, operations and procedures of charging, using Service Based Interface are specified in TS 32.290 [57].

-	The charging service of 5G system is specified in TS 32.291 [58].

All references, abbreviations, terms, descriptions, principles and requirements, used in the present document, that are common across 3GPP TSs, are defined in TR 21.905 [100]. Those that are common across charging management in 3GPP networks/domains, services or subsystems are provided in the umbrella TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [50]	Void.

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53] 	Void.

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55-56] 	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] - [199]	Void

[200]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[201]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[202]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[203]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[204]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[205] - [299]	Void

[300] - [399]	Void.

[400] - [499]	Void.

[500] - [599]	Void.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [100] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [100].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bam	Reference point for the CDR file transfer from the 5G connection and mobility CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

N41	Reference point between AMF and the H-CHF.

N42	Reference point between AMF and the V-CHF.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

5GC	5G Core Network

5GS	5G System

5G-AN	5G Access Network

AMF	Access and Mobility Management Function

BD	Billing Domain

CCS	Converged Charging System

CDF	Charging Data Function

CGF	Charging Gateway Function

CHF	Charging Function

CTF	Charging Trigger Function

DNN	Data Network Name

ECUR	Event Charging with Unit Reservation

GPSI	Generic Public Subscription Identifier

H-CHF	A CHF in HPLMN

IEC	Immediate Event Charging

GUAMI	Globally Unique AMF Identifier

MICO	Mobile Initiated Connection Only

N3IWF	Non-3GPP InterWorking Function

NF	Network Function

NRF	Network Repository Function

PEC	Post Event Charging

PEI	Permanent Equipment Identifier

SUPI	Subscription Permanent Identifier

UDM	Unified Data Management

UDR	Unified Data Repository

V-CHF	A CHF in VPLMN

# 4	Architecture considerations

## 4.1	High-level 5G System architecture

### 4.1.1	Non-roaming reference architecture

Figure 4.1.1.1 shows the 5G System high level architecture in the service-based representation, as defined in TS 23.501 [200], with AMF encompassing 5G registration, connection and mobility management functionalities:

Figure 4.1.1.1: 5G System architecture

### 4.1.2	Roaming reference architecture

Figure 4.1.2.1 shows the 5G System roaming architecture with local breakout in the service-based representation, as defined in TS 23.501 [200], with AMF encompassing 5G registration, connection and mobility management functionalities:

Figure 4.1.2.1: Roaming 5G System architecture- local breakout scenario in service-based interface representation

Figure 4.1.2.2 shows the 5G System roaming architecture in the case of home routed scenario the service-based representation, as defined in TS 23.501 [200], with AMF encompassing 5G registration, connection and mobility management functionalities:

Figure 4.1.2.2: Roaming 5G System architecture - home routed scenario in service-based interface representation

## 4.2	5G connection and mobility domain converged charging architecture

### 4.2.1	Non-roaming

Architectural options for 5G connection and mobility converged charging achieved by AMF embedding the CTF, are depicted in figure 4.2.1.1.

Figure 4.2.1.1: 5G connection and mobility converged charging architecture

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components. Ga is described in clause 5.2.4 and Bam in clause 5.2.5 of the present document, and Nchf is described in TS 32.290 [57].

### 4.2.2	Roaming

Figure 4.2.2.1 shows the 5G connection and mobility converged charging architecture in roaming service based representation:

Figure 4.2.2.1: 5G connection and mobility converged charging architecture in roaming

The architectural options of figure 4.2.1.1 apply with AMF located in VPLMN, V-CHF and corresponding Billing domain/CGF located in VPLMN.

The architectural options of figure 4.2.1.1 apply with AMF located in VPLMN, H-CHF and corresponding Billing domain/CGF located HPLMN. The Security Edge Protection Proxy (SEPP) specified in TS 23.501 [200] is used for inter-PLMN Nchf.

Figure 4.2.2.2 shows the 5G connection and mobility converged charging architecture in roaming, reference point representation:

Figure 4.2.2.2: 5G connection and mobility converged charging architecture in roaming - reference point representation

The N41 reference point is defined for the interactions between AMF and H-CHF in the reference point representation.

The N42 reference point is defined for the interactions between AMF and V-CHF in the reference point representation.

# 5	5G connection and mobility charging principles and scenarios

## 5.1	5G connection and mobility charging principles

### 5.1.1	General

The charging functions specified for the 5G connection and mobility charging, are based on functionalities supported by AMF specified in TS 23.501 [200]:

-	Registration Management;

-	Connection Management;

-	Location Reporting.

Location Reporting is applicable to NG-RAN only.

### 5.1.2	Requirements

The following are high-level charging requirements specific to the 5G connection and mobility charging:

-	The AMF shall support converged online and offline charging using service based interface.

-	The AMF shall collect charging information per UE registration to the 5GS.

-	The AMF shall collect charging information per UE N2 connection between the 5G-AN and the AMF.

-	The AMF shall collect charging information per UE location reporting.

-	In roaming scenario, the AMF shall collect charging information per in-bound roamer UE registration in 5GS, for both V-PLMN and Home PLMN.

### 5.1.3	CHF selection

The CHF Address(es) selection by the AMF is done during the Registration process based on the following options:

-	CHF Address(es) used for UE context transferred from old AMF.

-	CHF address(es) provided by the PCF as part of Access and mobility policy control information.

-	UDM provided charging characteristics.

-	NRF based discovery.

-	AMF locally provisioned charging characteristics.

The priority order between these options depends on Operator's policies.

Once selected, these CHF Address(es) shall be used as long as the UE is registered in the AMF.

### 5.1.4	UE Presence in Presence Reporting Area (PRA)

At UE initial registration, the CHF may subscribe to a list of Presence Reporting Area(s), to be reported with "Change of UE presence in Presence Reporting Area(s)" from AMF.

A Presence Reporting Area, as specified in TS 23.501 [200] clause 5.6.11, may represent:

-	A Core Network pre-configured Presence Reporting Area identified by a PRA Identifier;

-	A UE-dedicated Presence Reporting Area defined by CHF and identified by a PRA Identifier with its elements (i.e. list of TAs and/or NG-RAN nodes and/or cells identifiers).

The CHF remains subscribed to the list of Presence Reporting Area(s) as long as the UE is registered in the AMF until the list is updated or unsubscribed-to by subsequent exchanges between the AMF and the CHF.

This functionality is detailed in clause 5.2.1.3.

### 5.1.5	Roaming

#### 5.1.5.1	General

Based on roaming agreements between the VPLMN and the HPLMN, for each UE determined as in-bound roamer, the AMF in VPLMN shall be able to collect charging information per UE registration, and interact with a V-CHF for CDR generation in VPLMN.

In addition, for this registration, the AMF in VPLMN may be able to interact with a H-CHF for CDR generation in HPLMN.

#### 5.1.5.2	CHF selection

During the Registration process, for UE determined as in-bound roamer, the AMF in VPLMN can select:

-	the CHF Address(es) in HPLMN based on the following options:

-	UDM provided charging characteristics.

-	NRF based discovery using UE identifier.

-	AMF locally provisioned in charging characteristics.

-	the CHF Address(es) in VPLMN based on the following options:

-	NRF based discovery using the PLMN Id of the UE H-PLMN.

-	AMF locally provisioned in charging characteristics.

The priority order between these options depends on Operator's policies.

## 5.2	5G connection and mobility converged online and offline charging scenarios

### 5.2.1	Basic principles

#### 5.2.1.1	General

5G connection and mobility converged charging, when activated, may be performed by the AMF interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics, etc.), the AMF shall be able to perform converged charging for each of the following:

-	Charging data related to registration.

-	Charging data related to N2 connection between the AN and the AMF.

-	Charging data related to user location.

The AMF shall use the AMF charging profile defined in clause 5.2.1.2 to determine whether charging (converged) is activated or not. The default AMF charging profile is provided by Charging Characteristics applicable to AMF are described in Annex A.

The AMF shall be able to perform converged charging by interacting with CHF, for charging data related to registration, connection and location management. The Charging Data Request and Charging Data Response are exchanged between the AMF and the CHF, based on IEC, or PEC scenarios as specified in TS 32.290 [57]. The Charging Data Request is issued by the AMF towards the CHF when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

#### 5.2.1.2	Applicable Triggers in the AMF

##### 5.2.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the 5G connection and mobility converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.2.1.2.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the AMF when charging is active for the corresponding AMF functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from AMF towards the CHF.

Table 5.2.1.2.1.1: Default Trigger conditions in AMF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Registration |  |  |  |  |  |
| Registration checked | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Registration completed | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| N2 connection |  |  |  |  |  |
| Start of N2 connection | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| End of N2 connection | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Location reporting |  |  |  |  |  |
| User Location change | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Change of UE presence in Presence Reporting Area(s) | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

AMF functionality charging activation/deactivation is performed by CHF via AMF charging profile update as defined in clause 5.2.1.2.3.

The AMF charging profile is defined in clause 5.2.1.2.2 and shall be used by the AMF to determine whether converged charging is active or not, and which scenario is applicable when active.

During UE initial registration in the AMF, if registration charging is active in the Charging Characteristics, a Charging Data Request [Event, Registration] or Charging Data Request [Initial, Registration] shall be sent to CHF in the corresponding charging scenario (i.e. IEC, PEC or ECUR) when applicable triggers are met. At N2 connection, if N2 connection charging is active in the AMF charging profile, a Charging Data Request [Event, N2 connection] shall be sent to CHF in PEC scenario when applicable triggers are met.

At N2 connection, if location reporting charging is active in the AMF charging profile, the AMF shall start the location reporting control procedure towards NG-RAN specified in clause 4.10 of TS 23.502 [202] for a continuous reporting whenever the UE location changes. A Charging Data Request [Event, location reporting] shall be sent by AMF upon receiving the location report from NG-RAN with current UE location.

If the AMF receives in a Charging Data Response [Event/Initial] from CHF, an updated AMF charging profile deactivating location reporting charging, and the AMF has a N2 connection, the AMF shall cancel the location reporting control procedure towards NG-RAN.

##### 5.2.1.2.2	AMF Charging profile

An AMF charging profile is defined to allow AMF determination of applicable trigger conditions for each functionality (i.e Registration, N2 connection, Location Reporting), and includes:

-	charging active or inactive;

-	IEC, ECUR or PEC scenario.

For location reporting charging it includes in addition, the level for location change to be reported (i.e. TAI change and/or Cell Change), or UE presence in Presence Reporting Area(s) to be reported. The UE Presence in Presence Reporting Area (PRA) functionality is detailed in clause 5.2.1.3.

IEC and ECUR scenarios are applicable for registration only.

The AMF charging profile is per UE and per AMF.

The default AMF charging profile is provided by the Charging Characteristics which is described in Annex A.

##### 5.2.1.2.3	Independent triggers activation

During the UE registration lifetime in the AMF, the CHF may activate/deactivate charging for any AMF functionality (i.e. Registration, N2 connection, Location Reporting) each time the CHF is invoked for a given AMF functionality. The set of triggers enabled/disabled via charging activation/deactivation by CHF are independent from the invoked AMF functionality. The triggers activation/deactivation take effect at the next AMF functionality invoking the CHF.

Table 5.2.1.2.3.1 describes the applicable messages for CHF to activate/deactivate triggers via update of AMF Charging profile:

Table 5.2.1.2.3.1: AMF charging profile update by CHF

| Functionality | AMF Charging Profile field which can be updated | Message from CHF to AMF |
| --- | --- | --- |
| Registration | - charging "active or inactive" | Charging Data Response [Event/Initial] |
| N2 connection | - charging "active or inactive" | Charging Data Response [Event/Initial] |
| Location Reporting | - charging "active or inactive" - level for location - list of Presence Reporting Area(s) | Charging Data Response [Event/Initial] |

In ECUR scenario, any update of AMF Charging Profile received in Charging Data Response [Initial] will take effect after the corresponding Charging Data Response [Termination].

#### 5.2.1.3	UE Presence in Presence Reporting Area (PRA)

At UE initial registration in the AMF, the CHF may subscribe to changes of UE presence status(es) in PRA(s) in the Charging Data Response [Event, Registration] or Charging Data Response [Initial, Registration] respective IEC or ECUR scenarios. This subscription is performed, by activation of location reporting charging in the AMF charging profile with a list of PRAs supplied in addition.

The PRAs list may be composed of:

-	Core Network pre-configured Presence Reporting Area(s) identified by PRA Identifier(s);

-	UE-dedicated Presence Reporting Area(s) defined by a PRA Identifier(s) each provided with its elements.

Once location reporting charging is activated, the registration is completed, the AMF starts the location reporting procedure towards to NG-RAN specified in clause 4.10 of TS 23.502 [202] with parameters associated to the PRA(s), as described in clause 5.2.2.4.2.

When a Change of UE presence in Presence Reporting Area(s) is detected for PRA(s) subscribed-to by the CHF, the AMF sends Charging Data Request [Event, location reporting] to CHF with the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s).

The CHF may update the list of Presence Reporting Area(s) subscription or remove the list of Presence Reporting Area(s), in any Charging Data Response [Event/Initial] as per table 5.2.1.2.3.1.

### 5.2.2	Message flows

#### 5.2.2.1	General

The flows in the present document specify the interaction between the AMF and the CHF for 5G connection and mobility converged charging functionality, in different scenarios, based on TS 23.501 [200] and TS 23.502 [201] procedures and flows.

This interaction is based on Charging Data Request /Response specified in TS 32.290 [57], exchanged between the AMF embedding the CTF and the CHF.

As a general principle, the steps in the figures for the message flows below correspond to the steps of figures in TS 23.502 [202], which is the reference. The present document specifies the charging specific extension part.

#### 5.2.2.2	Registration management charging from AMF

##### 5.2.2.2.1	General

The subclause below describes registration management charging.

For registration procedures, the following scenarios specified in TS 32.290 [57] are supported:

- 	PEC;

-	IEC;

-	ECUR.

For deregistration procedures, only PEC scenario is supported.

##### 5.2.2.2.2	General Registration – PEC charging

The following figure 5.2.2.2.2.1 describes a Registration charging message flow in PEC charging, based on figure 4.2.2.2.2-1 of TS 23.502 [202] description and, applicable to:

-	Initial Registration in 5GS;

-	Mobility Registration Update; or

-	Periodic Registration Update; or

-	Emergency Registration.

Figure 5.2.2.2.2.1: Registration – PEC charging

1-14c.	Registration procedure initiated by UE.

10: The new AMF notifies the old AMF that the registration of the UE in the new AMF is completed.

14d: If necessary, the old AMF is requested by UDM to delete Registration Management contexts and PDU Sessions.

14dCh: The old AMF interacts with the CHF as per deregistration procedure described in figure 5.2.2.2.8.1.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE successful registration.

21ch-b. The CHF creates the CDR for this registration.

21ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

##### 5.2.2.2.3	General Registration – IEC

The following figure 5.2.2.2.3.1 describes a Registration charging message flows in IEC, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.2.3.1: Registration – IEC

1-14c.	Registration procedure initiated by UE.

15-16: AM policy association procedure.

16ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE registration to be granted authorization, rated and accounted.

16ch-b. Account and Rating control by the CHF for the registration and CDR creation.

16ch-c. The CHF acknowledges by sending Charging Data Response [Event] and the AMF, granting authorization to the registration.

17 to 24. Same steps as in Figure 5.2.2.2.2.1.

##### 5.2.2.2.4	General Registration – ECUR

The following figure 5.2.2.2.4.1 describes a Registration charging message flows in ECUR, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.2.4.1: Registration – ECUR

1-14c.	Registration procedure initiated by UE.

15-16: AM policy association procedure.

16ch-a. The AMF sends Charging Data Request [Initial] to CHF for the UE registration to be granted authorization.

16ch-b. Account and Rating control by the CHF for the registration and open a CDR.

16ch-c. The CHF acknowledges by sending Charging Data Response [Initial] and the AMF, granting authorization to the registration.

17 to 21. Same steps as in Figure 5.2.2.2.2.1.

21ch-a. The AMF sends Charging Data Request [Termination] to CHF for the UE successful registration.

21ch-b. The CHF closes the CDR for this registration.

21ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the AMF.

##### 5.2.2.2.5	Registration with AMF re-allocation – PEC charging

The following figure 5.2.2.2.5.1 describes a registration with AMF re-allocation PEC charging, based on figure 4.2.2.2.3-1 TS 23.502 [202] description:

Figure 5.2.2.2.5.1: Registration with AMF re-allocation– PEC charging

1-7b.	Registration with AMF re-allocation procedure.

21ch-a to 21ch-c: same steps as per Figure 5.2.2.2.2.1.

4-24b. Registration procedure in PEC charging in clause 5.2.2.2.2.

##### 5.2.2.2.6	Registration with AMF re-allocation – IEC

AMF re-allocation IEC charging is described by the figure 5.2.2.2.5.1 with the Registration procedure in steps 4-24b. referring to IEC charging in clause 5.2.2.2.3.

##### 5.2.2.2.7	Registration with AMF re-allocation – ECUR

AMF re-allocation IEC charging is described by the figure 5.2.2.2.5.1 with the Registration procedure in steps 4-24b. referring to ECUR charging in clause 5.2.2.2.4.

##### 5.2.2.2.8	Deregistration – PEC charging

The Deregistration message flows are applicable to:

-	UE-initiated deregistration

-	Network-initiated Deregistration.

The following figure 5.2.2.2.8.1 describes a UE-initiated deregistration in PEC charging, based on figure 4.2.2.3.2-1 of TS 23.502 [202] description:

Figure 5.2.2.2.8.1: UE-initiated Deregistration – PEC charging

1.	Deregistration procedure initiated by UE.

1ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE deregistration.

1ch-b. The CHF creates the CDR for this deregistration.

1ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

7: The AMF sends NAS message Deregistration Accept to UE. In case of Deregistration type is switch-off, the Deregistration Accept message is not sent.

The following figure 5.2.2.2.8.2 describes a Network-initiated Deregistration in PEC charging, based on figure 4.2.2.3.3-1 of TS 23.502 [202] description:

Figure 5.2.2.2.8.2: Network-initiated Deregistration – PEC charging

1.	AMF receives Nudm_UECM_DeregistrationNotification from UDM.

2.	Deregistration Request sent towards the UE. In case of Implicit Deregistration, the Deregistration Request message is not sent.

2ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE successful deregistration.

2ch-b. The CHF creates the CDR for this deregistration.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

[3-8]. Deregistration procedure steps.

##### 5.2.2.2.9	Registration – Untrusted non-3GPP access - PEC charging

The following figure 5.2.2.2.9.1 describes UE Registration via Untrusted non-3GPP Access, in PEC charging, based on figure 4.12.2.2-1 of TS 23.502 [202] description.

Figure 5.2.2.2.9.1: Registration via Untrusted non-3GPP Access – PEC

1-11b. UE registration procedure via N3IWF.

12.	Upon successful procedure, NAS Registration Accept sent to N3IWF by AMF.

12ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE successful registration.

12ch-b. The CHF creates the CDR for this registration.

12ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

##### 5.2.2.2.10	Registration – Untrusted non-3GPP access - IEC

The following figure 5.2.2.2.10.1 describes UE Registration via Untrusted non-3GPP Access in IEC, based on figure 4.12.2.2-1 of TS 23.502 [202] description.

Figure 5.2.2.2.10.1: Registration via Untrusted non-3GPP Access – IEC

1-11b. UE registration procedure via N3IWF.

11c. Registration procedure in IEC with AMF interactions with PCF, UDM, as specified in clause 5.2.2.2.3.

11cch-a. to 11cch-c. Same steps as 16ch-a to 16ch-c of Figure 5.2.2.2.3.1.

##### 5.2.2.2.11	Registration – Untrusted non-3GPP access - ECUR

The following figure 5.2.2.2.11.1 describes UE Registration via Untrusted non-3GPP Access in ECUR, based on figure 4.12.2.2-1 of TS 23.502 [202] description.

Figure 5.2.2.2.11.1: Registration via Untrusted non-3GPP Access – ECUR

1-11b. UE registration procedure via N3IWF.

11c. Registration procedure with AMF interactions with PCF, UDM, as specified in clause 5.2.2.2.3.

11cch-a. to 11cch-c. Same steps as 16ch-a to 16ch-c of Figure 5.2.2.2.4.1.

12ch-a. to 12ch-c. Same steps as 21ch-a to 21ch-c of Figure 5.2.2.2.4.1.

##### 5.2.2.2.12	Registration for SMS over NAS – PEC charging

The following figure 5.2.2.2.12.1 describes a registration in 5GS for SMS over NAS in PEC charging, based on figure 4.13.3.1-1 TS 23.502 [202], where the "SMS supported" indication is included in Registration Request from the UE.

Figure 5.2.2.2.12.1: Registration for SMS over NAS – PEC charging

Steps 1-7: SMS service activation in the SMSF

Step 8: SMSF answers to the AMF with Nsmsf_SMService_Activate service

9ch-a to 9ch-c: Same steps as 21ch-a to 21ch-c of Figure 5.2.2.2.2.1. The "SMS allowed" indication is included in addition.

##### 5.2.2.2.13	Registration for SMS over NAS – IEC

The following figure 5.2.2.2.13.1 describes a registration in 5GS for SMS over NAS in IEC, based on clause 5.2.2.2.12.

Figure 5.2.2.2.13.1: Registration for SMS over NAS – IEC

Steps 1-4: UE Registration for SMS.

4ch-a to 4ch-c: Same steps as 16ch-a to 16ch-c of Figure 5.2.2.2.2.1. The "SMS allowed" indication is included.

Step 5-9: Registration for SMS procedure

##### 5.2.2.2.14	Registration for SMS over NAS – ECUR

The following figure 5.2.2.2.14.1 describes a registration in 5GS for SMS over NAS in ECUR, based on clause 5.2.2.2.12.

Figure 5.2.2.2.14.1: Registration for SMS over NAS – ECUR

Steps 1-4: UE Registration for SMS.

4ch-a to 4ch-c: Same steps as 16ch-a to 16ch-c of Figure 5.2.2.2.4.1. The "SMS allowed" indication is included.

Step 5-9: Registration for SMS procedure

9ch-a to 9ch-c: Same steps as 21ch-a to 21ch-c of Figure 5.2.2.2.4.1.

##### 5.2.2.2.15	Deregistration for SMS over NAS – PEC charging

Deregistration procedures for SMS over NAS are specified in TS 23.502 [202] clause 4.13.3.2:

-	When based on subsequent Registration Request message from the UE, the registration charging scenario in clause 5.2.2.2.2 or 5.2.2.2.3 applies.

-	When based on procedures in which UE is determined as deregistered by the AMF, the Network-initiated deregistration charging scenario in clause 5.2.2.2.8 applies.

#### 5.2.2.3	Connection management charging from AMF

##### 5.2.2.3.1	General

The clause below describes connection management charging, which includes:

-	N2 connection for this UE between the AN and the AMF for 3GPP access and untrusted non-3GPP access.

The TS 23.502 [201] service request procedures and flows are used to describe one of the N2 connection establishment cases. The procedures for N2 connection establishment are specified in TS 38.413 [203] for 3GPP access and exceptions in TS 29.413 [204] for untrusted non-3GPP, under the establishment of UE-associated logical NG-connection.

##### 5.2.2.3.2	UE Triggered Service Request

The following figure 5.2.2.3.2.1 describes UE N2 connection during UE Triggered Service Request, in event based offline charging, based on figure 4.2.3.2-1 of TS 23.502 [202] description.

Figure 5.2.2.3.2.1: UE Triggered Service Request –PEC  charging

1-13.	Service request procedure initiated by UE.

14. The (R)AN sends N2 Request ACK to AMF.

14ch-a. The AMF sends Charging Data Request [Event] to CHF for the N2 connection established for the UE.

14ch-b. The CHF creates the CDR for this N2 connectionestablishment.

14ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

15-22.	Service request procedure continuation.

In case of untrusted non-3GPP, the (R)AN corresponds to an N3IWF, and the figure 4.2.3.2-1 of TS 23.502 [202] description applies with the exceptions specified in clause 4.12.4.1 of TS 23.502 [202].

##### 5.2.2.3.3	Network Triggered Service Request

Based on figure 4.2.3.3-1 of TS 23.502 [202] description, a Service Request procedure from the UE is triggered from the Network, and corresponding N2 connection event offline charging in clause 5.2.2.3.2 applies.

##### 5.2.2.3.4	AN release

The following figure 5.2.2.3.4.1 describes UE N2 connection release in event based offline charging, based on figure 4.2.6-1 of TS 23.502 [202] description.

Figure 5.2.2.3.4.1: AN release –PEC  charging

1a-3.	Release initiated by the (R)AN.

4. The AMF receives an N2 UE Context Release Complete.

4ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE N2 connection release.

4ch-b. The CHF creates the CDR for this N2 connection release.

4ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

5-7.	AN release procedure continuation.

In case of untrusted non-3GPP, the procedure is specified in figure 4.12.4.2-1 of TS 23.502 [202], and the figure 5.2.2.3.4.1 applies with following differences:

-	(R)AN corresponds to an N3IWF.

-	step 1a is replaced by conditions in figure 4.12.4.2-1 of TS 23.502 [202]

#### 5.2.2.4	Location reporting charging

##### 5.2.2.4.1	General

The clause below describes location reporting charging.

##### 5.2.2.4.2	Location reporting

The following figure 5.2.2.4.2.1 describes Location reporting charging, in Post Event Charging.

Figure 5.2.2.4.2.1: Location reporting – Post Event Charging

1. A New N2 connection is established between NG-RAN and AMF and Location Reporting charging is activated in the AMF charging profile, potentially with a list of Presence Reporting Area(s).

2. The AMF starts the procedure based on figure 4.10-1 of TS 23.502 [202] description and sends "Location Reporting Control" to NG-RAN indicating UE changes of location or presence in Presence Reporting Area(s) need to be reported.

3. Location Report received by AMF includes the UE current location, and potentially presence in Presence Reporting Area(s).

3ch-a. The AMF sends Charging Data Request [Event, location reporting] to CHF with the current user location, and initial status(es) of presence in subscribed PRA(s) if requested in step 1.

3ch-b. The CHF creates the CDR for this location reporting.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event, location reporting] to the AMF.

4. Change in UE Location or presence in Presence Reporting Area(s) occur per AMF criteria provided in step 2.

5. Location Report is received by AMF.

5ch-a. The AMF sends Charging Data Request [Event, location reporting] to CHF with the new user location, and PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s) if any.

5ch-b. The CHF creates the CDR for this location reporting.

5ch-c. The CHF acknowledges by sending Charging Data Response [Event, location reporting] to the AMF.

#### 5.2.2.5	Roaming - Registration management charging from AMF

##### 5.2.2.5.1	General

In roaming, for registration management charging, the AMF in VPLMN:

-	may support PEC, IEC or ECUR scenario with H-CHF in HPLMN;

-	shall support PEC scenario with V-CHF in VPLMN.

##### 5.2.2.5.2	Registration - PEC to HPLMN

The following figure 5.2.2.5.2.1 describes a Registration charging message flow in PEC to HPLMN scenario for roaming, based on figure 4.2.2.2.2-1 of TS 23.502 [202] description

Figure 5.2.2.5.2.1: Roaming Registration – PEC to HPLMN

1-20a.	Registration procedure initiated by UE per steps 1 to 20a of figure 4.2.2.2.2-1 of TS 23.502, with PCF, AUSF and UDM in HPLMN.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a. Upon Registration completed trigger, the AMF in VPLMN may send Charging Data Request [Event] to H-CHF for the UE successful registration, indicating "roamer out".

21ch-b. The H-CHF creates the CDR for this registration.

21ch-c. The H-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21ch-d. The AMF in VPLMN sends Charging Data Request [Event] to the V-CHF for the UE successful registration, indicating "roamer in".

21ch-e. The V-CHF creates the CDR for this registration.

21ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

##### 5.2.2.5.3	Registration – IEC to HPLMN

The following figure 5.2.2.5.3.1 describes a Registration charging message flows in IEC to HPLMN scenario for roaming, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.5.3.1: Roaming Registration – IEC to HPLMN

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502, with PCF, AUSF and UDM in HPLMN.

16ch-a. Upon Registration checked trigger, the AMF in VPLMN sends Charging Data Request [Event] to H-CHF for the UE registration to be granted authorization, rated and accounted, indicating "roamer out".

16ch-b. Accounting and Rating control by the H-CHF for the registration and CDR creation.

16ch-c. The H-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN, granting authorization to the registration.

21ch-a. Upon Registration completed trigger, the AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful registration, indicating "roamer in".

21ch-b. The V-CHF creates the CDR for this registration.

21ch-c. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

17 to 24. Same steps as in Figure 5.2.2.2.2.1.

##### 5.2.2.5.4	Registration – ECUR to HPLMN

The following figure 5.2.2.5.4.1 describes a Registration charging message flows in ECUR to HPLMN scenario for roaming, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.5.4.1: Roaming Registration – ECUR to HPLMN

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502, with PCF, AUSF and UDM in HPLMN.

16ch-a. Upon Registration checked trigger, the AMF sends Charging Data Request [Initial] to H-CHF for the UE registration to be granted authorization, indicating "roamer out".

16ch-b. Account and Rating control by the H-CHF for the registration and open a CDR.

16ch-c. The H-CHF acknowledges by sending Charging Data Response [Initial] to the AMF in VPLMN, granting authorization to the registration.

17 to 21. Same steps as in Figure 5.2.2.2.2.1.

21ch-a. The AMF in VPLMN sends Charging Data Request [Termination] to H-CHF for the UE successful registration.

21ch-b. The H-CHF closes the CDR for this registration.

21ch-c. The H-CHF acknowledges by sending Charging Data Response [Termination] to the AMF in VPLMN.

21ch-d. The AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful registration, indicating "roamer in".

21ch-e. The V-CHF creates the CDR for this registration.

21ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

##### 5.2.2.5.5	Deregistration – PEC to HPLMN

The following figure 5.2.2.5.5.1 describes a deregistration UE-initiated or Network-initiated in PEC to HPLMN for roaming, based on figure 4.2.2.3.2-1 and figure 4.2.2.3.3-1 of TS 23.502 [202] description:

Figure 5.2.2.5.5.1: Roaming Deregistration – PEC to HPLMN

1.	Deregistration Request from the UE or towards the UE.

1ch-a. Upon Deregistration trigger, the AMF in VPLMN sends Charging Data Request [Event] to H-CHF for the UE successful deregistration, indicating "roamer out".

1ch-b. The H-CHF creates the CDR for this deregistration.

1ch-c. The H-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

1ch-d. The AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful deregistration, indicating "roamer in".

1ch-e. The V-CHF creates the CDR for this deregistration.

1ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN

[6-8]. Deregistration procedure steps.

### 5.2.3	CDR generation

#### 5.2.3.1	Introduction

- The CHF CDRs for 5G connection and mobility charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

- The following clauses describe in detail the conditions for generating the CHF CDR, which shall be supported by the CHF.

#### 5.2.3.2	Triggers for CHF CDR

##### 5.2.3.2.1	General

A 5G connection and mobility charging CHF CDR is used to collect charging information related to 5G connection and mobility chargeable events for PEC, IEC and ECUR.

##### 5.2.3.2.2	Triggers for CHF CDR generation

A CHF CDR shall be generated by the CHF for each received Charging Data Request [Event].

##### 5.2.3.2.3	Triggers for CHF CDR opening

A CHF CDR shall be opened when the CHF receives Charging Data Request [Initial].

##### 5.2.3.2.4	Triggers for CHF CDR closure

The CHF CDR shall be closed when the CHF receives Charging Data Request [Termination].

### 5.2.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [54].

### 5.2.5	Bam CDR file transfer

Details of the Bam protocol application are specified in TS 32.297 [52].

# 6.	Definition of charging information

## 6.1	Data description for 5G connection and mobility charging

### 6.1.1	Message contents

#### 6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [57] and include charging information. The Charging Data Request can be of type [Event].

Table 6.1.1.1.1 describes the use of these messages for converged charging.

Table 6.1.1.1.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | AMF | CHF |
| Charging Data Response | CHF | AMF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the AMF as used for 5G connection and mobility converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] In case Subscriber Identifier (i.e. SUPI) is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Charging Identifier | - | This field is not applicable. |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Notify URI | - | This field is not applicable. |
| Service Specification Information | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G connection and mobility specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57]. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| AMF Charging Profile | OC | This field holds the applicable AMF Charging Profile |
| Registration Charging Information | OM | This field holds the registration specific information described in clause 6.2.1.2 |
| N2 connection Charging Information | OM | This field holds the N2 connection specific information described in clause 6.2.1.3 |
| Location Reporting Charging information | OM | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G connection and mobility converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | This field is not applicable. |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G connection and mobility specific triggers described in clause 5.2.1. |
| Multiple Unit Information | OC | This field is applicable for ECUR. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| AMF Charging Profile | OC | This field holds the AMF Charging Profile to be applied |
| Location Reporting Charging information | Oc | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

### 6.1.2	Ga message contents

See clause 5.2.4.

### 6.1.3	CDR description on the Bam interface

#### 6.1.3.1	General

This clause describes the CDR content and format generated for 5G connection and mobility charging

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [51].

#### 6.1.3.2	Registration charging CHF CDR data

If enabled, CHF CDRs for Registration charging shall be produced for each registration and deregistration

The fields of Registration charging CHF CDR are specified in table 6.1.3.2.1.

Table 6.1.3.2.1: Registration charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. |
| NF Consumer Information | M | This field holds the information of the AMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. AMF) |
| NF Name | OC | This field holds the name of the AMF used. |
| NF Address | OC | This field holds the IP Address of the AMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the AMF. |
| Record Opening Time | M | Described in TS 32.298 [51] |
| Duration | M | Described in TS 32.298 [51] |
| Record Sequence Number | C | Described in TS 32.298 [51] |
| Cause for Record Closing | M | Described in TS 32.298 [51] |
| Diagnostics | OM | Described in TS 32.298 [51] |
| Local Record Sequence Number | OM | Described in TS 32.298 [51] |
| Record Extensions | OC | Described in TS 32.298 [51] |
| Registration Charging Information | OM | This field holds the 5G registration specific information defined in clause 6.2.1.2. |

#### 6.1.3.3	N2 connection charging Information CHF CDR data

If enabled, CHF CDRs for N2 connection charging shall be produced for each N2 connection establishment and release.

The fields of N2 connection charging CHF CDR are specified in table 6.1.3.3.1.

Table 6.1.3.3.1: N2 connection charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. |
| NF Consumer Information | M | This field holds the information of the AMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. AMF) |
| NF Name | OC | This field holds the name of the AMF used. |
| NF Address | OC | This field holds the IP Address of the AMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the AMF. |
| Record Opening Time | OC | Described in TS 32.298 [51] |
| Duration | M | Described in TS 32.298 [51] |
| Record Sequence Number | C | Described in TS 32.298 [51] |
| Cause for Record Closing | M | Described in TS 32.298 [51] |
| Local Record Sequence Number | OM | Described in TS 32.298 [51] |
| Record Extensions | OC | Described in TS 32.298 [51] |
| N2 connection charging Information | OM | This field holds the N2 connection specific information defined in clause 6.2.1.3 |

#### 6.1.3.4	Location reporting charging CHF CDR data

If enabled, CHF CDRs for Location reporting charging shall be produced for each Location reporting.

The fields of Location reporting charging CHF CDR are specified in table 6.1.3.4.1.

Table 6.1.3.4.1: Location reporting charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. |
| NF Consumer Information | M | This field holds the information of the AMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. AMF) |
| NF Name | OC | This field holds the name of the AMF used. |
| NF Address | OC | This field holds the IP Address of the AMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the AMF. |
| Record Opening Time | M | Described in TS 32.298 [51] |
| Duration | M | Described in TS 32.298 [51] |
| Record Sequence Number | C | Described in TS 32.298 [51] |
| Cause for Record Closing | M | Described in TS 32.298 [51] |
| Local Record Sequence Number | OM | Described in TS 32.298 [51] |
| Record Extensions | OC | Described in TS 32.298 [51] |
| Location reporting charging Information | OM | This field holds the Location reporting specific information defined in clause 6.2.1.4. |

## 6.2	5G connection and mobility charging specific parameters

### 6.2.1	Definition of 5G connection and mobility charging information

#### 6.2.1.1	General

The Charging Information parameter used for 5G connection and mobility charging is provided in the following clauses.

#### 6.2.1.2	Definition of Registration charging information

Specific charging information used for 5G registration charging is provided within the Registration Charging Information.

The detailed structure of the Registration Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of Registration Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Registration message type | M | This field holds the message type received by the AMF associated to registration: registration (initial, mobility, periodic, emergency), deregistration. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It may be used for identifying the user and shall be present in case Subscriber Identifier is not present. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| Roamer In Out | OC | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |
| UE 5GMM Core Network Capability | OC | This field holds the UE 5GMM Core Network Capability included in UE MM Core network capabilities, as specified in clause 5.4.4a TS 23.501 [200] |
| MICO Mode Indication | OC | This field indicates whether the MICO Mode is applicable or not. |
| SMS Supported Indication | OC | This field indicates whether SMS is supported or not. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT) currently serving the UE. |
| TAI List | OC | This field holds a set of tracking areas composing the Registration Area. |
| User Location Info | OC | This field indicates details of where the UE is currently located (access-specific user location information). |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Mobility Restrictions | OC | This field holds the Mobility Restrictions applicable to the UE: RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. |
| Requested NSSAI | OC | This field holds the requested NSSAI. |
| Allowed NSSAI | OC | This field holds the allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |
| Rejected NSSAI | OC | This field holds the rejected NSSAI. |
| NSSAI mapping list | OC | This field holds mapping of each S-NSSAI of the Allowed NSSAI to the S-NSSAIs of the Subscribed S-NSSAIs. |
| AMF UE NGAP ID | OM | This fields holds the UE association over the N2 interface within the AMF. |
| RAN UE NGAP ID | OM | This fields holds the RAN UE NGAP ID over N2 interface. |
| RAN Node Id | OM | This fields holds the Global RAN Node ID. |

#### 6.2.1.3	Definition of N2 Connection charging information

Specific charging information used for 5G N2 Connection charging is provided within the N2 connection charging Information.

The detailed structure of the N2 connection Charging Information can be found in table 6.2.1.3.1.

Table 6.2.1.3.1: Structure of N2 Connection Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| N2 Connection message type | M | This field holds the N2 message type received by the AMF. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| User Location Info | OC | This field indicates details of where the UE is currently located (access-specific user location information). |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT). |
| AMF UE NGAP ID | OM | This fields holds the UE association over the N2 interface within the AMF. |
| RAN UE NGAP ID | OM | This fields holds the RAN UE NGAP ID over N2 interface. |
| RAN Node Id | OM | This fields holds the Global RAN Node ID. |
| Mobility Restrictions | OC | This field holds the Mobility Restrictions applicable to the UE: RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. |
| Allowed NSSAI | OC | This field holds the allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |
| RRC Establishment Cause | OC | This fields holds the reason for RRC Connection Establishment as received from the UE. |

#### 6.2.1.4	Definition of Location Reporting charging information

Specific charging information used for 5G Location Reporting charging is provided within the Location reporting charging Information.

The detailed structure of the Location reporting charging Information can be found in table 6.2.1.4.1.

Table 6.2.1.4.1: Structure of Location Reporting Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Location reporting message type | M | This field holds the location reporting message type received by the AMF. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| User Location Info | OM | This field indicates details of where the UE is currently located under the access type. |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Presence Reporting Area Information | OC | This field contains the Presence Reporting Area Information of UE as defined in TS 23.501 [200], comprising the Presence Reporting Area identifier(s) and an indication on whether the UE is inside or outside the Presence Reporting Area, if available. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT). |

### 6.2.2	Formal 5G connection and mobility charging parameter description

#### 6.2.2.1	5G connection and mobility CHF CDR parameters

The detailed definitions, abstract syntax and encoding of 5G connection and mobility CHF CDRs parameters are specified in TS 32.298 [51].

#### 6.2.2.2	5G connection and mobility resources attributes

The detailed definitions of resources attributes used for 5G connection and mobility charging are specified in TS 32.291 [58].

### 6.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by AMF for 5G connection and mobility converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.3.1 defines the basic structure of the supported fields in the Charging Data Request message for AMF converged charging.

Table 6.2.3.1: Supported fields in Charging Data Request message

| Information Element | Functionality of AMF | Reg. | N2 cnt | Loc. Report. |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | I/T/E | E | E |
| Session Identifier |  | ITE | E | E |
| Subscriber Identifier |  | ITE | E | E |
| NF Consumer Identification |  | ITE | E | E |
| Charging Identifier |  | - | - | - |
| Invocation Timestamp |  | ITE | E | E |
| Invocation Sequence Number |  | ITE | E | E |
| Retransmission Indicator |  | - | - | - |
| One-time Event |  | --E | E | E |
| One-time Event Type |  | --E | E | E |
| Notify URI |  | - | - | - |
| Service Specification Information |  | ITE | E | E |
| Triggers |  | ITE | - | - |
| Multiple Unit Usage |  | ITE | - | - |
| AMF Identifier |  | ITE | E | E |
| AMF Charging Profile |  | I-E | E | E |
| Registration Charging Information |  |  |  |  |
| Registration Message type |  | ITE | - | - |
| User Information |  | ITE | - | - |
| UE 5GMM Core Network Capability |  | ITE | - | - |
| MICO Mode Indication |  | ITE | - | - |
| SMS Supported Indication |  | ITE | - | - |
| Access Type |  | ITE | - | - |
| RAT Type |  | ITE | - | - |
| TAI List |  | ITE | - | - |
| User Location Info |  | ITE | - | - |
| PSCell Information |  | ITE | - | - |
| UE Time Zone |  | ITE | - | - |
| Mobility Restrictions |  | ITE | - | - |
| Requested NSSAI |  | ITE | - | - |
| Allowed NSSAI |  | ITE | - | - |
| Rejected NSSAI |  | ITE | - | - |
| NSSAI mapping list |  | ITE | - | - |
| AMF UE NGAP ID |  | ITE | - | - |
| RAN UE NGAP ID |  | ITE | - | - |
| RAN Node Id |  | ITE | - | - |
| N2 Connection Charging Information |  |  |  |  |
| N2 Connection Message type |  | - | E | - |
| User Information |  | - | E | - |
| User Location Info |  | - | E | - |
| PSCell Information |  | - | E | - |
| UE Time Zone |  | - | E | - |
| RAT Type |  | - | E | - |
| AMF UE NGAP ID |  | - | E | - |
| RAN UE NGAP ID |  | - | E | - |
| RAN Node Id |  | - | E | - |
| Mobility Restrictions |  | - | E | - |
| Allowed NSSAI |  | - | E | - |
| RRC Establishment Cause |  | - | E | - |
| Location Reporting Charging information |  |  |  |  |
| Location reporting Message type |  | - | - | E |
| User Information |  | - | - | E |
| User Location Info |  | - | - | E |
| PSCell Information |  | - | - | E |
| UE Time Zone |  | - | - | E |
| Presence Reporting Area Information |  | - | - | E |
| RAT Type |  | - | - | E |

Table 6.2.3.2 defines the basic structure of the supported fields in the Charging Data Response message for AMF converged charging.

Table 6.2.3.2: Supported fields in Charging Data Response message

| Information Element | Functionality of AMF | Reg. | N2 cnt | Loc. Report. |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | ITE | E | E |
| Session Identifier |  | ITE | E | E |
| Invocation Timestamp |  | ITE | E | E |
| Invocation Result |  | ITE | E | E |
| Invocation Sequence Number |  | ITE | E | E |
| Session Failover |  | I-- | - | - |
| Triggers |  | - | E | E |
| Multiple Unit information |  | - | - | - |
| AMF Charging Profile |  | I-E | E | E |
| Location Reporting Charging information. |
| Location reporting Message type |  | - | - | - |
| User Information |  | - | - | - |
| User Location Info |  | - | - | - |
| PSCell Information |  | - | - | - |
| UE Time Zone |  | - | - | - |
| Presence Reporting Area Information |  | - | - | E |
| RAT Type |  | - | - | - |

## 6.3	Bindings for 5G connection and mobility converged charging

This mapping between the Information Elements, resource attributes and CHF CDR parameters for 5G connection and mobility converged charging is described in clause 7 of TS 32.291 [58].

Annex A (normative):
Charging Characteristics

# A.1	General

A subscriber may have Charging Characteristics assigned to his subscription. Default Charging Characteristics may also be pre-provisioned on the AMF.

During UE Registration, the subscribed Charging Characteristics, if any, shall override the AMF pre-provisioned Charging Characteristics. The subscribed Charging Characteristics may be transferred from old AMF during UE context transfer or retrieved from UDM.

The Charging Characteristics parameter consists of a string of 16 bits designated as Behaviours (B), freely defined by Operators, as shown in TS 32.298 [51]. Each bit corresponds to an index to specific charging behaviour which is configured on a per operator basis and active when the bit is set to "1" value.

The following Table A.1.1 provides an example of Charging Characteristics for AMF:

Table A.1.1: Example of Charging Characteristics behaviours for AMF

|  |  | Registration charging |  | N2 connections charging |  | Location reporting charging |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Behaviour Index | Primary and Secondary CHF addresses | Active | Charging scenario | Active | Charging scenario | Active | Charging scenario | Level |
| 0 | URI 1 URI 2 | Yes | ECUR | No | - | No | - |  |
| 1 | URI 1 URI 2 | Yes | IEC | Yes | PEC | Yes | PEC | TAI, Cell |
| 2 | URI 1 URI 2 | Yes | PEC | No | - | Yes | PEC | Cell |
| 3 | URI 1 URI 2 | No | - | Yes | PEC | Yes | PEC | TAI |
| … | … | … | … | … | … | … | … | … |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-12 | SA#86 | SP-191183 |  |  |  | Presented for approval | 2.0.0 |
| 2019-12 | SA#86 |  |  |  |  | Change control version | 16.0.0 |
| 2020-07 | SA#88-e | SP-200509 | 0001 | 1 | F | Add the Retransmission Indicator | 16.1.0 |
| 2020-07 | SA#88-e | SP-200509 | 0002 | 1 | F | Correct the Event offline charging scenarios | 16.1.0 |
| 2021-03 | SA#91e | SP-210163 | 0003 | - | F | Correction on Registration flows and message content | 16.2.0 |
| 2021-03 | SA#91e | SP-210163 | 0004 | - | F | Correction on UE MM Core Network Capability | 16.2.0 |
| 2022-12 | SA#98e | SP-221171 | 0008 | - | F | Correction on the Triggers for AMF | 16.3.0 |