# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part 3 of a multi-part conformance test specification for UE. The specification contains a TTCN (TTCN -2 and TTCN-3) design frame work and the detailed test specifications in TTCN for UE at the Uu interface.

3GPP TS 34.123-1 [1]: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

3GPP TS 34.123-2 [2]: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

3GPP TS 34.123-3: "Abstract Test Suite (ATS)" (the present document).

# 1	Scope

The present document specifies the protocol conformance testing in TTCN for the 3GPP User Equipment (UE) at the Uu interface.

The present document is the 3rd part of a multi-part test specification, 3GPP TS 34.123. The following TTCN test specification and design considerations can be found in the present document:

-	the overall test suite structure;

-	the testing architecture;

-	the test methods and PCO definitions;

-	the test configurations;

-	the design principles, assumptions, and used interfaces to the TTCN tester (System Simulator);

-	TTCN styles and conventions;

-	the partial PIXIT proforma;

-	the TTCN.MP and TTCN.GR forms for the mentioned protocols tests.

The Abstract Test Suites designed in the document are based on the test cases specified in prose (3GPP TS 34.123-1 [1]).

The present document is valid for UE implemented according to 3GPP releases starting from Release 1999 up to the Release indicated on the cover page of the present document.

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[2]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[3]	3GPP TS 34.108: "Common test environments for User Equipment (UE) conformance testing".

[4]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[5]	3GPP TR 21.905: "Vocabulary for 3GPP specifications".

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 23.101: "General UMTS architecture".

[8]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[9]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[10]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[11]	3GPP TS 24.012: "Short Message Service Cell Broadcast (SMSCB) support on the mobile radio interface".

[12]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[13]	3GPP TS 25.224: "Physical layer procedures (TDD)".

[14]	3GPP TS 25.301: "Radio interface protocol architecture".

[15]	3GPP TS 25.303: "Interlayer procedures in connected mode".

[16]	3GPP TS 25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[17]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[18]	3GPP TS 25.322: "Radio Link Control (RLC) protocol specification".

[19]	3GPP TS 25.323: "Packet Data Convergence Protocol (PDCP) specification".

[20]	3GPP TS 25.324: "Broadcast/Multicast Control (BMC)".

[21]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[22]	3GPP TS 27.005: "Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE-DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[23]	3GPP TS 27.007: "AT command set for 3G User Equipment (UE)".

[24]	3GPP TS 27.060: "Packet domain; Mobile Station (MS) supporting Packet Switched services".

[25]	3GPP TS 33.102: "3G security; Security architecture".

[26]	3GPP TS 51.010-1: "Mobile Station (MS) conformance specification; Part 1: Conformance specification".

[27]	ETSI TR 101 666 (V1.0.0): "Information technology; Open Systems Interconnection Conformance testing methodology and framework; The Tree and Tabular Combined Notation (TTCN) (Ed. 2++)".

[28]	ITU-T Recommendation X.691 (1997) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[29]	ISO/IEC 8824 (all parts): "Information technology - Abstract Syntax Notation One (ASN.1)".

[30]	IETF RFC 2507: "IP Header Compression".

[31]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".
3GPP TS 05.02: "Digital cellular telecommunications system (Phase 2+); Multiplexing and multiple access on the radio path".

[32]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".
3GPP TS 04.60: "Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[33]	3GPP TS 44.064: "Mobile Station - Serving GPRS Support Node (MS-SGSN) Logical Link Control (LLC) layer specification".

[34]	3GPP TS 23.038: "Alphabets and language-specific information".

[35]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)".

[36]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[37]	ETSI ETR 141: "Methods for Testing and Specification (MTS); Protocol and profile conformance testing specifications; The Tree and Tabular Combined Notation (TTCN) style guide".

[38]	ETSI TR 101 101: "Methods for Testing and Specification (MTS); TTCN interim version including ASN.1 1994 support [ISO/IEC 9646-3] (Second Edition Mock-up for JTC1/SC21 Review)".

[39]	ITU-T Recommendation X.680: "Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[40]	3GPP TS 25.211: "Physical channels and mapping of transport channels onto physical channels (FDD)".

[41]	ISO/IEC 9646 (all parts): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework".

[42]	3GPP TS 44.006: "Mobile Station - Base Stations System (MS - BSS) Interface Data Link (DL) layer specification".

[43]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".
3GPP TS 04.18: "Digital cellular telecommunications system (Phase 2+); Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".

[44]	3GPP TR 25.925: "Radio interface for Broadcast/Multicast Services".

[45]	ITU-T Recommendation O.153: "Basic parameters for the measurement of error performance at bit rates below the primary rate".

[46]	IETF RFC 1144: "Compressing TCP/IP headers for low-speed serial links".

[47]	ITU-T Recommendation V.42bis: "Data compression procedures for data circuit-terminating equipment (DCE) using error correction procedures".

[48]	ITU-T Recommendation V.44: "Data compression procedures".

[49]	3GPP TS 44.008: "Mobile radio interface layer 3 specification".
3GPP TS 04.08: "Digital cellular telecommunications system (Phase 2+); Mobile radio interface layer 3 specification".

[50]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[51]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[52]	ITU-T Recommendation Q.773: "Signalling System No. 7 - Transaction Capabilities Formats and Encoding".

[53]	ITU-T Recommendation X.880: "Information Technology - Remote Operations: Concepts, Model and Notation".

[54]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

[55]	3GPP TS 34.022: "Radio Link Protocol (RLP) for circuit switched bearer and teleservices".

[56]	Void

[57]	3GPP TS 25.433: "UTRAN Iub interface Node B Application Part (NBAP) signalling".

[58]	3GPP TS 25.212: "Multiplexing and channel coding (FDD)".

[59]	3GPP TS 25.306: "UE Radio Access capabilities".

[60]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[61]	Void

[62]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[63]	3GPP TS 37.571-2: “Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance”.

[64]	3GPP TS 37.571-4: "Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 4: Test suites".

[65]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[66]	3GPP TS 25.213: "Spreading and modulation (FDD)".

[67]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[68]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) ".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 34.123-1 [1] apply.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TS 34.123-1 [1], 3GPP TS 24.008 [9], 3GPP TS 25.331 [21] and TR 101 666 [27] apply.

# 4	Requirements on the TTCN development

A number of requirements are identified for the development and production of TTCN specification for 3GPP UE at Uu interface.

1.	Top-down design, following 3GPP TS 34.123-1 [1], 3GPP TS 34.108 [3] and 3GPP TS 34.109 [4].

2.	A unique testing architecture and test method for testing all protocol layers of UE.

3.	Uniform TTCN style and naming conventions.

4.	Improve TTCN readability.

5.	Using TTCN-2++ (TR 101 666 [27]) for R99, Release 4, Release 5, Release 6, Release 7, Release 8 and Release 9 and TTCN-3 (ETSI ES 201 873-1[65]) for Release 10 and later Releases.

6.	TTCN specification feasible, implementable and compilable.

7.	Test cases shall be designed in a way for easily adaptable, upwards compatible with the evolution of the 3GPP core specifications and the future Releases.

8.	The test declarations, data structures and data values shall be largely reusable.

9.	Modularity and modular working method.

10.	NAS ATS should be designed being independent from the radio access technologies.

11.	Minimizing the requirements of intelligence on the emulators of the lower testers. Especially the functionality of the RRC emulator in the TTCN tester should be reduced and simplified; the behaviours should be standardized as the TTCN RRC test steps in the TTCN modular library.

12.	Giving enough design freedom to the test equipment manufacturers.

13.	Maximizing reuse of ASN.1 definitions from the relevant core specifications.

In order to fulfil these requirements and to ensure the investment of the test equipment manufacturers having a stable testing architecture for a relatively long period, a unique testing architecture and test method are applied to the 3GPP UE protocol tests.

# 5	TTCN-2 ATS structure

The total TTCN specification for the UE testing is structured in a number of separate layered ATSs. The number of ATS being produced corresponds to the number of the 3GPP core specifications referred. The separation of ATSs reduces the size of ATSs. The layer-specific test preambles and test data can be confined to one test suite and parallel development of test suites can be facilitated. The separation of ATSs enables also easily to follow the evolution of the core specifications.

-	NAS ATSs:

1)	GSM MAP L3 ATS including MM, CC, GMM, SM test groups;

2)	SMS ATS;

3)	A-GPS ATS

-	AS ATSs:

1)	RRC ATS including Single cell and multicell test group;

2)	RLC ATS;

3)	MAC ATS;

4)	BMC ATS;

5)	PDCP ATS;

6)	RAB ATS;

7)	IR_U ATS;

8)	HSD_ENH ATS (Rel-5 or later);

9)	HSU_ENH ATS (Rel-6 or later);

10)	MBMS ATS (Rel-6 or later);

11)	HSPA7_ENH ATS (Rel-7 or later);

12)	HSPA8_ENH ATS (Rel-8 or later);

13)	HSPA9_ENH ATS (Rel-9 or later).

## 5.1	Modularity

The modular TTCN approach is used for the development of the 3GPP ATS specification work. Three modules, BasicM, RRC_M and L3M are installed.

### 5.1.1	Module structure

The module structure is shown in Figure 5.1.1.

Figure 5.1.1: Module structure

The BasicM (Basic Module) is a minimum module commonly for the layer 2 and layer 3 testing. The L3M (Layer 3 Module) contains all the items to be shared by the RRC, NAS, SMS, RAB, IR_U/G, A-GPS, HSD_ENH, HSU_ENH and MBMS ATSs. NAS is applied to the NAS, A-GPS, HSD_ENH, HSU_ENH, MBMS, HSPA7_ENH, HSPA8_ENH and HSPA9_ENH ATSs. The RRC_M is a module containing common object for RRC, RAB, IR_U/G, SMS, A-GPS, HSD_ENH, HSU_ENH, MBMS, HSPA7_ENH, HSPA8_ENH and HSPA9_ENH ATSs.

### 5.1.2	Contents of the modules

The BasicM module includes objects related to the RRC, the layer 2 and the physical layer. It includes also all test steps needed by the layer 2 and layer 3 test cases for configurations and all objects related to the definition of the steps:

-	Common test steps and default test steps defined as generic procedures in 3GPP TS 34.108 [3];

-	RRC declarations related to the steps: types, timers, PDU types, ASP type, PCOs, TSOs, constants;

-	Related ICS and IXIT parameters needed for testing and respectively defined in 3GPP TS 34.123-2 [2] and the present document;

-	Defaults constraints based on the default message contents defined in 3GPP TS 34.108 [3];

-	MMI PCO and ASPs;

-	All TTCN objects related to the SS configuration, e.g. PCOs, declaration of the components.

The L3M module includes the NAS configuration steps and all related TTCN objects:

-	Common test steps and default test steps defined as generic procedures in 3GPP TS 34.108 [3];

-	NAS declarations related to these steps: types, PDU, ASP, PCOs, TSOs, constants;

-	Related ICS and IXIT parameters needed for testing and respectively defined in 3GPP TS 34.123-2 [2] and the present document;

-	Default constraints based on the default message contents defined in 3GPP TS 34.108 [3].

The RRC_M module includes the RRC steps common to RRC and RAB test cases and all related TTCN objects.

### 5.1.3	Example of a working platform

Figure 5.1.3 shows the working platform for the user that is writing the SMS test cases.

Figure 5.1.3: An example of working platform for SMS

# 6	Test method and testing architecture

## 6.1	Test method

The distributed single party test method is used for the UE testing. The lower tester configures the emulator and communicates with the UE under test via the emulator. An upper tester interfaces UE as (E)MMI.

All common parts in 3GPP TS 34.108 [3], 3GPP TS 34.109 [4] and 3GPP TS 34.123-2 [2] are developed in a TTCN library including the declarations, default constraints, preambles and postambles. They have the following characteristics:

-	Very complex;

-	Worked in different layers;

-	Including data representing the radio parameters for SS setting and the data representing the UE capabilities (PICS parameters);

-	Including the generic procedures to bring the UE into certain test states or a test mode (C-plane);

-	Setting RABs at U-plane and SRBs in C-plane;

-	Being used by every test cases no matter which layer the test case belongs to;

-	No affect on the test verdict of PASS or FAIL.

The layer-specific test cases have the characteristics:

-	relatively simple and straight forward;

-	having narrow test scope and test purposes;

-	test scenarios in a single layer (one PCO);

-	assigning the test verdict.

## 6.2	TTCN-2 Testing architecture

A unique testing architecture is shown in Figure 6.2.

Figure 6.2: A unique testing architecture

### 6.2.1	Lower Tester (LT)

The Lower Tester (LT) provides the test means for the execution of the test cases for CC, SM, MM, GMM, SMS, RRC, RLC, MAC, PDCP, BMC, RAB, IR_U/G, A-GPS, HSD_ENH, HSU_ENH, MBMS, HSPA7_ENH, HSPA8_ENH and HSPA9_ENH. The LT provides also the RLC, MAC and PHY emulators to communicate with the UE. The configuration and initialization of the emulators are control by the TTCN via ASPs.

### 6.2.2	Configuration and initialization

A number of TTCN test steps are designed for the generic setting.

1)	Configuration of L1 of the tester, such as the cells, Physical channels and common transport channels via CPHY-PCO, configuration of MAC via CMAC-PCO and configuration of RLC layer via CRLC-PCO.

2)	Sending system information via TR-PCO.

3)	Establishment RRC connection via AM or UM-PCO.

4)	Assigning a radio bearer via AM-PCO.

5)	MM /GMM registration via Dc-PCO.

6)	Establishment of a CS call or a PDP context via Dc-PCO.

7)	Setting security parameters and control of integrity via CRLC- and ciphering via CRLC- and CMAC-PCO.

### 6.2.3	Upper Tester (UT)

An Upper Tester (UT) exists in the test system. The UT interfaces toward UE with any optional EMMI (3GPP TS 34.109 [4], clause 7). TTCN communicates with the UT by passing coordination primitives via a Ut PCO. The primitives can either contain AT commands aiming at the automatic tests, or some informal commands as MMI, in order to request the UE for certain actions and to provide simple means for observations of UE.

### 6.2.4	TTCN-2

TTCN-2 is used as specification language based on TR 101 666 [27] (TTCN 2++). The importation of ASN.1 modules and modular TTCN-2 are two of the most important features used in the design of the ATSs.

The TTCN test suites have been designed to maximize the portability from the language TTCN-2 to TTCN-3.

### 6.2.5	Model extension

If a test case needs to handle a concurrent situation two or more LTs can be configured at the same time. The following test scenarios identified may require multiple testers in the test configuration.

### 6.2.6	Multiplexing of RLC services

For the RRC and NAS testing, the TTCN RRC test steps (on RB1 and RB2) and the RRC emulator (on RB3 and RB4 for the NAS messages) share the same service access point (AM SAP). The RLC emulator shall provide separate message queues (buffers) for the TTCN RRC test steps and the RRC emulator for the TTCN NAS test cases, according to the signalling radio bearer identities.

## 6.3	NAS test method and architecture

### 6.3.1	Test configuration

The NAS test method is shown in Figure 6.3.1.

Figure 6.3.1: NAS testing architecture

The single layer distributed test method is used.

The Point of Control and Observation (PCO) are defined as the Dc (Dedicated control) SAP. The NAS test verdicts are assigned depending on the behaviours observed at the PCO.

The TTCN tester provides the NAS TTCN test cases and steps with a simple RRC direct transfer function which buffers the NAS PDU data, converts the data from the NAS TTCN table format into ASN.1, or in reverse way, and delivers all lower layer services of AM-SAP for RB3 and RB4.

The NAS TTCN test cases make also intensively use of the RRC TTCN test steps, in order to:

-	Configure, initialize and control the L2 emulator;

-	Initialize the UE for testing.

The RRC test steps, which are called by the NAS test cases or steps, interface with the RLC PCOs (UM, AM and TR), the control PCOs CRLC, CMAC and CPHY.

The General control (Gc) SAP and the Notification (Nt) SAP are not applied. Messages exchanged via these SAPs will be replaced with the corresponding RRC TTCN test steps.

The Ut PCO (so called logical interface [4]) is served as the interface to the UE EMMI to allow a remote control of operations, which have to be performed during execution of a test case such as to switch the UE on/off, initiate a call, etc.

### 6.3.2	Routing UL NAS massages in SS

The UL NAS messages are embedded in RRC messages INITIAL / UL DIRECT TRANSFER. In the UE test, the received UL NAS messages can either be routed to the Dc PCO and verified at the NAS message level, or routed to AM PCO and verified at the RRC message level.

1)	RBid =3 at the SS side indicates that the UL NAS high priority messages to be routed to Dc PCO. RB3 applies to RRC_DataInd/Req.

2)	RBid= -16 at the SS side indicates the received messages to be routed to RLC AM PCO. RB-16 applies to RLC_DataInd/Req.

The RB3 and RB-16 do not coexist. The TTCN writer uses the MAC and RLC reconfigurations to re-map the RB and the corresponding logical channels. If RB3 has been configured, but a test case needs to re-map the logical channel from RB3 to RB-16 the following way is to replace RB3 with RB-16.

-	CMAC_CONFIG_REQ (reconfiguration, RB-16).

Re-mapping on RB-16 which appears in the transport channel and logical channel mapping list.

-	CRLC_CONFIG_REQ (reconfiguration, RB-16).

RB-16 appears in the routing info, in order to replace the original mapping on RB3.

Mapping from RB-16 to RB3 is done in the reverse way.

## 6.4	RRC and RAB test method and architecture

### 6.4.1	Test configuration

Figure 6.4.1: RRC testing architecture

The single layer distributed test method is used.

The PCOs are defined as the AM (Acknowledged Mode), UM (Unacknowledged Mode) and TM (Transparent Mode) SAPs. The RRC test verdicts are assigned depending on the behaviours observed at the PCO. The RRC TTCN interface also with the control PCOs CRLC, CMAC and CPHY, for the configuration, initialization and control of the System Simulator.

The RRC TTCN test cases also make use of the NAS TTCN test steps in order to:

-	Bring UE to Idle state;

-	Bring UE to state U10.

The NAS test steps, which are called by the RRC test cases or steps, interface with the Dc PCO.

The Ut PCO (so called logical interface [4]) is served as the interface to the UE EMMI to allow a remote control of operations, which have to be performed during execution of a test case such as to switch the UE on/off, initiate a call, etc.

According to 3GPP TS 25.331 [21], clause 12.1.1, the encoding of RRC PDUs is obtained by applying UNALIGNED PER to the abstract syntax value as specified in ITU-T Recommendation X.691 [28]. The two tables below show the declaration of the encoding rule and an example of the use in the definition of an RRC PDU.

Table 6.4.1.1: PER_Unaligned Encoding Rule

| Encoding Rule Name | PER_Unaligned |
| --- | --- |
| Reference | ITU-T Recommendation X.691 [28] |
| Default |  |
| Comments | Packet encoding rules (ITU-T Recommendation X.691 [28]) unaligned and with adapted padding |

Table 6.4.1.2: Definition of the RRC ASN.1 DL_DCCH_Message type by reference

| PDU Name | DL_DCCH_Message |
| --- | --- |
| PCO Type | DSAP |
| Type Reference | DL-DCCH-Message |
| Module Identifier | Class-definitions |
| Enc Rule | PER_Unaligned |
| Enc Variation |  |

### 6.4.2	RAB test method

#### 6.4.2.1	Sending data on the same TTI

The RAB test requires a specific test method to send the test data on the same TTI. The TFC restriction method is used in this case. A specific TFC subset is allowed to ensure the test data are sent on different RBs on the same TTI. The downlink restriction can be used to ensure that the SS uses a specific TFC for transmission of data, by only allowing the "No data" TFC, and the "desired" TFC. It may also be necessary to include one or more "signalling only" TFCs to allow signalling to occur. The uplink restriction can be used to verify that the UE has used a specific TFC. Any data received by the SS using a forbidden TFCI shall be discarded.

#### 6.4.2.2	Sending continuous data on consecutive TTIs

The RBS ATS is developed using the tabular TTCN notation. In order to test of multiple-RB combinations and simultaneous signalling, the SS shall be capable of sending continues test data in every TTI using the downlink transport format combination under test. A specific TSO is designed to request the SS sending continuous data. The information about the number of RLC SDUs and their sizes for each RAB will be provided to the system simulator through TSO.

## 6.5	RLC test method and architecture

### 6.5.1	Testing architecture

Figure 6.5.1 illustrates a typical realization of the RLC ATS.

Figure 6.5.1: RLC ATS single party test method

The single party test method is used for RLC testing.

Separation of TTCN test cases from the configuration of the tester and initialization of the UE is achieved by using test steps. For each RLC test case, common test steps will be used to perform the configuration of the tester and the appropriate generic setup procedures as described in 3GPP TS 34.108 [3]. These test steps will make use of PCOs AM, UM, TM, CRLC, CMAC, and CPHY.

Three PCOs are provided at the top of the RLC emulation in the tester, one corresponding to each of the available RLC modes: acknowledged, unacknowledged, and transparent. Routing information for different radio bearers used at these PCOs will be provided in ASP parameters.

The queues shown in the RLC emulation in Figure 6.5.1 indicate that normal RLC transmit and receive buffering will be used to isolate the TTCN test suite from the real time issues involved if messages are sent directly to the MAC layer.

The RLC TTCN test cases make also use of the NAS TTCN test steps in order to bring UE to Idle state. The NAS test steps, which are called by the RLC test cases or steps, interface with the Dc PCO.

### 6.5.2	Test method

Figure 6.5.2.1 illustrates an example configuration for downlink UM testing. Uplink and AM tests will use similar configurations. A Tr-Entity is established on the tester side using a CRLC-CONFIG-REQ. A corresponding UM-Entity is created in the UE by sending a Radio Bearer Setup PDU. RLC PDUs are specified in the TTCN test suite, and sent to TM PCO. These PDUs shall be carefully designed so that the Tr-Entity will not perform any segmentation. The system simulator is responsible for direct encoding the abstract representation of transmitted PDUs into a bitstring to be sent by the Transmitting Tr entity. Direct encoding is performed by concatenation of all of the present fields in the abstract representation. It is the TTCN author's responsibility to ensure that the PDU is valid. To test reassembly in the UE side, the segmentation must be explicitly coded in TTCN. To test various aspects of the RLC header (e.g. sequence numbering, length indications, etc.), the RLC header must be explicitly coded in TTCN. Ciphering will not be tested using this approach, and will be disabled in the UE UM Entity.

The segmentation block in the SS Tr-entity is shown in grey to indicate that the functionality is present in the SS, but the test cases shall be carefully designed to ensure that segmentation is not used in the SS Tr-entity for RLC testing.

The deciphering block in the UE UM-entity is shown in grey to indicate that the functionality may be present in the UE, but shall be disabled for RLC testing.

Figure 6.5.2.1: Example configuration for downlink RLC UM testing

The TFCS used for RLC testing must guarantee that Tr mode segmentation will not occur. This is to prevent transmission of more than one Tr PDU per TTI.

All RLC tests that require uplink data will make use of the UE test loop mode 1 defined in 3GPP TS 34.109 [4]. The UE test loop mode 1 function provides all Upper Tester (UT) functionality required, so an UT PCO is not required for RLC tests. Test Loop mode 1 is only available in the user plane, so all RLC tests will be performed in the user plane, using DTCH and DCCH logical channels mapped to DCH transport channels.

Ciphering will be disabled for all RLC test cases. Ciphering will be tested implicitly by other test cases that have ciphering enabled.

Figure 6.5.2.2 illustrates an example configuration for uplink UM testing, and reception of an example UMD PDU. Figure 6.5.2.3 illustrates an example configuration for uplink AM testing, reception of an example STATUS_PDU, and the use of the superFields and superFieldsRec fields.

The ciphering and deciphering blocks in the UE RLC entities are shown in grey to indicate that the functionality may be present in the UE, but shall be disabled for RLC testing.

The reassembly blocks in the SS Tr-entities are shown in grey to indicate that the functionality is present in the SS, but the test cases shall be carefully designed to ensure that reassembly is not used in the SS Tr-entity for RLC testing.

Figure 6.5.2.2: Example configuration for uplink RLC UM testing

Figure 6.5.2.3: Example configuration for uplink RLC AM testing

Uplink data uses a similar approach to downlink, but the received data must be decoded in the correct way, depending on the current UE configuration. In the example in Figure 6.5.2.2, the SS must decode the data received at the TM PCO into an abstract representation of the structure defined in the TTCN for a UMD_PDU, using 7 bit length indicators. This structure is then compared with an abstract representation of the expected data to see if the receive event is successful. Refer to TR 101 666 [27], clause B.5.2.10 for more information.

For RLC testing, the following RB Ids are used within the system simulator, depending on the RLC mode, and length indicator size being simulated.

| RLC mode | LI Size | RB Id |
| --- | --- | --- |
| UM | 7 | -10 |
| UM | 15 | -11 |
| AM | 7 | -12 |
| AM | 15 | -13 |
| UM/AM (Rel-7 or later ) | 7/15 | -25 |

The SS decoder can use the RB Id to determine which abstract structure to create during the decode process. The SS decoder must also understand the RLC peer-to-peer protocol enough to determine which fields are present.

EXAMPLE 1:	The semantics of LI extension bits must be known to determine how many LIs are present.

EXAMPLE 2:	The contents of the LIs must be interpreted to determine how many octets of data, and how many octets of padding are present.

The SUFI list and any subsequent padding in a received STATUS_PDU or PiggyBackedSTATUS_PDU shall be decoded as a HEXSTRING, and put in the 'superFieldsRec' field of the abstract representation of the STATUS PDU. The "superFields" and "padding" fields shall be omitted for received STATUS PDUs. This is illustrated in Figure 6.5.2.3.

As in downlink testing, the TFCS must be defined to guarantee that the Tr entity does not perform any reassembly. This is to prevent reception of more than one Tr PDU per TTI so that the TTCN does not need to manage possible interleaving problems due to multiple PDUs received at the same time (i.e. they may be placed on the PCO queue in any order).

#### 6.5.2.1	Handling SUFIs in TTCN

The SUFIs are a very flexible set of information elements contained in the RLC protocol. The order of the fields varies, the existence of a field may depend upon the presence of another one. A field can be present multiple times. For matching received SUFIs, it is convenient to define the SUFIs as a HEXSTRING which is treated by a TSO o_SUFI_Handler.

Depending upon which SUFIs and which aspects of SUFIs are to be checked, the TSO is provided with the information (SUFI_Params) on what checking it is expected to perform. If the check is successful the result TRUE will be returned, otherwise FALSE. Additionally the TSO will return an object which is structured as the SUFIs used in transmission (SuperFields). This will allow to make use of information received and needed to establish SUFIs to be transmitted.

The input parameters to o_SUFI_Handler to be used as checking criteria are collected in tabular data structure SUFI_Params which is filled each time before the TSO is called. These data are to allow the checking of the presence and the value of SUFIs. All entries shall be set to well-defined values if these are to be used by o_SUFI_Handler. As a principle values specifically set are used as criteria for checking, values omitted are used as AnyOrOmit values. The resulting SUFI list is established by o_SUFI_Handler and can be retrieved in the data structure returned by the TSO. Details have to be defined in the TSO itself.

Tasks o_SUFI_Handler has to perform:

-	Transfer the SUFIs received into the structure of SuperFields; this is the SUFI list structure existing today.

-	If multiple occurrences of SUFI are found then use the last one to fill the SuperFields structure. The LIST SUFI is an exception: multiple SUFIs may be used to transfer the complete LIST information.

-	Check for all parameters in SUFI_Params set to a specific expected value that one of the SUFIs using this value is present and that the value received matches the specific expected value.

-	Check that if SUFIs are received for which an expected value of Any is specified, the SUFI is consistent if that SUFI is received.

-	Check that if SUFIs are received for the presence of which no entry is specified in SUFI_Params, the SUFI is consistent.

-	Check that sequence numbers are in the range between LB and UB if specific values are set.

Entries in SUFI_Params.

| Element Name | Significance | Comment |
| --- | --- | --- |
| LB | Lower bound of sequence number range | Lowest SN for checking SNs acknowledged |
| UB | Upper bound of sequence number range | Highest SN for checking SNs acknowledged |
| WSN_presence | Window Size SUFI present | To check the presence of the Window Size SUFI |
| MRW_presence | Move Receive Window SUFI present | To check the presence of the MRW SUFI |
| Nack1 | SN of 1st PDU negatively acknowledged | For the NackList to check SN to be negatively acknowledged |
| Nack2 | SN of 2nd PDU negatively acknowledged | For the NackList to check SN to be negatively acknowledged |
| Nack3 | SN of 3rd PDU negatively acknowledged | For the NackList to check SN to be negatively acknowledged |

More entries may be required in the future if specific SUFI field values are to be checked. The concept allows to add more fields easily.

#### 6.5.2.2	Void

## 6.6	SMS test method and architecture

### 6.6.1	SMS CS test method and architecture

The test method used for SMS CS tests is the same as the NAS test method, see clause 6.3, and the same ASPs, see clause 7.1.2.

### 6.6.2	SMS PS test method and architecture

The test method used for SMS PS tests is the same as the NAS test method, see clause 6.3, and the same ASPs, see clause 7.1.2.

### 6.6.3	SMS Cell broadcasting test method and architecture

The test method used for SMS CB tests is the same as the BMC test method, see clause 6.8, and the same ASPs, see clause 7.3.1.1.

## 6.7	MAC test method and architecture

### 6.7.1	Testing architecture

Figure 6.7.1 illustrates a typical realization of the MAC ATS.

Figure 6.7.1: MAC ATS single party test method

### 6.7.2	Test method

The single party test method is used for MAC testing.

Separation of TTCN test cases from the configuration of the tester and initialization of the UE is achieved by using test steps. For each MAC test case, common test steps will be used to perform the configuration of the tester and the appropriate generic setup procedures as described in 3GPP TS 34.108 [3]. These test steps will make use of PCOs AM, UM, TM, CRLC, CMAC, and CPHY.

Three PCOs are provided at the top of the RLC emulation in the tester, one corresponding to each of the available RLC modes: acknowledged, unacknowledged, and transparent. Routing information for different radio bearers used at these PCOs will be provided in ASP parameters.

The queues shown in the RLC emulation in Figure 6.5.2.2 indicate that normal RLC transmit and receive buffering will be used to isolate the TTCN test suite from the real time issues involved if messages are sent directly to the MAC layer.

A flag is required within the CMAC Config Req to indicate that the SS MAC emulation must not add or remove any MAC header information, even if header fields should be present according to the configured channels. This flag shall allow control of the MAC header on a per logical channel basis. For example, it shall be possible to configure 4 DCCHs and a DTCH mapped to a DCH, such that the MAC will add / remove header information for the DCCHs, but not for the DTCH.

The MAC TTCN test cases make also use of the NAS TTCN test steps in order to bring UE to Idle state. The NAS test steps, which are called by the MAC test cases or steps, interface with the Dc PCO.

For MAC testing, the following RB Ids are used for the high priority NAS RB within the system simulator depending on the MAC configuration being simulated.

| RB Id | Simulated configuration |
| --- | --- |
| -14 | DCCH mapped to FACH |
| -15 | DCCH mapped to DCH |
| -18 | CCCH mapped to FACH |

The SS decoder can use the RB Id to determine which MAC header fields are present, and create the appropriate abstract structure during the decode process. The SS decoder must understand enough of the MAC peer-to-peer protocol to determine which fields are present.

For example, the semantics of the UE Id Type field must be known to determine how many bits should be present in the UE Id field.

The MAC PDUs for MAC testing will always contain an AM RLC PDU (data or status) using 7 bit length indicators. See the RLC test method for further information on the SS decoder requirements for RLC PDUs.

Ciphering shall be disabled for all MAC tests.

#### 6.7.2.1	Abnormal decoding situations

If the SS decoder cannot convert the received data into the supported structure, the SS shall terminate the test case immediately and indicate that a test case error has occurred.

#### 6.7.2.2	MAC_es/e test method (Rel-6 or later)

MAC test method for MAC_es/e is depictured in the following figure. In the UE side the RLC entity is AM mode, in the SS the mode of RLC in downlink direction is TM, the AM mode functions are implemented in TTCN. In the uplink direction, the mapping between RB identity and logical channel identity  is configured in the RLC entity, the RLC entity passes any data block received on the logical channel to the RB identified by tsc_RB_DTCH_E_DCH_MAC(value is -20), tsc_RB_DTCH_E_DCH_MAC1(-21), or tsc_RB_DTCH_E_DCH_MAC2(-22). Whenever a RLC pdu received through one of the specified RB identifiers, the TTCN shall generate a RLC ack for it and send it on the downlink direction.

Figure 6.7.2.2: MAC_es/e testing model

#### 6.7.2.3	MAC_is/i test method (Rel-8 or later)

MAC test method for MAC is/i is depictured in the following figure. In the UE side the RLC entity is AM or UM mode, in the SS the mode of RLC in downlink direction is TM, the AM or UM mode functions are implemented in TTCN. In the uplink direction, the mapping between RB identity and logical channel identity is configured in the RLC entity, the RLC entity passes any data block received on the logical channel to the RB identified by tsc_RB_DTCH_E_DCH_MAC(value is -20), tsc_RB_DTCH_E_DCH_MAC1(-21), or tsc_RB_DTCH_E_DCH_MAC2(-22). Whenever a RLC pdu received through one of the specified RB identifiers, the TTCN shall generate a RLC ack for it and send it on the downlink direction.

Figure 6.7.2.3: MAC_is/i testing model

## 6.8	BMC test method and architecture

Figure 6.8: BMC testing architecture single party method

### 6.8.1	BMC test architecture

The single party test method is used for BMC testing, i.e. it does not exist an Upper Tester. BMC emulation is used as shown in Figure 6.8. The BMC emulation makes use of two PCOs. The CBMC PCO is defined, to pass configuration information for a BMC entity. The BMC PCO is defined for BMC message data transfer.

Separation of TTCN test cases from the configuration of the tester and initialization of the UE is achieved by using test steps. For BMC test cases, common test steps and newly defined test steps for BMC configuration will be used to perform the configuration of the tester and on UE side. These test steps make use of PCOs, CRLC, CMAC, and CPHY.

The UE shall be able to activate and deactivate a certain CB MessageID according CB data to be sent while testing.

BMC messages are sent in BMC message blocks on the CTCH. For sending BMC messages (BMC Scheduling Message (Level 2, DRX) and BMC CBS Message ) a configuration in downlink direction shall be performed to map the CTCH (RB#30) onto the FACH - S-CCPCH.

### 6.8.2	BMC test method

For BMC testing, only PS Cell Broadcast Service as distributed BMC service is applied. CBS Messages and BMC Schedule Messages are only sent in downlink direction. No uplink is used for BMC testing. The BMC test data with necessary CBS information shall be given by PIXIT parameter with a description of the indication on the display.

There are two level of BMC scheduling, Level 1 for CTCH configuration and Level 2 for DRX. The BMC scheduling information is conveyed to both BMC and MAC layer.

Level 1 scheduling is used to configure the CTCH on the S-CCPCH. For BMC testing (FDD), the Level 1 scheduling parameter MTTI contains one radio frame in the TTI of the FACH used for CTCH. Therefore, only Level 1 scheduling information N (period of CTCH allocation on S-CCPCH) and K (CBS frame offset to synchronize to the SFN cycle (0 to 4 095 frames per cycle)) are necessary to configure the CTCH onto the S-CCPCH.

The Level 1 scheduling is done in the SS MAC layer, therefore this information is given by using the primitive "CMAC_BMCscheduling_REQ" to inform the MAC on SS side about K and N. The Level 1 scheduling information, K and N, is broadcast as system information in SIB 5 and SIB 6. After having performed the CTCH configuration as Level 1 scheduling, the SS is configured to send BMC messages and the UE has to listen to each CTCH for a BMC message.

Segmentation of BMC messages is performed by RLC in UM according to the payload size on RB#30 (152 bits).

If only one CB data as BMC CBS message is sent and repeated for a BMC test case, Level 1 scheduling is adequate, i.e. no BMC Scheduling Message (Level 2) is needed. Then, no level 2 scheduling information are included in the "CMAC_BMCscheduling_REQ" primitive. If more then one BMC CBS message are transmitted and repeated or if BMC Scheduling Messages are transmitted, BMC scheduling Level 2 message shall be performed. Level 2 scheduling is used to predict the sent event of the next BMC message blocks and the BS index contents.

BMC scheduling Level 2 predicts exactly, which information is contained on a certain CTCH block set with an aligned Block Set index number and how many CTCH blocks are given as offset, before the next BMC message block will be sent. Figure 6.8.2.1 shows an example, how the messages flow shall be done for BMC scheduling Level 2.

Figure 6.8.2.1: BMC Scheduling

The BMC test method makes use of the primitive: "BMC_Data_REQ" to transmit the BMC Messages to RLC. If BMC Scheduling Level 2 is used, it includes BMC CBS PDUs or BMC Schedule PDUs, created by the BMC TTCN and forwarded to the BMC emulation. The transmission of BMC PDU is confirmed through the primitive BMC_Data_CNF.

According to the K and N value, the MAC layer at SS side determines the CTCH blocks for the BMC use. The CTCH blocks are indexed (i = 1 ... 256).

If BMC DRX is needed, the BMC scheduling Level 2 information indicates the occupancy/spare of the available CTCH blocks by using a DRX_Selection_Bitmap and the list of CTCH BS index offsets. In the bitmap each bit, set to '1', corresponds to an actually available CTCH block belonging to the DRX period for the SS transmission. The all consecutive occupied CTCH blocks constitutes a BMC DRX period, whilst the consecutive spared blocks indicate the DRX offset as spare CTCH slot. Following the DRX_Selection_Bitmap, the segmented BMC messages are transmitted.  Each BMC PDU is transmitted in a BMC_Data_REQ primitive in the order of expected transmission. The list of CTCH BS index offset indicating the offset start of the relative to the previous message transmitted is included in the CMAC_BMC_Scheduling_REQ. If any, the retransmission is handled by the BMC TTCN.

The figure 6.8.2.2 shows an example of DRX level2 scheduling when sending 4 BMC messages (either BMC schedule or CBS BMC data messages) with the CTCH BS Index Offset List={0,6,2,11} and the Bitmap=‘107002’O.

Figure 6.8.2.2: Example of BMC DRX scheduling: CTCH BS index offset list and Bitmap handling

## 6.9	PDCP test

Figure 6.9: PDCP testing architecture 1: single party test method, with test loop mode 1

### 6.9.1	PDCP test architecture

The single party test method is used for PDCP testing. All PDCP tests that require uplink data will make use of the UE test loop mode 1 defined in 3GPP TS 34.109 [4]. Test Loop mode 1 is only available in the user plane, so all PDCP tests will be performed in the user plane, using the same logical channels mapped to transport channels as defined in RLC test cases, except for test case, clause 7.3.2.2.4, where a configuration of combined radio bearers used only for this test case is defined.

Separation of TTCN test cases from the configuration of the tester and initialization of the UE is achieved by using test steps. For PDCP test cases, common test steps and newly defined test steps for PDCP configuration will be used to perform the configuration of the tester and the appropriate generic setup procedures as described in 3GPP TS 34.108 [3] and in clause 7.3 of 3GPP TS 34.123-1 [1]. These test steps will make use of PCOs RLC AM, RLC UM, CRLC, CMAC, and CPHY.

The PDCP TTCN test cases make also use of the NAS TTCN test steps in order to setup a PS session.

For PDCP testing, the IP Header Compression protocol as described in RFC 2507 [30] is used as optimization method. The IP header compression and decompression mechanisms as described in RFC 2507 [30] is not part of PDCP TTCN. PDCP testing make use of uncompressed, compressed and decompressed TCP/IP header packets of a certain packet stream and uncompressed, compressed and decompressed UDP/IP header packets of a certain generation. This parameters are given as test parameter (PIXIT information).

PDCP testing includes transmission/reception of compressed/decompressed IP header packets, PDCP sequence numbering while lossless SRNS relocation and PID assignment rules as well as PDCP configuration tests as described in 3GPP TS 25.323 [19]. It does not test optimization specific protocol behaviour as error recovery and packet reordering as described in RFC 2507 [30].

### 6.9.2	PDCP test method

For PDCP testing, the RB test mode is used with test loop mode 1. After establishing a PS session with RB in RLC UM or/and AM, the UE is configured to support a negotiated PDCP configuration. UDP/IP header packets are used as Non-TCP/IP header packets as PDCP test data.

There are different input parameter as PIXIT values necessary for PDCP testing.

For TCP/IP header packets, uncompressed TCP/IP header packets shall be defined as PIXIT input parameter. In addition, there are the corresponding RFC 2507 [30] FULL_HEADER packet, COMPRESSED_TCP packet and COMPRESSED_TCP_NONDELTA packet given for each TCP/IP header packet as PIXIT information.

For UDP/IP header packets, uncompressed UDP/IP header packets shall be defined as PIXIT input parameter. In addition, there are the corresponding RFC 2507 [30] FULL_HEADER packet and COMPRESSED_NON_TCP packet given for each UDP/IP header packet as PIXIT information.

To check the use of certain PID values assigned to IP compressed header types, a given IP header packet (PIXIT) will be sent to the UE. The UE shall return a appropriate valid IP header packet type, which corresponds to the previous sent IP header packet. The usage of valid compressed/uncompressed IP header packets shall be checked by comparing the given PIXIT IP header packet types for each IP header packet previously sent.

The IP header packet order as described in RFC 2507 [30] shall be applied within a test case.

If for example an TCP/IP header packet of type "COMPRESSED_TCP" shall be sent, the TTCN uses the given TCP/IP header packet (PIXIT) for transmission to the UE. The UE shall decompress the received packets appropriate, afterwards it will be returned by the loop back entity and it shall be sent by applying IP header compression rules as described in RFC 2507 [30] and as configured. Then, the SS receives  returned IP header packets and compares it with all valid IP header packets given as PIXIT parameter corresponding to the previously sent IP header packet. It is checked, whether or not the IP header packet with assigned PID is valid and a configured PDCP PDU where used for transmission. In this way, it is checked, that the UE performs IP header compression as configured and is able to assign the correct PID values.

#### 6.9.2.1	CS voice over HSPA

For PDCP CS voice over HSPA tests, the RB test mode used is test loop mode 1 with loopback of PDCP SDUs (as per 3GPP TS 34.109 [4], clause 5.3.2.6.1). The CS domain voice RAB is associated with one RB and one PDCP entity.  The two RLC entities (DL/UL) are configured in UM with SN_delivery mode. The PDCP entity serving CS service does not use header compression, therefore no ROHC is configured.

#### 6.9.2.2	Network initiated secondary PDP context

Figure 6.9.2.2: Network initiated secondary PDP testing architecture

For the network initiated secondary PDP context tests using data loopback, the UE test loop mode 4 is applied with loopback of IP PDUs (as per 3GPP TS 34.109 [4], clause 5.3.2.8.1). No header compression is tested, therefore  no ROHC is configured.

## 6.10	Multi-RAT Handover Test Model

### 6.10.1	Overview

The test model is shown in Figure 6.10.2.9.6. The SS in the model consists of UTRAN emulation part and GERAN emulation part, GERAN emulation part includes protocol emulation modules for GSM CS services and protocol emulation modules for GPRS service. Protocol stack L1 (GERAN), L2 is for GSM CS service function emulation, protocol stack L1, RLC/MAC, LLC, SNDCP is for GPRS service function emulation. SNDCP emulation model and relevant PCO's can be removed if "traffic channel gets through" is not tested.

L1 (GERAN) provides necessary physical layer functionality for both GSM and GPRS. A control PCO and a set of ASP's are defined for configuring and controlling its protocol behaviour required in the test cases. L1 (GERAN) provides services to L2 and RLC/MAC emulation modules, the interfaces between them are not specified in this test model, it is implementation dependent and shall follow the relevant GSM and GPRS specifications.

L2 emulates necessary GSM L2 protocol functionality used in testing. A data PCO and a set of ASP's are defined for this module and used for transmitting and receiving layer 3 signalling messages and use data. The definition of the PCO and these ASP's are based on the logical channel concept of GSM specification. A control PCO and related ASP's are also defined for L2, they are used to introduce abnormal layer 2 behaviour required by the test purposes.

RLC/MAC is emulation module for GPRS Radio Link Control/Medium Access Control protocol. Two PCO's and related ASP's are defined for the module. Control PCO is used to set TBF and assign physical resources to it, actual physical resources (packet channels) are created by L1 (GERAN) ASP's beforehand. Data PCO is for transmitting and receiving RLC control messages (RLC control block). Before any RLC data or control block is sent (or received) a proper TBF shall be configured. In addition RLC/MAC module provides service to LLC emulation module, the interface between them is determined by implementation and shall be compliant with relevant core specification.

LLC performs GPRS Logical Link Control protocol emulation. Its data PCO and ASP's are used for exchange GMM signalling messages between TTCN and the UE under test. The current defined ASP's on control PCO are subset of the primitives defined in core specification, they are used to assign, un-assign TLLI and ciphering parameters, or get status report.

### 6.10.2	ASP function description

#### 6.10.2.1	Identities

-	Within the SS, a cell is identified by cell identifier (cellId), which is of TTCN type CellId (INTEGER).

-	Within a cell, a basic physical channel is identified by physical channel identifier (physicalChId), which is of TTCN type PhysicalChId (INTEGER). In multislot configuration a basic physical channel is identified by physical channel identifier (physicalChId) and timeslot, which is of TTCN type TN (INTEGER).

-	Within a physical channel, logical channel is identified by logical channel type (g_LogicChType), which is of TTCN type G_LogicChType (INTEGER). When multiple logical channels of same type are carried by (mapped to) the same basic physical channel, they are differentiated by sub-channel number (subChannel), which is of TTCN type SubChannelNumber (INTEGER).

-	At the top boundary of L2 emulation module two service access points (SAP) are available, they are identified by SAPI. SAPI=3 is used for short message service; SAPI=0 is used for L3 signalling messages and user data.

EXAMPLE:	If G_L2_DATA_REQ ASP has the following parameter setting:

-	cellId = tsc_CellA;

-	sAPI = tsc_SAPI_0;

-	physicalChId = tsc_PhyCh0;

-	g_LogicChType = tsc_SDCCH4; and

-	subChannel = tsc_SubChannel1;

it sends PDU on the SDCCH4(1) logical channel which is carried by the physical channel tsc_PhyCh0 in cell A.

#### 6.10.2.2	Cell configuration and control

In GSM each base station has a base station identity code BSIC, it consists of network colour code and base station colour code (NCC + BCC). BSIC is continuously broadcasted on the SCH channel, and it shall be used  as the training sequence code for broadcast and common control channels.

In the test model the function of G_CL1_CreateCell_REQ ASP is to create a cell and pass parameter BSIC to it. This ASP establishes the cell identifier which shall be used in the ASP's related to this cell.

This is the first step to configure L1 (GERAN) emulation module of the SS.

#### 6.10.2.3	L1 (GERAN) configuration and control

Configuration and control functions identified for L1 (GERAN) of a cell are:

-	creation of basic physical channels;

-	creation of multislot configuration;

-	release of basic physical channel;

-	modifications of channel mode, ciphering parameters and transmission power level;

-	reporting of L1 header of SACCH channel;

-	pickup a frame in near future, which can carry L3 message.

##### 6.10.2.3.1	Basic physical channel configuration

A basic physical channel uses a combination of frequency and time domain resources, therefore, the definition of a particular basic physical channel consists of a description in the frequency domain and a description in the time domain. In time domain the resource is called Time Slot, there are 8 time slots in one frame, numbered from 0 to 7. In frequency domain a basic physical channel may use only one frequency or may use multiple frequencies in frequency hopping.

Basic physical channel carrying FCCH + SCH + BCCH + CCCH (PCH, AGCH, RACH) or FCCH + SCH + BCCH + CCCH + SDCCH4 logical channels shall be located in time slot 0, and uses single frequency (non-hopping). The basic physical channel carrying additional BCCH, CCCH (PCH, AGCH, RACH) logical channels shall be located in time slot 2, 4, 6 and uses the same single frequency as the frequency used by the physical channel carrying FCCH, SCH.

GSM specification defines 24 permitted combinations of different logical channels, which can be mapped on to a basic physical channel. The combination defines which logical channels are carried by a basic physical channel, and it is also an indication of which modulation (GMSK or 8PSK) is used for the basic physical channel.

Training Sequence Code (TSC) is another parameter needed by physical channel. Common control and broadcast channel have to use BCC as its TSC.

Dedicated control channel and dedicated traffic channel need more parameters to configure. Parameter "Channel Mode" is needed to specify channel coding (therefore the user data rate). Ciphering related parameters are required to define the ciphering behaviour of the channel.

Common control channels need parameters to configure where in the 51-multiframe  paging and access grant blocks are located.

Transmission power level is provided as per physical channel parameter, power level of each physical channel can be controlled independently.

The function of ASP G_CL1_CreateBasicPhyCh_REQ is to create a basic physical channel which has the required property defined by all the parameters mentioned above.

In the process of L1 (GERAN) configuration, calling the ASP is the next step after calling G_CL1_CreateCell_REQ.

##### 6.10.2.3.2	Multislot configuration for circuit or packet switched channels

Multislot configuration for circuit switched connection consists of multiple circuit switched traffic channels, in L1 point of view these traffic channels are independent basic physical channels with the same frequency parameters (ARFCN or MA, MAIO, HSN) and the same training sequence code but located in different time slots, one of the basic physical channels is the main channel of the configuration carrying the main signalling (FACCH, SACCH, IACCH) for the configuration. The main channel shall be bi-directional channel and with channelCombanition TCH/F+FACCH/F+SACCH/M or E-TCH/F+E-IACCH/F+E-FACCH/F+E-SACCH/M. When transmitting user data (not signalling message) stream is divided into substreams, each substream is transmitted independently on a channel in the configuration. At the receiving side all substreams are combined back to user stream.

According to the test model creation of a multislot configuration for circuit switched connection needs two ASP calls. Firstly, G_L1_CreatedBasicPhyCh_REQ is called to establish the main channel, then G_L1_CreateMultiSlotConfig_REQ is called to allocate more timeslots to the channel established by the previous ASP. A substream of a multislot configuration is identified with the physicalChId and timeslot.

Multislot configuration for packet switched connection consists of multiple PDCHs which can carry PDTCH/Us or PDTCH/Ds. All these PDCHs use the same frequency parameters (ARFCN or MA, MAIO, HSN) and the same training sequence code, but are located on different timeslots.

Similarly, a multislot configuration for packet switched connection is created with two ASP calls. First G_L1_CreatedBasicPhyCh_REQ is called to establish the first PDCH channel, then G_L1_CreateMultiSlotConfig_REQ is called to allocate more timeslots to the channel established by the previous ASP. All data ASP on packet data channel use physicalChId and timeslot to address the physical channels.

##### 6.10.2.3.3	Frame in the near future

ASP G_CL1_ComingFN_REQ is defined to request L1 (GERAN) return the reduced frame number (FN modulo 42432) which is far enough in the future from current frame number and is able to carry L3 message on the specified channel. "far enough" means that there is enough time left for TTCN to prepare a L3 message to be sent on that frame. When calculating startingTime, this ASP could  be useful. The starting time usually is set to a frame number in a time distance from current frame number. TTCN writer can use G_CL1_ComingFN_REQ to get a frame number in the future then add a certain number of frames as time distance to it and use the result as the value for startingTime.

##### 6.10.2.3.4	L1 header

The layer 1 header of SACCH from UE to network carries information of timing advance and UE uplink transmission power level, verifying L1 header contents is required in some test cases, ASP G_CL1_L1Header_REQ and G_CL1_L1Header_CNF are defined for fulfilling this requirement.

#### 6.10.2.4	L2 configuration and control

For normal operation there is no parameter configurable in L2. Some abnormal L2 behaviours are required in test cases. In the test model two ASP's are currently defined to introduce abnormal L2 behaviour. When creating a dedicated channel the initial SACCH header is set to the values in powerLevel and timingAdvance fields of DedCH_Info.

##### 6.10.2.4.1	Don't response to some handover access bursts

In non-synchronized handover procedure UE/MS, having received handover command, sends handover access bursts on the target channel repeatedly till it receives PHYSICAL INFORMATION message from network or T3124 times out. Normally network replies PHYSICAL INFORMATION as soon as it receives handover access burst. Some test cases require that the SS ignores several incoming handover access bursts then responses to the one that follows. ASP G_CL2_HoldPhyInfo_REQ is defined for fulfilling this requirement. It is used together with and before a data ASP sending PHYSICAL INFORMATION message. When SS receives the G_CL2_HoldPhyInfo_REQ, it does not transmit the PHYSICAL INFORMATION message until n handover access bursts have been received.

##### 6.10.2.4.2	No UA reply to SABM

GSM L2 protocol is adapted from LAPD (HDLC subset). The multiframe operation mode is established through exchange of supervisory frame SABM and unnumbered frame UA between peer entities, and SABM is always sent by UE/MS, UA is always sent by network. UE/MS will repeatedly transmit SABM till it receives UA or retransmission counter is reached. Some handover test cases require that the SS does not response to the incoming SABM, so handover fails. G_CL2_NoUAforSABM_REQ is used for such purpose, it commands the SS not to send UA response to the UE when SABM is received.

#### 6.10.2.5	System Information sending

There are 17 different SYSTEM INFORMATION messages on BCCH and 4 different SYSTEM INFORMATION messages on SACCH defined for circuit switched services in GSM specification. In a particular test case not all of them are required. SYSTEM INFORMATION messages on BCCH shall be broadcasted periodically by the SS, SYSTEM INFORMATION TYPE 5, 6 and optionally 5bis and 5ter messages shall be sent on SACCH by the SS when nothing else has to be sent on that channel.

G_L2_SYSINFO_REQ is defined to deliver a SYSTEM INFORMATION message and its type SysInfoType to the SS, SS shall store the SYSTEM INFORMATION and transmit it periodically according to the scheduling rules specified in 3GPP TS 45.002 [31], clause 6.3.1.3. SYSTEM INFORMATION message newly delivered shall override the same type SYSTEM IFORMATION message previously stored in the SS.

SYSTEM INFORMATION message type 18, 19, 20 are scheduled by scheduling information in SYSTEM INFORMATION type 9. ASP for scheduling these messages has not been defined yet because these messages are not required in current test cases.

#### 6.10.2.6	Paging

Paging message for a particular UE/MS shall be sent on the right CCCH_GROUP and PAGING_GROUP which are determined by IMSI of the UE/MS and other parameters. In the test model TTCN code is responsible to calculate the value of CCCH_GROUP and the value of PAGING_GROUP.

TTCN selects the right channel according to the value of CCCH_GROUP, then PAGING REQUEST message and the value of PAGING_GROUP are passed to the SS by using:

-	ASP G_L2_Paging_REQ in case of UE/MS in idle mode or the UE/MS not supporting  SPLIT_PG_CYCLE on CCCH when it is in GPRS attached mode.

The SS shall determine the position where the paging block is located using the value PAGING_GROUP and other CCCH parameters configured by G_CL1_CreateBasicPhyCH_REQ, then send the PAGING REQUEST message according the parameter pagingMode in the ASP:

-	send the message on the paging block determined by PAGING_GROUP if pagingMode = "normal paging";

-	send the message on the paging block determined by PAGING_GROUP and the "next but one" position on the PCH  if pagingMode = "extended paging";

-	send the message on all paging blocks if pagingMode ="paging reorganization".

#### 6.10.2.7	Generic procedures for GPRS signalling

Two channel combinations are applied to configure a GERAN cell for the GPRS signalling:

-	The channel combinations 5 + 13, (FCCH + SCH + BCCH + CCCH + SDCCH/4(0..3) + SACCH/C4(0..3)) + (PDTCH/F+PACCH/F+PTCCH/F), are considered as default at the interRAT tests and GERAN to UTRAN Inter-RAT test cases in clause 42.4.7 of TS 51.010-1.

The following generic procedures show the usages of GPRS ASP's for the GPRS generic attach procedures, the generic cell change order within a TBF and the GSM ciphering procedure.

##### 6.10.2.7.1	GPRS generic attach procedures and ciphering mode control

6.10.2.7.1.1	GPRS attach procedure in channel combinations 5 and 13

| Direction | ASP | message | Comments |
| --- | --- | --- | --- |
| SS | G_CL1_CreateCell_REQ |  | Create the cell |
| SS | G_CL1_CreateBasicPhyCh_REQ |  | Create the physical channel combination 5 for FCCH+SCH+BCCH+CCCH+SDCCH/4(0..3)+SACCH/C4(0..3) |
| SS | G_CL1_CreateBasicPhyCh_REQ |  | Create the physical channel combination 13 for PDTCH/F+PACCH/F+PTCCH/F |
| SS -> MS | G_L2_SYSINFO_REQ | SYSTEM INFORMATION TYPE1, SYSTEM INFORMATION TYPE2, SYSTEM INFORMATION TYPE2quater, SYSTEM INFORMATION TYPE3, SYSTEM INFORMATION TYPE4, SYSTEM INFORMATION TYPE13 | Broadcast system information messages : SI 1~4; SI 13 |
| SS | G_CRLC_CreateRLC_MAC_REQ |  | Create RLC/MAC emulation entity |
| SS | G_CLLC_CreateLLE_REQ |  | Create LLC emulation entity |
| SS | MMI_CmdReq |  | Power on the UE/MS |
| MS-> SS | G_L2_ACCESS_IND | CHANNEL REQUEST | RACH, TBF establishment with Establishment Cause = one phase packet access. |
| SS | G_CRLC_UL_TBF_Config_REQ |  | Set up uplink TBF in RLC/MAC entity in SS, this TBF is corresponding to what indicated in IMMEDIATE ASSIGNMENT. |
| SS -> MS | G_L2_UNITDATA_REQ | IMMEDIATE ASSIGNMENT | Assign the uplink resources (uplink TBF) to MS. Polling bit and Starting Time are set |
| MS -> SS | G_RLC_ControlMsg_IND | PACKET CONTROL ACKNOWLEDGEMENT |  |
| SS | G_CLLC_Assign_REQ |  | Assign TLLI, ciphering key and algorithm. The ciphering algorithm = "ciphering not used". The value of ciphering key shall be the one generated in the following authentication procedure. If there is no user data traffic in acknowledged mode before authentication procedure the ciphering algorithm may be set to one of the GPRS ciphering algorithm, and the late G_CLLC_Assign_REQ shall be not used. |
| MS -> SS | G_LLC_UNITDATA_IND | ATTACH REQUEST | MS uses the assigned uplink TBF to transmit the L3 message to SS, the SS manages the operation of the TBF without TTCN intervention and releases the TBF automatically according the countdown procedure. The SS reassembles the received data blocks into the L3 message and passes it to the LLC DATA PCO G_LLC. |
| SS | G_CRLC_DL_TBF_Config_REQ |  | Set up downlink TBF in RLC/MAC entity in SS |
| SS -> MS | G_L2_Paging_REQ | IMMEDIATE ASSIGNMENT | Downlink TBF establishment |
| SS -> MS | G_LLC_UNITDATA_REQ | AUTHENTICATION AND CIPHERING REQUEST |  |
| MS-> SS | G_L2_ACCESS_IND | CHANNEL REQUEST | RACH, TBF establishment with Establishment Cause = one phase packet access. |
| SS | G_CRLC_UL_TBF_Config_REQ |  | Set up uplink TBF in RLC/MAC entity in SS, this TBF is corresponding to what indicated in IMMEDIATE ASSIGNMENT. |
| SS -> MS | G_L2_UNITDATA_REQ | IMMEDIATE ASSIGNMENT | Assign the uplink resources (uplink TBF) to MS. Polling bit and Starting Time are set |
| MS -> SS | G_RLC_ControlMsg_IND | PACKET CONTROL ACKNOWLEDGEMENT |  |
| SS | G_CLLC_Assign_REQ |  | Assign TLLI, if changed |
| MS -> SS | G_LLC_UNITDATA_IND | AUTHENTICATION AND CIPHERING RESPONSE |  |
| SS | G_CLLC_Assign_REQ |  | Keep TLLI unchanged, ciphering algorithm = one of the GPRS ciphering algorithm. The value of ciphering key shall be the one generated in the authentication procedure. If no user data traffic in acknowledged mode before authentication procedure, this ASP is not needed. |
| SS | G_CRLC_DL_TBF_Config_REQ |  | Set up downlink TBF in RLC/MAC entity in SS |
| SS -> MS | G_L2_Paging_REQ | IMMEDIATE ASSIGNMENT | Downlink TBF establishment |
| SS -> MS | G_LLC_UNITDATA_REQ | ATTACH ACCEPT | SS uses the established downlink TBF to transmit the L3 message to MS, the SS manages the operation of the TBF without TTCN intervention and releases the TBF automatically after all data blocks of the L3 message are transmitted |
| MS-> SS | G_L2_ACCESS_IND | CHANNEL REQUEST | RACH, TBF establishment with Establishment Cause = one phase packet access. |
| SS | G_CRLC_UL_TBF_Config_REQ |  | Set up uplink TBF in RLC/MAC entity in SS |
| SS -> MS | G_L2_UNITDATA_REQ | IMMEDIATE ASSIGNMENT | Assign the uplink resources (uplink TBF) to MS. Polling bit and Starting Time are set |
| MS -> SS | G_RLC_ControlMsg_IND | PACKET CONTROL ACKNOWLEDGEMENT |  |
| SS | G_CLLC_Assign_REQ |  | Assign new TLLI |
| MS -> SS | G_LLC_UNITDATA_IND | ATTACH COMPLETE | MS uses the assigned uplink TBF to transmit the L3 message to SS, the SS manages the operation of the TBF without TTCN intervention and releases the TBF automatically according the countdown procedure |
| SS | G_CRLC_DeleteRLC_MAC_REQ |  | Release resources in the SS for RLC/MAC emulation entity |
| SS | G_CLLC_DeleteLLE_REQ |  | Release resources in the SS for LLC emulation entity |
| SS | G_CL1_DeleteChannel_REQ |  | Release SS resources of channel combination 13 |
| SS | G_CL1_DeleteChannel_REQ |  | Release SS resources of channel combination 5 |
| SS | G_CL1_DeleteCell_REQ |  |  |

6.10.2.7.1.2	Void

##### 6.10.2.7.2	Cell change order within a TBF

6.10.2.7.2.1	Cell change order procedure in channel combinations 5 and 13

| Direction | ASP | message | Comments |
| --- | --- | --- | --- |
| SS | G_CL1_CreateCell_REQ |  |  |
| SS | G_CL1_CreateBasicPhyCh_REQ |  | Create the physical channel combination 5 for FCCH+SCH+BCCH+CCCH+SDCCH/4(0..3)+SACCH/C4(0..3) |
| SS | G_CL1_CreateBasicPhyCh_REQ |  | Create the physical channel combination 13 for PDTCH/F+PACCH/F+PTCCH/F |
| SS -> MS | G_L2_SYSINFO_REQ | SYSTEM INFORMATION TYPE1, SYSTEM INFORMATION TYPE2, SYSTEM INFORMATION TYPE2quater, SYSTEM INFORMATION TYPE3, SYSTEM INFORMATION TYPE4, SYSTEM INFORMATION TYPE13 | Broadcast system information messages: SI 1~4; SI 13 |
| SS | G_CRLC_CreateRLC_MAC_REQ |  | Create RLC/MAC emulation entity |
| SS | G_CLLC_CreateLLE_REQ |  | Create LLC emulation entity |
| SS | G_CLLC_Assign_REQ |  | Assign TLLI, ciphering key and algorithm |
| MS |  |  | MS is GPRS attached, PDP context activated, then trigger MS to send two SNDCP PDU on LLC SAPI 3, each with 500 bytes user data. |
| MS-> SS | G_L2_ACCESS_IND | CHANNEL REQUEST | RACH, TBF establishment with Establishment Cause = one phase packet access. |
| SS | G_CRLC_UL_TBF_Config_REQ |  | Set up uplink TBF in RLC/MAC entity in SS, this TBF is corresponding to what indicated in the next IMMEDIATE ASSIGNMENT. The USFRate is set to 5 USF per second. |
| SS -> MS | G_L2_UNITDATA_REQ | IMMEDIATE ASSIGNMENT | Assign the uplink resources (uplink TBF) to MS |
| MS -> SS | G_LLC_UNITDATA_IND | User data on SAPI 3, the first SNDCP PDU | The TBF shall not be in countdown process |
| SS -> MS | G_RLC_ControlMsg_REQ | PACKET MEASUREMENT ORDER | This is within the TBF established above, which is in the process handling the second SNDCP PDU REPORT_TYPE = 1 |
| MS -> SS | G_RLC_ControlMsg_IND | PACKET MEASUREMENT REPORT | MS sends the PACKET MEASUREMENT REPORT |
| SS -> MS | G_RLC_ControlMsg_REQ | PACKET CELL CHANGE ORDER | This is within the TBF established above |
|  |  |  | what follows are in UTRAN cell, not present here |

6.10.2.7.2.2	Void

#### 6.10.2.8	Generic configuration procedure for GSM ciphering mode control

| Direction | ASP | message | Comments |
| --- | --- | --- | --- |
|  | . . . |  | Other necessary configuration ASP's |
| SS | G_CL1_CreateBasicPhyCh_REQ |  | Create a dedicated physical channel, e.g. combination 1 with ciphering not started: This ASP download Kc and ciphering algorithm to the SS with startingCiph = 0 in cipherMode. If there is no authentication procedure before CIPHERING MODE COMMAND, the value of Kc in this ASP shall be the one generated in previous authentication procedure, otherwise the value of Kc shall be the one generated by forthcoming authentication procedure. |
|  | . . . |  | Any other signalling message sending/receiving or configuration ASP's |
| SS | G_CL1_CipheringControl_REQ |  | rcvCipherMode ='1' , the SS starts ciphering on receiving |
| SS | G_CL1_CipheringControl_CNF |  |  |
| SS -> MS | G_L2_DATA_REQ | CIPHERING MODE COMMAND | Sent without ciphering |
| SS |  |  | Before this point both transmitting and receiving in the SS are not ciphered. |
| MS -> SS | G_L2_DATA_IND | CIPHERING MODE COMPLETE | After receiving this message the SS shall start ciphering on transmitting, The CIPHERING MODE COMPLETE is ciphered |
|  | . . . |  | Any signalling message or user data sending/receiving in ciphered mode |

#### 6.10.2.9	L|H bits convention and bit padding in DL

##### 6.10.2.9.1	GERAN DL RLC/MAC message bit padding

The length of a GPRS RLC/MAC control messages is an integer number of RLC/MAC control blocks. Padding bits are necessary to fill the message up to the desired length. The padding bits may be the 'null' string. Otherwise, the padding bits starts with bit '0', followed by "spare padding". The padding sequence used for "spare padding" in the present document, is a repetition of octet '00101011', starting on an octet boundary.

< padding bits > ::= { null | 0 < spare padding >

"<spare padding> ::= <spare L> {null | < spare padding>}"

In the TTCN a specific encoding variation - encoding rule 1 -  is defined according to the rules described above.  This shall be used in the definition of the message itself.  No 'padding bits' field will be defined in the TTCN. The implementation shall ensure that after encoding the message contents defined in the TTCN, the remainder of the message shall be filled with 'padding bits'.

##### 6.10.2.9.2	GSM DL message spare padding

A number of GPRS information elements are defined in the rest octets of certain GSM DL messages, for instance, IA Rest Octets, SI 2quater Rest Octets, SI 3 Rest Octets, SI 4 Rest Octets, SI 13 Rest Octets, etc. These rest octets were filled in a repetition of bit padding '00101011' or '2B'O, starting on an octet boundary to a certain length.

In the TTCN, a second encoding variation - encoding rule 2 - shall be used in the definition of the message itself, which shall be of a fixed length (always 23 octets). No "spare padding" field will be defined in the TTCN. The implementation shall ensure that after encoding the message contents defined in the TTCN, the remainder of the message, up to the defined fixed length, shall be filled with "spare padding".

##### 6.10.2.9.3	L | H convention in rest octets of GSM DL messages

A number of GPRS information elements are defined in the rest octets of certain GSM DL messages. The special notations "L" and "H" are used to denote respectively the bit's logical value corresponding to the padding spare bit for that position, and the other value. The actual value of the bit transmitted by SS therefore depends upon its position within the octet - this involves counting bits.

In the TTCN a third encoding variation - encoding rule 3 - is defined for this purpose. This encoding variation is applied to those specific TTCN Rest Octets definitions which contain the L|H convention.

##### 6.10.2.9.4	Spare Bits

Where the IE definition of RLC/MAC blocks contains bits defined to be 'spare bits', these bits shall set to the value '0' by the TTCN writers, according to the defined length indicator.

##### 6.10.2.9.5	GSM System Information messages on SACCH

Certain GSM System Information messages, for instance, SI 5 and SI 6 are sent as a B4 frame on the SACCH. These messages are defined in 3GPP TS 44.006 [42], clause 8.8.3, to have a maximum of 19 octets.

In the TTCN a fourth encoding variation - encoding rule 4 - shall be used in the definition of the message itself.  The implementation shall ensure that after encoding the message contents defined in the TTCN, the remainder of the message, up to the fixed length of 19 octets, shall be filled with "spare padding".

##### 6.10.2.9.6	GSM Measurement Information messages on SACCH

The GSM Measurement Information message is sent as a Bter UI frame on the SACCH.  This messages is defined in 3GPP TS 44.006 [42], clause 8.8.3 to have a maximum of 21 octets.

In the TTCN a fifth encoding variation - encoding rule 5 - shall be used in the definition of the message itself. The implementation shall ensure that after encoding the message contents defined in the TTCN, the remainder of the message, up to the fixed length of 21 octets, shall be filled with "spare padding".

Figure 6.10.2.9.6: The model of multi-RAT handover testing

## 6.11	DCH-DSCH model (R99 or Rel-4)

The model illustrates the relationship between various channels from logical channel to physical channels. DCH are associated with DSCH.

Figure 6.11: Associated DCH-DSCH model

The model associating DCH with DSCH enable in the SS:

-	to define DSCH transport channel;

-	to define TFCI(field2) for DSCH;

-	to configure PDSCH;

-	to define DSCH-RNTI value.

## 6.12	DCH with HS-DSCH (MAC-hs) model (FDD, Rel-5 or later)

The test model illustrates the relationship between various channels from logical channels to physical channels. All DCH are associated with a single HS-DSCH.

Figure 6.12: Associated DCH with HS-DSCH model

Associating DCH with HS-DSCH, the model enables in the SS:

-	to define MAC-hs and multiplexing of logical channels DTCHs onto MAC-d flows;

-	to configure HS-DSCH transport channel and MAC-d flows;

-	to configure HS-PDSCHs and HS-SCCHs;

-	to define the H-RNTI value.

## 6.12a	DCH with HS-DSCH model for 1.28 Mcps TDD (Rel-5 or later)

Figure 6.12a: Associated DCH with HS-DSCH model for 1.28 Mcps TDD

Associating DCH with HS-DSCH, the model enables in the SS:

-	to define MAC-hs and multiplexing of logical channels DTCHs onto MAC-d flows;

-	to configure HS-DSCH transport channel and MAC-d flows;

-	to configure HS-PDSCHs and HS-SCCHs;

-	to define the H-RNTI value.

## 6.12b	DCH with HS-DSCH (MAC-ehs) model (FDD, Rel-7 or later)

The test model illustrates the relationship between various channels from logical channels to physical channels. All DCH are associated with a single HS-DSCH.

Figure 6.12b: Associated DCH with HS-DSCH model

Associating DCH with HS-DSCH, the model enables in the SS:

-	to define MAC-ehs and multiplexing of logical channels DTCHs & DCCHs onto MAC-d flows;

-	to configure HS-DSCH transport channel and MAC-d flows;

-	to configure HS-PDSCHs and HS-SCCHs;

-	to define the H-RNTI value.

-	to configure MIMO;

## 6.12c	HS-DSCH (MAC-hs/ehs) model (FDD, Rel-7 or later)(No DCH Associated)

The test model illustrates the relationship between various channels from logical channels to physical channels.

Figure 6.12c: HS-DSCH model without DCH associated

The model enables in the SS:

-	to define MAC-ehs/hs and multiplexing of logical channels DTCHs & DCCHs onto MAC-d flows;

-	to configure HS-DSCH transport channel and MAC-d flows/MAC-ehs Queues;

-	to configure HS-PDSCHs and HS-SCCHs;

-	to define the H-RNTI value(s);

-	to configure MIMO.

-	during the active set updating (soft handover), the test case configurations involves more than one cell; but one MAC-ehs entity is configured.

## 6.12d	HS-DSCH (MAC-ehs) model for DC/4C -HSDPA (FDD, Rel-8 or later)

The test model illustrates the relationship between various channels from logical channels to physical channels.

Figure 6.12d-1: HS-DSCH model with intra-NodeB DC-HSDPA

Figure 6.12d-2: HS-DSCH model for 4C-HSDPA with 3 secondary cells configured

The model enables in the SS to configure DC-HSDPA or 3C/4C-HSDPA:

-	MAC-ehs and the served RLC are cell-independent and are configured by using the cell-id =-1. During reconfigurations, cell changes and state transitions, the relevant counters in the RLC are maintained

-	to define MAC-ehs and multiplexing of logical channels DTCHs & DCCHs onto MAC-ehs queues;

-	to configure HS-DSCH transport channel and MAC-ehs Queues;

-	to configure HS-PDSCHs and HS-SCCHs on the serving cell and a secondary HS-DSCH cell for DC-HSDPA;

-	to configure HS-DPCCH in the serving cell;

-	to define the H-RNTI value(s);

-	MIMO and DC-HSDPA do not co-exist in Rel-8;

-	to configure combined DC-HSDPA and MIMO in Rel-9 or later Releases.

-	to configure HS-PDSCHs and HS-SCCHs on the serving cell and two or three secondary HS-DSCH cells for  4C-HSDPA in Rel-10 or later Releases. MIMO can be configured in the serving and/or secondary cells. When DC-HSUPA is configured in uplink, the first secondary serving HS-DSCH cell is the cell associated with the secondary uplink frequency.

The secondary cells are configured with P-CPICH common channel only according to TS 25.214 [12] section 4.2.4.

## 6.12e	HS-DSCH (MAC-ehs) model for Multiflow Operation (FDD, Rel-11 or later)

The test model illustrates the relationship between various channels from logical channels to physical channels.

Figure 6.12e-1: Multiflow intra-NodeB HS-DSCH model

Figure 6.12e-2: Multiflow inter-NodeB HS-DSCH model

The model enables in the SS to configure Multiflow:

- to configure HS-PDSCHs and HS-SCCHs on the serving cell, assisting serving HS-DSCH cell, secondary serving HS-DSCH cell and assisting secondary serving HS-DSCH cell;

- to configure HS-DPCCH in the time reference cell only; either in the serving HS-DSCH cell or in the assisting serving HS-DSCH cell. For example in figures 6.12e-1 and 6.12e-2 the assisting serving HS-DSCH cell is the time reference cell;

- to configure one MAC-ehs entity for intra-NodeB configuration or two MAC-ehs entities for inter-NodeB configuration. The MAC-ehs entities are referred to a nodeB_Id;

- for each MAC-ehs entity, to configure HS-DSCH transport channel and MAC-ehs queues;

- to define the H-RNTI value(s);

- to define multiplexing of logical channels DTCHs & DCCHs onto MAC-ehs queues;

- to map cells on a MAC-ehs entity indicated by nodeB_Id. When a cell is mapped to the MAC-ehs entity then data can be transmitted on the HS-PDSCH of that cell. When a cell is unmapped then it is deactivated and no data can be sent on HS-PDSCH.

- the served RLC is nodeB and cell independent and is configured using cell-id=-1. During reconfigurations, cell changes and state transitions, the relevant counters in the RLC are maintained. In inter-NodeB configuration the RLC entity is connected to two MAC-ehs entities.

The assisting serving, secondary serving and assisting secondary serving HS-DSCH cells are configured with all common channels. The assisting serving cell is part of the UE active set. The serving and secondary serving HS-DSCH cells have the same downlink timing, and the same applies to the assisting serving and assisting secondary serving HS-DSCH cells. MIMO is not considered.

For RLC test cases, the test method specified in clause 6.5 is applied, the RAB is configured in TM mode on SS side. The following RB Ids are used within the system simulator, depending on the RLC mode, and length indicator size being simulated.

| UE RLC mode | LI Size | RB Id |
| --- | --- | --- |
| AM | 7 | tsc_RB_AM_7_RLC (-12) |

## 6.13	E-DCH model (Rel-6 or later)

### 6.13.1	MAC-e/MAC-es test model

The E-DCH model illustrates the relationship between various channels from logical channel to physical channels. In this model the TTCN writer can:

-	define MAC-e/es and multiplexing of logical channels onto MAC-d flows;

-	configure E-DCH transport channel and MAC-d flows;

-	configure E-DPDCH, E-DPCCH, E-HICH, E-RGCH and E-AGCH.

MAC-es and the served RLC are cell-independent and are configured by using the cell-id = -1. During reconfigurations, cell changes and state transitions, the relevant counters in the RLC are maintained.

For the reason of simplicity, the E-DCH testing model does not shown the relation between E-DCH and related DCH and HS-DPCH, however the TTCN writer shall understand that the E-DCH active set is a subset of the DCH active set, when configuring E-DCH in the SS the TTCN writer shall keep this requirement respected.

During the active set updating (soft handover), the test case configurations may involve more than one cell. Those cells are under the control of the same Node B (intra-node) or under several Node B's (inter-node). For the signalling testing no macro diversity is required in the SS. In such test configurations only one E-DPDCH is necessary to be configured (together with corresponding E-DCH) for each Node B. Preferably, the E-DPDCH in the serving E-DCH cell controlled by the serving Node B is chosen for the configuration. In the inter-node soft handover cases, the E-DPDCH in a Non-serving RL cell of another Node B may require to be configured, instead of the one in the serving E-DCH cell. When the configuration involves several Node B's only one MAC-e (Node B) is necessary to be connected to the MAC-es. All possible connections are represented by dashed line and the preferable connection is the connection between MAC-es and the Node B controlling the serving E-DCH cell.

Since the UL-DPCCH is needed as reference channel for the E-DPCCH the UL-DPCH is configured in every cell where an E-DCH is configured (i.e. in serving and non-serving cell). In order to simplify the implementation and to avoid macro diversity in all non-serving cells the UL-DPCH is configured without the associated  transport channel configuration, i.e. the physical channel is not connected to MAC-d.

Figure 6.13.1: The model of E-DCH testing with MAC-e/MAC-es

### 6.13.2	MAC-i/MAC-is test model (Rel-8 or later)

For channel configuration and activate set update, the same requirement apply as for MAC-e/Mac-es in clause 6.13.1.

The MAC-i/is E-DCH test model is provided in Figure 6.13.2.

Figure 6.13.2: The model of E-DCH testing with MAC-i/MAC-is

#### 6.13.2.1	MAC-i/MAC-is test model for Enhanced UL in Cell_FACH (Rel-8 or later)

The MAC-i/is E-DCH test model for enhanced UL in cell_FACH and further enhanced cell_FACH (Rel-11 or later) is provided in Figure 6.13.2.1.

Figure 6.13.2.1: The model of E-DCH testing with MAC-i/MAC-is
for enhanced uplink in CELL_FACH state

If the UE is in CELL_FACH state or Idle mode, the  E-DCH active set consists of the serving E-DCH cell.  When CCH is configured, MAC-is entity is configured per E-DCH cell basis, i.e. configured in the serving E-DCH cell, as well as in the neighbour E-DCH cells.

#### 6.13.2.2	MAC-i/MAC-is test model for DC-HSUPA (Rel-9 or later)

The E-DCH model illustrates the relationship between various channels from logical channel to physical channels in DC-HSUPA configuration. In this model the TTCN can:

-	define MAC-i/-is and multiplexing of logical channels onto MAC-d flows;

-	configure E-DCH transport channel and MAC-d flows on all cells;

-	configure E-DPDCH, E-DPCCH, E-HICH, E-RGCH and E-AGCH on primary and secondary uplink frequency.

MAC-is and the served RLC are cell-independent and are configured by using the cell-id = -1. During reconfigurations, cell changes and state transitions, the relevant counters in the RLC are maintained.

In DC-HSUPA, a single MAC-i entity is shared by the primary and the secondary serving radio link. The MAC-i entity is mapped to a NodeB id and is connected to the MAC-is entity.

During the active set updating (soft handover) involving cells under the control of several Node Bs (inter-node), two MAC-i entities are configured. Only one of the two MAC-i entities is connected to the MAC-is entity. T he same design in clause 6.13.1 is applied.

For the reason of simplicity, the E-DCH testing model does not show the relation between E-DCH and related DCH and HS-DPCH, however, it is understood that the E-DCH active set is a subset of the DCH active set, when configuring E-DCH in the SS. The HS-DPCCH channel is configured only in the primary uplink frequency cell. Since the UL-DPCCH is needed as reference channel for the E-DPCCH the UL-DPCH is configured in every cell where ver an E-DCH is configured (i.e. in serving and secondary serving cell).

During the active set updating (soft handover), the added RL are configured in the same way as for MAC-e/es.

Figure 6.13.2.2: The model of E-DCH testing with MAC-i/MAC-is for DC-HSUPA

## 6.14	MBMS model (Rel-6 or later)

The MBMS test model illustrates the relationship between various channels, from logical channel to physical channels applied to the MBMS test. The MBMS-dedicated stand-alone SCCPCH, MICH, MAC-m, MCCH, MSCH and MTCH are configured by the TTCN.

During softcombining, MTCHs which have the same logical channel identity but in different cells are connected to the same UM RLC.

Figure 6.14: The model of MBMS testing

### 6.14.1	MBMS RLC test model

#### 6.14.1.1	RLC test model for MTCH test

For RLC tests on MTCH a TR radio bearer is configured (tsc_RB_MTCH_RLC_TR). Similar to the UM mode, data scheduling is applied at the test.

When reconfiguring back from TR to UM mode, MAC is reconfigured and RLC is released/reconfigured again. It implies that UM will restart with sequence number 0; the corresponding RLC state variables are set to 0. Sequence numbers between the one used in the last PDU sent in TR mode and the sequence number 0 are considered by the UE as lost sequence numbers and shall have no impact on the test in the UM mode.

#### 6.14.1.2	RLC test model for MCCH test

For RLC tests the MCCH critical messages can be sent as a DL sequence of PER encoded UM RLC PDUs in RLC TR mode. To achieve this, the normal UM radio bearer on MCCH is replaced with a TR radio bearer configured with a negative RB Id (tsc_RB_MCCH_RLC_TR). This is achieved by reconfiguring MAC and releasing the existing RLC UM entity /configuring a new TR RLC entity. It results in only one RLC entity being able to map on MCCH at the test.

It is assumed that the necessary MCCH data are completely sent out before the reconfiguration procedure mentioned above takes place.

When changing from UM to TR the care should be taken on the sequence numbers used in the RLC PDUs. That can be achieved by querying the SN from SS with CRLC_SequenceNumber_REQ. When changing back to UM the RLC may continue with the sequence number following the last sequence number used before changing to TR mode. That implies, the UM part of the RLC in SS does not need to take care of the UM PDUs sent in TR mode. The UE will regard it as PDU lost.

To support re-synchronization the 'specialLI' of the RLC_UM_CriticalMCCHMsg_REQ following TR mode can be set to TRUE.

## IP signalling

In the TTCN-2 implementation, no IP signalling is handled because of the limitation of the test language. For IPv4 or IPv4IPv6 UE, only an IPv4 address is assigned in the NAS signalling in the Activate PDP context Accept message. For IPv6 only UE, a full IPv6 address is assigned in the NAS signalling in the Activate PDP context Accept message.

In the TTCN-3 implementation, IP signalling is referred to 3GPP TS 36.523-3 [62] clause 4.4.1.1. For IPv4 or IPv6 UE, an IPv4 or a full IPv6 address is respectively assigned in the Activate PDP context Accept message. For IPv4IPv6 UE the both IP addresses are assigned.

## 6.16	Supplementary Service test method and architecture

### 6.16.1	Test configuration

The SS test method is shown in Figure 6A.2.

The upper tester triggers the UE to perform the supplementary service task required by the test case.

The supplementary services are encoded in a TCAP [52] ROSE component [53] of the facility information element [51] conveyed in the NAS messages. The facility information element is encoded in BER in TTCN referred to 3GPP TS 24.080 [50], clause 3.6.1 and 3GPP TS 29.002 [51], clause 17.1.1.

## 6.17	UTRAN-WLAN Inter working Test Model

Refer to 3GPP TS 36.523-3 [62], clause 4.4.6.2.

# 6A	TTCN-3 Test method and testing architecture

The following signalling conformance tests are specified in TTCN-3:

Rel-10 or later;

UE capability audit;

Network Initiated Secondary PDP context;

Supplementary Service.

The system architecture and the test model are based on 3GPP TS 36.523-3 [62].

NOTE: The TTCN-3 specification of A-GNSS test cases is defined in 37.571-4 [64].

## 6A.1	Test system architecture

### 6A.1.1	General system architecture

Refer to 3GPP TS 36.523-3 [62], clause 4.1.1.

### 6A.1.2	Component architecture

The component architecture as defined in 3GPP TS 36.523-3 [62], clause 4.1.2 is applied. The TTCN-3 master test component (MTC) and the UTRAN TTCN-3 parallel component (PTC) are configured.

## 6A.2	Test model

The test model defined in clause 6 of the present test specification is applied; the figure below shows how it is mapped on the TTCN-3 component architecture.

Figure 6A.2: UTRAN TTCN-3 Test Model

UTRAN PDCP, if necessary, is configured according to 3GPP TS 36.523-3 [62] clause 4.4.1.1.

## 6A.3	ASP specifications

### 6A.3.1	ASPs for Control Primitive Transmission

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CPHY_CONFIG_REQ |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CPHY |  |
| CPHY_RL_Setup_FDD_REQ |  | clause 7.3.2.2.11 |
| CPHY_RL_Setup_TDD_REQ |  | clause 7.3.2.3.1 |
| CPHY_RL_Modify_FDD_REQ |  | clause 7.3.2.2.9 |
| CPHY_RL_Modify_TDD_REQ |  | clause 7.3.2.3.1 |
| CPHY_RL_Release_REQ |  | clause 7.3.2.2.10 |
| CPHY_TrCH_Config_FDD_REQ |  | clause 7.3.2.2.13 |
| CPHY_TrCH_Config_TDD_REQ |  | clause 7.3.2.2.13 |
| CPHY_TrCH_Release_REQ |  | clause 7.3.2.2.14 |
| CPHY_Cell_Config_FDD_REQ |  | clause 7.3.2.2.2 |
| CPHY_Cell_Config_TDD_REQ |  | clause 7.3.2.3.1 |
| CPHY_Cell_Release_REQ |  | clause 7.3.2.2.3, see note |
| CPHY_Ini_REQ |  | clause 7.3.2.2.4 |
| CPHY_Cell_TxPower_Modify_REQ |  | clause 7.3.2.2.5 |
| CPHY_Frame_Number_REQ |  | clause 7.3.2.2.6 |
| CPHY_HS_DPCCH_CQI_MultiCell_REQ |  | clause 6A.3.1.3.1 |
| CPHY_HS_DPCCH_CQI_MultiCell_IND |  | clause 6A.3.1.3.1 |
| CPHY_MeasurementConfig_REQ |  | clause 6A.3.1.3.2 |
| CPHY_PRACH_Measurement_REQ |  | clause 7.3.2.2.8 |
| CPHY_AICH_NegAckModeSet_REQ |  | clause 6A.3.1.3.7 |
| NOTE: The Cell Release ASP can only ever be called at the end of the UTRAN side of the test case. Nothing else will occur on this cell within the test case after this ASP has been called. |  |  |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CPHY_CONFIG_CNF |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CPHY |  |
| CPHY_RL_Setup_CNF |  | clause 7.3.2.2.11 |
| CPHY_RL_Modify_CNF |  | clause 7.3.2.2.9 |
| CPHY_RL_Release_CNF |  | clause 7.3.2.2.10 |
| CPHY_TrCH_Config_CNF |  | clause 7.3.2.2.13 |
| CPHY_TrCH_Release_CNF |  | clause 7.3.2.2.14 |
| CPHY_Cell_Config_CNF |  | clause 7.3.2.2.2 |
| CPHY_Cell_Release_CNF |  | clause 7.3.2.2.3 |
| CPHY_Ini_CNF |  | clause 7.3.2.2.4 |
| CPHY_Cell_TxPower_Modify_CNF |  | clause 7.3.2.2.5 |
| CPHY_Frame_Number_CNF |  | clause 7.3.2.2.6 |
| CPHY_Sync_IND |  | clause 7.3.2.2.12 |
| CPHY_Out_of_Sync_IND |  | clause 7.3.2.2.7 |
| CPHY_HS_DPCCH_CQI_MultiCell_CNF |  | clause 6A.3.1.3.1 |
| CPHY_MeasurementConfig_CNF |  | clause 6A.3.1.3.2 |
| CPHY_PRACH_Measurement_CNF |  | clause 7.3.2.2.8 |
| CPHY_PRACH_Measurement_Report_IND |  | clause 7.3.2.2.8 |
| CPHY_AICH_NegAckModeSet_CNF |  | clause 6A.3.1.3.7 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CMAC_CONFIG_REQ |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CMAC |  |
| CMAC_Config_FDD_REQ |  | clause 7.3.2.2.17 |
| CMAC_Config_TDD_REQ |  | clause 7.3.2.2.17 |
| CMAC_SYSINFO_Config_REQ |  | clause 7.3.2.2.22 |
| CMAC_SecurityMode_Config_REQ |  | clause 7.3.2.2.20 |
| CMAC_Ciphering_Activate_REQ |  | clause 7.3.2.2.16 |
| CMAC_PAGING_Config_FDD_REQ |  | clause 7.3.2.2.18 |
| CMAC_PAGING_Config_TDD_REQ |  | clause 7.3.2.2.18 |
| CMAC_MACes_Config_REQ |  | clause 7.3.2.2.17d |
| CMAC_MACe_Config_FDD_REQ |  | clause 7.3.2.2.17b |
| CMAC_MACe_Config_TDD_REQ |  | clause 7.3.2.2.17b |
| CMAC_MACe_NodeB_CellMapping_REQ |  | clause 7.3.2.2.17c |
| CMAC_MAChs_MACehs_TFRCconfigure_FDD_REQ |  | clause 7.3.2.2.17a |
| CMAC_MAChs_MACehs_TFRCconfigure_TDD_REQ |  | clause 7.3.2.3.1 |
| CMAC_MACi_Config_FDD_REQ |  | clause 7.3.2.2.17k |
| CMAC_MACi_NodeB_CellMapping_REQ |  | clause 7.3.2.2.17l |
| CMAC_MACis_Config_REQ |  | clause 7.3.2.2.17m |
| CMAC_MACi_AG_REQ |  | clause 7.3.2.2.17n |
| CMAC_MACi_E_TFC_Restriction_REQ |  | clause 7.3.2.2.17p |
| CMAC_MACehs_HARQAssign_MultiFlows_REQ |  | clause 7.3.2.2.17a1 |
| CMAC_ConfigMACehs_NodeB_REQ |  | clause 6A.3.1.3.3 |
| CMAC_MACehs_NodeB_CellMapping_REQ |  | clause 6A.3.1.3.4 |
| CMAC_MACehs_HARQAssign_NodeB_REQ |  | clause 6A.3.1.3.5 |
| CMAC_FACH_MeasOccas_REQ |  | clause 7.3.2.2.16a |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CMAC_CONFIG_CNF |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CMAC |  |
| CMAC_Config_CNF |  | clause 7.3.2.2.17 |
| CMAC_SYSINFO_Config_CNF |  | clause 7.3.2.2.22 |
| CMAC_SecurityMode_Config_CNF |  | clause 7.3.2.2.20 |
| CMAC_Ciphering_Activate_CNF |  | clause 7.3.2.2.16 |
| CMAC_PAGING_Config_CNF |  | clause 7.3.2.2.18 |
| CMAC_MACes_Config_CNF |  | clause 7.3.2.2.17d |
| CMAC_MACe_Config_CNF |  | clause 7.3.2.2.17b |
| CMAC_MACe_NodeB_CellMapping_CNF |  | clause 7.3.2.2.17c |
| CMAC_MAChs_MACehs_TFRCconfigure_CNF |  | clause 7.3.2.2.17a |
| CMAC_MACi_Config_CNF |  | clause 7.3.2.2.17k |
| CMAC_MACi_NodeB_CellMapping_CNF |  | clause 7.3.2.2.17l |
| CMAC_MACis_Config_CNF |  | clause 7.3.2.2.17m |
| CMAC_MACi_AG_CNF |  | clause 7.3.2.2.17n |
| CMAC_MACi_E_TFC_Restriction_CNF |  | clause 7.3.2.2.17p |
| CMAC_MACehs_HARQAssign_MultiFlows_CNF |  | clause 7.3.2.2.17a1 |
| CMAC_ConfigMACehs_NodeB_CNF |  | clause 6A.3.1.3.3 |
| CMAC_MACehs_NodeB_CellMapping_CNF |  | clause 6A.3.1.3.4 |
| CMAC_MACehs_HARQAssign_NodeB_CNF |  | clause 6A.3.1.3.5 |
| CMAC_FACH_MeasOccas_CNF |  | clause 7.3.2.2.16a |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CRLC_CONFIG_REQ |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CRLC |  |
| CRLC_Config_FDD_REQ |  | clause 7.3.2.2.24 |
| CRLC_Config_TDD_REQ |  | clause 7.3.2.2.24 |
| CRLC_Sequence_Number_REQ |  | clause 7.3.2.2.29 |
| CRLC_SecurityMode_Config_REQ |  | clause 7.3.2.2.28 |
| CRLC_Ciphering_Activate_REQ |  | clause 7.3.2.2.23 |
| CRLC_Integrity_Activate_REQ |  | clause 7.3.2.2.25 |
| CRLC_SetRRC_MessageSN_REQ |  | clause 7.3.2.2.28a |
| CRLC_RRC_MessageSN_REQ |  | clause 7.3.2.2.27a |
| CRLC_Resume_REQ |  | clause 7.3.2.2.27 |
| CRLC_Suspend_REQ |  | clause 7.3.2.2.31 |
| CRLC_ProhibitRLC_Ack_REQ |  | clause 7.3.2.2.26a |
| CRLC_BindTestDataInMultipleMACehs_PDU_MultiFlows_REQ |  | clause 7.3.2.2.22c |
| CRLC_ReportReceivedCellId_REQ |  | clause 7.3.2.2.26d |
| CRLC_BindTestDataInMultipleMACehs_PDU_NodeB_REQ |  | clause 6A.3.1.3.6 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CRLC_CONFIG_CNF |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CRLC |  |
| CRLC_Config_CNF |  | clause 7.3.2.2.24 |
| CRLC_Sequence_Number_CNF |  | clause 7.3.2.2.29 |
| CRLC_SecurityMode_Config_CNF |  | clause 7.3.2.2.28 |
| CRLC_Ciphering_Activate_CNF |  | clause 7.3.2.2.23 |
| CRLC_integrity_Activate_CNF |  | clause 7.3.2.2.25 |
| CRLC_Integrity_Failure_IND |  | clause 7.3.2.2.26 |
| CRLC_SetRRC_MessageSN_CNF |  | clause 7.3.2.2.28a |
| CRLC_RRC_MessageSN_CNF |  | clause 7.3.2.2.27a |
| CRLC_Resume_CNF |  | clause 7.3.2.2.27 |
| CRLC_Suspend_CNF |  | clause 7.3.2.2.31 |
| CRLC_ProhibitRLC_Ack_CNF |  | clause 7.3.2.2.26a |
| CRLC_BindTestDataInMultipleMACehs_PDU_MultiFlows_CNF |  | clause 7.3.2.2.22c |
| CRLC_ReportReceivedCellId_CNF |  | clause 7.3.2.2.26d |
| CRLC_BindTestDataInMultipleMACehs_PDU_NodeB_CNF |  | clause 6A.3.1.3.6 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CPDCP_CONFIG_REQ |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CPDCP |  |
| CPDCP_Config_REQ |  | clause 7.3.6.2.2 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_CPDCP_CONFIG_CNF |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_CPDCP |  |
| CPDCP_Config_CNF |  | clause 7.3.6.2.2 |

#### 6A.3.1.1	FDD Control ASP extension types

The control ASP extension types used in the relAspTypeExtension fields are defined for Rel-10 or later, they are implemented in TTCN-3 ATSs only. In TTCN-2, the relAspTypeExtension fields are defined as SEQUENCE{}.

##### 6A.3.1.1.1	CPHY_RL_Setup extension

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DPCHInfo_r10OrLaterExtensionType |
| Comment | Applicable Rel-10 or later rel12_AspExt is applied when DTCH is mapped on DL-DPCH |
| Type Definition |  |
| CHOICE { rel10_AspExt SEQUENCE { ul_DPCHInfo UL_DPCH_Info_r7 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r10 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo_r10 OPTIONAL, ss_UL_DPCCH_DRX_Info SS_UL_DPCCH_DRX_Info OPTIONAL, ss_DRX_Info DTX_Info OPTIONAL }, rel11_AspExt SEQUENCE { ul_DPCHInfo UL_DPCH_Info_r11 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r11 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo_r10 OPTIONAL, ss_UL_DPCCH_DRX_Info SS_UL_DPCCH_DRX_Info OPTIONAL, ss_DRX_Info DTX_Info OPTIONAL }, rel12_AspExt SEQUENCE { -- Rel-12 and later ul_DPCHInfo UL_DPCH_Info_r11 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r12 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo_r10 OPTIONAL, ss_UL_DPCCH_DRX_Info SS_UL_DPCCH_DRX_Info OPTIONAL, ss_DRX_Info DTX_Info OPTIONAL }, rel13_AspExt SEQUENCE { -- Rel-13 and later ul_DPCHInfo UL_DPCH_Info_r11 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r13 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo_r10 OPTIONAL, ss_UL_DPCCH_DRX_Info SS_UL_DPCCH_DRX_Info OPTIONAL, ss_DRX_Info DTX_Info OPTIONAL } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r10 |
| Comment | Applicable Rel-10 or later |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r10, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r7, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r7 }, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r11 |
| Comment | Applicable Rel-11 or later |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r11, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r7, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r7 }, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r12 |
| Comment | Applicable Rel-12 or later Used only when DTCH is mapped on DL-DPCH |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r12, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r12, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r7 }, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r13 |
| Comment | Applicable Rel-13 or later Used only when DTCH is mapped on DL-DPCH |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r12, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r12, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r13 }, rl_Information RL_Information } |  |

| Type Name | HS_DPCCHInfo_r10 |
| --- | --- |
| Comment | Applicable to 4C-HSDPA Rel-10 or later secondaryServing_HS_DSCH_Cell indicates if the HS-DSCH secondary serving cell is activated (see 3GPP TS 25.214 [12], clause 6A.1) The HS-DPCCH code allocation and slot format are set according to 3GPP TS 25.213 [66], clause 4.3.1.2.2. |
| Type Definition |  |
| SEQUENCE { cqi_RepetitionFactor CQI_RepetitionFactor, ackNackRepetitionFactor ACK_NACK_repetitionFactor, mimoStatus BOOLEAN DEFAULT FALSE, firstSecondaryServing_HS_DSCH_Cell ENUMERATED { deactivated (0), activated (1) } DEFAULT deactivated, secondSecondaryServing_HS_DSCH_Cell ENUMERATED { deactivated (0), activated (1) } DEFAULT deactivated, thirdSecondaryServing_HS_DSCH_Cell ENUMERATED { deactivated (0), activated (1) } DEFAULT deactivated -- Relevant when third secondary cell is configured } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r10OrLaterExtensionType |
| Comment | Rel-10 or later. hS_PDSCH_Info rel10 extension is applied to 4C HSDPA configurations. The IEs related to HS-DSCH physical layer categories are mutually exclusive. One of the IE shall be present in the SS configuration. The IE "hsdsch_physical_layer_category_ext4" is present when 2 secondary cells are configured. The IE "hsdsch_physical_layer_category_ext5" is present when 3 secondary cells are configured. The IE "hsdsch_physical_layer_category_ext6" is present when multi-cell operation on 5 or 6 cells are configured. The IE "hsdsch_physical_layer_category_ext7" is present when multi-cell operation on 7 or 8 cells are configured. The IE "hsdsch_physical_layer_category_ext8" is present when MIMO with 4 transmit antenna are configured. |
| Type Definition |  |
| CHOICE { rel10_AspExt SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, hsdsch_physical_layer_category_ext2 HSDSCH_physical_layer_category_ext2 OPTIONAL, hsdsch_physical_layer_category_ext3 HSDSCH_physical_layer_category_ext3 OPTIONAL, hsdsch_physical_layer_category_ext4 HSDSCH_physical_layer_category_ext4 OPTIONAL, hsdsch_physical_layer_category_ext5 HSDSCH_physical_layer_category_ext5 OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r9, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH ss_DTX_Info DRX_Info OPTIONAL, hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimo_Parameters MIMO_Parameters_r9 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, hs_SCCH_SttdIndicator BOOLEAN DEFAULT FALSE }, rel11_AspExt SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, hsdsch_physical_layer_category_ext2 HSDSCH_physical_layer_category_ext2 OPTIONAL, hsdsch_physical_layer_category_ext3 HSDSCH_physical_layer_category_ext3 OPTIONAL, hsdsch_physical_layer_category_ext4 HSDSCH_physical_layer_category_ext4 OPTIONAL, hsdsch_physical_layer_category_ext5 HSDSCH_physical_layer_category_ext5 OPTIONAL, hsdsch_physical_layer_category_ext6 HSDSCH_physical_layer_category_ext6 OPTIONAL, hsdsch_physical_layer_category_ext7 HSDSCH_physical_layer_category_ext7 OPTIONAL, hsdsch_physical_layer_category_ext8 HSDSCH_physical_layer_category_ext8 OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r11, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH ss_DTX_Info DRX_Info OPTIONAL, hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimo_Parameters MIMO_Parameters_r9 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, hs_SCCH_SttdIndicator BOOLEAN DEFAULT FALSE }, commonRel11_AspExt SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo HS_SCCH_SystemInfo, hs_dsch_PagingSystemInformation HS_DSCH_PagingSystemInformation OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, ss_HS_DSCH_DtxCellFach HS_DSCH_DrxCellfach_info OPTIONAL, ss_HS_DSCH_DtxCellFach_SecondDtx HS_DSCH_DrxCellfach_SecondDrx_info OPTIONAL }, rel12_AspExt SEQUENCE { -- Rel-12 or later hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, hsdsch_physical_layer_category_ext2 HSDSCH_physical_layer_category_ext2 OPTIONAL, hsdsch_physical_layer_category_ext3 HSDSCH_physical_layer_category_ext3 OPTIONAL, hsdsch_physical_layer_category_ext4 HSDSCH_physical_layer_category_ext4 OPTIONAL, hsdsch_physical_layer_category_ext5 HSDSCH_physical_layer_category_ext5 OPTIONAL, hsdsch_physical_layer_category_ext6 HSDSCH_physical_layer_category_ext6 OPTIONAL, hsdsch_physical_layer_category_ext7 HSDSCH_physical_layer_category_ext7 OPTIONAL, hsdsch_physical_layer_category_ext8 HSDSCH_physical_layer_category_ext8 OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r12, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH ss_DTX_Info DRX_Info_r12 OPTIONAL, hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimo_Parameters MIMO_Parameters_r9 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, hs_SCCH_SttdIndicator BOOLEAN DEFAULT FALSE } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_HICH_Info_r10OrLaterExtensionType |
| Comment | Rel-11 or later. Only one of e_HICHInfo, e_HICHInfoCommonEDCH or e_HICHInfo_r11 can be present. |
| Type Definition |  |
| CHOICE { rel11_AspExt SEQUENCE { e_HICHInfo_r11 E_HICH_Information_r11 OPTIONAL } } |  |

##### 6A.3.1.1.1a	CPHY_RL_Modify extension

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyRlModifyReq_FDD_r10OrLaterExtensionType |
| Comment | Applicable Rel-12 or later Used only for DCH Enhancements |
| Type Definition |  |
| CHOICE { rel12_AspExt SEQUENCE { dch_EnhancementsInfo_FDD DCH_Enhancements_Info_FDD OPTIONAL } } |  |

##### 6A.3.1.1.2	CMAC_MACehs_HARQAssign_MultiFlows extension

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HARQAssign_MultiFlows_r10OrLaterExtensionType |
| Comment | Rel-10 or later Extension to assign the HARQ processes for the simultaneous transmission of the MAC-ehs PDUs on the different flows on the second and third secondary cells on the same TTI when 4C-HSDPA is configured. In 4C-HSDPA, the primaryFlow corresponds to the serving cell, the secPrimaryFlow corresponds to the first secondary cell, the thirdPrimaryFlow corresponds to the second secondary cell, and the fourthPrimaryFlow corresponds to the third secondary cell, when configured. In 4C-HSDPA and MIMO: the primaryFlow and secondaryFlow corresponds to the serving cell. The secPrimaryFlow and secSecondaryFlow corresponds to the first secondary cell. The thirdPrimaryFlow and thirdSecondaryFlow corresponds to the second secondary cell. The fourthPrimaryFlow and fourthSecondaryFlow corresponds to the third secondary cell, when configured. |
| Type Definition |  |
| CHOICE { rel10_AspExt SEQUENCE { thirdPrimaryFlowHarqProcessId INTEGER(0..15¦31), -- of third cell thirdSecondaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, -- for MIMO in third cell fourthPrimaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, -- of fourth cell for 4C-HSDPA fourthSecondaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL -- for MIMO in fourth cell for 4C-HSDPA } } |  |

##### 6A.3.1.1.3	CMAC_MAChs_MACehs_TFRCconfigure extension

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TFRCconfigure_r10OrLaterExtensionType |
| Comment | Rel-10 or later Extension to configure the TFRC selection in the MAC-ehs entity when 4C-HSDPA or Multiflow is configured. |
| Type Definition |  |
| CHOICE { rel10_AspExt CHOICE { sS_Configured_4C SEQUENCE { -- Rel-10 3C/4C HSDPA serving_TFRC TFRC_Configured_Type, firstSecondary_TFRC TFRC_Configured_Type, secondSecondary_TFRC TFRC_Configured_Type, thirdSecondary_TFRC TFRC_Configured_Type OPTIONAL, -- for the third secondary cell when configured iniHS_PDSCH_TxPower DL_TxPower, -- default offset related -- to p-CPICH or s-CPICH hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL }, explicitlyConfigured_4C SEQUENCE { -- Rel-10 3C/4C HSDPA serving_TFRC TFRC_Explicit_Type OPTIONAL, -- when omitted then no data is sent on the serving HS-DSCH cell firstSecondary_TFRC TFRC_Explicit_Type OPTIONAL, -- when omitted then no data is sent on the secondary HS-DSCH cell secondSecondary_TFRC TFRC_Explicit_Type OPTIONAL, -- when omitted then no data is sent on the secondary HS-DSCH cell thirdSecondary_TFRC TFRC_Explicit_Type OPTIONAL -- for the third secondary cell when configured -- when omitted then no data is sent on the secondary HS-DSCH cell }, explicitlyConfigured_4C_MIMO SEQUENCE { -- Rel-10 3C/4C HSDPA serving_TFRC TFRC_ExplicitMIMO_Type OPTIONAL, -- when omitted then no data is sent on the serving HS-DSCH cell firstSecondary_TFRC TFRC_ExplicitMIMO_Type OPTIONAL, -- when omitted then no data is sent on the secondary HS-DSCH cell secondSecondary_TFRC TFRC_ExplicitMIMO_Type OPTIONAL, -- when omitted then no data is sent on the secondary HS-DSCH cell thirdSecondary_TFRC TFRC_ExplicitMIMO_Type OPTIONAL -- for the third secondary cell when configured -- when omitted then no data is sent on the secondary HS-DSCH cell } }, rel11_AspExt CHOICE { sS_Configured_MF SEQUENCE { -- Rel-11 Multiflow HSDPA nodeB_Id INTEGER(0..63), serving_TFRC TFRC_Configured_Type OPTIONAL, -- serving HS-DSCH cell assistingServing_TFRC TFRC_Configured_Type OPTIONAL, -- assisting serving HS-DSCH cell secondary_TFRC TFRC_Configured_Type OPTIONAL, -- secondary serving HS-DSCH cell assistingSecondary_TFRC TFRC_Configured_Type OPTIONAL, -- assisting secondary serving HS-DSCH cell iniHS_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH }, explicitlyConfigured_MF SEQUENCE { -- Rel-11 Multiflow HSDPA nodeB_Id INTEGER(0..63), serving_TFRC TFRC_Explicit_Type OPTIONAL, -- when omitted then no data is sent on the serving HS-DSCH cell assistingServing_TFRC TFRC_Explicit_Type OPTIONAL, -- when omitted then no data is sent on the assisting serving -- HS-DSCH cell secondary_TFRC TFRC_Explicit_Type OPTIONAL, -- when omitted then no data is sent on the secondary serving -- HS-DSCH cell assistingSecondary_TFRC TFRC_Explicit_Type OPTIONAL -- when omitted then no data is sent on the assisting secondary -- serving HS-DSCH cell } } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TFRC_Configured_Type |
| Comment | Rel-10 or later. |
| Type Definition |  |
| SEQUENCE { minChannelisationCodeOffset INTEGER (1..15), maxNoOfChannelisationCodes INTEGER (1..15), mimoStatus BOOLEAN DEFAULT FALSE } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TFRC_Explicit_Type |
| Comment | Rel-10 or later. |
| Type Definition |  |
| SEQUENCE{ modulationScheme ModulationScheme, channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), tbSizeIndexOnHS_SCCH INTEGER (0..63), minimumInterTTIinterval INTEGER (1..3), redundancyVersions RedundancyVersionList, hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TFRC_ExplicitMIMO_Type |
| Comment | Rel-10 or later. |
| Type Definition |  |
| SEQUENCE{ modulationSchemeAndNumTB INTEGER(0..7), -- set according to table 14 of 25.212 -- Values 1,2 and 5 are used for 64QAM+MIMO channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), precodingWeight2 INTEGER (0..3), -- set according to table 14a of 25.212 primaryTB_SizeIndexOnHS_SCCH INTEGER (0..63), secondaryTB_SizeIndexOnHS_SCCH INTEGER (0..63) OPTIONAL, -- present only if second TB is to be tx -- as per modulationSchemeAndNumTB minimumInterTTIinterval INTEGER (1..3), primaryRedundancyVersions RedundancyVersionList, secondaryRedundancyVersions RedundancyVersionList OPTIONAL, -- present only if second TB is to be tx -- as per modulationSchemeAndNumTB hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH } |  |

##### 6A.3.1.1.4	CRLC_BindTestDataInMultipleMACehs_PDU_MultiFlows extension

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | BindTestDataInMultipleMACehs_r10OrLaterExtensionType |
| PCO Type | CSAP |
| Comment | Extension to request of binding subsequent data sending on the third and fourth secondary cells. |
| Type Definition |  |
| CHOICE { rel10_AspExt SEQUENCE { -- Extension for 4C-HSDPA with or without MIMO thirdPrimaryFlowNumOfSDUs INTEGER, -- Number of RLC SDU's for primary flow of third cell thirdSecondaryFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for secondary flow of third cell fourthPrimaryFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for fourth cell primary flow fourthSecondaryFlowNumOfSDUs INTEGER OPTIONAL -- Number of RLC SDU's for fourth cell secondary flow } } |  |

##### 6A.3.1.1.5	CMAC_Config

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCHInfo_r10OrLaterExtensionType |
| Comment | Rel-11 or later For Rel-11 MAC-ehs configuration: When ulconnectedTrCHList, ulTFCS, dlconnectedTrCHList and dlTFCS are omitted and ehs_DSCH_Flows or E-HS-DSCH_Common Flows or ehs_DSCH_Flows_r11 is present this ASP configures an MAC-ehs entity. Only one of hsDSCHMacdFlows or ehs_DSCH_Flows or ehs_DSCH_Flows_r9 or ehs_DSCH_Flows_r11 can be present. For Rel-12, UL DCH configuration: Only one of ulTFCS in TrCHInfo and ulTFCS _r12 can be present. For Rel-13 MAC-hs or MAC-ehs configuration: Only one of hsDSCHMacdFlows or ehs_DSCH_Flows or ehs_DSCH_Flows_r9 or ehs_DSCH_Flows_r11 or ehs_DSCH_Flows_r13 can be present. |
| Type Definition |  |
| CHOICE { rel11_AspExt SEQUENCE { -- Rel-11 or later ehs_DSCH_Flows_r11 EHS_DSCH_Flows_r11 OPTIONAL }, rel12_AspExt SEQUENCE { -- Rel-12 or later ulTFCS_r12 TFCS_r12 OPTIONAL }, rel13_AspExt SEQUENCE { -- Rel-13 or later ehs_DSCH_Flows_r13 EHS_DSCH_Flows_r13 OPTIONAL } } |  |

| Type Name | EHS_DSCH_Flows_r11 |
| --- | --- |
| Comment | Applicable Rel-11 or later |
| Type Definition |  |
| SEQUENCE { harqInfo_r11 HARQ_Info_r11 OPTIONAL, addOrReconfMAC_ehs_ReordQ SS_MAC_ehs_AddReconfReordQueueList_r11 OPTIONAL, ackNackRepetitionFactor ACK_NACK_repetitionFactor OPTIONAL, mimoStatus BOOLEAN OPTIONAL } |  |

| Type Name | EHS_DSCH_Flows_r13 |
| --- | --- |
| Comment | Applicable Rel-13 or later |
| Type Definition |  |
| SEQUENCE { harqInfo_r13 HARQ_Info_r13 OPTIONAL, dl_MAC_HeaderType CHOICE { mac_hs SS_AddOrReconfMAC_dFlow, mac_ehs SS_MAC_ehs_AddReconfReordQueueList_r11 }, ackNackRepetitionFactor ACK_NACK_repetitionFactor OPTIONAL, mimoStatus BOOLEAN OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_ehs_AddReconfReordQueueList_r11 |
| Comment | Applicable Rel-11 or later |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxQueueIDs)) OF SS_MAC_ehs_AddReconfReordQ_r11 |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_ehs_AddReconfReordQ_r11 |
| Comment | Applicable Rel-11or later The priority of PriorityQueue shall set according to the priority of logical channels which is mapped on to this priority queue. NOTE: The range of priority of PriorityQueue is from 0 to 7 and 0 is the lowest priority. |
| Type Definition |  |
| SEQUENCE { mAC_ehs_AddReconfReordQ MAC_ehs_AddReconfReordQ_r11, priority INTEGER(0..7) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacConfigReq_r10OrLaterExtensionType |
| Comment | Applicable Rel-10 or later servingSecondaryCells is provided in 4C configurations |
| Type Definition |  |
| CHOICE { rel10_AspExt SEQUENCE { servingSecondaryCells SEQUENCE { -- 3C/4C HSDPA configuration servingCellId INTEGER(0..63), secondaryCellActivationList SecondaryCellActivationList_Type } OPTIONAL } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryCellActivationList_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..3)) OF SecondaryCellActivation_Type |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryCellActivation_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE { secondaryCellId INTEGER(0..63), secondaryServingStatus ENUMERATED { deactivated (0), activated (1) } DEFAULT deactivated } |  |

##### 6A.3.1.1.6	CRLC_Config

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_RLC_Info_r10OrLaterExtensionType |
| Comment | Rel-11 or later |
| Type Definition |  |
| CHOICE { rel11_AspExt SEQUENCE { -- Rel-11 or later sS_ul_RLC_Mode DL_RLC_Mode_r11 OPTIONAL, sS_dl_RLC_Mode SS_DL_RLC_Mode OPTIONAL, rlc_OneSidedReEst BOOLEAN DEFAULT FALSE, altE_bitInterpretation ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for UM RLC mode useSpecialValueOfHEField ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for AM RLC mode uM_SN_DeliveryMode ENUMERATED { nonConfigured(0), configured(1)} DEFAULT nonConfigured } } |  |

#### 6A.3.1.2	TDD Control ASP extension types

No type specific to TDD is needed.

#### 6A.3.1.3	FDD and TDD Control ASP types

##### 6A.3.1.3.1	CPHY_HS_DPCCH_CQI_MultiCell (Rel-10 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_MultiCell_CNF |
| PCO Type | CSAP |
| Comment | To Confirm CPHY_HS_DPCCH_CQI_MultiCell_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_MultiCell_REQ |
| PCO Type | CSAP |
| Comment | To enable the SS to start reporting N times of the CQI value received on the HS-DPCCH on cellId when 3C/4C-HSDPA is configured. After N times the SS stops reporting. N is specified as numberOfReports. At the SS initialization reporting of CQI values is disabled. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, numberOfReports INTEGER(1..32) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_MultiCell_IND |
| PCO Type | CSAP |
| Comment | SS generates one indication per subframe when a CQI/PCI value is received on HS-DPCCH after invocation of ASP CPHY_HS_DPCCH_CQI_MultiCell_REQ and before the numberOfReports is reached. This ASP is used for verifying whether the UE has configured the HS-DSCH and starts reception of HS-DSCH (3GPP TS 25.331 [21], subclause 8.6.6.34 and 3GPP TS 25.212 [58], subclause 4.7.4.2). |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, cfn INTEGER (0..255), subframe INTEGER (0..4), cqiPci_List cqiPci_List_Type } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | cqiPci_List_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..2)) OF cqiPci_Type |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | cqiPci_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE { cqi INTEGER (0..1023), pci INTEGER (0..3) OPTIONAL, -- MIMO cqiType ENUMERATED {cqiTypeA(0), cqiTypeB(1)} OPTIONAL -- MIMO } |  |

##### 6A.3.1.3.2	CPHY_MeasurementConfig

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_MeasurementConfig_REQ |
| PCO Type | CSAP |
| Comment | To configure measurement parameters cellDCHMeasOccasionInfo_TDD128: to configure SS for CELL_DCH measurement occasion according to TS 25.331[21] clause 8.5.11a and 8.6.7.26. No DL transmission shall be performed in the calculated timeslot. The activationTime is set to ‘activateNow’. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), activationTime SS_ActivationTime, measurement CHOICE { rel9 SEQUENCE { cellDCHMeasOccasionInfo_TDD128 CellDCHMeasOccasionInfo_TDD128_r9 -– LCR TDD Rel-9 } } } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_MeasurementConfig_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CPHY_MeasurementConfig_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

##### 6A.3.1.3.3	CMAC_ConfigMACehs_NodeB (Rel-11 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_ConfigMACehs_NodeB_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_ConfigMACehs_NodeB_REQ successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_ConfigMACehs_NodeB_REQ |
| PCO Type | CSAP |
| Comment | FDD rel-11 or later To configure a MAC-ehs entity. - setup is used for creation of the MAC-ehs entity; - release the MAC-ehs entity; - reconfiguration is used to modify the MAC-ehs parameters. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, activationTime SS_ActivationTime, configMessage CHOICE { setup CmacConfigReq_r11OrLater, reconfigure CmacConfigReq_r11OrLater, release NULL } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacConfigReq_r11OrLater |
| Comment |  |
| Type Definition |  |
| CHOICE { rel11 SEQUENCE { uE_Info UE_Info, trCHInfo TrCHInfo, trCH_LogCHMapping TrCH_LogCHMappingList1, associatedPhychConfigToFollow BOOLEAN DEFAULT FALSE, tSN_FieldExtension BOOLEAN DEFAULT FALSE } } |  |

##### 6A.3.1.3.4	CMAC_MACehs_NodeB_CellMapping (Rel-11 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_NodeB_CellMapping_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACehs_NodeB_CellMapping_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_NodeB_CellMapping_REQ |
| PCO Type | CSAP |
| Comment | FDD rel-11 or later. To put a set of cells under the control of a MAC-ehs entity indicated by nodeB_Id, which is configured by CMAC_ConfigMACehs_NodeB_REQ. This ASP establishes the routing relation between HS-DSCH related channels in these cells with a MAC-ehs entity. When a cell is mapped to the MAC-ehs entity then data can be transmitted on the HS-PDSCH of that cell. When a cell is unmapped then it is deactivated. A cell is mapped to only one NodeB. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), activationTime SS_ActivationTime, cellList SEQUENCE (SIZE (1..4)) OF MappingCellInfo_Type } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MappingCellInfo_Type |
| Comment | For Multiflow configuration: - multiflowCellConfiguration is included. - timeReferenceCell is set to true for the time reference cell: either the serving or the assisting serving HS-DSCH cell. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), multiflowCellConfiguration MultiflowCellConfiguration_Type OPTIONAL, timeReferenceCell ENUMERATED { true } OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MultiflowCellConfiguration_Type |
| Comment |  |
| Type Definition |  |
| ENUMERATED { serving, assistingServing, secondaryServing, assistingSecondaryServing } |  |

##### 6A.3.1.3.5	CMAC_MACehs_HARQAssign_NodeB (Rel-11 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_HARQAssign_NodeB_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-11 or later Confirm a previous CMAC_MACehs_HARQAssign_NodeB_REQ being successful. |
| Type Definition |  |
| SEQUENCE { } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_HARQAssign_NodeB_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-11 or later To assign the HARQ processes for the simultaneous transmission of the MAC-ehs PDUs on the different flows in the same TTI (assisting serving and assisting secondary serving being shifted by cell timing offset). After successful transmission the MAC-ehs entities return back to normal operation. In the normal operation suitable HARQ processes are selected by HARQ entities in the MAC-ehs to serve the MAC-ehs PDU without TTCN intervening. |
| Type Definition |  |
| SEQUENCE{ harqFlowsPerNodeB SEQUENCE (SIZE (1..2)) OF NodeB_HarqFlowList_Type } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | NodeB_HarqFlowList_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), servingFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, assistingServingFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, secondaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, assistingSecondaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL } |  |

##### 6A.3.1.3.6	CRLC_BindTestDataInMultipleMACehs_PDU_NodeB (Rel-11 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInMultipleMACehs_PDU_NodeB_CNF |
| PCO Type | CSAP |
| Comment | To confirm the request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(-1..63), routingInfo RoutingInfo, -- RB ID result ENUMERATED{failure(0), success(1)} } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInMultipleMACehs_PDU_NodeB_REQ |
| PCO Type | CSAP |
| Comment | To request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on up to 4 HS-DSCH configured on one or two NodeB in simultaneous MAC-ehs PDUs. On the request, the transmission of the test data is temporarily suppressed on the radio bearer till all SDUs for all flows are received by RLC layer on the Radio Bearer. After receiving all SDU's the RLC layer submits to MAC such that all of them are sent in MAC-ehs PDUs at the same timing. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(-1..63), routingInfo RoutingInfo, -- RB ID desired to be given bindingNodeB SEQUENCE (SIZE (1..2)) OF NodeB_FlowList_Type } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | NodeB_FlowList_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), servingFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for the serving HS-DSCH cell assistingServingFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for the assisting serving HS-DSCH cell secondaryFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for the secondary serving HS-DSCH cell assistingSecondaryFlowNumOfSDUs INTEGER OPTIONAL -- Number of RLC SDU's for the assisting secondary serving HS-DSCH cell } |  |

##### 6A.3.1.3.7	CPHY_AICH_NegAckModeSet (Rel-11 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_AICH_NegAckModeSet_REQ |
| PCO Type | CSAP |
| Comment | To configure SS to transmit a specific number of Negative AICH Acknowledge - numNegACK defines how many access preambles are negatively acknowledged - nextAICH_Mod is the AICH acknowledgement mode applied after the numNegACK negative acknowledgements have been transmitted. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, numNegACK INTEGER(1..100), nextAICH_Mode AICH_Mode } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_AICH_NegAckModeSet_CNF |
| PCO Type | CSAP |
| Comment | To confirm setting of Negative AICH Acknowledge Mode |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

### 6A.3.2	ASPs for Data Transmission and Reception

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_RLC_AM_REQ |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_AM |  |
| RLC_AM_DATA_REQ |  | clause 7.3.2.2.34 |
| RLC_AM_TestDataReq |  | clause 7.3.3.1 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_RLC_AM_IND |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_AM |  |
| RLC_AM_DATA_CNF |  | clause 7.3.2.2.34 |
| RLC_AM_DATA_IND |  | clause 7.3.2.2.34 |
| RLC_AM_TestDataInd |  | clause 7.3.3.1 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_RLC_TR_REQ |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_TM |  |
| RLC_TR_DATA_REQ |  | clause 7.3.2.2.33 |
| RLC_TR_TestDataReq |  | clause 7.3.3.1 |

| TTCN-3 ASP Definition |  |  |
| --- | --- | --- |
| Type Name | U_RLC_TR_IND |  |
| TTCN-3 Type | union |  |
| Port | UTRAN_TM |  |
| RLC_TR_DATA_IND |  | clause 7.3.2.2.33 |
| RLC_TR_TestDataInd |  | clause 7.3.3.1 |

| TTCN-3 ASP Definition | Port | Defined in |
| --- | --- | --- |
| RLC_UM_DATA_REQ | UTRAN_UM | clause 7.3.2.2.35 |
| RLC_UM_DATA_IND | UTRAN_UM | clause 7.3.2.2.35 |
| RRC_DataReq | UTRAN_Dc | clause 7.1.2 |
| RRC_DataReqInd | UTRAN_Dc | clause 7.1.2 |
| PDCP_DATA_REQ | UTRAN_PDCP | clause 7.3.6.3.2 |
| PDCP_DATA_IND | UTRAN_PDCP | clause 7.3.6.3.2 |

The Invalid_DL_DCCH_Message type is replaced with:

| Type Name | Invalid_DL_DCCH_Message |
| --- | --- |
| TTCN-3 Type | NULL |

## 6A.4	Upper Tester Interface

Refer to 3GPP TS 36.523-3 [62], clause 5.

## 6A.5	IXIT Proforma

Refer to Annex B of the present test specification.

# 7	PCO and ASP definitions

## 7.1	NAS PCO and ASP definitions

### 7.1.1	NAS PCO Definitions

Table 7.1.1.1: Dc PCO Type Declarations

| PCO Type Declarations |  |
| --- | --- |
| PCO Type | Dc_SAP |
| Role | LT |
| Comments | The PCO type for NAS testing |

Table 7.1.1.2: Dc PCO Declarations

| PCO Declarations |  |
| --- | --- |
| PCO Name | Dc |
| PCO Type | Dc_SAP |
| Role | LT |
| Comments | Carry transmission and reception of NAS messages |

### 7.1.2	Primitives used at Dc PCO

The Dc PCO is used to transmit and receive NAS (MM, CC, SM, SS) messages. Two categories of primitives are operated at the Dc PCO:

-	RRC_DataReq for transmission of a NAS PDU;

-	RRC_DataInd for reception of a NAS PDU.

These primitives are declared in TTCN tabular form, see Tables 7.1.2.1-1 and 7.1.2.1-2.

Table 7.1.2.1-1: Primitive RRC_DataInd used at the Dc PCO

| ASP Name | RRC_DataInd |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | Dc_SAP |  |  |  |
| Comments | The ASP is used to indicate the receipt of the NAS PDU message using acknowledged operation (NAS <- RRC). |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER | Cell Id |
| rB_Id |  |  | SS_RB_Identity | RB identity (RB3 or RB4) |
| ch |  |  | LogicChGERAN | Logical channel (used for interworking with GERAN) |
| sapId |  |  | SapId | RRC SAP identifier (SAP0) |
| cN_Domain |  |  | SS_CN_DomainIdentity | CN domain identity |
| start |  |  | START_Value | Mandatory in INITIAL DIRECT TRANSFER |
| msg |  |  | PDU | NAS PDU |
| Detailed Comments |  |  |  |  |

Table 7.1.2.1-2: Primitive RRC_DataReq used at the Dc PCO

| ASP Name | RRC_DataReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | Dc_SAP |  |  |  |
| Comments | The ASP is used to request the transmission of the NAS PDU message using acknowledged operation (NAS -> RRC). |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER | Cell Id |
| rB_Id |  |  | SS_RB_Identity | RB identity (RB3 or RB4) |
| ch |  |  | LogicChGERAN | Logical channel (used for interworking with GERAN) |
| sapId |  |  | SapId | RRC SAP identifier (SAP0) |
| cN_Domain |  |  | SS_CN_DomainIdentity | CN domain identity |
| msg |  |  | PDU | NAS PDU |
| Detailed Comments |  |  |  |  |

The RB Identity and CN domain parameters defined in the primitives are mandatory for UTRAN and not applicable for GERAN.

The START parameter is mandatory in INITIAL DIRECT TRANSFER; each time when it is received the new START shall be downloaded to the SS to reinitialize counters-C and counters-I.

The LogicChGSM and SapId parameters are mandatory for GERAN and not applicable for UTRAN. They are defined because they may be used for future TTCN test cases.

Except the initial, uplink and downlink direct transfer procedures, the NAS TTCN specification uses the TTCN test steps to realize all RRC functions for testing. The single layer test concept is kept for the NAS tests.

A simple RRC emulation shall be maintained for the NAS tests. It has four functions:

-	Emulate the three direct transfer procedures.

-	Convert the NAS downlink messages defined in 3GPP TS 24.008 [9] in table format to the NAS message in ASN.1 octet string specified in 3GPP TS 25.331 [21]. Convert the NAS uplink message in the reverse way.

-	PER encoding and decoding.

-	Have the integrity protection.

RB3 and RB4 are specifically used for the NAS signalling. When an uplink message entered the receiving buffer at AM-SAP from the RLC emulation, either an RRC test step if running will take it out; or the RRC emulation if running will pick the received message from the buffer. Activation of any RRC test steps and activation of any NAS test steps at the same time shall be excluded in TTCN (no concurrency between them).

## 7.2	Ut PCO and ASP definitions

### 7.2.1	Ut PCO Declarations

The Ut PCO is served as the interface to the UE EMMI for remote control of operations, which have to be performed during execution of a test case such as to switch the UE on/off, initiate a call, etc.

Table 7.2.1.1: Declaration of the upper tester PCO type

| PCO Type Declarations |  |
| --- | --- |
| PCO Type | MMI |
| Role | UT |
| Comments | The PCO type for MMI or EMMI of the upper tester |

Table 7.2.1.2: Declaration of the Ut PCO

| PCO Declarations |  |
| --- | --- |
| PCO Name | Ut |
| PCO Type | MMI |
| Role | UT |
| Comments | Carry transmission commands and reception of results for the upper tester |

### 7.2.2	Primitives used at Ut PCO

The Ut PCO is used to indicate to the upper tester actions and to receive the acknowledgement of these actions. The AT commands are used wherever the suitable commands exist within 3GPP TS 27.007 [23], 3GPP TS 27.005 [22] and 3GPP TS 27 060 [24]. An MMI command is used, when AT commands does not exit for the action to performed. The primitives used at the Ut PCO, are declared in TTCN tabular form, see Table 7.2.2.1.

Table 7.2.2.1: Primitives used at the Ut PCO

| Primitive | Parameters | Use |
| --- | --- | --- |
| AT_CmdReq | Command: IA5String SMS_BlockMode: HEXSTRING | Request an AT command to the upper tester. |
| AT_CmdInd | Command: IA5String SMS_BlockMode: HEXSTRING | Indication of a result from the upper tester. |
| AT_CmdCnf | Result: BOOLEAN ResultString: IA5String SMS_BlockMode: HEXSTRING | Return a positive or negative result from the command previously sent. Both the Boolean result and String parameter are optional. |
| MMI_CmdReq | Command: IA5String | Request a command to the upper tester. |
| MMI_CmdCnf | Result: BOOLEAN ResultString: IA5String | Return a positive or negative result from the command previously sent. The String parameter is optional. |

The AT_CmdReq primitive for sending AT commands is mostly used to trigger electronically an uplink access, such as initiating of a call, attaching or detaching, starting packet data transfer etc. The MMI_primitive is defined mainly for observation of some test events via a test operator, such as checking DTMF tone or checking called party number, etc.

The AT_CmdInd primitive for receiving AT commands is mostly used to transfer unsolicited result codes from the UE to the lower tester.

The SMS_BlockMode parameter is used to control and observe the Block mode procedure for SMS. This parameter is not yet used; it is defined for future development. The Command and SMS_BlockMode parameters are mutually exclusive

For the Command in the AT_CmdReq and AT_CmdInd primitives, the verbose format is used as defined in 3GPP TS 27.007 [23]. For the Command in MMI_CmdReq, just a descriptive IA5 string line, like "Check DTMF tone" is used.

## 7.3	RRC PCO and ASP definitions

### 7.3.1	AM/UM/TM PCO and ASP definitions

#### 7.3.1.1	SAP and PCO for data transmission and reception

Table 7.3.1.1.1: Declaration of the RRC PCO Type

| PCO Type Definition |  |
| --- | --- |
| PCO Type | DSAP |
| Role | LT |
| Comment | DATA transmission and reception |

Table 7.3.1.1.2: PCO TM declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Name | TM |
| PCO Type | DSAP |
| Role | LT |
| Comment | Carry Transparent Mode RLC PDU |

Table 7.3.1.1.3: PCO AM declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Name | AM |
| PCO Type | DSAP |
| Role | LT |
| Comment | Carry Acknowledged Mode RLC PDU |

Table 7.3.1.1.4: PCO UM declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Name | UM |
| PCO Type | DSAP |
| Role | LT |
| Comment | Carry Unacknowledged Mode RLC PDU |

Table 7.3.1.1.5: PCO BMC declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Name | BMC |
| PCO Type | DSAP |
| Role | LT |
| Comment | Provide Unacknowledged Mode BMC data transmission service |

### 7.3.2	Control PCO and ASP

#### 7.3.2.1	SAP and PCO for control primitives transmission and reception

Table 7.3.2.1.1: SAP declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Type | CSAP |
| Role | LT |
| Comment | Control primitives transmission and reception |

Table 7.3.2.1.2: PCO CPHY

| PCO Definition |  |
| --- | --- |
| PCO Name | CPHY |
| PCO Type | CSAP |
| Role | LT |
| Comment | Control Physical Layer |

Table 7.3.2.1.3: PCO CRLC

| PCO Definition |  |
| --- | --- |
| PCO Name | CRLC |
| PCO Type | CSAP |
| Role | LT |
| Comment | Control RLC Layer |

Table 7.3.2.1.4: PCO CMAC

| PCO Definition |  |
| --- | --- |
| PCO Name | CMAC |
| PCO Type | CSAP |
| Role | LT |
| Comment | Control MAC Layer |

Table 7.3.2.1.5: PCO CBMC

| PCO Definition |  |
| --- | --- |
| PCO Name | CBMC |
| PCO Type | CSAP |
| Role | LT |
| Comment | Control BMC Layer |

Table 7.3.2.1.6: ExternalAsn1Codec declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Type | ExternalAsn1Codec |
| Role | LT |
| Comment | Control decoder primitives transmission and reception |

Table 7.3.2.1.7: PCO CCodec

| PCO Definition |  |
| --- | --- |
| PCO Name | CCodec |
| PCO Type | ExternalAsn1Codec |
| Role | LT |
| Comment | Control asn.1 CONTAINING decoder |

Table 7.3.2.1.8: ExternalStructure Codec declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Type | ExternalStructureCodec |
| Role | LT |
| Comment | Control decoder from a BITSTRING to a structure type primitives transmission and reception |

Table 7.3.2.1.9: PCO CCodecS

| PCO Definition |  |
| --- | --- |
| PCO Name | CCodecS |
| PCO Type | ExternalStructureCodec |
| Role | LT |
| Comment | Control BITSTRING to structure type decoder |

#### 7.3.2.2	Control ASP Type Definition

##### 7.3.2.2.1	CPHY_AICH_AckModeSet

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_AICH_AckModeSet_REQ |
| PCO Type | CSAP |
| Comment | To request for setting of AICH Acknowledge Mode |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, aICH_Mode AICH_Mode } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_AICH_AckModeSet_CNF |
| PCO Type | CSAP |
| Comment | To confirm setting of AICH Acknowledge Mode |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AICH_Mode |
| Comment | Normal operation: The AICH will operate as normal, and will acknowledge or negatively acknowledge on all UE RACH/E-DCH transmission attempts, appropriately. No Acknowledge: The AICH shall not transmit acknowledge or Negative Acknowledge on all UE RACH/E-DCH transmission attempts. Negative Acknowledge: The AICH shall transmit Negative Acknowledge on all UE RACH/E-DCH transmission attempts |
| Type Definition |  |
| ENUMERATED { normal (0), noAck (1), negACK (2) } |  |

##### 7.3.2.2.2	CPHY_Cell_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm to setup the cell parameter |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to setup the cell parameter. The unit of tcell is chip; the unit of sfnOffset is frame number. The sfnOffset is defined as the number of frames the SFN shall be shifted, i.e. the frames lagging behind or in advance, in comparison to a system reference time. The both interpretations are valid for the test. The primary scrambling code number of the cell is 16*primaryScramblingCode_SS. The unit of dLTxAttenuationLevel is dB; If set to 123 the cell becomes a non-suitable off cell (CPICH_Ec ≤ -122 dBm/3.84 MHz of an off cell). |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), tcell INTEGER(0..38399), sfnOffset INTEGER(0..4095), frequencyInfo FrequencyInfo, primaryScramblingCode_SS INTEGER(0..511), cellTxPowerLevel CellTxPowerLevel, dLTxAttenuationLevel INTEGER(0..30¦123), frequencyBandIndicator FrequencyBandFDD, relAspTypeExtension SEQUENCE {} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CellTxPowerLevel |
| Comment | The defaultCellTxPowerLvl is a default setting and is used for the most signalling tests. The real total cell DL Tx power level equals to the sum of the DL Tx power of the individual physical channels configured. The totalCellTxPowerLvl applies to e.g. the idle mode tests in a non-default multi-cell radio environment. |
| Type Definition |  |
| CHOICE { defaultCellTxPowerLvl NULL, totalCellTxPowerLvl DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | FrequencyBandFDD |
| Comment | The frequency band indicator indicates how to interpret the radio frequency broadcast. |
| Type Definition |  |
| CHOICE { frequencyBandIndicator RadioFrequencyBandFDD, frequencyBandIndicator2 RadioFrequencyBandFDD2, frequencyBandIndicator3 RadioFrequencyBandFDD3, relAspTypeExtension SEQUENCE{} } |  |

##### 7.3.2.2.3	CPHY_Cell_Release

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Release_CNF |
| PCO Type | CSAP |
| Comment | The confirmation to the CPHY_Cell_Release_Req |
| Type Definition |  |
| SEQUENCE { soft_Reset BOOLEAN, cell_ID_List SEQUENCE (SIZE (1..8)) OF INTEGER(0..63) -- cell IDs } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Release_REQ |
| PCO Type | CSAP |
| Comment | 1. This Primitive with "Soft_Reset" flag ON gives a common known starting point/state of SS for a test case. The SS performs the following whenever it receives this primitive with "Soft_Reset" flag ON: Releases all configured Channels and cells (if any) irrespective of Cell ID list IE. 2. Releases the associated Memory Buffers (if any). 3. Cancels all active timers (if any) With "Soft_Reset" flag OFF: 1. Releases cells listed in IE Cell_ID_List and associated configured Channels (if any) 2. Releases the Memory Buffers(if any) associated with Cells listed in IE Cell_ID_List 3. Cancels all active timers (if any) associated with Cells listed in IE Cell_ID_List. Presence of rbConfig_List IE indicates the configured Channels in the associated cells in the list to be released. If rbConfig_List IE is present, then for each entry in cell_ID_List a corresponding entry in rbConfig_List shall be present. The configuration values applied in rbConfig_List follows RB_ConfigType. |
| Type Definition |  |
| SEQUENCE { soft_Reset BOOLEAN, cell_ID_List SEQUENCE (SIZE (1..8)) OF INTEGER(0..63) , -- cell IDs rbConfig_List SEQUENCE (SIZE (1..8)) OF INTEGER OPTIONAL } |  |

##### 7.3.2.2.3a	CPHY_Cell_TimingAdjust

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_TimingAdjust_CNF |
| PCO Type | CSAP |
| Comment | To confirm to set the cell timing parameter |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_TimingAdjust_REQ |
| PCO Type |  |
| Comment | To request the cell identified by cellId to adjust its timing to the amount of deltTcell given in this ASP. Usage: The deltTcell is a relative value, which specifies the timing difference between the original timing (i.e. before calling this ASP) and the timing after calling this ASP. Example: assume the cell 1 is initially 5 chips advance of cell 2 in timing, the test case requires timing change of cell 2 being -19 chips delay with respect to cell 1 (i.e. cell 2 is 19 chips advance of cell 1 in timing) , TTCN can use this ASP with deltTcell = -24 and celId = cell 2 to adjust the timing; later on the test case requires timing change again: cell 2 being 19 chips delay with respect to cell 1, TTCN can use this ASP with deltTcell = 38 and cellId = cell 2 to achieve the required timing difference. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), deltTcell INTEGER(-128..127) } |  |

##### 7.3.2.2.3b	CPHY_Detect_TFCI

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_DetectTFCI_CNF |
| PCO Type | CSAP |
| Comment | To confirm to CPHY_DetetTFCI_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) ), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_DetectTFCI_REQ |
| PCO Type | CSAP |
| Comment | To set the mode of the SS for detecting whether the specified list of TFCI values occurred. Usage: At the SS initialization, the default mode is stop. When the mode is set to start, the SS shall detect whether the specified list of TFCI values (tfci_List) happens on the specified uplink physical channel. When happened the SS generates a CPHY_TFCI_Detected_IND and stop further detection. Otherwise keeps monitoring until a CPHY_DetectTFCI_REQ with mode = stop received. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, mode ENUMERATED{start(0), stop(1)}, tfci_List TFCI_List } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TFCI_Detected_IND |
| PCO Type | CSAP |
| Comment | To indicate the TFCI value specified in the CPHY_DetectTFCI_REQ has been detected. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, tfciValue INTEGER (0..1023) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TFCI_List |
| Comment |  |
| Type Definition |  |
| SEQUENCE ( SIZE (1..1024)) OF INTEGER (0..1023) |  |

##### 7.3.2.2.4	CPHY_Ini

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Ini_REQ |
| PCO Type | CSAP |
| Comment | Request to initialize the test |
| Type Definition |  |
| ENUMERATED { defaultRadioEnvironment(0), nonDefaultMultiCell(1) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Ini_CNF |
| PCO Type | CSAP |
| Comment | Confirm the test initialization |
| Type Definition |  |
| SEQUENCE { confirmation NULL } |  |

##### 7.3.2.2.5	CPHY_Cell_TxPower_Modify

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_TxPower_Modify_CNF |
| PCO Type | CSAP |
| Comment | To confirm to change the DL power |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_TxPower_Modify_REQ |
| PCO Type | CSAP |
| Comment | To request to change the DL power If the Tx attenuation level value is set to 123, the cell becomes a non-suitable off cell (CPICH_Ec ≤ -122 dBm/3.84 MHz of an off cell). |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), dLTxAttenuationLevel INTEGER(0..40¦123) } |  |

##### 7.3.2.2.6	CPHY_Frame_Number

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Frame_Number_CNF |
| PCO Type | CSAP |
| Comment | To return the requested connection frame number. The routingInfo indicates a physical channel. The frameNumber is set to (SFN mod 256) when requested on S-CCPCH in PCH/FACH. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, frameNumber INTEGER (0..255) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Frame_Number_REQ |
| PCO Type | CSAP |
| Comment | To request the physical layer to return a connection frame number on which the next message can be sent at the specified PCO on the specified logical channel. The return frame number shall leave time from current frame number in order to leave some execution time for TTCN preparing next message. The routingInfo indicates a physical channel |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.6a	CPHY_SFN (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_SFN_CNF |
| PCO Type | CSAP |
| Comment | To return the requested system frame number of the cell. The routingInfo indicates the P-CCPCH physical channel. In MBMS the MICH Connection Frame Number (CFN) corresponds to the Cell SFN of the frame in which the start of the S-CCPCH frame is located. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, currentSFN INTEGER(0..4095) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_SFN_REQ |
| PCO Type | CSAP |
| Comment | To request the physical layer to return the current SFN of the cell. The routingInfo indicates the P-CCPCH physical channel. In MBMS the MICH Connection Frame Number (CFN) corresponds to the Cell SFN of the frame in which the start of the S-CCPCH frame is located. The timing of S-CCPCH relative to P-CCPCH can be configured as timingOffSet in steps of 256 chips. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.6b	CPHY_MBMS_MICH_q (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_MBMS_MICH_q_CNF |
| PCO Type | CSAP |
| Comment | To confirm CPHY_MBMS_MICH_q_REQ. The routingInfo indicates the MICH physical channel. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_MBMS_MICH_q_REQ |
| PCO Type | CSAP |
| Comment | To request the physical layer to transmit MBMS notification Indicators on the MICH physical channel. The transmission shall start on the SFN specified by the parameter mICHCFN and be continued for one modification period indicated by parameter modiCoefficent. The routingInfo indicates the MICH physical channel. The notification indicators to be transmitted are specified by the parameter indicatorList. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, mICHCFN MICH_CFN, modiPeriodCoefficent INTEGER(7..10), indicatorList MBMS_q_List } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MBMS_q_List |
| Comment | The maximum number of notification indicators per frame for the nn18 alternative is 18. The maximum number of notification indicators per frame for the nn36 alternative is 36. The maximum number of notification indicators per frame for the nn72 alternative is 72. The maximum number of notification indicators per frame for the nn144 alternative is 144. |
| Type Definition |  |
| CHOICE { nn18 Q_List18, nn36 Q_List36, nn72 Q_List72, nn144 Q_List144 } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Q_List18 |
| Comment | This type is a list which holds 128 or 256 or 512 or 1024 values of ListOf18q, each value of ListOf18q is also a list of (1..18) values of INTEGER. The first value of ListOf18q is corresponding to the MICH frame which starts the modification period, the second value of ListOf18q is corresponding to the next MICH frame in the modification period and so on, the last value of ListOf18q is corresponding to the last MICH frame of the modification period. Be noted that for different modification period configurations the number of values of ListOf18q are different and shall match the configuration. The SS sets the bits {b0, …, b287} of the MICH frame according to the INTEGER values in the ListOf18q corresponding to the MICH frame (see 3GPP TS 25.211 [40], subclause 5.3.3.15) |
| Type Definition |  |
| CHOICE { fRM128 SEQUENCE SIZE((128)) OF ListOf18q, fRM256 SEQUENCE SIZE((256)) OF ListOf18q, fRM512 SEQUENCE SIZE((512)) OF ListOf18q, fRM1024 SEQUENCE SIZE((1024)) OF ListOf18q } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ListOf18q |
| Comment | This type holds (1..18) values of the "q" for a MICH frame which MICH CFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs The values of q are calculated by TTCN according to formula: q=C×NI⊕C×SFNmodGmodG×NnG where: G =216, C = 25033; NI = Notification Indicator (0..65535) is computed by the TTCN for each TMGI according to the formula: NI=TMGI+TMGIGmodG where G = 216; the number of TMGI could be 1 to 18; SFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs; Nn = the number of notification indicators per frame: |
| Type Definition |  |
| SEQUENCE (SIZE(1..18)) OF INTEGER (0..17) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Q_List36 |
| Comment | This type is a list which holds 128 or 256 or 512 or 1024 values of ListOf36q, each value of ListOf36q is also a list of (1..36) values of INTEGER. The first value of ListOf36q is corresponding to the MICH frame which starts the modification period, the second value of ListOf36q is corresponding to the next MICH frame in the modification period and so on, the last value of ListOf36q is corresponding to the last MICH frame of the modification period. Be noted that for different modification period configurations the number of values of ListOf36q are different and shall match the configuration. The SS sets the bits {b0, …, b287} of the MICH frame according to the INTEGER values in the ListOf36q corresponding to the MICH frame (see 3GPP TS 25.211 [40], subclause 5.3.3.15). |
| Type Definition |  |
| CHOICE { fRM128 SEQUENCE (SIZE(128)) OF ListOf36q, fRM256 SEQUENCE (SIZE(256)) OF ListOf36q, fRM512 SEQUENCE (SIZE(512)) OF ListOf36q, fRM1024 SEQUENCE (SIZE(1024)) OF ListOf36q } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ListOf36q |
| Comment | This type holds (1..36) values of the "q" for a frame which MICH CFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs The values of q are calculated by TTCN according to formula: q=C×NI⊕C×SFNmodGmodG×NnG where: G = 216, C = 25033; NI = Notification Indicator (0..65535) is computed by the TTCN for each TMGI according to the formula: NI=TMGI+TMGIGmodG where G = 216; the number of TMGI could be 1 to 36; SFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs; Nn = the number of notification indicators per frame: |
| Type Definition |  |
| SEQUENCE (SIZE(1..36)) OF INTEGER (0..35) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Q_List72 |
| Comment | This type is a list which holds 128 or 256 or 512 or 1024 values of ListOf72q, each value of ListOf72q is also a list of (1..72) values of INTEGER. The first value of ListOf72q is corresponding to the MICH frame which starts the modification period, the second value of ListOf72q is corresponding to the next MICH frame in the modification period and so on, the last value of ListOf72q is corresponding to the last MICH frame of the modification period. Be noted that for different modification period configurations the number of values of ListOf72q are different and shall match the configuration. The SS sets the bits {b0, …, b287} of the MICH frame according to the INTEGER values in the ListOf72q corresponding to the MICH frame (see 3GPP TS 25.211 [40], subclause 5.3.3.15). |
| Type Definition |  |
| CHOICE { fRM128 SEQUENCE (SIZE(128)) OF ListOf72q, fRM256 SEQUENCE (SIZE(256)) OF ListOf72q, fRM512 SEQUENCE (SIZE(512)) OF ListOf72q, fRM1024 SEQUENCE (SIZE(1024)) OF ListOf72q } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ListOf72q |
| Comment | This type holds (1..72) values of the "q" for a frame which MICH CFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs The values of q are calculated by TTCN according to formula: q=C×NI⊕C×SFNmodGmodG×NnG where: G = 216, C = 25033; NI = Notification Indicator (0..65535) is computed by the TTCN for each TMGI according to the formula: NI=TMGI+TMGIGmodG where G = 216; the number of TMGI could be 1 to 72; SFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs; Nn = the number of notification indicators per frame: |
| Type Definition |  |
| SEQUENCE (SIZE(1..72)) OF INTEGER (0..71) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Q_List144 |
| Comment | This type is a list which holds 128 or 256 or 512 or 1024 values of ListOf144q, each value of ListOf144q is also a list of (1..144) values of INTEGER. The first value of ListOf144q is corresponding to the MICH frame which starts the modification period, the second value of ListOf144q is corresponding to the next MICH frame in the modification period and so on, the last value of ListOf144q is corresponding to the last MICH frame of the modification period. Be noted that for different modification period configurations the number of values of ListOf144q are different and shall match the configuration. The SS sets the bits {b0, …, b287} of the MICH frame according to the INTEGER values in the ListOf144q corresponding to the MICH frame (see 3GPP TS 25.211 [40], subclause 5.3.3.15). |
| Type Definition |  |
| CHOICE { fRM128 SEQUENCE (SIZE(128)) OF ListOf144q, fRM256 SEQUENCE (SIZE(256)) OF ListOf144q, fRM512 SEQUENCE (SIZE(512)) OF ListOf144q, fRM1024 SEQUENCE (SIZE(1024)) OF ListOf144q } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ListOf144q |
| Comment | This type holds (1..144) values of the "q" for a frame which MICH CFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs The values of q are calculated by TTCN according to formula: q=C×NI⊕C×SFNmodGmodG×NnG where: G = 216, C = 25033; NI = Notification Indicator (0..65535) is computed by the TTCN for each TMGI according to the formula: NI=TMGI+TMGIGmodG where G = 216; the number of TMGI could be 1 to 144; SFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs; Nn = the number of notification indicators per frame: |
| Type Definition |  |
| SEQUENCE (SIZE(1..144)) OF INTEGER (0..143) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MICH_CFN |
| Comment | Subclause 9.2.1.46a of 3GPP TS 25.433 [57] |
| Type Definition |  |
| INTEGER (0..4095) |  |

##### 7.3.2.2.6c	CPHY_MBMS_NI (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_MBMS_NI_CNF |
| PCO Type | CSAP |
| Comment | To confirm CPHY_MBMS_NI_REQ. The routingInfo indicates the MICH physical channel. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_MBMS_NI_REQ |
| PCO Type | CSAP |
| Comment | To request the physical layer to transmit MBMS Notification Indicators on the MICH physical channel. The transmission shall start on the SFN specified by the parameter startingTime and be continued for one modification period indicated by parameter modiPeriodCoefficent. The routingInfo indicates the MICH physical channel. The notification indicators to be transmitted are specified by the parameter indicatorList. If value of invert is TRUE, all notification indicators Nq are set to "0" and all other indicators are set to "1". |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, startingTime MICH_CFN, modiPeriodCoefficent INTEGER(7..10), nIList MBMSIndicatorList, invert BOOLEAN DEFAULT FALSE } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MBMSIndicatorList |
| Comment | The TTCN calculates Notification Indicator (NI, 0..65535) for each TMGI according to the formula: NI=TMGI+TMGIGmodG where G = 216 and sends these NI to the SS by the MBMSIndicatorList; The SS shall calculate the q values for every MICH frame of a modification period according to formula: q=C×NI⊕C×SFNmodGmodG×NnG where: G = 216, C = 25033; SFN = the SFN of the P-CCPCH radio frame during which the start of the MICH radio frame occurs; Nn = the number of notification indicators per frame: Then the SS sets the bits {b0, …, b287} of the MICH frame according to the calculated q values which corresponds to the MICH frame (see 3GPP TS 25.211 [40], subclause 5.3.3.15). |
| Type Definition |  |
| CHOICE { nn18 SEQUENCE (SIZE(1..18)) OF SEQUENCE { nI INTEGER (0..65535)}, nn36 SEQUENCE (SIZE(1..36)) OF SEQUENCE { nI INTEGER (0.. 65535)}, nn72 SEQUENCE (SIZE(1..72)) OF SEQUENCE { nI INTEGER (0.. 65535)}, nn144 SEQUENCE (SIZE(1..144)) OF SEQUENCE { nI INTEGER (0.. 65535)} } |  |

##### 7.3.2.2.7	CPHY_Out_of_Sync

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Out_of_Sync_IND |
| PCO Type | CSAP |
| Comment | To report that the physical channel synchronization (in FDD mode, sync with uplink DPCCH) was lost as detected by the SS receiver. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.8	CPHY_PRACH_Measurement

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_PRACH_Measurement_CNF |
| PCO Type | CSAP |
| Comment | To Confirm PRACH Measurement Req |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_PRACH_Measurement_REQ |
| PCO Type | CSAP |
| Comment | To request for Start or Stop of PRACH Measurements to be done every PRACH PREAMBLE or MESSAGE received. If reportSFN is set to TRUE, SS shall report in the IE PRACH_MeasurementReport the SFN value at which the event is received. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, pRACH_MeasurementInd PRACH_MeasurementInd, reportSFN BOOLEAN DEFAULT FALSE –- Rel-8 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PRACH_MeasurementInd |
| Comment | 1) StartMeas: The SS shall start the sending PRACH parameters Measurement report on CPHY PCO, for each PRACH Preamble or MESSAGE received from the UE by primitive CPHY_PRACH_Measurement_Report_IND on CPHY PCO. 2) StopMeas: The SS shall stop sending of PRACH parameters Measurement report on CPHY PCO, for each PRACH Preamble or MESSAGE received from the UE by primitive CPHY_PRACH_Measurement_Report_IND on CPHY PCO. |
| Type Definition |  |
| ENUMERATED { startMeas (0), stopMeas (1) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_PRACH_Measurement_Report_IND |
| PCO Type | CSAP |
| Comment | SS indicates a PRACH parameters measurement report for each PRACH Preambles or MESSAGE received from the UE |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, measurementReport PRACH_MeasurementReport } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PRACH_MeasurementReport |
| Comment | sfn is included if reportSFN is TRUE in CPHY_PRACH_Measurement_REQ. |
| Type Definition |  |
| SEQUENCE { usedPRACH_AcessSlot INTEGER (0..14), usedPRACH_Signature INTEGER (0..15) OPTIONAL, sfn INTEGER (0..4095) OPTIONAL –- Rel-8 or later } |  |

##### 7.3.2.2.9	CPHY_RL_Modify

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Modify_CNF |
| PCO Type | CSAP |
| Comment | To confirm to modify the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Modify_REQ |
| PCO Type | CSAP |
| Comment | To request to modify the Radio Link HardHandover (PhysicalChannelReconfig) ChannelizationCodeChange FrequencyChange PhysicalChannelModifyForTrCHReconfig CompressedMode( PhysicalChannelReconfig) Re_Synchronized HardHandover Softhandover |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, modifyMessage CphyRlModifyReq } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyRlModifyReq |
| Comment |  |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, physicalChannelInfo CHOICE { dpch_CompressedModeStatusInfo Dpch_CompressedModeStatusInfo, secondaryCCPCHInfo SecondaryCCPCHInfo, pRACHInfo PRACHInfo, dPCHInfo DPCHInfo, dPCHInfo_r5 DPCHInfo_r5OrLater, -- Rel-5 or later hS_PDSCHInfo HS_PDSCHInfo_r5OrLater, -- Rel-5 or later e_DPCHInfo SS_E_DPCH_Info_r6OrLater, -- Rel-6 or later e_AGCHInfo SS_E_AGCH_Info, -- Rel-6 or later e_HICHInfo SS_E_HICH_Info, -- Rel-6 or later e_RGCHInfo SS_E_RGCH_Info, -- Rel-6 or later mBMS_MICHInfo SS_MBMS_MICHConfigurationInfo_r6, -- Rel-6 or later aICHInfo AICHInfo, relAspTypeExtension SEQUENCE{} -- Rel-10 or later }, trchConfigToFollow BOOLEAN DEFAULT TRUE, relAspTypeExtension CphyRlModifyReq _ FDD _r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_ActivationTime |
| Comment |  |
| Type Definition |  |
| CHOICE { activationCFN ActivationTime, activateNow NULL } |  |

##### 7.3.2.2.10	CPHY_RL_Release

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Release_CNF |
| PCO Type | CSAP |
| Comment | PHY emulator confirms that a specified physical channel has been released. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Release_REQ |
| PCO Type | CSAP |
| Comment | To request to release the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, activationTime SS_ActivationTime } |  |

##### 7.3.2.2.11	CPHY_RL_Setup

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Setup_CNF |
| PCO Type | CSAP |
| Comment | To confirm to setup the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Setup_REQ |
| PCO Type | CSAP |
| Comment | To request to setup the associated transport channels and the Radio Link itself. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, setupMessage CphyRlSetupReq } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyRlSetupReq |
| Comment | To request to setup the Radio Link |
| Type Definition |  |
| SEQUENCE { physicalChannelInfo CHOICE { primaryCPICHInfo PrimaryCPICHInfo, secondaryCPICHInfo SecondaryCPICHInfo, primarySCHInfo PrimarySCHInfo, secondarySCHInfo SecondarySCHInfo, primaryCCPCHInfo PrimaryCCPCHInfo, secondaryCCPCHInfo SecondaryCCPCHInfo, pRACHInfo PRACHInfo, pICHInfo PICHInfo, aICHInfo AICHInfo, dPCHInfo DPCHInfo, pDSCHInfo PDSCHInfo, dPCHInfo_r5 DPCHInfo_r5OrLater, -- Rel-5 or later hS_PDSCHInfo HS_PDSCHInfo_r5OrLater, -- Rel-5 or later e_DPCHInfo SS_E_DPCH_Info_r6OrLater, -- Rel-6 or later e_AGCHInfo SS_E_AGCH_Info, -- Rel-6 or later e_HICHInfo SS_E_HICH_Info, -- Rel-6 or later e_RGCHInfo SS_E_RGCH_Info, -- Rel-6 or later mBMS_MICHInfo SS_MBMS_MICHConfigurationInfo_r6, -- Rel-6 or later relAspTypeExtension SEQUENCE{} -- Rel-10 or later }, activationTime SS_ActivationTime, trchConfigToFollow BOOLEAN DEFAULT TRUE, relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PrimaryCPICHInfo |
| Comment | mimoAntenna2CPICH indicates if MIMO is either not configured or configured with a P-CPICH or with an S-CPICH on the MIMO antenna2 (see figure 7 of 3GPP TS 25.214 [12], clause 9). dC_SecondaryServingCell indicates if the cell is configured as secondary HS-DSCH serving cell, without other common channel than P-CPICH. MIMO and DC-HSDPA do not co-exist. |
| Type Definition |  |
| SEQUENCE { dl_TxPower_PCPICH DL_TxPower_PCPICH, tx_diversityIndicator BOOLEAN, mimoAntenna2CPICH MimoAntennaConfigType DEFAULT noMimo, -- Rel-7 or later dC_SecondaryServingCell BOOLEAN DEFAULT FALSE, -- Rel-8 or later relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryCPICHInfo |
| Comment | The channel can be configured for the MIMO antenna2. The value antenna2pCPICH is not applicable. |
| Type Definition |  |
| SEQUENCE { scramblingCode INTEGER(0..15), dl_ChannelizationCode SF512_AndCodeNumber, dl_TxPower DL_TxPower, mimoAntenna MimoAntennaConfigType DEFAULT noMimo -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MimoAntennaConfigType |
| Comment | Rel-7 or later |
| Type Definition |  |
| ENUMERATED {noMimo(0), antenna2pCPICH(1), antenna2sCPICH(2), spare(3)} |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PrimarySCHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { tstdIndicator BOOLEAN, dl_TxPower DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondarySCHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { tstdIndicator BOOLEAN, dl_TxPower DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PrimaryCCPCHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { sttd_Indicator BOOLEAN, dl_TxPower DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryCCPCHInfo |
| Comment | The range for powerOffsetOfTFCI_PO1 and powerOffsetOfPILOT_PO3 is 0 dB to 6 dB, 0.25 dB per step. mbms_softCombTimingOffset is present when L1-combination applies When configuring SCCPCH for MBMS "positionFixedOrFlexible" shall be set to Flexible, and sCCPCHSlotFormat shall be chosen to the one without pilot bits |
| Type Definition |  |
| SEQUENCE { scramblingCode INTEGER(0..15), dl_ChannelizationCode SF256_AndCodeNumber, sCCPCHSlotFormat SCCPCHSlotFormat, timingOffset INTEGER (0..149), positionFixedOrFlexible PositionFixedOrFlexible, sttd_Indicator BOOLEAN, dl_TxPower DL_TxPower, powerOffsetOfTFCI_PO1 INTEGER (0..24), powerOffsetOfPILOT_PO3 INTEGER (0..24), mbms_softCombTimingOffset MBMS_SoftComb_TimingOffset OPTIONAL –-Rel-6 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PRACHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { fdd_tdd CHOICE { fdd SEQUENCE { preambleSignature AvailableSignatures, spreadingFactorForDataPart SF_PRACH, preambleScramblingCode PreambleScramblingCodeWordNumber, puncturingLimit PuncturingLimit, accessSlot AvailableSubChannelNumbers }, tdd SEQUENCE { -- TDD not applicable }, fdd_prachForEnhUL SEQUENCE { -- Rel-8 or later -- For rel-99 PRACH preambleSignature AvailableSignatures, spreadingFactorForDataPart SF_PRACH, preambleScramblingCode PreambleScramblingCodeWordNumber, puncturingLimit PuncturingLimit, accessSlot AvailableSubChannelNumbers, -- For Enhanced Uplink E-DCH transmission commonEDCH_PreambleSignature AvailableSignatures, -- access preamble signature for E-DCH transmission commonEDCH_PreambleScramblingCode PreambleScramblingCodeWordNumber, commonEDCH_AccessSlot AvailableSubChannelNumbers } } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PICHInfo |
| Comment | When the PICH is associated to the HS-SCCH, sccpchId_associated is set to 32 (Rel-7 or later). |
| Type Definition |  |
| SEQUENCE { pichinfo PICH_Info, dl_TxPower PICH_PowerOffset, sccpchId_associated INTEGER (0..32) -- Value 32 used in Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AICHInfo |
| Comment | When e_AI_Info is present: - upon receipt of the access preamble signature corresponding to a PRACH message transmission i.e. preambleSignature, the Acquisition Indicators is transmitted on AICH - upon receipt of the access preamble signature corresponding to an E-DCH transmission i.e. commonEDCH_PreambleSignature, the Extended Acquisition Indicators is transmitted on AICH. |
| Type Definition |  |
| SEQUENCE { aichinfo AICH_Info, dl_TxPower AICH_PowerOffset, e_AI_Info E_AI_InfoType OPTIONAL -- Rel-8 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DPCHInfo |
| Comment | At least one of the fields shall be present. |
| Type Definition |  |
| SEQUENCE { ul_DPCHInfo UL_DPCH_Info OPTIONAL, dl_DPCHInfo DL_DPCHInfo OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation, dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_DPCCHInfo |
| Comment | mimoStatus represents status of MIMO and shall be set only if mimo_Parameters are provided in HS-PDSCH configuration |
| Type Definition |  |
| SEQUENCE { cqi_RepetitionFactor CQI_RepetitionFactor, ackNackRepetitionFactor ACK_NACK_repetitionFactor, mimoStatus BOOLEAN DEFAULT FALSE -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_DPCCHInfo_r8 |
| Comment | mimoStatus represents status of MIMO and shall be set only if mimo_Parameters are provided in HS-PDSCH configuration. secondaryServing_HS_DSCH_Cell indicates if the HS-DSCH secondary serving cell is activated (see 3GPP TS 25.214 [12], clause 6A.1) |
| Type Definition |  |
| SEQUENCE { cqi_RepetitionFactor CQI_RepetitionFactor, ackNackRepetitionFactor ACK_NACK_repetitionFactor, mimoStatus BOOLEAN DEFAULT FALSE, secondaryServing_HS_DSCH_Cell ENUMERATED { deactivated (0), activated (1) } DEFAULT deactivated -- Rel-8 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r5 |
| Comment | Applicable Rel-5 or later |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r5, dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r5, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_TxPower_PCPICH |
| Comment | Absolute Tx Power of PCPICH |
| Type Definition |  |
| INTEGER (-60..-30) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_TxPower |
| Comment | Downlink Tx Power relative to PCPICH |
| Type Definition |  |
| INTEGER (-35..+15) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SCCPCHSlotFormat |
| Comment | Reference to 3GPP TS25.211 [40] |
| Type Definition |  |
| INTEGER (0..17) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PDSCHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { fdd_tdd CHOICE { fdd SEQUENCE { pdsch_CodeMapping PDSCH_CodeMapping }, tdd SEQUENCE { -- TDD not applicable }, }, dl_TxPower DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DPCHInfo_r5OrLater |
| Comment | Applicable Rel-5 or later At least one of the first two fields ul_DPCHInfo or dl_DPCHInfo shall be present. Presence of hs_DPCCHInd means that the HS-DPCCH shall be configured in the uplink DPCH. If hs_DPCCHInd is absent no HS-DPCCH shall be configured in the uplink DPCH, or the configured HS-DPCCH shall be removed in the modify ASP. In the active set which has radio links from more than one cell the HS-DPCCH is configured only in the HS-DSCH serving cell. Three combinations are valid: ul_DPCH_Info only, dl_DPCHInfo only and ul_DPCH_Info + hs_DPCCHInd. Presence of ss_UL_DPCCH_DRX_Info if UL DPCH being configured and presence of ss_DRX_Info if DL F-DPCH being configured indicates that UL_DTX is enabled. ss_DRX_Info is provided as F-DPCH TPC commands need not be transmitted during gaps due to UL-DPCCH DTX (3GPP TS 25.214 [12], subclause 5.1.2.2.1). ul_DPCH_TimingOffset is the timing offset relative to P-CCPCH, it is provided in Rel-7 only when Enhanced Cell FACH Downlink (common_HS_PDSCH_Info) is configured. |
| Type Definition |  |
| CHOICE { r5 SEQUENCE { ul_DPCHInfo UL_DPCH_Info_r5 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r5 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo OPTIONAL }, r6 SEQUENCE { ul_DPCHInfo UL_DPCH_Info_r6 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r6 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo OPTIONAL }, r7 SEQUENCE{ ul_DPCHInfo UL_DPCH_Info_r7 OPTIONAL, ul_DPCH_TimingOffset INTEGER (0..38399) OPTIONAL, -- timing offset relative to P-CCPCH -- applicable to Rel-7 when Enhanced Cell FACH DL is configured dl_DPCHInfo DL_DPCHInfo_r7 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo OPTIONAL, ss_UL_DPCCH_DRX_Info SS_UL_DPCCH_DRX_Info OPTIONAL, ss_DRX_Info DTX_Info OPTIONAL }, r8 SEQUENCE{ ul_DPCHInfo UL_DPCH_Info_r7 OPTIONAL, dl_DPCHInfo DL_DPCHInfo_r8 OPTIONAL, hs_DPCCHInd HS_DPCCHInfo_r8 OPTIONAL, ss_UL_DPCCH_DRX_Info SS_UL_DPCCH_DRX_Info OPTIONAL, ss_DRX_Info DTX_Info OPTIONAL }, relAspTypeExtension DPCHInfo_r10OrLaterExtensionType -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r6 |
| Comment | Applicable Rel-6 For SS reconfiguration from DL-DPCH to F-DPCH and vice versa the dl_CommonInformation shall include the defaultDPCH_OffsetValue as additional information for SS even when this is not included in the corresponding RRC message sent to the UE and the timing is maintained. |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r6, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r6, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r6 }, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r7 |
| Comment | Applicable Rel-7 or later |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r7, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r7, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r7 }, rl_Information RL_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r8 |
| Comment | Applicable Rel-8 or later |
| Type Definition |  |
| CHOICE { r8 SEQUENCE { dl_CommonInformation DL_CommonInformation_r8, dl_Dpch_InfoPerRL CHOICE { dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r7, dl_FDPCH_InfoPerRL DL_FDPCH_InfoPerRL_r7 }, rl_Information RL_Information }, spare1 NULL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_DPCCH_DRX_Info |
| Comment |  |
| Type Definition |  |
| SEQUENCE { ss_DRX_Info DTX_Info OPTIONAL, dtx_DRX_timingInfo DTX_DRX_TimingInfo_r7 OPTIONAL, uplink_DPCCHSlotFormatInformation Uplink_DPCCH_Slot_Format_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r5OrLater |
| Comment | Applicable Rel-5 or later When CHY_RL_Setup_REQ is called with CHOICE of hS_PDSCHInfo HS_PDSCH and HS-SCCH shall be configured in SS. The following HS-DSCH related parameters are passed to the SS implicitly by HSDSCH_physical_layer_category: - Maximum number of HS-DSCH codes can be received by UE. - Minimum inter-TTI interval. - Maximum number of bits of an HS-DSCH transport block within an HS-DSCH TTI. - Total number of soft channel bits". HSDSCH_physical_Layer_category is also used for interpretation of the meaning of CQI value. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. |
| Type Definition |  |
| CHOICE { r5 SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, r6 SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r6, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, r7 HS_PDSCHInfo_r7, r8 HS_PDSCHInfo_r8, r9 HS_PDSCHInfo_r9, relAspTypeExtension HS_PDSCHInfo_r10OrLaterExtensionType -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r7 |
| Comment | Rel-7 or later. Choice of hS_PDSCH_Info will be used for configuring HSD-DSCH with 64QAM and/or CPC. Choice of common_HS_PDSCH_Info will be used for Enhanced Cell_FACH. commonOrDedicated_H_RNTI indicates, the H-RNTI that will be used by UE (either dedicated H-RNTI if provided in RRC message, or selected common H-RNTI transmitted in SIB5). The transmission of BCCH on HS-DSCH is performed by using BCCH specific H-RNTI on the first indexed HS-SCCH code indicated in system information broadcast. Presence of ss_DTX_Info makes DL DRX to be enabled. Presence of hs_scch_LessInfo makes HS-SCCH less operation enabled. Presence of mimo_Parameters indicate MIMO is to be started. MIMO and HS-SCCH less operation do not co-exist. MIMO is not applicable for non DCH states. The two IE "hSDSCHPhysicalLayerCategory" and "hsdsch_physical_layer_category_ext" are mutually exclusive. One of the IE shall be present in the SS configuration. The IE "hsdsch_physical_layer_category_ext" is present when MAC-ehs is configured. If commonOrDedicated_H_RNTI is omitted, HS-SCCHless is to be applied. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. In common_HS_PDSCH_Info, hs_DPCCHToFollow is set to FALSE unless HS-DPCCH is required in specific configuration. |
| Type Definition |  |
| CHOICE { hS_PDSCH_Info SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r7, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH ss_DTX_Info DRX_Info OPTIONAL, hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimo_Parameters MIMO_Parameters_r7 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, hs_SCCH_SttdIndicator BOOLEAN DEFAULT FALSE }, common_HS_PDSCH_Info SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo HS_SCCH_SystemInfo, hs_dsch_PagingSystemInformation HS_DSCH_PagingSystemInformation OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, spare2 SEQUENCE {}, spare3 SEQUENCE {} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r8 |
| Comment | Rel-8 or later. Choice of hS_PDSCH_Info will be used for configuring HSD-DSCH with 64QAM and/or CPC. Choice of common_HS_PDSCH_Info will be used for Enhanced Cell_FACH. commonOrDedicated_H_RNTI indicates, the H-RNTI that will be used by UE (either dedicated H-RNTI if provided in RRC message, or selected common H-RNTI transmitted in SIB5). The transmission of BCCH on HS-DSCH is performed by using BCCH specific H-RNTI on the first indexed HS-SCCH code indicated in system information broadcast. Presence of ss_DTX_Info makes DL DRX to be enabled. Presence of hs_scch_LessInfo makes HS-SCCH less operation enabled. Presence of mimo_Parameters indicate MIMO is to be started. MIMO and HS-SCCH less operation do not co-exist. MIMO is not applicable for non DCH states. The three IE "hSDSCHPhysicalLayerCategory", "hsdsch_physical_layer_category_ext" and "hsdsch_physical_layer_category_ext2" are mutually exclusive. One of the IE shall be present in the SS configuration. The IE "hsdsch_physical_layer_category_ext2" is present when DC-HSDPA is configured. If commonOrDedicated_H_RNTI is omitted, HS-SCCHless is to be applied. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. Presence of ss_HS_DSCH_DtxCellFach makes HS-DSCH DRX operation enabled and SS shall transmit any requested HS-SCCH and HS-DSCH only in occasions when UE will be listening. |
| Type Definition |  |
| CHOICE { hS_PDSCH_Info SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, hsdsch_physical_layer_category_ext2 HSDSCH_physical_layer_category_ext2 OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r8, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH ss_DTX_Info DRX_Info OPTIONAL, hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimo_Parameters MIMO_Parameters_r8 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, hs_SCCH_SttdIndicator BOOLEAN DEFAULT FALSE }, common_HS_PDSCH_Info SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo HS_SCCH_SystemInfo, hs_dsch_PagingSystemInformation HS_DSCH_PagingSystemInformation OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, ss_HS_DSCH_DtxCellFach HS_DSCH_DrxCellfach_info OPTIONAL }, spare2 SEQUENCE {}, spare3 SEQUENCE {} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r9 |
| Comment | Rel-9 or later. Choice of hS_PDSCH_Info will be used for configuring HSD-DSCH with 64QAM and/or CPC. Choice of common_HS_PDSCH_Info will be used for Enhanced Cell_FACH. commonOrDedicated_H_RNTI indicates, the H-RNTI that will be used by UE (either dedicated H-RNTI if provided in RRC message, or selected common H-RNTI transmitted in SIB5). The transmission of BCCH on HS-DSCH is performed by using BCCH specific H-RNTI on the first indexed HS-SCCH code indicated in system information broadcast. Presence of ss_DTX_Info makes DL DRX to be enabled. Presence of hs_scch_LessInfo makes HS-SCCH less operation enabled. Presence of mimo_Parameters indicate MIMO is to be started. MIMO and HS-SCCH less operation do not co-exist. MIMO is not applicable for non DCH states. The IEs "hSDSCHPhysicalLayerCategory", "hsdsch_physical_layer_category_ext", "hsdsch_physical_layer_category_ext2" and "hsdsch_physical_layer_category_ext3" are mutually exclusive. One of the IE shall be present in the SS configuration. The IE "hsdsch_physical_layer_category_ext2" is present when DC-HSDPA is configured. The IE "hsdsch_physical_layer_category_ext3" is present when DC-HSDPA with MIMO is configured. If commonOrDedicated_H_RNTI is omitted, HS-SCCHless is to be applied. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. Presence of ss_HS_DSCH_DtxCellFach makes HS-DSCH DRX operation enabled and SS shall transmit any requested HS-SCCH and HS-DSCH only in occasions when UE will be listening. |
| Type Definition |  |
| CHOICE { hS_PDSCH_Info SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, hsdsch_physical_layer_category_ext2 HSDSCH_physical_layer_category_ext2 OPTIONAL, hsdsch_physical_layer_category_ext3 HSDSCH_physical_layer_category_ext3 OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r9, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH ss_DTX_Info DRX_Info OPTIONAL, hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimo_Parameters MIMO_Parameters_r9 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, hs_SCCH_SttdIndicator BOOLEAN DEFAULT FALSE }, common_HS_PDSCH_Info SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo HS_SCCH_SystemInfo, hs_dsch_PagingSystemInformation HS_DSCH_PagingSystemInformation OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, ss_HS_DSCH_DtxCellFach HS_DSCH_DrxCellfach_info OPTIONAL }, spare2 SEQUENCE {}, spare3 SEQUENCE {} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_DPCH_Info_r6OrLater |
| Comment | Rel-6 or later. If there is an UL-DPCH configured in the cell the E-DPCH shall use the same scramblingCodeType and scramblingCode as the UL-DPCH. Presence of ul_16QAM_Settings enables usage of 16QAM in UL. With 16QAM configured the value of "E-TFCI table index" is increased by 2, and indices in the SG-Table refer to Scheduling Grant Table 2 in 3GPP TS 25.321 [17]. The IEs "edch_PhysicalLayerCategory" , "edch_PhysicalLayerCategory_extension" and "edch_PhysicalLayerCategory_extension2" are mutually exclusive. One of the IE shall be present in the SS configuration. The IE "edch_PhysicalLayerCategory_extension" is present only for UE categories greater than or equal to 7. "edch_PhysicalLayerCategory_extension2" is present when Dual Cell E-DCH operation is configured. |
| Type Definition |  |
| CHOICE { r6 SEQUENCE { e_DPCCH_Info E_DPCCH_Info, e_DPDCH_Info E_DPDCH_Info, scramblingCodeType ScramblingCodeType OPTIONAL, scramblingCode UL_ScramblingCode OPTIONAL, tti E_DCH_TTI OPTIONAL, edch_PhysicalLayerCategory INTEGER (1..16) }, r7 SEQUENCE { e_DPCCH_Info E_DPCCH_Info_r7, e_DPDCH_Info E_DPDCH_Info_r7, scramblingCodeType ScramblingCodeType OPTIONAL, scramblingCode UL_ScramblingCode OPTIONAL, tti E_DCH_TTI OPTIONAL, edch_PhysicalLayerCategory INTEGER (1..16) OPTIONAL, edch_PhysicalLayerCategory_extension INTEGER (7) OPTIONAL, ul_16QAM_Settings UL_16QAM_Settings OPTIONAL }, r8 SEQUENCE { e_DPCCH_Info E_DPCCH_Info_r7, e_DPDCH_Info E_DPDCH_Info_r8, scramblingCodeType ScramblingCodeType OPTIONAL, scramblingCode UL_ScramblingCode OPTIONAL, tti E_DCH_TTI OPTIONAL, edch_PhysicalLayerCategory INTEGER (1..16) OPTIONAL, edch_PhysicalLayerCategory_extension INTEGER (7) OPTIONAL, ul_16QAM_Settings UL_16QAM_Settings OPTIONAL, s_offset INTEGER (0..9) OPTIONAL -– Used for enhFACH UL }, r9 SEQUENCE { e_DPCCH_Info E_DPCCH_Info_r7, e_DPDCH_Info E_DPDCH_Info_r8, scramblingCodeType ScramblingCodeType OPTIONAL, scramblingCode UL_ScramblingCode OPTIONAL, tti E_DCH_TTI OPTIONAL, edch_PhysicalLayerCategory INTEGER (1..16) OPTIONAL, edch_PhysicalLayerCategory_extension INTEGER (7) OPTIONAL, edch_PhysicalLayerCategory_extension2 INTEGER (8..9) OPTIONAL, ul_16QAM_Settings UL_16QAM_Settings OPTIONAL, s_offset INTEGER (0..9) OPTIONAL -– Used for enhFACH UL }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_AGCH_Info |
| Comment | Rel-6 or later. Presence of ss_DTX_Info indicates DL_DRX is enabled and SS shall transmit any requested AGCH only in occasions when UE will be listening. |
| Type Definition |  |
| CHOICE { r6 SEQUENCE { e_AGCHInfo E_AGCH_Information, tti E_DCH_TTI, e_AGCH_PowerOffset INTEGER (0..255) -- Range and Step are acc to 25.433, 9.2.2.13Id -- Offset relative to P-CPICH, e_RNTI_Primary E_RNTI OPTIONAL, e_RNTI_Secondary E_RNTI OPTIONAL, ss_DTX_Info DRX_Info OPTIONAL, -- Rel-7 or later sttdIndicator BOOLEAN DEFAULT FALSE }, r8 SEQUENCE { -- Rel-8 or later e_AGCHInfo E_AGCH_Information_r8, tti E_DCH_TTI, e_AGCH_PowerOffset INTEGER (0..255), -- Range and Step are acc to 25.433, 9.2.2.13Id -- Offset relative to P-CPICH e_RNTI_Primary E_RNTI OPTIONAL, e_RNTI_Secondary E_RNTI OPTIONAL, ss_DTX_Info DRX_Info OPTIONAL, initialServingGrantValue INTEGER (0..37) OPTIONAL, -- Used for enhFACH UL sttdIndicator BOOLEAN DEFAULT FALSE }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_HICH_Info |
| Comment | Rel-6 or later. Only one of e_HICHInfo and e_HICHInfoCommonEDCH can be present. |
| Type Definition |  |
| SEQUENCE { e_HICHInfo E_HICH_Information OPTIONAL, e_HICHInfoCommonEDCH E_HICH_Information_CommonEdch OPTIONAL, -- Rel-8 or later tti E_DCH_TTI, e_HICH_PowerOffset INTEGER (0..255), -- PowerOffset = -32 + offset * 0.25 -- Unit dB, Range -32dB .. +31.75dB, Step +0.25dB -- (25.433, 9.2.2.13If), offset relative to P-CPICH sttdIndicator BOOLEAN DEFAULT FALSE, relAspTypeExtension SS_E_HICH_Info_r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_RGCH_Info |
| Comment | Rel-6 or later. Presence of ss_DTX_Info indicates DL_DRX is enabled and SS shall transmit any requested RGCH only in occasions when UE is going to listen. Only one of e_RGCHInfo and e_RGCHInfoCommonEDCH can be present. |
| Type Definition |  |
| SEQUENCE { e_RGCHInfo E_RGCH_Information OPTIONAL, e_RGCHInfoCommonEDCH E_RGCH_Information_CommonEdch OPTIONAL, -- Rel-8 or later tti E_DCH_TTI, e_RGCH_PowerOffset INTEGER (0..255), -- Range Range:-32 .. +31.75 dB, Step: 0.25 dB -- Offset relative to P-CPICH ss_DTX_Info DRX_Info OPTIONAL, -- Rel-7 or later sttdIndicator BOOLEAN DEFAULT FALSE, relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RL_Information |
| Comment | The range for powerOffsetOfTPC_PO2 and powerOffsetOfTFCI_PO1 and powerOffsetOfPILOT_PO3 is 0 dB to 6 dB, 0,25 dB per step. The IE cfnTgtSfnFrameOffset is applied when adding another RL or moving the UE to another cell in DCH state in the timing-maintained hard handover or in the softhandover. The cfnTgtSfnFrameOffset is defined as being the time difference between the CFN and the SFN of the cell in which the RL is to be added. The use of cfnTgtSfnFrameOffset and DOFF (Default DPCH Offset Value) is mutually exclusive. The IE cfnTgtSfnFrameOffset is omitted when configuring the 1st RL, or configuring the RL in the timing re-initialized hard handover where the required synchronization information is provided in defaultDPCH_offsetValue in DL_CommonInformation. In addition, TmTgt value is provided to dpch_FrameOffset in DL_DPCH_InfoPerRL_r5 or DL_DPCH_InfoPerRL_r6 as CFNchipOffset_Tgt. TmTgt can be observed by the UE, or calculated by the TTCN. If it is calculated, TmTgt = (DOFF*512 + 38400 + TCell_Ref - TCell_Tgt) Mod 38400. dl_FDPCH_ShoConfig set to TRUE, indicates that F-DPCH is configured in softHO. |
| Type Definition |  |
| SEQUENCE { powerOffsetOfTFCI_PO1 INTEGER (0..24), powerOffsetOfTPC_PO2 INTEGER (0..24), powerOffsetOfPILOT_PO3 INTEGER (0..24), dl_TxPower DL_TxPower, dl_TxPowerMax DL_TxPower, dl_TxPowerMin DL_TxPower, cfnTgtSfnFrameOffset CfnTgtSfnFrameOffset OPTIONAL, dl_FDPCH_ShoConfig BOOLEAN DEFAULT FALSE –- Rel-6 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CfnTgtSfnFrameOffset |
| Comment | 1. The observedValue is provided if the UE reads SFN when measuring "Cell synchronization information" and sends it to the SS in MEASUREMENT REPORT message. The OFF value in IE COUNT-C-SFN frame difference is applied to the observedValue. 2. If the synchronization between the target cell and the reference cell is calculated the calculatedAbsoluteValue is applied. Depending upon how to interpret SfnOffset two valid calculations exist. 2.1 If SfnOffset is interpreted as the frames lagging behind: calculated_sfnLagging = (((4096 * 38400) + SfnOffset_Ref * 38400 +TCell_Ref + DOFF*512 - (SfnOffset_Tgt * 38400 + TCell_Tgt)) mod (256 * 38400)) / 38400 2.2 If SfnOffset is interpreted as the frames in advance: calculated_sfnInAdvance = (((4096 + SfnOffset_Tgt - SfnOffset_Ref) * 38400 +TCell_Ref + DOFF*512 - TCell_Tgt) mod (256 * 38400)) / 38400 The formula can be further simplified in the default condition if SfnOffset_Ref and TCell_Ref are equal to 0: calculated_sfnLagging = (((4096 * 38400) + DOFF*512 - (SfnOffset_Tgt * 38400 + TCell_Tgt)) mod (256 * 38400)) / 38400 calculated_sfnInAdvance = ((DOFF*512 + SfnOffset_Tgt * 38400 - TCell_Tgt) Mod (256 * 38400)) / 38400 calculated_sfnInAdvance = (calculated_sfnLagging + 2* SfnOffset_Tgt) mod 256 calculated_sfnLagging = (calculated_sfnInAdvance + (4096 - SfnOffset_Tgt) * 2) mod 256 The TTCN provides calculated_sfnLagging. |
| Type Definition |  |
| SEQUENCE { referenceCellId INTEGER(0..63), cfnFrameOffset CHOICE { observed INTEGER (0..255), calculated INTEGER (0..255) } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MBMS_MICHConfigurationInfo_r6 |
| Comment | Rel-6 or later. |
| Type Definition |  |
| SEQUENCE { michinfo MBMS_MICHConfigurationInfo_r6, sccpchId_associated INTEGER (0..31) } |  |

| Type Name | E_AI_InfoType |
| --- | --- |
| Comment | defaultE_DCH_ResourceIndex is the index (range 0 to 31) in SIB5/5bis of the E-DCH resource to be used by SS when providing the resource configuration index to UE in the E-AI indication. defaultE_DCH_ResourceIndex = -1 indicates that any E-DCH resource index is used as appropriate and as indicated by UE in the access preamble. The totalNumberE_DCH is the total number of E-DCH resources configured in SIB5/5bis. |
| Type Definition |  |
| SEQUENCE { defaultE_DCH_ResourceIndex INTEGER (-1..31), totalNumberE_DCH INTEGER (1..32) } |  |

##### 7.3.2.2.12	CPHY_Sync

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Sync_IND |
| PCO Type | CSAP |
| Comment | To indicate that physical channel synchronization (in FDD mode, sync with DPCCH) has been achieved. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.12a	CPHY_HS_DPCCH_AckNack (Rel-5 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_AckNack_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To Confirm CPHY_HS_DPCCH_AckNack_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_AckNack_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To request for start or stop reporting Ack/Nack received on the HS-DPCCH for the HARQ process hARQProcessId. Harq Process ID 0..15 represent the respective process Id with HS-SCCH type 1 operation. Value 31 represents all HARQ process IDs. E.g. HS-SCCH less/HS-SCCH Type 2 operation uses value 31. At the initialization the SS is at the "sTOPRep" state without reporting any Ack/Nack |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, ackNackReportReq AckNackReportReq, hARQProcessId INTEGER(0..15¦31) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AckNackReportReq |
| Comment | Applicable Rel-5 or later startRep: The SS shall start reporting the HARQ-ACK information received on HS-DPCCH by primitive CPHY_HS_DPCCH_AckNack_IND on CPHY PCO. stopRep: The SS shall stop reporting. |
| Type Definition |  |
| ENUMERATED { startRep (0), stopRep (1) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_AckNack_IND |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later SS reportes the HARQ-ACK information received on HS_DPCCH, each received Ack/Nack generates a CPHY_HS_DPCCH_AckNack_IND Harq Process ID 0..15 represent the process Id with HS-SCCH type 1 operation. Value 31 represents all HARQ process IDs. E.g. HS-SCCH less/HS-SCCH Type 2 operation uses value 31. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, hARQ_ACKInfo ENUMERATED {ack(0), nack (1)}, hARQProcessId INTEGER(0..15¦31) } |  |

##### 7.3.2.2.12b	CPHY_HS_DPCCH_CQI (Rel-5 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To Confirm CPHY_HS_DPCCH_CQI_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To enable the SS to start reporting N times of the CQI value received on the HS-DPCCH. After N times the SS stops reporting. N is specified in numberOfReports. At the SS initialization reporting of CQI values is disabled |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, numberOfReports INTEGER(1..32) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_IND |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later SS generates the indication when a CQI value is received on HS_DPCCH after invocation of ASP CPHY_HS_DPCCH_CQI_REQ and before the numberOfReports is reached. This ASP is used for verifying whether the UE has configured the HS-DSCH and starts reception of HS-DSCH (3GPP TS 25.331 [21], subclause 8.6.6.34). ‘second_cqi’ shall be reported when MIMO is configured and dual transport block type A CQI report is received. When second_cqi is present, allowed values for ‘cqi’ are 0..14. In all other cases (i.e. non MIMO operation, MIMO operation but with CQI type B or single transport block type A CQI reports] ‘second_cqi’ shall not be reported and range for cqi is 0..30. ‘pci’ and cqiType shall be present when MIMO is configured. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, cfn INTEGER (0..255), -- Rel-7 or later subframe INTEGER (0..4), -- Rel-7 or later cqi INTEGER (0..30), cqiExt CQI_ExtensionType } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CQI_ExtensionType |
| Comment | Rel-7 or later The value -1 for second_cqi indicates the absence of the second CQI. |
| Type Definition |  |
| SEQUENCE { second_cqi INTEGER (-1..14) OPTIONAL, pci INTEGER (0..3) OPTIONAL, -- MIMO cqiType ENUMERATED {cqiTypeA(0), cqiTypeB(1)} OPTIONAL -- MIMO } |  |

##### 7.3.2.2.12b1	CPHY_HS_DPCCH_CQI_DC (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_DC_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later To Confirm CPHY_HS_DPCCH_CQI_DC_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_DC_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later To enable the SS to start reporting N times of the CQI value received on the HS-DPCCH on cellId when DC-HSDPA is configured. After N times the SS stops reporting. N is specified in numberOfReports. At the SS initialization reporting of CQI values is disabled. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, numberOfReports INTEGER(1..32) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DPCCH_CQI_DC_IND |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later SS generates the indication when a CQI value is received on HS-DPCCH after invocation of ASP CPHY_HS_DPCCH_CQI_REQ and before the numberOfReports is reached. This ASP is used for verifying whether the UE has configured the HS-DSCH and starts reception of HS-DSCH (3GPP TS 25.331 [21], subclause 8.6.6.34 and 3GPP TS 25.212 [58], subclause 4.7.3.A2). |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, cfn INTEGER (0..255), subframe INTEGER (0..4), cqi_dCHSDPA INTEGER (0..1023) } |  |

##### 7.3.2.2.12c	CPHY_HS_DSCH_CRC_Mode (Rel-5 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DSCH_CRC_Mode_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later Confirm a previous CPHY_HS_DSCH_CRC_Mode_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_DSCH_CRC_Mode_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To set the CRC calculation mode for HS-DSCH. If mode = normal, the SS generates the correct CRC. If mode = erroneous, the SS always generates any wrong CRC value which is different from the correct one on the specified MACdFlow/mac-ehs Queue. If mode = error1AndNormal, the SS generates wrong CRC for first transmission and correct CRC on first retransmission. Later SS operates in normal mode. If mode = error2AndNormal, the SS generates wrong CRC for first transmission, first retransmission and correct CRC second retransmission. The corrupted bits in first error transmission and second error transmission are different, so that when UE combines the data from three transmission, shall result in correct data. Later SS operates in normal mode. As default, the normal mode is applied. When the HS-DSCH first configured or reconfigured the SS enters the normal CRC calculation mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) , routingInfo RoutingInfo, flow_Queue_Id Flow_Queue_ID, mode ENUMERATED {normal(0), erroneous(1), error1AndNormal(2), error2AndNormal(3)} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Flow_Queue_ID |
| Comment | Choice MAC_d_Flow is used when MAC-hs is configured and MAC_ehs_Queue is used when MAC-ehs is configured. |
| Type Definition |  |
| CHOICE { mac_dFlowId MAC_d_FlowIdentity, mac_ehs_QueueId NULL } |  |

##### 7.3.2.2.13	CPHY_TrCH_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TrCH_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm to configure the transport channel |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TrCH_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to configure the transport channel |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, trchConfigType TrchConfigType, configMessage CphyTrchConfigReq } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyTrchConfigReq |
| Comment | To request to configure the transport channel. The same TFCS information should be provided to the PHY and MAC layers at all times. When a CPHY_TrCH_Config_REQ is used to configure the PHY layer, a corresponding CMAC_Config_REQ should be sent to the MAC layer to ensure that the configuration is consistent. HS-DSCH MAC-d/EHS-DSCH/EHS-DSCH-Common flows or E-DCH MAC-d flows shall be separately configured whilst the ulconnectedTrCHList, ulTFCS, dlconnectedTrCHList and dlTFCS are omitted. For configuring MBMS if the default TFCS used in the RRC message the dLTFCS shall use the "complete" CHOICE in "ExplicitTFCS-Configuration " IE and the TFCS configuration shall be in line with clause 14.10.1.1/14.10.1.2 of 3GPP TS 25.331 [21]. ehs_DSCH_Flow or ehs_DSCH_Flows_r9 is used when MAC-ehs is to be configured in Cell_DCH state. ehs_DSCH_CommonFlows shall be used for configuring MAC_ehs layer in common connected mode states. Only one of hsDSCHMacdFlows, ehs_DSCH_Flows, ehs_DSCH_CommonFlows, ehs_DSCH_Flows_r9, can be present. e_DCHMacd_CommonFlows shall be used for configuring MACi/is in common connected mode states. |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, ulconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, ul_TransportChannelType SS_UL_TransportChannelType, transportChannelInfo CommonOrDedicatedTFS } OPTIONAL, ulTFCS TFCS OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, dl_TransportChannelType SS_DL_TransportChannelType, transportChannelInfo CommonOrDedicatedTFS } OPTIONAL, dlTFCS TFCS OPTIONAL, hsDSCHMacdFlows HS_DSCHMACdFlows OPTIONAL, -- Rel-5 or later e_DCHMacdFlows E_DCHMACdFlows OPTIONAL, -- Rel-6 or later ehs_DSCH_Flows EHS_DSCH_Flows OPTIONAL, -- Rel-7 or later ehs_DSCH_CommonFlows EHS_DSCH_CommonFlows OPTIONAL, -- Rel-7 or later e_DCHMacd_CommonFlows CommonE_DCHMACdFlows OPTIONAL, -- Rel-8 or later ehs_DSCH_Flows_r9 EHS_DSCH_Flows_r9 OPTIONAL, -- Rel-9 or later relAspTypeExtension TrCHInfo_r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RoutingInfo |
| Comment | To route between each channels. |
| Type Definition |  |
| CHOICE physicalChannelIdentity INTEGER {0..31}, transportChannelIdentity TransportChannelIdentity, logicalChannelIdentity LogicalChannelIdentity, rB_Identity INTEGER {-31..32}, cn_DomainIdentity CN_DomainIdentity } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RatType |
| Comment | To select route between each channels. |
| Type Definition |  |
| ENUMERATED { fdd (0), tdd128 (1) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTFS |
| Comment | Transport Format Set |
| Type Definition |  |
| SEQUENCE { tti CHOICE { tti10 CommonOrDedicatedTF_InfoList, tti20 CommonOrDedicatedTF_InfoList, tti40 CommonOrDedicatedTF_InfoList, tti80 CommonOrDedicatedTF_InfoList, dynamic CommonOrDedicatedTF_InfoList_DynamicTTI }, semistaticTF_Information SemistaticTF_Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTF_InfoList |
| Comment | Transport Format Set |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxTF)) OF CommonOrDedicatedTF_Info |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTF_Info |
| Comment | Transport Format Set |
| Type Definition |  |
| SEQUENCE { tb_Size INTEGER (0..5035), numberOfTbSizeList SEQUENCE (SIZE (1..maxTF)) OF NumberOfTransportBlocks, logicalChannelList SS_LogicalChannelList } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTF_InfoList_DynamicTTI |
| Comment | Transport Format Set for TDD mode |
| Type Definition |  |
| SEQUENCE { tb_Size INTEGER (0..5035), numberOfTbSizeList SEQUENCE (SIZE (1..maxTF)) OF NumberOfTransportBlocks, logicalChannelList SS_LogicalChannelList } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrchConfigType |
| Comment |  |
| Type Definition |  |
| CHOICE { nonDch NULL, dch ENUMERATED {normal(0), softHO(1)} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_DSCHMACdFlows |
| Comment | Applicable Rel-5 or later Within the ACK/NACK repetition period indicated by ackNackRepetitionFactor the SS shall not transmit MAC-hs PDU's on HS-PDSCH. Only one among harqInfo and harqInfo_r7 shall be present. Harq_Info_r7 shall be used for Rel-7 or later. mimoStatus represents status of MIMO, and shall be set only if mimo_Parameters are provided in HS-PDSCH configuration |
| Type Definition |  |
| SEQUENCE { harqInfo HARQ_Info OPTIONAL, addOrReconfMACdFlow SS_AddOrReconfMAC_dFlow OPTIONAL, ackNackRepetitionFactor ACK_NACK_repetitionFactor OPTIONAL, harqInfo_r7 HARQ_Info_r7 OPTIONAL -- to be used for Rel-7 or later. } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | EHS_DSCH_Flows |
| Comment | Applicable Rel-7 or later Within the ACK/NACK repetition period indicated by ackNackRepetitionFactor the SS shall not transmit MAC-hs PDU's on HS-PDSCH. mimoStatus represents status of MIMO, and shall be set only if mimo_Parameters are provided in HS-PDSCH configuration |
| Type Definition |  |
| SEQUENCE { harqInfo_r7 HARQ_Info_r7 OPTIONAL, addOrReconfMAC_ehs_ReordQ SS_MAC_ehs_AddReconfReordQueueList OPTIONAL, ackNackRepetitionFactor ACK_NACK_repetitionFactor OPTIONAL, mimoStatus BOOLEAN OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_ehs_AddReconfReordQueueList |
| Comment | Applicable Rel-7 or later |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxQueueIDs)) OF SS_MAC_ehs_AddReconfReordQ |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_ehs_AddReconfReordQ |
| Comment | Applicable Rel-7 or later The priority of PriorityQueue shall set according to the priority of logical channels which is mapped on to this priority queue. NOTE: The range of priority of PriorityQueue is from 0 to 7 and 0 is the lowest priority. |
| Type Definition |  |
| SEQUENCE { mAC_ehs_AddReconfReordQ MAC_ehs_AddReconfReordQ, priority INTEGER(0..7) } |  |

| Type Name | EHS_DSCH_Flows_r9 |
| --- | --- |
| Comment | Applicable Rel-9 or later Within the ACK/NACK repetition period indicated by ackNackRepetitionFactor the SS shall not transmit MAC-hs PDU's on HS-PDSCH. mimoStatus represents status of MIMO, and is set only if mimo_Parameters are provided in HS-PDSCH configuration. In dedicated MAC configuration mimostatus is not applicable. |
| Type Definition |  |
| SEQUENCE { harqInfo_r7 HARQ_Info_r7 OPTIONAL, addOrReconfMAC_ehs_ReordQ SS_MAC_ehs_AddReconfReordQueueList_r9 OPTIONAL, ackNackRepetitionFactor ACK_NACK_repetitionFactor OPTIONAL, mimoStatus BOOLEAN OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_ehs_AddReconfReordQueueList_r9 |
| Comment | Applicable Rel-9 or later |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxQueueIDs)) OF SS_MAC_ehs_AddReconfReordQ_r9 |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_ehs_AddReconfReordQ_r9 |
| Comment | Applicable Rel-9 or later The priority of PriorityQueue shall set according to the priority of logical channels which is mapped on to this priority queue. NOTE: The range of priority of PriorityQueue is from 0 to 7 and 0 is the lowest priority. |
| Type Definition |  |
| SEQUENCE { mAC_ehs_AddReconfReordQ MAC_ehs_AddReconfReordQ_r9, priority INTEGER(0..7) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | EHS_DSCH_CommonFlows |
| Comment | Applicable Rel-7 or later |
| Type Definition |  |
| SEQUENCE { harqInfo_r7 HARQ_Info_r7 OPTIONAL, common_MAC_ehs_ReorderingQueueList SS_Common_MAC_ehs_ReorderingQueueList OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_Common_MAC_ehs_ReorderingQueueList |
| Comment | Applicable Rel-7 or later |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxQueueIDs)) OF SS_Common_MAC_ehs_ReorderingQueue |  |

| Type Name | SS_Common_MAC_ehs_ReorderingQueue |
| --- | --- |
| Comment | Applicable Rel-7 or later The priority of PriorityQueue shall set according to the priority of logical channels which is mapped on to this priority queue. NOTE: The range of priority of PriorityQueue is from 0 to 7 and 0 is the lowest priority. |
| Type Definition |  |
| SEQUENCE { common_MAC_ehs_ReorderingQueue Common_MAC_ehs_ReorderingQueue, priority INTEGER(0..7) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_AddOrReconfMAC_dFlow |
| Comment | Applicable Rel-5 or later |
| Type Definition |  |
| SEQUENCE { mac_hs_AddReconfQueue_List SEQUENCE (SIZE(1..maxQueueIDs)) OF SEQUENCE { mac_hs_AddReconfQueue SS_MAC_hs_AddReconfQueue} OPTIONAL, mac_hs_DelQueue_List SEQUENCE (SIZE(1..maxQueueIDs)) OF SEQUENCE { mac_hsQueueId INTEGER(0..7)} OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MAC_hs_AddReconfQueue |
| Comment | Applicable Rel-5 or later The priority of PriorityQueue shall set according to the priority of logical channels which is mapped on to this priority queue. NOTE: The range of priority of PriorityQueue is from 0 to 7 and 0 is the lowest priority. DiscardTimer defines the time (unit ms) to live for a MAC-hs SDU starting from the instant of its arrival into an HSDPA Priority Queue. The SS shall use this information to discard out-of-data MAC-hs SDUs from the HSDPA Priority Queues. |
| Type Definition |  |
| SEQUENCE { mAChsAddReconfQueue MAC_hs_AddReconfQueue, logicalChannelList SEQUENCE OF LogicalChannelIdentity, -- logical channels mapping onto the priority queue -- which is specified in maChsAddReconfQueue priority INTEGER(0..7), discardTimer ENUMERATED { v20(0),v40(1),v60(2),v80(3),v100(4),v120(5),v140(6),v160(7),v180(8),v200(9), v250(10),v300(11),v400(12),v500(13),v750(14),v1000(15),v1250(16),v1500(17),v1750(18),v2000(19),v2500(20),v3000(21), v3500(22),v4000(23),v4500(24),v5000(25), v7500(26) } OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | E_DCHMACdFlows |
| Comment | Rel-6 or later addReconf_MAC_d_FlowList and addReconf_MAC_d_FlowList_r7 are mutually exclusive. One of these shall be present. |
| Type Definition |  |
| SEQUENCE { tti E_DCH_TTI, harq_Info ENUMERATED {rv0 (0), rvtable (1)}, addReconf_MAC_d_FlowList E_DCH_AddReconf_MAC_d_FlowList OPTIONAL, -- Rel-6 addReconf_MAC_d_FlowList_r7 E_DCH_AddReconf_MAC_d_FlowList_r7 OPTIONAL -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonE_DCHMACdFlows |
| Comment | Rel-6 or later addReconf_MAC_d_FlowList and addReconf_MAC_d_FlowList_r7 are mutually exclusive. One of these shall be present. |
| Type Definition |  |
| SEQUENCE { tti E_DCH_TTI, harq_Info ENUMERATED {rv0 (0), rvtable (1)}, addReconfMAC_d_FlowList Common_E_DCH_MAC_d_FlowsList } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_LogicalChannelList |
| Comment |  |
| Type Definition |  |
| CHOICE { allSizes NULL, configured NULL, explicitList SEQUENCE (SIZE (1..15 ) ) OF SS_LogicalChannelByRB } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_LogicalChannelByRB |
| Comment | logChOfRb is an optional and currently unused |
| Type Definition |  |
| SEQUENCE { rb_Identity INTEGER (-31..32), logChOfRb INTEGER (0..1) OPTIONAL } |  |

##### 7.3.2.2.14a	CPHY_UL_PowerModify

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_UL_PowerModify_CNF |
| PCO Type | CSAP |
| Comment | To confirm the increase/decrease in UE uplink DPCH power transmission or send the TPC commands as instructed. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_UL_PowerModify_REQ |
| PCO Type | CSAP |
| Comment | To request increase/decrease in the UE uplink DPCH transmission by the delta value given in dB, from the existing transmission level or make UE to transmit at maximum or minimum power level. It is assumed that the UE UL DPCH transmission power level is set to -20 dbm by default at beginning of each test. For routing Info the Dl DPCH Physical channel ID shall be used. For IE ul_DPCH_Id, the physical channel ID of associated UL DPCH shall be given. SS can use it or neglect it. Ul_UE_TxPower gives either the value in dB, by which SS shall increase/decrease the uplink transmission power of UE from the existing transmission power, when this primitive is called or Start transmission of TPC commands on DL DPCCH as configured |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ul_DPCH_Id INTEGER(0..31), ul_UE_Tx_Power Ul_UE_Tx_Power } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Ul_UE_Tx_Power |
| Comment | Choice delta gives the value in dB, by which the existing UE UL DPCH transmission power level is to be increased or decreased. After reaching the new desired level SS shall make UE to maintain this new transmission power level. WithChoice maxMin, and ENUM 'tpc_Up' selection, SS shall start transmitting TPC commands on the DL DPCCH, as '1' every slot so as to ask UE to increase the transmission power. With Choice maxMin, and ENUM 'tpc_Down' selection, SS shall start transmitting TPC commands on the DL DPCCH, as '0' every slot so as to ask UE to decrease the transmission power. With Choice maxMin, and ENUM 'tpc_Maintain' selection, SS will start transmitting TPC commands on the DL DPCCH, as alternate '0' and '1' in alternate slots so as to maintain the UE uplink transmission power |
| Type Definition |  |
| CHOICE { deltaINTEGER (-64..63), maxMin ENUMERATED{ tpc_Up(0), tpc_Down(1), tpc_Maintain(2) } } |  |

##### 7.3.2.2.14	CPHY_TrCH_Release

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TrCH_Release_REQ |
| PCO Type | CSAP |
| Comment | To request to release the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, trchConfigType TrchConfigType, activationTime SS_ActivationTime } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TrCH_Release_CNF |
| PCO Type | CSAP |
| Comment | To confirm to release the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.15	CMAC_BMC_Scheduling

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_BMC_Scheduling_CNF |
| PCO Type | CSAP |
| Comment | To confirm the BMC scheduling. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_BMC_Scheduling_REQ |
| PCO Type | CSAP |
| Comment | Send the BMC scheduling information to the MAC. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, schedulingInfo BMC_SchedulingInfo } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | BMC_SchedulingInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { level1Info BMC_SchedulingLevel1Info, level2Info BMC_SchedulingLevel2Info OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | BMC_SchedulingLevel2Info |
| Comment | ctchBsIndexList contains a CTCH BS index offset value for each BMC PDU transmitted, the offset is relative to the previous BMC PDU transmitted. The offset for the first BMC PDU is set to 0. |
| Type Definition |  |
| SEQUENCE { ctchBsIndexList SEQUENCE (SIZE(1..256)) OF INTEGER (0..255), drxSelectionBitmap OCTET STRING } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | BMC_SchedulingLevel1Info |
| Comment | 0  K  N-1 (3GPP TS 25.331 [21], subclause 8.5.16) |
| Type Definition |  |
| SEQUENCE { ctchAllocationPeriod INTEGER (1..256), -- N cbsFrameOffset INTEGER (0..255) -- K } |  |

##### 7.3.2.2.16	CMAC_Ciphering_Activate

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Ciphering_Activate_CNF |
| PCO Type | CSAP |
| Comment | To confirm to activate or inactivate the ciphering |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Ciphering_Activate_REQ |
| PCO Type | CSAP |
| Comment | To request to start or restart downlink ciphering or uplink deciphering. The physicalChannelIdentity of DPCH applies to routingInfo. Initialize the 20 MSB of HFN component of COUNT-C to the START value stored. If the value of incHFN is set to "NotInc" the SS initializes the remaining LSBs of HFN component in COUNT-C to zero and the SS shall not increment HFN part of COUNT-C at every CFN cycle. If the value of incHFN is set to "IncPerCFN_Cycle" the SS initializes the remainingLSBs of HFN component in COUNT-C accordingly. If it is absent the SS initialize the LSBs of HFN component in COUNT-C to zero, increments the HFN component in COUNT-C by one and then starts the increment HFN part of COUNT-C at every CFN cycle. Only one among cipheringModeInfo and CipheringModeInfo_r7 shall be present. The later being applicable from Rel 7 onwards. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, ratType RatType, cn_DomainIdentity CN_DomainIdentity, cipheringModeInfo CipheringModeInfo OPTIONAL, cipheringModeInfo_r7 CipheringModeInfo_r7 OPTIONAL, -- Rel 7 or later incHFN Increment_Mode } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Increment_Mode |
| Comment |  |
| Type Definition |  |
| ENUMERATED {incPerCFN_Cycler(0), notInc(1), incByOne_IncPerCFN_Cycle(2)} |  |

##### 7.3.2.2.16a	CMAC_FACH_MeasOccas

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_FACH_MeasOccas_CNF |
| PCO Type | CSAP |
| Comment | To confirm to config FACH Measurement Occasions |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_FACH_MeasOccas_REQ |
| PCO Type | CSAP |
| Comment | To request MAC layer to configure FACH Measurement Occasions (FMO). If IE "Inter-frequency FDD measurement indicator" is set to TRUE, the SS will start inter-frequency FMO handling at the activationTime. If this IE is set to FALSE, the SS inter-frequency FMO handling will be stopped at the activation Time. For the FDD test, the IE "Inter-frequency TDD measurement indicator" is set to FALSE. For GSM or LTE measurement, if IE "Inter-RAT measurement indicators" is included, independent of the RAT-Type, the SS will start inter-RAT FMO handling at the activationTime. If this IE is omitted, the SS inter-RAT FMO handling will be stopped at the activation Time. largest_TTI_Number is the TTI (in number of 10ms frames) of the FACH having the largest TTI on the SCCPCH. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, fMO FACH_MeasurementOccasionInfo, largest_TTI INTEGER(1..8), activationTime SS_ActivationTime } |  |

##### 7.3.2.2.17	CMAC_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Config_CNF |
| PCO Type | CSAP |
| Comment | For MAC emulator to report that a previous attempt to setup, reconfigure or release a logical channel is successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to configure MAC entity. Setup is used for creation of the MAC instances or the MAC resources. Release is used for free the all MAC resources. The reconfiguration is to change the MAC parameters, it is not the MAC modification. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, ratType RatType, configMessage CHOICE { setup CmacConfigReq, reconfigure CmacConfigReq, release SS_ActivationTime } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacConfigReq |
| Comment | To request to configure MAC The IE associatedPhychConfigToFollow should be set to TRUE when there is a associated physical channel configuration to follow (e.g configuration of PICH or MICH). The implementation of the IE "associatedPhychConfigToFollow" can be left to the SS either to wait for the associated physical channel before MAC is configured or to continue the configuration of MAC without waiting for the associated physical channel. If Dual Cell is configured, IE servingAndSecondaryCellActivation is included. Presence of tSN_FieldExtension indicates that the TSN field extension is used as defined in TS 25.321[17]; the absence indicates that 6 bits TSN is used. For 4C-HSDPA tSN_FieldExtension is set to TRUE. |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, uE_Info UE_Info, trCHInfo TrCHInfo, trCH_LogCHMapping TrCH_LogCHMappingList1, associatedPhychConfigToFollow BOOLEAN DEFAULT FALSE, servingAndSecondaryCellActivation ServingAndSecondaryCellActivation_Type OPTIONAL, -- for Dual Cell Rel-8 or later tSN_FieldExtension BOOLEAN DEFAULT FALSE, -- Rel-9 or later relAspTypeExtension CmacConfigReq_r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | UE_Info |
| Comment | The value of c_RNTI_DSCH_RNTI is 16 bits, used either for C-RNTI or DSCH-RNTI. DSCH is configured if the physical channel in CMAC_config_REQ is a PDSCH. Otherwise, C-RNTI is applied. At the MAC-hs configuration both u_RNTI and c_RNTI_DSCH_RNTI are omitted. If MAC is being configured with Common_MAC_ehs flows, UE Info U-RNTI shall be provided when the common H-RNTI is applied and is applied in MAC-c header on SRB#1. |
| Type Definition |  |
| SEQUENCE { u_RNTI U_RNTI OPTIONAL, c_RNTI C_RNTI OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCH_LogCHMappingList1 |
| Comment | maxulTrCH = maxdlTrCH = 16 dlconnectedMACdFlows is used for MAC-hs and dlconnectedMAC_ehsFlows for MAC-ehs configurations in dedicated. dlconnectedMAC_ehsCommonFlow is used for MAC-ehs in Common state. For BCCH and PCCH in common state, the mac_ehs_QueueId is omitted, no reordering is applied and the TSN and SI fields are always set to zero (according to 3GPP TS 25.321 [17], subclauses 9.1.4 and 11.6.4.4) dlconnectedMAC_ehsFlowsDC is used when dual cell or 4C-HSDPA or Multiflow is activated. |
| Type Definition |  |
| SEQUENCE { ulconnectedTrCHList SEQUENCE (SIZE (1..maxulTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, trCH_LogCHMappingList TrCH_LogCHMappingList } OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (1..maxdlTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, trCH_LogCHMappingList TrCH_LogCHMappingList } OPTIONAL, dlconnectedMACdFlows SEQUENCE (SIZE (1..8)) OF SEQUENCE { mac_dFlowId MAC_d_FlowIdentity, trCH_LogCHMappingList TrCH_LogCHMappingList } OPTIONAL, -- Rel-5 or later dlconnectedMAC_ehsFlows SEQUENCE (SIZE (1..8)) OF SEQUENCE { mac_ehs_QueueId MAC_ehs_QueueId, trCH_LogCHMappingList TrCH_LogCHMappingList } OPTIONAL, -- Rel-7 or later dlconnectedMAC_ehsCommonFlows SEQUENCE (SIZE (1..8)) OF SEQUENCE { mac_ehs_QueueId MAC_ehs_QueueId OPTIONAL, -- not present for BCCH/PCCH trCH_LogCHMappingList TrCH_LogCHMappingList } OPTIONAL, -- Rel-7 or later dlconnectedMAC_ehsFlowsDC SEQUENCE (SIZE (1..8)) OF SEQUENCE { mac_ehs_QueueId MAC_ehs_QueueId, trCH_LogCHMappingList TrCH_LogCHMappingList, } OPTIONAL, -- Rel-8 or later relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCH_LogCHMappingList |
| Comment | maxLogCHperTrCH = 15 |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxLogCHperTrCH)) OF TrCH_LogicalChannelMapping |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCHInfo |
| Comment | The same TFCS information should be provided to the PHY and MAC layers at all times. When a CMAC_Config_REQ is used to configure the MAC layer, a corresponding CPHY_TrCH_Config_REQ should be sent to the PHY layer to ensure that the configuration is consistent. For MAC-hs configuration: When ulconnectedTrCHList, ulTFCS, dlconnectedTrCHList and dlTFCS are omitted and hsDSCHMacdFlows is present this ASP configures an MAC-hs entity. For MAC-ehs configuration: When ulconnectedTrCHList, ulTFCS, dlconnectedTrCHList and dlTFCS are omitted and ehs_DSCH_Flows or E-HS-DSCH_Common Flows or ehs_DSCH_Flows_r9 is present this ASP configures an MAC-ehs entity. Only one of hsDSCHMacdFlows or ehs_DSCH_Flows or ehs_DSCH_Flows_r9 can be present (Only one of MAC-hs or MAC-ehs layer can be configured). |
| Type Definition |  |
| SEQUENCE { ulconnectedTrCHList SEQUENCE (SIZE (1..maxulTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, transportChannelInfo CommonOrDedicatedTFS } OPTIONAL, ulTFCS TFCS OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (1..maxdlTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, transportChannelInfo CommonOrDedicatedTFS } OPTIONAL, dlTFCS TFCS OPTIONAL, hsDSCHMacdFlows HS_DSCHMACdFlows OPTIONAL, -- Rel-5 or later ehs_DSCH_Flows EHS_DSCH_Flows OPTIONAL, -- Rel-7 or later ehs_DSCH_CommonFlows EHS_DSCH_CommonFlows OPTIONAL, -- Rel-7 or later ehs_DSCH_Flows_r9 EHS_DSCH_Flows_r9 OPTIONAL, -- Rel-9 or later relAspTypeExtension TrCHInfo_r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCH_LogicalChannelMapping |
| Comment | When used for logical channel to MAC_d flow mapping dl_LogicalChannelMapping shall be chosen, |
| Type Definition |  |
| SEQUENCE { logicalChannel_Mapping CHOICE { ul_LogicalChannelMapping SS_UL_LogicalChannelMapping, dl_LogicalChannelMapping SS_DL_LogicalChannelMapping }, rB_Identity INTEGER (-31..32) OPTIONAL, cn_DomainIdentity CN_DomainIdentity OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_LogicalChannelMapping |
| Comment | If the macHeaderManipulation field is 'NormalMacHeader', then data received on the transport channel supporting this logical channel shall have it's MAC header inspected to determine the appropriate routing, and removed as normal. The MAC SDU shall be passed to the appropriate logical channel. If the macHeaderManipulation field is 'OmitMacHeader', then data received on the transport channel supporting this logical channel shall have it's MAC header inspected to determine the appropriate routing, but the MAC layer shall not remove the MAC header. Thus the entire MAC PDU shall be passed to the appropriate logical channel, and the MAC header can be checked by the TTCN. |
| Type Definition |  |
| SEQUENCE { macHeaderManipulation MAC_HeaderManipulation, ul_TransportChannelType SS_UL_TransportChannelType, logicalChannelIdentity LogicalChannelIdentity, logicalChannelType LogicalChannelType } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DL_LogicalChannelMapping |
| Comment | If the macHeaderManipulation field is 'NormalMacHeader', then data transmitted on this logical channel shall have an appropriate MAC header added before it is sent to lower layers for transmission. If the macHeaderManipulation field is 'OmitMacHeader', then data transmitted on this logical channel shall not have any MAC header information added, even if the logical channel type and mapping indicates that there should be a MAC header present. This allows the entire MAC PDU to be specified in the TTCN, so individual fields in the MAC header can be modified. When used for DTCH mapping to MAC_d flow, rlc_SizeList shall choose "configured" according to the configured mAChsAddReconfQueue values. When the logical channel is MTCH, the logicalChannelIdentity shall be consistent with MBMS_LogicalChIdentity in MBMS_PTM_RBInformation_N and MBMS_PTM_RBInformation_C. |
| Type Definition |  |
| SEQUENCE { macHeaderManipulation MAC_HeaderManipulation, dlTransportChannelType SS_DL_TransportChannelType, logicalChannelIdentity LogicalChannelIdentity, logicalChannelType LogicalChannelType, rlc_SizeList CHOICE { allSizes NULL, configured NULL, explicitList RLC_SizeExplicitList}, mac_LogicalChannelPriority MAC_LogicalChannelPriority OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_TransportChannelType |
| Comment |  |
| Type Definition |  |
| ENUMERATED { dch (0), rach (1), cpch (2), usch (3), edch (4) -- Rel-6 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DL_TransportChannelType |
| Comment |  |
| Type Definition |  |
| ENUMERATED { dch (0), fach (1), bch (2), pch (3), dsch (4), hsdsch (5) -- Rel-5 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | LogicalChannelType |
| Comment |  |
| Type Definition |  |
| ENUMERATED { bCCH (0), pCCH (1), cCCH (2), cTCH (3), dCCH (4), dTCH (5), sHCCH (6), mTCH (7), -- Rel-6 or later mCCH (8), -- Rel-6 or later mSCH (9) -- Rel-6 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MAC_HeaderManipulation |
| Comment |  |
| Type Definition |  |
| ENUMERATED { normalMacHeader (0), omitMacHeader (1) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ServingAndSecondaryCellActivation_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE { servingCellId INTEGER(0..63), secondaryCellId INTEGER(0..63), secondaryServing_HS_DSCH_Cell ENUMERATED { deactivated (0), activated (1) } DEFAULT deactivated } |  |

##### 7.3.2.2.17a	CMAC_MAChs_MACehs_TFRCconfigure (Rel-5 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_MACehs_TFRCconfigure_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later Confirm a previous CMAC_MAChs_MACehs_TFRCconfigure_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_MACehs_TFRCconfigure_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To configure the TFRC selection in the MAC-hs entity, channelisationCodeOffset + noOfChannelisationCodes shall not be great than 15. If explicitlyConfigured is selected in tfrcConfigMode, the SS shall use all the parameter values specified to configure a correct transport format and radio resources. This configuration is used for HS-SCCH associated HS-DSCH transmission. If sS_Configured is selected, the parameter value range is specified. SS shall dynamically select the suitable values for the parameters "modulationScheme", "channelisationCodeOffset", "noOfChannelisationCodes ", .tbSizeIndexOnHS_SCCH", "redundancyVersion" and "hs_PDSCH_TxPower" according to UE's capability category and CQI information reported by the UE. As HS-SCCH less operation and MIMO cannot be simultaneously configured, only one among hs_scch_LessInfo and mimoStatus can be present. When both are absent non MIMO, non HS-SCCH less operation is configured. explicitHS_SCCH_LessMode is used to force SS to use HS-SCCH less operation. explicitMIMO is used for MIMO dual stream transmission. Presence of ss_DTX_Info indicates DL_DRX is enabled. If minimumInterTTIinterval is set to 1, Data is sent every TTI. If it is set to 2, every TTI with Data shall be followed by at least 1 TTI without Data. If it is set to 3, every TTI with Data shall be followed by at least 2 TTI without Data. This field needs to be set as per UE category as defined in Table 5.1a of 3GPP TS 25.306 [59]. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), tfrcConfigMode CHOICE { explicitlyConfigured SEQUENCE { modulationScheme ModulationScheme, channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), tbSizeIndexOnHS_SCCH INTEGER (0..63), minimumInterTTIinterval INTEGER (1..3), redundancyVersions RedundancyVersionList, hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH }, sS_Configured SEQUENCE { minChannelisationCodeOffset INTEGER (1..15), maxNoOfChannelisationCodes INTEGER (1..15), iniHS_PDSCH_TxPower DL_TxPower, -- default offset related -- to p-CPICH or s-CPICH hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL, mimoStatus BOOLEAN DEFAULT FALSE }, explicitHS_SCCH_LessMode SEQUENCE{ hs_pdsch_CodeIndex INTEGER (1..15), hs_scch_LessTFI INTEGER (1..90), hs_scch_LessSecondCodeApplicability BOOLEAN, tbs INTEGER (0..3), -- the index of tbs for HS-SCCH less operation -- The value should be consistent with code index, TFI and second -- code applicability minimumInterTTIinterval INTEGER (1..3), hs_PDSCH_TxPower DL_TxPower -- default offset related to p-CPICH or s-CPICH }, explicitMIMO SEQUENCE { modulationSchemeAndNumTB INTEGER(0..7), -- set according to table 14 of 25.212 -- Values 1,2 and 5 are used for 64QAM+MIMO. Rel-8 or later channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), precodingWeight2 INTEGER (0..3), -- set according to table 14a of 25.212 primaryTB_SizeIndexOnHS_SCCH INTEGER (0..63), secondaryTB_SizeIndexOnHS_SCCH INTEGER (0..63) OPTIONAL, --present only if second TB is to be tx as per modulationSchemeAndNumTB minimumInterTTIinterval INTEGER (1..3), primaryRedundancyVersions RedundancyVersionList, secondaryRedundancyVersions RedundancyVersionList OPTIONAL, --present only if second TB is to be tx as per modulationSchemeAndNumTB hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH }, sS_Configured_DC SEQUENCE { serving_minChannelisationCodeOffset INTEGER (1..15), serving_maxNoOfChannelisationCodes INTEGER (1..15), serving_mimoStatus BOOLEAN DEFAULT FALSE, –- Rel-9 or later sec_minChannelisationCodeOffset INTEGER (1..15), sec_maxNoOfChannelisationCodes INTEGER (1..15), sec_mimoStatus BOOLEAN DEFAULT FALSE, –- Rel-9 or later iniHS_PDSCH_TxPower DL_TxPower, -- default offset related -- to p-CPICH or s-CPICH hs_scch_LessInfo HS_SCCH_LessInfo_r7 OPTIONAL }, explicitlyDC SEQUENCE { servingTFRC SEQUENCE{ modulationScheme ModulationScheme, channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), tbSizeIndexOnHS_SCCH INTEGER (0..63), minimumInterTTIinterval INTEGER (1..3), redundancyVersions RedundancyVersionList, hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH } OPTIONAL, -- when omitted then no data is sent on the serving HS-DSCH cell secondaryTFRC SEQUENCE{ modulationScheme ModulationScheme, channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), tbSizeIndexOnHS_SCCH INTEGER (0..63), minimumInterTTIinterval INTEGER (1..3), redundancyVersions RedundancyVersionList, hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH } OPTIONAL -- when omitted then no data is sent on the secondary HS-DSCH cell }, explicitlyDC_MIMO SEQUENCE { -- Rel-9 or later servingTFRC SEQUENCE{ modulationSchemeAndNumTB INTEGER(0..7), -- set according to table 14 of 25.212 -- Values 1,2 and 5 are used for 64QAM+MIMO. Rel-8 or later channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), precodingWeight2 INTEGER (0..3), -- set according to table 14a of 25.212 primaryTB_SizeIndexOnHS_SCCH INTEGER (0..63), secondaryTB_SizeIndexOnHS_SCCH INTEGER (0..63) OPTIONAL, -- present only if second TB is to be tx -- as per modulationSchemeAndNumTB minimumInterTTIinterval INTEGER (1..3), primaryRedundancyVersions RedundancyVersionList, secondaryRedundancyVersions RedundancyVersionList OPTIONAL, -- present only if second TB is to be tx -- as per modulationSchemeAndNumTB hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH } OPTIONAL -- when omitted then no data is sent on the serving HS-DSCH cell secondaryTFRC SEQUENCE{ modulationSchemeAndNumTB INTEGER(0..7), -- set according to table 14 of 25.212 -- Values 1,2 and 5 are used for 64QAM+MIMO. Rel-8 or later channelisationCodeOffset INTEGER (1..15), noOfChannelisationCodes INTEGER (1..15), precodingWeight2 INTEGER (0..3), -- set according to table 14a of 25.212 primaryTB_SizeIndexOnHS_SCCH INTEGER (0..63), secondaryTB_SizeIndexOnHS_SCCH INTEGER (0..63) OPTIONAL, -- present only if second TB is to be tx -- as per modulationSchemeAndNumTB minimumInterTTIinterval INTEGER (1..3), primaryRedundancyVersions RedundancyVersionList, secondaryRedundancyVersions RedundancyVersionList OPTIONAL, -- present only if second TB is to be tx -- as per modulationSchemeAndNumTB hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-CPICH or s-CPICH } OPTIONAL -- when omitted then no data is sent on the secondary HS-DSCH cell }, relAspTypeExtension TFRCconfigure_r10OrLaterExtensionType -- Rel-10 or later }, activationTime SS_ActivationTime, ss_DTX_Info DRX_Info OPTIONAL hs_DSCH_TBSizeTable HS_DSCH_TBSizeTable OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RedundancyVersionList |
| Comment | Gives the Redundancy and constellation version coding sequence (Xrv) to be used for every transmission / retransmission. The SIZE (number of Xrv elements in there) of the SEQUENCE implies the number of HARQ transmission / retransmissions to be required. |
| Type Definition |  |
| SEQUENCE (SIZE (1..8)) OF INTEGER (0..7) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ModulationScheme |
| Comment |  |
| Type Definition |  |
| ENUMERATED {qpsk (0), qam16 (1), qam64 (2), spare1 (3)} |  |

##### 7.3.2.2.17a0	CMAC_MAChs_MACehs_HARQprocAssign

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_MACehs_HARQprocAssign_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later Confirm a previous CMAC_MAChs_MACehs_HARQprocAssign_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_MACehs_HARQprocAssign_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To assign a HARQ process handling the next MAC-hs PDU transmission. This ASP provides TTCN the ability to select an HARQ process serving the next MAC-hs PDU which follows the ASP. After successful transmission the MAC-hs returns back to normal operation. In the normal operation a suitable HARQ process is selected by HARQ entity in the MAC-hs to serve the MAC-hs PDU without TTCN intervening. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), harqProcessId INTEGER(0..15¦31) } |  |

##### 7.3.2.2.17a1	CMAC_MACehs_HARQAssign_MultiFlows (Rel-7 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_HARQAssign_MultiFlows_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later Confirm a previous CMAC_MACehs_HARQAssign_MultiFlows_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_HARQAssign_MultiFlows_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later To assign the HARQ processes for the simultaneous transmission of the MAC-ehs PDUs on the different flows on the same TTI . After successful transmission the MAC-ehs returns back to normal operation. In the normal operation suitable HARQ processes are selected by HARQ entities in the MAC-ehs to serve the MAC-ehs PDU without TTCN intervening. For Dual Cell, the primaryFlow corresponds to the serving cell, the secPrimaryFlow corresponds to the secondary cell. For MIMO, the primary and secondary HARQ process identities (of serving cell) are set as per 3GPP TS 25.212 [58], subclause 4.6B.2.5. For combined DC-HSDPA and MIMO the primaryFlow and secondary flow correspond to the serving cell, the secPrimaryFlow and secSecondaryFlow to the secondary cell. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), primaryFlowHarqProcessId INTEGER(0..15¦31), -- of serving cell secondaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, -- of serving cell secPrimaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, -- of secondary cell Rel-9 or later secSecondaryFlowHarqProcessId INTEGER(0..15¦31) OPTIONAL, -- of secondary cell Rel-9 or later relAspTypeExtension HARQAssign_MultiFlows_r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

##### 7.3.2.2.17aa	CMAC _MACehs_HS_SCCH_Orders (Rel-7 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_HS_SCCH_Orders_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later Confirm a previous CMAC _MACehs_HS_SCCH_Orders_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo -- Physical Channel ID } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACehs_HS_SCCH_Orders_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later To instruct SS to transmit requested HS-SCCH orders at requested time: OrderType, drx_order, dtx_Order are as per 3GPP TS 25.212 [58], subclause 4.6c.2. activationTime and subframe together indicate the time on which the HS-SCCH order is to be transmitted. When activationTime is set to ‘activateNow’, subframe is omitted. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, -- Physical Channel ID activationTime SS_ActivationTime, subframe INTEGER (0..4) OPTIONAL, ordertype INTEGER (0..7), xord1 INTEGER (0..1), xord2 INTEGER (0..1), xord3 INTEGER (0..1), h_RNTI H_RNTI } |  |

##### 7.3.2.2.17b	CMAC_MACe_Config (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_Config_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACe_Config_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_Config_REQ |
| PCO Type | CSAP |
| Comment |  |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), configMessage CHOICE { setup MACeConfig, reconfig MACeConfig, reset NULL, release SS_ActivationTime } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACeConfig |
| Comment | If the macHeaderManipulation field is 'NormalMacHeader' in ddiMappingList, then data received on the E-DCH (MAC_e PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, and the MACes PDU shall be passed to the MAC_es together with the relevant DDI, N, CFN and subframe number. If the macHeaderManipulation field field is 'OmitMacHeader', then data received on the E-DCH (MAC_e PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, then the MAC_e layer shall delivery the MAC-es PDU, SI and the related CFN, subframe number to the MAC_es entity. connectedToMAC_es field is used to provide the possibility that the E-DCH-MACdFlows from only one MAC_e entity are connected to the MAC_es entity in the inter node B soft handover test cases. The IEs ddiMappinglist and e_DCHMacdFlows can be OMITted when changing the serving cell MAC-e without modification of MAC-e configurations. It will be applied in EDCH SHO. ss_DRX_MAC_Info presence indicates UL DRX shall be applied. |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, ddiMappinglist DDI_MappingList OPTIONAL, e_DCHMacdFlows E_DCHMACdFlows OPTIONAL, connectedToMAC_es BOOLEAN DEFAULT TRUE, –- can be set to FALSE in inter nodeB SHO ss_DRX_MAC_Info SS_DRX_MAC_Info OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DRX_MAC_Info |
| Comment | Consistent with E-DCH TTI, either of mac_dtx_Cycle_2ms or mac_dtx_Cycle_10ms shall be present. |
| Type Definition |  |
| SEQUENCE { mac_InactivityThreshold MAC_InactivityThreshold, mac_dtx_Cycle_2ms MAC_DTX_Cycle_2ms OPTIONAL, mac_dtx_Cycle_10ms MAC_DTX_Cycle_10ms OPTIONAL, timingInfo NewTiming } |  |

##### 7.3.2.2.17c	CMAC_MACe_NodeB_CellMapping (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_NodeB_CellMapping_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACe_NodeB_CellMapping_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_NodeB_CellMapping_REQ |
| PCO Type | CSAP |
| Comment | To put a set of cells under the control of a MAC_e entity indicated by nodeB_Id, which is configured by CMAC_MAC_e_Config_REQ. This ASP establishes the routing relation between E-DCH related channels in these cells with a MAC_e entity. A cell is mapped to only one NodeB, and the cellId allocation is unique in a test. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), celllist SEQUENCE OF INTEGER (0..63) } |  |

##### 7.3.2.2.17d	CMAC_MACes_Config (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACes_Config_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACes_Config_REQ being successful. cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACes_Config_REQ |
| PCO Type | CSAP |
| Comment | This ASP is used for creating and configuring, reconfiguring, resetting or releasing an MAC_es, a cell / nodeB-independent entity in the SS. cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), configMessage CHOICE { setup MACesConfig, reconfig MACesConfig, reset NULL, release SS_ActivationTime} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACesConfig |
| Comment | MACesConfig establishes the mapping between logical channels and E-DCH_MACd_Flows. When the macTestMode is TRUE, the re-ordering entity shall not eliminate the duplicated packets, but passes them to RLC. macTestMode = "TRUE" is used for testing the retransmission function of HARQ process. |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, ddiMappinglist DDI_MappingList, macTestMode BOOLEAN DEFAULT FALSE } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DDI_MappingList |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..31)) OF DDI_Mapping |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DDI_Mapping |
| Comment | Both SRBs and RBs can be mapped onto E-DCH. The mechanism for control of MAC header manipulation (macHeaderManipulation) is applied to individual logical channels to be mapped on E-DCH. Typically, the SRBs are configured in 'NormalMacHeader" mode while the RBs in user plane can be configured either in 'NormalMacHeader" or in "OmitMacHeader" mode. If more than one UL RLC PDU size is configured for the RB (represent by logicalChannelIdentity), the different sizes will use subsequent DDI values starting from the DDI value in this table. If the value of macHeaderManipulation field is 'NormalMacHeader', then data received on the E-DCH MACd flows supporting this logical channel shall have its MAC header inspected to determine the appropriate routing, and removed as normal. The MACes SDU shall be passed to the appropriate logical channel. If the value of macHeaderManipulation field is 'OmitMacHeader', then data received on the E-DCH MACd flows supporting this logical channel shall have it's MAC header inspected to determine the appropriate routing, then the MAC_es layer shall deliver the MAC-es SDU, SI and the related CFN, subframe number, HARQ process identity to the appropriate logical channel. The TTCN receives these fields by RLC_TR_MACesDATA_IND, then these fields can be checked by the TTCN. HARQ ID = CFN mod 4 for TTI=10ms; HARQ ID = (5*CFN + subframe number) mod 8 for TTI=2ms |
| Type Definition |  |
| SEQUENCE { macHeaderManipulation MAC_HeaderManipulation, logicalChannelIdentity LogicalChannelIdentity, e_DCH_MAC_d_FlowIdentity E_DCH_MAC_d_FlowIdentity, ddi DDI, rlc_PDU_SizeList RLC_PDU_SizeList, includeInSchedulingInfo BOOLEAN, mac_LogicalChannelPriority MAC_LogicalChannelPriority, logicalChannelType LogicalChannelType, rB_Identity INTEGER (-31..32) OPTIONAL } |  |

##### 7.3.2.2.17e	CMAC_MACe_AG (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_AG_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACe_AG_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_AG_REQ |
| PCO Type | CSAP |
| Comment | The hARQProcId shall be converted to the nearest CFN (and subframe number if TTI = 2 ms) by the SS, and the Absolute Grant is sent in that CFN (and subframe number if TTI = 2 ms) |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), grantType ENUMERATED {primary(0), secondary(1)}, absoluteGrantValue BIT STRING(SIZE(5)), absoluteGrantScope BIT STRING(SIZE(1)), hARQProcId INTEGER (0..7), activationTime SS_ActivationTime } |  |

##### 7.3.2.2.17f	CMAC_MACe_AckNack (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_AckNack_CNF |
| PCO Type | CSAP |
| Comment | To Confirm CMAC_e_AckNack_REQ |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_AckNack_REQ |
| PCO Type | CSAP |
| Comment | To request the SS to set operation mode of the Ack/Nack function for the HARQ process hARQProcId. The harqProcId, between 0 to 3 for 10 ms TTI or 0 to 7 for 2 msTTI, is individually applied to the configuration for the normal / nack mode. If the special hARQProcId -1 is used, all active HARQ processes ( 0..3 for 10 ms TTI and 0..7 for 2 ms TTI) will be configured according to ackNackFunction. At the SS initialization Ack/Nack function is in normal operation mode |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), hARQProcId INTEGER (-1¦0..7), ackNackFunction AckNackFunction } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AckNackFunction |
| Comment | normal : put the HARQ process in normal operation mode, it generats the ACK or NACK according to whether the received TB block can be decoded correctly and delivery the correctly decoded data to higher layer. When the MAC_e is configured the HARQ process is in normal operation mode. nack : put the HARQ process in the special operation mode in which the HARQ process always sends NACK for the received TB blcok till the number of the retransmissions reaches the number indicated in this field. The HARQ process is back to the normal operation mode after the number is reached or received a normal mode request. Except each received TB shall be passed to higher layer, other operations are the same as a real NACK occured. |
| Type Definition |  |
| CHOICE { normal NULL, nack E_DCH_MAC_d_FlowMaxRetrans } |  |

##### 7.3.2.2.17g	CMAC_MACe_E_TFC_Restriction (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_E_TFC_Restriction_CNF |
| PCO Type | CSAP |
| Comment | For MAC emulator to report that a previous attempt of restricting TFCs have been successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_E_TFC_Restriction_REQ |
| PCO Type | CSAP |
| Comment | To request to configure MACe entity. The field restrictAllowedTFCs is provided to allow the E-TFCI to be restricted. The IE fullE_TFCS will be used to remove any previous E_TFCS restriction configured. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER (0..63), restrictAllowedTFCs CHOICE { e_TFCS_Restriction E_TFCS_Restriction, fullE_TFCS NULL } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | E_TFCS_Restriction |
| Comment | The E_TFCS restriction is a list of E-TFCIs, and can be used to verify that the UE has used a specific TFC. Any data received by the SS using a forbidden TFCI shall be discarded. |
| Type Definition |  |
| SEQUENCE OF INTEGER (0..127) |  |

##### 7.3.2.2.17h	CMAC_MACe_RG (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_RG_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACe_RG_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_RG_REQ |
| PCO Type | CSAP |
| Comment | For non-serving RL the value for relativeGrant is limited to "down" and "hold". The SS shall convert the hARQProcId to the nearest CFN (and subframe number if TTI = 2 ms) by the SS and send the Relative Grant in that CFN (and subframe number if TTI = 2 ms) |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), relativeGrant ENUMERATED{up(0), down(1), hold(2)}, hARQProcId INTEGER (0..7), activationTime SS_ActivationTime } |  |

##### 7.3.2.2.17ha	Void

##### 7.3.2.2.17i	CMAC_MACes_SI_IND (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACes_SI_IND |
| PCO Type | CSAP |
| Comment | This ASP is used for MACes delivering scheduling information in MAC_es testing. cellId=-1. If the SI was sent alone in a MAC-e PDU or sent together with other MAC-es PDU in a MAC-e PDU but without a special DDI associated the value of specialDDIpresense is set to absent; If the SI was sent together with other MAC-es PDU in a MAC-e PDU with a special DDI (DDI 63) associated the specialDDIpresence is set to present. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), cfn INTEGER (0..255), subframe INTEGER (0..4¦7), -- 0..4 when TTI=2ms, 7 when TTI=10ms specialDDIpresence ENUMERATED {absent (0), present (1)}, uePowerHeadRoom BIT STRING (SIZE(5)), totalE_DCHBufferStatus BIT STRING (SIZE(5)), highestPriorityLogChBS BIT STRING (SIZE(4)), highestPriorityLogChId BIT STRING (SIZE(4)) } |  |

##### 7.3.2.2.17j	CMAC_MACes_SI_Config (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACes_SI_Config_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-6 or later To Confirm CMAC_MACes_SI_Config_REQ, cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACes_SI_Config_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-6 or later To configure the SS to enable / disable to report the reception of Scheduling Information in MAC-es PDU's via primitive CMAC_MACes_SI_IND. At the SS initialization, the default mode is SI reporting disabled. cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), sI_reportEnable ENUMERATED {disable(0), enable(1)} DEFAULT disable } |  |

##### 7.3.2.2.17k	CMAC_MACi_Config (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_Config_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACi_Config_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_Config_REQ |
| PCO Type | CSAP |
| Comment |  |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), configMessage CHOICE { setup MACiConfig, reconfig MACiConfig, reset NULL, release SS_ActivationTime } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACiConfig |
| Comment | If the macHeaderManipulation field is 'NormalMacHeader' in lCH_Mappinglist, then data received on the E-DCH (MAC_i PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, and the MACis PDU shall be passed to the MAC_is together with the relevant LCH_ID, length, F, CFN and subframe number. If the macHeaderManipulation field field is 'OmitMacHeader', then data received on the E-DCH (MAC-i PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, then the MAC-i layer shall deliver the MAC-is PDU, SI and the related CFN, subframe number to the MAC-is entity. connectedToMAC_is field is used to provide the possibility that the E-DCH-MACdFlows from only one MAC-i entity are connected to the MAC_is entity in the inter node B soft handover test cases. When MAC-i is configured in enhanced FACH UL using commonMac_i_r8, the MAC-i entity is always connected to MAC-is. In DC-HSUPA configuration, one MAC-i entity is connected to the MAC-is entity. In DC-HSUPA, e_DCHMacdFlows is applied to the two E-DCH transport channels in the primary and secondary UL frequency. The IEs lCH_Mappinglist and e_DCHMacdFlows can be OMITted when changing the serving cell MAC-i without modification of MAC-i configurations. It will be applied in EDCH SHO. ss_DRX_MAC_Info presence indicates UL DRX shall be applied. Presence of tSN_FieldExtension indicates that the TSN field extension is used as defined in TS 25.321[17] clause 9.2.4.3; the absence indicates that 6 bits TSN is used. |
| Type Definition |  |
| CHOICE { mAC_i_r8 SEQUENCE { -- Rel-8 or later activationTime SS_ActivationTime, lCH_Mappinglist LCH_MappingList OPTIONAL, e_DCHMacdFlows E_DCHMACdFlows OPTIONAL, connectedToMAC_is BOOLEAN DEFAULT TRUE, –- can be set to FALSE in inter nodeB SHO ss_DRX_MAC_Info SS_DRX_MAC_Info OPTIONAL }, commonMac_i_r8 SEQUENCE { activationTime SS_ActivationTime, lCH_Mappinglist LCH_MappingList OPTIONAL, e_DCHMacdFlows CommonE_DCHMACdFlows OPTIONAL, additional_E_DCH_TransmitBackoff INTEGER (0..15), max_CCCH_ResourceAllocation ENUMERATED { tti8(0), tti12(1), tti16(2), tti20(3), tti24(4), tti32(5), tti40(6), tti80(7)}, max_PeriodForCollisionResolution INTEGER (8..24) }, mAC_i_r9 SEQUENCE { -- Rel-9 or later activationTime SS_ActivationTime, lCH_Mappinglist LCH_MappingList OPTIONAL, e_DCHMacdFlows E_DCHMACdFlows OPTIONAL, connectedToMAC_is BOOLEAN DEFAULT TRUE, –- can be set to FALSE in inter nodeB SHO ss_DRX_MAC_Info SS_DRX_MAC_Info OPTIONAL, tSN_FieldExtension BOOLEAN DEFAULT FALSE }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

##### 7.3.2.2.17l	CMAC_MACi_NodeB_CellMapping (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_NodeB_CellMapping_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACi_NodeB_CellMapping_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_NodeB_CellMapping_REQ |
| PCO Type | CSAP |
| Comment | To put a set of cells under the control of a MAC_i entity indicated by nodeB_Id, which is configured by CMAC_MAC_i_Config_REQ. This ASP establishes the routing relation between E-DCH related channels in these cells with a MAC_i entity. A cell is mapped to only one NodeB, and the cellId allocation is unique in a test. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), celllist SEQUENCE OF INTEGER (0..63) } |  |

##### 7.3.2.2.17m	CMAC_MACis_Config (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACis_Config_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACis_Config_REQ being successful. cellId=-1 except when CCCH is configured to MAC-i is entity. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACis_Config_REQ |
| PCO Type | CSAP |
| Comment | This ASP is used for creating and configuring, reconfiguring, resetting or releasing a MAC_is, a cell / nodeB-independent entity in the SS. cellId=-1 except when CCCH is configured to MAC-i is entity. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), configMessage CHOICE { setup MACisConfig, reconfig MACisConfig, reset NULL, release SS_ActivationTime} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACisConfig |
| Comment | MACisConfig establishes the mapping between logical channels and E-DCH_MACd_Flows. When the macTestMode is TRUE, the re-ordering entity shall not eliminate the duplicated packets, but passes them to RLC. macTestMode = "TRUE" is used for testing the retransmission function of HARQ process. Presence of tSN_FieldExtension indicates that the TSN field extension is used as defined in TS 25.321[17] clause 9.2.4.3; the absence indicates that 6 bits TSN is used. |
| Type Definition |  |
| CHOICE { mACis_r8 SEQUENCE { -- Rel-8 or later activationTime SS_ActivationTime, lCHMappinglist LCH_MappingList, macTestMode BOOLEAN DEFAULT FALSE }, mACis_r9 SEQUENCE { -- Rel-9 or later activationTime SS_ActivationTime, lCHMappinglist LCH_MappingList, macTestMode BOOLEAN DEFAULT FALSE, tSN_FieldExtension BOOLEAN DEFAULT FALSE }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | LCH_MappingList |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..31)) OF LCH_Mapping |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | LCH_Mapping |
| Comment | Both SRBs and RBs can be mapped onto E-DCH. The mechanism for control of MAC header manipulation (macHeaderManipulation) is applied to individual logical channels to be mapped on E-DCH. Typically, the SRBs are configured in 'NormalMacHeader" mode while the RBs in user plane can be configured either in 'NormalMacHeader" or in "OmitMacHeader" mode. If the value of macHeaderManipulation field is 'NormalMacHeader', then data received on the E-DCH MACd flows supporting this logical channel shall have its MAC header inspected to determine the appropriate routing, and removed as normal. The MACis SDU shall be passed to the appropriate logical channel. If the value of macHeaderManipulation field is 'OmitMacHeader', then data received on the E-DCH MACd flows supporting this logical channel shall have it's MAC header inspected to determine the appropriate routing, then the MAC_is layer shall deliver the MAC-is SDU, LCH_ID, Length, F,SI and the related CFN, subframe number, HARQ process identity to the appropriate logical channel. The TTCN receives these fields by RLC_TR_MACis_DATA_IND, then these fields can be checked by the TTCN. HARQ ID = CFN mod 4 for TTI=10ms; HARQ ID = (5*CFN + subframe number) mod 8 for TTI=2ms |
| Type Definition |  |
| SEQUENCE { macHeaderManipulation MAC_HeaderManipulation, logicalChannelIdentity LogicalChannelIdentity, e_DCH_MAC_d_FlowIdentity E_DCH_MAC_d_FlowIdentity, rlc_PDU_Size CHOICE { flexibleSize RLC_PDU_SizeConstraint, fixedSize RLC_PDU_SizeList }, includeInSchedulingInfo BOOLEAN, mac_LogicalChannelPriority MAC_LogicalChannelPriority, logicalChannelType LogicalChannelType, rB_Identity INTEGER (-31..32) OPTIONAL } |  |

##### 7.3.2.2.17n	CMAC_MACi_AG (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_AG_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACi_AG_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_AG_REQ |
| PCO Type | CSAP |
| Comment | The hARQProcId shall be converted to the nearest CFN (and subframe number if TTI = 2 ms) by the SS, and the Absolute Grant is sent in that CFN (and subframe number if TTI = 2 ms). The absolute grant is applied on the A-GCH configured in the referred cellId. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63), grantType ENUMERATED {primary(0), secondary(1)}, absoluteGrantValue BIT STRING(SIZE(5)), absoluteGrantScope BIT STRING(SIZE(1)), hARQProcId INTEGER (0..7), activationTime SS_ActivationTime } |  |

##### 7.3.2.2.17o	CMAC_MACi_AckNack (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_AckNack_CNF |
| PCO Type | CSAP |
| Comment | To Confirm CMAC_i_AckNack_REQ |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_AckNack_REQ |
| PCO Type | CSAP |
| Comment | To request the SS to set operation mode of the Ack/Nack function for the HARQ process hARQProcId, this is applied on the HARQ process in the referred cell Id. The harqProcId, between 0 to 3 for 10 ms TTI or 0 to 7 for 2 msTTI, is individually applied to the configuration for the normal / nack mode. If the special hARQProcId -1 is used, all active HARQ processes ( 0..3 for 10 ms TTI and 0..7 for 2 ms TTI) will be configured according to ackNackFunction. At the SS initialization Ack/Nack function is in normal operation mode. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63), hARQProcId INTEGER (-1¦0..7), ackNackFunction AckNackFunction } |  |

##### 7.3.2.2.17p	CMAC_MACi_E_TFC_Restriction (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_E_TFC_Restriction_CNF |
| PCO Type | CSAP |
| Comment | For MAC emulator to report that a previous attempt of restricting TFCs have been successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_E_TFC_Restriction_REQ |
| PCO Type | CSAP |
| Comment | To request to configure MACi entity. The field restrictAllowedTFCs is provided to allow the E-TFCI to be restricted on the referred cell Id. The IE fullE_TFCS will be used to remove any previous E_TFCS restriction configured. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63), restrictAllowedTFCs CHOICE { e_TFCS_Restriction E_TFCS_Restriction, fullE_TFCS NULL } } |  |

##### 7.3.2.2.17q	CMAC_MACi_RG (Rel-8 or later)

NOTE:	These ASPs are currently not used in ATSs

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_RG_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CMAC_MACi_RG_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), } cellId INTEGER(0..63) |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_RG_REQ |
| PCO Type | CSAP |
| Comment | For non-serving RL the value for relativeGrant is limited to "down" and "hold". The SS shall convert the hARQProcId to the nearest CFN (and subframe number if TTI = 2 ms) by the SS and send the Relative Grant in that CFN (and subframe number if TTI = 2 ms). The relative grant is applied on the R-GCH configured in the referred cellId. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), cellId INTEGER(0..63), relativeGrant ENUMERATED{up(0), down(1), hold(2)}, hARQProcId INTEGER (0..7), activationTime SS_ActivationTime } |  |

##### 7.3.2.2.17r	Void

##### 7.3.2.2.17s	CMAC_MACis_SI_IND

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACis_SI_IND |
| PCO Type | CSAP |
| Comment | This ASP is used for MACis delivering scheduling information in MAC_is testing, cellId=-1. cellId is set to the received cell if the sI_ReportReceivedCellId is enabled in CMAC_MACis_SI_Config_REQ. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (-1..63), cfn INTEGER (0..255), subframe INTEGER (0..4¦7), -- 0..4 when TTI=2ms, 7 when TTI=10ms uePowerHeadRoom BIT STRING (SIZE(5)), totalE_DCHBufferStatus BIT STRING (SIZE(5)), highestPriorityLogChBS BIT STRING (SIZE(4)), highestPriorityLogChId BIT STRING (SIZE(4)) } |  |

##### 7.3.2.2.17t	CMAC_MACis_SI_Config (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACis_SI_Config_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later To Confirm CMAC_MACis_SI_Config_REQ, cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACis_SI_Config_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later To configure the SS to enable / disable to report the reception of Scheduling Information in MAC-is PDU's via primitive CMAC_MACis_SI_IND. At the SS initialization, the default mode is SI reporting disabled cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), sI_reportEnable ENUMERATED {disable(0), enable(1)} DEFAULT disable, sI_ReportReceivedCellId ENUMERATED {disable(0), enable(1)} DEFAULT disable -- Used in DC-HSUPA } |  |

##### 7.3.2.2.17u	CMAC_MBMS_ConfigInfo (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MBMS_ConfigInfo_CNF |
| PCO Type | CSAP |
| Comment | To confirm CMAC_MBMS_ConfigInfo_REQ. The routingInfo indicates the physical channel which carries logical channel of type: MCCH, MSCH. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MBMS_ConfigInfo_REQ |
| PCO Type | CSAP |
| Comment | To provide the SS MCCH or MSCH configuration information. The routingInfo indicates the physical channel which carries logical channel of type : MCCH or MSCH. This ASP shall be called after the ASP CMAC_Config_REQ used for MCCH or MSCH configuration. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, mCCH_ConfigInfo MBMS_MCCH_ConfigurationInfo_r6 OPTIONAL, mSCH_ConfigInfo MBMS_MSCH_ConfigurationInfo_r6 OPTIONAL } |  |

##### 7.3.2.2.18	CMAC_PAGING_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_PAGING_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm to setup the paging message |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_PAGING_Config_REQ |
| PCO Type | CSAP |
| Comment | To request MAC layer to send the Paging message on the specified configuration. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, configMessage CmacPagingConfigReq } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacPagingConfigReq |
| Comment | The IE t_pich_T_sccpch is obsolete for the purpose of the UE conformance test in all Releases. The timing relation of PICH / S-CCPCH and PICH/HS-SCCH are specified in 3GPP TS 25.211 [40], subclauses 7.1, 7.2 and 7.2A. A desired clean correction is to remove this IE. However, for the backwards /forwards compatibility, the proposed solution is to set this IE always to FALSE. The SS can ignore this IE, but shall behave according to the corresponding core spec. |
| Type Definition |  |
| SEQUENCE { pI_BitMapInfo CHOICE { e18 BIT STRING (SIZE (18)), e36 BIT STRING (SIZE (36)), e72 BIT STRING (SIZE (72)), e144 BIT STRING (SIZE (144)) }, dRX_CycleLength INTEGER {3..9}, iMSI SEQUENCE (SIZE (6..15)) OF Digit, t_pich_T_sccpch BOOLEAN -- T_pich>T_sccpch then FALSE } |  |

##### 7.3.2.2.19	CMAC_Restriction

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Restriction_CNF |
| PCO Type | CSAP |
| Comment | For MAC emulator to report that a previous attempt of restricting TFCs have been successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Restriction_REQ |
| PCO Type | CSAP |
| Comment | To request to configure MAC entity. The field restrictAllowedTFCs is provided to allow the UL and/or DL SS TFCS to be restricted for a specific transport channel. This information only needs to be sent to the MAC layer, since it is the MAC layer's responsibility to determine the set of valid TFCs each TTI. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (-1..63), routingInfo RoutingInfo, ratType RatType, restrictAllowedTFCs TFC_Restriction } |  |

| ASN.1 Type Definition |  |  |
| --- | --- | --- |
| Type Name |  | TFC_Restriction |
| Comment |  | This type is used to specify the allowed TFCs within the current TFCS. A TFC restriction is applicable until a subsequent TFC restriction is applied. TFC restrictions are not cumulative, so each TFC restriction completely replaces the previous TFC restriction. The downlink restriction can be used to ensure that the SS uses a specific TFC for transmission of data, by only allowing the 'No data' TFC, and the 'desired' TFC. It may also be necessary to include one or more 'signalling only' TFCs to allow signalling to occur. The uplink restriction can be used to verify that the UE has used a specific TFC. Any data received by the SS using a forbidden TFCI shall be discarded. |
| Type Definition |  |  |
| SEQUENCE { ulTFCI_Restriction TFC_Subset OPTIONAL, dlTFCI_Restriction TFC_Subset OPTIONAL } |  |  |
| Detailed Comments | SS requirements for downlink. - The SS MAC layer shall not use a restrictednon-allowed TFC for DL. - The SS MAC layer shall not use a TFC that requires the SS RLC layer to provide padding PDUs (3GPP TS 25.322 [18]) - In the case that there is data pending on one or more RLC entities, but not enough to use one of the allowed TFCs: - The SS MAC layer shall use the 'No data' TFC until there is enough data in the RLC to use another allowed TFC. - The SS RLC layer shall buffer the data until there is enough data in the RLC entities for the MAC layer to use an allowed TFC other than the 'No data' TFC for transmission of the data. NOTE: The TTCN author is responsible for ensuring: - The SDU discard function is not configured for TM and UM entities in the UE, and is configured to no_discard for AM entities in the UE. - That RLC SDUs that are expected to be sent in the same TTI (due to a TFC restriction) are sent as quickly as possible to minimize the number of 'no data' TFCs used by the MAC layer, and the amount of buffering that must be performed by the RLC layer. SS requirements for uplink: The SS shall discard all data received using a restricted non-allowed TFC. |  |

##### 7.3.2.2.20	CMAC_SecurityMode_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_SecurityMode_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm to configure the MAC security mode |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_SecurityMode_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to configure the MAC security mode. If there are several CMAC_Ciphering_Activate_REQ follow this ASP, the SS shall take a serial of specified actions on the same contents in this ASP at the activation time indicated in each CMAC_Ciphering_Activate_REQ. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), macCipheringInfo SecurityInfo } |  |

##### 7.3.2.2.21	CMAC_SequenceNumber

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Sequence_Number_CNF |
| PCO Type | CSAP |
| Comment | To return the requested counter sequence number on MAC-d DCH. The physicalChannelIdentity of DPCH applies to routingInfo. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, count_C_MSB_UL COUNT_C_MSB , count_C_MSB_DL COUNT_C_MSB } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_SequenceNumber_REQ |
| PCO Type | CSAP |
| Comment | To request the MAC layer to return current counter sequence numbers. The physicalChannelIdentity of DPCH applies to routingInfo. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.22	CMAC_SYSINFO_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_SYSINFO_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm to setup the system information block |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_SYSINFO_Config_REQ |
| PCO Type | CSAP |
| Comment | To request MAC layer to send the BCCH message on the specified configuration. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, ratType RatType, configMessage CmacSysinfoConfigReq } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacSysinfoConfigReq |
| Comment |  |
| Type Definition |  |
| SEQUENCE { sg_REP INTEGER (2..12), -- Repetition period is the sg_REP-th power of 2. sg_POS INTEGER (0..2047), -- The position of each segment is 2 * sg_POS. bcch_ModificationTime BCCH_ModificationTime OPTIONAL } |  |

##### 7.3.2.2.22a	CRLC_Bind_TestData_TTI

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Bind_TestData_TTI_CNF |
| PCO Type | CSAP |
| Comment | To confirm the request of binding subsequent data sending RLC_TR_TestDataReq on the different DL RBs in the same TTI. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), result ENUMERATED{failure(0), success(1)} } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Bind_TestData_TTI_REQ |
| PCO Type | CSAP |
| Comment | To request binding subsequent data sending RLC_TR_TestDataReq on the different DL RBs in the same TTI. On the request, the transmission of the test data is temporarily suppressed on those radio bearers which follow subsequently this CRLC_Bind_TestData_TTI_REQ and have 'numOfDiffRb' different RB IDs. Having received the number 'numOfDiffRb' of RLC_TR_TestDataReq, the SS RLC sends the test data on those RBs in the same TTI according to the allowed DL TFCS. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), numOfDiffRb INTEGER(2..6) -- Number of different RB IDs } |  |

##### 7.3.2.2.22b	CRLC_BindTestDataInOneMAChs_MACehs_PDU (Rel-5 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInOneMAChs_MACehs_PDU_CNF |
| PCO Type | CSAP |
| Comment | To confirm the request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH in the same MAC-hs/MAC-ehs PDU. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(-1..63), routingInfo RoutingInfo, -- RB ID desired to be given result ENUMERATED{failure(0), success(1)} } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInOneMAChs_MACehs_PDU_REQ |
| PCO Type | CSAP |
| Comment | To request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH in the same MAC-hs/MAC-ehs PDU. On the request, the transmission of the test data is temporarily suppressed on the radio bearers till 'numOfSDU's' are received by RLC layer on the Radio Bearer. After receiving all SDU's the RLC layer submits to MAC such that all of them are sent in one MAC-hs/ MAC-ehs PDU. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(-1..63), routingInfo RoutingInfo, -- RB ID desired to be given numOfSDUs INTEGER -- Number of RLC SDU's } |  |

##### 7.3.2.2.22c	CRLC_BindTestDataInMultipleMACehs_PDU_MultiFlows (Rel-7 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInMultipleMACehs_PDU_MultiFlows_CNF |
| PCO Type | CSAP |
| Comment | To confirm the request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(-1..63), routingInfo RoutingInfo, -- RB ID desired to be given result ENUMERATED{failure(0), success(1)} } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInMultipleMACehs_PDU_MultiFlows_REQ |
| PCO Type | CSAP |
| Comment | To request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH in two MAC-ehs PDUs, one for each primary and secondary flow. On the request, the transmission of the test data is temporarily suppressed on the radio bearer till all SDUs for all flows are received by RLC layer on the Radio Bearer. After receiving all SDU's the RLC layer submits to MAC such that all of them are sent in two MAC-ehs PDUs. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(-1..63), routingInfo RoutingInfo, -- RB ID desired to be given primaryFlowNumOfSDUs INTEGER, -- Number of RLC SDU's for primary flow of serving cell secondaryFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for secondary flow of serving cell secPrimaryFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for secondary cell primary flow Rel-9 or later secSecondaryFlowNumOfSDUs INTEGER OPTIONAL, -- Number of RLC SDU's for secondary cell secondary flow Rel-9 or later relAspTypeExtension BindTestDataInMultipleMACehs_r10OrLaterExtensionType OPTIONAL -- Rel-10 or later } |  |

##### 7.3.2.2.23	CRLC_Ciphering_Activate

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Ciphering_Activate_CNF |
| PCO Type | CSAP |
| Comment | To confirm to activate or inactivate the ciphering |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Ciphering_Activate_REQ |
| PCO Type | CSAP |
| Comment | To request to start orrestart downlink ciphering or uplink deciphering. Each call of the ASP includes one RLC SN in rb-DL-CiphActivationTimeInfo for the corresponding rb-identity. Initialize the 20 MSB of HFN component of COUNT-C to the START value stored. For RLC_UM COUNT-C: - If the value of incHFN is set to "NotInc" the SS initialiszes the remaining LSBs of HFN component in UM COUNT-C to zero. - If the value of incHFN is set to "Inc" the SS initializes the remaining LSBs of HFN component in UM COUNT-C to zero, then increments the HFN by one. For RLC_AM COUNT-C: - If the value of incHFN is set to "NotInc" no further action is needed. - If the value of incHFN is set to "Inc" the SS increments the HFN by one. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), ratType RatType, cn_DomainIdentity CN_DomainIdentity, ciphActivationInfo CiphActivationInfo, incHFN RLC_IncMode } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CiphActivationInfo |
| Comment | DL or UL ciphering activation info If RB is omitted in rB_UL_CiphActivationTimeInfo the SS takes no action on this RB and the ciphering configuration keeps unchanged on this RB. CipheringModeCommand = dummy NULL means no ciphering. |
| Type Definition |  |
| CHOICE { cipheringModeInfo CipheringModeInfo, rb_UL_CipheringActivationTimeInfo RB_ActivationTimeInfoList, cipheringModeInfo_r7 CipheringModeInfo_r7 -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RLC_IncMode |
| Comment |  |
| Type Definition |  |
| ENUMERATED{notInc(0), inc(1)} |  |

##### 7.3.2.2.24	CRLC_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Config_CNF |
| PCO Type | CSAP |
| Comment | For RLC emulator to confirm that a previous attempt to establish, re_configure or release a radio bearer has been successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to setup, reconfigure or release RLC entity |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, ratType RatType, configMessage CrlcConfigReq, activationTime SS_ActivationTime OPTIONAL -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CrlcConfigReq |
| Comment | To request to setup, re_configure release RLC entity The Stop parameter indicates that the RLC entity shall not transmit or receive RLC PDUs. The Continue parameter indicates that the RLC entity shall continue transmission and reception of RLC PDUs. When the RLC entity is stopped, the all protocol parameters, such as the protocol variables, RLC timers and status are not affected. Triggered polls and status transmissions are delayed until the RLC entity is continued. |
| Type Definition |  |
| CHOICE { setup RBInfo, reconfigure RBInfo, release NULL, sS_stop NULL, sS_continue NULL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RBInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { sS_rlc_Info SS_RLC_Info OPTIONAL, sS_rlc_Info_r8 SS_RLC_Info_r8OrLater OPTIONAL, -- Rel-8 or Later rB_LogCH_Mapping RB_LogCH_Mapping relAspTypeExtension SEQUENCE{} OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RB_LogCH_Mapping |
| Comment | Provide mapping information between RB, logical channel and CN domain. When the logical channel is MTCH, the logicalChannelIdentity shall be consistent with MBMS_LogicalChIdentity in MBMS_PTM_RBInformation_N and MBMS_PTM_RBInformation_C being sent to the UE. In rel-11 Multiflow inter-NodeB configuration RB is mapped to logical channel on two MAC-ehs entities. |
| Type Definition |  |
| SEQUENCE { uLlogicalChannelIdentity LogicalChannelIdentity OPTIONAL, dLlogicalChannelIdentity LogicalChannelIdentity OPTIONAL, logicalChannelType LogicalChannelType OPTIONAL, cn_DomainIdentity CN_DomainIdentity OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_RLC_Info |
| Comment | UL and DL have been swapped intentionally in this type definition. This is to maximize re-use of the type definitions in 3GPP TS 25.331 [21] which are intended to configure a UE, where UL is transmission, and DL is reception. For the SS, UL is reception, and DL is transmission. For example, consider configuring a DL AM RLC entity (transmitter) in the SS. The transmission parameters to be configured include PollingInformation, Transmission-RLC-Discard etc. If the DL-AM-RLC-Mode type definition is used to configure this entity, it is only possible to configure reception parameters such as StatusInformation, and receiving window size. By swapping UL and DL, it is possible to configure the DL AM RLC entity using the existing type definition UL-AM-RLC-Info, which contains all of the required transmission parameters. When uM_SN_DeliveryMode is set to configured, the RLC entity does not concatenate nor segment RLC SDUs (see 3GPP TS 25.322 [18], subclause 8.2). If the IE useSpecialValueOfHEField is set to true, the last octet of the PDU is the last octet of an SDU and there is no SDU concatenation inside the PDU. Applicable for Rel-99 to Rel-7 |
| Type Definition |  |
| SEQUENCE { sS_ul_RLC_Mode DL_RLC_Mode OPTIONAL, sS_dl_RLC_Mode SS_DL_RLC_Mode OPTIONAL, rlc_OneSidedReEst BOOLEAN DEFAULT FALSE, altE_bitInterpretation ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for UM RLC mode of Rel-7 or later useSpecialValueOfHEField ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for AM RLC mode of Rel-7 or later uM_SN_DeliveryMode ENUMERATED { nonConfigured(0), configured(1)} DEFAULT nonConfigured -- applicable for UM RLC mode of Rel-7 or later } |  |

| Type Name | SS_RLC_Info_r8OrLater |
| --- | --- |
| Comment | Applicable for Rel-8 or Later. UL and DL have been swapped intentionally in this type definition. This is to maximize re-use of the type definitions in 3GPP TS 25.331 [21], which are intended to configure a UE, where UL is transmission, and DL is reception. For the SS, UL is reception, and DL is transmission. For example, consider configuring a DL AM RLC entity (transmitter) in the SS. The transmission parameters to be configured include PollingInformation, Transmission-RLC-Discard etc. If the DL-AM-RLC-Mode type definition is used to configure this entity, it is only possible to configure reception parameters such as StatusInformation, and receiving window size. By swapping UL and DL, it is possible to configure the DL AM RLC entity using the existing type definition UL-AM-RLC-Info, which contains all of the required transmission parameters. When uM_SN_DeliveryMode is set to configured, the RLC entity does not concatenate nor segment RLC SDUs (see 3GPP TS 25.322 [18], subclause 8.2). If the IE useSpecialValueOfHEField is set to true, the last octet of the PDU is the last octet of an SDU and there is no SDU concatenation inside the PDU. |
| Type Definition |  |
| CHOICE { r8 SEQUENCE { sS_ul_RLC_Mode DL_RLC_Mode_r7 OPTIONAL, sS_dl_RLC_Mode SS_DL_RLC_Mode OPTIONAL, rlc_OneSidedReEst BOOLEAN DEFAULT FALSE, altE_bitInterpretation ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for UM RLC mode useSpecialValueOfHEField ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for AM RLC mode uM_SN_DeliveryMode ENUMERATED { nonConfigured(0), configured(1)} DEFAULT nonConfigured -- applicable for UM RLC mode of Rel-7 or later }, relAspTypeExtension SS_RLC_Info_r10OrLaterExtensionType -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DL_RLC_Mode |
| Comment | "dl_UM_outOfSeqDelivery" is present only for the DL_RLC entity connected to MCCH, and in the configuration with dl_UM_outOfSeqDelivery present the UM RLC can transmit RLC PDU containing SDU of ACCESS INFORMATION message out of sequence when it is necessary Maximum one among dl_RLC_PDU_size & dl_PayloadSize shall be included. For RLC UM configuration, with altE_bitInterpretation set to TRUE, neither dl_PayloadSize nor dl_RLC_PDU_size can be present. |
| Type Definition |  |
| SEQUENCE { dl_PayloadSize PayloadSize OPTIONAL, dl_RLCModeInfo UL_RLC_Mode, dl_UM_RLC_LI_size DL_UM_RLC_LI_size OPTIONAL, -- only for UM RLC configuration of Rel-5 or later dl_UM_outOfSeqDelivery UM_RLC_OutOSeqDelivery_Info_r6 OPTIONAL, –- Rel-6 or later dl_RLC_PDU_size CHOICE { fixedSize OctetModeRLC_SizeInfoType1, flexibleSize SS_FlexibleSize } OPTIONAL -- Only for AM RLC Configuration of Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PayloadSize |
| Comment |  |
| Type Definition |  |
| INTEGER (0..4992) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_FlexibleSize |
| Comment | Rel-7 or later If max_RLC_DataField_Size is present, SS shall guarantee that the size of data field of DL RLC PDU does not exceed the size specified in Bytes. |
| Type Definition |  |
| SEQUENCE { li_Size ENUMERATED { size7 (0), size15 (1)}, -- Size of length indicator max_RLC_DataField_Size INTEGER (1..1503) OPTIONAL - Maximum size of data field of RLC PDU (25.322, 9.2.2.9 and 25.433, 9.2.1.38c) } |  |

##### 7.3.2.2.25	CRLC_Integrity_Activate

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_integrity_Activate_CNF |
| PCO Type | CSAP |
| Comment | To confirm to activate or inactivate the integrity protection |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Integrity_Activate_REQ |
| PCO Type | CSAP |
| Comment | To request to start or to modify the downlink or uplink integrity protection. The ASP shall be called before send SECURITY MODE COMMAND. It activates the integrity on all SRBs in DL. The SS initializes the 20 MSB of HFN component of COUNT-I to the START value stored and set the remaining LSBs of HFN component in COUNT-I to zero. If integrityModeCommand in ASP is set to "startIntegrityProtection", the SS shall start the downlink integrity protection from the first downlink RRC message. If the integrityModeCommand in ASP is set to "modify", the SS shall start the downlink integrity protection at the RRC message sequence number specified in "dl_IntegrityProtActivationInfo". |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), cn_DomainIdentity CN_DomainIdentity, integrityActivationInfo IntegrityActivationInfo } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | IntegrityActivationInfo |
| Comment | DL or UL integrity activation info At the RRC message sequence numbers specified in the ul_IntegProtActivationInfo the SS shall initialize COUNT-I for the SRB's indicated in the ul_IntegrityProtActivationInfo and start using the new configuration on uplink for the indicated SRB's. If the START value is omitted in the CRLC_SecurityMode_Config_REQ above COUNT-I initialization shall not be performed. |
| Type Definition |  |
| CHOICE { integrityProtectionModeInfo IntegrityProtectionModeInfo, ul-IntegProtActivationInfo IntegrityProtActivationInfoList, integrityProtectionModeInfo_r7 IntegrityProtectionModeInfo_r7 -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | IntegrityProtActivationInfoList |
| Comment | List of SS IntegrityProtActivationInfo |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxRB ) ) OF SS_IntegrityProtActivationTimeInfo |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_IntegrityProtActivationTimeInfo |
| Comment | Omitting rrc_MessageSequenceNumber means activation time set to "now". |
| Type Definition |  |
| SEQUENCE { rb_Identity INTEGER (-31..32), rrc_MessageSequenceNumber RRC_MessageSequenceNumber OPTIONAL } |  |

##### 7.3.2.2.26	CRLC_Integrity_Failure

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Integrity_Failure_IND |
| PCO Type | CSAP |
| Comment | RLC emulator reports the occurrences of a failure in integrity protection, i.e. reception of an integrity-protected RLC AM/UM SDU containing a non-matching X-MAC value compared to the desired. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, failureCause ENUMERATED { codeNotMatched(0) } -- the enumerated types of failure cause field is ffs } |  |

##### 7.3.2.2.26a	CRLC_MAC_I_Mode

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_MAC_I_Mode_CNF |
| PCO Type | CSAP |
| Comment | Confirm a previous CRLC_MAC_I_Mode_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), srbId INTEGER(0..4) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_MAC_I_Mode_REQ |
| PCO Type | CSAP |
| Comment | To set the MAC-I calculation mode. The ASP does not affect the UL integrity calculation. If mode = normal, the SS generates the correct MAC-I. If mode = erroneous, the SS generates any wrong MAC-I value different from the one it shall be. As default, when the integrity protection is jswitched on the SS enters the normal MAC-I calculation mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), srbId INTEGER (0..4), mode ENUMERATED {normal(0), erroneous(1)} } |  |

##### 7.3.2.2.26b	CRLC_NotAckNxtRxSDU

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_NotAckNxtRxSDU_CNF |
| PCO Type | CSAP |
| Comment | To confirm that the next received SDU has not been acknowledged. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_NotAckNxtRxSDU_REQ |
| PCO Type | CSAP |
| Comment | To request that the next received SDU is not acknowledged. The received SDU is passed to the upper layers. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, mode ENUMERATED{start(0)} } |  |

##### 7.3.2.2.26c	CRLC_ProhibitRLC_Ack

The use of the pair of ASPs should be restricted to each start of  SRB3 Uplink ciphering only. The SS behaviours are not specified if the ASPs are used in any other procedures.

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_ProhibitRLC_Ack_CNF |
| PCO Type | CSAP |
| Comment | To confirm that the reception of a CRLC_ProhibitRLC_Ack_REQ. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, supportFlag SupportFlag DEFAULT noNeed } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_ProhibitRLC_Ack_REQ |
| PCO Type | CSAP |
| Comment | To request the SS to prohibit/Continue acknowledging RLC SDUs. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, mode ENUMERATED{prohibit(0), continue(1)} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SupportFlag |
| Comment | The default value noNeed indicates that the SS does not perform the operation mentioned in CRLC_ProhibitRLC_Ack_REQ, but performs the suspension / resume of UL RLC PDU data. If the non default values are taken, the SS has either prohibited, or continued acknowledging RLC SDUs. |
| Type Definition |  |
| ENUMERATED {ackProhibited(0), ackContinued(1), noNeed (2)} |  |

##### 7.3.2.2.26d	CRLC_ReportDataReceivedCellId (Rel-9 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_ReportReceivedCellId_CNF |
| PCO Type | CSAP |
| Comment | To confirm to activate or deactivate the received data cellId. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_ReportReceivedCellId_ REQ |
| PCO Type | CSAP |
| Comment | Rel-9 or later. To request to start or stop reporting the received cellId in the test data ASPs: RLC_AM_TestDataInd, RLC_UM_TestDataInd and RLC_TR_TestDataInd. When activated, the IE cellId in the test data ASPs indicates the physical cellId on which the data is received, i.e. cellId=-1 is not used. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, reportReceivedCellId ENUMERATED { activate(0), deactivate(1)} } |  |

##### 7.3.2.2.27	CRLC_Resume

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Resume_CNF |
| PCO Type | CSAP |
| Comment | To confirm the resume request |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Resume_REQ |
| PCO Type | CSAP |
| Comment | To request to resume data transmission. If the SS implemented the optional suspension of UL data PDUs, then the processing in the UL of data PDUs shall be resumed. Any suspended UL control PDUs and Piggybacked Status shall be preceded or resumed. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.27a	CRLC_RRC_MessageSN

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_RRC_MessageSN_CNF |
| PCO Type | CSAP |
| Comment | To return the counter I values (HFN and RRC message sequence number) for sending the next DL RRC message or for receiving the next UL RRC message on the concerned SRB. COUNT_I_MSB is the 28 MSB of the COUNT-I (HFN) |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, count_I_MSB_UL COUNT_I_MSB, count_I_LSB_UL RRC_SequenceNumber, count_I_MSB_DL COUNT_I_MSB, count_I_LSB_DL RRC_SequenceNumber } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | COUNT_I_MSB |
| Comment | 28 bits long |
| Type Definition |  |
| INTEGER (0..268435455) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RRC_SequenceNumber |
| Comment | 4 bits long |
| Type Definition |  |
| INTEGER (0..15) |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_RRC_MessageSN_REQ |
| PCO Type | CSAP |
| Comment | To request the SS to return the values in COUNT-I for sending the next DL RRC message or for receiving the next UL RRC message on the concerned SRB. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.28	CRLC_SecurityMode_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SecurityMode_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm to configure the RLC security mode If several subsequent CRLC_Integrity_Activate_REQ or CRLC_Ciphering_Activate_REQ follow this ASP, the SS shall take a serial of specified actions on the same contents in this ASP at the activation time indicated in each CRLC_Integrity (or Ciphering)_Activate_REQ. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SecurityMode_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to configure the RLC security mode |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), rlcSecurityInfo SecurityInfo } |  |

| ASN.1 Type Definition |  |  |
| --- | --- | --- |
| Type Name |  | SecurityInfo |
| Comment |  | The integrityKey is not applicable to MAC |
| Type Definition |  |  |
| SEQUENCE{ Cn_DomainIdentity CN_DomainIdentity, startValue START_VALUE OPTIONAL, cipheringKey BITSTRING(128) OPTIONAL, integrityKey BITSTRING(128) OPTIONAL, gsmCipheringKey BITSTRING(64) OPTIONAL } |  |  |
| Detailed Comments | When the SS receives SecurityInfo, the SS first stores the contents. The SecurityInfo contents is not activated until receiving the subsequent ASP, CRLC_Ciphering_Activate_REQ, CMAC_Ciphering_Activate_REQ or CRLC_Integrity_Activate_REQ. Omitted fields of SecurityInfo shall not be affected by the subsequent ASP at the activation time. EXAMPLE: Omitting of startValue indicates not to re-initialize the relevant COUNT-C or COUNT-I, omitting of cipheringKey indicates that the current ciphering key is valid. |  |

##### 7.3.2.2.28a	CRLC_SetRRC_MessageSN

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SetRRC_MessageSN_CNF |
| PCO Type | CSAP |
| Comment | To confirm the RRC message sequence number setting request |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SetRRC_MessageSN_REQ |
| PCO Type | CSAP |
| Comment | To request the SS to set the RRC message sequence number in COUNT-I to the value specified in this ASP. The ASP is used to initialize SS RRC SN. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, count_I_LSB_UL RRC_SequenceNumber OPTIONAL, count_I_LSB_DL RRC_SequenceNumber OPTIONAL } |  |

##### 7.3.2.2.28b	CRLC_Set_Count_I

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Set_Count_I_CNF |
| PCO Type | CSAP |
| Comment | To confirm the count_I_MSB and the RRC message sequence number setting request |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Set_Count_I_REQ |
| PCO Type | CSAP |
| Comment | To request the SS to set the 28 MSB and 4 LSB (RRC message sequence number) in COUNT-I according to the parameter values specified in this ASP. Parameters omitted in this ASP shall leave the corresponding bits in the SS COUNT-I unchanged. Typically the parameters count_I_MSB_UL and count_I_MSB_DL are omitted. They are only applied in a few specific security test cases requiring restoration of the used integrity context. NOTE: The 28 MSBs are initialized with the UE-provided START value plus 8 bits set to 0, using a different ASP (CRLC_SecurityMode_Config_REQ). |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, count_I_LSB_UL RRC_SequenceNumber OPTIONAL, count_I_LSB_DL RRC_SequenceNumber OPTIONAL, count_I_MSB_UL COUNT_I_MSB OPTIONAL, count_I_MSB_DL COUNT_I_MSB OPTIONAL } |  |

.

##### 7.3.2.2.29	CRLC_SequenceNumber

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Sequence_Number_CNF |
| PCO Type | CSAP |
| Comment | To return the requested counter sequence number to which the next DL PDU to be sent or the expected UL PDU to be received. The length of count_C_MSB_UL/DL and count_C_LSB_UL/DL are according to the long and short sequence number in 3GPP TS 33.102 [25], subclause 6.6.4.1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, count_C_MSB_UL COUNT_C_MSB, count_C_LSB_UL RLC_SequenceNumber, count_C_MSB_DL COUNT_C_MSB, count_C_LSB_DL RLC_SequenceNumber } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SequenceNumber_REQ |
| PCO Type | CSAP |
| Comment | To request the RLC layer to return current counter sequence numbers to which the next DL PDU to be sent or the expected UL PDU to be received. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

##### 7.3.2.2.29a	CRLC_SendContinuousData_TTI

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SendContinuousData_CNF |
| PCO Type | CSAP |
| Comment | Confirm sending data in every TTI on each requested RB |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), result ENUMERATED{failure(0), success(1)} } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_SendContinuousData_REQ |
| PCO Type | CSAP |
| Comment | To request sending data in every TTI on each RB identified. After the CMAC_Restriction_REQ, the TFC under test will be the one corresponding to the maximum CTFC value in the Restricted list, so that SS can select the number of Transport blocks and the size of Transport blocks on individual Transport channels derived from this CTFC. SS shall take care about all kind of discard info in all RLC modes and the final goal is that the DL TFCs under test shall be selected in downlink for sending data on the request RBs in each TTI. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), rabTxInfo RabTxInfo } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RabTxInfo |
| Comment | Provide test data, number of RBs, and RB Tx info of each RB (RB id, SDU size and number of SDUs) to be transmitted in consecutive TTIs |
| Type Definition |  |
| SEQUENCE { testData BIT STRING (SIZE (8..163840)), rbTxInfoList SEQUENCE (SIZE (1..6)) OF RbTxInfo } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RbTxInfo |
| Comment | Info on RB id and the actual DL test data size (SDU_Size * number of SDUs). The actual test data is extracted from the first (SDU_Size * number of SDUs) bits in the raw testData buffer. SS shall transmit the actual test data in every TTI. The value nomOfSdu = T / TTI , whereby T=1200 is the duration of the data transmitting in the RAB test, taking into account the test tolerance (+50 %) of the UE loop back delay (< 800 ms). |
| Type Definition |  |
| SEQUENCE { rB_Identity INTEGER (-31..32), sduSize INTEGER (1..163840), nomOfSdu INTEGER (0..255) -- 0 is set for no data on this RB } |  |

##### 7.3.2.2.30	CRLC_Status

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Status_IND |
| PCO Type | CSAP |
| Comment | To report the occurrence of certain events to RRC. NOTE: The possible event types to be defined for this ASP is FFS. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, ratType RatType, statusInd CrlcStatusInd } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CrlcStatusInd |
| Comment |  |
| Type Definition |  |
| ENUMERATED { dataLinkFailure (0) maxRESET (1), sDUDiscarded (2) -- More event types are to be added here } |  |

##### 7.3.2.2.31	CRLC_Suspend

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Suspend_CNF |
| PCO Type | CSAP |
| Comment | To confirm the suspension of data transmission. The parameter vt indicates either the value of the Send State Variable VT(S) for AM, or the value of Data State Varialble VT(US) for UM. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, vt RLC_SequenceNumber } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_Suspend_REQ |
| PCO Type | CSAP |
| Comment | To request the suspension of data transmission. The parameter n indicates that an RLC entity will not send a PDU with "Sequence Number"VT(S)+N for AM and "Sequence Number"VT(US)+N for UM, where N is a non-negative integer. Optionally an SS may start immediate suspension of processing of data PDUs in the UL. The UL control PDUs and Piggybacked Status may optionally be processed. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, n RLC_SequenceNumber } |  |

##### 7.3.2.2.31a	CRLC_MTCH_Scheduling (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_MTCH_Scheduling_CNF |
| PCO Type | CSAP |
| Comment | To confirm the CRLC_MTCH_Scheduling_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_MTCH_Scheduling_REQ |
| PCO Type | CSAP |
| Comment | Applied to the RLC entity carrying MTCH. MBMS serviceSchedulingInfo can contain a list of MBMS ServiceSchedulingInfo for multiple consecutive scheduling periods of discountinous MBMS services. mSCH_REPconfiguration provides the timing of scheduling periods. serviceShedulingInfos provides a list of SS_ServiceSchedulingInfo corresponding to multiple scheduling periods. On or after the start and within the duration of a MBMS session, the RLC behaves as normal entity. Outside of these ranges the RLC regards the buffer occupancy as being zero, prohibiting the MAC from requesting PDU's. The same is valid for the service silence period (noServcieData). Each call of the ASP replaces the existing whole scheduling information list or creates a new scheduling information list if the list does not exist. The absence of IE mSCH_REPconfiguration and schedulingInfoInfos indicates continuous MBMS services. The SS shall delete the existing scheduling information list if it has existed. The RLC entity behaves as normal. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, mSCH_REPconfiguration MSCH_REPconfiguration OPTIONAL, serviceShedulingInfos ServiceSchedulingInfoList OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ServiceSchedulingInfoList |
| Comment | Multiple ServiceSchedulingInfo can be submitted to the SS. Each ServiceSchedulingInfo corresponds to a MSCH scheduling period. |
| Type Definition |  |
| SEQUENCE (SIZE(1.. MaxNumMSCHMsgs))OF SS_ServiceSchedulingInfo |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_ServiceSchedulingInfo |
| Comment | The IE is applied to the discontinuous MBMS service and contains pairs of "start" and "duration" within a scheduling period. The start value indicates the start of the service transmission in number of 4-frames relative to: either the 1st TTI on which the MBMS SCHEDULING INFORMATION message of the corresponding scheduling period is sent if MSCH is configured; or the IE scheduledSFN value in MSCH_REPconfiguration if MSCH is not configured. The duration value indicates how long the service is transmitted in unit of 4-frames. noServiceData is applied to the scheduling period when no MBMS service data are sent on that MTCH. |
| Type Definition |  |
| CHOICE { mbms_ServiceTransmInfoList MBMS_ServiceTransmInfoList, noServiceData NULL } |  |

##### 7.3.2.2.32	CBMC_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CBMC_Config_CNF |
| PCO Type | CSAP |
| Comment | To confirm the BMC configuration, reconfiguration or release. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo -- RBid } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CBMC_Config_REQ |
| PCO Type | CSAP |
| Comment | To request the configuration, reconfiguration or release of BMC. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, -- RBid configMessage CHOICE { setup BMC_SchedulingInfo, release NULL} } |  |

##### 7.3.2.2.32b	DEC_PERbitstring

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | DEC_PERbitstring_CNF |
| PCO Type | ExternalAsn1Codec |
| Comment | To receive the decoded BIT STRING. |
| Type Definition |  |
| SEQUENCE { containedType ContainedType } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | DEC_PERbitstring_REQ |
| PCO Type | ExternalAsn1Codec |
| Comment | To request decoding of the BITSTRING recived from UE in receivedBITSTRING with the type specified in containingType. |
| Type Definition |  |
| SEQUENCE { receivedBITSTRING BIT STRING, containingType ContainingPERbitstringType } |  |

| ASN.1 PDU Type Definition |  |
| --- | --- |
| Type Name | ContainedType |
| Comment |  |
| Type Definition |  |
| CHOICE { ue_CapabilityContainer_IEs UE_CapabilityContainer_IEs, rrcConnectionSetupComplete_r3_add_ext_IEs RRCConnectionSetupComplete_r3_add_ext_IEs, ueCapabilityInformation_r3_add_ext_IEs UECapabilityInformation_r3_add_ext_IEs, interRATHandoverInfo_r3_add_ext InterRATHandoverInfo_r3_add_ext_IEs, interRATHandoverInfo InterRATHandoverInfo, uE_RadioAccessCapabilityInfo UE_RadioAccessCapabilityInfo } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ContainingPERbitstringType |
| Comment |  |
| Type Definition |  |
| ENUMERATED { ue_CapabilityContainer_IEs (0), rrcConnectionSetupComplete_r3_add_ext_IEs (1), ueCapabilityInformation_r3_add_ext_IEs (2), interRATHandoverInfo_r3_add_ext_IEs (3), interRATHandoverInfo (4), uE_RadioAccessCapabilityInfo (5) } |  |

##### 7.3.2.2.32c	ENC_PERbitstring

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | ENC_PERbitstring_CNF |
| PCO Type | ExternalAsn1Codec |
| Comment | To receive the encoded BIT STRING. |
| Type Definition |  |
| SEQUENCE { encodedBITSTRING BIT STRING } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | ENC_PERbitstring_REQ |
| PCO Type | ExternalAsn1Codec |
| Comment | To request encoding of asn.1 PDU or IE. |
| Type Definition |  |
| CHOICE { mcchMessage MCCH_Message, radioBearerSetup_r7_add_ext RadioBearerSetup_r7_add_ext_IEs } |  |

##### 7.3.2.2.33	RLC_TR_DATA

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_DATA_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit DATA using transparent mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, tM_Message CHOICE { dL_DCCH_Message DL_DCCH_Message, dL_CCCH_Message DL_CCCH_Message, pCCH_Message PCCH_Message, dL_SHCCH_Message DL_SHCCH_Message, bCCH_FACH_Message BCCH_FACH_Message, bCCH_BCH_Message BCCH_BCH_Message } } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_DATA_IND |
| PCO Type | DSAP |
| Comment | To indicate to receive DATA using transparent mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, tM_Message CHOICE { uL_DCCH_Message UL_DCCH_Message, uL_CCCH_Message UL_CCCH_Message, uL_SHCCH_Message UL_SHCCH_Message} } |  |

##### 7.3.2.2.34	RLC_AM_DATA

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_AM_DATA_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit DATA using acknowledged mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (-1..63), routingInfo RoutingInfo, confirmationRequest AmConfirmationRequest, aM_Message CHOICE { dL_DCCH_Message DL_DCCH_Message, dL_CCCH_Message DL_CCCH_Message, pCCH_Message PCCH_Message, dL_SHCCH_Message DL_SHCCH_Message, bCCH_FACH_Message BCCH_FACH_Message, bCCH_BCH_Message BCCH_BCH_Message, invalid_dL_DCCH_Message Invalid_DL_DCCH_Message} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AmConfirmationRequest |
| Comment | If the noConfirmationRequested option is used, then an RLC_AM_DATA_CNF is not expected from the RLC AM entity. If the confirmationRequested option is used, then the RLC AM entity is being requested to provide an RLC_AM_DATA_CNF primitive containing the same Mui value. |
| Type Definition |  |
| CHOICE { noConfirmationRequest NULL, confirmationRequested Mui } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Mui |
| Comment |  |
| Type Definition |  |
| INTEGER {0..4095} |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_AM_DATA_IND |
| PCO Type | DSAP |
| Comment | To indicate to receive DATA using acknowledged mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, integrityResult IntegrityResult, aM_Message CHOICE { uL_DCCH_Message UL_DCCH_Message, uL_CCCH_Message UL_CCCH_Message, uL_SHCCH_Message UL_SHCCH_Message} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | IntegrityResult |
| Comment |  |
| Type Definition |  |
| CHOICE { integrityNotUsed NULL, integrityUsed IntegrityStatus } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | IntegrityStatus |
| Comment |  |
| Type Definition |  |
| ENUMERATED { i_pass(0), i_fail(1) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_AM_DATA_CNF |
| PCO Type | DSAP |
| Comment | For RLC emulator to report to the upper layer that a previously transmitted SDU has been acknowledged correctly by the UE |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, mui Mui } |  |

##### 7.3.2.2.34a	RLC_UM_ACCESSinfo (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_UM_ACCESSinfo_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit ACCESS INFORMATION messages using unacknowledged mode. This ASP is valid for the RLC entity configured for the logical channel MCCH. When an RLC_UM_ACCESSinfo_REQ with uM_Messages present is received the ongoing transmission of ACCESS INFORMATION, if any, shall be stopped in the modification period indicated by startingTime. At the same time, the SS starts transmitting the ACCESS INFORMATION messages passed by the ASP, then repeats the transmission in each next modification period. When an RLC_UM_ACCESSinfo_REQ without uM_Messages is received the SS stops the ongoing ACCESS INFORMATION transmission at the modification period specified by startingTime. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, startingTime INTEGER(0..4095), -- pointing to the first frame of a modification uM_Messages AI_MsgList OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AI_MsgList |
| Comment | AI_MsgList is an ordered list of AI messages. The order corresponds to the AI_Msg transmission timing in a modification period. A modification period can have 1, 2, 4 or 8 access information periods depending on MCCH configuration. The size of the list shall be consistent with the MCCH configuration. |
| Type Definition |  |
| SEQUENCE (SIZE(1¦2¦4¦8))OF AI_Msg |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | AI_Msg |
| Comment | The aI_Message is sent on the first TTI of the access information period. If the corresponding aI_Message is empty there is no ACCESS INFORMATION scheduled for that access information period. |
| Type Definition |  |
| CHOICE { aI_Message MBMSAccessInformation, aI_EmptyMsg NULL } |  |

##### 7.3.2.2.34b	RLC_UM_CriticalMCCHMsg (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_UM_CriticalMCCHMsg_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit critical MCCH messageList using unacknowledged mode. This ASP is valid only for the RLC entity configured for the logical channel MCCH. When an RLC_UM_CriticalMCCHMsg_REQ with non-empty uM_Messages is received the SS stops ongoing critical MCCH information transmission in the modification period indicated by startingTime. At the same time, the SS starts transmitting the set of critical MCCH messageList passed by the ASP in the same order as they appear in the uM_MessageList, and then repeats the transmission in each next repetition period until another RLC_UM_CriticalMCCHMsg_REQ is received to modify the critical messages at start of the next modification period. When an RLC_UM_CriticalMCCHMsg_REQ without uM_Messages is received the SS stops the ongoing critical MCCH message transmission at the modification period specified by startingTime. If specialLI is set to TRUE all SUDs sent within the RLC_UM_CriticalMCCHMsg_REQ have the special LI set to indicate beginning of the RLC SDU. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, startingTime INTEGER(0..4095), -- pointing to the first frame of a modification uM_Messages MCCH_MessageList OPTIONAL, specialLI BOOLEAN DEFAULT FALSE } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MCCH_MessageList |
| Comment | MBMSAccessInformation shall not be included in the MCCH_MessageList. |
| Type Definition |  |
| SEQUENCE (SIZE(1..maxNumMCCHMsgs))OF MCCH_MessageType |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | maxNumMCCHMsgs |
| Comment | For covering the configuration with 20 neighbouring cells |
| Type Definition |  |
| INTEGER (25) |  |

##### 7.3.2.2.34c	RLC_TR_SeqOfRlcPdus

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_SeqOfRlcPdus_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit a sequence of RLC PDUs using transparent mode: The first PDU is sent in the frame at startingTime, the other PDUs are subsequently sent in the following frames. This primitive can be used e.g. to send fully coded RLC PDUs of critical messages at the beginning of a repetition period. Each sequence of RLC PDUs is sent just once i.e. not repeated at the beginning of the next repetition period. Therefore the sequence may also contain Access Information. Furthermore the sequence may contain corrupted PDUs. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, startingTime INTEGER(0..4095), -- pointing to the first frame of a modification seqOfPdus MCCH_RlcPduList } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MCCH_RlcPduList |
| Comment | Each RLC PDU is completely encoded and consists of RLC UM header and RLC SDU |
| Type Definition |  |
| SEQUENCE (SIZE(1..maxNumMCCHRlcPdus)) OF BIT STRING |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | maxNumMCCHRlcPdus |
| Comment | Maximum number of RLC PDUs in RLC tests of MCCH |
| Type Definition |  |
| INTEGER (64) |  |

##### 7.3.2.2.35	RLC_UM_DATA

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_UM_DATA_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit DATA using unacknowledged mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, uM_Message CHOICE { dL_DCCH_Message DL_DCCH_Message, dL_CCCH_Message DL_CCCH_Message, pCCH_Message PCCH_Message, dL_SHCCH_Message DL_SHCCH_Message, bCCH_FACH_Message BCCH_FACH_Message, bCCH_BCH_Message BCCH_BCH_Message invalid_dL_DCCH_Message Invalid_DL_DCCH_Message}, specialLI BOOLEAN } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_UM_DATA_IND |
| PCO Type | DSAP |
| Comment | To indicate to receive DATA using unacknowledged mode. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, integrityResult IntegrityResult, uM_Message CHOICE { uL_DCCH_Message UL_DCCH_Message, uL_CCCH_Message UL_CCCH_Message, uL_SHCCH_Message UL_SHCCH_Message} } |  |

##### 7.3.2.2.35a	RLC_UM_MSCH_Msg (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_UM_MSCH_Msg_REQ |
| PCO Type | DSAP |
| Comment | To request to transmit MSCH_MessageList using unacknowledged mode. The ASP is applied to the RLC entity configured for the logical channel MSCH. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, mSCH_REPconfiguration MSCH_REPconfiguration, uM_Messages MSCH_MessageList } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MSCH_REPconfiguration |
| Comment | MSCH_REPconfiguration describes how the scheduled data to be transmitted. If MSCH is configured MSCH_REPconfiguration specifies when series of MSCH scheduling repetitions start and how long the scheduling period is. The scheduledSFN fulfils: scheduledSFN = ((SFNss / MSCH_REP + 1) * MSCH_REP + MSCH_OFF + (SCTO / 10ms)) mod 4096, where SFNss is the value of currentSFN provided by SS via CPHY_SFN_CNF. The SS shall start sending the 1st SCHEDULING INFORMATION message on the frame indicated by scheduledSFN and successively send the remaining MSCH messages in the list on the 1st TTI of every mSCH_REP. If MSCH is not configured the scheduledSFN fulfils: scheduledSFN = (SFNss + (SCTO / 10ms)) mod 4096, mSCH_REP is omitted. |
| Type Definition |  |
| SEQUENCE { scheduledSFN INTEGER(0..4095), mSCH_REP ENUMERATED { sp32(0), sp64(1), sp128(2), sp256(3), sp512(4), sp1024(5) } OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MSCH_MessageList |
| Comment | Multiple MSCH messages can be submitted to the SS. Every scheduling period a new message in the sequence is transmitted according to the appearing order in the sequence. |
| Type Definition |  |
| SEQUENCE (SIZE(1.. maxNumMSCHMsgs))OF SS_MSCH_Message |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_MSCH_Message |
| Comment | noSend of SS_MSCH_Message is applied to the scheduling period when no MBMS service data are sent on all MTCH. |
| Type Definition |  |
| CHOICE { mSCH_Message MSCH_MessageType, noSend NULL} |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | maxNumMSCHMsgs |
| Comment | Covering a sufficiently long duration of multiple MSCH scheduling periods for test |
| Type Definition |  |
| INTEGER (64) |  |

##### 7.3.2.2.36	RLC_TR_MACesDATA_IND (Rel-6 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_MACesDATA_IND |
| PCO Type | DSAP |
| Comment | This ASP is used for MACes delivering data in MAC_es testing. The IE cellId = -1, The routingInfo is RB identity, corresponding to RLC in TM, (tsc_RB_DTCH_E_DCH_MAC0(-20), tsc_RB_DTCH_E_DCH_MAC1(-21), or tsc_RB_DTCH_E_DCH_MAC2(-22)). The cfn and subframe indicate the CFN and sub-frame number on which the mACesSDUs (RLC PDUs) were received. The ddi, tsn and n are the reported values in the header of each MAC-es PDU that carries the mACesSDUs. If SI is received together with other MACes PDUs in a MAC-e PDU but without a special DDI associated, the SS shall split SI from MACes data and the latter ones are delivered with the ASP. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, cfn INTEGER (0..255), subframe INTEGER (0..4¦7), -- Value 7 applied when TTI=10ms happyBit ENUMERATED {happy(0), unhappy(1)}, ddi INTEGER (0..62), tsn INTEGER (0..63), n INTEGER (0..63), mACesSDUs MACesSDU_List } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACesSDU_List |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..63)) OF MACesSDU |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACesSDU |
| Comment |  |
| Type Definition |  |
| BIT STRING -- RLC PDU in TM |  |

##### 7.3.2.2.36a	RLC_TR_MACisDATA_IND (Rel-8 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_MACisDATA_IND |
| PCO Type | DSAP |
| Comment | This ASP is used for MACis delivering data in MAC_is testing. The IE cellId = -1, the routingInfo is RB identity, corresponding to RLC in TM. The cfn and subframe indicate the CFN and sub-frame number on which the mACisSDUs (RLC PDUs) were received. The LCH_Id, length, F are the reported values in the header of each MAC-is SDU. The SS, tsn are the reported values in the header of each MAC-is PDU that carries the mACisSDUs. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, cfn INTEGER (0..255), subframe INTEGER (0..4¦7), -- Value 7 applied when TTI=10ms happyBit ENUMERATED {happy(0), unhappy(1)}, mACi_Header MACi_HeaderList_Type, ss INTEGER (0.. 3), tsn INTEGER (0..63), cRC_Present BOOLEAN DEFAULT FALSE, mACisSDUs MACisSDU_List } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACi_HeaderList_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..63)) OF MACi_Header_Type |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACi_Header_Type |
| Comment |  |
| Type Definition |  |
| SEQUENCE { lCH_ID INTEGER (0..15), length INTEGER (0..2047) OPTIONAL, f INTEGER (0..1) OPTIONAL, eRNTI BIT STRING (SIZE (16)) OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACisSDU_List |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..63)) OF MACisSDU |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACisSDU |
| Comment |  |
| Type Definition |  |
| BIT STRING -- RLC PDU in TM |  |

##### 7.3.2.2.36b	RLC_TR_MACisDATA_ExtTSN_IND (Rel-9 or later)

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_MACisDATA_ExtTSN _IND |
| PCO Type | DSAP |
| Comment | Rel-9 or later This ASP is used for MACis delivering data in MAC_is testing when extended TSN is configured. The IE cellId is set to the received cell , the routingInfo is RB identity, corresponding to RLC in TM. The cfn and subframe indicate the CFN and sub-frame number on which the mACisSDUs (RLC PDUs) were received. The LCH_Id, length, F are the reported values in the header of each MAC-is SDU. The SS, tsn are the reported values in the header of each MAC-is PDU that carries the mACisSDUs. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, cfn INTEGER (0..255), subframe INTEGER (0..4¦7), -- Value 7 applied when TTI=10ms happyBit ENUMERATED {happy(0), unhappy(1)}, mACi_Header MACi_HeaderList_Type, ss INTEGER (0.. 3), tsn INTEGER (0..16383), mACisSDUs MACisSDU_List } |  |

#### 7.3.2.3	Specific ASP and IE definitions for 1.28 Mcps TDD (Rel-4 or later)

The ASP definitions in 7.3.2.2 are applied to 1.28 Mcps TDD with the exceptions.

-	The ASP definition CPHY_AICH_AckModeSet is not applied.

-	Specific IE definitions in this clause replace the definitions in clause 7.3.2.2.

##### 7.3.2.3.1	Specific ASP definitions

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Config_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-4 or later To request to setup the cell parameter. The unit of tcell is chip; the unit of sfnOffset is frame number; the primary scambling code number of the cell is 16*primaryScramblingCode_SS; the unit of dLTxAttenuationLevel is dB. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63), sfnOffset INTEGER (0 .. 4095), frequencyInfo FrequencyInfo, cellTxPowerLevel CellTxPowerLevel, dLTxAttenuationLevel INTEGER(0..30 ¦123), cellParametersID CellParametersID, timeSlotConfigurationList_LCR TimeSlotConfigurationList_LCR, dwPCHInfo DwPCHInfo, transmissionDiversityApplied ENUMERATED {notApplied (0),applied (1)}OPTIONAL, secondaryFrequencyInfo SecondaryFrequencyInfoList OPTIONAL, upPCHposition UpPCHposition_LCR OPTIONAL, relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_SICH_AckNack_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To Confirm CPHY_HS_SICH_AckNack_REQ |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_SICH_AckNack_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To request for Start or Stop of reporting Ack/Nack received on the SICH for the HARQ process hARQProcessId. At the SS initialization reporting of Ack/Nack is in "STOP" state |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(0..63), ratType RatType, ackNackReportReq AckNackReportReq, hARQProcessId INTEGER(0..7) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_SICH_AckNack_IND |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later SS reports the HARQ-ACK information received on HS_DPCCH, each received Ack/Nack generates a CPHY_HS_DPCCH_AckNack_IND |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, hARQ_ACKInfo ENUMERATED {ack(0), nack (1)}, hARQProcessId INTEGER(0..7) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_SICH_CQI_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. To Confirm CPHY_HS_SICH_CQI_REQ |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_SICH_CQI_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. To enable the SS to start reporting N times of the CQI value received on the HS-SICH. At the SS initialization reporting of CQI values is disabled |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(0..63), ratType RatType, cQIReport ENUMERATED {startRep (0),stopRep (1) } } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_HS_SICH_CQI_IND |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. SS generates the indication when the CQI information is received on HS_SICH after invocation of ASP CPHY_HS_SICH_CQI_REQ. This ASP is used for verifying whether the UE has configured the HS-DSCH and starts reception of HS-DSCH. (3GPP TS 25.331 [21], subclause 8.6.6.34) |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), ratType RatType, rMS ENUMERATED {qpsk (0),qam16 (1) }, rTB INTEGER(0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_TFRCconfigure_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. Confirm a previous CMAC_MAChs_TFRCconfigure_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| Type Name | CMAC_MAChs_TFRCconfigure_REQ |
| --- | --- |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later To configure the TFRC selection in the MAC-hs entity If explicitlyConfigured is selected in tfrcConfigMode, the SS shall use all the parameter values specified to configure a correct transport format and radio resources. If sS_Configured is selected, the parameter value range is specified. SS shall dynamically select the suitable values for the parameters"modulationScheme", "dLTimeslotandCode", "noOfChannelisationCodes ", tbSizeIndexOnHS_SCCH", "redundancyVersion" and "hs_PDSCH_TxPower" according to UE's capability category and CQI information reported by the UE. MaxnoofDLtsLCR=6 |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), tfrcConfigMode CHOICE { explicitlyConfigured SEQUENCE { modulationScheme ModulationScheme, noOfTimeSlots B5, startCode HS_ChannelisationCode_LCR, stopCode HS_ChannelisationCode_LCR, noOfCodesPerTimeslot INTEGER (0..16), tbSizeIndexOnHS_SCCH INTEGER (0..63), redundancyVersions RedundancyVersionList, hs_PDSCH_TxPower DL_TxPower -- default offset related -- to p-PCCPCH }, sS_Configured SEQUENCE { numofTimeslots INTEGER (0..6), startCode HS_ChannelisationCode_LCR, stopCode HS_ChannelisationCode_LCR, numofCodesPerTimeslot INTEGER (0..16), iniHS_PDSCH_TxPower DL_TxPower -- default offset related -- to p-PCCPCH } } } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_MACehs_TFRCconfigure_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later To configure the TFRC selection in the MAC-hs entity, If explicitlyConfigured is selected in tfrcConfigMode, the SS shall use all the parameter values specified to configure a correct transport format and radio resources. This configuration is used for HS-SCCH associated HS-DSCH transmission. If sS_Configured is selected, the parameter value range is specified. SS shall dynamically select the suitable values for the parameters "modulationScheme", "channelisationCodeOffset", "noOfChannelisatonCodes ", .tbSizeIndexOnHS_SCCH", "redundancyVersion" and "hs_PDSCH_TxPower" according to UE's capability category and CQI information reported by the UE. As sps_Information_TDD128 and MIMO cannot be simultaneously configured, only one among sps_Information_TDD128 and mimoStatus can be present. When both are absent non MIMO, non sps_Information_TDD128 is configured. If minimumInterTTIinterval is set to 1, Data is sent every TTI. If it is set to 2, every TTI with Data shall be followed by at least 1 TTI without Data. If it is set to 3, every TTI with Data shall be followed by at least 2 TTI without Data. This field needs to be set as per UE category as defined in Table 5.1a of 25.306. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), tfrcConfigMode CHOICE { explicitlyConfigured SEQUENCE { modulationScheme ModulationScheme, noOfTimeSlots B5, startCode HS_ChannelisationCode_LCR, stopCode HS_ChannelisationCode_LCR, noOfCodesPerTimeslot INTEGER (0..16), tbSizeIndexOnHS_SCCH INTEGER (0..63), redundancyVersions RedundancyVersionList, hs_PDSCH_TxPower DL_TxPower --default offset related to p-PCCPCH }, sS_Configured SEQUENCE { numofTimeslots INTEGER (0..6), startCode HS_ChannelisationCode_LCR, stopCode HS_ChannelisationCode_LCR, numofCodesPerTimeslot INTEGER (0..16), iniHS_PDSCH_TxPower DL_TxPower --default offset related to p-PCCPCH mimoStatus BOOLEAN DEFAULT FALSE, sps_Information_TDD128 SPS_Information_TDD128_r8 OPTIONAL }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later }, activationTime SS_ActivationTime OPTIONAL, } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_HARQprocAssign_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. To assign a HARQ process for handling next one MAC-hs PDU transmission. In normal operation a suitable HARQ process is, without TTCN intervening, selected by HARQ entity in the MAC-hs to service the MAC-hs PDU and the HARQ process identifier is set accordingly. This ASP provides TTCN the ability of selecting an HARQ process servicing the next one MAC-hs PDU which follows the ASP. After successful transmission the MAC-hs returns back to normal operation. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), harqProcessId INTEGER(0..7) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_HARQprocAssign_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. Confirm a previous CMAC_MAChs_HARQprocAsign_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_Reset_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. To reset the MAC-hs entity. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MAChs_Reset_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-5 or later. Confirm a previous CMAC_MAChs_Reset_REQ being successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| Type Name | CMAC_MACe_AG_REQ |
| --- | --- |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), absoluteGrantValue BIT STRING(SIZE(5)), channelisationCode UL_TS_ChannelisationCode, tRRI BIT STRING(SIZE(5)), rDI INTEGER(0..7), eCSN INTEGER(0..7), eI INTEGER(0..3), eNI INTEGER(0..7), hARQProcId INTEGER(0..7), activationTime SS_ActivationTime } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_SI_Config_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later To confirm CMAC_MACe_SI_Config_REQ, cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_SI_Config_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-7 or later To configure the SS to enable / disable to report the reception of Scheduling Information in MAC-es PDU's via primitive CMAC_MACe_SI_IND. At the SS initialization, the default mode is SI reporting disabled. cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), sI_reportEnable ENUMERATED {disable(0), enable(1)} DEFAULT disable } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACe_SI_IND |
| PCO Type | CSAP |
| Comment | This ASP is used for MAC-e delivering scheduling information in MAC-e testing. cellId=-1. If the SI was sent alone in a MAC-e PDU or sent together with other MAC-e PDU in a MAC-e PDU but without a special DDI associated the value of specialDDIpresense is set to absent; If the SI was sent together with other MAC-es PDU in a MAC-e PDU with a special DDI (DDI 63) associated the specialDDIpresence is set to present. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), cfn INTEGER (0..255), subframe INTEGER (0..1), specialDDIpresence ENUMERATED {absent (0), present (1)}, sNPL BIT STRING (SIZE(5)), uePowerHeadRoom BIT STRING (SIZE(5)), totalE_DCHBufferStatus BIT STRING (SIZE(5)), highestPriorityLogChBS BIT STRING (SIZE(4)), highestPriorityLogChId BIT STRING (SIZE(4)) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_MACesDATA_IND |
| PCO Type | DSAP |
| Comment | This ASP is used for MACes delivering data in MAC_es testing. The IE cellId = -1, The routingInfo is RB identity, corresponding to RLC in TM, (tsc_RB_DTCH_E_DCH_MAC0(-20), tsc_RB_DTCH_E_DCH_MAC1(-21), or tsc_RB_DTCH_E_DCH_MAC2(-22)). The cfn and subframe indicate the CFN and sub-frame number on which the mACesSDUs (RLC PDUs) were received. The ddi, tsn and n are the reported values in the header of each MAC-es PDU that carries the mACesSDUs. If SI is received together with other MACes PDUs in a MAC-e PDU but without a special DDI associated, the SS shall split SI from MACes data and the latter ones are delivered with the ASP. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, cfn INTEGER (0..255), subframe INTEGER (0..1), ddi INTEGER (0..62), tsn INTEGER (0..63), n INTEGER (0..63), mACesSDUs MACesSDU_List } |  |

| Type Name | CMAC_MACi_AG_REQ |
| --- | --- |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63), absoluteGrantValue BIT STRING(SIZE(5)), channelisationCode UL_TS_ChannelisationCode, tRRI BIT STRING(SIZE(5)), rDI INTEGER(0..7), eCSN INTEGER(0..7), eI INTEGER(0..3), eNI INTEGER(0..7), hARQProcId INTEGER(0..7), activationTime SS_ActivationTime } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_SI_Config_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later. To confirm CMAC_MACi_SI_Config_REQ, cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_SI_Config_REQ |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later To configure the SS to enable / disable to report the reception of Scheduling Information in MAC-is PDU's via primitive CMAC_MACi_SI_IND. At the SS initialization, the default mode is SI reporting disabled. cellId=-1. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), sI_reportEnable ENUMERATED {disable(0), enable(1)} DEFAULT disable } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_SI_IND |
| PCO Type | CSAP |
| Comment | Applicable Rel-8 or later This ASP is used for MAC-e delivering scheduling information in MAC-i testing. cellId=-1. If the SI was sent alone in a MAC-e PDU or sent together with other MAC-i PDU in a MAC-i PDU but without a special DDI associated the value of specialDDIpresense is set to absent; If the SI was sent together with other MAC-is PDU in a MAC-i PDU with a special DDI (DDI 63) associated the specialDDIpresence is set to present. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), cfn INTEGER (0..255), subframe INTEGER (0..1), sNPL BIT STRING (SIZE(5)), uePowerHeadRoom BIT STRING (SIZE(5)), totalE_DCHBufferStatus BIT STRING (SIZE(5)), highestPriorityLogChBS BIT STRING (SIZE(4)), highestPriorityLogChId BIT STRING (SIZE(4)) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | RLC_TR_MACisDATA_IND |
| PCO Type | DSAP |
| Comment | Applicable Rel-8 or later This ASP is used for MACis delivering data in MAC_is testing. The IE cellId = -1, The routingInfo is RB identity, corresponding to RLC in TM, (tsc_RB_DTCH_E_DCH_MAC0(-20), tsc_RB_DTCH_E_DCH_MAC1(-21), or tsc_RB_DTCH_E_DCH_MAC2(-22)). The cfn and subframe indicate the CFN and sub-frame number on which the mACisSDUs (RLC PDUs) were received. The ddi, tsn and n are the reported values in the header of each MAC-is PDU that carries the mACisSDUs. If SI is received together with other MACis PDUs in a MAC-i PDU but without a special DDI associated, the SS shall split SI from MACes data and the latter ones are delivered with the ASP. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, cfn INTEGER (0..255), subframe INTEGER (0..1), mACi_Header MACi_HeaderList_Type, ss INTEGER (0.. 3), tsn INTEGER (0..63), cRC_Present BOOLEAN DEFAULT FALSE, mACisSDUs MACisSDU_List } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Setup_REQ |
| PCO Type | CSAP |
| Comment | To request to setup the associated transport channels and the Radio Link itself. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63), secondaryFrequency UARFCN OPTIONAL, routingInfo RoutingInfo, ratType RatType, setupMessage CphyRlSetupReq } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Modify_REQ |
| PCO Type | CSAP |
| Comment | To request to modify the Radio Link HardHandover (PhysicalChannelReconfig) ChannelisationCodeChange FrequencyChange PhysicalChannelModifyForTrCHReconfig Re_Synchronized HardHandover |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63), secondaryFrequency UARFCN OPTIONAL, routingInfo RoutingInfo, ratType RatType, setupMessage CphyRlSetupReq } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_UpPCH_IND |
| PCO Type | CSAP |
| Comment | To indicate SS the UpPCH and PRACH received from the UE. sync_UL is the summary of sync UL UE sent in a period. pRACH is the summary of PRACH UE sent in a period. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), sync_UL INTEGER(0..256), pRACH INTEGER(0..256) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_FPACH_REQ |
| PCO Type | CSAP |
| Comment | To request for Start or Stop of FPACH to reponse UpPCH received. If send_FPACH is FALSE, SS should not send FPACH to UE, else SS should send FPACH.If fPACH_Channel_Code is FALSE, SS should send FPACH to UE with wrong channel code, else SS should send FPACH with correct channel code.If signature is FALSE, SS should send FPACH to UE with wrong signature, else SS should send FPACH with correct signature. If subFrame_Number is FALSE, SS should send FPACH to UE with wrong subFrame Number, else SS should send FPACH with correct subFrame Number. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), send_FPACH BOOLEAN, fPACH_Channel_Code BOOLEAN, signature BOOLEAN, subFrame_Number BOOLEAN } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Tx_Timing_Modify_REQ |
| PCO Type | CSAP |
| Comment | Applicable to Rel-4 or later for LCR TDD To request to modify Tx timing of a cell. The unit of tcell is 0.125 chip. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63), tcell INTEGER(0..127) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_Cell_Tx_Timing_Modify_CNF |
| PCO Type | CSAP |
| Comment | Applicable to Rel-4 or later for LCR TDD The confirmation to the CPHY_Cell_Tx_Timing_Modify_REQ |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_RL_Release_REQ |
| PCO Type | CSAP |
| Comment | To request to release the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, activationTime SS_ActivationTime OPTIONAL } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TrCH_Release_CNF |
| PCO Type | CSAP |
| Comment | To confirm to release tthe Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER (0..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_TrCH_Release_REQ |
| PCO Type | CSAP |
| Comment | To request to release the Radio Link |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..63), routingInfo RoutingInfo, trchConfigType TrchConfigType, activationTime SS_ActivationTime OPTIONAL } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInOneMAChs_PDU_CNF |
| PCO Type | CSAP |
| Comment | To confirm the request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH in the same MAChs PDU. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, --RB ID desired to be given result ENUMERATED{failure(0), success(1)} } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CRLC_BindTestDataInOneMAChs_PDU_REQ |
| PCO Type | CSAP |
| Comment | To request of binding subsequent data sending RLC_TR/UM/AM_TestDataReq on the specified RB mapped on HS-DSCH in the same Mac-HS PDU. On the request, the transmission of the test data is temporarily suppressed on the radio bearers till 'numOfSDU's' are received by RLC layer on the Radio Bearer. After receiving all SDU's the RLC layer submits to MAC such that all of them are sent in one MAC-Hs PDU. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, --RB ID desired to be given numOfSDUs INTEGER -- Number of RLC SDU's } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPHY_E_RUCCH_IND |
| PCO Type | CSAP |
| Comment | To indicate the E-RNTI received from UE in E-RUCCH. |
| Type Definition |  |
| SEQUENCE{ cellId INTEGER(0..63), e_RNTI E_RNTI } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_MACi_AG_CNF |
| PCO Type | CSAP |
| Comment | Applicable Rel-9 or later Confirm a previous CMAC_MACi_AG_REQ being successful. |
| Type Definition |  |
| SEQUENCE { nodeB_Id INTEGER(0..63) } |  |

##### 7.3.2.3.2	Specific IE definitions

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyRlModifyReq |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, physicalChannelInfo CHOICE { secondaryCCPCHInfo SecondaryCCPCHInfo, pRACHInfo PRACHInfo, dPCHInfo DPCHInfo, dPCHInfo_r5 DPCHInfo_r5 OrLater, hS_DPSCHInfo HS_DPSCHInfo_r5OrLater, e_DCHInfo SS_E_DCH_Info_r7, e_DCH_Non_ScheduledTransGrantInfo SS_Non_ScheduledTransGrantInfoTDD, e_AGCHInfo SS_E_AGCH_Info, e_HICHInfo SS_E_HICH_Info, mBMS_MICHInfo SS_MBMS_MICHConfigurationInfo_r6, relAspTypeExtension SEQUENCE{} -- Rel-10 or later }, trchConfigToFollow BOOLEAN DEFAULT TRUE, relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyRlSetupReq |
| Comment | Applicable Rel-4 or later for LCR TDD [1] To request to setup the Radio Link; [2] Physical channel of FPACH is defined inside PRACHInfo. |
| Type Definition |  |
| SEQUENCE { physicalChannelInfo CHOICE { primaryCCPCHInfo PrimaryCCPCHInfo, secondaryCCPCHInfo SecondaryCCPCHInfo, pRACHInfo PRACHInfo, pICHInfo PICHInfo, dPCHInfo DPCHInfo, pDSCHInfo PDSCHInfo, pUSCHInfo PUSCHInfo, dPCHInfo_r5 DPCHInfo_r5 OrLater, hS_DPSCHInfo HS_DPSCHInfo_r5OrLater, e_DCHInfo SS_E_DCH_Info_r7, e_DCH_Non_ScheduledTransGrantInfo SS_Non_ScheduledTransGrantInfoTDD, e_AGCHInfo SS_E_AGCH_Info, e_HICHInfo SS_E_HICH_Info, mBMS_MICHInfo SS_MBMS_MICHConfigurationInfo_r6, relAspTypeExtension SEQUENCE{} -- Rel-10 or later }, activationTime SS_ActivationTime OPTIONAL, trchConfigToFollow BOOLEAN DEFAULT TRUE, relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacConfigReq |
| Comment | To request to configure MAC. The IE associatedPhychConfigToFollow should be set to TRUE when there is a associated physical channel configuration to follow (e.g configuration of PICH or MICH), The implementation of the IE "associatedPhychConfigToFollow" can be left to the SS either to wait for the associated physical channel before MAC is configured or to continue the configuration of MAC without waiting for the associated physical channel. If Dual Cell is configured, IE servingAndSecondaryCellActivation is included. |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, uE_Info UE_Info, trCHInfo TrCHInfo, trCH_LogCHMapping TrCH_LogCHMappingList1, associatedPhychConfigToFollow SEQUENCE (SIZE(1..4)) OF INTEGER OPTIONAL, tSN_FieldExtension BOOLEAN DEFAULT FALSE, -- Rel-9 or later relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PrimaryCCPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { sctd_Indicator ENUMERATED {notApplied (0), applied (1)}, tstd_Indicator ENUMERATED {notApplied (0), applied (1)}, commonTimeSlotInfo CommonTimeslotInfo, dL_TxPower_PCCPCH DL_TxPower_PCCPCH, relAspTypeExtension SEQUENCE{} OPTIONAL -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryCCPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD The range for powerOffsetOfTFCI_PO1 is 0-6 dB, 0.25 dB per step. |
| Type Definition |  |
| SEQUENCE { tstd_Indicator ENUMERATED {notApplied (0), applied (1)}, sctd_Indicator ENUMERATED {notApplied (0), applied (1)}, dl_TxPower DL_TxPower, commonTimeSlotInfo CommonTimeslotInfoSCCPCH, channelisationCode SCCPCH_ChannelisationCodeList, individualTimeslotInfo IndividualTimeslotInfo_LCR_r4, powerOffsetOfTFCI_PO1 INTEGER (0..24) OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PRACHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { pRACH_RACH_Info_LCR_r4 PRACH_RACH_Info_LCR_r4, accessServiceClass_TDD_LCR AccessServiceClass_TDD_LCR_r4, fPACH_Power DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD The range for powerOffsetOfTPC_PO2 and powerOffsetOfTFCI_PO1 is 0 dB to 6 dB, 0,25 dB per step. |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r4, dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r4, powerOffsetOfTFCI_PO1 INTEGER (0..24), powerOffsetOfTPC_PO2 INTEGER (0..24), dl_TxPower DL_TxPower, dl_TxPowerMax DL_TxPower, dl_TxPowerMin DL_TxPower, dL_TimeslotISCPInfoLCR TimeslotListWithISCP } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_DPCHInfo_r5 |
| Comment | Applicable Rel-4 or later for LCR TDD The range for powerOffsetOfTPC_PO2 and powerOffsetOfTFCI_PO1 is 0 dB to 6 dB, 0,25 dB per step. |
| Type Definition |  |
| SEQUENCE { dl_CommonInformation DL_CommonInformation_r5, dl_DPCH_InfoPerRL DL_DPCH_InfoPerRL_r5, powerOffsetOfTFCI_PO1 INTEGER (0..24), powerOffsetOfTPC_PO2 INTEGER (0..24), dl_TxPower DL_TxPower, dl_TxPowerMax DL_TxPower, dl_TxPowerMin DL_TxPower, dL_TimeslotISCPInfoLCR TimeslotListWithISCP } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PDSCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { pdsch_Identity PDSCH_Identity, pdsch_Info PDSCH_Info_r4, pdsch_PowerControlInfo PDSCH_PowerControlInfo OPTIONAL, dl_TxPower DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_DSCHMACdFlows |
| Comment | Applicable Rel-5 or later. Within the ACK/NACK repetition period indicated by ackNackRepetitionFactor the SS shall not transmit MAC-hs PDU's on HS-PDSCH. |
| Type Definition |  |
| SEQUENCE { harqInfo HARQ_Info OPTIONAL, addOrReconfMACdFlow SS_AddOrReconfMAC_dFlow OPTIONAL, harqInfo_r7 HARQ_Info_r7 OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTFS |
| Comment | Applicable Rel-4 or later for LCR TDD Transport Format Set |
| Type Definition |  |
| SEQUENCE { tfsMode CHOICE { dedicatedTransChTFS DedicatedTransChTFS, commonTransChTFS CommonTransChTFS, commonTransChTFS_LCR CommonTransChTFS_LCR } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_TxPower_PCCPCH |
| Comment | Applicable Rel-4 or later for LCR TDD Absolute Tx Power of PCCPCH |
| Type Definition |  |
| INTEGER (-60..-30 ) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { ul_DPCHInfo UL_DPCHInfo OPTIONAL, dl_DPCHInfo DL_DPCHInfo OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DwPCH_Power |
| Comment | Applicable Rel-4 or later for LCR TDD DwPCH_power = power * 10 Unit dBm, Range -15dBm .. +40 dBm, Step +0.1dB |
| Type Definition |  |
| INTEGER (-150..400) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DwPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { tstd_Indicator ENUMERATED {notApplied (0),applied (1)}, dwPCH_Power DwPCH_Power } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Four_PICH_pI_BitmapInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| CHOICE { e88 BIT STRING (SIZE (88)), e176 BIT STRING (SIZE (176)), e352 BIT STRING (SIZE (352)) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Two_PICH_pI_BitmapInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| CHOICE { e44 BIT STRING (SIZE (44)), e88 BIT STRING (SIZE (88)), e176 BIT STRING (SIZE (176)) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PDSCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { pdsch_Identity PDSCH_Identity, pdsch_Info PDSCH_Info_r4, pdsch_PowerControlInfo PDSCH_PowerControlInfo OPTIONAL, dl_TxPower DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PICHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { Pichinfo PICH_Info_LCR_r4, dl_TxPower PICH_PowerOffset, sccpchId_associated INTEGER (0..32) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PRACHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { pRACH_RACH_Info_LCR_r4 PRACH_RACH_Info_LCR_r4, accessServiceClass_TDD_LCR AccessServiceClass_TDD_LCR_r4, fPACH_Power DL_TxPower } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PrimaryCCPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { sctd_Indicator ENUMERATED {NotApplied(0),Applied(1)}, tstd_Indicator ENUMERATED {NotApplied(0),Applied(1)}, commonTimeSlotInfo CommonTimeslotInfo, dL_TxPower_PCCPCH DL_TxPower_PCCPCH } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PUSCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { pusch_Identity PUSCH_Identity, pusch_Info PUSCH_Info_r4, pusch_PowerControlInfo PUSCH_PowerControlInfo_r4 OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RatType |
| Comment | Applicable Rel-4 or later for LCR TDD To select route between each channels |
| Type Definition |  |
| ENUMERATED { fdd(0), tdd128(1) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryCCPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { tstd_Indicator ENUMERATED {NotApplied(0),Applied(1)}, sctd_Indicator ENUMERATED {NotApplied(0),Applied(1)}, dl_TxPower DL_TxPower, commonTimeSlotInfo CommonTimeslotInfoSCCPCH, channelisationCode SCCPCH_ChannelisationCodeList, individualTimeslotInfo IndividualTimeslotInfo_LCR_r4, powerOffsetOfTFCI_PO1 INTEGER (0..24) OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_TransportChannelType |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| ENUMERATED { dch(0), rach(1), usch(2), edch(3) -- Rel-6 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TimeSlotConfiguration_LCR |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { timeSlotLCR TimeslotNumber_LCR_r4, timeSlotStatus ENUMERATED {active (0), notActive (1)}, timeSlotDirection ENUMERATED {downlink (0), uplink (1)} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TimeSlotConfigurationList_LCR |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxTS_LCR)) OF TimeSlotConfiguration_LCR |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TimeslotListWithISCP_LCR |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxTS ) ) OF TimeslotWithISCP_LCR |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TimeslotWithISCP_LCR |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { Timeslot TimeslotNumber_LCR_r4, timeslotISCP TimeslotISCP } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | UL_DPCHInfo |
| Comment | Applicable Rel-4 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { uL_DPCH_Info UL_DPCH_Info_r4 } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCH_ChannelisationCodeInfo_LCR |
| Comment |  |
| Type Definition |  |
| SEQUENCE { timeslotNumber TimeslotNumber_LCR_r4, startCode HS_ChannelisationCode_LCR, stopCode HS_ChannelisationCode_LCR } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r5OrLater |
| Comment | Applicable Rel-5 or later for LCR TDD When CHY_RL_Setup_REQ is called with CHOICE of hS_PDSCHInfo HS_PDSCH and HS-SCCH shall be configured in SS. The following HS-DSCH related parameters are passed to the SS implicitly by HSDSCH_physical_layer_category: - Maximum number of HS-DSCH codes can be received by UE, - Minimum inter-TTI interval, - Maximum number of bits of an HS-DSCH transport block within an HS-DSCH TTI - Total number of soft channel bits". HSDSCH_physical_Layer_category is also used for interpretation of the meaning of CQI value. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. |
| Type Definition |  |
| CHOICE { r5 SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to pilot bits on DL-DPCCH hs_PDSCHChannelisationCodeInfo SEQUENCE (SIZE (1..maxPDSCHtimeslot ) ) OF HS_PDSCH_ChannelisationCodeInfo_LCR, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, r6 SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r6, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to pilot bits on DL-DPCCH hs_PDSCHChannelisationCodeInfo SEQUENCE (SIZE (1..maxPDSCHtimeslot)) OF HS_PDSCH_ChannelisationCodeInfo_LCR, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, r7 HS_PDSCHInfo_r7, r8 HS_PDSCHInfo_r8, r9 HS_PDSCHInfo_r9, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

| Type Name | HS_PDSCHInfo_r7 |
| --- | --- |
| Comment | Rel-7 or later. Choice of hS_PDSCH_Info will be used for configuring HSD-DSCH with 64QAM and/or CPC. Choice of common_HS_PDSCH_Info will be used for Enhanced Cell_FACH. commonOrDedicated_H_RNTI indicates, the H-RNTI that will be used by UE (either dedicated H-RNTI if provided in RRC message, or selected common H-RNTI transmitted in SIB5). The transmission of BCCH on HS-DSCH is performed by using BCCH specific H-RNTI on the first indexed HS-SCCH code indicated in system information broadcast. Presence of ss_DTX_Info makes DL DRX to be enabled. Presence of hs_scch_LessInfo makes HS-SCCH less operation enabled. Presence of mimo_Parameters indicate MIMO is to be started. MIMO and HS-SCCH less operation do not co-exist. MIMO is not applicable for non DCH states. The two IE “hSDSCHPhysicalLayerCategory” and “hsdsch_physical_layer_category_ext” are mutually exclusive. One of the IE shall be present in the SS configuration. The IE “hsdsch_physical_layer_category_ext” is present when MAC-ehs is configured. If commonOrDedicated_H_RNTI is omitted, HS-SCCHless is to be applied. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. In common_HS_PDSCH_Info, hs_DPCCHToFollow is set to FALSE unless HS-DPCCH is required in specific configuration. |
| Type Definition |  |
| CHOICE { hS_PDSCH_Info SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r7, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_PDSCHChannelisationCodeInfo SEQUENCE (SIZE (1..maxPDSCHtimeslot ) ) OF HS_PDSCH_ChannelisationCodeInfo_LCR, ss_DTX_Info DRX_Info OPTIONAL, mimo_Parameters MIMO_Parameters_r7 OPTIONAL, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, common_HS_PDSCH_Info SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo HS_SCCH_SystemInfo, hs_dsch_PagingSystemInformation HS_DSCH_PagingSystemInformation OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE }, spare2 SEQUENCE {}, spare3 SEQUENCE {} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r8 |
| Comment | Applicable Rel-8 or later. Choice of hS_PDSCH_Info will be used for configuring HSD-DSCH with 64QAM and/or CPC. Choice of common_HS_PDSCH_Info will be used for Enhanced Cell_FACH. commonOrDedicated_H_RNTI indicates, the H-RNTI that will be used by UE (either dedicated H-RNTI if provided in RRC message, or selected common H-RNTI transmitted in SIB5). The transmission of BCCH on HS-DSCH is performed by using BCCH specific H-RNTI on the first indexed HS-SCCH code indicated in system information broadcast. Presence of controlChannelDRXInfo_TDD128 makes DL DRX to be enabled. Presence of sps_Information_TDD128 makes SPS operation enabled. Presence of mimo_Parameters indicate MIMO is to be started. MIMO and HS-SCCH less operation do not co-exist. MIMO is not applicable for non DCH states. The three IE “hSDSCHPhysicalLayerCategory”, “hsdsch_physical_layer_category_ext” and “hsdsch_physical_layer_category_ext2” are mutually exclusive. One of the IE shall be present in the SS configuration. The IE If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. Presence of ss_HS_DSCH_DtxCellFach makes HS-DSCH DRX operation enabled and SS shall transmit any requested HS-SCCH and HS-DSCH only in occasions when UE will be listening. hs_PDSCHChannelisationCodeInfo present in TDD configurations. |
| Type Definition |  |
| CHOICE { hS_PDSCH_Info SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, multiCarrier_physical_layer_category MultiCarrier_HSDSCH_physical_layer_category OPTIONAL, multiCarrier_physicallayer_category_extension MultiCarrier_HSDSCH_physical_layer_category_extension OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r8, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH mimo_Parameters MIMO_Parameters_r8 OPTIONAL, hs_PDSCHChannelisationCodeInfo SEQUENCE (SIZE (1..maxPDSCHtimeslot ) ) OF HS_PDSCH_ChannelisationCodeInfo_LCR, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, controlChannelDRXInfo_TDD128 ControlChannelDRXInfo_TDD128_r8 OPTIONAL, sps_Information_TDD128 SPS_Information_TDD128_r8 OPTIONAL }, common_HS_PDSCH_Info SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo_tdd128 HS_SCCH_SystemInfo_TDD128 OPTIONAL,, hs_pdsch_MidambleConfiguration HS_PDSCH_Midamble_Configuration_TDD128 OPTIONAL, hs_dsch_PagingSystemInformation_tdd128 HS_DSCH_PagingSystemInformation_TDD128 OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, ss_HS_DSCH_DtxCellFach HS_DSCH_DrxCellfach_info_TDD128 OPTIONAL }, spare2 SEQUENCE {}, spare3 SEQUENCE {} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | HS_PDSCHInfo_r9 |
| Comment | Applicable Rel-9 or later. Choice of hS_PDSCH_Info will be used for configuring HSD-DSCH with 64QAM and/or CPC. Choice of common_HS_PDSCH_Info will be used for Enhanced Cell_FACH. commonOrDedicated_H_RNTI indicates, the H-RNTI that will be used by UE (either dedicated H-RNTI if provided in RRC message, or selected common H-RNTI transmitted in SIB5). The transmission of BCCH on HS-DSCH is performed by using BCCH specific H-RNTI on the first indexed HS-SCCH code indicated in system information broadcast. Presence of controlChannelDRXInfo_TDD128 makes DL DRX to be enabled. Presence of sps_Information_TDD128 makes SPS operation enabled. Presence of mimo_Parameters indicate MIMO is to be started. The IEs “hSDSCHPhysicalLayerCategory”, “hsdsch_physical_layer_category_ext”, are mutually exclusive. One of the IE shall be present in the SS configuration. The IE If commonOrDedicated_H_RNTI is omitted, HS-SCCHless is to be applied. If hs_DPCCHToFollow is FALSE, the hs_DPCCHInd IE shall not be present when ul_DPCHInfo is configured or reconfigured. Presence of ss_HS_DSCH_DtxCellFach makes HS-DSCH DRX operation enabled and SS shall transmit any requested HS-SCCH and HS-DSCH only in occasions when UE will be listening. hs_PDSCHChannelisationCodeInfo present in TDD configurations. |
| Type Definition |  |
| CHOICE { hS_PDSCH_Info SEQUENCE { hSDSCHPhysicalLayerCategory HSDSCH_physical_layer_category OPTIONAL, hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext OPTIONAL, multiCarrier_physical_layer_category MultiCarrier_HSDSCH_physical_layer_category OPTIONAL, multiCarrier_physicallayer_category_extension MultiCarrier_HSDSCH_physical_layer_category_extension OPTIONAL, h_RNTI H_RNTI, dlHSPDSCHInformation DL_HSPDSCH_Information_r9, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH mimo_Parameters MIMO_Parameters_r9 OPTIONAL, hs_PDSCHChannelisationCodeInfo SEQUENCE (SIZE (1..maxPDSCHtimeslot ) ) OF HS_PDSCH_ChannelisationCodeInfo_LCR, hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, controlChannelDRXInfo_TDD128 ControlChannelDRXInfo_TDD128_r8 OPTIONAL, sps_Information_TDD128 SPS_Information_TDD128_r8 OPTIONAL }, common_HS_PDSCH_Info SEQUENCE { hsdsch_physical_layer_category_ext HSDSCH_physical_layer_category_ext, commonOrDedicated_H_RNTI H_RNTI OPTIONAL, bcchSpecific_H_RNTI H_RNTI, hs_scch_SystemInfo_tdd128 HS_SCCH_SystemInfo_TDD128 OPTIONAL,, hs_pdsch_MidambleConfiguration HS_PDSCH_Midamble_Configuration_TDD128 OPTIONAL, hs_dsch_PagingSystemInformation_tdd128 HS_DSCH_PagingSystemInformation_TDD128 OPTIONAL, sttd_Indicator BOOLEAN, hs_SCCH_TxPower DL_TxPower, -- offset related to CPICH hs_DPCCHToFollow BOOLEAN DEFAULT TRUE, ss_HS_DSCH_DtxCellFach HS_DSCH_DrxCellfach_info_TDD128 OPTIONAL }, spare2 SEQUENCE {}, spare3 SEQUENCE {} } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_DCH_Info_r7 |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { e_RUCCH_Info E_RUCCH_Info OPTIONAL, e_PUCH_Info SS_E_PUCH_Info OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_AGCH_Info |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| CHOICE { r7 SEQUENCE { e_AGCH_ID_TDD INTEGER (0..255), e_AGCHInfo E_AGCH_Information_r7, e_AGCH_PowerOffset INTEGER (0..255), -- Range and Step are acc to 25.433, 9.2.2.13Id -- Offset relative to P-CPICH maximum_E_AGCH_Power INTEGER (-350..150), carrier_Frequency UARFCN OPTIONAL, e_RNTI E_RNTI OPTIONAL, ss_DTX_Info DRX_Info OPTIONAL }, r8 SEQUENCE { -- Rel-8 or later e_AGCH_ID_TDD INTEGER (0..255), e_AGCHInfo E_AGCH_Information_r8, e_AGCH_PowerOffset INTEGER (0..255), -- Range and Step are acc to 25.433, 9.2.2.13Id -- Offset relative to P-CPICH maximum_E_AGCH_Power INTEGER (-350..150), carrier_Frequency UARFCN OPTIONAL, e_RNTI E_RNTI OPTIONAL, ss_E_AGCH_Drx_Info E_AGCH_DRX_Info_TDD128 OPTIONAL }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_HICH_Info |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { n_E_HICH INTEGER (4..15 ), maximum_E_HICH_Power INTEGER (-350..150), e_HICHInfoList SS_E_HICH_Info_List } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | E_DCHMACdFlows |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { harq_Info ENUMERATED {rv0 (0), rvtable (1)}, addReconf_MAC_d_FlowList_r7 E_DCH_AddReconf_MAC_d_FlowList_r7 OPTIONAL -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACeConfig |
| Comment | Applicable Rel-7 or later for LCR TDD If the macHeaderManipulation field is 'NormalMacHeader' in ddiMappingList, then data received on the E-DCH (MAC_e PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, and the MACes PDU shall be passed to the MAC_es together with the relevant DDI, N, CFN and subframe number. If the macHeaderManipulation field field is 'OmitMacHeader', then data received on the E-DCH (MAC_e PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, then the MAC_e layer shall delivery the MAC-es PDU, SI and the related CFN, subframe number to the MAC_es entity. |
| Type Definition |  |
| SEQUENCE { activationTime SS_ActivationTime, ddiMappinglist DDI_MappingList OPTIONAL, e_DCHMacdFlows E_DCHMACdFlows OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MACiConfig |
| Comment | If the macHeaderManipulation field is 'NormalMacHeader' in lCH_Mappinglist, then data received on the E-DCH (MAC_i PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, and the MACis PDU shall be passed to the MAC_is together with the relevant LCH_ID, length, F, CFN and subframe number. If the macHeaderManipulation field field is 'OmitMacHeader', then data received on the E-DCH (MAC-i PDU) shall have it's MAC header inspected to de-multiplex and to determine the appropriate routing, then the MAC-i layer shall deliver the MAC-is PDU, SI and the related CFN, subframe number to the MAC-is entity. connectedToMAC_is field is used to provide the possibility that the E-DCH-MACdFlows from only one MAC-i entity are connected to the MAC_is entity in the inter node B soft handover test cases. When MAC-i is configured in enhanced FACH UL using commonMac_i_r8, the MAC-i entity is always connected to MAC-is. The IEs lCH_Mappinglist and e_DCHMacdFlows can be OMITted when changing the serving cell MAC-i without modification of MAC-i configurations. It will be applied in EDCH SHO. ss_DRX_MAC_Info presence indicates UL DRX shall be applied. |
| Type Definition |  |
| CHOICE { mAC_i_r8 SEQUENCE { -- Rel-8 or later activationTime SS_ActivationTime, lCH_Mappinglist LCH_MappingList OPTIONAL, e_DCHMacdFlows E_DCHMACdFlows OPTIONAL, connectedToMAC_is BOOLEAN DEFAULT TRUE –- can be set to FALSE in inter nodeB SHO }, commonMac_i_r8 SEQUENCE { activationTime SS_ActivationTime, lCH_Mappinglist LCH_MappingList OPTIONAL, e_DCHMacdFlows CommonE_DCHMACdFlows OPTIONAL }, relAspTypeExtension SEQUENCE{} -- Rel-10 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_PUCH_Info |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { lTGI_Presence BOOLEAN, e_TFCS_Info E_TFCS_Info, snpl_ReportType ENUMERATED {type1(0), type2(1)}, prxBASEdes INTEGER (-112..-50 ), beaconPLEst BOOLEAN OPTIONAL, tpc_StepSize TPC_StepSizeTDD, pebase_PowerControlGAP PowerControlGAP OPTIONAL, ul_SynchronisationParameters UL_SynchronisationParameters_r4 OPTIONAL, minimum_Allowed_Code_Rate INTEGER (0..63 ), maximum_Allowed_Code_Rate INTEGER (0..63 ), maximumNumOfRetransSchedInfo INTEGER (0..15 ), retransTimerForSchedInfo ENUMERATED {ms10(0), ms15(1), ms20(2), ms25(3), ms30(4), ms35(5), ms40(6), ms45(7), ms50(8), ms55(9), ms60(10), ms65(11), ms70(12), ms75(13), ms80(14), ms85(15), ms90(16), ms95(17), ms100(18), ms110(19), ms120(20), ms140(21), ms160(22), ms200(23), ms240(24), ms280(25), ms320(26), ms400(27), ms480(28), ms560(29)}, powerOffsetForSchedInfo INTEGER (0..6 ) OPTIONAL, e_DCH_phyLayCategory INTEGER (1..6 ) OPTIONAL, e_PUCH_TS_Information_per_UARFCN SEQUENCE (SIZE (1..maxTS_LCR_1 ) ) OF SS_E_PUCH_Timeslot_Info, carrier_Frequency UARFCN OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_PUCH_Timeslot_Info |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { timeslotNumber TimeslotNumber_LCR_r4, midambleAllocationMode CHOICE { defaultMidamble NULL, ueSpecificMidamble INTEGER (0..15 ) }, midambleConfiguration INTEGER (1..8 ), channelisation_Code UL_TS_ChannelisationCode } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_Non_ScheduledTransGrantInfoTDD |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| CHOICE { tdd384_768 SEQUENCE { timeslotResourceRelatedInfo BIT STRING (SIZE (13 ) ), powerResourceRelatedInfo INTEGER (1..32 ), activationTime ActivationTime, repetitionPeriodAndLength RepetitionPeriodAndLength OPTIONAL, codeResourceInfo UL_TS_ChannelisationCode }, tdd128 SEQUENCE { n_E_UCCH INTEGER (1..8 ) OPTIONAL, n_E_HICH INTEGER (4..15 ) OPTIONAL, timeslotResourceRelatedInfo BIT STRING (SIZE (5 ) ), powerResourceRelatedInfo INTEGER (1..32 ), activationTime ActivationTime, sfnNum INTEGER (0..1 ), repetitionPeriodAndLength RepetitionPeriodAndLength OPTIONAL, codeResourceInfo UL_TS_ChannelisationCode, e_HICH_Info SEQUENCE { e_HICH_ID_TDD INTEGER (0..255 ), signature_Sequence_Group_Index INTEGER (0..19 ) } } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_HICH_Info_LCR |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE { e_HICH_ID_TDD INTEGER (0..255), e_HICH_Type ENUMERATED {scheduled (0), non_scheduled(1)}, e_HICHInfo E_HICH_Information_LCR, carrier_Frequency UARFCN OPTIONAL, e_HICH_PowerOffset INTEGER (0..255) -- PowerOffset = -32 + offset * 0.25 -- Unit dB, Range -32dB .. +31.75dB, Step +0.25dB -- (25.433, 9.2.2.13If), offset relative to P-CPICH } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_E_HICH_Info_List |
| Comment | Applicable Rel-7 or later for LCR TDD |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxNumE_HICH ) ) OF SS_E_HICH_Info_LCR |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | E_TFCS_Restriction |
| Comment | Applicable Rel-7 or later for LCR TDD The E_TFCS restriction is a list of E-TFCIs, and can be used to verify that the UE has used a specific TFC. Any data received by the SS using a forbidden TFCI shall be discarded. |
| Type Definition |  |
| SEQUENCE OF INTEGER (0..63) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_RLC_Info |
| Comment | Applicable to Rel-4, Rel-5, Rel-6, Rel-7 for LCR TDD. UL and DL have been swapped intentionally in this type definition. This is to maximize re-use of the type definitions in 3GPP TS 25.331 [21] which are intended to configure a UE, where UL is transmission, and DL is reception. For the SS, UL is reception, and DL is transmission. For example, consider configuring a DL AM RLC entity (transmitter) in the SS. The transmission parameters to be configured include PollingInformation, Transmission-RLC-Discard etc. If the DL-AM-RLC-Mode type definition is used to configure this entity, it is only possible to configure reception parameters such as StatusInformation, and receiving window size. By swapping UL and DL, it is possible to configure the DL AM RLC entity using the existing type definition UL-AM-RLC-Info, which contains all of the required transmission parameters. When uM_SN_DeliveryMode is set to configured, the RLC entity does not concatenate nor segment RLC SDUs (see 3GPP TS 25.322 [18], subclause 8.2). If the IE useSpecialValueOfHEField is set to true, the last octet of the PDU is the last octet of an SDU and there is no SDU concatenation inside the PDU. sS_ul_PayloadSize is applied to RB0 RLC UL configuration; if it is omitted in the configuration the dl_PayloadSize is applied to UL RLC entity. |
| Type Definition |  |
| SEQUENCE { sS_ul_RLC_Mode DL_RLC_Mode OPTIONAL, sS_ul_PayloadSize PayloadSize OPTIONAL, sS_dl_RLC_Mode SS_DL_RLC_Mode OPTIONAL, rlc_OneSidedReEst BOOLEAN DEFAULT FALSE, altE_bitInterpretation ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for UM RLC mode of Rel-7 or later useSpecialValueOfHEField ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for AM RLC mode of Rel-7 or later uM_SN_DeliveryMode ENUMERATED { nonConfigured(0), configured(1)} DEFAULT nonConfigured -- applicable for UM RLC mode of Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacPagingConfigReq |
| Comment |  |
| Type Definition |  |
| SEQUENCE { pI_BitMapInfo CHOICE { two_PICH_frame Two_PICH_pI_BitmapInfo, four_PICH_frame Four_PICH_pI_BitmapInfo }, dRX_CycleLength INTEGER (3..9), iMSI IMSI_GSM_MAP, t_pich_T_sccpch BOOLEAN -- T_pich > T_sccpch then FALSE } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonE_DCHMACdFlows |
| Comment | Rel-7 or later |
| Type Definition |  |
| SEQUENCE { harq_Info ENUMERATED {rv0 (0), rvtable (1)}, addReconfMAC_d_FlowList Common_E_DCH_MAC_d_FlowsList } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | PRACH_MeasurementReport |
| Comment | sfn is included if reportSFN is TRUE in CPHY_PRACH_Measurement_REQ. |
| Type Definition |  |
| SEQUENCE { sync_UL_Code BIT STRING(SIZE (8)), sfn INTEGER (0..4095) OPTIONAL -- Rel-8 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryFrequencyInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { frequencyInfo UARFCN, dLTxAttenuationLevel INTEGER(0..30) } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SecondaryFrequencyInfoList |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE(1..maxNumOfSecondFrequency)) OF SecondaryFrequencyInfo |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RL_Information |
| Comment | The range for powerOffsetOfTPC_PO2 and powerOffsetOfTFCI_PO1 and powerOffsetOfPILOT_PO3 is 0 dB to 6 dB, 0,25 dB per step. The IE cfnTgtSfnFrameOffset is applied when adding another RL or moving the UE to another cell in DCH state in the timing-maintained hard handover or in the soft handover. The cfnTgtSfnFrameOffset is defined as being the time difference between the CFN and the SFN of the cell in which the RL is to be added. The use of cfnTgtSfnFrameOffset and DOFF (Default DPCH Offset Value) is mutually exclusive. The IE cfnTgtSfnFrameOffset is omitted when configuring the 1st RL, or configuring the RL in the timing re-initialized hard handover where the required synchronization information is provided in defaultDPCH_offsetValue in DL_CommonInformation. In addition, TmTgt value is provided to dpch_FrameOffset in DL_DPCH_InfoPerRL_r5 or DL_DPCH_InfoPerRL_r6 as CFNchipOffset_Tgt. TmTgt can be observed by the UE, or calculated by the TTCN. If it is calculated, TmTgt = (DOFF*512 + 38400 + TCell_Ref - TCell_Tgt) Mod 38400. dL_TimeslotISCPInfoLCR present in TDD configuration. |
| Type Definition |  |
| SEQUENCE { powerOffsetOfTFCI_PO1 INTEGER (0..24), powerOffsetOfTPC_PO2 INTEGER (0..24), powerOffsetOfPILOT_PO3 INTEGER (0..24), dl_TxPower DL_TxPower, dl_TxPowerMax DL_TxPower, dl_TxPowerMin DL_TxPower, cfnTgtSfnFrameOffset CfnTgtSfnFrameOffset OPTIONAL, dL_TimeslotISCPInfoLCR TimeslotListWithISCP OPTIONAL } |  |

| Type Name | SS_RLC_Info_r8OrLater |
| --- | --- |
| Comment | Applicable for Rel-8 or Later. UL and DL have been swapped intentionally in this type definition. This is to maximize re-use of the type definitions in 3GPP TS 25.331 [21], which are intended to configure a UE, where UL is transmission, and DL is reception. For the SS, UL is reception, and DL is transmission. For example, consider configuring a DL AM RLC entity (transmitter) in the SS. The transmission parameters to be configured include PollingInformation, Transmission-RLC-Discard etc. If the DL-AM-RLC-Mode type definition is used to configure this entity, it is only possible to configure reception parameters such as StatusInformation, and receiving window size. By swapping UL and DL, it is possible to configure the DL AM RLC entity using the existing type definition UL-AM-RLC-Info, which contains all of the required transmission parameters. When uM_SN_DeliveryMode is set to configured, the RLC entity does not concatenate nor segment RLC SDUs (see 3GPP TS 25.322 [18], subclause 8.2). If the IE useSpecialValueOfHEField is set to true, the last octet of the PDU is the last octet of an SDU and there is no SDU concatenation inside the PDU. sS_ul_PayloadSize is applied to RB0 RLC UL configuration; if it is omitted in the configuration the dl_PayloadSize is applied to UL RLC entity. |
| Type Definition |  |
| CHOICE { r8 SEQUENCE { sS_ul_RLC_Mode DL_RLC_Mode_r7 OPTIONAL, sS_ul_PayloadSize PayloadSize OPTIONAL, sS_dl_RLC_Mode SS_DL_RLC_Mode OPTIONAL, rlc_OneSidedReEst BOOLEAN DEFAULT FALSE, altE_bitInterpretation ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for UM RLC mode useSpecialValueOfHEField ENUMERATED {false (0), true (1)} DEFAULT false, -- applicable only for AM RLC mode uM_SN_DeliveryMode ENUMERATED { nonConfigured(0), configured(1)} DEFAULT nonConfigured -- applicable for UM RLC mode of Rel-7 or later }, relAspTypeExtension SS_RLC_Info_r10OrLaterExtensionType -- Rel-10 or later } |  |

### 7.3.3	TTCN primitives

#### 7.3.3.1	UTRAN TTCN primitives

The tables below show the primitives that are used for RLC, BMC ,RB and PDCP tests, these primitives are defined in TTCN tabular form.

| ASP Name | RLC_TR_TestDataReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To request the transmission of unstructered data using transparent mode in the downlink direction. TTCN writter is requires to send data according to the transport block size allowed. |  |  |  |
| Parameter Name |  | Parameter Type |  | Comments |
| cellId |  | INTEGER |  |  |
| rB_Id |  | SS_RB_Identity |  |  |
| data |  | PDU |  |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_TR_TestDataInd |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To indicate the reception of unstructered data using transparent mode in the uplink direction |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_UM_TestDataReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To request the transmission of unstructered data using unacknowledged mode in the downlink direction |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_UM_TestDataInd |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To indicate the reception of unstructered data using unacknowledged mode in the uplink direction |  |  |  |
| Parameter Name |  | Parameter Type |  | Comments |
| cellId |  | INTEGER |  |  |
| rB_Id |  | SS_RB_Identity |  |  |
| data |  | PDU |  |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_UM_ScheduledDataReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | The ASP is used to request the transmission of unstructured data using unacknowledged mode in the downlink direction. The CFN/Subframe indicates the CFN and sub-frame values on which the Data is transmitted. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity |  |
| cFN |  |  | ActivationTime |  |
| subframe |  |  | Subframe_Type |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_UM_ScheduledDataInd |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | The ASP is used to indicate the reception of unstructured data using unacknowledged mode in the uplink direction. The CFN/Subframe indicates the CFN and sub-frame values on which the Data is received. If an SDU is segmented into multiple PDUs, the CFN/Subframe of the last PDU is to be reported. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity |  |
| cFN |  |  | ActivationTime |  |
| subframe |  |  | Subframe_Type |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_AM_TestDataReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To request the transmission of unstructered data using acknowledged mode in the downlink direction |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_AM_TestDataInd |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To indicate the reception of unstructered data using acknowledged mode in the uplink direction |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | BMC_DataReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | BSAP |  |  |  |
| Comments | The ASP is used to request the transmission of unstructured BMC data or scheduling message, using unacknowledged mode and the BMC PCO in the downlink direction. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| routingInfo |  |  | RoutingInfo |  |
| data |  |  | PDU |  |
| Detailed Comments |  |  |  |  |

| ASP Name | BMC_DataCnf |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | BSAP |  |  |  |
| Comments | The ASP is used to confirm the previous reception of BMC_DataReq |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| routingInfo |  |  | RoutingInfo |  |
| Detailed Comments |  |  |  |  |

| ASP Name | RLC_HandoverReq |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | The ASP is used to request the transmission of the HandoverFromUTRANCommand_GSM message using acknowledged operation (AM). The Meta PDU in turn consists of 2 components. 1) The ASN.1 PER encoded HandoverFromUTRANCommand, without any 1 bit to 7 bits of padding. 2) The GSM Handover command. The SS shall take care of inserting the MAC and RLC sequence number of Integrity check info, as in the case of other RRC DL PDU's |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | INTEGER |  |
| rB_Id |  |  | SS_RB_Identity | RB Identity |
| msg |  |  | PDU | HandoverFromUTRANCommandGSM message |
| Detailed Comments |  | This ASP is sent to RRC integrity emulation for generation of the correct IntegrityCheckInfo value. |  |  |

The TTCN tabular format applies to the primitive definitions.

### 7.3.4	GERAN PCO and ASP definitions

#### 7.3.4.1	PCO Type definitions

##### 7.3.4.1.1	PCO type for data transmission and reception in GERAN

Table 7.3.4.1.1: Declaration of the G_DSAP PCO Type

| PCO Type Definition |  |
| --- | --- |
| PCO Type | G_DSAP |
| Role | LT |
| Comment | DATA transmission and reception |

##### 7.3.4.1.2	PCO type for configuration and control in GERAN

Table 7.3.4.1.2: Declaration of the G_CSAP PCO Type

| PCO Type Definition |  |
| --- | --- |
| PCO Type | G_CSAP |
| Role | LT |
| Comment | Transmission and reception of control primitives |

#### 7.3.4.2	PCO definitions

##### 7.3.4.2.1	PCOs for data transmission and reception in GERAN

7.3.4.2.1.1	PCO for data transmission and reception through GERAN L2

Table 7.3.4.2.1.1: Declaration of G_L2 PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_L2 |
| PCO Type | G_DSAP |
| Role | LT |
| Comment | Control and observation point of GERAN L3 messages and user data |

7.3.4.2.1.2	PCO for data transmission and reception through GPRS RLC

Table 7.3.4.2.1.2: Declaration of G_RLC PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_RLC |
| PCO Type | G_DSAP |
| Role | LT |
| Comment | Control and observation point of GPRS GRR signalling messages |

7.3.4.2.1.3	PCO for data transmission and reception through GPRS LLC

Table 7.3.4.2.1.3: Declaration of LLC PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_LLC |
| PCO Type | G_DSAP |
| Role | LT |
| Comment | Control and observation point of GPRS GMM signalling messages |

7.3.4.2.1.4	Void

##### 7.3.4.2.2	PCOs for control primitives transmission and reception in GERAN

7.3.4.2.2.1	PCO for GERAN L1control primitives transmission and reception

Table 7.3.4.2.2.1: Declaration of G_CL1 PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_CL1 |
| PCO Type | G_CSAP |
| Role | LT |
| Comment | Control GERAN Physical Layer (L1) |

7.3.4.2.2.2	PCO for GERAN L2 control primitives transmission and reception

Table 7.3.4.2.2.2: Declaration of G_CL2 PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_CL2 |
| PCO Type | G_CSAP |
| Role | LT |
| Comment | Control GERAN L2 |

7.3.4.2.2.3	PCO for GPRS RLC control primitives transmission and reception

Table 7.3.4.2.2.3: Declaration of G_CRLC PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_CRLC |
| PCO Type | G_CSAP |
| Role | LT |
| Comment | Control GPRS RLC/MAC layer |

7.3.4.2.2.4	PCO for GPRS LLC control primitives transmission and reception

Table 7.3.4.2.2.4: Declaration of G_CLLC PCO

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_CLLC |
| PCO Type | G_CSAP |
| Role | LT |
| Comment | Control GPRS LLC layer |

7.3.4.2.2.5	Void

#### 7.3.4.3	GERAN ASP Definitions

##### 7.3.4.3.1	ASPs for data transmission and reception in GERAN

7.3.4.3.1.1	ASPs for data transmission and reception through GERAN L2

| ASP Name | G_L2_DATA_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send L3 signalling message on the signalling channels or user data on the traffic channels to the UE/MS in acknowledged mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| sAPI |  |  | SAPI | 0 or 3 |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType |  |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  |  | RFN | The reduced frame number of the first frame on which this message is sent. This field is not applicable and the SS shall ignore it if the field t2 of rfn is coded as '11111'B. |
| msg |  |  | PDU | Signalling message or user data to be sent |
| Detailed Comments |  | Parameter rfn is only used in the test cases that require L3 message to be sent on specified frame number. |  |  |

| ASP Name | G_L2_DATA_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive a L3 signalling message on the signalling channels or user data on the traffic channels from the UE/MS in acknowledged mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| sAPI |  |  | SAPI | 0 or 3 |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType |  |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  |  | RFN | The reduced frame number of the first frame carrying the message |
| msg |  |  | PDU | Signalling message or user data received |
| Detailed Comments |  |  |  |  |

| ASP Name | G_L2_L2Estab_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive an indication of that L2 multiple frame operation on the specified channel has been established. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType |  |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: FACCH/H, SDCCH/8 and SDCCH/4, This field shall be coded as 15 if it is not applicable. |
| sAPI |  |  | SAPI | 0,3 |
| establish_mode |  |  | OCTETSTRING[1] |  |
| rfn |  |  | RFN | The reduced frame number of the first frame carries the L2 SABM frame |
| msg |  |  | PDU | this field is present only when the establish mode is CoRes (collision resolution) |
| Detailed Comments |  | see 3GPP TS 44.006 [42], clauses 7.1.1 and 7.1.3 |  |  |

| ASP Name | G_L2_UNITDATA_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send L3 signalling message on the signalling channels or send user data on the traffic channels to the UE/MS in unacknowledged mode. |  |  |  |
| Parameter Name |  | Parameter Type |  | Comments |
| cellId |  | CellId |  |  |
| sAPI |  | SAPI |  | 0 or 3 |
| physicalChId |  | PhysicalChId |  | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |  |
| subChannel |  | SubChannelNumber |  | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  | RFN |  | The reduced frame number of the first frame on which this message is sent. This field is not applicable and the SS shall ignore it if the field t2 of rfn is coded as '11111'B. |
| msg |  | PDU |  | Signalling message or user data to be sent |
| Detailed Comments |  |  | Parameter fn is only used in the test cases that require specific L3 message to be sent on specified frame number. |  |

| ASP Name | G_L2_UNITDATA_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive a L3 signalling message on the signalling channels or user data on the traffic channels from the UE/MS in unacknowledged mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| sAPI |  |  | SAPI | 0 or 3 |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType |  |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  |  | RFN | The reduced frame number of the first frame carrying the message |
| msg |  |  | PDU | Signalling message or user data received |
| Detailed Comments |  |  |  |  |

| ASP Name | G_L2_ACCESS_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive a random access or handover access burst on the specified channel. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType | RACH, FACCH, SDCCH/8, SDCCH/4. RACH is used for random access burst; others are used for handover access burst |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: FACCH/H, SDCCH/8, SDCCH/4. This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  |  | RFN | The reduced frame number of the first frame carrying the burst |
| burst |  |  | PDU | Random access burst or handover access burst |
| Detailed Comments |  |  |  |  |

| ASP Name | G_L2_Paging_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send a paging message on the specified paging group of the specified paging channel to the UE/MS, when the UE/MS is in idle mode or the UE/MS not supporting SPLIT_PG_CYCLE on CCCH is in GPRS attached mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| sAPI |  |  | SAPI | 0 |
| physicalChId |  |  | PhysicalChId | Channel identifier of the right CCCH_GROUP |
| g_LogicChType |  |  | G_LogicChType | PCH |
| pagingGroup |  |  | PAGING_GROUP |  |
| pagingMode |  |  | PagingMode | 0-normal paging; 1-extended paging; 2-paging reorganization. |
| msg |  |  | PDU | Paging message |
| Detailed Comments |  | The SS is required to send valid layer 3 messages continuously on all paging subchannels on CCCH where paging can appear. For "normal paging" the SS send the paging message in the specified pagingGroup; For "extended paging" " the SS send the paging message in the specified pagingGroup and in the "next but one" position on the PCH, following the block corresponding to pagingGroup; For "paging reorganization" the SS send the paging message in all paging subchannels. The required 51-multiframe occurs when: pagingGroup div (N div BS_PA_MFRMS) = (FN div 51) mod (BS_PA_MFRMS) The index to the required paging block in the 51-multiframe determined above: Paging block index = pagingGroup mod (N div BS_PA_MFRMS) N = (9-BS_AG_BLKS_RES) * BS_PA_MFRMS CCCH not combined or N = (3-BS_AG_BLKS_RES) * BS_PA_MFRMS CCCH + SDCCH combined |  |  |

| ASP Name | G_L2_PagingGPRS_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send a paging message on the specified paging group of the specified paging channel to the UE/MS, when the UE/MS supporting SPLIT_PG_CYCLE on CCCH is in GPRS attached mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| sAPI |  |  | SAPI | 0 |
| physicalChId |  |  | PhysicalChId | Channel identifier of the right CCCH_GROUP |
| g_LogicChType |  |  | G_LogicChType | PCH |
| pagingGroup |  |  | PAGING_GROUP |  |
| pagingMode |  |  | PagingMode | 0-normal paging; 1-extended paging; 2-paging reorganization. |
| splitPGcycleCode |  |  | INTEGER | 0 -- No Split Paging, 1..32 -- Split Paging |
| msg |  |  | PDU | Paging message |
| Detailed Comments |  | The SS is required to send valid layer 3 messages continuously on all paging subchannels on CCCH where paging can appear. For "normal paging" the SS send the paging message in the specified pagingGroup; For "extended paging" " the SS send the paging message in the specified pagingGroup and in the "next but one" position on the PCH, following the block corresponding to pagingGroup; For "paging reorganization" the SS send the paging message in all paging subchannels. The required 51-multiframe occurs when: pagingGroup div (M div 64) = (FN div 51) mod 64 The index to the required paging block in the 51-multiframe determined above: Paging block index = pagingGroup mod (M div 64) M = (9-BS_AG_BLKS_RES) × 64 CCCH not combined or M = (3-BS_AG_BLKS_RES) × 64 CCCH + SDCCH combined |  |  |
| NOTE: This ASP may not be implemented if the MS/UE does not support SPLIT_PG_CYCLE on CCCH. |  |  |  |  |

| Type Name | CellId |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments |  |

| Type Name | SAPI |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | Service access point identifier for GERAN L2 and LLC |

| Type Name | PhysicalChId |
| --- | --- |
| Type Definition | INTEGER(0..31) |
| Type Encoding |  |
| Comments | Physical channel identifier in GERAN |

| Type Name | G_LogicChType |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | GERAN logical channel type: 0-BCCH; 1-RACH; 2-PCH; 3-AGCH; 4-SDCCH/4; 5-SACCH/C4; 6-SDCCH/8; 7-SACCH/C8; 8-TCH/F; 9-FACCH/F; 10-SACCH/TF; 11-TCH/H; 12-FACCH/H; 13-SACCH/TH; 14-Void; 15-PRACH; 16-PPCH; 17-PAGCH; 18-PDTCH/F; 19-PACCH/F; 20-PTCCH/F; 21-E-TCH/F; 22-E-IACCH/F; 23-E-FACCH/F; 24-SACCH/M; 25-SACCH/MD |

| Type Name | SubChannelNumber |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | Subchannel number for TCH/H, FACCH/H, SACCH/TH, SDCCH/4, SDCCH/C4, SDCCH/8 and SDCCH/C8. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); For SDCCH/4 and SACCH/C4 value is (0..3). |

| Type Name | PAGING_GROUP |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | 3GPP TS 05.02 or 3GPP TS 45.002 [31], clauses 6.5.2 and 6.5.6 |

| Type Name | PagingMode |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | 0 - normal paging; 1 - extended paging; 2 - paging reorganization. |

| Type Name | RFN |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | The reduced frame number, its range is 0 -- 42431 (FN modulo 42432) about 195.8 s |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| t1_ | BITSTRING[5] |  | (FN div 1326) mod 32 |
| t3 | BITSTRING[6] |  | FN mod 51 |
| t2 | BITSTRING[5] |  | FN mod 26 |
| Detailed Comments | see 3GPP TS 04.18 or 3GPP TS 44.018 [43], clause 10.5.2.38. The reduced frame number, FN modulo 42432 can be calculated in the following formula: 51 × ((t3 - t2) mod 26) + t3 + 1326 × t1_. RFN is used for starting time and TBF starting time. |  |  |

| ASP Name | G_L2_Release_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |
| Comments | This ASP from L2, indicates that the multiple frame operation release was successful. This means that the UA message was received in response to L2 DISC command. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| sAPI |  | SAPI | 0 or 3 |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| releaseMode |  | BITSTRING[1] | 0 = normal release; 1 = local release. |
| Detailed Comments |  |  |  |

| ASP Name | G_L2_Release_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |
| Comments | This ASP requests L2 to send Layer 2 DISC command on the indicated SAPI. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| sAPI |  | SAPI | 0 or 3 |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| releaseMode |  | BITSTRING[1] | 0 = normal release; 1 = local release. |
| Detailed Comments |  |  |  |

| ASP Name | G_L2_Release_IND |  |  |
| --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |
| Comments | The ASP is used to receive an indication of the termination of an established multiple frame operation or an indication of an unsuccessful establishment attempt. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| sAPI |  | SAPI | 0 |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); for SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). |
| releaseMode |  | BITSTRING[1] | 0 = normal release; 1 = local end release |
| outstanding_Indicator |  | BOOLEAN | whether or not there are outstanding acknowledgements or unsolved G_L2_DATA_REQ primitives. |
| Detailed Comments |  |  |  |

| ASP Name | G_L2_SYSINFO_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send system information messages to the lower layer emulator. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| sAPI |  |  | SAPI | 0 |
| physicalChId |  |  | PhysicalChId |  |
| g_LogicChType |  |  | G_LogicChType | BCCH or SACCH |
| instanceIndex |  |  | INTEGER | To indicate the instance of the system information messages. For SYSTEM INFORMATION Type 2ter, 18, 19, 20 the value is (0..7); for type 14, 15 the value is (0..3); for type 2quater the value is (0..15); for all other type the value is 0. |
| sysInfoType |  |  | SysInfoType | SYSTEM INFORMATION Type 5, 5bis, 5ter, and 6 are sent on SACCH, the other SYSTEM INFORMATION 's are sent on BCCH. |
| BCCHExt |  |  | B1 | '0' indicates message sent on BCCH Norm, '1' indicates message sent on BCCH Ext. Only valid for SI 2quater, 7, 8, 13, 15, 16, 17. Default value '0' |
| msg |  |  | PDU | This field contains SYSTEM INFORMATION message. See 3GPP TS 44.018 [43], clause 9.1.31 to clause 9.1.43h for SYSTEM INFORMATION message definitions. |
| Detailed Comments |  | The lower layer emulator shall store the SYSTEM INFORMATION's, and transmit them periodically according to the rules specified in clause 6.3.1.3 of 3GPP TS 05.02 or 3GPP TS 45.002 [31]. The msg shall override the same type system information message previous stored in the lower layer emulator. |  |  |

| Type Name | SysInfoType |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | 25--SYSTEM INFORMATION TYPE 1 26--SYSTEM INFORMATION TYPE 2 2 -- SYSTEM INFORMATION TYPE 2bis 3 -- SYSTEM INFORMATION TYPE 2ter 7 -- SYSTEM INFORMATION TYPE 2quater 27--SYSTEM INFORMATION TYPE 3 28--SYSTEM INFORMATION TYPE 4 29--SYSTEM INFORMATION TYPE 5 5 -- SYSTEM INFORMATION TYPE 5bis 6 -- SYSTEM INFORMATION TYPE 5ter 30--SYSTEM INFORMATION TYPE 6 31--SYSTEM INFORMATION TYPE 7 24--SYSTEM INFORMATION TYPE 8 4 -- SYSTEM INFORMATION TYPE 9 0 -- SYSTEM INFORMATION TYPE 13 61--SYSTEM INFORMATION TYPE 16 62--SYSTEM INFORMATION TYPE 17 64--SYSTEM INFORMATION TYPE 18 65--SYSTEM INFORMATION TYPE 19 66--SYSTEM INFORMATION TYPE 20 67—SYSTEM INFORMATION TYPE 15 |

7.3.4.3.1.2	ASPs for data transmission and reception through GERAN RLC

| ASP Name | G_RLC_ControlMsg_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to transmit a RLC/MAC control message to the UE/MS on the specified channel. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| physicalChId |  |  | PhysicalChId |  |
| g_LogicChType |  |  | G_LogicChType | PACCH |
| tBF_Direction |  |  | INTEGER | 1-downlink TBF; 0-uplink TBF |
| tFI |  |  | TFI | Temporary flow identity |
| rRBP |  |  | RRBP | Relative reserved block period |
| s_P_Bit |  |  | S_P_Bit | Supplementary/polling bit |
| rfn |  |  | RFN | The reduced frame number of the first frame on which this message is sent. This field is not applicable and the SS shall ignore it if the field t2 of rfn is coded as '11111'B. |
| pagingGroup |  |  | PAGING_GROUP | this field shall be omitted |
| pagingMode |  |  | PagingMode | this field shall be omitted |
| msg |  |  | PDU | Down link RLC/MAC control message |
| Detailed Comments |  | This ASP provides values for "RRBP" and "S/P" fields in MAC header for TTCN controlling the response from the UE, the value for "PayloadType" and "USF" fields in MAC header shall be filled by the SS. If a RLC/MAC control message can not be fitted into one RLC/MAC control block, the SS RLC/MAC entity shall take the responsibility of segmentation of the message, and set the correct "PayloadType" and optional octet1 (and optional octet2). |  |  |

| Type Name | RRBP |
| --- | --- |
| Type Definition | BITSTRING[2] |
| Type Encoding |  |
| Comments | 3GPP TS 04.60 or 3GPP TS 44.060 [32], clause 10.4.5 |

| Type Name | S_P_Bit |
| --- | --- |
| Type Definition | BITSTRING[1] |
| Type Encoding |  |
| Comments | 0 - RRBP field is not valid; 1 - RRBP field is valid. |

| ASP Name | G_RLC_ControlMsg_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive an uplink RLC/MAC control block sent by the UE/MS on the specified channel. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| physicalChId |  |  | PhysicalChId |  |
| g_LogicChType |  |  | G_LogicChType | PACCH or PDTCH |
| tBF_Direction |  |  | INTEGER | 1 - downlink TBF; 0 - uplink TBF |
| tFI |  |  | TFI | Temporary flow identity |
| rfn |  |  | RFN | The reduced frame number of the frame carrying the message |
| msg |  |  | PDU | Uplink RLC/MAC control message |
| Detailed Comments |  | Logical channel type PDTCH is valid for PACKET ENHANCED MEARSUREMENT REPORT message only. |  |  |

7.3.4.3.1.3	ASPs for data transmission and reception through GERAN LLC

| ASP Name | G_LLC_UNITDATA_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send L3 PDU to the UE/MS in LLC unconfirmed transmission. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId |  |
| tLLI |  |  | TLLI |  |
| sAPI |  |  | SAPI |  |
| protectMode |  |  | BITSTRING[1] | 0 -- unprotected; 1 -- protected |
| cipherMode |  |  | BITSTRING[1] | 0 -sent without encryption; 1 -sent with encryption |
| msg |  |  | PDU | L3 PDU |
| Detailed Comments |  | 3GPP TS 04.64 or 3GPP TS 44.064 [33], clause 8.4.1 After the ciphering function is started in the SS by G_CLLC_Assign_REQ, the SS shall encrypt the "msg" when cipherMode = '1', and the SS shall not encrypt the "msg" if cipherMode = '0'. |  |  |

| Type Name | LLMEId |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | The identifier of the Logical Link Management Entity in SGSN |

| ASP Name | G_LLC_UNITDATA_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive a L3 PDU from the UE/MS in LLC unconfirmed transmission. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId |  |
| tLLI |  |  | TLLI |  |
| sAPI |  |  | SAPI |  |
| msg |  |  | PDU | L3 PDU |
| Detailed Comments |  | 3GPP TS 04.64 or 3GPP TS 44.064 [33], clause 8.4.2 |  |  |

| ASP Name | G_LLC_XID_RES |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to send to the UE/MS the negotiated XID parameters agreed by the SS. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId |  |
| tLLI |  |  | TLLI |  |
| sAPI |  |  | SAPI |  |
| xID_Info |  |  | XID_Info | the negotiated XID parameters agreed by the SS |
| Detailed Comments |  |  |  |  |

| Type Name | XID_Info |
| --- | --- |
| Type Definition | OCTETSTRING |
| Type Encoding |  |
| Comments | Exchange Identification Information |

| ASP Name | G_LLC_XID_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used to receive the XID requested by the UE/MS. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId |  |
| tLLI |  |  | TLLI |  |
| sAPI |  |  | SAPI |  |
| xID_Info |  |  | XID_Info | the XID parameters requested by the UE/MS |
| Detailed Comments |  |  |  |  |

| ASP Name | G_LLC_NULL |
| --- | --- |
| PCO Type | G_DSAP |
| Comments | Please refer to TS 36.523-3 [62], clause 6.4.2 for the definition. This ASP is not used in ATSs in TTCN2. |

7.3.4.3.1.4	Void

7.3.4.3.1.5	ASPs for data transmission and reception through GERAN DTM

| ASP Name | G_L2_GTTP_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used for DTM to send an LLC signalling message on the DCCH in acknowledged mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| SAPI |  |  | SAPI |  |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType |  |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| protectMode |  |  | BITSTRING[1] | 0 -- unprotected; 1 -- protected |
| cipherMode |  |  | BITSTRING[1] | 0 - sent without encryption; 1 - sent with encryption |
| skipIndicator |  |  | B4 | GTTP header, unciphered |
| GTTPProtocolDiscriminator |  |  | B4 | GTTP header, unciphered |
| msgType |  |  | B8 | GTTP header, unciphered |
| TLLI |  |  | TLLI | GTTP header, unciphered |
| gmmSmPDULength |  |  | Length | GTTP header, unciphered |
| gmmSmPDU |  |  | PDU | GMM or SM PDU - ciphered |
| Detailed Comments |  | Ciphering must be started in the SS by G_CLLC_Assign_REQ , and also by either G_CL1_CipheringControl_REQ, G_CL1_CipherModeModify_REQ, or G_CL1_CreateBasicPhyCh_REQ. When cipherMode = '1' the SS shall encrypt the "LLCPDU", using the algorithm specified in px_GPRS_CipherAlg, and then encrypt the whole outgoing message using the algorithm specified in px_GSM_CipherAlg. The SS shall not encrypt the message at all if cipherMode = '0'. |  |  |

| ASP Name | G_L2_GTTP_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |  |
| Comments | The ASP is used for DTM to receive an LLC signalling message on DCCH acknowledged mode. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| SAPI |  |  | SAPI |  |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| g_LogicChType |  |  | G_LogicChType |  |
| subChannel |  |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  |  | RFN | The reduced frame number of the first frame carrying the message |
| TLLI |  |  | TLLI |  |
| gmmSmPDU |  |  | PDU | Deciphered GMM or SM signalling message received |
| Detailed Comments |  | If ciphering is used, the SS will take care to ensure the "LLC PDU" is deciphered |  |  |

##### 7.3.4.3.2	ASPs for control primitive transmission and reception in GERAN

7.3.4.3.2.1	ASPs for configuration and control of GERAN L1

| ASP Name | G_CL1_CreateCell_REQ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |  |  |
| Comments | The ASP is used to create a cell in GERAN |  |  |  |  |  |
| Parameter Name |  |  |  | Parameter Type |  | Comments |
| cellId |  |  |  | CellId |  |  |
| baseId |  |  |  | BITSTRING[6] |  | base transceiver station identity code = NCC+BCC. see 3GPP TS 23.003 [6] |
| timingAdvance |  |  | BITSTRING[8] |  | The SS sets the timing of uplink direction in advance of downlink direction timing by this value. |  |
| Detailed Comments |  |  |  |  |  |  |

| ASP Name | G_CL1_CreateCell_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to get the confirmation of a G_CL1_CreateCell_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The cell created |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CL1_DeleteCell_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to delete a cell in GERAN |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The cell to be deleted |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CL1_DeleteCell_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to get the confirmation of a G_CL1_DeleteCell_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The cell deleted |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CL1_CreateBasicPhyCh_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to create a basic physical channel in GERAN |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The cell which the channel to be created belongs to |
| physicalChId |  | PhysicalChId | identifier of the physical channel in the SS. |
| channelCombination |  | ChannelCombination | Logical channels combined onto the basic physical channel. |
| frqInfo |  | FrqInfo | Parameters for Description of the physical channel in frequency domain |
| timeSlot |  | TN | The timeslot number of the physical channel |
| tsc |  | TSC | Training sequence code. For common control and broadcast channels the value of tsc must be equal to BCC (base station colour code) |
| channelSpecificInfo |  | ChannelSpecificInfo | Specific parameters related to individual channel |
| txPower |  | TX_Power | The transmission power level in dBVemf() |
| bandIndicator |  | BITSTRING[1] | Parameter for DCS or PCS frequency band selection. A value 0 for frqInfo.arfcn interpreted as DCS1800. A value 1 for frqInfo.arfcn interpreted as PCS1900. If omitted, the value in frqInfo.arfcn interpreted as DCS1800. |
| Detailed Comments |  | The value of channelCombination permitted currently: 1 TCH/F + FACCH/F + SACCH/TF 2 TCH/H(0,1) + FACCH/H(0,1) + SACCH/TH(0,1) 3 TCH/H(0,0) + FACCH/H(0,1) + SACCH/TH(0,1) + TCH/H(1,1) 4 FCCH + SCH + BCCH + CCCH 5 FCCH + SCH + BCCH + CCCH + SDCCH/4(0..3) + SACCH/C4(0..3) 6 BCCH + CCCH 7 SDCCH/8(0..7) + SACCH/C8(0.. 7) 8 TCH/F + FACCH/F + SACCH/M 9 TCH/F + SACCH/M 10 TCH/FD + SACCH/MD 11 Void 12 Void 13 PDTCH/F+PACCH/F+PTCCH/F 18 E-TCH/F + E-IACCH/F + E-FACCH/F + SACCH/TF 19 E-TCH/F + E-IACCH/F + E-FACCH/F + SACCH/M 20 E-TCH/F + E-IACCH/F + SACCH/M 21 E-TCH/FD + E-IACCH/F + SACCH/MD |  |

| ASP Name | G_CL1_CreateBasicPhyCh_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to get the confirmation of a G_CL1_CreateBasicPhyCh_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The cell which the created channel belongs to |
| physicalChId |  |  | PhysicalChId | The physical channel created. |
| Detailed Comments |  |  |  |  |

| Type Name | FrqInfo |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Parameters for Description of basic physical channel in frequency domain. |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| h | BITSTRING[1] |  | h=1:hopping channel h=0: non-hopping channel |
| spr | BITSTRING [3] |  | '000'B |
| spr1 | BITSTRING [2] |  | '00'B if h = 0, otherwise OMIT |
| maio | BITSTRING [6] |  | mobile allocation index offset if h = 1, otherwise OMIT |
| hsn | BITSTRING [6] |  | hopping sequence number if h = 1, otherwise OMIT |
| arfcn | BITSTRING [10] |  | absolute RF channel number if h = 0, otherwise OMIT |
| hoppingFreqList | FrequencyList |  | hopping frequency list if h = 1, otherwise OMIT. The definition see 3GPP TS 44.018 [43] or 3GPP TS 04.18, clause 10.5.2.13 |
| Detailed Comments |  |  |  |

| Type Name | ChannelSpecificInfo |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Parameters for individual channel |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| dedCH_Info | DedCH_Info |  | Parameters for dedicated channel. Valid for combination:1, 2, 3, 5, 7, 8, 9, 10 |
| cCCH_Info | CCCH_Info |  | Parameters for common control channels: PCH, SCH, etc. Valid for combination: 4, 5, 6 |
| Detailed Comments |  |  |  |

| Type Name | DedCH_Info |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Parameters for dedicated channel |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| chMod | ChMode |  | Definition see 3GPP TS 04.18 or 3GPP TS 44.018 [43], clause 10.5.2.6 |
| cipherMode | CipherModeSetting |  | Definition see 3GPP TS 04.18 or 3GPP TS 44.018 [43], clause 10.5.2.9 |
| cipherKey | BITSTRING[64] |  | (see note 1) |
| cipherKey128 | BITSTRING[128] |  | (see note 1) |
| powerLevel | BITSTRING[5] |  | Initial MS uplink transmission power level. This value is used in the L1 header of SACCH. |
| timingAdvance | BITSTRING[8] |  | Initial timing advance. This value is used in the L1 header of SACCH. This field shall be set to the same value as in timingAdvance of G_CL1_CreateCell_REQ. |
| Detailed Comments | In addition to ciphering algorithm the cipherMode specifies the initial ciphering mode of the physical channel in both transmission and receiving direction by startingCiph bit. During ciphering mode setting procedure the ciphering mode of receiving direction can be changed by G_CL1_CipheringControl_REQ. NOTE 1: The value of cipherMode dictates which of cipherKey or cipherKey128 shall be used by the SS. |  |  |

| Type Name | CCCH_Info |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Parameters for common control channels |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| bS_PA_MFRMS | BITSTRING[3] |  | the number of 51-multiframes between transmissions of paging messages. Definition see 3GPP TS 04.18 or 3GPP TS 44.018 [43], clause 10.5.2.11 |
| bS_AG_BLKS_RES | BITSTRING[3] |  | the number of blocks on each common control channel reserved for access grant messages. Definition see 3GPP TS 04.18 or 3GPP TS 44.018 [43], clause 10.5.2.11 |
| Detailed Comments |  |  |  |

| Type Name | MultiSlotAllocation |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Used in multi-slot configuration |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| tN0 | BOOLEAN |  | TRUE - time slot 0 is allocated; FALSE -- not allocated |
| channelCombination0 | ChannelCombination |  | Channel combination for time slot 0; not applicable if tN0 = FALSE |
| tN1 | BOOLEAN |  | TRUE - time slot 1 is allocated; FALSE -- not allocated |
| channelCombination 1 | ChannelCombination |  | Channel Combination for time slot 1; not applicable if tN1 = FALSE |
| tN2 | BOOLEAN |  | TRUE - time slot 2 is allocated; FALSE -- not allocated |
| channelCombination 2 | ChannelCombination |  | Channel Combination for time slot 2; not applicable if tN2 = FALSE |
| tN3 | BOOLEAN |  | TRUE - time slot 3 is allocated; FALSE -- not allocated |
| channelCombination 3 | ChannelCombination |  | Channel Combination for time slot 3; not applicable if tN3 = FALSE |
| tN4 | BOOLEAN |  | TRUE - time slot 4 is allocated; FALSE -- not allocated |
| channelCombination 4 | ChannelCombination |  | Channel Combination for time slot 4; not applicable if tN4 = FALSE |
| tN5 | BOOLEAN |  | TRUE - time slot 5 is allocated; FALSE -- not allocated |
| channelCombination 5 | ChannelCombination |  | Channel Combination for time slot 5; not applicable if tN5 = FALSE |
| tN6 | BOOLEAN |  | TRUE - time slot 6 is allocated; FALSE -- not allocated |
| channelCombination 6 | ChannelCombination |  | Channel Combination for time slot 6; not applicable if tN6 = FALSE |
| tN7 | BOOLEAN |  | TRUE - time slot 7 is allocated; FALSE -- not allocated |
| channelCombination 7 | ChannelCombination |  | Channel Combination for time slot 7; not applicable if tN7 = FALSE |
| Detailed Comments | Multislot configuration is referred to 3GPP TS 05.02 or 3GPP TS 45.002 [31], clause 6.4.2. The timeslot for which G_CL1_CreateBasicPhyCh_REQ has set the channel combination shall be set to FALSE. |  |  |

| ASP Name | G_CL1_CipheringControl_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to set the ciphering mode of the physical channel in receiving direction, the kc and ciphering algorithm was set by the G_CL1_CreateBasicPhyCh_REQ for the physical channel before calling the ASP. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| rcvCipherMode |  | BITSTRING[1] | Ciphering Mode in SS receiving direction: 0 not ciphered 1 ciphered |
| Detailed Comments |  | For GSM dedicated physical channel, the ciphering mode of the SS shall be changed in three steps: (3GPP TS 44.018 [43], clause 3.4.7) Before the SS sending CIPHERING MODE COMMAND the SS is transmitting and receiving in old ciphering mode (for example, not ciphered), after the SS sending CIPHERING MODE COMMAND the SS changes its receiving ciphering mode to new ciphering mode (for example, ciphered) and keeps transmitting in old ciphering mode; then after receiving CIPHERING MODE COMPLETE or any correct L2 frame in new ciphering mode the SS changes the transmitting ciphering mode to the new mode. TTCN writer shall use this ASP before sending the CIPHERING MODE COMMAND to ensure the ciphering mode of the physical channel, in sufficient time, according to the 3 step procedure outlined above. |  |

| ASP Name | G_CL1_CipheringControl_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to confirm that the G_CL1_CipheringControl_REQ is executed correctly. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| physicalChId |  |  | PhysicalChId | Channel identifier |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CL1_ComingFN_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to request lower layer return the reduced frame number (FN modulo 42432) which is far enough in the future from current frame number and is able to carry L3 message on the specified channel. The requirement of "far enough" is that there is enough time left for TTCN to prepare a L3 message to send before that frame. The ASP could also be used in the calculation of a value for starting time |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_ComingFN_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to receive the result of G_CL1_ComingFN_REQ. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| rfn |  | RFN | the reduced frame number (FN modulo 42432) which is about 0.7 seconds later than current frame number and is able to carry L3 message on the channel specified by "physicalChId"+"G_LogicChType"+"subChannel" |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_DeleteChannel_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to delete a basic physical channel or an multi-slot configuration |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The identifier of the cell which the channel to be deleted belongs to |
| physicalChId |  | PhysicalChId | The physical channel or the multi-slot configuration to be deleted. |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_DeleteChannel_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to get the confirmation of a G_CL1_DeleteChannel_REQ |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The identifier of the cell which the deleted channel belongs to |
| physicalChId |  | PhysicalChId | The physical channel or multi-slot configuration deleted. |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_CipherModeModify_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to modify cipher mode of a dedicated channel |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The identifier of the cell |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| cipherMode |  | CipherModeSetting | The new cipher mode. Definition see 3GPP TS 04.18 or 3GPP TS 44.018 [43], clause 10.5.2.9 |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_CipherModeModify_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to get the confirmation of a G_CL1_CipherModeModify_REQ |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The identifier of the cell |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: TCH/H, FACCH/H, SACCH/TH, SDCCH/8, SACCH/C8, SDCCH/4, and SACCH/C4. For TCH/H, FACCH/H and SACCH/TH value is (0..1); For SDCCH/8 and SACCH/C8 value is (0..7); for SDCCH/4 and SACCH/C4 value is (0..3). This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_ChangePowerLevel_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to change the transmission power level of a physical channel |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The identifier of the cell which the physical channel belongs to |
| physicalChId |  | PhysicalChId | Channel using the new transmission power level |
| txPower |  | TX_Power | The new transmission power level in dBVemf() |
| Detailed Comments |  |  |  |

| ASP Name | G_CL1_ChangePowerLevel_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to get the confirmation of a G_CL1_ChangePowerLevel_REQ |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The identifier of the cell |
| physicalChId |  | PhysicalChId | The physical channel which uses the new transmission power level |
| Detailed Comments |  |  |  |

7.3.4.3.2.2	ASPs for configuration and control of GERAN L2

| ASP Name | G_CL2_HoldPhyInfo_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP commands the SS to hold the PHYSICAL INFORMATION message, which will be sent on PCO G_L2 following the current ASP. The PHYSICAL INFORMATION message shall be sent to the UE/MS within T3124 from the time when the SS has received n handover access bursts. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: FACCH/H, SDCCH/8 and SDCCH/4, This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| n |  | INTEGER | The number of handover access bursts to be received |
| Detailed Comments |  | T3124 is defined in 3GPP TS 04.18 or 3GPP TS 44.018 [43], clauses 3.4.4.2.2 and 11.1.1 |  |

| ASP Name | G_CL2_HoldPhyInfo_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to get a confirmation of the G_CL2_HoldPhyInfo_REQ. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType |  |
| subChannel |  | SubChannelNumber | Valid only for logical channel types: FACCH/H, SDCCH/8 and SDCCH/4. This field is not applicable and the SS shall ignore it if this field is coded as 15. |
| Detailed Comments |  |  |  |

| ASP Name | G_CL2_MeasRptControl_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to enable or disable the reporting of received Measurement Reports to the TTCN |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| g_LogicChType |  | G_LogicChType | Valid only for logical channel types: SACCH/TF, SACCH/TH, SACCH/C8 and SACCH/C4 |
| subChannel |  | SubChannelNumber | For SACCH/TH value is (0..1); for SACCH/C8 value is (0..7); for SACCH/C4 value is (0..3). |
| sendMeasRpts |  | BOOLEAN | Whether or not to report received Measurement Reports to the TTCN. |
| Detailed Comments |  | Per default, this will be set to FALSE |  |

| ASP Name | G_CL2_MeasRptControl_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to confirm that G_CL2_MeasRptControl_REQ was executed correctly |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| Detailed Comments |  |  |  |

| ASP Name | G_CL2_Release_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used request the SS stop L2 transmission on a channel. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |
| Detailed Comments |  |  |  |

| ASP Name | G_CL2_Release_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to confirm that the G_CL2_Release_REQ is executed correctly |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| physicalChId |  | PhysicalChId | Channel identifier |

7.3.4.3.2.3	ASPs for configuration and control of GERAN RLC/MAC

| ASP Name | G_CRLC_CreateRLC_MAC_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to create a RLC/MAC entity in GERAN RLC/MAC emulation module. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The identifier of the cell |
| Detailed Comments |  | One RLC/MAC entity per cell can exist, cellId will be used for couping LLC layer module to the RLC/MAC emulation module.. The packet channel description given in the ChannelSpecificInfo of G_CL1_CreateBasicPhyCh_REQ shall be used to configure this layer. This ASP shall be called after the G_CL1_CreateBasicPhyCh_REQ ASP. |  |  |

| ASP Name | G_CRLC_CreateRLC_MAC_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to confirm the G_CRLC_CreateRLC_MAC_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The identifier of the cell |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CRLC_DeleteRLC_MAC_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to delete a RLC/MAC entity in GERAN emulation module. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The identifier of the cell |
| Detailed Comments |  | This ASP is used to release any resource used for the RLC/MAC emulation entity in the SS. |  |  |

| ASP Name | G_CRLC_DeleteRLC_MAC_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to confirm the G_CRLC_CreateRLC_MAC_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId | The identifier of the cell |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CRLC_UL_TBF_Config_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to configure a TBF used for uplink packet data transfer |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| tFI |  | TFI |  |
| tBF_Mode |  | BITSTRING[1] | 0 - GPRS; 1 - EGPRS |
| channelCoding |  | ChannelCoding |  |
| tLLI_BlockChannelCoding |  | BITSTRING[1] | 0 - CS-1 or MCS-1(EGPRS); 1 - same as channelCoding |
| rLC_Mode |  | BITSTRING[1] | 0 - acknowledged mode; 1 - unacknowledged mode |
| startingTime |  | RFN | This field is not applicable and the SS shall ignore it if the field t2 of rfn is coded as '11111'B. |
| uSF_Rate |  | INTEGER | This parameter controls the speed of the UL TBF transferring data blocks by controlling the USF rate: 1---> implementation dependent. TTCN does not specify the USF generating rate; 2---> 10 USF's per second; 3---> 5 USF's per second; 4---> 1 USF per second; 5---> 1 USF per 2 seconds; 6---> 1 USF per 3 seconds; 7---> 1 USF per 4 seconds. |
| dynamicAllocation |  | dynamicAllocation | dynamic allocation and other parameters. |
| Detailed Comments |  | For GPRS channel coding can be: CS-1, CS-2, CS-3 and CS-4; For EGPRS channel coding can be : MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, MCS-7, MCS-8, MCS-9, MCS-5-7 and MCS-6-9. Due to one cell currently has only one RLC/MAC emulation module, this ASP does not contain RLC/MAC identity parameter to indicate which RLC/MAC emulation module this TBF is established for, instead, the parameter cellId implicitly indicates the RLC/MAC module, which is created by G_CRLC_CreateRLC_MAC_REQ in the cell. The higher layer (LLC emulation module) uses rLC/MAC_MappingInfo (with type of CellId) to address the RLC/MAC emulation module to which it connects |  |

| ASP Name | G_CRLC_UL_TBF_Config_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to get the confirmation of a G_CRLC_UL_TBF_Config_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| tFI |  |  | TFI |  |
| Detailed Comments |  |  |  |  |

| Type Name | ChannelCoding |
| --- | --- |
| Type Definition | INTEGER |
| Type Encoding |  |
| Comments | 1 - CS-1; 2 - CS-2; 3 - CS-3; 4 -- CS-4; 5 - MCS-1; 6 - MCS-2; 7 - MCS-3; 8 - MCS-4; 9 - MCS-5; 10 - MCS-6; 11 - MCS-7; 12 - MCS-8; 13 - MCS-9; 14 - MCS-5-7; 15 - MCS-6-9. |

| Type Name | DynamicAllocation |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Used for up link TBF; dynamic allocation or extended dynamic allocation |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| extendedAllocation | BITSTRING[1] |  | 0 - dynamic allocation; 1 - extended dynamic allocation |
| uSFGranularity | BITSTRING[1] |  | 0 - one block; 1 - four blocks |
| physicalChId | PhysicalChId |  | Single PDCH or multislot-configured PDCHs |
| tN0 | BOOLEAN |  | TRUE - time slot 0 is allocated; FALSE -- not allocated |
| uSF_TN0 | BITSTRING[3] |  | USF value for slot 0 |
| tN1 | BOOLEAN |  | TRUE - time slot 1 is allocated; FALSE -- not allocated |
| uSF_TN1 | BITSTRING[3] |  | USF value for slot 1 |
| tN2 | BOOLEAN |  | TRUE - time slot 2 is allocated; FALSE -- not allocated |
| uSF_TN2 | BITSTRING[3] |  | USF value for slot 2 |
| tN3 | BOOLEAN |  | TRUE - time slot 3 is allocated; FALSE -- not allocated |
| uSF_TN3 | BITSTRING[3] |  | USF value for slot 3 |
| tN4 | BOOLEAN |  | TRUE - time slot 4 is allocated; FALSE -- not allocated |
| uSF_TN4 | BITSTRING[3] |  | USF value for slot 4 |
| tN5 | BOOLEAN |  | TRUE - time slot 5 is allocated; FALSE -- not allocated |
| uSF_TN5 | BITSTRING[3] |  | USF value for slot 5 |
| tN6 | BOOLEAN |  | TRUE - time slot 6 is allocated; FALSE -- not allocated |
| uSF_TN6 | BITSTRING[3] |  | USF value for slot 6 |
| tN7 | BOOLEAN |  | TRUE - time slot 7 is allocated; FALSE -- not allocated |
| uSF_TN7 | BITSTRING[3] |  | USF value for slot 7 |
| Detailed Comments | The uSF_TNx field is not applicable when tNx = FALSE. |  |  |

| ASP Name | G_CRLC_DL_TBF_Config_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to configure a TBF used for down link packet data transfer |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| tFI |  | TFI |  |
| tBF_Mode |  | BITSTRING[1] | 0 - GPRS; 1 - EGPRS |
| channelCoding |  | ChannelCoding |  |
| rLC_Mode |  | BITSTRING[1] | 0 - acknowledged mode; 1 - unacknowledged mode |
| timeSlotAllocation |  | TimeSlotAllocation | Downlink TBF time slot allocation |
| startingTime |  | RFN | This field is not applicable and the SS shall ignore it if the field t2 of rfn is coded as '11111'B. |
| dataBlockRate |  | INTEGER | This parameter controls the speed of the DL TBF sending RLC/MAC data blocks on the assigned PDCH's: 1---> implementation dependent. TTCN does not specify the data block rate; 2---> 10 data blocks per second; 3---> 5 data blocks per second; 4---> 1 data block per second; 5---> 1 data block per 2 seconds; 6---> 1 data block per 3 seconds; 7---> 1 data block per 4 seconds. |
| Detailed Comments |  | For GPRS channel coding can be: CS-1, CS-2, CS-3 and CS-4; For EGPRS channel coding can be : MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, MCS-7, MCS-8, MCS-9, MCS-5-7 and MCS-6-9. |  |

| ASP Name | G_CRLC_DL_TBF_Config_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to get the confirmation of a G_CRLC_DL_TBF_Config_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| tFI |  |  | TFI |  |
| Detailed Comments |  |  |  |  |

| Type Name | TimeSlotAllocation |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | Used for downlink and up link TBF |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| physicalChId | PhysicalChId |  | single PDCH or multislot-configured PDCHs |
| tN0 | BOOLEAN |  | Timeslot 0; TRUE - allocated; FALSE - not allocated. |
| tN1 | BOOLEAN |  | Timeslot 1; TRUE - allocated; FALSE - not allocated. |
| tN2 | BOOLEAN |  | Timeslot 2; TRUE - allocated; FALSE - not allocated. |
| tN3 | BOOLEAN |  | Timeslot 3; TRUE - allocated; FALSE - not allocated. |
| tN4 | BOOLEAN |  | Timeslot 4; TRUE - allocated; FALSE - not allocated. |
| tN5 | BOOLEAN |  | Timeslot 5; TRUE - allocated; FALSE - not allocated. |
| tN6 | BOOLEAN |  | Timeslot 6; TRUE - allocated; FALSE - not allocated. |
| tN7 | BOOLEAN |  | Timeslot 7; TRUE - allocated; FALSE - not allocated. |
| Detailed Comments |  |  |  |

7.3.4.3.2.4	ASPs for configuration and control of GERAN LLC

| ASP Name | G_CLLC_CreateLLE_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to create an LLE (LLC Entity) in GERAN emulation part of the SS and connects the created LLE to the RLC/MAC emulation module pointed by rLC/MAC_MappingInfo.. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| lLMEId |  | LLMEId | Logical Layer Management Entity Id |
| rLC/MAC_MappingInfo |  | CellId | This parameter indicates the RLC/MAC emulation module in the cell, not the cell itself. |
| Detailed Comments |  | The RLC/MAC emulation module needs to be created prior to this ASP by G_CRLC_CreateRLC_MAC_REQ ASP. |  |

| ASP Name | G_CLLC_CreateLLE_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to confirm the G_CLLC_CreateLLE_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId | The identifier of the cell Logical Layer Management Entity Id |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CLLC_DeleteLLE_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to delete an LLE (LLC Entity) in GERAN LLC emulation module. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId | Logical Layer Management Entity Id |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CLLC_DeleteLLE_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | The ASP is used to confirm the G_CLLC_DeleteLLE_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId | Logical Layer Management Entity Id |
| Detailed Comments |  |  |  |  |

| ASP Name | G_CLLC_Assign_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to assign, change, or unassign the TLLI, the ciphering key (Kc) and the ciphering algorithm of GERAN LLC emulation module. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| lLMEId |  | LLMEId | Logical Layer Management Entity Id |
| oldTLLI |  | TLLI | OCTETSTRING[4] |
| newTLLI |  | TLLI |  |
| cipherKey |  | BITSTRING[64] | (see note 1) |
| cipherKey128 |  | BITSTRING[128] | (see note 1) |
| cipherAlgorithm |  | GPRS_CipherAlg | BITSTRING[3], see 3GPP TS 24.008 [9], clause 10.5.5.3 |
| Detailed Comments |  | This ASP is used to assign, change, or unassign the TLLI, the ciphering key (Kc) and the ciphering algorithm. 1. The oldTLLI and newTLLI parameters shall be interpreted as follows: - If oldTLLI = all 1's and newTLLI  all 1's then newTLLI is assigned and used when (re-)transmitting LLC frames. If an oldTLLI  all 1's was assigned to the LLME, then oldTLLI is unassigned. Only newTLLI is accepted when received from the peer. It shall be treated as a TLLI change. If oldTLLI = all 1's was assigned to the LLME, then this shall be treated as a TLLI assignment, and this ASP shall be the first ASP sent to the SS in order to enable LLC to process requests from layer 3. - If oldTLLI  all 1's and newTLLI  all 1's then oldTLLI and newTLLI are assigned, and newTLLI shall be used when (re-)transmitting LLC frames. Both oldTLLI and newTLLI shall be accepted when received from the peer. It shall be treated as a TLLI change. - If oldTLLI  all 1's and newTLLI = all 1's then oldTLLI shall be unassigned. It shall be treated as a TLLI unassignment, and this ASP shall be the last ASP sent to the SS in order to disable LLC to not process requests from layer 3 any longer. 2. Kc and Ciphering Algorithm are associated with newTLLI (and with oldTLLI if assigned): - If Ciphering Algorithm indicates no ciphering, then the ciphering function shall be disabled. - Otherwise, the ciphering function shall be enabled. If a Ciphering Algorithm was already associated with newTLLI or oldTLLI, then the new Kc shall replace the previous Kc, and Ciphering Algorithm shall replace the previous algorithm selection. All I frames, and UI frames with the E bit set to 1, shall use the new Kc and algorithm for ciphering. All unacknowledged I frames shall be ciphered using the new Kc and algorithm before retransmission. As an implementation option, the previous Kc and algorithm may be used to decipher received frames. NOTE 1: The value of cipherAlgorithm dictates which of cipherKey or cipherKey128 shall be used by the SS. |  |

| ASP Name | G_CLLC_Assign_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |  |
| Comments | the ASP is used to get confirmation of G_CLLC_Assign_REQ |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| lLMEId |  |  | LLMEId | Logical Layer Management Entity Id |
| Detailed Comments |  |  |  |  |

7.3.4.3.2.5	Void

### 7.3.5	A-GPS Upper tester, PCO and ASP definitions

This clause is for A-GPS test model in Release 99, the A-GNSS test model is referred to 3GPP TS 37.571-4 [64].

#### 7.3.5.1	Upper tester

In order to perform A-GPS test, an Upper Tester is defined to have two basic functional unites:

-	Satellite simulator generating and broadcasting satellite signals;

-	Assistance data source storing the data simulating a number of pre-defined GPS test scenarios.

Under the TTCN command, the upper tester loads a pre-defined or re-loads another pre-defined GPS test scenario to the satellite simulator. The generated satellite signals shall simulate a sufficient number satellites. The signal shall be sufficiently strong, in order to enable the UE to do the positioning measurement.

The SS also sends the GPS assistance data to the UE through RRC signalling to facilitate the UE acquiring and tracking satellites. Such assistance data shall be consistent to within 2 seconds with the satellite signals generated.

The assistance data source shall provide the assistance data consistent to +1/-0 second with the GPS test scenario currently running in the satellite simulator (i.e. the data shall be up to 1 second in advance of the scenario); this allows for a further 2 seconds of latency in the SS.

#### 7.3.5.2	SV PCO

The upper tester has an ASP interface through a PCO in type of SatS PCO defined in the table.

| PCO Type Declarations |  |
| --- | --- |
| PCO Type | SatS |
| Role | UT |
| Comments | PCO type used for the Satellite Simulator and the assistance data source in the upper tester |

| PCO Declarations |  |
| --- | --- |
| PCO Name | SV |
| PCO Type | SatS |
| Role | UT |
| Comments | Carry control, configuration and GPS assistance data to/from satellite simulator and assistance data source in the upper tester |

#### 7.3.5.3	A-GPS Primitives

The primitives at SV PCO are used to

-	load a pre-defined  test scenario into the satellite simulator;

-	start or stop generating and broadcasting satellite signals from the satellite simulator;

-	retrieve the GPS assistance data from assistance data source, the table below is the summary of these primitives.

| Primitive | Parameters | Use |
| --- | --- | --- |
| Satellite_StartStop_REQ | Mode: start or stop | Start or stop generating satellite signals in the satellite simulator. |
| Satellite_StartStop_CNF | Null | Confirm the Satellite_StartStop_Req. |
| Load_ GPS_Scenario_REQ | GPS test scenario number | Requests to load a pre-defined test scenario into the satellite simulator |
| Load_ GPS_Scenario_CNF | Null | Confirm the load_GPS_Scenario_Req |
| Retri_GPS_AssistanceData_REQ | Indication of which assistance data elements to be retrieved | Request the assistance data source to provide the next (in time) valid GPS assistance data elements. |
| Retri_GPS_AssistanceData_CNF | GPS assistance data elements | Return the GPS assistance data retrieved |

##### 7.3.5.3.1	Control ASP Type Definition

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | Satellite_StartStop_CNF |
| PCO Type | SatS |
| Comment | To confirm successful of Satellite_StartStop_REQ |
| Type Definition |  |
| SEQUENCE { confirm NULL } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | Satellite_StartStop_REQ |
| PCO Type | SatS |
| Comment | To start or stop generating satellite signals in the satellite simulator "start" starts broadcasting satellite signals; "stop" stops broadcasting satellite signals If used for start (0), this ASP shall be called 2 s. after the ASP Load_GPS_Scenario_REQ for loading or reloading a pre-defined GPS test scenario. |
| Type Definition |  |
| SEQUENCE { satelliteSignals ENUMERATED {startSatSignal (0), stopSatSignal (1)} } |  |

##### 7.3.5.3.2	Data ASP Type Definition

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | Load_ GPS_Scenario_CNF |
| PCO Type | SatS |
| Comment | To confirm the Load_GPS_Scenario_REQ |
| Type Definition |  |
| SEQUENCE { dummy NULL } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | Load_ GPS_Scenario_REQ |
| PCO Type | SatS |
| Comment | To request the upper tester to load the required pre-defined GPS test scenario. |
| Type Definition |  |
| SEQUENCE { gps_Scenario INTEGER(0..31) } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | Retri_GPS_AssistanceData_CNF |
| PCO Type | SatS |
| Comment | To return the next valid GPS assistance data elements as requested in the Retri_GPS_AssistanceData_REQ. The returned GPS assistance data (all or part) will be used as assistance data sent to UE in RRC messages for A-GPS positioning. The returned Almanac information is split into two fields: - Almanac for satellites 1 to 12:in 'assistanceData' together with other information; - Almanac for satellites 13 to 24: in 'almanacSat13To24' |
| Type Definition |  |
| SEQUENCE { assistanceData UE_Positioning_GPS_AssistanceData, almanacSat13To24 AlmanacSatInfoList OPTIONAL } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | Retri_GPS_AssistanceData_REQ |
| PCO Type | SatS |
| Comment | To request the GPS assistance data source to provide the next valid GPS assistance data elements, consistent with the running GPS test scenario. The parameter navModelAddDataRequest in the assistanceDataReq shall be omitted. Another three parameters, utcModelRequest, dgpsCorrectionsRequest and realTimeIntegrityRequest in the assistanceDataReq are not applicable and shall be set to "FALSE". |
| Type Definition |  |
| SEQUENCE { assistanceDataReq UE_Positioning_GPS_AdditionalAssistanceDataRequest } |  |

### 7.3.6	ROHC test model and ASP

#### 7.3.6.1	ROHC test method

The ROHC test architecture illustrates the relationship between various compressor and de-compressor entities. No de-compressor is implemented in PDCP on the uplink direction in the SS.

Figure 7.3.6.1: ROHC testing architecture

#### 7.3.6.2	ASP and PCO for control primitives transmission and reception

##### 7.3.6.2.1	PCO definition

Table 7.3.6.2.1: PCO CPDCP declaration

| PCO Definition |  |
| --- | --- |
| PCO Name | CPDCP |
| PCO Type | CSAP |
| Role | LT |
| Comment | Provide PDCP Layer configuration service |

##### 7.3.6.2.2	CPDCP_Config

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPDCP_Config_CNF |
| PCO Type | CSAP |
| Comment | For PDCP emulator to report that a previous attempt to setup, reconfigure or release a logical channel is successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPDCP_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to configure PDCP entity. Setup is used for creation of the PDCP instances or the PDCP resources. Release is used for free the all PDCP resources. The reconfiguration is to change the PDCP parameters. PDCP entity does not belong to a particular cell, the cellId shall assign the value -1. the routingInfo is RB identity which is used to connect this PDCP entity to the RLC entity with the same RB identity and it is also used for PDCP_DATA_REQ ASP to transmit data through this PDCP entity. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, ratType RatType, configMessage CHOICE { setup SS_PDCP_Info, reconfigure SS_PDCP_InfoReconfig, release NULL } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_PDCP_Info |
| Comment | When configuring downlink direction of the SS, the UL_RFC3095-r4 shall be used; when configuring uplink direction of the SS the DL_RFC3095-r4 shall be used. |
| Type Definition |  |
| SEQUENCE { pDCP_ROHC_TargetMode PDCP_ROHC_TargetMode OPTIONAL, pDCP_Info PDCP_Info_r4, cSVoHS_Info CS_HSPA_Information OPTIONAL -- Rel-7 or later } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_PDCP_InfoReconfig |
| Comment | When configuring downlink direction of the SS, the UL_RFC3095-r4 shall be used; when configuring uplink direction of the SS the DL_RFC3095-r4 shall be used. |
| Type Definition |  |
| SEQUENCE { pDCP_ROHC_TargetMode PDCP_ROHC_TargetMode OPTIONAL, pDCP_Info PDCP_InfoReconfig_r4, cSVoHS_Info CS_HSPA_Information OPTIONAL -- Rel_7 or later } |  |

##### 7.3.6.2.3	CPDCP_ComProtocolControl

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPDCP_ComProtocolControl_CNF |
| PCO Type | CSAP |
| Comment | For PDCP emulator to report that a previous attempt to control the compression protocol is successful. |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo } |  |

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CPDCP_ComProtocolControl_REQ |
| PCO Type | CSAP |
| Comment | To request to reinitialize/context-relocate the compression protocol. PDCP entity does not belong to a particular cell; the cellId shall assign the value -1. The routingInfo is RB identity which is used to connect this PDCP entity to the RLC entity with the same RB identity and it is also used for PDCP_DATA_REQ ASP to transmit data through this PDCP entity. For reinitialization: - Configured compression parameters remain valid. - All compression state information is initialized. - The PDCP sequence numbers are not changed. - Actions specified in section 6.3.1 of RFC 3095 [54]. For contextRelocation: - initialize the context with the parameter in the ASP - valid for RFC3095 compression only |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(-1..63), routingInfo RoutingInfo, ratType RatType, controlMessage CHOICE { reinitialisation NULL, contextRelocation Context } } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | Context |
| Comment | Adapted from ASN1 type: RFC3095_Context_List |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxRFC3095_CID)) OF SEQUENCE { dl_RFC3095_Context DL_RFC3095_Context OPTIONAL, ul_RFC3095_Context UL_RFC3095_Context OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | DL_RFC3095_Context |
| Comment | For the compressor in the SS |
| Type Definition |  |
| SEQUENCE { rfc3095_Context_Identity INTEGER (0..16383), dl_mode ENUMERATED {u, o, r}, dl_ref_ir OCTET STRING ( SIZE (1..3000)), dl_ref_time INTEGER (0..4294967295) OPTIONAL, dl_curr_time INTEGER (0..4294967295) OPTIONAL, dl_syn_offset_id INTEGER (0..65535) OPTIONAL, dl_syn_slope_ts INTEGER (0..4294967295) OPTIONAL, dl_dyn_changed BOOLEAN } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | UL_RFC3095_Context |
| Comment | For the de-compressor in the SS |
| Type Definition |  |
| SEQUENCE { rfc3095_Context_Identity INTEGER (0..16383), ul_mode ENUMERATED {u, o, r}, ul_ref_ir OCTET STRING ( SIZE (1..3000)), ul_ref_time INTEGER (0..4294967295) OPTIONAL, ul_curr_time INTEGER (0..4294967295) OPTIONAL, ul_syn_offset_id INTEGER (0..65535) OPTIONAL, ul_syn_slope_ts INTEGER (0..4294967295) OPTIONAL, ul_ref_sn_1 INTEGER (0..65535) OPTIONAL } |  |

#### 7.3.6.3	ASP and PCO for data transmission and reception

##### 7.3.6.3.1	PCO definition

Table 7.3.6.3.1: PCO PDCP declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Name | PDCP |
| PCO Type | DSAP |
| Role | LT |
| Comment | Provide PDCP data transfer service |

##### 7.3.6.3.2	PDCP_DATA

| ASP Name | PDCP_DATA_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To request to transmit data (PDCP SDU) |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| rB_Id |  |  | SS_RB_Identity | Radio bearer identifier |
| pDCP_SDU |  |  | PDU | IPv4/UDP/RTP or IPv6/UDP/RTP PDUs |
| Detailed Comments |  |  |  |  |

| ASP Name | PDCP_DATA_IND |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To indicate to receive data (PDCP SDU) |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| rB_Id |  |  | SS_RB_Identity | Radio bearer identifier |
| pDCP_SDU |  |  | PDU | IPv4/UDP/RTP or IPv6/UDP/RTP PDUs |
| Detailed Comments |  |  |  |  |

##### 7.3.6.3.3	PDCP_DL_FeedBack

| ASP Name | PDCP_DL_FeedBack_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | For the SS to confirm a previous PDCP_DL_FeedBack_REQ. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| rB_Id |  |  | SS_RB_Identity | Radio bearer identifier |
| Detailed Comments |  |  |  |  |

| ASP Name | PDCP_DL_FeedBack_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To request the SS to send a feedback packet to the compressor in the UE. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| rB_Id |  |  | SS_RB_Identity | Radio bearer identifier |
| feedBackPacket1 |  |  | FeedBackPacket1 | either of feedBackPacket1 or feedBackPacket2 is presented |
| feedBackPacket2 |  |  | FeedBackPacket2 |  |
| Detailed Comments |  |  |  |  |

| ASP Name | PDCP_UL_FeedBack_CNF |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | For the SS to confirm a previous PDCP_UL_FeedBack_REQ. |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| rB_Id |  |  | SS_RB_Identity | Radio bearer identifier |
| Detailed Comments |  |  |  |  |

| ASP Name | PDCP_UL_FeedBack_REQ |  |  |  |
| --- | --- | --- | --- | --- |
| PCO Type | DSAP |  |  |  |
| Comments | To request the SS to pass a feedback packet received on uplink to the compressor in the SS. When a feedback packet is received at the uplink direction, TTCN uses this ASP to pass the received feedback packet to the downlink compressor of the SS |  |  |  |
| Parameter Name |  |  | Parameter Type | Comments |
| cellId |  |  | CellId |  |
| rB_Id |  |  | SS_RB_Identity | Radio bearer identifier |
| feedBackPacket1 |  |  | FeedBackPacket1 | either of feedBackPacket1 or feedBackPacket2 is presented |
| feedBackPacket2 |  |  | FeedBackPacket2 |  |
| Detailed Comments |  |  |  |  |

| Type Name | FeedBackPacket1 |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | For ROHC RTP |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| feedBackType | BITSTRING[5] |  | '11110'B |
| code | BITSTRING[3] |  |  |
| size | OCTETSTRING[1] |  | Present if code = 0 |
| addCIDpart1 | BITSTRING[4] |  | Present if small CID and CID <>0; this shall be "1110" |
| addCIDpart2 | BITSTRING[4] |  | Present if small CID and CID <>0; This is the CID value |
| largeCID | OCTETSTRING[1..2] |  | Present if large CID |
| sequenceNumber | BITSTRING[8] |  |  |
| Detailed Comments |  |  |  |

| Type Name | FeedBackPacket2 |  |  |
| --- | --- | --- | --- |
| Encoding Variation |  |  |  |
| Comments | For ROHC RTP |  |  |
| Element Name | Type Definition | Field Encoding | Comments |
| feedBackType | BITSTRING[5] |  | '11110'B |
| code | BITSTRING[3] |  |  |
| size | OCTETSTRING[1] |  | Present if code = 0 |
| addCIDpart1 | BITSTRING[4] |  | Present if small CID and CID <>0; this shall be "1110" |
| addCIDpart2 | BITSTRING[4] |  | Present if small CID and CID <>0; This is the CID value |
| largeCID | OCTETSTRING[1..2] |  | Present if large CID |
| ackType | BITSTRING[2] |  | 0 = ACK; 1 = NACK; 2 = STATIC-NACK; 3 = reserved |
| mode | BITSTRING[2] |  |  |
| sequenceNumber | BITSTRING[12] |  |  |
| optioncode | BITSTRING[4] |  |  |
| optionLength | BITSTRING[4] |  | Only 0 and 1 are valid values |
| optionData | BITSTRING[8] |  | Present if optionLength = 1 |
| Detailed Comments | See section 5.7.6.1 of RFC 3095 [54] for the coding of optionalLength and optionalData |  |  |

### 7.3.7	Handling RLP for CS non-transparent data

After the establishment of a CS non-transparent data call during the test, the UE may attempt to initiate a Radio Link Protocol (RLP) connection or start XID exchange before the RLP connection if the UE has an RLP installed. The RLP frames exchanges shall be handled by the SS, in order to carry on the test. Otherwise, the UE may disconnect the CS call.

For the purpose of handling UE originated RLP frames the SS has installed an RLC codec. The RLP codec supports RLP Version 0,1, and 2, detects the version number at the first XID exchange. Without any prior XID exchange, the default version 0 applies. According to the RLP version number, the codec decodes / encodes U, S, I+S frames including header, information and FCS (Frame Check Sequence) fields. The RLP has a fixed bit frame size that is set to 576 bits. The SS RLP codec calculates the FCS value and inserts it in the FCS fields in each DL RLP frames. The FCS values in the UL frames are irrelevant in the TTCN. The I frames are decoded and acknowledged in TTCN, and the other RLP frames are decoded and discarded in the default behaviour trees in TTCN.

#### 7.3.7.1	UTRAN cell

In a UTRAN cell, an RLP codec can be activated on the top of the SS TM RLC entity if the RLP protocol is applied to the UE. In addition of PCO TM, two new PCOs are defined. The PCO CRLP is used for the control and configuration of the RLP codec and the PCO RLP is used for the transferring of the RLP frames. PCO TM is not applied if PCO RLP is used in the test.

Figure 7.3.7.1: RLP codec model

An activate / deactivate command is required within the CRLP_Config_REQ to configure and activate the RLP codec and RLP PCO. The deactivate command switches off the RLP PCO and the SS discards all received RLP frames.

#### 7.3.7.2	GERAN cell

Figure 7.3.7.2: GERAN RLP codec model

After the establishment of a CS non-transparent data call in a GERAN cell during the test, the UE may attempt to initiate a Radio Link Protocol (RLP) connection or start XID exchange before the RLP connection if the UE has an RLP installed. The RLP frame exchanges shall be handled by the SS, in order to carry on the test. Otherwise, the UE may disconnect the CS call.

In a GERAN cell, an RLP codec can be activated on the traffic channel if the RLP protocol is applied to the UE. Two new PCOs are defined: G_CRLP is used for the control and configuration of the RLP codec and G_RLP is used for the transferring of the RLP frames in the GERAN cell.

For the purposes of interRAT handover testing, only the frame length of 576 bits is required. The 240 bits frame length and the REMAP function are not required.

#### 7.3.7.3	ASP and PCO for control primitives

Table 7.3.7.3: PCO CRLP

| PCO Definition |  |
| --- | --- |
| PCO Name | CRLP |
| PCO Type | CSAP |
| Role | LT |
| Comment | Control RLP codec. |

| ASP Name | CRLP_Config_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | CSAP |  |  |
| Comments | The ASP is used to activate or deactivate the RLP codec. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The cell which the RB identity belongs to |
| rB_Identity |  | SS_RB_Identity | Identifier of the RB identity in the SS |
| command |  | INTEGER | 0: activate 1: deactivate |
| Detailed Comments |  |  |  |

| ASP Name | CRLP_Config_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | CSAP |  |  |
| Comments | For RLP codec to confirm that a previous attempt to activate or deactivate has been successful. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The cell which the RB identity belongs to |
| rB_Identity |  | SS_RB_Identity | Identifier of the RB identity in the SS |
| Detailed Comments |  |  |  |

| PCO Definition |  |
| --- | --- |
| PCO Name | G_CRLP |
| PCO Type | G_CSAP |
| Role | LT |
| Comment | Control RLP codec in GERAN cell |

| ASP Name | G_CRLP_Config_REQ |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | The ASP is used to activate or deactivate the RLP agent in the GERAN cell and establish a pipe between the TTCN and the RLP agent on the TCH. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The cell which the RB identity belongs to |
| channelId |  | PhysicalChId | Identifier of the TCH in the SS |
| command |  | INTEGER | 0: activate 1: deactivate |
| Detailed Comments |  |  |  |

| ASP Name | G_CRLP_Config_CNF |  |  |
| --- | --- | --- | --- |
| PCO Type | G_CSAP |  |  |
| Comments | For the RLP agent to confirm that a previous attempt to activate or deactivate has been successful. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId | The cell which the TCH belongs to |
| channelId |  | PhysicalChId | Identifier of the TCH in the SS |
| Detailed Comments |  |  |  |

#### 7.3.7.4	ASP and PCO for data transmission and reception

Table 7.3.7.4: PCO RLP declaration

| PCO Type Definition |  |
| --- | --- |
| PCO Name | RLP |
| PCO Type | DSAP |
| Role | LT |
| Comment | Carry RLP frame. |

| ASP Name | RLP_FrameReq |  |  |
| --- | --- | --- | --- |
| PCO Type | DSAP |  |  |
| Comments | The ASP is used to request the transmission of the RLP frame. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| rB_Identity |  | SS_RB_Identity | Identifier of the RB identity in the SS |
| data |  | PDU | Meta type PDU |
| Detailed Comments |  |  |  |

| ASP Name | RLP_FrameInd |  |  |
| --- | --- | --- | --- |
| PCO Type | DSAP |  |  |
| Comments | The ASP is used to indicate the reception of an RLP frame. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| rB_Identity |  | SS_RB_Identity | Identifier of the RB identity in the SS |
| data |  | PDU | Meta type PDU |
| Detailed Comments |  |  |  |

| PCO Type Definition |  |
| --- | --- |
| PCO Name | G_RLP |
| PCO Type | G_DSAP |
| Role | LT |
| Comment | Carry RLP frame. |

| ASP Name | G_RLP_FrameReq |  |  |
| --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |
| Comments | The ASP is used to request the transmission of the RLP frame. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| channelId |  | PhysicalChId | Identifier of the TCH in the SS |
| data |  | PDU | Meta type PDU |
| Detailed Comments |  |  |  |

| ASP Name | G_RLP_FrameInd |  |  |
| --- | --- | --- | --- |
| PCO Type | G_DSAP |  |  |
| Comments | The ASP is used to indicate the reception of an RLP frame. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| cellId |  | CellId |  |
| channelId |  | PhysicalChId | Identifier of the TCH in the SS |
| data |  | PDU | Meta type PDU |
| Detailed Comments |  |  |  |

# 8	Design Considerations

## 8.1	Channel mapping

Figure 8.1 shows the channel type mapping that is used for the configuration of the SS.

Figure 8.1: Channel mapping in SS-

## 8.2	Channel and RB identity

The TTCN addresses the TTCN tester by using a channel identifier:

-	Either Physical channel identifier (PhyCh id); or

-	Transport channel identifier (TrCh id); or

-	Radio bearer identifier (RB id).

The selected channel identifier identifies uniquely:

-	a channel within a cell;

-	a total path of the address in the lower layers concerned.

Having taken out the cell id and PCO id (AM, UM and TM), a complete address, as RoutingInfo in the RRC ASP definition, should have at least five fields, CN domain id, RB id, LogCH id, TrCH id and PhyCH id. For simplified application of CHOICE of the routing information, a TTCN writer must carefully follow a number of rules assigning the channel identifiers.

General requirements:

-	a structured scheme of planning all channel identifiers assigned;

-	the scheme shall meet the requirements for all test cases in 3GPP TS 34.123-1 [1] including TDD channels;

-	the scheme can apply to all radio bearer configurations in 3GPP TS 34.108 [3], clause 6.10;

-	a clear multiplex mapping between a PhyCH id to TrCH ids and a TrCH id to LogCH ids, RB ids is needed.

Requirements on identification of RB in a test case:

-	unique identification of the individual SRBs;

-	unique identification of the individual sub-flows of a RABs in CS and PS domain.;

-	an assigned RB id can represent UL and DL.

Requirements on identification of Logical Channel in a test case:

-	it is an instance number of the individual logical channel; and

-	uniquely identifies among all the Logical Channel mapped onto a Transport Channel.

Requirements on identification of Transport Channel in a test case:

-	unique identification of the individual Transport Channel;

-	assign different identities for UL and DL of a same Transport Channel type;

-	the order of the Transport Channel id assigned in a cell shall follow the TFCS definitions in the 3GPP TS 34.108 [3], clause 6.10.

EXAMPLE:	Transport Channel ids are assigned in the ascending order for (RABsubflow#1, RABsubflow#2, RABsubflow#3, 64kRAB, DCCH).

Requirements on identification of Physical Channel in a test case:

-	unique identification of the individual Physical Channel;

-	assign different identities for UL and DL of a same Physical Channel type;

-	each S-CCPCH or PRACH has a unique identifier;

-	for 2 Mbps PS data radio link (in case of demux of a Transport Channel), three DPCH are needed for high-speed data. A single Physical Channel id is assigned to a bundle of the three physical channels.

Table 8.2 shows which type of channel identity is chosen for the individual primitives. In table 8.2, the ASN.1 primitives use a CHOICE type for channel identity, while TTCN primitives use an explicit channel identity.

Table 8.2: Primitives and the associated channel identity type

| Primitive name | Channel Identity | Releases |
| --- | --- | --- |
| ASN.1 Primitives |  |  |
| CPHY_AICH_AckModeSet_CNF | Physical Channel Identity |  |
| CPHY_AICH_AckModeSet_REQ | Physical Channel Identity |  |
| CPHY_Cell_Config_CNF | No Routing Info Field Present |  |
| CPHY_Cell_Config_REQ | No Routing Info Field Present |  |
| CPHY_Cell_Ini_CNF | No Routing Info Field Present |  |
| CPHY_Cell_Ini_REQ | No Routing Info Field Present |  |
| CPHY_Cell_TxPower_Modify_CNF | No Routing Info Field Present |  |
| CPHY_Cell_TxPower_Modify_REQ | No Routing Info Field Present |  |
| CPHY_Cell_Release_CNF | No Routing Info Field Present |  |
| CPHY_Cell_Release_REQ | No Routing Info Field Present |  |
| CPHY_DetectTFCI_CNF | Physical Channel Identity |  |
| CPHY_DetectTFCI_IND | Physical Channel Identity |  |
| CPHY_DetectTFCI_REQ | Physical Channel Identity |  |
| CPHY_Frame_Number_CNF | Physical Channel Identity |  |
| CPHY_Frame_Number_REQ | Physical Channel Identity |  |
| CPHY_SFN_CNF | Physical Channel Identity | Rel-6 or later |
| CPHY_SFN_REQ | Physical Channel Identity | Rel-6 or later |
| CPHY_MBMS_MICH_q_CNF | Physical Channel Identity | Rel-6 or later |
| CPHY_MBMS_MICH_q_REQ | Physical Channel Identity | Rel-6 or later |
| CPHY_MBMS_NI_CNF | Physical Channel Identity | Rel-6 or later |
| CPHY_MBMS_NI_REQ | Physical Channel Identity | Rel-6 or later |
| CPHY_Out_of_Sync_IND | Physical Channel Identity |  |
| CPHY_PRACH_Measurement_CNF | Physical Channel Identity |  |
| CPHY_PRACH_Measurement_REQ | Physical Channel Identity |  |
| CPHY_PRACH_Measurement_Report_IND | Physical Channel Identity |  |
| CPHY_RL_Modify_CNF | Physical Channel Identity |  |
| CPHY_RL_Modify_REQ | Physical Channel Identity |  |
| CPHY_RL_Release_CNF | Physical Channel Identity |  |
| CPHY_RL_Release_REQ | Physical Channel Identity |  |
| CPHY_RL_Setup_CNF | Physical Channel Identity |  |
| CPHY_RL_Setup_REQ | Physical Channel Identity |  |
| CPHY_Sync_IND | Physical Channel Identity |  |
| CPHY_TrCH_Config_CNF | Physical Channel Identity |  |
| CPHY_TrCH_Config_REQ | Physical Channel Identity |  |
| CPHY_TrCH_Release_CNF | Physical Channel Identity |  |
| CPHY_TrCH_Release_REQ | Physical Channel Identity |  |
| CPHY_HS_DPCCH_AckNack_CNF | No Routing Info Field Present | Rel-5 or later |
| CPHY_HS_DPCCH_AckNack_REQ | No Routing Info Field Present | Rel-5 or later |
| CPHY_HS_DPCCH_AckNack_IND | No Routing Info Field Present | Rel-5 or later |
| CPHY_HS_DPCCH_CQI_CNF | No Routing Info Field Present | Rel-5 or later |
| CPHY_HS_DPCCH_CQI_REQ | No Routing Info Field Present | Rel-5 or later |
| CPHY_HS_DPCCH_CQI_IND | No Routing Info Field Present | Rel-5 or later |
| CPHY_HS_DPCCH_CQI_DC_CNF | No Routing Info Field Present | Rel-8 or later |
| CPHY_HS_DPCCH_CQI_DC_REQ | No Routing Info Field Present | Rel-8 or later |
| CPHY_HS_DPCCH_CQI_DC_IND | No Routing Info Field Present | Rel-8 or later |
| CPHY_HS_DSCH_CRC_Mode_CNF | Physical Channel Identity | Rel-5 or later |
| CPHY_HS_DSCH_CRC_Mode_REQ | Physical Channel Identity | Rel-5 or later |
| CPHY_HS_SICH_AckNack_CNF | No Routing Info Field Present | Rel-5 or later (LCR TDD) |
| CPHY_HS_SICH_AckNack_REQ | No Routing Info Field Present | Rel-5 or later (LCR TDD) |
| CPHY_HS_SICH_AckNack_IND | No Routing Info Field Present | Rel-5 or later (LCR TDD) |
| CPHY_HS_SICH_CQI_CNF | No Routing Info Field Present | Rel-5 or later (LCR TDD) |
| CPHY_HS_SICH_CQI_REQ | No Routing Info Field Present | Rel-5 or later (LCR TDD) |
| CPHY_HS_SICH_CQI_IND | No Routing Info Field Present | Rel-5 or later (LCR TDD) |
| CPHY_UL_PowerModify_CNF | Physical Channel Identity |  |
| CPHY_UL_PowerModify_REQ | Physical Channel Identity |  |
| CMAC_BMC_Scheduling_CNF | Physical Channel Identity |  |
| CMAC_BMC_Scheduling_REQ | Physical Channel Identity |  |
| CMAC_Ciphering_Activate_CNF | Physical Channel Identity of DPCH |  |
| CMAC_Ciphering_Activate_REQ | Physical Channel Identity of DPCH |  |
| CMAC_Config_CNF | Physical Channel Identity |  |
| CMAC_Config_REQ | Physical Channel Identity |  |
| CMAC_FACH_MeasOccas_CNF | Physical Channel Identity |  |
| CMAC_FACH_MeasOccas_REQ | Physical Channel Identity |  |
| CMAC_PAGING_Config_CNF | Physical Channel Identity |  |
| CMAC_PAGING_Config_REQ | Physical Channel Identity |  |
| CMAC_Restriction_CNF | Physical Channel Identity |  |
| CMAC_Restriction_REQ | Physical Channel Identity |  |
| CMAC_SecurityMode_Config_CNF | No Routing Info Field Present (applies to all RB Ids) |  |
| CMAC_SecurityMode_Config_REQ | No Routing Info Field Present (applies to all RB Ids) |  |
| CMAC_SequenceNumber_CNF | Physical Channel Identity |  |
| CMAC_SequenceNumber_REQ | Physical Channel Identity |  |
| CMAC_SYSINFO_Config_CNF | RB Identity |  |
| CMAC_SYSINFO_Config_REQ | RB Identity |  |
| CMAC_MAChs_MACehs _Reset_CNF | No Routing Info Field Present | Rel-5 or later |
| CMAC_MAChs_MACehs _Reset_REQ | No Routing Info Field Present | Rel-5 or later |
| CMAC_MAChs_MACehs _HARQprocAsign_CNF | No Routing Info Field Present | Rel-5 or later |
| CMAC_MAChs_MACehs _HARQprocAsign_REQ | No Routing Info Field Present | Rel-5 or later |
| CMAC_MAChs_MACehs _TFRCconfigure_CNF | No Routing Info Field Present | Rel-5 or later |
| CMAC_MAChs_MACehs _TFRCconfigure_REQ | No Routing Info Field Present | Rel-5 or later |
| CMAC_MACehs_HS_SCCH_OrdersCNF | No Routing Info Field Present | Rel-7 or later |
| CMAC_MACehs_HS_SCCH_OrdersREQ | No Routing Info Field Present | Rel-7 or later |
| CMAC_MACe_Config_CNF | Node B Identity | Rel-6 or later |
| CMAC_MACe_Config_REQ | Node B Identity | Rel-6 or later |
| CMAC_MACe_NodeB_CellMapping_CNF | Node B Identity | Rel-6 or later |
| CMAC_MACe_NodeB_CellMapping_REQ | Node B Identity | Rel-6 or later |
| CMAC_MACi_NodeB_CellMapping_CNF | Node B Identity | Rel-8 or later |
| CMAC_MACi_NodeB_CellMapping_REQ | Node B Identity | Rel-8 or later |
| CMAC_MACes_Config_CNF | No Routing Info Field Present | Rel-6 or later |
| CMAC_MACes_Config_REQ | No Routing Info Field Present | Rel-6 or later |
| CMAC_MACis_Config_CNF | No Routing Info Field Present | Rel-8 or later |
| CMAC_MACis_Config_REQ | No Routing Info Field Present | Rel-8 or later |
| CMAC_MACi_Config_CNF | No Routing Info Field Present | Rel-8 or later |
| CMAC_MACi_Config_REQ | No Routing Info Field Present | Rel-8 or later |
| CMAC_MACe_AG_CNF | Node B Identity | Rel-6 or later |
| CMAC_MACe_AG_REQ | Node B Identity | Rel-6 or later |
| CMAC_MACi_AG_CNF | Node B Identity | Rel-8 or later |
| CMAC_MACi_AG_REQ | Node B Identity | Rel-8 or later |
| CMAC_MACe_AckNack_CNF | Node B Identity | Rel-6 or later |
| CMAC_MACe_AckNack_REQ | Node B Identity | Rel-6 or later |
| CMAC_MACi_AckNack_CNF | Node B Identity | Rel-8 or later |
| CMAC_MACi_AckNack_REQ | Node B Identity | Rel-8 or later |
| CMAC_MACe_RG_CNF | Node B Identity | Rel-6 or later |
| CMAC_MACe_RG_REQ | Node B Identity | Rel-6 or later |
| CMAC_MACi_RG_CNF | Node B Identity | Rel-8 or later |
| CMAC_MACi_RG_REQ | Node B Identity | Rel-8 or later |
| CMAC_MACe_E_TFC_Restriction_CNF | Node B Identity | Rel-6 or later |
| CMAC_MACe_E_TFC_Restriction_REQ | Node B Identity | Rel-6 or later |
| CMAC_MACi_E_TFC_Restriction_CNF | Node B Identity | Rel-8 or later |
| CMAC_MACi_E_TFC_Restriction_REQ | Node B Identity | Rel-8 or later |
| CMAC_MACes_SI_IND | No Routing Info Field Present | Rel-6 or later |
| CMAC_MACes_SI_Config_CNF | No Routing Info Field Present | Rel-6 or later |
| CMAC_MACis_SI_IND | No Routing Info Field Present | Rel-8 or later |
| CMAC_MACis_SI_Config_CNF | No Routing Info Field Present | Rel-8 or later |
| CMAC_MBMS_ConfigInfo_CNF | Physical Channel Identity | Rel-6 or later |
| CMAC_MBMS_ConfigInfo_REQ | Physical Channel Identity | Rel-6 or later |
| CRLC_Bind_TestData_TTI_CNF | No Routing Info Field Present |  |
| CRLC_Bind_TestData_TTI_REQ | No Routing Info Field Present |  |
| CRLC_Ciphering_Activate_CNF | No Routing Info Field Present (applies to all RB Ids) |  |
| CRLC_Ciphering_Activate_REQ | No Routing Info Field Present (applies to all RB Ids) |  |
| CRLC_MAC_I_Mode_CNF | RB Identity |  |
| CRLC_MAC_I_Mode_REQ | RB Identity |  |
| CRLC_Config_CNF | RB Identity |  |
| CRLC_Config_REQ | RB Identity |  |
| CRLC_Integrity_Activate_CNF | No Routing Info Field Present (applies to all RB Ids) |  |
| CRLC_Integrity_Activate_REQ | No Routing Info Field Present (applies to all RB Ids) |  |
| CRLC_Integrity_Failure_IND | RB Identity |  |
| CRLC_NotAckNxtRxSDU_CNF | RB Identity |  |
| CRLC_NotAckNxtRxSDU_REQ | RB Identity |  |
| CRLC_ProhibitRLC_Ack_CNF | RB Identity |  |
| CRLC_ProhibitRLC_Ack_REQ | RB Identity |  |
| CRLC_Resume_CNF | RB Identity (applies to all suspended RB Ids) |  |
| CRLC_Resume_REQ | RB Identity (applies to all suspended RB Ids) |  |
| CRLC_RRC_MessageSN_CNF | RB Identity |  |
| CRLC_RRC_MessageSN_REQ | RB Identity |  |
| CRLC_SecurityMode_Config_CNF | No Routing Info Field Present (applies to all RB Ids) |  |
| CRLC_SecurityMode_Config_REQ | No Routing Info Field Present (applies to all RB Ids) |  |
| CRLC_SendContinuousData_CNF | No Routing Info Field Present |  |
| CRLC_SendContinuousData_REQ | No Routing Info Field Present |  |
| CRLC_SendTestDataInOneMAC_Hs_PDU_CNF | RB Identity | Rel-5 or later |
| CRLC_SendTestDataInOneMAC_Hs_PDU_REQ | RB Identity | Rel-5 or later |
| CRLC_SequenceNumber_CNF | RB Identity |  |
| CRLC_SequenceNumber_REQ | RB Identity |  |
| CRLC_SetRRC_MessageSN_CNF | RB Identity |  |
| CRLC_SetRRC_MessageSN_REQ | RB Identity |  |
| CRLC_Set_Count_I_CNF | RB Identity |  |
| CRLC_Set_Count_I_REQ | RB Identity |  |
| CRLC_Status_Ind | RB Identity |  |
| CRLC_Suspend_CNF | RB Identity |  |
| CRLC_Suspend_REQ | RB Identity |  |
| CRLC_MTCH_Scheduling_REQ | RB Identity | Rel-6 or later |
| CRLC_MTCH_Scheduling_CNF | RB Identity | Rel-6 or later |
| CBMC_Config_CNF | RB Identity |  |
| CBMC_Config_REQ | RB Identity |  |
| RLC_AM_DATA_CNF | RB Identity |  |
| RLC_AM_DATA_IND | RB Identity |  |
| RLC_AM_DATA_REQ | RB Identity |  |
| RLC_TR_DATA_IND | RB Identity |  |
| RLC_TR_DATA_REQ | RB Identity |  |
| RLC_UM_ACCESSinfo_REQ | RB Identity | Rel-6 or later |
| RLC_UM_CriticalMCCHMsg_REQ | RB Identity | Rel-6 or later |
| RLC_UM_DATA_IND | RB Identity |  |
| RLC_UM_DATA_REQ | RB Identity |  |
| RLC_UM_MSCH_Msg_REQ | RB Identity | Rel-6 or later |
| RLC_TR_MACesDATA_IND | RB Identity | Rel-6 or later |
| RLC_TR_MACisDATA_IND | RB Identity | Rel-8 or later |
| TTCN Primitives |  |  |
| RLC_AM_TestDataInd | RB Identity |  |
| RLC_AM_TestDataReq | RB Identity |  |
| RLC_TR_TestDataInd | RB Identity |  |
| RLC_TR_TestDataReq | RB Identity |  |
| RLC_UM_TestDataInd | RB Identity |  |
| RLC_UM_TestDataReq | RB Identity |  |
| BMC_DataReq | RB Identity |  |

### 8.2.1	Physical channels

Table 8.2.1: Physical channel identities

| Type | Min. No. | Current Config. | Identities (value assigned) | Direction | Comment |
| --- | --- | --- | --- | --- | --- |
| P-CCPCH | 1 | 1 | tsc_P_CCPCH (4) | downlink | Primary Common Control Physical Channel. For Broadcasting System Information messages, using the Primary Scrambling Code for the Cell. |
| P-CPICH | 1 | 1 | tsc_P_CPICH (0) | downlink | Primary Common Pilot Channel using the Primary Scrambling Code for the Cell. |
| S-CPICH | 1 | 1 | tsc_S_CPICH (3) | downlink | Secondary Common Pilot Channel, used as the phase reference for some RF tests. |
| P-SCH | 1 | 1 | tsc_P_SCH (1) | downlink | Primary Synchronization Channel |
| S-SCH | 1 | 1 | tsc_S_SCH (2) | downlink | Secondary Synchronization Channel |
| S-CCPCH | 3 | 1 | tsc_S_CCPCH1 (5) tsc_S_CCPCH2 (10) tsc_S_CCPCH3 (13) | downlink | Secondary Common Control Physical Channel. |
| PICH | 1 | 1 | tsc_PICH1 (6) tsc_PICH2 (11) | downlink | To identify whether the UE should access the PCCH for Paging Messages. |
| AICH | 1 | 1 | tsc_AICH1 (7) tsc_AICH2 (12) | downlink | General Acquisition Indicator Channel, can be used for: - Acquisition Indicator Channel, for PRACH - Access Preamble Acquisition Indicator Channel (AP-ICH), for PCPCH - Collision-Detection/Channel- Assignment Indicator Channel (CD/CA-ICH), for PCPCH |
| DPCH | 3 | 1 | tsc_DL_DPCH1 (26) tsc_DL_DPCH2 (27) | downlink | Downlink Physical Data Channel. Layer 1 signalling is transmitted only on the first DPCH. This number is for the First Cell. Additional Cells may define a lower number which should be at least 1. |
| DPDCH | 1 | 1 | tsc_UL_DPCH1 (20) tsc_UL_DPCH2 (21) | uplink | Uplink Dedicated Physical Channel. A single DPCCH associated with all the DPDCHs used for Layer 1 signalling. |
| PRACH | 2 | 1 | tsc_PRACH1 (8) tsc_PRACH2 (9) | uplink | Physical Random Access Channel. |
| PCPCH | 1 | FFS |  | uplink | Physical Common Packet Channel. |
| CSICH | 1 | FFS |  | downlink | CPCH Status Indicator Channel |
| HS-PDSCH | 1 |  | tsc_HSPDSCH(18) | downlink | Rel-5 or later High speed physical downlink shared channel |
| E-AGCH | 1 |  | tsc_E_AGCH (14) | downlink | Rel-6 or later E-DCH Absolute Grant Channel |
| E-HICH | 1 |  | tsc_E_HICH (15) | downlink | Rel-6 or later E-DCH HARQ Acknowledgement Indicator Channel |
| E-RGCH | 1 |  | tsc_E_RGCH (16) | downlink | Rel-6 or later E-DCH Relative Grant Channel |
| E-DPDCH | 1 |  | tsc_E_DPCH (22) | uplink | Rel-6 or later Enhanced Dedicated Physical Channel |
| F-DPCH | 1 |  | tsc_F_DPCH (28) | downlink | Rel-6 or later Fractional Dedicated Physical Channel |
| MICH | 1 |  | tsc_MICH (29) | downlink | Rel-6 or later MBMS notification Indicator Channel |

The Physical Channel values 20 to 25 are assigned to uplink DPCHs and the values 26 to 31 are assigned to downlink DPCHs/ FDPCHs.

### 8.2.2	Transport channels

Table 8.2.2: Transport channel identities

| Type | Min. No. | Current Config. | Identities (value assigned) | Direction | Comments |
| --- | --- | --- | --- | --- | --- |
| BCH | 1 | 1 | tsc_BCH1 (11) | downlink |  |
| FACH | 1 | 1 | tsc_FACH1 (13) tsc_FACH2 (14) tsc_FACH3 (16) tsc_FACH4 (17) tsc_FACH5 (23) | downlink |  |
| PCH | 1 | 1 | tsc_PCH1 (12) tsc_PCH2 (30) | downlink |  |
| DCH | n | 4 | tsc_UL_DCH1 (1) tsc_UL_DCH2 (2) tsc_UL_DCH3 (3) tsc_UL_DCH4 (4) tsc_UL_DCH5 (5) tsc_UL_DCH6 (21) | uplink | tsc_UL_DCH1 for RAB1-1 or RAB1, tsc_UL_DCH2 for RAB1-2 or RAB2, tsc_UL_DCH3 for RAB1-3, tsc_UL_DCH4 RAB2, tsc_UL_DCH5 for SRB/RAB3, tsc_UL_DCH6 for SRB. |
| DCH | n | 4 | tsc_DL_DCH1 (6) tsc_DL_DCH2 (7) tsc_DL_DCH3 (8) tsc_DL_DCH4 (9) tsc_DL_DCH5 (10) tsc_DL_DCH6 (22) | downlink | tsc_DL_DCH1 for RAB1-1 or RAB1, tsc_DL_DCH2 for RAB1-2 or RAB2, tsc_DL_DCH3 for RAB1-3, tsc_DL_DCH4 for RAB2, tsc_DL_DCH5 for SRB, tsc_DL_DCH6 for SRB. |
| USCH | 1 | N/A | tsc_USCH1(20) | uplink | TDD only |
| DSCH | 1 | N/A | tsc_DSCH (19) | downlink |  |
| RACH | 2 | 1 | tsc_RACH1 (15) tsc_RACH2 (31) | uplink |  |
| CPCH | 1 | N/A | tsc_CPCH1(32) | uplink |  |
| FAUSCH | N/A | N/A | tsc_FAUSCH1(18) | uplink | Not in Release 99 |
| HSDSCH | 1 | 1 | N/A | downlink | Rel-5 or later |
| E-DCH | 1 | 1 | N/A | uplink | Rel-6 or later |

#### 8.2.2.1	Support of Default Configurations

In test cases using default configurations according to 3GPP TS 25.331 [21], clause 13.7, the configuration of the system simulator follows the same parameter values as defined for the UE side with the following exceptions:

-	UL/DL transport channel ids;

-	E-DCH MAC-d flow id / HS-DSCH MAC-d flow id.

As the transport channel identities and the MAC-d flow identities have the local significance, the TTCN implementations follow the test model.

### 8.2.3	Logical Channels

Table 8.2.3 shows the logical channels identities.

Table 8.2.3: Logical channel identities

| Type | Min. No. | Current Config. | Identities (value assigned) | Direction | Comments |
| --- | --- | --- | --- | --- | --- |
| BCCH_BCH | 1 | 1 | tsc_BCCH1 (1) | downlink |  |
| BCCH_FACH | 1 | 1 | tsc_BCCH6 (6) | downlink |  |
| CCCH | 1 | 1 | tsc_DL_CCCH5 (5) | downlink |  |
| CCCH | 1 | 2 | tsc_UL_CCCH5 (5) tsc_UL_CCCH_EFUL(15) tsc_UL_CCCH6 (6) | uplink |  |
| DCCH | 4 | 4 | tsc_DL_DCCH1 (1) tsc_DL_DCCH2 (2) tsc_DL_DCCH3 (3) tsc_DL_DCCH4 (4) tsc_DL_DCCH5 (5) | downlink | tsc_DL_DCCH1 for SRB1, tsc_DL_DCCH2 for SRB2, tsc_DL_DCCH3 for SRB3, tsc_DL_DCCH4 for SRB4, tsc_DL_DCCH5 for SRB5 |
| DCCH | 4 | 4 | tsc_UL_DCCH1 (1) tsc_UL_DCCH2 (2) tsc_UL_DCCH3 (3) tsc_UL_DCCH4 (4) | uplink | tsc_UL_DCCH1 for SRB1, tsc_UL_DCCH2 for SRB2, tsc_UL_DCCH3 for SRB3, tsc_UL_DCCH4 for SRB4 |
| PCCH | 1 | 2 | tsc_PCCH1 (1) tsc_PCCH2 (2) tsc_PCCH_FACH (11) | downlink |  |
| DTCH | n | 4 | tsc_UL_DTCH1 (7) tsc_UL_DTCH2 (8) tsc_UL_DTCH3 (9) tsc_UL_DTCH4 (10) tsc_UL_DTCH5 (13) | uplink | tsc_UL_DTCH1 for RAB1-1 or RAB 1, tsc_UL_DTCH2 for RAB1-2 or RAB 2, tsc_UL_DTCH3 for RAB1-3' tsc_UL_DTCH4 for RAB2, tsc_UL_DTCH5 for RAB3 |
| DTCH | n | 4 | tsc_DL_DTCH1 (7) tsc_DL_DTCH2 (8) tsc_DL_DTCH3 (9) tsc_DL_DTCH4 (10) | downlink | tsc_DL_DTCH1for RAB1-1 or RAB 1, tsc_DL_DTCH2 for RAB1-2 or RAB 2, tsc_DL_DTCH3 for RAB-3, tsc_DL_DTCH4 for RAB2 |
| CTCH | 1 | 2 | tsc_CTCH1 (11) tsc_CTCH2 (12) | downlink |  |
| MTCH | 1 | 4 | tsc_MTCH1(1) tsc_MTCH2(2) tsc_MTCH3(3) tsc_MTCH4(4) | downlink | MBMS_LogicalChIdentity |
| MCCH | 1 | 1 | tsc_MCCH1(1) | downlink | One and only one for each cell |
| MSCH | 0 | 1 | tsc_MSCH1(1) | downlink |  |

### 8.2.4	Radio bearers

Table 8.2.4.1: Radio Bearers

| Identities (value assigned) | Direction | Type | RLC mode | Service domain | Comments |
| --- | --- | --- | --- | --- | --- |
| tsc_RB_BCCH (-1) | downlink |  | TM | NA | BCCH-BCH |
| tsc_RB_PCCH (-2) | downlink |  | TM | NA | PCCH PCH |
| tsc_RB_BCCH_FACH (-3) | downlink |  | TM | NA | BCCH FACH |
| tsc_RB_2ndPCCH (-4) | downlink |  | TM | NA | Second PCCH PCH SCPCCH |
| tsc_RB_2ndCCCH (-5) | uplink |  | TM | NA | Second CCCH RACH PRACH |
| tsc_RB_MTCH_RLC_TR (-6) | downlink |  | TM | PS | Rel-6 or later For TM MTCH RLC tests |
| tsc_RB_UM_7_RLC (-10) | downlink | RAB | TM | CS | For UM RLC tests using 7 bit LIs |
| tsc_RB_UM_7_RLC (-10) | uplink | RAB | TM | CS | For UM RLC tests using 7 bit LIs |
| tsc_RB_UM_15_RLC (-11) | downlink | RAB | TM | CS | For UM RLC tests using 15 bit LIs |
| tsc_RB_UM_15_RLC (-11) | uplink | RAB | TM | CS | For UM RLC tests using 15 bit LIs |
| tsc_RB_AM_7_RLC (-12) | downlink | RAB | TM | CS | For AM RLC tests using 15 bit LIs |
| tsc_RB_AM_7_RLC (-12) | uplink | RAB | TM | CS | For AM RLC tests using 7 bit LIs |
| tsc_RB_AM_15_RLC (-13) | downlink | RAB | TM | CS | For AM RLC tests using 15 bit LIs |
| tsc_RB_AM_15_RLC (-13) | uplink | RAB | TM | CS | For AM RLC tests using 15 bit LIs |
| tsc_RB_DCCH_FACH_MAC (-14) | downlink | SRB3 | TM | CS | For MAC tests using DCCH mapped to FACH |
| tsc_RB_DCCH_FACH_MAC (-14) | uplink | SRB3 | TM | CS | For MAC tests using DCCH mapped to FACH |
| tsc_RB_DCCH_DCH_MAC (-15) | downlink | SRB3 | TM | CS | For MAC tests using DCCH mapped to DCH |
| tsc_RB_DCCH_FACH_MAC (-15) | uplink | SRB3 | TM | CS | For MAC tests using DCCH mapped to DCH |
| tsc_RB3_DCCH_RRC_(-16) | uplink | SRB3 | AM | CS or PS | For RRC test cases to route UL NAS messages |
| tsc_RB_CCCH_FACH_MAC (-18) | downlink | SRB0 | TM | CS or PS | For MAC test using downlink SRB0 on TM |
| tsc_RB_BCCH_FACH_RAB (-19) | downlink |  | TM | NA | BCCH FACH |
| tsc_RB_DTCH_E_DCH_MAC(-20) | uplink | RAB | TM | PS | For MAC_es_e tests |
| tsc_RB_DTCH_E_DCH_MAC1(-21) | uplink | RAB | TM | PS | For MAC_es_e tests |
| tsc_RB_DTCH_E_DCH_MAC2(-22) | uplink | RAB | TM | PS | For MAC_es_e tests |
| tsc_RB_MAC_HS(-25) | uplink | RAB | TM | PS | For MAC_hs/ehs tests |
| tsc_RB_MAC_HS(-25) | downlink | RAB | TM | PS | For MAC_hs/ehs tests |
| tsc_RB_MAC_ehs_26 (-26) | uplink | RAB | TM | PS | For MAC_ehs tests |
| tsc_RB_MAC_ehs_26 (-26) | downlink | RAB | TM | PS | For MAC_ehs tests |
| tsc_RB_MAC_ehs_27(-27) | uplink | RAB | TM | PS | For MAC_ehs tests |
| tsc_RB_MAC_ehs_27(-27) | downlink | RAB | TM | PS | For MAC_ehs tests |
| tsc_RB0 (0) | uplink | SRB0 | TM | CS or PS | The service domain for which the most recent security negotiation took place. CCCH |
| tsc_RB0_EFUL(-28) | uplink | SRB0 | TM | CS or PS | For CCCH on common E-DCH |
| tsc_RB0 (0) | downlink | SRB0 | UM | CS or PS | CCCH |
| tsc_RB1 (1) | uplink | SRB1 | UM | CS or PS | DCCH |
| tsc_RB1 (1) | downlink | SRB1 | UM | CS or PS | DCCH |
| tsc_RB2 (2) | uplink | SRB2 | AM | CS or PS | DCCH |
| tsc_RB2 (2) | downlink | SRB2 | AM | CS or PS | DCCH |
| tsc_RB3 (3) | uplink | SRB3 | AM | CS or PS | DCCH |
| tsc_RB3 (3) | downlink | SRB3 | AM | CS or PS | DCCH |
| tsc_RB4 (4) | uplink | SRB4 | AM | CS or PS | DCCH |
| tsc_RB4 (4) | downlink | SRB4 | AM | CS or PS | DCCH |
| tsc_RB5 (5) | uplink |  | TM |  | DCCH |
| tsc_RB5 (5) | downlink |  | TM |  | DCCH |
| tsc_RB_MCCH(8) | downlink | SRB | UM | PS | Rei-6 or later |
| tsc_RB_MCCH_RLC_TR(-8) | downlink | SRB | TM | PS | Rel-6 or later |
| tsc_RB_MSCH(9) | downlink | SRB | UM | PS | Rel-6 or later |
| tsc_RB10 (10) | uplink | RAB#1-1 | TM | CS | or RAB1 |
| tsc_RB10 (10) | downlink | RAB#1-1 | TM | CS | or RAB1 |
| tsc_RB11 (11) | uplink | RAB#1-2 | TM | CS | or RAB2 |
| tsc_RB11 (11) | downlink | RAB#1-2 | TM | CS | or RAB2 |
| tsc_RB12 (12) | uplink | RAB#1-3 | TM | CS |  |
| tsc_RB12 (12) | downlink | RAB#1-3 | TM | CS |  |
| tsc_RB13 (13) | uplink | RAB#2 | TM | CS |  |
| tsc_RB13 (13) | downlink | RAB#2 | TM | CS |  |
| tsc_RB_MTCH1(14) | downlink | RAB | UM | PS | Rel-6 or later, media contents |
| tsc_RB_MTCH2(15) | downlink | RAB | UM | PS | Rel-6 or later, media contents |
| tsc_RB_MTCH3(16) | downlink | RAB | UM | PS | Rel-6 or later, media contents |
| tsc_RB17 (17) | uplink | RAB#2 | AM | PS | Rel-5 or later, 2nd AM RAB for HS |
| tsc_RB17 (17) | downlink | RAB#2 | AM | PS | Rel-5 or later, 2nd AM RAB for HS |
| tsc_RB20 (20) | uplink | RAB#1 | AM | PS |  |
| tsc_RB20 (20) | downlink | RAB#1 | AM | PS |  |
| tsc_RB21 (21) | uplink | RAB#2 | UM | PS |  |
| tsc_RB21 (21) | downlink | RAB#2 | UM | PS |  |
| tsc_RB22 (22) | uplink | RAB#2 | AM | PS |  |
| tsc_RB22 (22) | downlink | RAB#2 | AM | PS |  |
| tsc_RB23 (23) | uplink | RAB#2 | AM | PS | 2nd AM RAB for PS |
| tsc_RB23 (23) | downlink | RAB#2 | AM | PS | 2nd AM RAB for PS |
| tsc_RB24 (24) | uplink | RAB#2 | AM | PS | 2nd AM RAB for PS |
| tsc_RB24 (24) | downlink | RAB#2 | AM | PS | 2nd AM RAB for PS |
| tsc_RB25 (25) | uplink | RAB#1 | AM | PS | Rel-5 or later: DTCH on DPCH associated HS-DSCH Rel-6 or later: DTCH on E-DCH |
| tsc_RB25 (25) | downlink | RAB#1 | AM | PS | Rel-5 or later DTCH on HS-DSCH |
| tsc_RB26 (26) | uplink | RAB#1 | UM | PS | Rel-5 or later |
| tsc_RB26 (26) | downlink | RAB#1 | UM | PS | Rel-5 or later |
| tsc_RB27 (27) | uplink | RAB#2 | UM | PS | Rel-5 or later |
| tsc_RB27 (27) | downlink | RAB#2 | UM | PS | Rel-5 or later |
| tsc_RB28 (28) | uplink | RAB#3 | AM | PS | Rel-5 or later |
| tsc_RB28 (28) | downlink | RAB#3 | AM | PS | Rel-5 or later |
| tsc_RB29 (29) | downlink | SRB0 | AM | PS | RB Id for Radio bearer that carries the 2nd CCCH in the DL |
| tsc_RB30 (30) | downlink |  | UM |  | CTCH FACH |
| tsc_RB31 (31) | downlink |  | UM |  | Second CTCH FACH |

The RB values 0 to 5 are used for the signalling bearers. The values 10 to 15 are assigned to the CS RAB sub-flows. The values 15 to 29 are assigned to the PS RAB sub-flows. The value 30 is assigned to the CBSMS/BMC service.

Table 8.2.4.2: RB identities mapping between 34.123-1 & 34.123-3

| RAB Combinations | 34.123-1 | 34.123-3 |
| --- | --- | --- |
| Single CS RAB | RB5 | tsc_RB10 |
|  | RB6 | tsc_RB11 |
|  | RB7 | tsc_RB12 |
| Single PS RAB | RB5 | tsc_RB20 |
|  | RB7 | tsc_RB20 |
|  | RB8 | tsc_RB20 |
| CS+PS Multi RABs | RB5 | tsc_RB10 |
|  | RB6 | tsc_RB11, tsc_RB20 |
|  | RB7 | tsc_RB12 |
|  | RB8 | tsc_RB20 |
|  | RB9 | tsc_RB22 |
| CS+CS Multi RABs | RB5 | tsc_RB10 |
|  | RB6 | tsc_RB11 |
|  | RB7 | tsc_RB12 |
|  | RB8 | tsc_RB13 |
| PS+PS Multi RABs | RB5 | tsc_RB20 |
|  | RB6 | tsc_RB22 |
|  | RB7 | tsc_RB20 |
|  | RB8 | tsc_RB24 |
| Single PS (HSDPA/HSUPA) RAB | RB5 | tsc_RB25 |
| PS+PS Multi (HSDPA) RABs | RB5 | tsc_RB26 |
|  | RB6 | tsc_RB27 |
|  | RB7 | tsc_RB25 |
|  | RB8 | tsc_RB28 |
|  | RB9 | tsc_RB17 |
| Single PS (HSUPA) RAB | RB5 | tsc_RB25 |
| CS + PS Multi (HSDPA/HSUPA) RABs | RB5 | tsc_RB10 |
|  | RB6 | tsc_RB11, tsc_RB25 |
|  | RB7 | tsc_RB12 |
|  | RB8 | tsc_RB25 |
|  | RB9 | tsc_RB17 |
| PS (HSUPA) + Speech Multi RABs | RB5 | tsc_RB10 |
|  | RB6 | tsc_RB11 |
|  | RB7 | tsc_RB12 |
|  | RB8 | tsc_RB25 |
| PS AM + PS AM Multi (HSUPA) RABs | RB5 | tsc_RB25 |
|  | RB6 | tsc_RB17 |
| PS UM + PS AM Multi (HSUPA) RABs | RB5 | tsc_RB27 |
|  | RB6 | tsc_RB25 |
| PS UM + PS AM + PS AM Multi (HSUPA) RABs | RB5 | tsc_RB27 |
|  | RB6 | tsc_RB25 |
|  | RB7 | tsc_RB17 |

### 8.2.5	Scrambling and channelization codes

Table 8.2.5.1 shows the primary/secondary scrambling codes and the channelization codes for downlink channels.

Table 8.2.5.1: Primary/secondary scrambling codes and channelization codes for downlink channels

| Type | Identities (value assigned) | Primary scrambling code | Secondary scrambling code | Channelization Code |
| --- | --- | --- | --- | --- |
| P-CCPCH | tsc_P_CCPCH (4) | (px_PriScrmCode+ 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_P_CCPCH_ChC (256:1) |
| P-CPICH | tsc_P_CPICH (0) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_P_CPICH_ChC (256:0) |
| S-CPICH | tsc_S_CPICH (3) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512 | NA | tsc_S_CPICH_ChC (256:12) |
| S-CCPCH | tsc_S_CCPCH1 (5) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA (carrying PCH) | tsc_S_CCPCH1_ChC (64:1 or 4 or 6 depending on the channels configuration) tsc_S_CCPCH1_ChC_MBMS (64:1) for MBMS testing |
|  | tsc_S_CCPCH2 (10) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA (carrying PCH) | tsc_S_CCPCH2_ChC (64:1) tsc_S_CCPCH2_ChC_MBMS (256:9) for MBMS testing |
|  | tsc_S_CCPCH3 (13) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA (carrying PCH) | tsc_S_CCPCH3_ChC (64:2) tsc_S_CCPCH3_ChC_MBMS (8:1 or 16:1 or 32:2 depending on the channels configuration) for MBMS testing |
| PICH | tsc_PICH1 (6) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_PICH1_ChC (256:2) |
|  | tsc_PICH2 (11) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_PICH2_ChC (256:12) |
| AICH | tsc_AICH1 (7) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_AICH1_ChC (256:3) |
|  | tsc_AICH2 (12) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_AICH2_ChC (256:13) |
| DPCH | tsc_DL_DPCH1 (26) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | tsc_DL_DPCH1_2ndScrC (1) This value is related to the primary scrambling code of the cell | Depending on the configuration: tsc_DL_DPCH1_ChC_SRB (128:9) tsc_DL_DPCH1_ChC_Speech (128:0) tsc_DL_DPCH1_ChC_Streaming (32:0) tsc_DL_DPCH1_ChC_64k_CS (32:0) tsc_DL_DPCH1_ChC_64k_PS (32:0) |
|  | tsc_DL_DPCH2 (27) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | tsc_DL_DPCH2_2ndScrC (1) This value is related to the primary scrambling code of the cell | Depending on the configuration: tsc_DL_DPCH2_ChC_SRB (256:1) tsc_DL_DPCH2_ChC_Speech (128:1) tsc_DL_DPCH2_ChC_Streaming (32:1) tsc_DL_DPCH2_ChC_64k_CS (32:1) tsc_DL_DPCH2_ChC_64k_PS (32:1) |
| HS-PDSCH | tsc_HSPDSCH(18) | Same as HS-SCCH | Same as HS-SCCH | Rel-5 or later SF= 16 Number of codes depending on the configuration, at most 15 codes |
| HS-SCCH | NA | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | - | Rel-5 or later tsc_HS_SCCH_ChC (128:7) |
| E-AGCH | tsc_E_AGCH (14) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | Rel-6 or later |
| E-HICH | tsc_E_HICH (15) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | Rel-6 or later |
| E-RGCH | tsc_E_RGCH (16) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | Rel-6 or later |
| F-DPCH | tsc_F_DPCH (28) | (px_PriScrmCode + 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | Rel-6 or later |
| MICH | tsc_MICH (29) | (px_PriScrmCode+ 50 × ( cell No -1) ) mod 512, (px_PriScrmCode+20+ 50 × ( cell No -21) ) mod 512 for MBMS testing | NA | tsc_MICH_ChC (256:8) |

Table 8.2.5.2 shows the scrambling codes, the signatures and the spreading factors for uplink channels.

Table 8.2.5.2: Scrambling codes, signatures and spreading factor for uplink channels

| Type | Identities (value assigned) | Scrambling code | Signature | Spreading factor |
| --- | --- | --- | --- | --- |
| DPDCH | tsc_UL_DPCH1 (20) | (px_UL_ScramblingCode + 1000*( cell No -1)) MOD 16777216 | NA | If only one DPDCH and depending on the configuration tsc_UL_DPDCH_SF_SRB (64) tsc_UL_DPDCH_SF_Speech (64) tsc_UL_DPDCH_SF_Streaming (16) tsc_UL_DPDCH_SF_64k_CS (16) tsc_UL_DPDCH_SF_64k_PS (16) If more than one DPDCH tsc_UL_DPDCH_SF_4 (4:1) |
|  | tsc_UL_DPCH2 (21) | (px_UL_ScramblingCode + 1 000 × ( cell No -1)) MOD 16 777 216 | NA | If only one DPDCH and depending on the configuration tsc_UL_DPDCH_SF_SRB (64) tsc_UL_DPDCH_SF_Speech (64) tsc_UL_DPDCH_SF_Streaming (16) tsc_UL_DPDCH_SF_64k_CS (16) tsc_UL_DPDCH_SF_64k_PS (16) If more than one DPDCH tsc_UL_DPDCH_SF_4 (4:1) |
| PRACH | tsc_PRACH1 (8) | tsc_PRACH1_ScrC (0) | tsc_PRACH1_Signatures ('0000000011111111'B) | tsc_PRACH1_SF (64) |
|  | tsc_PRACH2 (9) | tsc_PRACH2_ScrC (1) | tsc_PRACH2_Signatures ('0000000011111111'B) | tsc_PRACH2_SF (64) |
| HS-DPCCH | NA | Same as DPDCH | NA | Rel-5 or later Depending on the number of DPDCHs: If only one DPDCH: C256,64; If 2 or 4 or 6 DPDCHs: C256,1; If 3 or 5 DPDCHs: C256,32. |
| E-DPCH | tsc_E_DPCH (22) | Same as DPDCH | NA | Rel-6 or later |

### 8.2.6	MAC-d

MAC-d and the served RLC are cell-independent and are configured by using the cell-id = -1. During reconfigurations, cell changes and state transitions, the relevant counters in the RLC and MAC-d are maintained.

For the active set updating, the DL DCH with the same channel Id in the different cells are implicitly connected to form the DL multiple paths.

#### 8.2.6.1	MAC-d configuration examples

The following example shows how the MAC and RLC ASP are used to configure different configurations.

The 1st parameter in ASP represents the cell identity: p_CellId corresponds to the current cell identity, tsc_CellDedicated corresponds to the cell independent (-1). The 2nd parameter represents the channel Id, this parameter is not needed in the CRLC ASP).

1. Cell_DCH_StandAloneSRB: configuration of DL/UL-DPCH1

CPHY!CPHY_RL_Setup_REQ		( p_CellId, tsc_DL_DPCH1)				-- Cell concerned

CPHY?CPHY_RL_Setup_CNF		( p_CellId, tsc_DL_DPCH1)				-- Cell concerned

CPHY!CPHY_TrCH_Config_REQ	( p_CellId, tsc_DL_DPCH1)				-- Cell concerned

CPHY?CPHY_TrCH_Config_CNF	( p_CellId, tsc_DL_DPCH1 )				-- Cell concerned

CMAC ! CMAC_Config_REQ		( tsc_CellDedicated,  tsc_DL_DPCH1)	-- Cell independent (-1)

CMAC ? CMAC_Config_CNF		( tsc_CellDedicated, tsc_DL_DPCH1)		-- Cell independent (-1)

CPHY!CPHY_RL_Setup_REQ		( p_CellId, tsc_UL_DPCH1) 				-- Cell concerned

CPHY?CPHY_RL_Setup_CNF		( p_CellId, tsc_UL_DPCH1) 				-- Cell concerned

CPHY!CPHY_TrCH_Config_REQ	( p_CellId, tsc_UL_DPCH1 ) 			-- Cell concerned

CPHY?CPHY_TrCH_Config_CNF	( p_CellId, tsc_UL_DPCH1 ) 			-- Cell concerned

CMAC ! CMAC_Config_REQ		( tsc_CellDedicated,  tsc_UL_DPCH1) 	-- Cell independent (-1)

CMAC ? CMAC_Config_CNF		( tsc_CellDedicated, tsc_UL_DPCH1 ) 	-- Cell independent (-1)

CRLC ! CRLC_Config_REQ 		( tsc_CellDedicated )				-- Cell independent (-1)

CRLC ? CRLC_Config_CNF 		( tsc_CellDedicated )				-- Cell independent (-1)

2. Cell_FACH: configuration of S-CCPCH1

CPHY!CPHY_RL_Setup_REQ 			( p_CellId, tsc_S_CCPCH1) 		-- Cell concerned

CPHY?CPHY_RL_Setup_CNF 			( p_CellId, tsc_S_CCPCH1) 		-- Cell concerned t

CPHY!CPHY_TrCH_Config_REQ 		( p_CellId, tsc_S_CCPCH1) 			-- Cell concerned

CPHY ? CPHY_TrCH_Config_CNF 	( p_CellId, tsc_S_CCPCH1) 			-- Cell concerned

CMAC ! CMAC_Config_REQ 			( p_CellId,  tsc_S_CCPCH1) 	-- Cell concerned

CMAC ? CMAC_Config_CNF 			( p_CellId,  tsc_S_CCPCH1 ) 	-- Cell concerned

CPHY!CPHY_RL_Setup_REQ 			( p_CellId, tsc_PICH1			-- Cell concerned

CPHY?CPHY_RL_Setup_CNF 			( p_CellId, tsc_PICH1)			-- Cell concerned

CRLC ! CRLC_Config_REQ 			( tsc_CellDedicated )			-- Cell independent (-1)

CRLC ? CRLC_Config_CNF 			( tsc_CellDedicated )			-- Cell independent (-1)

3. Cell_FACH: configuration of  P-CCPCH

CPHY!CPHY_RL_Setup_REQ 			( p_CellId, tsc_P_CPICH ) 		-- Cell concerned

CPHY?CPHY_RL_Setup_CNF 			( p_CellId, tsc_P_CPICH ) 		-- Cell concerned

CPHY!CPHY_RL_Setup_REQ 			( p_CellId, tsc_P_SCH) 		-- Cell concerned

CPHY?CPHY_RL_Setup_CNF 			( p_CellId, tsc_P_SCH ) 		-- Cell concerned

CPHY!CPHY_RL_Setup_REQ 			( p_CellId, tsc_P_SCH) 		-- Cell concerned

CPHY?CPHY_RL_Setup_CNF 			( p_CellId, tsc_S_SCH ) 		-- Cell concerned

CPHY!CPHY_RL_Setup_REQ 			( p_CellId, tsc_P_CCPCH) 		-- Cell concerned

CPHY?CPHY_RL_Setup_CNF 			( p_CellId, tsc_P_CCPCH ) 		-- Cell concerned

CPHY!CPHY_TrCH_Config_REQ 		( p_CellId, tsc_P_CCPCH )  		-- Cell concerned

CPHY?CPHY_TrCH_Config_CNF 		( p_CellId, tsc_P_CCPCH ) 			-- Cell concerned

CMAC!CMAC_Config_REQ 			( p_CellId, tsc_P_CCPCH) 			-- Cell concerned

CMAC?CMAC_Config_CNF 			( p_CellId, tsc_P_CCPCH ) 			-- Cell concerned

CRLC! CRLC_Config_REQ			( p_CellId)							-- Cell concerned

CRLC? CRLC_Config_CNF			( p_CellId)							-- Cell concerned

### 8.2.7	Configuration of compressed mode

#### 8.2.7.1	UE Side

Two IE are available for the configuration of the compressed mode for the UE.

a)	DPCH_CompressedModeInfo.

b)	DPCH_CompressedModeStatusInfo.

Compressed mode initiation at UE side can be divided into 2 steps:

a)	Downloading compressed mode parameters.

b)	Activating the compressed mode.

Both of them can be done in one shot.

#### 8.2.7.2	SS Side

Compressed mode configuration at SS side shall be maintained the same status as that on the UE side. So there are 3 different types of compressed mode configuration states both on UE and SS side.

-	Configuration of compressed mode parameters (Use of DPCH_CompressedModeInfo) without the activation.

-	Configuration of compressed mode parameters and simultaneous activation (use of DPCH_CompressedModeInfo).

-	Only activation (use of DPCH_CompressedModeStatusInfo).

If compressed mode parameters are to be downloaded to the UE without actually activation, it shall be configured on the SS side by any one of the following two procedures.

-	If DPCH channel on which compressed mode is to be downloaded is not already configured, primitive "CPHY_RL_Setup_REQ", with "CphyRlSetupReq. PhysicalChannelInfo" which is of choice, chosen to dPCHInfo shall be called. The procedure is used to pre-configure all compressed patterns necessary for test, but deactivate the all patterns configured at the beginning of the test. This procedure has not been implemented in the TTCN.

-	If DPCH channel on which compressed mode is to be downloaded is already configured, the primitive "CPHY_RL_Modify_REQ" with "CphyRlModifyReq. PhysicalChannelInfo" which is of choice, chosen to dPCHInfo shall be called. This procedure in generally used in the TTCN.

If compressed mode parameters are to be configured and simultaneously activated, the same procedure as for the configuration of compressed mode without activation shall be used.

Activation of the compressed mode, whose parameters are already configured shall be achieved by the primitive "CPHY_RL_Modify_REQ" with "CphyRlModifyReq. PhysicalChannelInfo" which is of choice, chosen to dpch_CompressedModeStatusInfo.

### 8.2.8	Use of U-RNTI and C-RNTI

The uRNTI and cRNTI are optional when configuring the MAC (CMAC_Config_REQ). Table 8.2.8.1 gives indication on when uRNTI and cRNTI are needed.

Table 8.2.8.1: cRNTI and uRNTI in CMAC-Config_REQ

|  | P-CCPCH | S-CCPCH with mapped DL-DCCH/DTCH (UE in cell_FACH) | S-CCPCH without mapped DL-DCCH/DTCH (UE in cell_DCH) | PRACH with mapped DL-DCCH/DTCH (UE in cell_FACH) | PRACH without mapped DL-DCCH/DTCH (UE in cell_DCH) | DPCH |
| --- | --- | --- | --- | --- | --- | --- |
| uRNTI | - | Included | - | Omit | - | - |
| cRNTI | - | Included | - | Included | - | - |
| CMAC-Config_REQ | OMIT both | Download cRNTI and uRNTI | OMIT both | Download cRNTI | OMIT both | OMIT both |

In the case of DL-DCCH/DTCH mapped on S-CCPCH, cRNTI and uRNTI are downloaded to the MAC layer. As default, SS MAC shall use cRNTI as UE id. At the CMAC configuration of the beginning of test cases, the RLC payload size is configured, as default on cRNTI for the MAC header calculation. If uRNTI is to be used the SS RLC payload size shall be reconfigured as cRNTI and uRNTI do not have the same length (16 bits and 32 bits respectively).

CELL UPDATE CONFIRM or URA UPDATE CONFIRM shall be sent on DCCH at the test for the ciphering reason except the periodic update without carrying the UE identity information. In this case the CELL UPDATE CONFIRM or URA UPDATE CONFIRM is sent on CCCH at the test.

Table 8.2.8.2: Relationship between cell update cause, UE state and RLC size reconfiguration

| Cell update cause | UE State (before cell update) | CELL UPDATE CONFIRM | CRLC_Reconf RLC_Size Needed | Valid UE ID |
| --- | --- | --- | --- | --- |
| Cell reselection | CELL_PCH / CELL_FACH | DCCH | Y | U_RNTI |
| Periodical cell update | CELL_PCH | DCCH or CCCH | Y (for DCCH) | U_RNTI |
| Periodical cell update | CELL_FACH | DCCH or CCCH | N | C_RNTI |
| Uplink data transmission | CELL_PCH / URA _PCH | DCCH | Y | U_RNTI |
| UTRAN paging response | CELL_PCH / URA_PCH | DCCH | Y | U_RNTI |
| Re-entered service area | CELL_PCH / URA_PCH | DCCH | Y | U_RNTI |
| Re-entered service area | CELL_FACH | DCCH | N | C_RNTI |
| Radio Link failure | CELL_DCH | DCCH | Y | U_RNTI |
| RLC_unrecoverable error | CELL_DCH / CELL_FACH | DCCH | Y N (selected the same cell in CELL_FACH) | U_RNTI C_RNTI |

## 8.3	Channels configurations

### 8.3.1	Configuration of Cell_FACH

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH, CELL_PCH and URA_PCH. They need a minimum radio configuration for testing.

Table 8.3.1.1: Uplink configuration of Cell_FACH

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |

Table 8.3.1.2: Downlink configuration of Cell_FACH

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |

### 8.3.1a	Configuration of Cell_FACH_NoDedicated

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states for a non serving cell without DCCH/DTCH.

Table 8.3.1a.1: Uplink configuration of Cell_FACH_NoDedicated

| RB Identity | tsc_RB0 (0) |
| --- | --- |
| LogCh Type | CCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) |
| RLC mode | TM |
| TrCH Type | RACH |
| TrCH identity | tsc_RACH1 (15) |
| PhyCh Type | PRACH |
| PhyCH identity | tsc_PRACH1 (8) |

Table 8.3.1a.2: Downlink configuration of Cell_FACH_NoDedicated

| RB Identity | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- |
| LogCh Type | CCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | UM | TM | TM |
| MAC priority | 1 | 6 | 1 |
| TrCH Type | FACH |  | PCH |
| TrCH identity | tsc_FACH1 (13) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |

### 8.3.2	Configuration of Cell_DCH_StandAloneSRB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1. 3. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to the RRC and NAS signalling tests in the DCH state without RAB.

Table 8.3.2.1: Uplink configuration of Cell_DCH_StandAloneSRB

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB0 (0) |  |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH | CCCH |  |
| LogCh Identity | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) | tsc_UL_CCCH5 (5) |  |
| RLC mode | UM | AM | AM | AM | TM | AM |
| TrCH Type | DCH |  |  |  | RACH |  |
| TrCH identity | tsc_UL_DCH5 (5) |  |  |  | tsc_RACH1 (15) |  |
| PhyCh Type | DPDCH |  |  |  | PRACH |  |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |  |

Table 8.3.2.2: Downlink configuration of Cell_DCH_StandAloneSRB

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB0 (0) | tsc_RB_PCCH (-2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH | CCCH | PCCH |  |
| LogCh Identity | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_DL_CCCH5 (5) | tsc_PCCH1 (1) |  |
| RLC mode | UM | AM | AM | AM | UM | TM | AM |
| MAC priority | 1 | 2 | 3 | 4 | 1 | 1 | 1 |
| TrCH Type | DCH |  |  |  | FACH | PCH | FACH |
| TrCH identity | tsc_DL_DCH5 (10) |  |  |  | tsc_FACH1 (13) | tsc_PCH1 (12) | tsc_FACH2 (14) |
| PhyCh Type | DPCH |  |  |  | Secondary CCPCH |  |  |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |  |  |

### 8.3.3	Configuration of Cell_DCH_Speech

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.4 and 6.10.2.4.1.5. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those RRC and NAS signalling tests in the DCH state where a CS voice service, such as narrowband speech, emergency speech call or TS 61 for speech, is established.

Table 8.3.3.1: Uplink configuration of Cell_DCH_Speech

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | TM | TM | TM |  |  |
| TrCH Type | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) |  |  |
| PhyCh Type | DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.3.2: Downlink configuration of Cell_DCH_Speech

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | TM | TM | TM |  |  |
| MAC priority | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH3 (8) |  |  |
| PhyCh Type | DPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.4	Configuration of Cell_DCH_64kCS_RAB_SRB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.13 for the conversational unknown quality class. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those RRC and NAS signalling tests in the DCH state where one of the following CS transparent data services is established:

-	Multimedia call 28,8 kbit/s, 3,1 kHz Audio;

-	Multimedia call 32 kbit/s, UDI;

-	Multimedia call 33,6 kbit/s, 3,1 kHz Audio;

-	Multimedia call 56 kbit/s, RDI;

-	Multimedia call 64 kbit/s, UDI;

-	Asynchronous 3,1 kHz Audio 28,8 kbit/s;

-	Synchronous 3,1 kHz Audio 28,8 kbit/s;

-	Synchronous V.110 UDI up to 56 kbit/s;

-	BTM RDI 56 kbit/s;

-	BTM UDI 64 bit/s.

Table 8.3.4.1: Uplink configuration of Cell_DCH_64kCS_RAB_SRB

| RB Identity | tsc_RB10 (10) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.4.2: Downlink configuration of Cell_DCH_64kCS_RAB_SRB

| RB Identity | tsc_RB10 (10) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.5	Configuration of Cell_DCH_57_6kCS_RAB_SRB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.17 for the streaming unknown quality class. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those RRC and NAS signalling tests in the DCH state where one of the following CS non-transparent data services is established:

-	Asynchronous 3,1 kHz Audio up to 19,2 kbit/s;

-	Asynchronous 3,1 kHz Audio modem auto-bauding;

-	Asynchronous V.110 UDI up to 38,4 kbit/s, except 28,8 kbit/s;

-	Asynchronous V.120 up to 56 kbit/s;

-	Asynchronous PIAFS up to 64 kbit/s;

-	Asynchronous FTM up to 64 kbit/s;

-	Synchronous 3,1 kHz Audio up to 19,2 kbit/s;

-	Synchronous V.110 UDI up to 56 kbit/s, except 28,8 kbit/s;

-	Synchronous X.31 Flags Stuffing UDI up to 56 kbit/s;

-	Synchronous V.120 up to 56 kbit/s;

-	Synchronous BTM up to 64 kbit/s;

-	TS61 FAX.

Table 8.3.5.1: Uplink configuration of Cell_DCH_57_6kCS_RAB_SRB

| RB Identity | tsc_RB10 (10) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.5.2: Downlink configuration of Cell_DCH_57_6kCS_RAB_SRB

| RB Identity | tsc_RB10 (10) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.6	Configuration of Cell_RLC_DCH_RAB

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.1, 6.11.2, 6.11.3, and 6.11.4 for the RLC AM and UM tests with 7 and 15 bit length indicators. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The RB Ids used for the DTCH depend on the RLC mode and length indicator size being simulated (reference clause 6.5.2, RLC test method). Table 8.3.6.1 shows the test suite constants used for each RLC mode, and length indicator size.

Table 8.3.6.1: RB Ids used for DTCH depending on RLC mode and LI size

| RLC mode | LI Size | TSC | RB Id |
| --- | --- | --- | --- |
| UM | 7 | tsc_RB_UM_7_RLC | -10 |
| UM | 15 | tsc_RB_UM_15_RLC | -11 |
| AM | 7 | tsc_RB_AM_7_RLC | -12 |
| AM | 15 | tsc_RB_AM_15_RLC | -13 |

Table 8.3.6.2: Uplink configuration of Cell_RLC_DCH_RAB

| RB Identity | See table 8.3.6.1 | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.6.3: Downlink configuration of Cell_RLC_DCH_RAB

| RB Identity | See table 8.3.6.1 | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.7	Configuration of Cell_FACH_BMC

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 without RAB/DTCH for uplink. A RB30/CTCH is configured. The configuration is applied to the BMC and CBSMS tests.

The uplink configuration of Cell_FACH_BMC is the same as the uplink configuration of Cell_FACH.

Table 8.3.7.1: Downlink configuration of Cell_FACH_BMC

| RB Identity |  | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | Tsc_RB30 (30) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type |  | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | CTCH | PCCH |
| LogCh Identity |  | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | Tsc_CTCH (11) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | UM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |  |

### 8.3.8	Configuration of PS Cell_DCH_64kPS_RAB_SRB and Cell_PDCP_AM_RAB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.26. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those RRC and NAS signalling tests in the DCH state where a PS RAB on DTCH is setup for the interactive or background service class. The configuration is applied to PDCP test cases in acknowledge mode.

Table 8.3.8.1: Uplink configuration of PS Cell_DCH_64kPS_RAB_SRB SRB and Cell_PDCP_AM_RAB

| RB Identity | tsc_RB20 (20) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.8.2: Downlink configuration of PS Cell_DCH_64kPS_RAB_SRB SRB and Cell_PDCP_AM_RAB

| RB Identity | tsc_RB20 (20) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.9	Configuration of Cell_Two_DTCH

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.6 to 6.10.2.4.1.11. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.9.1: Uplink configuration of Cell_Two_DTCH

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) |  |  |
| PhyCh Type | DPCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPDCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.9.2: Downlink configuration of Cell_Two_DTCH

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| MAC priority | 1 | 1 |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) |  |  |
| PhyCh Type | DPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.10	Configuration of Cell_Single_DTCH (CS)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.12 to 6.10.2.4.1.22. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.10.1: Uplink configuration of Cell_Single_DTCH (CS)

| RB Identity | tsc_RB10 (10) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.10.2: Downlink configuration of Cell_Single_DTCH (CS)

| RB Identity | tsc_RB10 (10) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.11	Configuration of PS Cell_PDCP_UM_RAB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.26. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to PDCP test cases in unacknowledge mode.

Table 8.3.11.1: Uplink configuration of PS Cell_PDCP_UM_RAB

| RB Identity | tsc_RB21 (21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | UM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.11.2: Downlink configuration of PS Cell_PDCP_UM_RAB

| RB Identity | tsc_RB21 (21) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | UM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.12	Configuration of PS Cell_PDCP_AM_UM_RAB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.26. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to PDCP test cases using both the acknowledged and unacknowledged mode.

Table 8.3.12.1: Uplink configuration of PS Cell_PDCP_AM_UM_RAB

| RB Identity | tsc_RB20 (20) | tsc_RB21 (21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM | UM |  |  |
| TrCH Type | DCH |  |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |  |
| PhyCh Type | DPDCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.12.2: Downlink configuration of PS Cell_PDCP_AM_UM_RAB

| RB Identity | tsc_RB20 (20) | tsc_RB21 (21) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | AM | UM |  |  |
| MAC priority | 1 | 1 |  |  |
| TrCH Type | DCH |  |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |  |
| PhyCh Type | DPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.13	Configuration of Cell_2SCCPCH_BMC

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 without RAB/DTCH for uplink. RB30/CTCH and RB31/CTCH as well as two PCCH are configured. The configuration is applied to the BMC and CBSMS tests.

Table 8.3.13.1: Uplink configuration of Cell_2SCCPCH_BMC

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | Tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |

Table 8.3.13.2: Downlink configuration of Cell_2SCCPCH_BMC: second S-CCPCH

| RB Identity | Tsc_RB31 (31) | tsc_RB_2ndPCCH (-4) |
| --- | --- | --- |
| LogCh Type | CTCH | PCCH |
| LogCh Identity | Tsc_CTCH2 (12) | tsc_PCCH2 (2) |
| RLC mode | UM | TM |
| MAC priority | 1 | 1 |
| TrCH Type | FACH | PCH |
| TrCH identity | tsc_FACH1 (13) | tsc_PCH2 (30) |
| PhyCh Type | Secondary CCPCH |  |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |

Table 8.3.13.3: Downlink configuration of Cell_2SCCPCH_BMC: first S-CCPCCH

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | Tsc_RB30 (30) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | CTCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (6) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | Tsc_CTCH1 (11) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | UM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  |  | PCH |
| TrCH identity | Tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |  |

### 8.3.14	Configuration of Cell_Four_DTCH_CS_PS, Cell_Four_DTCH_PS_CS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.40. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.14.1: Uplink configuration of Cell_Four_DTCH_CS_PS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB20 (20) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) | tsc_UL_DTCH4 (10) |  |  |
| RLC mode | TM | TM | TM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) | tsc_UL_DCH4 (4) |  |  |
| PhyCh Type | DPDCH |  |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  |  | tsc_PRACH1 (8) |

Table 8.3.14.2: Downlink configuration of Cell_Four_DTCH_CS_PS, Cell_Four_DTCH_PS_CS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB20 (20) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) |  |  |
| RLC mode | TM | TM | TM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | Tsc_DL_DCH3 (8) | tsc_DL_DCH4 (9) |  |  |
| PhyCh Type | DPCH |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.14a	Configuration of Cell_Five_DTCH_CS_PS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.5.4.1.67. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to LCR TDD RB tests.

Table 8.3.14a.1: Uplink configuration of Cell_Five_DTCH_CS_PS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB20 (20) | tsc_RB22 (22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) | tsc_UL_DTCH4 (10) | tsc_UL_DTCH5 (13) |  |  |
| RLC mode | TM | TM | TM | AM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) | tsc_UL_DCH4 (4) | tsc_UL_DCH5 (5) |  |  |
| PhyCh Type | DPDCH |  |  |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  |  |  | tsc_PRACH1 (8) |

Table 8.3.14a.2: Downlink configuration of Cell_Five_DTCH_CS_PS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB20 (20) | tsc_RB22 (22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) | tsc_UL_DTCH5 (11) |  |  |
| RLC mode | TM | TM | TM | AM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | Tsc_DL_DCH3 (8) | tsc_DL_DCH4 (9) | tsc_DL_DCH5 (10) |  |  |
| PhyCh Type | DPCH |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.15	Configuration of Cell_Two_DTCH_CS_PS, Cell_Two_DTCH_PS_CS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.51 and 6.10.2.4.1.53. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.15.1:Uplink configuration of Cell_Two_DTCH_CS_PS, Cell_Two_DTCH_PS_CS

| RB Identity | tsc_RB10 (10) | tsc_RB20 (20) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | TM | AM |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) |  |  |
| PhyCh Type | DPDCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.15.2: Downlink configuration of Cell_Two_DTCH_CS_PS

| RB Identity | tsc_RB10 (10) | tsc_RB20 (20) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | TM | AM |  |  |
| MAC priority | 1 | 1 |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) |  |  |
| PhyCh Type | DPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.16	Configuration of Cell_Four_DTCH_CS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.49. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.16.1: Uplink configuration of Cell_Four_DTCH_CS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB13 (13) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (1) | tsc_UL_DTCH2 (2) | tsc_UL_DTCH3 (3) | tsc_UL_DTCH4 (4) |  |  |
| RLC mode | TM | TM | TM | TM |  |  |
| MAC priority | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (6) | tsc_UL_DCH2 (7) | tsc_UL_DCH3 (8) | tsc_UL_DCH4 (9) |  |  |
| PhyCh Type | DPDCH |  |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  |  | tsc_PRACH1 (8) |

Table 8.3.16.2: Downlink configuration of Cell_Four_DTCH_CS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB13 (13) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) |  |  |
| RLC mode | TM | TM | TM | TM |  |  |
| MAC priority | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH3 (8) | tsc_DL_DCH4 (9) |  |  |
| PhyCh Type | DPCH |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.17	Configuration of Cell_DCH_MAC_SRB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1. 3. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1; except that RB3 is mapped on TM mode.

The configuration is applied to the MAC tests.

Table 8.3.17.1: Uplink configuration of Cell_DCH_MAC_SRB

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB_DCCH_DCH_MAC (-15) | tsc_RB4 (4) | tsc_RB0 (0) |  |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH | CCCH |  |
| LogCh Identity | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) | tsc_UL_CCCH5 (5) |  |
| RLC mode | UM | AM | TM | AM | TM | AM |
| TrCH Type | DCH |  |  |  | RACH |  |
| TrCH identity | tsc_UL_DCH5 (5) |  |  |  | tsc_RACH1 (15) |  |
| PhyCh Type | DPDCH |  |  |  | PRACH |  |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |  |

Table 8.3.17.2: Downlink configuration of Cell_DCH_MAC_SRB

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB_DCCH_DCH_MAC (-15) | tsc_RB4 (4) | tsc_RB0 (0) | tsc_RB_PCCH (-2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH | CCCH | PCCH |  |
| LogCh Identity | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_DL_CCCH5 (5) | tsc_PCCH1 (1) |  |
| RLC mode | UM | AM | TM | AM | UM | TM | AM |
| MAC priority | 1 | 2 | 3 | 4 | 1 | 1 | 1 |
| TrCH Type | DCH |  |  |  | FACH | PCH | FACH |
| TrCH identity | tsc_DL_DCH5 (10) |  |  |  | tsc_FACH1 (13) | tsc_PCH1 (12) | tsc_FACH2 (14) |
| PhyCh Type | DPCH |  |  |  | Secondary CCPCH |  |  |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |  |  |

### 8.3.18	Configuration of Cell_FACH_MAC_SRB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink; except that RB3 is mapped on TM mode.

The configuration is applied to the MAC tests.

Table 8.3.18.1: Uplink configuration of Cell_FACH_MAC_SRB

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB_DCCH_FACH_MAC (-14) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | TM | UM | AM | TM | AM |
| TrCH Type | RACH |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |

Table 8.3.18.2: Downlink configuration of Cell_FACH_MAC_SRB

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB_DCCH_FACH_MAC (-14) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (6) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | TM | AM | TM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |

### 8.3.19	Configuration of Cell_FACH_MAC_SRB0

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink; except that the downlink SRB0 is mapped on TM mode.

The configuration is applied to the MAC tests.

The uplink configuration of Cell_FACH_MAC_SRB0 is the same as the uplink configuration of Cell_FACH.

Table 8.3.19: Downlink configuration of Cell_FACH_MAC_SRB0

| RB Identity | tsc_RB20 (20) | tsc_RB_CCCH_FACH_MAC (-18) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (6) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | AM | TM | UM | AM | AM | AM | TM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |

### 8.3.20	Configuration of Cell_FACH_2SCCPCH_StandAlonePCH

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3] except the mapping of PCH, clause 6.10.2.4.4.1.1.1 for uplink.

The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_2SCCPCH_StandAlonePCH is the same as the uplink configuration of Cell_FACH.

Table 8.3.20: Downlink configuration of Cell_FACH_2SCCPCH_StandAlonePCH

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (6) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.21	Configuration of PS Cell_DCH_2AM_PS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.26 and 6.10.2.4.1.57. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 with 2 AM RAB and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to MAC and RAB test cases.

Table 8.3.21.1: Uplink configuration of Cell_DCH_2AM_PS

| RB Identity | tsc_RB20 (20) | tsc_RB22 (22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| TrCH Type | DCH |  |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |  |
| PhyCh Type | DPDCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.21.2: Downlink configuration of Cell_DCH_2AM_PS

| RB Identity | tsc_RB20 (20) | tsc_RB22 (22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| MAC priority | 1 | 1 |  |  |
| TrCH Type | DCH |  |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |  |
| PhyCh Type | DPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.21a	Configuration of Cell_DCH_3AM_PS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.5.4.1.70. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to LCR TDD RB tests.

Table 8.3.21a.1: Uplink configuration of Cell_DCH_3AM_PS

| RB Identity | tsc_RB20 (20) | tsc_RB22 (22) | tsc_RB24 (24) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | AM | AM |  |  |
| TrCH Type | DCH |  |  |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |  |  |
| PhyCh Type | DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.21a.2: Downlink configuration of Cell_DCH_3AM_PS

| RB Identity | tsc_RB20 (20) | tsc_RB22 (22) | tsc_RB24 (24) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | AM | AM | AM |  |  |
| MAC priority | 1 | 1 | 1 |  |  |
| TrCH Type | DCH |  |  |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |  |  |
| PhyCh Type | DPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.22	Configuration of PS Cell_DCH_2_PS_Call

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.56  and 6.10.2.4.1.58. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.22.1: Uplink configuration of Cell_DCH_2_PS_Call

| RB Identity | tsc_RB20 (20) | tsc_RB22 (22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) |  |  |
| PhyCh Type | DPDCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.22.2: Downlink configuration of Cell_DCH_2_PS_Call

| RB Identity | tsc_RB20 (20) | tsc_RB22 (22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| MAC priority | 1 | 1 |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2(7) |  |  |
| PhyCh Type | DPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.23	Configuration of Cell_FACH_3_SCCPCH_4_FACH_Cnfg1

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_3_SCCPCH_4_FACH Cnfg1 is the same as the uplink configuration of Cell_FACH.

Table 8.3.23.1: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_Cnfg1:
1st & 2nd S-CCPCH

| RB Identity |  | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- |
| LogCh Type |  | CCCH | BCCH | PCCH |
| LogCh Identity |  | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode |  | UM | TM | TM |
| MAC priority |  | 1 | 6 | 1 |
| TrCH Type | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  | tsc_S_CCPCH1 (5) |

Table 8.3.23.2: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_Cnfg1: 3rd S-CCPCH

| RB Identity | tsc_RB20 (20) | tsc_RB29 (29) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH_RAB (-19) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH6 (6) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH7 (7) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 |
| TrCH Type | FACH | FACH |  |  |  |  |  |
| TrCH identity | tsc_FACH4 (17) | tsc_FACH3 (16) |  |  |  |  |  |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH3 (13) |  |  |  |  |  |  |

### 8.3.24	Configuration of Cell_FACH_3_SCCPCH_4_FACH_Cnfg2

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_3_SCCPCH_4_FACH Cnfg2 is the same as the uplink configuration of Cell_FACH.

Table 8.3.24.1: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_Cnfg2: 2nd S-CCPCH

| RB Identity | tsc_RB20 (20) | tsc_RB29 (29) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH_RAB (-19) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH6 (6) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH7 (7) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 |
| TrCH Type | FACH | FACH |  |  |  |  |  |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  |  |  |  |  |

Table 8.3.24.2: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_Cnfg2:
1st & 3rd S-CCPCH

| RB Identity |  | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- |
| LogCh Type |  | CCCH | BCCH | PCCH |
| LogCh Identity |  | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode |  | UM | TM | TM |
| MAC priority |  | 1 | 6 | 1 |
| TrCH Type | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH4 (17) | tsc_FACH3 (16) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH3 (13) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.25	Configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH  is the same as the uplink configuration of Cell_FACH.

Table 8.3.25.1: Downlink configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH:
1st & 2nd S-CCPCH

| RB Identity | tsc_RB30 (30) | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- |
| LogCh Type | CTCH | CCCH | BCCH | PCCH |
| LogCh Identity | tsc_CTCH1 (11) | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | UM | UM | TM | TM |
| MAC priority | 7 | 1 | 6 | 1 |
| TrCH Type | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  | tsc_S_CCPCH1 (5) |

Table 8.3.25.2: Downlink configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH: 3rd S-CCPCH

| RB Identity | tsc_RB20 (20) | tsc_RB29 (29) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH_RAB (-19) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH6 (6) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (5) | tsc_BCCH7 (7) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 |
| TrCH Type | FACH | FACH |  |  |  |  |  |
| TrCH identity | tsc_FACH4 (17) | tsc_FACH3 (16) |  |  |  |  |  |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH3 (13) |  |  |  |  |  |  |

### 8.3.26	Configuration of PS Cell_DCH_DSCH_PS_RAB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.2.1. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those RAB signalling tests where a PS RAB on DTCH is setup for the interactive or background service class is mapped on to DSCH.

The uplink configuration is same in clause 8.3.8.

Table 8.3.26: Downlink configuration of PS Cell_DCH_DSCH_PS_RAB

| RB Identity | tsc_RB20 (20) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DSCH |  |  |
| TrCH identity | tsc_DSCH1 (19) |  |  |
| PhyCh Type | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_DL_PDSCH1 (16) | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.27	Configuration of Cell_DCH_DSCH_CS_PS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.2.4. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

The Uplink configuration is similar to clause 8.3.14.

Table 8.3.27: Downlink configuration of Cell_DCH_DSCH_CS_PS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB20 (20) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) |  |  |
| RLC mode | TM | TM | TM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DSCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | Tsc_DL_DCH3 (8) | tsc_DL_DSCH1 (19) |  |  |
| PhyCh Type | DPCH |  |  | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  | tsc_DL_PDSCH1 (16) | tsc_DL_DPCH1 (20) | tsc_S_CCPCH1 (5) |

### 8.3.28	Configuration of Cell_FACH_2SCCPCH_StandAlonePCH_2a

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2a for downlink and 3GPP TS 34.108 [3] except the mapping of PCH, clause 6.10.2.4.4.2 for uplink. The configuration is applied to the RAB tests.

Table 8.3.28.1: Uplink configuration of Configuration of Configuration of Cell_FACH_2SCCPCH_StandAlonePCH_2a

| RB Identity | tsc_RB24 (24) | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_DTCH4 (10) | tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |  |

Table 8.3.28.2: Downlink configuration of Cell_FACH_2SCCPCH_StandAlonePCH_2a

| RB Identity | tsc_RB20 (20) | tsc_RB24 (24) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_2ndPCCH (-4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH4 (10) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | AM | AM | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 1 | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH | FACH | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) |  | tsc_FACH1(13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.29	Configuration of Cell_FACH_3_SCCPCH_4_FACH_2a_Cnfg1

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2a for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.2 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_3_SCCPCH_4_FACH Cnfg1 is the same as the uplink configuration of Cell_FACH_2_SCCPCH_StandAlonePCH_2a.

Table 8.3.29.1: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_2a_Cnfg1:
1st & 2nd S-CCPCH

| RB Identity |  |  | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) |  | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type |  |  | CCCH | BCCH |  | PCCH |
| LogCh Identity |  |  | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) |  | tsc_PCCH1 (1) |
| RLC mode |  |  | UM | TM |  | TM |
| MAC priority |  |  | 1 | 6 |  | 1 |
| TrCH Type | FACH | FACH | FACH |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) |  | tsc_FACH1 (13) |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  | Secondary CCPCH |  |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  |  | tsc_S_CCPCH1 (5) |  |

Table 8.3.29.2: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_2a_Cnfg1:
3rd S-CCPCH

| RB Identity | tsc_RB24 (24) | tsc_RB20 (20) | tsc_RB29 (29) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH_RAB (-19) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH4 (10) | tsc_DL_DTCH1 (7) | tsc_DL_CCCH6 (6) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH7 (7) |
| RLC mode | AM | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 1 | 1 | 2 | 3 | 4 | 5 | 6 |
| TrCH Type | FACH |  | FACH |  |  |  |  |  |
| TrCH identity | tsc_FACH4 (17) |  | tsc_FACH3 (16) |  |  |  |  |  |
| PhyCh Type | Secondary CCPCH tsc_S_CCPCH3 (13) |  |  |  |  |  |  |  |
| PhyCH identity |  |  |  |  |  |  |  |  |

### 8.3.30	Configuration of Cell_FACH_3_SCCPCH_4_FACH_2a_Cnfg2

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2a for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.2 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_3_SCCPCH_4_FACH Cnfg2 is the same as the uplink configuration of Cell_FACH_2_SCCPCH_StandAlonePCH_2a.

Table 8.3.30.1: Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_2a_Cnfg2: 2nd S-CCPCH

| RB Identity | tsc_RB21 (24) | tsc_RB20 (20) | tsc_RB29 (29) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH_RAB (-19) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH2 (10) | tsc_DL_DTCH1 (7) | tsc_DL_CCCH6 (6) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH7 (7) |
| RLC mode | AM | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 1 | 1 | 2 | 3 | 4 | 5 | 6 |
| TrCH Type | FACH | FACH | FACH |  |  |  |  |  |
| TrCH identity | tsc_FACH2 (14) |  | tsc_FACH1 (13) |  |  |  |  |  |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  |  |  |  |  |  |

Table 8.3.30.2 Downlink configuration of Cell_FACH_3_SCCPCH_4_FACH_2a_Cnfg2:
1st & 3rd S-CCPCH

| RB Identity |  |  | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- |
| LogCh Type |  |  | CCCH | BCCH | PCCH |
| LogCh Identity |  |  | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode |  |  | UM | TM | TM |
| MAC priority |  |  | 1 | 6 | 1 |
| TrCH Type | FACH | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH4 (17) |  | tsc_FACH3 (16) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH3 (13) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.31	Configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH_2a

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.2 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH_2a  is the same as the uplink configuration of Cell_FACH Cell_FACH_3_SCCPCH_4_FACH Cnfg1.

Table 8.3.31.1: Downlink configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH_2a:
1st & 2nd S-CCPCH

| RB Identity | tsc_RB30 (30) | tsc_RB0 (0) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- |
| LogCh Type | CTCH | CCCH | BCCH | PCCH |
| LogCh Identity | tsc_CTCH1 (11) | tsc_DL_CCCH5 (5) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | UM | UM | TM | TM |
| MAC priority | 7 | 1 | 6 | 1 |
| TrCH Type | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  | tsc_S_CCPCH1 (5) |

Table 8.3.31.2 Downlink configuration of Cell_FACH_3_SCCPCH_3_FACH_CTCH_2a: 3rd S-CCPCH

| RB Identity | tsc_RB24 (24) | tsc_RB20 (20) | tsc_RB29 (29) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH_RAB (-19) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH4(10) | tsc_DL_DTCH1 (7) | tsc_DL_CCCH6 (6) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (5) | tsc_BCCH7 (7) |
| RLC mode | AM | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 1 | 1 | 2 | 3 | 4 | 5 | 6 |
| TrCH Type | FACH | FACH | FACH |  |  |  |  |  |
| TrCH identity | tsc_FACH4 (17) |  | tsc_FACH3 (16) |  |  |  |  |  |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH3 (13) |  |  |  |  |  |  |  |

### 8.3.32	Configuration of Cell_DCH_HS_DSCH (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.5.1 or 6.10.2.4.5.2. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those RRC and NAS signalling tests in the DCH state where a PS RAB on DTCH mapped on HS-DSCH is setup for the interactive or background service class.

Table 8.3.32.1: Uplink configuration of Cell_DCH_HS_DSCH

| RB Identity | tsc_RB25 (25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.32.2: Downlink configuration of Cell_DCH_HS_DSCH

| RB Identity | tsc_RB25 (25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 |  |  |
| PhyCh Type | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.32a	Configuration of Cell_DCH_E_DPCH_PS

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.5.4.7.8. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to LCR TDD RB tests.

Table 8.3.32a.1: Uplink configuration of Cell_DCH_E_DPCH_PS

| RB Identity | tsc_RB25 (25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity/Mac-d Flow Id | 2 |  |  |
| PhyCh Type | E-DPDCH | DPDCH | PRACH |
| PhyCH identity | tsc_E_DPCH (22) | tsc_UL_DPCH1 (20) | tsc_PRACH1 (8) |

Table 8.3.32a.2: Downlink configuration of PS Cell_DCH_E_DPCH_PS

| RB Identity | tsc_RB25 (25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.33	Configuration of cell_One_DTCH_HS_DSCH_MAC (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.5.1. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those MAC-hs/MAC-ehs[Rel-7 or later] Signalling tests in the DCH state where a PS RAB on DTCH mapped on HS-DSCH is setup for the interactive or background service class.

Table 8.3.33.1: Uplink configuration of cell_One_DTCH_HS_DSCH_MAC

| RB Identity | tsc_RB_MAC_HS (-25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.33.2: Downlink configuration of Cell_DCH_HS_DSCH

| RB Identity | tsc_RB_MAC_HS (-25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 |  |  |
| PhyCh Type | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.33a	Configuration of cell_Three_DTCH_1Q_HS_DSCH_MAC (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4f. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those MAC-ehs Signalling tests in the DCH state where a 3 PS RAB on DTCH mapped on HS-DSCH [1 MAC-ehs Queue] is setup for the interactive or background service class.

Table 8.3.33a.1: Uplink configuration of cell_Three_DTCH_1Q_HS_DSCH_MAC

| RB Identity | tsc_RB_MAC_HS (-25) | tsc_RB_MAC_ehs_26 (-26) | tsc_RB_MAC_ehs_27 (-27) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |  |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |  |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |  |
| RLC mode | TM | TM | TM |  |  |  |
| TrCH Type | DCH |  |  |  |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |  |  |  |
| PhyCh Type | DPDCH |  |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  |  | tsc_PRACH1 (8) |

Table 8.3.33a.2: Downlink configuration of cell_Three_DTCH_1Q_HS_DSCH_MAC

| RB Identity | tsc_RB_MAC_HS (-25) | tsc_RB_MAC_ehs_26 (-26) | tsc_RB_MAC_ehs_27 (-27) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | TM | TM | TM |  |  |
| MAC priority | 8 | 8 | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |  |  |
| TrCH identity /QueueID | 0 |  |  |  |  |
| PhyCh Type | PDSCH |  |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.33b	Configuration of cell_Three_DTCH_3Q_HS_DSCH_MAC (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4f. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to those MAC-ehs Signalling tests in the DCH state where a 3 PS RAB on DTCH mapped on HS-DSCH [1 MAC-ehs Queue] is setup for the interactive or background service class.

The uplink configuration of cell_Three_DTCH_3Q_HS_DSCH_MAC is the same as the uplink configuration of cell_Three_DTCH_1Q_HS_DSCH_MAC

Table 8.3.33b: Downlink configuration of cell_Three_DTCH_3Q_HS_DSCH_MAC

| RB Identity | Tsc_RB_MAC_HS (-25) | tsc_RB_MAC_ehs_26 (-26) | tsc_RB_MAC_ehs_27 (-27) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | TM | TM | TM |  |  |
| MAC priority | 8 | 8 | 8 |  |  |
| TrCH Type | HS-DSCH | HS-DSCH | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 | 1 | 2 |  |  |
| PhyCh Type | PDSCH |  |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.33c	Configuration of Cell_E_HS_SRB_MAC_TM_RAB (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.3, with RAB configured in TM mode on SS side. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A14):

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

Table 8.3.33c.1: Uplink configuration of Cell_E_HS_SRB_MAC_TM_RAB

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity/ Mac-d Flow Id | 2 | 1 |  |
| PhyCh Type | E-DPDCH |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  | tsc_PRACH1 (8) |

Table 8.3.33c.2: Downlink configuration of Cell_E_HS_SRB_MAC_TM_RAB

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |  |
| RLC mode | TM |  |  |  |
| MAC priority | 8 |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 | 1 |  |  |
| PhyCh Type | PDSCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.34	Configuration of Cell_2UM_3AM_DCH_HS_DSCH (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4a The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to MAC test case 7.1.5.2.

Table 8.3.34.1: Uplink configuration of Cell_2UM_3AM_DCH_HS_DSCH

| RB Identity | tsc_RB26 (26) | tsc_RB27 (27) | tsc_RB25 (25) | tsc_RB28 (28) | tsc_RB17 (17) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) | tsc_UL_DTCH4 (10) | tsc_UL_DTCH5 (13) |  |  |
| RLC mode | UM | UM | AM | AM | AM |  |  |
| TrCH Type | DCH |  |  |  |  |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |  |  |  |  |
| PhyCh Type | DPDCH |  |  |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  |  |  | tsc_PRACH1 (8) |

Table 8.3.34.2: Downlink configuration of Cell_2UM_3AM_DCH_HS_DSCH

| RB Identity | tsc_RB26 (26) | tsc_RB27 (27) | tsc_RB25 (25) | tsc_RB28 (28) | tsc_RB17 () | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) | tsc_DL_DTCH5 (11) |  |  |
| RLC mode | UM | UM | AM | AM | AM |  |  |
| MAC priority | 8 | 8 | 8 | 8 | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |  |  |  |  |
| TrCH identity /QueueID | 0 |  | 1 |  | 2 |  |  |
| PhyCh Type | PDSCH |  |  |  |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.35	Configuration of Cell_DCH_Speech_WAMR (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.62. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108[3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RAB test 14.2.62.

Table 8.3.35.1: Uplink configuration of Cell_DCH_Speech_WAMR

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) |  |  |
| PhyCh Type | DPDCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.35.2: Downlink configuration of Cell_DCH_Speech_WAMR

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB5 (5) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DCCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DCCH5 (5) |  |  |
| RLC mode | TM | TM | TM |  |  |
| MAC priority | 1 | 1 | 5 |  |  |
| TrCH Type | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH6 (22) |  |  |
| PhyCh Type | DPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.36	Configuration of PS Cell_Four_DTCH_HS_CS and Cell_Four_DTCH_CS_HS (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.5.3 and 6.10.2.4.5.3a. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The uplink configuration is same in clause 8.3.14 except a HS-DPCCH shall be included in the UL_DPCH and tsc_RB25 shall be used instead of tsc_RB20.

Table 8.3.36: Downlink configuration of PS Cell_Four_DTCH_HS_CS and Cell_Four_DTCH_CS_HS

| RB Identity | tsc_RB25 (25) | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH4 (10) | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | AM | TM | TM | TM |  |  |
| MAC priority | 8 | 1 | 1 | 1 |  |  |
| TrCH Type | HS_DSCH | DCH | DCH | DCH |  |  |
| TrCH identity | N/A | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH3 (8) |  |  |
| PhyCh Type | HS-PDSCH | DPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.37	Configuration of PS Cell_Two_DTCH_HS_CS (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.5.4 and 6.10.2.4.5.4a. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The uplink configuration is same in clause 8.3.15 except a HS-DPCCH shall be included in the UL_DPCH and tsc_RB25 shall be used instead of tsc_RB20.

Table 8.3.37: Downlink configuration of PS Cell_Two_DTCH_HS_CS

| RB Identity | tsc_RB25 (25) | tsc_RB10 (10) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH4 (10) | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM | TM |  |  |
| MAC priority | 8 | 1 |  |  |
| TrCH Type | HS_DSCH | DCH |  |  |
| TrCH identity | N/A | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | HS-PDSCH | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (20) |  | tsc_S_CCPCH1 (5) |

### 8.3.38	Configuration of PS Cell_DCH_64kPS_RAB_SRB_HS (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.26. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

Table 8.3.38.1: Uplink configuration of PS Cell_DCH_64kPS_RAB_SRB_HS

| RB Identity | tsc_RB25 (25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.38.2: Downlink configuration of PS Cell_DCH_64kPS_RAB_SRB SRB

| RB Identity | tsc_RB25 (25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.39	Configuration of PS Cell_DCH_2AM_HS_DSCH (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.26 and 6.10.2.4.1.57. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 with 2 AM RAB and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to MAC and RAB test cases.

Table 8.3.39.1: Uplink configuration of Cell_DCH_2AM_HS_DSCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) |  |  |
| PhyCh Type | DPDCH |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  | tsc_PRACH1 (8) |

Table 8.3.39.2: Downlink configuration of Cell_DCH_2AM_HS_DSCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| MAC priority | 8 | 8 |  |  |
| TrCH Type | HS-DSCH | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 | 1 |  |  |
| PhyCh Type | PDSCH |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.39a	Configuration of Cell_DCH_2AM_E_DPCH

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.5.4.7.12. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to LCR TDD RB tests.

Table 8.3.39a.1: Uplink configuration of Cell_DCH_2AM_E_DPCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity/Mac-d Flow Id | 2 | 3 |  |  |
| PhyCh Type | E-DPDCH |  | DPDCH | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  | tsc_UL_DPCH1 (20) | tsc_PRACH1 (8) |

Table 8.3.39a.2: Downlink configuration of PS Cell_DCH_2AM_E_DPCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| MAC priority | 1 | 1 |  |  |
| TrCH Type | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) |  |  |
| PhyCh Type | DPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.40	Configuration of Cell_Three_DTCH_5SRB (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.1.62. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

The uplink configuration is same in clause 8.3.3 Cell_DCH_Speech.

Table 8.3.40: Downlink configuration of Cell_Three_DTCH_5SRB

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB5 (5) | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DCCH | DCCH | DCCH | DCCH | DCCH |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_DL_DCCH5 () |  |
| RLC mode | TM | TM | TM | UM | AM | AM | AM | TM |  |
| MAC priority | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 |  |
| TrCH Type | DCH | DCH | DCH | DCH |  |  |  | DCH |  |
| TrCH/ Q- identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH3 (8) | tsc_DL_DCH5 (10) |  |  |  | tsc_DL_DCH6 (22) |  |
| PhyCh Type | DPCH |  |  |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.41	Configuration of Cell_Five_DTCH_CS_HS (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.5.7. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

Table 8.3.41.1: Uplink configuration of Cell_Five_DTCH_CS_HS and Cell_Five_DTCH_CS_HS

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB25 (25) | tsc_RB17 (17) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH except TrCH Identity is tsc_UL_DCH6 (21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) | tsc_UL_DTCH4 (10) | tsc_UL_DTCH5 (13) |  |  |
| RLC mode | TM | TM | TM | AM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) | tsc_UL_DCH4 (4) | tsc_UL_DCH5 (5) |  |  |
| PhyCh Type | DPDCH |  |  |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  |  |  | tsc_PRACH1 (8) |

Table 8.3.41.2: Downlink configuration of PS Cell_Five_DTCH_HS_CS and Cell_Five_DTCH_CS_HS

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH4 (10) | tsc_DL_DTCH5 (11) | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | AM | AM | TM | TM | TM |  |  |
| MAC priority | 8 | 8 | 1 | 1 | 1 |  |  |
| TrCH Type | HS_DSCH | HS_DSCH | DCH | DCH | DCH |  |  |
| TrCH identity | N/A | N/A | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH3 (8) |  |  |
| PhyCh Type | HS-PDSCH |  | DPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.41a	Configuration of Cell_FiveDTCH_E_DPCH

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.5.4.7.15. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to LCR TDD RB tests.

Table 8.3.41a.1: Uplink configuration of Cell_FiveDTCH_E_DPCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) |  | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH4 (10) | tsc_UL_DTCH5 (13) |  | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | AM |  | TM | TM | TM |  |  |
| TrCH Type | E-DCH |  |  | DCH | DCH | DCH |  |  |
| TrCH identity/Mac-d Flow Id | 2 |  | 3 | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) |  |  |
| PhyCh Type | E-DPDCH |  |  | DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.41a.2: Downlink configuration of PS Cell_FiveDTCH_E_DPCH

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB25 (25) | tsc_RB17 (17) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) | tsc_UL_DTCH5 (13) |  |  |
| RLC mode | TM | TM | TM | AM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | Tsc_DL_DCH3 (8) | tsc_DL_DCH4 (9) | tsc_DL_DCH5 (10) |  |  |
| PhyCh Type | DPCH |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.42	Configuration of Cell_DCH_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.1. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB on DTCH mapped on E-DCH in uplink and HS-DSCH in downlink is setup for the streaming or interactive or background service class (A12).

The downlink configuration is same in clause 8.3.32 Cell_DCH_HS_DSCH.

Table 8.3.42: Uplink configuration of Cell_DCH_E_HS

| RB Identity | tsc_RB25 (25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity/Mac-d Flow Id | 2 |  |  |
| PhyCh Type | E-DPDCH | DPDCH | PRACH |
| PhyCH identity | tsc_E_DPCH (22) | tsc_UL_DPCH1 (20) | tsc_PRACH1 (8) |

### 8.3.43	Configuration of Cell_DCH_dlSRB_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.2. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A13):

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

The downlink configuration is same in clause 8.3.32 Cell_DCH_HS_DSCH.

Table 8.3.43: Uplink configuration of Cell_DCH_dlSRB_E_HS

| RB Identity | tsc_RB25 (25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH |  | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |  |
| RLC mode | AM |  |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity/ Mac-d Flow Id | 2 |  | 1 |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

### 8.3.44	Configuration of Cell_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.3. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A14):

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

The uplink configuration is same in clause 8.3.43 Cell_DCH_dlSRB_E_HS. In the downlink F-DPCH is configured.

Table 8.3.44: Downlink configuration of Cell_E_HS

| RB Identity | tsc_RB25 (25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |  |
| RLC mode | AM |  |  |  |
| MAC priority | 8 |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 |  | 1 |  |
| PhyCh Type | PDSCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.45	Configuration of PS Cell_Four_DTCH_E_HS_CS and Cell_Four_DTCH_CS_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.4. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class:

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	CS RAB on DTCH are mapped on DCH in uplink and downlink.

-	Uplink SRBs on DCCH are mapped on DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

The downlink configuration is same as in clause 8.3.36 Cell_Four_DTCH_HS_CS and Cell_Four_DTCH_CS_HS.

Table 8.3.45: Uplink configuration of Cell_Four_DTCH_E_HS_CS and Cell_Four_DTCH_CS_E_HS

| RB Identity | tsc_RB25 (25) | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH4 (10) | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | TM | TM | TM |  |  |
| MAC priority |  | 1 | 1 | 1 |  |  |
| TrCH Type | E-DCH | DCH | DCH | DCH |  |  |
| TrCH identity / Mac-d Flow Id | 2 | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) |  |  |
| PhyCh Type | E-DPDCH | DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |

### 8.3.45a	Configuration of Cell_FourDTCH_E_DPCH

The configuration is based on 3GPP TS 34.108 [3], clauses 6.11.5.4.7.13. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to LCR TDD RB tests.

Table 8.3.45a.1: Uplink configuration of Cell_FourDTCH_E_DPCH

| RB Identity | tsc_RB25 (25) | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH4 (10) | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | TM | TM | TM |  |  |
| TrCH Type | E-DCH | DCH | DCH | DCH |  |  |
| TrCH identity/Mac-d Flow Id | 2 | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) | tsc_UL_DCH3 (3) |  |  |
| PhyCh Type | E-DPDCH | DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.45a.2: Downlink configuration of PS Cell_FourDTCH_E_DPCH

| RB Identity | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB25 (25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DTCH4 (10) |  |  |
| RLC mode | TM | TM | TM | AM |  |  |
| MAC priority | 1 | 1 | 1 | 1 |  |  |
| TrCH Type | DCH | DCH | DCH | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | Tsc_DL_DCH3 (8) | tsc_DL_DCH4 (9) |  |  |
| PhyCh Type | DPCH |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (20) |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.46	Configuration of Cell_2DCH_2AM_dlSRB_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.5.2. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A15):

-	2 AM PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

The downlink configuration is same in clause 8.3.39 Cell_2DCH_2AM_HS_DSCH

Table 8.3.46: Uplink configuration of Cell_2DCH_2AM_dlSRB_E_HS

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM | AM |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 3 | 1 |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

### 8.3.47	Configuration of Cell_E_HS_MAC_TM_RAB (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4c, with RAB configured in TM mode on SS side. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to MAC(e/es) signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A12):

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

Table 8.3.47.1: Uplink configuration of Cell_E_HS_MAC_TM_RAB

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity/Mac-d Flow Id | 2 |  |  |
| PhyCh Type | E-DPDCH | DPDCH | PRACH |
| PhyCH identity | tsc_E_DPCH (22) | tsc_UL_DPCH1 (20) | tsc_PRACH1 (8) |

Table 8.3.47.2: Downlink configuration of Cell_E_HS_MAC_TM_RAB

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 |  |  |
| PhyCh Type | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.48	Configuration of Cell_2DCH_MAC_2TM_dlSRB_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4d. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. (A13).

The configuration is applied to MAC(e/es)  signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A15):

-	2 TM PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

Table 8.3.48.1: Uplink configuration of Cell_2DCH_MAC_2TM_dlSRB_E_HS

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 3 | 1 |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

Table 8.3.48.2: Downlink configuration of Cell_2DCH_MAC_2TM_dlSRB_E_HS

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| MAC priority | 8 | 8 |  |  |
| TrCH Type | HS-DSCH | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 | 1 |  |  |
| PhyCh Type | PDSCH |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.49	Configuration of Cell_2DCH_1AM_1UM_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.6. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where a PS RAB is setup for the streaming or interactive or background service class and another UM PS Bearer is setup for conversational / unknown or speech (A16):

-	1 AM PS RAB and 1 UM PS RAB on DTCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

In the downlink F-DPCH is configured.

Table 8.3.49.1: Uplink configuration of Cell_2DCH_1AM_1UM_E_HS

| RB Identity | tsc_RB25 (25) | tsc_RB27 (27) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | UM |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 4 | 1 |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

Table 8.3.49.2: Downlink configuration of Cell_2DCH_1AM_1UM_E_HS

| RB Identity | tsc_RB25 (25) | tsc_RB27 (27) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH3 (9) |  |  |  |  |
| RLC mode | AM | UM |  |  |  |  |
| MAC priority | 8 | 8 |  |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 |  | 3 |  | 1 |  |
| PhyCh Type | PDSCH |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.50	Configuration of Cell_3DCH_2AM_1UM_E_HS (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.7. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RRC signalling tests in the DCH state where two PS RABs are setup for the streaming or interactive or background service class and another UM PS Bearer is setup for conversational / unknown or speech:

-	2 AM PS RABs and 1 UM PS RAB on DTCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

In the downlink F-DPCH is configured.

Table 8.3.50.1: Uplink configuration of Cell_2DCH_1AM_1UM_E_HS

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | tsc_RB27 (27) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | AM | UM |  |  |
| TrCH Type | E-DCH |  |  |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 3 | 4 | 1 |  |
| PhyCh Type | E-DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.50.2: Downlink configuration of Cell_2DCH_1AM_1UM_E_HS

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | tsc_RB27 (27) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |  |  |
| RLC mode | AM | AM | UM |  |  |  |  |
| MAC priority | 8 | 8 | 8 |  |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 | 2 |  | 3 |  | 1 |  |
| PhyCh Type | PDSCH |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.51	Configuration of Cell_Four_DTCH_CS_E_HS_5SRB (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses6.10.2.4.6.8. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

The uplink configuration is same as in clause 8.3.45 Cell_Four_DTCH_E_HS_CS and Cell_Four_DTCH_CS_E_HS.

The downlink configuration is the same as in clause 8.3.52 Cell_Four_DTCH_HS_5SRB.

### 8.3.52	Configuration of Cell_Four_DTCH_HS_5SRB (Rel-5 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.5.8. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RB tests.

The uplink configuration is same in clause 8.3.36 Cell_Four_DTCH_HS_CS and Cell_Four_DTCH_CS_HS.

Table 8.3.52: Downlink configuration of Cell_Four_DTCH_HS_5SRB

| RB Identity | tsc_RB25 (25) | tsc_RB10 (10) | tsc_RB11 (11) | tsc_RB12 (12) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB5 (5) | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH | DTCH | DCCH | DCCH | DCCH | DCCH | DCCH |  |
| LogCh Identity | tsc_DL_DTCH41 (107) | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_DL_DCCH5 (5) |  |
| RLC mode | AM | TM | TM | TM | UM | AM | AM | AM | TM |  |
| MAC priority | 8 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 |  |
| TrCH Type | HS-DSCH | DCH | DCH | DCH | DCH |  |  |  | DCH |  |
| TrCH/ Q- identity | 0 | Tsc_DL_DCH1 (6) | tsc_DL_DCH2 (7) | tsc_DL_DCH3 (8) | tsc_DL_DCH5 (10) |  |  |  | tsc_DL_DCH6 (22) |  |
| PhyCh Type | PDSCH | DPCH |  |  |  |  |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) |  |  |  |  |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.53	Configuration of Cell_E_HS_StandAloneSRB/ Cell_E_HS_StandAloneSRB_NoConn (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.1a. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to stand-alone SRB RRC signalling tests in the DCH:

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

Additionally UL-DPCCH and HS-DPCCH are configured in UL as well as F-DPCH is configured in DL.

Table 8.3.53.1: Uplink configuration of Cell_E_HS_StandAloneSRB/ Cell_E_HS_StandAloneSRB_NoConn

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH |  |
| LogCh Identity | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |  |
| RLC mode | UM | AM | AM | AM |  |
| TrCH Type | E-DCH |  |  |  |  |
| TrCH identity / Mac-d Flow Id | tsc_E_DCH_MAC_d_FlowId_DCCH (1) |  |  |  |  |
| PhyCh Type | E-DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.53.2: Downlink configuration of Cell_E_HS_StandAloneSRB/ Cell_E_HS_StandAloneSRB_NoConn

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH |  |
| LogCh Identity | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) |  |
| RLC mode | UM | AM | AM | AM |  |
| MAC priority | 1 | 2 | 3 | 4 |  |
| TrCH Type | HS-DSCH |  |  |  |  |
| TrCH identity / Mac-d Flow Id | tsc_HS_DSCH_MAC_d_FlowId_DCCH (1) |  |  |  |  |
| PhyCh Type | HS-PDSCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.54	MBMS channel configuration (Rel-6 or later)

The MBMS channel configurations are configured in addition to any existing configurations defined in clause 8.3.

#### 8.3.54.1	Configuration cell_MBMS_MCCH (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.

The configuration is applied to the MBMS tests.

Table 8.3.54.1: cell_MBMS_MCCH

| RB Identity | tsc_RB_MCCH (8) |
| --- | --- |
| LogCh Type | MCCH |
| LogCh Identity | tsc_MCCH1 (1) |
| RLC mode | UM |
| MAC priority | 1 |
| TrCH Type | FACH |
| TrCH identity | tsc_FACH3 (16) |
| PhyCh Type | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |

#### 8.3.54.2	Configuration cell_MBMS_MCCH_One_MTCH (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.

The configuration is applied to the MBMS tests.

Table 8.3.54.2: cell_MBMS_MCCH_One_MTCH

| RB Identity | tsc_RB_MCCH (8) | tsc_RB_MTCH1 (14) |
| --- | --- | --- |
| LogCh Type | MCCH | MTCH |
| LogCh Identity | tsc_MCCH1 (1) | tsc_MTCH1 (1) |
| RLC mode | UM | UM |
| MAC priority | 1 | 1 |
| TrCH Type | FACH | FACH |
| TrCH identity | tsc_FACH3 (16) | tsc_FACH4 (17) |
| PhyCh Type | Secondary CCPCH | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) | tsc_S_CCPCH3 (13) |

### 8.3.55	Configuration of PS Cell_DCH_64kPS_AM_RAB

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.1.26. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to MBMS tests in the DCH state where a PS RAB on DTCH is setup for the interactive or background service class.

Table 8.3.55.1: Uplink configuration of PS Cell_DCH_64kPS_AM_RAB

| RB Identity | tsc_RB22 (22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_UL_DCH1 (1) |  |  |
| PhyCh Type | DPDCH |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  | tsc_PRACH1 (8) |

Table 8.3.55.2: Downlink configuration of PS Cell_DCH_64kPS_AM_RAB

| RB Identity | tsc_RB22 (22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.56	Configuration of PS Cell_MBMS_PTPRB

The configuration is based on, clause 6.10.2.4.1.58. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to MBMS PTP RB test cases.

The uplink configuration is same in clause 8.3.2 Cell_DCH_StandAloneSRB.

Table 8.3.56: Downlink configuration of Cell_MBMS_PTPRB

| RB Identity | tsc_RB21 (21) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH2 (8) |  |  |
| RLC mode | UM |  |  |
| MAC priority | 1 |  |  |
| TrCH Type | DCH |  |  |
| TrCH identity | tsc_DL_DCH2 (7) |  |  |
| PhyCh Type | DPCH |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  | tsc_S_CCPCH1 (5) |

### 8.3.57	Configuration of PS Cell_MBMS_PTPRB_AM

The configuration is based on, clause 6.10.2.4.1.58. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to MBMS PTP RB test cases, with additional PS RAB established.

The uplink configuration is same in clause 8.3.8 Cell_DCH_64kPS_RAB_SRB and Cell_PDCP_AM_RAB.

Table 8.3.57: Downlink configuration of PS Cell_MBMS_PTPRB_AM

| RB Identity | tsc_RB20 (20) | tsc_RB21 (21) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |  |
| RLC mode | AM | UM |  |  |  |
| MAC priority | 1 | 1 |  |  |  |
| TrCH Type | DCH |  | DCH |  |  |
| TrCH identity | tsc_DL_DCH1 (6) |  | tsc_DL_DCH2 (7) |  |  |
| PhyCh Type | DPCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_DL_DPCH1 (26) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.58	Configuration of Cell_FACH_MCCH_SRB / Cell_FACH_MCCH_NoDedicated

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.9 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the MBMS tests related in the states CELL_FACH, CELL_PCH and URA_PCH.

The uplink configuration is same in clause 8.3.1 Cell_FACH.

Table 8.3.58: Downlink configuration of Cell_FACH_MCCH_NoConn / 
Cell_FACH_MCCH_SRB / Cell_FACH_MCCH_NoDedicated

| RB Identity | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_MCCH (8) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | MCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | Tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_MCCH1 (1) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | AM | AM | UM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 1 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  | FACH | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  | tsc_FACH3 (16) | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |

### 8.3.59	Configuration of Cell_DCH_MCCH_PS

The configuration is based on 3GPP TS 34.108 [3], clause . The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to the MBMS signalling tests in the DCH state.

The uplink configuration is same in clause 8.3.8 Cell_DCH_64kPS_RAB_SRB

Table 8.3.59: Downlink configuration of Cell_DCH_MCCH_PS

| RB Identity | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB20 (20) | tsc_RB0 (0) | tsc_RB_PCCH (-2) | tsc_RB_MCCH (8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DCCH | DCCH | DCCH | DCCH | DTCH | CCCH | PCCH | MCCH |  |
| LogCh Identity | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_DL_DTCH1 (7) | tsc_DL_CCCH5 (5) | tsc_PCCH1 (1) | tsc_MCCH1 (1) |  |
| RLC mode | UM | AM | AM | AM | AM | UM | TM | UM | AM |
| MAC priority | 1 | 2 | 3 | 4 | 1 | 1 | 1 | 1 | 1 |
| TrCH Type | DCH |  |  |  | DCH | FACH | PCH | FACH | FACH |
| TrCH identity | tsc_DL_DCH5 (10) |  |  |  | tsc_DL_DCH1 (6) | tsc_FACH1 (13) | tsc_PCH1 (12) | tsc_FACH3 (16) | tsc_FACH2 (16) |
| PhyCh Type | DPCH tsc_DL_DPCH1 (26) |  |  |  |  | Secondary CCPCH |  |  |  |
| PhyCH identity |  |  |  |  |  | tsc_S_CCPCH1 (5) |  |  |  |

### 8.3.60	Configuration of PS Cell_DCH_1AM_2AM_HS_DSCH (Rel-6 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.5.10. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 with 2 AM RAB and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1. The configuration is applied to RAB test cases.

Table 8.3.60.1: Uplink configuration of Cell_DCH_1AM_2AM_HS_DSCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | tsc_RB28 (28) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | AM | AM |  |  |  |
| TrCH Type | DCH | DCH |  |  |  |
| TrCH identity | tsc_UL_DCH1 (1) | tsc_UL_DCH2 (2) |  |  |  |
| PhyCh Type | DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_UL_DPCH1 (20) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.60.2: Downlink configuration of Cell_DCH_1AM_2AM_HS_DSCH

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) | tsc_RB28 (28) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | AM | AM | AM |  |  |
| MAC priority | 8 | 8 | 8 |  |  |
| TrCH Type | HS-DSCH | HS-DSCH | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 | 2 | 3 |  |  |
| PhyCh Type | PDSCH |  |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.61	Configuration of Cell_FACH_enhDL_PCH (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.4.3 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_PCH and URA_PCH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the PCH states where a PS RAB is setup for the interactive or background service class (A):

-	PS RAB on DTCH is mapped on S-CCPCH in uplink and HS-DSCH in downlink.

- Downlink SRBs on DCCH are mapped on HS-DSCH;

- Uplink SRBs on DCCH are mapped on PRACH.

Table 8.3.61.1: Uplink configuration of Cell_FACH_enhDL_PCH

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |

Table 8.3.61.2: Downlink configuration of Cell_FACH_enhDL_PCH

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH2 (2) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH_FACH (11) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 8 | 1 | 2 | 3 | 4 | 5 | 1 | 1 |
| TrCH Type | HS-DSCH | HS-DSCH | HS-DSCH |  |  |  | HS-DSCH | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 2 | 0 | 1 |  |  |  | NA | NA |
| PhyCh Type | PDSCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  |  |  |

### 8.3.62	Configuration of Cell_FACH_enhDL_PS (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.4.3 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the FACH state where a PS RAB is setup for the interactive or background service class (A):

-	PS RAB on DTCH is mapped on PRACH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH and CCCH are mapped on PRACH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

Table 8.3.62.1: Uplink configuration of Cell_FACH_enhDL_PS

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |

Table 8.3.62.2: Downlink configuration of Cell_FACH_enhDL_PS

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH2 (2) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 8 | 1 | 2 | 3 | 4 | 5 | 1 |
| TrCH Type | HS-DSCH | HS-DSCH | HS-DSCH |  |  |  | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 2 | 0 | 1 |  |  |  | NA |
| PhyCh Type | PDSCH |  |  |  |  |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  |  |

### 8.3.63	Configuration of Cell_E_HS_UM (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clauses 6.10.2.4.6.9 and 6.10.2.4.6.10. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to signalling tests in the DCH state where a PS RAB is setup for CS voice over HSPA(A23):

-	UM PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	SRBs on DCCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

Table 8.3.63.1: Uplink configuration of Cell_E_HS_UM

| RB Identity | tsc_RB26 (26) |  | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |  |
| RLC mode | UM |  |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity/ Mac-d Flow Id | 2 | 1 |  |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

Table 8.3.63.2: Downlink configuration of Cell_E_HS_UM

| RB Identity | tsc_RB26 (26) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |  |
| RLC mode | UM |  |  |  |
| MAC priority | 8 |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 | 1 |  |  |
| PhyCh Type | PDSCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.64	Configuration of Cell_FACH_enhDL_SRB (Rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.4.3 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the FACH state with a signalling connection:

-	Uplink SRBs on DCCH are mapped on PRACH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

Table 8.3.64.1: Uplink configuration of Cell_FACH_enhDL_SRB

| RB Identity | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |

Table 8.3.64.2: Downlink configuration of Cell_FACH_enhDL_SRB

| RB Identity | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_CCCH2 (2) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) |
| RLC mode | UM | UM | AM | AM | AM | TM |
| MAC priority | 1 | 2 | 3 | 4 | 5 | 1 |
| TrCH Type | HS-DSCH | HS-DSCH |  |  |  | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 0 | 1 |  |  |  | NA |
| PhyCh Type | PDSCH |  |  |  |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  |

### 8.3.65	Configuration of Cell_DCH_3TM_dlSRB_E_HS (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.2. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to MAC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A26):

-	3 PS UM RAB on DTCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

Table 8.3.65.1: Uplink configuration of Cell_DCH_3TM_dlSRB_E_HS

| RB Identity | tsc_RB_DTCH_E_DCH_MAC0 (-20) | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) | tsc_UL_DTCH3 (9) |  |  |
| RLC mode | TM | TM | TM |  |  |
| TrCH Type | E-DCH |  |  |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 3 | 4 | 1 |  |
| PhyCh Type | E-DPDCH |  |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  |  | tsc_PRACH1 (8) |

Table 8.3.65.2: Downlink configuration of Cell_DCH_3TM_dlSRB_E_HS

| RB Identity | tsc_RB_DTCH_E_DCH_MAC0 (-20) | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) | tsc_DL_DTCH3 (9) |  |  |
| RLC mode | TM | TM | TM |  |  |
| MAC priority | 8 | 8 | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |  |  |
| TrCH identity /QueueID | 2 | 3 | 4 |  |  |
| PhyCh Type | PDSCH |  |  | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.66	Configuration of Cell_E_HS_TM (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.9 and 6.10.2.4.6.10. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to MAC(i/is) signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A27):

-	TM PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	SRBs on DCCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

Table 8.3.66.1: Uplink configuration of Cell_E_HS_TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) |  | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |  |
| RLC mode | TM |  |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity/ Mac-d Flow Id | 2 | 1 |  |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

Table 8.3.66.2: Downlink configuration of Cell_E_HS_TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |  |
| RLC mode | TM |  |  |  |
| MAC priority | 8 |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 | 1 |  |  |
| PhyCh Type | PDSCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  | tsc_S_CCPCH1 (5) |

### 8.3.67	Dual cell configurations (Rel-8 or later)

#### 8.3.67.1	Configuration of cell_SecondaryDualCell_SRB (Rel-8 or later)

The configuration is applied to the HS-DSCH secondary serving cell where a PS RAB is setup with dual cell activated (A25):

-	Downlink PS RAB is mapped on HS-DSCH in downlink.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

There is no uplink configuration. The related HS-DSCH serving cell is configured in cell_E_HS.

Table 8.3.67.1: Downlink cell_SecondaryDualCell_SRB

| RB Identity | tsc_RB25 (25) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH |  |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |
| TrCH identity / Mac-d Flow Id | 0 |  | 1 |
| PhyCh Type | PDSCH |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |

#### 8.3.67.2	Configuration of cell_SecondaryDualCell_RAB (Rel-8 or later)

The configuration is applied to the HS-DSCH secondary serving cell where a PS RAB is setup with dual cell activated:

-	Downlink PS RAB is mapped on HS-DSCH in downlink.

There is no uplink configuration. The related HS-DSCH serving cell is configured in cell_DCH_HS_DSCH or cell_DCH_E_HS or Cell_DCH_dlSRB_E_HS.

Table 8.3.67.2: Downlink cell_SecondaryDualCell_RAB

| RB Identity | tsc_RB25 (25) |
| --- | --- |
| LogCh Type | DTCH |
| LogCh Identity | tsc_DL_DTCH1 (7) |
| RLC mode | AM |
| MAC priority | 8 |
| TrCH Type | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 0 |
| PhyCh Type | PDSCH |
| PhyCH identity | tsc_HSPDSCH (18) |

#### 8.3.67.3	Configuration of cell_SecondaryDualCell_2RAB (Rel-8 or later)

The configuration is applied to the HS-DSCH secondary serving cell where 2 PS RAB are setup with dual cell activated:

-	2 Downlink PS AM RABs are mapped on HS-DSCH in downlink.

There is no uplink configuration. The related HS-DSCH serving cell is configured in Cell_DCH_2AM_HS_DSCH.

Table 8.3.67.3: Downlink cell_SecondaryDualCell_2RAB

| RB Identity | tsc_RB25 (25) | tsc_RB17 (17) |
| --- | --- | --- |
| LogCh Type | DTCH | DTCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |
| RLC mode | AM | AM |
| MAC priority | 8 | 8 |
| TrCH Type | HS-DSCH | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 0 | 1 |
| PhyCh Type | PDSCH | PDSCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_HSPDSCH (18) |

#### 8.3.67.4	Void

#### 8.3.67.5	Configuration of cell_SecondaryDCU_SRB_2TM (Rel-9 or later)

The configuration is applied to the HS-DSCH secondary serving cell where 2 PS RAB are setup with dual cell HSUPA activated:

-	2 Downlink PS RABs are mapped on HS-DSCH in downlink.

-	2 Uplink PS RABs are mapped on E-DCH.

-	SRBs on DCCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

The related HS-DSCH serving cell is configured in  cell_E_HS_2TM.

Table 8.3.67.5.1: Uplink cell_SecondaryDCU_SRB_2TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) |  | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity/ Mac-d Flow Id | 2 | 3 | 1 |  |
| PhyCh Type | E-DPDCH |  |  |  |
| PhyCH identity | tsc_E_DPCH (22) |  |  |  |

Table 8.3.67.5.2: Downlink cell_SecondaryDCU_SRB_2TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |
| RLC mode | TM | TM |  |
| MAC priority | 8 | 8 |  |
| TrCH Type | HS-DSCH | HS-DSCH | HS-DSCH |
| TrCH identity /QueueID | 0 | 2 | 1 |
| PhyCh Type | PDSCH |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |

#### 8.3.67.6	Configuration of cell_SecondaryDCU_SRB_RAB (Rel-9 or later)

The configuration is applied to the HS-DSCH secondary serving cell where 1 PS RAB is setup with dual cell HSUPA activated:

-	The downlink configuration is the same as cell_SecondaryDualCell_SRB

-	1 Uplink PS RAB is mapped on E-DCH.

-	Uplink SRBs on DCCH are mapped on E-DCH.

The related HS-DSCH serving cell is configured in Cell_E_HS

Table 8.3.67.6.1: Uplink cell_SecondaryDCU_SRB_RAB

| RB Identity | tsc_RB25 (25) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH |  |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | AM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity/ Mac-d Flow Id | 2 |  | 1 |
| PhyCh Type | E-DPDCH |  |  |
| PhyCH identity | tsc_E_DPCH (22) |  |  |

#### 8.3.67.7	Configuration of cell_SecondaryDCU_SRB_TM (Rel-9 or later)

The configuration is applied to the HS-DSCH secondary serving cell where 1 PS RAB is setup with dual cell HSUPA activated:

-	1 Downlink PS RAB is mapped on HS-DSCH in downlink.

-	1 Uplink PS RAB is mapped on E-DCH.

-	SRBs on DCCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

The related HS-DSCH serving cell is configured in Cell_E_HS_MAC_TM_RAB

Table 8.3.67.7.1: Uplink cell_SecondaryDCU_SRB_TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH |
| --- | --- | --- |
| LogCh Type | DTCH |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |
| RLC mode | TM |  |
| TrCH Type | E-DCH |  |
| TrCH identity/ Mac-d Flow Id | 2 | 1 |
| PhyCh Type | E-DPDCH |  |
| PhyCH identity | tsc_E_DPCH (22) |  |

Table 8.3.67.7.2: Downlink cell_SecondaryDCU_SRB_TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH |
| --- | --- | --- |
| LogCh Type | DTCH |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |
| RLC mode | TM |  |
| MAC priority | 8 |  |
| TrCH Type | HS-DSCH | HS-DSCH |
| TrCH identity /QueueID | 0 | 1 |
| PhyCh Type | PDSCH |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |

### 8.3.68	Enhanced FACH Uplink configurations (Rel-8 or later)

#### 8.3.68.1	Configuration of Cell_FACH_UL_SRB (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.7.1 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the FACH state where a signalling connection is setup:

-	Uplink SRBs on DCCH and CCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

The downlink configuration is same as in clause 8.3.64 Cell_FACH_enhDL_SRB

Table 8.3.68.1-1: Uplink configuration of Cell_FACH_UL_SRB

| RB Identity | tsc_RB0 (0) | tsc_RB0_EFUL (0-28) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) | tsc_UL_CCCH_EFUL5 (15) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | TM | TM | UM | AM | AM | AM |
| TrCH Type | RACH | E-DCH |  |  |  |  |
| TrCH identity/ Mac-d Flow Id | tsc_RACH1 (15) | 7 | 1 |  |  |  |
| PhyCh Type | PRACH | E-DPDCH |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) | tsc_E_DPCH (22) |  |  |  |  |

#### 8.3.68.2	Configuration of Cell_FACH_UL_PS (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.7.1 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the FACH state where a PS RAB is setup:

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH and CCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

The downlink configuration is same as in clause 8.3.62 Cell_FACH_enhDL_PS.

Table 8.3.68.2: Uplink configuration of Cell_FACH_UL_PS

| RB Identity | tsc_RB0 (0) | tsc_RB25 (25) | tsc_RB0_EFUL (0-28) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5_EFUL (15) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | TM | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH | E-DCH |  |  |  |  |  |
| TrCH identity/ Mac-d Flow Id | tsc_RACH1 (15) | 0 | 7 | 1 |  |  |  |
| PhyCh Type | PRACH | E-DPDCH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) | tsc_E_DPCH (22) |  |  |  |  |  |

#### 8.3.68.3	Configuration of Cell_FACH_UL_TM_PS (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.7.1 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the FACH state where a PS RAB is setup:

-	PS RAB in TM on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH and CCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on HS-DSCH.

Table 8.3.68.3.1: Uplink configuration of Cell_FACH_UL_TM_PS

| RB Identity | tsc_RB0 (0) | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB0_EFUL (0-28) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) | tsc_UL_DTCH1 (7) | tsc_UL_CCCH5_EFUL (15) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | TM | TM | TM | UM | AM | AM | AM |
| TrCH Type | RACH | E-DCH |  |  |  |  |  |
| TrCH identity/ Mac-d Flow Id | tsc_RACH1 (15) | 0 | 7 | 1 |  |  |  |
| PhyCh Type | PRACH | E-DPDCH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) | tsc_E_DPCH (22) |  |  |  |  |  |

Table 8.3.68.3.2: Downlink configuration of Cell_FACH_UL_TM_PS

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH2 (2) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) |
| RLC mode | TM | UM | UM | AM | AM | AM | TM |
| MAC priority | 8 | 1 | 2 | 3 | 4 | 5 | 1 |
| TrCH Type | HS-DSCH | HS-DSCH | HS-DSCH |  |  |  | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 2 | 0 | 1 |  |  |  | NA |
| PhyCh Type | PDSCH |  |  |  |  |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  |  |

#### 8.3.68.4	Configuration of Cell_FACH_UL_NoDedicated (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.7.1 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink.

The configuration is applied to signalling tests in the enhanced FACH state for a non serving cell where:

-	Uplink SRBs on CCCH are mapped on E-DCH.

The downlink configuration is same as in clause 8.3.1a Cell_FACH_NoDedicated

Table 8.3.68.4.1: Uplink configuration of Cell_FACH_UL_NoDedicated

| RB Identity | tsc_RB0 (0) | tsc_RB0_EFUL (-28) |
| --- | --- | --- |
| LogCh Type | CCCH | CCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) | tsc_UL_CCCH_EFUL (15) |
| RLC mode | TM | TM |
| TrCH Type | RACH | E-DCH |
| TrCH identity/ Mac-d Flow Id | tsc_RACH1 (15) | 7 |
| PhyCh Type | PRACH | E-DPDCH |
| PhyCH identity | tsc_PRACH1 (8) | tsc_E_DPCH (22) |

#### 8.3.68.5	Configuration of Cell_FACH_UL_FallBack (Rel-11 or later)

The configuration is based on 3GPP TS 34.108[3], clause 6.10.2.4.7.1 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the MAC tests related in the states further enhanced CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to MAC signalling tests in the further enhanced FACH state where a PS RAB is setup:

-	PS RAB on DTCH is mapped on PRACH in uplink and HS-DSCH in downlink

-	Uplink SRBs on DCCH and CCCH are mapped on PRACH

-	Downlink SRBs on DCCH are mapped on HS-DSCH

The downlink configuration is same as in clause 8.3.62 Cell_FACH_enhDL_PS.

Table 8.3.68.5-1: Uplink configuration of Cell_FACH_UL_FallBack

| RB Identity | tsc_RB0_EFUL (0-28) | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_CCCH5_EFUL (15) | tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | TM | AM | TM | UM | AM | AM | AM |
| TrCH Type | E-DCH | RACH |  |  |  |  |  |
| TrCH identity/ Mac-d Flow Id | 7 | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | E-DPDCH | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_E_DPCH (22) | tsc_PRACH1 (8) |  |  |  |  |  |

#### 8.3.68.6	Configuration of Cell_FACH_UL_SRB_NoConn (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.7.1 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH. They need a minimum radio configuration for testing.

The configuration is applied to RRC signalling tests in the FACH state when UE is in idle mode:

-	Uplink SRBs on DCCH and CCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on S-CCPCH.

Table 8.3.68.6-1: Uplink configuration of Cell_FACH_UL_SRB_NoConn

| RB Identity | tsc_RB0 (0) | tsc_RB0_EFUL (0-28) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | tsc_UL_CCCH5 (5) | tsc_UL_CCCH_EFUL5 (15) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | TM | TM | UM | AM | AM | AM |
| TrCH Type | RACH | E-DCH |  |  |  |  |
| TrCH identity/ Mac-d Flow Id | tsc_RACH1 (15) | 7 | 1 |  |  |  |
| PhyCh Type | PRACH | E-DPDCH |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) | tsc_E_DPCH (22) |  |  |  |  |

Table 8.3.68.6-2: Downlink configuration of Cell_FACH_UL_SRB_NoConn

| RB Identity | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH1 (13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |

### 8.3.69	Configuration of Cell_FACH_2_SCCPCH_CTCHenhDL_PS (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_2_SCCPCH_CTCH enhDL_PS is the same as the uplink configuration of Cell_FACH_enhDL_PS.

Table 8.3.69.1: Downlink configuration of Cell_FACH_2_SCCPCH_CTCHenhDL_PS:
1st & 2nd S-CCPCH

| RB Identity | tsc_RB30 (30) |  |  |  |
| --- | --- | --- | --- | --- |
| LogCh Type | CTCH |  |  |  |
| LogCh Identity | tsc_CTCH1 (11) |  |  |  |
| RLC mode | UM | UM | TM | TM |
| MAC priority | 7 | 1 | 6 | 1 |
| TrCH Type | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  | tsc_S_CCPCH1 (5) |

Table 8.3.69.2: Downlink configuration of Cell_FACH_2_SCCPCH_CTCHenhDL_PS: HS-PDSCH

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH2 (2) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM |
| MAC priority | 8 | 1 | 2 | 3 | 4 | 5 | 1 |
| TrCH Type | HS-DSCH | HS-DSCH |  | HS-DSCH |  |  | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 2 | 0 |  | 1 |  |  | NA |
| PhyCh Type | PDSCH |  |  |  |  |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  |  |

### 8.3.70	Configuration of Cell_FACH_2_SCCPCH_CTCHenhDL_PCH (Rel-8 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RAB tests.

The uplink configuration of Cell_FACH_2_SCCPCH_CTCH enhDL_PCH is the same as the uplink configuration of Cell_FACH_enhDL_PCH.

Table 8.3.70.1: Downlink configuration of Cell_FACH_2_SCCPCH_CTCHenhDL_PCH:
1st & 2nd S-CCPCH

| RB Identity | tsc_RB30 (30) |  |  |  |
| --- | --- | --- | --- | --- |
| LogCh Type | CTCH |  |  |  |
| LogCh Identity | tsc_CTCH1 (11) |  |  |  |
| RLC mode | UM | UM | TM | TM |
| MAC priority | 7 | 1 | 6 | 1 |
| TrCH Type | FACH | FACH |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  | Secondary CCPCH |
| PhyCH identity | tsc_S_CCPCH2 (10) |  |  | tsc_S_CCPCH1 (5) |

Table 8.3.70.2: Downlink configuration of Cell_FACH_2_SCCPCH_CTCHenhDL_PCH: HS-PDSCH

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH2 (2) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH_FACH (11) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 8 | 1 | 2 | 3 | 4 | 5 | 1 | 1 |
| TrCH Type | HS-DSCH | HS-DSCH |  | HS-DSCH |  |  | HS-DSCH | HS-DSCH |
| TrCH identity / Mac-d Flow Id | 2 | 0 |  | 1 |  |  | NA | NA |
| PhyCh Type | PDSCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  |  |  |  |  |

### 8.3.71	Configuration of Cell_FACH_HS (rel-7 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 for downlink and 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1 for uplink. The configuration is applied to the RRC tests related in the states CELL_FACH, CELL_PCH and URA_PCH. They need a minimum radio configuration for testing.

Table 8.3.71.1: Uplink configuration of Cell_FACH_HS

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) |
| --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH |
| LogCh Identity | Tsc_UL_DTCH1 (7) | tsc_UL_CCCH5 (5) | tsc_UL_DCCH1 (1) | tsc_UL_DCCH2 (2) | tsc_UL_DCCH3 (3) | tsc_UL_DCCH4 (4) |
| RLC mode | AM | TM | UM | AM | AM | AM |
| TrCH Type | RACH |  |  |  |  |  |
| TrCH identity | tsc_RACH1 (15) |  |  |  |  |  |
| PhyCh Type | PRACH |  |  |  |  |  |
| PhyCH identity | tsc_PRACH1 (8) |  |  |  |  |  |

Table 8.3.71.2: Downlink configuration of Cell_FACH_HS

| RB Identity | tsc_RB25 (25) | tsc_RB0 (0) | tsc_RB1 (1) | tsc_RB2 (2) | tsc_RB3 (3) | tsc_RB4 (4) | tsc_RB_BCCH_FACH (-3) | tsc_RB_PCCH (-2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | CCCH | DCCH | DCCH | DCCH | DCCH | BCCH | PCCH |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_CCCH5 (5) | tsc_DL_DCCH1 (1) | tsc_DL_DCCH2 (2) | tsc_DL_DCCH3 (3) | tsc_DL_DCCH4 (4) | tsc_BCCH6 (6) | tsc_PCCH1 (1) |
| RLC mode | AM | UM | UM | AM | AM | AM | TM | TM |
| MAC priority | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 1 |
| TrCH Type | FACH | FACH |  |  |  |  |  | PCH |
| TrCH identity | tsc_FACH2 (14) | tsc_FACH1 (13) |  |  |  |  |  | tsc_PCH1 (12) |
| PhyCh Type | Secondary CCPCH |  |  |  |  |  |  |  |
| PhyCH identity | tsc_S_CCPCH1 (5) |  |  |  |  |  |  |  |

### 8.3.72	Configuration of Cell_E_HS_MAC_TM_dlSRB (Rel-9 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4d. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [33], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to MAC(i/is) signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class:

-	1 TM PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on E-DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

Table 8.3.72.1: Uplink configuration of Cell_E_HS_MAC_TM_dlSRB

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 1 |  |
| PhyCh Type | E-DPDCH |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  | tsc_PRACH1 (8) |

Table 8.3.72.2: Downlink configuration of Cell_E_HS_MAC_TM_dlSRB

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 |  |  |
| PhyCh Type | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

### 8.3.73	Configuration of Cell_E_HS_2TM (Rel-9 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.10.2.4.6.9 and 6.10.2.4.6.10. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to MAC(i/is) signalling tests in the DCH state where two PS RAB are setup for the interactive or background service class:

-	2 TM PS RAB on DTCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

-	SRBs on DCCH are mapped on E-DCH in uplink and HS-DSCH in downlink.

Table 8.3.73.1: Uplink configuration of Cell_E_HS_2TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) | tsc_UL_DTCH2 (8) |  |  |
| RLC mode | TM | TM |  |  |
| TrCH Type | E-DCH |  |  |  |
| TrCH identity//Mac-d Flow Id | 2 | 3 | 1 |  |
| PhyCh Type | E-DPDCH |  |  | PRACH |
| PhyCH identity | tsc_E_DPCH (22) |  |  | tsc_PRACH1 (8) |

Table 8.3.73.2: Downlink configuration of Cell_E_HS_2TM

| RB Identity | tsc_RB_DTCH_E_DCH_MAC1 (-21) | tsc_RB_DTCH_E_DCH_MAC2 (-22) |  | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- | --- | --- |
| LogCh Type | DTCH | DTCH |  |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) | tsc_DL_DTCH2 (8) |  |  |  |
| RLC mode | TM | TM |  |  |  |
| MAC priority | 8 | 8 |  |  |  |
| TrCH Type | HS-DSCH |  |  |  |  |
| TrCH identity / Mac-d Flow Id | 0 | 2 | 1 |  |  |
| PhyCh Type | PDSCH |  |  |  | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) |  |  |  | tsc_S_CCPCH1 (5) |

### 8.3.74	Configuration of Cell_DCH_E_HS_TM_RAB (Rel-11 or later)

The configuration is based on 3GPP TS 34.108 [3], clause 6.11.4c, with RAB configured in TM mode on SS side. The RB0/UM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.3.2.1.2 and RB0/TM-CCCH is referred to 3GPP TS 34.108 [3], clause 6.10.2.4.4.1.1.1.

The configuration is applied to RLC signalling tests in the DCH state where a PS RAB is setup for the interactive or background service class (A44 or A45):

-	PS RAB on DTCH is mapped on E-DCH in uplink and HS-DSCH in downlink.

-	Uplink SRBs on DCCH are mapped on DCH.

-	Downlink SRBs on DCCH are mapped on DCH.

Table 8.3.74.1: Uplink configuration of Cell_E_HS_TM_RAB

| RB Identity | tsc_RB_AM_7_RLC (-12) | Same as uplink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as uplink configuration of Cell_DCH_StandAloneSRB on PRACH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_UL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| TrCH Type | E-DCH |  |  |
| TrCH identity/Mac-d Flow Id | 2 |  |  |
| PhyCh Type | E-DPDCH | DPDCH | PRACH |
| PhyCH identity | tsc_E_DPCH (22) | tsc_UL_DPCH1 (20) | tsc_PRACH1 (8) |

Table 8.3.74.2: Downlink configuration of Cell_E_HS_TM_RAB

| RB Identity | tsc_RB_AM_7_RLC (-12) | Same as downlink configuration of Cell_DCH_StandAloneSRB on DPCH | Same as downlink configuration of Cell_DCH_StandAloneSRB on sCCPCH |
| --- | --- | --- | --- |
| LogCh Type | DTCH |  |  |
| LogCh Identity | tsc_DL_DTCH1 (7) |  |  |
| RLC mode | TM |  |  |
| MAC priority | 8 |  |  |
| TrCH Type | HS-DSCH |  |  |
| TrCH identity /QueueID | 0 |  |  |
| PhyCh Type | PDSCH | DPCH | Secondary CCPCH |
| PhyCH identity | tsc_HSPDSCH (18) | tsc_DL_DPCH1 (26) | tsc_S_CCPCH1 (5) |

## 8.4	System information blocks scheduling

All SIBs specified in 3GPP TS 34.108 [3] are broadcast for all test cases in the present document. The repeat period of broadcasting of a complete SIB configuration is 64 frames (0,64 s) as the default configuration.

Except MIB and SB1, they have the highest scheduling rates, SIB7 has also a higher scheduling rate.

According to the default SIB contents in 3GPP TS 34.108 [3], SIB11 and SIB12 have 3 segments. SIB5/SIB5bis has 4 segments for FDD and 5 segments for 1.28 Mcps TDD. SIB 6 has 4 segments. MIB, SB1, SIB1, SIB2, SIB3, SIB4, SIB7 and SIB18 are not segmented, i.e. one segment for each. For the PDCP tests, SIB16 has 7 segments.

Use CMAC_SYSINFO_CONFIG_REQ, CMAC_SYSINFO_CONFIG_CNF and RLC_TR_DATA_REQ as interface to SS for broadcasting.

Two TSOs are defined, one for PER encoding function, the other for segmentation function. The TSOs shall be implemented in the tester.

### 8.4.1	Grouping SIBs for testing

The grouping of SIBs is defined in 3GPP TS 34.108 [3], clause 6.1.0a.1.

### 8.4.2	SIB configurations

SIB configurations are defined in 3GPP TS 34.108 [3], clause 6.1.0a.2.

### 8.4.3	Test SIB default schedule

The SIB default schedule is defined in 3GPP TS 34.108 [3], clause 6.1.0a.3.

#### 8.4.3.1	Test SIB schedule for idle mode, measurement and Inter-RAT UTRAN to GERAN test cases

The SIB schedule is defined in 3GPP TS 34.108 [3], clause 6.1.0a.4.2.

### 8.4.4	Test SIB special schedule

#### 8.4.4.1	Test SIB schedule for two S-CCPCH or two PRACH

The SIB schedule for two S-CCPCH or two PRACH is defined in 3GPP TS 34.108 [3], clause 6.1.0a.4.1.

#### 8.4.4.2	Test SIB schedule for Inter-Rat Handover from GERAN to UTRAN Test

The SIB schedule for Inter-Rat Handover from GERAN to UTRAN Test is defined in 3GPP TS 34.108 [3], clause 6.1.0a.4.3.

### 8.4.5	Handling the transmission of SIB

According to the SIB repeat periods, SIBs need to be transmitted on a very regular basis during the operation of a test case. This transmission usually has no direct bearing on the operation of the test case, although the carried information ensures the correct configuration and operation of the UE during the test case.

To send this information repeatedly directly from each test case would make the test cases very complex to implement, difficult to understand and place real-time requirements upon them that are beyond the capabilities of most TTCN driven test engines.

Management of scheduling of System Information messages is performed by the system simulator. The SIB contents, usually determined in part by the individual tests, come from the TTCN test cases.

#### 8.4.5.1	Delivery of System Information content

The content of the System Information messages is delivered as a fully encoded bit string to the TM-RLC SAP from the message content defined in the TTCN test case.

The IE 'SFNprime' in the SI messages is set to 0 by the TTCN, and the correct value of 'SFNprime' shall be inserted by the System Simulator prior to transmission of a SI message.

SI messages are ASN.1 packed encoded through a TTCN TSO and segmented another TTCN TSO into SIBs in the TTCN and sent only once to the TM-RLC SAP. Repetition of the SIB is the responsibility of the System Simulator lower layers.

SIBs are considered to be cached. That is, sending a SIB to the TM-RLC SAP will cause a previously sent copy of the SIB to be lost, and all future transmissions of the SIB will be the most recently sent version. This allows for the updating of System Information during the operation of a test case.

#### 8.4.5.2	Scheduling of system Information blocks

The schedule for the transmission of SIBs is provided by the TTCN test case. It is sent using the CMAC_SYSINFO_CONFIG_REQ primitive sent to the CMAC SAP (CMAC_PCO).

Each CMAC_SYSINFO_CONFIG_REQ primitive carries scheduling information for the next SIB sent from the TTCN. Each primitive is followed by an associated SIB. Sending two CMAC_SYSINFO_CONFIG_REQ primitives in succession may cause an unspecified result.

#### 8.4.5.3	Example of usage

The following example shows how the MIB, SB1 and all SIBs in subclause 8.4.3 are sent to the System Simulator lower layers for broadcasting. The 1st parameter in CMAC_SYSINFO_CONFIG_REQ represents the repeat period in power of 2. The 2nd parameter represents the repetition position. Two consecutive frames represent an available repetition position.

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (3, 0)

TM_PCO:	MIB

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (4, 1)

TM_PCO:	SB1

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 2)

TM_PCO:	SIB7

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 3)

TM_PCO:	SIB6 (segment 1 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 5)

TM_PCO:	SIB6 (segment 2 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 6)

TM_PCO:	SIB6 (segment 3 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 7)

TM_PCO:	SIB6 (segment 4 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 10)

TM_PCO:	SIB7 + SIB3 (concatenation)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 11)

TM_PCO:	SIB1 + SIB2 (concatenation)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 13)

TM_PCO:	SIB12 (segment 1 of 3)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 14)

TM_PCO:	SIB12 (segment 2 of 3)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 15)

TM_PCO:	SIB12 (segment 3 of 3)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 18)

TM_PCO:	SIB7 + SIB18 (concatenation)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 19)

TM_PCO:	SIB5/SIB5bis (segment 1 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 21)

TM_PCO:	SIB5/SIB5bis (segment 2 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 22)

TM_PCO:	SIB5/SIB5bis (segment 3 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 23)

TM_PCO:	SIB5/SIB5bis (segment 4 of 4)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 26)

TM_PCO:	SIB7 + SIB4 (concatenation)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 27)

TM_PCO:	No segment

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 29)

TM_PCO:	SIB11 (segment 1 of 3)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 30)

TM_PCO:	SIB11 (segment 3 of 3)

CMAC_PCO:	CMAC_SYSINFO_CONFIG_REQ (6, 31)

TM_PCO:	SIB11 (segment 3 of 3)