3GPP TS 45.003 V16.0.0 (2020-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

GSM/EDGE Channel coding

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	18

1	Scope	19

1.1	References	19

1.2	Abbreviations	20

2	General	20

2.1	General organization	20

2.2	Naming Convention	40

3	Traffic Channels (TCH)	43

3.1	Speech channel at full rate (TCH/FS and TCH/EFS)	43

3.1.1	Preliminary channel coding for EFR only	43

3.1.1.1	CRC calculation	43

3.1.1.2	Repetition bits	44

3.1.1.3	Correspondence between input and output of preliminary channel coding	44

3.1.2	Channel coding for FR and EFR	44

3.1.2.1	Parity and tailing for a speech frame	44

3.1.2.2	Convolutional encoder	45

3.1.3	Interleaving	45

3.1.4	Mapping on a Burst	45

3.2	Speech channel at half rate (TCH/HS)	46

3.2.1	Parity and tailing for a speech frame	46

3.2.2	Convolutional encoder	46

3.2.3	Interleaving	47

3.2.4	Mapping on a burst	47

3.3	Data channel at full rate, 12.0 kbit/s radio interface rate (9.6 kbit/s services (TCH/F9.6))	47

3.3.1	Interface with user unit	48

3.3.2	Block code	48

3.3.3	Convolutional encoder	48

3.3.4	Interleaving	48

3.3.5	Mapping on a Burst	48

3.4	Data channel at full rate, 6.0 kbit/s radio interface rate (4.8 kbit/s services (TCH/F4.8))	49

3.4.1	Interface with user unit	49

3.4.2	Block code	49

3.4.3	Convolutional encoder	49

3.4.4	Interleaving	49

3.4.5	Mapping on a Burst	49

3.5	Data channel at half rate, 6.0 kbit/s radio interface rate (4.8 kbit/s services (TCH/H4.8))	50

3.5.1	Interface with user unit	50

3.5.2	Block code	50

3.5.3	Convolutional encoder	50

3.5.4	Interleaving	50

3.5.5	Mapping on a Burst	50

3.6	Data channel at full rate, 3.6 kbit/s radio interface rate (2.4 kbit/s and less services (TCH/F2.4))	50

3.6.1	Interface with user unit	50

3.6.2	Block code	50

3.6.3	Convolutional encoder	50

3.6.4	Interleaving	51

3.6.5	Mapping on a Burst	51

3.7	Data channel at half rate, 3.6 kbit/s radio interface rate (2.4 kbit/s and less services (TCH/H2.4))	51

3.7.1	Interface with user unit	51

3.7.2	Block code	51

3.7.3	Convolutional encoder	51

3.7.4	Interleaving	51

3.7.5	Mapping on a Burst	52

3.8	Data channel at full rate, 14.5 kbit/s radio interface rate (14.4 kbit/s services (TCH/F14.4))	52

3.8.1	Interface with user unit	52

3.8.2	Block code	52

3.8.3	Convolutional encoder	52

3.8.4	Interleaving	52

3.8.5	Mapping on a Burst	52

3.9	Adaptive multi rate speech channel at full rate (TCH/AFS)	52

3.9.1	SID_UPDATE	53

3.9.1.1	Coding of in-band data	53

3.9.1.2	Parity and convolutional encoding for the comfort noise parameters	53

3.9.1.3	Identification marker	54

3.9.1.4	Interleaving	55

3.9.1.5	Mapping on a Burst	55

3.9.2	SID_FIRST	55

3.9.2.1	Coding of in-band data	55

3.9.2.2	Identification marker	55

3.9.2.3	Interleaving	55

3.9.2.4	Mapping on a Burst	55

3.9.3	ONSET	55

3.9.3.1	Coding of in-band data	56

3.9.3.2	Interleaving	56

3.9.3.3	Mapping on a Burst	56

3.9.4	SPEECH	56

3.9.4.1	Coding of the in-band data	56

3.9.4.2	Ordering according to subjective importance	57

3.9.4.3	Parity for speech frames	57

3.9.4.4	Convolutional encoder	59

3.9.4.5	Interleaving	65

3.9.4.6	Mapping on a Burst	66

3.9.5	RATSCCH	66

3.9.5.1	Coding of in-band data	66

3.9.5.2	Parity and convolutional encoding for the RATSCCH message	66

3.9.5.3	Identification marker	67

3.9.5.4	Interleaving	67

3.9.5.5	Mapping on a Burst	67

3.10	Adaptive multi rate speech channel at half rate (TCH/AHS)	67

3.10.1	SID_UPDATE	68

3.10.1.1	Coding of in-band data	68

3.10.1.2	Parity and convolutional encoding for the comfort noise parameters	68

3.10.1.3	Identification marker	69

3.10.1.4	Interleaving	69

3.10.1.5	Mapping on a Burst	69

3.10.2	SID_UPDATE_INH	70

3.10.2.1	Coding of in-band data	70

3.10.2.2	Identification marker	70

3.10.2.3	Interleaving	70

3.10.2.4	Mapping on a Burst	70

3.10.3	SID_FIRST_P1	71

3.10.3.1	Coding of in-band data	71

3.10.3.2	Identification marker	71

3.10.3.3	Interleaving	71

3.10.3.4	Mapping on a Burst	71

3.10.4	SID_FIRST_P2	71

3.10.4.1	Coding of in-band data	71

3.10.4.2	Interleaving	71

3.10.4.3	Mapping on a Burst	72

3.10.5	SID_FIRST_INH	72

3.10.5.1	Coding of in-band data	72

3.10.5.2	Identification marker	72

3.10.5.3	Interleaving	72

3.10.5.4	Mapping on a Burst	72

3.10.6	ONSET	72

3.10.6.1	Coding of in-band data	72

3.10.6.2	Interleaving	73

3.10.6.3	Mapping on a Burst	73

3.10.7	SPEECH	73

3.10.7.1	Coding of the in-band data	73

3.10.7.2	Ordering according to subjective importance	73

3.10.7.3	Parity for speech frames	74

3.10.7.4	Convolutional encoder	75

3.10.7.5	Interleaving	79

3.10.7.6	Mapping on a Burst	79

3.10.8	RATSCCH_MARKER	79

3.10.8.1	Coding of in-band data	79

3.10.8.2	Identification marker	80

3.10.8.3	Interleaving	80

3.10.8.4	Mapping on a Burst	80

3.10.9	RATSCCH_DATA	80

3.10.9.1	Coding of in-band data	80

3.10.9.2	Parity and convolutional encoding for the RATSCCH message	80

3.10.9.3	Interleaving	81

3.10.9.4	Mapping on a Burst	81

3.11	Data channel for ECSD at full rate, 29.0 kbit/s radio interface rate (28.8 kbit/s services (E-TCH/F28.8))	81

3.11.1	Interface with user unit	81

3.11.2	Block code	81

3.11.2.1	Repetition bits	81

3.11.2.2	Reed Solomon encoder	81

3.11.3	Convolutional encoder	83

3.11.3.1	Tailing bits for a data frame	83

3.11.3.2	Convolutional encoding for a data frame	83

3.11.4	Interleaving	83

3.11.5	Mapping on a Burst	84

3.12	Data channel for ECSD at full rate, 32.0 kbit/s radio interface rate (32.0 kbit/s services (E-TCH/F32.0))	84

3.12.1	Interface with user unit	84

3.12.2	Void	85

3.12.3	Convolutional encoder	85

3.12.3.1	Tailing bits for a data frame	85

3.12.3.2	Convolutional encoding for a data frame	85

3.12.4	Interleaving	86

3.12.5	Mapping on a Burst	86

3.13	Data channel for ECSD at full rate, 43.5 kbit/s radio interface rate (43.2 kbit/s services (E-TCH/F43.2))	86

3.13.1	Interface with user unit	86

3.13.2	Convolutional encoder	86

3.13.2.1	Tailing bits for a data frame	86

3.13.2.2	Convolutional encoding for a data frame	86

3.13.3	Interleaving	87

3.13.4	Mapping on a Burst	87

3.14	Wideband Adaptive multi rate speech channel at full rate (TCH/WFS)	87

3.14.1	SID_UPDATE	87

3.14.2	SID_FIRST	88

3.14.3	ONSET	88

3.14.4	SPEECH	88

3.14.4.1	Coding of the in-band data	88

3.14.4.2	Ordering according to subjective importance	88

3.14.4.3	Parity for speech frames	89

3.14.4.4	Convolutional encoder	90

3.14.4.5	Interleaving	92

3.14.4.6	Mapping on a Burst	92

3.14.5	RATSCCH	92

3.15	Adaptive multi rate speech channel at 8-PSK half rate (O-TCH/AHS)	92

3.15.1	SID_UPDATE	93

3.15.1.1	Coding of in-band data	93

3.15.1.2	Parity and convolutional encoding for the comfort noise parameters	93

3.15.1.3	Identification marker	93

3.15.1.4	Repetition	93

3.15.1.5	Interleaving	93

3.15.1.6	Mapping on a Burst	93

3.15.2	SID_UPDATE_INH	94

3.15.2.1	Coding of in-band data	94

3.15.2.2	Identification marker	94

3.15.2.3	 Repetition	94

3.15.2.4	Interleaving	94

3.15.2.5	Mapping on a Burst	94

3.15.3	SID_FIRST_P1	94

3.15.3.1	Coding of in-band data	94

3.15.3.2	Identification marker	94

3.15.3.3	 Repetition	94

3.15.3.4	Interleaving	95

3.15.3.5	Mapping on a Burst	95

3.15.4	SID_FIRST_P2	95

3.15.4.1	Coding of in-band data	95

3.15.4.2	 Repetition	95

3.15.4.3	Interleaving	95

3.15.4.4	Mapping on a Burst	95

3.15.5	SID_FIRST_INH	95

3.15.5.1	Coding of in-band data	95

3.15.5.2	Identification marker	96

3.15.5.3	 Repetition	96

3.15.5.4	Interleaving	96

3.15.5.5	Mapping on a Burst	96

3.15.6	ONSET	96

3.15.6.1	Coding of in-band data	96

3.15.6.2	 Repetition	96

3.15.6.3	Interleaving	96

3.15.6.4	Mapping on a Burst	96

3.15.7	SPEECH	96

3.15.7.1	Coding of the in-band data	96

3.15.7.2	Ordering according to subjective importance	97

3.15.7.3	Parity for speech frames	97

3.15.7.4	Convolutional encoder	99

3.15.7.5	Interleaving	114

3.15.7.6	Mapping on a Burst	114

3.15.8	RATSCCH_MARKER	114

3.15.8.1	Coding of in-band data	114

3.15.8.2	Identification marker	114

3.15.8.3	Interleaving	114

3.15.8.4	Mapping on a Burst	115

3.15.9	RATSCCH_DATA	115

3.15.9.1	Coding of in-band data	115

3.15.9.2	Parity and convolutional encoding for the RATSCCH message	115

3.15.9.3	Interleaving	115

3.15.9.4	Mapping on a Burst	115

3.16	Wideband Adaptive multi rate speech channel at 8-PSK full rate (O-TCH/WFS)	115

3.16.1	SID_UPDATE	116

3.16.1.1	Coding of in-band data	116

3.16.1.2	Parity and convolutional encoding for the comfort noise parameters	116

3.16.1.3	Identification marker	116

3.16.1.4	Repetition	116

3.16.1.5	Interleaving	116

3.16.1.6	Mapping on a Burst	116

3.16.2	SID_FIRST	116

3.16.2.1	Coding of in-band data	116

3.16.2.2	Identification marker	117

3.16.2.3	 Repetition	117

3.16.2.4	Interleaving	117

3.16.2.5	Mapping on a Burst	117

3.16.3	ONSET	117

3.16.3.1	Coding of in-band data	117

3.16.3.2	 Repetition	117

3.16.3.3	Interleaving	117

3.16.3.4	Mapping on a Burst	117

3.16.4	SPEECH	117

3.16.4.1	Coding of the in-band data	118

3.16.4.2	Ordering according to subjective importance	118

3.16.4.3	Parity for speech frames	118

3.16.4.4	Convolutional encoder	120

3.16.4.5	Interleaving	134

3.16.4.6	Mapping on a Burst	134

3.16.5	RATSCCH	134

3.16.5.1	Coding of in-band data	134

3.16.5.2	 Parity and convolutional encoding for the RATSCCH message	135

3.16.5.3	Identification marker	135

3.16.5.4	Interleaving	135

3.16.5.5	Mapping on a Burst	135

3.17	Wideband Adaptive multi rate speech channel at 8-PSK half rate (O-TCH/WHS)	135

3.17.1	SID_UPDATE	136

3.17.1.1	Coding of in-band data	136

3.17.1.2	Parity and convolutional encoding for the comfort noise parameters	136

3.17.1.3	Identification marker	136

3.17.1.4	Repetition	136

3.17.1.5	Interleaving	136

3.17.1.6	Mapping on a Burst	136

3.17.2	SID_UPDATE_INH	136

3.17.2.1	Coding of in-band data	136

3.17.2.2	Identification marker	137

3.17.2.3	 Repetition	137

3.17.2.4	Interleaving	137

3.17.2.5	Mapping on a Burst	137

3.17.3	SID_FIRST_P1	137

3.17.3.1	Coding of in-band data	137

3.17.3.2	Identification marker	137

3.17.3.3	Repetition	137

3.17.3.4	Interleaving	137

3.17.3.5	Mapping on a Burst	137

3.17.4	SID_FIRST_P2	137

3.17.4.1	Coding of in-band data	137

3.17.4.2	Repetition	137

3.17.4.3	Interleaving	138

3.17.4.4	Mapping on a Burst	138

3.17.5	SID_FIRST_INH	138

3.17.5.1	Coding of in-band data	138

3.17.5.2	Identification marker	138

3.17.5.3	Repetition	138

3.17.5.4	Interleaving	138

3.17.5.5	Mapping on a Burst	138

3.17.6	ONSET	138

3.17.6.1	Coding of in-band data	138

3.17.6.2	Repetition	138

3.17.6.3	Interleaving	138

3.17.6.4	Mapping on a Burst	138

3.17.7	SPEECH	139

3.17.7.1	Coding of the in-band data	139

3.17.7.2	Ordering according to subjective importance	139

3.17.7.3	Parity for speech frames	139

3.17.7.4	Convolutional encoder	140

3.17.7.5	Interleaving	146

3.17.7.6	Mapping on a Burst	146

3.17.8	RATSCCH_MARKER	146

3.17.8.1	Coding of in-band data	146

3.17.8.2	Identification marker	146

3.17.8.3	Interleaving	146

3.17.8.4	Mapping on a Burst	146

3.17.9	RATSCCH_DATA	146

3.17.9.1	Coding of in-band data	146

3.17.9.2	Parity and convolutional encoding for the RATSCCH message	147

3.17.9.3	Interleaving	147

3.17.9.4	Mapping on a Burst	147

4	Control Channels	147

4.1	Slow associated control channel (SACCH)	147

4.1.1	Block constitution	147

4.1.2	Block code	147

4.1.3	Convolutional encoder	147

4.1.4	Interleaving	148

4.1.5	Mapping on a Burst	148

4.2	Fast associated control channel at full rate (FACCH/F)	148

4.2.1	Block constitution	148

4.2.2	Block code	148

4.2.3	Convolutional encoder	148

4.2.4	Interleaving	148

4.2.5	Mapping on a Burst	149

4.3	Fast associated control channel at half rate (FACCH/H)	149

4.3.1	Block constitution	149

4.3.2	Block code	149

4.3.3	Convolutional encoder	149

4.3.4	Interleaving	149

4.3.5	Mapping on a Burst	150

4.4	Broadcast control, Paging, Access grant, Notification and Cell broadcast channels (BCCH, PCH, AGCH, NCH, CBCH), CTS Paging and Access grant channels (CTSPCH, CTSAGCH)	150

4.5	Stand-alone dedicated control channel (SDCCH)	151

4.6	Random access channel (RACH)	151

4.6.1	RACH carrying 8 information bits	151

4.6.2	(EC-)RACH carrying 30 information bits	152

4.6.2.1	Block constitution	152

4.6.2.2	Parity bits	152

4.6.2.3	Tail bits	152

4.6.2.4	Convolutional code	152

4.6.2.5	Burst mapping	152

4.7	Synchronization channel (SCH), Compact synchronization channel (CSCH), CTS Beacon and Access request channels (CTSBCH-SB, CTSARCH)	153

4.7a	Extended Coverage Synchronization channel (EC-SCH)	153

4.7a.1	Block constitution	153

4.7a.2	Coding	153

4.7a.3	Blind physical layer transmission	154

4.7a.4	Cyclic shift	154

4.7a.5	Mapping onto a physical channel	154

4.8	Access Burst on circuit switched channels other than RACH	154

4.9	Access Bursts for uplink access on a channel used for VGCS or VBS	154

4.10a	Fast associated control channel at ECSD E-TCH/F (E-FACCH/F)	155

4.10a.1	Block constitution	155

4.10a.2	Block code	155

4.10a.3	Convolutional encoder	155

4.10a.4	Interleaving	155

4.10a.5	Mapping on a Burst	155

4.10b	Octal fast associated control channel at half rate (O-FACCH/H)	155

4.10b.1	 Block constitution	155

4.10b.2	 Block code	156

4.10b.3	 Convolutional encoder	156

4.10b.4	 Reordering	156

4.10b.5	 Interleaving	157

4.10b.6	 Mapping on a burst	157

4.10c	Octal fast associated control channel at full rate (O-FACCH/F)	157

4.10c.1	 Block constitution	157

4.10c.2	 Block code	157

4.10c.3	 Convolutional encoder	157

4.10c.4	 Reordering	157

4.10c.5	 Interleaving	158

4.10c.6	 Mapping on a burst	158

4.11	Slow associated control channel with embedded enhanced power control (SACCH/TP)	158

4.11.1	Block constitution	158

4.11.2	Block code	158

4.11.3	Convolutional encoder	159

4.11.4	Dummy bits insertion	159

4.11.5	Interleaving	160

4.11.6	Mapping on a Burst	160

4.12	Enhanced power control channel (EPCCH)	160

4.12.1	Block code	160

4.12.2	Mapping on a Burst	160

5	Packet Switched Channels	161

5.1	Packet data traffic channel (PDTCH)	161

5.1.1	Packet data block type 1 (CS-1)	161

5.1.2	Packet data block type 2 (CS-2)	162

5.1.2.1	Block constitution	162

5.1.2.2	Block code	162

5.1.2.3	Convolutional encoder	162

5.1.2.4	Interleaving	163

5.1.2.5	Mapping on a burst	163

5.1.3	Packet data block type 3 (CS-3)	163

5.1.3.1	Block constitution	163

5.1.3.2	Block code	163

5.1.3.3	Convolutional encoder	163

5.1.3.4	Interleaving	164

5.1.3.5	Mapping on a burst	164

5.1.4	Packet data block type 4 (CS-4)	164

5.1.4.1	Block constitution	164

5.1.4.2	Block code	164

5.1.4.3	Convolutional encoder	165

5.1.4.4	Interleaving	165

5.1.4.5	Mapping on a burst	165

5.1.4a	Packet data block type 5a (MCS-0)	165

5.1.4a.1	Downlink (MCS-0 DL)	165

5.1.4a.1.1	Block constitution	165

5.1.4a.1.2	USF precoding	165

5.1.4a.1.3	Data coding	165

5.1.4a.1.4	Header coding	166

5.1.4a.1.5	Interleaving	166

5.1.4a.1.6	Mapping on a burst	167

5.1.5	Packet data block type 5 (MCS-1)	167

5.1.5.1	Downlink (MCS-1 DL)	167

5.1.5.1.1	Block constitution	167

5.1.5.1.2	USF precoding	167

5.1.5.1.2.1	BTTI configuration	167

5.1.5.1.2.2	RTTI configuration	167

5.1.5.1.3	Header coding	168

5.1.5.1.4	Data coding	169

5.1.5.1.4a	Piggy-backed Ack/Nack coding	170

5.1.5.1.5	Interleaving	171

5.1.5.1.6	Mapping on a burst	171

5.1.5.1.6.1	BTTI configuration	171

5.1.5.1.6.2	RTTI configuration	171

5.1.5.2	Uplink (MCS-1 UL)	173

5.1.5.2.1	Block constitution	173

5.1.5.2.2	Header coding	173

5.1.5.2.3	Data coding	173

5.1.5.2.3a	Piggy-backed Ack/Nack coding	173

5.1.5.2.4	Interleaving	174

5.1.5.2.5	Mapping on a burst	174

5.1.6	Packet data block type 6 (MCS-2)	174

5.1.6.1	Downlink (MCS-2 DL)	174

5.1.6.1.1	Block constitution	174

5.1.6.1.2	USF precoding	175

5.1.6.1.2.1	BTTI configuration	175

5.1.6.1.2.2	RTTI configuration	175

5.1.6.1.3	Header coding	175

5.1.6.1.4	Data coding	175

5.1.6.1.4a	Piggy-backed Ack/Nack coding	176

5.1.6.1.5	Interleaving	176

5.1.6.1.6	Mapping on a burst	176

5.1.6.2	Uplink (MCS-2 UL)	176

5.1.6.2.1	Block constitution	176

5.1.6.2.2	Header coding	176

5.1.6.2.3	Data coding	176

5.1.6.2.3a	Piggy-backed Ack/Nack coding	177

5.1.6.2.4	Interleaving	177

5.1.6.2.5	Mapping on a burst	177

5.1.7	Packet data block type 7 (MCS-3)	177

5.1.7.1	Downlink (MCS-3 DL)	177

5.1.7.1.1	Block constitution	177

5.1.7.1.2	USF precoding	177

5.1.7.1.2.1	BTTI configuration	177

5.1.7.1.2.2	RTTI configuration	177

5.1.7.1.3	Header coding	177

5.1.7.1.4	Data coding	177

5.1.7.1.4a	Piggy-backed Ack/Nack coding	179

5.1.7.1.5	Interleaving	179

5.1.7.1.6	Mapping on a burst	179

5.1.7.2	Uplink (MCS-3 UL)	179

5.1.7.2.1	Block constitution	179

5.1.7.2.2	Header coding	179

5.1.7.2.3	Data coding	179

5.1.7.2.3a	Piggy-backed Ack/Nack coding	179

5.1.7.2.4	Interleaving	179

5.1.7.2.5	Mapping on a burst	179

5.1.8	Packet data block type 8 (MCS-4)	179

5.1.8.1	Downlink (MCS-4 DL)	179

5.1.8.1.1	Block constitution	179

5.1.8.1.2	USF precoding	180

5.1.8.1.2.1	BTTI configuration	180

5.1.8.1.2.2	RTTI configuration	180

5.1.8.1.3	Header coding	180

5.1.8.1.4	Data coding	180

5.1.8.1.5	Interleaving	181

5.1.8.1.6	Mapping on a burst	181

5.1.8.2	Uplink (MCS-4 UL)	181

5.1.8.2.1	Block constitution	181

5.1.8.2.2	Header coding	181

5.1.8.2.3	Data coding	181

5.1.8.2.4	Interleaving	181

5.1.8.2.5	Mapping on a burst	181

5.1.9	Packet data block type 9 (MCS-5)	181

5.1.9.1	Downlink (MCS-5 DL)	181

5.1.9.1.1	Block constitution	181

5.1.9.1.2	USF precoding	182

5.1.9.1.2.1	BTTI configuration	182

5.1.9.1.2.2	RTTI configurations	182

5.1.9.1.3	Header coding	182

5.1.9.1.4	Data coding	183

5.1.9.1.4a	Piggy-backed Ack/Nack coding	184

5.1.9.1.5	Interleaving	185

5.1.9.1.6	Mapping on a burst	186

5.1.9.2	Uplink (MCS-5 UL)	188

5.1.9.2.1	Block constitution	188

5.1.9.2.2	Header coding	188

5.1.9.2.3	Data coding	189

5.1.9.2.3a	Piggy-backed Ack/Nack coding	189

5.1.9.2.4	Interleaving	189

5.1.9.2.5	Mapping on a burst	189

5.1.10	Packet data block type 10 (MCS-6)	190

5.1.10.1	Downlink (MCS-6 DL)	190

5.1.10.1.1	Block constitution	190

5.1.10.1.2	USF precoding	190

5.1.10.1.2.1	BTTI configuration	190

5.1.10.1.2.2	RTTI configuration	190

5.1.10.1.3	Header coding	190

5.1.10.1.4	Data coding	191

5.1.10.1.4a	Piggy-backed Ack/Nack coding	192

5.1.10.1.5	Interleaving	192

5.1.10.1.6	Mapping on a burst	192

5.1.10.2	Uplink (MCS-6 UL)	192

5.1.10.2.1	Block constitution	192

5.1.10.2.2	Header coding	192

5.1.10.2.3	Data coding	192

5.1.10.2.3a	Piggy-backed Ack/Nack coding	192

5.1.10.2.4	Interleaving	193

5.1.10.2.5	Mapping on a burst	193

5.1.11	Packet data block type 11 (MCS-7)	193

5.1.11.1	Downlink (MCS-7 DL)	193

5.1.11.1.1	Block constitution	193

5.1.11.1.2	USF precoding	193

5.1.11.1.2.2	RTTI configuration	193

5.1.11.1.3	Header coding	193

5.1.11.1.4	Data coding	194

5.1.11.1.4a	Piggy-backed Ack/Nack coding	196

5.1.11.1.5	Interleaving	197

5.1.11.1.6	Mapping on a burst	197

5.1.11.2	Uplink (MCS-7 UL)	198

5.1.11.2.1	Block constitution	198

5.1.11.2.2	Header coding	199

5.1.11.2.3	Data coding	199

5.1.11.2.3a	Piggy-backed Ack/Nack coding	199

5.1.11.2.4	Interleaving	200

5.1.11.2.5	Mapping on a burst	200

5.1.12	Packet data block type 12 (MCS-8)	201

5.1.12.1	Downlink (MCS-8 DL)	201

5.1.12.1.1	Block constitution	201

5.1.12.1.2	USF precoding	201

5.1.12.1.2.1	BTTI configuration	201

5.1.12.1.2.2	RTTI configuration	201

5.1.12.1.3	Header coding	201

5.1.12.1.4	Data coding	201

5.1.12.1.4a	Piggy-backed Ack/Nack coding	202

5.1.12.1.5	Interleaving	203

5.1.12.1.6	Mapping on a burst	204

5.1.12.2	Uplink (MCS-8 UL)	204

5.1.12.2.1	Block constitution	204

5.1.12.2.2	Header coding	205

5.1.12.2.3	Data coding	205

5.1.12.2.3a	Piggy-backed Ack/Nack coding	205

5.1.12.2.4	Interleaving	205

5.1.12.2.5	Mapping on a burst	205

5.1.13	Packet data block type 13 (MCS-9)	206

5.1.13.1	Downlink (MCS-9 DL)	206

5.1.13.1.1	Block constitution	206

5.1.13.1.2	USF precoding	206

5.1.13.1.2.1	BTTI configuration	206

5.1.13.1.2.2	RTTI configuration	206

5.1.13.1.3	Header coding	206

5.1.13.1.4	Data coding	206

5.1.13.1.5	Interleaving	207

5.1.13.1.6	Mapping on a burst	207

5.1.13.2	Uplink (MCS-9 UL)	207

5.1.13.2.1	Block constitution	207

5.1.13.2.2	Header coding	207

5.1.13.2.3	Data coding	207

5.1.13.2.4	Interleaving	207

5.1.13.2.5	Mapping on a burst	207

5.1a	Packet data traffic channels (PDTCH) for EGPRS2	208

5.1a.1	General descriptions of channel coding functions	208

5.1a.1.1	Header	208

5.1a.1.2	Data encoded with convolutional code	209

5.1a.1.3	Data encoded with turbo code	209

5. 1a.1.3.1	Parity bits	209

5.1a.1.3.2	Turbo encoding	209

5.1a.1.3.3	Trellis termination for Turbo coder	211

5.1a.1.3.4	Turbo code internal interleaver	211

5.1a.1.3.4.1	Bits-input to rectangular matrix with padding	211

5.1a.1.3.5	Turbo code puncturing	214

5.1a.1.3.5.2.1	P1 – first puncturing version	215

5.1a.1.3.5.2.2	P2 – second puncturing version – Type 1	215

5.1a.1.3.5.2.3	P2 – second puncturing version – Type 2	215

5.1a.1.3.5.2.4	P3 – third puncturing version	216

5.1a.1.3.5.3	PAN Parameters Handling	216

5.1a.1.4	PAN	221

5.1a.2	General descriptions of interleaving functions	222

5.1a.2.1	Interleaver type 1	222

5.1a.2.2	Interleaver type 2	222

5.1a.3	Packet data block type 14 (UAS-7)	222

5.1a.3.1	Block constitution	222

5.1a.3.2	Header coding	222

5.1a.3.3	Data coding	223

5.1a.3.4	PAN coding	223

5.1a.3.5	Interleaving	223

5.1a.3.6	Mapping on a burst	224

5.1a.4	Packet data block type 15 (UAS-8)	225

5.1a.4.1	Block constitution	225

5.1a.4.2	Header coding	226

5.1a.4.3	Data coding	226

5.1a.4.4	PAN coding	226

5.1a.4.5	Interleaving	226

5.1a.4.6	Mapping on a burst	226

5.1a.5	Packet data block type 16 (UAS-9)	227

5.1a.5.1	Block constitution	227

5.1a.5.2	Header coding	227

5.1a.5.3	Data coding	227

5.1a.5.4	PAN coding	228

5.1a.5.5	Interleaving	228

5.1a.5.6	Mapping on a burst	228

5.1a.6	Packet data block type 17 (UAS-10)	228

5.1a.6.1	Block constitution	228

5.1a.6.2	Header coding	228

5.1a.6.3	Data coding	228

5.1a.6.4	PAN coding	229

5.1a.6.5	Interleaving	229

5.1a.6.6	Mapping on a burst	229

5.1a.7	Packet data block type 18 (UAS-11)	231

5.1a.7.1	Block constitution	231

5.1a.7.2	Header coding	232

5.1a.7.3	Data coding	232

5.1a.7.4	PAN coding	232

5.1a.7.5	Interleaving	232

5.1a.7.6	Mapping on a burst	233

5.1a.8	Packet data block type 19 (UBS-5)	235

5.1a.8.1	Block constitution	235

5.1a.8.2	Header coding	235

5.1a.8.3	Data coding	235

5.1a.8.4	PAN coding	235

5.1a.8.5	Interleaving	236

5.1a.8.6	Mapping on a burst	236

5.1a.9	Packet data block type 20 (UBS-6)	236

5.1a.9.1	Block constitution	236

5.1a.9.2	Header coding	237

5.1a.9.3	Data coding	237

5.1a.9.4	PAN coding	237

5.1a.9.5	Interleaving	237

5.1a.9.6	Mapping on a burst	237

5.1a.10	Packet data block type 21 (UBS-7)	237

5.1a.10.1	Block constitution	237

5.1a.10.2	Header coding	238

5.1a.10.3	Data coding	238

5.1a.10.4	PAN coding	238

5.1a.10.5	Interleaving	238

5.1a.10.6	Mapping on a burst	239

5.1a.11	Packet data block type 22 (UBS-8)	241

5.1a.11.1	Block constitution	241

5.1a.11.2	Header coding	241

5.1a.11.3	Data coding	241

5.1a.11.4	PAN coding	242

5.1a.11.5	Interleaving	242

5.1a.11.6	Mapping on a burst	242

5.1a.12	Packet data block type 23 (UBS-9)	242

5.1a.12.1	Block constitution	242

5.1a.12.2	Header coding	242

5.1a.12.3	Data coding	242

5.1a.12.4	PAN coding	243

5.1a.12.5	Interleaving	243

5.1a.12.6	Mapping on a burst	244

5.1a.13	Packet data block type 24 (UBS-10)	246

5.1a.13.1	Block constitution	246

5.1a.13.2	Header coding	246

5.1a.13.3	Data coding	246

5.1a.13.4	PAN coding	247

5.1a.13.5	Interleaving	247

5.1a.13.6	Mapping on a burst	247

5.1a.14	Packet data block type 25 (UBS-11)	250

5.1a.14.1	Block constitution	250

5.1a.14.2	Header coding	250

5.1a.14.3	Data coding	250

5.1a.14.4	PAN coding	251

5.1a.14.5	Interleaving	251

5.1a.14.6	Mapping on a burst	252

5.1a.15	Packet data block type 26 (UBS-12)	253

5.1a.15.1	Block constitution	253

5.1a.15.2	Header coding	253

5.1a.15.3	Data coding	253

5.1a.15.4	PAN coding	254

5.1a.15.5	Interleaving	254

5.1a.15.6	Mapping on a burst	254

5.1a.16	Packet data block type 27 (DAS-5)	255

5.1a.16.1	Block constitution	255

5.1a.16.2	USF coding	255

5.1a.16.3	Header coding	255

5.1a.16.4	Data coding	255

5.1a.16.5	PAN coding	255

5.1a.16.6	Interleaving	255

5.1a.16.7	Mapping on a burst	256

5.1a.17	Packet data block type 28 (DAS-6)	256

5.1a.17.1	Block constitution	256

5.1a.17.2	USF coding	256

5.1a.17.3	Header coding	256

5.1a.17.4	Data coding	256

5.1a.17.5	PAN coding	257

5.1a.17.6	Interleaving	257

5.1a.17.7	Mapping on a burst	257

5.1a.18	Packet data block type 29 (DAS-7)	257

5.1a.18.1	Block constitution	257

5.1a.18.2	USF coding	257

5.1a.18.3	Header coding	257

5.1a.18.4	Data coding	257

5.1a.18.5	PAN coding	258

5.1a.18.6	Interleaving	258

5.1a.18.7	Mapping on a burst	258

5.1a.19	Packet data block type 30 (DAS-8)	258

5.1a.19.1	Block constitution	258

5.1a.19.2	USF coding	259

5.1a.19.2.1	BTTI configuration	259

5.1a.19.2.2	RTTI configurations	259

5.1a.19.3	Header coding	260

5.1a.19.4	Data coding	260

5.1a.19.5	PAN coding	260

5.1a.19.6	Interleaving	260

5.1a.19.7	Mapping on a burst	261

5.1a.20	Packet data block type 31 (DAS-9)	263

5.1a.20.1	Block constitution	263

5.1a.20.2	USF coding	263

5.1a.20.3	Header coding	263

5.1a.20.4	Data coding	263

5.1a.20.5	PAN coding	264

5.1a.20.6	Interleaving	264

5.1a.20.7	Mapping on a burst	264

5.1a.21	Packet data block type 32 (DAS-10)	264

5.1a.21.1	Block constitution	264

5.1a.21.2	USF coding	265

5.1a.21.2.1	BTTI configuration	265

5.1a.21.2.2	RTTI configurations	265

5.1a.21.3	Header coding	265

5.1a.21.4	Data coding	266

5.1a.21.5	PAN coding	266

5.1a.21.6	Interleaving	266

5.1a.21.7	Mapping on a burst	266

5.1a.22	Packet data block type 33 (DAS-11)	269

5.1a.22.1	Block constitution	269

5.1a.22.2	USF coding	269

5.1a.22.3	Header coding	269

5.1a.22.4	Data coding	270

5.1a.22.5	PAN coding	270

5.1a.22.6	Interleaving	270

5.1a.22.7	Mapping on a burst	271

5.1a.23	Packet data block type 34 (DAS-12)	273

5.1a.23.1	Block constitution	273

5.1a.23.2	USF coding	273

5.1a.23.3	Header coding	273

5.1a.23.4	Data coding	273

5.1a.23.5	PAN coding	274

5.1a.23.6	Interleaving	274

5.1a.23.7	Mapping on a burst	274

5.1a.24	Packet data block type 35 (DBS-5)	276

5.1a.24.1	Block constitution	276

5.1a.24.2	USF coding	277

5.1a.24.2.1	BTTI configuration	277

5.1a.24.2.2	RTTI configurations	277

5.1a.24.3	Header coding	277

5.1a.24.4	Data coding	278

5.1a.24.5	PAN coding	278

5.1a.24.6	Interleaving	278

5.1a.24.7	Mapping on a burst	278

5.1a.25	Packet data block type 36 (DBS-6)	279

5.1a.25.1	Block constitution	279

5.1a.25.2	USF coding	279

5.1a.25.3	Header coding	279

5.1a.25.4	Data coding	279

5.1a.25.5	PAN coding	280

5.1a.25.6	Interleaving	280

5.1a.25.7	Mapping on a burst	280

5.1a.26	Packet data block type 37 (DBS-7)	280

5.1a.26.1	Block constitution	280

5.1a.26.2	USF coding	280

5.1a.26.2.1	BTTI configuration	280

5.1a.26.2.2	RTTI configurations	280

5.1a.26.3	Header coding	281

5.1a.26.4	Data coding	281

5.1a.26.5	PAN coding	282

5.1a.26.6	Interleaving	282

5.1a.26.7	Mapping on a burst	282

5.1a.27	Packet data block type 38 (DBS-8)	284

5.1a.27.1	Block constitution	284

5.1a.27.2	USF coding	285

5.1a.27.3	Header coding	285

5.1a.27.4	Data coding	285

5.1a.27.5	PAN coding	285

5.1a.27.6	Interleaving	285

5.1a.27.7	Mapping on a burst	285

5.1a.28	Packet data block type 39 (DBS-9)	285

5.1a.28.1	Block constitution	285

5.1a.28.2	USF coding	286

5.1a.28.3	Header coding	286

5.1a.28.4	Data coding	286

5.1a.28.5	PAN coding	286

5.1a.28.6	Interleaving	286

5.1a.28.7	Mapping on a burst	287

5.1a.29	Packet data block type 40 (DBS-10)	289

5.1a.29.1	Block constitution	289

5.1a.29.2	USF coding	289

5.1a.29.2.1	BTTI configuration	289

5.1a.29.2.2	RTTI configurations	290

5.1a.29.3	Header coding	290

5.1a.29.4	Data coding	290

5.1a.29.5	PAN coding	291

5.1a.29.6	Interleaving	291

5.1a.29.7	Mapping on a burst	291

5.1a.30	Packet data block type 41 (DBS-11)	294

5.1a.30.1	Block constitution	294

5.1a.30.2	USF coding	294

5.1a.30.3	Header coding	294

5.1a.30.4	Data coding	294

5.1a.30.5	PAN coding	295

5.1a.30.6	Interleaving	295

5.1a.30.7	Mapping on a burst	296

5.1a.31	Packet data block type 42 (DBS-12)	297

5.1a.31.1	Block constitution	297

5.1a.31.2	USF coding	297

5.1a.31.3	Header coding	297

5.1a.31.4	Data coding	297

5.1a.31.5	PAN coding	298

5.1a.31.6	Interleaving	298

5.1a.31.7	Mapping on a burst	299

5.1b	Extended Coverage Packet data traffic channel (EC-PDTCH)	299

5.1b.1	General	299

5.1b.2	Downlink MCS-1/M	300

5.1b.2.1	Block constitution	300

5.1b.2.2	Encoding and blind physical layer transmission	300

5.1b.2.3	Mapping onto PDCHs	300

5.1b.3	Uplink MCS-1/M	301

5.1b.3.1	Block constitution	301

5.1b.3.2	Encoding and blind physical layer transmission	301

5.1b.3.3	Mapping onto PDCHs	301

5.1b.4	Uplink MCS-1’/48	301

5.1b.4.1	Block constitution	301

5.1b.4.2	Header coding	301

5.1b.4.3	Data coding	302

5.1b.4.4	Interleaving	303

5.1b.4.5	Mapping on a burst	303

5.1b.4.6	Blind physical layer transmission	303

5.1b.4.7	Mapping onto PDCHs	304

5.2	Packet control channels (PACCH, PBCCH, PAGCH, PPCH, PTCCH, CPBCCH, CPAGCH and CPPCH)	304

5.2a	Extended Coverage broadcast control channel (EC-BCCH)	304

5.2a.1	Block constitution	304

5.2a.2	Coding	305

5.2a.3	Blind physical layer transmission	305

5.2a.4	Mapping onto a physical channel	305

5.2b	Extended coverage common control channel downlink (EC-CCCH/D/M) , Extended coverage paging indication channel (EC-PICH) and Extended coverage packet associated control channel (EC-PACCH/M)	305

5.2b.1	Block constitution	305

5.2b.2	Data coding	305

5.2b.3	USF coding (only for EC-PACCH/D)	306

5.2b.4	Mapping on a Burst	306

5.2b.4.1	EC-CCCH/D/M and EC-PICH	306

5.2b.4.2	EC-PACCH/D/M	306

5.2b.4.3	EC-PACCH/U/M	307

5.2b.5	Blind physical layer transmission	307

5.2b.6	Bit stealing for USF (only for EC-PACCH/D)	307

5.2b.7	Mapping onto physical channels	308

5.2b.7.1	EC-PACCH	308

5.2b.7.2	EC-CCCH/D and EC-PICH	308

5.3	Packet random access channel (PRACH, CPRACH and MPRACH)	308

5.3.1	Packet Access Burst	308

5.3.2	Extended Packet Access Burst	308

5.3.2.1	Block constitution	308

5.3.2.2	Parity bits	308

5.3.2.3	Addition of BSIC	308

5.3.2.4	Convolutional code	309

5.3a	Extended coverage random access channel (EC-RACH/M)	309

5.3a.0	General	309

5.3a.1	Block constitution	309

5.3a.2	Coding	309

5.3a.3	Blind physical layer transmissions	310

5.3a.4	Mapping onto physical channel(s)	310

5.3a.4.1	1 TS EC-RACH	310

5.3a.4.2	2 TS EC-RACH	310

5.3a.5	EC-RACH carrying 30 information bits	310

5.3a.6	Extended coverage random access channel (EC-RACH/66)	310

5.3a.6.0	General	310

5.3a.6.1	Block constitution	310

5.3a.6.2	Parity bits	310

5.3a.6.3	Addition of BSIC	311

5.3a.6.4	Tail biting Convolutional code	311

5.3a.6.5	Blind physical layer transmissions	311

5.3a.6.6	Mapping onto physical channel(s)	311

5.3a.7	Extended coverage random access channel (EC-RACH/132)	312

5.3a.7.0	General	312

5.3a.7.1	Block constitution	312

5.3a.7.2	Parity bits	312

5.3a.7.3	Addition of BSIC	312

5.3a.7.4	Convolutional code	312

5.3a.7.5	Blind physical layer transmissions	312

5.3a.7.6	Mapping onto physical channel(s)	312

5.4	Access Burst on packet switched channels other than PRACH, CPRACH and MPRACH	313

6	Flexible Layer One	332

6.1	General	332

6.2	Transport channel coding/multiplexing	332

6.2.1	CRC Attachment	333

6.2.2	Channel Coding	334

6.2.3	Rate Matching	335

6.2.4	Transport Channel multiplexing	337

6.2.5	TFCI Encoding	338

6.2.6	In-band signalling encoding	338

6.2.7	Radio packet mapping	339

6.2.8	Interleaving	339

6.2.9	Mapping on a Burst	341

6.2.10	Signalling on Half Rate Channels	342

Annex A (informative):	Summary of Channel Types	343

Annex B (informative):	Summary of Polynomials Used for Convolutional Codes and Turbo Codes	346

Annex C (informative):	Change history	347

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

A reference configuration of the transmission chain is shown in 3GPP TS 45.001. According to this reference configuration, the present document specifies the data blocks given to the encryption unit.

It includes the specification of encoding, reordering, interleaving and the stealing flag. It does not specify the channel decoding method.

The definition is given for each kind of logical channel, starting from the data provided to the channel encoder by the speech coder, the data terminal equipment, or the controller of the Mobile Station (MS) or Base Transceiver Station (BTS). The definitions of the logical channel types used in this technical specification are given in 3GPP TS 45.002, a summary is in annex A.

Additionally, the present document describes the characteristics of the coding/multiplexing unit for the Flexible Layer One (FLO) starting from the transport blocks provided by higher layers. An overview of FLO is given in 3GPP TR 45.902.

## 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.090: "AMR speech Codec; Transcoding Functions".

[3]	3GPP TS 26.190: "Mandatory Speech Codec speech processing functions AMR Wideband speech codec; Transcoding functions".

[4]	3GPP TS 44.018: "Mobile radio interface layer 3 specification, Radio Resource Control Protocol".

[5]	3GPP TS 44.021: "Rate adaption on the Mobile Station - Base Station System (MS - BSS) interface".

[6]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/ Medium Access Control (RLC/MAC) protocol".

[7]	3GPP TS 45.001: "Physical Layer on the Radio Path (General Description)".

[8]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[9]	3GPP TS 45.004: "Modulation".

[10]	3GPP TS 45.008: "Radio subsystem link control".

[11]	3GPP TS 45.009: "Link adaptation".

[12]	3GPP TR 45.902: "Flexible Layer One".

[13]	3GPP TS 46.010: "Full rate speech transcoding".

[14]	3GPP TS 46.020: "Half rate speech transcoding".

[15]	3GPP TS 46.060: "Enhanced full rate speech transcoding".

## 1.2	Abbreviations

Abbreviations used in the present document are listed in 3GPP TR 21.905. In addition to abbreviations in 3GPP TR 21.905 the following abbreviations apply:

BTTI	Basic Transmission Time Interval

ESAB	Extended Synchronization Access Burst

EDAB	Extended Dual slot Access Burst

eTFI	Extended Temporary Flow Identity

FANR	Fast Ack/Nack Reporting

PAN	Piggy-backed Ack/Nack

PANI	Piggy-backed Ack/Nack Indicator

RTTI	Reduced Transmission Time Interval

TFI	Temporary Flow Identity

TTI	Transmission Time Interval

# 2	General

## 2.1	General organization

Each channel has its own coding and interleaving scheme. However, the channel coding and interleaving is organized in such a way as to allow, as much as possible, a unified decoder structure.

Each channel uses the following sequence and order of operations:

-	the information bits are coded with a systematic block code, building words of information + parity bits;

-	these information + parity bits are encoded with a convolutional code or a turbo code, building the coded bits;

-	reordering and interleaving the coded bits, and adding a stealing flag, gives the interleaved bits.

All these operations are made block by block, the size of which depends on the channel. However, most of the channels use a block of either 456 coded bits or 1368 coded bits, corresponding to 456 coded symbols, which is interleaved and mapped onto bursts in a very similar way for all of them. This block of 456 coded symbols is the basic structure of the channel coding scheme. Figures 1a, 1b, 1c, 1d, 1ea, 1eb, 1f, 1g, 1h, 1i, 1j, 1k, 1l, 2 and 2aa give diagrams showing the general structure of the channel coding.

In the case of full rate speech TCH, a block of 456 coded bits carries the information of one speech frame. In case of control channels, it carries one message.

In the case of half rate speech TCH, the information of one speech frame is carried in a block of 228 coded bits.

In the case of the Enhanced full rate speech the information bits coming out of the source codec first go through a preliminary channel coding. Then the channel coding as described above takes place.

In the case of 8-PSK modulated speech TCH, the information of one speech frame is carried in a block of 1368 coded bits (456 coded symbols) for full rate channels or 684 coded bits (228 coded symbols) for half rate channels.

In the case of a packet switched channel the block of 456, 1096, 1384, 1848, 2200, 2312 or 2748 coded bits carries one RLC/MAC block.

In the case of E-TCH/F28.8 or E-TCH/F43.2, the block of 1368 coded bits (456 coded symbols) carries one radio interface data block. In the case of E-TCH/F32.0, the block of 1392 coded bits (464 coded symbols) carries one radio interface data block.

In the case of FACCH, a coded message block of 456 bits is divided into eight sub-blocks. The first four sub-blocks are sent by stealing the even numbered bits of four timeslots in consecutive frames used for the TCH. The other four sub-blocks are sent by stealing the odd numbered bits of the relevant timeslot in four consecutive used frames delayed 2 or 4 frames relative to the first frame. Along with each block of 456 coded bits there is, in addition, a stealing flag (8 bits), indicating whether the block belongs to the TCH or to the FACCH. In the case of SACCH, BCCH, CCCH or CTSCCH, this stealing flag is dummy. In the case of a packet switched channel, these bits are used to indicate the coding scheme used.

In the case of E-FACCH/F, a coded message block of 456 bits is divided into four sub-blocks. The four sub-blocks are sent by stealing all symbols of four timeslots in consecutive frames used for the E-TCH and using GMSK modulation. The indication of the E-FACCH/F is based on the identification of the modulation.  Along with each block of 456 coded bits there is, in addition, a stealing flag (8 bits), indicating whether the block belongs to the E-FACCH, FACCH or TCH.

Some cases do not fit in the general organization, and use short blocks of coded bits which are sent completely in one timeslot. They are the random access messages of:

-	the RACH;

-	or PRACH, CPRACH and MPRACH;

on uplink and the synchronization information broadcast on the SCH or CSCH on the downlink. In CTS, they are the access request message of the CTSARCH on uplink and the information broadcast on the CTSBCH-SB on downlink.

In the coding/multiplexing unit of FLO, error detection, forward error correction and rate matching is applied to each transport channel independently. However the transport channels share a common multiplexing, TFCI mapping, interleaving and burst mapping. All these operations are made every transmission time interval and the number of coded bits produced by the coding/multiplexing unit depends on the basic physical subchannel. In the case of full rate GMSK basic physical subchannel, blocks of 464 bits are produced. In the case of half rate GMSK basic physical subchannel, blocks of 232 bits are produced. In the case of full rate 8PSK basic physical subchannel, blocks of 1392 bits are produced. In the case of half rate 8PSK basic physical subchannel, blocks of 696 bits are produced.

For EC-GSM-IoT, the same nine modulation and coding schemes as for EGPRS are defined for the extended coverage packet data traffic channels (EC-PDTCH). In addition three modulation and coding schemes employing MCS-1 and blind physical layer transmissions are defined for EC-PDTCH. For extended coverage common control channels in downlink (EC-CCCH/D) and the extended coverage packet associated control channel (EC-PACCH/U and EC-PACCH/D), three coding schemes having the same structure, differing mainly in message size and puncturing, are defined. The encoded bits are mapped onto a single burst, which is transmitted using blind physical layer transmissions. The extended coverage broadcast control channel (EC-BCCH) uses the same coding scheme as the broadcast control channel (BCCH) with subsequent blind physical layer transmissions. The extended coverage synchronization channel (EC-SCH) uses the same coding scheme as the synchronization channel (SCH), with subsequent blind physical layer transmissions and cyclic bit shifting. The extended coverage random access channel (EC-RACH) uses the same coding scheme as the random access channel (RACH), with subsequent blind physical layer transmissions, with the exception of the EC-RACH used for coverage class CC5, that applies modified coding schemes with subsequent blind physical layer transmissions. Figures 1m, 1n, 2ab, 2ac, 2ad, 2ae,2af and 2ag give diagrams showing the general structure of the channel coding for EC-channels.

Figure 1a: Channel Coding and Interleaving Organization for speech, circuit switched data and GPRS packet data channels

In each box, the last line indicates the subclause defining the function. In the case of data TCHs, N0, N1 and n depend on the type of data TCH. In the case of PDTCH, Q0, Q1 and n depend on the coding scheme.

Interfaces:

0)	speech bits from the speech encoder (s);

1)	information bits (d);

2)	information + parity + tail bits (u);

3)	coded bits (c);

4)	interleaved bits (e).

Figure 1b: Channel Coding and Interleaving Organization, adaptive multi-rate speech

In each box, the last line indicates the subclause defining the function.

Interfaces:

0)	speech bits from the speech encoder (s);

1)	reordered speech bits (d);

2)	speech + parity + tail bits (u);

3)	coded bits (c);

4)	interleaved bits (e).

Figure 1c: Channel Coding and Interleaving Organization, wide-band adaptive multi-rate speech

Figure 1d: Channel Coding and Interleaving Organization for ECSD 8-PSK modulated signals

In each box, the last line indicates the subclause defining the function.

Figure 1ea: Channel Coding and Interleaving Organization for EGPRS Uplink Packet Data Channels

In each box, the last line indicates the subclause defining the function.

Figure 1eb: Channel Coding and Interleaving Organization for EGPRS Downlink Packet Data Channels

In each box, the last line indicates the subclause defining the function.

Figure 1f: Channel Coding and Interleaving Organization for EGPRS2-A Uplink Packet Data Channels

In each box, the last line indicates the subclause defining the function.

Figure 1g: Channel Coding and Interleaving Organization for EGPRS2-B Uplink Packet Data Channels, UBS-5 to UBS-9

In each box, the last line indicates the subclause defining the function.

Figure 1h: Channel Coding and Interleaving Organization for EGPRS2-B Uplink Packet Data Channels, UBS-10 to UBS-12

In each box, the last line indicates the subclause defining the function.

Figure 1i: Channel Coding and Interleaving Organization for EGPRS2-A Downlink Packet Data Channels, DAS-5 to DAS-9

In each box, the last line indicates the subclause defining the function.

Figure 1j: Channel Coding and Interleaving Organization for EGPRS2-A Downlink Packet Data Channels, DAS-10 to DAS-12

In each box, the last line indicates the subclause defining the function.

Figure 1k: Channel Coding and Interleaving Organization for EGPRS2-B Downlink Packet Data Channels, DBS-5 to DBS-9

Figure 1l: Channel Coding and Interleaving Organization for EGPRS2-B Downlink Packet Data Channels, DBS-10 to DBS-12

In each box, the last line indicates the subclause defining the function.

Figure 1m: Channel Coding and Interleaving Organization for EC-GSM-IoT Downlink Packet Data Traffic Channels

Figure 1n: Channel Coding and Interleaving Organization for EC-GSM-IoT Uplink Packet Data Traffic Channels (MCS-1/M is used for CC1 to CC4, MCS-1'/48 for CC5.)

In each box, the last line indicates the subclause defining the function. M denotes the number of blind physical layer transmissions, see subclause 5.1b.2.2 and 5.1b.3.2 for the downlink and uplink respectively.

Figure 2: Channel Coding and Interleaving Organization for control channels except PACCH, EC-BCCH, EC-SCH, EC-CCCH and EC-PACCH

In each box, the last line indicates the subclause defining the function. In the case of RACH, PRACH and of MPRACH using Packet Access Burst, P0 = 8 and P1 = 18; in the case of PRACH and of MPRACH using Extended Packet Access Burst, P0 = 11 and P1 = 21; in the case of SCH, CSCH, CTSBCH-SB and CTSARCH, P0 = 25 and P1 = 39.

Figure 2aa: Channel Coding and Interleaving Organization for PACCH

In each box, the last line indicates the subclause defining the function.

Figure 2ab: Channel Coding and Interleaving Organization for EC-BCCH

Figure 2ac: Channel Coding and Interleaving Organization for EC-SCH

Figure 2ad: Channel Coding and Interleaving Organization for EC-CCCH/D and EC-PACCH/U

Figure 2ae: Channel Coding and Interleaving Organization for EC-PACCH/D

Figure 2af: Channel Coding and Interleaving Organization for EC-RACH (M=1, 4, 16, 48 is used for CC1, CC2, CC3, CC4 and M=132 for CC5).

Figure 2ag: Channel Coding Organization for EC-RACH (M=66)

In each box, the last line indicates the subclause defining the function. M denotes the number of blind physical layer transmissions.

## 2.2	Naming Convention

For ease of understanding a naming convention for bits is given for use throughout the technical specification:

-	General naming:

"k" and "j" for numbering of bits or symbols in data blocks and bursts;

"Kx" gives the amount of bits or symbols in one block, where "x" refers to the data type;

"n" is used for numbering of delivered data blocks where;

"N" marks a certain data block;

"m" is used for numbering of blind physical layer transmissions where;

"M" gives the number of transmissions;

"B" is used for numbering of bursts or blocks where;

"B0" marks the first burst or block carrying bits from the data block with n = 0 (first data block in the transmission).

-	Data delivered to the preliminary channel encoding unit (for EFR only):

s(k)			for	k = 1..., Ks

-	Data delivered by the preliminary channel encoding unit (for EFR only) before bits rearrangement

w(k)			for	k = 1..., Kw

-	Data bits delivered to the encoding unit (interface 1 in figures 1a, 1b, 1c, 1ea, 1eb, 2 and 2aa):

d(k)			for	k = 0,1,...,Kd-1

-	Data bits delivered to the encoding unit for EC-GSM-IoT MCS-1/M:

a(k)			for	k = 0,1,...,Ka-1

-	Data symbols delivered to the encoding unit (interface 1 in figure 1d):

D(k)			for	k = 0,1,...,KD-1

-	Input in-band data bits (for TCH/AMR only):

id(k)			for	k = 0, 1

-	Encoded in-band data bits (for TCH/AMR only):

ic(k)			for	k = 0, 1,...,3 TCH/AHS speech frames or

k = 0, 1,...,7 TCH/AFS or TCH/WFS speech frames or

k = 0, 1,...,11 O-TCH/AHS or O-TCH/WHS speech frames or

k = 0, 1,…,23 O-TCH/WFS speech frames or

k = 0, 1,...,15 TCH/AMR SID and RATSCCH frames

-	Code identifying the used coding scheme (for packet switched channels only):

q(k)			for k = 0,1,..., 7

-	Data bits after the first encoding step (block code, cyclic code; interface 2 in figures 1a, 1b, 1c, 1ea, 1eb, 2 and 2aa):

u(k)			for	k = 0,1,...,Ku-1

-	Data symbols after the first encoding step (block code; interface 2 in figure 1d):

U(k)			for	k = 0,1,...,KU-1

-	Data put into the shift register of the convolutional code and calculated from the data bits u(k) and the feedback bits in recursive systematic convolutional codes

r(k)			for k= 0,1,..., Kr-1

-	Data after the second encoding step (convolutional code ; interface 3 in figures 1a, 1b, 1c, 1d, 1ea, 1eb, 2 and 2aa):

c(n,k) or c(k)	for	k = 0,1,...,Kc-1

n = 0,1,...,N,N+1,...

-	Interleaved data bits:

i(B,k)			for	k = 0,1,...,Ki-1

B = B0, B0+1,....

-	Interleaved data symbols:

I(B,k)			for	k = 0,1,...,KI-1

- B = B0, B0+1,....

-	Bits in one burst (interface 4 in figures 1a, 1b, 1c, 1ea, 1eb, 2 and 2aa):

e(B,k)			for	k = 0,1,...,114,115

B = B0,B0+1,...

-	Symbols in one burst (interface 4 in figure 1d):

E(B,k)			for	k = 0,1,...,114,115

B = B0,B0+1,...

-	Bits in transmission m of burst B after blind physical layer transmissions:

R(m,B,k)		for	k = 0,1,...,KR-1

B = 0,1,...

m = 0,...,M-1

-	E-IACCH messages delivered to the block coding of inband signalling (for ECSD only):

im(k) or im(n,k)

for	k = 0,1,2

n = 0,1,...,N,N+1,...

-	E-IACCH bits delivered to the mapping on one burst (for ECSD only):

ib(B,k)		for	k = 0,1,...,5

B = B0, B0+1,....

-	E-IACCH symbols in one burst (for ECSD only):

HL(B) and HU(B)

for	B = B0, B0+1,....

-	EPCCH messages delivered to the block coding (for SACCH/TP only):

pm(k) or pm(n,k)

for	k = 0,1,2

n = 0,1,...,N,N+1,...

-	EPCCH bits delivered to the mapping on one burst (for SACCH/TP only):

pb(B,k)		for	k = 0,1,...,11

B = B0, B0+1,....

# 3	Traffic Channels (TCH)

Two kinds of traffic channel are considered: speech and data. Both of them use the same general structure (see figure 1), and in both cases, a piece of information can be stolen by the FACCH.

## 3.1	Speech channel at full rate (TCH/FS and TCH/EFS)

The speech coder (whether Full rate or Enhanced full rate) delivers to the channel encoder a sequence of blocks of data. In case of a full rate and enhanced full rate speech TCH, one block of data corresponds to one speech frame.

For the full rate coder each block contains 260 information bits, including 182 bits of class 1 (protected bits), and 78 bits of class 2 (no protection), (see table 2).

The bits delivered by the speech coder are received in the order indicated in 3GPP TS 46.010 and have to be rearranged according to table 2 before channel coding as defined in subclauses 3.1.1 to 3.1.4. The rearranged bits are labelled {d(0),d(1),...,d(259)}, defined in the order of decreasing importance.

For the EFR coder each block contains 244 information bits. The block of 244 information bits, labelled s(1).., s(244), passes through a preliminary stage, applied only to EFR (see figure 1) which produces 260 bits corresponding to the 244 input bits and 16 redundancy bits. Those 16 redundancy bits correspond to 8 CRC bits and 8 repetition bits, as described in subclause 3.1.1. The 260 bits, labelled w(1)..w(260), have to be rearranged according to table 6 before they are delivered to the channel encoding unit which is identical to that of the TCH/FS. The 260 bits block includes 182 bits of class 1(protected bits) and 78 bits of class 2 (no protection). The class 1 bits are further divided into the class 1a and class 1b, class 1a bits being protected by a cyclic code and the convolutional code whereas the class 1b are protected by the convolutional code only.

### 3.1.1	Preliminary channel coding for EFR only

#### 3.1.1.1	CRC calculation

An 8-bit CRC is used for error-detection. These 8 parity bits (bits w253-w260) are generated by the cyclic generator polynomial: g(D) = D8 + D4 + D3 + D2 + 1 from the 65 most important bits (50 bits of class 1a and 15 bits of class 1b). These 65 bits (b(1)-b(65)) are taken from the table 5 in the following order (read row by row, left to right):

| s39 | s40 | s41 | s42 | s43 | s44 | s48 | s87 | s45 | s2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s3 | s8 | s10 | s18 | s19 | s24 | s46 | s47 | s142 | s143 |
| s144 | s145 | s146 | s147 | s92 | s93 | s195 | s196 | s98 | s137 |
| s148 | s94 | s197 | s149 | s150 | s95 | s198 | s4 | s5 | s11 |
| s12 | s16 | s9 | s6 | s7 | s13 | s17 | s20 | s96 | s199 |
| s1 | s14 | s15 | s21 | s25 | s26 | s28 | s151 | s201 | s190 |
| s240 | s88 | s138 | s191 | s241 |  |  |  |  |  |

The encoding is performed in a systematic form, which means that, in GF(2), the polynomial:

-	b(1)D72 + b(2)D71 +...+b(65)D8 + p(1)D7 + p(2)D6 +...+ p(7)D1 + p(8);

-	p(1) - p(8): the parity bits (w253-w260);

-	b(1) - b(65) = the data bits from the table above;

when divided by g(D), yields a remainder equal to 0.

#### 3.1.1.2	Repetition bits

The repeated bits are s70, s120, s173 and s223. They correspond to one of the bits in each of the PULSE_5, the most significant one not protected by the channel coding stage.

#### 3.1.1.3	Correspondence between input and output of preliminary channel coding

The preliminary coded bits w(k) for k = 1 to 260 are hence defined by:

w(k) = s(k)	for k = 1 to 71

w(k) = s(k-2) for k = 74 to 123

w(k) = s(k-4) for k = 126 to 178

w(k) = s(k-6) for k = 181 to s230

w(k) = s(k-8) for k = 233 to s252

Repetition bits:

w(k) = s(70)	for k = 72 and 73

w(k) = s(120) for k = 124 and 125

w(k) = s(173) for k = 179 and 180

w(k) = s(223) for k = 231 and 232

Parity bits:

w(k = p(k-252) for k = 253 to 260

### 3.1.2	Channel coding for FR and EFR

#### 3.1.2.1	Parity and tailing for a speech frame

a)	Parity bits:

The first 50 bits of class 1 (known as class 1a for the EFR) are protected by three parity bits used for error detection. These parity bits are added to the 50 bits, according to a degenerate (shortened) cyclic code (53,50,2), using the generator polynomial:

g(D) = D3 + D + 1

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D52 + d(1)D51 +... + d(49)D3 + p(0)D2 + p(1)D+ p(2)

where p(0), p(1), p(2) are the parity bits, when divided by g(D), yields a remainder equal to:

1 + D + D2

b)	Tailing bits and reordering:

The information and parity bits of class 1 are reordered, defining 189 information + parity + tail bits of class 1, {u(0),u(1),...,u(188)} defined by:

u(k)	= d(2k)	and	u(184-k) = d(2k+1)		for k = 0,1,...,90

u(91+k)	= p(k)											for k = 0,1,2

u(k)	= 0												for k = 185,186,187,188 (tail bits)

#### 3.1.2.2	Convolutional encoder

The class 1 bits are encoded with the ½ rate convolutional code defined by the polynomials:

G0 = 1 + D3+ D4

G1 = 1 + D + D3+ D4

The coded bits {c(0), c(1),..., c(455)} are then defined by:

- class 1:	c(2k)	=		u(k) + u(k-3) + u(k-4)

c(2k+1)	=	u(k) + u(k-1) + u(k-3) + u(k-4)		for	k = 0,1,...,188

u(k) = 0 for k < 0

-	class 2:	c(378+k)	= d(182+k)			for	k = 0,1,....,77

### 3.1.3	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k),	for	k  = 0,1,...,455

n  = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 8)

j   = 2((49k) mod 57) + ((k mod 8) div 4)

See table 1. The result of the interleaving is a distribution of the reordered 456 bits of a given data block, n = N, over 8 blocks using the even numbered bits of the first 4 blocks (B = B0 + 4N + 0, 1, 2, 3) and odd numbered bits of the last 4 blocks (B = B0 + 4N + 4, 5, 6, 7). The reordered bits of the following data block, n = N+1, use the even numbered bits of the blocks B = B0 + 4N + 4, 5, 6, 7 (B = B0 + 4(N+1) + 0, 1, 2, 3) and the odd numbered bits of the blocks B = B0 + 4(N+1) + 4, 5, 6, 7. Continuing with the next data blocks shows that one block always carries 57 bits of data from one data block (n = N) and 57 bits of data from the next block (n = N+1), where the bits from the data block with the higher number always are the even numbered data bits, and those of the data block with the lower number are the odd numbered bits.

The block of coded data is interleaved "block diagonal", where a new data block starts every 4th block and is distributed over 8 blocks.

### 3.1.4	Mapping on a Burst

The mapping is given by the rule:

e(B,j)	= i(B,j)	and	e(B,59+j) = i(B,57+j)		for j = 0,1,...,56

and

e(B,57) = hl(B)	and	e(B,58) = hu(B)

The two bits, labelled hl(B) and hu(B) on burst number B are flags used for indication of control channel signalling. For each TCH/FS block not stolen for signalling purposes:

hu(B) = 0	for the first 4 bursts	(indicating status of even numbered bits)

hl(B)  = 0	for the last 4 bursts	(indicating status of odd numbered bits)

For the use of hl(B) and hu(B) when a speech frame is stolen for signalling purposes see subclause 4.2.5.

## 3.2	Speech channel at half rate (TCH/HS)

The speech coder delivers to the channel encoder a sequence of blocks of data. In case of a half rate speech TCH, one block of data corresponds to one speech frame. Each block contains 112 bits, including 95 bits of class 1 (protected bits), and 17 bits of class 2 (no protection), see tables 3a and 3b.

The bits delivered by the speech coder are received in the order indicated in 3GPP TS 46.020 and have to be arranged according to either table 3a or table 3b before channel encoding as defined in subclauses 3.2.1 to 3.2.4. The rearranged bits are labelled {d(0),d(1),...,d(111)}. Table 3a has to be taken if parameter Mode = 0 (which means that the speech encoder is in unvoiced mode), while table 3b has to be taken if parameter Mode = 1, 2 or 3 (which means that the speech encoder is in voiced mode).

### 3.2.1	Parity and tailing for a speech frame

a)	Parity bits:

The most significant 22 class 1 bits d(73),d(74),...,d(94) are protected by three parity bits used for error detection. These bits are added to the 22 bits, according to a cyclic code using the generator polynomial:

g(D) = D3 + D + 1

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(73)D24 + d(74)D23 + ... + d(94)D3 + p(0)D2 + p(1)D + p(2)

where p(0), p(1), p(2) are the parity bits, when divided by g(D), yields a remainder equal to:

1 + D + D2.

b)	Tail bits and reordering:

The information and parity bits of class 1 are reordered, defining 104 information + parity + tail bits of class 1, {u(0),u(1),...,u(103)} defined by:

u(k) = d(k)		for k = 0,1,...,94

u(k) = p(k-95)	for k = 95,96,97

u(k) = 0			for k = 98,99,...,103 (tail bits)

### 3.2.2	Convolutional encoder

The class 1 bits are encoded with the punctured convolutional code defined by the mother polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G5 = 1 + D + D4 + D6

G6 = 1 + D + D2 + D3 + D4 + D6

and the puncturing matrices:

(1,0,1)		for {u(0),u(1),...,u(94)} (class 1 information bits);

and {u(98),u(99),...,u(103)} (tail bits).

(1,1,1)		for {u(95),u(96),u(97)} (parity bits)

In the puncturing matrices, a 1 indicates no puncture and a 0 indicates a puncture.

The coded bits {c(0),c(1),...,c(227)} are then defined by:

class 1 information bits:

c(2k)		= u(k)+u(k-2)+u(k-3)+ (k-5)+u(k-6)

c(2k+1)	= u(k)+u(k-1)+u(k-2)+u(k-3)+u(k-4)+u(k-6)			for k = 0,1,...,94;u(k) = 0 for k<0

parity bits:

c(3k-95)	= u(k)+u(k-2)+u(k-3)+u(k-5)+u(k-6)

c(3k-94)	= u(k)+u(k-1)+u(k-4)+u(k-6)

c(3k-93)	= u(k)+u(k-1)+u(k-2)+u(k-3)+u(k-4)+u(k-6)			for k = 95,96,97

tail bits:

c(2k+3)	= u(k)+u(k-2)+u(k-3)+u(k-5)+u(k-6)

c(2k+4)	= u(k)+u(k-1)+u(k-2)+u(k-3)+u(k-4)+u(k-6)			for k = 98,99,...,103

class 2 information bits:

c(k+211)	= d(k+95)	for k = 0,1,...,16

### 3.2.3	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)		for		k = 0,1,...,227

n = 0,1,...,N,N+1,...

B = B0 + 2n + b

The values of b and j in dependence of k are given by table 4.

The result of the interleaving is a distribution of the reordered 228 bits of a given data block, n = N, over 4 blocks using the even numbered bits of the first 2 blocks (B = B0+2N+0,1) and the odd numbered bits of the last 2 blocks (B = B0+2N+2,3). The reordered bits of the following data block, n = N + 1, use the even numbered bits of the blocks B = B0 + 2N + 2,3 (B = B0+2(N+1)+0,1) and the odd numbered bits of the blocks B = B0 + 2(N+1) + 2,3. Continuing with the next data blocks shows that one block always carries 57 bits of data from one data block (n = N) and 57 bits from the next block (n = N+1), where the bits from the data block with the higher number always are the even numbered data bits, and those of the data block with the lower number are the odd numbered bits. The block of coded data is interleaved "block diagonal", where a new data block starts every 2nd block and is distributed over 4 blocks.

### 3.2.4	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j) and e(B,59+j) = i(B,57+j) for j = 0,1,...,56

and

e(B,57) = hl(B) and e(B,58) = hu(B)

The two bits, labelled hl(B) and hu(B) on burst number B are flags used for indication of control channel signalling. For each TCH/HS block not stolen for signalling purposes:

hu(B) = 0	for the first 2 bursts (indicating status of the even numbered bits)

hl(B)  = 0	for the last 2 bursts (indicating status of the odd numbered bits)

For the use of hl(B) and hu(B) when a speech frame is stolen for signalling purposes, see subclause 4.3.5.

## 3.3	Data channel at full rate, 12.0 kbit/s radio interface rate (9.6 kbit/s services (TCH/F9.6))

The definition of a 12.0 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.3.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 60 information bits (data frames) every 5 ms. Four such blocks are dealt with together in the coding process {d(0),...,d(239)}. For non-transparent services those four blocks shall align with one 240-bit RLP frame.

### 3.3.2	Block code

The block of 4 * 60 information bits is not encoded, but only increased with 4 tail bits equal to 0 at the end of the block.

u(k) = d(k)		for k = 0,1,...,239

u(k) = 0			for k = 240,241,242,243 (tail bits)

### 3.3.3	Convolutional encoder

This block of 244 bits {u(0),...,u(243)} is encoded with the ½ rate convolutional code defined by the following polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3+ D4

resulting in 488 coded bits {C(0), C(1),..., C(487)} with

C(2k)		= u(k) + u(k-3) + u(k-4)

C(2k+1)	= u(k) + u(k-1) + u(k-3) + u(k-4)	for k = 0,1,...,243 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following 32 coded bits:

{C(11+15j) for j = 0,1,...,31} are not transmitted.

The result is a block of 456 coded bits, {c(0),c(1),..., c(455)}

### 3.3.4	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)	for	k  = 0,1,...,455	
				n  = 0,1,...,N,N + 1,...	
				B  = B0 +4n + (k mod 19) + (k div 114)	
				j   = (k mod 19) + 19 (k mod 6)

The result of the interleaving is a distribution of the reordered 114 bit of a given data block, n = N, over 19 blocks, 6 bits equally distributed in each block, in a diagonal way over consecutive blocks.

Or in other words the interleaving is a distribution of the encoded, reordered 456 bits from four given input data blocks, which taken together give n = N, over 22 bursts, 6 bits equally distributed in the first and 22nd bursts, 12 bits distributed in the second and 21st bursts, 18 bits distributed in the third and 20th bursts and 24 bits distributed in the other 16 bursts.

The block of coded data is interleaved "diagonal", where a new block of coded data starts with every fourth burst and is distributed over 22 bursts.

### 3.3.5	Mapping on a Burst

The mapping is done as specified for TCH/FS in subclause 3.1.4. On bitstealing by a FACCH, see subclause 4.2.5.

## 3.4	Data channel at full rate, 6.0 kbit/s radio interface rate (4.8 kbit/s services (TCH/F4.8))

The definition of a 6.0 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.4.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 60 information bits (data frames) every 10 ms, {d(0),d(1),...,d(59)}.

In the case where the user unit delivers to the encoder a bit stream organized in blocks of 240 information bits every 40 ms (e.g. RLP frames), the bits {d(0),d(1),...,d(59),d(60),...,d(60+59), d(2*60),...,d(2*60+59), d(3*60),...,d(3*60+59)} shall be treated as four blocks of 60 bits each as described in the remainder of this clause. To ensure end-to-end synchronization of the 240 bit blocks, the resulting block after coding of the first 120 bits {d(0),d(1),...,d(60+59)} shall be transmitted in one of the transmission blocks B0, B2, B4 of the channel mapping defined in 3GPP TS 45.002.

### 3.4.2	Block code

Sixteen bits equal to 0 are added to the 60 information bits, the result being a block of 76 bits, {u(0),u(1),...,u(75)}, with:

u(19k+p) = d(15k+p)	for k = 0,1,2,3 and p = 0,1,...,14;

u(19k+p) = 0				for k = 0,1,2,3 and p = 15,16,17,18.

Two such blocks forming a block of 152 bits {u'(0),u'(1),...,u'(151)} are dealt with together in the rest of the coding process:

u'(k)		= u1(k),	k = 0,1,...,75 (u1 = 1st block)

u'(k+76)	= u2(k),	k = 0,1,...,75 (u2 = 2nd block)

### 3.4.3	Convolutional encoder

This block of 152 bits is encoded with the convolutional code of rate 1/3 defined by the following polynomials:

G1 = 1 + D + D3 + D4

G2 = 1 + D2 + D4

G3 = 1 + D + D2 + D3 + D4

The result is a block of 3 * 152 = 456 coded bits, {c(0),c(1),...,c(455)}:

c(3k)		= u'(k) + u'(k-1) + u'(k-3) + u'(k-4)

c(3k+1)	= u'(k) + u'(k-2) + u'(k-4)

c(3k+2)	= u'(k) + u'(k-1) + u'(k-2) + u'(k-3) + u'(k-4)	for	k = 0,1,...,151;

u'(k) = 0 for k < 0

### 3.4.4	Interleaving

The interleaving is done as specified for the TCH/F9.6 in subclause 3.3.4.

### 3.4.5	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4. On bitstealing for signalling purposes by a FACCH, see subclause 4.2.5.

## 3.5	Data channel at half rate, 6.0 kbit/s radio interface rate (4.8 kbit/s services (TCH/H4.8))

The definition of a 6.0 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.5.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 60 information bits (data frames) every 10 ms. Four such blocks are dealt with together in the coding process, {d(0),d(1),...,d(239)}.

For non-transparent services those four blocks shall align with one complete 240-bit RLP frame.

### 3.5.2	Block code

The block encoding is done as specified for the TCH/F9.6 in subclause 3.3.2.

### 3.5.3	Convolutional encoder

The convolutional encoding is done as specified for the TCH/F9.6 in subclause 3.3.3.

### 3.5.4	Interleaving

The interleaving is done as specified for the TCH/F9.6 in subclause 3.3.4.

### 3.5.5	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4. On bitstealing for signalling purposes by a FACCH, see subclause 4.3.5.

## 3.6	Data channel at full rate, 3.6 kbit/s radio interface rate (2.4 kbit/s and less services (TCH/F2.4))

The definition of a 3.6 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.6.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 36 information bits (data frames) every 10 ms. Two such blocks are dealt with together in the coding process, {d(0),d(1),...,d(71)}.

### 3.6.2	Block code

This block of 72 information bits is not encoded, but only increased with four tail bits equal to 0 at the end of the block.

u(k) = d(k),	k = 0,1,...,71

u(k) = 0    ,	k = 72,73,74,75 (tail bits);

### 3.6.3	Convolutional encoder

This block of 76 bits {u(0),u(1),...,u(75)} is encoded with the convolutional code of rate 1/6 defined by the following polynomials:

G1 = 1 + D + D3 +D4

G2 = 1 + D2 + D4

G3 = 1 + D + D2 + D3 + D4

G1 = 1 + D + D3 +  D4

G2 = 1 + D2 + D4

G3 = 1 + D + D2 + D3 + D4

The result is a block of 456 coded bits:

{c(0), c(1),...,c(455)}, defined by

c(6k)		= c(6k+3) = u(k) + u(k-1) + u(k-3) + u(k-4)

c(6k+1)	= c(6k+4) = u(k) + u(k-2) + u(k-4)

c(6k+2)	= c(6k+5) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-4),	for		k	= 0,1,...,75;

u(k)	= 0 for k < 0

### 3.6.4	Interleaving

The interleaving is done as specified for the TCH/FS in subclause 3.1.3.

### 3.6.5	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4.

## 3.7	Data channel at half rate, 3.6 kbit/s radio interface rate (2.4 kbit/s and less services (TCH/H2.4))

The definition of a 3.6 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.7.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 36 information bits (data frames) every 10 ms. Two such blocks are dealt with together in the coding process, {d(0),d(1),...,d(71)}.

### 3.7.2	Block code

The block of 72 information bits is not encoded, but only increased with 4 tail bits equal to 0, at the end of the block.

Two such blocks forming a block of 152 bits {u(0),u(1),...,u(151)} are dealt with together in the rest of the coding process.

u(k)		= d1(k),		k = 0,1,...,75 (d1 = 1st information block)

u(k+76)	= d2(k),		k = 0,1,...,75 (d2 = 2nd information block)

u(k)		= 0,			k = 72,73,74,75,148,149,150,151 (tail bits)

### 3.7.3	Convolutional encoder

The convolutional encoding is done as specified for the TCH/F4.8 in subclause 3.4.3.

### 3.7.4	Interleaving

The interleaving is done as specified for the TCH/F9.6 in subclause 3.3.4.

### 3.7.5	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4. On bit stealing for signalling purposes by a FACCH, see subclause 4.3.5.

## 3.8	Data channel at full rate, 14.5 kbit/s radio interface rate (14.4 kbit/s services (TCH/F14.4))

The definition of a 14.5 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.8.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 290 information bits (data frames) every 20 ms.

### 3.8.2	Block code

The block of 290 information bits is not encoded, but only increased with 4 tail bits equal to 0 at the end of the block.

u(k) = d(k)		for k = 0,1,...,289

u(k) = 0		for k = 290,291,292,293 (tail bits)

### 3.8.3	Convolutional encoder

This block of 294 bits {u(0),...,u(293)} is encoded with the ½ rate convolutional code defined by the following polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3+ D4

resulting in 588 coded bits {C(0), C(1),..., C(587)} with

C(2k) = u(k) + u(k-3) + u(k-4)

C(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)	for k = 0,1,...,293 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following 132 coded bits:

{C(18*j+1), C(18*j+6), C(18*j+11), C(18*j+15) for j = 0,1,...,31} and the bits C(577), C(582), C(584) and C(587) are not transmitted.

The result is a block of 456 coded bits, {c(0),c(1),..., c(455)}

### 3.8.4	Interleaving

The interleaving is done as specified for the TCH/F9.6 in section 3.3.4.

### 3.8.5	Mapping on a Burst

The mapping is done as specified for TCH/FS in section 3.1.4. On bitstealing by a FACCH, see section 4.2.5.

## 3.9	Adaptive multi rate speech channel at full rate (TCH/AFS)

This section describes the coding for the different frame formats used for TCH/AFS. The formats used are (in the order they are described):

SID_UPDATE	Used to convey comfort noise parameters during DTX

SID_FIRST		Marker to define end of speech, start of DTX

ONSET			Used to signal the Codec mode for the first speech frame after DTX

SPEECH			Speech frames

RATSCCH		Frames used to convey RATSCCH messages

In this chapter, sub chapters 3.9.1 to 3.9.5 describe the channel coding for the different formats listed above.

Common to all the formats is that in-band information is conveyed, the coding for the in-band channel is described in the table below.

| Identifier (defined in 3GPP TS 45.009) | Received in-band data id(1), id(0) | Encoded in-band data for SID and RATSCCH frames ic(15),.., ic(0) | Encoded in-band data for speech frames ic(7),.., ic(0) |
| --- | --- | --- | --- |
| CODEC_MODE_1 | 00 | 0101001100001111 | 00000000 |
| CODEC_MODE_2 | 01 | 0011111010111000 | 10111010 |
| CODEC_MODE_3 | 10 | 1000100001100011 | 01011101 |
| CODEC_MODE_4 | 11 | 1110010111010100 | 11100111 |

### 3.9.1	SID_UPDATE

The speech encoder delivers 35 bits of comfort noise parameters. Also delivered is two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands or Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 CN bits which are then coded by a rate ¼ RSC coder to 212 bits. Finally a 212 bit identification field is added thereby giving a total size of 456 bits. These 456 bits are then block interleaved in the same way as SACCH frames.

#### 3.9.1.1	Coding of in-band data

The two in-band data fields, id0(0,1) and id1(0,1), are encoded, giving ic0(0..15) and ic1(0..15).

The ic0 and ic1 data is moved to the coded data c as:

c(k) = ic0(k)				for k = 0,1,2,3

c(k) = ic1(k-4)			for k = 4, 5, 6, 7

c(k) = ic0(k-4)			for k = 8, 9, 10, 11

c(k) = ic1(k-8)			for k = 12, 13, 14, 15

c(k) = ic0(k-8)			for k = 16, 17, 18, 19

c(k) = ic1(k-12)			for k = 20, 21, 22, 23

c(k) = ic0(k-12)			for k = 24, 25, 26, 27

c(k) = ic1(k-16)			for k = 28, 29, 30, 31

#### 3.9.1.2	Parity and convolutional encoding for the comfort noise parameters

a)	Parity bits:

A 14-bit CRC is used for error-detection. These 14 parity bits are generated by the cyclic generator polynomial: g(D) =  D14 + D13 + D5 + D3 + D2 +1 from the 35 comfort noise parameter bits. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(48) + d(1)D(47) +... + d(34)D(14) + p(0)D(13) +…+ p(12)D+ p(13)

where p(0), p(1) … p(13) are the parity bits, when divided by g(D), yields a remainder equal to 1+ D + D2 + D3 + D4 + D5 + D6 + D7 + D8 + D9 + D10 + D11 + D12+ D13

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, 34

u(k) = p(k-35)			for k = 35, 36, …, 48

b)	Convolutional encoder

The comfort noise parameters with parity bits (u(0..48)) are encoded with the ¼ rate

convolutional code defined by the polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

G3/G3 = 1

resulting in 212 coded bits, {C(0)… C(211)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(4k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= u(k)

C(4k+3)	= u(k)					for k = 0, 1, ..., 48; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)

C(4k+3)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)				for k = 49, 50, ..., 52

This block of data is moved to the coded data (c) as:

c(8*k+32)	= C(4*k)

c(8*k+33)	= C(4*k+1)

c(8*k+34)	= C(4*k+2)

c(8*k+35)	= C(4*k+3)				for k = 0, 1, ..., 52

#### 3.9.1.3	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 9-bit sequence: { 0, 1, 0, 0, 1, 1, 1, 1, 0 } 24 times and then discarding the last 4 bits. This block of data is moved to the coded data (c) as:

c(8*k+36)	= IM(4*k)

c(8*k+37)	= IM(4*k+1)

c(8*k+38)	= IM(4*k+2)

c(8*k+39)	= IM(4*k+3)				for k = 0, 1, ..., 52

#### 3.9.1.4	Interleaving

The interleaving is done as specified for the SACCH in subclause 4.1.4.

#### 3.9.1.5	Mapping on a Burst

The interleaving is done as specified for the SACCH in subclause 4.1.5 with the exception that hl(B) and hu(B) is set to "0".

### 3.9.2	SID_FIRST

This frame type contains no source data from the speech coder, what is transmitted is the in-band channel (signalling Mode Indication or Mode Command/Mode Request depending on the current frame number) and an identification marker.

#### 3.9.2.1	Coding of in-band data

The in-band data, id(0,1), is encoded to ic(0..15) which is moved to the coded data c as:

c(k) = ic(k)			for k = 0,1,2,3

c(k) = ic(k-4)			for k = 8, 9, 10, 11

c(k) = ic(k-8)			for k = 16, 17, 18, 19

c(k) = ic(k-12)		for k = 24, 25, 26, 27

#### 3.9.2.2	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 9-bit sequence: { 0, 1, 0, 0, 1, 1, 1, 1, 0 } 24 times and then discarding the last 4 bits. This block of data is moved to the coded data (c) as:

c(8*k+32)	= IM(4*k)

c(8*k+33)	= IM(4*k+1)

c(8*k+34)	= IM(4*k+2)

c(8*k+35)	= IM(4*k+3)			for k = 0, 1, ..., 52

#### 3.9.2.3	Interleaving

The interleaving is done as specified for the TCH/FS in subclause 3.1.3.

#### 3.9.2.4	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4. The last 4 bursts shall not be transmitted unless the SID_FIRST frame is immediately followed by a speech frame.

### 3.9.3	ONSET

Onset frames are used to preset the interleaver buffer after a period of no speech activity in DTX mode. This frame type contains no source data from the speech coder, what is transmitted is the in-band channel signalling the Mode Indication for the speech frame following the onset marker.

#### 3.9.3.1	Coding of in-band data

The in-band data, Mode Indication id1(0,1),  is encoded to ic1(0..15). This sequence is then repeated 14 times more, and the last 12 bits are discarded (15*16-12=228) giving the sequence ic1(0..227).

This sequence is then moved to c as:

c(8*k+4)	= ic1(4*k)

c(8*k+5)	= ic1(4*k+1)

c(8*k+6)	= ic1(4*k+2)

c(8*k+7)	= ic1(4*k+3)			for k = 0, 1, ..., 56

#### 3.9.3.2	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k),	for	k  = 4,5,6,7, 12,13,14,15,20,21,22,23 ...,455

n  = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 8) - 4

j   = 2((49k) mod 57) + ((k mod 8) div 4)

See table 1. The result of the interleaving is a distribution of the defined 228 bits of a given data block of size 456 bits, n = N, over 4 blocks using the odd numbered bits. The even numbered bits of these 4 blocks will be filled by the speech frame for which this frame is the ONSET.

#### 3.9.3.3	Mapping on a Burst

The mapping is given by the rule:

e(B,j)	= i(B,j)	and	e(B,59+j) = i(B,57+j)		for j = 0,1,...,56

and

e(B,57) = hl(B)

The bit labelled hl(B) on burst number B is a flag used for indication of control channel signalling. For each ONSET block not stolen for signalling purposes:

hl(B)  = 0	for the 4 bursts	(indicating status of odd numbered bits)

For the use of hl(B) when an ONSET is stolen for signalling purposes see subclause 4.2.5.

### 3.9.4	SPEECH

The speech coder delivers to the channel encoder a sequence of blocks of data. One block of data corresponds to one speech frame and the block length is different in each of the eight channel codec modes. Adjoining each block of data is information of the channel codec mode to use when encoding the block. Also delivered is the in-band data id(0,1) representing Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.9.4.1	Coding of the in-band data

The two input in-band bits (id(0,1)) are coded to eight coded in-band bits (ic(0..7)).

The encoded in-band bits are moved to the coded bits, c, as

c(k) = ic(k)		for k = 0, 1, ..., 7.

#### 3.9.4.2	Ordering according to subjective importance

The bits delivered by the speech encoder, {s(1),s(2),...,s(Ks)},  are rearranged according to subjective importance before channel coding. Tables 7 to 16 define the correct rearrangement for the speech codec modes 12.2 kbit/s, 10.2 kbit/s, 7.95 kbit/s, 7.40 kbit/s, 6.70 kbit/s, 5.90 kbit/s, 5.15 kbit/s and 4.75 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to 3GPP TS 26.090 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see below:

| Codec mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| TCH/AFS12.2 | 244 |
| TCH/AFS10.2 | 204 |
| TCH/AFS7.95 | 159 |
| TCH/AFS7.4 | 148 |
| TCH/AFS6.7 | 134 |
| TCH/AFS5.9 | 118 |
| TCH/AFS5.15 | 103 |
| TCH/AFS4.75 | 95 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1	d(j) := s(table(j)+1);			where table(j) is read line by line left to right

The rearranged bits are further divided into two different classes to perform unequal error protection for different bits according to subjective importance.

The protection classes are:

1a	-	Data protected with the CRC and the convolution code.
1b	-	Data protected with the convolution code.
No unprotected bits are used.

The number of class 1 (sum of class 1a and 1b), class 1a and class 1b bits for each codec mode is shown below:

| Codec Mode | Number of speech bits delivered per block | Number of class 1 bits per block | Number of class 1a bits per block | Number of class 1b bits per block |
| --- | --- | --- | --- | --- |
| TCH/AFS12.2 | 244 | 244 | 81 | 163 |
| TCH/AFS10.2 | 204 | 204 | 65 | 139 |
| TCH/AFS7.95 | 159 | 159 | 75 | 84 |
| TCH/AFS7.4 | 148 | 148 | 61 | 87 |
| TCH/AFS6.7 | 134 | 134 | 55 | 79 |
| TCH/AFS5.9 | 118 | 118 | 55 | 63 |
| TCH/AFS5.15 | 103 | 103 | 49 | 54 |
| TCH/AFS4.75 | 95 | 95 | 39 | 56 |

#### 3.9.4.3	Parity for speech frames

The basic parameters for each codec mode for the first encoding step are shown below:

| Codec mode | Speech encoded bits (Kd) | CRC protected bits (Kd1a) | Number of bits after first encoding step (Ku = Kd + 6) |
| --- | --- | --- | --- |
| TCH/AFS12.2 | 244 | 81 | 250 |
| TCH/AFS10.2 | 204 | 65 | 210 |
| TCH/AFS7.95 | 159 | 75 | 165 |
| TCH/AFS7.4 | 148 | 61 | 154 |
| TCH/AFS6.7 | 134 | 55 | 140 |
| TCH/AFS5.9 | 118 | 55 | 124 |
| TCH/AFS5.15 | 103 | 49 | 109 |
| TCH/AFS4.75 | 95 | 39 | 101 |

A 6-bit CRC is used for error-detection. These 6 parity bits are generated by the cyclic generator polynomial: g(D) = D6 + D5 + D3 + D2 + D1 + 1 from the first Kd1a bits of class 1, where Kd1a refers to number of bits in protection class 1a as shown above for each codec mode. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(Kd1a+5) + d(1)D(Kd1a+4) +... + d(Kd1a-1)D(6) + p(0)D(5) +…+ p(4)D+ p(5)

where p(0), p(1) … p(5) are the parity bits, when divided by g(D), yields a remainder equal to:

1+ D + D2 + D3 + D4 + D5.

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, Kd1a-1

u(k) = p(k-Kd1a)			for k = Kd1a, Kd1a+1, …, Kd1a+5

u(k) = d(k-6)				for k = Kd1a+6, Kd1a+7, …, Ku-1

Thus, after the first encoding step u(k) will be defined by the following contents for each codec mode:

TCH/AFS12.2:

u(k) = d(k)				for k = 0, 1, …, 80

u(k) = p(k-81)			for k = 81, 82, …, 86

u(k) = d(k-6)				for k = 87, 88, …, 249

TCH/AFS10.2:

u(k) = d(k)				for k = 0, 1, ..., 64

u(k) = p(k-65)			for k = 65, 66, ..., 70

u(k) = d(k-6)				for k = 71, 72, ..., 209

TCH/AFS7.95:

u(k) = d(k)				for k = 0, 1, …, 74

u(k) = p(k-75)			for k = 75, 76, …, 80

u(k) = d(k-6)				for k = 81, 82, …, 164

TCH/AFS7.4:

u(k) = d(k)				for k = 0, 1, …, 60

u(k) = p(k-61)			for k = 61, 62, …, 66

u(k) = d(k-6)				for k = 67, 68, …, 153

TCH/AFS6.7:

u(k) = d(k)				for k = 0, 1, …, 54

u(k) = p(k-55)			for k = 55, 56, …, 60

u(k) = d(k-6)				for k = 61, 62, …, 139

TCH/AFS5.9:

u(k) = d(k)				for k = 0, 1, …, 54

u(k) = p(k-55)			for k = 55, 56, …, 60

u(k) = d(k-6)				for k = 61, 62, …, 123

TCH/AFS5.15:

u(k) = d(k)				for k = 0, 1, …, 48

u(k) = p(k-49)			for k = 49, 50, …, 54

u(k) = d(k-6)				for k = 55, 56, …, 108

TCH/AFS4.75:

u(k) = d(k)				for k = 0, 1, ..., 38

u(k) = p(k-39)			for k = 39, 40, ..., 44

u(k) = d(k-6)				for k = 45, 46, ..., 100

#### 3.9.4.4	Convolutional encoder

The bits from the first encoding step (u(k)) are encoded with the recursive systematic convolutional codes as summarised below. The number of output bits after puncturing is 448 for all codec modes.

| Codec mode | Rate | Number of input bits to conv. coder | Number of output bits from conv. coder | Number of punctured bits |
| --- | --- | --- | --- | --- |
| TCH/AFS12.2 | ½ | 250 | 508 | 60 |
| TCH/AFS10.2 | 1/3 | 210 | 642 | 194 |
| TCH/AFS7.95 | 1/3 | 165 | 513 | 65 |
| TCH/AFS7.4 | 1/3 | 154 | 474 | 26 |
| TCH/AFS6.7 | ¼ | 140 | 576 | 128 |
| TCH/AFS5.9 | ¼ | 124 | 520 | 72 |
| TCH/AFS5.15 | 1/5 | 109 | 565 | 117 |
| TCH/AFS4.75 | 1/5 | 101 | 535 | 87 |

Below the coding for each codec mode is specified in detail.

TCH/AFS12.2:

The block of 250 bits {u(0)… u(249)} is encoded with the ½ rate convolutional code defined by	the following polynomials:

G0/G0 = 1

G1/G0 = 1 + D + D3+ D4 / 1 + D3 + D4

resulting in 508 coded bits, {C(0)… C(507)} defined by:

r(k)		= u(k) + r(k-3) + r(k-4)

C(2k)		= u(k)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 0, 1, ..., 249; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(2k)		= r(k-3) + r(k-4)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 250, 251, ..., 253

The code is punctured in such a way that the following 60 coded bits:

C(321), C(325), C(329), C(333), C(337), C(341), C(345), C(349), C(353), C(357), C(361), C(363), C(365), C(369), C(373), C(377), C(379), C(381), C(385), C(389), C(393), C(395), C(397), C(401), C(405), C(409), C(411), C(413), C(417), C(421), C(425), C(427), C(429), C(433), C(437), C(441), C(443), C(445), C(449), C(453), C(457), C(459), C(461), C(465), C(469), C(473), C(475), C(477), C(481), C(485), C(489), C(491), C(493), C(495), C(497), C(499), C(501), C(503), C(505) and C(507)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which	are appended to the in-band bits in c as

c(k+8) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS10.2:

The block of 210 bits {u(0)... u(209)} is encoded with the 1/3 rate convolutional code defined by	the following polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

resulting in 642 coded bits, {C(0)... C(641)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(3k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(3k+1)	= r(k)+r(k-2)+r(k-4)

C(3k+2)	= u(k)					for k = 0, 1, ..., 209

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(3k+1)	= r(k)+r(k-2)+r(k-4)

C(3k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)	for k = 210, 211, ..., 213

The code is punctured in such a way that the following 194 bits:

C(1), C(4), C(7), C(10), C(16), C(19), C(22), C(28), C(31), C(34), C(40), C(43), C(46), C(52), C(55), C(58), C(64), C(67), C(70), C(76), C(79), C(82), C(88), C(91), C(94), C(100), C(103), C(106), C(112), C(115), C(118), C(124), C(127), C(130), C(136), C(139), C(142), C(148), C(151), C(154), C(160), C(163), C(166), C(172), C(175), C(178), C(184), C(187), C(190), C(196), C(199), C(202), C(208), C(211), C(214), C(220), C(223), C(226), C(232), C(235), C(238), C(244), C(247), C(250), C(256), C(259), C(262), C(268), C(271), C(274), C(280), C(283), C(286), C(292), C(295), C(298), C(304), C(307), C(310), C(316), C(319), C(322), C(325), C(328), C(331), C(334), C(337), C(340), C(343), C(346), C(349), C(352), C(355), C(358), C(361), C(364), C(367), C(370), C(373), C(376), C(379), C(382), C(385), C(388), C(391), C(394), C(397), C(400), C(403), C(406), C(409), C(412), C(415), C(418), C(421), C(424), C(427), C(430), C(433), C(436), C(439), C(442), C(445), C(448), C(451), C(454), C(457), C(460), C(463), C(466), C(469), C(472), C(475), C(478), C(481), C(484), C(487), C(490), C(493), C(496), C(499), C(502), C(505), C(508), C(511), C(514), C(517), C(520), C(523), C(526), C(529), C(532), C(535), C(538), C(541), C(544), C(547), C(550), C(553), C(556), C(559), C(562), C(565), C(568), C(571), C(574), C(577), C(580), C(583), C(586), C(589), C(592), C(595), C(598), C(601), C(604), C(607), C(609), C(610), C(613), C(616), C(619), C(621), C(622), C(625), C(627), C(628), C(631), C(633), C(634), C(636), C(637), C(639) and C(640)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which	are appended to the in-band bits in c as:

c(k+8) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS7.95:

The block of 165 bits {u(0)… u(164)} is encoded with the 1/3 rate convolutional code defined by	the following polynomials:

G4/G4 = 1

G5/G4 = 1 + D + D4 + D6/ 1 + D2  + D3 + D5 + D6

G6/G4 = 1 + D  + D2 + D3 + D4 + D6/ 1 + D2  + D3 + D5 + D6

resulting in 513 coded bits, {C(0)… C(512)} defined by:

r(k)		= u(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k)		= u(k)

C(3k+1)	= r(k)+r(k-1)+r(k-4)+r(k-6)

C(3k+2)	= r(k)+r(k-1)+ r(k-2)+r(k-3)+r(k-4)+r(k-6)		for k = 0, 1, ..., 164; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k)+r(k-1)+r(k-4)+r(k-6)

C(3k+2)	= r(k)+r(k-1)+ r(k-2)+r(k-3)+r(k-4)+r(k-6)	for k = 165, 166, ..., 170

The code is punctured in such a way that the following 65 coded bits:

C(1), C(2), C(4), C(5), C(8), C(22), C(70), C(118), C(166), C(214), C(262), C(310), C(317), C(319), C(325), C(332), C(334), C(341), C(343), C(349), C(356), C(358), C(365), C(367), C(373), C(380), C(382), C(385), C(389), C(391), C(397), C(404), C(406), C(409), C(413), C(415), C(421), C(428), C(430), C(433), C(437), C(439), C(445), C(452), C(454), C(457), C(461), C(463), C(469), C(476), C(478), C(481), C(485), C(487), C(490), C(493), C(500), C(502), C(503), C(505), C(506), C(508), C(509), C(511) and C(512)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which	are appended to the in-band bits in c as

c(k+8) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS7.4:

The block of 154 bits {u(0)... u(153)} is encoded with the 1/3 rate convolutional code defined by	the following polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

resulting in 474 coded bits, {C(0)... C(473)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(3k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(3k+1)	= r(k)+r(k-2)+r(k-4)

C(3k+2)	= u(k)					for k = 0, 1, ..., 153

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(3k+1)	= r(k)+r(k-2)+r(k-4)

C(3k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)	for k = 154, 155, ..., 157

The code is punctured in such a way that the following 26 bits:

C(0), C(355), C(361), C(367), C(373), C(379), C(385), C(391), C(397), C(403), C(409), C(415), C(421), C(427), C(433), C(439), C(445), C(451), C(457), C(460), C(463), C(466), C(468), C(469), C(471) and C(472)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which	are appended to the in-band bits in c as:

c(k+8) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS6.7:

The block of 140 bits {u(0)… u(139)} is encoded with the ¼ rate convolutional code defined by	the following polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

G3/G3 = 1

resulting in 576 coded bits, {C(0)… C(575)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(4k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= u(k)

C(4k+3)	= u(k)					for k = 0, 1, ..., 139; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)

C(4k+3)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)				for k = 140, 141, ..., 143

The code is punctured in such a way that the following 128 coded bits:

C(1), C(3), C(7), C(11), C(15), C(27), C(39), C(55), C(67), C(79), C(95), C(107), C(119), C(135), C(147), C(159), C(175), C(187), C(199), C(215), C(227), C(239), C(255), C(267), C(279), C(287), C(291), C(295), C(299), C(303), C(307), C(311), C(315), C(319), C(323), C(327), C(331), C(335), C(339), C(343), C(347), C(351), C(355), C(359), C(363), C(367), C(369), C(371), C(375), C(377), C(379), C(383), C(385), C(387), C(391), C(393), C(395), C(399), C(401), C(403), C(407), C(409), C(411), C(415), C(417), C(419), C(423), C(425), C(427), C(431), C(433), C(435), C(439), C(441), C(443), C(447), C(449), C(451), C(455), C(457), C(459), C(463), C(465), C(467), C(471), C(473), C(475), C(479), C(481), C(483), C(487), C(489), C(491), C(495), C(497), C(499), C(503), C(505), C(507), C(511), C(513), C(515), C(519), C(521), C(523), C(527), C(529), C(531), C(535), C(537), C(539), C(543), C(545), C(547), C(549), C(551), C(553), C(555), C(557), C(559), C(561), C(563), C(565), C(567), C(569), C(571), C(573) and C(575)

are not transmitted. The result is a block of 448 coded bits, P(0)...P(447) which are appended to	the in-band bits in c as

c(k+8) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS5.9:

The block of 124 bits {u(0)… u(123)} is encoded with the ¼ rate convolutional code defined by	the following polynomials:

G4/G6 =  1 + D2  + D3 + D5 + D6  / 1 + D + D2  + D3 + D4 + D6

G5/G6 = 1 + D + D4 + D6  / 1 + D + D2  + D3 + D4 + D6

G6/G6 = 1

G6/G6 = 1

resulting in 520 coded bits, {C(0)… C(519)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= u(k)

C(4k+3)	= u(k)

for k = 0, 1, ..., 123; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k)+r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k)+r(k-1)+r(k-4)+r(k-6)

C(4k+2)	= r(k-1)+r(k-2)+ r(k-3)+r(k-4)+r(k-6)

C(4k+3)	= r(k-1)+r(k-2)+ r(k-3)+r(k-4)+r(k-6)

for k = 124, 125, ..., 129

The code is punctured in such a way that the following 72 coded bits:

C(0), C(1), C(3), C(5), C(7), C(11), C(15), C(31), C(47), C(63), C(79), C(95), C(111), C(127), C(143), C(159), C(175), C(191), C(207), C(223), C(239), C(255), C(271), C(287), C(303), C(319), C(327), C(331), C(335), C(343), C(347), C(351), C(359), C(363), C(367), C(375), C(379), C(383), C(391), C(395), C(399), C(407), C(411), C(415), C(423), C(427), C(431), C(439), C(443), C(447), C(455), C(459), C(463), C(467), C(471), C(475), C(479), C(483), C(487), C(491), C(495), C(499), C(503), C(507), C(509), C(511), C(512), C(513), C(515), C(516), C(517)  and C(519)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which are appended to the in-band bits in c as

c(8+k) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS5.15:

The block of 109 bits {u(0)… u(108)} is encoded with the 1/5 rate convolutional code defined by	the following polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

G3/G3 = 1

resulting in 565 coded bits, {C(0)… C(564)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(5k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(5k+1)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(5k+2)	= r(k)+r(k-2)+r(k-4)

C(5k+3)	= u(k)

C(5k+4)	= u(k)

for k = 0, 1, ..., 108; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(5k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(5k+1)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(5k+2)	= r(k)+r(k-2)+r(k-4)

C(5k+3)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)

C(5k+4)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)		for k = 109, 110, ..., 112

The code is punctured in such a way that the following 117 coded bits:

C(0), C(C(4), C(5), C(9), C(10), C(14), C(15), C(20), C(25), C(30), C(35), C(40), C(50), C(60), C(70), C(80), C(90), C(100), C(110), C(120), C(130), C(140), C(150), C(160), C(170), C(180),C(190), C(200), C(210), C(220), C(230), C(240), C(250), C(260), C(270), C(280),C(290), C(300), C(310), C(315), C(320), C(325), C(330), C(334), C(335), C(340), C(344), C(345), C(350), C(354), C(355), C(360), C(364), C(365), C(370), C(374), C(375), C(380), C(384), C(385), C(390), C(394), C(395), C(400), C(404), C(405), C(410), C(414), C(415), C(420), C(424), C(425), C(430), C(434), C(435), C(440), C(444), C(445), C(450), C(454), C(455), C(460), C(464), C(465), C(470), C(474), C(475), C(480), C(484), C(485), C(490), C(494), C(495), C(500), C(504), C(505), C(510), C(514), C(515), C(520), C(524), C(525), C(529), C(530), C(534), C(535), C(539), C(540), C(544), C(545), C(549), C(550), C(554), C(555), C(559), C(560) and C(564)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which	are appended to the in-band bits in c as

c(8+k) = P(k)		for k = 0, 1, ..., 447.

TCH/AFS4.75:

The block of 101 bits {u(0)... u(100)} is encoded with the 1/5 rate convolutional code defined by	the following polynomials:

G4/G6 =  1 + D2  + D3 + D5 + D6  / 1 + D + D2  + D3 + D4 + D6

G4/G6 =  1 + D2  + D3 + D5 + D6  / 1 + D + D2  + D3 + D4 + D6

G5/G6 = 1 + D + D4 + D6  / 1 + D + D2  + D3 + D4 + D6

G6/G6 = 1

G6/G6 = 1

resulting in 535 coded bits, {C(0)… C(534)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= u(k)

C(5k+4)	= u(k)

for k = 0, 1, ..., 100; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(5k)		= r(k)+r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k)+r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k)+r(k-1)+r(k-4)+r(k-6)

C(5k+3)	= r(k-1)+r(k-2)+ r(k-3)+r(k-4)+r(k-6)

C(5k+4)	= r(k-1)+r(k-2)+ r(k-3)+r(k-4)+r(k-6)

for k = 101, 102, ..., 106

The code is punctured in such a way that the following 87 coded bits:

C(0), C(1), C(2), C(4), C(5), C(7), C(9), C(15), C(25), C(35), C(45), C(55), C(65), C(75), C(85), C(95), C(105), C(115), C(125), C(135), C(145), C(155), C(165), C(175), C(185), C(195), C(205), C(215), C(225), C(235), C(245), C(255), C(265), C(275), C(285), C(295), C(305), C(315), C(325), C(335), C(345), C(355), C(365), C(375), C(385), C(395), C(400), C(405), C(410), C(415), C(420), C(425), C(430), C(435), C(440), C(445), C(450), C(455), C(459), C(460), C(465), C(470), C(475), C(479), C(480), C(485), C(490), C(495), C(499), C(500), C(505), C(509), C(510), C(515), C(517), C(519), C(520), C(522), C(524), C(525), C(526), C(527), C(529), C(530), C(531), C(532) and C(534)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which are appended to the inband bits in c as

c(8+k) = P(k)		for k = 0, 1, ..., 447.

#### 3.9.4.5	Interleaving

The interleaving is done as specified for the TCH/FS in subclause 3.1.3.

#### 3.9.4.6	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4.

### 3.9.5	RATSCCH

The RATSCCH message consists of 35 bits. Also delivered are two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands or Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 RATSCCH bits which are then coded by a rate ¼ RSC coder to 212 bits. Finally a 212 bit identification field is added thereby giving a total size of 456 bits. These 456 bits are then block interleaved in the same way as a normal speech frame.

#### 3.9.5.1	Coding of in-band data

The two in-band data fields, id0(0,1) and id1(0,1), are encoded, giving ic0(0..15) and ic1(0..15).

These bits are moved to the coded bits c as:

c(k) = ic1(k)	for k = 0,1, ..., 15

c(k+228) = ic0(k) for k = 0, 1, …, 15

#### 3.9.5.2	Parity and convolutional encoding for the RATSCCH message

a)	Parity bits:

A 14-bit CRC is used for error-detection. These 14 parity bits are generated by the cyclic generator polynomial: g(D) =  D14 + D13 + D5 + D3 + D2 +1 from the 35 comfort noise parameter bits. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(48) + d(1)D(47) +... + d(34)D(14) + p(0)D(13) +…+ p(12)D+ p(13)

where p(0), p(1) … p(13) are the parity bits, when divided by g(D), yields a remainder equal to 1+ D + D2 + D3 + D4 + D5 + D6 + D7 + D8 + D9 + D10 + D11 + D12+ D13

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, 34

u(k) = p(k-35)			for k = 35, 36, …, 48

b)	Convolutional encoder

The comfort noise parameters with parity and tail bits (u(0..48)) are encoded with the ¼ rate

convolutional code defined by the polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

G3/G3 = 1

resulting in 212 coded bits, {C(0)… C(211)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(4k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= u(k)

C(4k+3)	= u(k)					for k = 0, 1, ..., 48; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)

C(4k+3)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)				for k = 49, 50, ..., 52

This block of data is moved to the coded data (c) as:

c(k+244) = C(k)			for k = 0, 1, ..., 211

#### 3.9.5.3	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 11-bit sequence: { 1, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1 } 20 times and then discarding the last 8 bits. This block of data is moved to the coded data (c) as:

c(k+16) = IM(k)			for k = 0, 1, .., 211

#### 3.9.5.4	Interleaving

The interleaving is done as specified for the TCH/FS in subclause 3.1.3.

#### 3.9.5.5	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4.

## 3.10	Adaptive multi rate speech channel at half rate (TCH/AHS)

This section describes the coding for the different frame formats used for TCH/AHS. The formats used are (in the order they are described):

SID_UPDATE			Used to convey comfort noise parameters during DTX

SID_UPDATE_INH		Used to inhibit the second part of a SID_UPDATE frame if there is a speech onset

SID_FIRST_P1			First part of marker to define end of speech, start of DTX

SID_FIRST_P2			Second part of marker to define end of speech, start of DTX

SID_FIRST_INH		Used to inhibit the second part of a SID_FIRST_P1 frame if there is a speech onset

ONSET					Used to signal the Codec mode for the first speech frame after DTX

SPEECH					Speech frames

RATSCCH_MARKER	Marker to identify RATSCCH frames

RATSCCH_DATA		Frame that conveys the actual RATSCCH message

In this chapter, sub chapters 3.10.1 to 3.10.9 describe the channel coding for the different formats listed above.

Common to all the formats is that in-band information is conveyed, the coding for the in-band channel is described in the table below:

| Identifier (defined in 3GPP TS 45.009) | Received in-band data id(1), id(0) | Encoded in-band data for SID and RATSCCH frames ic(15),.., ic(0) | Encoded in-band data for speech framesic(3),.., ic(0) |
| --- | --- | --- | --- |
| CODEC_MODE_1 | 00 | 0101001100001111 | 0000 |
| CODEC_MODE_2 | 01 | 0011111010111000 | 1001 |
| CODEC_MODE_3 | 10 | 1000100001100011 | 0111 |
| CODEC_MODE_4 | 11 | 1110010111010100 | 1110 |

### 3.10.1	SID_UPDATE

The speech encoder delivers 35 bits of comfort noise parameters. Also delivered is two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands/Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 CN bits which are then coded by a rate ¼ RSC coder to 212 bits. Finally a 212 bit identification field is added thereby giving a total size of 456 bits. These 456 bits are block interleaved over 4 bursts.

#### 3.10.1.1	Coding of in-band data

The two in-band data fields, id0(0,1) and id1(0,1), are encoded, giving ic0(0..15) and ic1(0..15).

The ic0 and ic1 data is moved to the coded data c as:

c(k) = ic1(k)		for k = 0,1, .., 15

c(k) = ic0(k-228)	for k = 228, 229, .., 243

#### 3.10.1.2	Parity and convolutional encoding for the comfort noise parameters

a)	Parity bits:

A 14-bit CRC is used for error-detection. These 14 parity bits are generated by the cyclic generator polynomial: g(D) =  D14 + D13 + D5 + D3 + D2 +1 from the 35 comfort noise parameter bits. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(48) + d(1)D(47) +... + d(34)D(14) + p(0)D(13) +…+ p(12)D+ p(13)

where p(0), p(1) … p(13) are the parity bits, when divided by g(D), yields a remainder equal to 
1+ D + D2 + D3 + D4 + D5 + D6 + D7 + D8 + D9 + D10 + D11 + D12+ D13

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, 34

u(k) = p(k-35)				for k = 35, 36, …, 48

b)	Convolutional encoder

The comfort noise parameters with parity bits (u(0..48)) are encoded with the ¼ rate

convolutional code defined by the polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

G3/G3 = 1

resulting in 212 coded bits, {C(0)… C(211)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(4k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= u(k)

C(4k+3)	= u(k)					for k = 0, 1, ..., 48; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)

C(4k+3)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)				for k = 49, 50, ..., 52

This block of data is moved to the coded data (c) as:

c(k+244)	= C(k)				for k = 0, 1, ..., 211

#### 3.10.1.3	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 9-bit sequence: 
{ 1, 0, 1, 1, 0, 0, 0, 0, 1 } 24 times and then discarding the last 4 bits. This block of data is moved to the coded data (c) as:

c(k+16)	= IM(k)				for k = 0, 1, ..., 211

#### 3.10.1.4	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)	for	k = 0,1,...,227

n = 0,1,...,N,N+1,...

B = B0 + 2n + b

i(B,j) = c(n,k+228)	for	k = 0,1,...,227

n = 0,1,...,N,N+1,...

B = B0 + 2n + ((b + 2) mod 4)

The values of b and j in dependence of k are given by table 4.

The result of the interleaving is a distribution of the 456 bits of a given data block, n = N, over 4 blocks using all bits for each block. The block of coded data is interleaved "block rectangular" where a new data block starts every 4th block and is distributed over 4 blocks.

#### 3.10.1.5	Mapping on a Burst

The mapping is given by the rule:

e(B,j) = i(B,j) and e(B,59+j) = i(B,57+j) for j = 0,1,...,56

and

e(B,57) = hl(B)  and  e(B,58) = hu(B)

The two bits, labelled hl(B) and hu(B) on burst number B are flags used for indication of control channel signalling. For each block not stolen for FACCH signalling purposes:

hu(B)	= 0		for all 4  bursts

hl(B)	= 0	for all 4 bursts

For the use of hl(B) and hu(B) when frame is stolen for signalling purposes, see subclause 4.3.5.

### 3.10.2	SID_UPDATE_INH

This special frame is used when the first 2 burst of a SID_UPDATE frame have been transmitted but the second two bursts cannot be transmitted due to a speech frame. The general coding is as: the in-band data (Note that this must be the same Mode Indication bits as id1(0,1) for the SID_UPDATE frame that is being inhibited) is encoded, a marker that is the opposite of the SID_UPDATE marker is appended and the data is interleaved in such a way that the odd bits of two bursts are filled.

#### 3.10.2.1	Coding of in-band data

The in-band data, Mode Indication id1(0,1), is encoded to ic1(0..15) which is moved to the coded data c as:

c(k) = ic1(k)			for k = 0,1, .., 15

#### 3.10.2.2	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 9-bit sequence: 
{ 0, 1, 0, 0, 1, 1, 1, 1, 0 } 24 times and then discarding the last 4 bits. This block of data is moved to the coded data (c) as:

c(k+16)	= IM(k)				for k = 0, 1, ..., 211

#### 3.10.2.3	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)			for			k = 1,3,5,7,...,227

n = 0,1,...,N,N+1,...

B = B0 + 2n + b - 2

The values of b and j in dependence of k are given by table 4.

The result of the interleaving is a distribution of 114 of the reordered 228 bits of a given data block, n = N, over 2 blocks using the odd numbered bits. The even numbered bits of these 2 blocks will be filled by the speech frame that following immediately after this frame.

#### 3.10.2.4	Mapping on a Burst

The mapping is given by the rule:

e(B,j) = i(B,j) and e(B,59+j) = i(B,57+j) for j = 0,1,...,56

and

e(B,57) = hl(B)

The bit labelled hl(B) on burst number B is a flag used for indication of control channel signalling. For each SID_FIRST_INH block not stolen for signalling purposes:

hl(B) = 0	for the 2 bursts (indicating status of the odd numbered bits)

For the use of  hl(B) when a SID_UPDATE_INH  is stolen for signalling purposes, see subclause 4.3.5.

### 3.10.3	SID_FIRST_P1

This frame type contains no source data from the speech coder. What is generated is the in-band channel and an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.10.3.1	Coding of in-band data

The in-band data, id(0,1), is encoded to ic(0..15) which is moved to the coded data c as:

c(k)		= ic (k)				for k = 0,1, .., 15

#### 3.10.3.2	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 9-bit sequence: 
{ 0, 1, 0, 0, 1, 1, 1, 1, 0 } 24 times and then discarding the last 4 bits. This block of data is moved to the coded data (c) as:

c(k+16)	= IM(k)				for k = 0, 1, ..., 211

#### 3.10.3.3	Interleaving

The interleaving is done as specified for the TCH/HS in subclause 3.2.3.

#### 3.10.3.4	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4.

### 3.10.4	SID_FIRST_P2

This frame type contains no source data from the speech coder. What is generated is the in-band channel and, derived from that, an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.10.4.1	Coding of in-band data

The in-band data, id(0,1), is encoded to ic(0..15). This sequence is then repeated 7 times more, and the last 14 bits are discarded (8*16-14=114) giving the sequence ic(0..113).

This sequence is then moved to c as:

c(2*k)	= ic(k)			for k = 0, 1, ..., 113

#### 3.10.4.2	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)			for			k = 0,2,4,6,...,226

n = 0,1,...,N,N+1,...

B = B0 + 2n + b

The values of b and j in dependence of k are given by table 4.

The result of the interleaving is a distribution of 114 of the reordered 228 bits of a given data block, n = N, over 2 blocks using the even numbered bits. The odd numbered bits of these 2 blocks have already been filled by the SID_FIRST_P1 frame.

#### 3.10.4.3	Mapping on a Burst

The mapping is given by the rule:

e(B,j) = i(B,j) and e(B,59+j) = i(B,57+j) for j = 0,1,...,56

and

e(B,58) = hu(B)

The bit labelled hu(B) on burst number B is a flag used for indication of control channel signalling. For each SID_FIRST_P2 block not stolen for signalling purposes:

hu(B) = 0	for the 2 bursts (indicating status of the even numbered bits)

For the use of  hu(B) when a SID_FIRST_P2  is stolen for signalling purposes, see subclause 4.3.5.

### 3.10.5	SID_FIRST_INH

This special frame is used when the first 2 burst of a SID_FIRST_P1 frame have been transmitted but the second two bursts cannot be transmitted due to a SPEECH frame. The general coding is as: the in-band data (Note that this must be the same data as for the SID_FIRST_P1 frame that is being inhibited) is encoded, a marker that is the opposite of the SID_FIRST_P1 marker is appended and the data is interleaved in such a way that the odd bits of two bursts are filled.

#### 3.10.5.1	Coding of in-band data

The coding of the in-band data is done as specified for the SID_FIRST_P1 frame in subclause 3.10.3.1.

#### 3.10.5.2	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 9-bit sequence: 
{ 1, 0, 1, 1, 0, 0, 0, 0, 1 } 24 times and then discarding the last 4 bits. This block of data is moved to the coded data (c) as:

c(k+16)	= IM(k)				for k = 0, 1, ..., 211

#### 3.10.5.3	Interleaving

The interleaving is done as specified for the SID_UPDATE_INH in subclause 3.10.2.3.

#### 3.10.5.4	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE_INH in subclause 3.10.2.4.

### 3.10.6	ONSET

Onset frames are used to preset the interleaver buffer after a period of no speech activity in DTX mode. This frame type contains no source data from the speech coder. What is transmitted is the in-band channel signalling the Mode Indication for the speech frame following the onset marker.

#### 3.10.6.1	Coding of in-band data

The in-band data, Mode Indication id1(0,1),  will be encoded to ic1(0..15). This sequence is then repeated 7 times more, and the last 14 bits are discarded (8*16-14=114) giving the sequence ic1(0..113).

This sequence is then moved to c as:

c(2*k+1)	= ic1(k)			for k = 0, 1, ..., 113

#### 3.10.6.2	Interleaving

The interleaving is done as specified for the SID_UPDATE_INH in subclause 3.10.2.3.

#### 3.10.6.3	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE_INH in subclause 3.10.2.4.

### 3.10.7	SPEECH

The speech coder delivers to the channel encoder a sequence of blocks of data. One block of data corresponds to one speech frame and the block length is different in each of the six channel codec modes. Adjoining each block of data is information of the channel codec mode to use when encoding the block Also delivered is the in-band data id(0,1) representing Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.10.7.1	Coding of the in-band data

The two bits to be in-band encoded, id(0,1), are encoded into ic(0..3).

The encoded in-band data (4 bits) are then moved to c(k) as:

c(k) = ic(k)			for k = 0, 1, …, 3

#### 3.10.7.2	Ordering according to subjective importance

The bits delivered by the speech encoder, {s(1),s(2),...,s(Ks)},  are rearranged according to subjective importance before channel coding, Tables 9, 10, 11, 12, 13, 14 define the correct rearrangement for the speech codec modes 7.95 kbit/s, 7.40 kbit/s, 6.70 kbit/s, 5.90 kbit/s, 5.15 kbit/s and 4.75 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to 3GPP TS 26.090 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see below:

| Codec mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| TCH/AHS7.95 | 159 |
| TCH/AHS7.4 | 148 |
| TCH/AHS6.7 | 134 |
| TCH/AHS5.9 | 118 |
| TCH/AHS5.15 | 103 |
| TCH/AHS4.75 | 95 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1	d(j) := s(table(j)+1);		where table(j) is read line by line left to right

The rearranged bits are further divided into three different classes to perform unequal error protection for different bits according to subjective importance.

The protection classes are:

1a	-	Data protected with the CRC and the convolution code.
	1b	-	Data protected with the convolution code.

2	-	Data sent without protection.

The number of class 1 (sum of class 1a and 1b), class 1a, class 1b and class 2 bits for each codec mode is shown below:

| Codec mode | Number of speech bits delivered per block | Number of class 1 bits per block | Number of class 1a bits per block | Number of class 1b bits per block | Number of class 2 bits per block |
| --- | --- | --- | --- | --- | --- |
| TCH/AHS7.95 | 159 | 123 | 67 | 56 | 36 |
| TCH/AHS7.4 | 148 | 120 | 61 | 59 | 28 |
| TCH/AHS6.7 | 134 | 110 | 55 | 55 | 24 |
| TCH/AHS5.9 | 118 | 102 | 55 | 47 | 16 |
| TCH/AHS5.15 | 103 | 91 | 49 | 42 | 12 |
| TCH/AHS4.75 | 95 | 83 | 39 | 44 | 12 |

#### 3.10.7.3	Parity for speech frames

The basic parameters for each codec mode for the first encoding step are shown below:

| Mode number | Number of class 1 bits (Kd1) | CRC protected bits (Kd1a) | Number of output bits from first encoding step (Ku = Kd1 + 6) |
| --- | --- | --- | --- |
| TCH/AHS7.95 | 123 | 67 | 129 |
| TCH/AHS7.4 | 120 | 61 | 126 |
| TCH/AHS6.7 | 110 | 55 | 116 |
| TCH/AHS5.9 | 102 | 55 | 108 |
| TCH/AHS5.15 | 91 | 49 | 97 |
| TCH/AHS4.75 | 83 | 39 | 89 |

A 6-bit CRC is used for error-detection. These 6 parity bits are generated by the cyclic generator polynomial:
g(D) = D6 + D5 + D3 + D2 + D1 + 1 from the first Kd1a bits of class 1, where Kd1a refers to number of bits in protection class 1a. The value of Kd1a for each codec mode is shown above.

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:
d(0)D(Kd1a+5) + d(1)D(Kd1a+4) +... + d(Kd1a-1)D(6) + p(0)D(5) +…+ p(4)D+ p(5)

where p(0), p(1) … p(5) are the parity bits, when divided by g(D), yields a remainder equal to:

1+ D + D2 + D3 + D4 + D5.

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, Kd1a-1

u(k) = p(k-Kd1a)			for k = Kd1a, Kd1a+1, …, Kd1a+5

u(k) = d(k-6)				for k = Kd1a+6, Kd1a+7, …, Ku-1

Thus, after the first encoding step u(k) will be defined by the following contents for each codec mode:

TCH/AHS7.95:

u(k) = d(k)				for k = 0, 1, …, 66

u(k) = p(k-67)			for k = 67, 68, …, 72

u(k) = d(k-6)				for k = 73, 74, …, 128

TCH/AHS7.4:

u(k) = d(k)				for k = 0, 1, …, 60

u(k) = p(k-61)			for k = 61, 62, …, 66

u(k) = d(k-6)				for k = 67, 68, …, 125

TCH/AHS6.7:

u(k) = d(k)				for k = 0, 1, …, 54

u(k) = p(k-55)			for k = 55, 56, …, 60

u(k) = d(k-6)				for k = 61, 62, …, 115

TCH/AHS5.9:

u(k) = d(k)				for k = 0, 1, …, 54

u(k) = p(k-55)			for k = 55, 56, …, 60

u(k) = d(k-6)				for k = 61, 62, …, 107

TCH/AHS5.15:

u(k) = d(k)				for k = 0, 1, …, 48

u(k) = p(k-49)			for k = 49, 50, …, 54

u(k) = d(k-6)				for k = 55, 56, …, 96

TCH/AHS4.75:

u(k) = d(k)				for k = 0, 1, ..., 38

u(k) = p(k-39)			for k = 39, 40, ..., 44

u(k) = d(k -6)			for k = 45, 46, ..., 88

#### 3.10.7.4	Convolutional encoder

The bits from the first encoding step (u(k)) are encoded with the recursive systematic convolutional code as summarised below:

| Codec mode | Number of input bits to conv. code | Rate | Number of output bits from conv. code | Number of punctured bits |
| --- | --- | --- | --- | --- |
| TCH/AHS7.95 | 129 | ½ | 266 | 78 |
| TCH/AHS7.4 | 126 | ½ | 260 | 64 |
| TCH/AHS6.7 | 116 | ½ | 240 | 40 |
| TCH/AHS5.9 | 108 | ½ | 224 | 16 |
| TCH/AHS5.15 | 97 | 1/3 | 303 | 91 |
| TCH/AHS4.75 | 89 | 1/3 | 285 | 73 |

Below the coding for each codec mode is specified in detail.

TCH/AHS7.95:

The block of 129 bits {u(0)… u(128)} is encoded with the ½ rate convolutional code defined by	the following polynomials:

G0/G0 = 1

G1/G0 = 1 + D + D3+ D4 / 1 + D3 + D4

resulting in 266 coded bits, {C(0)… C(265)} defined by:

r(k)		= u(k) + r(k-3) + r(k-4)

C(2k)		= u(k)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 0, 1, ..., 128; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(2k)		= r(k-3) + r(k-4)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 129, 130 ..., 132

The code is punctured in such a way that the following 78 coded bits:

C(1), C(3), C(5), C(7), C(11), C(15), C(19), C(23), C(27), C(31), C(35), C(43), C(47), C(51), C(55), C(59), C(63), C(67), C(71), C(79), C(83), C(87), C(91), C(95), C(99), C(103), C(107), C(115), C(119), C(123), C(127), C(131), C(135), C(139), C(143), C(151), C(155), C(159), C(163), C(167), C(171), C(175), C(177), C(179), C(183), C(185), C(187), C(191), C(193), C(195), C(197), C(199), C(203), C(205), C(207), C(211), C(213), C(215), C(219), C(221), C(223), C(227), C(229), C(231), C(233), C(235), C(239), C(241), C(243), C(247), C(249), C(251), C(255), C(257), C(259), C(261), C(263) and C(265)

are not transmitted. The result is a block of 188 coded and punctured bits, P(0)...P(187) which	are appended to the in-band bits in c as

c(k+4) = P(k)		for k = 0, 1, ..., 187.

Finally the 36 class 2 bits are appended to c

c(192+k ) = d(123+k)	for k = 0, 1, ..., 35.

TCH/AHS7.4:

The block of 126 bits {u(0)… u(125)} is encoded with the ½ rate convolutional code defined by	the following polynomials:

G0/G0 = 1

G1/G0 = 1 + D + D3+ D4 / 1 + D3 + D4

resulting in 260 coded bits, {C(0)… C(259)} defined by:

r(k)		= u(k) + r(k-3) + r(k-4)

C(2k)		= u(k)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 0, 1, ..., 125; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(2k)		= r(k-3) + r(k-4)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 126, 127 ..., 129

The code is punctured in such a way that the following 64 coded bits:

C(1), C(3), C(7), C(11), C(19), C(23), C(27), C(35), C(39), C(43), C(51), C(55), C(59), C(67), C(71), C(75), C(83), C(87), C(91), C(99), C(103), C(107), C(115), C(119), C(123), C(131), C(135), C(139), C(143), C(147), C(151), C(155), C(159), C(163), C(167), C(171), C(175), C(179), C(183), C(187), C(191), C(195), C(199), C(203), C(207), C(211), C(215), C(219), C(221), C(223), C(227), C(229), C(231), C(235), C(237), C(239), C(243), C(245), C(247), C(251), C(253), C(255), C(257) and C(259)

are not transmitted. The result is a block of 196 coded and punctured bits, P(0)...P(195) which	are appended to the in-band bits in c as

c(k+4) = P(k)		for k = 0, 1, ..., 195.

Finally the 28 class 2 bits are appended to c

c(200+k ) = d(120+k)	for k = 0, 1, ..., 27.

TCH/AHS6.7:

The block of 116 bits {u(0)… u(115)} is encoded with the ½ rate convolutional code defined by	the following polynomials:

G0/G0 = 1

G1/G0 = 1 + D + D3+ D4 / 1 + D3 + D4

resulting in 240 coded bits, {C(0)… C(239)} defined by:

r(k)		= u(k) + r(k-3) + r(k-4)

C(2k)		= u(k)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 0, 1, ..., 115; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(2k)		= r(k-3) + r(k-4)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 116, 117 ..., 119

The code is punctured in such a way that the following 40 coded bits:

C(1), C(3), C(9), C(19), C(29), C(39), C(49), C(59), C(69), C(79), C(89), C(99), C(109), C(119), C(129), C(139), C(149), C(159), C(167), C(169), C(177), C(179), C(187), C(189), C(197), C(199), C(203), C(207), C(209), C(213), C(217), C(219), C(223), C(227), C(229), C(231), C(233), C(235), C(237) and C(239)

are not transmitted. The result is a block of 200 coded and punctured bits, P(0)...P(199) which	are appended to the in-band bits in c as

c(k+4) = P(k)		for k = 0, 1, ..., 199.

Finally the 24 class 2 bits are appended to c

c(204+k ) = d(110+k)	for k = 0, 1, ..., 23.

TCH/AHS5.9:

The block of 108 bits {u(0)… u(107)} is encoded with the ½ rate convolutional code defined by	the following polynomials:

G0/G0 = 1

G1/G0 = 1 + D + D3+ D4 / 1 + D3 + D4

resulting in 224 coded bits, {C(0)… C(223)} defined by:

r(k)		= u(k) + r(k-3) + r(k-4)

C(2k)		= u(k)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 0, 1, ..., 107; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(2k)		= r(k-3) + r(k-4)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)	      for k = 108, 109 ..., 111

The code is punctured in such a way that the following 16 coded bits:

C(1), C(15), C(71), C(127), C(139), C(151), C(163), C(175), C(187), C(195), C(203), C(211), C(215), C(219), C(221) and C(223)

are not transmitted. The result is a block of 208 coded and punctured bits, P(0)...P(207) which	are appended to the in-band bits in c as

c(k+4) = P(k)		for k = 0, 1, ..., 207.

Finally the 16 class 2 bits are appended to c

c(212+k ) = d(102+k)	for k = 0, 1, ..., 15.

TCH/AHS5.15:

The block of 97 bits {u(0)… u(96)} is encoded with the 1/3 rate convolutional code defined by	the	following polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

resulting in 303 coded bits, {C(0)… C(302)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(3k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(3k+1)	= r(k)+r(k-2)+r(k-4)

C(3k+2)	= u(k)					for k = 0, 1, ..., 96

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(3k+1)	= r(k)+r(k-2)+r(k-4)

C(3k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)	for k = 97, 98, ..., 100

The code is punctured in such a way that the following 91 coded bits:

C(0), C(1), C(3), C(4), C(6), C(9), C(12), C(15), C(18), C(21), C(27), C(33), C(39), C(45), C(51), C(54), C(57), C(63), C(69), C(75), C(81), C(87), C(90), C(93), C(99), C(105), C(111), C(117), C(123), C(126), C(129), C(135), C(141), C(147), C(153), C(159), C(162), C(165), C(168), C(171), C(174), C(177), C(180), C(183), C(186), C(189), C(192), C(195), C(198), C(201), C(204), C(207), C(210), C(213), C(216), C(219), C(222), C(225), C(228), C(231), C(234), C(237), C(240), C(243), C(244), C(246), C(249), C(252), C(255), C(256), C(258), C(261), C(264), C(267), C(268), C(270), C(273), C(276), C(279), C(280), C(282), C(285), C(288), C(289), C(291), C(294), C(295), C(297), C(298), C(300) and C(301)

are not transmitted. The result is a block of 212 coded and punctured bits, P(0)...P(211) which	are appended to the in-band bits in c as

c(k+4) = P(k)		for k = 0, 1, ..., 211.

Finally the 12 class 2 bits are appended to c

c(216+k ) = d(91+k)	for k = 0, 1, ..., 11.

TCH/AHS4.75:

The block of 89 bits {u(0)… u(88)} is encoded with the 1/3 rate convolutional code defined by the	following polynomials:

G4/G4 = 1

G5/G4 = 1 + D + D4 + D6/ 1 + D2  + D3 + D5 + D6

G6/G4 = 1 + D  + D2 + D3 + D4 + D6/ 1 + D2  + D3 + D5 + D6

resulting in 285 coded bits, {C(0)… C(284)} defined by:

r(k)		= u(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k)		= u(k)

C(3k+1)	= r(k)+r(k-1)+r(k-4)+r(k-6)

C(3k+2)	= r(k)+r(k-1)+ r(k-2)+r(k-3)+r(k-4)+r(k-6)	for k = 0, 1, ..., 88; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k)+r(k-1)+r(k-4)+r(k-6)

C(3k+2)	= r(k)+r(k-1)+ r(k-2)+r(k-3)+r(k-4)+r(k-6)	for k = 89, 90, ..., 94

The code is punctured in such a way that the following 73 coded bits:

C(1), C(2), C(4), C(5), C(7), C(8), C(10), C(13), C(16), C(22), C(28), C(34), C(40), C(46), C(52), C(58), C(64), C(70), C(76), C(82), C(88), C(94), C(100), C(106), C(112), C(118), C(124), C(130), C(136), C(142), C(148), C(151), C(154), C(160), C(163), C(166), C(172), C(175), C(178), C(184), C(187), C(190), C(196), C(199), C(202), C(208), C(211), C(214), C(220), C(223), C(226), C(232), C(235), C(238), C(241), C(244), C(247), C(250), C(253), C(256), C(259), C(262), C(265), C(268), C(271), C(274), C(275), C(277), C(278), C(280), C(281), C(283) and C(284)

are not transmitted. The result is a block of 212 coded and punctured bits, P(0)...P(211) which	are appended to the in-band bits in c as

c(k+4) = P(k)		for k = 0, 1, ..., 211.

Finally the 12 class 2 bits are appended to c

c(216+k ) = d(83+k)	for k = 0, 1, ..., 11.

#### 3.10.7.5	Interleaving

The interleaving is done as specified for the TCH/HS in subclause 3.2.3.

#### 3.10.7.6	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4.

### 3.10.8	RATSCCH_MARKER

This frame type contains the in-band channel and an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.10.8.1	Coding of in-band data

The in-band data, ic(0,1), is encoded to ic(0..15) which is moved to the coded data c as:

c(k)		= ic (k)				for k = 0,1, .., 15

#### 3.10.8.2	Identification marker

The identification marker, IM(0..211), is constructed by repeating the following 11-bit sequence: 
{ 1, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1 } 20 times and then discarding the last 8 bits. This block of data is moved to the coded data (c) as:

c(k+16) = IM(k)			for k = 0, 1, .., 211

#### 3.10.8.3	Interleaving

The interleaving is done as specified for the TCH/HS in subclause 3.2.3.

#### 3.10.8.4	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4.

### 3.10.9	RATSCCH_DATA

This frame contains the RATSCCH data and an inband channel. The RATSCCH data consists of 35 bits. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.10.9.1	Coding of in-band data

The in-band data, ic(0,1), is encoded to ic(0..15) which is moved to the coded data c as:

c(k)		= ic (k)				for k = 0,1, .., 15

#### 3.10.9.2	Parity and convolutional encoding for the RATSCCH message

a)	Parity bits:

A 14-bit CRC is used for error-detection. These 14 parity bits are generated by the cyclic generator polynomial: g(D) =  D14 + D13 + D5 + D3 + D2 +1 from the 35 comfort noise parameter bits. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(48) + d(1)D(47) +... + d(34)D(14) + p(0)D(13) +…+ p(12)D+ p(13)

where p(0), p(1) … p(13) are the parity bits, when divided by g(D), yields a remainder equal to 1+ D + D2 + D3 + D4 + D5 + D6 + D7 + D8 + D9 + D10 + D11 + D12+ D13

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, 34

u(k) = p(k-35)			for k = 35, 36, …, 48

b)	Convolutional encoder

The comfort noise parameters with parity and tail bits (u(0..48)) are encoded with the ¼ rate

convolutional code defined by the polynomials:

G1/G3  = 1 + D + D3 + D4 / 1 + D + D2 + D3 + D4

G2/G3  = 1 + D2 + D4 / 1 + D + D2 + D3 + D4

G3/G3 = 1

G3/G3 = 1

resulting in 212 coded bits, {C(0)… C(211)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4)

C(4k)		= r(k) + r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= u(k)

C(4k+3)	= u(k)					for k = 0, 1, ..., 48; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k)+r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2)+r(k-4)

C(4k+2)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)

C(4k+3)	= r(k-1)+r(k-2)+r(k-3)+r(k-4)				for k = 49, 50, ..., 52

This block of data is moved to the coded data (c) as:

c(k+16) = C(k)			for k = 0, 1, ..., 211

#### 3.10.9.3	Interleaving

The interleaving is done as specified for the TCH/HS in subclause 3.2.3.

#### 3.10.9.4	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4.

## 3.11	Data channel for ECSD at full rate, 29.0 kbit/s radio interface rate (28.8 kbit/s services (E-TCH/F28.8))

The definition of a 28.8 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.11.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 580 information bits (data frames) every 20 ms.

### 3.11.2	Block code

#### 3.11.2.1	Repetition bits

To match to RS alphabet 4 extra data bits are added to the end of each block of 580 bits: d(k)=0, k=580,…583.

#### 3.11.2.2	Reed Solomon encoder

The block of 584 information bits is encoded by shortened systematic Reed Solomon (RS) code over Galois field GF(28). The Galois field GF(28) is built as an extension of GF(2). The characteristic of GF(28) is equal to 2.

The code used is systematic RS8 (85,73), which is shortened systematic RS8(255,243) code over GF(28) with the primitive polynomial  p(x)=x8+x4+x3+x2+1. The primitive element a is the root of the primitive polynomial, i.e.

a8 = a4 + a3 + a2 + 1.

Generator polynomial for RS8(255,243) code is:

g(x)=; that  results in symmetrical form for the generator polynomial with coefficients given in decimal notation

g(x)= x12 +18x11 + 157x10 + 162x9 + 134x8 + 157x7 + 253x6 + 157x5 + 134x4 + 162x3 + 157x2 + 18x + 1

where binary presentation of polynomial coefficients in GF(256) is {a7, a6, a5, a4, a3, a2, a, 1}.

Specifically, decimal, power and polynomial presentations for the generator polynomial coefficients are the following:

x12:  1

x11:  18 = a224 =  a4 + a

x10: 157  = a32 =  a7 + a4 + a3 + a2 + 1

x9:  162 = a209 =  a7 + a5 + a

x8:  134 = a99 = a7 + a2 + a

x7:  157 = a32 = a7 + a4 + a3 + a2 + 1

x6:  253 = a80 = a7 + a6 + a5 + a4 + a3 + a2 + 1

x5:  157 = a32 = a7 + a4 + a3 + a2 + 1

x4:  134 = a99 = a7 + a2 + a

x3:  162 =a209 =  a7 + a5 +a

x2:  157 =a32 = a7 + a4 + a3 + a2 + 1

x1:  18 = a224 = a4 +  a

x0:  1= a255 = 1

The RS encoding is performed in the following three steps:

a)	Bit to symbol conversion

The information bits {d(0),d(1),…,d(583)} are converted into 73 information 8-bit symbols {D(0),…,D(72)} as the following:

D(k) = 128d(8k+7) + 64d(8k+6) + 32d(8k+5) + 16d(8k+4) + 8d(8k+3) + 4d(8k+2) + 2d(8k+1) + d(8k)
for k = 0,1,...,72

Resulting 8-bit symbols are presented as

D(k) = {d(8k+7), d(8k+6), d(8k+5), d(8k+4), d(8k+3), d(8k+2), d(8k+1), d(8k)}		for k = 0,1,...,72

where d(8k+7),…,d(8k) are ordered from the most significant bit (MSB) to the less significant bit (LSB).

The polynomial representation of a single information symbol over GF(28) in terms of a is given by

Da(k) = a7d(8k+7) + a6d(8k+6) + a5d(8k+5) + a4d(8k+4) + a3d(8k+3) + a2d(8k+2) + ad(8k+1) + d(8k)

b)	Encoding

The information symbols D(0)…D(72) are encoded by shortened systematic RS8(85,73) code with  output symbols    U(0)…U(84) ordered as

U(k)=D(k) for k=0,1,..72; U(k)=R(k) for k=73,74,…,84;

where R(k) are parity check symbols added by RS8(85,73) encoder.

Information symbols are ordered in the descending polynomial order such that Da (72) corresponds to the lowest

degree term of D(x) = Da(72) + Da (71)x + …+ Da (1)x71 + Da (0)x72, where D(x) is the polynomial representation of

information symbols {D(0),D(1),…,D(72)} over Galois field .

Parity check symbols in polynomial representation over Galois field are ordered in the descending polynomial order such that Ra(84) corresponds to the lowest degree of R(x)=Ra(84) + Ra(83)x + … + Ra(74)x10 + Ra(73)x11. The parity check symbols are calculated as R(x) = remainder [x12 D(x)/g(x)], and U(x) = R(x) + x12 D(x), i.e.,

Ua(k) = Da(k) for k=0,1,..72; Ua(k) = Ra(k) for k=73,74,…,84.

The encoding operation with the shortened RS8(85,73) code may be presented as the following:

-	Expanding 73 information symbols to the block of 243 symbols by adding 170 dump (zero) symbols

-	Encoding 243 symbols by systematic RS8(255,243) encoder with outer block of 255 symbols

-	Removing 170 dump symbols, resulting in the output block of 85 symbols.

c)	Symbol to bit conversion

The output symbols {Ua(0),…,Ua(84)}are converted back into symbols {U(0),…,U(84)} and then back into binary form with LSB coming out first, resulting in the block of 680 bits {u(0),…u(679)}.

### 3.11.3	Convolutional encoder

#### 3.11.3.1	Tailing bits for a data frame

Before convolutional encoding 6 tail bits {u(k)=0, k=680,…685}are added to the end of each data block .

#### 3.11.3.2	Convolutional encoding for a data frame

This block of 686 bits {u(0),...,u(685)} is encoded with the ½ rate convolutional code defined by the following polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

resulting in 1372 coded bits {c(0), c(1),..., c(1371)} with

c(2k)= u(k)+u(k-2)+u(k-3)+ u(k-5)+u(k-6);

c(2k+1)= u(k)+u(k-1)+ u(k-2)+ u(k-3)+ u(k-6)  for k = 0,1,...,685;  u(k) = 0 for k<0

The code is punctured in such a way that the following 4 coded bits:

c(363), c(723), c(1083) and c(1299) are not transmitted.

The result is a block of 1368 coded bits, {c(0),c(1),..., c(1367)}.

### 3.11.4	Interleaving

The interleaving scheme is presented below.

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k),	for	k  = 0,1,...,1367

n  = 0,1,...,N,N+1,…

B = B0 + 4n + (k mod 19) + (k div 342)

j   = (k mod 19) + 19(k mod 18)

The result of the interleaving is a distribution of the reordered 342 bit of a given data block, n = N, over 19 blocks, 18 bits equally distributed in each block, in a diagonal way over consecutive blocks.

Or in other words the interleaving is a distribution of the encoded, reordered 1368 bits from four given input data blocks, which taken together give n = N, over 22 bursts, 18 bits equally distributed in the first and 22nd bursts, 36 bits distributed in the second and 21st bursts, 54 bits distributed in the third and 20th bursts and 72 bits distributed in the other 16 bursts.

The block of coded data is interleaved "diagonal", where a new block of coded data starts with every fourth burst and is distributed over 22 bursts.

### 3.11.5	Mapping on a Burst

Before mapping on a burst the interleaved bits {i(0)…i(1367)} are converted into 3-bit symbols {I(0),I(1), …,I(455)} according  to Table 1 in 3GPP TS 45.004, the symbol I(k) depends on i(3k+2), i(3k+1) and i(3k) for k=0,1,…,455.

The E-IACCH message delivered to the encoder on every 20ms has a fixed size of 3 information bits {im(0), im(1), im(2)}. The contents of the bits are defined in 3GPP TS 45.008 for both uplink and downlink.

The E-IACCH information bits {im(n,0),im(n,1),im(n,2)} are coded into 24 bits
ib(B,k), B0 + 4n  B < B0 + 4n + 4, k = 0,1,...5 according to the following table:

| im(n,0),im(n,1),im(n,2) | ib(B0+4n,0),...,ib(B0+4n,5),..., ib(B0+4n+3,0),...,ib(B0+4n+3,5) |
| --- | --- |
| 000 | 000000 000000 000000 000000 |
| 001 | 001111 110100 100101 110100 |
| 010 | 011100 010111 111001 100011 |
| 011 | 010011 100011 011100 010111 |
| 100 | 100110 011001 110110 001101 |
| 101 | 101001 101101 010011 111001 |
| 110 | 111010 001110 001111 101110 |
| 111 | 110101 111010 101010 011010 |

Before mapping on a burst the E-IACCH bits {ib(B,0)…ib(B,5)} are converted into 3-bit symbols
{HL(B),HU(B)} according  to Table 1 in 3GPP TS 45.004. The symbol HL(B) depends on ib(B,2), ib(B,1) and ib(B,0) and ,

the symbol HU(B) on ib(B,5), ib(B,4) and ib(B,3).

The mapping is given by the rule:

E(B,j)	= I(B,j)	and	E(B,59+j) = I(B,57+j)		for j = 0,1,...,56

and

E(B,57) = HL(B)   and	E(B,58) = HU(B).

The two symbols, labelled HL(B) and HU(B) on burst number B are flags used for E-IACCH.

## 3.12	Data channel for ECSD at full rate, 32.0 kbit/s radio interface rate (32.0 kbit/s services (E-TCH/F32.0))

The definition of a 32.0 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.12.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 640 information bits (data frames) every 20 ms.

### 3.12.2	Void

### 3.12.3	Convolutional encoder

#### 3.12.3.1	Tailing bits for a data frame

Before convolutional encoding 6 tail bits {d(k)=0, k=640,…,645} are added to the end of each data block.

#### 3.12.3.2	Convolutional encoding for a data frame

This block of 646 bits {d(0),...,d(645)} is encoded with the 1/3 rate convolutional code (the same code as for MCS-1) defined by the following polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

resulting in 1938 coded bits {c(0), c(1),..., c(1937)} with

c(3k) = d(k) + d(k-2) + d(k-3) + d(k-5) + d(k-6) ;

c(3k+1) = d(k) + d(k-1) + d(k-2) + d(k-3) + d(k-6) ;

c(3k+2) = d(k) + d(k-1) + d(k-4) + d(k-6) ;

for k = 0,1,...,645 ; d(k) = 0 for k<1

The code is punctured using the rate matching algorithm of clause 6.2.3, such that the following 546 coded bits:

c(0), c(3), c(7), c(10), c(14), c(17), c(21), c(24), c(28), c(31), c(35), c(39), c(42), c(46), c(49), c(53), c(56), c(60), c(63), c(67), c(70), c(74), c(78), c(81), c(85), c(88), c(92), c(95), c(99), c(102), c(106), c(110), c(113), c(117), c(120), c(124), c(127), c(131), c(134), c(138), c(141), c(145), c(149), c(152), c(156), c(159), c(163), c(166), c(170), c(173), c(177), c(181), c(184), c(188), c(191), c(195), c(198), c(202), c(205), c(209), c(212), c(216), c(220), c(223), c(227), c(230), c(234), c(237), c(241), c(244), c(248), c(252), c(255), c(259), c(262), c(266), c(269), c(273), c(276), c(280), c(283), c(287), c(291), c(294), c(298), c(301), c(305), c(308), c(312), c(315), c(319), c(323), c(326), c(330), c(333), c(337), c(340), c(344), c(347), c(351), c(354), c(358), c(362), c(365), c(369), c(372), c(376), c(379), c(383), c(386), c(390), c(393), c(397), c(401), c(404), c(408), c(411), c(415), c(418), c(422), c(425), c(429), c(433), c(436), c(440), c(443), c(447), c(450), c(454), c(457), c(461), c(464), c(468), c(472), c(475), c(479), c(482), c(486), c(489), c(493), c(496), c(500), c(504), c(507), c(511), c(514), c(518), c(521), c(525), c(528), c(532), c(535), c(539), c(543), c(546), c(550), c(553), c(557), c(560), c(564), c(567), c(571), c(575), c(578), c(582), c(585), c(589), c(592), c(596), c(599), c(603), c(606), c(610), c(614), c(617), c(621), c(624), c(628), c(631), c(635), c(638), c(642), c(646), c(649), c(653), c(656), c(660), c(663), c(667), c(670), c(674), c(677), c(681), c(685), c(688), c(692), c(695), c(699), c(702), c(706), c(709), c(713), c(716), c(720), c(724), c(727), c(731), c(734), c(738), c(741), c(745), c(748), c(752), c(756), c(759), c(763), c(766), c(770), c(773), c(777), c(780), c(784), c(787), c(791), c(795), c(798), c(802), c(805), c(809), c(812), c(816), c(819), c(823), c(827), c(830), c(834), c(837), c(841), c(844), c(848), c(851), c(855), c(858), c(862), c(866), c(869), c(873), c(876), c(880), c(883), c(887), c(890), c(894), c(898), c(901), c(905), c(908), c(912), c(915), c(919), c(922), c(926), c(929), c(933), c(937), c(940), c(944), c(947), c(951), c(954), c(958), c(961), c(965), c(969), c(972), c(976), c(979), c(983), c(986), c(990), c(993), c(997), c(1000), c(1004), c(1008), c(1011), c(1015), c(1018), c(1022), c(1025), c(1029), c(1032), c(1036), c(1039), c(1043), c(1047), c(1050), c(1054), c(1057), c(1061), c(1064), c(1068), c(1071), c(1075), c(1079), c(1082), c(1086), c(1089), c(1093), c(1096), c(1100), c(1103), c(1107), c(1110), c(1114), c(1118), c(1121), c(1125), c(1128), c(1132), c(1135), c(1139), c(1142), c(1146), c(1150), c(1153), c(1157), c(1160), c(1164), c(1167), c(1171), c(1174), c(1178), c(1181), c(1185), c(1189), c(1192), c(1196), c(1199), c(1203), c(1206), c(1210), c(1213), c(1217), c(1221), c(1224), c(1228), c(1231), c(1235), c(1238), c(1242), c(1245), c(1249), c(1252), c(1256), c(1260), c(1263), c(1267), c(1270), c(1274), c(1277), c(1281), c(1284), c(1288), c(1292), c(1295), c(1299), c(1302), c(1306), c(1309), c(1313), c(1316), c(1320), c(1323), c(1327), c(1331), c(1334), c(1338), c(1341), c(1345), c(1348), c(1352), c(1355), c(1359), c(1362), c(1366), c(1370), c(1373), c(1377), c(1380), c(1384), c(1387), c(1391), c(1394), c(1398), c(1402), c(1405), c(1409), c(1412), c(1416), c(1419), c(1423), c(1426), c(1430), c(1433), c(1437), c(1441), c(1444), c(1448), c(1451), c(1455), c(1458), c(1462), c(1465), c(1469), c(1473), c(1476), c(1480), c(1483), c(1487), c(1490), c(1494), c(1497), c(1501), c(1504), c(1508), c(1512), c(1515), c(1519), c(1522), c(1526), c(1529), c(1533), c(1536), c(1540), c(1544), c(1547), c(1551), c(1554), c(1558), c(1561), c(1565), c(1568), c(1572), c(1575), c(1579), c(1583), c(1586), c(1590), c(1593), c(1597), c(1600), c(1604), c(1607), c(1611), c(1615), c(1618), c(1622), c(1625), c(1629), c(1632), c(1636), c(1639), c(1643), c(1646), c(1650), c(1654), c(1657), c(1661), c(1664), c(1668), c(1671), c(1675), c(1678), c(1682), c(1685), c(1689), c(1693), c(1696), c(1700), c(1703), c(1707), c(1710), c(1714), c(1717), c(1721), c(1725), c(1728), c(1732), c(1735), c(1739), c(1742), c(1746), c(1749), c(1753), c(1756), c(1760), c(1764), c(1767), c(1771), c(1774), c(1778), c(1781), c(1785), c(1788), c(1792), c(1796), c(1799), c(1803), c(1806), c(1810), c(1813), c(1817), c(1820), c(1824), c(1827), c(1831), c(1835), c(1838), c(1842), c(1845), c(1849), c(1852), c(1856), c(1859), c(1863), c(1867), c(1870), c(1874), c(1877), c(1881), c(1884), c(1888), c(1891), c(1895), c(1898), c(1902), c(1906), c(1909), c(1913), c(1916), c(1920), c(1923), c(1927), c(1930), c(1934)

are not transmitted.

The result is a block of 1392 coded bits, {c(0),c(1),..., c(1391)}.

### 3.12.4	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k),	for	k  = 0,1,...,1391

n  = 0,1,...,N,N+1,…

B = B0 + 4n + (k mod 12)

j = 3*[(49*(k+int(k/348)) mod 116) + int[(k mod 12)/4]

The result of the interleaving is a distribution of the reordered 348 bits of a given data block, n = N, over 12 blocks, 29 bits equally distributed in each block. The block of coded data is interleaved "diagonal", where a new block of coded data starts with every fourth burst and is distributed over 12 bursts.

### 3.12.5	Mapping on a Burst

The mapping is given by the rule:

e(B,j)	= i(B,j)	for j = 0,1,…,347

NOTE:	No stealing flags are used.

## 3.13	Data channel for ECSD at full rate, 43.5 kbit/s radio interface rate (43.2 kbit/s services (E-TCH/F43.2))

The definition of a 43.5 kbit/s radio interface rate data flow for data services is given in 3GPP TS 44.021.

### 3.13.1	Interface with user unit

The user unit delivers to the encoder a bit stream organized in blocks of 870 information bits (data frames) every 20 ms.

### 3.13.2	Convolutional encoder

#### 3.13.2.1	Tailing bits for a data frame

Before convolutional encoding 6 tail bits {d(k)=0, k=870,…875} are added to the end of each data block .

#### 3.13.2.2	Convolutional encoding for a data frame

This block of 876 bits {d(0),...,d(875)} is encoded with the ½ rate convolutional code defined by the following polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

resulting in 1752 coded bits {c(0), c(1),..., c(1751)} with

c(2k)= d(k)+d(k-2)+d(k-3)+ d(k-5)+d(k-6);

c(2k+1)= d(k)+d(k-1)+ d(k-2)+ d(k-3)+ d(k-6)  for k = 0,1,...,875;  u(k) = 0 for k<0

The code is punctured in such a way that the following 384 coded bits:

c(2+8(k-1)) for k=1:219; c(4+16(k-1)) for k=1:110;  c(6+32(k-1)) for k=1:55

are not transmitted.

The result is a block of 1368 coded bits, {c(0),c(1),..., c(1367)}.

### 3.13.3	Interleaving

The interleaving is done as specified for E-TCH/F28.8 in subclause 3.11.4.

### 3.13.4	Mapping on a Burst

The mapping is done as specified for E-TCH/F28.8 in subclause 3.11.5.

## 3.14	Wideband Adaptive multi rate speech channel at full rate (TCH/WFS)

This section describes the coding for the different frame formats used for TCH/WFS. The formats used are (in the order they are described):

SID_UPDATE	Used to convey comfort noise parameters during DTX

SID_FIRST		Marker to define end of speech, start of DTX

ONSET			Used to signal the Codec mode for the first speech frame after DTX

SPEECH			Speech frames

RATSCCH		Frames used to convey RATSCCH messages

In this chapter, sub chapters 3.14.1 to 3.14.4 describe the channel coding for the different formats listed above.

Common to all the formats is that in-band information is conveyed, the coding for the in-band channel is described in the table below.

| Identifier (defined in 3GPP TS 45.009) | Received in-band data id(1), id(0) | Encoded in-band data for SID and RATSCCH frames ic(15),.., ic(0) | Encoded in-band data for speech frames ic(7),.., ic(0) |
| --- | --- | --- | --- |
| CODEC_MODE_1 | 00 | 0101001100001111 | 00000000 |
| CODEC_MODE_2 | 01 | 0011111010111000 | 10111010 |
| CODEC_MODE_3 | 10 | 1000100001100011 | 01011101 |
| CODEC_MODE_4 | 11 | 1110010111010100 | 11100111 |

### 3.14.1	SID_UPDATE

The SID_UPDATE frames are handled as specified for the TCH/AFS in subclause 3.9.1.

### 3.14.2	SID_FIRST

The SID_FIRST frames are handled as specified for the TCH/AFS in subclause 3.9.2.

### 3.14.3	ONSET

The Onset frames are handled as specified for the TCH/AFS in subclause 3.9.3.

### 3.14.4	SPEECH

The speech coder delivers to the channel encoder a sequence of blocks of data. One block of data corresponds to one speech frame and the block length is different in each of the seven channel codec modes. Adjoining each block of data is information of the channel codec mode to use when encoding the block. Also delivered is the in-band data id(0,1) representing Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.14.4.1	Coding of the in-band data

The two input in-band bits (id(0,1)) are coded to eight coded in-band bits (ic(0..7)).

The encoded in-band bits are moved to the coded bits, c, as

c(k) = ic(k)		for k = 0, 1, ..., 7.

#### 3.14.4.2	Ordering according to subjective importance

The bits delivered by the speech encoder, {s(1),s(2),...,s(Ks)},  are rearranged according to subjective importance before channel coding. Tables 16 to 18 define the correct rearrangement for the speech codec modes 12.65 kbit/s, 8.85 kbit/s and 6.60 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to 3GPP TS 26.190 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see below:

| Codec mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| TCH/WFS12.65 | 253 |
| TCH/WFS8.85 | 177 |
| TCH/WFS6.60 | 132 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1	d(j) := s(table(j)+1);			where table(j) is read line by line left to right

The rearranged bits are further divided into two different classes to perform unequal error protection for different bits according to subjective importance.

The protection classes are:

1a	-	Data protected with the CRC and the convolution code.
	1b	-	Data protected with the convolution code.

The number of class 1 (sum of class 1a and 1b), class 1a and class 1b bits for each codec mode is shown below:

| Codec mode | Number of speech bits delivered per block | Number of class 1 bits per block | Number of class 1a bits per block | Number of class 1b bits per block |
| --- | --- | --- | --- | --- |
| TCH/WFS12.65 | 253 | 253 | 72 | 181 |
| TCH/WFS8.85 | 177 | 177 | 64 | 113 |
| TCH/WFS6.60 | 132 | 132 | 54 | 78 |

#### 3.14.4.3	Parity for speech frames

The basic parameters for each codec mode for the first encoding step are shown below:

| Codec mode | Number of class 1 bits (Kd1) | CRC Protected bits (Kd1a) | CRC bits | Number of bits after first encoding step (Ku = Kd + 8/6) |
| --- | --- | --- | --- | --- |
| TCH/WFS12.65 | 253 | 72 | 6 | 259 |
| TCH/WFS8.85 | 177 | 64 | 6 | 183 |
| TCH/WFS6.60 | 132 | 54 | 8 | 140 |

A 8-bit or 6-bit CRC is used for error-detection. These parity bits are generated by the cyclic generator polynomial: g8(D) = D8 + D4 + D3 + D2 + 1 or g6(D) = D6 + D5 + D3 + D2 + D1 + 1 respectively from the first Kd1a bits of class 1, where Kd1a refers to number of bits in protection class 1a as shown above for each codec mode. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(Kd1a+n-1) + d(1)D(Kd1a+n-2) +... + d(Kd1a-1)D(n) + p(0)D(n-1) +…+ p(n-2)D+ p(n-1)

where p(0), p(1) … p(n) are the parity bits (n=8 or 6), when divided by g8(D) or g6(D), yields a remainder equal to:

1+ D + … + Dn-1.

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, Kd1a-1

u(k) = p(k-Kd1a)			for k = Kd1a, Kd1a+1, …, Kd1a+ (n–1)

u(k) = d(k-n)				for k = Kd1a+n, Kd1a+n+1, …, Ku-1

Thus, after the first encoding step u(k) will be defined by the following contents for each codec mode:

TCH/WFS12.65:

u(k) = d(k)				for k = 0, 1, ..., 71

u(k) = p(k-72)			for k = 72, 73, …, 77

u(k) = d(k-6)				for k = 78, 79, …, 258

TCH/WFS8.85:

u(k) = d(k)				for k = 0, 1, …, 63

u(k) = p(k-64)			for k = 64, 65, …, 69

u(k) = d(k-6)				for k = 70, 71, …, 182

TCH/WFS6.60:

u(k) = d(k)				for k = 0, 1, ..., 53

u(k) = p(k-54)			for k = 54, 55, ..., 61

u(k) = d(k-8)				for k = 62, 63, ..., 139

#### 3.14.4.4	Convolutional encoder

The bits from the first encoding step (u(k)) are encoded with the recursive systematic convolutional codes as summarised below. The number of output bits after puncturing is 448 for all codec modes.

| Codec Mode | Rate | Number of input bits to conv. coder | Number of output bits from conv. Coder | Number Of punctured bits |
| --- | --- | --- | --- | --- |
| TCH/WFS12.65 | ½ | 259 | 526 | 78 |
| TCH/WFS8.85 | 1/3 | 183 | 561 | 113 |
| TCH/WFS6.60 | ¼ | 140 | 576 | 128 |

Below the coding for each codec mode is specified in detail. The puncturing for each mode is designed to give an even protection of the class 1A bits while the protection within class 1B is not equal to reflect the individual error sensitivity of the class 1B bits.

TCH/WFS12.65:

The block of 259 bits {u(0)… u(258)} is encoded with the ½ rate convolutional code defined by	the following polynomials:

G0/G0 = 1

G1/G0 = 1 + D + D3+ D4 / 1 + D3 + D4

resulting in 518 coded bits, {C(0)… C(517)} defined by:

r(k)		= u(k) + r(k-3) + r(k-4)

C(2k)		= u(k)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)			for k = 0, 1, ..., 258; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(2k)		= r(k-3) + r(k-4)

C(2k+1)	= r(k)+r(k-1)+r(k-3)+r(k-4)			for k = 259, 260, ..., 262

The code is punctured in such a way that the following 78 coded bits:

C(1), C(17), C(33), C(191), C(207), C(223), C(239), C(251), C(253), C(255), C(267), C(269), C(271), C(283), C(285), C(287), C(297), C(299), C(301), C(303), C(313), C(315), C(317), C(319), C(329), C(331), C(333), C(335), C(345), C(347), C(349), C(351), C(361), C(363), C(365), C(367), C(377), C(379), C(381), C(383), C(393), C(395), C(397), C(399), C(409), C(411), C(413), C(415), C(425), C(427), C(429), C(431), C(441), C(443), C(445), C(447), C(457), C(459), C(461), C(463), C(473), C(475), C(477), C(479), C(487), C(489), C(491), C(493), C(495), C(503), C(505), C(507), C(509), C(511), C(519), C(521), C(523), C(525)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which are appended to the in-band bits in c as

c(8+k) = P(k)		for k = 0, 1, ..., 447.

TCH/WFS8.85:

The block of 183 bits {u(0)… u(182)} is encoded with the 1/3 rate convolutional code defined by	the following polynomials:

G1/G1  = 1

G2/G1  = 1 + D2 + D4 / 1 + D + D3 + D4

G3/G1 = 1 + D + D2 + D3 + D4  / 1 + D + D3 + D4

resulting in 549 coded bits, {C(0)… C(548)} defined by:

r(k)		= u(k) + r(k-1) +r(k-3) + r(k-4)

C(3k)		= u(k)

C(3k+1)		= r(k)+r(k-2) + r(k-4)

C(3k+2)	= r(k) + r(k-1)+r(k-2)+r(k-3)+r(k-4)			for k = 0, 1, ..., 182; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k-1) + r(k-3) + r(k-4)

C(3k+1)		= r(k)+r(k-2) + r(k-4)

C(3k+2)	= r(k) + r(k-1)+r(k-2)+r(k-3)+r(k-4)			for k = 183, 184, ..., 186

The code is punctured in such a way that the following 113 coded bits:

C(2), C(20), C(23), C(44), C(47), C(71), C(95), C(119), C(143), C(167), C(191), C(212), C(215), C(227), C(230), C(233), C(236), C(239), C(251), C(254), C(257), C(260), C(263), C(275), C(278), C(281), C(284), C(287), C(299), C(302), C(305), C(308), C(311), C(323), C(326), C(329), C(332), C(335), C(341), C(344), C(347), C(350), C(353), C(356), C(359), C(365), C(368), C(371), C(374), C(377), C(380), C(383), C(386), C(389), C(392), C(395), C(398), C(401), C(404), C(407), C(410), C(413), C(416), C(419), C(422), C(425), C(428), C(431), C(434), C(437), C(440), C(443), C(446), C(449), C(452), C(455), C(458), C(461), C(464), C(467), C(470), C(473), C(476), C(479), C(485), C(488), C(491), C(494), C(497), C(500), C(503), C(506), C(509), C(512), C(515), C(518), C(521), C(524), C(527), C(530), C(533), C(536), C(539), C(542), C(545), C(548), C(551), C(553), C(554), C(556), C(557), C(559), C(560)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which	are appended to the in-band bits in c as

c(8+k) = P(k)		for k = 0, 1, ..., 447.

TCH/WFS6.60:

The block of 140 bits {u(0)... u(139)} is encoded with the ¼ rate convolutional code defined by	the following polynomials:

G1/G1  = 1

G2/G1  = 1 + D2 + D4 / 1 + D + D3 + D4

G3/G1 = 1 + D + D2 + D3 + D4  / 1 + D + D3 + D4

G1/G1 = 1

resulting in 560 coded bits, {C(0)… C(559)} defined by:

r(k)		= u(k) + r(k-1) +r(k-3) + r(k-4)

C(4k)		= u(k)

C(4k+1)	= r(k) + r(k-2) + r(k-4)

C(4k+2)	= r(k) + r(k-1)+r(k-2)+ r(k-3)+r(k-4)

C(4k+3)	= u(k)			for k = 0, 1, ..., 139; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k-1) + r(k-3) + r(k-4)

C(4k+1)	= r(k)+r(k-2) + r(k-4)

C(4k+2)	= r(k) + r(k-1)+r(k-2)+ r(k-3)+r(k-4)

C(4k+3)	= r(k-1) + r(k-3) + r(k-4)			for k = 140, 141, ..., 143

The code is punctured in such a way that the following 128 coded bits:

C(3), C(7), C(11), C(15), C(27), C(31), C(35), C(39), C(51), C(55), C(59), C(75), C(79), C(83), C(99), C(103), C(107), C(123), C(127), C(131), C(147), C(151), C(155), C(171), C(175), C(179), C(195), C(199), C(203), C(219), C(223), C(227), C(231), C(243), C(247), C(251), C(255), C(267), C(271), C(275), C(279), C(283), C(291), C(295), C(299), C(303), C(307), C(311), C(315), C(319), C(323), C(327), C(331), C(335), C(339), C(343), C(347), C(351), C(355), C(359), C(363), C(367), C(371), C(375), C(379), C(382), C(383), C(387), C(391), C(395), C(399), C(403), C(406), C(407), C(411), C(415), C(419), C(423), C(427), C(430), C(431), C(435), C(439), C(443), C(447), C(451), C(454), C(455), C(459), C(463), C(467), C(471), C(475), C(478), C(479), C(483), C(487), C(491), C(495), C(499), C(502), C(503), C(507), C(511), C(515), C(519), C(523), C(526), C(527), C(531), C(535), C(539), C(543), C(547), C(550), C(551), C(555), C(559), C(562), C(563), C(566), C(567), C(569), C(570), C(571), C(573), C(574), C(575)

are not transmitted. The result is a block of 448 coded and punctured bits, P(0)...P(447) which are appended to the inband bits in c as

c(8+k) = P(k)		for k = 0, 1, ..., 447.

#### 3.14.4.5	Interleaving

The interleaving is done as specified for the TCH/FS in subclause 3.1.3.

#### 3.14.4.6	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4.

### 3.14.5	RATSCCH

The RATSCCH frames are handled as specified for TCH/AFS in subclause 3.9.5.

## 3.15	Adaptive multi rate speech channel at 8-PSK half rate (O-TCH/AHS)

This section describes the coding for the different frame formats used for O-TCH/AHS. The formats used are (in the order they are described):

SID_UPDATE			Used to convey comfort noise parameters during DTX

SID_UPDATE_INH		Used to inhibit the second part of a SID_UPDATE frame if there is a speech onset

SID_FIRST_P1			First part of marker to define end of speech, start of DTX

SID_FIRST_P2			Second part of marker to define end of speech, start of DTX

SID_FIRST_INH		Used to inhibit the second part of a SID_FIRST_P1 frame if there is a speech onset

ONSET					Used to signal the Codec mode for the first speech frame after DTX

SPEECH					Speech frames

RATSCCH_MARKER	Marker to identify RATSCCH frames

RATSCCH_DATA		Frame that conveys the actual RATSCCH message

In this chapter, sub chapters 3.15.1 to 3.15.9 describe the channel coding for the different formats listed above.

Common to all the formats is that in-band information is conveyed, the coding for the in-band channel is described in the table below.

| Identifier (defined in 3GPP 45.009) | Received in-band data id(1), id(0) | Encoded in-band data for SID and RATSCCH frames ic(15),.., ic(0) | Encoded in-band data for speech frames ic(11),.., ic(0) |
| --- | --- | --- | --- |
| CODEC_MODE_1 | 00 | 0101001100001111 | 000000000000 |
| CODEC_MODE_2 | 01 | 0011111010111000 | 110110101110 |
| CODEC_MODE_3 | 10 | 1000100001100011 | 101101110101 |
| CODEC_MODE_4 | 11 | 1110010111010100 | 011011011011 |

### 3.15.1	SID_UPDATE

The speech encoder delivers 35 bits of comfort noise parameters. Also delivered is two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands/Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 CN bits which are then coded by a rate ¼ RSC coder to 212 bits. A 212 bit identification field is added thereby giving a total size of 456 bits. Finally each bit is repeated 3 times and then converted into 3-bit symbols giving a total size of 456 symbols. These 456 symbols are block interleaved over 4 bursts.

#### 3.15.1.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_UPDATE frame at half rate in subclause 3.10.1.1.

#### 3.15.1.2	Parity and convolutional encoding for the comfort noise parameters

The parity and convolutional encoding for the comfort noise parameters are done as specified for the SID_UPDATE frame at half rate in subclause 3.10.1.2.

#### 3.15.1.3	Identification marker

The identification marker is constructed as specified for the SID_UPDATE frame at half rate in subclause 3.10.1.3.

#### 3.15.1.4	Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,455

#### 3.15.1.5	Interleaving

Before interleaving the coded bits {c'(0) … c'(1367)} are converted into 3-bit symbols {C(0) … C(455)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,455.

The interleaving is done as specified for the SID_UPDATE frame at half rate in subclause 3.10.1.4. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.15.1.6	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE frame at half rate in subclause 3.10.1.5 with exception that it is done by symbols instead of single bits.

### 3.15.2	SID_UPDATE_INH

This special frame is used when the first 2 burst of a SID_UPDATE frame have been transmitted but the second two bursts cannot be transmitted due to a speech frame. The general coding is as: the in-band data (Note that this must be the same Mode Indication bits as id1(0,1) for the SID_UPDATE frame that is being inhibited) is encoded, a marker that is the opposite of the SID_UPDATE marker is appended and the data is interleaved in such a way that the odd symbols of two bursts are filled.

#### 3.15.2.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_UPDATE_INH frame at half rate in subclause 3.10.2.1.

#### 3.15.2.2	Identification marker

The identification marker is constructed as specified for the SID_UPDATE_INH frame at half rate in subclause 3.10.2.2.

#### 3.15.2.3		Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

#### 3.15.2.4	Interleaving

Before interleaving the coded bits {c'(0) … c'(683)} are converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the SID_UPDATE_INH frame at half rate in subclause 3.10.2.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables. The result of the interleaving is a distribution of 114 of the reordered 228 symbols of a given data block over 2 blocks using the odd numbered bits. The even numbered symbols of these 2 blocks will be filled by the speech frame that following immediately after this frame.

#### 3.15.2.5	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE_INH frame at half rate in subclause 3.10.2.4 with exception that it is done by symbols instead of single bits.

### 3.15.3	SID_FIRST_P1

This frame type contains no source data from the speech coder. What is generated is the in-band channel and an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.15.3.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_FIRST_P1 frame at half rate in subclause 3.10.3.1.

#### 3.15.3.2	Identification marker

The identification marker is constructed as specified for the SID_FIRST_P1 frame at half rate in subclause 3.10.3.2.

#### 3.15.3.3		Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

#### 3.15.3.4	Interleaving

Before interleaving the coded bits {c'(0) … c'(683)} are converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the TCH/HS in subclause 3.2.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.15.3.5	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4 with exception that it is done by symbols instead of single bits.

### 3.15.4	SID_FIRST_P2

This frame type contains no source data from the speech coder. What is generated is the in-band channel and, derived from that, an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.15.4.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_FIRST_P2 frame at half rate in subclause 3.10.4.1.

#### 3.15.4.2		Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,2,4…,226

#### 3.15.4.3	Interleaving

Before interleaving the coded bits {c'(0), c'(1), c'(2), c'(6), c'(7), c'(8) … c'(678), c'(679), c'(680)} are converted into 3-bit symbols {C(0), C(2) , C(4) … C(226)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,2,4,…,226.

The interleaving is done as specified for the SID_FIRST_P2 frame at half rate in subclause 3.10.4.2. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables. The result of the interleaving is a distribution of 114 of the reordered 228 symbols of a given data block over 2 blocks using the even numbered symbols. The odd numbered symbols of these 2 blocks have already been filled by the SID_FIRST_P1 frame.

#### 3.15.4.4	Mapping on a Burst

The mapping is done as specified for the SID_FIRST_P2 frame at half rate in subclause 3.10.4.3 with exception that it is done by symbols instead of single bits.

### 3.15.5	SID_FIRST_INH

This special frame is used when the first 2 burst of a SID_FIRST_P1 frame have been transmitted but the second two bursts cannot be transmitted due to a SPEECH frame. The general coding is as: the in-band data (Note that this must be the same data as for the SID_FIRST_P1 frame that is being inhibited) is encoded, a marker that is the opposite of the SID_FIRST_P1 marker is appended and the data is interleaved in such a way that the odd symbols of two bursts are filled.

#### 3.15.5.1	Coding of in-band data

The coding of the in-band data is done as specified for the SID_FIRST_P1 frame in subclause 3.10.3.1.

#### 3.15.5.2	Identification marker

The identification marker is done as specified for the SID_FIRST_INH frame at half rate in subclause 3.10.5.2.

#### 3.15.5.3		Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

#### 3.15.5.4	Interleaving

Before interleaving the coded bits {c'(0) … c'(683)} are converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the SID_UPDATE_INH in subclause 3.10.2.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.15.5.5	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE_INH in subclause 3.10.2.4 with exception that it is done by symbols instead of single bits.

### 3.15.6	ONSET

Onset frames are used to preset the interleaver buffer after a period of no speech activity in DTX mode. This frame type contains no source data from the speech coder. What is transmitted is the in-band channel signalling the Mode Indication for the speech frame following the onset marker.

#### 3.15.6.1	Coding of in-band data

The coding of in-band data is done as specified for the ONSET frame at half rate in subclause 3.10.6.1.

#### 3.15.6.2		Repetition

The repetition is done as specified for the SID_UPDATE_INH frame in subclause 3.15.2.3.

#### 3.15.6.3	Interleaving

The interleaving is done as specified for the SID_UPDATE_INH frame in subclause 3.15.2.4.

#### 3.15.6.4	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE_INH frame in subclause 3.15.2.5.

### 3.15.7	SPEECH

The speech coder delivers to the channel encoder a sequence of blocks of data. One block of data corresponds to one speech frame and the block length is different in each of the eight channel codec modes. Adjoining each block of data is information of the channel codec mode to use when encoding the block. Also delivered is the in-band data id(0,1) representing Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.15.7.1	Coding of the in-band data

The two input in-band bits (id(0,1)) are coded to twelve coded in-band bits (ic(0..11)).

#### 3.15.7.2	Ordering according to subjective importance

The bits delivered by the speech encoder, {s(1),s(2),...,s(Ks)},  are rearranged according to subjective importance before channel coding. Tables 7 to 14 define the correct rearrangement for the speech codec modes 12.2 kbit/s, 10.2 kbit/s, 7.95 kbit/s, 7.4 kbit/s, 6.7 kbit/s, 5.9 kbit/s, 5.15 kbit/s and 4.75 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to 3GPP TS 26.090 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see below:

| Codec mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| O-TCH/AHS12.2 | 244 |
| O-TCH/AHS10.2 | 204 |
| O-TCH/AHS7.95 | 159 |
| O-TCH/AHS7.4 | 148 |
| O-TCH/AHS6.7 | 134 |
| O-TCH/AHS5.9 | 118 |
| O-TCH/AHS5.15 | 103 |
| O-TCH/AHS4.75 | 95 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1	d(j) := s(table(j)+1);			where table(j) is read line by line left to right

The rearranged bits are further divided into three different classes to perform unequal error protection for different bits according to subjective importance.

The protection classes are:

1a	-	Data protected with the CRC and the convolution code.
	1b	-	Data protected with the convolution code.

The number of class 1 (sum of class 1a and 1b), class 1a and class 1b bits for each codec mode is shown below:

| Codec mode | Number of speech bits delivered per block | Number of class 1a bits per block | Number of class 1b bits per block |
| --- | --- | --- | --- |
| O-TCH/AHS12.2 | 244 | 81 | 163 |
| O-TCH/AHS10.2 | 204 | 65 | 139 |
| O-TCH/AHS7.95 | 159 | 75 | 84 |
| O-TCH/AHS7.4 | 148 | 61 | 87 |
| O-TCH/AHS6.7 | 134 | 55 | 79 |
| O-TCH/AHS5.9 | 118 | 55 | 63 |
| O-TCH/AHS5.15 | 103 | 49 | 54 |
| O-TCH/AHS4.75 | 95 | 39 | 56 |

#### 3.15.7.3	Parity for speech frames

The basic parameters for each codec mode for the first encoding step are shown below:

| Codec mode | Speech encoded bits (Kd) | CRC protected bits (Kd1a) | Number of bits after first encoding step (Ku = Kd + 6) |
| --- | --- | --- | --- |
| O-TCH/AHS12.2 | 244 | 81 | 250 |
| O-TCH/AHS10.2 | 204 | 65 | 210 |
| O-TCH/AHS7.95 | 159 | 75 | 165 |
| O-TCH/AHS7.4 | 148 | 61 | 154 |
| O-TCH/AHS6.7 | 134 | 55 | 140 |
| O-TCH/AHS5.9 | 118 | 55 | 124 |
| O-TCH/AHS5.15 | 103 | 49 | 109 |
| O-TCH/AHS4.75 | 95 | 39 | 101 |

A 6-bit CRC is used for error-detection. These 6 parity bits are generated by the cyclic generator polynomial: g(D) = D6 + D5 + D3 + D2 + D1 + 1 from the first Kd1a bits of class 1, where Kd1a refers to number of bits in protection class 1a as shown above for each codec mode. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(Kd1a+5) + d(1)D(Kd1a+4) +... + d(Kd1a-1)D(6) + p(0)D(5) +…+ p(4)D+ p(5)

where p(0), p(1) … p(5) are the parity bits, when divided by g(D), yields a remainder equal to:

1+ D + D2 + D3 + D4 + D5.

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, Kd1a-1

u(k) = p(k-Kd1a)			for k = Kd1a, Kd1a+1, …, Kd1a+5

u(k) = d(k-6)				for k = Kd1a+6, Kd1a+7, …, Ku-1

Thus, after the first encoding step u(k) will be defined by the following contents for each codec mode:

O-TCH/AHS12.2:

u(k) = d(k)				for k = 0, 1, …, 80

u(k) = p(k-81)			for k = 81, 82, …, 86

u(k) = d(k-6)				for k = 87, 88, …, 249

O-TCH/AHS10.2:

u(k) = d(k)				for k = 0, 1, ..., 64

u(k) = p(k-65)			for k = 65, 66, ..., 70

u(k) = d(k-6)				for k = 71, 72, ..., 209

O-TCH/AHS7.95:

u(k) = d(k)				for k = 0, 1, …, 74

u(k) = p(k-75)			for k = 75, 76, …, 80

u(k) = d(k-6)				for k = 81, 82, …, 164

O-TCH/AHS7.4:

u(k) = d(k)				for k = 0, 1, …, 60

u(k) = p(k-61)			for k = 61, 62, …, 66

u(k) = d(k-6)				for k = 67, 68, …, 153

O-TCH/AHS6.7:

u(k) = d(k)				for k = 0, 1, …, 54

u(k) = p(k-55)			for k = 55, 56, …, 60

u(k) = d(k-6)				for k = 61, 62, …, 139

O-TCH/AHS5.9:

u(k) = d(k)				for k = 0, 1, …, 54

u(k) = p(k-55)			for k = 55, 56, …, 60

u(k) = d(k-6)				for k = 61, 62, …, 123

O-TCH/AHS5.15:

u(k) = d(k)				for k = 0, 1, …, 48

u(k) = p(k-49)			for k = 49, 50, …, 54

u(k) = d(k-6)				for k = 55, 56, …, 108

O-TCH/AHS4.75:

u(k) = d(k)				for k = 0, 1, ..., 38

u(k) = p(k-39)			for k = 39, 40, ..., 44

u(k) = d(k-6)				for k = 45, 46, ..., 100

#### 3.15.7.4	Convolutional encoder

The bits from the first encoding step (u(k)) are encoded with the recursive systematic convolutional codes as summarised below. The number of output bits after puncturing is 672 for all codec modes.

| Codec Mode | Rate | Number of input bits to conv. coder | Number of output bits from conv. coder | Number of punctured bits |
| --- | --- | --- | --- | --- |
| O-TCH/AHS12.2 | 1/3 | 250 | 768 | 96 |
| O-TCH/AHS10.2 | ¼ | 210 | 864 | 192 |
| O-TCH/AHS7.95 | ¼ | 165 | 684 | 12 |
| O-TCH/AHS7.4 | 1/5 | 154 | 800 | 128 |
| O-TCH/AHS6.7 | 1/5 | 140 | 730 | 58 |
| O-TCH/AHS5.9 | 1/6 | 124 | 780 | 108 |
| O-TCH/AHS5.15 | 1/6 | 109 | 690 | 18 |
| O-TCH/AHS4.75 | 1/7 | 101 | 749 | 77 |

Below the coding for each codec mode is specified in detail.

O-TCH/AHS12.2:

The block of 250 bits {u(0)… u(249)} is encoded with the 1/3 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 768 coded bits, {C(0)… C(767)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(3k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(3k+2)	= u(k)	for k = 0, 1, ..., 249;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(3k+2)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 250, 251, ..., 255

The following 448 coded bits are moved to data block PG:

C(2), C(3), C(5), C(6), C(8), C(9), C(11), C(12), C(14), C(15), C(17), C(18), C(20), C(21), C(22), C(23), C(24), C(26), C(27), C(29), C(30), C(32), C(33), C(34), C(35), C(36), C(38), C(39), C(41), C(42), C(44), C(45), C(46), C(47), C(48), C(50), C(51), C(53), C(54), C(56), C(57), C(58), C(59), C(60), C(62), C(63), C(65), C(66), C(68), C(69), C(70), C(71), C(72), C(74), C(75), C(77), C(78), C(80), C(81), C(82), C(83), C(84), C(86), C(87), C(89), C(90), C(92), C(93), C(94), C(95), C(96), C(98), C(99), C(101), C(102), C(104), C(105), C(106), C(107), C(108), C(110), C(111), C(113), C(114), C(116), C(117), C(118), C(119), C(120), C(122), C(123), C(125), C(126), C(128), C(129), C(130), C(131), C(132), C(134), C(135), C(137), C(138), C(140), C(141), C(142), C(143), C(144), C(146), C(147), C(149), C(150), C(152), C(153), C(154), C(155), C(156), C(158), C(159), C(161), C(162), C(164), C(165), C(166), C(167), C(168), C(170), C(171), C(173), C(174), C(176), C(177), C(178), C(179), C(180), C(182), C(183), C(185), C(186), C(188), C(189), C(190), C(191), C(192), C(194), C(195), C(197), C(198), C(200), C(201), C(202), C(203), C(204), C(206), C(207), C(209), C(210), C(212), C(213), C(214), C(215), C(216), C(218), C(219), C(221), C(222), C(224), C(225), C(226), C(227), C(228), C(230), C(231), C(233), C(234), C(236), C(237), C(238), C(239), C(240), C(242), C(243), C(245), C(246), C(248), C(249), C(250), C(251), C(252), C(254), C(255), C(257), C(258), C(260), C(261), C(262), C(263), C(264), C(266), C(267), C(269), C(270), C(272), C(273), C(275), C(276), C(278), C(279), C(281), C(282), C(284), C(285), C(287), C(288), C(290), C(291), C(293), C(294), C(296), C(297), C(299), C(300), C(302), C(303), C(305), C(306), C(308), C(309), C(311), C(312), C(314), C(315), C(317), C(318), C(320), C(321), C(323), C(324), C(326), C(327), C(329), C(330), C(332), C(333), C(335), C(336), C(338), C(339), C(341), C(342), C(344), C(345), C(347), C(348), C(350), C(351), C(353), C(354), C(356), C(357), C(359), C(360), C(362), C(363), C(365), C(366), C(368), C(369), C(371), C(372), C(374), C(375), C(377), C(378), C(380), C(381), C(383), C(384), C(386), C(387), C(389), C(390), C(392), C(393), C(395), C(396), C(398), C(399), C(401), C(402), C(404), C(405), C(407), C(410), C(411), C(413), C(414), C(416), C(419), C(420), C(422), C(423), C(425), C(428), C(431), C(434), C(435), C(437), C(438), C(440), C(443), C(444), C(446), C(449), C(450), C(452), C(455), C(458), C(461), C(464), C(467), C(468), C(470), C(471), C(473), C(476), C(479), C(482), C(483), C(485), C(486), C(488), C(491), C(494), C(497), C(498), C(500), C(503), C(506), C(509), C(512), C(515), C(516), C(518), C(519), C(521), C(524), C(527), C(530), C(531), C(533), C(534), C(536), C(539), C(542), C(545), C(546), C(548), C(551), C(554), C(557), C(560), C(563), C(564), C(566), C(567), C(569), C(572), C(575), C(578), C(579), C(581), C(582), C(584), C(587), C(590), C(593), C(594), C(596), C(599), C(602), C(605), C(608), C(611), C(612), C(614), C(615), C(617), C(620), C(623), C(626), C(627), C(629), C(630), C(632), C(635), C(638), C(641), C(642), C(644), C(647), C(650), C(653), C(656), C(659), C(660), C(662), C(663), C(665), C(668), C(671), C(674), C(675), C(677), C(678), C(680), C(683), C(686), C(689), C(690), C(692), C(695), C(698), C(701), C(704), C(707), C(708), C(710), C(711), C(713), C(716), C(719), C(722), C(723), C(725), C(726), C(728), C(731), C(734), C(737), C(738), C(740), C(743), C(746), C(749), C(755), C(758), C(761), C(764)

And the following 224 coded bits are moved to data block PB:

C(16), C(19), C(25), C(28), C(31), C(37), C(40), C(49), C(52), C(55), C(61), C(64), C(67), C(73), C(76), C(79), C(85), C(88), C(97), C(100), C(103), C(109), C(112), C(115), C(121), C(124), C(127), C(133), C(136), C(145), C(148), C(151), C(157), C(160), C(163), C(169), C(172), C(175), C(181), C(184), C(193), C(196), C(199), C(205), C(208), C(211), C(217), C(220), C(223), C(229), C(232), C(241), C(244), C(247), C(253), C(256), C(259), C(265), C(268), C(271), C(274), C(277), C(280), C(283), C(286), C(289), C(295), C(301), C(307), C(310), C(316), C(322), C(328), C(337), C(343), C(349), C(355), C(358), C(364), C(370), C(376), C(385), C(391), C(397), C(403), C(406), C(408), C(412), C(417), C(418), C(424), C(426), C(429), C(432), C(433), C(439), C(441), C(445), C(447), C(451), C(453), C(454), C(456), C(459), C(460), C(462), C(465), C(466), C(472), C(474), C(477), C(480), C(481), C(487), C(489), C(492), C(493), C(495), C(499), C(501), C(502), C(504), C(507), C(508), C(510), C(513), C(514), C(520), C(522), C(525), C(528), C(529), C(535), C(537), C(540), C(541), C(543), C(547), C(549), C(550), C(552), C(555), C(556), C(558), C(561), C(562), C(568), C(570), C(573), C(576), C(577), C(583), C(585), C(588), C(589), C(591), C(595), C(597), C(598), C(600), C(603), C(604), C(606), C(609), C(610), C(616), C(618), C(621), C(624), C(625), C(631), C(633), C(636), C(637), C(639), C(643), C(645), C(646), C(648), C(651), C(652), C(654), C(657), C(658), C(664), C(666), C(669), C(672), C(673), C(679), C(681), C(684), C(685), C(687), C(691), C(693), C(694), C(696), C(699), C(700), C(702), C(705), C(706), C(712), C(714), C(717), C(720), C(721), C(727), C(729), C(732), C(733), C(735), C(739), C(741), C(742), C(744), C(747), C(750), C(752), C(753), C(756), C(759), C(767)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS10.2:

The block of 210 bits {u(0)… u(209)} is encoded with the ¼ rate convolutional code defined by the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 864 coded bits, {C(0)… C(863)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k+3) = u(k)	for k = 0, 1, ..., 209;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k+3)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 210, 211, ..., 215

The following 448 coded bits are moved to data block PG:

C(3), C(7), C(8), C(11), C(12), C(15), C(16), C(19), C(20), C(23), C(24), C(27), C(28), C(31), C(32), C(33), C(35), C(36), C(39), C(40), C(43), C(44), C(45), C(47), C(48), C(51), C(52), C(55), C(56), C(57), C(59), C(60), C(63), C(64), C(67), C(68), C(69), C(71), C(72), C(75), C(76), C(79), C(80), C(81), C(83), C(84), C(87), C(88), C(91), C(92), C(93), C(95), C(96), C(99), C(100), C(103), C(104), C(105), C(107), C(108), C(111), C(112), C(115), C(116), C(117), C(119), C(120), C(123), C(124), C(127), C(128), C(129), C(131), C(132), C(135), C(136), C(139), C(140), C(141), C(143), C(144), C(147), C(148), C(151), C(152), C(153), C(155), C(156), C(159), C(160), C(163), C(164), C(165), C(167), C(168), C(171), C(172), C(175), C(176), C(177), C(179), C(180), C(183), C(184), C(187), C(188), C(189), C(191), C(192), C(195), C(196), C(199), C(200), C(201), C(203), C(204), C(207), C(208), C(211), C(212), C(213), C(215), C(216), C(219), C(220), C(223), C(224), C(225), C(227), C(228), C(231), C(232), C(235), C(236), C(237), C(239), C(240), C(243), C(244), C(247), C(248), C(251), C(252), C(255), C(256), C(259), C(260), C(261), C(263), C(264), C(267), C(268), C(271), C(272), C(275), C(276), C(279), C(280), C(283), C(284), C(285), C(287), C(288), C(291), C(292), C(295), C(296), C(299), C(300), C(303), C(304), C(307), C(308), C(311), C(312), C(315), C(316), C(319), C(320), C(323), C(324), C(327), C(328), C(331), C(332), C(335), C(336), C(339), C(340), C(343), C(344), C(347), C(348), C(351), C(352), C(355), C(356), C(359), C(360), C(363), C(364), C(367), C(368), C(371), C(372), C(375), C(376), C(379), C(380), C(383), C(384), C(387), C(388), C(391), C(392), C(395), C(396), C(399), C(400), C(403), C(404), C(407), C(408), C(411), C(412), C(415), C(416), C(419), C(420), C(423), C(424), C(427), C(428), C(431), C(432), C(435), C(436), C(439), C(440), C(443), C(444), C(447), C(448), C(451), C(452), C(455), C(456), C(459), C(460), C(463), C(464), C(467), C(468), C(471), C(472), C(475), C(476), C(479), C(480), C(483), C(484), C(487), C(488), C(491), C(492), C(495), C(496), C(499), C(500), C(503), C(504), C(507), C(508), C(511), C(512), C(515), C(516), C(519), C(520), C(523), C(524), C(527), C(528), C(531), C(532), C(535), C(536), C(539), C(540), C(543), C(544), C(547), C(548), C(551), C(552), C(555), C(556), C(559), C(560), C(563), C(564), C(567), C(568), C(571), C(572), C(575), C(576), C(579), C(580), C(583), C(584), C(587), C(588), C(591), C(592), C(595), C(596), C(599), C(600), C(603), C(604), C(607), C(608), C(611), C(612), C(615), C(616), C(619), C(620), C(623), C(624), C(627), C(628), C(631), C(632), C(635), C(636), C(639), C(640), C(643), C(644), C(647), C(648), C(651), C(652), C(655), C(656), C(659), C(660), C(663), C(664), C(667), C(668), C(671), C(672), C(675), C(676), C(679), C(680), C(683), C(684), C(687), C(688), C(691), C(692), C(695), C(696), C(699), C(700), C(703), C(704), C(707), C(708), C(711), C(712), C(715), C(716), C(719), C(720), C(723), C(724), C(727), C(728), C(731), C(732), C(735), C(736), C(739), C(740), C(743), C(744), C(747), C(748), C(751), C(752), C(755), C(756), C(759), C(760), C(763), C(764), C(767), C(768), C(771), C(772), C(775), C(776), C(779), C(780), C(783), C(784), C(787), C(788), C(791), C(792), C(795), C(796), C(799), C(800), C(803), C(804), C(807), C(808), C(811), C(812), C(815), C(816), C(819), C(820), C(823), C(824), C(827), C(828), C(831), C(832), C(835), C(836), C(839), C(840), C(843), C(844), C(847), C(848), C(851), C(852), C(855), C(859), C(863)

And the following 224 coded bits are moved to data block PB:

C(4), C(17), C(21), C(25), C(26), C(29), C(34), C(37), C(41), C(42), C(46), C(49), C(50), C(53), C(58), C(61), C(65), C(66), C(70), C(73), C(77), C(82), C(85), C(89), C(90), C(94), C(97), C(101), C(106), C(109),  C(113), C(114), C(118), C(121), C(125), C(130), C(133), C(137), C(138), C(142), C(145), C(149), C(154), C(157), C(161), C(162), C(166), C(169), C(173), C(178), C(181), C(185), C(186), C(190), C(193), C(197), C(202), C(205), C(209), C(210), C(214), C(217), C(221), C(226), C(229), C(233), C(234), C(238), C(241), C(245), C(249), C(250), C(253), C(257), C(258), C(262), C(265), C(269), C(273), C(274), C(277), C(281), C(282), C(286), C(289), C(293), C(297), C(298), C(301), C(305), C(306), C(309), C(310), C(313), C(317), C(321), C(325), C(329), C(333), C(337), C(341), C(345), C(349), C(353), C(357), C(361), C(365), C(369), C(373), C(377), C(381), C(385), C(389), C(393), C(397), C(401), C(405), C(409), C(413), C(417), C(421), C(425), C(429), C(433), C(437), C(441), C(445), C(449), C(453), C(457), C(461), C(465), C(469), C(473), C(477), C(481), C(485), C(489), C(493), C(497), C(501), C(505), C(509), C(513), C(517), C(521), C(525), C(529), C(533), C(537), C(541), C(545), C(549), C(553), C(557), C(561), C(565), C(569), C(573), C(577), C(581), C(585), C(589), C(593), C(597), C(601), C(605), C(609), C(613), C(617), C(621), C(625), C(629), C(633), C(637), C(641), C(645), C(649), C(653), C(657), C(661), C(665), C(669), C(673), C(677), C(681), C(685), C(689), C(693), C(697), C(701), C(705), C(709), C(713), C(717), C(721), C(725), C(729), C(733), C(737), C(741), C(745), C(749), C(753), C(757), C(761), C(765), C(769), C(773), C(777), C(781), C(785), C(789), C(793), C(797), C(801), C(805), C(809), C(813), C(817), C(821), C(825), C(829), C(833)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS7.95:

The block of 165 bits {u(0)… u(164)} is encoded with the ¼ rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 684 coded bits, {C(0)… C(683)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k+3)	= u(k)	for k = 0, 1, ..., 164;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k+3)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 165, 166, ..., 170

The following 448 coded bits are moved to data block PG:

C(3), C(7), C(11), C(15), C(16), C(19), C(20), C(23), C(24), C(27), C(28), C(29), C(31), C(32), C(33), C(35), C(36), C(37), C(39), C(40), C(41), C(43), C(44), C(47), C(48), C(49), C(51), C(52), C(53), C(55), C(56), C(57), C(59), C(60), C(61), C(63), C(64), C(65), C(67), C(68), C(71), C(72), C(73), C(75), C(76), C(77), C(79), C(80), C(81), C(83), C(84), C(85), C(87), C(88), C(89), C(91), C(92), C(95), C(96), C(97), C(99), C(100), C(101), C(103), C(104), C(105), C(107), C(108), C(109), C(111), C(112), C(113), C(115), C(116), C(119), C(120), C(121), C(123), C(124), C(125), C(127), C(128), C(129), C(131), C(132), C(133), C(135), C(136), C(137), C(139), C(140), C(143), C(144), C(145), C(147), C(148), C(149), C(151), C(152), C(153), C(155), C(156), C(157), C(159), C(160), C(161), C(163), C(164), C(167), C(168), C(169), C(171), C(172), C(173), C(175), C(176), C(177), C(179), C(180), C(181), C(183), C(184), C(185), C(187), C(188), C(191), C(192), C(193), C(195), C(196), C(197), C(199), C(200), C(201), C(203), C(204), C(205), C(207), C(208), C(209), C(211), C(212), C(215), C(216), C(217), C(219), C(220), C(221), C(223), C(224), C(225), C(227), C(228), C(229), C(231), C(232), C(233), C(235), C(236), C(239), C(240), C(241), C(243), C(244), C(245), C(247), C(248), C(249), C(251), C(252), C(253), C(255), C(256), C(257), C(259), C(260), C(263), C(264), C(265), C(267), C(268), C(269), C(271), C(272), C(273), C(275), C(276), C(277), C(279), C(280), C(281), C(283), C(284), C(287), C(288), C(289), C(291), C(292), C(293), C(295), C(296), C(297), C(299), C(300), C(301), C(303), C(304), C(305), C(307), C(308), C(311), C(312), C(313), C(315), C(316), C(317), C(319), C(320), C(321), C(323), C(324), C(325), C(327), C(328), C(329), C(331), C(332), C(335), C(336), C(337), C(339), C(340), C(341), C(343), C(344), C(345), C(347), C(348), C(349), C(351), C(352), C(353), C(355), C(356), C(359), C(360), C(361), C(363), C(364), C(365), C(367), C(368), C(369), C(371), C(372), C(373), C(375), C(376), C(377), C(379), C(380), C(383), C(384), C(385), C(387), C(388), C(389), C(391), C(392), C(393), C(395), C(396), C(397), C(399), C(400), C(401), C(403), C(404), C(407), C(408), C(409), C(411), C(412), C(413), C(415), C(416), C(417), C(419), C(420), C(421), C(423), C(424), C(425), C(427), C(428), C(431), C(432), C(433), C(435), C(436), C(437), C(439), C(440), C(441), C(443), C(444), C(445), C(447), C(448), C(449), C(451), C(452), C(455), C(456), C(457), C(459), C(460), C(461), C(463), C(464), C(465), C(467), C(468), C(469), C(471), C(472), C(473), C(475), C(476), C(479), C(480), C(481), C(483), C(484), C(487), C(488), C(489), C(491), C(492), C(495), C(496), C(497), C(499), C(500), C(503), C(504), C(505), C(507), C(508), C(511), C(512), C(513), C(515), C(516), C(519), C(520), C(521), C(523), C(524), C(527), C(528), C(529), C(531), C(532), C(535), C(536), C(537), C(539), C(540), C(543), C(544), C(545), C(547), C(548), C(551), C(552), C(553), C(555), C(556), C(559), C(560), C(561), C(563), C(564), C(567), C(568), C(569), C(571), C(572), C(575), C(576), C(577), C(579), C(580), C(583), C(584), C(585), C(587), C(588), C(591), C(592), C(593), C(595), C(596), C(599), C(600), C(601), C(603), C(604), C(607), C(608), C(609), C(611), C(612), C(615), C(616), C(617), C(619), C(620), C(623), C(624), C(625), C(627), C(628), C(631), C(632), C(633), C(635), C(636), C(639), C(640), C(641), C(643), C(644), C(647), C(648), C(651), C(655), C(656), C(659), C(660), C(663), C(664), C(667), C(671), C(675), C(679), C(683)

And the following 224 coded bits are moved to data block PB:

C(0), C(4), C(8), C(9), C(10), C(12), C(13), C(14), C(17), C(18), C(21), C(22), C(25), C(26), C(30), C(34), C(38), C(42), C(45), C(46), C(50), C(54), C(58), C(62), C(66), C(69), C(70), C(74), C(78), C(82), C(86), C(90), C(93), C(94), C(98), C(102), C(106), C(110), C(114), C(117), C(118), C(122), C(126), C(130), C(134), C(138), C(141), C(142), C(146), C(150), C(154), C(158), C(162), C(165), C(166), C(170), C(174), C(178), C(182), C(186), C(189), C(190), C(194), C(198), C(202), C(206), C(210), C(213), C(214), C(218), C(222), C(226), C(230), C(234), C(237), C(238), C(242), C(246), C(250), C(254), C(258), C(261), C(262), C(266), C(270), C(274), C(278), C(282), C(285), C(286), C(290), C(294), C(298), C(302), C(306), C(309), C(310), C(314), C(318), C(322), C(326), C(330), C(333), C(334), C(338), C(342), C(346), C(350), C(354), C(357), C(358), C(362), C(366), C(370), C(374), C(378), C(381), C(382), C(386), C(390), C(394), C(398), C(402), C(405), C(406), C(410), C(414), C(418), C(422), C(426), C(429), C(430), C(434), C(438), C(442), C(446), C(450), C(453), C(454), C(458), C(462), C(466), C(470), C(474), C(477), C(478), C(482), C(485), C(486), C(490), C(493), C(494), C(498), C(501), C(502), C(506), C(509), C(510), C(514), C(517), C(518), C(522), C(525), C(526), C(530), C(533), C(534), C(538), C(541), C(542), C(546), C(549), C(550), C(554), C(557), C(558), C(562), C(565), C(566), C(570), C(573), C(574), C(578), C(581), C(582), C(586), C(589), C(590), C(594), C(597), C(598), C(602), C(605), C(606), C(610), C(613), C(614), C(618), C(621), C(622), C(626), C(629), C(630), C(634), C(637), C(638), C(642), C(645), C(646), C(649), C(650), C(652), C(653), C(654), C(657), C(658), C(661), C(662), C(665), C(666), C(668), C(669), C(670), C(672)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS7.4:

The block of 154 bits {u(0)… u(153)} is encoded with the 1/5 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 800 coded bits, {C(0)… C(799)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= u(k)	for k = 0, 1, ..., 153;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 154, 155, ..., 159

The following 448 coded bits are moved to data block PG:

C(4), C(9), C(14), C(17), C(19), C(22), C(24), C(27), C(29), C(32), C(34), C(37), C(39), C(42), C(43), C(44), C(47), C(48), C(49), C(52), C(53), C(54), C(57), C(58), C(59), C(62), C(63), C(64), C(67), C(68), C(69), C(72), C(73), C(74), C(77), C(78), C(79), C(82), C(83), C(84), C(87), C(88), C(89), C(92), C(93), C(94), C(97), C(98), C(99), C(102), C(103), C(104), C(107), C(108), C(109), C(112), C(113), C(114), C(117), C(118), C(119), C(122), C(123), C(124), C(127), C(128), C(129), C(132), C(133), C(134), C(137), C(138), C(139), C(142), C(143), C(144), C(147), C(148), C(149), C(152), C(153), C(154), C(157), C(158), C(159), C(162), C(163), C(164), C(167), C(168), C(169), C(172), C(173), C(174), C(177), C(178), C(179), C(182), C(183), C(184), C(187), C(188), C(189), C(192), C(193), C(194), C(197), C(198), C(199), C(202), C(203), C(204), C(207), C(208), C(209), C(212), C(213), C(214), C(217), C(218), C(219), C(222), C(223), C(224), C(227), C(228), C(229), C(232), C(233), C(234), C(237), C(238), C(239), C(242), C(243), C(244), C(247), C(248), C(249), C(252), C(253), C(254), C(257), C(258), C(259), C(262), C(263), C(264), C(267), C(268), C(269), C(272), C(273), C(274), C(277), C(278), C(279), C(282), C(283), C(284), C(287), C(288), C(289), C(292), C(293), C(294), C(297), C(298), C(299), C(302), C(303), C(304), C(307), C(308), C(309), C(312), C(313), C(314), C(317), C(318), C(319), C(322), C(323), C(324), C(327), C(328), C(329), C(332), C(333), C(334), C(337), C(338), C(339), C(342), C(343), C(344), C(347), C(348), C(349), C(352), C(353), C(354), C(357), C(358), C(359), C(362), C(363), C(364), C(367), C(368), C(369), C(372), C(373), C(374), C(377), C(378), C(379), C(382), C(383), C(384), C(387), C(389), C(392), C(393), C(394), C(397), C(398), C(399), C(402), C(403), C(404), C(407), C(408), C(409), C(412), C(413), C(414), C(417), C(418), C(419), C(422), C(423), C(424), C(427), C(428), C(429), C(432), C(433), C(434), C(437), C(439), C(442), C(443), C(444), C(447), C(448), C(449), C(452), C(453), C(454), C(457), C(458), C(459), C(462), C(463), C(464), C(467), C(468), C(469), C(472), C(473), C(474), C(477), C(478), C(479), C(482), C(483), C(484), C(487), C(489), C(492), C(493), C(494), C(497), C(498), C(499), C(502), C(503), C(504), C(507), C(508), C(509), C(512), C(513), C(514), C(517), C(518), C(519), C(522), C(523), C(524), C(527), C(528), C(529), C(532), C(533), C(534), C(537), C(539), C(542), C(543), C(544), C(547), C(548), C(549), C(552), C(553), C(554), C(557), C(558), C(559), C(562), C(563), C(564), C(567), C(568), C(569), C(572), C(573), C(574), C(577), C(578), C(579), C(582), C(583), C(584), C(587), C(589), C(592), C(593), C(594), C(597), C(598), C(599), C(602), C(603), C(604), C(607), C(608), C(609), C(612), C(613), C(614), C(617), C(618), C(619), C(622), C(623), C(624), C(627), C(628), C(629), C(632), C(633), C(634), C(637), C(639), C(642), C(643), C(644), C(647), C(648), C(649), C(652), C(653), C(654), C(657), C(658), C(659), C(662), C(663), C(664), C(667), C(668), C(669), C(672), C(673), C(674), C(677), C(678), C(679), C(682), C(683), C(684), C(687), C(689), C(692), C(693), C(694), C(697), C(698), C(699), C(702), C(703), C(704), C(707), C(708), C(709), C(712), C(713), C(714), C(717), C(718), C(719), C(722), C(723), C(724), C(728), C(729), C(733), C(734), C(737), C(739), C(742), C(743), C(744), C(747), C(748), C(749), C(752), C(753), C(754), C(758), C(759), C(763), C(764), C(767), C(768), C(769), C(773), C(774), C(778), C(779), C(783), C(784), C(789), C(794), C(799)

And the following 224 coded bits are moved to data block PB:

C(12), C(13), C(16), C(18), C(21), C(23), C(26), C(28), C(31), C(33), C(36), C(38), C(41), C(45), C(46), C(50), C(51), C(55), C(56), C(60), C(61), C(65), C(66), C(71), C(75), C(76), C(80), C(81), C(85), C(86), C(90), C(91), C(95), C(96), C(100), C(101), C(105), C(106), C(110), C(111), C(115), C(116), C(121), C(125), C(126), C(130), C(131), C(135), C(136), C(140), C(141), C(145), C(146), C(150), C(151), C(155), C(156), C(160), C(161), C(165), C(166), C(171), C(175), C(176), C(180), C(181), C(185), C(186), C(190), C(191), C(195), C(196), C(200), C(201), C(205), C(206), C(210), C(211), C(215), C(216), C(221), C(225), C(226), C(230), C(231), C(235), C(236), C(240), C(241), C(245), C(246), C(250), C(251), C(255), C(256), C(260), C(261), C(265), C(266), C(271), C(275), C(276), C(280), C(281), C(285), C(286), C(290), C(291), C(296), C(300), C(301), C(305), C(306), C(310), C(311), C(315), C(316), C(321), C(325), C(326), C(330), C(331), C(335), C(336), C(340), C(341), C(346), C(350), C(351), C(355), C(356), C(360), C(361), C(365), C(366), C(371), C(375), C(376), C(380), C(381), C(385), C(386), C(388), C(390), C(391), C(396), C(400), C(401), C(405), C(406), C(410), C(411), C(415), C(416), C(421), C(425), C(426), C(430), C(431), C(435), C(436), C(438), C(440), C(441), C(446), C(450), C(451), C(455), C(456), C(460), C(461), C(465), C(466), C(471), C(475), C(476), C(480), C(481), C(485), C(486), C(488), C(491), C(496), C(500), C(501), C(505), C(506), C(510), C(511), C(516), C(521), C(536), C(538), C(546), C(561), C(571), C(586), C(588), C(596), C(611), C(621), C(636), C(638), C(646), C(661), C(671), C(686), C(688), C(696), C(711), C(721), C(727), C(732), C(736), C(738), C(746), C(757), C(762), C(772), C(777), C(782), C(787), C(788), C(793)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS6.7:

The block of 140 bits {u(0)… u(139)} is encoded with the 1/5 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 730 coded bits, {C(0)… C(729)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= u(k)	for k = 0, 1, ..., 139;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 140, 141, ..., 145

The following 448 coded bits are moved to data block PG:

C(4), C(9), C(12), C(13), C(14), C(17), C(18), C(19), C(21), C(22), C(23), C(24), C(27), C(28), C(29), C(32), C(33), C(34), C(36), C(37), C(38), C(39), C(42), C(43), C(44), C(46), C(47), C(48), C(49), C(52), C(53), C(54), C(57), C(58), C(59), C(61), C(62), C(63), C(64), C(67), C(68), C(69), C(71), C(72), C(73), C(74), C(77), C(78), C(79), C(82), C(83), C(84), C(86), C(87), C(88), C(89), C(92), C(93), C(94), C(96), C(97), C(98), C(99), C(102), C(103), C(104), C(107), C(108), C(109), C(111), C(112), C(113), C(114), C(117), C(118), C(119), C(121), C(122), C(123), C(124), C(127), C(128), C(129), C(132), C(133), C(134), C(136), C(137), C(138), C(139), C(142), C(143), C(144), C(146), C(147), C(148), C(149), C(152), C(153), C(154), C(157), C(158), C(159), C(161), C(162), C(163), C(164), C(167), C(168), C(169), C(171), C(172), C(173), C(174), C(177), C(178), C(179), C(182), C(183), C(184), C(186), C(187), C(188), C(189), C(192), C(193), C(194), C(196), C(197), C(198), C(199), C(202), C(203), C(204), C(207), C(208), C(209), C(211), C(212), C(213), C(214), C(217), C(218), C(219), C(221), C(222), C(223), C(224), C(227), C(228), C(229), C(232), C(233), C(234), C(236), C(237), C(238), C(239), C(242), C(243), C(244), C(246), C(247), C(248), C(249), C(252), C(253), C(254), C(257), C(258), C(259), C(261), C(262), C(263), C(264), C(267), C(268), C(269), C(271), C(272), C(273), C(274), C(277), C(278), C(279), C(282), C(283), C(284), C(286), C(287), C(288), C(289), C(292), C(293), C(294), C(296), C(297), C(298), C(299), C(302), C(303), C(304), C(307), C(308), C(309), C(311), C(312), C(313), C(314), C(317), C(318), C(319), C(321), C(322), C(323), C(324), C(327), C(328), C(329), C(332), C(333), C(334), C(336), C(337), C(338), C(339), C(342), C(343), C(344), C(347), C(348), C(349), C(352), C(353), C(354), C(357), C(358), C(359), C(362), C(363), C(364), C(367), C(368), C(369), C(372), C(373), C(374), C(377), C(378), C(379), C(382), C(383), C(384), C(387), C(388), C(389), C(392), C(393), C(394), C(397), C(398), C(399), C(402), C(403), C(404), C(407), C(408), C(409), C(412), C(413), C(414), C(417), C(418), C(419), C(422), C(423), C(424), C(427), C(428), C(429), C(432), C(433), C(434), C(437), C(438), C(439), C(442), C(443), C(444), C(447), C(448), C(449), C(452), C(453), C(454), C(457), C(458), C(459), C(462), C(463), C(464), C(467), C(468), C(469), C(472), C(473), C(474), C(477), C(478), C(479), C(482), C(483), C(484), C(487), C(488), C(489), C(492), C(493), C(494), C(498), C(499), C(502), C(503), C(504), C(507), C(508), C(509), C(512), C(513), C(514), C(517), C(518), C(519), C(523), C(524), C(527), C(528), C(529), C(532), C(533), C(534), C(537), C(538), C(539), C(542), C(543), C(544), C(548), C(549), C(552), C(553), C(554), C(557), C(558), C(559), C(562), C(563), C(564), C(567), C(568), C(569), C(573), C(574), C(577), C(578), C(579), C(582), C(583), C(584), C(587), C(588), C(589), C(592), C(593), C(594), C(598), C(599), C(602), C(603), C(604), C(607), C(608), C(609), C(612), C(613), C(614), C(617), C(618), C(619), C(623), C(624), C(627), C(628), C(629), C(632), C(633), C(634), C(637), C(638), C(639), C(642), C(643), C(644), C(648), C(649), C(652), C(653), C(654), C(657), C(658), C(659), C(662), C(663), C(664), C(667), C(668), C(669), C(673), C(674), C(677), C(678), C(679), C(682), C(683), C(684), C(687), C(688), C(689), C(692), C(693), C(694), C(698), C(699), C(702), C(703), C(704), C(707), C(708), C(709), C(712), C(713), C(714), C(718), C(719), C(723), C(724), C(728), C(729)

And the following 224 coded bits are moved to data block PB:

C(16), C(25), C(26), C(30), C(31), C(35), C(40), C(41), C(45), C(50), C(51), C(55), C(56), C(60), C(65), C(66), C(70), C(75), C(76), C(80), C(81), C(85), C(90), C(91), C(95), C(100), C(101), C(105), C(106), C(110), C(115), C(116), C(120), C(125), C(126), C(130), C(131), C(135), C(140), C(141), C(145), C(150), C(151), C(155), C(156), C(160), C(165), C(166), C(170), C(175), C(176), C(180), C(181), C(185), C(190), C(191), C(195), C(200), C(201), C(205), C(206), C(210), C(215), C(216), C(220), C(225), C(226), C(230), C(231), C(235), C(240), C(241), C(245), C(250), C(251), C(255), C(256), C(260), C(265), C(266), C(270), C(275), C(276), C(280), C(281), C(285), C(290), C(291), C(295), C(300), C(301), C(305), C(306), C(310), C(315), C(316), C(320), C(325), C(326), C(330), C(331), C(335), C(340), C(341), C(345), C(346), C(350), C(351), C(355), C(356), C(360), C(361), C(365), C(366), C(370), C(371), C(375), C(376), C(380), C(381), C(385), C(386), C(390), C(391), C(395), C(396), C(400), C(401), C(405), C(406), C(410), C(411), C(415), C(416), C(420), C(421), C(425), C(426), C(430), C(431), C(435), C(436), C(440), C(441), C(445), C(446), C(450), C(451), C(455), C(456), C(460), C(461), C(465), C(466), C(470), C(471), C(475), C(476), C(480), C(481), C(485), C(486), C(490), C(491), C(495), C(496), C(497), C(501), C(505), C(506), C(511), C(515), C(516), C(521), C(522), C(526), C(530), C(531), C(536), C(540), C(541), C(546), C(547), C(551), C(555), C(556), C(561), C(565), C(566), C(571), C(572), C(576), C(580), C(581), C(586), C(590), C(591), C(596), C(597), C(601), C(605), C(606), C(611), C(615), C(616), C(621), C(622), C(626), C(630), C(631), C(636), C(640), C(641), C(646), C(647), C(651), C(656), C(661), C(666), C(671), C(672), C(676), C(681), C(697)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS5.9:

The block of 124 bits {u(0)… u(123)} is encoded with the 1/6 rate convolutional code defined by	the following polynomials:

G4/G6 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D4 + D6

G4/G6 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D4 + D6

G5/G6 = 1 + D + D4 + D6/1 + D + D2 + D3 + D4 + D6

G6/G6 = 1

G6/G6 = 1

G7/G6 = 1 + D + D2 + D3 + D6/1 + D + D2 + D3 + D4 + D6

resulting in 780 coded bits, {C(0)… C(779)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(6k+3)	= u(k)

C(6k+4)	= u(k)

C(6k+5)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6) for k = 0, 1, ..., 123; r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(6k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(6k+3)	= r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+4)	= r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+5)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)   for k = 124, 125, ..., 129

The following 448 coded bits are moved to data block PG:

C(4), C(9), C(10), C(13), C(14), C(15), C(16), C(19), C(20), C(21), C(22), C(25), C(26), C(27), C(28), C(31), C(32), C(33), C(34), C(37), C(38), C(39), C(40), C(43), C(44), C(45), C(46), C(49), C(50), C(51), C(52), C(55), C(56), C(57), C(58), C(61), C(62), C(63), C(64), C(67), C(68), C(69), C(70), C(73), C(74), C(75), C(76), C(79), C(80), C(81), C(82), C(85), C(86), C(87), C(88), C(91), C(92), C(93), C(94), C(97), C(98), C(99), C(100), C(103), C(104), C(105), C(106), C(109), C(110), C(111), C(112), C(115), C(116), C(117), C(118), C(121), C(122), C(123), C(124), C(127), C(128), C(129), C(130), C(133), C(134), C(135), C(136), C(139), C(140), C(141), C(142), C(145), C(146), C(147), C(148), C(151), C(152), C(153), C(154), C(157), C(158), C(159), C(160), C(163), C(164), C(165), C(166), C(169), C(170), C(171), C(172), C(175), C(176), C(177), C(178), C(181), C(182), C(183), C(184), C(187), C(188), C(189), C(190), C(193), C(194), C(195), C(196), C(199), C(200), C(201), C(202), C(205), C(206), C(207), C(208), C(211), C(212), C(213), C(214), C(217), C(218), C(219), C(220), C(223), C(224), C(225), C(226), C(229), C(230), C(231), C(232), C(235), C(236), C(237), C(238), C(241), C(242), C(243), C(244), C(247), C(248), C(249), C(250), C(253), C(254), C(255), C(256), C(259), C(260), C(261), C(262), C(265), C(266), C(267), C(268), C(271), C(272), C(273), C(274), C(277), C(278), C(279), C(280), C(283), C(284), C(285), C(286), C(289), C(290), C(291), C(292), C(295), C(296), C(297), C(298), C(301), C(302), C(303), C(304), C(307), C(308), C(309), C(310), C(313), C(314), C(315), C(316), C(319), C(320), C(321), C(322), C(325), C(326), C(327), C(328), C(331), C(332), C(333), C(334), C(337), C(338), C(339), C(340), C(343), C(344), C(345), C(346), C(349), C(350), C(351), C(352), C(355), C(356), C(357), C(358), C(361), C(362), C(363), C(364), C(367), C(368), C(369), C(370), C(373), C(374), C(375), C(376), C(379), C(380), C(381), C(382), C(385), C(386), C(387), C(388), C(391), C(392), C(393), C(394), C(397), C(398), C(399), C(400), C(403), C(404), C(405), C(406), C(409), C(410), C(411), C(412), C(415), C(416), C(417), C(418), C(421), C(422), C(423), C(424), C(427), C(428), C(429), C(430), C(434), C(435), C(436), C(439), C(440), C(441), C(442), C(446), C(447), C(448), C(451), C(452), C(453), C(454), C(458), C(459), C(460), C(463), C(464), C(465), C(466), C(470), C(471), C(472), C(475), C(476), C(477), C(478), C(482), C(483), C(484), C(487), C(488), C(489), C(490), C(494), C(495), C(496), C(499), C(500), C(501), C(502), C(506), C(507), C(508), C(511), C(513), C(514), C(518), C(519), C(520), C(523), C(525), C(526), C(530), C(531), C(532), C(535), C(537), C(538), C(542), C(543), C(544), C(547), C(549), C(550), C(554), C(555), C(556), C(559), C(561), C(562), C(566), C(567), C(568), C(571), C(573), C(574), C(578), C(579), C(580), C(583), C(585), C(586), C(590), C(591), C(592), C(595), C(597), C(598), C(602), C(603), C(604), C(607), C(609), C(610), C(614), C(615), C(616), C(619), C(621), C(622), C(626), C(627), C(628), C(631), C(633), C(634), C(638), C(639), C(640), C(643), C(645), C(646), C(650), C(651), C(652), C(655), C(657), C(658), C(662), C(663), C(664), C(667), C(669), C(670), C(674), C(675), C(676), C(679), C(681), C(682), C(686), C(687), C(688), C(691), C(693), C(694), C(698), C(699), C(700), C(703), C(705), C(706), C(710), C(711), C(712), C(717), C(718), C(722), C(723), C(724), C(729), C(730), C(736), C(741), C(742), C(748), C(753), C(754), C(760), C(765), C(766), C(777), C(778)

And the following 224 coded bits are moved to data block PB:

C(3), C(7), C(8), C(12), C(18), C(23), C(24), C(30), C(35), C(36), C(42), C(47), C(48), C(54), C(59), C(60), C(66), C(71), C(72), C(78), C(83), C(84), C(90), C(95), C(96), C(102), C(107), C(108), C(114), C(119), C(120), C(126), C(131), C(132), C(138), C(143), C(144), C(150), C(155), C(156), C(162), C(167), C(168), C(174), C(179), C(180), C(186), C(191), C(192), C(198), C(203), C(204), C(210), C(215), C(216), C(222), C(227), C(228), C(234), C(239), C(240), C(246), C(251), C(252), C(258), C(263), C(264), C(270), C(275), C(276), C(282), C(287), C(288), C(294), C(299), C(300), C(306), C(311), C(312), C(318), C(323), C(324), C(330), C(335), C(336), C(342), C(347), C(348), C(354), C(359), C(360), C(366), C(371), C(372), C(378), C(383), C(384), C(390), C(395), C(396), C(402), C(407), C(408), C(414), C(419), C(420), C(426), C(431), C(432), C(433), C(438), C(443), C(444), C(445), C(450), C(455), C(456), C(457), C(462), C(467), C(468), C(469), C(474), C(479), C(480), C(481), C(486), C(491), C(492), C(493), C(498), C(503), C(504), C(505), C(512), C(515), C(516), C(517), C(524), C(527), C(528), C(529), C(536), C(539), C(540), C(541), C(548), C(551), C(552), C(553), C(560), C(563), C(564), C(565), C(572), C(575), C(576), C(577), C(584), C(587), C(588), C(589), C(596), C(599), C(600), C(601), C(608), C(611), C(612), C(613), C(620), C(623), C(624), C(625), C(632), C(635), C(636), C(637), C(644), C(647), C(648), C(649), C(656), C(659), C(660), C(661), C(668), C(671), C(672), C(673), C(680), C(683), C(684), C(685), C(692), C(695), C(696), C(697), C(704), C(707), C(708), C(709), C(715), C(716), C(720), C(721), C(727), C(728), C(734), C(735), C(739), C(740), C(746), C(747), C(751), C(752), C(758), C(759), C(763), C(764), C(770), C(771), C(772), C(775)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS5.15:

The block of 109 bits {u(0)… u(108)} is encoded with the 1/6 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 690 coded bits, {C(0)… C(689)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(6k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(6k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+5)	= u(k)	for k = 0, 1, ..., 108;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(6k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(6k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+5)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 109, 110, ..., 114

The following 448 coded bits are moved to data block PG:

C(5), C(8), C(9), C(11), C(14), C(15), C(17), C(20), C(21), C(23), C(26), C(27), C(29), C(30), C(32), C(33), C(35), C(36), C(38), C(39), C(41), C(42), C(44), C(45), C(47), C(48), C(50), C(51), C(53), C(54), C(56), C(57), C(59), C(60), C(61), C(62), C(63), C(65), C(66), C(68), C(69), C(71), C(72), C(73), C(74), C(75), C(77), C(78), C(80), C(81), C(83), C(84), C(86), C(87), C(89), C(90), C(92), C(93), C(95), C(96), C(98), C(99), C(101), C(102), C(104), C(105), C(107), C(108), C(109), C(110), C(111), C(113), C(114), C(116), C(117), C(119), C(120), C(121), C(122), C(123), C(125), C(126), C(128), C(129), C(131), C(132), C(134), C(135), C(137), C(138), C(140), C(141), C(143), C(144), C(146), C(147), C(149), C(150), C(152), C(153), C(155), C(156), C(157), C(158), C(159), C(161), C(162), C(164), C(165), C(167), C(168), C(169), C(170), C(171), C(173), C(174), C(176), C(177), C(179), C(180), C(182), C(183), C(185), C(186), C(188), C(189), C(191), C(192), C(194), C(195), C(197), C(198), C(200), C(201), C(203), C(204), C(205), C(206), C(207), C(209), C(210), C(212), C(213), C(215), C(216), C(217), C(218), C(219), C(221), C(222), C(224), C(225), C(227), C(228), C(230), C(231), C(233), C(234), C(236), C(237), C(239), C(240), C(242), C(243), C(245), C(246), C(248), C(249), C(251), C(252), C(253), C(254), C(255), C(257), C(258), C(260), C(261), C(263), C(264), C(265), C(266), C(267), C(269), C(270), C(272), C(273), C(275), C(276), C(278), C(279), C(281), C(282), C(284), C(285), C(287), C(288), C(290), C(291), C(293), C(294), C(296), C(297), C(299), C(300), C(301), C(302), C(303), C(305), C(306), C(308), C(309), C(311), C(312), C(313), C(314), C(315), C(317), C(318), C(320), C(321), C(323), C(324), C(326), C(327), C(329), C(330), C(332), C(333), C(335), C(336), C(338), C(339), C(341), C(342), C(344), C(345), C(347), C(348), C(349), C(350), C(351), C(353), C(354), C(356), C(357), C(359), C(360), C(361), C(362), C(363), C(365), C(366), C(368), C(369), C(371), C(372), C(374), C(375), C(377), C(378), C(380), C(381), C(383), C(384), C(386), C(387), C(389), C(390), C(392), C(393), C(395), C(396), C(397), C(398), C(399), C(401), C(402), C(404), C(405), C(407), C(408), C(409), C(410), C(411), C(413), C(414), C(416), C(417), C(419), C(420), C(422), C(423), C(425), C(426), C(428), C(429), C(431), C(432), C(434), C(435), C(437), C(438), C(440), C(441), C(443), C(444), C(446), C(447), C(449), C(450), C(452), C(453), C(455), C(456), C(458), C(459), C(461), C(462), C(464), C(465), C(467), C(468), C(470), C(471), C(473), C(474), C(476), C(477), C(479), C(480), C(482), C(483), C(485), C(486), C(488), C(489), C(491), C(494), C(495), C(497), C(498), C(500), C(501), C(503), C(504), C(506), C(507), C(509), C(510), C(512), C(513), C(515), C(518), C(519), C(521), C(522), C(524), C(525), C(527), C(528), C(530), C(531), C(533), C(534), C(536), C(537), C(539), C(542), C(543), C(545), C(546), C(548), C(549), C(551), C(552), C(554), C(555), C(557), C(558), C(560), C(561), C(563), C(566), C(567), C(569), C(570), C(572), C(573), C(575), C(576), C(578), C(579), C(581), C(582), C(584), C(585), C(587), C(590), C(591), C(593), C(594), C(596), C(597), C(599), C(600), C(602), C(603), C(605), C(606), C(608), C(609), C(611), C(614), C(615), C(617), C(618), C(620), C(621), C(623), C(624), C(626), C(627), C(629), C(630), C(632), C(633), C(635), C(638), C(639), C(641), C(642), C(644), C(645), C(647), C(648), C(650), C(651), C(653), C(654), C(656), C(657), C(662), C(663), C(666), C(668), C(669), C(671), C(675)

And the following 224 coded bits are moved to data block PB:

C(2), C(3), C(13), C(24), C(28), C(31), C(34), C(37), C(40), C(43), C(46), C(49), C(52), C(55), C(58), C(64), C(67), C(70), C(76), C(79), C(82), C(85), C(88), C(91), C(94), C(97), C(100), C(103), C(106), C(112), C(115), C(118), C(124), C(127), C(130), C(133), C(136), C(139), C(142), C(145), C(148), C(151), C(154), C(160), C(163), C(166), C(172), C(175), C(178), C(181), C(184), C(187), C(190), C(193), C(196), C(199), C(202), C(208), C(211), C(214), C(220), C(223), C(226), C(229), C(232), C(235), C(238), C(241), C(244), C(247), C(250), C(256), C(259), C(262), C(268), C(271), C(274), C(277), C(280), C(283), C(286), C(289), C(292), C(295), C(298), C(304), C(307), C(310), C(316), C(319), C(322), C(325), C(328), C(331), C(334), C(337), C(340), C(343), C(346), C(352), C(355), C(358), C(364), C(367), C(370), C(373), C(376), C(379), C(382), C(385), C(388), C(391), C(394), C(400), C(403), C(406), C(412), C(415), C(418), C(421), C(424), C(427), C(430), C(433), C(436), C(439), C(442), C(445), C(448), C(451), C(454), C(457), C(460), C(463), C(466), C(469), C(472), C(475), C(478), C(481), C(484), C(487), C(490), C(492), C(493), C(496), C(499), C(502), C(505), C(508), C(511), C(514), C(516), C(517), C(520), C(523), C(526), C(529), C(532), C(535), C(538), C(540), C(541), C(544), C(547), C(550), C(553), C(556), C(559), C(562), C(564), C(565), C(568), C(571), C(574), C(577), C(580), C(583), C(586), C(588), C(589), C(592), C(595), C(598), C(601), C(604), C(607), C(610), C(612), C(613), C(616), C(619), C(622), C(625), C(628), C(631), C(634), C(636), C(637), C(640), C(643), C(646), C(649), C(652), C(655), C(658), C(659), C(660), C(661), C(664), C(665), C(667), C(670), C(672), C(673), C(676), C(677), C(681), C(682), C(683), C(686), C(687), C(688), C(689)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/AHS4.75:

The block of 101 bits {u(0)… u(100)} is encoded with the 1/7 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 749 coded bits, {C(0)… C(748)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(7k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(7k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(7k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(7k+3)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(7k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(7k+5)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(7k+6)	= u(k)	for k = 0, 1, ..., 100;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(7k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(7k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(7k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(7k+3)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(7k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(7k+5)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(7k+6)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 101, 102, ..., 106

The following 448 coded bits are moved to data block PG:

C(6), C(13), C(15), C(18), C(20), C(22), C(25), C(27), C(30), C(32), C(34), C(36), C(37), C(39), C(41), C(42), C(43), C(44), C(46), C(48), C(49), C(51), C(53), C(55), C(56), C(57), C(58), C(60), C(62), C(63), C(64), C(65), C(67), C(69), C(70), C(72), C(74), C(76), C(77), C(78), C(79), C(81), C(83), C(84), C(85), C(86), C(88), C(90), C(91), C(93), C(95), C(97), C(98), C(99), C(100), C(102), C(104), C(105), C(106), C(107), C(109), C(111), C(112), C(114), C(116), C(118), C(119), C(120), C(121), C(123), C(125), C(126), C(127), C(128), C(130), C(132), C(133), C(135), C(137), C(139), C(140), C(141), C(142), C(144), C(146), C(147), C(148), C(149), C(151), C(153), C(154), C(156), C(158), C(160), C(161), C(162), C(163), C(165), C(167), C(168), C(169), C(170), C(172), C(174), C(175), C(177), C(179), C(181), C(182), C(183), C(184), C(186), C(188), C(189), C(190), C(191), C(193), C(195), C(196), C(198), C(200), C(202), C(203), C(204), C(205), C(207), C(209), C(210), C(211), C(212), C(214), C(216), C(217), C(219), C(221), C(223), C(224), C(225), C(226), C(228), C(230), C(231), C(232), C(233), C(235), C(237), C(238), C(240), C(242), C(244), C(245), C(246), C(247), C(249), C(251), C(252), C(253), C(254), C(256), C(258), C(259), C(261), C(263), C(265), C(266), C(267), C(268), C(270), C(272), C(273), C(274), C(275), C(277), C(279), C(280), C(282), C(284), C(286), C(287), C(288), C(289), C(291), C(293), C(294), C(295), C(296), C(298), C(300), C(301), C(303), C(305), C(307), C(308), C(309), C(310), C(312), C(314), C(315), C(316), C(317), C(319), C(321), C(322), C(324), C(326), C(328), C(329), C(330), C(331), C(333), C(335), C(336), C(337), C(338), C(340), C(342), C(343), C(345), C(347), C(349), C(350), C(351), C(352), C(354), C(356), C(357), C(358), C(359), C(361), C(363), C(364), C(366), C(368), C(370), C(371), C(372), C(373), C(375), C(377), C(378), C(379), C(380), C(382), C(384), C(385), C(387), C(389), C(391), C(392), C(393), C(394), C(396), C(398), C(399), C(400), C(401), C(403), C(405), C(406), C(408), C(410), C(412), C(413), C(414), C(415), C(417), C(419), C(420), C(421), C(422), C(424), C(426), C(427), C(429), C(431), C(433), C(434), C(436), C(438), C(440), C(441), C(443), C(445), C(447), C(448), C(450), C(452), C(454), C(455), C(456), C(457), C(459), C(461), C(462), C(463), C(464), C(466), C(468), C(469), C(471), C(473), C(475), C(476), C(478), C(480), C(482), C(483), C(485), C(487), C(489), C(490), C(492), C(494), C(496), C(497), C(498), C(499), C(501), C(503), C(504), C(505), C(506), C(508), C(510), C(511), C(513), C(515), C(517), C(518), C(520), C(522), C(524), C(525), C(527), C(529), C(531), C(532), C(534), C(536), C(538), C(539), C(540), C(541), C(543), C(545), C(546), C(547), C(548), C(550), C(552), C(553), C(555), C(557), C(559), C(560), C(562), C(564), C(566), C(567), C(569), C(571), C(573), C(574), C(576), C(578), C(580), C(581), C(582), C(583), C(585), C(587), C(588), C(590), C(592), C(594), C(595), C(597), C(599), C(601), C(602), C(604), C(606), C(608), C(609), C(611), C(613), C(615), C(616), C(618), C(620), C(622), C(623), C(625), C(627), C(629), C(630), C(632), C(634), C(636), C(637), C(639), C(641), C(643), C(644), C(646), C(648), C(650), C(653), C(655), C(657), C(658), C(660), C(662), C(664), C(667), C(669), C(671), C(674), C(676), C(678), C(679), C(681), C(683), C(685), C(688), C(690), C(692), C(695), C(697), C(699), C(700), C(702), C(704), C(706), C(709), C(711), C(713), C(716), C(718), C(720), C(721), C(723), C(725), C(727), C(730), C(732), C(734), C(742)

And the following 224 coded bits are moved to data block PB:

C(1), C(4), C(8), C(9), C(11), C(24), C(26), C(29), C(31), C(33), C(40), C(45), C(47), C(50), C(52), C(54), C(61), C(68), C(71), C(73), C(75), C(80), C(82), C(87), C(89), C(92), C(94), C(96), C(103), C(110), C(113), C(115), C(117), C(122), C(124), C(129), C(131), C(134), C(136), C(138), C(145), C(152), C(155), C(157), C(159), C(164), C(166), C(171), C(173), C(176), C(178), C(180), C(187), C(194), C(197), C(199), C(201), C(206), C(208), C(213), C(215), C(218), C(220), C(222), C(229), C(236), C(239), C(241), C(243), C(248), C(250), C(255), C(257), C(260), C(262), C(264), C(271), C(278), C(281), C(283), C(285), C(292), C(299), C(302), C(304), C(306), C(313), C(320), C(323), C(325), C(327), C(334), C(341), C(344), C(346), C(348), C(355), C(362), C(365), C(367), C(369), C(376), C(383), C(386), C(388), C(390), C(397), C(404), C(407), C(409), C(411), C(418), C(425), C(428), C(430), C(432), C(435), C(439), C(442), C(446), C(449), C(451), C(453), C(460), C(467), C(470), C(472), C(474), C(477), C(481), C(484), C(488), C(491), C(493), C(495), C(502), C(509), C(512), C(514), C(516), C(519), C(523), C(526), C(530), C(533), C(535), C(537), C(544), C(551), C(554), C(556), C(558), C(561), C(565), C(568), C(572), C(575), C(577), C(579), C(586), C(589), C(593), C(596), C(598), C(600), C(603), C(607), C(610), C(614), C(617), C(619), C(621), C(624), C(628), C(631), C(635), C(638), C(640), C(642), C(645), C(649), C(651), C(652), C(656), C(659), C(661), C(663), C(665), C(666), C(670), C(672), C(673), C(677), C(680), C(682), C(684), C(686), C(687), C(691), C(693), C(694), C(698), C(701), C(703), C(705), C(707), C(708), C(712), C(714), C(715), C(719), C(722), C(724), C(726), C(728), C(729), C(733), C(737), C(739), C(741), C(743), C(744), C(746), C(748)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

#### 3.15.7.5	Interleaving

Before interleaving the bits {PC'(0) … PC'(683)} are converted into 3-bit symbols {c(0) … c(227)} according to table 1 in 3GPP TS 45.004, the symbol c(k) consists of d3k=PC'(k), d3k+1=PC'(k+228) and d3k+2=PC'(k+456) for k=0,1,…,227. The interleaving is done as specified for the TCH/HS in subclause 3.2.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.15.7.6	Mapping on a Burst

The mapping is given by the rule:

e(B,j) = i(B,j) and e(B,59+j) = i(B,57+j) for j = 0,1,...,56

and

e(B,57) = hl(B) and e(B,58) = hu(B)

The two symbols, labelled hl(B) and hu(B) on burst number B are flags used for indication of control channel signalling. For each O-TCH/AHS block not stolen for signalling purposes:

hu(B) = {0,0,0}	for the first 2 bursts (indicating status of the even numbered symbols)

hl(B)  = {0,0,0}	for the last 2 bursts (indicating status of the odd numbered symbols)

where {0,0,0} is the mapping of the three bits 0,0,0 onto a 3-bit symbol according to table 1 in 3GPP TS 45.004.

For the use of hl(B) and hu(B) when a speech frame is stolen for signalling purposes, see subclause 4.11.6.

### 3.15.8	RATSCCH_MARKER

This frame type contains the in-band channel and an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.15.8.1	Coding of in-band data

The coding of in-band data is done as specified for the RATSCCH_MARKER frame at half rate in subclause 3.10.8.1.

#### 3.15.8.2	Identification marker

The identification marker is done as specified for the RATSCCH_MARKER frame at half rate in subclause 3.10.8.2.

#### 3.15.8.3	Interleaving

Before interleaving the bits are repeated 3 times:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

The bits {c'(0) … c'(683)} are then converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the TCH/HS in subclause 3.2.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.15.8.4	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4 with exception that it is done by symbols instead of single bits.

### 3.15.9	RATSCCH_DATA

This frame contains the RATSCCH data and an inband channel. The RATSCCH data consists of 35 bits. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.15.9.1	Coding of in-band data

The coding of in-band data is done as specified for the RATSCCH_DATA frame at half rate in subclause 3.10.9.1.

#### 3.15.9.2	Parity and convolutional encoding for the RATSCCH message

The parity and convolutional encoding for the RATSCCH message are done as specified for the RATSCCH_DATA frame at half rate in subclause 3.10.9.2.

#### 3.15.9.3	Interleaving

Before interleaving the bits are repeated 3 times:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

The bits {c'(0) … c'(683)} are then converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the TCH/HS in subclause 3.2.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.15.9.4	Mapping on a Burst

The mapping is done as specified for the TCH/HS in subclause 3.2.4 with exception that it is done by symbols instead of single bits.

## 3.16	Wideband Adaptive multi rate speech channel at 8-PSK full rate (O-TCH/WFS)

This section describes the coding for the different frame formats used for O-TCH/WFS. The formats used are (in the order they are described):

SID_UPDATE	Used to convey comfort noise parameters during DTX

SID_FIRST		Marker to define end of speech, start of DTX

ONSET			Used to signal the Codec mode for the first speech frame after DTX

SPEECH			Speech frames

RATSCCH		Frames used to convey RATSCCH messages

In this chapter, sub chapters 3.16.1 to 3.16.5 describe the channel coding for the different formats listed above.

Common to all the formats is that in-band information is conveyed, the coding for the in-band channel is described in the table below.

| Identifier (defined in 3GPP TS 45.009) | Received in-band data id(1), id(0) | Encoded in-band data for SID and RATSCCH frames ic(15),.., ic(0) | Encoded in-band data for speech frames ic(23),.., ic(0) |
| --- | --- | --- | --- |
| CODEC_MODE_1 | 00 | 0101001100001111 | 000000000000000000000000 |
| CODEC_MODE_2 | 01 | 0011111010111000 | 110110101110110110101110 |
| CODEC_MODE_3 | 10 | 1000100001100011 | 101101110101101101110101 |
| CODEC_MODE_4 | 11 | 1110010111010100 | 011011011011011011011011 |

### 3.16.1	SID_UPDATE

The speech encoder delivers 35 bits of comfort noise parameters. Also delivered is two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands or Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 CN bits which are then coded by a rate ¼ RSC coder to 212 bits. Finally a 212 bits identification field is added thereby giving a total size of 456 bits. These 456 bits are then block interleaved in the same way as SACCH frames.

#### 3.16.1.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_UPDATE frame in TCH/AFS (subclause 3.9.1.1).

#### 3.16.1.2	Parity and convolutional encoding for the comfort noise parameters

The parity and convolutional encoding for the comfort noise parameters are done as specified for the SID_UPDATE frame in TCH/AFS (subclause 3.9.1.2).

#### 3.16.1.3	Identification marker

The identification marker is constructed as specified for the SID_UPDATE frame in TCH/AFS (subclause 3.9.1.3).

#### 3.16.1.4	Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,455

#### 3.16.1.5	Interleaving

Before interleaving the coded bits {c'(0) … c'(1367)} are converted into 3-bit symbols {C(0) … C(455)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,455.

The interleaving is done as specified for the SID_UPDATE frame in TCH/AFS (subclause 3.9.1.4). The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.16.1.6	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE frame in TCH/AFS (subclause 3.9.1.5) with exception that it is done by symbols instead of single bits.

### 3.16.2	SID_FIRST

This frame type contains no source data from the speech coder, what is transmitted is the in-band channel (signalling Mode Indication or Mode Command/Mode Request depending on the current frame number) and an identification marker.

#### 3.16.2.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_FIRST frame in TCH/AFS (subclause 3.9.2.1).

#### 3.16.2.2	Identification marker

The identification marker is constructed as specified for the SID_FIRST frame in TCH/AFS (subclause 3.9.2.2).

#### 3.16.2.3		Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

#### 3.16.2.4	Interleaving

Before interleaving the coded bits {c'(0) … c'(683)} are converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the in TCH/AFS (subclause 3.9.2.3). The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.16.2.5	Mapping on a Burst

The mapping is done as specified for the TCH/AFS (subclause 3.9.2.4) with exception that it is done by symbols instead of single bits.

### 3.16.3	ONSET

Onset frames are used to preset the interleaver buffer after a period of no speech activity in DTX mode. This frame type contains no source data from the speech coder. What is transmitted is the in-band channel signalling the Mode Indication for the speech frame following the onset marker.

#### 3.16.3.1	Coding of in-band data

The coding of in-band data is done as specified for the ONSET frame in TCH/AFS (subclause 3.9.3.1).

#### 3.16.3.2		Repetition

The coded bits (c) are repeated according to the following rule:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,227

#### 3.16.3.3	Interleaving

Before interleaving the coded bits {c'(0) … c'(683)} are converted into 3-bit symbols {C(0) … C(227)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,227.

The interleaving is done as specified for the ONSET frame in TCH/AFS (subclause 3.9.3.2). The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables. The result of the interleaving is a distribution of 114 of the reordered 228 symbols of a given data block over 2 blocks using the odd numbered bits. The even numbered symbols of these 2 blocks will be filled by the speech frame that following immediately after this frame.

#### 3.16.3.4	Mapping on a Burst

The mapping is done as specified for the ONSET frame in TCH/AFS (subclause 3.9.3.3) with exception that it is done by symbols instead of single bits.

### 3.16.4	SPEECH

The speech coder delivers to the channel encoder a sequence of blocks of data. One block of data corresponds to one speech frame and the block length is different in each of the nine channel codec modes. Adjoining each block of data is information of the channel codec mode to use when encoding the block. Also delivered is the in-band data id(0,1) representing Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.16.4.1	Coding of the in-band data

The two input in-band bits (id(0,1)) are coded to twenty four coded in-band bits (ic(0..23)).

The encoded in-band bits are moved to the coded bits, c, as

c(k) = ic(k)		for k = 0, 1, ..., 23.

#### 3.16.4.2	Ordering according to subjective importance

The bits delivered by the speech encoder, {s(1),s(2),...,s(Ks)},  are rearranged according to subjective importance before channel coding. Tables 16 to 20 define the correct rearrangement for the speech codec modes 12.65 kbit/s, 8.85 kbit/s 6.60 kbit/s, 23.85 kbit/s and 15.85 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to 3GPP TS 26.190 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see below:

| Codec mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| O-TCH/WFS23.85 | 477 |
| O-TCH/WFS15.85 | 317 |
| O-TCH/WFS12.65 | 253 |
| O-TCH/WFS8.85 | 177 |
| O-TCH/WFS6.60 | 132 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1	d(j) := s(table(j)+1);			where table(j) is read line by line left to right

The rearranged bits are further divided into two different classes to perform unequal error protection for different bits according to subjective importance.

The protection classes are:

1a	-	Data protected with the CRC and the convolution code.
	1b	-	Data protected with the convolution code.

The number of class 1 (sum of class 1a and 1b), class 1a and class 1b bits for each codec mode is shown below:

| Codec mode | Number of speech bits delivered per block | Number of class 1 bits per block | Number of Class 1a bits per block | Number of class 1b bits per block |
| --- | --- | --- | --- | --- |
| O-TCH/WFS23.85 | 477 | 477 | 72 | 405 |
| O-TCH/WFS15.85 | 317 | 317 | 72 | 245 |
| O-TCH/WFS12.65 | 253 | 253 | 72 | 181 |
| O-TCH/WFS8.85 | 177 | 177 | 64 | 113 |
| O-TCH/WFS6.60 | 132 | 132 | 54 | 78 |

#### 3.16.4.3	Parity for speech frames

The basic parameters for each codec mode for the first encoding step are shown below:

| Codec mode | Number of class 1 bits (Kd1) | CRC Protected bits (Kd1a) | CRC bits | Number of bits after first encoding step (Ku = Kd + 6) |
| --- | --- | --- | --- | --- |
| O-TCH/WFS23.85 | 477 | 72 | 6 | 483 |
| O-TCH/WFS15.85 | 317 | 72 | 6 | 323 |
| O-TCH/WFS12.65 | 253 | 72 | 6 | 259 |
| O-TCH/WFS8.85 | 177 | 64 | 6 | 183 |
| O-TCH/WFS6.60 | 132 | 54 | 6 | 138 |

A 6-bit CRC is used for error-detection. These parity bits are generated by the cyclic generator polynomial: 
g(D) = D6 + D5 + D3 + D2 + D1 + 1 from the first Kd1a bits of class 1, where Kd1a refers to number of bits in protection class 1a as shown above for each codec mode. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(Kd1a+5) + d(1)D(Kd1a+4) +... + d(Kd1a-1)D(6) + p(0)D(5) +…+ p(4)D+ p(5)

where p(0), p(1) … p(5) are the parity bits, when divided by g(D), yields a remainder equal to:

1+ D + D2 + D3 + D4 + D5.

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, Kd1a-1

u(k) = p(k-Kd1a)			for k = Kd1a, Kd1a+1, …, Kd1a+5

u(k) = d(k-6)				for k = Kd1a+6, Kd1a+7, …, Ku-1

O-TCH/WFS23.85:

u(k) = d(k)				for k = 0, 1, ..., 71

u(k) = p(k-72)			for k = 72, 73, …, 77

u(k) = d(k-6)				for k = 78, 79, …, 482

O-TCH/WFS15.85:

u(k) = d(k)				for k = 0, 1, ..., 71

u(k) = p(k-72)			for k = 72, 73, …, 77

u(k) = d(k-6)				for k = 78, 79, …, 322

O-TCH/WFS12.65:

u(k) = d(k)				for k = 0, 1, ..., 71

u(k) = p(k-72)			for k = 72, 73, …, 77

u(k) = d(k-6)				for k = 78, 79, …, 258

O-TCH/WFS8.85:

u(k) = d(k)				for k = 0, 1, …, 63

u(k) = p(k-64)			for k = 64, 65, …, 69

u(k) = d(k-6)				for k = 70, 71, …, 182

O-TCH/WFS6.60:

u(k) = d(k)				for k = 0, 1, ..., 53

u(k) = p(k-54)			for k = 54, 55, ..., 59

u(k) = d(k-6)				for k = 60, 61, ..., 137

#### 3.16.4.4	Convolutional encoder

The bits from the first encoding step (u(k)) are encoded with the recursive systematic convolutional codes as summarised below. The number of output bits after puncturing is 1344 for all codec modes.

| Codec Mode | Rate | Number of input bits to conv. coder | Number of output bits from conv. Coder | Number Of Punctured bits |
| --- | --- | --- | --- | --- |
| O-TCH/WFS23.85 | 1/3 | 483 | 1467 | 123 |
| O-TCH/WFS15.85 | 1/5 | 323 | 1645 | 301 |
| O-TCH/WFS12.65 | 1/6 | 259 | 1590 | 246 |
| O-TCH/WFS8.85 | 1/8 | 183 | 1512 | 168 |
| O-TCH/WFS6.60 | 1/10 | 138 | 1440 | 96 |

Below the coding for each codec mode is specified in detail. The puncturing for each mode is designed to give an even protection of the class 1A bits while the protection within class 1B is not equal to reflect the individual error sensitivity of the class 1B bits.

O-TCH/WFS23.85:

The block of 483 bits {u(0)… u(482)} is encoded with the 1/3 rate convolutional code defined by	the following

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 1467 coded bits, {C(0)… C(1466)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(3k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(3k+2)	= u(k)	for k = 0, 1, ..., 482;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(3k+2)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 483, 484, ..., 488

The following 896 coded bits are moved to data block PG:

C(2), C(3), C(5), C(6), C(8), C(9), C(11), C(12), C(14), C(15), C(17), C(18), C(20), C(21), C(23), C(24), C(26), C(27), C(29), C(30), C(32), C(33), C(35), C(36), C(38), C(39), C(41), C(42), C(44), C(45), C(47), C(48), C(50), C(51), C(53), C(54), C(56), C(57), C(59), C(60), C(62), C(63), C(65), C(66), C(68), C(69), C(71), C(72), C(74), C(75), C(77), C(78), C(80), C(81), C(83), C(84), C(86), C(87), C(89), C(90), C(92), C(93), C(95), C(96), C(98), C(99), C(101), C(102), C(104), C(105), C(107), C(108), C(110), C(111), C(113), C(114), C(116), C(117), C(119), C(120), C(122), C(123), C(125), C(126), C(128), C(129), C(131), C(132), C(134), C(135), C(137), C(138), C(140), C(141), C(143), C(144), C(146), C(147), C(149), C(150), C(152), C(153), C(155), C(156), C(158), C(159), C(161), C(162), C(164), C(165), C(167), C(168), C(170), C(171), C(173), C(174), C(176), C(177), C(179), C(180), C(182), C(183), C(185), C(186), C(188), C(189), C(191), C(192), C(194), C(195), C(197), C(198), C(200), C(201), C(203), C(204), C(206), C(207), C(209), C(210), C(212), C(213), C(215), C(216), C(218), C(219), C(221), C(222), C(224), C(225), C(227), C(228), C(230), C(231), C(233), C(234), C(236), C(237), C(239), C(240), C(242), C(243), C(245), C(246), C(248), C(249), C(251), C(252), C(254), C(255), C(257), C(258), C(260), C(261), C(263), C(264), C(266), C(267), C(269), C(270), C(272), C(273), C(275), C(276), C(278), C(279), C(281), C(282), C(284), C(285), C(287), C(288), C(290), C(291), C(293), C(294), C(296), C(297), C(299), C(300), C(302), C(303), C(305), C(306), C(308), C(309), C(311), C(312), C(314), C(315), C(317), C(318), C(320), C(321), C(323), C(324), C(326), C(327), C(329), C(330), C(332), C(333), C(335), C(336), C(338), C(339), C(341), C(342), C(344), C(345), C(347), C(348), C(350), C(351), C(353), C(354), C(356), C(357), C(359), C(360), C(362), C(363), C(365), C(366), C(368), C(369), C(371), C(372), C(374), C(375), C(377), C(378), C(380), C(381), C(383), C(384), C(386), C(387), C(389), C(390), C(392), C(393), C(395), C(396), C(398), C(399), C(401), C(402), C(404), C(405), C(407), C(408), C(410), C(411), C(413), C(414), C(416), C(417), C(419), C(420), C(422), C(423), C(425), C(426), C(428), C(429), C(431), C(432), C(434), C(435), C(437), C(438), C(440), C(441), C(443), C(444), C(446), C(447), C(449), C(450), C(452), C(453), C(455), C(456), C(458), C(459), C(461), C(462), C(464), C(465), C(467), C(468), C(470), C(471), C(473), C(474), C(476), C(477), C(479), C(480), C(482), C(483), C(485), C(486), C(488), C(489), C(491), C(492), C(494), C(495), C(497), C(498), C(500), C(501), C(503), C(504), C(506), C(507), C(509), C(510), C(512), C(513), C(515), C(518), C(519), C(521), C(522), C(524), C(525), C(527), C(530), C(531), C(533), C(534), C(536), C(537), C(539), C(542), C(543), C(545), C(546), C(548), C(549), C(551), C(554), C(555), C(557), C(558), C(560), C(561), C(563), C(566), C(567), C(569), C(570), C(572), C(573), C(575), C(578), C(579), C(581), C(582), C(584), C(585), C(587), C(590), C(591), C(593), C(594), C(596), C(597), C(599), C(602), C(603), C(605), C(606), C(608), C(609), C(611), C(614), C(615), C(617), C(618), C(620), C(621), C(623), C(626), C(627), C(629), C(630), C(632), C(633), C(635), C(638), C(639), C(641), C(642), C(644), C(645), C(647), C(650), C(651), C(653), C(654), C(656), C(657), C(659), C(662), C(663), C(665), C(666), C(668), C(669), C(671), C(674), C(675), C(677), C(678), C(680), C(681), C(683), C(686), C(687), C(689), C(690), C(692), C(693), C(695), C(698), C(699), C(701), C(702), C(704), C(705), C(707), C(710), C(711), C(713), C(714), C(716), C(717), C(719), C(722), C(723), C(725), C(726), C(728), C(729), C(731), C(734), C(735), C(737), C(738), C(740), C(741), C(743), C(746), C(747), C(749), C(750), C(752), C(753), C(755), C(758), C(759), C(761), C(762), C(764), C(765), C(767), C(770), C(771), C(773), C(774), C(776), C(777), C(779), C(782), C(783), C(785), C(786), C(788), C(789), C(791), C(794), C(795), C(797), C(798), C(800), C(801), C(803), C(806), C(807), C(809), C(810), C(812), C(813), C(815), C(818), C(819), C(821), C(822), C(824), C(825), C(827), C(830), C(831), C(833), C(834), C(836), C(837), C(839), C(842), C(843), C(845), C(846), C(848), C(849), C(851), C(854), C(855), C(857), C(858), C(860), C(861), C(863), C(866), C(867), C(869), C(870), C(872), C(873), C(875), C(878), C(879), C(881), C(882), C(884), C(885), C(887), C(890), C(891), C(893), C(894), C(896), C(897), C(899), C(902), C(903), C(905), C(906), C(908), C(909), C(911), C(914), C(915), C(917), C(918), C(920), C(921), C(923), C(926), C(927), C(929), C(930), C(932), C(933), C(935), C(938), C(939), C(941), C(942), C(944), C(945), C(947), C(950), C(951), C(953), C(954), C(956), C(957), C(959), C(962), C(963), C(965), C(966), C(968), C(969), C(971), C(974), C(975), C(977), C(978), C(980), C(981), C(983), C(986), C(987), C(989), C(990), C(992), C(993), C(995), C(998), C(999), C(1001), C(1002), C(1004), C(1005), C(1007), C(1010), C(1011), C(1013), C(1014), C(1016), C(1017), C(1019), C(1022), C(1023), C(1025), C(1026), C(1028), C(1029), C(1031), C(1034), C(1035), C(1037), C(1038), C(1040), C(1041), C(1043), C(1046), C(1047), C(1049), C(1050), C(1052), C(1053), C(1055), C(1058), C(1059), C(1061), C(1062), C(1064), C(1065), C(1067), C(1070), C(1071), C(1073), C(1074), C(1076), C(1077), C(1079), C(1082), C(1083), C(1085), C(1086), C(1088), C(1089), C(1091), C(1094), C(1095), C(1097), C(1098), C(1100), C(1101), C(1103), C(1106), C(1107), C(1109), C(1110), C(1112), C(1113), C(1115), C(1118), C(1119), C(1121), C(1122), C(1124), C(1125), C(1127), C(1130), C(1131), C(1133), C(1134), C(1136), C(1137), C(1139), C(1142), C(1143), C(1145), C(1146), C(1148), C(1149), C(1151), C(1154), C(1155), C(1157), C(1158), C(1160), C(1161), C(1163), C(1166), C(1167), C(1169), C(1170), C(1172), C(1173), C(1175), C(1178), C(1179), C(1181), C(1182), C(1184), C(1185), C(1187), C(1190), C(1191), C(1193), C(1194), C(1196), C(1197), C(1199), C(1202), C(1203), C(1205), C(1206), C(1208), C(1209), C(1211), C(1214), C(1215), C(1217), C(1218), C(1220), C(1221), C(1223), C(1226), C(1227), C(1229), C(1230), C(1232), C(1233), C(1235), C(1238), C(1239), C(1241), C(1242), C(1244), C(1245), C(1247), C(1250), C(1251), C(1253), C(1254), C(1256), C(1257), C(1259), C(1262), C(1263), C(1265), C(1266), C(1268), C(1269), C(1271), C(1274), C(1275), C(1277), C(1278), C(1280), C(1281), C(1283), C(1286), C(1287), C(1289), C(1290), C(1292), C(1293), C(1295), C(1298), C(1299), C(1301), C(1302), C(1304), C(1305), C(1307), C(1310), C(1311), C(1313), C(1314), C(1316), C(1317), C(1319), C(1322), C(1323), C(1325), C(1326), C(1328), C(1329), C(1331), C(1334), C(1335), C(1337), C(1338), C(1340), C(1341), C(1343), C(1346), C(1347), C(1349), C(1350), C(1352), C(1353), C(1355), C(1358), C(1359), C(1361), C(1362), C(1364), C(1365), C(1367), C(1370), C(1371), C(1373), C(1374), C(1376), C(1377), C(1379), C(1382), C(1383), C(1385), C(1386), C(1388), C(1389), C(1391), C(1394), C(1395), C(1397), C(1398), C(1400), C(1401), C(1403), C(1406), C(1407), C(1409), C(1410), C(1412), C(1413), C(1415), C(1418), C(1419), C(1421), C(1422), C(1424), C(1425), C(1427), C(1430), C(1431), C(1433), C(1434), C(1436), C(1437), C(1439), C(1442), C(1443), C(1445), C(1446), C(1448), C(1449), C(1451), C(1454), C(1455), C(1457), C(1458), C(1460), C(1463), C(1466)

And the following 448 coded bits are moved to data block PB:

C(1), C(4), C(7), C(10), C(13), C(16), C(19), C(22), C(25), C(28), C(31), C(34), C(37), C(40), C(43), C(46), C(49), C(52), C(55), C(58), C(61), C(64), C(67), C(70), C(73), C(76), C(79), C(82), C(85), C(88), C(91), C(94), C(97), C(100), C(103), C(106), C(109), C(112), C(115), C(118), C(121), C(124), C(127), C(130), C(133), C(136), C(139), C(142), C(145), C(148), C(151), C(154), C(157), C(160), C(163), C(166), C(169), C(172), C(175), C(178), C(181), C(184), C(187), C(190), C(193), C(196), C(199), C(202), C(205), C(208), C(211), C(214), C(217), C(220), C(223), C(226), C(229), C(232), C(235), C(238), C(241), C(244), C(247), C(250), C(253), C(256), C(259), C(262), C(265), C(268), C(271), C(274), C(277), C(280), C(283), C(286), C(289), C(292), C(295), C(298), C(301), C(304), C(307), C(310), C(313), C(316), C(319), C(322), C(325), C(328), C(331), C(334), C(337), C(340), C(343), C(346), C(349), C(352), C(355), C(358), C(361), C(364), C(367), C(370), C(373), C(376), C(379), C(382), C(385), C(388), C(391), C(394), C(397), C(400), C(403), C(406), C(409), C(412), C(415), C(418), C(421), C(424), C(427), C(430), C(433), C(436), C(439), C(442), C(445), C(448), C(451), C(454), C(457), C(460), C(463), C(466), C(469), C(472), C(475), C(478), C(481), C(484), C(487), C(490), C(493), C(496), C(499), C(502), C(505), C(508), C(511), C(514), C(516), C(517), C(520), C(523), C(526), C(528), C(529), C(532), C(535), C(538), C(540), C(541), C(544), C(547), C(550), C(552), C(553), C(556), C(559), C(562), C(564), C(565), C(568), C(571), C(574), C(576), C(577), C(580), C(583), C(586), C(588), C(589), C(592), C(595), C(598), C(600), C(601), C(604), C(607), C(610), C(612), C(613), C(616), C(619), C(622), C(624), C(625), C(628), C(631), C(634), C(636), C(637), C(640), C(643), C(646), C(648), C(649), C(652), C(655), C(658), C(660), C(661), C(664), C(667), C(670), C(672), C(673), C(676), C(679), C(682), C(684), C(685), C(688), C(691), C(694), C(696), C(697), C(700), C(703), C(706), C(708), C(709), C(712), C(715), C(718), C(720), C(721), C(724), C(727), C(730), C(732), C(733), C(736), C(739), C(742), C(744), C(745), C(748), C(754), C(756), C(760), C(766), C(768), C(772), C(778), C(780), C(784), C(790), C(792), C(796), C(802), C(804), C(808), C(814), C(816), C(820), C(826), C(828), C(832), C(838), C(840), C(844), C(850), C(852), C(856), C(862), C(864), C(868), C(874), C(876), C(880), C(886), C(888), C(892), C(898), C(900), C(904), C(910), C(912), C(916), C(922), C(924), C(928), C(934), C(936), C(940), C(946), C(948), C(952), C(958), C(960), C(964), C(970), C(972), C(976), C(982), C(984), C(988), C(994), C(996), C(1000), C(1006), C(1008), C(1012), C(1018), C(1020), C(1024), C(1030), C(1032), C(1036), C(1042), C(1044), C(1048), C(1054), C(1056), C(1060), C(1066), C(1068), C(1072), C(1078), C(1080), C(1084), C(1090), C(1092), C(1096), C(1102), C(1104), C(1108), C(1114), C(1116), C(1120), C(1126), C(1128), C(1132), C(1138), C(1140), C(1144), C(1150), C(1152), C(1156), C(1162), C(1164), C(1168), C(1174), C(1176), C(1180), C(1186), C(1188), C(1192), C(1198), C(1200), C(1204), C(1210), C(1212), C(1216), C(1222), C(1224), C(1228), C(1234), C(1236), C(1240), C(1246), C(1248), C(1252), C(1258), C(1260), C(1264), C(1270), C(1272), C(1276), C(1282), C(1284), C(1288), C(1294), C(1296), C(1300), C(1306), C(1308), C(1312), C(1318), C(1320), C(1324), C(1330), C(1332), C(1336), C(1342), C(1344), C(1348), C(1354), C(1356), C(1360), C(1366), C(1368), C(1372), C(1378), C(1380), C(1384), C(1390), C(1392), C(1396), C(1402), C(1404), C(1408), C(1414), C(1416), C(1420), C(1426), C(1428), C(1432), C(1438), C(1440), C(1444), C(1450), C(1452), C(1456), C(1462)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3, 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 0, 1, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11, 12, 13, 14, 15

PC' (k+16) = PG(k)			for k = 448, 449, ..., 895

PC' (k+896) = ic(k)			for k = 16, 17, 18, 19, 20, 21, 22, 23

PC' (k+920) = PB(k)			for k = 0, 1, ..., 447

O-TCH/WFS15.85:

The block of 323 bits {u(0)… u(322)} is encoded with the 1/5 rate convolutional code defined by the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 1645 coded bits, {C(0)… C(1644)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= u(k)	for k = 0, 1, ..., 322;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 323, 324, ..., 238

The following 896 coded bits are moved to data block PG:

C(4), C(9), C(11), C(12), C(14), C(16), C(17), C(18), C(19), C(21), C(22), C(23), C(24), C(26), C(27), C(28), C(29), C(31), C(32), C(34), C(36), C(37), C(38), C(39), C(41), C(42), C(43), C(44), C(46), C(47), C(49), C(51), C(52), C(53), C(54), C(56), C(57), C(58), C(59), C(61), C(62), C(64), C(66), C(67), C(68), C(69), C(71), C(72), C(73), C(74), C(76), C(77), C(79), C(81), C(82), C(83), C(84), C(86), C(87), C(88), C(89), C(91), C(92), C(94), C(96), C(97), C(98), C(99), C(101), C(102), C(103), C(104), C(106), C(107), C(109), C(111), C(112), C(113), C(114), C(116), C(117), C(118), C(119), C(121), C(122), C(124), C(126), C(127), C(128), C(129), C(131), C(132), C(133), C(134), C(136), C(137), C(139), C(141), C(142), C(143), C(144), C(146), C(147), C(148), C(149), C(151), C(152), C(154), C(156), C(157), C(158), C(159), C(161), C(162), C(163), C(164), C(166), C(167), C(169), C(171), C(172), C(173), C(174), C(176), C(177), C(178), C(179), C(181), C(182), C(184), C(186), C(187), C(188), C(189), C(191), C(192), C(193), C(194), C(196), C(197), C(199), C(201), C(202), C(203), C(204), C(206), C(207), C(208), C(209), C(211), C(212), C(214), C(216), C(217), C(218), C(219), C(221), C(222), C(223), C(224), C(226), C(227), C(229), C(231), C(232), C(233), C(234), C(236), C(237), C(238), C(239), C(241), C(242), C(244), C(246), C(247), C(248), C(249), C(251), C(252), C(253), C(254), C(256), C(257), C(259), C(261), C(262), C(263), C(264), C(266), C(267), C(268), C(269), C(271), C(272), C(274), C(276), C(277), C(278), C(279), C(281), C(282), C(283), C(284), C(286), C(287), C(289), C(291), C(292), C(293), C(294), C(296), C(297), C(298), C(299), C(301), C(302), C(304), C(306), C(307), C(308), C(309), C(311), C(312), C(313), C(314), C(316), C(317), C(319), C(321), C(322), C(323), C(324), C(326), C(327), C(328), C(329), C(331), C(332), C(334), C(336), C(337), C(338), C(339), C(341), C(342), C(343), C(344), C(346), C(347), C(349), C(351), C(352), C(353), C(354), C(356), C(357), C(358), C(359), C(361), C(362), C(364), C(366), C(367), C(369), C(371), C(372), C(374), C(376), C(377), C(379), C(381), C(384), C(386), C(387), C(389), C(391), C(394), C(396), C(397), C(399), C(401), C(404), C(406), C(407), C(409), C(411), C(414), C(416), C(417), C(419), C(421), C(424), C(426), C(427), C(429), C(431), C(434), C(436), C(437), C(439), C(441), C(444), C(446), C(447), C(449), C(451), C(454), C(456), C(457), C(459), C(461), C(464), C(466), C(467), C(469), C(471), C(474), C(476), C(477), C(479), C(481), C(484), C(486), C(487), C(489), C(491), C(494), C(496), C(497), C(499), C(501), C(504), C(506), C(507), C(509), C(511), C(514), C(516), C(517), C(519), C(521), C(524), C(526), C(527), C(529), C(531), C(534), C(536), C(537), C(539), C(541), C(544), C(546), C(547), C(549), C(551), C(554), C(556), C(557), C(559), C(561), C(564), C(566), C(567), C(569), C(571), C(574), C(576), C(577), C(579), C(581), C(584), C(586), C(587), C(589), C(591), C(594), C(596), C(597), C(599), C(601), C(604), C(606), C(607), C(609), C(611), C(614), C(616), C(617), C(619), C(621), C(624), C(626), C(627), C(629), C(631), C(634), C(636), C(637), C(639), C(641), C(644), C(646), C(647), C(649), C(651), C(654), C(656), C(657), C(659), C(661), C(664), C(666), C(667), C(669), C(671), C(674), C(676), C(677), C(679), C(681), C(684), C(686), C(687), C(689), C(691), C(694), C(696), C(697), C(699), C(701), C(704), C(706), C(707), C(709), C(711), C(714), C(716), C(717), C(719), C(721), C(724), C(726), C(727), C(729), C(731), C(734), C(736), C(737), C(739), C(741), C(744), C(746), C(747), C(749), C(751), C(754), C(756), C(757), C(759), C(761), C(764), C(766), C(767), C(769), C(771), C(774), C(776), C(777), C(779), C(781), C(784), C(786), C(787), C(789), C(791), C(794), C(796), C(797), C(799), C(801), C(804), C(806), C(807), C(809), C(811), C(814), C(816), C(817), C(819), C(821), C(824), C(826), C(827), C(829), C(831), C(834), C(836), C(837), C(839), C(841), C(844), C(846), C(847), C(849), C(851), C(854), C(856), C(857), C(859), C(861), C(864), C(866), C(867), C(869), C(871), C(874), C(876), C(877), C(879), C(881), C(884), C(886), C(887), C(889), C(891), C(894), C(896), C(897), C(899), C(901), C(904), C(906), C(907), C(909), C(911), C(914), C(916), C(917), C(919), C(921), C(924), C(926), C(927), C(929), C(931), C(934), C(936), C(937), C(939), C(941), C(944), C(946), C(947), C(949), C(951), C(954), C(956), C(957), C(959), C(961), C(964), C(966), C(967), C(969), C(971), C(974), C(976), C(977), C(979), C(981), C(984), C(986), C(987), C(989), C(991), C(994), C(996), C(997), C(999), C(1001), C(1004), C(1006), C(1007), C(1009), C(1011), C(1014), C(1016), C(1017), C(1019), C(1021), C(1024), C(1026), C(1027), C(1029), C(1031), C(1034), C(1036), C(1037), C(1039), C(1041), C(1044), C(1046), C(1047), C(1049), C(1051), C(1054), C(1056), C(1057), C(1059), C(1061), C(1064), C(1066), C(1067), C(1069), C(1071), C(1074), C(1076), C(1077), C(1079), C(1081), C(1084), C(1086), C(1087), C(1089), C(1091), C(1094), C(1096), C(1097), C(1099), C(1101), C(1104), C(1106), C(1107), C(1109), C(1111), C(1114), C(1116), C(1117), C(1119), C(1121), C(1124), C(1126), C(1127), C(1129), C(1131), C(1134), C(1136), C(1137), C(1139), C(1141), C(1144), C(1146), C(1147), C(1149), C(1151), C(1154), C(1156), C(1157), C(1159), C(1161), C(1164), C(1166), C(1167), C(1169), C(1171), C(1174), C(1176), C(1177), C(1179), C(1181), C(1184), C(1186), C(1187), C(1189), C(1191), C(1194), C(1196), C(1197), C(1199), C(1201), C(1204), C(1206), C(1207), C(1209), C(1211), C(1214), C(1216), C(1217), C(1219), C(1221), C(1224), C(1226), C(1227), C(1229), C(1231), C(1234), C(1236), C(1237), C(1239), C(1241), C(1244), C(1246), C(1247), C(1249), C(1251), C(1254), C(1256), C(1257), C(1259), C(1261), C(1264), C(1266), C(1267), C(1269), C(1271), C(1274), C(1276), C(1277), C(1279), C(1281), C(1284), C(1286), C(1287), C(1289), C(1291), C(1294), C(1296), C(1297), C(1299), C(1301), C(1304), C(1306), C(1307), C(1309), C(1311), C(1314), C(1316), C(1317), C(1319), C(1321), C(1324), C(1326), C(1327), C(1329), C(1331), C(1334), C(1336), C(1337), C(1339), C(1341), C(1344), C(1346), C(1347), C(1349), C(1351), C(1354), C(1356), C(1357), C(1359), C(1361), C(1364), C(1366), C(1367), C(1369), C(1371), C(1374), C(1376), C(1377), C(1379), C(1381), C(1384), C(1386), C(1387), C(1389), C(1391), C(1394), C(1396), C(1397), C(1399), C(1401), C(1404), C(1406), C(1407), C(1409), C(1411), C(1414), C(1416), C(1417), C(1419), C(1421), C(1424), C(1426), C(1427), C(1429), C(1431), C(1434), C(1436), C(1437), C(1439), C(1441), C(1444), C(1446), C(1447), C(1449), C(1451), C(1454), C(1456), C(1457), C(1459), C(1461), C(1464), C(1466), C(1467), C(1469), C(1471), C(1474), C(1476), C(1477), C(1479), C(1481), C(1484), C(1486), C(1487), C(1489), C(1491), C(1494), C(1496), C(1497), C(1499), C(1501), C(1504), C(1506), C(1507), C(1509), C(1511), C(1514), C(1516), C(1517), C(1519), C(1521), C(1524), C(1526), C(1527), C(1529), C(1531), C(1534), C(1536), C(1537), C(1539), C(1541), C(1544), C(1546), C(1547), C(1549), C(1551), C(1554), C(1556), C(1557), C(1559), C(1561), C(1564), C(1566), C(1567), C(1569), C(1571), C(1574), C(1576), C(1577), C(1579), C(1581), C(1584), C(1586), C(1587), C(1589), C(1591), C(1594), C(1596), C(1597), C(1599), C(1601), C(1604), C(1606), C(1609), C(1614), C(1616), C(1619), C(1624), C(1626), C(1629), C(1631), C(1634), C(1636), C(1639), C(1644)

And the following 448 coded bits are moved to data block PB:

C(30), C(33), C(35), C(40), C(45), C(48), C(50), C(55), C(60), C(63), C(65), C(70), C(75), C(78), C(80), C(85), C(90), C(93), C(95), C(100), C(105), C(108), C(110), C(115), C(120), C(123), C(125), C(130), C(135), C(138), C(140), C(145), C(150), C(153), C(155), C(160), C(165), C(168), C(170), C(175), C(180), C(183), C(185), C(190), C(195), C(198), C(200), C(205), C(210), C(213), C(215), C(220), C(225), C(228), C(230), C(235), C(240), C(243), C(245), C(250), C(255), C(258), C(260), C(265), C(270), C(273), C(275), C(280), C(285), C(288), C(290), C(295), C(300), C(303), C(305), C(310), C(315), C(318), C(320), C(325), C(330), C(333), C(335), C(340), C(345), C(348), C(350), C(355), C(363), C(368), C(373), C(378), C(382), C(383), C(388), C(392), C(393), C(398), C(402), C(403), C(408), C(412), C(413), C(418), C(422), C(423), C(428), C(432), C(433), C(438), C(442), C(443), C(448), C(452), C(453), C(458), C(462), C(463), C(468), C(472), C(473), C(478), C(482), C(483), C(488), C(492), C(493), C(498), C(502), C(503), C(508), C(512), C(513), C(518), C(522), C(523), C(528), C(532), C(533), C(538), C(542), C(543), C(548), C(552), C(553), C(558), C(562), C(563), C(568), C(572), C(573), C(578), C(582), C(583), C(588), C(592), C(593), C(598), C(602), C(608), C(612), C(613), C(618), C(622), C(623), C(628), C(632), C(633), C(638), C(642), C(643), C(648), C(652), C(653), C(658), C(662), C(668), C(672), C(673), C(678), C(682), C(683), C(688), C(692), C(698), C(702), C(703), C(708), C(712), C(713), C(718), C(722), C(723), C(728), C(732), C(733), C(738), C(742), C(743), C(748), C(752), C(758), C(762), C(763), C(768), C(772), C(773), C(778), C(782), C(788), C(792), C(793), C(798), C(802), C(803), C(808), C(812), C(813), C(818), C(822), C(823), C(828), C(832), C(833), C(838), C(842), C(848), C(852), C(853), C(858), C(862), C(863), C(868), C(872), C(878), C(882), C(883), C(888), C(892), C(893), C(898), C(902), C(903), C(908), C(912), C(913), C(918), C(922), C(923), C(928), C(932), C(938), C(942), C(943), C(948), C(952), C(953), C(958), C(962), C(968), C(972), C(973), C(978), C(982), C(983), C(988), C(992), C(993), C(998), C(1002), C(1003), C(1008), C(1012), C(1013), C(1018), C(1022), C(1028), C(1032), C(1033), C(1038), C(1042), C(1043), C(1048), C(1052), C(1058), C(1062), C(1063), C(1068), C(1072), C(1073), C(1078), C(1082), C(1083), C(1088), C(1092), C(1093), C(1098), C(1102), C(1103), C(1108), C(1112), C(1118), C(1122), C(1123), C(1128), C(1132), C(1133), C(1138), C(1142), C(1148), C(1152), C(1153), C(1158), C(1162), C(1163), C(1168), C(1172), C(1173), C(1178), C(1182), C(1183), C(1188), C(1192), C(1193), C(1198), C(1202), C(1208), C(1212), C(1213), C(1218), C(1222), C(1223), C(1228), C(1232), C(1238), C(1242), C(1243), C(1248), C(1252), C(1253), C(1258), C(1262), C(1263), C(1268), C(1272), C(1273), C(1278), C(1282), C(1283), C(1288), C(1292), C(1298), C(1302), C(1303), C(1308), C(1312), C(1313), C(1318), C(1322), C(1328), C(1332), C(1333), C(1338), C(1342), C(1343), C(1348), C(1352), C(1353), C(1358), C(1362), C(1363), C(1368), C(1372), C(1373), C(1378), C(1382), C(1388), C(1392), C(1393), C(1398), C(1402), C(1403), C(1408), C(1412), C(1418), C(1422), C(1423), C(1428), C(1432), C(1433), C(1438), C(1442), C(1443), C(1448), C(1452), C(1453), C(1458), C(1462), C(1463), C(1468), C(1472), C(1478), C(1482), C(1483), C(1488), C(1492), C(1493), C(1498), C(1502), C(1508), C(1512), C(1513), C(1518), C(1522), C(1523), C(1528), C(1532), C(1533), C(1538), C(1542), C(1543), C(1548), C(1552), C(1553), C(1558), C(1562), C(1568), C(1572), C(1573), C(1578), C(1582), C(1583), C(1588), C(1592), C(1598), C(1602), C(1603), C(1607), C(1611), C(1612), C(1617), C(1621), C(1622), C(1623), C(1627), C(1632), C(1637), C(1641)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3, 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 0, 1, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11, 12, 13, 14, 15

PC' (k+16) = PG(k)			for k = 448, 449, ..., 895

PC' (k+896) = ic(k)			for k = 16, 17, 18, 19, 20, 21, 22, 23

PC' (k+920) = PB(k)			for k = 0, 1, ..., 447

O-TCH/WFS12.65:

The block of 259 bits {u(0)… u(258)} is encoded with the 1/6 rate convolutional code defined by the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 1590 coded bits, {C(0)… C(1589)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(6k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(6k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+5)	= u(k)	for k = 0, 1, ..., 258;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(6k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(6k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(6k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(6k+5)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 259, 260, ..., 264

The following 896 coded bits are moved to data block PG:

C(5), C(7), C(11), C(13), C(14), C(17), C(19), C(20), C(23), C(25), C(26), C(27), C(29), C(31), C(32), C(35), C(36), C(37), C(38), C(39), C(41), C(42), C(43), C(44), C(45), C(47), C(48), C(49), C(50), C(51), C(53), C(54), C(55), C(56), C(59), C(60), C(61), C(62), C(63), C(65), C(66), C(67), C(68), C(71), C(72), C(73), C(74), C(75), C(77), C(78), C(79), C(80), C(81), C(83), C(84), C(85), C(86), C(87), C(89), C(90), C(91), C(92), C(95), C(96), C(97), C(98), C(99), C(101), C(102), C(103), C(104), C(107), C(108), C(109), C(110), C(111), C(113), C(114), C(115), C(116), C(117), C(119), C(120), C(121), C(122), C(123), C(125), C(126), C(127), C(128), C(131), C(132), C(133), C(134), C(135), C(137), C(138), C(139), C(140), C(143), C(144), C(145), C(146), C(147), C(149), C(150), C(151), C(152), C(153), C(155), C(156), C(157), C(158), C(159), C(161), C(162), C(163), C(164), C(167), C(168), C(169), C(170), C(171), C(173), C(174), C(175), C(176), C(179), C(180), C(181), C(182), C(183), C(185), C(186), C(187), C(188), C(189), C(191), C(192), C(193), C(194), C(195), C(197), C(198), C(199), C(200), C(203), C(204), C(205), C(206), C(207), C(209), C(210), C(211), C(212), C(215), C(216), C(217), C(218), C(219), C(221), C(222), C(223), C(224), C(225), C(227), C(228), C(229), C(230), C(231), C(233), C(234), C(235), C(236), C(239), C(240), C(241), C(242), C(243), C(245), C(246), C(247), C(248), C(251), C(252), C(253), C(254), C(255), C(257), C(258), C(259), C(260), C(261), C(263), C(264), C(265), C(266), C(267), C(269), C(270), C(271), C(272), C(275), C(276), C(277), C(278), C(279), C(281), C(282), C(283), C(284), C(287), C(288), C(289), C(290), C(291), C(293), C(294), C(295), C(296), C(297), C(299), C(300), C(301), C(302), C(303), C(305), C(306), C(307), C(308), C(311), C(312), C(313), C(314), C(315), C(317), C(318), C(319), C(320), C(323), C(324), C(325), C(326), C(327), C(329), C(330), C(331), C(332), C(333), C(335), C(336), C(337), C(338), C(339), C(341), C(342), C(343), C(344), C(347), C(348), C(349), C(350), C(351), C(353), C(354), C(355), C(356), C(359), C(360), C(361), C(362), C(363), C(365), C(366), C(367), C(368), C(369), C(371), C(372), C(373), C(374), C(375), C(377), C(378), C(379), C(380), C(383), C(384), C(385), C(386), C(387), C(389), C(390), C(391), C(392), C(395), C(396), C(397), C(398), C(399), C(401), C(402), C(403), C(404), C(405), C(407), C(408), C(409), C(410), C(411), C(413), C(414), C(415), C(416), C(419), C(420), C(421), C(422), C(423), C(425), C(426), C(427), C(428), C(431), C(432), C(433), C(434), C(435), C(437), C(438), C(439), C(440), C(441), C(443), C(444), C(445), C(446), C(447), C(449), C(450), C(451), C(452), C(455), C(456), C(457), C(458), C(459), C(461), C(462), C(463), C(464), C(467), C(468), C(469), C(470), C(471), C(473), C(474), C(475), C(476), C(477), C(479), C(480), C(481), C(482), C(483), C(485), C(486), C(487), C(488), C(491), C(492), C(493), C(494), C(495), C(497), C(498), C(499), C(500), C(503), C(505), C(506), C(509), C(511), C(512), C(515), C(517), C(518), C(521), C(523), C(524), C(527), C(529), C(530), C(533), C(535), C(536), C(539), C(541), C(542), C(545), C(547), C(548), C(551), C(553), C(554), C(557), C(559), C(560), C(563), C(565), C(566), C(569), C(571), C(572), C(575), C(577), C(578), C(581), C(583), C(584), C(587), C(589), C(590), C(593), C(595), C(596), C(599), C(601), C(602), C(605), C(607), C(608), C(611), C(613), C(614), C(617), C(619), C(620), C(623), C(625), C(626), C(629), C(631), C(632), C(635), C(637), C(638), C(641), C(643), C(644), C(647), C(649), C(650), C(653), C(655), C(656), C(659), C(661), C(662), C(665), C(667), C(668), C(671), C(673), C(674), C(677), C(679), C(680), C(683), C(685), C(686), C(689), C(691), C(692), C(695), C(697), C(698), C(701), C(703), C(704), C(707), C(709), C(710), C(713), C(715), C(716), C(719), C(721), C(722), C(725), C(727), C(728), C(731), C(733), C(734), C(737), C(739), C(740), C(743), C(745), C(746), C(749), C(751), C(752), C(755), C(757), C(758), C(761), C(763), C(764), C(767), C(769), C(770), C(773), C(775), C(776), C(779), C(781), C(782), C(785), C(787), C(788), C(791), C(793), C(794), C(797), C(799), C(800), C(803), C(805), C(806), C(809), C(811), C(812), C(815), C(817), C(818), C(821), C(823), C(824), C(827), C(829), C(830), C(833), C(835), C(836), C(839), C(841), C(842), C(845), C(847), C(848), C(851), C(853), C(854), C(857), C(859), C(860), C(863), C(865), C(866), C(869), C(871), C(872), C(875), C(877), C(878), C(881), C(883), C(884), C(887), C(889), C(890), C(893), C(895), C(896), C(899), C(901), C(902), C(905), C(907), C(908), C(911), C(913), C(914), C(917), C(919), C(920), C(923), C(925), C(926), C(929), C(931), C(932), C(935), C(937), C(938), C(941), C(943), C(944), C(947), C(949), C(950), C(953), C(955), C(956), C(959), C(961), C(962), C(965), C(967), C(968), C(971), C(973), C(974), C(977), C(979), C(980), C(983), C(985), C(986), C(989), C(991), C(992), C(995), C(997), C(998), C(1001), C(1003), C(1004), C(1007), C(1009), C(1013), C(1015), C(1016), C(1019), C(1021), C(1022), C(1025), C(1027), C(1028), C(1031), C(1033), C(1034), C(1037), C(1039), C(1040), C(1043), C(1045), C(1049), C(1051), C(1052), C(1055), C(1057), C(1058), C(1061), C(1063), C(1064), C(1067), C(1069), C(1070), C(1073), C(1075), C(1076), C(1079), C(1081), C(1085), C(1087), C(1088), C(1091), C(1093), C(1094), C(1097), C(1099), C(1100), C(1103), C(1105), C(1106), C(1109), C(1111), C(1112), C(1115), C(1117), C(1121), C(1123), C(1124), C(1127), C(1129), C(1130), C(1133), C(1135), C(1136), C(1139), C(1141), C(1142), C(1145), C(1147), C(1148), C(1151), C(1153), C(1157), C(1159), C(1160), C(1163), C(1165), C(1166), C(1169), C(1171), C(1172), C(1175), C(1177), C(1178), C(1181), C(1183), C(1184), C(1187), C(1189), C(1193), C(1195), C(1196), C(1199), C(1201), C(1202), C(1205), C(1207), C(1208), C(1211), C(1213), C(1214), C(1217), C(1219), C(1220), C(1223), C(1225), C(1229), C(1231), C(1232), C(1235), C(1237), C(1238), C(1241), C(1243), C(1244), C(1247), C(1249), C(1250), C(1253), C(1255), C(1256), C(1259), C(1261), C(1265), C(1267), C(1268), C(1271), C(1273), C(1274), C(1277), C(1279), C(1280), C(1283), C(1285), C(1286), C(1289), C(1291), C(1292), C(1295), C(1297), C(1301), C(1303), C(1304), C(1307), C(1309), C(1310), C(1313), C(1315), C(1316), C(1319), C(1321), C(1322), C(1325), C(1327), C(1328), C(1331), C(1333), C(1337), C(1339), C(1340), C(1343), C(1345), C(1346), C(1349), C(1351), C(1352), C(1355), C(1357), C(1358), C(1361), C(1363), C(1364), C(1367), C(1369), C(1373), C(1375), C(1376), C(1379), C(1381), C(1382), C(1385), C(1387), C(1388), C(1391), C(1393), C(1394), C(1397), C(1399), C(1400), C(1403), C(1405), C(1409), C(1411), C(1412), C(1415), C(1417), C(1418), C(1421), C(1423), C(1424), C(1427), C(1429), C(1430), C(1433), C(1435), C(1436), C(1439), C(1441), C(1445), C(1447), C(1448), C(1451), C(1453), C(1454), C(1457), C(1459), C(1460), C(1463), C(1465), C(1466), C(1469), C(1471), C(1472), C(1475), C(1477), C(1481), C(1483), C(1484), C(1487), C(1489), C(1490), C(1493), C(1495), C(1496), C(1499), C(1501), C(1502), C(1505), C(1507), C(1508), C(1511), C(1513), C(1517), C(1519), C(1520), C(1523), C(1525), C(1529), C(1531), C(1532), C(1535), C(1537), C(1541), C(1543), C(1544), C(1547), C(1549), C(1553), C(1555), C(1559), C(1565), C(1567), C(1571), C(1573), C(1577), C(1583), C(1589)

And the following 448 coded bits are moved to data block PB:

C(21), C(33), C(34), C(46), C(57), C(58), C(69), C(70), C(82), C(93), C(94), C(105), C(106), C(118), C(129), C(130), C(141), C(142), C(154), C(165), C(166), C(177), C(178), C(190), C(201), C(202), C(213), C(214), C(226), C(237), C(238), C(249), C(250), C(262), C(273), C(274), C(285), C(286), C(298), C(309), C(310), C(321), C(322), C(334), C(345), C(346), C(357), C(358), C(370), C(381), C(382), C(393), C(394), C(406), C(417), C(418), C(429), C(430), C(442), C(453), C(454), C(465), C(466), C(478), C(489), C(490), C(501), C(502), C(504), C(507), C(510), C(513), C(516), C(519), C(522), C(525), C(528), C(531), C(534), C(537), C(540), C(543), C(546), C(549), C(552), C(555), C(558), C(561), C(564), C(567), C(570), C(573), C(576), C(579), C(582), C(585), C(588), C(591), C(594), C(597), C(600), C(603), C(606), C(609), C(612), C(615), C(618), C(621), C(624), C(627), C(630), C(633), C(636), C(639), C(642), C(645), C(648), C(651), C(654), C(657), C(660), C(663), C(666), C(669), C(672), C(675), C(678), C(681), C(684), C(687), C(690), C(693), C(696), C(699), C(702), C(705), C(708), C(711), C(714), C(717), C(720), C(723), C(726), C(729), C(732), C(735), C(738), C(741), C(744), C(747), C(750), C(753), C(756), C(759), C(762), C(765), C(768), C(771), C(774), C(777), C(780), C(783), C(786), C(789), C(792), C(795), C(798), C(801), C(804), C(807), C(810), C(813), C(816), C(819), C(822), C(825), C(828), C(831), C(834), C(837), C(840), C(843), C(846), C(849), C(852), C(855), C(858), C(861), C(864), C(867), C(870), C(873), C(876), C(879), C(882), C(885), C(888), C(891), C(894), C(897), C(900), C(903), C(906), C(909), C(912), C(915), C(918), C(921), C(924), C(927), C(930), C(933), C(936), C(939), C(942), C(945), C(948), C(951), C(954), C(957), C(960), C(963), C(966), C(969), C(972), C(975), C(978), C(981), C(984), C(987), C(990), C(993), C(996), C(999), C(1002), C(1005), C(1008), C(1010), C(1011), C(1014), C(1017), C(1020), C(1023), C(1026), C(1029), C(1032), C(1035), C(1038), C(1041), C(1044), C(1046), C(1047), C(1050), C(1053), C(1056), C(1059), C(1062), C(1065), C(1068), C(1071), C(1074), C(1077), C(1080), C(1082), C(1083), C(1086), C(1089), C(1092), C(1095), C(1098), C(1101), C(1104), C(1107), C(1110), C(1113), C(1116), C(1118), C(1119), C(1122), C(1125), C(1128), C(1131), C(1134), C(1137), C(1140), C(1143), C(1146), C(1149), C(1152), C(1154), C(1155), C(1158), C(1161), C(1164), C(1167), C(1170), C(1173), C(1176), C(1179), C(1182), C(1185), C(1188), C(1190), C(1191), C(1194), C(1197), C(1200), C(1203), C(1206), C(1209), C(1212), C(1215), C(1218), C(1221), C(1224), C(1226), C(1227), C(1230), C(1233), C(1236), C(1239), C(1242), C(1245), C(1248), C(1251), C(1254), C(1257), C(1260), C(1262), C(1263), C(1266), C(1269), C(1272), C(1275), C(1278), C(1281), C(1284), C(1287), C(1290), C(1293), C(1296), C(1298), C(1299), C(1302), C(1305), C(1308), C(1311), C(1314), C(1317), C(1320), C(1323), C(1326), C(1329), C(1332), C(1334), C(1335), C(1338), C(1341), C(1344), C(1347), C(1350), C(1353), C(1356), C(1359), C(1362), C(1365), C(1368), C(1370), C(1371), C(1374), C(1377), C(1380), C(1383), C(1386), C(1389), C(1392), C(1395), C(1398), C(1401), C(1404), C(1406), C(1407), C(1410), C(1413), C(1416), C(1419), C(1422), C(1425), C(1428), C(1431), C(1434), C(1437), C(1440), C(1442), C(1443), C(1446), C(1449), C(1452), C(1455), C(1458), C(1461), C(1464), C(1467), C(1470), C(1473), C(1476), C(1478), C(1479), C(1482), C(1485), C(1488), C(1491), C(1494), C(1497), C(1500), C(1503), C(1506), C(1509), C(1512), C(1514), C(1515), C(1518), C(1521), C(1524), C(1526), C(1527), C(1530), C(1533), C(1536), C(1538), C(1539), C(1542), C(1545), C(1548), C(1550), C(1551), C(1554), C(1556), C(1560), C(1561), C(1562), C(1563), C(1566), C(1568), C(1572), C(1574), C(1579), C(1585)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3, 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 0, 1, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11, 12, 13, 14, 15

PC' (k+16) = PG(k)			for k = 448, 449, ..., 895

PC' (k+896) = ic(k)			for k = 16, 17, 18, 19, 20, 21, 22, 23

PC' (k+920) = PB(k)			for k = 0, 1, ..., 447

O-TCH/WFS8.85:

The block of 183 bits {u(0)… u(182)} is encoded with the 1/8 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

G7/G7 = 1

resulting in 1512 coded bits, {C(0)… C(1511)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(8k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(8k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(8k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(8k+3)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(8k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(8k+5)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(8k+6)	= u(k)

C(8k+7)	= u(k)	for k = 0, 1, ..., 182;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(8k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(8k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(8k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(8k+3)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(8k+4)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(8k+5)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(8k+6)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(8k+7)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 183, 184, ..., 188

The following 896 coded bits are moved to data block PG:

C(1), C(2), C(6), C(9), C(10), C(14), C(16), C(17), C(18), C(20), C(22), C(24), C(25), C(26), C(29), C(30), C(32), C(33), C(34), C(36), C(38), C(40), C(41), C(42), C(43), C(44), C(45), C(46), C(48), C(49), C(50), C(52), C(54), C(56), C(57), C(58), C(60), C(61), C(62), C(64), C(65), C(66), C(67), C(68), C(70), C(72), C(73), C(74), C(76), C(77), C(78), C(80), C(81), C(82), C(83), C(84), C(86), C(88), C(89), C(90), C(92), C(93), C(94), C(96), C(97), C(98), C(100), C(102), C(104), C(105), C(106), C(108), C(109), C(110), C(112), C(113), C(114), C(115), C(116), C(118), C(120), C(121), C(122), C(124), C(125), C(126), C(128), C(129), C(130), C(131), C(132), C(134), C(136), C(137), C(138), C(140), C(141), C(142), C(144), C(145), C(146), C(148), C(150), C(152), C(153), C(154), C(156), C(157), C(158), C(160), C(161), C(162), C(163), C(164), C(166), C(168), C(169), C(170), C(172), C(173), C(174), C(176), C(177), C(178), C(179), C(180), C(182), C(184), C(185), C(186), C(188), C(189), C(190), C(192), C(193), C(194), C(196), C(198), C(200), C(201), C(202), C(204), C(205), C(206), C(208), C(209), C(210), C(211), C(212), C(214), C(216), C(217), C(218), C(220), C(221), C(222), C(224), C(225), C(226), C(227), C(228), C(230), C(232), C(233), C(234), C(236), C(237), C(238), C(240), C(241), C(242), C(244), C(246), C(248), C(249), C(250), C(252), C(253), C(254), C(256), C(257), C(258), C(259), C(260), C(262), C(264), C(265), C(266), C(268), C(269), C(270), C(272), C(273), C(274), C(275), C(276), C(278), C(280), C(281), C(282), C(284), C(285), C(286), C(288), C(289), C(290), C(292), C(294), C(296), C(297), C(298), C(300), C(301), C(302), C(304), C(305), C(306), C(307), C(308), C(310), C(312), C(313), C(314), C(316), C(317), C(318), C(320), C(321), C(322), C(323), C(324), C(326), C(328), C(329), C(330), C(332), C(333), C(334), C(336), C(337), C(338), C(340), C(342), C(344), C(345), C(346), C(348), C(349), C(350), C(352), C(353), C(354), C(355), C(356), C(358), C(360), C(361), C(362), C(364), C(365), C(366), C(368), C(369), C(370), C(371), C(372), C(374), C(376), C(377), C(378), C(380), C(381), C(382), C(384), C(385), C(386), C(388), C(390), C(392), C(393), C(394), C(396), C(397), C(398), C(400), C(401), C(402), C(403), C(404), C(406), C(408), C(409), C(410), C(412), C(413), C(414), C(416), C(417), C(418), C(419), C(420), C(422), C(424), C(425), C(426), C(428), C(429), C(430), C(432), C(433), C(434), C(436), C(438), C(440), C(441), C(442), C(444), C(445), C(446), C(448), C(449), C(450), C(451), C(452), C(454), C(456), C(457), C(458), C(460), C(461), C(462), C(464), C(465), C(466), C(467), C(468), C(470), C(472), C(473), C(474), C(476), C(477), C(478), C(480), C(481), C(482), C(484), C(486), C(488), C(489), C(490), C(492), C(493), C(494), C(496), C(497), C(498), C(499), C(500), C(502), C(504), C(505), C(506), C(508), C(509), C(510), C(512), C(513), C(514), C(515), C(516), C(518), C(520), C(521), C(522), C(524), C(525), C(526), C(528), C(529), C(530), C(532), C(534), C(536), C(537), C(538), C(540), C(541), C(542), C(544), C(545), C(546), C(547), C(548), C(550), C(552), C(553), C(554), C(556), C(557), C(558), C(560), C(561), C(562), C(563), C(564), C(566), C(568), C(569), C(570), C(572), C(573), C(574), C(576), C(577), C(578), C(580), C(582), C(584), C(585), C(586), C(588), C(589), C(590), C(592), C(593), C(594), C(595), C(596), C(598), C(600), C(601), C(602), C(604), C(605), C(606), C(608), C(609), C(610), C(611), C(612), C(614), C(616), C(617), C(618), C(620), C(621), C(622), C(624), C(625), C(626), C(628), C(630), C(632), C(633), C(634), C(636), C(637), C(638), C(640), C(641), C(642), C(643), C(644), C(646), C(648), C(649), C(650), C(652), C(653), C(654), C(656), C(657), C(658), C(659), C(660), C(662), C(664), C(665), C(666), C(668), C(669), C(670), C(673), C(674), C(676), C(678), C(681), C(682), C(684), C(686), C(689), C(690), C(692), C(694), C(697), C(698), C(700), C(702), C(705), C(706), C(708), C(710), C(713), C(714), C(716), C(718), C(721), C(722), C(724), C(726), C(729), C(730), C(732), C(734), C(737), C(738), C(740), C(742), C(745), C(746), C(748), C(750), C(753), C(754), C(756), C(758), C(761), C(762), C(764), C(766), C(769), C(770), C(772), C(774), C(777), C(778), C(780), C(782), C(785), C(786), C(788), C(790), C(793), C(794), C(796), C(798), C(801), C(802), C(804), C(806), C(809), C(810), C(812), C(814), C(817), C(818), C(820), C(822), C(825), C(826), C(828), C(830), C(833), C(834), C(836), C(838), C(841), C(842), C(844), C(846), C(849), C(850), C(852), C(854), C(857), C(858), C(860), C(862), C(865), C(866), C(868), C(870), C(873), C(874), C(876), C(878), C(881), C(882), C(884), C(886), C(889), C(890), C(892), C(894), C(897), C(898), C(900), C(902), C(905), C(906), C(908), C(910), C(913), C(914), C(916), C(918), C(921), C(922), C(924), C(926), C(929), C(930), C(932), C(934), C(937), C(938), C(940), C(942), C(945), C(946), C(948), C(950), C(953), C(954), C(956), C(958), C(961), C(962), C(964), C(966), C(969), C(970), C(972), C(974), C(977), C(978), C(980), C(982), C(985), C(986), C(988), C(990), C(993), C(994), C(996), C(998), C(1001), C(1002), C(1004), C(1006), C(1009), C(1010), C(1012), C(1014), C(1017), C(1018), C(1020), C(1022), C(1025), C(1026), C(1028), C(1030), C(1033), C(1034), C(1036), C(1038), C(1041), C(1042), C(1044), C(1046), C(1049), C(1050), C(1052), C(1054), C(1057), C(1058), C(1060), C(1062), C(1065), C(1066), C(1068), C(1070), C(1073), C(1074), C(1076), C(1078), C(1081), C(1082), C(1084), C(1086), C(1089), C(1090), C(1092), C(1094), C(1097), C(1098), C(1100), C(1102), C(1105), C(1106), C(1108), C(1110), C(1113), C(1114), C(1116), C(1118), C(1121), C(1122), C(1124), C(1126), C(1129), C(1130), C(1132), C(1134), C(1137), C(1138), C(1140), C(1142), C(1145), C(1146), C(1148), C(1150), C(1153), C(1154), C(1156), C(1158), C(1161), C(1162), C(1164), C(1166), C(1169), C(1170), C(1172), C(1174), C(1177), C(1178), C(1180), C(1182), C(1185), C(1186), C(1188), C(1190), C(1193), C(1194), C(1196), C(1198), C(1201), C(1202), C(1204), C(1206), C(1209), C(1210), C(1212), C(1214), C(1217), C(1218), C(1220), C(1222), C(1225), C(1226), C(1228), C(1230), C(1233), C(1234), C(1236), C(1238), C(1241), C(1242), C(1244), C(1246), C(1249), C(1250), C(1252), C(1254), C(1257), C(1258), C(1260), C(1262), C(1265), C(1266), C(1268), C(1270), C(1273), C(1274), C(1276), C(1278), C(1281), C(1282), C(1284), C(1286), C(1289), C(1290), C(1292), C(1294), C(1297), C(1298), C(1300), C(1302), C(1305), C(1306), C(1308), C(1310), C(1313), C(1314), C(1316), C(1318), C(1321), C(1322), C(1324), C(1326), C(1329), C(1330), C(1332), C(1334), C(1337), C(1338), C(1340), C(1342), C(1345), C(1346), C(1348), C(1350), C(1353), C(1354), C(1356), C(1358), C(1361), C(1362), C(1364), C(1366), C(1369), C(1370), C(1372), C(1374), C(1377), C(1378), C(1380), C(1382), C(1385), C(1386), C(1388), C(1390), C(1393), C(1394), C(1396), C(1398), C(1401), C(1402), C(1404), C(1406), C(1409), C(1410), C(1412), C(1414), C(1417), C(1418), C(1420), C(1422), C(1425), C(1426), C(1428), C(1430), C(1433), C(1434), C(1436), C(1438), C(1441), C(1442), C(1446), C(1449), C(1450), C(1454), C(1457), C(1458), C(1462), C(1465), C(1466), C(1470), C(1473), C(1474), C(1478), C(1481), C(1482), C(1484), C(1486), C(1489), C(1490), C(1492), C(1494), C(1497), C(1498), C(1502), C(1505), C(1506), C(1510)

And the following 448 coded bits are moved to data block PB:

C(51), C(53), C(55), C(59), C(63), C(69), C(71), C(75), C(79), C(85), C(87), C(91), C(95), C(99), C(101), C(103), C(107), C(111), C(117), C(119), C(123), C(127), C(133), C(135), C(139), C(143), C(147), C(149), C(151), C(155), C(159), C(165), C(167), C(171), C(175), C(181), C(183), C(187), C(191), C(195), C(197), C(199), C(203), C(207), C(213), C(215), C(219), C(223), C(229), C(231), C(235), C(239), C(243), C(245), C(247), C(251), C(255), C(261), C(263), C(267), C(271), C(277), C(279), C(283), C(287), C(291), C(293), C(295), C(299), C(303), C(309), C(311), C(315), C(319), C(325), C(327), C(331), C(335), C(339), C(341), C(343), C(347), C(351), C(357), C(359), C(363), C(367), C(373), C(375), C(379), C(383), C(387), C(389), C(391), C(395), C(399), C(405), C(407), C(411), C(415), C(421), C(423), C(427), C(431), C(435), C(437), C(439), C(443), C(447), C(453), C(455), C(459), C(463), C(469), C(471), C(475), C(479), C(483), C(485), C(487), C(491), C(495), C(501), C(503), C(507), C(511), C(517), C(519), C(523), C(527), C(531), C(533), C(535), C(539), C(543), C(549), C(551), C(555), C(559), C(565), C(567), C(571), C(575), C(579), C(581), C(583), C(587), C(591), C(597), C(599), C(603), C(607), C(613), C(615), C(619), C(623), C(627), C(629), C(631), C(635), C(639), C(645), C(647), C(651), C(655), C(661), C(663), C(667), C(671), C(672), C(675), C(677), C(680), C(683), C(685), C(688), C(691), C(693), C(696), C(699), C(701), C(704), C(707), C(709), C(712), C(715), C(717), C(720), C(723), C(725), C(728), C(731), C(733), C(736), C(739), C(741), C(744), C(747), C(749), C(752), C(755), C(757), C(760), C(763), C(765), C(768), C(771), C(773), C(776), C(779), C(781), C(784), C(787), C(789), C(792), C(795), C(797), C(800), C(803), C(805), C(808), C(811), C(813), C(816), C(819), C(821), C(824), C(827), C(829), C(832), C(835), C(837), C(840), C(843), C(845), C(848), C(851), C(853), C(856), C(859), C(861), C(864), C(867), C(869), C(872), C(875), C(877), C(880), C(883), C(885), C(888), C(891), C(893), C(896), C(899), C(901), C(904), C(907), C(909), C(912), C(915), C(917), C(920), C(923), C(925), C(928), C(931), C(933), C(936), C(939), C(941), C(944), C(947), C(949), C(952), C(955), C(957), C(960), C(963), C(965), C(968), C(971), C(973), C(976), C(979), C(981), C(984), C(987), C(989), C(992), C(995), C(997), C(1000), C(1003), C(1005), C(1008), C(1013), C(1016), C(1019), C(1021), C(1024), C(1029), C(1032), C(1035), C(1037), C(1040), C(1045), C(1048), C(1051), C(1053), C(1056), C(1061), C(1064), C(1067), C(1069), C(1072), C(1077), C(1080), C(1083), C(1085), C(1088), C(1093), C(1096), C(1099), C(1101), C(1104), C(1109), C(1112), C(1115), C(1117), C(1120), C(1125), C(1128), C(1131), C(1133), C(1136), C(1141), C(1144), C(1147), C(1149), C(1152), C(1157), C(1160), C(1163), C(1165), C(1168), C(1173), C(1176), C(1179), C(1181), C(1184), C(1189), C(1192), C(1195), C(1197), C(1200), C(1205), C(1208), C(1211), C(1213), C(1216), C(1221), C(1224), C(1227), C(1229), C(1232), C(1237), C(1240), C(1243), C(1245), C(1248), C(1253), C(1256), C(1259), C(1261), C(1264), C(1269), C(1272), C(1275), C(1277), C(1280), C(1285), C(1288), C(1291), C(1293), C(1296), C(1301), C(1304), C(1307), C(1309), C(1312), C(1317), C(1320), C(1323), C(1325), C(1328), C(1333), C(1336), C(1339), C(1341), C(1344), C(1349), C(1352), C(1355), C(1357), C(1360), C(1365), C(1368), C(1371), C(1373), C(1376), C(1381), C(1384), C(1387), C(1389), C(1392), C(1397), C(1400), C(1403), C(1405), C(1408), C(1413), C(1416), C(1419), C(1421), C(1424), C(1429), C(1432), C(1435), C(1437), C(1440), C(1444), C(1445), C(1448), C(1451), C(1452), C(1453), C(1456), C(1460), C(1461), C(1464), C(1467), C(1468), C(1469), C(1472), C(1476), C(1480), C(1488)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3, 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 0, 1, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11, 12, 13, 14, 15

PC' (k+16) = PG(k)			for k = 448, 449, ..., 895

PC' (k+896) = ic(k)			for k = 16, 17, 18, 19, 20, 21, 22, 23

PC' (k+920) = PB(k)			for k = 0, 1, ..., 447

O-TCH/WFS6.60:

The block of 138 bits {u(0)… u(137)} is encoded with the 1/10 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 1440 coded bits, {C(0)… C(1439)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(10k)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(10k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(10k+2)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(10k+3)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(10k+4)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(10k+5)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(10k+6)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(10k+7)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(10k+8)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(10k+9)	= u(k)	for k = 0, 1, ..., 182;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(10k)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(10k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(10k+2)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(10k+3)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(10k+4)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(10k+5)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(10k+6)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(10k+7)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(10k+8)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(10k+9)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 183, 184, ..., 188

The following 896 coded bits are moved to data block PG:

C(0), C(6), C(9), C(10), C(16), C(19), C(20), C(23), C(26), C(29), C(30), C(33), C(34), C(36), C(37), C(39), C(40), C(41), C(42), C(43), C(44), C(46), C(47), C(49), C(50), C(51), C(52), C(53), C(54), C(56), C(57), C(59), C(60), C(61), C(62), C(63), C(64), C(65), C(66), C(67), C(69), C(70), C(71), C(72), C(73), C(74), C(76), C(77), C(79), C(80), C(81), C(83), C(84), C(86), C(87), C(89), C(90), C(91), C(93), C(94), C(96), C(97), C(99), C(100), C(101), C(103), C(104), C(105), C(106), C(107), C(109), C(110), C(111), C(113), C(114), C(116), C(117), C(119), C(120), C(121), C(123), C(124), C(126), C(127), C(129), C(130), C(131), C(133), C(134), C(136), C(137), C(139), C(140), C(141), C(143), C(144), C(145), C(146), C(147), C(149), C(150), C(151), C(153), C(154), C(156), C(157), C(159), C(160), C(161), C(163), C(164), C(166), C(167), C(169), C(170), C(171), C(173), C(174), C(176), C(177), C(179), C(180), C(181), C(183), C(184), C(186), C(187), C(189), C(190), C(191), C(193), C(194), C(196), C(197), C(199), C(200), C(201), C(203), C(204), C(206), C(207), C(209), C(210), C(211), C(213), C(214), C(216), C(217), C(219), C(220), C(221), C(223), C(224), C(226), C(227), C(229), C(230), C(231), C(233), C(234), C(236), C(237), C(239), C(240), C(241), C(243), C(244), C(246), C(247), C(249), C(250), C(251), C(253), C(254), C(256), C(257), C(259), C(260), C(261), C(263), C(264), C(266), C(267), C(269), C(270), C(271), C(273), C(274), C(276), C(277), C(279), C(280), C(281), C(283), C(284), C(286), C(287), C(289), C(290), C(291), C(293), C(294), C(296), C(297), C(299), C(300), C(301), C(303), C(304), C(306), C(307), C(309), C(310), C(311), C(313), C(314), C(316), C(317), C(319), C(320), C(321), C(323), C(324), C(326), C(327), C(329), C(330), C(331), C(333), C(334), C(336), C(337), C(339), C(340), C(341), C(343), C(344), C(346), C(347), C(349), C(350), C(351), C(353), C(354), C(356), C(357), C(359), C(360), C(361), C(363), C(364), C(366), C(367), C(369), C(370), C(371), C(373), C(374), C(376), C(377), C(379), C(380), C(381), C(383), C(384), C(386), C(387), C(389), C(390), C(391), C(393), C(394), C(396), C(397), C(399), C(400), C(401), C(403), C(404), C(406), C(407), C(409), C(410), C(411), C(413), C(414), C(416), C(417), C(419), C(420), C(421), C(423), C(424), C(426), C(427), C(429), C(430), C(431), C(433), C(434), C(436), C(437), C(439), C(440), C(441), C(443), C(444), C(446), C(447), C(449), C(450), C(451), C(453), C(454), C(456), C(457), C(459), C(460), C(461), C(463), C(464), C(466), C(467), C(469), C(470), C(471), C(473), C(474), C(476), C(477), C(479), C(480), C(481), C(483), C(484), C(486), C(487), C(489), C(490), C(491), C(493), C(494), C(496), C(497), C(499), C(500), C(501), C(503), C(504), C(506), C(507), C(509), C(510), C(511), C(513), C(514), C(516), C(517), C(519), C(520), C(521), C(523), C(524), C(526), C(527), C(529), C(530), C(531), C(533), C(534), C(536), C(537), C(539), C(540), C(541), C(543), C(544), C(546), C(547), C(549), C(550), C(551), C(553), C(554), C(556), C(557), C(559), C(560), C(561), C(563), C(564), C(566), C(567), C(569), C(570), C(571), C(573), C(574), C(576), C(577), C(579), C(580), C(581), C(583), C(584), C(586), C(587), C(589), C(590), C(591), C(593), C(594), C(596), C(597), C(599), C(600), C(601), C(603), C(604), C(606), C(607), C(609), C(610), C(611), C(613), C(614), C(616), C(617), C(619), C(620), C(621), C(623), C(624), C(626), C(627), C(629), C(630), C(631), C(633), C(634), C(636), C(637), C(639), C(640), C(641), C(643), C(644), C(646), C(647), C(649), C(650), C(651), C(653), C(654), C(656), C(657), C(659), C(660), C(661), C(663), C(664), C(666), C(667), C(669), C(670), C(671), C(673), C(674), C(676), C(677), C(679), C(680), C(681), C(683), C(686), C(687), C(689), C(690), C(691), C(693), C(696), C(697), C(699), C(700), C(701), C(703), C(706), C(707), C(709), C(710), C(711), C(713), C(716), C(719), C(720), C(721), C(723), C(726), C(727), C(729), C(730), C(731), C(733), C(736), C(737), C(739), C(740), C(741), C(743), C(746), C(747), C(749), C(750), C(751), C(753), C(756), C(759), C(760), C(761), C(763), C(766), C(767), C(769), C(770), C(771), C(773), C(776), C(777), C(779), C(780), C(781), C(783), C(786), C(787), C(789), C(790), C(791), C(793), C(796), C(799), C(800), C(801), C(803), C(806), C(807), C(809), C(810), C(811), C(813), C(816), C(817), C(819), C(820), C(821), C(823), C(826), C(827), C(829), C(830), C(831), C(833), C(836), C(839), C(840), C(841), C(843), C(846), C(847), C(849), C(850), C(851), C(853), C(856), C(857), C(859), C(860), C(861), C(863), C(866), C(867), C(869), C(870), C(871), C(873), C(876), C(879), C(880), C(881), C(883), C(886), C(887), C(889), C(890), C(891), C(893), C(896), C(897), C(899), C(900), C(901), C(903), C(906), C(907), C(909), C(910), C(911), C(913), C(916), C(919), C(920), C(921), C(923), C(926), C(927), C(929), C(930), C(931), C(933), C(936), C(937), C(939), C(940), C(941), C(943), C(946), C(947), C(949), C(950), C(951), C(953), C(956), C(959), C(960), C(961), C(963), C(966), C(967), C(969), C(970), C(971), C(973), C(976), C(977), C(979), C(980), C(981), C(983), C(986), C(987), C(989), C(990), C(991), C(993), C(996), C(999), C(1000), C(1001), C(1003), C(1006), C(1007), C(1009), C(1010), C(1011), C(1013), C(1016), C(1017), C(1019), C(1020), C(1021), C(1023), C(1026), C(1027), C(1029), C(1030), C(1031), C(1033), C(1036), C(1039), C(1040), C(1041), C(1043), C(1046), C(1047), C(1049), C(1050), C(1051), C(1053), C(1056), C(1057), C(1059), C(1060), C(1061), C(1063), C(1066), C(1067), C(1069), C(1070), C(1071), C(1073), C(1076), C(1079), C(1080), C(1081), C(1083), C(1086), C(1087), C(1089), C(1090), C(1091), C(1093), C(1096), C(1097), C(1099), C(1100), C(1101), C(1103), C(1106), C(1107), C(1109), C(1110), C(1111), C(1113), C(1116), C(1119), C(1120), C(1121), C(1123), C(1126), C(1127), C(1129), C(1130), C(1131), C(1133), C(1136), C(1137), C(1139), C(1140), C(1141), C(1143), C(1146), C(1147), C(1149), C(1150), C(1151), C(1153), C(1156), C(1159), C(1160), C(1161), C(1163), C(1166), C(1167), C(1169), C(1170), C(1171), C(1173), C(1176), C(1177), C(1179), C(1180), C(1181), C(1183), C(1186), C(1187), C(1189), C(1190), C(1191), C(1193), C(1196), C(1199), C(1200), C(1201), C(1203), C(1206), C(1207), C(1209), C(1210), C(1211), C(1213), C(1216), C(1217), C(1219), C(1220), C(1221), C(1223), C(1226), C(1227), C(1229), C(1230), C(1231), C(1233), C(1236), C(1239), C(1240), C(1241), C(1243), C(1246), C(1247), C(1249), C(1250), C(1251), C(1253), C(1256), C(1257), C(1259), C(1260), C(1261), C(1263), C(1266), C(1267), C(1269), C(1270), C(1271), C(1273), C(1276), C(1279), C(1280), C(1281), C(1283), C(1286), C(1289), C(1290), C(1291), C(1293), C(1296), C(1299), C(1300), C(1301), C(1303), C(1306), C(1309), C(1310), C(1311), C(1313), C(1316), C(1319), C(1320), C(1321), C(1323), C(1326), C(1329), C(1330), C(1331), C(1333), C(1336), C(1339), C(1340), C(1341), C(1343), C(1346), C(1349), C(1350), C(1351), C(1353), C(1356), C(1359), C(1360), C(1361), C(1363), C(1366), C(1369), C(1370), C(1371), C(1373), C(1376), C(1379), C(1380), C(1381), C(1383), C(1386), C(1389), C(1390), C(1391), C(1393), C(1396), C(1399), C(1400), C(1401), C(1403), C(1406), C(1409), C(1410), C(1411), C(1413), C(1416), C(1419), C(1420), C(1421), C(1423), C(1426), C(1429), C(1430), C(1431), C(1433), C(1436), C(1439)

And the following 448 coded bits are moved to data block PB:

C(45), C(48), C(55), C(58), C(68), C(75), C(78), C(82), C(85), C(88), C(92), C(95), C(98), C(102), C(108), C(112), C(115), C(118), C(122), C(125), C(128), C(132), C(135), C(138), C(142), C(148), C(152), C(155), C(158), C(162), C(165), C(168), C(172), C(175), C(178), C(182), C(185), C(188), C(192), C(195), C(198), C(202), C(205), C(208), C(212), C(215), C(218), C(222), C(225), C(228), C(232), C(235), C(238), C(242), C(245), C(248), C(252), C(255), C(258), C(262), C(265), C(268), C(272), C(275), C(278), C(282), C(285), C(288), C(292), C(295), C(298), C(302), C(305), C(308), C(312), C(315), C(318), C(322), C(325), C(328), C(332), C(335), C(338), C(342), C(345), C(348), C(352), C(355), C(358), C(362), C(365), C(368), C(372), C(375), C(378), C(382), C(385), C(388), C(392), C(395), C(398), C(402), C(405), C(408), C(412), C(415), C(418), C(422), C(425), C(428), C(432), C(435), C(438), C(442), C(445), C(448), C(452), C(455), C(458), C(462), C(465), C(468), C(472), C(475), C(478), C(482), C(485), C(488), C(492), C(495), C(498), C(502), C(505), C(508), C(512), C(515), C(518), C(522), C(525), C(528), C(532), C(535), C(538), C(542), C(545), C(548), C(552), C(555), C(558), C(562), C(565), C(568), C(572), C(575), C(578), C(582), C(585), C(588), C(592), C(595), C(598), C(602), C(605), C(608), C(612), C(615), C(618), C(622), C(625), C(628), C(632), C(635), C(638), C(642), C(648), C(652), C(655), C(658), C(662), C(668), C(672), C(675), C(678), C(682), C(684), C(688), C(692), C(694), C(698), C(702), C(704), C(708), C(712), C(714), C(715), C(717), C(718), C(722), C(724), C(728), C(732), C(734), C(738), C(742), C(744), C(748), C(752), C(754), C(755), C(757), C(758), C(762), C(764), C(768), C(772), C(774), C(778), C(782), C(784), C(788), C(792), C(794), C(795), C(797), C(798), C(802), C(804), C(808), C(812), C(814), C(818), C(822), C(824), C(828), C(832), C(834), C(835), C(837), C(838), C(842), C(844), C(848), C(852), C(854), C(858), C(862), C(864), C(868), C(872), C(874), C(875), C(877), C(878), C(882), C(884), C(888), C(892), C(894), C(898), C(902), C(904), C(908), C(912), C(914), C(915), C(917), C(918), C(922), C(924), C(928), C(932), C(934), C(938), C(942), C(944), C(948), C(952), C(954), C(955), C(957), C(958), C(962), C(964), C(968), C(972), C(974), C(978), C(982), C(984), C(988), C(992), C(994), C(995), C(997), C(998), C(1002), C(1004), C(1008), C(1012), C(1014), C(1018), C(1022), C(1024), C(1028), C(1032), C(1034), C(1035), C(1037), C(1038), C(1042), C(1044), C(1048), C(1052), C(1054), C(1058), C(1062), C(1064), C(1068), C(1072), C(1074), C(1075), C(1077), C(1078), C(1082), C(1084), C(1088), C(1092), C(1094), C(1098), C(1102), C(1104), C(1108), C(1112), C(1114), C(1115), C(1117), C(1118), C(1122), C(1124), C(1128), C(1132), C(1134), C(1138), C(1142), C(1144), C(1148), C(1152), C(1154), C(1155), C(1157), C(1158), C(1162), C(1164), C(1168), C(1172), C(1174), C(1178), C(1182), C(1184), C(1188), C(1192), C(1194), C(1195), C(1197), C(1198), C(1202), C(1204), C(1208), C(1212), C(1214), C(1218), C(1222), C(1224), C(1228), C(1232), C(1234), C(1235), C(1237), C(1238), C(1242), C(1244), C(1248), C(1252), C(1254), C(1258), C(1262), C(1264), C(1268), C(1272), C(1274), C(1275), C(1277), C(1278), C(1282), C(1284), C(1287), C(1288), C(1292), C(1294), C(1297), C(1298), C(1302), C(1304), C(1307), C(1308), C(1312), C(1314), C(1315), C(1317), C(1318), C(1322), C(1324), C(1327), C(1328), C(1332), C(1334), C(1337), C(1338), C(1342), C(1344), C(1347), C(1348), C(1352), C(1354), C(1355), C(1357), C(1358), C(1362), C(1364), C(1367), C(1368), C(1372), C(1374), C(1377), C(1378), C(1382), C(1384), C(1387), C(1394), C(1397), C(1404), C(1407), C(1414), C(1417), C(1424), C(1427), C(1434), C(1437)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3, 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 0, 1, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11, 12, 13, 14, 15

PC' (k+16) = PG(k)			for k = 448, 449, ..., 895

PC' (k+896) = ic(k)			for k = 16, 17, 18, 19, 20, 21, 22, 23

PC' (k+920) = PB(k)			for k = 0, 1, ..., 447

#### 3.16.4.5	Interleaving

Before interleaving the bits {PC'(0) … PC'(1367)} are converted into 3-bit symbols {c(0) … c(455)} according to table 1 in 3GPP TS 45.004, the symbol c(k) consists of d3k=PC'(k), d3k+1=PC'(k+456) and d3k+2=PC'(k+912) for k=0,1,…,456. The interleaving is done as specified for the TCH/FS in subclause 3.1.3. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.16.4.6	Mapping on a Burst

The mapping is done as specified for the TCH/FS in subclause 3.1.4 with exception that it is done by symbols instead of single bits.

### 3.16.5	RATSCCH

The RATSCCH message consists of 35 bits. Also delivered are two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands or Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 RATSCCH bits which are then coded by a rate ¼ RSC coder to 212 bits. Finally a 212 bit identification field is added thereby giving a total size of 456 bits. These 456 bits are then block interleaved in the same way as a normal speech frame.

#### 3.16.5.1	Coding of in-band data

The coding of in-band data is done as specified for the RATSCCH message in TCH/AFS (subclause 3.9.5.1).

#### 3.16.5.2		Parity and convolutional encoding for the RATSCCH message

The parity and convolutional encoding for the RATSCCH message are done as specified for the RATSCCH message in TCH/AFS (subclause 3.9.5.2).

#### 3.16.5.3	Identification marker

The identification marker is done as specified for the RATSCCH message in TCH/AFS (subclause 3.9.5.3).

#### 3.16.5.4	Interleaving

Before interleaving the bits are repeated 3 times:

c'(3k+2) = c'(3k+1) = c'(3k) = c(k)		for k=0,…,455

The bits {c'(0) … c'(1367)} are then converted into 3-bit symbols {C(0) … C(455)} according to table 1 in 3GPP TS 45.004, the symbol C(k) depends on c'(3k+2), c'(3k+1) and c'(3k) for k=0,1,…,455.

The interleaving is done as specified for the TCH/AFS (subclause 3.9.5.4). The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

#### 3.16.5.5	Mapping on a Burst

The mapping is done as specified for the TCH/AFS (subclause 3.9.5.5) with exception that it is done by symbols instead of single bits.

## 3.17	Wideband Adaptive multi rate speech channel at 8-PSK half rate (O-TCH/WHS)

This section describes the coding for the different frame formats used for O-TCH/WHS. The formats used are (in the order they are described):

SID_UPDATE			Used to convey comfort noise parameters during DTX

SID_UPDATE_INH		Used to inhibit the second part of a SID_UPDATE frame if there is a speech onset

SID_FIRST_P1			First part of marker to define end of speech, start of DTX

SID_FIRST_P2			Second part of marker to define end of speech, start of DTX

SID_FIRST_INH		Used to inhibit the second part of a SID_FIRST_P1 frame if there is a speech onset

ONSET					Used to signal the Codec mode for the first speech frame after DTX

SPEECH					Speech frames

RATSCCH_MARKER	Marker to identify RATSCCH frames

RATSCCH_DATA		Frame that conveys the actual RATSCCH message

In this chapter, sub chapters 3.17.1 to 3.17.9 describe the channel coding for the different formats listed above.

Common to all the formats is that in-band information is conveyed, the coding for the in-band channel is described in the table below.

| Identifier (defined in 3GPP 45.009) | Received in-band data id(1), id(0) | Encoded in-band data for SID and RATSCCH frames ic(15),.., ic(0) | Encoded in-band data for speech frames ic(11),.., ic(0) |
| --- | --- | --- | --- |
| CODEC_MODE_1 | 00 | 0101001100001111 | 000000000000 |
| CODEC_MODE_2 | 01 | 0011111010111000 | 110110101110 |
| CODEC_MODE_3 | 10 | 1000100001100011 | 101101110101 |
| CODEC_MODE_4 | 11 | 1110010111010100 | 011011011011 |

### 3.17.1	SID_UPDATE

The speech encoder delivers 35 bits of comfort noise parameters. Also delivered is two in-band channels, id0(0,1) and id1(0,1), id0 corresponding to Mode Commands/Mode Requests and id1 to Mode Indication. The general coding is as: the two in-band data channels are coded to 16 bits each, a 14-bit CRC is added to the 35 CN bits which are then coded by a rate ¼ RSC coder to 212 bits. A 212 bit identification field is added thereby giving a total size of 456 bits. Finally each bit is repeated 3 times and then converted into 3-bit symbols giving a total size of 456 symbols. These 456 symbols are block interleaved over 4 bursts.

#### 3.17.1.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_UPDATE frame in O-TCH/AHS (subclause 3.15.1.1).

#### 3.17.1.2	Parity and convolutional encoding for the comfort noise parameters

The parity and convolutional encoding for the comfort noise parameters are done as specified for the SID_UPDATE frame in O-TCH/AHS (subclause 3.15.1.2).

#### 3.17.1.3	Identification marker

The identification marker is constructed as specified for the SID_UPDATE frame in O-TCH/AHS (subclause 3.15.1.3).

#### 3.17.1.4	Repetition

The repetition is done as specified for the SID_UPDATE frame in O-TCH/AHS (subclause 3.15.1.4).

#### 3.17.1.5	Interleaving

The interleaving is done as specified for the SID_UPDATE frame in O-TCH/AHS (subclause 3.15.1.5).

#### 3.17.1.6	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE frame in O-TCH/AHS (subclause 3.15.1.6).

### 3.17.2	SID_UPDATE_INH

This special frame is used when the first 2 burst of a SID_UPDATE frame have been transmitted but the second two bursts cannot be transmitted due to a speech frame. The general coding is as: the in-band data (Note that this must be the same Mode Indication bits as id1(0,1) for the SID_UPDATE frame that is being inhibited) is encoded, a marker that is the opposite of the SID_UPDATE marker is appended and the data is interleaved in such a way that the odd symbols of two bursts are filled.

#### 3.17.2.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_UPDATE_INH frame in O-TCH/AHS (subclause 3.15.2.1).

#### 3.17.2.2	Identification marker

The identification marker is constructed as specified for the SID_UPDATE_INH frame in O-TCH/AHS (subclause 3.15.2.2).

#### 3.17.2.3		Repetition

The repetition is done as specified for the SID_UPDATE_INH frame in O-TCH/AHS (subclause 3.15.2.3).

#### 3.17.2.4	Interleaving

The interleaving is done as specified for the SID_UPDATE_INH frame in O-TCH/AHS (subclause 3.15.2.4).

#### 3.17.2.5	Mapping on a Burst

The mapping is done as specified for the SID_UPDATE_INH frame in O-TCH/AHS (subclause 3.15.2.5).

### 3.17.3	SID_FIRST_P1

This frame type contains no source data from the speech coder. What is generated is the in-band channel and an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.17.3.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_FIRST_P1 frame in O-TCH/AHS (subclause 3.15.3.1).

#### 3.17.3.2	Identification marker

The identification marker is constructed as specified for the SID_FIRST_P1 frame in O-TCH/AHS (subclause 3.15.3.2).

#### 3.17.3.3	Repetition

The repetition is done as specified for the SID_FIRST_P1 frame in O-TCH/AHS (subclause 3.15.3.3).

#### 3.17.3.4	Interleaving

The interleaving is done as specified for the SID_FIRST_P1 frame in O-TCH/AHS (subclause 3.15.3.4).

#### 3.17.3.5	Mapping on a Burst

The mapping is done as specified for the SID_FIRST_P1 frame in O-TCH/AHS (subclause 3.15.3.5).

### 3.17.4	SID_FIRST_P2

This frame type contains no source data from the speech coder. What is generated is the in-band channel and, derived from that, an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.17.4.1	Coding of in-band data

The coding of in-band data is done as specified for the SID_FIRST_P2 frame in O-TCH/AHS (subclause 3.15.4.1).

#### 3.17.4.2	Repetition

The repetition is done as specified for the SID_FIRST_P2 frame in O-TCH/AHS (subclause 3.15.4.2).

#### 3.17.4.3	Interleaving

The interleaving is done as specified for the SID_FIRST_P2 frame in O-TCH/AHS (subclause 3.15.4.3).

#### 3.17.4.4	Mapping on a Burst

The mapping is done as specified for the SID_FIRST_P2 frame in O-TCH/AHS (subclause 3.15.4.4).

### 3.17.5	SID_FIRST_INH

This special frame is used when the first 2 burst of a SID_FIRST_P1 frame have been transmitted but the second two bursts cannot be transmitted due to a SPEECH frame. The general coding is as: the in-band data (Note that this must be the same data as for the SID_FIRST_P1 frame that is being inhibited) is encoded, a marker that is the opposite of the SID_FIRST_P1 marker is appended and the data is interleaved in such a way that the odd symbols of two bursts are filled.

#### 3.17.5.1	Coding of in-band data

The coding of the in-band data is done as specified for the SID_FIRST_INH frame in O-TCH/AHS (subclause 3.15.5.1).

#### 3.17.5.2	Identification marker

The identification marker is done as specified for the SID_FIRST_INH frame in O-TCH/AHS (subclause 3.15.5.2).

#### 3.17.5.3	Repetition

The repetition is done as specified for the SID_FIRST_INH frame in O-TCH/AHS (subclause 3.15.5.3).

#### 3.17.5.4	Interleaving

The interleaving is done as specified for the SID_FIRST_INH frame in O-TCH/AHS (subclause 3.15.5.4).

#### 3.17.5.5	Mapping on a Burst

The mapping is done as specified for the SID_FIRST_INH frame in O-TCH/AHS (subclause 3.15.5.5).

### 3.17.6	ONSET

Onset frames are used to preset the interleaver buffer after a period of no speech activity in DTX mode. This frame type contains no source data from the speech coder. What is transmitted is the in-band channel signalling the Mode Indication for the speech frame following the onset marker.

#### 3.17.6.1	Coding of in-band data

The coding of in-band data is done as specified for the ONSET frame in O-TCH/AHS (subclause 3.15.6.1).

#### 3.17.6.2	Repetition

The repetition is done as specified for the ONSET frame in O-TCH/AHS (subclause 3.15.6.2).

#### 3.17.6.3	Interleaving

The interleaving is done as specified for the ONSET frame in O-TCH/AHS (subclause 3.15.6.3).

#### 3.17.6.4	Mapping on a Burst

The mapping is done as specified for the ONSET frame in O-TCH/AHS (subclause 3.15.6.4).

### 3.17.7	SPEECH

The speech coder delivers to the channel encoder a sequence of blocks of data. One block of data corresponds to one speech frame and the block length is different in each of the nine channel codec modes. Adjoining each block of data is information of the channel codec mode to use when encoding the block. Also delivered is the in-band data id(0,1) representing Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.17.7.1	Coding of the in-band data

The two input in-band bits (id(0,1)) are coded to twelve coded in-band bits (ic(0..11)).

The encoded in-band bits are moved to the coded bits, c, as

c(k) = ic(k)		for k = 0, 1, ..., 11.

#### 3.17.7.2	Ordering according to subjective importance

The bits delivered by the speech encoder, {s(1),s(2),...,s(Ks)},  are rearranged according to subjective importance before channel coding. Tables 16 to 18 define the correct rearrangement for the speech codec modes 12.65 kbit/s, 8.85 kbit/s and 6.60 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to 3GPP TS 26.190 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see below:

| Codec mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| O-TCH/WHS12.65 | 253 |
| O-TCH/WHS8.85 | 177 |
| O-TCH/WHS6.60 | 132 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1	d(j) := s(table(j)+1);			where table(j) is read line by line left to right

The rearranged bits are further divided into two different classes to perform unequal error protection for different bits according to subjective importance.

The protection classes are:

1a	-	Data protected with the CRC and the convolution code.
	1b	-	Data protected with the convolution code.

The number of class 1 (sum of class 1a and 1b), class 1a and class 1b bits for each codec mode is shown below:

| Codec mode | Number of speech bits delivered per block | Number of class 1 bits per block | Number of Class 1a bits per block | Number of class 1b bits per block |
| --- | --- | --- | --- | --- |
| O-TCH/WHS12.65 | 253 | 253 | 72 | 181 |
| O-TCH/WHS8.85 | 177 | 177 | 64 | 113 |
| O-TCH/WHS6.60 | 132 | 132 | 54 | 78 |

#### 3.17.7.3	Parity for speech frames

The basic parameters for each codec mode for the first encoding step are shown below:

| Codec mode | Number of class 1 bits (Kd1) | CRC Protected bits (Kd1a) | CRC bits | Number of bits after first encoding step (Ku = Kd + 6) |
| --- | --- | --- | --- | --- |
| O-TCH/WHS12.65 | 253 | 72 | 6 | 259 |
| O-TCH/WHS8.85 | 177 | 64 | 6 | 183 |
| O-TCH/WHS6.60 | 132 | 54 | 6 | 138 |

A 6-bit CRC is used for error-detection. These parity bits are generated by the cyclic generator polynomial: 
g(D) = D6 + D5 + D3 + D2 + D1 + 1 from the first Kd1a bits of class 1, where Kd1a refers to number of bits in protection class 1a as shown above for each codec mode. The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D(Kd1a+5) + d(1)D(Kd1a+4) +... + d(Kd1a-1)D(6) + p(0)D(5) +…+ p(4)D+ p(5)

where p(0), p(1) … p(5) are the parity bits, when divided by g(D), yields a remainder equal to:

1+ D + D2 + D3 + D4 + D5.

The information and parity bits are merged:

u(k) = d(k)				for k = 0, 1, …, Kd1a-1

u(k) = p(k-Kd1a)			for k = Kd1a, Kd1a+1, …, Kd1a+5

u(k) = d(k-6)				for k = Kd1a+6, Kd1a+7, …, Ku-1

O-TCH/WHS12.65:

u(k) = d(k)				for k = 0, 1, ..., 71

u(k) = p(k-72)			for k = 72, 73, …, 77

u(k) = d(k-6)				for k = 78, 79, …, 258

O-TCH/WHS8.85:

u(k) = d(k)				for k = 0, 1, …, 63

u(k) = p(k-64)			for k = 64, 65, …, 69

u(k) = d(k-6)				for k = 70, 71, …, 182

O-TCH/WHS6.60:

u(k) = d(k)				for k = 0, 1, ..., 53

u(k) = p(k-54)			for k = 54, 55, ..., 59

u(k) = d(k-6)				for k = 60, 61, ..., 137

#### 3.17.7.4	Convolutional encoder

The bits from the first encoding step (u(k)) are encoded with the recursive systematic convolutional codes as summarised below. The number of output bits after puncturing is 672 for all codec modes.

| Codec Mode | Rate | Number of input bits to conv. coder | Number of output bits from conv. Coder | Number Of Punctured bits |
| --- | --- | --- | --- | --- |
| O-TCH/WHS12.65 | 1/3 | 259 | 795 | 123 |
| O-TCH/WHS8.85 | ¼ | 183 | 756 | 84 |
| O-TCH/WHS6.60 | 1/5 | 138 | 720 | 48 |

Below the coding for each codec mode is specified in detail. The puncturing for each mode is designed to give an even protection of the class 1A bits while the protection within class 1B is not equal to reflect the individual error sensitivity of the class 1B bits.

O-TCH/WHS12.65:

The block of 259 bits {u(0)… u(258)} is encoded with the 1/3 rate convolutional code defined by the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 795 coded bits, {C(0)… C(794)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(3k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(3k+2)	= u(k)	for k = 0, 1, ..., 258;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(3k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(3k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(3k+2)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 259, 260, ..., 264

The following 448 coded bits are moved to data block PG:

C(2), C(5), C(6), C(8), C(9), C(11), C(12), C(14), C(15), C(17), C(18), C(20), C(21), C(23), C(24), C(26), C(27), C(29), C(30), C(32), C(33), C(35), C(36), C(38), C(39), C(41), C(42), C(44), C(45), C(47), C(48), C(50), C(51), C(53), C(54), C(56), C(57), C(59), C(60), C(62), C(63), C(65), C(66), C(68), C(69), C(71), C(72), C(74), C(75), C(77), C(78), C(80), C(81), C(83), C(84), C(86), C(87), C(89), C(90), C(92), C(93), C(95), C(96), C(98), C(99), C(101), C(102), C(104), C(105), C(107), C(108), C(110), C(111), C(113), C(114), C(116), C(117), C(119), C(120), C(122), C(123), C(125), C(126), C(128), C(129), C(131), C(132), C(134), C(135), C(137), C(138), C(140), C(141), C(143), C(144), C(146), C(147), C(149), C(150), C(152), C(153), C(155), C(156), C(158), C(159), C(161), C(162), C(164), C(165), C(167), C(168), C(170), C(171), C(173), C(174), C(176), C(177), C(179), C(180), C(182), C(183), C(185), C(186), C(188), C(189), C(191), C(192), C(194), C(195), C(197), C(198), C(200), C(201), C(203), C(204), C(206), C(207), C(209), C(210), C(212), C(213), C(215), C(216), C(218), C(219), C(221), C(222), C(224), C(225), C(227), C(228), C(230), C(231), C(233), C(234), C(236), C(237), C(239), C(240), C(242), C(243), C(245), C(246), C(248), C(249), C(251), C(252), C(254), C(255), C(257), C(258), C(260), C(261), C(263), C(266), C(267), C(269), C(270), C(272), C(275), C(276), C(278), C(279), C(281), C(282), C(284), C(287), C(290), C(291), C(293), C(294), C(296), C(299), C(300), C(302), C(303), C(305), C(306), C(308), C(311), C(314), C(315), C(317), C(318), C(320), C(323), C(324), C(326), C(327), C(329), C(330), C(332), C(335), C(338), C(339), C(341), C(342), C(344), C(347), C(348), C(350), C(351), C(353), C(354), C(356), C(359), C(362), C(363), C(365), C(366), C(368), C(371), C(372), C(374), C(375), C(377), C(378), C(380), C(383), C(386), C(387), C(389), C(390), C(392), C(395), C(396), C(398), C(399), C(401), C(402), C(404), C(407), C(410), C(411), C(413), C(414), C(416), C(419), C(420), C(422), C(423), C(425), C(426), C(428), C(431), C(434), C(435), C(437), C(438), C(440), C(443), C(444), C(446), C(447), C(449), C(450), C(452), C(455), C(458), C(459), C(461), C(462), C(464), C(467), C(468), C(470), C(471), C(473), C(474), C(476), C(479), C(482), C(483), C(485), C(486), C(488), C(491), C(492), C(494), C(495), C(497), C(498), C(500), C(503), C(506), C(507), C(509), C(510), C(512), C(515), C(516), C(518), C(519), C(521), C(522), C(524), C(527), C(530), C(531), C(533), C(534), C(536), C(539), C(540), C(542), C(543), C(545), C(546), C(548), C(551), C(554), C(555), C(557), C(558), C(560), C(563), C(564), C(566), C(567), C(569), C(570), C(572), C(575), C(578), C(579), C(581), C(582), C(584), C(587), C(588), C(590), C(591), C(593), C(594), C(596), C(599), C(602), C(603), C(605), C(606), C(608), C(611), C(612), C(614), C(615), C(617), C(618), C(620), C(623), C(626), C(627), C(629), C(630), C(632), C(635), C(636), C(638), C(639), C(641), C(642), C(644), C(647), C(650), C(651), C(653), C(654), C(656), C(659), C(660), C(662), C(663), C(665), C(666), C(668), C(671), C(674), C(675), C(677), C(678), C(680), C(683), C(684), C(686), C(687), C(689), C(690), C(692), C(695), C(698), C(699), C(701), C(702), C(704), C(707), C(708), C(710), C(711), C(713), C(714), C(716), C(719), C(722), C(723), C(725), C(726), C(728), C(731), C(734), C(737), C(740), C(743), C(746), C(749), C(752), C(755), C(758), C(761), C(764), C(767), C(770), C(773), C(776), C(779), C(782), C(785), C(788), C(791), C(794)

And the following 224 coded bits are moved to data block PB:

C(10), C(13), C(16), C(19), C(22), C(25), C(28), C(31), C(34), C(37), C(40), C(43), C(46), C(49), C(52), C(55), C(58), C(61), C(64), C(67), C(70), C(73), C(76), C(79), C(82), C(85), C(88), C(91), C(94), C(97), C(100), C(103), C(106), C(109), C(112), C(115), C(118), C(121), C(124), C(127), C(130), C(133), C(136), C(139), C(142), C(145), C(148), C(151), C(154), C(157), C(160), C(163), C(166), C(169), C(172), C(175), C(178), C(181), C(184), C(187), C(190), C(193), C(196), C(199), C(202), C(205), C(208), C(211), C(214), C(217), C(220), C(223), C(226), C(229), C(232), C(235), C(238), C(241), C(244), C(247), C(250), C(253), C(256), C(259), C(262), C(264), C(265), C(268), C(271), C(273), C(274), C(277), C(280), C(283), C(285), C(286), C(288), C(289), C(292), C(295), C(297), C(298), C(301), C(304), C(307), C(309), C(310), C(312), C(313), C(316), C(319), C(321), C(322), C(325), C(333), C(334), C(336), C(337), C(345), C(346), C(357), C(358), C(360), C(361), C(369), C(370), C(381), C(384), C(385), C(393), C(394), C(405), C(408), C(409), C(417), C(418), C(429), C(432), C(433), C(441), C(442), C(453), C(456), C(457), C(465), C(466), C(477), C(480), C(481), C(489), C(490), C(501), C(504), C(505), C(513), C(514), C(525), C(528), C(529), C(537), C(538), C(549), C(552), C(553), C(561), C(562), C(573), C(576), C(577), C(585), C(586), C(597), C(600), C(601), C(609), C(610), C(621), C(624), C(625), C(633), C(634), C(645), C(648), C(649), C(657), C(658), C(669), C(672), C(673), C(681), C(693), C(696), C(697), C(705), C(717), C(720), C(721), C(729), C(730), C(732), C(735), C(736), C(738), C(741), C(744), C(745), C(747), C(750), C(753), C(754), C(756), C(759), C(762), C(763), C(765), C(768), C(771), C(772), C(774), C(777), C(780), C(781), C(783), C(786)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/WHS8.85:

The block of 183 bits {u(0)… u(182)} is encoded with the ¼ rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 756 coded bits, {C(0)… C(755)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k+3)	= u(k)	for k = 0, 1, ..., 182;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(4k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(4k+1)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(4k+2)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(4k+3)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 183, 184, ..., 188

The following 448 coded bits are moved to data block PG:

C(3), C(7), C(11), C(15), C(16), C(19), C(20), C(23), C(24), C(27), C(28), C(31), C(32), C(33), C(35), C(36), C(39), C(40), C(41), C(43), C(44), C(47), C(48), C(49), C(51), C(52), C(55), C(56), C(57), C(59), C(60), C(63), C(64), C(65), C(67), C(68), C(71), C(72), C(73), C(75), C(76), C(79), C(80), C(81), C(83), C(84), C(87), C(88), C(89), C(91), C(92), C(95), C(96), C(97), C(99), C(100), C(103), C(104), C(105), C(107), C(108), C(111), C(112), C(113), C(115), C(116), C(119), C(120), C(121), C(123), C(124), C(127), C(128), C(129), C(131), C(132), C(135), C(136), C(137), C(139), C(140), C(143), C(144), C(145), C(147), C(148), C(151), C(152), C(153), C(155), C(156), C(159), C(160), C(161), C(163), C(164), C(167), C(168), C(169), C(171), C(172), C(175), C(176), C(177), C(179), C(180), C(183), C(184), C(185), C(187), C(188), C(191), C(192), C(193), C(195), C(196), C(199), C(200), C(201), C(203), C(204), C(207), C(208), C(209), C(211), C(212), C(215), C(216), C(217), C(219), C(220), C(223), C(224), C(225), C(227), C(228), C(231), C(232), C(233), C(235), C(236), C(239), C(240), C(241), C(243), C(244), C(247), C(248), C(249), C(251), C(252), C(255), C(256), C(257), C(259), C(260), C(263), C(264), C(265), C(267), C(268), C(271), C(272), C(273), C(275), C(276), C(279), C(280), C(281), C(283), C(284), C(287), C(288), C(289), C(291), C(292), C(295), C(296), C(297), C(299), C(300), C(303), C(304), C(305), C(307), C(308), C(311), C(312), C(313), C(315), C(316), C(319), C(320), C(321), C(323), C(324), C(327), C(328), C(329), C(331), C(332), C(335), C(336), C(337), C(339), C(340), C(343), C(344), C(345), C(347), C(348), C(351), C(352), C(353), C(355), C(356), C(359), C(360), C(361), C(363), C(364), C(367), C(368), C(369), C(371), C(372), C(375), C(376), C(377), C(379), C(380), C(383), C(384), C(385), C(387), C(388), C(391), C(392), C(393), C(395), C(396), C(399), C(400), C(401), C(403), C(404), C(407), C(408), C(409), C(411), C(412), C(415), C(416), C(417), C(419), C(420), C(423), C(424), C(425), C(427), C(428), C(431), C(432), C(433), C(435), C(436), C(439), C(440), C(441), C(443), C(444), C(447), C(448), C(449), C(451), C(452), C(455), C(456), C(457), C(459), C(460), C(463), C(464), C(465), C(467), C(468), C(471), C(472), C(473), C(475), C(476), C(479), C(480), C(481), C(483), C(484), C(487), C(488), C(489), C(491), C(492), C(495), C(496), C(497), C(499), C(500), C(503), C(504), C(505), C(507), C(508), C(511), C(512), C(513), C(515), C(516), C(519), C(520), C(521), C(523), C(524), C(527), C(528), C(529), C(531), C(532), C(535), C(536), C(537), C(539), C(540), C(543), C(544), C(545), C(547), C(548), C(551), C(552), C(555), C(556), C(559), C(560), C(561), C(563), C(564), C(567), C(568), C(569), C(571), C(572), C(575), C(576), C(577), C(579), C(580), C(583), C(584), C(585), C(587), C(588), C(591), C(592), C(595), C(596), C(599), C(600), C(603), C(604), C(607), C(608), C(609), C(611), C(612), C(615), C(616), C(617), C(619), C(620), C(623), C(624), C(625), C(627), C(628), C(631), C(632), C(633), C(635), C(636), C(639), C(640), C(643), C(644), C(647), C(648), C(651), C(652), C(655), C(656), C(657), C(659), C(660), C(663), C(664), C(665), C(667), C(668), C(671), C(672), C(673), C(675), C(676), C(679), C(680), C(681), C(683), C(684), C(687), C(688), C(691), C(692), C(695), C(696), C(699), C(700), C(703), C(704), C(705), C(707), C(708), C(711), C(712), C(713), C(715), C(716), C(719), C(720), C(723), C(727), C(728), C(731), C(732), C(735), C(736), C(739), C(743), C(747), C(751), C(755)

And the following 224 coded bits are moved to data block PB:

C(4), C(8), C(12), C(13), C(17), C(21), C(22), C(25), C(26), C(29), C(30), C(34), C(37), C(38), C(42), C(45), C(46), C(50), C(53), C(54), C(58), C(61), C(62), C(66), C(69), C(70), C(74), C(77), C(78), C(82), C(85), C(86), C(90), C(93), C(94), C(98), C(101), C(102), C(106), C(109), C(110), C(114), C(117), C(118), C(122), C(125), C(126), C(130), C(133), C(134), C(138), C(141), C(142), C(146), C(149), C(150), C(154), C(157), C(158), C(162), C(165), C(166), C(170), C(173), C(174), C(178), C(181), C(182), C(186), C(189), C(190), C(194), C(197), C(198), C(202), C(205), C(206), C(210), C(213), C(214), C(218), C(221), C(222), C(226), C(229), C(230), C(234), C(237), C(238), C(242), C(245), C(246), C(250), C(253), C(254), C(258), C(261), C(262), C(266), C(269), C(270), C(274), C(277), C(278), C(282), C(285), C(286), C(290), C(293), C(294), C(298), C(301), C(302), C(306), C(309), C(310), C(314), C(317), C(318), C(322), C(325), C(326), C(330), C(333), C(334), C(338), C(341), C(342), C(346), C(349), C(350), C(354), C(357), C(358), C(362), C(365), C(366), C(370), C(373), C(374), C(378), C(381), C(382), C(386), C(389), C(394), C(397), C(402), C(405), C(413), C(418), C(421), C(429), C(434), C(437), C(445), C(450), C(453), C(461), C(466), C(469), C(477), C(482), C(485), C(493), C(498), C(501), C(509), C(514), C(517), C(525), C(530), C(533), C(541), C(549), C(553), C(557), C(562), C(565), C(570), C(573), C(578), C(581), C(589), C(593), C(597), C(601), C(605), C(610), C(613), C(618), C(621), C(626), C(629), C(637), C(641), C(645), C(649), C(653), C(658), C(661), C(666), C(669), C(674), C(677), C(685), C(689), C(693), C(697), C(701), C(706), C(709), C(714), C(717), C(721), C(722), C(724), C(725), C(729), C(733), C(737), C(740), C(741), C(744)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

O-TCH/WHS6.60:

The block of 138 bits {u(0)… u(137)} is encoded with the 1/5 rate convolutional code defined by	the following polynomials:

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G4/G7 = 1 + D2  + D3 + D5 + D6/1 + D + D2 + D3 + D6

G5/G7 = 1 + D + D4 + D6/1 + D + D2 + D3 + D6

G6/G7 = 1 + D + D2 + D3 + D4 + D6/1 + D + D2 + D3 + D6

G7/G7 = 1

resulting in 720 coded bits, {C(0)… C(719)} defined by:

r(k)		= u(k) + r(k-1) + r(k-2) + r(k-3) + r(k-6)

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= u(k)	for k = 0, 1, ..., 137;	r(k) = 0 for k<0

and (for termination of the coder):

r(k)		= 0

C(5k)		= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+1)	= r(k) + r(k-2) + r(k-3) + r(k-5) + r(k-6)

C(5k+2)	= r(k) + r(k-1) + r(k-4) + r(k-6)

C(5k+3)	= r(k) + r(k-1) + r(k-2) + r(k-3) + r(k-4) + r(k-6)

C(5k+4)	= r(k-1) + r(k-2) + r(k-3) + r(k-6)	for k = 138, 139, ..., 143

The following 448 coded bits are moved to data block PG:

C(4), C(9), C(11), C(12), C(14), C(16), C(17), C(19), C(21), C(22), C(24), C(26), C(27), C(29), C(31), C(32), C(34), C(36), C(37), C(38), C(39), C(41), C(42), C(44), C(46), C(47), C(48), C(49), C(51), C(52), C(54), C(56), C(57), C(59), C(60), C(61), C(62), C(63), C(64), C(66), C(67), C(69), C(71), C(72), C(73), C(74), C(76), C(77), C(79), C(81), C(82), C(84), C(85), C(86), C(87), C(88), C(89), C(91), C(92), C(94), C(96), C(97), C(98), C(99), C(101), C(102), C(104), C(106), C(107), C(109), C(111), C(112), C(113), C(114), C(116), C(117), C(119), C(121), C(122), C(123), C(124), C(126), C(127), C(129), C(131), C(132), C(134), C(135), C(136), C(137), C(138), C(139), C(141), C(142), C(144), C(146), C(147), C(148), C(149), C(151), C(152), C(154), C(156), C(157), C(159), C(161), C(162), C(163), C(164), C(166), C(167), C(169), C(171), C(172), C(173), C(174), C(176), C(177), C(179), C(181), C(182), C(184), C(186), C(187), C(188), C(189), C(191), C(192), C(194), C(196), C(197), C(198), C(199), C(201), C(202), C(204), C(206), C(207), C(209), C(210), C(211), C(212), C(213), C(214), C(216), C(217), C(219), C(221), C(222), C(223), C(224), C(226), C(227), C(229), C(231), C(232), C(234), C(236), C(237), C(238), C(239), C(241), C(242), C(244), C(246), C(247), C(248), C(249), C(251), C(252), C(254), C(256), C(257), C(259), C(261), C(262), C(263), C(264), C(266), C(267), C(269), C(271), C(272), C(273), C(274), C(276), C(277), C(279), C(281), C(282), C(284), C(285), C(286), C(287), C(288), C(289), C(291), C(292), C(294), C(296), C(297), C(298), C(299), C(301), C(302), C(304), C(306), C(307), C(309), C(311), C(312), C(313), C(314), C(316), C(317), C(319), C(321), C(322), C(323), C(324), C(326), C(327), C(329), C(331), C(332), C(334), C(336), C(337), C(338), C(339), C(341), C(342), C(344), C(346), C(347), C(349), C(351), C(352), C(354), C(356), C(357), C(359), C(361), C(362), C(364), C(366), C(367), C(369), C(371), C(372), C(374), C(376), C(377), C(379), C(381), C(382), C(384), C(386), C(387), C(389), C(391), C(392), C(394), C(396), C(397), C(399), C(401), C(402), C(404), C(406), C(407), C(409), C(411), C(412), C(414), C(416), C(417), C(419), C(421), C(422), C(424), C(426), C(427), C(429), C(431), C(432), C(434), C(436), C(437), C(439), C(441), C(442), C(444), C(446), C(447), C(449), C(451), C(452), C(454), C(456), C(457), C(459), C(461), C(462), C(464), C(466), C(467), C(469), C(471), C(472), C(474), C(476), C(477), C(479), C(481), C(482), C(484), C(486), C(487), C(489), C(491), C(492), C(494), C(496), C(499), C(501), C(502), C(504), C(506), C(507), C(509), C(511), C(512), C(514), C(516), C(517), C(519), C(521), C(524), C(526), C(527), C(529), C(531), C(532), C(534), C(536), C(537), C(539), C(541), C(542), C(544), C(546), C(549), C(551), C(552), C(554), C(556), C(557), C(559), C(561), C(562), C(564), C(566), C(567), C(569), C(571), C(574), C(576), C(577), C(579), C(581), C(582), C(584), C(586), C(587), C(589), C(591), C(592), C(594), C(596), C(599), C(601), C(602), C(604), C(606), C(607), C(609), C(611), C(612), C(614), C(616), C(617), C(619), C(621), C(624), C(626), C(627), C(629), C(631), C(632), C(634), C(636), C(637), C(639), C(641), C(642), C(644), C(646), C(649), C(651), C(652), C(654), C(656), C(657), C(659), C(661), C(662), C(664), C(666), C(667), C(669), C(671), C(674), C(676), C(677), C(679), C(681), C(682), C(684), C(686), C(687), C(689), C(691), C(692), C(694), C(696), C(699), C(701), C(702), C(704), C(706), C(707), C(709), C(711), C(712), C(714), C(716), C(719)

And the following 224 coded bits are moved to data block PB:

C(18), C(23), C(25), C(28), C(30), C(33), C(35), C(40), C(43), C(45), C(50), C(53), C(55), C(58), C(65), C(68), C(70), C(75), C(78), C(80), C(83), C(90), C(93), C(95), C(100), C(103), C(105), C(108), C(110), C(115), C(118), C(120), C(125), C(128), C(130), C(133), C(140), C(143), C(145), C(150), C(153), C(155), C(158), C(160), C(165), C(168), C(170), C(175), C(178), C(180), C(183), C(185), C(190), C(193), C(195), C(200), C(203), C(205), C(208), C(215), C(218), C(220), C(225), C(228), C(230), C(233), C(235), C(240), C(243), C(245), C(250), C(253), C(255), C(258), C(260), C(265), C(268), C(270), C(275), C(278), C(280), C(283), C(290), C(293), C(295), C(300), C(303), C(305), C(308), C(310), C(315), C(318), C(320), C(325), C(328), C(330), C(333), C(335), C(340), C(343), C(345), C(348), C(350), C(353), C(355), C(358), C(360), C(363), C(365), C(368), C(370), C(373), C(375), C(378), C(380), C(383), C(385), C(388), C(390), C(393), C(395), C(398), C(400), C(403), C(405), C(408), C(410), C(413), C(415), C(418), C(420), C(423), C(425), C(428), C(430), C(433), C(435), C(438), C(440), C(443), C(445), C(448), C(450), C(453), C(455), C(458), C(460), C(463), C(465), C(468), C(470), C(473), C(475), C(478), C(480), C(483), C(485), C(488), C(490), C(493), C(495), C(497), C(498), C(503), C(505), C(508), C(510), C(513), C(515), C(518), C(522), C(523), C(528), C(530), C(533), C(538), C(540), C(543), C(547), C(548), C(553), C(555), C(558), C(560), C(563), C(565), C(568), C(572), C(573), C(578), C(580), C(583), C(588), C(590), C(593), C(597), C(598), C(603), C(605), C(608), C(610), C(613), C(615), C(618), C(622), C(623), C(628), C(630), C(633), C(638), C(640), C(643), C(647), C(648), C(653), C(658), C(660), C(663), C(668), C(672), C(673), C(678), C(683), C(697)

The vectors PG and PB of coded and punctured bits is combined with in band bits to vector PC' as

PC' (k) = ic(k)					for k = 0, 1, 2, 3

PC' (k+4) = PG(k)				for k = 0, 1, ..., 223

PC' (k+224) = ic(k)			for k = 4, 5, 6, 7

PC' (k+8) = PG(k)				for k = 224, 225, ..., 447

PC' (k+448) = ic(k)			for k = 8, 9, 10, 11

PC' (k+460) = PB(k)			for k = 0, 1, ..., 223

#### 3.17.7.5	Interleaving

The interleaving is done as specified for the O-TCH/AHS (subclause 3.15.7.5).

#### 3.17.7.6	Mapping on a Burst

The mapping is done as specified for the O-TCH/AHS (subclause 3.15.7.6).

### 3.17.8	RATSCCH_MARKER

This frame type contains the in-band channel and an identification marker. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.17.8.1	Coding of in-band data

The coding of in-band data is done as specified for the RATSCCH_MARKER frame in O-TCH/AHS (subclause 3.15.8.1).

#### 3.17.8.2	Identification marker

The identification marker is done as specified for the RATSCCH_MARKER frame in O-TCH/AHS (subclause 3.15.8.2).

#### 3.17.8.3	Interleaving

The interleaving is done as specified for the RATSCCH_MARKER frame in O-TCH/AHS (subclause 3.15.8.3).

#### 3.17.8.4	Mapping on a Burst

The mapping is done as specified for the RATSCCH_MARKER frame in O-TCH/AHS (subclause 3.15.8.4).

### 3.17.9	RATSCCH_DATA

This frame contains the RATSCCH data and an inband channel. The RATSCCH data consists of 35 bits. The in-band data id(0,1) represents Mode Indication or Mode Command/Mode Request depending on the current frame number.

#### 3.17.9.1	Coding of in-band data

The coding of in-band data is done as specified for the RATSCCH_DATA frame in O-TCH/AHS (subclause 3.15.9.1).

#### 3.17.9.2	Parity and convolutional encoding for the RATSCCH message

The parity and convolutional encoding for the RATSCCH message are done as specified for the RATSCCH_DATA frame in O-TCH/AHS (subclause 3.15.9.2).

#### 3.17.9.3	Interleaving

The interleaving is done as specified for the RATSCCH_DATA frame in O-TCH/AHS (subclause 3.15.9.3).

#### 3.17.9.4	Mapping on a Burst

The mapping is done as specified for the RATSCCH_DATA frame in O-TCH/AHS (subclause 3.15.9.4).

# 4	Control Channels

## 4.1	Slow associated control channel (SACCH)

### 4.1.1	Block constitution

The message delivered to the encoder has a fixed size of 184 information bits {d(0),d(1),...,d(183)}. It is delivered on a burst mode.

### 4.1.2	Block code

a)	Parity bits:

The block of 184 information bits is protected by 40 extra bits used for error correction and detection. These bits are added to the 184 bits according to a shortened binary cyclic code (FIRE code) using the generator polynomial:

g(D) = (D23 + 1)*(D17 + D3 + 1)

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

d(0)D223 + d(1)D222 +...+d(183)D40 + p(0)D39 + p(1)D38 +...+p(38)D + p(39)

where {p(0),p(1),...,p(39)} are the parity bits , when divided by g(D) yields a remainder equal to:

1 + D + D2 +...+ D39.

b)	Tail bits

Four tail bits equal to 0 are added to the information and parity bits, the result being a block of 228 bits.

u(k) = d(k)		for k= 0,1,...,183

u(k) = p(k-184)	for k = 184,185,...,223

u(k) = 0			for k = 224,225,226,227 (tail bits)

### 4.1.3	Convolutional encoder

This block of 228 bits is encoded with the ½ rate convolutional code (identical to the one used for TCH/FS) defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

This results in a block of 456 coded bits: {c(0),c(1),...,c(455)} defined by:

c(2k)		= u(k) + u(k-3) + u(k-4)

c(2k+1)		= u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,227 ; u(k) = 0 for k < 0

### 4.1.4	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)	for	k = 0,1,...,455

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 4)

j  = 2((49k) mod 57) + ((k mod 8) div 4)

See table 1. The result of the reordering of bits is the same as given for a TCH/FS (subclause 3.1.3) as can be seen from the evaluation of the bit number-index j, distributing the 456 bits over 4 blocks on even numbered bits and 4 blocks on odd numbered bits. The resulting 4 blocks are built by putting blocks with even numbered bits and blocks with odd numbered bits together into one block.

The block of coded data is interleaved "block rectangular" where a new data block starts every 4th block and is distributed over 4 blocks.

### 4.1.5	Mapping on a Burst

The mapping is given by the rule:

e(B,j)		= i(B,j)	and	e(B,59+j) = i(B,57+j)	for j = 0,1,...,56

and

e(B,57)		= hl(B)	and	e(B,58) = hu(B)

The two bits labelled hl(B) and hu(B) on burst number B are flags used for indication of control channel signalling. They are set to "1" for a SACCH.

## 4.2	Fast associated control channel at full rate (FACCH/F)

### 4.2.1	Block constitution

The message delivered to the encoder has a fixed size of 184 information bits. It is delivered on a burst mode.

### 4.2.2	Block code

The block encoding is done as specified for the SACCH in subclause 4.1.2.

### 4.2.3	Convolutional encoder

The convolutional encoding is done as specified for the SACCH in subclause 4.1.3.

### 4.2.4	Interleaving

The interleaving is done as specified for the TCH/FS in subclause 3.1.3.

### 4.2.5	Mapping on a Burst

A FACCH/F frame of 456 coded bits is mapped on 8 consecutive bursts as specified for the TCH/FS in subclause 3.1.4. As a FACCH is transmitted on bits which are stolen in a burst from the traffic channel, the even numbered bits in the first 4 bursts and the odd numbered bits of the last 4 bursts are stolen.

To indicate this to the receiving device the flags hl(B) and hu(B) have to be set according to the following rule:

hu(B) = 1 for the first 4 bursts	(even numbered bits are stolen);

hl(B)  = 1 for the last 4 bursts	(odd numbered bits are stolen).

The consequences of this bitstealing by a FACCH/F is for a:

-	speech channel (TCH/FS) and data channel (TCH/F2.4):

One full frame of data is stolen by the FACCH.

-	Data channel (TCH/F14.4):

The bitstealing by a FACCH/F disturbs a maximum of 96 of the 456 coded bits generated from an input data block of 290 bits.

-	Data channel (TCH/F9.6):

The bitstealing by a FACCH/F disturbs a maximum of 96 coded bits generated from an input frame of four data blocks. A maximum of 24 of the 114 coded bits resulting from one input data block of 60 bits may be disturbed.

-	Data channel (TCH/F4.8):

The bit stealing by FACCH/F disturbs a maximum of 96 coded bits generated from an input frame of two data blocks. A maximum of 48 of the 228 coded bits resulting from one input data block of 60 bits may be disturbed.

NOTE:	In the case of consecutive stolen frames, a number of bursts will have both the even and the odd bits stolen and both flags hu(B) and hl(B) must be set to 1.

## 4.3	Fast associated control channel at half rate (FACCH/H)

### 4.3.1	Block constitution

The message delivered to the encoder has a fixed size of 184 information bits. It is delivered on a burst mode.

### 4.3.2	Block code

The block encoding is done as specified for the SACCH in subclause 4.1.2.

### 4.3.3	Convolutional encoder

The convolutional encoding is done as specified for the SACCH in subclause 4.1.3.

### 4.3.4	Interleaving

The coded bits are reordered and interleaved according to the following rule:

i(B,j) = c(n,k)	for	k = 0,1,...,455

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 8) - 4((k mod 8) div 6)

j  = 2((49k) mod 57) + ((k mod 8) div 4)

See table 1. The result of the reordering of bits is the same as given for a TCH/FS (subclause 3.1.3) as can be seen from the evaluation of the bit number-index j, distributing the 456 bits over 4 blocks on even numbered bits and 4 blocks on odd numbered bits. The 2 last blocks with even numbered bits and the 2 last blocks with odd numbered bits are put together into 2 full middle blocks.

The block of coded data is interleaved "block diagonal" where a new data block starts every 4th block and is distributed over 6 blocks.

### 4.3.5	Mapping on a Burst

A FACCH/H frame of 456 coded bits is mapped on 6 consecutive bursts by the rule:

e(B,j) = i(B,j)		and	e(B,59+j) = i(B,57+j)	for	j = 0,1,...,56

and

e(B,57) = hl(B)	and	e(B,58) = hu(B)

As a FACCH/H is transmitted on bits which are stolen from the traffic channel, the even numbered bits of the first 2 bursts, all bits of the middle 2 bursts and the odd numbered bits of the last 2 bursts are stolen.

To indicate this to the receiving device the flags hl(B) and hu(B) have to be set according to the following rule:

hu(B) = 1					for the first 2 bursts (even numbered bits are stolen)

hu(B) = 1 and hl(B) = 1	for the middle 2 bursts (all bits are stolen)

hl(B) = 1					for the last 2 bursts (odd numbered bits are stolen)

The consequences of this bitstealing by a FACCH/H is for a:

-	speech channel (TCH/HS):

two full consecutive speech frames are stolen by a FACCH/H.

-	data channel (TCH/H4.8):

The bitstealing by FACCH/H disturbs a maximum of 96 coded bits generated from an input frame of four data blocks. A maximum of 24 out of the 114 coded bits resulting from one input data block of 60 bits may be disturbed.

-	data channel (TCH/H2.4):

The bitstealing by FACCH/H disturbs a maximum of 96 coded bits generated from an input frame of four data blocks. A maximum of 24 out of the 114 coded bits resulting from one input data block of 36 bits may be disturbed.

NOTE:	In the case of consecutive stolen frames, two overlapping bursts will have both the even and the odd numbered bits stolen and both flags hu(B) and hl(B) must be set to 1.

## 4.4	Broadcast control, Paging, Access grant, Notification and Cell broadcast channels (BCCH, PCH, AGCH, NCH, CBCH), CTS Paging and Access grant channels (CTSPCH, CTSAGCH)

The coding scheme used for the broadcast control , paging, access grant, notification and cell broadcast messages is the same as for the SACCH messages, specified in subclause 4.1. In CTS, the coding scheme used for the paging and access grant messages is also the same as for the SACCH messages, specified in subclause 4.1.

## 4.5	Stand-alone dedicated control channel (SDCCH)

The coding scheme used for the dedicated control channel messages is the same as for SACCH messages, specified in subclause 4.1.

## 4.6	Random access channel (RACH)

Three coding schemes are specified for the burst carrying the random access uplink message:

the access burst containing 8 information bits,

the access burst containing 11 information bits and

the access burst containing 30 information bits.

The encoding of the access burst containing 8 information bits is as defined in subclause 4.6.1.

The encoding of the access burst containing 11 information bits is as defined in sub-clause 5.3.2 for the packet random access channel (PRACH, CPRACH and MPRACH).

The encoding of the access burst containing 30 information bits is as defined in sub-clause 4.6.2. It is used for the Multilateration Timing Advance procedure when the Extended Access Burst method is used (see 3GPP TS 43.059).

### 4.6.1	RACH carrying 8 information bits

The encoding of the access burst containing 8 information bits is defined as follows. It contains 8 information bits 
d(0),d(1),...,d(7).

Six parity bits p(0),p(1),...,p(5) are defined in such a way that in GF(2) the binary polynomial:

d(0)D13 +...+ d(7)D6 + p(0)D5 +...+ p(5), when divided by D6 + D5 + D3 + D2 + D + 1 yields a remainder equal to D5 + D4 + D3 + D2 + D + 1.

The six bits of the BSIC, {B(0),B(1),...,B(5)}, of the BS to which the Random Access is intended, are added bitwise modulo 2 to the six parity bits, {p(0),p(1),...,p(5)}. This results in six colour bits, C(0) to C(5) defined as C(k) = b(k) + p(k) (k = 0 to 5) where:

b(0) = MSB of PLMN colour code

b(5) = LSB of BS colour code.

This defines {u(0),u(1),..., u(17)} by:

u(k)	= d(k)		for k = 0,1,...,7

u(k)	= C(k-8)	for k = 8,9,...,13

u(k)	= 0			for k = 14,15,16,17 (tail bits)

The bits {e(0),e(1),..., e(35)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

e(2k)		= u(k) + u(k-3) + u(k-4)

e(2k+1)	= u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,17 ; u(k) = 0 for k < 0

### 4.6.2	(EC-)RACH carrying 30 information bits

In case of the Multilateration Timing Advance procedure when the Extended Access Burst method is used, indicated by the network, the mobile station first sends a random access message carrying 11 information bits, using the encoding for the access burst containing 11 bits, as specified in sub-clause 5.3.2, and after access grant by the network sends a subsequent random access message carrying 30 information bits as soon as possible after receiving the access grant while still using the principles for (EC-)RACH transmission opportunity selection described in 3GPP TS 44.018. The encoding of the 30 information bits is as given in the following sub-clauses.

#### 4.6.2.1	Block constitution

The burst carrying the extended random access uplink message for connectionless Multilateration Positioning contains 30 information bits d(0),d(1),...,d(29). The first 11 bits d(0),...,d(10) are encoded in the same way as specified for the access burst containing 11 information bits in sub-clause 5.3.2 resulting in a block of 36 coded bits {e(0), e(1),...,e(35)}. The encoding of the remaining 19 information bits d(11),...,d(29) is specified in the following sub-clauses.

#### 4.6.2.2	Parity bits

Six parity bits p(0),p(1),...,p(5) are defined in such a way that in GF(2) the binary polynomial:

d(11)D24 +...+ d(29)D6 + p(0)D5 +...+ p(5), when divided by D6 + D5 + D3 + D2 + D + 1 yields a remainder equal to D5 + D4 + D3 + D2 + D + 1.

#### 4.6.2.3	Tail bits

Four tail bits are added to information bits and parity bits defining the bit sequence{u(0),u(1),..., u(28)} as follows:

u(k-11) = d(k)		for k = 11,12,...,29

u(k+19) = p(k)		for k = 0,...,5

u(k+25) = 0			for k = 0,1,2,3 (tail bits)

#### 4.6.2.4	Convolutional code

The coded bits {c(0),c(1),..., c(57)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

c(2k) = u(k) + u(k-3) + u(k-4)

c(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,28 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bit:

c(57) is not transmitted.

This results in a block of 57 coded bits, {e(36),...,e(92)} with

e(k+36) = c(k)	for k = 0,1,...,56

#### 4.6.2.5	Burst mapping

Both data blocks {e(0),...,e(35)} and {e(36),...,e(92)} are mapped onto the Extended Access burst as specified in 3GPP TS 45.002.

## 4.7	Synchronization channel (SCH), Compact synchronization channel (CSCH), CTS Beacon and Access request channels (CTSBCH-SB, CTSARCH)

The burst carrying the synchronization information on the downlink BCCH, the downlink CPBCCH for Compact, and in CTS the information of the CTSBCH-SB and the access request message of the CTSARCH, has a different structure. It contains 25 information bits {d(0),d(1),..., d(24)}, 10 parity bits {p(0),p(1),..., p(9)} and 4 tail bits. The precise ordering of the information bits is given in 3GPP TS 44.018.

The ten parity bits {p(0),p(1),,...,p(9)} are defined in such a way that in GF(2) the binary polynomial:

d(0)D34 +...+ d(24)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D8 + D6 + D5 + D4 + D2 + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 +  D5 +  D4 + D3 + D2 + D+ 1.

Thus the encoded bits {u(0),u(1),...,u(38)} are:

u(k)	= d(k)			for k = 0,1,...,24

u(k)	= p(k-25)		for k = 25,26,...,34

u(k)	= 0				for k = 35,36,37,38 (tail bits)

The bits {e(0),e(1),..., e(77)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

e(2k)		= u(k) + u(k-3) + u(k-4)

e(2k+1)	= u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,....,38 ; u(k) = 0 for k < 0

## 4.7a	Extended Coverage Synchronization channel (EC-SCH)

### 4.7a.1	Block constitution

The burst carrying the synchronization information on the downlink EC-SCH contains 30 information bits {d(0),d(1),..., d(29)}.

### 4.7a.2	Coding

The 30 information bits {d(0),d(1),..., d(29)} are encoded as follows:

a)	Parity bits:

Ten parity bits {p(0),p(1),,...,p(9)} are defined in such a way that in GF(2) the binary polynomial:

d(0)D39 +...+ d(29)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D8 + D6 + D5 + D4 + D2 + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 +  D5 +  D4 + D3 + D2 + D+ 1.

b)	Tail bits:

Four tail bits equal to 0 are added to the information and parity bits, the results being a block of 44 bits {u(0),u(1),...,u(43)}:

u(k)	= d(k)			for k = 0,1,...,29

u(k)	= p(k-30)		for k = 30,31,...,39

u(k)	= 0				for k = 40,41,42,43 (tail bits)

c)	Convolutional encoder:

This block of 44 bits {u(0),u(1),...,u(43)} is encoded with the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

This results in a block of 88 coded bits {C(0),C(1),...,C(87)} defined by:

C(2k)		= u(k) + u(k-3) + u(k-4)

C(2k+1)	= u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,....,43 ; u(k) = 0 for k < 0

The block is punctured such that the bits C(k), k=[0,10,19,29,39,48,58,68,77,87] are not transmitted, resulting in a block of 78 encoded bits {e(0),e(1),..., e(77)}.

### 4.7a.3	Blind physical layer transmission

The encoded bits e(j), j=0,…,77 are transmitted 28 times, generating the repeated bursts R(m,B,j)=e(j), m=0,…,27, B=0, j=0,…,77.

### 4.7a.4	Cyclic shift

The repeated bursts are cyclically shifted as:

R’(m,B,j) = R(m,B,(j + T2'') mod 78)

where T2'' = (FN div 51) mod 4, i.e. it refers to the specific 51-multiframe FN, in which the burst will be sent in the set of four contiguous 51-multiframes as defined in TS 45.002 [8].

### 4.7a.5	Mapping onto a physical channel

Burst R’(m,B,j), is mapped onto burst B’ of the timeslot carrying the EC-SCH, where

B’ = m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

## 4.8	Access Burst on circuit switched channels other than RACH

The encoding of this burst is as defined in subclause 4.6 for the 8 bits access burst on the random access channel (RACH). The BSIC used shall be the BSIC of the BTS to which the burst is intended.

## 4.9	Access Bursts for uplink access on a channel used for VGCS or VBS

The encoding of this burst is as defined in subclause 4.6 for the 8 bits access burst on the RACH. The BSIC used by the Mobile Station shall be the BSIC indicated by network signalling, or if not thus provided, the last received BSIC on the SCH of the current cell.

## 4.10a	Fast associated control channel at ECSD E-TCH/F (E-FACCH/F)

### 4.10a.1	Block constitution

The message delivered to the encoder has a fixed size of 184 information bits. It is delivered on a burst mode.

### 4.10a.2	Block code

The block encoding is done as specified for the SACCH in subclause 4.1.2.

### 4.10a.3	Convolutional encoder

The convolutional encoding is done as specified for the SACCH in subclause 4.1.3.

### 4.10a.4	Interleaving

The interleaving is done as specified for the SACCH in subclause 4.1.4.

### 4.10a.5	Mapping on a Burst

A E-FACCH/F frame of 456 coded bits is mapped on 4 full consecutive bursts. As a E-FACCH/F is transmitted on bits, which are stolen in a burst from the ECSD traffic channel, the four full bursts are stolen.

The mapping on is given by the rule:

e(B,j)=i(B,j) and e(B,59+j)=i(B,57+j)	for j=0,1,…,56

and

e(B,57)=hl(B) and e(B,58)=hu(B).

To indicate to the receiving device the flags hl(B) and hu(B) have to be set according to the following rule:

hu(B)=1 and  hl(B)=1 for the all 4 bursts (4 full bursts are stolen).

The consequences of this bitstealing by a E-FACCH/F is for a:

-	Data channel (E-TCH/F43.2)

The bitstealing by a E-FACCH/F disturbs a maximum of 288 of the 1368 coded bits generated from an input data block of 870 bits.

-	Data channel (E-TCH/F32.0)

The bitstealing by a E-FACCH/F disturbs 464 of the 1392 coded bits generated from an input data block of 640 bits.

-	Data channel (E-TCH/F28.8)

The bitstealing by a E-FACCH/F disturbs a maximum of 288 of the 1368 coded bits generated from an input data block of 580 bits.

## 4.10b	Octal fast associated control channel at half rate (O-FACCH/H)

### 4.10b.1		Block constitution

The message delivered to the encoder has a fixed size of 184 information bits. It is delivered on a burst mode.

### 4.10b.2		Block code

a)	Parity bits:

The block of 184 information bits is protected by 40 extra bits used for error correction and detection. These bits are added to the 184 bits according to a shortened binary cyclic code (FIRE code) using the generator polynomial:

G(D)=(D23 + 1)(D17 + D3 + 1)

The encoding of the cyclic code is performed in a systematic form, which means that, in GF(2), the polynomial:

D(0)D223 + d(1)D222 +…+ d(183)D40 + p(0)D39 + p(1)D38 +…+ p(38)D + p(39)

where {p(0),p(1),…,p(39)} are the parity bits, when divided by g(D) yields a remainder equal to:

1 + D + D2 +… +D39.

b)	Tail bits

Six tail bits equal to zero are added to the information and parity bits, the result being a block of 230 bits.

u(k) = d(k)	for k = 0,1,…,183

u(k) = p(k-184)	for k = 184,185,…,223

u(k) = 0	for k = 224,225,226,227,228,229 (tail bits)

### 4.10b.3		Convolutional encoder

This block of 230 bits is encoded with the rate 1/6 convolutional code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G4 = 1 + D2 + D3 + D5 + D6

G5 = 1 + D + D4 + D6

G5 = 1 + D + D4 + D6

G6 = 1 + D + D2 + D3 + D4 + D6

G7 = 1 + D + D2 + D3 + D6

This results in a block of 1380 encoded bits {C(0),C(1),…C(1379)} defined by

C(6k) = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(6k+1) = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(6k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)

C(6k+3) = u(k) + u(k-1) + u(k-4) + u(k-6)

C(6k+4) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-4) + u(k-6)

C(6k+5) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)		for k = 0,1,...,229 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits:

{C(21+114k) for k=0,1,..,11} are not transmitted.

The result is a block of 1368 coded bits {c(0),c(1),…,c(1367)}.

### 4.10b.4		Reordering

The coded bits are reordered according to the following rule:

r(j) = c(k),	for	k  = 0,1,...,1367

j   = k div 36 + 38*(k mod 36)

NOTE:	The reordering is a simple block interleaver: a 38 rows x 36 columns matrix which is filled in by row and read out by column.

### 4.10b.5		Interleaving

Before interleaving the reordered coded bits {r(0),r(1),…,r(1367)} are converted into 3-bit symbols {Rs(0),Rs(1),…,Rs(455)} according to Table 1 in 3GPP TS 45.004, the symbol Rs(k) depends on r(3k+2), r(3k+1), and r(3k) for k=0,1,…,455. The interleaving is done as specified for the FACCH at half rate in subclause 4.3.4. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

### 4.10b.6		Mapping on a burst

As an O-FACCH is transmitted on symbols which are stolen in a burst from the traffic channel, the even numbered symbols in the first 2 bursts, all symbols in the middle 2 bursts, and the odd numbered symbols in the last 2 bursts are stolen.

The mapping is given by the rule:

E(B,j) = I(B,j) and E(B,59+j) = I(B,57+j) for j=0,1,…,56

and

E(B,57) = HL(B) and E(B,58) = HU(B).

To indicate the stealing to the receiving device the symbols HL(B) and HU(B) have to be set according to the following rule:

HU(B) = {1,1,1} for the first two bursts (even numbered symbols are stolen)

HU(B) = {1,1,1} and HL(B) = {1,1,1} for the middle two bursts (all symbols are stolen)

HL(B) = {1,1,1} for the last two burts (odd numbered symbols are stolen).

As a consequence, two full consecutive speech frames of an O-TCH/AHS are stolen by an O-FACCH/H.

## 4.10c	Octal fast associated control channel at full rate (O-FACCH/F)

### 4.10c.1		Block constitution

The message delivered to the encoder has a fixed size of 184 information bits. It is delivered on a burst mode.

### 4.10c.2		Block code

The block encoding is done as specified for the O-FACCH/H in subclause 4.10b.2

### 4.10c.3		Convolutional encoder

The convolutional encoding is done as specified for the O-FACCH/H in subclause 4.10b.3.

### 4.10c.4		Reordering

The reordering is done as specified for the O-FACCH/H in subclause 4.10b.4.

### 4.10c.5		Interleaving

Before interleaving the reordered coded bits {r(0),r(1),…,r(1367)} are converted into 3-bit symbols {Rs(0),Rs(1),…,Rs(455)} according to Table 1 in 3GPP TS 45.004, the symbol Rs(k) depends on r(3k+2), r(3k+1), and r(3k) for k=0,1,…,455. The interleaving is done as specified for the FACCH at full rate in subclause 4.2.4. The difference is that the interleaving is done by symbols instead of single bits, reusing the existing interleaving tables.

### 4.10c.6		Mapping on a burst

As an O-FACCH is transmitted on symbols which are stolen in a burst from the traffic channel, the even numbered symbols in the first four bursts and the odd numbered symbols in the last four bursts are stolen.

The mapping is given by the rule:

E(B,j) = I(B,j) and E(B,59+j) = I(B,57+j) for j=0,1,…,56

and

E(B,57) = HL(B) and E(B,58) = HU(B).

To indicate the stealing to the receiving device the symbols HL(B) and HU(B) have to be set according to the following rule:

HU(B) = {1,1,1} for the first four bursts (even numbered symbols are stolen)

HL(B) = {1,1,1} for the last four burts (odd numbered symbols are stolen).

As a consequence, one speech frame of an O-TCH/F is stolen by an O-FACCH/F.

## 4.11	Slow associated control channel with embedded enhanced power control (SACCH/TP)

### 4.11.1	Block constitution

The message delivered to the encoder has a fixed size of 184 information bits {d(0),d(1),...,d(183)}. It is delivered on a burst mode.

### 4.11.2	Block code

a)	Parity bits:

Eighteen parity bits p(0),p(1),...,p(17) are defined in such a way that in GF(2) the binary polynomial:

d(0)D201 +...+ d(183)D18 + p(0)D17+...+ p(17), when divided by:

D18 + D17 + D14 + D13 + D11 + D10 + D8 + D7 + D6 + D3 + D2 + 1, yields a remainder equal to:

D17 + D16 + D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail bits

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 208 bits.

u(k) = d(k)		for k= 0,1,...,183

u(k) = p(k-184)	for k = 184,185,...,201

u(k) = 0			for k = 202,203,204,205,206,207 (tail bits)

### 4.11.3	Convolutional encoder

This block of 208 bits is encoded with the ½ rate convolutional code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7= 1 + D + D2 + D3 + D6

This results in a block of 416 coded bits: {c'(0),c'(1),...,c'(415)} defined by:

c'(2k)		= u(k) + u(k-2) + u(k-3) +  u(k-5) + u(k-6)

c'(2k+1)		= u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)		for k = 0,1,...,207 ; u(k) = 0 for k < 0

### 4.11.4	Dummy bits insertion

Forty dummy bits are first inserted to the coded bits according to the following rule:

c(k)		= c'(k)			for k = 0,1,2

c(k)		= c'(k-1)		for k = 4,...,31

c(k)		= c'(k-2)		for k = 33,...,39

c(k)		= c'(k-3)		for k = 41,...,45

c(k)		= c'(k-5)		for k = 48,...,67

c(k)		= c'(k-6)		for k = 69,...,88

c(k)		= c'(k-7)		for k = 90,...,95

c(k)		= c'(k-9)		for k = 98,...,102

c(k)		= c'(k-10)		for k = 104,...,123

c(k)		= c'(k-12)		for k = 126,...,131

c(k)		= c'(k-13)		for k = 133,...,145

c(k)		= c'(k-14)		for k = 147,...,152

c(k)		= c'(k-16)		for k = 155,...,180

c(k)		= c'(k-18)		for k = 183,...,188

c(k)		= c'(k-19)		for k = 190,...,202

c(k)		= c'(k-20)		for k = 204,...,209

c(k)		= c'(k-22)		for k = 212,...,231

c(k)		= c'(k-23)		for k = 233,...,237

c(k)		= c'(k-25)		for k = 240,...,245

c(k)		= c'(k-26)		for k = 247,...,266

c(k)		= c'(k-27)		for k = 268,...,287

c(k)		= c'(k-29)		for k = 290,...,294

c(k)		= c'(k-30)		for k = 296,...,302

c(k)		= c'(k-31)		for k = 304,...,331

c(k)		= c'(k-32)		for k = 333,...,344

c(k)		= c'(k-34)		for k = 347,...,387

c(k)		= c'(k-36)		for k = 390,...,401

c(k)		= c'(k-38)		for k = 404,...,444

c(k)		= c'(k-40)		for k = 447,...,455

c(k)	= 0		for k = 3, 32, 40, 46 , 47, 68, 89, 96, 97, 103, 124, 125, 132, 146, 153, 154, 181, 182, 189, 203, 210, 211, 232, 238, 239, 246, 267, 288, 289, 295, 303, 332, 345, 346, 388, 389, 402, 403, 445, 446

### 4.11.5	Interleaving

The interleaving is done as specified for the SACCH in subclause 4.1.4.

### 4.11.6	Mapping on a Burst

The mapping is given by the rule:

e(B,j)		= i(B,j)	and	e(B,59+j) = i(B,57+j)	for j = 0,1,...,56

NOTE:	The bits e(B,57) and e(B,58) on burst number B do not need to be set as they are used by the EPCCH (see subclause 4.12).

## 4.12	Enhanced power control channel (EPCCH)

### 4.12.1	Block code

The EPCCH message delivered to the encoder on every 120ms, and has a fixed size of 3 information bits {pm(0), pm(1), pm(2)}. The contents of the bits are defined in 3GPP TS 45.008 for both uplink and downlink.

The EPCCH information bits {pm(n,0),pm(n,1),pm(n,2)} are coded into 12 bits
pb(B,k), k = 0,1,...11 according to the following table (identical to the one used for USF in section 5.1.4.2):

| pm(n,0),pm(n,1),pm(n,2) | pb(B,0),..., pb(B,11) |
| --- | --- |
| 000 | 000 000 000 000 |
| 001 | 000 011 011 101 |
| 010 | 001 101 110 110 |
| 011 | 001 110 101 011 |
| 100 | 110 100 001 011 |
| 101 | 110 111 010 110 |
| 110 | 111 001 111 101 |
| 111 | 111 010 100 000 |

### 4.12.2	Mapping on a Burst

The EPCCH message is mapped on the SACCH/TP burst.

The mapping is given by the rule:

e(B,j)		= pb(B,k)		for respectively j = 44, 47, 50, 53, 55, 57, 58, 60, 62, 65, 68, 71, and k= 0,1, ..., 11

# 5	Packet Switched Channels

## 5.1	Packet data traffic channel (PDTCH)

Thirteen coding schemes are specified for the packet data traffic channels. For the coding schemes CS-2 to CS-4 and MCS-1 to MCS-4, the first three bits (USF-bits) of the data block are encoded such that the first twelve coded bits are representing the same bit pattern, irrespective of the coding scheme, depending only on the USF-bits. For these coding schemes, the USF-bits can therefore always be decoded from these twelve bits in the same way. It should be noted that the USF precoding is done in the uplink direction for coding schemes CS-2 – CS-4, despite the fact that uplink RLC data block structure (3GPP TS 44.060) does not define USF-field.

For the nine coding schemes MCS-1 to MCS-9, the block structure differs between uplink and downlink since header sizes before coding are not the same.

In BTTI configuration, the RLC/MAC layer delivers to the encoder one data block every 20 ms. In RTTI configuration, the RLC/MAC layer delivers to the encoder one data block every 10 ms or, if BTTI USF mode is used (see 3GPP TS 45.002), the RLC/MAC layer in the downlink may deliver to the encoder two data blocks every 20 ms.

In the downlink direction, if BTTI USF mode is used (see 3GPP TS 45.002), one value of the USF per PDCH is delivered to the encoder every 20 ms; if RTTI USF mode is used (see 3GPP TS 45.002), one value of the USF per corresponding downlink PDCH-pair is delivered to the encoder every 10 ms.

If BTTI USF mode is used when sending downlink data blocks in RTTI configuration, then the USF need not be delivered to the encoder as the first three bits of a data block. In this case, the first three bits of a data block are set to an unspecified value (see 3GPP TS 44.060).

NOTE:	How the USFs are delivered to the encoder in this case is implementation dependent.

If BTTI USF mode is used when sending downlink data blocks in RTTI configuration, then both data blocks sent in a 20 ms block period shall be encoded using coding schemes with the same modulation, unless

-	Reduced Latency is supported by the MS to which the USF is intended or

-	EGPRS2 is supported by the MS to which the USF is intended or

-	The USF to be transmitted is set to an unused value (see 3GPP TS 45.002).

If an MS, that neither supports Reduced Latency nor EGPRS2, receives in a 20ms block period data blocks sent using different modulations, the MS shall ignore the USF.

If BTTI USF mode is used when sending downlink data blocks in RTTI configuration and different modulations are used in the two data blocks sent in a 20 ms block period, the USF will be sent with mixed modulation. In this case, the coding of the USF bits sent in the first 10 ms block period is according to the MCS used in that block period, while the coding of the USF bits sent in the second 10 ms block period is according to the MCS used in that block period. The network shall use only MCSs supported by the MS to which the USF is intended.

### 5.1.1	Packet data block type 1 (CS-1)

The coding scheme used for packet data block type 1 is the same as for SACCH as specified in section 4.1.

The flags hl(B) and hu(B) set to "1" identify the coding scheme CS-1.

In RTTI configuration with RTTI USF mode, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

### 5.1.2	Packet data block type 2 (CS-2)

#### 5.1.2.1	Block constitution

The message delivered to the encoder has a fixed size of 271 information bits {d(0),d(1),...,d(270)}. It is delivered on a burst mode.

#### 5.1.2.2	Block code

a)	USF precoding:

The first three bits d(0),d(1),d(2) are precoded into six bits u’(0),u’(1),...,u’(5) according to the following table:

| d(0),d(1),d(2) | u’(0),u’(1),...,u’(5) |
| --- | --- |
| 000 | 000 000 |
| 001 | 001 011 |
| 010 | 010 110 |
| 011 | 011 101 |
| 100 | 100 101 |
| 101 | 101 110 |
| 110 | 110 011 |
| 111 | 111 000 |

b)	Parity bits:

Sixteen parity bits p(0),p(1),...,p(15) are defined in such a way that in GF(2) the binary polynomial:

d(0)D286 +...+ d(270)D16 + p(0)D15 +...+ p(15), when divided by:

D16 + D12 + D5 + 1, yields a remainder equal to:

D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D+1.

c)	Tail bits:

Four tail bits equal to 0 are added to the information and parity bits, the result being a block of 294 bits  {u(0),u(1),...,u(293)}:

u(k)	= u’(k)			for k = 0,1,...,5

u(k)	= d(k-3)		for k = 6,7,...,273

u(k)	= p(k-274)	for k = 274,275,...,289

u(k)	= 0				for k = 290,291,292,293 (tail bits)

#### 5.1.2.3	Convolutional encoder

This block of 294 bits {u(0),u(1),...,u(293)} is encoded with the ½ rate convolutional code (identical to the one used for TCH/FS) defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

This results in a block of 588 coded bits: {C(0),C(1),...,C(587)} defined by:

C(2k)     = u(k) + u(k-3) + u(k-4)

C(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,293 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits:

{C(3+4j) for j = 3,4,...,146 except for j = 9,21,33,45,57,69,81,93,105,117,129,141} are not transmitted

The result is a block of 456 coded bits, {c(0),c(1),...,c(455)}.

#### 5.1.2.4	Interleaving

The interleaving is done as specified for SACCH in section 4.1.4.

#### 5.1.2.5	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,59+j) = i(B,57+j)			for j = 0,1,...,56

and

e(B+m,57) = q(2m)  and  e(B+m,58) = q(2m+1)	for m = 0,1,2,3

where

q(0),q(1),...,q(7) = 1,1,0,0,1,0,0,0 identifies the coding scheme CS-2.

### 5.1.3	Packet data block type 3 (CS-3)

#### 5.1.3.1	Block constitution

The messages delivered to the encoder has a fixed size of 315 information bits {d(0),d(1),...,d(314)}. It is delivered on a burst mode.

#### 5.1.3.2	Block code

a)	USF precoding:

The first three bits d(0),d(1),d(2) are precoded into six bits u’(0),u’(1),...,u’(5) as specified for CS-2 in section 5.1.2.2.a).

b)	Parity bits:

Sixteen parity bits p(0),p(1),...,p(15) are defined in such a way that in GF(2) the binary polynomial:

d(0)D330 +...+ d(314)D16 + p(0)D15 +...+ p(15), when divided by:

D16 + D12 + D5 + 1, yields a remainder equal to:

D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D+1.

c)	Tail bits:

Four tail bits equal to 0 are added to the information and parity bits, the result being a block of 338 bits  {u(0),u(1),...,u(337)}:

u(k)	= u’(k)			for k = 0,1,...,5

u(k)	= d(k-3)		for k = 6,7,...,317

u(k)	= p(k-318)	for k = 318,319,...,333

u(k)	= 0				for k = 334,335,336,337 (tail bits)

#### 5.1.3.3	Convolutional encoder

This block of 338 bits {u(0),u(1),...,u(337)} is encoded with the ½ rate convolutional code (identical to the one used for TCH/FS) defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

This results in a block of 676 coded bits: {C(0),C(1),...,C(675)} defined by:

C(2k)     = u(k) + u(k-3) + u(k-4)

C(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)	for k = 0,1,...,337 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits:

{C(3+6j) and C(5+6j) for j = 2,3,...,111} are not transmitted

The result is a block of 456 coded bits, {c(0),c(1),...,c(455)}.

#### 5.1.3.4	Interleaving

The interleaving is done as specified for SACCH in subclause 4.1.4.

#### 5.1.3.5	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,59+j) = i(B,57+j)			for j = 0,1,...,56

and

e(B+m,57) = q(2m)  and  e(B+m,58) = q(2m+1)	for m = 0,1,2,3

where

q(0),q(1),...,q(7) = 0,0,1,0,0,0,0,1 identifies the coding scheme CS-3.

### 5.1.4	Packet data block type 4 (CS-4)

#### 5.1.4.1	Block constitution

The message delivered to the encoder has a fixed size of 431 information bits {d(0),d(1),...,d(430)}. It is delivered on a burst mode.

#### 5.1.4.2	Block code

a)	USF precoding:

The first three bits d(0),d(1),d(2) are block coded into twelve bits u’(0),u’(1),...,u’(11) according to the following table:

| d(0),d(1),d(2) | u’(0),u’(1),...,u’(11) |
| --- | --- |
| 000 | 000 000 000 000 |
| 001 | 000 011 011 101 |
| 010 | 001 101 110 110 |
| 011 | 001 110 101 011 |
| 100 | 110 100 001 011 |
| 101 | 110 111 010 110 |
| 110 | 111 001 111 101 |
| 111 | 111 010 100 000 |

b)	Parity bits:

Sixteen parity bits p(0),p(1),...,p(15) are defined in such a way that in GF(2) the binary polynomial:

d(0)D446 +...+ d(430)D16 + p(0)D15 +...+ p(15), when divided by:

D16 + D12 + D5 + 1, yields a remainder equal to:

D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D+1.

The result is a block of 456 coded bits, {c(0),c(1),...,c(455)}:

c(k)	= u’(k)			for k = 0,1,...,11

c(k)	= d(k-9)		for k = 12,13,...,439

c(k)	= p(k-440)	for k = 440,441,...,455

#### 5.1.4.3	Convolutional encoder

No convolutional coding is done.

#### 5.1.4.4	Interleaving

The interleaving is done as specified for SACCH in section 4.1.4.

#### 5.1.4.5	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,59+j) = i(B,57+j)			for j = 0,1,...,56

and

e(B+m,57) = q(2m)  and  e(B+m,58) = q(2m+1)	for m = 0,1,2,3

where

q(0),q(1),...,q(7) = 0,0,0,1,0,1,1,0 identifies the coding scheme CS-4.

### 5.1.4a	Packet data block type 5a (MCS-0)

#### 5.1.4a.1	Downlink (MCS-0 DL)

##### 5.1.4a.1.1	Block constitution

The message delivered to the encoder has a fixed size of 207 information bits {d(0),d(1),...,d(206)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 210 information bits {d(0),d(1),...,d(209)}, if an eTFI field is included (see 3GPP TS 44.060).

##### 5.1.4a.1.2	USF precoding

Twelve bits u’(0),u’(1),...,u’(11) are generated as described for MCS-1 DL in subclause 5.1.5.1.2.2.

##### 5.1.4a.1.3	Data coding

a)	Parity bits:

Eighteen data parity bits p(0),p(1),...,p(17) are defined in such a way that in GF(2) the binary polynomial:

d(31)D193 +...+ d(206)D18 + p(0)D17 +...+ p(17), when divided by:

D18 + D17 + D14 + D13 + D11 + D10 + D8 + D7 + D6 + D3 + D2 + 1, yields a remainder equal to:

D17 + D16 + D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

In case an eTFI field (see 3GPP TS 44.060) is included the three bits {d(207),d(208),d(209} are added bit-wise modulo 2 to the last three parity bits {p(15), p(16),p(17)}. This results in eighteen modified parity bits {pt(0),…,pt(17)} defined as:

pt(k) = p(k)					for k=0,…,14

pt(k) = d(k+192) + p(k)		for k=15,…,17

b)	Tail bits:

Six tail bits equal to 0 are added to the information bits, the result being a block of 182 bits 
{u(0),u(1),...,u(181)}:

u(k)	= d(k+31)		for k = 0,1,...,175

u(k)	= 0				for k = 176,177,…,181 (tail bits)

c)	Convolutional encoder

This block of 182 bits {u(0),u(1),...,u(181)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 546 coded bits: {C(0),C(1),...,C(545)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,181; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits are not transmitted:

{C(2+3j) for j = 0,1,...,181} are not transmitted except {C(k) for k = 35, 104, 173, 242, 308, 377, 446, 515} which are transmitted

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

##### 5.1.4a.1.4	Header coding

The header bits {d(3),d(4),...,d(30)} shall be coded as for Packet data block type 5 (MCS-1) in subclause 5.1.5.1.3.

When applying the coding in subclause 5.1.5.1.3, the operations specific to eTFI inclusion therein shall be ignored.

Before coding {d(8)..d(23)} is replaced by {p(0),..,p(15) and {d(29),d(30)} is replaced by {p(16),p(17)}, where {p (0),..,p(17)} is defined in 5.1.4a.1.3.

In case an eTFI field is included p(0),…,p(17) is replaced by pt(0),…pt(17) defined in 5.1.4a.1.3.

The result is a block of 68 coded bits, {hc(0),hc(1),...,hc(67)}.

##### 5.1.4a.1.5	Interleaving

The interleaving is done as specified for MCS-1 DL in subclause 5.1.5.1.5.

##### 5.1.4a.1.6	Mapping on a burst

The mapping is done as specified for MCS-1 DL in subclause 5.1.5.1.6.2.

### 5.1.5	Packet data block type 5 (MCS-1)

#### 5.1.5.1	Downlink (MCS-1 DL)

##### 5.1.5.1.1	Block constitution

The message delivered to the encoder has a fixed size of 209 information bits {d(0),d(1),...,d(208)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 212 information bits {d(0),d(1),...,d(211)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 234 information bits {d(0),d(1),...,d(233)}, if a PAN field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 237 information bits {d(0),d(1),...,d(236)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.5.1.2	USF precoding

###### 5.1.5.1.2.1	BTTI configuration

The first three bits d(0),d(1),d(2) are block coded into twelve bits u’(0),u’(1),...,u’(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2.

###### 5.1.5.1.2.2	RTTI configuration

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the first three bits d(0),d(1),d(2) are block coded into twelve bits u’(0),u’(1),...,u’(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into twelve bits uL(0),uL(1),...,uL(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into twelve bits uH(0),uH(1),...,uH(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then d(0),d(1),d(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then

u’(i) = uL(i)		for i = 0, 4, 8;

u’(i) = uH(i-1)		for i = 1, 5, 9;

u’(i) = uL(i-1)		for i = 2, 6, 10;

u’(i) = uH(i-2)		for i = 3, 7, 11.

If the data block is sent in the second 10ms of a 20ms block period, then

u’(i) = uL(i+2)	for i = 0, 4, 8;

u’(i) = uH(i+1)	for i = 1, 5, 9;

u’(i) = uL(i+1)	for i = 2, 6, 10;

u’(i) = uH(i)		for i = 3, 7, 11.

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20 ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

##### 5.1.5.1.3	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(3)D35 +...+ d(30)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

In case an eTFI field (see 3GPP TS 44.060) is included and a PAN field is not the three bits {d(209),d(210),d(211)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. This results in the eight modified header parity bits {pt(0),…,pt(7)} defined as:

pt(k) = p(k)					for k=0,…,4

pt(k) = d(k+204) + p(k)		for k=5,…,7

In case an eTFI field (see 3GPP TS 44.060) and a PAN field are included the three bits {d(234),d(235),d(236)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. This results in the eight modified header parity bits {pt(0),…,pt(7)} defined as:

pt(k) = p(k)					for k=0,…,4

pt(k) = d(k+229) + p(k)		for k=5,…,7

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 42 bits {u"(-6),…,u"(0),u"(1),...,u"(35)} with six negative indexes. In case an eTFI field is not included this operation is defined as:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k+3)		for k = 0,1,...,27

u"(k)		= p(k-28)		for k = 28,29,...,35

In case an eTFI field is included p(k) is replaced by pt(k) in the above operation.

c)	Convolutional encoder

This block of 42 bits {u"(-6),…,u"(0),u"(1),...,u"(35)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 108 coded bits: {C(0),C(1),...,C(107)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,35

The code is punctured in such a way that the following coded bits:

{C(2+3j) for j = 0,1,...,35} as well as {C(k) for k = 34,58,82,106} are not transmitted

The result is a block of 68 coded bits, {hc(0),hc(1),...,hc(67)}.

##### 5.1.5.1.4	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(31)D189 +...+ d(208)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 196 bits 
{u(0),u(1),...,u(195)}:

u(k)	= d(k+31)		for k = 0,1,...,177

u(k)	= p(k-178)	for k = 178,179,...,189

u(k)	= 0				for k = 190,191,…,195 (tail bits)

c)	Convolutional encoder

This block of 196 bits {u(0),u(1),...,u(195)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 588 coded bits: {C(0),C(1),...,C(587)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,195; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits:

| P1 | {C(2+21j), C(5+21j), C(8+21j), C(10+21j), C(11+21j), C(14+21j), C(17+21j), C(20+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 73,136,199,262,325,388,451,514} which are transmitted |
| --- | --- |
| P2 | {C(1+21j), C(4+21j), C(7+21j), C(9+21j), C(13+21j), C(15+21j), C(16+21j), C(19+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 78,141,204,267,330,393,456,519} which are transmitted |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(3+21j), C(12+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 33,96,159,222,369,432,495,558} which are transmitted |
| --- | --- |
| P2 | {C(6+21j), C(18+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 39,102,165,228,375,438,501,564} which are transmitted |

The result is a block of 324 coded bits {pc(0),pc(1),...,pc(323)}.

##### 5.1.5.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

a)	Parity bits

Ten PAN parity bits p(0),p(1),...,p(9) are defined in such a way that in GF(2) the binary polynomial:

d(209)D29 +...+ d(228)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D9 + D5 + D4 + D + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D1 + 1.

The five bits {d(229),...,d(233)} (TFI value or 00000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the 5 last parity bits {p(5),…,p(9)}. This results in the ten modified PAN parity bits {pt(0),…,pt(9)} defined as:

pt(k) = p(k)					for k=0,…,4

pt(k) = d(k+224) + p(k)		for k=5,…,9

b)	Tail biting:

The six last modified PAN parity bits are added before information and modified PAN parity bits, the result being a block of 36 {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} bits with six negative indexes:

u’’(k-6)	= pt(k+4)		for k = 0,1,...,5

u’’(k)		= d(k+209)	for k = 0,1,...,19

u’’(k)		= pt(k-20)		for k = 20,21,...,29

c)	Convolutional encoder

The block of 36 bits {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 90 coded bits {C(0),C(1),...,C(89)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,29

The block of 90 coded bits is punctured in such way that the following coded bits:

{ C(15k), C(2+15k), C(4+15k), C(6+15k), C(7+15k), C(10+15k), C(13+15k) for k = 0,1,...,5} are not transmitted.

The result is a block of 48 coded bits {ac(0),ac(1),...,ac(47)}.

The data coded bits {pc(0),pc(1),...,pc(323)} are appended to the PAN coded bits by the following rule:

dc(k) = ac(k)			for k = 0,1,...,47

dc(k) = pc(k-48)		for k = 48,49,...,371

The result is a block of 372 coded bits {dc(0),dc(1),...,dc(371)}.

##### 5.1.5.1.5	Interleaving

The USF, header and data are put together as one entity as described by the following rule:

c(k) = u’(k)			for k = 0,1,...,11

c(k) = hc(k-12)		for k = 12,13,...,79

c(k) = dc(k-80)		for k = 80,81,...,451

c’(n,k) = c(n,k)		for k = 0,1,...,24

c’(n,k) = c(n,k-1)		for k = 26,27,...,81

c’(n,k) = c(n,k-2)		for k = 83,84,...,138

c’(n,k) = c(n,k-3)		for k = 140,141,...,423

c’(n,k) = c(n,k-4)		for k = 425,426,...,455

c’(n,25) = q(8)		c’(n,82) = q(9)		c’(n,139) = q(10)		c’(n,424) = q(11)

c(n,k) are the coded bits and q(8),q(9),…,q(11) = 0,0,0,0 are four extra stealing flags

The resulting block is interleaved according to the following rule:

i(B,j) = c’(n,k)	for	k = 0,1,...,455

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 4)

j = 2((49k) mod 57) + ((k mod 8) div 4)

##### 5.1.5.1.6	Mapping on a burst

###### 5.1.5.1.6.1	BTTI configuration

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,59+j) = i(B,57+j)			for j = 0,1,...,56

and

e(B+m,57) = q(2m)  and  e(B+m,58) = q(2m+1)	for m = 0,1,2,3

where

q(0),q(1),...,q(7) = 0,0,0,1,0,1,1,0.

Note:	For a standard GPRS MS, bits q(0),...,q(7) indicates that the USF is coded as for CS-4.

###### 5.1.5.1.6.2	RTTI configuration

a)	Bit swapping

After the interleaving the following bits are swapped:

If the RTTI radio block is sent in the first 10ms of a 20ms block period:

Swap i(B+1,98) with i(B+1,0)

Swap i(B+1,35) with i(B+1,51)

Swap i(B+1,84) with i(B+1,100)

Swap i(B+2,98) with i(B+2,82)

Swap i(B+2,35) with i(B+2,19)

Swap i(B+2,84) with i(B+2,68)

Swap i(B+3,35) with i(B+3,3)

Swap i(B+3,84) with i(B+3,52)

Swap i(B+3,98) with i(B+3,66)

If the RTTI radio block is sent in the second 10ms of a 20ms block period:

Swap i(B,19) with i(B,51)

Swap i(B,68) with i(B,100)

Swap i(B,82) with i(B,0)

Swap i(B+1,19) with i(B+1,35)

Swap i(B+1,68) with i(B+1,84)

Swap i(B+1,82) with i(B+1,98)

Swap i(B+2,19) with i(B+2,3)

Swap i(B+2,68) with i(B+2,52)

Swap i(B+2,82) with i(B+2,66)

b)	Mapping on bursts

The mapping is given by the rule:

e(B,j) = i(B,j)	and	e(B,59+j) = i(B,57+j)	for j = 0,1,...,56

and

e(B+m,57) = q(2m)  and  e(B+m,58) = q(2m+1)	for m = 0,1,2,3

where q(0),q(1),...,q(7) are set according to the following table, depending on the USF mode (see 3GPP TS 45.002):

|  | in the first 10ms of a 20ms block period | in the second 10ms of a 20ms block period |
| --- | --- | --- |
| USF sent in BTTI USF mode | 0,0,0,0,0,1,0,1 | 0,1,0,1,1,0,1,0 |
| USF sent in RTTI USF mode | 0,0,0,1,0,1,1,0 |  |

c)	Mapping on PDCHs

The bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

#### 5.1.5.2	Uplink (MCS-1 UL)

##### 5.1.5.2.1	Block constitution

The block constitution is the same as for MCS-1 DL specified in subclause 5.1.5.1.1, with the exception that an eTFI field can only be included in combination with a PAN field, see sub-clause 5.1.5.2.3a.

##### 5.1.5.2.2	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(0)D38 +...+ d(30)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 45 bits {u"(-6),…,u"(0),u"(1),...,u"(38)} with six negative indexes:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k)		for k = 0,1,...,30

u"(k)		= p(k-31)	for k = 31,32,...,38

c)	Convolutional encoder

This block of 45 bits {u"(-6),…,u"(0),u"(1),...,u"(38)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 117 coded bits: {C(0),C(1),...,C(116)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,38

The code is punctured in such a way that the following coded bits:

{C(5+12j), C(8+12j), C(11+12j), for j = 0,1,...,8} as well as {C(k) for k = 26,38,50,62,74,86,98,110,113,116} are not transmitted

The result is a block of 80 coded bits, {hc(0),hc(1),...,hc(79)}.

##### 5.1.5.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.5.1.4.

##### 5.1.5.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, the PAN coding is the same as for the downlink as specified in subclause 5.1.5.1.4a.

If a PAN field and an eTFI field are included, the PAN coding is the same as for the downlink as specified in subclause 5.1.5.1.4a, with the addition that the five bits {d(229),d(230),d(231),d(232),d(233)} (TFI value or 00000, see 3GPP TS 44.060) and the three bits {d(234),d(235),d(236)} (eTFI value or 000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the PAN parity bits {p(2),p(3),p(4),p(5),p(6),p(7),p(8),p(9)} resulting in the ten modified PAN parity bits defined as:

pt(k) = p(k)					for k=0,…,1

pt(k) = d(k+232) + p(k)		for k=2,…,4

pt(k) = d(k+224) + p(k)		for k=5,…,9

##### 5.1.5.2.4	Interleaving

The header and data are put together as one entity as described by the following rule:

c(k)	= hc(k)		for k = 0,1,...,79

c(k)	= dc(k-80)	)		for k = 80,81,...,451

c’(n,k) = c(n,k)		for k = 0,1,...,24

c’(n,k) = c(n,k-1)		for k = 26,27,...,81

c’(n,k) = c(n,k-2)		for k = 83,84,...,138

c’(n,k) = c(n,k-3)		for k = 140,141,...,423

c’(n,k) = c(n,k-4)		for k = 425,426,...,455

c’(n,25) = q(8)		c’(n,82) = q(9)		c’(n,139) = q(10)	c’(n,424) = q(11)

c(n,k) are the coded bits and q(8),q(9),…,q(11) = 0,0,0,0 are four extra stealing flags

The resulting block is interleaved according to the following rule:

i(B,j) = c’(n,k)	for	k = 0,1,...,455

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 4)

j = 2((49k) mod 57) + ((k mod 8) div 4)

##### 5.1.5.2.5	Mapping on a burst

In BTTI configuration, the mapping is the same as for MCS-1 DL as specified in subclause 5.1.5.1.6.1.

NOTE:	This mapping is also applied in RTTI configuration.

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

### 5.1.6	Packet data block type 6 (MCS-2)

#### 5.1.6.1	Downlink (MCS-2 DL)

##### 5.1.6.1.1	Block constitution

The message delivered to the encoder has a fixed size of 257 information bits {d(0),d(1),...,d(256)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 260 information bits {d(0),d(1),...,d(259)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 282 information bits {d(0),d(1),...,d(281)} if a PAN field is included.

The message delivered to the encoder will have a fixed size of 285 information bits {d(0),d(1),...,d(284)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.6.1.2	USF precoding

###### 5.1.6.1.2.1	BTTI configuration

The first three bits d(0),d(1),d(2) are block coded into twelve bits u’(0),u’(1),...,u’(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2.

###### 5.1.6.1.2.2	RTTI configuration

Twelve bits u’(0),u’(1),...,u’(11) are generated as described for MCS-1 DL in subclause 5.1.5.1.2.2.

##### 5.1.6.1.3	Header coding

A block of 68 coded bits {hc(0),hc(1),...,hc(67)} is derived from {d(3),d(4),…,d(30)} as described for MCS-1 DL in subclause 5.1.5.1.3, with the exception that eTFI field bits are now located in {d(257),d(258),d(259)} if included when PAN is not included, and located in {d(282),d(283),d(284)} if included in combination with a PAN field.

##### 5.1.6.1.4	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(31)D237 +...+ d(256)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 244 bits {u(0),u(1),...,u(243)}:

u(k)	= d(k+31)		for k = 0,1,...,225

u(k)	= p(k-226)	for k = 226,227,...,237

u(k)	= 0				for k = 238,239,…,243 (tail bits)

c)	Convolutional encoder

This block of 244 bits {u(0),u(1),...,u(243)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 732 coded bits: {C(0),C(1),...,C(731)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,243; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits:

| P1 | {C(6j), C(1+6j), C(5+6j) for j = 0,1,...,121} and {C(k) for k = 57,171,285,399,513,627} are transmitted |
| --- | --- |
| P2 | {C(2+6j), C(3+6j), C(4+6j) for j = 0,1,...,121} and {C(k) for k = 108,222,336,450,564,678} are transmitted |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(18+30j), C(30+30j) for j = 0,1,...,23} are not transmitted |
| --- | --- |
| P2 | {C(9+30j), C(27+30j) for j = 0,1,...,23} are not transmitted |

The result is a block of 324 coded bits {pc(0),pc(1),...,pc(323)}.

##### 5.1.6.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

A block of 48 coded bits {ac(0),ac(1),...,ac(47)} is derived from {d(257),d(258),...,d(281)} as described for MCS-1 DL in subclause 5.1.5.1.4a, with bits {d(209),d(210),...,d(233)} replaced by bits {d(257),d(258),...,d(281)}.

The data coded bits {pc(0),pc(1),...,pc(323)} are appended to the PAN coded bits as described for MCS-1 DL in subclause 5.1.5.1.4a. The result is a block of 372 coded bits {dc(0),dc(1),...,dc(371)}.

##### 5.1.6.1.5	Interleaving

The interleaving is done as specified for MCS-1 DL in subclause 5.1.5.1.5.

##### 5.1.6.1.6	Mapping on a burst

The mapping is done as specified for MCS-1 DL in subclause 5.1.5.1.6.

#### 5.1.6.2	Uplink (MCS-2 UL)

##### 5.1.6.2.1	Block constitution

The block constitution is the same as for MCS-2 DL specified in subclause 5.1.6.1.1, with the exception that an eTFI field can only be included in combination with a PAN field, see sub-clause 5.1.6.2.3a.

##### 5.1.6.2.2	Header coding

A block of 80 coded bits {hc(0),hc(1),...,hc(79)} is derived from {d(0),d(1),…,d(30)} as described for MCS-1 UL in subclause 5.1.5.2.2.

##### 5.1.6.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.6.1.4.

##### 5.1.6.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, its coding is the same as for downlink as specified in subclause 5.1.6.1.4a.

If a PAN field and an eTFI field are included, the coding of these fields are the same as for the downlink as specified in subclause 5.1.6.1.4a, with the addition that the TFI and eTFI values being added bit-wise modulo 2 to the last eight PAN parity bits as specified in subclause 5.1.5.2.3a.

##### 5.1.6.2.4	Interleaving

The interleaving is the same as for MCS-1 UL as specified in subclause 5.1.5.2.4.

##### 5.1.6.2.5	Mapping on a burst

The mapping is the same as for MCS-1 UL as specified in subclause 5.1.5. 2.5.

### 5.1.7	Packet data block type 7 (MCS-3)

#### 5.1.7.1	Downlink (MCS-3 DL)

##### 5.1.7.1.1	Block constitution

The message delivered to the encoder has a fixed size of 329 information bits {d(0),d(1),...,d(328)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 332 information bits {d(0),d(1),...,d(331)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 354 information bits {d(0),d(1),...,d(353)} if a PAN field is included.

The message delivered to the encoder will have a fixed size of 357 information bits {d(0),d(1),...,d(356)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.7.1.2	USF precoding

###### 5.1.7.1.2.1	BTTI configuration

The first three bits d(0),d(1),d(2) are block coded into twelve bits u’(0),u’(1),...,u’(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2.

###### 5.1.7.1.2.2	RTTI configuration

Twelve bits u’(0),u’(1),...,u’(11) are generated as described for MCS-1 DL in subclause 5.1.5.1.2.2.

##### 5.1.7.1.3	Header coding

A block of 68 coded bits {hc(0),hc(1),...,hc(67)} is derived from {d(3),d(4),…,d(30)} as described for MCS-1 DL in subclause 5.1.5.1.3, with the exception that eTFI field bits are now located in {d(329),d(330),d(331)} if included when PAN is not included, and located in {d(354),d(355),d(356)} if included in combination with a PAN field.

##### 5.1.7.1.4	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(31)D309 +...+ d(328)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 316 bits 
{u(0),u(1),...,u(315)}:

u(k)	= d(k+31)		for k = 0,1,...,297

u(k)	= p(k-298)	for k = 298,299,...,309

u(k)	= 0				for k = 310,311,…,315 (tail bits)

c)	Convolutional encoder

This block of 316 bits {u(0),u(1),...,u(315)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 948 coded bits: {C(0),C(1),...,C(947)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,315; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits:

| P1 | {C(18j), C(1+18j), C(3+18j), C(6+18j), C(10+18j), C(14+18j), C(17+18j) for j = 0,1,...,51} and {C(k) for k = 241,475,709, 936,937,939,942,946 } are transmitted |
| --- | --- |
| P2 | {C(2+18j), C(5+18j), C(6+18j), C(7+18j), C(9+18j), C(12+18j), C(16+18j) for j = 0,1,...,51} and {C(k) for k = 121,355,589, 938,941,942,943,945 } are transmitted |
| P3 | {C(18j), C(4+18j), C(8+18j), C(11+18j), C(12+18j), C(13+18j), C(15+18j) for j = 0,1,...,51} and {C(k) for k = 181,289,523,811, 936,940,944,947 } are transmitted |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(18j) for j = 0,1,...,51} are not transmitted except {C(k) for k = 108,342,576,810} which are transmitted |
| --- | --- |
| P2 | {C(6+18j) for j = 0,1,...,51} are not transmitted except {C(k) for k = 186,294,528,762} which are transmitted |
| P3 | {C(12+18j) for j = 0,1,...,51} are not transmitted except {C(k) for k = 66,390,642,876} which are transmitted |

The result is a block of 324 coded bits {pc(0),pc(1),...,pc(323)}.

##### 5.1.7.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

A block of 48 coded bits {ac(0),ac(1),...,ac(47)} is derived from {d(329),d(330),...,d(353)} as described for MCS-1 DL in subclause 5.1.5.1.4a, with bits {d(209),d(210),...,d(233)} replaced by bits {d(329),d(330),...,d(353)}.

The data coded bits {pc(0),pc(1),...,pc(323)} are appended to the PAN coded bits as described for MCS-1 DL in subclause 5.1.5.1.4a. The result is a block of 372 coded bits {dc(0),dc(1),...,dc(371)}.

##### 5.1.7.1.5	Interleaving

The interleaving is done as specified for MCS-1 DL in subclause 5.1.5.1.5.

##### 5.1.7.1.6	Mapping on a burst

The mapping is done as specified for MCS-1 DL in subclause 5.1.5.1.6.

#### 5.1.7.2	Uplink (MCS-3 UL)

##### 5.1.7.2.1	Block constitution

The block constitution is the same as for MCS-3 DL specified in subclause 5.1.7.1.1, with the exception that an eTFI field can only be included in combination with a PAN field, see sub-clause 5.1.7.2.3a.

##### 5.1.7.2.2	Header coding

A block of 80 coded bits {hc(0),hc(1),...,hc(79)} is derived from {d(0),d(1),…,d(30)} as described for MCS-1 UL in subclause 5.1.5.2.2.

##### 5.1.7.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.7.1.4.

##### 5.1.7.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, its coding is the same as for downlink as specified in subclause 5.1.7.1.4a.

If a PAN field and an eTFI field are included, the coding of theses fields are the same as for the downlink as specified in subclause 5.1.7.1.4a, with the addition that the TFI and eTFI values being added bit-wise modulo 2 to the last eight PAN parity bits as specified in subclause 5.1.5.2.3a.

##### 5.1.7.2.4	Interleaving

The interleaving is the same as for MCS-1 UL as specified in subclause 5.1.5.2.4.

##### 5.1.7.2.5	Mapping on a burst

The mapping is the same as for MCS-1 UL as specified in subclause 5.1.5. 2.5.

### 5.1.8	Packet data block type 8 (MCS-4)

#### 5.1.8.1	Downlink (MCS-4 DL)

##### 5.1.8.1.1	Block constitution

The message delivered to the encoder has a fixed size of 385 information bits {d(0),d(1),...,d(384)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 388 information bits {d(0),d(1),...,d(387)}, if an eTFI field is included (see 3GPP TS 44.060).

##### 5.1.8.1.2	USF precoding

###### 5.1.8.1.2.1	BTTI configuration

The first three bits d(0),d(1),d(2) are block coded into twelve bits u’(0),u’(1),...,u’(11) as for Packet data block type 4 (CS-4) in subclause 5.1.4.2.

###### 5.1.8.1.2.2	RTTI configuration

Twelve bits u’(0),u’(1),...,u’(11) are generated as described for MCS-1 DL in subclause 5.1.5.1.2.2.

##### 5.1.8.1.3	Header coding

A block of 68 coded bits {hc(0),hc(1),...,hc(67)} is derived from {d(3),d(4),…,d(30)} as described for MCS-1 DL in subclause 5.1.5.1.3, with the exception that eTFI field bits are now located in {d(385),d(386),d(387)} if included.

##### 5.1.8.1.4	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(31)D365 +...+ d(384)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 372 bits {u(0),u(1),...,u(371)}:

u(k)	= d(k+31)		for k = 0,1,...,353

u(k)	= p(k-354)	for k = 354,355,...,365

u(k)	= 0				for k = 366,367,…,371 (tail bits)

c)	Convolutional encoder

This block of 372 bits {u(0),u(1),...,u(371)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 1116 coded bits: {C(0),C(1),...,C(1115)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,..., 371; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits:

| P1 | {C(3j) for j = 0,1,...,371} are transmitted |
| --- | --- |
| P2 | {C(1+3j) for j = 0,1,...,371} are transmitted |
| P3 | {C(2+3j) for j = 0,1,...,371} are transmitted |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

##### 5.1.8.1.5	Interleaving

The interleaving is done as specified for MCS-1 DL in subclause 5.1.5.1.5.

##### 5.1.8.1.6	Mapping on a burst

The mapping is done as specified for MCS-1 DL in subclause 5.1.5.1.6.

#### 5.1.8.2	Uplink (MCS-4 UL)

##### 5.1.8.2.1	Block constitution

The message delivered to the encoder has a fixed size of 385 information bits {d(0),d(1),...,d(384)}. It is delivered on a burst mode.

##### 5.1.8.2.2	Header coding

A block of 80 coded bits {hc(0),hc(1),...,hc(79)} is derived from {d(0),d(1),…,d(30)} as described for MCS-1 UL in subclause 5.1.5.2.2.

##### 5.1.8.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.8.1.4.

##### 5.1.8.2.4	Interleaving

The interleaving is the same as for MCS-1 UL as specified in subclause 5.1.5.2.4.

##### 5.1.8.2.5	Mapping on a burst

The mapping is the same as for MCS-1 UL as specified in subclause 5.1.5. 2.5.

### 5.1.9	Packet data block type 9 (MCS-5)

#### 5.1.9.1	Downlink (MCS-5 DL)

##### 5.1.9.1.1	Block constitution

The message delivered to the encoder has a fixed size of 478 information bits {d(0),d(1),...,d(477)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 481 information bits {d(0),d(1),...,d(480)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 503 information bits {d(0),d(1),...,d(502)} if a PAN field is included.

The message delivered to the encoder will have a fixed size of 506 information bits {d(0),d(1),...,d(505)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.9.1.2	USF precoding

###### 5.1.9.1.2.1	BTTI configuration

The first three bits d(0),d(1),d(2) are block coded into 36 bits u’(0),u’(1),...,u’(35) according to the following table:

| d(0),d(1),d(2) | u’(0),u’(1),...,u’(35) |  |  |  |
| --- | --- | --- | --- | --- |
|  | burst 0 | burst 1 | burst 2 | burst 3 |
| 000 | 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 |
| 001 | 1 1 1 1 1 0 0 0 0 | 1 1 1 1 0 0 0 0 0 | 1 1 1 1 1 1 0 0 0 | 1 1 1 1 1 0 0 0 1 |
| 010 | 1 1 1 0 0 1 1 1 0 | 1 1 1 0 1 1 1 0 0 | 1 1 0 0 0 0 1 1 0 | 1 1 0 0 0 1 1 0 0 |
| 011 | 1 0 0 1 1 1 1 0 0 | 1 1 0 0 0 0 0 1 1 | 1 0 1 1 1 0 1 1 1 | 0 0 1 0 0 1 1 1 1 |
| 100 | 0 0 0 1 1 0 0 1 1 | 0 0 1 0 1 1 0 1 0 | 1 0 0 0 0 1 1 0 1 | 1 1 1 1 1 1 1 1 0 |
| 101 | 1 1 0 1 0 1 0 1 1 | 0 0 0 1 1 0 1 0 1 | 0 1 1 1 0 1 0 1 1 | 1 0 0 1 0 1 0 1 1 |
| 110 | 0 0 1 0 0 1 1 0 1 | 1 0 1 1 1 1 1 1 1 | 0 1 1 0 1 0 0 0 1 | 0 0 1 1 1 0 1 0 0 |
| 111 | 0 1 1 0 1 0 1 1 1 | 0 1 0 1 0 1 1 1 1 | 0 0 0 1 1 1 1 1 0 | 0 1 0 0 1 0 0 1 1 |

###### 5.1.9.1.2.2	RTTI configurations

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the first three bits d(0),d(1),d(2) are block coded into 36 bits u’(0),u’(1),...,u’(35) as described in subclause 5.1.9.1.2.1.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into 36 bits uL(0),uL(1),...,uL(35) as described in subclause 5.1.9.1.2.1; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into 36 bits uH(0),uH(1),...,uH(35) as described in subclause 5.1.9.1.2.1.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then d(0),d(1),d(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then:

u’(j)=uL(j),		j=0…8

u’(j)=uH(j-9),		j=9…17

u’(j)=uL(j-9),		j=18…26

u’(j)=uH(j-18)		j=27…35

If the data block is sent in the second 10ms of a 20ms block period, then:

u’(j)=uL(j+18),	j=0…8

u’(j)=uH(j+9),		j=9…17

u’(j)=uL(j+9),		j=18…26

u’(j)=uH(j)			j=27…35

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20 ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

##### 5.1.9.1.3	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(3)D32 +...+ d(27)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

In case an eTFI field (see 3GPP TS 44.060) is included and a PAN field is not included the three bits {d(478),d(479),d(480)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. In case an eTFI field and a PAN field are included the three bits {d(503),d(504),d(505)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. This result in eight modified header parity bits {pt(0),…,pt(7)} as specified in subclause 5.1.5.1.3.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 39 bits {u"(-6),…,u"(0),u"(1),...,u"(32)} with six negative indexes. In case an eTFI field is not included this operation is defined as:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k+3)		for k = 0,1,...,24

u"(k)		= p(k-25)	for k = 25,26,...,32

In case an eTFI field is included p(k) is replaced by pt(k) in the above operation.

c)	Convolutional encoder

This block of 39 bits {u"(-6),…,u"(0),u"(1),...,u"(32)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 99 coded bits: {C(0),C(1),...,C(98)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,32

A spare bit is added at the end of this block:

hc(k) = C(k) for k = 0,1,…,98

hc(99) = C(98)

The result is a block of 100 coded bits, {hc(0),hc(1),...,hc(99)}.

##### 5.1.9.1.4	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(28)D461 +...+ d(477)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 468 bits {u(0),u(1),...,u(467)}:

u(k)	= d(k+28)		for k = 0,1,...,449

u(k)	= p(k-450)	for k = 450,451,...,461

u(k)	= 0				for k = 462,463,…,467 (tail bits)

c)	Convolutional encoder

This block of 468 bits {u(0),u(1),...,u(467)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 1404 coded bits: {C(0),C(1),...,C(1403)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,467; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits:

| P1 | {C(2+9j) for j = 0,1,...,153} as well as {C(1388+3j) for j = 0,1,...,5}are not transmitted except {C(k) for k = 47,371,695,1019} which are transmitted |
| --- | --- |
| P2 | {C(1+9j) for j = 0,1,...,153} as well as {C(1387+3j) for j = 0,1,...,5}are not transmitted except {C(k) for k = 136,460,784,1108} which are transmitted |

The result is a block of 1248 coded bits, {dc(0),dc(1),...,dc(1247)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(4+18j) for j = 0,1,...,76} are not transmitted except {C(k) for k = 526} which is transmitted |
| --- | --- |
| P2 | {C(14+18j) for j = 0,1,...,76} are not transmitted except {C(k) for k = 626} which is transmitted |

The result is a block of 1172 coded bits {pc(0),pc(1),...,pc(1171)}.

##### 5.1.9.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

a)	Parity bits

Ten PAN parity bits p(0), p(1),...,p(9) are defined in such a way that in GF(2) the binary polynomial:

d(478)D29 +...+ d(497)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D9 + D5 + D4 + D + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D1 + 1.

The five bits {d(498),...,d(502)} (TFI value or 00000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the 5 last parity bits {p(5),…,p(9)}. This results in the ten modified PAN parity bits {pt(0),…,pt(9)} defined as:

pt(k) = p(k)					for k=0,…,4

pt(k) = d(k+493) + p(k)		for k=5,…,9

b)	Tail biting:

The six last modified PAN parity bits are added before information and modified PAN parity bits, the result being a block of 36 {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} bits with six negative indexes:

u’’(k-6)	= pt(k+4)			for k = 0,1,...,5

u’’(k)		= d(k+478)		for k = 0,1,...,19

u’’(k)		= pt(k-20)			for k = 20,21,...,29

c)	Convolutional encoder

The block of 36 bits {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 90 coded bits {C(0),C(1),...,C(89)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,29

The block of 90 coded bits is punctured in such way that the following coded bits:

{ C(5+6k), C(50+6k) for k = 0,1,...,6} are not transmitted.

The result is a block of 76 coded bits {ac(0),ac(1),...,ac(75)}.

The data coded bits {pc(0),pc(1),...,pc(1171)} are appended to the PAN coded bits by the following rule:

dc(k) = ac(k)			for k = 0,1,...,75

dc(k) = pc(k-76)		for k = 76,49,...,1247

The result is a block of 1248 coded bits {dc(0),dc(1),...,dc(1247)}.

##### 5.1.9.1.5	Interleaving

a)	Header

The 100 coded bits of the header, {hc(0),hc(1),...,hc(99)}, are interleaved according to the following rule:

hi(j) = hc(k)	for	k = 0,1,...,99

j = 25(k mod 4) + ((17k) mod 25)

b)	Data

There is no closed expression describing the interleaver, but it has been derived taking the following approach:

1.	A block interleaver with a 1392 bit block size is defined:

The kth input data bit is mapped to the jth bit of the Bth burst, where

k = 0,…,1391

B = mod(k,4)

d = mod(k,464)

j = 3*(2mod(25d,58) + div(mod(d,8),4) + 2(-1)Bdiv(d,232)) + mod(k,3)

2.	The data bit positions being mapped onto header positions in the interleaved block are removed (the header positions are j = 156,157,…,191 when the header is placed next to the training sequence. This leaves 1248 bits in the mapping.

3.	The bits are renumbered to fill out the gaps both in j and k, without changing the relative order

The resulting interleaver transform the block of 1248 coded bits, {dc(0),dc(1),...,dc(1247)} into a block of 1248 interleaved bits, {di(0),di(1),...,di(1247)}.

di(j’) = dc(k’)	for	k’ = 0,1,...,1247

(An explicit relation between j’ and k’ is given in table 15)

##### 5.1.9.1.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(312B+j)		for j = 0,1,...,155

e(B,j) = hi(25B+j-156)	for j = 156,157,...,167

e(B,j) = u’(9B+j-168)	for j = 168,169,...,173

e(B,j) = q(2B+j-174)		for j = 174,175

e(B,j) = u’(9B+j-170)	for j = 176,177,178

e(B,j) = hi(25B+j-167)	for j = 179,180,...,191

e(B,j) = di(312B+j-36)	for j = 192,193,...,347

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme MCS-5 or MCS-6.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,142) with e(B,155)

Swap e(B,144) with e(B,158)

Swap e(B,145) with e(B,161)

Swap e(B,147) with e(B,164)

Swap e(B,148) with e(B,167)

Swap e(B,150) with e(B,170)

Swap e(B,151) with e(B,173)

Swap e(B,176) with e(B,195)

Swap e(B,179) with e(B,196)

Swap e(B,182) with e(B,198)

Swap e(B,185) with e(B,199)

Swap e(B,188) with e(B,201)

Swap e(B,191) with e(B,202)

Swap e(B,194) with e(B,204).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c) PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,50) with e(B,49)

Swap e(B,113) with e(B,112)

Swap e(B,167) with e(B,154)

Swap e(B,221) with e(B,220)

Swap e(B,278) with e(B,277)

Swap e(B,341) with e(B,340)

For B = 1

Swap e(B,8) with e(B,7)

Swap e(B,59) with e(B,58)

Swap e(B,71) with e(B,70)

Swap e(B,116) with e(B,115)

Swap e(B,173) with e(B,154)

Swap e(B,182) with e(B,193)

Swap e(B,236) with e(B,235)

Swap e(B,299) with e(B,298)

For B = 2

Swap e(B,17) with e(B,16)

Swap e(B,74) with e(B,73)

Swap e(B,137) with e(B,136)

Swap e(B,257) with e(B,256)

Swap e(B,302) with e(B,301)

Swap e(B,314) with e(B,313)

For B = 3

Swap e(B,32) with e(B,31)

Swap e(B,95) with e(B,94)

Swap e(B,152) with e(B,154)

Swap e(B,215) with e(B,214)

Swap e(B,260) with e(B,259)

Swap e(B,272) with e(B,271)

Swap e(B,323) with e(B,322)

#### 5.1.9.2	Uplink (MCS-5 UL)

##### 5.1.9.2.1	Block constitution

The message delivered to the encoder has a fixed size of 487 information bits {d(0),d(1),...,d(486)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 512 information bits {d(0),d(1),...,d(511)} if a PAN field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 515 information bits {d(0),d(1),...,d(514)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.9.2.2	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(0)D44 +...+ d(36)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 51 bits {u"(-6),…,u"(0),u"(1),...,u"(44)} with six negative indexes:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k)		for k = 0,1,...,36

u"(k)		= p(k-37)	for k = 37,38,...,44

c)	Convolutional encoder

This block of 51 bits {u"(-6),…,u"(0),u"(1),...,u"(44)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 135 coded bits: {C(0),C(1),...,C(134)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,44

The code is punctured in such a way that the following coded bits:

hc(k) = C(k) for k = 0,1,…,134

hc(135) = C(134)

The result is a block of 136 coded bits, {hc(0),hc(1),...,hc(135)}.

##### 5.1.9.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.9.1.4 where bits {d(28),d(29),…,d(477)} are replaced by bits {d(37),d(38),…,d(486)}.

##### 5.1.9.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, the coding of the PAN field is the same as for the downlink as specified in subclause 5.1.9.1.4a where bits {d(478), d(479),...,d(502)} are replaced by bits {d(487), d(488),...,d(511)}.

If a PAN field and an eTFI field are included, the PAN coding is the same as for the downlink as specified in sub-clause 5.1.9.1.4a, with the exception that the five bits {d(507),d(508),d(509),d(510),d(511)} (TFI value or 00000, see 3GPP TS 44.060) and the three bits {d(512),d(513),d(514)} (eTFI value or 000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the PAN parity bits {p(2),p(3),p(4),p(5),p(6),p(7),p(8),p(9)} resulting in the ten modified PAN parity bits defined as:

pt(k) = p(k)					for k=0,…,1

pt(k) = d(k+510) + p(k)		for k=2,…,4

pt(k) = d(k+502) + p(k)		for k=5,…,9

The data coded bits {pc(0),pc(1),...,pc(1171)} are appended to the PAN coded bits as described for the downlink in subclause 5.1.9.1.4a. The result is a block of 1248 coded bits {dc(0),dc(1),...,dc(1247)}.

##### 5.1.9.2.4	Interleaving

a)	Header

The 136 coded bits of the header, {hc(0),hc(1),...,hc(135)}, are interleaved according to the following rule:

hi(j) = hc(k)	for	k = 0,1,...,135

j = 34(k mod 4) + 2((11k) mod 17) + [(k mod 8)/4]

b)	Data

The data interleaving is the same as for MCS-5 DL as specified in subclause 5.1.9.1.5.

##### 5.1.9.2.5	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(312B+j)		for j = 0,1,...,155

e(B,j) = hi(34B+j-156)	for j = 156,157,...,173

e(B,j) = q(2B+j-174)		for j = 174,175

e(B,j) = hi(34B+j-158)	for j = 176,177,...,191

e(B,j) = di(312B+j-36)	for j = 192,193,...,347

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme MCS-5 or MCS-6.

b)	Bit swapping

The bit swapping is the same as for MCS-5 DL as specified in subclause 5.1.9.1.6 b).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c) PAN bit swapping

In case a PAN is included in the radio block, additional bits are swapped as specified in subclause 5.1.9.1.6 c).

### 5.1.10	Packet data block type 10 (MCS-6)

#### 5.1.10.1	Downlink (MCS-6 DL)

##### 5.1.10.1.1	Block constitution

The message delivered to the encoder has a fixed size of 622 information bits {d(0),d(1),...,d(621)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 625 information bits {d(0),d(1),...,d(624)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 647 information bits {d(0),d(1),...,d(646)} if a PAN field is included.

The message delivered to the encoder will have a fixed size of 650 information bits {d(0),d(1),...,d(649)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.10.1.2	USF precoding

###### 5.1.10.1.2.1	BTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived from {d(0),d(1),d(2)} as described for MCS-5 DL in subclause 5.1.9.1.2. 1.

###### 5.1.10.1.2.2	RTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is generated as described for MCS-5 DL in subclause 5.1.9.1.2.2.

##### 5.1.10.1.3	Header coding

A block of 100 coded bits {hc(0),hc(1),...,hc(99)} is derived from {d(3),d(4),…,d(27)} as described for MCS-5 DL in subclause 5.1.9.1.3, with the exception that eTFI field bits are now located in {d(622),d(623),d(624)} if included when PAN is not included, and located in {d(647),d(648),d(649)} if included in combination with a PAN field.

##### 5.1.10.1.4	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(28)D605 +...+ d(621)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 612 bits {u(0),u(1),...,u(611)}:

u(k)	= d(k+28)		for k = 0,1,...,593

u(k)	= p(k-594)	for k = 594,595,...,605

u(k)	= 0				for k = 606,607,…,611 (tail bits)

c)	Convolutional encoder

This block of 612 bits {u(0),u(1),...,u(611)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 1836 coded bits: {C(0),C(1),...,C(1835)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,611; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits:

| P1 | {C(2+3j) for j = 0,1,...,611} are not transmitted except {C(k) for k = 32,98,164,230,296,428,494,560, 626,692,824,890,956,1022,1088,1220,1286,1352,1418,1484,1616,1682,1748,1814} which are transmitted |
| --- | --- |
| P2 | {C(1+3j) for j = 0,1,...,611} are not transmitted except {C(k) for k = 16,82,148,214,280,412,478,544, 610,676,808,874,940,1006,1072,1204,1270,1336,1402,1468,1600,1666,1732,1798} which are transmitted |

The result is a block of 1248 coded bits, {dc(0),dc(1),...,dc(1247)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(6+24j) for j = 0,1,...,75} are not transmitted |
| --- | --- |
| P2 | {C(18+24j) for j = 0,1,...,75} are not transmitted |

The result is a block of 1172 coded bits {pc(0),pc(1),...,pc(1171)}.

##### 5.1.10.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

A block of 76 coded bits {ac(0),ac(1),...,ac(75)} is derived from {(622),d(623),...,d(646)} as described for MCS-5 DL in subclause 5.1.9.1.4a, with bits {d(478),d(479),...,d(502)} replaced by bits {d(622),d(623),...,d(646)}.

The data coded bits {pc(0),pc(1),...,pc(1171)} are appended to the PAN coded bits as described for MCS-5 DL in subclause 5.1.9.1.4a. The result is a block of 1248 coded bits {dc(0),dc(1),...,dc(1247)}.

##### 5.1.10.1.5	Interleaving

The interleaving is done as specified for MCS-5 DL in subclause 5.1.9.1.5.

##### 5.1.10.1.6	Mapping on a burst

The mapping is done as specified for MCS-5 DL in subclause 5.1.9.1.6.

#### 5.1.10.2	Uplink (MCS-6 UL)

##### 5.1.10.2.1	Block constitution

The message delivered to the encoder has a fixed size of 631 information bits {d(0),d(1),...,d(630)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 656 information bits {d(0),d(1),...,d(655)} if a PAN field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 659 information bits {d(0),d(1),...,d(658)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.10.2.2	Header coding

A block of 136 coded bits {hc(0),hc(1),...,hc(135)} is derived from {d(0),d(1),…,d(36)} as described for MCS-5 UL in subclause 5.1.9.2.2.

##### 5.1.10.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.10.1.4 where bits {d(28),d(29),…,d(621)} are replaced by bits {d(37),d(38),…,d(630)}.

##### 5.1.10.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, the coding of the PAN field is the same as for the MCS-5 DL as specified in subclause 5.1.9.1.4a where bits {d(478), d(479),...,d(502)} are replaced by bits {d(631),d(632),...,d(655)}.

If a PAN field and an eTFI field are included, the PAN coding is the same as for the downlink as specified in sub-clause 5.1.10.1.4a, with the exception that the five bits {d(651),d(652),d(653),d(654),d(655)} (TFI value or 00000, see 3GPP TS 44.060) and the three bits {d(656),d(657),d(658)} (eTFI value or 000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the PAN parity bits {p(2),p(3),p(4),p(5),p(6),p(7),p(8),p(9)} resulting in the ten modified PAN parity bits defined as:

pt(k) = p(k)					for k=0,…,1

pt(k) = d(k+654) + p(k)		for k=2,…,4

pt(k) = d(k+646) + p(k)		for k=5,…,9

The data coded bits {pc(0),pc(1),...,pc(1171)} are appended to the PAN coded bits as described for MCS-5 DL in subclause 5.1.9.1.4a. The result is a block of 1248 coded bits {dc(0),dc(1),...,dc(1247)}.

##### 5.1.10.2.4	Interleaving

The interleaving is the same as for MCS-5 UL as specified in subclause 5.1.9.2.4.

##### 5.1.10.2.5	Mapping on a burst

The mapping is the same as for MCS-5 UL as specified in subclause 5.1.9.2.5.

### 5.1.11	Packet data block type 11 (MCS-7)

#### 5.1.11.1	Downlink (MCS-7 DL)

##### 5.1.11.1.1	Block constitution

The message delivered to the encoder has a fixed size of 940 information bits {d(0),d(1),...,d(939)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 943 information bits {d(0),d(1),...,d(942)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 965 information bits {d(0),d(1),...,d(964)} if a PAN field is included.

The message delivered to the encoder will have a fixed size of 968 information bits {d(0),d(1),...,d(967)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.11.1.2	USF precoding

5.1.11.1.2.1	BTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived from {d(0),d(1),d(2)} as described for MCS-5 DL in subclause 5.1.9.1.2.1.

###### 5.1.11.1.2.2	RTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is generated as described for MCS-5 DL in subclause 5.1.9.1.2.2.

##### 5.1.11.1.3	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(3)D44 +...+ d(39)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

In case an eTFI field (see 3GPP TS 44.060) is included and a PAN field is not included the three bits {d(940),d(941),d(942)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. In case an eTFI field and a PAN field are included the three bits {d(965),d(966),d967)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. This result in eight modified header parity bits {pt(0),…,pt(7)} as specified in subclause 5.1.5.1.3.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 51 bits {u"(-6),…,u"(0),u"(1),...,u"(44)} with six negative indexes. In case an eTFI field is not included this operation is defined as:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k+3)		for k = 0,1,...,36

u"(k)		= p(k-37)	for k = 37,38,...,44

In case an eTFI field is included p(k) is replaced by pt(k) in the above operation.

c)	Convolutional encoder

This block of 51 bits {u"(-6),…,u"(0),u"(1),...,u"(44)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 135 coded bits: {C(0),C(1),...,C(134)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,44

The code is punctured in such a way that the following coded bits:

{C(k) for k = 14,23,33,50,59,69,86,95,105,122,131} are not transmitted

The result is a block of 124 coded bits, {hc(0),hc(1),...,hc(123)}.

##### 5.1.11.1.4	Data coding

I)	First half:

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(40)D461 +...+ d(489)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 468 bits {u(0),u(1),...,u(467)}:

u(k)	= d(k+40)		for k = 0,1,...,449

u(k)	= p(k-450)	for k = 450,451,...,461

u(k)	= 0				for k = 462,463,…,467 (tail bits)

c)	Convolutional encoder

This block of 468 bits {u(0),u(1),...,u(467)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 1404 coded bits: {C(0),C(1),...,C(1403)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,467; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits:

| P1 | {C(18j), C(1+18j), C(4+18j), C(8+18j), C(11+18j), C(12+18j), C(13+18j), C(15+18j) for j = 0,1,...,77} are transmitted except {C(k) for k = 1,19,37,235,415,595,775,955,1135,1351,1369,1387} which are not transmitted |
| --- | --- |
| P2 | {C(2+18j), C(3+18j), C(5+18j), C(6+18j), C(10+18j), C(14+18j), C(16+18j), C(17+18j) for j = 0,1,...,77} are transmitted except {C(k) for k = 16,34,52,196,376,556,736,916,1096,1366,1384,1402} which are not transmitted |
| P3 | {C(2+18j), C(5+18j), C(6+18j), C(7+18j), C(9+18j), C(12+18j), C(13+18j), C(16+18j) for j = 0,1,...,77} are transmitted except {C(k) for k = 13,31,49,301,481,661,841,1021,1201,1363,1381,1399} which are not transmitted |

The result is a block of 612 coded bits, {c1(0),c1(1),...,c1(611)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(13+36j) for j = 0,1,…,38} are not transmitted except {C(k) for k = 13,49,1381} which are transmitted |
| --- | --- |
| P2 | {C(5+36j) for j = 0,1,…,38} are not transmitted except {C(k) for k = 185,545,1085} which are transmitted |
| P3 | {C(6+36j) for j = 0,1,…,38} are not transmitted except {C(k) for k = 294,654,1194} which are transmitted |

The result is a block of 576 coded bits {pc1(0),pc1(1),...,pc1(575)}.

II)	Second half:

The same data coding as for first half is proceeded with bits {d(40),d(41),…,d(489)} replaced by bits {d(490),d(491),…,d(939)}. The result is a block of 612 coded bits, {c2(0),c2(1),...,c2(611)}.

If the PANI field is set to 1, additional bits are punctured as for the first half. The result is a block of 576 coded bits {pc2(0),pc2(1),...,pc2(575)}.

##### 5.1.11.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

a)	Parity bits

Ten PAN parity bits p(0), p(1),...,p(9) are defined in such a way that in GF(2) the binary polynomial:

d(940)D29 +...+ d(959)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D9 + D5 + D4 + D + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D1 + 1.

The five bits {d(960),...,d(964)} (TFI value or 00000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the 5 last parity bits {p(5),…,p(9)}. This results in the ten modified PAN parity bits {pt(0),…,pt(9)} defined as:

pt(k) = p(k)					for k=0,…,4

pt(k) = d(k+955) + p(k)		for k=5,…,9

b)	Tail biting:

The six last modified PAN parity bits are added before information and modified PAN parity bits, the result being a block of 36 {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} bits with six negative indexes:

u’’(k-6)	= pt(k+4)		for k = 0,1,...,5

u’’(k)		= d(k+940)	for k = 0,1,...,19

u’’(k)		= pt(k-20)		for k = 20,21,...,29

c)	Convolutional encoder

The block of 36 bits {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 90 coded bits {C(0),C(1),...,C(89)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,29

The block of 90 coded bits is punctured in such way that the following coded bits:

{ C(2+15k), C(8+15k), C(14+15k) for k = 0,1,...,5} are not transmitted.

The result is a block of 72 coded bits {ac(0),ac(1),...,ac(71)}.

The data coded bits {pc1(0),pc1(1),...,pc1(575)} and {pc2(0),pc2(1),...,pc2(575)} are appended to the PAN coded bits by the following rule:

c1(k) = ac(k)				for k = 0,1,...,71

c1(k) = pc1(k-72)		for k = 72,73,...,611

c2(k) = pc1(k+540)		for k = 0,1,...,35

c2(k) = pc2(k-36)		for k = 36,37,...,611

The result is two blocks of 612 coded bits {c1(0),c1(1),...,c1(611)} and {c2(0),c2(1),...,c2(611)}.

##### 5.1.11.1.5	Interleaving

a)	Header

The 124 coded bits of the header, {hc(0),hc(1),...,hc(123)}, are interleaved according to the following rule:

hi(j) = hc(k)	for	k = 0,1,...,123

j = 31(k mod 4) + ((17k) mod 31)

b)	Data

Data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,1,...,611

dc(k) = c2(k-612)	for k = 612,613,...,1223

The resulting block is interleaved according to the following rule:

di(j) = dc(k)	for	k = 0,1,...,1223

j = 306(k mod 4) + 3((44k) mod 102 + (k div 4) mod 2) + (k + 2 – (k div 408)) mod 3

##### 5.1.11.1.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(306B+j)		for j = 0,1,...,152

e(B,j) = hi(31B+j-153)	for j = 153,154,...,167

e(B,j) = u’(9B+j-168)	for j = 168,169,...,173

e(B,j) = q(2B+j-174)		for j = 174,175

e(B,j) = u’(9B+j-170)	for j = 176,177,178

e(B,j) = hi(31B+j-164)	for j = 179,180,...,194

e(B,j) = di(306B+j-42)	for j = 195,196,...,347

where

q(0),q(1),...,q(7) = 1,1,1,0,0,1,1,1 identifies the coding scheme MCS-7, MCS-8 or MCS-9.

b)	Bit swapping

The bit swapping is the same as for MCS-5 DL as specified in subclause 5.1.9.1.6 b).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c) PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,2) with e(B,1)

Swap e(B,59) with e(B,58)

Swap e(B,110) with e(B,109)

Swap e(B,209) with e(B,208)

Swap e(B,260) with e(B,259)

Swap e(B,317) with e(B,316)

For B = 1

Swap e(B,23) with e(B,22)

Swap e(B,74) with e(B,73)

Swap e(B,131) with e(B,130)

Swap e(B,314) with e(B,313)

Swap e(B,224) with e(B,223)

Swap e(B,281) with e(B,280)

Swap e(B,191) with e(B,205)

For B = 2

Swap e(B,38) with e(B,37)

Swap e(B,95) with e(B,94)

Swap e(B,146) with e(B,141)

Swap e(B,227) with e(B,226)

Swap e(B,278) with e(B,277)

Swap e(B,335) with e(B,334)

Swap e(B,176) with e(B,205)

For B = 3

Swap e(B,2) with e(B,1)

Swap e(B,59) with e(B,58)

Swap e(B,92) with e(B,91)

Swap e(B,149) with e(B,141)

Swap e(B,242) with e(B,241)

Swap e(B,299) with e(B,298)

#### 5.1.11.2	Uplink (MCS-7 UL)

##### 5.1.11.2.1	Block constitution

The message delivered to the encoder has a fixed size of 946 information bits {d(0),d(1),...,d(945)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 971 information bits {d(0),d(1),...,d(970)} if a PAN field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 974 information bits {d(0),d(1),...,d(973)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.11.2.2	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(0)D53 +...+ d(45)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 60 bits {u"(-6),…,u"(0),u"(1),...,u"(53)} with six negative indexes:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k)			for k = 0,1,...,45

u"(k)		= p(k-46)		for k = 46,47,...,53

c)	Convolutional encoder

This block of 60 bits {u"(-6),…,u"(0),u"(1),...,u"(53)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 162 coded bits: {C(0),C(1),...,C(161)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,53

The code is punctured in such a way that the following coded bits:

{C(k) for k = 35,131} are not transmitted

The result is a block of 160 coded bits, {hc(0),hc(1),...,hc(159)}.

##### 5.1.11.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.11.1.4 where bits {d(40),d(41),…,d(939)} are replaced by bits {d(46),d(47),…,d(945)}.

##### 5.1.11.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, the coding of the PAN field is the same as for the downlink as specified in subclause 5.1.11.1.4a where bits {d(940), d(941),...,d(964)} are replaced by bits {d(946),d(947),...,d(970)}.

If a PAN field and an eTFI field are included, the PAN coding is the same as for the downlink as specified in sub-clause 5.1.11.1.4a, with the exception that the five bits {d(966),d(967),d(968),d(969),d(970)} (TFI value or 00000, see 3GPP TS 44.060) and the three bits {d(971),d(972),d(973)} (eTFI value or 000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the PAN parity bits {p(2),p(3),p(4),p(5),p(6),p(7),p(8),p(9)} resulting in the ten modified PAN parity bits defined as:

pt(k) = p(k)					for k=0,…,1

pt(k) = d(k+969) + p(k)		for k=2,…,4

pt(k) = d(k+961) + p(k)		for k=5,…,9

The data coded bits {pc1(0),pc1(1),...,pc1(575)} and {pc2(0),pc2(1),...,pc2(575)} are appended to the PAN coded bits as described for the downlink in subclause 5.1.11.1.4a. The result is two blocks of 612 coded bits {c1(0),c1(1),...,c1(611)} and {c2(0),c2(1),...,c2(611)}.

##### 5.1.11.2.4	Interleaving

a)	Header

The 160 coded bits of the header, {hc(0),hc(1),...,hc(159)}, are interleaved according to the following rule:

hi(j) = hc(k)	for	k = 0,1,...,159

j = 40(k mod 4) + 2((13(k div 8)) mod 20) + ((k mod 8) div 4)

b)	Data

The data interleaving is the same as for MCS-7 DL as specified in subclause 5.1.11.1.5.

##### 5.1.11.2.5	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(306B+j)		for j = 0,1,...,152

e(B,j) = hi(40B+j-153)	for j = 153,154,...,173

e(B,j) = q(2B+j-174)		for j = 174,175

e(B,j) = hi(40B+j-155)	for j = 176,177,...,194

e(B,j) = di(306B+j-42)	for j = 195,196,...,347

where

q(0),q(1),...,q(7) = 1,1,1,0,0,1,1,1 identifies the coding scheme MCS-7, MCS-8 or MCS-9.

b)	Bit swapping

The bit swapping is the same as for MCS-5 DL as specified in subclause 5.1.9.1.6 b).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c) PAN bit swapping

In case a PAN is included in the radio block, additional bits are swapped as specified in subclause 5.1.11.1.6 c).

### 5.1.12	Packet data block type 12 (MCS-8)

#### 5.1.12.1	Downlink (MCS-8 DL)

##### 5.1.12.1.1	Block constitution

The message delivered to the encoder has a fixed size of 1132 information bits {d(0),d(1),...,d(1131)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 1135 information bits {d(0),d(1),...,d(1134)}, if an eTFI field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 1157 information bits {d(0),d(1),...,d(1156)} if a PAN field is included.

The message delivered to the encoder will have a fixed size of 1160 information bits {d(0),d(1),...,d(1159)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.12.1.2	USF precoding

###### 5.1.12.1.2.1	BTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived from {d(0),d(1),d(2)} as described for MCS-5 DL in subclause 5.1.9.1.2.1.

###### 5.1.12.1.2.2	RTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is generated as described for MCS-5 DL in subclause 5.1.9.1.2.2.

##### 5.1.12.1.3	Header coding

A block of 124 coded bits {hc(0),hc(1),...,hc(123)} is derived from {d(3),d(4),…,d(39)} as described for MCS-7 DL in subclause 5.1.11.1.3, with the exception that eTFI field bits are now located in {d(1132),d(1133),d(1134)} if included and PAN is not included, and located in {d(1157),d(1158),d(1159)} if included in combination with a PAN field.

##### 5.1.12.1.4	Data coding

I)	First half:

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(40)D557 +...+ d(585)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 564 bits {u(0),u(1),...,u(563)}:

u(k)	= d(k+40)		for k = 0,1,...,545

u(k)	= p(k-546)	for k = 546,547,...,557

u(k)	= 0				for k = 558,559,…,563 (tail bits)

c)	Convolutional encoder

This block of 564 bits {u(0),u(1),...,u(563)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 1692 coded bits: {C(0),C(1),...,C(1691)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,563; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits:

| P1 | {C(36j), C(2+36j), C(5+36j), C(6+36j), C(10+36j), C(13+36j), C(16+36j), C(20+36j), C(23+36j), C(24+36j), C(27+36j), C(31+36j), C(35+36j), for j = 0,1,...,46} as well as {C(845)} are transmitted |
| --- | --- |
| P2 | {C(1+36j), C(4+36j), C(8+36j), C(11+36j), C(12+36j), C(15+36j), C(17+36j), C(19+36j), C(22+36j), C(25+36j), C(28+36j), C(30+36j), C(33+36j), for j = 0,1,...,46} as well as {C(582)} are transmitted |
| P3 | {C(2+36j), C(3+36j), C(7+36j), C(9+36j), C(14+36j), C(17+36j), C(18+36j), C(21+36j), C(26+36j), C(27+36j), C(29+36j), C(32+36j), C(34+36j), for j = 0,1,...,46} as well as {C(1156)} are transmitted |

The result is a block of 612 coded bits, {c1(0),c1(1),...,c1(611)}.

For the FANR procedure, the code is punctured depending on the CPS field and the PANI field as defined in 3GPP TS 44.060. If the PANI field is set to 0, the puncturing is the same as for EGPRS. If the PANI field is set to 1, the puncturing schemes named P1 or P2 are applied in such a way that, in addition to the bits punctured for EGPRS, the following coded bits:

| P1 | {C(2+36j) for j = 0,1,…,46} are not transmitted except {C(k) for k = 38,182,326,470,614,758,902,1046,1190,1334,1478} which are transmitted |
| --- | --- |
| P2 | {C(17+36j) for j = 0,1,…,46} are not transmitted except {C(k) for k = 89,233,377,521,665,809,953,1097,1241,1385,1529} which are transmitted |
| P3 | {C(27+36j) for j = 0,1,…,46} are not transmitted except {C(k) for k = 135,279,423,567,711,855,999,1143,1287,1431,1575} which are transmitted |

The result is a block of 576 coded bits {pc1(0),pc1(1),...,pc1(575)}.

II)	Second half:

The same data coding as for first half is proceeded with bits {d(40),d(41),…,d(585)} replaced by bits {d(586),d(587),…,d(1131)}. The result is a block of 612 coded bits, {c2(0),c2(1),...,c2(611)}.

If the PANI field is set to 1, additional bits are punctured as for the first half. The result is a block of 576 coded bits {pc2(0),pc2(1),...,pc2(575)}.

##### 5.1.12.1.4a	Piggy-backed Ack/Nack coding

The operations in this subclause shall be carried out only if a PAN field is included.

a)	Parity bits

Ten PAN parity bits p(0), p(1),...,p(9) are defined in such a way that in GF(2) the binary polynomial:

d(1132)D29 +...+ d(1151)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D9 + D5 + D4 + D + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D1 + 1.

The five bits {d(1152),...,d(1156)} (TFI value or 00000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the 5 last parity bits {p(5),…,p(9)}. This results in the ten modified PAN parity bits {pt(0),…,pt(9)} defined as:

pt(k) = p(k)						for k=0,…,4

pt(k) = d(k+1147) + p(k)		for k=5,…,9

b)	Tail biting:

The six last modified PAN parity bits are added before information and modified PAN parity bits, the result being a block of 36 {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} bits with six negative indexes:

u’’(k-6)	= pt(k+4)			for k = 0,1,...,5

u’’(k)		= d(k+1132)	for k = 0,1,...,19

u’’(k)		= pt(k-20)		for k = 20,21,...,29

c)	Convolutional encoder

The block of 36 bits {u’’(-6),...,u’’(0),u’’(1),...,u’’(29)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 90 coded bits {C(0),C(1),...,C(89)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)     for k = 0,1,...,29

The block of 90 coded bits is punctured in such way that the following coded bits:

{ C(2+15k), C(8+15k), C(14+15k) for k = 0,1,...,5} are not transmitted.

The result is a block of 72 coded bits {ac(0),ac(1),...,ac(71)}.

The data coded bits {pc1(0),pc1(1),...,pc1(575)} and {pc2(0),pc2(1),...,pc2(575)} are appended to the PAN coded bits by the following rule:

c1(k) = ac(2k)			for k = 0,1,...,35

c1(k) = pc1(k-36)		for k = 36,37,...,611

c2(k) = ac(2k+1)			for k = 0,1,...,35

c2(k) = pc2(k-36)		for k = 36,37,...,611

The result is two blocks of 612 coded bits {c1(0),c1(1),...,c1(611)} and {c2(0),c2(1),...,c2(611)}.

##### 5.1.12.1.5	Interleaving

a)	Header

The header interleaving is the same as for MCS-7 DL as specified in subclause 5.1.11.1.5.

b)	Data

Data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,1,...,611

dc(k) = c2(k-612)	for k = 612,613,...,1223

The resulting block is interleaved according to the following rule:

di(j) = dc(k)	for	k = 0,1,...,1223

j = 306(2(k div 612) + (k mod 2)) + 3((74k) mod 102 + (k div 2) mod 2) + (k + 2 – (k div 204)) mod 3

##### 5.1.12.1.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is the same as for MCS-7 DL as specified in subclause 5.1.11.1.6 a).

b)	Bit swapping

The bit swapping is the same as for MCS-7 DL as specified in subclause 5.1.11.1.6 b).

c)  PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0,2

Swap e(B,2) with e(B,1)

Swap e(B,23) with e(B,22)

Swap e(B,113) with e(B,112)

Swap e(B,128) with e(B,127)

Swap e(B,155) with e(B,141)

Swap e(B,185) with e(B,205)

Swap e(B,260) with e(B,259)

Swap e(B,281) with e(B,280)

For B = 1,3

Swap e(B,59) with e(B,58)

Swap e(B,74) with e(B,73)

Swap e(B,176) with e(B,207)

Swap e(B,206) with e(B,205)

Swap e(B,227) with e(B,226)

Swap e(B,317) with e(B,316)

Swap e(B,332) with e(B,331)

#### 5.1.12.2	Uplink (MCS-8 UL)

##### 5.1.12.2.1	Block constitution

The message delivered to the encoder has a fixed size of 1138 information bits {d(0),d(1),...,d(1137)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 1163 information bits {d(0),d(1),...,d(1162)} if a PAN field is included (see 3GPP TS 44.060).

The message delivered to the encoder will have a fixed size of 1166 information bits {d(0),d(1),...,d(1165)}, if a PAN field and an eTFI field are included (see 3GPP TS 44.060).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

##### 5.1.12.2.2	Header coding

A block of 160 coded bits {hc(0),hc(1),...,hc(159)} is derived from {d(0),d(1),…,d(45)} as described for MCS-7 UL in subclause 5.1.11.2.2.

##### 5.1.12.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.12.1.4 where bits {d(40),d(41),…,d(1131)} are replaced by bits {d(46),d(47),…,d(1137)}.

##### 5.1.12.2.3a	Piggy-backed Ack/Nack coding

If a PAN field is included and an eTFI field is not included, the coding of the PAN field is the same as for the MCS-8 DL as specified in subclause 5.1.12.1.4a where bits {d(1132), d(1133),...,d(1156)} are replaced by bits {d(1138),d(1139),...,d(1162)}.

If a PAN field and an eTFI field are included, the PAN coding is the same as for the downlink as specified in sub-clause 5.1.12.1.4a, with the exception that the five bits {d(1158),d(1159),d(1160),d(1161),d(1162)} (TFI value or 00000, see 3GPP TS 44.060) and the three bits {d(1163),d(1164),d(1165)} (eTFI value or 000, see 3GPP TS 44.060) are added bit-wise modulo 2 to the PAN parity bits {p(2),p(3),p(4),p(5),p(6),p(7),p(8),p(9)} resulting in the ten modified PAN parity bits defined as:

pt(k) = p(k)					for k=0,…,1

pt(k) = d(k+1161) + p(k)	for k=2,…,4

pt(k) = d(k+1153) + p(k)	for k=5,…,9

The data coded bits {pc1(0),pc1(1),...,pc1(575)} and {pc2(0),pc2(1),...,pc2(575)} are appended to the PAN coded bits as described for MCS-8 DL in subclause 5.1.12.1.4a. The result is two blocks of 612 coded bits {c1(0),c1(1),...,c1(611)} and {c2(0),c2(1),...,c2(611)}.

##### 5.1.12.2.4	Interleaving

a)	Header

The header interleaving is the same as for MCS-7 UL as specified in subclause 5.1.11.2.4.

b)	Data

The data interleaving is the same as for MCS-8 DL as specified in subclause 5.1.12.1.5.

##### 5.1.12.2.5	Mapping on a burst

a)	Straightforward mapping

The mapping is the same as for MCS-7 UL as specified in subclause 5.1.11.2.5 a).

b)	Bit swapping

The bit swapping is the same as for MCS-7 UL as specified in subclause 5.1.11.2.5 b).

c)	PAN bit swapping

In case a PAN is included in the radio block, additional bits are swapped as specified in subclause 5.1.12.1.6 c).

### 5.1.13	Packet data block type 13 (MCS-9)

#### 5.1.13.1	Downlink (MCS-9 DL)

##### 5.1.13.1.1	Block constitution

The message delivered to the encoder has a fixed size of 1228 information bits {d(0),d(1),...,d(1227)}. It is delivered on a burst mode.

The message delivered to the encoder will have a fixed size of 1231 information bits {d(0),d(1),...,d(1230)}, if an eTFI field is included (see 3GPP TS 44.060).

##### 5.1.13.1.2	USF precoding

###### 5.1.13.1.2.1	BTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived from {d(0),d(1),d(2)} as described for MCS-5 DL in subclause 5.1.9.1.2.1.

###### 5.1.13.1.2.2	RTTI configuration

A block of 36 bits {u’(0),u’(1),...,u’(35)} is generated as described for MCS-5 DL in subclause 5.1.9.1.2.2.

##### 5.1.13.1.3	Header coding

A block of 124 coded bits {hc(0),hc(1),...,hc(123)} is derived from {d(3),d(4),…,d(39)} as described for MCS-7 DL in subclause 5.1.11.1.3, with the exception that eTFI field bits are now located in {d(1228),d(1229),d(1230)} if included.

##### 5.1.13.1.4	Data coding

I) First half:

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(40)D605 +...+ d(633)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 612 bits {u(0),u(1),...,u(611)}:

u(k)	= d(k+40)		for k = 0,1,...,593

u(k)	= p(k-594)	for k = 594,595,...,605

u(k)	= 0				for k = 606,607,…,611 (tail bits)

c)	Convolutional encoder

This block of 612 bits {u(0),u(1),...,u(611)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 1836 coded bits: {C(0),C(1),...,C(1835)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,611; u(k) = 0 for k < 0

The code is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits:

| P1 | {C(3j) for j = 0,1,...,611} are transmitted |
| --- | --- |
| P2 | {C(1+3j) for j = 0,1,...,611} are transmitted |
| P3 | {C(2+3j) for j = 0,1,...,611} are transmitted |

The result is a block of 612 coded bits, {c1(0),c1(1),...,c1(611)}.

II)	Second half:

The same data coding as for first half is proceeded with bits {d(40),d(41),…,d(633)} replaced by bits {d(634),d(635),…,d(1227)}. The result is a block of 612 coded bits, {c2(0),c2(1),...,c2(611)}.

##### 5.1.13.1.5	Interleaving

The interleaving is the same as for MCS-8 DL as specified in subclause 5.1.12.1.5.

##### 5.1.13.1.6	Mapping on a burst

The mapping is the same as for MCS-7 DL as specified in subclause 5.1.11.1.6.

#### 5.1.13.2	Uplink (MCS-9 UL)

##### 5.1.13.2.1	Block constitution

The message delivered to the encoder has a fixed size of 1234 information bits {d(0),d(1),...,d(1233)}. It is delivered on a burst mode.

##### 5.1.13.2.2	Header coding

A block of 160 coded bits {hc(0),hc(1),...,hc(159)} is derived from {d(0),d(1),…,d(45)} as described for MCS-7 UL in subclause 5.1.11.2.2.

##### 5.1.13.2.3	Data coding

The data coding is the same as for downlink as specified in subclause 5.1.13.1.4 where bits {d(40),d(41),…,d(1227)} are replaced by bits {d(46),d(47),…,d(1233)}.

##### 5.1.13.2.4	Interleaving

The interleaving is the same as for MCS-8 UL as specified in subclause 5.1.12.2.4.

##### 5.1.13.2.5	Mapping on a burst

The mapping is the same as for MCS-7 UL as specified in subclause 5.1.11.2.5.

## 5.1a	Packet data traffic channels (PDTCH) for EGPRS2

For each of EGPRS2-A downlink, EGPRS2-B downlink and EGPRS2-B uplink, eight additional coding schemes are specified for the packet data traffic channels. For EGPRS2-A uplink, five additional coding schemes are specified for the packet data traffic channels.

### 5.1a.1	General descriptions of channel coding functions

#### 5.1a.1.1	Header

a) Parity bits

Given a block of N bits, {h(0),…,h(N-1)}, eight header parity bits {p(0),p(1),...,p(7)} are defined in such a way that in GF(2) the binary polynomial:

h(0)D8+N-1 +...+ h(N-1)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

In case an eTFI field (see 3GPP TS 44.060) is included the three eTFI bits {et(0),et(1),et(2)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)}. This results in the eight modified header parity bits {pt(0),…,pt(7)} defined as:

pt(k) = p(k)				or k=0,…,4

pt(k) = et(k-5) + p(k)		for k=5,…,7

The parity bits are added after the block of N bits, the result being a block of N+8 bits, {b(0),…,b(N+7)}, in case an eTFI field is not included, defined as:

b(k)		= h(k)			for k = 0,1,...,N-1

b(k)		= p(k-N)		for k = N,...,N+7

In case an eTFI field is included, p(k) is replaced by pt(k) in the above operation.

b) Tail-biting convolutional encoder

Given the block of N+8 bits, {b(0),…,b(N+7)}, the six last bits are added before the block of N+8 bits, the result being a block of N+14 bits {c(-6),…,c(0),c(1),...,c(N+7)} with six negative indexes:

c(k)		= b(N+8+k)	for k = -6,...,-1

c(k)		= b(k)			for k = 0,1,...,N+7

This block of N+14 bits {c(-6),…,c(0),c(1),...,c(N+7)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 3(N+8) coded bits {C(0),...,C(3(N+8)-1)} defined by:

C(3k)     = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

C(3k+1) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

C(3k+2) = c(k) + c(k-1) + c(k-4) + c(k-6)     for k = 0,1,...,N+7

#### 5.1a.1.2	Data encoded with convolutional code

a) Parity bits

Given a block of N bits, {i(0),…,i(N-1)}, twelve data parity bits {p(0),p(1),...,p(11)} are defined in such a way that in GF(2) the binary polynomial:

i(0)D12+N-1 +...+ i(N-1)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

The parity bits are added after the block of N bits, the result being a block of N+12 bits, {b(0),…,b(N+11)}, defined as:

b(k)		= i(k)			for k = 0,1,...,N-1

b(k)		= p(k-N)		for k = N,...,N+11

b) Convolutional encoding

Given the block of N+12 bits, {b(0),…,b(N+11)}, six tail bits equal to 0 are added to the block of N+12 bits, the result being a block of N+18 bits {c(0),...,c(N+17)}:

c(k)		= b(k)			for k = 0,1,...,N+11

c(k)		= 0				for k = N+12,…,N+17 (tail bits)

This block of N+18 bits {c(0),...,c(N+17)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 3(N+18) coded bits, {C(0),...,C(3(N+18)-1)}, defined by:

C(3k)     = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

C(3k+1) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

C(3k+2) = c(k) + c(k-1) + c(k-4) + c(k-6)     for k = 0,1,...,N+17; c(k) = 0 for k < 0

#### 5.1a.1.3	Data encoded with turbo code

Given a block of N bits, {i(0),…,i(N-1)}, the following steps are taken:

##### 5. 1a.1.3.1	Parity bits

Parity bits are added as defined in subclause 5.1a.1.2, the result being a block of N+12 bits, {b(0),…,b(N+11)}.

##### 5.1a.1.3.2	Turbo encoding

The block of K=N+12 bits is encoded with a Turbo code. The input bits to the Turbo coder are defined as:

xi =b(i-1)	for i=1,…,K

The output bits from the Turbo coder are defined as:

C(3i-3)		= xi

C(3i-2)		= zi

C(3i-1)		= z'i     for i=1,…,K

and

C(3K+2i-2)	= xK+i

C(3K+2i-1)	= zK+i

C(3K+2i+4)	= x'K+i

C(3K+2i+5)	= z'K+i    for i=1,2,3

where zi, z'i and x'i are defined below.

The scheme of Turbo coder is a Parallel Concatenated Convolutional Code (PCCC) with two 8-state constituent encoders and one Turbo code internal interleaver.  The coding rate of Turbo coder is 1/3.  The structure of Turbo coder is illustrated in figure 2a.

The transfer function of the 8-state constituent code for PCCC is:

G(D) = 1,G9(D)G8(D),

where

G8(D) = 1 + D2 + D3,

G9(D) = 1 + D + D3.

The initial value of the shift registers of the 8-state constituent encoders shall be all zeros when starting to encode the input bits.

Output from the Turbo coder is

x1, z1, z'1, x2, z2, z'2, …, xK, zK, z'K,

where x1, x2, …, xK are the bits input to the Turbo coder i.e. both first 8-state constituent encoder and Turbo code internal interleaver, and K is the number of bits, and z1, z2, …, zK and z'1, z'2, …, z'K are the bits output from first and second 8-state constituent encoders, respectively.

The bits output from Turbo code internal interleaver are denoted by x'1, x'2, …, x'K, and these bits are to be input to the second 8-state constituent encoder.

Figure 2a: Structure of rate 1/3 Turbo coder (dotted lines apply for trellis termination only)

##### 5.1a.1.3.3	Trellis termination for Turbo coder

Trellis termination is performed by taking the tail bits from the shift register feedback after all information bits are encoded. Tail bits are padded after the encoding of information bits.

The first three tail bits shall be used to terminate the first constituent encoder (upper switch of figure 2a in lower position) while the second constituent encoder is disabled. The last three tail bits shall be used to terminate the second constituent encoder (lower switch of figure 2a in lower position) while the first constituent encoder is disabled.

The transmitted bits for trellis termination shall then be:

xK+1, zK+1, xK+2, zK+2, xK+3, zK+3, x'K+1, z'K+1, x'K+2, z'K+2, x'K+3, z'K+3.

##### 5.1a.1.3.4	Turbo code internal interleaver

The Turbo code internal interleaver consists of bits-input to a rectangular matrix with padding, intra-row and inter-row permutations of the rectangular matrix, and bits-output from the rectangular matrix with pruning. The bits input to the Turbo code internal interleaver are denoted by , where K is the integer number of the bits.

The following subclause specific symbols are used in subclauses 5.1a.1.3.4.1 to 5.1a.1.3.4.3:

K	Number of bits input to Turbo code internal interleaver

R	Number of rows of rectangular matrix

C	Number of columns of rectangular matrix

p	Prime number

v	Primitive root

Base sequence for intra-row permutation

qi	Minimum prime integers

ri	Permuted prime integers

Inter-row permutation pattern

Intra-row permutation pattern of i-th row

i	Index of row number of rectangular matrix

j	Index of column number of rectangularmatrix

k	Index of bit sequence

###### 5.1a.1.3.4.1	Bits-input to rectangular matrix with padding

The bit sequence  input to the Turbo code internal interleaver is written into the rectangular matrix as follows.

(1)	Determine the number of rows of the rectangular matrix R, such that:

.

The rows of rectangular matrix are numbered 0, 1, …, R - 1 from top to bottom.

(2)	Determine the prime number to be used in the intra-permutation, p, and the number of columns of rectangular matrix, C, such that:

if (481  K  530) then

p = 53 and C = p.

else

Find minimum prime number p from table 0 such that

,

and determine C such that

.

end if

The columns of rectangular matrix are numbered 0, 1, …, C - 1 from left to right.

Table 0: List of prime number p and associated primitive root v

| p | v | p | v | p | v | p | v | p | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 3 | 47 | 5 | 101 | 2 | 157 | 5 | 223 | 3 |
| 11 | 2 | 53 | 2 | 103 | 5 | 163 | 2 | 227 | 2 |
| 13 | 2 | 59 | 2 | 107 | 2 | 167 | 5 | 229 | 6 |
| 17 | 3 | 61 | 2 | 109 | 6 | 173 | 2 | 233 | 3 |
| 19 | 2 | 67 | 2 | 113 | 3 | 179 | 2 | 239 | 7 |
| 23 | 5 | 71 | 7 | 127 | 3 | 181 | 2 | 241 | 7 |
| 29 | 2 | 73 | 5 | 131 | 2 | 191 | 19 | 251 | 6 |
| 31 | 3 | 79 | 3 | 137 | 3 | 193 | 5 | 257 | 3 |
| 37 | 2 | 83 | 2 | 139 | 2 | 197 | 2 |  |  |
| 41 | 6 | 89 | 3 | 149 | 2 | 199 | 3 |  |  |
| 43 | 3 | 97 | 5 | 151 | 6 | 211 | 2 |  |  |

(3)	Write the input bit sequence  into the R C rectangular matrix row by row starting with bit y1 in column 0 of row 0:

where yk = xk for k = 1, 2, …, K and if RCK, the dummy bits are padded such that  for k = K + 1, K + 2, …, RC.  These dummy bits are pruned away from the output of the rectangular matrix after intra-row and inter-row permutations.

###### 5.1a.1.3.4.2	Intra-row and inter-row permutations

After the bits-input to the RC rectangular matrix, the intra-row and inter-row permutations for the RC rectangular matrix are performed stepwise by using the following algorithm with steps (1) – (6):

(1)	Select a primitive root v from table 0 in section 5.1a.1.3.4.1, which is indicated on the right side of the prime number p.

(2)	Construct the base sequence  for intra-row permutation as:

,	j = 1, 2,…, (p - 2), and s(0) = 1.

(3)	Assign q0 = 1 to be the first prime integer in the sequence , and determine the prime integer qi in the sequence  to be a least prime integer such that g.c.d(qi, p - 1) = 1, qi > 6, and qi > q(i - 1) for each i = 1, 2, …, R – 1.  Here g.c.d. is greatest common divisor.

(4)	Permute the sequence  to make the sequence  such that

rT(i) = qi,  i = 0, 1, …, R - 1,

where  is the inter-row permutation pattern defined as the one of the four kind of patterns, which are shown in table 0a, depending on the number of input bits K.

Table 0a: Inter-row permutation patterns for Turbo code internal interleaver

| Number of input bits K | Number of rows R | Inter-row permutation patterns <T(0), T(1), …, T(R - 1)> |
| --- | --- | --- |
| (40K159) | 5 | <4, 3, 2, 1, 0> |
| (160K200) or (481K530) | 10 | <9, 8, 7, 6, 5, 4, 3, 2, 1, 0> |
| (2281K2480) or (3161K3210) | 20 | <19, 9, 14, 4, 0, 2, 5, 7, 12, 18, 16, 13, 17, 15, 3, 1, 6, 11, 8, 10> |
| K = any other value | 20 | <19, 9, 14, 4, 0, 2, 5, 7, 12, 18, 10, 8, 13, 17, 3, 1, 16, 6, 15, 11> |

(5)	Perform the i-th (i = 0, 1, …, R - 1) intra-row permutation as:

if (C = p) then

,   j = 0, 1, …, (p - 2), and Ui(p - 1) = 0,

where Ui(j) is the original bit position of j-th permuted bit of i-th row.

end if

if (C = p + 1) then

,   j = 0, 1, …, (p - 2).  Ui(p - 1) = 0, and Ui(p) = p,

where Ui(j) is the original bit position of j-th permuted bit of i-th row, and

if (K = RC) then

Exchange UR-1(p) with UR-1(0).

end if

end if

if (C = p - 1) then

,    j = 0, 1, …, (p - 2),

where Ui(j) is the original bit position of j-th permuted bit of i-th row.

end if

(6) Perform the inter-row permutation for the rectangular matrix based on the pattern ,

where T(i) is the original row position of the i-th permuted row.

###### 5.1a.1.3.4.3	Bits-output from rectangular matrix with pruning

After intra-row and inter-row permutations, the bits of the permuted rectangular matrix are denoted by y'k:

The output of the Turbo code internal interleaver is the bit sequence read out column by column from the intra-row and inter-row permuted R  C rectangular matrix starting with bit y'1 in row 0 of column 0 and ending with bit y'CR in row R - 1 of column C - 1.  The output is pruned by deleting dummy bits that were padded to the input of the rectangular matrix before intra-row and inter row permutations, i.e. bits y'k that corresponds to bits yk with k > K are removed from the output.  The bits output from Turbo code internal interleaver are denoted by x'1, x'2, …, x'K, where x'1 corresponds to the bit y'k with smallest index k after pruning, x'2 to the bit y'k with second smallest index k after pruning, and so on. The number of bits output from Turbo code internal interleaver is K and the total number of pruned bits is:

R  C – K.

##### 5.1a.1.3.5	Turbo code puncturing

This section defines the generation of the puncturing sequences for Turbo coded schemes. The process is defined in 6 parts.

Section 5.1a.1.3.5.1 describes the notation used.

Section 5.1a.1.3.5.2 defines setup of the length parameters based on the properties of each Modulation and Coding scheme.

Section 5.1a.1.3.5.3 defines the modification of the parameters to handle the support of a PAN field.

Section 5.1a.1.3.5.4 defines calculation of the loop parameters explicitly used in the puncturing loop.

Section 5.1a.1.3.5.5 defines the puncturing loop operation.

Section 5.1a.1.3.5.6 gives a usage example for DAS-5.

5.1a.1.3.5.1	Notation

The following notation is used to denote the variables used in sub-sections of Section 5.1a.1.3.5.

swap	fraction of systematic bits not transmitted in P1; defined per DAS/DBS

stream 1	Vector of output bits from Turbo encoder, selected as 1,4,7,…

stream 2	Vector of output bits from Turbo encoder, selected as 2,5,8,…

stream 3	Vector of output bits from Turbo encoder, selected as 3,6,9,…

N			number of bits in stream

Ndata		number of data bits of each BSN transmitted after puncturing, with no PAN field present

Ndata2	number of data bits of each BSN transmitted after puncturing, with PAN field present

flip		logical Boolean to indicate logical XOR with P1 to map previously transmitted bits

Pr		Puncturing sequence version, r is the version number, 1,2, or 3

rmax		Number of puncturing sequences for a BSN, either 2 or 3

Xir,a	Internal variable used for parameter calculationsr is the PS number (1,2,or3)a is type 1,2 or 3

Ntr,a	Internal variable used for parameter calculationsr is the PS number (1,2,or3)a is type 1,2, or 3

5.1a.1.3.5.2	Puncturing Loop parameter setup

This section defines the calculation of the initial parameters for the puncturing sequences. There are 2 types of puncturing for P2; the type to be used is defined per Modulation and Coding scheme in its definition.

5.1a.1.3.5.2.1	P1 – first puncturing version

Set the parameter values as

Xi1,1=N, Xi1,2=N, Xi1,3=N.

Nt1,1=min(1−swap)⋅N,Ndata,

Nt1,2=Ndata−Nt1,12,

Nt1,3=Ndata−Nt1,12

flip=0.

5.1a.1.3.5.2.2	P2 – second puncturing version – Type 1

Set the parameter values as

Xi2,1=Nt1,1, Xi2,2=Nt1,2, Xi2,3=Nt1,3

Nt2,1=minNt1,1,2Ndata−3N,

Nt2,2=2Ndata−3N−Nt2,12,

Nt2,3=2Ndata−3N−Nt2,12

flip=1.

5.1a.1.3.5.2.3	P2 – second puncturing version – Type 2

Set the parameter values as

Xi2,1=N−Nt1,1, Xi2,2=N−Nt1,2, Xi2,3=N−Nt1,3

Nt2,1=N−Nt1,1,

Nt2,2=Ndata−Nt2,12,

Nt2,3=Ndata−Nt2,12

flip=0

5.1a.1.3.5.2.4	P3 – third puncturing version

Set the parameter values as for P1 with a fixed swap = 0.3 as depicted below.

Xi3,1=N, Xi3,2=N, Xi3,3=N.

Nt3,1=min(1−swap)⋅N,Ndata,

Nt3,2=Ndata−Nt3,12,

Nt3,3=Ndata−Nt3,12

flip=0.

##### 5.1a.1.3.5.3	PAN Parameters Handling

This section deals with the treatment of the loop to handle the inclusion of the PAN field.

Set Xir,1=Ntr,1, Xir,2=Ntr,2, Xir,3=Ntr,3

where r is the puncturing sequence number, 1,2 or 3.

If PAN is not included, then set

Ntr,1=Ntr,1,  Ntr,2=Ntr,2,  Ntr,3=Ntr,3

This has the effect of neutralising the part of the loop that punctures out the extra bits to leave space for the PAN field.

If PAN is included, then set

Ntr,1=Ntr,1−Ndata−Ndata2−Ntr,2−Ntr,2−Ntr,3−Ntr,3,

Ntr,2=maxNtr,2−Ndata−Ndata22,0,

Ntr,3=maxNtr,3−Ndata−Ndata22,0

5.1a.1.3.5.4	Puncturing Loop Parameter Calculation

The parameters eplus,  eminus,, e2plus and e2minus are as defined in Table 0a using the parameters calculated in Section 5.1a.1.3.5.3.

Table 0a: Rate Matching Loop Parameters

|  | eplus | eminus | e2plus | e2minus |
| --- | --- | --- | --- | --- |
| Stream 1 | Xir,1 | Xir,1−Ntr,1 | Xir,1 | Xir,1−Ntr,1 |
| Stream 2 | 2⋅Xir,2 | 2⋅Xir,2−Ntr,2 | 2⋅Xir,2 | 2⋅Xir,2−Ntr,2 |
| Stream 3 | Xir,3 | Xir,3−Ntr,3 | Xir,3 | Xir,3−Ntr,3 |

Also eini , e2ini are calculated for the stream 1 bits as

eini=Xir,1−(r−1)⋅eplusrmax−1 modeplus+1

e2ini=Xir,1−(r−1)⋅e2plusrmax−1 mode2plus+1

NOTE:	Given two integer numbers a and b, a mod b is defined as a – floor(a/b)*b if b is inequal to 0, where floor(a/b) rounds a/b to the nearest integer towards minus infinity. By convention, a mod 0 is a.

Similarly, the values of eini, e2ini are calculated for stream 2 and stream 3 bits.

For P3 puncturing scheme, the values of eini and e2ini shall be calculated with 'r' equal to 1 for all the streams.

5.1a.1.3.5.5	Puncturing Loop

This section describes the puncturing loop. The operation of the loop is based on the parameter setup and calculation described in Section 5.1a.1.3.5.3. In order to generate a puncturing sequence Pr , the puncturing  loop is run for each stream 1,2 and 3 using the parameters calculated in 5.1a.1.3.5.4.

Regardless of the presence of the PAN field, the P1 puncturing vector without PAN field is used explicitly in the generation of P2, both Types 1 and 2. For this purpose, the generated puncturing vector of P1 is denoted as variable Tm, m=1,…,Nsys. A logical value of Tm=1 indicates the bit is not punctured, and 0 if punctured. For puncturing versions P1 and P3 (if relevant), Tm=0for all values of m.

e = eini;

e2 = e2ini;

m=1;

while ( m <=  N )

if xor(T(m),  ~flip)

e = e - e_minus;

if (e <= 0)

puncture the bit

e = e + e_plus;

else

e2 = e2 – e2_minus;

if (e2 <= 0)

puncture the bit

e2 = e2 + e2_plus;

endif,

endif,

else

if (~flip)

puncture the bit

endif

endif,

m = m + 1;

end while

5.1a.1.3.5.6	Usage Example, 2 puncturing versions

This section gives a usage example for DAS-5, using the parameters defined for DAS-5 in Section 5.1a.16.4.

The parameter values used for rate matching are swap=0.05, Nsys=466,Ndata=1248 and Ndata2=1172. Using these parameters, we calculate the following parameters using the equations in section 5.1a.1.3.5.2.1:

Xi1,1=Xi1,2=Xi1,3=466,

Nt1,1=min(1−5100)⋅466,1248=443,

Nt1,2=1248−4432=402,

Nt1,3=1248−4432=403,

flip=0.

Assuming PAN field is not included, then

Xir,1=443, Xir,2=402, Xir,3=403,Nt,1=443 Nt,2=402, Nt,3=403.

Placing these values in Table 0a gives values as follows:

[e_ini_s,e_plus_s,e_minus_s] = [466,466,23]

[e_ini_p1,e_plus_p1,e_minus_p1] = [466,932,128]

[e_ini_p2,e_plus_p2,e_minus_p2] = [466,466,63]

[e2_ini_s,e2_plus_s,e2_minus_s] = [443,443,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [402,804,0]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [403,403,0]

For P1, flip=0 and T(m)=0 for every m. So, the first ten puncture pattern bits for each stream are

For stream 1 bits: 1 1 1 1 1 1 1 1 1 1

For stream 2 bits: 1 1 1 0 1 1 1 1 1 1

For stream 3 bits: 1 1 1 1 1 1 1 0 1 1

For the P2, Type 1 in this case as rmax=2, the parameters are calculated in a similar manner using the equation in section 5.1a.1.3.5.2.2:

[e_ini_s,e_plus_s,e_minus_s] = [222,443,0]

[e_ini_p1,e_plus_p1,e_minus_p1] = [804,804,148]

[e_ini_p2,e_plus_p2,e_minus_p2] = [202,403,76]

[e2_ini_s,e2_plus_s,e2_minus_s] = [222,443,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [656,656,0]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [164,327,0]

For P2, flip=1, and T(m) vector is the output from P1. Therefore the first ten puncture pattern bits are

For stream 1 bits: 1 1 1 1 1 1 1 1 1 1

For stream 2 bits: 1 1 1 1 1 1 0 1 1 1

For stream 3 bits: 1 1 0 1 1 1 1 1 0 1

5.1a.1.3.5.7	Usage Example, 3 puncturing versions

This section gives a usage example for DAS-12, using the parameters defined for DAS-12 in Section 5.1a.23.4.

5.1a.1.3.5.7.1		Without PAN

The parameter values used for rate matching are swap=0.0, Nsys=67,Ndata=700 and Ndata2=674. Using these parameters, we calculate the following parameters using the equations in section 5.1a.1.3.5.2.1:

Xi1,1=Xi1,2=Xi1,3=674,

Nt1,1=min(1−0100)⋅674,700=674,

Nt1,2=700−6742=13,

Nt1,3=700−6742=13,

flip=0.

Assuming PAN field is not included, then

Xir,1=674, Xir,2=13, Xir,3=13,Nt,1=674 Nt,2=13, Nt,3=13.

Placing these values in Table 0a gives values as follows:

[e_ini_s,e_plus_s,e_minus_s]        = [674, 674,0]

[e_ini_p1,e_plus_p1,e_minus_p1] = [674,1348,1322]

[e_ini_p2,e_plus_p2,e_minus_p2] = [674, 674,661]

[e2_ini_s,e2_plus_s,e2_minus_s] = [674, 674,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [13,26,0]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [13,13,0]

For P1, flip=0 and T(m)=0 for every m. So, the first ten puncture pattern bits for each stream are

For stream 1 bits: 1 1 1 1 1 1 1 1 1 1

For stream 2 bits: 0 0 0 0 0 0 0 0 0 0

For stream 3 bits: 1 0 0 0 0 0 0 0 0 0

For the P2, Type 2 is used in this case asrmax=3. The parameters are calculated in a similar manner using the equation in section 5.1a.1.3.5.2.3

[e_ini_s,e_plus_s,e_minus_s] = [0,0,0]

[e_ini_p1,e_plus_p1,e_minus_p1] = [221,1322,622]

[e_ini_p2,e_plus_p2,e_minus_p2] = [441,661,311]

[e2_ini_s,e2_plus_s,e2_minus_s] = [0,0,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [117,700,0]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [234,350,0]

For P2, flip=0, and T(m) vector is the output from P1. Therefore the first ten puncture pattern bits are

For stream 1 bits: 0 0 0 0 0 0 0 0 0 0

For stream 2 bits: 0 1 0 1 0 1 0 1 0 1

For stream 3 bits: 0 1 0 1 0 1 0 1 0 1

For the P3, the parameters are calculated in a similar manner using the equation in section 5.1a.1.3.5.2.4 (with a fixed swap = 0.3), where the ‘r’ value in calculating eini and e2ini in subsection 5.1a.1.3.5.4 is equal 1.

[e_ini_s,e_plus_s,e_minus_s] = [674, 674,202]

[e_ini_p1,e_plus_p1,e_minus_p1] = [674,1348,1120]

[e_ini_p2,e_plus_p2,e_minus_p2] = [674, 674,560]

[e2_ini_s,e2_plus_s,e2_minus_s] = [472,472,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [114,228,0]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [114, 114,0]

For P3, flip=0, and T(m)=0 for every m. swap is 30%. Therefore, the first ten puncture pattern bits are

For stream 1 bits: 1 1 1 0 1 1  0 1 1 1

5.1a.1.3.5.7.2	With PAN

In case of PAN inclusion, all the parameters are the same except those that specified below:

[e2_ini_s,e2_plus_s,e2_minus_s] = [674, 674,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [13,26,26]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [13,13,13]

For P1, flip=0 and T(m)=0 for every m. So, the first ten puncture pattern bits for each stream are

For stream 1 bits: 1 1 1 1 1 1 1 1 1 1

For stream 2 bits: 0 0 0 0 0 0 0 0 0 0

For stream 3 bits: 0 0 0 0 0 0 0 0 0 0

For the P2, Type 2 is used in this case asrmax=3. The parameters are calculated in a similar manner using the equation in section 5.1a.1.3.5.2.3

[e2_ini_s,e2_plus_s,e2_minus_s] = [0,0,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [117,700,26]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [234,350,13]

For P2, flip=0, and T(m) vector is the output from P1 without PAN. Therefore the first ten puncture pattern bits are

For stream 1 bits: 0 0 0 0 0 0 0 0 0 0

For stream 2 bits: 0 1 0 1 0 1 0 1 0 0

For stream 3 bits: 0 1 0 1 0 1 0 1 0 1

For the P3, the parameters are calculated in a similar manner using the equation in section 5.1a.1.3.5.2.1, where the ‘r’ value in calculating eini and e2ini in subsection 5.1a.1.3.5.4 is equal to 1.

[e2_ini_s,e2_plus_s,e2_minus_s] = [472,472,0]

[e2_ini_p1,e2_plus_p1,e2_minus_p1] = [114,228,26]

[e2_ini_p2,e2_plus_p2,e2_minus_p2] = [114, 114,13]

For P3, flip=0, and T(m)=0 for every m. swap is 30%. Therefore, the first ten puncture pattern bits are

For stream 1 bits: 1 1 1 0 1 1  0 1 1 1

For stream 2 bits: 0 0 1 0 0 0  0 0 1 0

For stream 3 bits: 1 0 0 0 0 1  0 0 0 0

#### 5.1a.1.4	PAN

a) Parity bits

Given a block of 25 bits, {pn(0),…,pn(24)}, ten PAN parity bits {p(0),p(1),...,p(9)} are defined in such a way that in GF(2) the binary polynomial:

pn(0)D29 +...+ pn(19)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D9 + D5 + D4 + D + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

The five last bits of the PAN, {pn(20),…,pn(24)}, are added bit-wise modulo 2 to the 5 last parity bits {p(5),…,p(9)}. In case an eTFI field is included, the three eTFI bits {et(0),et(l),et(2)} are added bit-wise modulo 2 to the parity bits {p(2),p(3),p(4)}.

The modified parity bits are added after the block of 20 bits, the result being a block of 30 bits, {b(0),…,b(29)}, incase the eTFI field is included, defined as:

b(k)		= pn(k)					for k = 0,...,19

b(k)		= p(k-20)					for k = 20,...,21

b(k)		= p(k-20) + et(k-22)		for k = 22,...,24

b(k)		= p(k-20) + pn(k-5)		for k = 25,…,29

And incase the eTFI field is not included, defined as:

b(k)		= pn(k)					for k = 0,...,19

b(k)		= p(k-20)					for k = 20,...,24b(k)		= p(k-20) + pn(k-5)		for k = 25,…,29

b) Tail-biting convolutional encoder

The six last bits are added before the block of 30 bits, the result being a block of 36 bits {c(-6),…,c(0),c(1),...,c(29)} with six negative indices:

c(k)		= b(30+k)		for k = -6,...,-1

c(k)		= b(k)			for k = 0,1,...,29

This block of 36 bits is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 90 coded bits {C(0),...,C(89)} defined by:

C(3k)     = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

C(3k+1) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

C(3k+2) = c(k) + c(k-1) + c(k-4) + c(k-6)     for k = 0,1,...,29

### 5.1a.2	General descriptions of interleaving functions

##### 5.1a.2.1	Interleaver type 1

Given a block of NC bits, {c(0),...,c(NC-1)} and the parameter a, interleaving is performed according to the following rule:

i(j) = c(k)	for	k = 0,1,..., NC-1

j = NCB/4 + (((k div 4)+(NC div 16)B)a mod NC/4)

B = 2(k mod 2) + (k mod 4) div 2

This results in a block of NC bits {i(0),...,i(NC-1)}.

##### 5.1a.2.2	Interleaver type 2

Given a block of NC bits, {c(0),...,c(NC-1)} and the parameter a, interleaving is performed according to the following rule:

i(j) = c(k)	for	k = 0,1,..., NC-1

j = ka mod NC

This results in a block of NC bits {i(0),...,i(NC-1)}.

### 5.1a.3	Packet data block type 14 (UAS-7)

#### 5.1a.3.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 941 information bits {d(0),d(1),...,d(940)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 966 information bits {d(0),d(1),...,d(965). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 969 information bits {d(0),d(1),...,d(968).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,40

i1(k-41) = d(k)		for k = 41,...,490

i2(k-491) = d(k)		for k = 491,...,940

And if a PAN is included:

pn(k-941) = d(k)		for k = 941,...,965

And if a PAN and an eTFI are included:

et(k-966) = d(k)		for k = 966,...,968

#### 5.1a.3.2	Header coding

The header {h(0),...,h(40)} is coded as defined in subclause 5.1a.1.1, with N=41, resulting in a block of 147 bits, {C(0),...,C(146)}.

The code is punctured in such a way that the following coded bits:

{C(k) for k = 0, 14, 27, 41, 54, 67, 81, 94, 107, 121 and 134} are not transmitted

This results in a block of 136 bits, {hc(0),...,hc(135)}.

#### 5.1a.3.3	Data coding

Each data part, {i1(0),…,i1(449)} and {i2(0),…,i2(449)}, is coded as defined in subclause 5.1a.1.2, with N=450, resulting in two coded blocks of 1404 bits, {C1(0),...,C1(1403)} and {C2(0),...,C2(1403)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(33*k+j) for k=0,…,41, j=2, 5, 8, 10, 11, 13, 17, 19, 23, 25, 29, 31 and 32; and C(33*42+j) for j=2, 5, 8, 10, 11, 13 and 17, except C(33*k+10) for k=4, 13, 22, 31 and 40 that are not punctured. | C(33*k+10) for k=4, 13, 22, 31 and 40; and C(33*k) for k=0, 1, 2, 3, 4, 6, 7, 8, 9, 11, 12, 13, 14, 16, 17, 18, 19, 21, 22, 23, 24, 25, 27, 28, 29, 30, 32, 33, 34, 35, 37, 38, 39 and 40. |
| P2 | C(33*k+j) for k=0,…,41, j=1, 3, 6, 9, 12, 14, 15, 20, 22, 24, 26, 27 and 30; and C(33*42+j) for j=1, 3, 6, 9, 12, 14 and 15, except C(33*k+24) for k=1, 10, 19, 28 and 37 that are not punctured. | C(33*k+24) for k=1, 10, 19, 28 and 37; and C(33*k+4) for k=0, 1, 2, 4, 5, 6, 7, 9, 10, 11, 12, 14, 15, 16, 17, 19, 20, 21, 22, 23, 25, 26, 27, 28, 30, 31, 32, 33, 35, 36, 37, 38, 40 and 41. |

If a PAN is not included, the result is two blocks of 856 bits, {c1(0),...,c1(855)} and {c2(0),...,c2(855)}.

If a PAN is included, the result is two blocks of 817 bits, {c1(0),...,c1(816)} and {c2(0),...,c2(816)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.3.4	PAN coding

The PAN {pn(0),...,pn(24)}, if included, is coded as defined in subclause 5.1a.1.4, resulting in a block of 90 bits, {C(0),...,C(89)}.

The code is punctured in such a way that the following coded bits:

{C(6*k+5) , C(6*k+50) for k = 0,1,…6} are not transmitted; except C(23), C(68) which are transmitted.

This results in a block of 78 bits, {ac(0),...,ac(77)}.

#### 5.1a.3.5	Interleaving

a)	Header

The header, {hc(0),...,hc(135)}, is interleaved as defined in subclause 5.1a.2.1, with NC=136 and a=23, resulting in a block of 136 bits, {hi(0),...,hi(135)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,855

dc(k) = c2(k-856)	for k = 856,...,1711

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,77

dc(k) = c1(k-78)		for k = 78,…,894

dc(k) = c2(k-895)	for k = 895,…,1711

The block {dc(0),...,dc(1711)} is interleaved as defined in subclause 5.1a.2.1, with NC=1712 and a=187, resulting in a block of 1712 bits, {di(0),...,di(1711)}.

#### 5.1a.3.6	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(428B+j)		for j = 0,...,215

e(B,j) = hi(34B+j-216)	for j = 216,...,231

e(B,j) = q(2B+j-232)		for j = 232,233

e(B,j) = hi(34B+j-218)	for j = 234,...,251

e(B,j) = di(428B+j-36)	for j = 252,...,463

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme UAS-7, UAS-8 or UAS-9.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,200+k) with e(B,218+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 34, 35, 38, 39, 42, 43, 46, 47, 50 and 51.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,50) with e(B,40)

Swap e(B,79) with e(B,160)

Swap e(B,158) with e(B,81)

Swap e(B,183) with e(B,21)

Swap e(B,187) with e(B,177)

Swap e(B,230) with e(B,120)

Swap e(B,302) with e(B,405)

Swap e(B,327) with e(B,305)

Swap e(B,406) with e(B,280)

Swap e(B,410) with e(B,444)

Swap e(B,435) with e(B,368)

For B = 1

Swap e(B,26) with e(B,177)

Swap e(B,51) with e(B,81)

Swap e(B,130) with e(B,40)

Swap e(B,159) with e(B,120)

Swap e(B,231) with e(B,160)

Swap e(B,274) with e(B,368)

Swap e(B,299) with e(B,280)

Swap e(B,303) with e(B,444)

Swap e(B,382) with e(B,405)

Swap e(B,407) with e(B,305)

For B = 2

Swap e(B,23) with e(B,40)

Swap e(B,106) with e(B,120)

Swap e(B,131) with e(B,81)

Swap e(B,210) with e(B,0)

Swap e(B,214) with e(B,177)

Swap e(B,275) with e(B,405)

Swap e(B,354) with e(B,305)

Swap e(B,379) with e(B,280)

Swap e(B,383) with e(B,444)

Swap e(B,462) with e(B,368)

For B = 3

Swap e(B,78) with e(B,120)

Swap e(B,103) with e(B,40)

Swap e(B,107) with e(B,177)

Swap e(B,186) with e(B,160)

Swap e(B,211) with e(B,81)

Swap e(B,326) with e(B,280)

Swap e(B,330) with e(B,444)

Swap e(B,355) with e(B,368)

Swap e(B,434) with e(B,305)

Swap e(B,463) with e(B,405)

### 5.1a.4	Packet data block type 15 (UAS-8)

#### 5.1a.4.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1069 information bits {d(0),d(1),...,d(1068)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1094 information bits {d(0),d(1),...,d(1093). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1097 information bits {d(0),d(1),...,d(1096).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,40

i1(k-41) = d(k)		for k = 41,...,554

i2(k-555) = d(k)		for k = 555,...,1068

And if a PAN is included:

pn(k-1069) = d(k)	for k = 1069,...,1093

And if a PAN and an eTFI are included:

et(k-1094) = d(k)		for k = 1094,...,1096

#### 5.1a.4.2	Header coding

The header coding is the same as for UAS-7 as specified in subclause 5.1a.3.2.

#### 5.1a.4.3	Data coding

Each data part, {i1(0),…,i1(513)} and {i2(0),…,i2(513)}, is coded as defined in subclause 5.1a.1.2, with N=514, resulting in two coded blocks of 1596 bits, {C1(0),...,C1(1595)} and {C2(0),...,C2(1595)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(15*k+j) for k=0,…,105, j=2, 4, 8, 9, 11, 12 and 13; and C(15*106+j) for j=2 and 4, except C(15*k+9) for k=13, 40, 67 and 94 that are not punctured. | C(15*k+9) for k=13, 40, 67 and 94; and C(15*k+5) for k=0, 3, 6, 9, 12, 15, 18, 21, 24, 27, 30, 33, 36, 39, 42, 45, 48, 51, 54, 57, 60, 63, 66, 69, 72, 76, 79, 82, 85, 88, 91, 94, 97, 100 and 103 |
| P2 | C(15*k+j) for k=0,…,105, j=0, 1, 3, 6, 7, 10 and 14; and C(15*106+j) for j=0,1 and 3, except C(15*k+1) for k=8, 31, 54, 77 and 100 that are not punctured. | C(15*k+1) for k=8, 31, 54, 77 and 100; and C(15*k+13) for k=1, 4, 7, 10, 13, 16, 19, 22, 25, 28, 31, 34, 38, 41, 44, 47, 50, 53, 56, 59, 62, 65, 68, 71, 74, 77, 80, 83, 86, 89, 92, 95, 98 and 101 |

If a PAN is not included, the result is two blocks of 856 bits, {c1(0),...,c1(855)} and {c2(0),...,c2(855)}.

If a PAN is included, the result is two blocks of 817 bits, {c1(0),...,c1(816)} and {c2(0),...,c2(816)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.4.4	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.4.5	Interleaving

The interleaving is the same as for UAS-7 as specified in subclause 5.1a.3.5.

#### 5.1a.4.6	Mapping on a burst

The mapping is the same as for UAS-7 as specified in subclause 5.1a.3.6.

### 5.1a.5	Packet data block type 16 (UAS-9)

#### 5.1a.5.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1229 information bits {d(0),d(1),...,d(1228)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1254 information bits {d(0),d(1),...,d(1253). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1257 information bits {d(0),d(1),...,d(1256).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,40

i1(k-41) = d(k)		for k = 41,...,634

i2(k-635) = d(k)		for k = 635,...,1228

And if a PAN is included:

pn(k-1229) = d(k)	for k = 1229,...,1253

And if a PAN and an eTFI are included:

et(k-1254) = d(k)		for k = 1254,...,1256

#### 5.1a.5.2	Header coding

The header coding is the same as for UAS-7 as specified in subclause 5.1a.3.2.

#### 5.1a.5.3	Data coding

Each data part, {i1(0),…,i1(593)} and {i2(0),…,i2(593)}, is coded as defined in subclause 5.1a.1.2, with N=594, resulting in two coded blocks of 1836 bits, {C1(0),...,C1(1835)} and {C2(0),...,C2(1835)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(15*k+j) for k=0,…,121, j=1, 2, 3, 5, 8, 9, 10 and 14; and C(15*122+j) for j=1, 2, 3 and 5 | C(15*k+13) for k=0, 3, 6, 9, 12, 15, 18, 21, 25, 28, 31, 34, 37, 40, 43, 47, 50, 53, 56, 59, 62, 65, 69, 72, 75, 78, 81, 84, 87, 91, 94, 97, 100, 103, 106, 109, 112, 116 and 119 |
| P2 | C(15*k+j) for k=0,…,121, j=0, 4, 5, 6, 7, 11, 12 and 13; and C(15*122+j) for j=0, 4 and 5; and C(15*121+9) | C(15*k+9) for k=1, 4, 7, 10, 13, 16, 19, 23, 26, 29, 32, 35, 38, 41, 44, 48, 51, 54, 57, 60, 63, 66, 70, 73, 76, 79, 82, 85, 88, 92, 95, 98, 101, 104, 107, 110, 114, 117 and 120 |
| P3 | C(15*k+j) for k=0,…,121, j=2, 3, 4, 8, 10, 11, 12 and 14; and C(15*122+j) for j=2, 3 and 4; and C(6) | C(15*k+6) for k=2, 5, 8, 11, 14, 17, 20, 24, 27, 30, 33, 36, 39, 42, 46, 49, 52, 55, 58, 61, 64, 68, 71, 74, 77, 80, 83, 86, 89, 93, 96, 99, 102, 105, 108, 111, 115, 118 and 121 |

If a PAN is not included, the result is two blocks of 856 bits, {c1(0),...,c1(855)} and {c2(0),...,c2(855)}.

If a PAN is included, the result is two blocks of 817 bits, {c1(0),...,c1(816)} and {c2(0),...,c2(816)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.5.4	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.5.5	Interleaving

The interleaving is the same as for UAS-7 as specified in subclause 5.1a.3.5.

#### 5.1a.5.6	Mapping on a burst

The mapping is the same as for UAS-7 as specified in subclause 5.1a.3.6.

### 5.1a.6	Packet data block type 17 (UAS-10)

#### 5.1a.6.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1402 information bits {d(0),d(1),...,d(1401)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1427 information bits {d(0),d(1),...,d(1426). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1430 information bits {d(0),d(1),...,d(1429).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,51

i1(k-52) = d(k)		for k = 52,...,501

i2(k-502) = d(k)		for k = 502,...,951

i3(k-952) = d(k)		for k = 952,...,1401

And if a PAN is included:

pn(k-1402) = d(k)	for k = 1402,...,1426

And if a PAN and an eTFI are included:

et(k-1427) = d(k)		for k = 1427,...,1429

#### 5.1a.6.2	Header coding

The header {h(0),...,h(51)} is coded as defined in subclause 5.1a.1.1, with N=52, resulting in a block of 180 bits, {C(0),...,C(179)}.

The code is punctured in such a way that the following coded bits:

{ C(15*k+12) for k=0,…,11} are not transmitted

This results in a block of 168 bits, {hc(0),...,hc(167)}.

#### 5.1a.6.3	Data coding

Each data part, {i1(0),…,i1(449)}, {i2(0),…,i2(449)} and {i3(0),…,i3(449)}, is coded as defined in subclause 5.1a.1.2, with N=450, resulting in three coded blocks of 1404 bits, {C1(0),...,C1(1403)}, {C2(0),...,C2(1403)} and {C3(0),...,C3(1403)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(18*k+j) for k=0,…,77, j=2, 4, 5, 7, 8, 9, 11, 12, 13, 15 and 16, except C(18*k+7) for k=2, 8, 13, 19, 24, 30, 35, 41, 46, 52, 57, 63, 68 and 74 that are not punctured. | C(18*k+7) for k=2, 8, 13, 19, 24, 30, 35, 41, 46, 52, 57, 63, 68 and 74; and C(18*k) for k=0, 6, 13, 19, 26, 32, 39, 45, 52, 58, 65 and 71 |
| P2 | C(18*k+j) for k=0,…,77, j=0, 1, 3, 4, 6, 8, 10, 11, 13, 14 and 17, except C(18*k+3) for k=4, 10, 15, 21, 26, 32, 37, 43, 48, 54, 59, 65, 70 and 76 that are not punctured. | C(18*k+3) for k=4, 10, 15, 21, 26, 32, 37, 43, 48, 54, 59, 65, 70 and 76; and C(18*k+15) for k=2, 8, 15, 21, 28, 34, 41, 47, 54, 60, 67 and 73 |
| P3 | C(18*k+j) for k=0,…,77, j=1, 2, 3, 5, 6, 7, 9, 10, 14, 16 and 17, except C(18*k+1) for k=0, 6, 11, 17, 22, 28, 33, 39, 44, 50, 55, 61, 66 and 72 that are not punctured. | C(18*k+1) for k=0, 6, 11, 17, 22, 28, 33, 39, 44, 50, 55, 61, 66 and 72; and C(18*k+12) for k=4, 10, 17, 23, 30, 36, 43, 49, 56, 62, 69 and 75 |

If a PAN is not included, the result is three blocks of 560 bits, {c1(0),...,c1(559)}, {c2(0),...,c2(559)} and {c3(0),...,c3(559)}, where c1 corresponds to i1, c2 to i2 and c3 to i3.

If a PAN is included, the result is three blocks of 534 bits, {c1(0),...,c1(533)}, {c2(0),...,c2(533)} and {c3(0),...,c3(533)}, where c1 corresponds to i1, c2 to i2 and c3 to i3.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, and C3 and c3 to i3.

#### 5.1a.6.4	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.6.5	Interleaving

a)	Header

The header, {hc(0),...,hc(167)}, is interleaved as defined in subclause 5.1a.2.1, with NC=168 and a=25, resulting in a block of 168 bits, {hi(0),...,hi(167)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,559

dc(k) = c2(k-560)	for k = 560,...,1119

dc(k) = c3(k-1120)	for k = 1120,...,1679

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,...,77

dc(k) = c1(k-78)		for k = 78,…,611

dc(k) = c2(k-612)	for k = 612,…,1145

dc(k) = c3(k-1146)	for k = 1146,…,.1679

The block {dc(0),...,dc(1679)} is interleaved as defined in subclause 5.1a.2.1, with NC=1680 and a=173, resulting in a block of 1680 bits, {di(0),...,di(1679)}.

#### 5.1a.6.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(420B+j)		for j = 0,...,211

e(B,j) = hi(42B+j-212)	for j = 212,...,231

e(B,j) = q(2B+j-232)		for j = 232,233

e(B,j) = hi(42B+j-214)	for j = 234,...,255

e(B,j) = di(420B+j-44)	for j = 256,...,463

where

q(0),q(1),...,q(7) = 1,1,1,1,1,1,1,1 identifies the coding scheme UAS-10 or UAS-11.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,192+k) with e(B,214+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 16, 17, 42, 43, 46, 47, 50, 51, 54, 55, 58, 59, 62 and 63.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,50) with e(B,120)

Swap e(B,75) with e(B,81)

Swap e(B,99) with e(B,177)

Swap e(B,174) with e(B,40)

Swap e(B,198) with e(B,160)

Swap e(B,267) with e(B,368)

Swap e(B,366) with e(B,305)

Swap e(B,390) with e(B,444)

Swap e(B,391) with e(B,280)

Swap e(B,415) with e(B,405)

For B = 1

Swap e(B,7) with e(B,0)

Swap e(B,31) with e(B,177)

Swap e(B,106) with e(B,21)

Swap e(B,130) with e(B,120)

Swap e(B,155) with e(B,40)

Swap e(B,226) with e(B,160)

Swap e(B,251) with e(B,368)

Swap e(B,298) with e(B,280)

Swap e(B,322) with e(B,444)

Swap e(B,323) with e(B,321)

Swap e(B,347) with e(B,405)

Swap e(B,446) with e(B,305)

For B = 2

Swap e(B,38) with e(B,40)

Swap e(B,62) with e(B,160)

Swap e(B,87) with e(B,120)

Swap e(B,186) with e(B,81)

Swap e(B,210) with e(B,177)

Swap e(B,211) with e(B,0)

Swap e(B,279) with e(B,405)

Swap e(B,378) with e(B,368)

Swap e(B,403) with e(B,305)

Swap e(B,427) with e(B,444)

For B = 3

Swap e(B,19) with e(B,160)

Swap e(B,118) with e(B,81)

Swap e(B,143) with e(B,40)

Swap e(B,167) with e(B,177)

Swap e(B,214) with e(B,120)

Swap e(B,239) with e(B,280)

Swap e(B,310) with e(B,368)

Swap e(B,335) with e(B,305)

Swap e(B,359) with e(B,444)

Swap e(B,434) with e(B,321)

Swap e(B,458) with e(B,405)

### 5.1a.7	Packet data block type 18 (UAS-11)

#### 5.1a.7.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1594 information bits {d(0),d(1),...,d(1593)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1619 information bits {d(0),d(1),...,d(1618). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1622 information bits {d(0),d(1),...,d(1621).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,51

i1(k-52) = d(k)		for k = 52,...,565

i2(k-566) = d(k)		for k = 566,...,1079

i3(k-1080) = d(k)		for k = 1080,...,1593

And if a PAN is included:

pn(k-1594) = d(k)	for k = 1594,...,1618

And if a PAN and an eTFI are included:

et(k-1619) = d(k)		for k = 1619,...,1621

#### 5.1a.7.2	Header coding

The header coding is the same as for UAS-10 as specified in subclause 5.1a.6.2.

#### 5.1a.7.3	Data coding

Each data part, {i1(0),…,i1(513)}, {i2(0),…,i2(513)} and {i3(0),…,i3(513)}, is coded as defined in subclause 5.1a.1.2, with N=514, resulting in three coded blocks of 1596 bits, {C1(0),...,C1(1595)}, {C2(0),...,C2(1595)} and {C3(0),...,C3(1595)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(57*k+j) for k=0,…,27, j=0, 2, 3, 5, 6, 8, 9, 11, 12, 14, 15, 17, 19, 20, 22, 23, 25, 26, 28, 29, 31, 32, 34, 35, 37, 39, 40, 42, 43, 45, 46, 48, 49, 51, 52, 54 and 55 | C(57*k+38) for k=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25 and 26 |
| P2 | C(57*k+j) for k=0,…,27, j=1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18, 20, 21, 23, 24, 26, 27, 29, 30, 32, 33, 35, 36, 38, 40, 41, 43, 44, 46, 47, 49, 50, 52, 53, 55 and 56 | C(57*k+2) for k=0, 1, 2, 3, 4, 5, 6, 7, 8, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 24, 25, 26 and 27 |
| P3 | C(57*k+j) for k=0,…,27, j=0, 2, 4, 5, 7, 8, 10, 11, 13, 14, 16, 17, 19, 21, 22, 24, 25, 27, 28, 30, 31, 33, 34, 36, 37, 39, 41, 42, 44, 45, 47, 48, 50, 51, 53, 54 and 56 | C(57*k+18) for k=0, 1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 19, 20, 21, 22, 23, 24, 25, 26 and 27 |

If a PAN is not included, the result is three blocks of 560 bits, {c1(0),...,c1(559)}, {c2(0),...,c2(559)} and {c3(0),...,c3(559)}.

If a PAN is included, the result is three blocks of 534 bits, {c1(0),...,c1(533)}, {c2(0),...,c2(533)} and {c3(0),...,c3(533)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, and C3 and c3 to i3.

#### 5.1a.7.4	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.7.5	Interleaving

a)	Header

The interleaving is the same as for UAS-10 as specified in subclause 5.1a.6.5.

b)	Data and PAN

If a PAN is not included, the following definitions apply:

dc1(k) = c1(k)			for k = 0,...,559

dc2(k) = c2(k)			for k = 0,...,559

dc3(k) = c3(k)			for k = 0,...,559

If a PAN is included, the following definitions apply:

dc1(k) = ac(3k)			for k = 0,…,25

dc1(k) = c1(k-26)		for k = 26,…,559

dc2(k) = ac(3k+1)		for k = 0,…,25

dc2(k) = c2(k-26)		for k = 26,…,559

dc3(k) = ac(3k+2)		for k = 0,…,25

dc3(k) = c3(k-26)		for k = 26,…,559

The three blocks {dc1(0),...,dc1(559)}, {dc2(0),...,dc2(559)} and {dc3(0),...,dc3(559)} are separately interleaved as defined in subclause 5.1a.2.2, with NC=560 and a=359, resulting in the three blocks {di1(0),…,di1(559)}, {di2(0),…,di2(559)} and {di3(0),…,di3(559)}, where di1 corresponds to dc1, di2 to dc2 and di3 to dc3.

The blocks are put together as one entity as described by the following rule:

di(k) = di1(k)				for k = 0,...,559

di(k) = di2(k-560)		for k = 560,...,1119

di(k) = di3(k-1120)		for k = 1120,...,1679

#### 5.1a.7.6	Mapping on a burst

a)	Straightforward mapping

The mapping is the same as for UAS-10 as specified in subclause 5.1a.6.6 a).

b)	Bit swapping

The bit swapping is the same as for UAS-10 as specified in subclause 5.1a.6.6. b)

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,15) with e(B,40)

Swap e(B,58) with e(B,81)

Swap e(B,115) with e(B,160)

Swap e(B,158) with e(B,177)

Swap e(B,187) with e(B,120)

Swap e(B,238) with e(B,280)

Swap e(B,274) with e(B,405)

Swap e(B,303) with e(B,305)

Swap e(B,346) with e(B,368)

Swap e(B,403) with e(B,444)

For B = 1

Swap e(B,11) with e(B,160)

Swap e(B,54) with e(B,177)

Swap e(B,83) with e(B,81)

Swap e(B,126) with e(B,120)

Swap e(B,155) with e(B,21)

Swap e(B,198) with e(B,0)

Swap e(B,234) with e(B,368)

Swap e(B,299) with e(B,405)

Swap e(B,342) with e(B,444)

Swap e(B,371) with e(B,280)

Swap e(B,414) with e(B,305)

Swap e(B,443) with e(B,321)

For B = 2

Swap e(B,22) with e(B,81)

Swap e(B,79) with e(B,177)

Swap e(B,151) with e(B,120)

Swap e(B,194) with e(B,160)

Swap e(B,267) with e(B,405)

Swap e(B,310) with e(B,444)

Swap e(B,339) with e(B,280)

Swap e(B,382) with e(B,305)

Swap e(B,439) with e(B,368)

For B = 3

Swap e(B,18) with e(B,177)

Swap e(B,47) with e(B,120)

Swap e(B,90) with e(B,160)

Swap e(B,119) with e(B,0)

Swap e(B,162) with e(B,40)

Swap e(B,227) with e(B,81)

Swap e(B,263) with e(B,444)

Swap e(B,335) with e(B,368)

Swap e(B,378) with e(B,405)

Swap e(B,407) with e(B,280)

Swap e(B,450) with e(B,305)

### 5.1a.8	Packet data block type 19 (UBS-5)

#### 5.1a.8.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 479 information bits {d(0),d(1),...,d(478)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 504 information bits {d(0),d(1),...,d(503).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,28

i(k-29) = d(k)			for k = 29,...,478

And if a PAN is included:

pn(k-479) = d(k)		for k = 479,...,503

#### 5.1a.8.2	Header coding

The header {h(0),...,h(28)} is coded as defined in subclause 5.1a.1.1, with N=29, resulting in a block of 111 bits, {C(0),...,C(110)}.

The code is punctured in such a way that the following coded bits:

{ C(37*k+36) for k=0,…,2} are not transmitted

This results in a block of 108 bits, {hc(0),...,hc(107)}.

#### 5.1a.8.3	Data coding

The data, {i(0),…,i(449)}, is coded as defined in subclause 5.1a.1.2, with N=450, resulting in a coded block of 1404 bits, {C(0),...,C(1403)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(27*k+j) for k=0,…,51, j=5, 8, 11, 14, 17, 20, 23 and 26 | C(27*k) for k=0,...,51; and C(27*k+10) for k=0, 1, 3, 5, 7, 9, 11, 13, 14, 16, 18, 20, 22, 24, 26, 27, 29, 31, 33, 35, 37, 39, 40, 42, 44, 46, 48 and 50 |
| P2 | C(27*k+j) for k=0,…,51, j=2, 4, 6, 12, 13, 18, 22 and 24 | C(27*k+16) for k=0,...,51; and C(27*k+9) for k=0, 2, 4, 6, 8, 10, 12, 13, 15, 17, 19, 21, 23, 25, 26, 28, 30, 32, 34, 36, 38, 39, 41, 43, 45, 47, 49 and 51 |

If a PAN is not included, the result is a block of 988 bits, {c(0),...,c(987)}.

If a PAN is included, the result is a block of 908 bits, {c(0),...,c(907)}.

#### 5.1a.8.4	PAN coding

The PAN {pn(0),...,pn(24)}, if included, is coded as defined in subclause 5.1a.1.4, resulting in a block of 90 bits, {C(0),...,C(89)}.

The code is punctured in such a way that the following coded bits:

{C(11+k) , C(17+k), C(23+k,) C(32+k), C(41+k) for k = 0, 45} are not transmitted

This results in a block of 80 bits, {ac(0),...,ac(79)}.

#### 5.1a.8.5	Interleaving

a)	Header

The header, {hc(0),...,hc(107)}, is interleaved as defined in subclause 5.1a.2.1, with NC=108 and a=23, resulting in a block of 108 bits, {hi(0),...,hi(107)}.

b)	Data and PAN

If a PAN is not included, the following rule applies:

dc(k) = c(k)			for k = 0,...,987

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,79

dc(k) = c(k-80)			for k = 80,…,987

The block {dc(0),...,dc(987)} is interleaved as defined in subclause 5.1a.2.1, with NC=988 and a=108, resulting in a block of 988 bits, {di(0),...,di(987)}.

#### 5.1a.8.6	Mapping on a burst

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(247B+j)		for j = 0,...,123

e(B,j) = hi(27B+j-124)	for j = 124,...,137

e(B,j) = q(2B+j-138)		for j = 138,139

e(B,j) = hi(27B+j-126)	for j = 140,...,152

e(B,j) = di(247B+j-29)	for j = 153,...,275

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme UBS-5 or UBS-6.

### 5.1a.9	Packet data block type 20 (UBS-6)

#### 5.1a.9.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 623 information bits {d(0),d(1),...,d(622)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 648 information bits {d(0),d(1),...,d(647).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,28

i(k-29) = d(k)			for k = 29,...,622

And if a PAN is included:

pn(k-623) = d(k)		for k = 622,...,647

#### 5.1a.9.2	Header coding

The header coding is the same as for UBS-5 as specified in subclause 5.1a.8.2.

#### 5.1a.9.3	Data coding

The data, {i(0),…,i(593)}, is coded as defined in subclause 5.1a.1.2, with N=594, resulting in a coded block of 1836 bits, {C(0),...,C(1835)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(15*k+j) for k=0,…,121, j=2, 4, 8, 9, 11, 12 and 13; and C(15*122+j) for j=2 and 4, except C(15*k+9) for k=9, 24, 39, 54, 69, 84, 99 and 114 that are not punctured | C(15*k+9) for k=9, 24, 39, 54, 69, 84, 99 and 114; and C(15*k+5) for k=0, 1, 3, 5, 6, 8, 10, 11, 13, 15, 17, 18, 20, 22, 23, 25, 27, 28, 30, 32, 34, 35, 37, 39, 40, 42, 44, 45, 47, 49, 51, 52, 54, 56, 57, 59, 61, 62, 64, 66, 68, 69, 71, 73, 74, 76, 78, 79, 81, 83, 85, 86, 88, 90, 91, 93, 95, 96, 98, 100, 102, 103, 105, 107, 108, 110, 112, 113, 115, 117, 119 and 120 |
| P2 | C(15*k+j) for k=0,…,121, j=0, 1, 3, 6, 7, 10 and 14; and C(15*122+j) for j=0, 1 and 3, except C(15*k+1) for k=2, 17, 32, 47, 62, 77, 92, 107 and 122 that are not punctured | C(15*k+1) for k=2, 17, 32, 47, 62, 77, 92, 107 and 122; and C(15*k+13) for k=0, 2, 4, 5, 7, 9, 11, 12, 14, 16, 17, 19, 21, 22, 24, 26, 28, 29, 31, 33, 34, 36, 38, 39, 41, 43, 45, 46, 48, 50, 51, 53, 55, 56, 58, 60, 62, 63, 65, 67, 68, 70, 72, 73, 75, 77, 79, 80, 82, 84, 85, 87, 89, 90, 92, 94, 96, 97, 99, 101, 102, 104, 106, 107, 109, 111, 113, 114, 116, 118 and 119 |

If a PAN is not included, the result is a block of 988 bits, {c(0),...,c(987)}.

If a PAN is included, the result is a block of 908 bits, {c(0),...,c(907)}.

#### 5.1a.9.4	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.9.5	Interleaving

The interleaving is the same as for UBS-5 as specified in subclause 5.1a.8.5.

#### 5.1a.9.6	Mapping on a burst

The mapping is the same as for UBS-5 as specified in subclause 5.1a.8.6.

### 5.1a.10	Packet data block type 21 (UBS-7)

#### 5.1a.10.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 940 information bits {d(0),d(1),...,d(939)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 965 information bits {d(0),d(1),...,d(964).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,39

i1(k-40) = d(k)		for k = 40,...,489

i2(k-490) = d(k)		for k = 490,...,939

And if a PAN is included:

pn(k-940) = d(k)		for k = 940,...,959+lt

#### 5.1a.10.2	Header coding

The header {h(0),...,h(39)} is coded as defined in subclause 5.1a.1.1, with N=40, resulting in a block of 144 bits, {C(0),...,C(143)}.

No puncturing is applied. The coded header is defined as:

hc(k) = C(k)			for k = 0,...,143.

#### 5.1a.10.3	Data coding

Each data part, {i1(0),…,i1(449)} and {i2(0),…,i2(449)}, is coded as defined in subclause 5.1a.1.2, with N=450, resulting in two coded blocks of 1404 bits, {C1(0),...,C1(1403)} and {C2(0),...,C2(1403)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(33*k+j) for k=0,…,41, j=4, 8, 10, 14, 20, 23, 25, 29 and 30; and C(33*42+j) for j=4, 8, 10 and 14, except C(33*k+20) for k=6, 12, 18, 24, 30 and 36 that are not punctured | C(33*k+20) for k=6, 12, 18, 24, 30 and 36; and C(33*k+18) for k=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18, 20, 21, 22, 23, 25, 26, 27, 28, 30, 31, 32, 33, 35, 36, 37, 38, 40 and 41 |
| P2 | C(33*k+j) for k=0,…,41, j=2, 5, 7, 12, 17, 19, 24, 26 and 28; and C(33*42+j) for j=2, 5, 7, 12 and 17, except C(33*k+26) for k=3, 9, 15, 21, 27, 33 and 39 that are not punctured | C(33*k+26) for k=3, 9, 15, 21, 27, 33 and 39; and C(33*k+13) for k=0, 1, 3, 4, 5, 6, 8, 9, 10, 11, 13, 14, 15, 16, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 33, 34, 35, 36, 38, 39 and 40 |

If a PAN is not included, the result is two blocks of 1028 bits, {c1(0),...,c1(1027)} and {c2(0),...,c2(1027)}.

If a PAN is included, the result is two blocks of 988 bits, {c1(0),...,c1(987)} and {c2(0),...,c2(987)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.10.4	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.10.5	Interleaving

a)	Header

The header, {hc(0),...,hc(143)}, is interleaved as defined in subclause 5.1a.2.1, with NC=144 and a=29, resulting in a block of 144 bits, {hi(0),...,hi(143)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,1027

dc(k) = c2(k-1028)	for k = 1028,...,2055

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,79

dc(k) = c1(k-80)		for k = 80,…,1067

dc(k) = c2(k-1068)	for k = 1068,…,2055

The block {dc(0),...,dc(2055)} is interleaved as defined in subclause 5.1a.2.1, with NC=2056 and a=403, resulting in a block of 2056 bits, {di(0),...,di(2055)}.

#### 5.1a.10.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(514B+j)		for j = 0,...,257

e(B,j) = hi(36B+j-258)	for j = 258,...,275

e(B,j) = q(2B+j-276)		for j = 276,277

e(B,j) = hi(36B+j-260)	for j = 278,...,295

e(B,j) = di(514B+j-38)	for j = 296,...,551

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme UBS-7 or UBS-8.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,240+k) with e(B,258+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 16, 17, 38, 39, 42, 43, 46, 47, 50, 51, 54 and 55.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,58) with e(B,81)

Swap e(B,70) with e(B,177)

Swap e(B,99) with e(B,120)

Swap e(B,210) with e(B,201)

Swap e(B,251) with e(B,160)

Swap e(B,318) with e(B,321)

Swap e(B,330) with e(B,444)

Swap e(B,359) with e(B,500)

Swap e(B,470) with e(B,368)

Swap e(B,499) with e(B,340)

Swap e(B,511) with e(B,405)

For B = 1

Swap e(B,102) with e(B,201)

Swap e(B,131) with e(B,81)

Swap e(B,143) with e(B,160)

Swap e(B,242) with e(B,120)

Swap e(B,254) with e(B,177)

Swap e(B,362) with e(B,368)

Swap e(B,391) with e(B,520)

Swap e(B,403) with e(B,405)

Swap e(B,502) with e(B,340)

Swap e(B,514) with e(B,444)

Swap e(B,543) with e(B,500)

For B = 2

Swap e(B,23) with e(B,0)

Swap e(B,35) with e(B,201)

Swap e(B,134) with e(B,40)

Swap e(B,146) with e(B,160)

Swap e(B,175) with e(B,120)

Swap e(B,263) with e(B,81)

Swap e(B,275) with e(B,177)

Swap e(B,394) with e(B,500)

Swap e(B,406) with e(B,444)

Swap e(B,435) with e(B,368)

Swap e(B,546) with e(B,405)

For B = 3

Swap e(B,26) with e(B,40)

Swap e(B,38) with e(B,177)

Swap e(B,67) with e(B,120)

Swap e(B,178) with e(B,201)

Swap e(B,207) with e(B,0)

Swap e(B,219) with e(B,160)

Swap e(B,266) with e(B,81)

Swap e(B,327) with e(B,500)

Swap e(B,438) with e(B,405)

Swap e(B,467) with e(B,321)

Swap e(B,479) with e(B,444)

### 5.1a.11	Packet data block type 22 (UBS-8)

#### 5.1a.11.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1228 information bits {d(0),d(1),...,d(1227)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1253 information bits {d(0),d(1),...,d(1252).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,39

i1(k-40) = d(k)		for k = 40,...,633

i2(k-634) = d(k)		for k = 634,...,1227

And if a PAN is included:

pn(k-1228) = d(k)	for k = 1228,...,1252

#### 5.1a.11.2	Header coding

The header coding is the same as for UBS-7 as specified in subclause 5.1a.10.2.

#### 5.1a.11.3	Data coding

Each data part, {i1(0),…,i1(593)} and {i2(0),…,i2(593)}, is coded as defined in subclause 5.1a.1.2, with N=594, resulting in two coded blocks of 1836 bits, {C1(0),...,C1(1835)} and {C2(0),...,C2(1835)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(18*k+j) for k=0,…,101, j=2, 4, 8, 9, 10, 14, 16 and 17, except C(18*k+17) for k=4, 17, 30, 43, 56, 69, 82 and 95 that are not punctured | C(18*k+17) for k=4, 17, 30, 43, 56, 69, 82 and 95; and C(18*k+3) for k=0, 3, 6, 9, 12, 15, 19, 22, 25, 28, 31, 35, 38, 41, 44, 47, 51, 54, 57, 60, 63, 66, 70, 73, 76, 79, 82, 86, 89, 92, 95 and 98 |
| P2 | C(18*k+j) for k=0,…,101, j=0, 1, 5, 6, 7, 11, 12 and 13, except C(18*k+7) for k=8, 21, 34, 47, 60, 73, 86 and 99 that are not punctured | C(18*k+7) for k=8, 21, 34, 47, 60, 73, 86 and 99; and C(18*k+15) for k=1, 4, 7, 11, 14, 17, 20, 23, 27, 30, 33, 36, 39, 43, 46, 49, 52, 55, 58, 62, 65, 68, 71, 74, 78, 81, 84, 87, 90, 94, 97 and 100 |

If a PAN is not included, the result is two blocks of 1028 bits, {c1(0),...,c1(1027)} and {c2(0),...,c2(1027)}.

If a PAN is included, the result is two blocks of 988 bits, {c1(0),...,c1(987)} and {c2(0),...,c2(987)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.11.4	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.11.5	Interleaving

The interleaving is the same as for UBS-7 as specified in subclause 5.1a.10.5.

#### 5.1a.11.6	Mapping on a burst

The mapping is the same as for UBS-7 as specified in subclause 5.1a.10.6.

### 5.1a.12	Packet data block type 23 (UBS-9)

#### 5.1a.12.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1402 information bits {d(0),d(1),...,d(1401)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1427 information bits {d(0),d(1),...,d(1426).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,51

i1(k-52) = d(k)		for k = 52,...,501

i2(k-502) = d(k)		for k = 502,...,951

i3(k-952) = d(k)		for k = 952,...,1401

And if a PAN is included:

pn(k-1402) = d(k)	for k = 1402,...,1426

#### 5.1a.12.2	Header coding

The header {h(0),...,h(51)} is coded as defined in subclause 5.1a.1.1, with N=52, resulting in a block of 180 bits, {C(0),...,C(179)}.

Some bits of the code are repeated in the following way:

hc(k) = C(k) for k = 0, 1, … 179 and

hc(180) = C(8), hc(181) = C(53), hc(182) = C(98), hc(183) = C(143)

This results in a block of 184 bits, {hc(0),...,hc(183)}.

#### 5.1a.12.3	Data coding

Each data part, {i1(0),…,i1(449)}, {i2(0),…,i2(449)} and {i3(0),…,i3(449)}, is coded as defined in subclause 5.1a.1.2, with N=450, resulting in three coded blocks of 1404 bits, {C1(0),...,C1(1403)}, {C2(0),...,C2(1403)} and {C3(0),...,C3(1403)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(21*k+j) for k=0,…,65, j=2, 4, 5, 7, 8, 10, 13, 14, 16, 18 and 20; and C(21*66+j) for j=2, 4, 5, 7, 8, 10, 13, 14 and 16, except C(21*k+7) for k=16, 33 and 50 that are not punctured | C(21*k+7) for k=16, 33 and 50; and C(21*k+9) for k=0, 2, 5, 8, 10, 13, 16, 18, 21, 24, 26, 29, 32, 34, 37, 40, 42, 45, 48, 50, 53, 56, 58, 61 and 64 |
| P2 | C(21*k+j) for k=0,…,65, j=0, 1, 3, 6, 9, 11, 12, 14, 15, 17 and 19; and C(21*66+j) for j=0, 1, 3, 6, 9, 11, 12, 14, 15 and 17, except C(21*k+17) for k=4, 21, 38 and 55 that are not punctured | C(21*k+17) for k=4, 21, 38 and 55; and C(21*k+20) for k=0, 3, 6, 8, 11, 14, 16, 19, 22, 24, 27, 30, 32, 35, 38, 41, 43, 46, 49, 51, 54, 57, 59 and 62 |
| P3 | C(21*k+j) for k=0,…,65, j=0, 2, 5, 7, 8, 10, 11, 13, 16, 17 and 19; and C(21*66+j) for j=0, 2, 5, 7, 8, 10, 11, 13, 16 and 17, except C(21*k+10) for k=10, 27, 44 and 61 that are not punctured | C(21*k+10) for k=10, 27, 44 and 61; and C(21*k+12) for k=1, 4, 7, 9, 12, 15, 17, 20, 23, 25, 28, 31, 33, 36, 39, 41, 44, 47, 49, 52, 55, 57, 60 and 63 |

If a PAN is not included, the result is three blocks of 672 bits, {c1(0),...,c1(671)}, {c2(0),...,c2(671)} and {c3(0),...,c3(671)}.

If a PAN is included, the result is three blocks of 644 bits, {c1(0),...,c1(643)}, {c2(0),...,c2(643)} and {c3(0),...,c3(643)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, and C3 and c3 to i3.

#### 5.1a.12.4	PAN coding

The PAN {pn(0),...,pn(24)}, if included, is coded as defined in subclause 5.1a.1.4, resulting in a block of 90 bits, {C(0),...,C(89)}.

The code is punctured in such a way that the following coded bits:

{C(15*k+5) for k = 0,1,…5} are not transmitted

This results in a block of 84 bits, {ac(0),...,ac(83)}.

#### 5.1a.12.5	Interleaving

a)	Header

The header, {hc(0),...,hc(183)}, is interleaved as defined in subclause 5.1a.2.1, with NC=184 and a=33, resulting in a block of 184 bits, {hi(0),...,hi(183)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,671

dc(k) = c2(k-672)	for k = 672,...,1343

dc(k) = c3(k-1344)	for k = 1344,...,2015

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,..,83

dc(k) = c1(k-84)		for k = 84,…,727

dc(k) = c2(k-728)	for k = 728,…,1371

dc(k) = c3(k-1372)	for k = 1372,…,2015

The block {dc(0),...,dc(2015)} is interleaved as defined in subclause 5.1a.2.1, with NC=2016 and a=229, resulting in a block of 2016 bits, {di(0),...,di(2015)}.

#### 5.1a.12.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(504B+j)		for j = 0,...,251

e(B,j) = hi(46B+j-252)	for j = 252,...,275

e(B,j) = q(2B+j-276)		for j = 276,277

e(B,j) = hi(46B+j-254)	for j = 278,...,299

e(B,j) = di(504B+j-48)	for j = 300,...,551

where

q(0),q(1),...,q(7) = 1,1,1,1,1,1,1,1 identifies the coding scheme UBS-9.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,228+k) with e(B,254+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 16, 17, 20 and 21.

Swap e(B,278+k) with e(B,300+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 16, 17, 20 and 21.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,90) with e(B,40)

Swap e(B,91) with e(B,201)

Swap e(B,182) with e(B,81)

Swap e(B,183) with e(B,160)

Swap e(B,254) with e(B,120)

Swap e(B,255) with e(B,177)

Swap e(B,322) with e(B,500)

Swap e(B,367) with e(B,520)

Swap e(B,414) with e(B,405)

Swap e(B,459) with e(B,340)

Swap e(B,506) with e(B,444)

Swap e(B,551) with e(B,329)

For B = 1

Swap e(B,34) with e(B,160)

Swap e(B,79) with e(B,120)

Swap e(B,126) with e(B,177)

Swap e(B,170) with e(B,81)

Swap e(B,171) with e(B,201)

Swap e(B,310) with e(B,329)

Swap e(B,311) with e(B,405)

Swap e(B,402) with e(B,500)

Swap e(B,403) with e(B,444)

Swap e(B,494) with e(B,368)

Swap e(B,539) with e(B,340)

For B = 2

Swap e(B,22) with e(B,120)

Swap e(B,67) with e(B,0)

Swap e(B,114) with e(B,201)

Swap e(B,159) with e(B,40)

Swap e(B,206) with e(B,177)

Swap e(B,251) with e(B,81)

Swap e(B,278) with e(B,444)

Swap e(B,390) with e(B,329)

Swap e(B,391) with e(B,368)

Swap e(B,482) with e(B,500)

Swap e(B,483) with e(B,405)

For B = 3

Swap e(B,10) with e(B,40)

Swap e(B,11) with e(B,160)

Swap e(B,102) with e(B,81)

Swap e(B,103) with e(B,177)

Swap e(B,194) with e(B,120)

Swap e(B,239) with e(B,0)

Swap e(B,266) with e(B,201)

Swap e(B,334) with e(B,405)

Swap e(B,379) with e(B,500)

Swap e(B,426) with e(B,444)

Swap e(B,470) with e(B,329)

Swap e(B,471) with e(B,368)

### 5.1a.13	Packet data block type 24 (UBS-10)

#### 5.1a.13.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1834 information bits {d(0),d(1),...,d(1833)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1859 information bits {d(0),d(1),...,d(1858).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,51

i1(k-52) = d(k)		for k = 52,...,645

i2(k-646) = d(k)		for k = 646,...,1239

i3(k-1240) = d(k)		for k = 1240,...,1833

And if a PAN is included:

pn(k-1834) = d(k)	for k = 1834,...,1858

#### 5.1a.13.2	Header coding

The header {h(0),...,h(51)} is coded as defined in subclause 5.1a.1.1, with N=52, resulting in a block of 180 bits, {C(0),...,C(179)}.

The code is punctured in such a way that the following coded bits:

{ C(15*k+14) for k=0,…,11} are not transmitted.

This results in a block of 168 bits, {hc(0),...,hc(167)}.

#### 5.1a.13.3	Data coding

Each data part, {i1(0),…,i1(593)}, {i2(0),…,i2(593)} and {i3(0),…,i3(593)}, is coded as defined in subclause 5.1a.1.2, with N=594, resulting in three coded blocks of 1836 bits, {C1(0),...,C1(1835)}, {C2(0),...,C2(1835)} and {C3(0),...,C3(1835)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(15*k+j) for k=0,…,121, j=2, 4, 5, 6, 8, 11, 12 and 13; and C(15*122+j) for j=2, 4 and 5, except C(15*k+8) for k=10, 51 and 92 that are not punctured | C(15*k+8) for k=10, 51 and 92; and C(15*k+1) for k=0, 4, 9, 14, 19, 24, 29, 34, 39, 44, 48, 53, 58, 63, 68, 73, 78, 83, 88, 93, 97, 102, 107, 112 and 117 |
| P2 | C(15*k+j) for k=0,…,121, j=0, 1, 3, 7, 8, 9, 10 and 14; and C(15*122+j) for j=0, 1 and 3, except C(15*k) for k=20, 61 and 102 that are not punctured | C(15*k) for k=20, 61 and 102; and C(15*k+12) for k=1, 6, 11, 16, 21, 26, 31, 35, 40, 45, 50, 55, 60, 65, 70, 75, 79, 84, 89, 94, 99, 104, 109, 114 and 119 |
| P3 | C(15*k+j) for k=0,…,121, j=0, 2, 5, 6, 7, 11, 13 and 14; and C(15*122+j) for j=0, 2 and 5, except C(15*k+2) for k=30, 71 and 112 that are not punctured | C(15*k+2) for k=30, 71 and 112; and C(15*k+9) for k=3, 8, 13, 17, 22, 27, 32, 37, 42, 47, 52, 57, 62, 66, 71, 76, 81, 86, 91, 96, 101, 106, 110, 115 and 120 |

If a PAN is not included, the result is three blocks of 860 bits, {c1(0),...,c1(859)}, {c2(0),...,c2(859)} and {c3(0),...,c3(859)}.

If a PAN is included, the result is three blocks of 832 bits, {c1(0),...,c1(831)}, {c2(0),...,c2(831)} and {c3(0),...,c3(831)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, and C3 and c3 to i3.

#### 5.1a.13.4	PAN coding

The PAN coding is the same as for UBS-9 as specified in subclause 5.1a.12.4.

#### 5.1a.13.5	Interleaving

a)	Header

The header, {hc(0),...,hc(167)}, is interleaved as defined in subclause 5.1a.2.1, with NC=168 and a=29, resulting in a block of 168 bits, {hi(0),...,hi(167)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,859

dc(k) = c2(k-860)	for k = 860,...,1719

dc(k) = c3(k-1720)	for k = 1720,...,2579

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,83

dc(k) = c1(k-84)		for k = 84,…,915

dc(k) = c2(k-916)	for k = 916,…,1747

dc(k) = c3(k-1748)	for k = 1748,…,2579

The block {dc(0),...,dc(2579)} is interleaved as defined in subclause 5.1a.2.1, with NC=2580 and a=179, resulting in a block of 2580 bits, {di(0),...,di(2579)}.

#### 5.1a.13.6	Mapping on a burst

a)	Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(645B+j)		for j = 0,...,324

e(B,j) = hi(42B+j-325)	for j = 325,...,344

e(B,j) = q(3B+j-345)		for j = 345

e(B,j) = hi(42B+j-326)	for j = 346

e(B,j) = q(3B+j-346)		for j = 347,348

e(B,j) = hi(42B+j-328)	for j = 349,...,369

e(B,j) = di(645B+j-45)	for j = 370,...,689

where

q(0),q(1),...,q(11) = 0,0,0,0,0,0,0,0,0,0,0,0 identifies the coding scheme UBS-10.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,305+k) with e(B,326+k) for k=0, 3, 5, 8, 10, 13, 15 and 18.

Swap e(B,295+k) with e(B,327+k) for k=0 and 5

Swap e(B,298+k) with e(B,337+k) for k=0 and 5

Swap e(B,370+k) with e(B,346+k) for k=0, 3, 5, 8, 10, 13, 15, 18, 20 and 23.

Swap e(B,395+k) with e(B,362+k) for k=0 and 5.

Swap e(B,398+k) with e(B,352+k) for k=0 and 5.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,34) with e(B,170)

Swap e(B,71) with e(B,98)

Swap e(B,142) with e(B,110)

Swap e(B,176) with e(B,200)

Swap e(B,179) with e(B,80)

Swap e(B,284) with e(B,188)

Swap e(B,321) with e(B,158)

Swap e(B,357) with e(B,410)

Swap e(B,367) with e(B,688)

Swap e(B,437) with e(B,503)

Swap e(B,474) with e(B,480)

Swap e(B,582) with e(B,433)

Swap e(B,616) with e(B,530)

Swap e(B,687) with e(B,493)

For B = 1

Swap e(B,7) with e(B,158)

Swap e(B,44) with e(B,98)

Swap e(B,149) with e(B,200)

Swap e(B,152) with e(B,80)

Swap e(B,186) with e(B,170)

Swap e(B,257) with e(B,188)

Swap e(B,294) with e(B,110)

Swap e(B,376) with e(B,480)

Swap e(B,447) with e(B,503)

Swap e(B,481) with e(B,688)

Swap e(B,484) with e(B,433)

Swap e(B,589) with e(B,493)

Swap e(B,626) with e(B,530)

For B = 2

Swap e(B,17) with e(B,98)

Swap e(B,51) with e(B,188)

Swap e(B,159) with e(B,170)

Swap e(B,196) with e(B,110)

Swap e(B,267) with e(B,158)

Swap e(B,304) with e(B,80)

Swap e(B,359) with e(B,530)

Swap e(B,454) with e(B,688)

Swap e(B,457) with e(B,410)

Swap e(B,491) with e(B,503)

Swap e(B,562) with e(B,493)

Swap e(B,599) with e(B,480)

Swap e(B,636) with e(B,433)

For B = 3

Swap e(B,24) with e(B,103)

Swap e(B,27) with e(B,80)

Swap e(B,61) with e(B,188)

Swap e(B,132) with e(B,200)

Swap e(B,169) with e(B,170)

Swap e(B,206) with e(B,110)

Swap e(B,277) with e(B,158)

Swap e(B,311) with e(B,0)

Swap e(B,369) with e(B,480)

Swap e(B,427) with e(B,493)

Swap e(B,464) with e(B,530)

Swap e(B,501) with e(B,433)

Swap e(B,572) with e(B,503)

Swap e(B,609) with e(B,410)

### 5.1a.14	Packet data block type 25 (UBS-11)

#### 5.1a.14.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 2248 information bits {d(0),d(1),...,d(2247)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 2273 information bits {d(0),d(1),...,d(2272).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,63

i1(k-64) = d(k)		for k = 64,...,609

i2(k-610) = d(k)		for k = 610,...,1155

i3(k-1156) = d(k)		for k = 1156,...,1701

i4(k-1702) = d(k)		for k = 1702,...,2247

And if a PAN is included:

pn(k-2248) = d(k)	for k = 2248,...,2272

#### 5.1a.14.2	Header coding

The header {h(0),...,h(63)} is coded as defined in subclause 5.1a.1.1, with N=64, resulting in a block of 216 bits, {C(0),...,C(215)}.

The code is punctured in such a way that the following coded bits:

{ C(27*k+24) for k=0,…,7} are not transmitted

This results in a block of 208 bits, {hc(0),...,hc(207)}.

#### 5.1a.14.3	Data coding

Each data part, {i1(0),…,i1(545)}, {i2(0),…,i2(545)}, {i3(0),…,i3(545)} and {i4(0),…,i4(545)}, is coded as defined in subclause 5.1a.1.2, with N=546, resulting in four coded blocks of 1692 bits, {C1(0),...,C1(1691)}, {C2(0),...,C2(1691)}, {C3(0),...,C3(1691)} and {C4(0),...,C4(1691)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Always punctured | Punctured only if a PAN is included |
| --- | --- | --- |
| P1 | C(24*k+j) for k=0,…,69, j=2, 4, 5, 6, 8, 10, 11, 13, 14, 15, 16, 18, 19, 21 and 22; and C(24*70+j) for j=2, 4, 5, 6, 8, 10 and 11 | C(24*k) for k=0, 3, 7, 10, 14, 17, 21, 24, 28, 31, 35, 38, 42, 45, 49, 52, 56, 59, 63 and 66 |
| P2 | C(24*k+j) for k=0,…,69, j=0, 1, 3, 4, 6, 7, 9, 11, 12, 13, 15, 17, 18, 20 and 23; and C(24*70+j) for j=0, 1, 3, 4, 6, 7, 9 and 11, except C(24*35+3) that is not punctured | C(24*35+3); and C(24*k+22) for k=2, 5, 9, 12, 16, 19, 23, 27, 30, 34, 37, 41, 44, 48, 51, 55, 58, 62 and 65 |
| P3 | C(24*k+j) for k=0,…,69, j=1, 2, 3, 5, 7, 8, 10, 12, 14, 16, 17, 19, 20, 21 and 22; and C(24*70+j) for j=1, 2, 3, 5, 7, 8 and 10 | C(24*k+9) for k=1, 4, 8, 11, 15, 18, 22, 25, 29, 32, 36, 39, 43, 47, 50, 54, 57, 61, 64 and 68 |

If a PAN is not included, the result is four blocks of 635 bits, {c1(0),...,c1(634)}, {c2(0),...,c2(634)}, {c3(0),...,c3(634)} and {c4(0),...,c4(634)}.

If a PAN is included, the result is four blocks of 615 bits, {c1(0),...,c1(614)}, {c2(0),...,c2(614)}, {c3(0),...,c3(614)} and {c4(0),...,c4(614)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, C3 and c3 to i3, and C4 and c4 to i4.

#### 5.1a.14.4	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.14.5	Interleaving

a)	Header

The header, {hc(0),...,hc(207)}, is interleaved as defined in subclause 5.1a.2.1, with NC=208 and a=49, resulting in a block of 208 bits, {hi(0),...,hi(207)}.

b)	Data and PAN

If a PAN is not included, the following definitions apply:

dc1(k) = c1(k)			for k = 0,...,634

dc2(k) = c2(k)			for k = 0,...,634

dc3(k) = c3(k)			for k = 0,...,634

dc4(k) = c4(k)			for k = 0,...,634

If a PAN is included, the following definitions apply:

dc1(k) = ac(4k)			for k = 0,…,19

dc1(k) = c1(k-20)		for k = 20,…,634

dc2(k) = ac(4k+1)		for k = 0,…,19

dc2(k) = c2(k-20)		for k = 20,…,634

dc3(k) = ac(4k+2)		for k = 0,…,19

dc3(k) = c3(k-20)		for k = 20,…,634

dc4(k) = ac(4k+3)		for k = 0,…,19

dc4(k) = c4(k-20)		for k = 20,…,634

The four blocks {dc1(0),...,dc1(634)}, {dc2(0),...,dc2(634)}, {dc3(0),...,dc3(634)} and {dc4(0),...,dc4(634)} are separately interleaved as defined in subclause 5.1a.2.2, with NC=635 and a=177, resulting in the four blocks {di1(0),…,di1(634)}, {di2(0),…,di2(634)}, {di3(0),…,di3(634)} and {di4(0),…,di4(634)}, where di1 corresponds to dc1, di2 to dc2, di3 to dc3 and di4 to dc4.

The blocks are put together as one entity as described by the following rule:

di(k) = di1(k)				for k = 0,...,634

di(k) = di2(k-635)		for k = 635,...,1269

di(k) = di3(k-1270)		for k = 1270,...,1904

di(k) = di4(k-1905)		for k = 1905,...,2539

#### 5.1a.14.6	Mapping on a burst

Straightforward Mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(635B+j)		for j = 0,...,319

e(B,j) = hi(52B+j-320)	for j = 320,...,344

e(B,j) = q(3B+j-345)		for j = 345

e(B,j) = hi(52B+j-321)	for j = 346

e(B,j) = q(3B+j-346)		for j = 347,348

e(B,j) = hi(52B+j-323)	for j = 349,...,374

e(B,j) = di(635B+j-55)	for j = 375,...,689

where

q(0),q(1),...,q(11) = 1,1,1,1,1,1,1,1,1,1,1,1 identifies the coding scheme UBS-11 or UBS-12.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,295+k) with e(B,321+k) for k=0, 3, 5, 8, 10, 13, 15, 18, 20 and 23.

Swap e(B,280+k) with e(B,322+k) for k=0, 5 and 10.

Swap e(B,288+k) with e(B,337+k) for k=0 and 5.

Swap e(B,375+k) with e(B,346+k) for k=0, 3, 5, 8, 10, 13, 15, 18, 20, 23, 25 and 28.

Swap e(B,405+k) with e(B,362+k) for k=0, 5 and 10.

Swap e(B,408+k) with e(B,352+k) for k=0 and 5.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0, 1, 2 and 3

Swap e(B,11) with e(B,200)

Swap e(B,42) with e(B,110)

Swap e(B,146) with e(B,98)

Swap e(B,177) with e(B,80)

Swap e(B,219) with e(B,158)

Swap e(B,292) with e(B,170)

Swap e(B,349) with e(B,530)

Swap e(B,409) with e(B,420)

Swap e(B,451) with e(B,493)

Swap e(B,482) with e(B,480)

Swap e(B,524) with e(B,688)

Swap e(B,586) with e(B,433)

Swap e(B,659) with e(B,503)

### 5.1a.15	Packet data block type 26 (UBS-12)

#### 5.1a.15.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 2440 information bits {d(0),d(1),...,d(2439)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 2465 information bits {d(0),d(1),...,d(2464).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

h(k) = d(k)			for k = 0,...,63

i1(k-64) = d(k)		for k = 64,...,657

i2(k-658) = d(k)		for k = 658,...,1251

i3(k-1252) = d(k)		for k = 1252...,1845

i4(k-1846) = d(k)		for k = 1846,...,2439

And if a PAN is included:

pn(k-2440) = d(k)	for k = 2440,...,2464

#### 5.1a.15.2	Header coding

The header coding is the same as for UBS-11 as specified in subclause 5.1a.14.2.

#### 5.1a.15.3	Data coding

Each data part, {i1(0),…,i1(593)}, {i2(0),…,i2(593)}, {i3(0),…,i3(593)} and {i4(0),…,i4(593)}, is coded as defined in subclause 5.1a.1.2, with N=594, resulting in four coded blocks of 1836 bits, {C1(0),...,C1(1835)}, {C2(0),...,C2(1835)}, {C3(0),...,C3(1835)} and {C4(0),...,C4(1835)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied in such a way that the following coded bits are punctured:

|  | Transmitted bits when PAN not included | Transmitted bits when PAN is included |
| --- | --- | --- |
| P1 | C(399*k1+j1) for k1=0,…3, j1 = 2, 5, 8, 11, 14, 17, 20, 23, 26, 28, 31, 34, 37, 40, 43, 46, 49, 52, 54, 57, 60, 63, 66, 69, 72, 75, 78, 80, 83, 86, 89, 92, 95, 98, 101, 104, 106, 109, 112, 115, 118, 121, 124, 127, 130, 133, 135, 138, 141, 144, 147, 150, 153, 156, 159, 161, 164, 167, 170, 173, 176, 179, 182, 185, 187, 190, 193, 196, 199, 202, 205, 208, 211, 213, 216, 219, 222, 225, 228, 231, 234, 237, 239, 242, 245, 248, 251, 254, 257, 260, 263, 266, 268, 271, 274, 277, 280, 283, 286, 289, 292, 294, 297, 300, 303, 306, 309, 312, 315, 318, 320, 323, 326, 329, 332, 335, 338, 341, 344, 346, 349, 352, 355, 358, 361, 364, 367, 370, 372, 375, 378, 381, 384, 387, 390, 393, 396 ; and C(399*4+j2) for j2=0, 2, 5, 8, 11, 14, 17, 20, 23, 26, 28, 31, 34, 37, 40, 43, 46, 49, 52, 54, 57, 60, 63, 66, 69, 72, 75, 78, 80, 83, 86, 89, 92, 95, 98, 101, 104, 106, 109, 112, 115, 118, 121, 124, 127, 130, 133, 135, 138, 141, 144, 147, 150, 153, 156, 159, 161, 164, 167, 170, 173, 176, 179, 182, 185, 187, 190, 193, 196, 199, 202, 205, 208, 211, 213, 216, 219, 222, 225, 228, 231, 234, 237, 239; and C(399), C(798) and C(1197) | C(399*k2+j3) for k2=0,…3, j3=2, 5, 8, 11, 14, 17, 20, 23, 26, 28, 31, 34, 37, 40, 43, 46, 49, 52, 54, 57, 60, 63, 66, 69, 72, 75, 78, 83, 86, 89, 92, 95, 98, 101, 104, 106, 109, 112, 115, 118, 121, 124, 127, 130, 133, 135, 138, 141, 144, 147, 150, 153, 156, 159, 164, 167, 170, 173, 176, 179, 182, 185, 187, 190, 193, 196, 199, 202, 205, 208, 211, 213, 216, 219, 222, 225, 228, 231, 234, 237, 242, 245, 248, 251, 254, 257, 260, 263, 266, 268, 271, 274, 277, 280, 283, 286, 289, 292, 294, 297, 300, 303, 306, 309, 312, 315, 318, 323, 326, 329, 332, 335, 338, 341, 344, 346, 349, 352, 355, 358, 361, 364, 367, 370, 372, 375, 378, 381, 384, 387, 390, 393, 396 ; and C(399*4+j4) for j4=0, 2, 5, 8, 11, 14, 17, 20, 23, 26, 28, 31, 34, 37, 40, 43, 46, 49, 52, 54, 57, 60, 63, 66, 69, 72, 75, 78, 83, 86, 89, 92, 95, 98, 101, 104, 106, 109, 112, 115, 118, 121, 124, 127, 130, 133, 135, 138, 141, 144, 147, 150, 153, 156, 159, 164, 167, 170, 173, 176, 179, 182, 185, 187, 190, 193, 196, 199, 202, 205, 208, 211, 213, 216, 219, 222, 225, 228, 231, 234, 237; and C(798) and C(1197) |
| P2 | C(399*k1+(j1-1)), C(399*4+(j2-1)), C(398), C(797) and C(1196) where k1, j1 and j2 are as defined above for P1 | C(399*k2+(j3-1)) and C(399*4+(j4-1)) and C(797) and C(1196)where k2, j3 and j4 are as defined above for P1 |
| P3 | C(399*k1+(j1-2)), C(399*4+(j2-2)), C(397), C(796) and C(1195) where k1, j1 and j2 are as defined above for P1 | C(399*k2+(j3-2)) and C(399*4+(j4-2)) and C(796) and C(1195)where k2, j3 and j4 are as defined above for P1 |

If a PAN is not included, the result is four blocks of 635 bits, {c1(0),...,c1(634)}, {c2(0),...,c2(634)}, {c3(0),...,c3(634)} and {c4(0),...,c4(634)}.

If a PAN is included, the result is four blocks of 615 bits, {c1(0),...,c1(614)}, {c2(0),...,c2(614)}, {c3(0),...,c3(614)} and {c4(0),...,c4(614)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, C3 and c3 to i3, and C4 and c4 to i4.

#### 5.1a.15.4	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.15.5	Interleaving

The interleaving is the same as for UBS-11 as specified in subclause 5.1a.14.5.

#### 5.1a.15.6	Mapping on a burst

The mapping is the same as for UBS-11 as specified in subclause 5.1a.14.6.

### 5.1a.16	Packet data block type 27 (DAS-5)

#### 5.1a.16.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 478 information bits {d(0),d(1),...,d(477)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 481 information bits {d(0),d(1),...,d(480). If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 503 information bits {d(0),d(1),...,d(502). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 506 information bits {d(0),d(1),...,d(505).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

#### 5.1a.16.2	USF coding

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived as described for MCS-5 DL in subclause 5.1.9.1.2.

#### 5.1a.16.3	Header coding

A block of 100 coded bits {hc(0),hc(1),...,hc(99)} is derived from {d(3),d(4),…,d(27)} as described for MCS-5 DL in subclause 5.1.9.1.3.

In case an eTFI field is included and a PAN field is not included the three bits {d(478),d(479),d(480)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)} as specified in clause 5.1.9.1.3. In case an eTFI field and a PAN field are included the three bits {d(503),d(504),d(505)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)} as specified in clause 5.1.9.1.3.

#### 5.1a.16.4	Data coding

The data, defined as

i(k) = d(k+28)				for k = 0,...,449

is coded as defined in subclause 5.1a.1.3, with N=450, resulting in a coded block of 1398 bits, {C(0),...,C(1397)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0.05, N=466,Ndata=1248 and Ndata2=1172.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is a block of 1248 bits, {c(0),...,c(1247)}.

If a PAN is included, the result is a block of 1172 bits, {c(0),...,c(1171)}.

#### 5.1a.16.5	PAN coding

The PAN, if included, is defined as

pn(i) = d(478+i)    for i=0,…,24.

The PAN coding is the same as for MCS-5 DL as specified in subclause 5.1.9.1.4a.

#### 5.1a.16.6	Interleaving

a)	Header

The header interleaving is the same as for MCS-5 DL as specified in subclause 5.1.9.1.5, resulting in a block of 100 bits, {hi(0),...,hi(99)}.

b)	Data and PAN

If a PAN is not included, the following rule applies:

dc(k) = c(k)			for k = 0,...,1247

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,..., 75

dc(k) = c(k-76)	for k = 76,...,1247

The block {dc(0),...,dc(1247)} is interleaved as for MCS-5 DL as specified in subclause 5.1.9.1.5, resulting in a block of 1248 bits, {di(0),...,di(1247)}.

#### 5.1a.16.7	Mapping on a burst

The mapping is the same as for MCS-5 DL as specified in subclause 5.1.9.1.6.

NOTE:	In this case, the stealing flags q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identify the coding scheme DAS-5, DAS-6 or DAS-7.

### 5.1a.17	Packet data block type 28 (DAS-6)

#### 5.1a.17.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 574 information bits {d(0),d(1),...,d(573)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 577 information bits {d(0),d(1),...,d(576). If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 599 information bits {d(0),d(1),...,d(598). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 602 information bits {d(0),d(1),...,d(601).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

#### 5.1a.17.2	USF coding

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived as described for MCS-5 DL in subclause 5.1.9.1.2.

#### 5.1a.17.3	Header coding

A block of 100 coded bits {hc(0),hc(1),...,hc(99)} is derived from {d(3),d(4),…,d(27)} as described for MCS-5 DL in subclause 5.1.9.1.3.

In case an eTFI field is included and a PAN field is not included the three bits {d(574),d(575),d(576)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)} as specified in clause 5.1.9.1.3. In case an eTFI field and a PAN field are included the three bits {d(599),d(600),d(601)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)} as specified in clause 5.1.9.1.3.

#### 5.1a.17.4	Data coding

The data, defined as

i(k) = d(k+28)				for k = 0,...,545

is coded as defined in subclause 5.1a.1.3, with N=546, resulting in a coded block of 1686 bits, {C(0),...,C(1685)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0, N=562,Ndata=1248 and Ndata2=1172.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is a block of 1248 bits, {c(0),...,c(1247)}.

If a PAN is included, the result is a block of 1172 bits, {c(0),...,c(1171)}.

#### 5.1a.17.5	PAN coding

The PAN, if included, is defined as

pn(i) = d(574+i)    for i=0,…,24.

The PAN coding is the same as for MCS-5 DL as specified in subclause 5.1.9.1.4a.

#### 5.1a.17.6	Interleaving

The interleaving is the same as for DAS-5 as specified in subclause 5.1a.16.6.

#### 5.1a.17.7	Mapping on a burst

The mapping is the same as for MCS-5 DL as specified in subclause 5.1.9.1.6.

NOTE:	In this case, the stealing flags q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identify the coding scheme DAS-5, DAS-6 or DAS-7.

### 5.1a.18	Packet data block type 29 (DAS-7)

#### 5.1a.18.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 686 information bits {d(0),d(1),...,d(685)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 689 information bits {d(0),d(1),...,d(688).  If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 711 information bits {d(0),d(1),...,d(710). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 714 information bits {d(0),d(1),...,d(713).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

#### 5.1a.18.2	USF coding

A block of 36 bits {u’(0),u’(1),...,u’(35)} is derived as described for MCS-5 DL in subclause 5.1.9.1.2.

#### 5.1a.18.3	Header coding

A block of 100 coded bits {hc(0),hc(1),...,hc(99)} is derived from {d(3),d(4),…,d(27)} as described for MCS-5 DL in subclause 5.1.9.1.3.

In case an eTFI field is included and a PAN field is not included the three bits {d(686),d(687),d(688)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)} as specified in clause 5.1.9.1.3. In case an eTFI field and a PAN field are included the three bits {d(711),d(712),d(713)} are added bit-wise modulo 2 to the last three parity bits {p(5), p(6),p(7)} as specified in clause 5.1.9.1.3.

#### 5.1a.18.4	Data coding

The data, defined as

i(k) = d(k+28)				for k = 0,...,657

is coded as defined in subclause 5.1a.1.3, with N=658, resulting in a coded block of 2022 bits, {C(0),...,C(2021)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0, N=674,Ndata=1248 and Ndata2=1172.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is a block of 1248 bits, {c(0),...,c(1247)}.

If a PAN is included, the result is a block of 1172 bits, {c(0),...,c(1171)}.

#### 5.1a.18.5	PAN coding

The PAN, if included, is defined as

pn(i) = d(686+i)    for i=0,…,24.

The PAN coding is the same as for MCS-5 DL as specified in subclause  5.1.9.1.4a.

#### 5.1a.18.6	Interleaving

The interleaving is the same as for DAS-5 as specified in subclause 5.1a.16.6.

#### 5.1a.18.7	Mapping on a burst

The mapping is the same as for MCS-5 DL as specified in subclause 5.1.9.1.6.

NOTE:	In this case, the stealing flags q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identify the coding scheme DAS-5, DAS-6 or DAS-7.

### 5.1a.19	Packet data block type 30 (DAS-8)

#### 5.1a.19.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 941 information bits {d(0),d(1),...,d(940)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 944 information bits {d(0),d(1),...,d(943).  If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 966 information bits {d(0),d(1),...,d(965). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 969 information bits {d(0),d(1),...,d(968).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,40

i1(k-41) = d(k)		for k = 41,...,490

i2(k-491) = d(k)		for k = 491,...,940

And if an eTFI is included:

et(k-941) = d(k)		for k = 941,...,943

And if a PAN is included:

pn(k-941) = d(k)		for k = 941,...,965

And if a PAN and an eTFI are included:

et(k-966) = d(k)		for k = 966,...,968

#### 5.1a.19.2	USF coding

##### 5.1a.19.2.1	BTTI configuration

The USF bits {u(0),u(1),u(2)} are block coded into 48 bits u’(0),u’(1),...,u’(47) according to the following table:

| u(0),u(1),u(2) | u’(0),u’(1),...,u’(47) |  |  |  |
| --- | --- | --- | --- | --- |
|  | burst 0 | burst 1 | burst 2 | burst 3 |
| 000 | 0 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 1 0 1 1 | 0 0 1 1 0 1 1 1 0 1 1 1 | 1 1 1 1 1 0 1 1 1 0 1 1 |
| 001 | 0 0 1 1 0 0 1 1 0 1 1 1 | 0 0 1 1 0 1 1 1 0 1 1 1 | 0 0 1 1 0 0 1 1 0 1 1 1 | 0 0 1 1 0 0 1 1 0 1 1 1 |
| 010 | 0 1 1 1 0 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 0 1 1 1 | 1 1 1 1 1 1 1 1 1 0 1 1 | 0 0 1 1 0 1 1 1 1 1 1 1 |
| 011 | 1 1 1 1 1 0 1 1 0 0 1 1 | 1 0 1 1 0 0 1 1 0 0 1 1 | 0 1 1 1 0 1 1 1 1 1 1 1 | 1 0 1 1 1 0 1 1 0 0 1 1 |
| 100 | 0 0 1 1 0 1 1 1 0 1 1 1 | 0 1 1 1 1 1 1 1 1 1 1 1 | 1 0 1 1 1 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 1 0 1 1 |
| 101 | 1 0 1 1 0 0 1 1 0 0 1 1 | 1 0 1 1 1 0 1 1 1 0 1 1 | 0 1 1 1 1 1 1 1 1 1 1 1 | 0 1 1 1 1 1 1 1 1 1 1 1 |
| 110 | 1 1 1 1 1 1 1 1 1 0 1 1 | 1 1 1 1 1 0 1 1 0 0 1 1 | 1 0 1 1 0 0 1 1 0 0 1 1 | 0 1 1 1 0 1 1 1 0 1 1 1 |
| 111 | 1 0 1 1 1 0 1 1 1 0 1 1 | 0 1 1 1 0 1 1 1 1 1 1 1 | 1 1 1 1 1 0 1 1 1 0 1 1 | 1 0 1 1 0 0 1 1 0 0 1 1 |

##### 5.1a.19.2.2	RTTI configurations

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the USF bits {u(0),u(1),u(2)} are block coded into 48 bits u’(0),u’(1),...,u’(47) as described in subclause 5.1a.19.2.1.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into 48 bits uL(0),uL(1),...,uL(47) as described in subclause 5.1a.19.2.1; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into 48 bits uH(0),uH(1),...,uH(47) as described in subclause 5.1a.19.2.1.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then u(0),u(1),u(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then:

u’(j)=uL(j),		j=0…11

u’(j)=uH(j-12),	j=12…23

u’(j)=uL(j-12),	j=24…35

u’(j)=uH(j-24)		j=36…47

If the data block is sent in the second 10ms of a 20ms block period, then:

u’(j)=uL(j+24),	j=0…11

u’(j)=uH(j+12),	j=12…23

u’(j)=uL(j+12),	j=24…35

u’(j)=uH(j)			j=36…47

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20 ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

#### 5.1a.19.3	Header coding

The header {h(0),...,h(37)} is coded as defined in subclause 5.1a.1.1, with N=38, resulting in a block of 138 bits, {C(0),...,C(137)}.

The code is punctured in such a way that the following coded bits:

{C(k) for k = 8 and 77} are not transmitted

This results in a block of 136 bits, {hc(0),...,hc(135)}.

#### 5.1a.19.4	Data coding

Each data part, {i1(0),…,i1(449)} and {i2(0),…,i2(449)}, is coded as defined in subclause 5.1a.1.3, with N=450, resulting in two coded blocks of 1398 bits, {C1(0),...,C1(1397)} and {C2(0),...,C2(1397)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0, N=466,Ndata=832 and Ndata2=793.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is two blocks of 832 bits, {c1(0),...,c1(831)} and {c2(0),...,c2(831)}.

If a PAN is included, the result is two blocks of 793 bits, {c1(0),...,c1(792)} and {c2(0),...,c2(792)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.19.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.19.6	Interleaving

a)	Header

The header, {hc(0),...,hc(135)}, is interleaved as defined in subclause 5.1a.2.1, with NC=136 and a=23, resulting in a block of 136 bits, {hi(0),...,hi(135)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,831

dc(k) = c2(k-832)	for k = 832,...,1663

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,77

dc(k) = c1(k-78)		for k = 78,…,870

dc(k) = c2(k-871)	for k = 871,..,.1663

The block {dc(0),...,dc(1663)} is interleaved as defined in subclause 5.1a.2.1, with NC=1664 and a=199, resulting in a block of 1664 bits, {di(0),...,di(1663)}.

#### 5.1a.19.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(416B+j)		for j = 0,...,207

e(B,j) = hi(34B+j-208)	for j = 208,...,227

e(B,j) = q(2B+j-228)		for j = 228,229

e(B,j) = hi(34B+j-210)	for j = 230,...,231

e(B,j) = u’(12B+j-232)	for j = 232,...,243

e(B,j) = hi(34B+j-222)	for j = 244,...,255

e(B,j) = di(416B+j-48)	for j = 256,...,463

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme DAS-8 or DAS-9.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,184+k) with e(B,210+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 16, 17, 20, 21.

Swap e(B,256+k) with e(B,246+k) for k=0, 1, 4, 5, 8, 9.

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,55) with e(B,40)

Swap e(B,91) with e(B,81)

Swap e(B,127) with e(B,120)

Swap e(B,163) with e(B,160)

Swap e(B,199) with e(B,177)

Swap e(B,302) with e(B,268)

Swap e(B,338) with e(B,305)

Swap e(B,374) with e(B,368)

Swap e(B,410) with e(B,405)

Swap e(B,446) with e(B,444)

For B = 1

Swap e(B,23) with e(B,120)

Swap e(B,59) with e(B,160)

Swap e(B,95) with e(B,177)

Swap e(B,150) with e(B,12)

Swap e(B,186) with e(B,81)

Swap e(B,230) with e(B,181)

Swap e(B,415) with e(B,268)

Swap e(B,451) with e(B,305)

Swap e(B,270) with e(B,368)

Swap e(B,306) with e(B,405)

Swap e(B,342) with e(B,444)

For B = 2

Swap e(B,46) with e(B,40)

Swap e(B,82) with e(B,81)

Swap e(B,118) with e(B,120)

Swap e(B,154) with e(B,160)

Swap e(B,190) with e(B,177)

Swap e(B,311) with e(B,268)

Swap e(B,347) with e(B,305)

Swap e(B,383) with e(B,368)

Swap e(B,419) with e(B,405)

Swap e(B,455) with e(B,444)

For B = 3

Swap e(B,14) with e(B,120)

Swap e(B,50) with e(B,160)

Swap e(B,86) with e(B,180)

Swap e(B,159) with e(B,40)

Swap e(B,195) with e(B,81)

Swap e(B,406) with e(B,268)

Swap e(B,442) with e(B,305)

Swap e(B,279) with e(B,368)

Swap e(B,315) with e(B,405)

Swap e(B,351) with e(B,444)

### 5.1a.20	Packet data block type 31 (DAS-9)

#### 5.1a.20.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1133 information bits {d(0),d(1),...,d(1132)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 1136 information bits {d(0),d(1),...,d(1135).  If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 1158 information bits {d(0),d(1),...,d(1157). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1161 information bits {d(0),d(1),...,d(1160).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,40

i1(k-41) = d(k)		for k = 41,...,586

i2(k-587) = d(k)		for k = 587,...,1132

And if an eTFI is included:

et(k-1133) = d(k)		for k = 1133,...,1135

And if a PAN is included:

pn(k-1133) = d(k)		for k = 1133,...,1157

And if a PAN and an eTFI are included:

et(k-1158) = d(k)			for k = 1158,...,1160

#### 5.1a.20.2	USF coding

The USF coding is the same as for DAS-8 as specified in subclause 5.1a.19.2.

#### 5.1a.20.3	Header coding

The header coding is the same as for DAS-8 as specified in subclause 5.1a.19.3.

#### 5.1a.20.4	Data coding

Each data part, {i1(0),…,i1(545)} and {i2(0),…,i2(545)}, is coded as defined in subclause 5.1a.1.3, with N=546, resulting in two coded blocks of 1686 bits, {C1(0),...,C1(1685)} and {C2(0),...,C2(1685)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=562,Ndata=832 and Ndata2=793.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is two blocks of 832 bits, {c1(0),...,c1(831)} and {c2(0),...,c2(831)}.

If a PAN is included, the result is two blocks of 793 bits, {c1(0),...,c1(792)} and {c2(0),...,c2(792)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.20.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.20.6	Interleaving

The interleaving is the same as for DAS-8 as specified in subclause 5.1a.19.6.

#### 5.1a.20.7	Mapping on a burst

The mapping is the same as for DAS-8 as specified in subclause 5.1a.19.7.

### 5.1a.21	Packet data block type 32 (DAS-10)

#### 5.1a.21.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1355 information bits {d(0),d(1),...,d(1354)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 1358 information bits {d(0),d(1),...,d(1357).  If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 1380 information bits {d(0),d(1),...,d(1379). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1383 information bits {d(0),d(1),...,d(1382).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,38

i1(k-39) = d(k)		for k = 39,...,696

i2(k-697) = d(k)		for k = 697,...,1354

And if an eTFI is included:

et(k-1355) = d(k)		for k = 1355,...,1357

And if a PAN is included:

pn(k-1355) = d(k)		for k = 1355,...,1379

And if a PAN and an eTFI are included:

et(k-1380) = d(k)			for k = 1380,...,1382

#### 5.1a.21.2	USF coding

##### 5.1a.21.2.1	BTTI configuration

The USF bits {u(0),u(1),u(2)} are block coded into 60 bits u’(0),u’(1),...,u’(59) according to the following table:

| u(0),u(1),u(2) | u’(0),u’(1),...,u’(59) |  |  |  |
| --- | --- | --- | --- | --- |
|  | burst 0 | burst 1 | burst 2 | burst 3 |
| 000 | 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 1 0 1 0 0 | 1 0 0 1 0 0 0 1 1 0 0 0 1 1 0 | 0 0 0 0 0 1 0 1 0 0 1 0 1 0 0 |
| 001 | 1 0 0 1 0 1 0 0 1 0 0 0 1 1 0 | 1 0 0 1 0 0 0 1 1 0 0 0 1 1 0 | 1 0 0 1 0 1 0 0 1 0 0 0 1 1 0 | 1 0 0 1 0 1 0 0 1 0 0 0 1 1 0 |
| 010 | 0 0 1 1 0 0 0 1 1 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 0 0 1 1 0 | 0 0 0 0 0 0 0 0 0 0 1 0 1 0 0 | 1 0 0 1 0 0 0 1 1 0 0 0 0 0 0 |
| 011 | 0 0 0 0 0 1 0 1 0 0 1 0 0 1 0 | 1 0 1 0 0 1 0 0 1 0 1 0 0 1 0 | 0 0 1 1 0 0 0 1 1 0 0 0 0 0 0 | 1 0 1 0 0 1 0 1 0 0 1 0 0 1 0 |
| 100 | 1 0 0 1 0 0 0 1 1 0 0 0 1 1 0 | 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 | 1 0 1 0 0 1 0 1 0 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 1 0 1 0 0 |
| 101 | 0 0 0 0 0 0 0 0 0 0 1 0 1 0 0 | 0 0 0 0 0 1 0 1 0 0 1 0 0 1 0 | 1 0 1 0 0 1 0 0 1 0 1 0 0 1 0 | 0 0 1 1 0 0 0 1 1 0 0 0 1 1 0 |
| 110 | 1 0 1 0 0 1 0 0 1 0 1 0 0 1 0 | 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 | 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 | 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 |
| 111 | 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 | 0 0 1 1 0 0 0 1 1 0 0 0 0 0 0 | 0 0 0 0 0 1 0 1 0 0 1 0 1 0 0 | 1 0 1 0 0 1 0 0 1 0 1 0 0 1 0 |

##### 5.1a.21.2.2	RTTI configurations

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the USF bits {u(0),u(1),u(2)} are block coded into 60 bits u’(0),u’(1),...,u’(59) as described in subclause 5.1a.21.2.1.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into 60 bits uL(0),uL(1),...,uL(59) as described in subclause 5.1a.21.2.1; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into 60 bits uH(0),uH(1),...,uH(59) as described in subclause 5.1a.21.2.1.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then u(0),u(1),u(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then:

u’(j)=uL(j),		j=0…14

u’(j)=uH(j-15),	j=15…29

u’(j)=uL(j-15),	j=30…44

u’(j)=uH(j-30)		j=45…59

If the data block is sent in the second 10ms of a 20ms block period, then:

u’(j)=uL(j+30),	j=0…14

u’(j)=uH(j+15),	j=15…29

u’(j)=uL(j+15),	j=30…44

u’(j)=uH(j)			j=45…59

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20 ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

#### 5.1a.21.3	Header coding

The header {h(0),...,h(35)} is coded as defined in subclause 5.1a.1.1, with N=36, resulting in a block of 132 bits, {C(0),...,C(131)}.

The coded header is defined as:

hc(k) = C(k)			for k = 0,...,131

#### 5.1a.21.4	Data coding

Each data part, {i1(0),…,i1(657)} and {i2(0),…,i2(657)}, is coded as defined in subclause 5.1a.1.3, with N=658, resulting in two coded blocks of 2022 bits, {C1(0),...,C1(2021)} and {C2(0),...,C2(2021)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0.15, N=674,Ndata=1060 and Ndata2=1021.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5

If a PAN is not included, the result is two blocks of 1060 bits, {c1(0),...,c1(1059)} and {c2(0),...,c2(1059)}.

If a PAN is included, the result is two blocks of 1021 bits, {c1(0),...,c1(1020)} and {c2(0),...,c2(1020)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.21.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.21.6	Interleaving

a)	Header

The header, {hc(0),...,hc(131)}, is interleaved as defined in subclause 5.1a.2.1, with NC=132 and a=7, resulting in a block of 132 bits, {hi(0),...,hi(131)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,1059

dc(k) = c2(k-1060)	for k = 1060,...,2119

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,77

dc(k) = c1(k-78)		for k = 78,…,1098

dc(k) = c2(k-1099)	for k = 1099,..,2119

The block {dc(0),...,dc(2119)} is interleaved as defined in subclause 5.1a.2.1, with NC=2120 and a=301, resulting in a block of 2120 bits, {di(0),...,di(2119)}.

#### 5.1a.21.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(530B+j)		for j = 0,...,264

e(B,j) = hi(33B+j-265)	for j = 265,...,284

e(B,j) = q(2B+j-285)		for j = 285

e(B,j) = hi(33B+j-266)	for j = 286,...,287

e(B,j) = q(2B+j-287)		for j = 288

e(B,j) = hi(33B+j-267)	for j = 289

e(B,j) = u’(15B+j-290)	for j = 290,...,304

e(B,j) = hi(33B+j-282)	for j = 305,...,314

e(B,j) = di(530B+j-50)	for j = 315,...,579

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme DAS-10.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,240+k) with e(B,266+k) for k=0, 3, 5, 8, 10, 13, 15, 18, 20, 23.

Swap e(B,225+k) with e(B,267+k) for k=0, 5, 10.

Swap e(B,233+k) with e(B,282+k) for k=0, 5.

Swap e(B,315+k) with e(B,306+k) for k=0, 3, 5, 8.

Swap e(B,328) with e(B,312), Swap e(B,325) with e(B,307).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,46) with e(B,188)

Swap e(B,59) with e(B,158)

Swap e(B,72) with e(B,80)

Swap e(B,131) with e(B,170)

Swap e(B,144) with e(B,98)

Swap e(B,216) with e(B,110)

Swap e(B,307) with e(B,385)

Swap e(B,351) with e(B,330)

Swap e(B,397) with e(B,480)

Swap e(B,469) with e(B,503)

Swap e(B,482) with e(B,400)

Swap e(B,554) with e(B,433)

Swap e(B,567) with e(B,363)

For B = 1

Swap e(B,41) with e(B,110)

Swap e(B,54) with e(B,80)

Swap e(B,126) with e(B,98)

Swap e(B,257) with e(B,158)

Swap e(B,311) with e(B,450)

Swap e(B,379) with e(B,530)

Swap e(B,392) with e(B,433)

Swap e(B,464) with e(B,480)

Swap e(B,477) with e(B,363)

Swap e(B,536) with e(B,503)

Swap e(B,549) with e(B,400)

Swap e(B,562) with e(B,330)

For B = 2

Swap e(B,36) with e(B,80)

Swap e(B,82) with e(B,170)

Swap e(B,167) with e(B,110)

Swap e(B,239) with e(B,158)

Swap e(B,252) with e(B,98)

Swap e(B,306) with e(B,450)

Swap e(B,361) with e(B,530)

Swap e(B,374) with e(B,480)

Swap e(B,387) with e(B,363)

Swap e(B,446) with e(B,503)

Swap e(B,459) with e(B,400)

Swap e(B,531) with e(B,433)

Swap e(B,544) with e(B,330)

For B = 3

Swap e(B,64) with e(B,200)

Swap e(B,77) with e(B,158)

Swap e(B,149) with e(B,170)

Swap e(B,162) with e(B,98)

Swap e(B,221) with e(B,188)

Swap e(B,234) with e(B,110)

Swap e(B,247) with e(B,80)

Swap e(B,356) with e(B,433)

Swap e(B,369) with e(B,363)

Swap e(B,441) with e(B,400)

Swap e(B,526) with e(B,330)

Swap e(B,572) with e(B,480)

### 5.1a.22	Packet data block type 33 (DAS-11)

#### 5.1a.22.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1691 information bits {d(0),d(1),...,d(1690)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 1694 information bits {d(0),d(1),...,d(1693).  If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 1716 information bits {d(0),d(1),...,d(1715). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 1719 information bits {d(0),d(1),...,d(1718).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,52

i1(k-53) = d(k)		for k = 53,...,598

i2(k-599) = d(k)		for k = 599,...,1144

i3(k-1145) = d(k)		for k = 1145,...,1690

And if an eTFI is included:

et(k-1691) = d(k)		for k = 1691,...,1693

And if a PAN is included:

pn(k-1691) = d(k)		for k = 1691,...,1715

And if a PAN and an eTFI are included:

et(k-1716) = d(k)			for k = 1716,...,1718

#### 5.1a.22.2	USF coding

The USF coding is the same as for DAS-10 as specified in subclause 5.1a.21.2.

#### 5.1a.22.3	Header coding

The header {h(0),...,h(49)} is coded as defined in subclause 5.1a.1.1, with N=50, resulting in a block of 174 bits, {C(0),...,C(173)}.

The code is punctured in such a way that the following coded bits:

{C(15*k+j) for k = 0,…,10, j = 11 and 14} are not transmitted

This results in a block of 152 bits, {hc(0),...,hc(151)}.

#### 5.1a.22.4	Data coding

Each data part, {i1(0),…,i1(545)}, {i2(0),…,i2(545)} and {i3(0),…,i3(545)}, is coded as defined in subclause 5.1a.1.3, with N=546, resulting in three coded blocks of 1686 bits, {C1(0),...,C1(1685)}, {C2(0),...,C2(1685)} and {C3(0),...,C3(1685)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=562,Ndata=700 and Ndata2=674.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is three blocks of 700 bits, {c1(0),...,c1(699)}, {c2(0),...,c2(699)} and {c3(0),...,c3(699)}.

If a PAN is included, the result is three blocks of 674 bits, {c1(0),...,c1(673)}, {c2(0),...,c2(673)} and {c3(0),...,c3(673)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2 and C3 and c3 to i3.

#### 5.1a.22.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.22.6	Interleaving

a)	Header

The header, {hc(0),...,hc(151)}, is interleaved as defined in subclause 5.1a.2.1, with NC=152 and a=3, resulting in a block of 152 bits, {hi(0),...,hi(151)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,699

dc(k) = c2(k-700)	for k = 700,...,1399

dc(k) = c3(k-1400)	for k = 1400,...,2099

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,77

dc(k) = c1(k-78)		for k = 78,…,751

dc(k) = c2(k-752)	for k = 752,…,1425

dc(k) = c3(k-1426)	for k = 1426,…,2099

The block {dc(0),...,dc(2099)} is interleaved as defined in subclause 5.1a.2.1, with NC=2100 and a=47, resulting in a block of 2100 bits, {di(0),...,di(2099)}.

#### 5.1a.22.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(525B+j)		for j = 0,...,259

e(B,j) = hi(38B+j-260)	for j = 260,...,284

e(B,j) = q(2B+j-285)		for j = 285

e(B,j) = hi(38B+j-261)	for j = 286,...,287

e(B,j) = q(2B+j-287)		for j = 288

e(B,j) = hi(38B+j-262)	for j = 289

e(B,j) = u’(15B+j-290)	for j = 290,...,304

e(B,j) = hi(38B+j-277)	for j = 305,...,314

e(B,j) = di(525B+j-55)	for j = 315,...,579

where

q(0),q(1),...,q(7) = 1,1,1,1,1,1,1,1 identifies the coding scheme DAS-11 or DAS-12.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,230+k) with e(B,261+k) for k=0, 3, 5, 8, 10, 13, 15, 18, 20, 23, 25, 28.

Swap e(B,215+k) with e(B,262+k) for k=0, 5, 10.

Swap e(B,218+k) with e(B,277+k) for k=0, 5, 10.

Swap e(B,315+k) with e(B,306+k) for k=0, 3, 5, 8.

Swap e(B,325) with e(B,312), Swap e(B,328) with e(B,307).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)  PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,39) with e(B,158)

Swap e(B,86) with e(B,170)

Swap e(B,94) with e(B,98)

Swap e(B,141) with e(B,110)

Swap e(B,266) with e(B,200)

Swap e(B,329) with e(B,480)

Swap e(B,376) with e(B,503)

Swap e(B,384) with e(B,363)

Swap e(B,431) with e(B,400)

For B = 1

Swap e(B,84) with e(B,170)

Swap e(B,131) with e(B,188)

Swap e(B,139) with e(B,98)

Swap e(B,186) with e(B,110)

Swap e(B,264) with e(B,200)

Swap e(B,429) with e(B,480)

Swap e(B,476) with e(B,503)

Swap e(B,484) with e(B,363)

Swap e(B,531) with e(B,400)

For B = 2

Swap e(B,184) with e(B,110)

Swap e(B,231) with e(B,158)

Swap e(B,239) with e(B,80)

Swap e(B,341) with e(B,330)

Swap e(B,474) with e(B,503)

Swap e(B,482) with e(B,363)

Swap e(B,521) with e(B,530)

Swap e(B,529) with e(B,400)

Swap e(B,576) with e(B,433)

For B = 3

Swap e(B,41) with e(B,110)

Swap e(B,96) with e(B,80)

Swap e(B,229) with e(B,170)

Swap e(B,331) with e(B,573)

Swap e(B,339) with e(B,330)

Swap e(B,386) with e(B,363)

Swap e(B,574) with e(B,530)

### 5.1a.23	Packet data block type 34 (DAS-12)

#### 5.1a.23.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 2027 information bits {d(0),d(1),...,d(2026)}. If the message delivered to the encoder does not include a PAN but includes an eTFI, it has a fixed size of 2030 information bits {d(0),d(1),...,d(2029).  If the message delivered to the encoder includes a PAN but does not include an eTFI, it has a fixed size of 2052 information bits {d(0),d(1),...,d(2051). If the message delivered to the encoder includes a PAN and an eTFI, it has a fixed size of 2055 information bits {d(0),d(1),...,d(2054).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,52

i1(k-53) = d(k)		for k = 53,...,710

i2(k-711) = d(k)		for k = 711,...,1368

i3(k-1369) = d(k)		for k = 1369,...,2026

And if an eTFI is included:

et(k-2027) = d(k)		for k = 2027,...,2029

And if a PAN is included:

pn(k-2027) = d(k)		for k = 2027,...,2051

And if a PAN and an eTFI are included:

et(k-2052) = d(k)			for k = 2052,...,2054

#### 5.1a.23.2	USF coding

The USF coding is the same as for DAS-10 as specified in subclause 5.1a.21.2.

#### 5.1a.23.3	Header coding

The header coding is the same as for DAS-11 as specified in subclause 5.1a.22.3.

#### 5.1a.23.4	Data coding

Each data part, {i1(0),…,i1(657)}, {i2(0),…,i2(657)} and {i3(0),…,i3(657)}, is coded as defined in subclause 5.1a.1.3, with N=658, resulting in three coded blocks of 2022 bits, {C1(0),...,C1(2021)}, {C2(0),...,C2(2021)} and {C3(0),...,C3(2021)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=674,Ndata=700 and Ndata2=674.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is three blocks of 700 bits, {c1(0),...,c1(699)}, {c2(0),...,c2(699)} and {c3(0),...,c3(699)}.

If a PAN is included, the result is three blocks of 674 bits, {c1(0),...,c1(673)}, {c2(0),...,c2(673)} and {c3(0),...,c3(673)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2 and C3 and c3 to i3..

#### 5.1a.23.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.23.6	Interleaving

a)	Header

The header, {hc(0),...,hc(151)}, is interleaved as defined in subclause 5.1a.2.1, with NC=152 and a=3, resulting in a block of 152 bits, {hi(0),...,hi(151)}.

b)	Data and PAN

If a PAN is not included, the following definitions apply:

dc1(k) = c1(k)			for k = 0,...,699

dc2(k) = c2(k)			for k = 0,...,699

dc3(k) = c3(k)			for k = 0,...,699

If a PAN is included, the following definitions apply:

dc1(k) = ac(3k)			for k = 0,…,25

dc1(k) = c1(k-26)		for k = 26,…,699

dc2(k) = ac(3k+1)		for k = 0,…,25

dc2(k) = c2(k-26)		for k = 26,…,699

dc3(k) = ac(3k+2)		for k = 0,…,25

dc3(k) = c3(k-26)		for k = 26,…,699

The three blocks {dc1(0),...,dc1(699)}, {dc2(0),...,dc2(699)} and {dc3(0),...,dc3(699)} are separately interleaved as defined in subclause 5.1a.2.2, with NC=700 and a=129, resulting in the three blocks {di1(0),…,di1(699)}, {di2(0),…,di2(699)} and {di3(0),…,di3(699)}, where di1 corresponds to dc1, di2 to dc2 and di3 to dc3.

The blocks are put together as one entity as described by the following rule:

di(k) = di1(k)				for k = 0,...,699

di(k) = di2(k-700)		for k = 700,...,1399

di(k) = di3(k-1400)		for k = 1400,...,2099

#### 5.1a.23.7	Mapping on a burst

a)	Straightforward mapping

The mapping is the same as for DAS-11 as specified in subclause 5.1a.22.7 a).

b)	Bit swapping

The bit swapping is the same as for DAS-11 as specified in subclause 5.1a.22.7 b)

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c) PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,19) with e(B,158)

Swap e(B,74) with e(B,110)

Swap e(B,129) with e(B,80)

Swap e(B,167) with e(B,188)

Swap e(B,289) with e(B,98)

Swap e(B,332) with e(B,503)

Swap e(B,351) with e(B,350)

Swap e(B,406) with e(B,493)

Swap e(B,461) with e(B,530)

Swap e(B,516) with e(B,433)

Swap e(B,571) with e(B,363)

For B = 1

Swap e(B,84) with e(B,98)

Swap e(B,139) with e(B,80)

Swap e(B,194) with e(B,158)

Swap e(B,249) with e(B,110)

Swap e(B,314) with e(B,480)

Swap e(B,359) with e(B,330)

Swap e(B,452) with e(B,530)

Swap e(B,526) with e(B,493)

For B = 2

Swap e(B,1) with e(B,188)

Swap e(B,56) with e(B,170)

Swap e(B,111) with e(B,110)

Swap e(B,166) with e(B,98)

Swap e(B,259) with e(B,200)

Swap e(B,271) with e(B,158)

Swap e(B,369) with e(B,330)

Swap e(B,424) with e(B,433)

Swap e(B,479) with e(B,400)

Swap e(B,534) with e(B,363)

Swap e(B,572) with e(B,480)

For B = 3

Swap e(B,121) with e(B,200)

Swap e(B,157) with e(B,98)

Swap e(B,176) with e(B,188)

Swap e(B,231) with e(B,158)

Swap e(B,281) with e(B,0)

Swap e(B,341) with e(B,363)

Swap e(B,396) with e(B,330)

Swap e(B,489) with e(B,433)

Swap e(B,544) with e(B,400)

### 5.1a.24	Packet data block type 35 (DBS-5)

#### 5.1a.24.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 480 information bits {d(0),d(1),...,d(479)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 505 information bits {d(0),d(1),...,d(504).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,29

i(k-30) = d(k)		for k = 30,...,479

And if a PAN is included:

pn(k-480) = d(k)		for k = 480,...,504

#### 5.1a.24.2	USF coding

##### 5.1a.24.2.1	BTTI configuration

The USF bits {u(0),u(1),u(2)} are block coded into 32 bits u’(0),u’(1),...,u’(31) according to the following table:

| u(0),u(1),u(2) | u’(0),u’(1),...,u’(31) |  |  |  |
| --- | --- | --- | --- | --- |
|  | burst 0 | burst 1 | burst 2 | burst 3 |
| 000 | 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 |
| 001 | 1 1 0 0 1 1 0 0 | 0 0 1 1 0 0 1 1 | 0 0 1 1 1 1 0 0 | 1 1 0 0 0 0 1 1 |
| 010 | 1 1 0 0 0 0 1 1 | 1 1 0 0 0 0 1 1 | 1 1 0 0 0 0 1 1 | 1 1 0 0 0 0 1 1 |
| 011 | 0 0 1 1 1 1 0 0 | 1 1 0 0 0 0 1 1 | 1 1 0 0 1 1 0 0 | 0 0 1 1 0 0 1 1 |
| 100 | 1 1 0 0 0 0 1 1 | 0 0 1 1 1 1 0 0 | 0 0 1 1 0 0 1 1 | 1 1 0 0 1 1 0 0 |
| 101 | 0 0 1 1 1 1 0 0 | 0 0 1 1 1 1 0 0 | 0 0 1 1 1 1 0 0 | 0 0 1 1 1 1 0 0 |
| 110 | 0 0 1 1 0 0 1 1 | 1 1 0 0 1 1 0 0 | 1 1 0 0 0 0 1 1 | 0 0 1 1 1 1 0 0 |
| 111 | 1 1 0 0 1 1 0 0 | 1 1 0 0 1 1 0 0 | 1 1 0 0 1 1 0 0 | 1 1 0 0 1 1 0 0 |

##### 5.1a.24.2.2	RTTI configurations

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the USF bits {u(0),u(1),u(2)} are block coded into 32 bits u’(0),u’(1),...,u’(31) as described in subclause 5.1a.24.2.1.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into 32 bits uL(0),uL(1),...,uL(31) as described in subclause 5.1a.24.2.1; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into 32 bits uH(0),uH(1),...,uH(31) as described in subclause 5.1a.24.2.1.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then u(0),u(1),u(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then:

u’(j)=uL(j),		j=0…7

u’(j)=uH(j-8),		j=8…15

u’(j)=uL(j-8),		j=16…23

u’(j)=uH(j-16)		j=24…31

If the data block is sent in the second 10ms of a 20ms block period, then:

u’(j)=uL(j+16),	j=0…7

u’(j)=uH(j+8),		j=8…15

u’(j)=uL(j+8),		j=16…23

u’(j)=uH(j)			j=24…31

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20 ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

#### 5.1a.24.3	Header coding

The header {h(0),...,h(26)} is coded as defined in subclause 5.1a.1.1, with N=27, resulting in a block of 105 bits, {C(0),...,C(104)}.

Some bits of the code are repeated in the following way:

hc(k) = C(k) for k = 0, 1, … 104 and

hc(105) = C(0), hc(106) = C(33), hc(107) = C(70)

This results in a block of 108 bits, {hc(0),...,hc(107)}.

#### 5.1a.24.4	Data coding

The data part, {i(0),…,i(449)}, is coded as defined in subclause 5.1a.1.3, with N=450, resulting in a coded block of 1398 bits, {C(0),...,C(1397)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0.05, N=466,Ndata=956 and Ndata2=876.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is a block of 956 bits, {c(0),...,c(955)}.

If a PAN is included, the result is a block of 876 bits, {c(0),...,c(875)}.

#### 5.1a.24.5	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.24.6	Interleaving

a)	Header

The header, {hc(0),...,hc(107)}, is interleaved as defined in subclause 5.1a.2.1, with NC=108 and a=10, resulting in a block of 108 bits, {hi(0),...,hi(107)}.

b)	Data and PAN

If a PAN is not included, the following rule applies:

dc(k) = c(k)			for k = 0,...,955

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,79

dc(k) =c(k-80)		for k = 80,..,955

The block {dc(0),...,dc(955)} is interleaved as defined in subclause 5.1a.2.1, with NC=956 and a=173, resulting in a block of 956 bits, {di(0),...,di(955)}.

#### 5.1a.24.7	Mapping on a burst

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(239B+j)		for j = 0,...,119

e(B,j) = hi(27B+j-120)	for j = 120,...,135

e(B,j) = q(2B+j-136)		for j = 136,137

e(B,j) = u’(8B+j-138)	for j = 138,...,145

e(B,j) = hi(27B+j-130)	for j = 146,...,156

e(B,j) = di(239B+j-37)	for j = 157,...,275

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme DBS-5 or DBS-6.

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

### 5.1a.25	Packet data block type 36 (DBS-6)

#### 5.1a.25.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 624 information bits {d(0),d(1),...,d(623)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 649 information bits {d(0),d(1),...,d(648).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,29

i(k-30) = d(k)			for k = 30,...,623

And if a PAN is included:

pn(k-624) = d(k)		for k = 624,...,648

#### 5.1a.25.2	USF coding

The USF coding is the same as for DBS-5 as specified in subclause 5.1a.24.2.

#### 5.1a.25.3	Header coding

The header coding is the same as for DBS-5 as specified in subclause 5.1a.24.3.

#### 5.1a.25.4	Data coding

The data part, {i(0),…,i(593)}, is coded as defined in subclause 5.1a.1.3, with N=594, resulting in a coded block of 1830 bits, {C(0),...,C(1829)}.

The coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0.15, N=610,Ndata=956 and Ndata2=876.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is a block of 956 bits, {c(0),...,c(955)}.

If a PAN is included, the result is a block of 876 bits, {c(0),...,c(875)}.

#### 5.1a.25.5	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.25.6	Interleaving

The interleaving is the same as for DBS-5 as specified in subclause 5.1a.24.6.

#### 5.1a.25.7	Mapping on a burst

The mapping is the same as for DBS-5 as specified in subclause 5.1a.24.7.

### 5.1a.26	Packet data block type 37 (DBS-7)

#### 5.1a.26.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 941 information bits {d(0),d(1),...,d(940)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 966 information bits {d(0),d(1),...,d(965).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,40

i1(k-41) = d(k)		for k = 41,...,490

i2(k-491) = d(k)		for k = 491,...,940

And if a PAN is included:

pn(k-941) = d(k)		for k = 941,...,965

#### 5.1a.26.2	USF coding

##### 5.1a.26.2.1	BTTI configuration

The USF bits {u(0),u(1),u(2)} are block coded into 64 bits u’(0),u’(1),...,u’(63) according to the following table:

| u(0),u(1),u(2) | u’(0),u’(1),...,u’(63) |  |  |  |
| --- | --- | --- | --- | --- |
|  | burst 0 | burst 1 | burst 2 | burst 3 |
| 000 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 |
| 001 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 |
| 010 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 |
| 011 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 |
| 100 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 |
| 101 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 |
| 110 | 0 0 1 1 0 0 1 1 0 0 1 1 0 0 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 0 0 1 1 0 0 1 1 | 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 |
| 111 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 |

##### 5.1a.26.2.2	RTTI configurations

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the USF bits {u(0),u(1),u(2)} are block coded into 64 bits u’(0),u’(1),...,u’(63) as described in subclause 5.1a.26.2.1.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into 64 bits uL(0),uL(1),...,uL(63) as described in subclause 5.1a.26.2.1; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into 64 bits uH(0),uH(1),...,uH(63) as described in subclause 5.1a.26.2.1.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then u(0),u(1),u(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then:

u’(j)=uL(j),		j=0…15

u’(j)=uH(j-16),	j=16…31

u’(j)=uL(j-16),	j=32…47

u’(j)=uH(j-32)		j=48…63

If the data block is sent in the second 10ms of a 20ms block period, then:

u’(j)=uL(j+32),	j=0…15

u’(j)=uH(j+16),	j=16…31

u’(j)=uL(j+16),	j=32…47

u’(j)=uH(j)			j=48…63

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20 ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

#### 5.1a.26.3	Header coding

The header {h(0),...,h(37)} is coded as defined in subclause 5.1a.1.1, with N=38, resulting in a block of 138 bits, {C(0),...,C(137)}.

Some bits of the code are repeated in the following way:

hc(k) = C(k) for k = 0, 1, 2, … 137 and

hc(138) = C(0), hc(139) = C(22), hc(140) = C(49), hc(141) = C(69), hc(142) = C(91), hc(143) = C(118).

The result is a block of 144 coded bits, {hc(0),...,hc(143)}.

#### 5.1a.26.4	Data coding

Each data part, {i1(0),…,i1(449)} and {i2(0),…,i2(449)}, is coded as defined in subclause 5.1a.1.3, with N=450, resulting in two coded blocks of 1398 bits, {C1(0),...,C1(1397)} and {C2(0),...,C2(1397)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0, N=466,Ndata=996 and Ndata2=956.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is two blocks of 996 bits, {c1(0),...,c1(995)} and {c2(0),...,c2(995)}.

If a PAN is included, the result is two blocks of 956 bits, {c1(0),...,c1(955)} and {c2(0),...,c2(955)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.26.5	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.26.6	Interleaving

a)	Header

The header, {hc(0),...,hc(143)}, is interleaved as defined in subclause 5.1a.2.1, with NC=144 and a=5, resulting in a block of 144 bits, {hi(0),...,hi(143)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,995

dc(k) = c2(k-996)	for k = 996,...,1991

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,79

dc(k) = c1(k-80)		for k = 80,…,1035

dc(k) = c2(k-1036)	for k = 1036,…,1991

The block {dc(0),...,dc(1991)} is interleaved as defined in subclause 5.1a.2.1, with NC=1992 and a=325, resulting in a block of 1992 bits, {di(0),...,di(1991)}.

#### 5.1a.26.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(498B+j)		for j = 0,...,248

e(B,j) = hi(36B+j-249)	for j = 249,...,271

e(B,j) = q(2B+j-272)		for j = 272,273

e(B,j) = hi(36B+j-251)	for j = 274,...,275

e(B,j) = u’(16B+j-276)	for j = 276,...,291

e(B,j) = hi(36B+j-267)	for j = 292,...,302

e(B,j) = di(498B+j-54)	for j = 303,...,551

where

q(0),q(1),...,q(7) = 0,0,0,0,0,0,0,0 identifies the coding scheme DBS-7 or DBS-8.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,224+k) with e(B,250+k) for k=0, 4, 8, 12, 16, 20, 24.

Swap e(B,221+k) with e(B,251+k) for k=0, 4, 8, 12, 16, 20, 24.

Swap e(B,304+k) with e(B,294+k) for k=0, 1, 4, 5, 8.

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,47) with e(B,120)

Swap e(B,110) with e(B,160)

Swap e(B,131) with e(B,177)

Swap e(B,199) with e(B,81)

Swap e(B,271) with e(B,201)

Swap e(B,358) with e(B,368)

Swap e(B,379) with e(B,444)

Swap e(B,426) with e(B,340)

Swap e(B,447) with e(B,329)

Swap e(B,510) with e(B,500)

Swap e(B,531) with e(B,405)

For B = 1

Swap e(B,30) with e(B,40)

Swap e(B,51) with e(B,120)

Swap e(B,114) with e(B,177)

Swap e(B,182) with e(B,0)

Swap e(B,203) with e(B,81)

Swap e(B,250) with e(B,201)

Swap e(B,275) with e(B,160)

Swap e(B,430) with e(B,368)

Swap e(B,451) with e(B,405)

Swap e(B,514) with e(B,444)

For B = 2

Swap e(B,34) with e(B,120)

Swap e(B,55) with e(B,160)

Swap e(B,123) with e(B,40)

Swap e(B,186) with e(B,81)

Swap e(B,207) with e(B,201)

Swap e(B,254) with e(B,177)

Swap e(B,303) with e(B,444)

Swap e(B,350) with e(B,340)

Swap e(B,371) with e(B,500)

Swap e(B,434) with e(B,368)

Swap e(B,455) with e(B,405)

Swap e(B,523) with e(B,329)

For B = 3

Swap e(B,38) with e(B,160)

Swap e(B,106) with e(B,81)

Swap e(B,127) with e(B,120)

Swap e(B,190) with e(B,201)

Swap e(B,211) with e(B,177)

Swap e(B,302) with e(B,340)

Swap e(B,354) with e(B,500)

Swap e(B,375) with e(B,405)

Swap e(B,438) with e(B,444)

Swap e(B,506) with e(B,329)

Swap e(B,527) with e(B,368)

### 5.1a.27	Packet data block type 38 (DBS-8)

#### 5.1a.27.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1229 information bits {d(0),d(1),...,d(1228)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1254 information bits {d(0),d(1),...,d(1253).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,40

i1(k-41) = d(k)		for k = 41,...,634

i2(k-635) = d(k)		for k = 635,...,1228

And if a PAN is included:

pn(k-1229) = d(k)	for k = 1229,...,1253

#### 5.1a.27.2	USF coding

The USF coding is the same as for DBS-7 as specified in subclause 5.1a.26.2.

#### 5.1a.27.3	Header coding

The header coding is the same as for DBS-7 as specified in subclause 5.1a.26.3.

#### 5.1a.27.4	Data coding

Each data part, {i1(0),…,i1(593)} and {i2(0),…,i2(593)}, is coded as defined in subclause 5.1a.1.3, with N=594, resulting in two coded blocks of 1830 bits, {C1(0),...,C1(1829)} and {C2(0),...,C2(1829)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Two puncturing schemes named P1 or P2 are applied.

The parameter values used for rate matching are: swap=0.10, N=610,Ndata=996 and Ndata2=956.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 1) puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is two blocks of 996 bits, {c1(0),...,c1(995)} and {c2(0),...,c2(995)}.

If a PAN is included, the result is two blocks of 956 bits, {c1(0),...,c1(955)} and {c2(0),...,c2(955)}.

NOTE:	C1 and c1 correspond to i1, and C2 and c2 to i2.

#### 5.1a.27.5	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.27.6	Interleaving

The interleaving is the same as for DBS-7 as specified in subclause 5.1a.26.6.

#### 5.1a.27.7	Mapping on a burst

The mapping is the same as for DBS-7 as specified in subclause 5.1a.26.7.

### 5.1a.28	Packet data block type 39 (DBS-9)

#### 5.1a.28.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1403 information bits {d(0),d(1),...,d(1402)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1428 information bits {d(0),d(1),...,d(1427).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,52

i1(k-53) = d(k)		for k = 53,...,502

i2(k-503) = d(k)		for k = 503,...,952

i3(k-953) = d(k)		for k = 953,...,1402

And if a PAN is included:

pn(k-1403) = d(k)		for k = 1403,...,1427

#### 5.1a.28.2	USF coding

The USF coding is the same as for DBS-7 as specified in subclause 5.1a.26.2.

#### 5.1a.28.3	Header coding

The header {h(0),...,h(49)} is coded as defined in subclause 5.1a.1.1, with N=50, resulting in a block of 174 bits, {C(0),...,C(173)}.

The code is punctured in such a way that the following coded bits:

{C(87*k+j) for k = 0 and 1, j = 44, 65 and 86} are not transmitted

This results in a block of 168 bits, {hc(0),...,hc(167)}.

#### 5.1a.28.4	Data coding

Each data part, {i1(0),…,i1(449)}, {i2(0),…,i2(449)} and {i3(0),…,i3(449)}, is coded as defined in subclause 5.1a.1.3, with N=450, resulting in three coded blocks of 1398 bits, {C1(0),...,C1(1397)}, {C2(0),...,C2(1397)} and {C3(0),...,C3(1397)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=466,Ndata=656 and Ndata2=630.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is three blocks of 656 bits, {c1(0),...,c1(655)}, {c2(0),...,c2(655)} and {c3(0),...,c3(655)}.

If a PAN is included, the result is three blocks of 630 bits, {c1(0),...,c1(629)}, {c2(0),...,c2(629)} and {c3(0),...,c3(629)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2 and C3 and c3 to i3..

#### 5.1a.28.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.28.6	Interleaving

a)	Header

The header, {hc(0),...,hc(167)}, is interleaved as defined in subclause 5.1a.2.1, with NC=168 and a=17, resulting in a block of 168 bits, {hi(0),...,hi(167)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,655

dc(k) = c2(k-656)	for k = 656,...,1311

dc(k) = c3(k-1312)	for k = 1312,...,1967

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,...,77

dc(k) = c1(k-78)		for k = 78,…,707

dc(k) = c2(k-708)	for k = 708,…,1337

dc(k) = c3(k-1338)	for k = 1338,…,1967

The block {dc(0),...,dc(1967)} is interleaved as defined in subclause 5.1a.2.1, with NC=1968 and a=283, resulting in a block of 1968 bits, {di(0),...,di(1967)}.

#### 5.1a.28.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(492B+j)		for j = 0,...,245

e(B,j) = hi(42B+j-246)	for j = 246,...,271

e(B,j) = q(2B+j-272)		for j = 272,273

e(B,j) = hi(42B+j-248)	for j = 274,...,275

e(B,j) = u’(16B+j-276)	for j = 276,...,291

e(B,j) = hi(42B+j-264)	for j = 292,...,305

e(B,j) = di(492B+j-60)	for j = 306,...,551

where

q(0),q(1),...,q(7) = 1,1,1,1,1,1,1,1 identifies the coding scheme DBS-9.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,216+k) with e(B,246+k) for k=0, 1, 4, 5, 8, 9, 12, 13, 16, 17, 20, 21, 24, 25, 28, 29.

Swap e(B,308+k) with e(B,294+k) for k=0, 1, 4, 5, 8, 9.

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,26) with e(B,81)

Swap e(B,74) with e(B,177)

Swap e(B,87) with e(B,201)

Swap e(B,174) with e(B,40)

Swap e(B,222) with e(B,160)

Swap e(B,235) with e(B,120)

Swap e(B,343) with e(B,444)

Swap e(B,430) with e(B,368)

Swap e(B,443) with e(B,500)

Swap e(B,491) with e(B,405)

For B = 1

Swap e(B,38) with e(B,120)

Swap e(B,51) with e(B,40)

Swap e(B,99) with e(B,201)

Swap e(B,186) with e(B,81)

Swap e(B,234) with e(B,177)

Swap e(B,307) with e(B,500)

Swap e(B,442) with e(B,405)

Swap e(B,455) with e(B,329)

Swap e(B,503) with e(B,444)

For B = 2

Swap e(B,50) with e(B,160)

Swap e(B,63) with e(B,120)

Swap e(B,111) with e(B,177)

Swap e(B,198) with e(B,201)

Swap e(B,211) with e(B,81)

Swap e(B,306) with e(B,444)

Swap e(B,319) with e(B,368)

Swap e(B,406) with e(B,329)

Swap e(B,454) with e(B,405)

Swap e(B,467) with e(B,500)

For B = 3

Swap e(B,14) with e(B,40)

Swap e(B,62) with e(B,160)

Swap e(B,75) with e(B,120)

Swap e(B,123) with e(B,177)

Swap e(B,210) with e(B,201)

Swap e(B,223) with e(B,81)

Swap e(B,331) with e(B,368)

Swap e(B,418) with e(B,329)

Swap e(B,466) with e(B,444)

Swap e(B,479) with e(B,500)

### 5.1a.29	Packet data block type 40 (DBS-10)

#### 5.1a.29.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 1835 information bits {d(0),d(1),...,d(1834)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 1860 information bits {d(0),d(1),...,d(1859).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,52

i1(k-53) = d(k)		for k = 53,...,646

i2(k-647) = d(k)		for k = 647,...,1240

i3(k-1241) = d(k)		for k = 1241,...,1834

And if a PAN is included:

pn(k-1835) = d(k)		for k = 1835,...,1859

#### 5.1a.29.2	USF coding

##### 5.1a.29.2.1	BTTI configuration

The USF bits {u(0),u(1),u(2)} are block coded into 80 bits u’(0),u’(1),...,u’(79) according to the following table:

| u(0),u(1),u(2) | u’(0),u’(1),...,u’(79) |  |  |  |
| --- | --- | --- | --- | --- |
|  | burst 0 | burst 1 | burst 2 | burst 3 |
| 000 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 001 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 |
| 010 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 |
| 011 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 100 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 |
| 101 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 |
| 110 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 | 1 0 0 1 0 1 0 0 1 0 0 0 0 0 0 0 0 0 0 0 | 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 |
| 111 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 | 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 1 0 0 1 0 |

##### 5.1a.29.2.2	RTTI configurations

If the USF is sent in RTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the USF bits {u(0),u(1),u(2)} are block coded into 80 bits u’(0),u’(1),...,u’(79) as described in subclause 5.1a.29.2.1.

If the USF is sent in BTTI USF mode (see 3GPP TS 45.002) when data blocks are transmitted in RTTI configuration, then the three bits of the USF to be sent on the lower numbered PDCH of a corresponding downlink PDCH-pair are block coded into 80 bits uL(0),uL(1),...,uL(79) as described in subclause 5.1a.29.2.1; the three bits of the USF to be sent on the higher numbered PDCH of a corresponding downlink PDCH-pair are block coded into 80 bits uH(0),uH(1),...,uH(79) as described in subclause 5.1a.29.2.1.

NOTE:	If BTTI USF mode is used when sending data blocks in RTTI configuration, then u(0),u(1),u(2) need not contain a USF; in this case, they are ignored by the encoder. How the USFs are delivered to the encoder in this case is implementation dependent.

If the data block is sent in the first 10ms of a 20ms block period, then:

u’(j)=uL(j),		j=0…19

u’(j)=uH(j-20),	j=20…39

u’(j)=uL(j-20),	j=40…59

u’(j)=uH(j-40)		j=60…79

If the data block is sent in the second 10ms of a 20ms block period, then:

u’(j)=uL(j+40),	j=0…19

u’(j)=uH(j+20),	j=20…39

u’(j)=uL(j+20),	j=40…59

u’(j)=uH(j)			j=60…79

NOTE:	In case mixed modulation USF is used (see subclause 5.1), the USF bits sent during the other half of the 20ms block period may be sent with a different modulation. In this case, the half of uL and uH not sent in the present data block will be discarded.

#### 5.1a.29.3	Header coding

The header coding is the same as for DBS-9 as specified in subclause 5.1a.28.3.

#### 5.1a.29.4	Data coding

Each data part, {i1(0),…,i1(593)}, {i2(0),…,i2(593)} and {i3(0),…,i3(593)}, is coded as defined in subclause 5.1a.1.3, with N=594, resulting in three coded blocks of 1830 bits, {C1(0),...,C1(1829)}, {C2(0),...,C2(1829)} and {C3(0),...,C3(1829)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=610,Ndata=833 and Ndata2=807.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is three blocks of 833 bits, {c1(0),...,c1(832)}, {c2(0),...,c2(832)} and {c3(0),...,c3(832)}.

If a PAN is included, the result is three blocks of 807 bits, {c1(0),...,c1(806)}, {c2(0),...,c2(806)} and {c3(0),...,c3(806)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2 and C3 and c3 to i3..

#### 5.1a.29.5	PAN coding

The PAN coding is the same as for UAS-7 as specified in subclause 5.1a.3.4.

#### 5.1a.29.6	Interleaving

a)	Header

The header, {hc(0),...,hc(167)}, is interleaved as defined in subclause 5.1a.2.1, with NC=168 and a=19, resulting in a block of 168 bits, {hi(0),...,hi(167)}.

b)	Data and PAN

If a PAN is not included, data are put together as one entity as described by the following rule:

dc(k) = c1(k)			for k = 0,...,832

dc(k) = c2(k-833)	for k = 833,...,1665

dc(k) = c3(k-1666)	for k = 1666,...,2498

dc(k) = 0				for k = 2499

If a PAN is included, data and PAN are put together as one entity as described by the following rule:

dc(k) = ac(k)			for k = 0,…,77

dc(k) = c1(k-78)		for k = 78,…,884

dc(k) = c2(k-885)	for k = 885,…,1691

dc(k) = c3(k-1692)	for k = 1692,…,2498

dc(k) = 0				for k = 2499

The block {dc(0),...,dc(2499)} is interleaved as defined in subclause 5.1a.2.1, with NC=2500 and a=323, resulting in a block of 2500 bits, {di(0),...,di(2499)}.

#### 5.1a.29.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(625B+j)		for j = 0,...,312

e(B,j) = hi(42B+j-313)	for j = 313,...,339

e(B,j) = q(3B+j-340)		for j = 340

e(B,j) = hi(42B+j-314)	for j = 341

e(B,j) = q(3B+j-341)		for j = 342,...,343

e(B,j) = hi(42B+j-316)	for j = 344

e(B,j) = u’(20B+j-345)	for j = 345,...,364

e(B,j) = hi(42B+j-336)	for j = 365,...,377

e(B,j) = di(625B+j-65)	for j = 378,...,689

where

q(0),q(1),...,q(11) = 0,0,0,0,0,0,0,0,0,0,0,0 identifies the coding scheme DBS-10.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,285+k) with e(B,316+k) for k=0, 5, 10, 15, 20, 25.

Swap e(B,278+k) with e(B,314+k) for k=0, 5, 10, 15, 20, 25, 30.

Swap e(B,270+k) with e(B,317+k) for k=0, 5,10.

Swap e(B,268+k) with e(B,332+k) for k=0, 5.

Swap e(B,380+k) with e(B,366+k) for k=0, 5, 10.

Swap e(B,378+k) with e(B,369+k) for k=0, 5.

Swap e(B,388+k) with e(B,367+k) for k=0, 5.

Swap e(B,395) with e(B,377)

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0

Swap e(B,21) with e(B,80)

Swap e(B,42) with e(B,98)

Swap e(B,84) with e(B,110)

Swap e(B,126) with e(B,158)

Swap e(B,147) with e(B,170)

Swap e(B,189) with e(B,188)

Swap e(B,367) with e(B,410)

Swap e(B,409) with e(B,433)

Swap e(B,451) with e(B,480)

Swap e(B,472) with e(B,503)

Swap e(B,514) with e(B,530)

Swap e(B,556) with e(B,688)

Swap e(B,577) with e(B,640)

For B = 1

Swap e(B,86) with e(B,80)

Swap e(B,107) with e(B,98)

Swap e(B,149) with e(B,110)

Swap e(B,191) with e(B,158)

Swap e(B,212) with e(B,188)

Swap e(B,254) with e(B,200)

Swap e(B,474) with e(B,410)

Swap e(B,516) with e(B,433)

Swap e(B,537) with e(B,480)

Swap e(B,579) with e(B,503)

Swap e(B,621) with e(B,530)

Swap e(B,642) with e(B,493)

For B = 2

Swap e(B,17) with e(B,200)

Swap e(B,151) with e(B,80)

Swap e(B,172) with e(B,98)

Swap e(B,214) with e(B,110)

Swap e(B,256) with e(B,158)

Swap e(B,277) with e(B,170)

Swap e(B,334) with e(B,188)

Swap e(B,384) with e(B,493)

Swap e(B,539) with e(B,410)

Swap e(B,581) with e(B,433)

Swap e(B,602) with e(B,480)

Swap e(B,644) with e(B,503)

Swap e(B,686) with e(B,530)

For B = 3

Swap e(B,19) with e(B,158)

Swap e(B,61) with e(B,170)

Swap e(B,82) with e(B,188)

Swap e(B,237) with e(B,80)

Swap e(B,279) with e(B,98)

Swap e(B,331) with e(B,110)

Swap e(B,386) with e(B,503)

Swap e(B,407) with e(B,530)

Swap e(B,449) with e(B,493)

Swap e(B,604) with e(B,410)

Swap e(B,646) with e(B,433)

Swap e(B,667) with e(B,480)

### 5.1a.30	Packet data block type 41 (DBS-11)

#### 5.1a.30.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 2249 information bits {d(0),d(1),...,d(2248)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 2274 information bits {d(0),d(1),...,d(2273).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,64

i1(k-65) = d(k)		for k = 65,...,610

i2(k-611) = d(k)		for k = 611,...,1156

i3(k-1157) = d(k)		for k = 1157,...,1702

i4(k-1703) = d(k)		for k = 1703,...,2248

And if a PAN is included:

pn(k-2249) = d(k)		for k = 2249,...,2273

#### 5.1a.30.2	USF coding

The USF coding is the same as for DBS-10 as specified in subclause 5.1a.29.2.

#### 5.1a.30.3	Header coding

The header {h(0),...,h(61)} is coded as defined in subclause 5.1a.1.1, with N=62, resulting in a block of 210 bits, {C(0),...,C(209)}.

The code is punctured in such a way that the following coded bits:

{C(30*k+j) for k = 0,…,6, j = 17, 20 and 28, and C(10)} are not transmitted

This results in a block of 188 bits, {hc(0),...,hc(187)}.

#### 5.1a.30.4	Data coding

Each data part, {i1(0),…,i1(545)}, {i2(0),…,i2(545)}, {i3(0),…,i3(545)} and {i4(0),…,i4(545)}, is coded as defined in subclause 5.1a.1.3, with N=546, resulting in four coded blocks of 1686 bits, {C1(0),...,C1(1685)}, {C2(0),...,C2(1685)}, {C3(0),...,C3(1685)} and {C4(0),...,C4(1685)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=562,Ndata=620 and Ndata2=600.

P1 puncturing is generated according to 5.1a.1.3.5

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

If a PAN is not included, the result is four blocks of 620 bits, {c1(0),...,c1(619)}, {c2(0),...,c2(619)}, {c3(0),...,c3(619)} and {c4(0),...,c4(619)}.

If a PAN is included, the result is four blocks of 600 bits, {c1(0),...,c1(599)}, {c2(0),...,c2(599)}, {c3(0),...,c3(599)} and {c4(0),...,c4(599)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, C3 and c3 to i3, and C4 and c4 to i4.

#### 5.1a.30.5	PAN coding

The PAN coding is the same as for UBS-5 as specified in subclause 5.1a.8.4.

#### 5.1a.30.6	Interleaving

a)	Header

The header, {hc(0),...,hc(187)}, is interleaved as defined in subclause 5.1a.2.1, with NC=188 and a=3, resulting in a block of 188 bits, {hi(0),...,hi(187)}.

b)	Data and PAN

If a PAN is not included, the following definitions apply:

dc1(k) = c1(k)			for k = 0,...,619

dc2(k) = c2(k)			for k = 0,...,619

dc3(k) = c3(k)			for k = 0,...,619

dc4(k) = c4(k)			for k = 0,...,619

If a PAN is included, the following definitions apply:

dc1(k) = ac(4k)			for k = 0,…,19

dc1(k) = c1(k-20)		for k = 20,…,619

dc2(k) = ac(4k+1)		for k = 0,…,19

dc2(k) = c2(k-20)		for k = 20,…,619

dc3(k) = ac(4k+2)		for k = 0,…,19

dc3(k) = c3(k-20)		for k = 20,…,619

dc4(k) = ac(4k+3)		for k = 0,…,19

dc4(k) = c4(k-20)		for k = 20,…,619

The four blocks {dc1(0),...,dc1(619)}, {dc2(0),...,dc2(619)}, {dc3(0),...,dc3(619)} and {dc4(0),...,dc4(619)} are separately interleaved as defined in subclause 5.1a.2.2, with NC=620 and a=141, resulting in the four blocks {di1(0),…,di1(619)}, {di2(0),…,di2(619)}, {di3(0),…,di3(619)} and {di4(0),…,di4(619)}, where di1 corresponds to dc1, di2 to dc2, di3 to dc3 and di4 to dc4.

The blocks are put together as one entity as described by the following rule:

di(k) = di1(k)				for k = 0,...,619

di(k) = di2(k-620)		for k = 620,...,1239

di(k) = di3(k-1240)		for k = 1240,...,1859

di(k) = di4(k-1860)		for k = 1860,...,2479

#### 5.1a.30.7	Mapping on a burst

a)	Straightforward mapping

The mapping is given by the rule:

For B=0,1,2,3, let

e(B,j) = di(620B+j)		for j = 0,...,309

e(B,j) = hi(47B+j-310)	for j = 310,...,339

e(B,j) = q(3B+j-340)		for j = 340

e(B,j) = hi(47B+j-311)	for j = 341

e(B,j) = q(3B+j-341)		for j = 342,...,343

e(B,j) = hi(47B+j-313)	for j = 344

e(B,j) = u’(20B+j-345)	for j = 345,...,364

e(B,j) = hi(47B+j-333)	for j = 365,...,379

e(B,j) = di(620B+j-70)	for j = 380,...,689

where

q(0),q(1),...,q(11) = 1,1,1,1,1,1,1,1,1,1,1,1 identifies the coding scheme DBS-11 or DBS-12.

b)	Bit swapping

After this mapping the following bits are swapped:

For B = 0,1,2,3,

Swap e(B,275+k) with e(B,311+k) for k=0, 3, 5, 8, 10, 13, 15, 18, 20, 23, 25, 28, 30, 33.

Swap e(B,260+k) with e(B,312+k) for k=0, 5, 10.

Swap e(B,263+k) with e(B,327+k) for k=0, 5, 10.

Swap e(B,380+k) with e(B,366+k) for k=0, 3, 5, 8, 10, 13.

Swap e(B,395+k) with e(B,372+k) for k=0, 5.

Swap e(B,398) with e(B,367).

In RTTI configuration, the bursts with B = 0,2 shall be mapped on the PDCH having the lower timeslot number, whereas the bursts with B = 1,3 shall be mapped on the PDCH having the higher timeslot number, see 3GPP TS 45.002.

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0, 1, 2 and 3

Swap e(B,29) with e(B,110)

Swap e(B,114) with e(B,158)

Swap e(B,141) with e(B,80)

Swap e(B,199) with e(B,188)

Swap e(B,226) with e(B,98)

Swap e(B,381) with e(B,433)

Swap e(B,466) with e(B,480)

Swap e(B,634) with e(B,410)

### 5.1a.31	Packet data block type 42 (DBS-12)

#### 5.1a.31.1	Block constitution

If the message delivered to the encoder does not include a PAN, it has a fixed size of 2441 information bits {d(0),d(1),...,d(2440)}. If the message delivered to the encoder includes a PAN, it has a fixed size of 2466 information bits {d(0),d(1),...,d(2465).

NOTE:	The presence of the PAN is indicated by the PANI field in the header (see 3GPP TS 44.060).

The message is separated into the following parts:

u(k) = d(k)			for k = 0,...,2

h(k-3) = d(k)			for k = 3,...,64

i1(k-65) = d(k)		for k = 65,...,658

i2(k-659) = d(k)		for k = 659,...,1252

i3(k-1253) = d(k)		for k = 1253,...,1846

i4(k-1847) = d(k)		for k = 1847,...,2440

And if a PAN is included:

pn(k-2441) = d(k)		for k = 2441,...,2465

#### 5.1a.31.2	USF coding

The USF coding is the same as for DBS-10 as specified in subclause 5.1a.29.2.

#### 5.1a.31.3	Header coding

The header coding is the same as for DBS-11 as specified in subclause 5.1a.30.3.

#### 5.1a.31.4	Data coding

Each data part, {i1(0),…,i1(593)}, {i2(0),…,i2(593)}, {i3(0),…,i3(593)} and {i4(0),…,i4(593)}, is coded as defined in subclause 5.1a.1.3, with N=594, resulting in four coded blocks of 1830 bits, {C1(0),...,C1(1829)}, {C2(0),...,C2(1829)}, {C3(0),...,C3(1829)} and {C4(0),...,C4(1829)}.

Each coded block is punctured depending on the value of the CPS field as defined in 3GPP TS 44.060. Three puncturing schemes named P1, P2 or P3 are applied.

The parameter values used for rate matching are: swap=0, N=610,Ndata=620 and Ndata2=606.

P1 puncturing is generated according to 5.1a.1.3.5 using the parameters above if PAN is not included. If PAN is included in P1, then for generation of P1, N = 606 is used.

NOTE:	When PAN is included, P1 calculated as above results in transmission of all systematic bits.

P2 (Type 2) puncturing is generated according to 5.1a.1.3.5.

P3 puncturing is generated according to 5.1a.1.3.5.

NOTE:	P2 and P3 are always generated using N = 610 irrespective of whether or not P1 is with or without PAN.

If a PAN is not included, the result is four blocks of 620 bits, {c1(0),...,c1(619)}, {c2(0),...,c2(619)}, {c3(0),...,c3(619)} and {c4(0)...,c4(619)}.

If a PAN is included, the result is four blocks of 606 bits, {c1(0),...,c1(605)}, {c2(0),...,c2(605)}, {c3(0),...,c3(605)} and {c4(0),...,c4(605)}.

NOTE:	C1 and c1 correspond to i1, C2 and c2 to i2, C3 and c3 to i3, and C4 and c4 to i4.

#### 5.1a.31.5	PAN coding

The PAN {pn(0),...,pn(24)}, if included, is coded as defined in subclause 5.1a.1.4, resulting in a block of 90 bits, {C(0),...,C(89)}.

The code is punctured in such a way that the following coded bits:

{C(15*k) , C(15*k+2), C(15*k+4), C(15*k+7), C(15*k+10), C(15*k+13) for k = 0,1,...5} are not transmitted; except C(15), C(45) which are transmitted.

This results in a block of 56 bits, {ac(0),...,ac(55)}.

#### 5.1a.31.6	Interleaving

a)	Header

The header interleaving is the same as for DBS-11 as specified in subclause 5.1a.30.6.

b)	Data and PAN

If a PAN is not included, the following definitions apply:

dc1(k) = c1(k)			for k = 0,...,619

dc2(k) = c2(k)			for k = 0,...,619

dc3(k) = c3(k)			for k = 0,...,619

dc4(k) = c4(k)			for k = 0,...,619

If a PAN is included, the following definitions apply:

dc1(k) = ac(4k)			for k = 0,…,13

dc1(k) = c1(k-14)		for k = 14,…,619

dc2(k) = ac(4k+1)		for k = 0,…,13

dc2(k) = c2(k-14)		for k = 14,…,619

dc3(k) = ac(4k+2)		for k = 0,…,13

dc3(k) = c3(k-14)		for k = 14,…,619

dc4(k) = ac(4k+3)		for k = 0,…,13

dc4(k) = c4(k-14)		for k = 14,…,619

The four blocks {dc1(0),...,dc1(619)}, {dc2(0),...,dc2(619)}, {dc3(0),...,dc3(619)} and {dc4(0),...,dc4(619)} are separately interleaved as defined in subclause 5.1a.2.2, with NC=620 and a=141, resulting in the four blocks {di1(0),…,di1(619)}, {di2(0),…,di2(619)}, {di3(0),…,di3(619)} and {di4(0),…,di4(619)}, where di1 corresponds to dc1, di2 to dc2, di3 to dc3 and di4 to dc4.

The blocks are put together as one entity as described by the following rule:

di(k) = di1(k)				for k = 0,...,619

di(k) = di2(k-620)		for k = 620,...,1239

di(k) = di3(k-1240)		for k = 1240,...,1859

di(k) = di4(k-1860)		for k = 1860,...,2479

#### 5.1a.31.7	Mapping on a burst

a)	Straightforward mapping

The mapping is the same as for DBS-11 as specified in subclause 5.1a.30.7 a).

b)	Bit swapping

The bit swapping is the same as for DBS-11 as specified in subclause 5.1a.30.7.b).

c)	PAN bit swapping

In case a PAN is included in the radio block, the following additional bits are swapped after the bit swapping in b):

For B = 0, 1, 2 and 3

Swap e(B,29) with e(B,158)

Swap e(B,141) with e(B,80)

Swap e(B,226) with e(B,110)

Swap e(B,282) with e(B,98)

Swap e(B,381) with e(B,480)

Swap e(B,437) with e(B,433)

Swap e(B,522) with e(B,503)

Swap e(B,634) with e(B,410)

## 5.1b	Extended Coverage Packet data traffic channel (EC-PDTCH)

### 5.1b.1	General

Twelve coding schemes are specified for the Extended Coverage Packet data traffic channels. The coding schemes MCS-1 to MCS-9 are identical to those defined for PDTCH in subclause 5.1 and apply for downlink and uplink coverage class CC1. The coding schemes MCS-1/16, MCS-1/8 and MCS-1/4 for downlink and uplink coverage classes CC4, CC3 and CC2, respectively, are based on MCS-1 defined in subclause 5.1. The coding scheme MCS-1'/48 for uplink coverage class CC5 is specified in subclause 5.1b.4.

The following additions apply:

-	Blind physical layer transmissions are employed.

-	For downlink, multiple sequences of USF bits are delivered to the encoder. Each precoded USF sequence is mapped onto one instance of the repeated block.

The following restrictions apply:

-	Reduced Transmission Time Interval (RTTI) shall not be used on EC-PDTCH.

-	A PAN field shall not be included in the message delivered to the encoder.

-	For data coding, a reduced set of puncturing schemes is used on an EC-PDTCH, compared to a PDTCH. Only the puncturing schemes listed in table 0 shall be used.

Table 0: Allowed puncturing schemes on EC-PDTCH

| MCS | Allowed puncturing schemes in subclause 5.1 |
| --- | --- |
| MCS-1/16 MCS-1/8 MCS-1/4 MCS-1 MCS-2 | P1 |
| MCS-3 MCS-4 | P1 or P2 |
| Other MCSs | No restriction |

### 5.1b.2	Downlink MCS-1/M

#### 5.1b.2.1	Block constitution

The message delivered to the encoder has a fixed size of 206 information bits {a(0),a(1),...,a(205)}. In addition, M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8 and 4 for MCS-1/16, MCS-1/8 and MCS-1/4, respectively.

#### 5.1b.2.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions, the block is encoded as specified for MCS-1 in subclause 5.1.5.1. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission k is defined as

d(k)=u(m,k)		for		k=0,1,2;

d(k)=a(k-3)		for		k=3,…,208.

Denote the resulting bursts from encoding of blind physical layer transmissions m as R(m,B,j) = e(B,j) for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.2.3	Mapping onto PDCHs

Burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where

- for EC channels with 4 PDCHs assigned

B’ = B + 4(m div 4);

pn = (m mod 4).

- for EC channels with 2 PDCHs assigned

B’ = B + 4(m div 2);

pn = (m mod 2).

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on a PDCH. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8] and 3GPP TS 44.060 [6].

NOTE 2:	pn denotes the relative PDCH number within the set of PDCHs on which the EC-PDTCH is mapped, where PDCHs are numbered from lower to higher timeslot number. The absolute timeslot number depends on the assignment, see 3GPP TS 44.060 [6].

### 5.1b.3	Uplink MCS-1/M

#### 5.1b.3.1	Block constitution

The message delivered to the encoder has a fixed size of 209 information bits {a(0),a(1),...,a(208)}.

#### 5.1b.3.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions the block is encoded as specified for MCS-1 in subclause 5.1.5.2. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission m is defined as

d(k)=a(k)	for k=0,…,208.

Denote the resulting bursts from encoding of blind physical layer transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.3.3	Mapping onto PDCHs

The mapping is done as specified for MCS-1/M DL in subclause 5.1b.2.3, except when 2 PDCHs are assigned. In this case, burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where:

B’ = B(M div 2) + (m div 2);

pn = (m mod 2).

### 5.1b.4	Uplink MCS-1’/48

#### 5.1b.4.1	Block constitution

The message delivered to the encoder has a fixed size of 194 information bits {d(0),d(1),...,d(193)}. It is delivered on a burst mode.

#### 5.1b.4.2	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(0)D23 +...+ d(15)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 30 bits {u"(-6),…,u"(0),u"(1),...,u"(23)} with six negative indexes:

u"(k-6)	= p(k+2)		for k = 0,1,...,5

u"(k)		= d(k)			for k = 0,1,...,15

u"(k)		= p(k-16)		for k = 16,17,...,23

c)	Convolutional encoder

This block of 30 bits {u"(-6),…,u"(0),u"(1),...,u"(23)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 72 coded bits: {C(0),C(1),...,C(71)} defined by:

C(3k)     = u"(k) + u"(k-2) + u"(k-3) + u"(k-5) + u"(k-6)

C(3k+1) = u"(k) + u"(k-1) + u"(k-2) + u"(k-3) + u"(k-6)

C(3k+2) = u"(k) + u"(k-1) + u"(k-4) + u"(k-6)    		for k = 0,1,...,23

The code is punctured in such a way that the following coded bits:

C(2+12j), C(5+12j), C(8+12j), C(11+12j)  for j = 0,1,...,5} are not transmitted

The result is a block of 48 coded bits, {hc(0),hc(1),...,hc(47)}.

#### 5.1b.4.3	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(16)D189 +...+ d(193)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 196 bits 
{u(0),u(1),...,u(195)}:

u(k)	= d(k+16)		for k = 0,1,...,177

u(k)	= p(k-178)	for k = 178,179,...,189

u(k)	= 0				for k = 190,191,…,195 (tail bits)

c)	Convolutional encoder

This block of 196 bits {u(0),u(1),...,u(195)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 588 coded bits: {C(0),C(1),...,C(587)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,195; u(k) = 0 for k < 0

The code is punctured using always P1 as defined below.

| P1 | {C(2+21j), C(5+21j), C(8+21j), C(10+21j), C(11+21j), C(14+21j), C(17+21j), C(20+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 73,136,199,262,325,388,451,514} which are transmitted |
| --- | --- |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

#### 5.1b.4.4	Interleaving

The header and data are put together as one entity as described by the following rule:

c(k)	= hc(k)		for k = 0,1,...,47

c(k)	= dc(k-48)	)		for k = 48,49,...,419

c’(n,k) = c(n,k)		for k = 0,1,...,24

c’(n,k) = c(n,k-1)		for k = 26,27,...,81

c’(n,k) = c(n,k-2)		for k = 83,84,...,138

c’(n,k) = c(n,k-3)		for k = 140,141,...,400

c’(n,k) = c(n,k-4)		for k = 402,403,...,423

c’(n,25) = q(10)		c’(n,82) = q(11)		c’(n,139) = q(12)		c’(n,401) = q(13)

c(n,k) are the coded bits and q(10),q(11),q(12),q(13) = 0,0,0,0 are four extra stealing flags

The resulting block of 424 bits is interleaved according to the following rule:

i(B,j) = c’(n,k)	for	k = 0,1,...,423

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 4)

j = 2((49k) mod 53) + ((k mod 8) div 4)

#### 5.1b.4.5	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,63+j) = i(B,53+j)		for j = 0,1,...,52

and

e(B+m,53+i) = q(i) and  e(B+58+i) =q(i+5)  		for m = 0,1,2,3 and i = 0,1,2,...,4

where

q(0),q(1),..,q(9) = 0,0,0,0,0,0,0,0,0,0.

#### 5.1b.4.6	Blind physical layer transmission

The resulting bursts from encoding of blind physical layer transmission m are denoted as R(m,B,j) = e(B,j)  for m=0,…,47, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1b.4.5.

#### 5.1b.4.7	Mapping onto PDCHs

Burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where

- for EC channels with 4 PDCHs assigned

B’ = B + 4*(m div 4);

pn = (m mod 4).

- for EC channels with 2 PDCHs assignedB’ = B(M div 2) + (m div 2);

pn = (m mod 2).

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on a PDCH. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8] and 3GPP TS 44.060 [6].

NOTE 2:	pn denotes the relative PDCH number within the set of PDCHs on which the EC-PDTCH is mapped, where PDCHs are numbered from lower to higher timeslot number. The absolute timeslot number depends on the assignment, see 3GPP TS 44.060 [6].

## 5.2	Packet control channels (PACCH, PBCCH, PAGCH, PPCH, PTCCH, CPBCCH, CPAGCH and CPPCH)

The coding scheme used for PACCH, PBCCH, PAGCH, PPCH, downlink PTCCH, CPBCCH, CPAGCH and CPPCH is the same as for CS-1 as specified in subclause 5.1.1.

In RTTI configuration, the channel coding for the downlink PACCH may also be the one defined for MCS-0 in subclause 5.1.4a.

In a DLMC configuration (see 3GPP TS 44.060), the coding scheme for the uplink PACCH may also be the one defined for CS-3 in subclause 5.1.3.

The coding scheme used for uplink PTCCH is the same as for PRACH as specified in subclause 5.3.

In the case of a downlink PACCH including an eTFI field (see 3GPP TS 44.060) the message delivered to the encoder will have a fixed size of 187 information bits {d(0),d(1),...,d(186)}. The first 184 bits will be protected by 40 parity bits as specified in subclause 4.1.2 a). Bit-wise modulo 2 addition is performed using a subset of the parity bits. Which parity bits are used is dependent on the state of the three bits {d(184),d(185),d(186)}. These three bits represent the assigned eTFI value and are converted to their equivalent decimal representation, et ϵ {0, 1, 2…7}. d{186} represents the least significant bit.

This results in the forty modified parity bits {pt(0),…,pt(39)} defined as:

pt(k*10+et) = 1 + p(k*10+et)		for k=0,1,2

pt(32+et) = 1 + p(32+et)

pt(k) = p(k)							for the remaining 36 parity bits

Four tail bits equal to 0 are then added to the information and modified parity bits as specified in subclause 4.1.2 b with p(k) replaced by pt(k). Convolutional encoding, Interleaving and Mapping on a Burst then follows the process outlined in subclauses 4.1.3, 4.1.4 and 4.1.5 respectively.

## 5.2a	Extended Coverage broadcast control channel (EC-BCCH)

### 5.2a.1	Block constitution

The message delivered to the encoder has a fixed size of 184 information bits {d(0),d(1),...,d(183)}.

### 5.2a.2	Coding

The message is coded as for SACCH as specified in subclause 4.1, resulting in the bursts e(B,j), B=0,1,2,3 and j = 0,1,...,115.

### 5.2a.3	Blind physical layer transmission

Each burst is transmitted 16 times. Denote the resulting bursts of transmission m as R(m,B,j) = e(B,j)  for m=0,…,15, B=0,1,2,3 and  j=0,…,115.

### 5.2a.4	Mapping onto a physical channel

Burst R(m,B,j), j=0…,115, is mapped onto burst B’ of the timeslot carrying the EC-BCCH, where

B’ = B + 4m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

## 5.2b	Extended coverage common control channel downlink (EC-CCCH/D/M) , Extended coverage paging indication channel (EC-PICH) and Extended coverage packet associated control channel (EC-PACCH/M)

### 5.2b.1	Block constitution

The message delivered to the encoder has a fixed size of Kd information bits {d(0),d(1),...,d(Kd -1)}, where:

-	EC-CCCH/D, EC-PICH: Kd=88

-	EC-PACCH/D: Kd=80

-	EC-PACCH/U/M: Kd=64 for M=1,4,8,16

-	EC-PACCH/U/M: Kd=56 for M=48

In addition, for EC-PACCH/D, up to M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8, 4 or 1 depending on the number of blind physical layer transmissions.

NOTE:	USF bits need not be delivered for those blocks that are transmitted on timeslots no GPRS/EGPRS MS is monitoring for USF.

### 5.2b.2	Data coding

a)	Parity bits:

Eighteen data parity bits p(0),p(1),...,p(17) are defined in such a way that in GF(2) the binary polynomial:

d(0)D Kd +18-1 +...+ d(Kd-1)D18 + p(0)D17 +...+ p(17), when divided by:

D18 + D17 + D14 + D13 + D11 + D10 + D8 + D7 + D6 + D3 + D2 + 1, yields a remainder equal to:

D17 + D16 + D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

The parity bits are added after the block of Kd bits, the result being a block of Kd+18 bits, {b(0),…,b(Kd+18-1)}, defined as:

b(k)	= d(k)		for k = 0,1,..., Kd-1

b(k)	= p(k-Kd)	for k = Kd,..., Kd+18-1

b) Tail-biting convolutional encoder

The six last bits are added before the block of Kd+18 bits, the result being a block of Kd+24 bits {c(-6),…,c(0),c(1),...,c(Kd+18-1)} with six negative indices:

c(k)		= b(Kd+18+k)		for k = -6,...,-1

c(k)		= b(k)				for k = 0,1,..., Kd+18-1

This block of Kd+24 bits is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of (Kd+18)*3 coded bits {C(0),...,C((Kd+18)*3-1)} defined by:

C(3k)   = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

C(3k+1) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

C(3k+2) = c(k) + c(k-1) + c(k-4) + c(k-6)   for k = 0,1,..., Kd+18-1

The code is punctured in such a way that the following coded bits are not transmitted:

| EC-CCCH/D, EC-PICH | C(floor(n*318/202)) for n=0,…,201 |
| --- | --- |
| EC-PACCH/U (M=1,4,8,16) | C(floor(n*246/130)) for n=0,…,129 |
| EC-PACCH/D | C(floor(n*294/180)) for n=0,…,179 |
| EC-PACCH/U (M=48) | C(floor(n*222/116)) for n=0,…,115 |

The result is a block of Kpc coded bits {pc(0),pc(1),...,pc(Kpc-1)}, where Kpc=114 for EC-PACCH/D and Kpc=116 for EC-CCCH/D, EC-PICH and EC-PACCH/U/1, EC-PACCH/U/4, EC-PACCH/U/8, EC-PACCH/U/16 and Kpc=106 for EC-PACCH/U/48.

### 5.2b.3	USF coding (only for EC-PACCH/D)

For each of the up to M delivered USF sequences, the USF  bits are block coded as for Packet data block type 4 (CS-4) in subclause 5.1.4.2. The input d(0),d(1),d(2) in subclause 5.1.4.2 shall be {u(m,0),u(m,1),u(m,2)} for USF number m.

Denote the resulting twelve encoded bits of USF number m as {u’(m,0),u’(m,1),...,u’(m,11)}.

### 5.2b.4	Mapping on a Burst

#### 5.2b.4.1	EC-CCCH/D/M and EC-PICH

The mapping is given by the rule:

e(B, j)		= pc(j)			for j = 0,1,...,115 and B=0,1

#### 5.2b.4.2	EC-PACCH/D/M

The mapping is given by the rule:

e(B,j) = pc(j)  and  e(B,59+j) = pc(57+j)		for j = 0,1,...,56 and B=0,1,2,3

and

e(B,57) = q(2B)  and  e(B,58) = q(2B+1)		for B = 0,1,2,3

where

q(0),q(1),...,q(7) = 0,0,0,1,0,1,1,0.

NOTE:	For a standard GPRS MS, bits q(0),...,q(7) indicates that the USF is coded as for CS-4.

#### 5.2b.4.3	EC-PACCH/U/M

The mapping is given by the following rule for M = 1,4,8 or 16:

e(B, j)		= pc(j)			for j = 0,1,...,115 and B=0,1,2,3

The mapping is given by the following rule for M = 48:

e(B,j) = pc(j)  and  e(B,63+j) = pc(53+j)		for j = 0,1,...,52 and B=0,1,2,3

and

e(B,53+i) = q(i)  for i = 0,1,…,9

where

q(0),q(1),...,q(9) = 0,0,0,0,0,0,0,0,0,0

### 5.2b.5	Blind physical layer transmission

Each burst is transmitted M times, where M is the number of blind physical layer transmissions. For EC-PACCH/D, M=1, 4, 8 or 16. For EC-PACCH/U, M=1, 4, 8, 16 or 48. For EC-CCCH/D, M=1, 8, 16 or 32. For EC-PICH, M=1, i.e. no blind physical layer transmission is applied.

Denote the resulting bursts of transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,1 (EC-CCCH) or B=0,1,2,3 (EC-PACCH), and  j=0,…,115 or m=0, B=0,1 (EC-PICH).

### 5.2b.6	Bit stealing for USF (only for EC-PACCH/D)

For those m for which a USF has been delivered to the encoder, replace bits in the repeated bursts with encoded USF bits as follows:

for m=0,…,M-1

R(m,0,0) = u’(m,0)

R(m,0,51) = u’(m,4)

R(m,0,102) = u’(m,8)

R(m,1,100) = u’(m,1)

R(m,1,35) = u’(m,5)

R(m,1,86) = u’(m,9)

R(m,2,84) = u’(m,2)

R(m,2,19) = u’(m,6)

R(m,2,70) = u’(m,10)

R(m,3,68) = u’(m,3)

R(m,3,3) = u’(m,7)

R(m,3,52) = u’(m,11)

### 5.2b.7	Mapping onto physical channels

#### 5.2b.7.1	EC-PACCH

The mapping onto PDCHs is done as specified for downlink MCS-1/M in subclause 5.1b.2.3 for M=1,4,8,16. The mapping onto PDCHs is done as specified for uplink MCS-1’/48 in subclause 5.1b.4.3.

#### 5.2b.7.2	EC-CCCH/D and EC-PICH

Burst R(m,B,j), B=0,1, j=0…,115, is mapped onto burst B’ of the timeslot carrying the EC-CCCH/D or EC-PICH, where

B’ = B + 2m, for m=0,…,M-1 (EC-CCCH/D) and m=0 (EC-PICH)

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

## 5.3	Packet random access channel (PRACH, CPRACH and MPRACH)

Two coding schemes are specified for access bursts on the packet switched channels. The packet access burst containing 8 information bits and the extended packet access burst containing 11 information bits. Only the 11 information bits access burst may be transmitted on the CPRACH.

### 5.3.1	Packet Access Burst

The encoding of this burst is as defined in section 4.6 for the random access channel (RACH). The BSIC used shall be the BSIC of the BTS to which the burst is intended.

### 5.3.2	Extended Packet Access Burst

#### 5.3.2.1	Block constitution

The burst carrying the extended packet random access uplink message contains 11 information bits d(0),d(1),...,d(10).

#### 5.3.2.2	Parity bits

Six parity bits p(0),p(1),...,p(5) are defined in such a way that in GF(2) the binary polynomial:

d(0)D16 +...+ d(10)D6 + p(0)D5 +...+ p(5), when divided by D6 + D5 + D3 + D2 + D + 1 yields a remainder equal to D5 + D4 + D3 + D2 + D + 1.

#### 5.3.2.3	Addition of BSIC

An MS that has not enabled PEO or EC operation shall use the six bit BSIC of the BTS to which the access burst is intended. The six bits of the BSIC, {b(0),b(1),...,b(5)}, are added bitwise modulo 2 to the six parity bits, {p(0),p(1),...,p(5)}.

This results in six colour bits, C(0) to C(5) defined as C(k) = b(k) + p(k) (k = 0 to 5) where:

b(0) = MSB of PLMN colour code

b(5) = LSB of BS colour code.

This defines {u(0),u(1),..., u(20)} by:

u(k) = d(k)		for k = 0,1,...,10

u(k) = C(k-11)	for k = 11,12,...,16

u(k) = 0			for k = 17,18,19,20 (tail bits)

An MS that has enabled PEO or EC operation shall use the nine bit BSIC of the BTS to which the access burst is intended . The first six bits of the BSIC, {b(0),b(1),...,b(5)}, are added bitwise modulo 2 to the six parity bits while the last three bits of the BSIC {b(6),b(7),b(8)} are added bitwise modulo 2 to the three last information bits d(8), d(9) and d(10). The bitwise modulo 2 operation results in nine colour bits, C(0) to C(8), defined as C(k) = b(k) + p(k) (for k = 0 to 5) and C(k) = b(k) + d(k+2) (for k = 6 to 8) where:

b(0) = MSB of PLMN colour code

b(8) = LSB of Radio frequency colour code.

This defines {u(0),u(1),..., u(20)} by:

u(k) = d(k)		for k = 0,1,...,7

u(k) = C(k-2)		for k = 8,9,10

u(k) = C(k-11)	for k = 11,12,...,16

u(k) = 0			for k = 17,18,19,20 (tail bits)

#### 5.3.2.4	Convolutional code

The coded bits {c(0),c(1),..., c(41)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

c(2k) = u(k) + u(k-3) + u(k-4)

c(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,20 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits:

c(0), c(2), c(5), c(37), c(39), c(41) are not transmitted.

This results in a block of 36 coded bits, {e(0), e(1),...,e(35)}.

## 5.3a	Extended coverage random access channel (EC-RACH/M)

### 5.3a.0	General

Channel encoding for the random access message in EC operation containing 11 information bits is depicted in the following sub-clauses 5.3a.1 to 5.3a.4, except for EC-RACH/66 (for ESAB format) and EC-RACH/132 (for EDAB format) the channel encoding are specified in clause 5.3a.6 and 5.3a.7. Sub-clause 5.3a.5 deals with the channel encoding for the random access message in EC operation containing 30 information bits.

### 5.3a.1	Block constitution

The message delivered to the encoder has a fixed size of 11 information bits {d(0),d(1),...,d(10)}.

### 5.3a.2	Coding

The message is encoded as for the 11 information bit access defined in subclause 5.3.2, resulting in a block of 36 coded bits {e(0), e(1),...,e(35)}.

### 5.3a.3	Blind physical layer transmissions

The block is transmitted M times, where M=1, 4, 16 or 48 for 1 TS EC-RACH and M=4, 16 or 48 for 2 TS EC-RACH, resulting in the repeated bursts R(m,B,j)=e(j) for m=0,…,M-1, B=0 and j=0,…,35.

### 5.3a.4	Mapping onto physical channel(s)

#### 5.3a.4.1	1 TS EC-RACH

Burst R(m,B,j), B=0, j=0…,35, is mapped onto burst B’ of the timeslot carrying the EC-RACH, where

B’ = m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

#### 5.3a.4.2	2 TS EC-RACH

Burst R(m,B,j), B=0, j=0…,35, is mapped onto burst B’ of timeslot number tn of the timeslots carrying the EC-RACH, where

B’ = m div 2

tn = m mod 2

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

NOTE 2:	tn denotes the relative timeslot number within the timeslot pair on which the EC-RACH is mapped. The absolute timeslot number depends on the configuration, see 3GPP TS 44.018 [4].

### 5.3a.5	EC-RACH carrying 30 information bits

In case of performing the Multilateration Timing Advance procedure using the Extended Access Burst method in        EC operation, i.e. the mobile station has selected Coverage Class 1 for the uplink and the downlink, the MS sends a random access message carrying 11 information bits using the encoding according to the above sub-clauses, and after access grant by the network sends a subsequent random access message carrying 30 information bits. The encoding of the 30 information bits follows the one for (EC-)RACH specified in subclause 4.6.2.

### 5.3a.6	Extended coverage random access channel (EC-RACH/66)

#### 5.3a.6.0	General

If ESAB format (see 3GPP TS 45.002) is used for EC-RACH in CC5 operation the channel coding, blind physical layer transmissions and mapping onto physical channels are depicted in the subclause 5.3a.6.

Channel encoding, blind physical layer transmissions and mapping onto physical channels for the random access channel EC-RACH/66 used for CC5 in EC operation is depicted in the following sub-clauses 5.3a.6.1 to 5.3a.6.6.

#### 5.3a.6.1	Block constitution

The message delivered to the encoder has a fixed size of 11 information bits {d(0),d(1),...,d(10)}.

#### 5.3a.6.2	Parity bits

Six parity bits are added as defined in subclause 5.3.2.2

#### 5.3a.6.3	Addition of BSIC

The first six bits of the BSIC, {b(0),b(1),...,b(5)}, are added bitwise modulo 2 to the six parity bits while the last three bits of the BSIC {b(6),b(7),b(8)} are added bitwise modulo 2 to the three last information bits d(8), d(9) and d(10). The bitwise modulo 2 operation results in nine colour bits, C(0) to C(8), defined as C(k) = b(k) + p(k) (for k = 0 to 5) and C(k) = b(k) + d(k+2) (for k = 6 to 8) where:

b(0) = MSB of PLMN colour code

b(8) = LSB of Radio frequency colour code.

This defines a block of 17 bits {u(0),u(1),..., u(16)} by:

u(k) = d(k)		for k = 0,1,...,7

u(k) = C(k-2)		for k = 8,9,10

u(k) = C(k-11)	for k = 11,12,...,16

#### 5.3a.6.4	Tail biting Convolutional code

The six last bits are added before the block of 17 bits, the result being a block of 23 bits {c(-6),…,c(0),c(1),...,c(17-1)} with six negative indices:

c(k)		= u(23+k)		for k = -6,...,-1

c(k)		= u(k)			for k = 0, 1,...,17-1

The block of 23 bits is encoded with 1/6 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G4 = 1 + D2 + D3 + D5 + D6

G7= 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

G6 = 1 + D + D2 + D3 + D4 + D6

G6 = 1 + D + D2 + D3 + D4 + D6

This results in a block of 17*6 coded bits {e(0), e(1),...,e(101)} defined by:

e(6k)     = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

e(6k+1) = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

e(6k+2) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

e(6k+3) = c(k) + c(k-1) + c(k-4) + c(k-6)

e(6k+4) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-4) + c(k-6)

e(6k+5) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-4) + c(k-6)	for k = 0,1,...,16

#### 5.3a.6.5	Blind physical layer transmissions

The block is transmitted M times, where M=1, 4, 16 or 48 for 1 TS EC-RACH, M=4, 16 or 48 for 2 TS EC-RACH, resulting in the repeated bursts R(m,B,j)=e(j) for m=0,…,M-1, B=0 and j=0,…,35, and M=66 for EC-RACH CC5 using ESAB burst, resulting in the repeated ESAB bursts R(m,B,j)=e(j) for m=0,…,65, B=0 and j=0,…,101.

#### 5.3a.6.6	Mapping onto physical channel(s)

For EC-RACH CC5/66, burst R(m,B,j), B=0, j=0…,101, is mapped onto ESAB burst B’’ on timeslot pair tn’ carrying the EC-RACH CC5, where

B’’ = m

NOTE 3:	The burst number B’’ denotes the relative transmission order of the ESAB bursts on 2 timeslots. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

NOTE 4:	tn’ denotes the relative timeslot pair number on which the EC-RACH CC5 is mapped. The absolute timeslot pair depends on the configuration, see 3GPP TS 44.018 [4].

### 5.3a.7	Extended coverage random access channel (EC-RACH/132)

#### 5.3a.7.0	General

If EDAB format (see 3GPP TS 45.002) is used for EC-RACH in CC5 operation the channel coding, blind physical layer transmissions and mapping onto physical channels are depicted in the subclause 5.3a.7.

Channel encoding, blind physical layer transmissions and mapping onto physical channels for the random access channel EC-RACH/132 used for CC5 in EC operation is depicted in the following sub-clauses 5.3a.6.1 to 5.3a.6.6.

#### 5.3a.7.1	Block constitution

The message delivered to the encoder has a fixed size of 11 information bits {d(0),d(1),...,d(10)}.

#### 5.3a.7.2	Parity bits

Six parity bits are added as defined in subclause 5.3.2.2

#### 5.3a.7.3	Addition of BSIC

9 bits of BSIC information is added over 6 parity bits and 3 data bits as defined in subclause 5.3.2.3.

#### 5.3a.7.4	Convolutional code

The coded bits {c(0),c(1),..., c(41)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

c(2k) = u(k) + u(k-3) + u(k-4)

c(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,20 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits:

[c(0), c(3), c(7), c(12), c(14), c(17), c(24), c(27), c(31), c(34), c(39), c(41)] are not transmitted.

This results in a block of 30 coded bits, {e(0), e(1),...,e(29)}.

#### 5.3a.7.5	Blind physical layer transmissions

The block of encoded data is transmitted 264 times using the Segmented AB for the 2 TS EC-RACH format (see 3GPP TS 45.002[8]). The first burst with normal burst size carries 3 instances of the encoded data block and the second burst with access burst size carries one instance of the encoded data block. The bursts are transmitted in a timeslot pair which is repeated over 66 TDMA frames forming 132 bursts in total.

#### 5.3a.7.6	Mapping onto physical channel(s)

The data block {e(0),...,e(29)} is mapped onto the Segmented AB, repeated over 66 TDMA frames in total, as specified in 3GPP TS 45.002[8].

For tn = TN mod 2 = 0, the data block {e(0), e(1),...,e(29)} is mapped 3 times on the burst with normal burst size.

For tn = TN mod 2 = 1, the data block {e(0), e(1),...,e(29)} is mapped once on the burst with access burst size.

NOTE :	tn denotes the relative timeslot number within the timeslot pair on which the EC-RACH is mapped. The absolute timeslot number depends on the configuration, see 3GPP TS 44.018 [4].

## 5.4	Access Burst on packet switched channels other than PRACH, CPRACH and MPRACH

The encoding of this burst is as defined in subclause 5.3 for the packet random access channel (PRACH). The BSIC used shall be the BSIC of the BTS to which the burst is intended.

An MS in EC operation sending a block on EC-PACCH/U as four access bursts shall use blind physical layer transmissions of each access burst according to the assigned UL Coverage Class. The blind physical layer transmissions and mapping onto physical channels shall be as described for EC-PACCH/U in subclause 5.2b.

Table 1: Reordering and partitioning of a coded block of 456 bits into 8 sub-blocks

| k mod 8= | 0 | 1 | 2 | 3 |  | k mod 8= | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| j=0 | k=0 | 57 | 114 | 171 |  | j=1 | 228 | 285 | 342 | 399 |
| 2 | 64 | 121 | 178 | 235 |  | 3 | 292 | 349 | 406 | 7 |
| 4 | 128 | 185 | 242 | 299 |  | 5 | 356 | 413 | 14 | 71 |
| 6 | 192 | 249 | 306 | 363 |  | 7 | 420 | 21 | 78 | 135 |
| 8 | 256 | 313 | 370 | 427 |  | 9 | 28 | 85 | 142 | 199 |
| 10 | 320 | 377 | 434 | 35 |  | 11 | 92 | 149 | 206 | 263 |
|  | 384 | 441 | 42 | 99 |  |  | 156 | 213 | 270 | 327 |
|  | 448 | 49 | 106 | 163 |  |  | 220 | 277 | 334 | 391 |
|  | 56 | 113 | 170 | 227 |  |  | 284 | 341 | 398 | 455 |
|  | 120 | 177 | 234 | 291 |  |  | 348 | 405 | 6 | 63 |
| 20 | 184 | 241 | 298 | 355 |  | 21 | 412 | 13 | 70 | 127 |
|  | 248 | 305 | 362 | 419 |  |  | 20 | 77 | 134 | 191 |
|  | 312 | 369 | 426 | 27 |  |  | 84 | 141 | 198 | 255 |
|  | 376 | 433 | 34 | 91 |  |  | 148 | 205 | 262 | 319 |
|  | 440 | 41 | 98 | 155 |  |  | 212 | 269 | 326 | 383 |
| 30 | 48 | 105 | 162 | 219 |  | 31 | 276 | 333 | 390 | 447 |
|  | 112 | 169 | 226 | 283 |  |  | 340 | 397 | 454 | 55 |
|  | 176 | 233 | 290 | 347 |  |  | 404 | 5 | 62 | 119 |
|  | 240 | 297 | 354 | 411 |  |  | 12 | 69 | 126 | 183 |
|  | 304 | 361 | 418 | 19 |  |  | 76 | 133 | 190 | 247 |
| 40 | 368 | 425 | 26 | 83 |  | 41 | 140 | 197 | 254 | 311 |
|  | 432 | 33 | 90 | 147 |  |  | 204 | 261 | 318 | 375 |
|  | 40 | 97 | 154 | 211 |  |  | 268 | 325 | 382 | 439 |
|  | 104 | 161 | 218 | 275 |  |  | 332 | 389 | 446 | 47 |
|  | 168 | 225 | 282 | 339 |  |  | 396 | 453 | 54 | 111 |
| 50 | 232 | 289 | 346 | 403 |  | 51 | 4 | 61 | 118 | 175 |
|  | 296 | 353 | 410 | 11 |  |  | 68 | 125 | 182 | 239 |
|  | 360 | 417 | 18 | 75 |  |  | 132 | 189 | 246 | 303 |
|  | 424 | 25 | 82 | 139 |  |  | 196 | 253 | 310 | 367 |
|  | 32 | 89 | 146 | 203 |  |  | 260 | 317 | 374 | 431 |
| 60 | 96 | 153 | 210 | 267 |  | 61 | 324 | 381 | 438 | 39 |
|  | 160 | 217 | 274 | 331 |  |  | 388 | 445 | 46 | 103 |
|  | 224 | 281 | 338 | 395 |  |  | 452 | 53 | 110 | 167 |
|  | 288 | 345 | 402 | 3 |  |  | 60 | 117 | 174 | 231 |
|  | 352 | 409 | 10 | 67 |  |  | 124 | 181 | 238 | 295 |
| 70 | 416 | 17 | 74 | 131 |  | 71 | 188 | 245 | 302 | 359 |
|  | 24 | 81 | 138 | 195 |  |  | 252 | 309 | 366 | 423 |
|  | 88 | 145 | 202 | 259 |  |  | 316 | 373 | 430 | 31 |
|  | 152 | 209 | 266 | 323 |  |  | 380 | 437 | 38 | 95 |
|  | 216 | 273 | 330 | 387 |  |  | 444 | 45 | 102 | 159 |
| 80 | 280 | 337 | 394 | 451 |  | 81 | 52 | 109 | 166 | 223 |
|  | 344 | 401 | 2 | 59 |  |  | 116 | 173 | 230 | 287 |
|  | 408 | 9 | 66 | 123 |  |  | 180 | 237 | 294 | 351 |
|  | 16 | 73 | 130 | 187 |  |  | 244 | 301 | 358 | 415 |
|  | 80 | 137 | 194 | 251 |  |  | 308 | 365 | 422 | 23 |
| 90 | 144 | 201 | 258 | 315 |  | 91 | 372 | 429 | 30 | 87 |
|  | 208 | 265 | 322 | 379 |  |  | 436 | 37 | 94 | 151 |
|  | 272 | 329 | 386 | 443 |  |  | 44 | 101 | 158 | 215 |
|  | 336 | 393 | 450 | 51 |  |  | 108 | 165 | 222 | 279 |
|  | 400 | 1 | 58 | 115 |  |  | 172 | 229 | 286 | 343 |
| 100 | 8 | 65 | 122 | 179 |  | 101 | 236 | 293 | 350 | 407 |
|  | 72 | 129 | 186 | 243 |  |  | 300 | 357 | 414 | 15 |
|  | 136 | 193 | 250 | 307 |  |  | 364 | 421 | 22 | 79 |
|  | 200 | 257 | 314 | 371 |  |  | 428 | 29 | 86 | 143 |
|  | 264 | 321 | 378 | 435 |  |  | 36 | 93 | 150 | 207 |
| 110 | 328 | 385 | 442 | 43 |  | 111 | 100 | 157 | 214 | 271 |
| 112 | 392 | 449 | 50 | 107 |  | 113 | 164 | 221 | 278 | 335 |

Table 2: Subjective importance of encoded bits for the full rate speech TCH
(Parameter names and bit indices refer to 3GPP TS 46.010)

| Importance class | Parameter name | Parameter number | Bit index | Label | Class |
| --- | --- | --- | --- | --- | --- |
| 1 | Log area ratio 1 | 1 | 5 | d0 |  |
|  | block amplitude | 12,29,46,63 | 5 | d1, d2, d3, d4 |  |
|  | Log area ratio 1 | 1 | 4 |  |  |
| 2 | Log area ratio 2 | 2 | 5 |  |  |
|  | Log area ratio 3 | 3 | 4 |  |  |
|  | Log area ratio 1 | 1 | 3 |  |  |
|  | Log area ratio 2 | 2 | 4 |  |  |
|  | Log area ratio 3 | 3 | 3 |  |  |
|  | Log area ratio 4 | 4 | 4 |  |  |
| 3 | LPT lag | 9,26,43,60 | 6 |  | 1 |
|  | block amplitude | 12,29,46,63 | 4 |  | with |
|  | Log area ratio 2,5,6 | 2,5,6 | 3 |  | parity |
|  | LPT lag | 9,26,43,60 | 5 |  | check |
|  | LPT lag | 9,26,43,60 | 4 |  |  |
|  | LPT lag | 9,26,43,60 | 3 |  |  |
|  | LPT lag | 9,26,43,60 | 2 |  |  |
|  | block amplitude | 12,29,46,63 | 3 |  |  |
|  | Log area ratio 1 | 1 | 2 |  |  |
|  | Log area ratio 4 | 4 | 3 |  |  |
|  | Log area ratio 7 | 7 | 2 |  |  |
| 4 | LPT lag | 9,26,43,60 | 1 | ...d48, d49 |  |
|  | Log area ratio 5,6 | 5,6 | 2 | d50 |  |
|  | LPT gain | 10,27,44,61 | 1 |  |  |
|  | LPT lag | 9,26,43,60 | 0 |  |  |
|  | Grid position | 11,28,45,62 | 1 |  |  |
|  | Log area ratio 1 | 1 | 1 |  |  |
|  | Log area ratio 2,3,8,4 | 2,3,8,4 | 2 |  |  |
|  | Log area ratio 5,7 | 5,7 | 1 |  |  |
|  | LPT gain | 10,27,44,61 | 0 |  |  |
|  | block amplitude | 12,29,46,63 | 2 |  | 1 |
|  | RPE pulses | 13..25 | 2 |  | without |
|  | RPE pulses | 30..42 | 2 |  | parity |
| 5 | RPE pulses | 47..59 | 2 |  | check |
|  | RPE pulses | 64..76 | 2 |  |  |
|  | Grid position | 11,28,45,62 | 0 |  |  |
|  | block amplitude | 12,29,46,63 | 1 |  |  |
|  | RPE pulses | 13..25 | 1 |  |  |
|  | RPE pulses | 30..42 | 1 |  |  |
|  | RPE pulses | 47..59 | 1 |  |  |
|  | RPE pulses | 64..67 | 1 | ...d181 |  |
|  | RPE pulses | 68..76 | 1 | d182 |  |
|  | Log area ratio 1 | 1 | 0 |  |  |
|  | Log area ratio 2,3,6 | 2,3,6 | 1 |  |  |
|  | Log area ratio 7 | 7 | 0 |  |  |
|  | Log area ratio 8 | 8 | 1 |  |  |
|  | Log area ratio 8,3 | 8,3 | 0 |  |  |
| 6 | Log area ratio 4 | 4 | 1 |  | 2 |
|  | Log area ratio 4,5 | 4,5 | 0 |  |  |
|  | block amplitude | 12,29,46,63 | 0 |  |  |
|  | RPE pulses | 13..25 | 0 |  |  |
|  | RPE pulses | 30..42 | 0 |  |  |
|  | RPE pulses | 47..59 | 0 |  |  |
|  | RPE pulses | 64..76 | 0 |  |  |
|  | Log area ratio 2,6 | 2,6 | 0 | ...d259 |  |

Table 3a: Subjective importance of encoded bits for the half rate speech TCH for unvoiced speech frames (Parameter names and bit indices refer to 3GPP TS 46.020)

| Parameter | Bit | Label | Class |
| --- | --- | --- | --- |
| name | index |  |  |
| R0 | 1 | d0 |  |
| LPC 3 | 7 | d1 |  |
| GSP 0-1 | 2 | d2 |  |
| GSP 0-2 | 2 | d3 |  |
| GSP 0-3 | 2 | d4 |  |
| GSP 0-4 | 2 | d5 |  |
| LPC 1 | 0 | d6 |  |
| LPC 2 | 5...1 | d7...d11 |  |
| LPC 3 | 6...1 | d12... |  |
| Code 1-2 | 0 |  |  |
| Code 2-2 | 6...0 |  |  |
| Code 1-3 | 6...0 |  | 1 |
| Code 2-3 | 6...3 |  |  |
| LPC3 | 0 |  | without |
| R0 | 0 |  | parity |
| INT-LPC | 0 |  | check |
| Code 1-2 | 1...6 |  |  |
| Code 2-1 | 0...6 |  |  |
| Code 1-1 | 0...6 |  |  |
| GSP 0-4 | 0 |  |  |
| GSP 0-3 | 0 |  |  |
| GSP 0-2 | 0 |  |  |
| GSP 0-1 | 0 |  |  |
| LPC 2 | 0 |  |  |
| GSP 0-4 | 1 |  |  |
| GSP 0-3 | 1 |  |  |
| GSP 0-2 | 1 |  |  |
| GSP 0-1 | 1 |  |  |
| LPC 1 | 1...4 | ...d72 |  |
| LPC 1 | 5 | d73... |  |
| GSP 0-4 | 3 |  |  |
| GSP 0-3 | 3 |  |  |
| GSP 0-2 | 3 |  |  |
| GSP 0-1 | 3 |  |  |
| LPC2 | 6...8 |  | 1 |
| GSP 0-4 | 4 |  |  |
| GSP 0-3 | 4 |  | with |
| GSP 0-2 | 4 |  | parity |
| GSP 0-1 | 4 |  | check |
| LPC 1 | 6...9 |  |  |
| R0 | 2 |  |  |
| LPC 1 | 10 |  |  |
| R0 | 3,4 |  |  |
| Mode | 0,1 | ...d94 |  |
| Code 2-4 | 0...6 | d95... |  |
| Code 1-4 | 0...6 |  | 2 |
| Code 2-3 | 0...2 | ...d111 |  |

Table 3b: Subjective importance of encoded bits for the half rate speech TCH for voiced speech frames (Parameter names and bit indices refer to 3GPP TS 46.020)

| Parameter name | Bit index | Label | Class |  | Parameter name | Bit index | Label | Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LPC 1 | 2,1 | d0, d1 |  |  | LAG 3 | 3 | d73... |  |
| LPC 2 | 6...4 | d2... |  |  | LAG 2 | 3 |  |  |
| GSP 0-1 | 4 |  |  |  | LAG 1 | 3,4 |  | 1 |
| GSP 0-2 | 4 |  |  |  | LPC 2 | 7,8 |  |  |
| GSP 0-3 | 4 |  |  |  | LPC 1 | 3...6 |  | with |
| GSP 0-4 | 4 |  |  |  | R0 | 2 |  | parity |
| GSP 0-1 | 3 |  |  |  | LAG 1 | 5...7 |  | check |
| GSP 0-2 | 3 |  |  |  | LPC 1 | 7...10 |  |  |
| GSP 0-3 | 3 |  |  |  | R0 | 3,4 |  |  |
| GSP 0-4 | 3 |  |  |  | Mode | 0,1 | ...d94 |  |
| GSP 0-1 | 2 |  |  |  | Code 4 | 0...8 | d95... | 2 |
| GSP 0-2 | 2 |  |  |  | Code 3 | 0...7 | ...d111 |  |
| GSP 0-3 | 2 |  |  |  |  |  |  |  |
| GSP 0-4 | 2 |  |  |  |  |  |  |  |
| Code 1 | 8...0 |  |  |  |  |  |  |  |
| Code 2 | 8...5 |  |  |  |  |  |  |  |
| Code 2 | 2...0 |  |  |  |  |  |  |  |
| Code 3 | 8 |  |  |  |  |  |  |  |
| Code 2 | 4,3 |  |  |  |  |  |  |  |
| GSP 0-1 | 1 |  |  |  |  |  |  |  |
| GSP 0-2 | 1 |  |  |  |  |  |  |  |
| GSP 0-3 | 1 |  |  |  |  |  |  |  |
| GSP 0-4 | 1 |  | 1 |  |  |  |  |  |
| GSP 0-1 | 0 |  |  |  |  |  |  |  |
| GSP 0-2 | 0 |  | without |  |  |  |  |  |
| GSP 0-3 | 0 |  | parity |  |  |  |  |  |
| GSP 0-4 | 0 |  | check |  |  |  |  |  |
| INT-LPC | 0 |  |  |  |  |  |  |  |
| LPC 2 | 0 |  |  |  |  |  |  |  |
| LPC 3 | 0 |  |  |  |  |  |  |  |
| LAG 4 | 0 |  |  |  |  |  |  |  |
| LPC 3 | 1 |  |  |  |  |  |  |  |
| LPC 2 | 1 |  |  |  |  |  |  |  |
| LAG 4 | 1 |  |  |  |  |  |  |  |
| LAG 3 | 0 |  |  |  |  |  |  |  |
| LAG 2 | 0 |  |  |  |  |  |  |  |
| LAG 1 | 0 |  |  |  |  |  |  |  |
| LAG 4 | 2 |  |  |  |  |  |  |  |
| LAG 3 | 1 |  |  |  |  |  |  |  |
| LAG 2 | 1 |  |  |  |  |  |  |  |
| LAG 1 | 1 |  |  |  |  |  |  |  |
| LPC 3 | 2...4 |  |  |  |  |  |  |  |
| LPC 2 | 2 |  |  |  |  |  |  |  |
| LPC 3 | 5,6 |  |  |  |  |  |  |  |
| LPC 2 | 3 |  |  |  |  |  |  |  |
| R0 | 0 |  |  |  |  |  |  |  |
| LPC 3 | 7 |  |  |  |  |  |  |  |
| LPC 1 | 0 |  |  |  |  |  |  |  |
| LAG 4 | 3 |  |  |  |  |  |  |  |
| LAG 3 | 2 |  |  |  |  |  |  |  |
| LAG 2 | 2 |  |  |  |  |  |  |  |
| LAG 1 | 2 |  |  |  |  |  |  |  |
| R0 | 1 | ...d72 |  |  |  |  |  |  |

Table 4: Reordering and partitioning of a coded block of 228 bits into 4 sub-blocks for TCH/HS

| b= | 0 | 1 |  | b= | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- |
| j=0 | k=0 | 150 |  | j=1 | k=1 | 151 |
| 2 | 38 | 188 |  | 3 | 39 | 189 |
| 4 | 76 | 226 |  | 5 | 77 | 227 |
| 6 | 114 | 14 |  | 7 | 115 | 15 |
| 8 | 152 | 52 |  | 9 | 153 | 53 |
| 10 | 190 | 90 |  | 11 | 191 | 91 |
|  | 18 | 128 |  |  | 19 | 129 |
|  | 56 | 166 |  |  | 57 | 167 |
|  | 94 | 204 |  |  | 95 | 205 |
|  | 132 | 32 |  |  | 133 | 33 |
| 20 | 170 | 70 |  | 21 | 171 | 71 |
|  | 208 | 108 |  |  | 209 | 109 |
|  | 8 | 146 |  |  | 9 | 147 |
|  | 46 | 184 |  |  | 47 | 185 |
|  | 84 | 222 |  |  | 85 | 223 |
| 30 | 122 | 10 |  | 31 | 123 | 11 |
|  | 160 | 48 |  |  | 161 | 49 |
|  | 198 | 86 |  |  | 199 | 87 |
|  | 28 | 124 |  |  | 29 | 125 |
|  | 66 | 162 |  |  | 67 | 163 |
| 40 | 104 | 200 |  | 41 | 105 | 201 |
|  | 142 | 30 |  |  | 143 | 31 |
|  | 180 | 68 |  |  | 181 | 69 |
|  | 218 | 106 |  |  | 219 | 107 |
|  | 4 | 144 |  |  | 5 | 145 |
| 50 | 42 | 182 |  | 51 | 43 | 183 |
|  | 80 | 220 |  |  | 81 | 221 |
|  | 118 | 6 |  |  | 119 | 7 |
|  | 156 | 44 |  |  | 157 | 45 |
|  | 194 | 82 |  |  | 195 | 83 |
| 60 | 22 | 120 |  | 61 | 23 | 121 |
|  | 60 | 158 |  |  | 61 | 159 |
|  | 98 | 196 |  |  | 99 | 197 |
|  | 136 | 24 |  |  | 137 | 25 |
|  | 174 | 62 |  |  | 175 | 63 |
| 70 | 212 | 100 |  | 71 | 213 | 101 |
|  | 12 | 138 |  |  | 13 | 139 |
|  | 50 | 176 |  |  | 51 | 177 |
|  | 88 | 214 |  |  | 89 | 215 |
|  | 126 | 2 |  |  | 127 | 3 |
| 80 | 164 | 40 |  | 81 | 165 | 41 |
|  | 202 | 78 |  |  | 203 | 79 |
|  | 34 | 116 |  |  | 35 | 117 |
|  | 72 | 154 |  |  | 73 | 155 |
|  | 110 | 192 |  |  | 111 | 193 |
| 90 | 148 | 26 |  | 91 | 149 | 27 |
|  | 186 | 64 |  |  | 187 | 65 |
|  | 224 | 102 |  |  | 225 | 103 |
|  | 16 | 140 |  |  | 17 | 141 |
|  | 54 | 178 |  |  | 55 | 179 |
| 100 | 92 | 216 |  | 101 | 93 | 217 |
|  | 130 | 20 |  |  | 131 | 21 |
|  | 168 | 58 |  |  | 169 | 59 |
|  | 206 | 96 |  |  | 207 | 97 |
|  | 36 | 134 |  |  | 37 | 135 |
| 110 | 74 | 172 |  | 111 | 75 | 173 |
| 112 | 112 | 210 |  | 113 | 113 | 211 |

Table 5: Enhanced Full rate Source Encoder output parameters in order of occurrence and bit allocation within the speech frame of 244 bits/20 ms(Parameter names and bit indices refer to 3GPP TS 46.060)

| Bits (MSB-LSB) | Description |
| --- | --- |
| s1 - s7 | index of 1st LSF submatrix |
| s8 - s15 | index of 2nd LSF submatrix |
| s16 - s23 | index of 3rd LSF submatrix |
| s24 | sign of 3rd LSF submatrix |
| s25 - s32 | index of 4th LSF submatrix |
| s33 - s38 | index of 5th LSF submatrix |
| subframe 1 |  |
| s39 - s47 | adaptive codebook index |
| s48 - s51 | adaptive codebook gain |
| s52 | sign information for 1st and 6th pulses |
| s53 - s55 | position of 1st pulse |
| s56 | sign information for 2nd and 7th pulses |
| s57 - s59 | position of 2nd pulse |
| s60 | sign information for 3rd and 8th pulses |
| s61 - s63 | position of 3rd pulse |
| s64 | sign information for 4th and 9th pulses |
| s65 - s67 | position of 4th pulse |
| s68 | sign information for 5th and 10th pulses |
| s69 - s71 | position of 5th pulse |
| s72 - s74 | position of 6th pulse |
| s75 - s77 | position of 7th pulse |
| s78 - s80 | position of 8th pulse |
| s81 - s83 | position of 9th pulse |
| s84 - s86 | position of 10th pulse |
| s87 - s91 | fixed codebook gain |
| subframe 2 |  |
| s92 - s97 | adaptive codebook index (relative) |
| s98 - s141 | same description as s48 - s91 |
| subframe 3 |  |
| s142 - s194 | same description as s39 - s91 |
| subframe 4 |  |
| s195 - s244 | same description as s92 - s141 |

Table 6: Ordering of enhanced full rate speech parameters for the channel encoder
(subjective importance of encoded bits) (after preliminary channel coding)
(Parameter names refers to 3GPP TS 46.060)

| Description | Bits (Table 5) | Bit index within parameter |
| --- | --- | --- |
| CLASS 1a: 50 bits (protected by 3 bit TCH-FS CRC) |  |  |
| LTP-LAG 1 | w39 - w44 | b8, b7, b6, b5, b4, b3 |
| LTP-LAG 3 | w146 - w151 | b8, b7, b6, b5, b4, b3 |
| LTP-LAG 2 | w94 - w95 | b5, b4 |
| LTP-LAG 4 | w201 - w202 | b5, b4 |
| LTP-GAIN 1 | n48 | b3 |
| FCB-GAIN 1 | w89 | b4 |
| LTP-GAIN 2 | w100 | b3 |
| FCB-GAIN 2 | w141 | b4 |
| LTP-LAG 1 | w45 | b2 |
| LTP-LAG 3 | w152 | b2 |
| LTP-LAG 2 | w96 | b3 |
| LTP-LAG 4 | w203 | b3 |
| LPC 1 | w2 - w3 | b5, b4 |
| LPC 2 | w8 | b7 |
| LPC 2 | w10 | b5 |
| LPC 3 | w18 - w19 | b6, b5 |
| LPC 3 | w24 | b0 |
| LTP-LAG 1 | w46 - w47 | b1, b0 |
| LTP-LAG 3 | w153 - w154 | b1, b0 |
| LTP-LAG 2 | w97 | b2 |
| LTP-LAG 4 | w204 | b2 |
| LPC 1 | w4 - w5 | b3, b2 |
| LPC 2 | w11 - w12 | b4, b3 |
| LPC 3 | w16 | b8 |
| LPC 2 | w9 | b6 |
| LPC 1 | w6 - w7 | b1, b0 |
| LPC 2 | w13 | b2 |
| LPC 3 | w17 | b7 |
| LPC 3 | w20 | b4 |
| LTP-LAG 2 | w98 | b1 |
| LTP-LAG 4 | w205 | b1 |
| CLASS 1b: 132 bits (protected) |  |  |
| LPC 1 | w1 | b6 |
| LPC 2 | w14 - w15 | b1, b0 |
| LPC 3 | w21 | b3 |
| LPC 4 | w25 - w26 | b7, b6 |
| LPC 4 | w28 | b4 |
| LTP-GAIN 3 | w155 | b3 |
| LTP-GAIN 4 | w207 | b3 |
| FCB-GAIN 3 | w196 | b4 |
| FCB-GAIN 4 | w248 | b4 |
| FCB-GAIN 1 | w90 | b3 |
| FCB-GAIN 2 | w142 | b3 |
| FCB-GAIN 3 | w197 | b3 |
| FCB-GAIN 4 | w249 | b3 |
| CRC-POLY | w253 - w260 | b7, b6, b5, b4, b3, b2, b1, b0 |
| LTP-GAIN 1 | w49 | b2 |
| (continued) |  |  |

Table 6 (continued): Ordering of enhanced full rate speech parameters for the channel encoder
(subjective importance of encoded bits) (after preliminary channel coding)
(Parameter names refers to 3GPP TS 46.060)

| Description | Bits (Table 5) | Bit index within parameter |
| --- | --- | --- |
| LTP-GAIN 2 | w101 | b2 |
| LTP-GAIN 3 | w156 | b2 |
| LTP-GAIN 4 | w208 | b2 |
| LPC 3 | w22 - w23 | b2, b1 |
| LPC 4 | w27 | b5 |
| LPC 4 | w29 | b3 |
| PULSE 1_1 | w52 | b3 |
| PULSE 1_2 | w56 | b3 |
| PULSE 1_3 | w60 | b3 |
| PULSE 1_4 | w64 | b3 |
| PULSE 1_5 | w68 | b3 |
| PULSE 2_1 | w104 | b3 |
| PULSE 2_2 | w108 | b3 |
| PULSE 2_3 | w112 | b3 |
| PULSE 2_4 | w116 | b3 |
| PULSE 2_5 | w120 | b3 |
| PULSE 3_1 | w159 | b3 |
| PULSE 3_2 | w163 | b3 |
| PULSE 3_3 | w167 | b3 |
| PULSE 3_4 | w171 | b3 |
| PULSE 3_5 | w175 | b3 |
| PULSE 4_1 | w211 | b3 |
| PULSE 4_2 | w215 | b3 |
| PULSE 4_3 | w219 | b3 |
| PULSE 4_4 | w223 | b3 |
| PULSE 4_5 | w227 | b3 |
| FCB-GAIN 1 | w91 | b2 |
| FCB-GAIN 2 | w143 | b2 |
| FCB-GAIN 3 | w198 | b2 |
| FCB-GAIN 4 | w250 | b2 |
| LTP-GAIN 1 | w50 | b1 |
| LTP-GAIN 2 | w102 | b1 |
| LTP-GAIN 3 | w157 | b1 |
| LTP-GAIN 4 | w209 | b1 |
| LPC 4 | w30 - w32 | b2, b1, b0 |
| LPC 5 | w33 - w36 | b5, b4, b3, b2 |
| LTP-LAG 2 | w99 | b0 |
| LTP-LAG 4 | w206 | b0 |
| PULSE 1_1 | w53 | b2 |
| PULSE 1_2 | w57 | b2 |
| (continued) |  |  |

Table 6 (continued): Ordering of enhanced full rate speech parameters for the channel encoder (subjective importance of encoded bits) (after preliminary channel coding)
(Parameter names refers to 3GPP TS 46.060)

| Description | Bits (Table 5) | Bit index within parameter |
| --- | --- | --- |
| PULSE 1_3 | w61 | b2 |
| PULSE 1_4 | w65 | b2 |
| PULSE 1_5 | w69 | b2 |
| PULSE 2_1 | w105 | b2 |
| PULSE 2_2 | w109 | b2 |
| PULSE 2_3 | w113 | b2 |
| PULSE 2_4 | w117 | b2 |
| PULSE 2_5 | w121 | b2 |
| PULSE 3_1 | w160 | b2 |
| PULSE 3_2 | w164 | b2 |
| PULSE 3_3 | w168 | b2 |
| PULSE 3_4 | w172 | b2 |
| PULSE 3_5 | w176 | b2 |
| PULSE 4_1 | w212 | b2 |
| PULSE 4_2 | w216 | b2 |
| PULSE 4_3 | w220 | b2 |
| PULSE 4_4 | w224 | b2 |
| PULSE 4_5 | w228 | b2 |
| PULSE 1_1 | w54 | b1 |
| PULSE 1_2 | w58 | b1 |
| PULSE 1_3 | w62 | b1 |
| PULSE 1_4 | w66 | b1 |
| PULSE 2_1 | w106 | b1 |
| PULSE 2_2 | w110 | b1 |
| PULSE 2_3 | w114 | b1 |
| PULSE 2_4 | w118 | b1 |
| PULSE 3_1 | w161 | b1 |
| PULSE 3_2 | w165 | b1 |
| PULSE 3_3 | w169 | b1 |
| PULSE 3_4 | w173 | b1 |
| PULSE 4_1 | w213 | b1 |
| PULSE 4_3 | w221 | b1 |
| PULSE 4_4 | w225 | b1 |
| FCB-GAIN 1 | w92 | b1 |
| FCB-GAIN 2 | w144 | b1 |
| FCB-GAIN 3 | s199 | b1 |
| FCB-GAIN 4 | w251 | b1 |
| LTP-GAIN 1 | w51 | b0 |
| LTP-GAIN 2 | w103 | b0 |
| LTP-GAIN 3 | w158 | b0 |
| LTP-GAIN 4 | w210 | b0 |
| FCB-GAIN 1 | w93 | b0 |
| FCB-GAIN 2 | w145 | b0 |
| FCB-GAIN 3 | w200 | b0 |
| FCB-GAIN 4 | w252 | b0 |
| PULSE 1_1 | w55 | b0 |
| PULSE 1_2 | w59 | b0 |
| PULSE 1_3 | w63 | b0 |
| PULSE 1_4 | w67 | b0 |
| PULSE 2_1 | w107 | b0 |
| PULSE 2_2 | w111 | b0 |
| PULSE 2_3 | w115 | b0 |
| PULSE 2_4 | w119 | b0 |
| PULSE 3_1 | w162 | b0 |
| PULSE 3_2 | w166 | b0 |
| PULSE 3_3 | w170 | b0 |
| (continued) |  |  |

Table 6 (continued): Ordering of enhanced full rate speech parameters for the channel encoder (subjective importance of encoded bits) (after preliminary channel coding)
(Parameter names refers to 3GPP TS 46.060)

| Description | Bits (Table 5) | Bit index within parameter |
| --- | --- | --- |
| PULSE 3_4 | w174 | b0 |
| PULSE 4_1 | w214 | b0 |
| PULSE 4_3 | w222 | b0 |
| PULSE 4_4 | w226 | b0 |
| LPC 5 | w37 - w38 | b1, b0 |
| CLASS 2: 78 bits (unprotected) |  |  |
| PULSE 1_5 | w70 | b1 |
| PULSE 1_5 | w72 - w73 | b1, b1 |
| PULSE 2_5 | w122 | b1 |
| PULSE 2_5 | w124 - s125 | b1, b1 |
| PULSE 3_5 | w177 | b1 |
| PULSE 3_5 | w179 - w180 | b1, b1 |
| PULSE 4_5 | w229 | b1 |
| PULSE 4_5 | w231 - w232 | b1, b1 |
| PULSE 4_2 | w217 - w218 | b1, b0 |
| PULSE 1_5 | w71 | b0 |
| PULSE 2_5 | w123 | b0 |
| PULSE 3_5 | w178 | b0 |
| PULSE 4_5 | w230 | b0 |
| PULSE 1_6 | w74 | b2 |
| PULSE 1_7 | w77 | b2 |
| PULSE 1_8 | w80 | b2 |
| PULSE 1_9 | w83 | b2 |
| PULSE 1_10 | w86 | b2 |
| PULSE 2_6 | w126 | b2 |
| PULSE 2_7 | w129 | b2 |
| PULSE 2_8 | w132 | b2 |
| PULSE 2_9 | w135 | b2 |
| PULSE 2_10 | w138 | b2 |
| PULSE 3_6 | w181 | b2 |
| PULSE 3_7 | w184 | b2 |
| PULSE 3_8 | w187 | b2 |
| PULSE 3_9 | w190 | b2 |
| PULSE 3_10 | w193 | b2 |
| PULSE 4_6 | w233 | b2 |
| PULSE 4_7 | w236 | b2 |
| PULSE 4_8 | w239 | b2 |
| PULSE 4_9 | w242 | b2 |
| PULSE 4_10 | w245 | b2 |
| PULSE 1_6 | w75 | b1 |
| PULSE 1_7 | w78 | b1 |
| PULSE 1_8 | w81 | b1 |
| PULSE 1_9 | w84 | b1 |
| PULSE 1_10 | w87 | b1 |
| PULSE 2_6 | w127 | b1 |
| PULSE 2_7 | w130 | b1 |
| PULSE 2_8 | w133 | b1 |
| PULSE 2_9 | w136 | b1 |
| PULSE 2_10 | w139 | b1 |
| PULSE 3_6 | w182 | b1 |
| PULSE 3_7 | w185 | b1 |
| PULSE 3_8 | w188 | b1 |
| PULSE 3_9 | w191 | b1 |
| PULSE 3_10 | w194 | b1 |
| PULSE 4_6 | w234 | b1 |
| PULSE 4_7 | w237 | b1 |
| (continued) |  |  |

Table 6 (concluded): Ordering of enhanced full rate speech parameters for the channel encoder (subjective importance of encoded bits) (after preliminary channel coding)
(Parameter names refers to 3GPP TS 46.060)

| Description | Bits (Table 5) | Bit index within parameter |
| --- | --- | --- |
| PULSE 4_8 | w240 | b1 |
| PULSE 4_9 | w243 | b1 |
| PULSE 4_10 | w246 | b1 |
| PULSE 1_6 | w76 | b0 |
| PULSE 1_7 | w79 | b0 |
| PULSE 1_8 | w82 | b0 |
| PULSE 1_9 | w85 | b0 |
| PULSE 1_10 | w88 | b0 |
| PULSE 2_6 | w128 | b0 |
| PULSE 2_7 | w131 | b0 |
| PULSE 2_8 | w134 | b0 |
| PULSE 2_9 | w137 | b0 |
| PULSE 2_10 | w140 | b0 |
| PULSE 3_6 | w183 | b0 |
| PULSE 3_7 | w186 | b0 |
| PULSE 3_8 | w189 | b0 |
| PULSE 3_9 | w192 | b0 |
| PULSE 3_10 | w195 | b0 |
| PULSE 4_6 | w235 | b0 |
| PULSE 4_7 | w238 | b0 |
| PULSE 4_8 | w241 | b0 |
| PULSE 4_9 | w244 | b0 |
| PULSE 4_10 | w247 | b0 |

Table 7: Sorting of the speech encoded bits for TCH/AFS12.2

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 23 | 15 | 16 | 17 | 18 |
| 19 | 20 | 21 | 22 | 24 | 25 | 26 | 27 | 28 | 38 |
| 141 | 39 | 142 | 40 | 143 | 41 | 144 | 42 | 145 | 43 |
| 146 | 44 | 147 | 45 | 148 | 46 | 149 | 47 | 97 | 150 |
| 200 | 48 | 98 | 151 | 201 | 49 | 99 | 152 | 202 | 86 |
| 136 | 189 | 239 | 87 | 137 | 190 | 240 | 88 | 138 | 191 |
| 241 | 91 | 194 | 92 | 195 | 93 | 196 | 94 | 197 | 95 |
| 198 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 50 | 100 |
| 153 | 203 | 89 | 139 | 192 | 242 | 51 | 101 | 154 | 204 |
| 55 | 105 | 158 | 208 | 90 | 140 | 193 | 243 | 59 | 109 |
| 162 | 212 | 63 | 113 | 166 | 216 | 67 | 117 | 170 | 220 |
| 36 | 37 | 54 | 53 | 52 | 58 | 57 | 56 | 62 | 61 |
| 60 | 66 | 65 | 64 | 70 | 69 | 68 | 104 | 103 | 102 |
| 108 | 107 | 106 | 112 | 111 | 110 | 116 | 115 | 114 | 120 |
| 119 | 118 | 157 | 156 | 155 | 161 | 160 | 159 | 165 | 164 |
| 163 | 169 | 168 | 167 | 173 | 172 | 171 | 207 | 206 | 205 |
| 211 | 210 | 209 | 215 | 214 | 213 | 219 | 218 | 217 | 223 |
| 222 | 221 | 73 | 72 | 71 | 76 | 75 | 74 | 79 | 78 |
| 77 | 82 | 81 | 80 | 85 | 84 | 83 | 123 | 122 | 121 |
| 126 | 125 | 124 | 129 | 128 | 127 | 132 | 131 | 130 | 135 |
| 134 | 133 | 176 | 175 | 174 | 179 | 178 | 177 | 182 | 181 |
| 180 | 185 | 184 | 183 | 188 | 187 | 186 | 226 | 225 | 224 |
| 229 | 228 | 227 | 232 | 231 | 230 | 235 | 234 | 233 | 238 |
| 237 | 236 | 96 | 199 |  |  |  |  |  |  |

Table 8: Sorting of the speech encoded bits for TCH/AFS10.2

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 16 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 13 | 12 | 11 | 10 | 9 | 8 | 26 | 27 | 28 |
| 29 | 30 | 31 | 115 | 116 | 117 | 118 | 119 | 120 | 72 |
| 73 | 161 | 162 | 65 | 68 | 69 | 108 | 111 | 112 | 154 |
| 157 | 158 | 197 | 200 | 201 | 32 | 33 | 121 | 122 | 74 |
| 75 | 163 | 164 | 66 | 109 | 155 | 198 | 19 | 23 | 21 |
| 22 | 18 | 17 | 20 | 24 | 25 | 37 | 36 | 35 | 34 |
| 80 | 79 | 78 | 77 | 126 | 125 | 124 | 123 | 169 | 168 |
| 167 | 166 | 70 | 67 | 71 | 113 | 110 | 114 | 159 | 156 |
| 160 | 202 | 199 | 203 | 76 | 165 | 81 | 82 | 92 | 91 |
| 93 | 83 | 95 | 85 | 84 | 94 | 101 | 102 | 96 | 104 |
| 86 | 103 | 87 | 97 | 127 | 128 | 138 | 137 | 139 | 129 |
| 141 | 131 | 130 | 140 | 147 | 148 | 142 | 150 | 132 | 149 |
| 133 | 143 | 170 | 171 | 181 | 180 | 182 | 172 | 184 | 174 |
| 173 | 183 | 190 | 191 | 185 | 193 | 175 | 192 | 176 | 186 |
| 38 | 39 | 49 | 48 | 50 | 40 | 52 | 42 | 41 | 51 |
| 58 | 59 | 53 | 61 | 43 | 60 | 44 | 54 | 194 | 179 |
| 189 | 196 | 177 | 195 | 178 | 187 | 188 | 151 | 136 | 146 |
| 153 | 134 | 152 | 135 | 144 | 145 | 105 | 90 | 100 | 107 |
| 88 | 106 | 89 | 98 | 99 | 62 | 47 | 57 | 64 | 45 |
| 63 | 46 | 55 | 56 |  |  |  |  |  |  |

Table 9: Sorting of the speech encoded bits for TCH/AFS7.95 and TCH/AHS7.95

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 14 | 16 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 12 | 13 | 15 | 11 | 17 | 20 | 22 | 24 | 23 |
| 19 | 18 | 21 | 56 | 88 | 122 | 154 | 57 | 89 | 123 |
| 155 | 58 | 90 | 124 | 156 | 52 | 84 | 118 | 150 | 53 |
| 85 | 119 | 151 | 27 | 93 | 28 | 94 | 29 | 95 | 30 |
| 96 | 31 | 97 | 61 | 127 | 62 | 128 | 63 | 129 | 59 |
| 91 | 125 | 157 | 32 | 98 | 64 | 130 | 1 | 0 | 25 |
| 26 | 33 | 99 | 34 | 100 | 65 | 131 | 66 | 132 | 54 |
| 86 | 120 | 152 | 60 | 92 | 126 | 158 | 55 | 87 | 121 |
| 153 | 117 | 116 | 115 | 46 | 78 | 112 | 144 | 43 | 75 |
| 109 | 141 | 40 | 72 | 106 | 138 | 36 | 68 | 102 | 134 |
| 114 | 149 | 148 | 147 | 146 | 83 | 82 | 81 | 80 | 51 |
| 50 | 49 | 48 | 47 | 45 | 44 | 42 | 39 | 35 | 79 |
| 77 | 76 | 74 | 71 | 67 | 113 | 111 | 110 | 108 | 105 |
| 101 | 145 | 143 | 142 | 140 | 137 | 133 | 41 | 73 | 107 |
| 139 | 37 | 69 | 103 | 135 | 38 | 70 | 104 | 136 |  |

Table 10: Sorting of the speech encoded bits for TCH/AFS7.4  and TCH/AHS7.4

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 15 | 16 | 26 | 87 | 27 |
| 88 | 28 | 89 | 29 | 90 | 30 | 91 | 51 | 80 | 112 |
| 141 | 52 | 81 | 113 | 142 | 54 | 83 | 115 | 144 | 55 |
| 84 | 116 | 145 | 58 | 119 | 59 | 120 | 21 | 22 | 23 |
| 17 | 18 | 19 | 31 | 60 | 92 | 121 | 56 | 85 | 117 |
| 146 | 20 | 24 | 25 | 50 | 79 | 111 | 140 | 57 | 86 |
| 118 | 147 | 49 | 78 | 110 | 139 | 48 | 77 | 53 | 82 |
| 114 | 143 | 109 | 138 | 47 | 76 | 108 | 137 | 32 | 33 |
| 61 | 62 | 93 | 94 | 122 | 123 | 41 | 42 | 43 | 44 |
| 45 | 46 | 70 | 71 | 72 | 73 | 74 | 75 | 102 | 103 |
| 104 | 105 | 106 | 107 | 131 | 132 | 133 | 134 | 135 | 136 |
| 34 | 63 | 95 | 124 | 35 | 64 | 96 | 125 | 36 | 65 |
| 97 | 126 | 37 | 66 | 98 | 127 | 38 | 67 | 99 | 128 |
| 39 | 68 | 100 | 129 | 40 | 69 | 101 | 130 |  |  |

Table 11: Sorting of the speech encoded bits for TCH/AFS6.7 and TCH/AHS6.7

| 0 | 1 | 4 | 3 | 5 | 6 | 13 | 7 | 2 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 11 | 15 | 12 | 14 | 10 | 28 | 82 | 29 | 83 |
| 27 | 81 | 26 | 80 | 30 | 84 | 16 | 55 | 109 | 56 |
| 110 | 31 | 85 | 57 | 111 | 48 | 73 | 102 | 127 | 32 |
| 86 | 51 | 76 | 105 | 130 | 52 | 77 | 106 | 131 | 58 |
| 112 | 33 | 87 | 19 | 23 | 53 | 78 | 107 | 132 | 21 |
| 22 | 18 | 17 | 20 | 24 | 25 | 50 | 75 | 104 | 129 |
| 47 | 72 | 101 | 126 | 54 | 79 | 108 | 133 | 46 | 71 |
| 100 | 125 | 128 | 103 | 74 | 49 | 45 | 70 | 99 | 124 |
| 42 | 67 | 96 | 121 | 39 | 64 | 93 | 118 | 38 | 63 |
| 92 | 117 | 35 | 60 | 89 | 114 | 34 | 59 | 88 | 113 |
| 44 | 69 | 98 | 123 | 43 | 68 | 97 | 122 | 41 | 66 |
| 95 | 120 | 40 | 65 | 94 | 119 | 37 | 62 | 91 | 116 |
| 36 | 61 | 90 | 115 |  |  |  |  |  |  |

Table 12: Sorting of the speech encoded bits for TCH/AFS5.9 and TCH/AHS5.9

| 0 | 1 | 4 | 5 | 3 | 6 | 7 | 2 | 13 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 9 | 11 | 12 | 14 | 10 | 16 | 28 | 74 | 29 |
| 75 | 27 | 73 | 26 | 72 | 30 | 76 | 51 | 97 | 50 |
| 71 | 96 | 117 | 31 | 77 | 52 | 98 | 49 | 70 | 95 |
| 116 | 53 | 99 | 32 | 78 | 33 | 79 | 48 | 69 | 94 |
| 115 | 47 | 68 | 93 | 114 | 46 | 67 | 92 | 113 | 19 |
| 21 | 23 | 22 | 18 | 17 | 20 | 24 | 111 | 43 | 89 |
| 110 | 64 | 65 | 44 | 90 | 25 | 45 | 66 | 91 | 112 |
| 54 | 100 | 40 | 61 | 86 | 107 | 39 | 60 | 85 | 106 |
| 36 | 57 | 82 | 103 | 35 | 56 | 81 | 102 | 34 | 55 |
| 80 | 101 | 42 | 63 | 88 | 109 | 41 | 62 | 87 | 108 |
| 38 | 59 | 84 | 105 | 37 | 58 | 83 | 104 |  |  |

Table 13: Sorting of the speech encoded bits for TCH/AFS5.15 and TCH/AHS5.15

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 15 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 12 | 11 | 10 | 9 | 8 | 23 | 24 | 25 | 26 |
| 27 | 46 | 65 | 84 | 45 | 44 | 43 | 64 | 63 | 62 |
| 83 | 82 | 81 | 102 | 101 | 100 | 42 | 61 | 80 | 99 |
| 28 | 47 | 66 | 85 | 18 | 41 | 60 | 79 | 98 | 29 |
| 48 | 67 | 17 | 20 | 22 | 40 | 59 | 78 | 97 | 21 |
| 30 | 49 | 68 | 86 | 19 | 16 | 87 | 39 | 38 | 58 |
| 57 | 77 | 35 | 54 | 73 | 92 | 76 | 96 | 95 | 36 |
| 55 | 74 | 93 | 32 | 51 | 33 | 52 | 70 | 71 | 89 |
| 90 | 31 | 50 | 69 | 88 | 37 | 56 | 75 | 94 | 34 |
| 53 | 72 | 91 |  |  |  |  |  |  |  |

Table 14: Sorting of the speech encoded bits for TCH/AFS4.75 and TCH/AHS4.75

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 15 | 23 | 24 | 25 | 26 |
| 27 | 28 | 48 | 49 | 61 | 62 | 82 | 83 | 47 | 46 |
| 45 | 44 | 81 | 80 | 79 | 78 | 17 | 18 | 20 | 22 |
| 77 | 76 | 75 | 74 | 29 | 30 | 43 | 42 | 41 | 40 |
| 38 | 39 | 16 | 19 | 21 | 50 | 51 | 59 | 60 | 63 |
| 64 | 72 | 73 | 84 | 85 | 93 | 94 | 32 | 33 | 35 |
| 36 | 53 | 54 | 56 | 57 | 66 | 67 | 69 | 70 | 87 |
| 88 | 90 | 91 | 34 | 55 | 68 | 89 | 37 | 58 | 71 |
| 92 | 31 | 52 | 65 | 86 |  |  |  |  |  |

Table 15: Interleaving table for MCS5 and MCS6:

| m\n | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 463 | 890 | 1038 | 220 | 371 | 795 | 946 | 582 | 733 | 1160 | 63 | 490 | 641 | 277 | 428 |
| 1 | 852 | 1003 | 185 | 333 | 1223 | 120 | 547 | 698 | 1122 | 28 | 915 | 1066 | 242 | 390 | 817 | 968 |
| 2 | 610 | 761 | 1185 | 85 | 512 | 660 | 305 | 453 | 880 | 1031 | 204 | 355 | 782 | 1242 | 148 | 575 |
| 3 | 723 | 1150 | 50 | 474 | 625 | 1088 | 267 | 418 | 845 | 993 | 169 | 320 | 1207 | 113 | 537 | 688 |
| 4 | 1115 | 12 | 902 | 1050 | 232 | 383 | 807 | 958 | 594 | 745 | 1172 | 75 | 502 | 653 | 289 | 440 |
| 5 | 864 | 1015 | 197 | 345 | 1235 | 132 | 559 | 710 | 1134 | 40 | 927 | 1078 | 254 | 402 | 829 | 980 |
| 6 | 159 | 622 | 773 | 1197 | 97 | 524 | 672 | 1099 | 5 | 465 | 892 | 1043 | 216 | 367 | 794 | 942 |
| 7 | 587 | 735 | 1162 | 62 | 486 | 637 | 279 | 430 | 857 | 1005 | 181 | 332 | 1219 | 125 | 549 | 700 |
| 8 | 1127 | 24 | 914 | 1062 | 244 | 395 | 819 | 970 | 606 | 757 | 1184 | 87 | 514 | 665 | 301 | 452 |
| 9 | 876 | 1027 | 209 | 357 | 784 | 1247 | 144 | 571 | 722 | 1146 | 52 | 479 | 627 | 1090 | 266 | 414 |
| 10 | 841 | 992 | 171 | 322 | 1209 | 109 | 536 | 684 | 1111 | 17 | 904 | 1055 | 228 | 379 | 806 | 954 |
| 11 | 599 | 747 | 1174 | 74 | 498 | 649 | 291 | 442 | 869 | 1017 | 193 | 344 | 1231 | 137 | 561 | 712 |
| 12 | 1139 | 36 | 926 | 1074 | 256 | 407 | 831 | 982 | 158 | 618 | 769 | 1196 | 99 | 526 | 677 | 1101 |
| 13 | 7 | 458 | 894 | 1033 | 227 | 363 | 802 | 941 | 577 | 740 | 1152 | 70 | 485 | 645 | 284 | 420 |
| 14 | 859 | 998 | 189 | 328 | 1215 | 127 | 542 | 702 | 1117 | 35 | 922 | 1061 | 246 | 385 | 824 | 960 |
| 15 | 605 | 765 | 1180 | 92 | 504 | 667 | 309 | 448 | 887 | 1023 | 211 | 350 | 786 | 1237 | 155 | 567 |
| 16 | 730 | 1145 | 54 | 469 | 632 | 1080 | 274 | 413 | 849 | 988 | 176 | 312 | 1202 | 117 | 532 | 695 |
| 17 | 1107 | 19 | 906 | 1045 | 239 | 375 | 814 | 953 | 589 | 752 | 1164 | 82 | 497 | 657 | 296 | 432 |
| 18 | 871 | 1010 | 201 | 340 | 1227 | 139 | 554 | 714 | 1129 | 47 | 934 | 1073 | 258 | 397 | 836 | 972 |
| 19 | 166 | 617 | 777 | 1192 | 104 | 516 | 679 | 1094 | 9 | 460 | 899 | 1035 | 223 | 362 | 798 | 937 |
| 20 | 579 | 742 | 1157 | 66 | 481 | 644 | 286 | 425 | 861 | 1000 | 188 | 324 | 1214 | 129 | 544 | 707 |
| 21 | 1119 | 31 | 918 | 1057 | 251 | 387 | 826 | 965 | 601 | 764 | 1176 | 94 | 509 | 669 | 308 | 444 |
| 22 | 883 | 1022 | 213 | 352 | 791 | 1239 | 151 | 566 | 726 | 1141 | 59 | 471 | 634 | 1085 | 270 | 409 |
| 23 | 848 | 984 | 178 | 317 | 1204 | 116 | 528 | 691 | 1106 | 21 | 911 | 1047 | 235 | 374 | 810 | 949 |
| 24 | 591 | 754 | 1169 | 78 | 493 | 656 | 298 | 437 | 873 | 1012 | 200 | 336 | 1226 | 141 | 556 | 719 |
| 25 | 1131 | 43 | 930 | 1069 | 263 | 399 | 838 | 977 | 162 | 613 | 776 | 1188 | 106 | 521 | 681 | 1096 |
| 26 | 2 | 462 | 889 | 1040 | 219 | 370 | 797 | 945 | 584 | 732 | 1159 | 65 | 489 | 640 | 276 | 427 |
| 27 | 854 | 1002 | 184 | 335 | 1222 | 122 | 546 | 697 | 1124 | 27 | 917 | 1065 | 241 | 392 | 816 | 967 |
| 28 | 609 | 760 | 1187 | 84 | 511 | 662 | 304 | 455 | 879 | 1030 | 206 | 354 | 781 | 1244 | 147 | 574 |
| 29 | 725 | 1149 | 49 | 476 | 624 | 1087 | 269 | 417 | 844 | 995 | 168 | 319 | 1206 | 112 | 539 | 687 |
| 30 | 1114 | 14 | 901 | 1052 | 231 | 382 | 809 | 957 | 596 | 744 | 1171 | 77 | 501 | 652 | 288 | 439 |
| 31 | 866 | 1014 | 196 | 347 | 1234 | 134 | 558 | 709 | 1136 | 39 | 929 | 1077 | 253 | 404 | 828 | 979 |
| 32 | 161 | 621 | 772 | 1199 | 96 | 523 | 674 | 1098 | 4 | 467 | 891 | 1042 | 218 | 366 | 793 | 944 |
| 33 | 586 | 737 | 1161 | 61 | 488 | 636 | 281 | 429 | 856 | 1007 | 180 | 331 | 1218 | 124 | 551 | 699 |
| 34 | 1126 | 26 | 913 | 1064 | 243 | 394 | 821 | 969 | 608 | 756 | 1183 | 89 | 513 | 664 | 300 | 451 |
| 35 | 878 | 1026 | 208 | 359 | 783 | 1246 | 146 | 570 | 721 | 1148 | 51 | 478 | 629 | 1089 | 265 | 416 |
| 36 | 840 | 991 | 173 | 321 | 1211 | 108 | 535 | 686 | 1110 | 16 | 903 | 1054 | 230 | 378 | 805 | 956 |
| 37 | 598 | 749 | 1173 | 73 | 500 | 648 | 293 | 441 | 868 | 1019 | 192 | 343 | 1230 | 136 | 563 | 711 |
| 38 | 1138 | 38 | 925 | 1076 | 255 | 406 | 833 | 981 | 157 | 620 | 768 | 1195 | 101 | 525 | 676 | 1103 |
| 39 | 6 | 457 | 896 | 1032 | 226 | 365 | 801 | 940 | 576 | 739 | 1154 | 69 | 484 | 647 | 283 | 422 |
| 40 | 858 | 997 | 191 | 327 | 1217 | 126 | 541 | 704 | 1116 | 34 | 921 | 1060 | 248 | 384 | 823 | 962 |
| 41 | 604 | 767 | 1179 | 91 | 506 | 666 | 311 | 447 | 886 | 1025 | 210 | 349 | 788 | 1236 | 154 | 569 |
| 42 | 729 | 1144 | 56 | 468 | 631 | 1082 | 273 | 412 | 851 | 987 | 175 | 314 | 1201 | 119 | 531 | 694 |
| 43 | 1109 | 18 | 908 | 1044 | 238 | 377 | 813 | 952 | 588 | 751 | 1166 | 81 | 496 | 659 | 295 | 434 |
| 44 | 870 | 1009 | 203 | 339 | 1229 | 138 | 553 | 716 | 1128 | 46 | 933 | 1072 | 260 | 396 | 835 | 974 |
| 45 | 165 | 616 | 779 | 1191 | 103 | 518 | 678 | 1093 | 11 | 459 | 898 | 1037 | 222 | 361 | 800 | 936 |
| 46 | 581 | 741 | 1156 | 68 | 480 | 643 | 285 | 424 | 863 | 999 | 187 | 326 | 1213 | 131 | 543 | 706 |
| 47 | 1121 | 30 | 920 | 1056 | 250 | 389 | 825 | 964 | 600 | 763 | 1178 | 93 | 508 | 671 | 307 | 446 |
| 48 | 882 | 1021 | 215 | 351 | 790 | 1241 | 150 | 565 | 728 | 1140 | 58 | 473 | 633 | 1084 | 272 | 408 |
| 49 | 847 | 986 | 177 | 316 | 1203 | 115 | 530 | 690 | 1105 | 23 | 910 | 1049 | 234 | 373 | 812 | 948 |
| 50 | 593 | 753 | 1168 | 80 | 492 | 655 | 297 | 436 | 875 | 1011 | 199 | 338 | 1225 | 143 | 555 | 718 |
| 51 | 1133 | 42 | 932 | 1068 | 262 | 401 | 837 | 976 | 164 | 612 | 775 | 1190 | 105 | 520 | 683 | 1095 |
| 52 | 1 | 464 | 888 | 1039 | 221 | 369 | 796 | 947 | 583 | 734 | 1158 | 64 | 491 | 639 | 278 | 426 |
| 53 | 853 | 1004 | 183 | 334 | 1221 | 121 | 548 | 696 | 1123 | 29 | 916 | 1067 | 240 | 391 | 818 | 966 |
| 54 | 611 | 759 | 1186 | 86 | 510 | 661 | 303 | 454 | 881 | 1029 | 205 | 356 | 780 | 1243 | 149 | 573 |
| 55 | 724 | 1151 | 48 | 475 | 626 | 1086 | 268 | 419 | 843 | 994 | 170 | 318 | 1208 | 111 | 538 | 689 |
| 56 | 1113 | 13 | 900 | 1051 | 233 | 381 | 808 | 959 | 595 | 746 | 1170 | 76 | 503 | 651 | 290 | 438 |
| 57 | 865 | 1016 | 195 | 346 | 1233 | 133 | 560 | 708 | 1135 | 41 | 928 | 1079 | 252 | 403 | 830 | 978 |
| 58 | 160 | 623 | 771 | 1198 | 98 | 522 | 673 | 1100 | 3 | 466 | 893 | 1041 | 217 | 368 | 792 | 943 |
| 59 | 585 | 736 | 1163 | 60 | 487 | 638 | 280 | 431 | 855 | 1006 | 182 | 330 | 1220 | 123 | 550 | 701 |
| 60 | 1125 | 25 | 912 | 1063 | 245 | 393 | 820 | 971 | 607 | 758 | 1182 | 88 | 515 | 663 | 302 | 450 |
| 61 | 877 | 1028 | 207 | 358 | 785 | 1245 | 145 | 572 | 720 | 1147 | 53 | 477 | 628 | 1091 | 264 | 415 |
| 62 | 842 | 990 | 172 | 323 | 1210 | 110 | 534 | 685 | 1112 | 15 | 905 | 1053 | 229 | 380 | 804 | 955 |
| 63 | 597 | 748 | 1175 | 72 | 499 | 650 | 292 | 443 | 867 | 1018 | 194 | 342 | 1232 | 135 | 562 | 713 |
| 64 | 1137 | 37 | 924 | 1075 | 257 | 405 | 832 | 983 | 156 | 619 | 770 | 1194 | 100 | 527 | 675 | 1102 |
| 65 | 8 | 456 | 895 | 1034 | 225 | 364 | 803 | 939 | 578 | 738 | 1153 | 71 | 483 | 646 | 282 | 421 |
| 66 | 860 | 996 | 190 | 329 | 1216 | 128 | 540 | 703 | 1118 | 33 | 923 | 1059 | 247 | 386 | 822 | 961 |
| 67 | 603 | 766 | 1181 | 90 | 505 | 668 | 310 | 449 | 885 | 1024 | 212 | 348 | 787 | 1238 | 153 | 568 |
| 68 | 731 | 1143 | 55 | 470 | 630 | 1081 | 275 | 411 | 850 | 989 | 174 | 313 | 1200 | 118 | 533 | 693 |
| 69 | 1108 | 20 | 907 | 1046 | 237 | 376 | 815 | 951 | 590 | 750 | 1165 | 83 | 495 | 658 | 294 | 433 |
| 70 | 872 | 1008 | 202 | 341 | 1228 | 140 | 552 | 715 | 1130 | 45 | 935 | 1071 | 259 | 398 | 834 | 973 |
| 71 | 167 | 615 | 778 | 1193 | 102 | 517 | 680 | 1092 | 10 | 461 | 897 | 1036 | 224 | 360 | 799 | 938 |
| 72 | 580 | 743 | 1155 | 67 | 482 | 642 | 287 | 423 | 862 | 1001 | 186 | 325 | 1212 | 130 | 545 | 705 |
| 73 | 1120 | 32 | 919 | 1058 | 249 | 388 | 827 | 963 | 602 | 762 | 1177 | 95 | 507 | 670 | 306 | 445 |
| 74 | 884 | 1020 | 214 | 353 | 789 | 1240 | 152 | 564 | 727 | 1142 | 57 | 472 | 635 | 1083 | 271 | 410 |
| 75 | 846 | 985 | 179 | 315 | 1205 | 114 | 529 | 692 | 1104 | 22 | 909 | 1048 | 236 | 372 | 811 | 950 |
| 76 | 592 | 755 | 1167 | 79 | 494 | 654 | 299 | 435 | 874 | 1013 | 198 | 337 | 1224 | 142 | 557 | 717 |
| 77 | 1132 | 44 | 931 | 1070 | 261 | 400 | 839 | 975 | 163 | 614 | 774 | 1189 | 107 | 519 | 682 | 1097 |

This table describes the interleaving applied to MCS-5 and MCS-6

di(j’) = dc(k’)	for	k’ = 0,1,...,1223

k’ = 16*m + n

The value of j’ for a given k is in the cell located in the row m and in the column n.

Table 16: Sorting of the speech encoded bits for 
TCH/WFS12.65, O-TCH/WFS12.65 and O-TCH/WHS12.65

| 0 | 4 | 6 | 93 | 143 | 196 | 246 | 7 | 5 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | 48 | 49 | 50 | 51 | 150 | 151 | 152 | 153 | 154 |
| 94 | 144 | 197 | 247 | 99 | 149 | 202 | 252 | 96 | 146 |
| 199 | 249 | 97 | 147 | 200 | 250 | 100 | 203 | 98 | 148 |
| 201 | 251 | 95 | 145 | 198 | 248 | 52 | 2 | 1 | 101 |
| 204 | 155 | 19 | 21 | 12 | 17 | 18 | 20 | 16 | 25 |
| 13 | 10 | 14 | 24 | 23 | 22 | 26 | 8 | 15 | 53 |
| 156 | 31 | 102 | 205 | 9 | 33 | 11 | 103 | 206 | 54 |
| 157 | 28 | 27 | 104 | 207 | 34 | 35 | 29 | 46 | 32 |
| 30 | 55 | 158 | 37 | 36 | 39 | 38 | 40 | 105 | 208 |
| 41 | 42 | 43 | 44 | 45 | 56 | 106 | 159 | 209 | 57 |
| 66 | 75 | 84 | 107 | 116 | 125 | 134 | 160 | 169 | 178 |
| 187 | 210 | 219 | 228 | 237 | 58 | 108 | 161 | 211 | 62 |
| 112 | 165 | 215 | 67 | 117 | 170 | 220 | 71 | 121 | 174 |
| 224 | 76 | 126 | 179 | 229 | 80 | 130 | 183 | 233 | 85 |
| 135 | 188 | 238 | 89 | 139 | 192 | 242 | 59 | 109 | 162 |
| 212 | 63 | 113 | 166 | 216 | 68 | 118 | 171 | 221 | 72 |
| 122 | 175 | 225 | 77 | 127 | 180 | 230 | 81 | 131 | 184 |
| 234 | 86 | 136 | 189 | 239 | 90 | 140 | 193 | 243 | 60 |
| 110 | 163 | 213 | 64 | 114 | 167 | 217 | 69 | 119 | 172 |
| 222 | 73 | 123 | 176 | 226 | 78 | 128 | 181 | 231 | 82 |
| 132 | 185 | 235 | 87 | 137 | 190 | 240 | 91 | 141 | 194 |
| 244 | 61 | 111 | 164 | 214 | 65 | 115 | 168 | 218 | 70 |
| 120 | 173 | 223 | 74 | 124 | 177 | 227 | 79 | 129 | 182 |
| 232 | 83 | 133 | 186 | 236 | 88 | 138 | 191 | 241 | 92 |
| 142 | 195 | 245 |  |  |  |  |  |  |  |

Table 17: Sorting of the speech encoded bits for 
TCH/WFS8.85, O-TCH/WFS8.85 and O-TCH/WHS8.85

| 0 | 4 | 6 | 7 | 5 | 3 | 47 | 48 | 49 | 112 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 113 | 114 | 75 | 106 | 140 | 171 | 80 | 111 | 145 | 176 |
| 77 | 108 | 142 | 173 | 78 | 109 | 143 | 174 | 79 | 110 |
| 144 | 175 | 76 | 107 | 141 | 172 | 50 | 115 | 51 | 2 |
| 1 | 81 | 116 | 146 | 19 | 21 | 12 | 17 | 18 | 20 |
| 16 | 25 | 13 | 10 | 14 | 24 | 23 | 22 | 26 | 8 |
| 15 | 52 | 117 | 31 | 82 | 147 | 9 | 33 | 11 | 83 |
| 148 | 53 | 118 | 28 | 27 | 84 | 149 | 34 | 35 | 29 |
| 46 | 32 | 30 | 54 | 119 | 37 | 36 | 39 | 38 | 40 |
| 85 | 150 | 41 | 42 | 43 | 44 | 45 | 55 | 60 | 65 |
| 70 | 86 | 91 | 96 | 101 | 120 | 125 | 130 | 135 | 151 |
| 156 | 161 | 166 | 56 | 87 | 121 | 152 | 61 | 92 | 126 |
| 157 | 66 | 97 | 131 | 162 | 71 | 102 | 136 | 167 | 57 |
| 88 | 122 | 153 | 62 | 93 | 127 | 158 | 67 | 98 | 132 |
| 163 | 72 | 103 | 137 | 168 | 58 | 89 | 123 | 154 | 63 |
| 94 | 128 | 159 | 68 | 99 | 133 | 164 | 73 | 104 | 138 |
| 169 | 59 | 90 | 124 | 155 | 64 | 95 | 129 | 160 | 69 |
| 100 | 134 | 165 | 74 | 105 | 139 | 170 |  |  |  |

Table 18: Sorting of the speech encoded bits for 
TCH/WFS6.60, O-TCH/WFS6.60 and O-TCH/WHS6.60

| 0 | 5 | 6 | 7 | 61 | 84 | 107 | 130 | 62 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 4 | 37 | 38 | 39 | 40 | 58 | 81 | 104 | 127 |
| 60 | 83 | 106 | 129 | 108 | 131 | 128 | 41 | 42 | 80 |
| 126 | 1 | 3 | 57 | 103 | 82 | 105 | 59 | 2 | 63 |
| 109 | 110 | 86 | 19 | 22 | 23 | 64 | 87 | 18 | 20 |
| 21 | 17 | 13 | 88 | 43 | 89 | 65 | 111 | 14 | 24 |
| 25 | 26 | 27 | 28 | 15 | 16 | 44 | 90 | 66 | 112 |
| 9 | 11 | 10 | 12 | 67 | 113 | 29 | 30 | 31 | 32 |
| 34 | 33 | 35 | 36 | 45 | 51 | 68 | 74 | 91 | 97 |
| 114 | 120 | 46 | 69 | 92 | 115 | 52 | 75 | 98 | 121 |
| 47 | 70 | 93 | 116 | 53 | 76 | 99 | 122 | 48 | 71 |
| 94 | 117 | 54 | 77 | 100 | 123 | 49 | 72 | 95 | 118 |
| 55 | 78 | 101 | 124 | 50 | 73 | 96 | 119 | 56 | 79 |
| 102 | 125 |  |  |  |  |  |  |  |  |

Table 19: Sorting of the speech encoded bits for O-TCH/WFS15.85

| 0 | 4 | 6 | 109 | 175 | 244 | 310 | 7 | 5 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | 48 | 49 | 50 | 51 | 182 | 183 | 184 | 185 | 186 |
| 110 | 176 | 245 | 311 | 115 | 181 | 250 | 316 | 112 | 178 |
| 247 | 313 | 113 | 179 | 248 | 314 | 116 | 251 | 114 | 180 |
| 249 | 315 | 111 | 177 | 246 | 312 | 52 | 2 | 1 | 117 |
| 252 | 187 | 19 | 21 | 12 | 17 | 18 | 20 | 16 | 25 |
| 13 | 10 | 14 | 24 | 23 | 22 | 26 | 8 | 15 | 53 |
| 188 | 31 | 118 | 253 | 9 | 33 | 11 | 119 | 254 | 54 |
| 189 | 28 | 27 | 120 | 255 | 34 | 35 | 29 | 46 | 32 |
| 30 | 55 | 190 | 37 | 36 | 39 | 38 | 40 | 121 | 256 |
| 41 | 42 | 43 | 44 | 45 | 56 | 122 | 191 | 257 | 63 |
| 129 | 198 | 264 | 76 | 142 | 211 | 277 | 89 | 155 | 224 |
| 290 | 102 | 168 | 237 | 303 | 57 | 123 | 192 | 258 | 70 |
| 136 | 205 | 271 | 83 | 149 | 218 | 284 | 96 | 162 | 231 |
| 297 | 62 | 128 | 197 | 263 | 75 | 141 | 210 | 276 | 88 |
| 154 | 223 | 289 | 101 | 167 | 236 | 302 | 58 | 124 | 193 |
| 259 | 71 | 137 | 206 | 272 | 84 | 150 | 219 | 285 | 97 |
| 163 | 232 | 298 | 59 | 125 | 194 | 260 | 64 | 130 | 199 |
| 265 | 67 | 133 | 202 | 268 | 72 | 138 | 207 | 273 | 77 |
| 143 | 212 | 278 | 80 | 146 | 215 | 281 | 85 | 151 | 220 |
| 286 | 90 | 156 | 225 | 291 | 93 | 159 | 228 | 294 | 98 |
| 164 | 233 | 299 | 103 | 169 | 238 | 304 | 106 | 172 | 241 |
| 307 | 60 | 126 | 195 | 261 | 65 | 131 | 200 | 266 | 68 |
| 134 | 203 | 269 | 73 | 139 | 208 | 274 | 78 | 144 | 213 |
| 279 | 81 | 147 | 216 | 282 | 86 | 152 | 221 | 287 | 91 |
| 157 | 226 | 292 | 94 | 160 | 229 | 295 | 99 | 165 | 234 |
| 300 | 104 | 170 | 239 | 305 | 107 | 173 | 242 | 308 | 61 |
| 127 | 196 | 262 | 66 | 132 | 201 | 267 | 69 | 135 | 204 |
| 270 | 74 | 140 | 209 | 275 | 79 | 145 | 214 | 280 | 82 |
| 148 | 217 | 283 | 87 | 153 | 222 | 288 | 92 | 158 | 227 |
| 293 | 95 | 161 | 230 | 296 | 100 | 166 | 235 | 301 | 105 |
| 171 | 240 | 306 | 108 | 174 | 243 | 309 |  |  |  |

Table 20: Sorting of the speech encoded bits for O-TCH/WFS23.85

| 0 | 4 | 6 | 145 | 251 | 360 | 466 | 7 | 5 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | 48 | 49 | 50 | 51 | 262 | 263 | 264 | 265 | 266 |
| 146 | 252 | 361 | 467 | 151 | 257 | 366 | 472 | 148 | 254 |
| 363 | 469 | 149 | 255 | 364 | 470 | 156 | 371 | 150 | 256 |
| 365 | 471 | 147 | 253 | 362 | 468 | 52 | 2 | 1 | 157 |
| 372 | 267 | 19 | 21 | 12 | 17 | 18 | 20 | 16 | 25 |
| 13 | 10 | 14 | 24 | 23 | 22 | 26 | 8 | 15 | 53 |
| 268 | 31 | 152 | 153 | 154 | 155 | 258 | 259 | 260 | 261 |
| 367 | 368 | 369 | 370 | 473 | 474 | 475 | 476 | 158 | 373 |
| 9 | 33 | 11 | 159 | 374 | 54 | 269 | 28 | 27 | 160 |
| 375 | 34 | 35 | 29 | 46 | 32 | 30 | 55 | 270 | 37 |
| 36 | 39 | 38 | 40 | 161 | 376 | 41 | 42 | 43 | 44 |
| 45 | 56 | 162 | 271 | 377 | 185 | 196 | 174 | 79 | 57 |
| 411 | 90 | 163 | 305 | 389 | 378 | 283 | 68 | 187 | 400 |
| 294 | 198 | 307 | 92 | 70 | 186 | 413 | 176 | 59 | 91 |
| 58 | 412 | 380 | 165 | 81 | 164 | 272 | 175 | 80 | 401 |
| 402 | 390 | 391 | 197 | 306 | 69 | 274 | 273 | 379 | 285 |
| 296 | 284 | 295 | 188 | 60 | 199 | 82 | 93 | 71 | 381 |
| 414 | 177 | 166 | 456 | 308 | 403 | 98 | 76 | 286 | 61 |
| 275 | 386 | 135 | 423 | 171 | 102 | 392 | 204 | 87 | 182 |
| 65 | 94 | 208 | 124 | 72 | 350 | 193 | 313 | 393 | 408 |
| 445 | 309 | 230 | 419 | 297 | 241 | 113 | 219 | 189 | 128 |
| 317 | 415 | 116 | 328 | 200 | 339 | 382 | 434 | 178 | 64 |
| 404 | 83 | 437 | 223 | 134 | 192 | 444 | 112 | 439 | 139 |
| 287 | 167 | 448 | 212 | 459 | 222 | 240 | 233 | 97 | 302 |
| 397 | 234 | 170 | 276 | 181 | 455 | 229 | 438 | 101 | 280 |
| 138 | 127 | 298 | 117 | 355 | 203 | 426 | 95 | 140 | 244 |
| 422 | 407 | 213 | 129 | 291 | 354 | 105 | 245 | 449 | 86 |
| 316 | 460 | 207 | 353 | 190 | 107 | 224 | 427 | 342 | 327 |
| 106 | 321 | 118 | 123 | 73 | 211 | 433 | 218 | 396 | 385 |
| 450 | 62 | 383 | 349 | 75 | 461 | 172 | 331 | 168 | 246 |
| 428 | 332 | 312 | 201 | 343 | 416 | 279 | 63 | 195 | 333 |
| 96 | 173 | 235 | 288 | 320 | 191 | 418 | 84 | 205 | 100 |
| 67 | 394 | 179 | 344 | 206 | 338 | 277 | 405 | 388 | 395 |
| 301 | 315 | 421 | 183 | 293 | 322 | 310 | 384 | 410 | 194 |
| 184 | 89 | 99 | 103 | 236 | 78 | 88 | 77 | 136 | 399 |
| 169 | 202 | 406 | 125 | 180 | 440 | 74 | 387 | 242 | 231 |
| 66 | 281 | 290 | 141 | 314 | 424 | 114 | 85 | 130 | 356 |
| 119 | 299 | 304 | 398 | 237 | 409 | 311 | 417 | 292 | 457 |
| 435 | 225 | 214 | 209 | 462 | 108 | 282 | 446 | 220 | 351 |
| 345 | 142 | 247 | 329 | 420 | 463 | 318 | 300 | 120 | 109 |
| 289 | 451 | 278 | 441 | 340 | 303 | 430 | 215 | 323 | 226 |
| 334 | 131 | 442 | 248 | 335 | 357 | 429 | 324 | 143 | 346 |
| 452 | 238 | 110 | 216 | 464 | 249 | 121 | 431 | 358 | 227 |
| 132 | 453 | 336 | 425 | 325 | 347 | 126 | 104 | 137 | 458 |
| 352 | 243 | 447 | 115 | 341 | 210 | 330 | 221 | 232 | 436 |
| 465 | 319 | 359 | 111 | 454 | 228 | 217 | 122 | 443 | 348 |
| 239 | 250 | 133 | 144 | 432 | 337 | 326 |  |  |  |

# 6	Flexible Layer One

## 6.1	General

Data stream from higher layers (transport blocks) is encoded to offer transport services over the radio transmission link. The coding/multiplexing unit of FLO is a combination of error detection, forward error correction, rate matching, multiplexing, interleaving and burst mapping onto basic physical subchannel.

## 6.2	Transport channel coding/multiplexing

On transport channels, data arrives to the coding/multiplexing unit in form of transport blocks (TB) once every transmission time interval (TTI).

The following coding/multiplexing steps can be identified:

-	add CRC to each transport block (see subclause 6.2.1);

-	channel coding (see subclause 6.2.2);

-	rate matching (see subclause 6.2.3);

-	multiplexing of transport channels (see subclause 6.2.4);

-	TFCI encoding (see subclause 6.2.5);

-	(downlink only) mapping of in-band signalling bits (see subclause 6.2.6);

-	radio packet mapping (see subclause 6.2.7);

-	interleaving (see subclause 6.2.8);

-	mapping on a burst (see subclause 6.2.9).

The coding/multiplexing steps are shown in figure 3 below.

Figure 3: Transport channel coding/multiplexing

### 6.2.1	CRC Attachment

Error detection is provided on transport blocks through a Cyclic Redundancy Check (CRC). The size of the CRC to be used is 18, 12, 6 or 0 bits and it is configured by higher layers for each TrCH.

Transport blocks are delivered to the CRC attachment block. They are denoted by di,1,di,2,di,3,...,di,Di where i is the TrCH number and Di is the number of bits in the transport block.

The whole transport block is used to calculate the CRC parity bits. The parity bits are generated by one of the following cyclic generator polynomials:

-	gCRC18(D) = D18 + D17 + D14 + D13 + D11 + D10 + D8 + D7 + D6 + D3 + D2 + 1	same as for SACCH/TP

-	gCRC12(D) = D12 + D11 + D10 + D8 + D5 + D4 + 1	same as for MCS-1

-	gCRC6(D) = D6 + D5 + D3 + D2 + D1 + 1			same as for TCH/AFS

Denote the parity bits by pi,1,pi,2,pi,3,...,pi,Li. Li is the number of parity bits (size of the CRC) and can take the values 18, 12, 6, or 0.

The encoding is performed in a systematic form, which means that in GF(2), the polynomial:

di,1DDi+Li−1+di,2DDi+Li−2+...+di,DiDLi+pi,1DLi−1+pi,2DLi−2+...+pi,Li−1D1+pi,Li

-	when divided by gCRC18 yields a remainder equal to:

1 + D + D2 + D3 + D4 + D5 + D6 + D7 + D8 + D9 + D10 + D11 + D12 + D13 + D14 + D15 + D16 + D17

-	when divided by gCRC12 yields a remainder equal to:

1 + D + D2 + D3 + D4 + D5 + D6 + D7 + D8 + D9 + D10 + D11 + D12

-	when divided by gCRC6 yields a remainder equal to:

1 + D + D2 + D3 + D4 + D5

The result of CRC attachment is a code block of Ui bits ui,1,ui,2,ui,3,...,ui,Ui where:

Ui=Di+Li

ui,k=di,k				for k = 1,2,3,…, Di

ui,k=pi,(Li+1−(k−Di))		for k = Di+1, Di+2,…, Di+Li

If no transport blocks are input to the CRC calculation, no CRC attachment shall be done.

### 6.2.2	Channel Coding

Code blocks are delivered to the channel coding block. They are denoted by ui,1,ui,2,ui,3,...,ui,Ui where i is the TrCH number and Ui is the number of bits in the code block. After channel coding the bits are denoted by ci,1,ci,2,ci,3,...,ci,Ci(encoded blocks) where Ci is the number of encoded bits.

Before convolutional coding 6 tail bits with binary value 0 are added to the end of the code block:

ui,1,ui,2,ui,3,...,ui,Ui,0,0,0,0,0,0

The block is then encoded with the same 1/3 rate convolutional code as for MCS-1, defined by the following polynomials:

G4 = 1 + D2  + D3  + D5  + D6

G7 = 1 + D  + D2  + D3  + D6

G5 = 1 + D  + D4  + D6

resulting in an encoded block of Ci bits {ci,1,ci,2,ci,3,...,ci,Ci} with:

Ci=3×Ui+6

ci,3k+1=ui,k+1+ui,(k−1)+ui,(k−2)+ui,(k−4)+ui,(k−5) ;

ci,(3k+2)=ui,k+1+ui,k+ui,(k−1)+ui,(k−2)+ui,(k−5) ;

ci,(3k+3)=ui,k+1+ui,k+ui,(k−3)+ui,(k−5)	for k = 0,1,…, Ui + 5 and ui,k = 0 for k < 1.

### 6.2.3	Rate Matching

Rate matching means that bits of an encoded block on a transport channel are repeated or punctured. The number of bits on a transport channel can vary between different transmission time intervals. When the number of bits between different transmission time intervals is changed, bits are repeated or punctured to ensure that the total bit rate after TrCH multiplexing is identical to the total channel bit rate of the allocated dedicated basic physical subchannel.

Higher layers assign a rate-matching attribute to each transport channel. The rate matching attribute is used to calculate the number of bits to be repeated or punctured.

The input bit sequences before rate matching (encoded blocks) are denoted by ci,1,ci,2,ci,3,...,ci,Ci where i is the TrCH number and Ci is the number of bits. Only one radio frame per TrCH is delivered to the rate matching block.

Notation used:

x		Round x towards -, i.e. integer such that x−1<x≤x.

x		Absolute value of x.

I	Number of TrCHs in the coded composite transport channel (CCTrCH).

Ndata		Total number of bits that are available in a radio packet for the CCTrCH.

Ni,j		Number of bits in an encoded block before rate matching on TrCH i with transport format combination j.

ΔNi,j		If positive, ΔNi,j denotes the number of bits that have to be repeated in an encoded block on TrCH i with transport format combination j in order to produce a radio frame.

If negative,ΔNi,j denotes the number of bits that have to be punctured in an encoded block on TrCH i with transport format combination j in order to produce a radio frame.

If null, no bits have to be punctured nor repeated, i.e. the rate matching is transparent and the content of the radio frame is identical to the content of the encoded block on TrCH i with transport format combination j.

RMi	Semi-static rate matching attribute for transport channel i.

eini	Initial value of variable e in the rate matching pattern determination algorithm.

eplus	Increment of variable e in the rate matching pattern determination algorithm.

eminus	Decrement value of variable e in the rate matching pattern determination algorithm.

Zi,j	Intermediate calculation variable.

R	Redundancy pattern index used for the transmission of signalling transport blocks on half rate channels (see subclause 6.2.10). In all other cases R = 0.

For each radio packet using transport format combination j, the number of bits to be repeated or punctured Ni,j within one encoded block for each TrCH i is calculated with the following equations:

Z0,j=0

Zi,j=m=1iRMm×Nm,j×Ndatam=1IRMm×Nm,j	for all i = 1 … I

ΔNi,j=Zi,j−Zi−1,j−Ni,j	for all i = 1 … I

For the calculation of the rate matching pattern of each TrCH i the following relations are defined:

eplus  = 2×Ni,j

eminus  = 2×ΔNi,j

if ΔNi,j < 0

if  epluseminus≥2

d=epluseminus=Ni,jΔNi,j						-- average distance between punctured bits

eini=1+(Rmodd)×eminus

else

d=epluseplus−eminus=Ni,jNi,j−ΔNi,j	-- average distance between transmitted bits

eini=1+(Rmodd)×eplus−eminus

end if

else  eini  = 1

end if.

The rate matching rule is as follows:

if ΔNi,j < 0  				-- puncturing is to be performed

e = eini						-- initial error between current and desired puncturing ratio

m = 1						-- index of current bit

do while m≤Ni,j		-- for each bit of the encoded block of TrCHi

e = e – eminus			-- update error

if e≤0 then			-- check if bit number m should be punctured

puncture bit bim	-- bit is punctured

e = e + eplus		-- update error

end if

m = m + 1				--  next bit

end do

else if ΔNi,j > 0				-- repetition is to be performed

e = eini						-- initial error between current and desired puncturing ratio

m = 1						-- index of current bit

do while m≤Ni,j		-- for each bit of the encoded block of TrCHi

e = e – eminus			-- update error

do while e≤0		-- check if bit number m should be repeated

repeat bit bi,m		-- repeat bit

e = e + eplus		-- update error

end do

m = m + 1				--  next bit

end do

else							-- ΔNi,j = 0

do nothing				-- no repetition nor puncturing

end if.

For each TrCH i, the bit sequences output from the rate matching are denoted fi,1,fi,2,fi,3,…,fi,Vi, where i is the TrCH number and Vi is the number of bits in the radio frame of TrCH i (Vi=Ni,j+ΔNi,j).

### 6.2.4	Transport Channel multiplexing

For every transmitted radio packet, one radio frame from each active TrCH is delivered to the TrCH multiplexing. These radio frames are serially multiplexed into a coded composite transport channel (CCTrCH).

The input bit sequences to the TrCH multiplexing are denoted by fi,1,fi,2,fi,3,…,fi,Vi where i is the TrCH number and Vi is the number of bits in the radio frame of TrCH i. If TrCHi is inactive, Vi = 0. The number of TrCHs is denoted by I. The bits output from TrCH multiplexing are denoted m1,m2,m3,…,mNdata where Ndata is the total number of bits that are available in a radio packet for the CCTrCH, i.e. Ndata=iVi.

The TrCH multiplexing is defined by the following relations:

mk=f1,k			for k = 1,2,…,V1

mk=f2,(k−V1)		for k = V1 + 1,V1 + 2,…, V1 + V2

…

mk=fI,(k−(V1+V2+...+VI−1))	for k = (V1 + V2 + … + VI-1) + 1, (V1 + V2 + … + VI-1) + 2,…, (V1 + V2 + … + VI-1) + VI

NOTE:	when I = 1, the TrCH multiplexing block is transparent for the only radio frame of the only transport channel i and consequently the output bit sequence is identical to the input one.

### 6.2.5	TFCI Encoding

The TFCI informs the receiver about the transport format combination of the CCTrCH. As soon as the TFCI is detected, the transport format combination, and hence the transport formats of the individual transport channels are known. The size and values of the TFCI to be used on basic physical subchannels are configured by higher layers. The value of the TFCI can vary between different transmission time intervals. The size of the TFCI can only be changed through higher layer signalling.

The TFCI bit sequence is denoted by a1,...,aNTFCI with NTFCI∈1,2,3,4,5.

The TFCI information bits are first block coded. The coded TFCI bit sequence is denoted by b1,b2,b3,...,bNCTFCI with NCTFCI∈4,8,12,14,16,18,24,28,32,36,48,56,72. The block coding is done according to the following rules:

On GMSK full rate channels and 8PSK half rate channels, the coding of the TFCI shall be as follows:

-	1 bit TFCI shall be encoded to 8 bits according to Table 25;

-	2 bits TFCI shall be encoded to 16 bits according to Table 24;

-	3 bits TFCI shall be encoded to 24 bits according to Table 23;

-	4 bits TFCI shall be encoded to 28 bits according to Table 22;

-	5 bits TFCI shall be encoded to 36 bits according to Table 21.

On 8PSK full rate channels, the coding of the TFCI shall be obtained by repetition of the coding defined for GMSK full rate channels:

-	1 bit TFCI shall be encoded to 16 bits (concatenation of two identical coded sequences of 8 bits);

-	2 bits TFCI shall be encoded to 32 bits (concatenation of two identical coded sequences of 16 bits);

-	3 bits TFCI shall be encoded to 48 bits (concatenation of two identical coded sequences of 24 bits);

-	4 bits TFCI shall be encoded to 56 bits (concatenation of two identical coded sequences of 28 bits);

-	5 bits TFCI shall be encoded to 72 bits (concatenation of two identical coded sequences of 36 bits).

On GMSK half rate channels, the coding of the TFCI shall be obtained by using only the middle segment of the coding defined for GMSK full rate channels:

-	1 bit TFCI shall be encoded to 4 bits;

-	2 bits TFCI shall be encoded to 8 bits;

-	3 bits TFCI shall be encoded to 12 bits;

-	4 bits TFCI shall be encoded to 14 bits;

-	5 bits TFCI shall be encoded to 18 bits.

### 6.2.6	In-band signalling encoding

The in-band signalling bits are transmitted in the downlink direction only. The information contained in these bits is a TFCI sequence w1,w2,...,wNTFCIU. The number of in-band signalling bits in each radio packet, NTFCIU, is equal to the size of the uplink TFCI. The number of coded in-band signalling bits in each radio packet, NCINBAND, is equal to the size of the coded uplink TFCI (see subclause 6.2.5).

The coded in-band signalling bit sequence is denoted v1,v2,...,vNCINBAND.

### 6.2.7	Radio packet mapping

The input data bit sequence is denoted by m1,m2,m3,…,mNdata where Ndata  is the total number of bits that are available in a radio packet for the CCTrCH. After mapping on a radio packet the bits are denoted by h0,h1,h2,...,hNradio−1 where Nradio is the total number of bits that are available in a radio packet:

On GMSK full rate channels, Nradio = 464;

On GMSK half rate channels, Nradio = 232;

On 8PSK full rate channels, Nradio = 1392;

On 8PSK half rate channels, Nradio = 696.

The result of the radio packet mapping is a radio packet of Nradio bits {h0,h1,h2,...,hNradio−1} where:

-	in the uplink:

Nradio=Ndata+NCTFCI

hk=bk+1				for k = 0,1,2,…, NCTFCI – 1

hk=mk−NCTFCI+1		for k = NCTFCI, NCTFCI + 1, …, Nradio – 1

-	in the downlink:

Nradio=Ndata+NCTFCI+NCINBAND

hk=bk+1						for k = 0,1,2,…,NCTFCI – 1

hk=vk−NCTFCI+1					for k = NCTFCI, NCTFCI + 1, …, NCTFCI + NCINBAND – 1

hk=mk−NCTFCI−NCINBAND+1		for k = NCTFCI + NCINBAND, NCTFCI + NCINBAND + 1, …, Nradio –1

### 6.2.8	Interleaving

The interleaving type (block rectangular, block diagonal) and interleaving depth are configured by higher layers. The input bit sequence to the interleaving is denoted by h0,h1,h2,...,hNradio−1 where Nradio is the total number of bits that are available in a radio packet. Interleaved bits are noted i(B,jk) where B denotes the burst number and jk the position of the bit within the burst.

The interleaving for the nth radio packet is based on the following equations:

i(B,jk) = hk		for k = 0,1,2, …Nradio-1

n = 0,1,…,N,N+1,…

B=B0+4n+kmodD

for block diagonal interleaving:

a=GCDD,NradioD

if   a > 1    then s=intkNradioa

else s = 0

jk=DM×49×k+smodJDM+intkmodDM

for block rectangular interleaving:

a=GCD2D,Nradio2D

if    a > 1   then s=intkNradioa

else s = 0

jk=2DM×49×k+smodJ2DM+intkmod2DM

where:

jk	is the position of the bit k within the burst B;

D	is the interleaving depth in bursts;

J	is the burst size in bits (J = Nradio / M);

M	is the size of the radio packet in bursts (M = 4 for full rate channels, M = 2 for half rate channels);

GCD(m,n) is the greatest common divisor of m and n.

On 8PSK channels, bit swapping for the coded bits of the TFCI is performed:

cpt = 0										-- counter of the swapped bits

for k = 0,1,2,3,…,NCTFCI  - 1

if (jk+1)mod3=0					-- the coded bit is to be mapped on a weak bit of the 8PSK symbol

cpt = cpt + 1							-- increment the counter of swapped bits

if (cptmod2=0)

Swap bit hk with bit hk+80

else

Swap bit hk with bit hNradio−80+k

end if

end if

The value of Nradio is specified in subclause 6.2.7. On GMSK channels J = 116, whereas on 8PSK channels J = 348.

For diagonal interleaving over 40 ms (used on full rate channels), D = 8. The result of the interleaving is then a distribution of the reordered bits over 8 bursts, using the even numbered position of the first 4 bursts and the odd positions of the last 4 bursts.

For diagonal interleaving over 4 bursts (used on half rate channels), D = 4. The result of the interleaving is then a distribution of the reordered bits over 4 bursts, using the even numbered position of the first 2 bursts and the odd positions of the last 2 bursts.

For diagonal interleaving over 60 ms (used on full rate channels), D = 12. The result of the interleaving is then a distribution of the reordered bits over 12 bursts, allocating one third of the bits to each of three consecutive radio packets.

For diagonal interleaving over 60 ms (used on half rate channels), D = 6. The result of the interleaving is then a distribution of the reordered bits over 6 bursts, allocating one third of the bits to each of three consecutive radio packets.

For block rectangular interleaving over 20 ms (used on full rate channels), D = 4. The result of the interleaving is the distribution of the reordered bits over 2D = 8 blocks as in the case of block diagonal interleaving over 8 bursts, and the first 4 blocks are combined with the last 4 blocks: block 0 is combined with block 4, block 1 is combined with block 5, block 2 is combined with block 6, block 3 is combined with block 7.

Block diagonal interleaving over 60 ms shall be used for 8PSK modulation only.

### 6.2.9	Mapping on a Burst

The mapping is given by the rule:

e(B,j)	= i(B,j)	for j = 0,1,…,115 on GMSK channels;

for j = 0,1,…,347 on 8PSK channels.

NOTE:	No stealing flags are used with FLO.

Table 21: Block Code for 5 bits TFCI

| TFCI | Coded TFCI |
| --- | --- |
| 0,0,0,0,0 | 1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1 |
| 0,0,0,0,1 | 1,1,1,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,0,1,0,1,0,1,0 |
| 0,0,0,1,0 | 1,1,1,0,1,0,1,1,0,0,1,1,0,1,0,0,1,1,0,0,1,1,0,0,1,1,0,0,1,0,0,1,1,0,0,1 |
| 0,0,0,1,1 | 1,1,1,0,0,0,0,1,1,0,0,1,1,1,1,0,0,1,1,0,0,1,1,0,0,1,1,0,0,1,0,0,1,1,0,0 |
| 0,0,1,0,0 | 1,1,0,1,1,0,0,0,1,1,1,1,0,1,0,0,0,0,1,1,1,1,0,0,0,0,1,1,1,0,0,0,0,1,1,1 |
| 0,0,1,0,1 | 1,1,0,1,0,0,1,0,0,1,0,1,1,1,1,0,1,0,0,1,0,1,1,0,1,0,0,1,0,1,0,1,0,0,1,0 |
| 0,0,1,1,0 | 1,1,0,0,1,1,0,0,0,0,1,1,1,1,1,1,0,0,0,0,1,1,1,1,0,0,0,0,1,1,1,0,0,0,0,1 |
| 0,0,1,1,1 | 1,1,0,0,0,1,1,0,1,0,0,1,0,1,0,1,1,0,1,0,0,1,0,1,1,0,1,0,0,0,1,1,0,1,0,0 |
| 0,1,0,0,0 | 1,0,1,1,1,0,0,0,0,0,0,0,1,0,1,1,1,1,1,1,1,0,1,1,1,1,1,1,1,0,0,0,0,0,0,0 |
| 0,1,0,0,1 | 1,0,1,1,0,0,1,0,1,0,1,0,0,0,0,1,0,1,0,1,0,0,0,1,0,1,0,1,0,1,0,1,0,1,0,1 |
| 0,1,0,1,0 | 1,0,1,0,1,1,0,0,1,1,0,0,0,0,0,0,1,1,0,0,1,0,0,0,1,1,0,0,1,1,1,0,0,1,1,0 |
| 0,1,0,1,1 | 1,0,1,0,0,1,1,0,0,1,1,0,1,0,1,0,0,1,1,0,0,0,1,0,0,1,1,0,0,0,1,1,0,0,1,1 |
| 0,1,1,0,0 | 1,0,0,1,1,1,1,1,0,0,0,0,0,0,0,0,0,0,1,1,1,0,0,0,0,0,1,1,1,1,1,1,1,0,0,0 |
| 0,1,1,0,1 | 1,0,0,1,0,1,0,1,1,0,1,0,1,0,1,0,1,0,0,1,0,0,1,0,1,0,0,1,0,0,1,0,1,1,0,1 |
| 0,1,1,1,0 | 1,0,0,0,1,0,1,1,1,1,0,0,1,0,1,1,0,0,0,0,1,0,1,1,0,0,0,0,1,0,0,1,1,1,1,0 |
| 0,1,1,1,1 | 1,0,0,0,0,0,0,1,0,1,1,0,0,0,0,1,1,0,1,0,0,0,0,1,1,0,1,0,0,1,0,0,1,0,1,1 |
| 1,0,0,0,0 | 0,1,1,1,1,0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1,0,0,0,0,0,0,0,0,1,1,1,1,1,1,1 |
| 1,0,0,0,1 | 0,1,1,1,0,0,1,0,1,0,1,0,1,1,0,1,0,1,0,1,0,0,1,0,1,0,1,0,1,0,1,0,1,0,1,0 |
| 1,0,0,1,0 | 0,1,1,0,1,1,0,0,1,1,0,0,1,1,0,0,1,1,0,0,1,0,1,1,0,0,1,1,0,0,0,1,1,0,0,1 |
| 1,0,0,1,1 | 0,1,1,0,0,1,1,0,0,1,1,0,0,1,1,0,0,1,1,0,0,0,0,1,1,0,0,1,1,1,0,0,1,1,0,0 |
| 1,0,1,0,0 | 0,1,0,1,1,1,1,1,0,0,0,0,1,1,0,0,0,0,1,1,1,0,1,1,1,1,0,0,0,0,0,0,0,1,1,1 |
| 1,0,1,0,1 | 0,1,0,1,0,1,0,1,1,0,1,0,0,1,1,0,1,0,0,1,0,0,0,1,0,1,1,0,1,1,0,1,0,0,1,0 |
| 1,0,1,1,0 | 0,1,0,0,1,0,1,1,1,1,0,0,0,1,1,1,0,0,0,0,1,0,0,0,1,1,1,1,0,1,1,0,0,0,0,1 |
| 1,0,1,1,1 | 0,1,0,0,0,0,0,1,0,1,1,0,1,1,0,1,1,0,1,0,0,0,1,0,0,1,0,1,1,0,1,1,0,1,0,0 |
| 1,1,0,0,0 | 0,0,1,1,1,1,1,1,1,1,1,1,0,0,1,1,1,1,1,1,1,1,0,0,0,0,0,0,0,0,0,0,0,0,0,0 |
| 1,1,0,0,1 | 0,0,1,1,0,1,0,1,0,1,0,1,1,0,0,1,0,1,0,1,0,1,1,0,1,0,1,0,1,1,0,1,0,1,0,1 |
| 1,1,0,1,0 | 0,0,1,0,1,0,1,1,0,0,1,1,1,0,0,0,1,1,0,0,1,1,1,1,0,0,1,1,0,1,1,0,0,1,1,0 |
| 1,1,0,1,1 | 0,0,1,0,0,0,0,1,1,0,0,1,0,0,1,0,0,1,1,0,0,1,0,1,1,0,0,1,1,0,1,1,0,0,1,1 |
| 1,1,1,0,0 | 0,0,0,1,1,0,0,0,1,1,1,1,1,0,0,0,0,0,1,1,1,1,1,1,1,1,0,0,0,1,1,1,1,0,0,0 |
| 1,1,1,0,1 | 0,0,0,1,0,0,1,0,0,1,0,1,0,0,1,0,1,0,0,1,0,1,0,1,0,1,1,0,1,0,1,0,1,1,0,1 |
| 1,1,1,1,0 | 0,0,0,0,1,1,0,0,0,0,1,1,0,0,1,1,0,0,0,0,1,1,0,0,1,1,1,1,0,0,0,1,1,1,1,0 |
| 1,1,1,1,1 | 0,0,0,0,0,1,1,0,1,0,0,1,1,0,0,1,1,0,1,0,0,1,1,0,0,1,0,1,1,1,0,0,1,0,1,1 |

Table 22: Block Code for 4 bits TFCI

| TFCI | Coded TFCI |
| --- | --- |
| 0,0,0,0 | 1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1 |
| 0,0,0,1 | 1,1,1,1,1,0,1,0,0,1,0,0,0,0,0,1,1,1,1,1,1,1,0,0,0,0,0,0 |
| 0,0,1,0 | 1,1,1,0,0,1,0,1,0,0,1,1,0,0,0,1,1,1,0,0,0,0,1,1,1,1,0,0 |
| 0,0,1,1 | 1,1,1,0,0,0,0,0,1,0,0,0,1,1,1,1,1,1,0,0,0,0,0,0,0,0,1,1 |
| 0,1,0,0 | 1,0,0,1,1,1,0,1,1,0,0,0,1,0,0,1,0,0,1,1,0,0,1,1,0,0,1,1 |
| 0,1,0,1 | 1,0,0,1,0,0,1,1,0,0,1,0,0,1,1,1,0,0,1,1,0,0,0,0,1,1,0,0 |
| 0,1,1,0 | 1,0,0,0,1,0,0,0,1,1,1,1,0,1,0,1,0,0,0,0,1,1,1,1,0,0,0,0 |
| 0,1,1,1 | 1,0,0,0,0,1,1,0,0,1,0,1,1,0,1,1,0,0,0,0,1,1,0,0,1,1,1,1 |
| 1,0,0,0 | 0,1,0,1,1,1,0,0,0,0,0,1,0,1,1,0,1,0,1,0,1,0,1,0,1,0,1,0 |
| 1,0,0,1 | 0,1,0,1,0,0,1,0,1,0,1,1,1,0,0,0,1,0,1,0,1,0,0,1,0,1,0,1 |
| 1,0,1,0 | 0,1,0,0,1,0,0,1,0,1,1,0,1,0,1,0,1,0,0,1,0,1,1,0,1,0,0,1 |
| 1,0,1,1 | 0,1,0,0,0,1,1,1,1,1,0,0,0,1,0,0,1,0,0,1,0,1,0,1,0,1,1,0 |
| 1,1,0,0 | 0,0,1,1,0,1,0,0,1,1,1,0,0,0,1,0,0,1,1,0,0,1,1,0,0,1,1,0 |
| 1,1,0,1 | 0,0,1,1,0,0,0,1,0,1,0,1,1,1,0,0,0,1,1,0,0,1,0,1,1,0,0,1 |
| 1,1,1,0 | 0,0,1,0,1,1,1,0,0,0,1,0,1,1,0,0,0,1,0,1,1,0,1,0,0,1,0,1 |
| 1,1,1,1 | 0,0,1,0,1,0,1,1,1,0,0,1,0,0,1,0,0,1,0,1,1,0,0,1,1,0,1,0 |

Table 23: Block Code for 3 bits TFCI

| TFCI | Coded TFCI |
| --- | --- |
| 0,0,0 | 1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1 |
| 0,0,1 | 1,1,1,0,0,0,0,1,1,1,0,0,0,0,1,1,1,0,0,0,0,1,1,1 |
| 0,1,0 | 1,0,0,1,1,0,0,1,0,0,1,1,0,0,1,0,0,1,1,0,0,1,0,0 |
| 0,1,1 | 1,0,0,0,0,1,1,1,0,0,0,0,1,1,1,0,0,0,0,1,1,1,0,0 |
| 1,0,0 | 0,1,0,1,0,1,0,0,1,0,1,0,1,0,0,1,0,1,0,1,0,0,1,0 |
| 1,0,1 | 0,1,0,0,1,0,1,0,1,0,0,1,0,1,0,1,0,0,1,0,1,0,1,0 |
| 1,1,0 | 0,0,1,1,0,0,1,0,0,1,1,0,0,1,0,0,1,1,0,0,1,0,0,1 |
| 1,1,1 | 0,0,1,0,1,1,0,0,0,1,0,1,1,0,0,0,1,0,1,1,0,0,0,1 |

Table 24: Block Code for 2 bits TFCI

| TFCI | Coded TFCI |
| --- | --- |
| 0,0 | 1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1 |
| 0,1 | 1,0,0,1,0,0,1,0,0,1,0,0,1,0,0,1 |
| 1,0 | 0,1,0,0,1,0,0,1,0,0,1,0,0,1,0,0 |
| 1,1 | 0,0,1,0,0,1,0,0,1,0,0,1,0,0,1,0 |

Table 25: Block Code for 1 bit TFCI

| TFCI | Coded TFCI |
| --- | --- |
| 0 | 1,1,1,1,1,1,1,1 |
| 1 | 0,0,0,0,0,0,0,0 |

### 6.2.10	Signalling on Half Rate Channels

When a signalling transport block is sent on half rate channels, the value of the redundancy pattern index (R) to be used in rate matching is given by the TDMA frame number (see 3GPP TS 45.002) of the first burst carrying coded bits of the corresponding radio packet, and table 26 below.

NOTE:	A radio packet containing a signalling transport block is indicated by TFCI=0.

Table 26: R and TDMA frame number modulo 26

| TDMA frame number | R |
| --- | --- |
| 0, 1, 2, 3 | 0 |
| 4, 5, 6, 7 | 1 |
| 8, 9, 10, 11 | 0 |
| 13, 14, 15, 16 | 1 |
| 17, 18, 19, 20 | 0 |
| 21, 22, 23, 24 | 1 |

NOTE:	For a given signalling transport block, this could result in R=1 being used in rate matching during the first transmission and R=0 during the subsequent retransmission.

Annex A (informative):
Summary of Channel Types

TCH/EFS:			enhanced full rate speech traffic channel

TCH/FS:			full rate speech traffic channel

TCH/HS:			half rate speech traffic channel

TCH/AFS:			adaptive multirate full rate speech traffic channel

TCH/AFS12.2			adaptive multirate full rate speech, 12.2 kbit/s

TCH/AFS10.2			adaptive multirate full rate speech, 10.2 kbit/s

TCH/AFS7.95			adaptive multirate full rate speech, 7.95 kbit/s

TCH/AFS7.4			adaptive multirate full rate speech, 7.5 kbit/s

TCH/AFS6.7			adaptive multirate full rate speech, 6.7 kbit/s

TCH/AFS5.9			adaptive multirate full rate speech, 5.9 kbit/s

TCH/AFS5.15			adaptive multirate full rate speech, 5.15 kbit/s

TCH/AFS4.75			adaptive multirate full rate speech, 4.75 kbit/s

TCH/AHS:			adaptive multirate half rate speech traffic channel

TCH/AHS7.95			adaptive multirate half rate speech, 7.95 kbit/s

TCH/AHS7.4			adaptive multirate half rate speech, 7.5 kbit/s

TCH/AHS6.7			adaptive multirate half rate speech, 6.7 kbit/s

TCH/AHS5.9			adaptive multirate half rate speech, 5.9 kbit/s

TCH/AHS5.15			adaptive multirate half rate speech, 5.15 kbit/s

TCH/AHS4.75			adaptive multirate half rate speech, 4.75 kbit/s

O-TCH/AHS:			adaptive multirate half rate 8PSK speech traffic channel

O-TCH/AHS12.2			adaptive multirate half rate 8PSK speech, 12.2 kbit/s

O-TCH/AHS10.2			adaptive multirate half rate 8PSK speech, 10.2 kbit/s

O-TCH/AHS7.95			adaptive multirate half rate 8PSK speech, 7.95 kbit/s

O-TCH/AHS7.4			adaptive multirate half rate 8PSK speech, 7.5 kbit/s

O-TCH/AHS6.7			adaptive multirate half rate 8PSK speech, 6.7 kbit/s

O-TCH/AHS5.9			adaptive multirate half rate 8PSK speech, 5.9 kbit/s

O-TCH/AHS5.15			adaptive multirate half rate 8PSK speech, 5.15 kbit/s

O-TCH/AHS4.75			adaptive multirate half rate 8PSK speech, 4.75 kbit/s

TCH/WFS			wideband adaptive multirate full rate speech traffic channels

TCH/WFS12.65			wideband adaptive multirate full rate speech, 12.65 kbit/s

TCH/WFS8.85			wideband adaptive multirate full rate speech, 8.85 kbit/s

TCH/WFS6.60			wideband adaptive multirate full rate speech, 6.60 kbit/s

O-TCH/WFS:			adaptive multirate full rate 8PSK wideband speech traffic channel

O-TCH/WFS23.85	adaptive multirate full rate 8PSK wideband speech, 23.85 kbit/s

O-TCH/WFS15.85	adaptive multirate full rate 8PSK wideband speech, 15.85 kbit/s

O-TCH/WFS12.65	adaptive multirate full rate 8PSK wideband speech, 12.65 kbit/s

O-TCH/WFS8.85	adaptive multirate full rate 8PSK wideband speech, 8.85 kbit/s

O-TCH/WFS6.6			adaptive multirate full rate 8PSK wideband speech, 6.6 kbit/s

O-TCH/WHS:			adaptive multirate half rate 8PSK wideband speech traffic channel

O-TCH/WHS12.65	adaptive multirate half rate 8PSK wideband speech, 12.65 kbit/s

O-TCH/WHS8.85	adaptive multirate half rate 8PSK wideband speech, 8.85 kbit/s

O-TCH/WHS6.6			adaptive multirate half rate 8PSK wideband speech, 6.6 kbit/s

E-TCH/F43.2:			43.2 kbit/s full rate data traffic channel

E-TCH/F32.0:			32.0 kbit/s full rate data traffic channel

E-TCH/F28.8:			28.8 kbit/s full rate data traffic channel

TCH/F14.4			14.4 kbit/s full rate data traffic channel

TCH/F9.6:			9.6 kbit/s full rate data traffic channel

TCH/F4.8:			4.8 kbit/s full rate data traffic channel

TCH/H4.8:			4.8 kbit/s half rate data traffic channel

TCH/F2.4:			2.4 kbit/s full rate data traffic channel

TCH/H2.4:			2.4 kbit/s half rate data traffic channel

SACCH:			slow associated control channel

FACCH/F:			fast associated control channel at full rate

FACCH/H:			fast associated control channel at half rate

E-FACCH/F:			enhanced circuit switched fast associated control channel at full rate

O-FACCH/H			octal fast associated control channel at half rate

EPCCH:			Enhanced power control channel

SDCCH:			stand-alone dedicated control channel

BCCH:			broadcast control channel

PCH:			paging channel

AGCH			access grant channel

RACH:			random access channel

SCH:			synchronization channel

CBCH:			cell broadcast channel

CTSBCH-SB:			CTS beacon channel (synchronisation burst)

CTSPCH:			CTS paging channel

CTSARCH:			CTS access request channel

CTSAGCH:			CTS access grant channel

PDTCH			packet data traffic channel

PACCH			packet associated control channel

PBCCH			packet broadcast control channel

PAGCH			packet access grant channel

PPCH			packet paging channel

PTCCH			packet timing advance control channel

PRACH			packet random access channel

CFCCH			Compact Frequency Correction Channel

CPAGCH			Compact Packet Access Grant Channel

CPBCCH			Compact Packet Broadcast Control Channel

CPCCCH			Compact Packet Common Control Channel

CPPCH			Compact Packet Paging Channel

CPRACH			Compact Packet Random Access Channel

CSCH			Compact Synchronization Channel

MPRACH			MBMS Packet Random Access Channel

EC-SCH			Extended Coverage synchronization channel

EC-BCCH			Extended Coverage broadcast control channel

EC-PCH			Extended Coverage paging channel

EC-AGCH			Extended Coverage access grant channel

EC-RACH			Extended Coverage random access channel

EC-PDTCH			Extended Coverage packet data traffic channel

EC-PACCH			Extended Coverage packet associated control channel

Annex B (informative):
Summary of Polynomials Used for Convolutional Codes and Turbo Codes

G0 = 1+ D3 + D4	TCH/FS, TCH/EFS, TCH/AFS, TCH/WFS, TCH/AHS, TCH/F14.4, TCH/F9.6, TCH/H4.8, SDCCH, BCCH, PCH, SACCH, FACCH, E-FACCH, AGCH, RACH, SCH, CSCH, EC-SCH, CTSBCH-SB, CTSPCH, CTSARCH, CTSAGCH, PDTCH (CS-1, CS-2, CS3, CS-4), PACCH,PBCCH, PAGCH, PPCH, PTCCH, PRACH, CPBCCH, CPAGCH, CPPCH, MPRACH, EC-RACH (M=1, 4, 16, 48), EC-BCCH

G1 = 1 + D + D3 + D4	TCH/FS, TCH/EFS, TCH/AFS, TCH/WFS, TCH/AHS, TCH/F14.4, TCH/F9.6, TCH/H4.8, SACCH, FACCH, E-FACCH, SDCCH, BCCH,PCH, AGCH, RACH, SCH, CSCH, EC-SCH, TCH/F4.8, TCH/F2.4, TCH/H2.4,PDTCH(CS-1, CS-2, CS-3, CS-4), PACCH, PBCCH, PAGCH, PPCH, PTCCH, PRACH, CPBCCH, CPAGCH, CPPCH, MPRACH, EC-RACH (M=1, 4, 16, 48), EC-BCCH

G2 = 1 + D2 + D4	TCH/AFS, TCH/WFS, TCH/F4.8, TCH/F2.4, TCH/H2.4

G3 = 1 + D + D2 + D3 + D4	TCH/AFS, TCH/WFS, TCH/F4.8, TCH/F2.4, TCH/H2.4

G4 = 1 + D2 + D3 + D5 + D6	TCH/HS, TCH/AFS, TCH/AHS, O-TCH/AHS, O-TCH/WFS, O-TCH/WHS, E-TCH/F43.2, E-TCH/F32.0, E-TCH/F28.8, PDTCH(MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, MCS-7, MCS-8, MCS-9, UAS-7, UAS-8, UAS-9, UAS-10, UAS-11, UBS-5, UBS-6, UBS-7, UBS-8, UBS-9, UBS-10, UBS-11, UBS-12), SACCH/TP, O-FACCH/H, O-FACCH/F, EC-CCCH/D, EC-PACCH, EC-RACH (M=66)

G5 = 1 + D + D4 + D6	TCH/HS, TCH/AFS, TCH/AHS, O-TCH/AHS, O-TCH/WFS, O-TCH/WHS, E-TCH/F32.0, PDTCH(MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, MCS-7, MCS-8, MCS-9, UAS-7, UAS-8, UAS-9, UAS-10, UAS-11, UBS-5, UBS-6, UBS-7, UBS-8, UBS-9, UBS-10, UBS-11, UBS-12), O-FACCH/H, O-FACCH/F, EC-CCCH/D, EC-PACCH, EC-RACH (M=66)

G6 = 1 + D + D2 + D3 + D4 + D6	TCH/HS, TCH/AFS, TCH/AHS, O-TCH/AHS, O-TCH/WFS, O-TCH/WHS, O-FACCH/H, O-FACCH/F, EC-RACH (M=66)

G7= 1 + D + D2 + D3 + D6	O-TCH/AHS, O-TCH/WFS, O-TCH/WHS, E-TCH/F43.2, E-TCH/F32.0, E-TCH/F28.8, PDTCH(MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, MCS-7, MCS-8, MCS-9, UAS-7, UAS-8, UAS-9, UAS-10, UAS-11, UBS-5, UBS-6, UBS-7, UBS-8, UBS-9, UBS-10, UBS-11, UBS-12), SACCH/TP, O-FACCH/H, O-FACCH/F, EC-CCCH/D, EC-PACCH, EC-RACH (M=66)

G8 = 1 + D2 + D3	PDTCH(DAS-5, DAS-6, DAS-7, DAS-8, DAS-9, DAS-10, DAS-11, DAS-12, DBS-5, DBS-6, DBS-7, DBS-8, DBS-9, DBS-10, DBS-11, DBS-12)

G9 = 1 + D + D3	PDTCH(DAS-5, DAS-6, DAS-7, DAS-8, DAS-9, DAS-10, DAS-11, DAS-12, DBS-5, DBS-6, DBS-7, DBS-8, DBS-9, DBS-10, DBS-11, DBS-12)

Annex C (informative):
Change history

| SPEC | SMG# | CR | PHASE | VERS | NEW_VERS | SUBJECT |
| --- | --- | --- | --- | --- | --- | --- |
| 05.03 | s25 | A015 | R97 | 6.0.0 | 6.1.0 | 14.4kbps Data Service |
| 05.03 | s27 |  | R97 | 6.1.0 | 6.1.2 | Change of status to EN |
| 05.03 | s28 | A017 | R97 | 6.1.2 | 6.2.0 | Clarification on the definition of USF precoding |
| 05.03 | s28 | A016 | R98 | 6.2.0 | 7.0.0 | Introduction of CTS in 05.03 |
| 05.03 | s28 |  | R98 | 7.0.0 | 7.0.1 | Correction to Figure 1 |
| 05.03 | s29 | A021 | R98 | 7.0.1 | 7.1.0 | Introduction of AMR |
| 05.03 | s29 | A022 | R99 | 7.1.0 | 8.0.0 | Introduction of ECSD/EDGE |
| 05.03 | s30 | A023 | R99 | 8.0.0 | 8.1.0 | Introduction of Fast power Control for ECSD in 05.03 |
| 05.03 | s30 | A025 | R99 | 8.0.0 | 8.1.0 | EGPRS Channel Coding |
| 05.03 | s30 | A026 | R99 | 8.0.0 | 8.1.0 | AMR Channel Coding |
| 05.03 | s30 | A027 | R99 | 8.0.0 | 8.1.0 | EDGE Compact logical channels |
| 05.03 | s30 | A029 | R99 | 8.0.0 | 8.1.0 | Correction of several small bugs in the AMR section / Optimization of the transmission of the in-band parameter Mode Indication |
| 05.03 | s30 | A030 | R99 | 8.0.0 | 8.1.0 | E-FACCH/F interleaving |
| 05.03 | s30 | A032 | R99 | 8.0.0 | 8.1.0 | Introduction of RATSCCH for AMR |
| 05.03 | s30b | A033 | R99 | 8.1.0 | 8.2.0 | Correction of EGPRS channel coding |
| 05.03 | s31 | A035 | R99 | 8.2.0 | 8.3.0 | Correction concerning SID_FIRST and clarification concerning bit order of codec mode code words |
| 05.03 | s31 | A036 | R99 | 8.2.0 | 8.3.0 | Editorial correction for ECSD channel coding |
| 05.03 | s31 | A037 | R99 | 8.2.0 | 8.3.0 | Correction for EGPRS Channel Coding |
| 05.03 | S31b | A039 | R99 | 8.3.0 | 8.4.0 | Fast inband signalling: E-IACCH |
| 05.03 | S32 | A040 | R99 | 8.4.0 | 8.5.0 | Clarification of stealing bits for MCS-1 to 4 |
| 05.03 | S32 | A041 | R99 | 8.4.0 | 8.5.0 | Correction to the interleaving formula of MCS-8 case |
|  |  |  |  |  |  | GERAN#2 November 2000 |
| 05.03 | G02 | A043 | R99 | 8.5.0 | 8.6.0 | Correction of errors in coding schemes |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2001-01 | 03 | GP-010261 | A045 |  | CR 05.03-A045 Editorial Correction to SACCH Block Coding | 8.6.0 | 4.0.0 |
| 2001-01 | 03 | GP-010242 | A046 |  | CR 05.03-A046 Channel coding for TCH/WFS | 4.0.0 | 5.0.0 |
| 2001-06 | 05 | GP-011412 | 004 | 1 | Introduction of EPC and SACCH/TP | 5.0.0 | 5.1.0 |
| 2001-06 | 05 | GP-011264 | 005 |  | Channel coding of AMR-NB codec on O-TCH/H | 5.0.0 | 5.1.0 |
| 2001-08 | 06 | GP-011919 | 006 | 1 | Editorial changes due to the introduction of O-TCH/AHS | 5.1.0 | 5.2.0 |
| 2001-08 | 06 | GP-011778 | 007 |  | Channel coding for O-FACCH/H | 5.1.0 | 5.2.0 |
| 2001-08 | 06 | GP-011779 | 008 |  | AMR signaling frames for O-TCH/AHS | 5.1.0 | 5.2.0 |
| 2001-11 | 07 | GP-012771 | 010 | 1 | Correction of references to relevant 3GPP TSs | 5.2.0 | 5.3.0 |
| 2001-11 | 07 | GP-012650 | 012 | 1 | Update of channel coding and interleaving organization | 5.2.0 | 5.3.0 |
| 2001-11 | 07 | GP-012758 | 014 |  | Correction of interleaving index | 5.2.0 | 5.3.0 |
| 2002-02 | 08 | GP-020055 | 015 |  | Correction to channel coding for TCH/WFS | 5.3.0 | 5.4.0 |
| 2002-04 | 09 | GP-021203 | 018 | 1 | Cleaning & Updates | 5.4.0 | 5.5.0 |
| 2002-04 | 09 | GP-021169 | 020 | 1 | Alignment of number of codecs for WB-AMR to proposed set | 5.4.0 | 5.5.0 |
| 2002-06 | 10 | GP-022025 | 016 | 3 | Channel coding for AMR-WB on O-TCH | 5.5.0 | 5.6.0 |
| 2002-06 | 10 | GP-021435 | 021 |  | Corrections and clean up | 5.5.0 | 5.6.0 |
| 2002-06 | 10 | GP-021761 | 022 |  | Update of channel coding and interleaving organisation for AMR-WB 8-PSK | 5.5.0 | 5.6.0 |
| 2003-04 | 14 | GP-030758 | 024 |  | Padding for MCS-8 Retransmissions | 5.6.0 | 5.7.0 |
| 2003-06 | 15 | GP-031426 | 025 |  | Correction of SACCH/TP | 5.7.0 | 5.8.0 |
| 2003-06 | 15 | GP-031543 | 027 |  | Correction of reordering of bits for O-FACCH/H | 5.7.0 | 5.8.0 |
| 2003-08 | 16 | GP-031995 | 028 |  | Correction of parameters for TCH/FS | 5.8.0 | 6.0.0 |
| 2003-11 | 17 | GP-032767 | 026 | 3 | Coding/Multiplexing unit for the Flexible Layer One | 6.0.0 | 6.1.0 |
| 2003-11 | 17 | GP-032458 | 029 |  | Figure 1e | 6.0.0 | 6.1.0 |
| 2003-11 | 17 | GP-032616 | 030 |  | 11 information bits access burst on RACH | 6.0.0 | 6.1.0 |
| 2004-02 | 18 | GP-040167 | 031 |  | Correction of encoded in-band data bits | 6.1.0 | 6.2.0 |
| 2004-04 | 19 | GP-041165 | 033 | 1 | One TFC for signalling on HR channels | 6.2.0 | 6.3.0 |
| 2004-06 | 20 | GP-041666 | 034 | 1 | Signalling for Uplink TFC selection for FLO | 6.3.0 | 6.4.0 |
| 2004-06 | 20 | GP-041369 | 035 |  | Corrections for FLO | 6.3.0 | 6.4.0 |
| 2004-06 | 20 | GP-041554 | 036 |  | Small editorial correction to F.32 Channel Coding for ECSD | 6.3.0 | 6.4.0 |
| 2004-08 | 21 | GP-041935 | 037 |  | Addition of RATSCCH for TCH/WFS | 6.4.0 | 6.5.0 |
| 2004-11 | 22 | GP-042471 | 042 |  | Introduction of MBMS | 6.5.0 | 6.6.0 |
| 2004-11 | 22 | GP-042824 | 045 | 1 | FLO-compatible quick fix for VT over GERAN | 6.5.0 | 6.6.0 |
| 2004-11 | 22 | GP-042786 | 046 |  | Removal of PTM-M | 6.5.0 | 6.6.0 |
| 2005-01 | 23 | GP-050485 | 0047 | 1 | Inclusion of 60ms interleaving for FLO | 6.6.0 | 6.7.0 |
| 2005-01 | 23 | GP-050040 | 0050 |  | Correction to E-FACCH/F for E-TCH/F32.0 | 6.6.0 | 6.7.0 |
| 2005-01 | 23 | GP-050490 | 0053 |  | Interleaving for E-TCH/F32.0 | 6.6.0 | 6.7.0 |
| 2005-09 | 26 | GP-051984 | 0055 |  | Correction to stealing flags for SACCH/TP | 6.7.0 | 6.8.0 |
| 2006-01 | 28 | GP-060014 | 0060 |  | Correction to the text in SACCH/TP Convolutional code | 6.8.0 | 6.9.0 |
| 2006-04 | 29 | GP-060922 | 0063 | 1 | Correction of confusing text | 6.9.0 | 7.0.0 |
| 2007-02 | 33 | GP-070366 | 0067 | 1 | Correction to the channel coding of the synchronization channel | 7.0.0 | 7.1.0 |
| 2007-08 | 35 | GP-071549 | 0068 | 4 | Introduction of Reduced TTI | 7.1.0 | 7.2.0 |
| 2007-08 | 35 | GP-071500 | 0069 | 2 | Introduction of Fast Ack/Nack Reporting | 7.1.0 | 7.2.0 |
| 2007-08 | 35 | GP-071543 | 0070 | 2 | Introduction of channel coding for RED HOT and HUGE | 7.1.0 | 7.2.0 |
| 2007-11 | 36 | GP-071966 | 0072 | 1 | FANR instead of RL and miscellaneous corrections on Reduced Latency | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071676 | 0073 |  | Deletion of RL-EGPRS in EGPRS2 | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071689 | 0074 |  | Puncturing patterns for EGPRS PAN | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071691 | 0075 |  | Correction to stealing flag sequences for RTTI configurations | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071963 | 0076 | 1 | Corrections to PAN | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071858 | 0079 | 1 | Puncturing patterns for EGPRS2 PAN | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071958 | 0080 | 1 | Channel coding for EGPRS2 | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-072029 | 0081 | 2 | Introduction of EGPRS-2 (RED HOT rate matching) | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071745 | 0082 |  | USF coding for EGPRS2 | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071964 | 0084 | 1 | Bit swapping for EGPRS PAN | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071955 | 0085 |  | Channel coding for MCS-0 | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071953 | 0086 |  | Bit swapping for RED HOT A PAN | 7.2.0 | 7.3.0 |
| 2007-11 | 36 | GP-071974 | 0087 |  | Puncturing for UBS-12 | 7.2.0 | 7.3.0 |
| 2008-02 | 37 | GP-080362 | 0088 | 1 | LATRED and EGPRS2 | 7.3.0 | 7.4.0 |
| 2008-02 | 37 | GP-080115 | 0089 |  | Corrections for LATRED and HUGE | 7.3.0 | 7.4.0 |
| 2008-02 | 37 | GP-080128 | 0090 |  | Corrections for LATRED and EGPRS2 | 7.3.0 | 7.4.0 |
| 2008-02 | 37 | GP-080133 | 0091 |  | Modified puncturing for UBS-12 | 7.3.0 | 7.4.0 |
| 2008-02 | 37 | GP-080135 | 0092 |  | Modified USF coding for EGPRS2-B | 7.3.0 | 7.4.0 |
| 2008-02 | 37 | GP-080175 | 0094 |  | Corrections in EGPRS2 (RED HOT rate matching) | 7.3.0 | 7.4.0 |
| 2008-05 | 38 | GP-080665 | 0096 |  | Correction to burst mapping for HUGE A | 7.4.0 | 7.5.0 |
| 2008-05 | 38 | GP-080760 | 0097 | 1 | Miscellaneous corrections to EGPRS2 | 7.4.0 | 7.5.0 |
| 2008-08 | 39 | GP-081314 | 0102 | 1 | Clarification to EGPRS2 | 7.5.0 | 7.6.0 |
| 2008-08 | 39 | GP-081124 | 0103 |  | Correction to MCS-8 PAN | 7.5.0 | 7.6.0 |
| 2008-08 | 39 | GP-081125 | 0104 |  | Correction to PAN bit swapping for MCS-5/6 | 7.5.0 | 7.6.0 |
| 2008-08 | 39 | GP-081313 | 0105 | 1 | EGPRS2 corrections | 7.5.0 | 7.6.0 |
| 2008-11 | 40 | GP-081586 | 0099 | 2 | Mixed modulation USF | 7.6.0 | 7.7.0 |
| 2008-11 | 40 | GP-081462 | 0108 |  | Correction linked to the mapping on a burst for UBS-10 to UBS-12 | 7.6.0 | 7.7.0 |
| 2008-11 | 40 | GP-081483 | 0109 |  | Correction to speech channel at full rate | 7.6.0 | 7.7.0 |
| 2008-11 | 40 | GP-081599 | 0110 |  | Addition of PAN bit swapping for MCS-5-6 UL | 7.6.0 | 7.7.0 |
| 2008-12 | 40 |  |  |  | Version for Release 8 | 7.7.0 | 8.0.0 |
| 2009-02 | 41 | GP-090080 | 0112 |  | Corrections of RED HOT rate matching | 8.0.0 | 8.1.0 |
| 2009-02 | 41 | GP-090156 | 0114 |  | Correction to DBS-10, DBS-11 and DBS-12 mapping on a burst | 8.0.0 | 8.1.0 |
| 2009-02 | 41 | GP-090413 | 0115 | 1 | Correction to DAS11, DAS12, DBS9, and DBS10 punctured block count and DBS10 RTTI USF bit count | 8.0.0 | 8.1.0 |
| 2009-02 | 41 | GP-090415 | 0116 | 1 | PAN bit swapping order clarification | 8.0.0 | 8.1.0 |
| 2009-05 | 42 | GP-090988 | 0118 | 2 | New USF code words for EGPRS2-A | 8.1.0 | 8.2.0 |
| 2009-05 | 42 | GP-090661 | 0122 |  | Clarification on puncturing loop parameters calculation | 8.1.0 | 8.2.0 |
| 2009-05 | 42 | GP-090663 | 0124 |  | Miscellaneous corrections for EGPRS2 | 8.1.0 | 8.2.0 |
| 2009-05 | 42 | GP-090955 | 0125 | 1 | Corrections to EGPRS2 DL puncturing and UBS-6 coding | 8.1.0 | 8.2.0 |
| 2009-09 | 43 | GP-091138 | 0128 |  | Various minor corrections on EGPRS2 | 8.2.0 | 8.3.0 |
| 2009-09 | 43 | GP-091143 | 0130 |  | Correction of UBS-12 puncturing with PAN included | 8.2.0 | 8.3.0 |
| 2009-12 | 44 |  |  |  | Version for Release 9 | 8.3.0 | 9.0.0 |
| 2011-03 | 49 |  |  |  | Version for Release 10 | 9.0.0 | 10.0.0 |
| 2012-09 | 55 |  |  |  | Version for Release 11 | 10.0.0 | 11.0.0 |
| 2012-11 | 56 | GP-121314 | 0131 |  | TC RT: Uplink Reply Procedure | 11.0.0 | 11.1.0 |
| 2013-11 | 60 | GP-131125 | 0132 | 4 | Extended TFI Addressing space for DLMC | 11.1.0 | 12.0.0 |
| 2014-05 | 62 | GP-140347 | 0133 | 2 | DLMC – Introduction of CS-3 for PDAN | 12.0.0 | 12.1.0 |
| 2014-05 | 62 | GP-140388 | 0134 | 1 | DLMC - Correction of eTFI usage in PACCH | 12.0.0 | 12.1.0 |
| 2015-11 | 68 | GP-151180 | 0135 | 2 | Introduction of EC-EGPRS | 12.1.0 | 13.0.0 |
| 2016-02 | 69 | GP-160209 | 0136 | 3 | Corrections for EC-EGPRS | 13.0.0 | 13.1.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-05 | RP-70 | GP-160467 | 0139 | 3 | B | Introduction of Radio Frequency Colour Code, clarifications and miscellaneous corrections to EC-GSM-IoT (including name change) | 13.2.0 |
| 2016-09 | RP-73 | RP-161392 | 0141 | 2 | F | Miscellaneous corrections for EC-GSM-IoT | 13.3.0 |
| 2016-12 | RP-74 | RP-162066 | 0142 | - | F | Introduction of Alternative Mappings for Higher Coverage Classes with 2 PDCHs | 14.0.0 |
| 2017-03 | RP-75 | RP-170061 | 0143 | 1 | B | Introduction of compact burst mapping | 14.1.0 |
| 2017-03 | RP-75 | RP-170060 | 0144 | 2 | B | Introduction of Extended Access Burst for connectionless Multilateration Positioning | 14.1.0 |
| 2017-06 | RP-76 | RP-170924 | 0145 | 5 | B | Introduction of uplink coverage class CC5 for UL MCL improvement | 14.2.0 |
| 2017-06 | RP-76 | RP-170928 | 0148 | 1 | A | Correction of EC-PACCH mapping onto PDCHs | 14.2.0 |
| 2018-06 | RP-80 |  |  |  |  | Version for Release 15 | 15.0.0 |
| 2018-09 | RP-81 | RP-181593 | 0149 | - | B | Introduction of EC-PICH | 15.1.0 |
| 2019-03 | RP-83 | RP-190061 | 0152 | - | A | Correction to EC-SCH cyclic shift operation | 15.2.0 |
| 2020-07 | RP-88e | - | - | - | - | Upgrade to Rel-16 version without technical change | 16.0.0 |